Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM Protocol Stack

Multilayer Test Specification

AT Command Interpreter

(Edit via File/Properties/Summary!)

| Document Number: | xxxx.xxx.xx.xxx (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1997-Dec-17 |
| Last changed: | 2003-May-12 by XGUTTEFE |
| File Name: | ati.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1997-Dec-17 | LE, MS |  | 0.1 |  | 1 |
| 2003-May-12 | XGUTTEFE |  | 0.2 | Draft |  |

Notes:

- Initial version

Table of Contents

1.1	References	4

1.2	Abbreviations	7

1.3	Terms	9

2	Overview	10

3	Parameters	12

4	TEST CASES	58

4.1	Preambles	58

4.1.1	ATI001: Power On	58

4.1.2	ATI010: Request Subscriber Number	59

4.1.3	ATI011: Request Subscriber Number	59

4.1.4	ATI110: Storing of SMS Profile	59

4.1.5	ATI111: Changing of SMS Profile	59

4.1.6	ATI112: Restoring of SMS Profile	60

4.1.7	ATI113: Selecting ME Memory	61

4.1.8	ATI114: Write a Short Message to ME Memory	61

4.1.9	ATI020: Request Preferred PLMN List	61

4.1.10	ATI030: Read SIM binary field	62

4.1.11	ATI310: Testing of AT Command Extension Mechanism	62

4.1.12	ATI320: Testing of AT Command Extension Mechanism	63

Appendices	64

A.	Acronyms	64

B.	Glossary	64

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH

[50]	Service Access Point PH
	6147.112.97.100; Condat GmbH

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH

[58]	Users Guide
	6147.300.96.100; Condat GmbH

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

Error! Objects cannot be created from editing field codes.

Figure 1: Mobile-station protocol architecture

This document describes the tests for the whole protocol stack according to GSM 11.10 chapter 26.8.

# Parameters

#define REST_OCTET 0x2b

/*------------------------------------------------------------------*\

| GSM 11.10

| 10 Generic call set up procedure

| 10.1 Generic call setup-up procedure for mobile terminating speech calls

| 10.1.2 Definition of system information messages

\*------------------------------------------------------------------*/

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

/*---------------------------------------------------------------*\

| BCCH Frequency list:

| Indicates seven surrounding cells on any ARFCN of the supported

| band, excluding ARFCNs in or immediately adjacent to those

| specified in section 6.2 (GSM 11.10).

| From GSM 11.10, section6.2 the following ARFCN are given :

|     10, 14, 17, 18, 22, 24, 26, 30, 31, 34, 38, 42, 45, 46, 50,

|     52, 54, 58, 59, 62, 66, 70, 73, 74, 78, 80, 82, 86, 87, 90,

|     94, 98, 101, 102, 106, 108, 110, 114

| The following 7 cells are chosen :

|              121,117,       76,      48,   12,7,1

| Thus BA is : 01100000  00000800 00008000 00000841

\*---------------------------------------------------------------*/

IE_BEGIN(bcch_frequency_list)

BF(32,0x01100000,ACT_CHECK,ANONYMOUS,"bit 128 thru  97")

BF(32,0x00000800,ACT_CHECK,ANONYMOUS,"bit  96 thru  65")

BF(32,0x00008000,ACT_CHECK,ANONYMOUS,"bit  64 thru  33")

BF(32,0x00000841,ACT_CHECK,ANONYMOUS,"bit  32 thru   1")

IE_END(bcch_frequency_list)

IE_BEGIN(cell_channel_description)

BF(32,0x00000000,ACT_CHECK,ANONYMOUS,"Includes the       ")

BF(32,0x00000000,ACT_CHECK,ANONYMOUS,"hopping sequence   ")

BF(32,0x00000000,ACT_CHECK,ANONYMOUS,"ARFCNs, if hopping ")

BF(32,0x00000000,ACT_CHECK,ANONYMOUS,"is used.           ")

IE_END(cell_channel_description)

IE_BEGIN(cell_identity)

BF(16,0x0001,ACT_CHECK,ANONYMOUS,"CI VALUE 0001 hex (not relevant)")

IE_END(cell_identity)

IE_BEGIN(cell_identity_B)

BF(16,0x0002,ACT_CHECK,ANONYMOUS,"CI VALUE 0002 hex (not relevant)")

IE_END(cell_identity_B)

IE_BEGIN(cell_options)

BF(1,0,ACT_CHECK,ANONYMOUS              ,"spare                ")

BF(1,0,ACT_CHECK,pwrc                ,"power control not set")

BF(2,2,ACT_CHECK,dtx                 ,"MS must not use DTX  ")

BF(4,1,ACT_CHECK,radio_link_time_out ,"8                    ")

IE_END(cell_options)

IE_BEGIN(cell_selection_parameter)

BF(3,  0,ACT_CHECK,cell_reselect_hysteresis,"0 dB")

BF(5,  0,ACT_CHECK,ms_txpwr_max_cch        ,"Max. output power of MS")

BF(1,  0,ACT_CHECK,acs                     ,"no additional cell params")

BF(1,  0,ACT_CHECK,neci                    ,"New est. cause not supp.")

BF(6,-90+111,ACT_CHECK,rxlev_access_min    ,"-90 dBm")

IE_END(cell_selection_parameter)

IE_BEGIN(control_channel_description)

BF(1,0,ACT_CHECK,ANONYMOUS        ,"spare                             ")

BF(1,0,ACT_CHECK,att           ,"MS shall not apply (not relevant) ")

BF(3,0,ACT_CHECK,bs_ag_blks_res,"0 blocks reserved (not relevant)  ")

BF(3,1,ACT_CHECK,ccch_conf     ,"Combined CCCH/SDCCH (not relevant)")

BF(5,0,ACT_CHECK,ANONYMOUS        ,"spare                             ")

BF(3,3,ACT_CHECK,bs_pa_mfrms   ,"5 multiframes (not relevant)      ")

BF(8,0,ACT_CHECK,t3212         ,"Infinite                          ")

IE_END(control_channel_description)

IE_BEGIN(l2_pseudo_length_12)

BF(6,12,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_12)

IE_BEGIN(l2_pseudo_length_18)

BF(6,18,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_18)

IE_BEGIN(l2_pseudo_length_21)

BF(6,21,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_21)

IE_BEGIN(l2_pseudo_length_22)

BF(6,22,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_22)

#define  MCC   0x262    /* 262  decimal (not relevant)   */

#define  MNC   1        /* 01   decimal (not relevant)   */

#define  LAC   0x0001   /* 0001 hex (not relevant)       */

#define  LAC_B 0x0002   /* 0002 hex (not relevant)       */

IE_BEGIN(location_area_identification)

BF( 4,6     ,ACT_CHECK,mcc_dig_2,"digit 2 of mobile country code")

BF( 4,2     ,ACT_CHECK,mcc_dig_1,"digit 1 of mobile country code")

BF( 4,   0xF,ACT_CHECK,ANONYMOUS   ,"end of MCC                    ")

BF( 4,2     ,ACT_CHECK,mcc_dig_3,"digit 3 of mobile country code")

BF( 4,1     ,ACT_CHECK,mnc_dig_2,"digit 2 of mobile network code")

BF( 4,0     ,ACT_CHECK,mnc_dig_1,"digit 1 of mobile network code")

BF(16,   LAC,ACT_CHECK,lac      ,"Location area code            ")

IE_END(location_area_identification)

IE_BEGIN(location_area_identification_B)

BF( 4,6     ,ACT_CHECK,mcc_dig_2,"digit 2 of mobile country code")

BF( 4,2     ,ACT_CHECK,mcc_dig_1,"digit 1 of mobile country code")

BF( 4,   0xF,ACT_CHECK,ANONYMOUS   ,"end of MCC                    ")

BF( 4,2     ,ACT_CHECK,mcc_dig_3,"digit 3 of mobile country code")

BF( 4,1     ,ACT_CHECK,mnc_dig_2,"digit 2 of mobile network code")

BF( 4,0     ,ACT_CHECK,mnc_dig_1,"digit 1 of mobile network code")

BF(16, LAC_B,ACT_CHECK,lac      ,"Location area code            ")

IE_END(location_area_identification_B)

IE_BEGIN(mobile_identity_tmsi)
	BF(8,	5,	ACT_CHECK,length,	"five octets to come")
	BF(4,	M4(1,1,1,1),	ACT_CHECK,ANONYMOUS,"bits 5-8 of octet 3 ='1111'")
	BF( 1,	0,	ACT_CHECK,odd_even, 	"as applicable for TMSI")
	BF( 3,	M3(1,0,0),	ACT_CHECK,type,	"TMSI")
	BF( 8,	0x12,	ACT_CHECK,	ANONYMOUS,	SILENT)
	BF( 8,	0x34,	ACT_CHECK,	ANONYMOUS,	SILENT)
	BF( 8,	0x56,	ACT_CHECK,	ANONYMOUS,	SILENT)
	BF( 8,	0x78,	ACT_CHECK,	ANONYMOUS,	SILENT)
IE_END(mobile_identity_tmsi)

IE_BEGIN(tmsi)
	BF( 8,	0x12,	ACT_CHECK,	ANONYMOUS,	SILENT)
	BF( 8,	0x34,	ACT_CHECK,	ANONYMOUS,	SILENT)
	BF( 8,	0x56,	ACT_CHECK,	ANONYMOUS,	SILENT)
	BF( 8,	0x78,	ACT_CHECK,	ANONYMOUS,	SILENT)
IE_END(tmsi)

IE_BEGIN(tmsi_2)
	BF( 8,	0x12,	ACT_CHECK,	ANONYMOUS,	SILENT)
	BF( 8,	0x79,	ACT_CHECK,	ANONYMOUS,	SILENT)
	BF( 8,	0x13,	ACT_CHECK,	ANONYMOUS,	SILENT)
	BF( 8,	0x78,	ACT_CHECK,	ANONYMOUS,	SILENT)
IE_END(tmsi_2)

IE_BEGIN(ncc_permitted)

BF(8,0xFF,ACT_CHECK,ncc_permit,"e.g. all NCCs permitted")

IE_END(ncc_permitted)

IE_BEGIN(rach_control_parameter)

BF( 2,0,ACT_CHECK,max_retrans                ,"Any Value  ")

BF( 4,0,ACT_CHECK,tx_integer                 ,"Any Value  ")

BF( 1,0,ACT_CHECK,cell_bar_access            ,"Not barred ")

BF( 1,1,ACT_CHECK,call_re_establishment      ,"Not Allowed")

BF( 5,0,ACT_CHECK,access_control_class_15_11 ,"None Barred")

BF( 1,0,ACT_CHECK,emergency_call             ,"Allowed    ")

BF(10,0,ACT_CHECK,access_control_class_09_00 ,"None Barred")

IE_END(rach_control_parameter)

IE_BEGIN(rach_control_parameter_reest)

BF( 2,0,ACT_CHECK,max_retrans                ,"Any Value  ")

BF( 4,0,ACT_CHECK,tx_integer                 ,"Any Value  ")

BF( 1,0,ACT_CHECK,cell_bar_access            ,"Not barred ")

BF( 1,0,ACT_CHECK,call_re_establishment      ,"Allowed")

BF( 5,0,ACT_CHECK,access_control_class_15_11 ,"None Barred")

BF( 1,0,ACT_CHECK,emergency_call             ,"Allowed    ")

BF(10,0,ACT_CHECK,access_control_class_09_00 ,"None Barred")

IE_END(rach_control_parameter_reest)

IE_BEGIN(rr_management_protocol_discriminator)

BF(4, 6,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(rr_management_protocol_discriminator)

IE_BEGIN(skip_indicator)

BF(4, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(skip_indicator)

IE_BEGIN(si_1_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,"Spare Octets")

IE_END(si_1_rest_octets)

IE_BEGIN(si_3_rest_octets) /* optionally contains cell (re)select params */

BF(1,                0,ACT_CHECK,p1    ,"C2 parameters not present")

BF(7,REST_OCTET & 0x7F,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

IE_END(si_3_rest_octets)

IE_BEGIN(si_4_rest_octets) /* optionally contains cell (re)select params */

BF(1,                0,ACT_CHECK,p1    ,"C2 parameters not present")

BF(7,REST_OCTET & 0x7F,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

BF(8,REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT                     )

IE_END(si_4_rest_octets)

IE_BEGIN(system_information_type_1_message_type)

BF(8, 0x19,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_1_message_type)

IE_BEGIN(system_information_type_2_message_type)

BF(8, 0x1A,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_2_message_type)

IE_BEGIN(system_information_type_3_message_type)

BF(8, 0x1B,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_3_message_type)

IE_BEGIN(system_information_type_4_message_type)

BF(8, 0x1C,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_4_message_type)

IE_BEGIN(system_information_type_5_message_type)

BF(8, 0x1D,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_5_message_type)

IE_BEGIN(system_information_type_6_message_type)

BF(8, 0x1E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_6_message_type)

IE_BEGIN(paging_request_type_1_message_type)

BF(8, 0x21,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(paging_request_type_1_message_type)

IE_BEGIN(paging_request_type_2_message_type)

BF(8, 0x22,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(paging_request_type_2_message_type)

IE_BEGIN(paging_request_type_3_message_type)

BF(8, 0x24,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(paging_request_type_3_message_type)

IE_BEGIN(immediate_assignment_message_type)

BF(8, 0x3F,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(immediate_assignment_message_type)

IE_BEGIN(paging_response_message_type)

BF(8, 0x27,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(paging_response_message_type)

IE_BEGIN(ciphering_mode_command_message_type)

BF(8, 0x35,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(ciphering_mode_command_message_type)

IE_BEGIN(ciphering_mode_complete_message_type)

BF(8, 0x32,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(ciphering_mode_complete_message_type)

IE_BEGIN(handover_command_message_type)

BF(8, 0x2B,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(handover_command_message_type)

IE_BEGIN(handover_complete_message_type)

BF(8, 0x2C,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(handover_complete_message_type)

IE_BEGIN(handover_failure_message_type)

BF(8, 0x28,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(handover_failure_message_type)

IE_BEGIN(assignment_failure_message_type)

BF(8, 0x2F,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(assignment_failure_message_type)

IE_BEGIN(channel_mode_modify_message_type)

BF(8, 0x10,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(channel_mode_modify_message_type)

IE_BEGIN(channel_mode_modify_acknowledge_message_type)

BF(8, 0x17,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(channel_mode_modify_acknowledge_message_type)

IE_BEGIN(assignment_command_message_type)

BF(8, 0x2E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(assignment_command_message_type)

IE_BEGIN(assignment_complete_message_type)

BF(8, 0x29,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(assignment_complete_message_type)

IE_BEGIN(cm_service_request_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x24,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cm_service_request_message_type)

IE_BEGIN(cm_service_abort_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x23,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cm_service_abort_message_type)

IE_BEGIN(cm_service_reject_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x22,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cm_service_reject_message_type)

IE_BEGIN(cm_service_accept_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x21,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cm_service_accept_message_type)

IE_BEGIN(call_proceeding_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x02,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_proceeding_message_type)

IE_BEGIN(modify_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x17,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(modify_message_type)

IE_BEGIN(modify_reject_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x13,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(modify_reject_message_type)

/*------------------------------------------------------------------*\

| Messages

\*------------------------------------------------------------------*/

MSG3_BEGIN(system_information_type_1)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1)

MSG3_BEGIN(system_information_type_2)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(bcch_frequency_list)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2)

MSG3_BEGIN(system_information_type_3)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3)

MSG3_BEGIN(system_information_type_4)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4)

MSG3_BEGIN(system_information_type_5)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(bcch_frequency_list)

MSG3_END(system_information_type_5)

MSG3_BEGIN(system_information_type_6)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_6_message_type)

IE(cell_identity)

IE(location_area_identification)

IE(cell_options)

IE(ncc_permitted)

MSG3_END(system_information_type_6)

/*------------------------------------------------------------------*\

| GSM 11.10

| 10 Generic call set up procedure

| 10.1 Generic call setup-up procedure for mobile terminating speech calls

| 10.1.4 Specific message contents

\*------------------------------------------------------------------*/

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

#define  ARFCN_BCCH	 122

#define ARFCN_BCCH_B	48

#define  NCC	0x5

#define  BCC	0x6

#define  BSIC	((NCC<<3)|(BCC))

#define  RFN	0

IE_BEGIN(handover_reference)

BF(8,0,ACT_CHECK,reference_value,"used for handover")

IE_END(handover_reference)

IE_BEGIN(authentication_parameter_rand)

BF(32,0x80000000,ACT_CHECK,rand_127_096,SILENT)

BF(32,0x00000012,ACT_CHECK,rand_095_064,SILENT)

BF(32,0x34000000,ACT_CHECK,rand_063_032,SILENT)

BF(32,0x0000000F,ACT_CHECK,rand_031_000,SILENT)

IE_END(authentication_parameter_rand)

IE_BEGIN(authentication_parameter_sres)

BF(32,0x0000000F,ACT_NOP,sres_031_000,SILENT)

IE_END(authentication_parameter_sres)

IE_BEGIN(bearer_capability)

BF(8,1,ACT_CHECK,   length,                   SILENT)

BF(1,1,ACT_CHECK,   ext3,                     SILENT)

BF(2,1,ACT_CHECK,   radio_channel_requirement,SILENT)

BF(1,0,ACT_CHECK,   coding_standard,          SILENT)

BF(1,0,ACT_CHECK,   transfer_mode,            SILENT)

BF(3,0,ACT_CHECK,   info_transfer_capability, SILENT)

IE_END(bearer_capability)

IE_BEGIN(bearer_capability_efr)

BF(8,   3,ACT_CHECK,   length,                   SILENT)

BF(1,   0,ACT_CHECK,   ext3,                     SILENT)

BF(2,   1,ACT_CHECK,   radio_channel_requirement,SILENT)

BF(1,   0,ACT_CHECK,   coding_standard,          SILENT)

BF(1,   0,ACT_CHECK,   transfer_mode,            SILENT)

BF(3,   0,ACT_CHECK,   info_transfer_capability, SILENT)

BF(8,0x00,ACT_CHECK, octet_3a_1,                 SILENT)

BF(8,0x82,ACT_CHECK, octet_3a_2,                 SILENT)

IE_END(bearer_capability_efr)

IE_BEGIN(bearer_capability_data)

BF(8,7,ACT_CHECK,   length,                   SILENT)

BF(1,1,ACT_CHECK,   ext3,                     SILENT)

BF(2,1,ACT_CHECK,   radio_channel_requirement,SILENT)

BF(1,0,ACT_CHECK,   coding_standard,          SILENT)

BF(1,0,ACT_CHECK,   transfer_mode,            SILENT)

BF(3,2,ACT_CHECK,   info_transfer_capability, SILENT)

BF(1,1,ACT_CHECK,   ext4,                     SILENT)

BF(1,1,ACT_CHECK,   compression,              SILENT)

BF(2,0,ACT_CHECK,   structure,                SILENT)

BF(1,1,ACT_CHECK,   duplex_mode,              SILENT)

BF(1,0,ACT_CHECK,   configuration,            SILENT)

BF(1,0,ACT_CHECK,   NIRR,                     SILENT)

BF(1,0,ACT_CHECK,   establishment,            SILENT)

BF(1,1,ACT_CHECK,   ext5,                     SILENT)

BF(2,1,ACT_CHECK,   access_identifier,        SILENT)

BF(2,0,ACT_CHECK,   rate_adaption,            SILENT)

BF(3,1,ACT_CHECK,   access_protocol,          SILENT)

BF(1,0,ACT_CHECK,   ext6,                     SILENT)

BF(2,1,ACT_CHECK,   layer_1,                  SILENT)

BF(4,0,ACT_CHECK,   default_layer_1,          SILENT)

BF(1,1,ACT_CHECK,   sync_async,               SILENT)

BF(1,0,ACT_CHECK,   ext6a,                    SILENT)

BF(1,0,ACT_CHECK,   stop_bits,                SILENT)

BF(1,0,ACT_CHECK,   negotiation,              SILENT)

BF(1,1,ACT_CHECK,   data_bits,                SILENT)

BF(4,3,ACT_CHECK,   user_rate,                SILENT)

BF(1,0,ACT_CHECK,   ext6b,                    SILENT)

BF(2,3,ACT_CHECK,   intermediate_rate,        SILENT)

BF(1,0,ACT_CHECK,   NIC_TX,                   SILENT)

BF(1,0,ACT_CHECK,   NIC_RX,                   SILENT)

BF(3,3,ACT_CHECK,   parity,                   SILENT)

BF(1,1,ACT_CHECK,   ext6c,                    SILENT)

BF(2,1,ACT_CHECK,   connection_element,       SILENT)

BF(5,3,ACT_CHECK,   modem_type,               SILENT)

IE_END(bearer_capability_data)

IE_BEGIN(call_control_protocol_discriminator)

BF(4, 3,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_control_protocol_discriminator)

IE_BEGIN(supplementary_services_protocol_discriminator)

BF(4, M4(1,0,1,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(supplementary_services_protocol_discriminator)

IE_BEGIN(called_party_bcd_number)

BF( 8,7,ACT_CHECK,length                     ,"length of IE")

BF( 1,1,ACT_CHECK,ext                        ,"Extension Bit")

BF( 3,0,ACT_CHECK,type_of_number             ,"Unknown")

BF( 4,1,ACT_CHECK,numbering_plan             ,"ISDN/telephony")

BF( 4,3,ACT_CHECK,digit_2                    ,"Digit 2")

BF( 4,0,ACT_CHECK,digit_1                    ,"Digit 1")

BF( 4,3,ACT_CHECK,digit_4                    ,"Digit 4")

BF( 4,0,ACT_CHECK,digit_3                    ,"Digit 3")

BF( 4,0,ACT_CHECK,digit_6                    ,"Digit 6")

BF( 4,9,ACT_CHECK,digit_5                    ,"Digit 5")

BF( 4,4,ACT_CHECK,digit_8                    ,"Digit 8")

BF( 4,9,ACT_CHECK,digit_7                    ,"Digit 7")

BF( 4,1,ACT_CHECK,digit_10                   ,"Digit 10")

BF( 4,1,ACT_CHECK,digit_9                   ,"Digit 9")

BF( 4,0xF,ACT_CHECK,digit_12                  ,"Digit 12")

BF( 4,7,ACT_CHECK,digit_11                ,"Digit 11")

IE_END(called_party_bcd_number)

IE_BEGIN(calling_party_bcd_number)

BF( 8,8,ACT_CHECK,length                     ,"length of IE")

BF( 1,0,ACT_CHECK,ext                        ,"Extension Bit")

BF( 3,0,ACT_CHECK,type_of_number             ,"Unknown")

BF( 4,1,ACT_CHECK,numbering_plan             ,"ISDN/telephony")

BF( 1,1,ACT_CHECK,ext                        ,"Extension Bit")

BF( 3,0,ACT_CHECK,presentation_indicator     ,"Presentation allowed")

BF( 3,0,ACT_CHECK,spare                      ,"Spare")

BF( 1,0,ACT_CHECK,screening_indicator        ,"Screening indicator")

BF( 4,3,ACT_CHECK,digit_2                    ,"Digit 2")

BF( 4,0,ACT_CHECK,digit_1                    ,"Digit 1")

BF( 4,3,ACT_CHECK,digit_4                    ,"Digit 4")

BF( 4,0,ACT_CHECK,digit_3                    ,"Digit 3")

BF( 4,0,ACT_CHECK,digit_6                    ,"Digit 6")

BF( 4,9,ACT_CHECK,digit_5                    ,"Digit 5")

BF( 4,4,ACT_CHECK,digit_8                    ,"Digit 8")

BF( 4,9,ACT_CHECK,digit_7                    ,"Digit 7")

BF( 4,1,ACT_CHECK,digit_10                   ,"Digit 10")

BF( 4,1,ACT_CHECK,digit_9                    ,"Digit 9")

BF( 4,0xF,ACT_CHECK,digit_12                 ,"Digit 12")

BF( 4,7,ACT_CHECK,digit_11                   ,"Digit 11")

IE_END(calling_party_bcd_number)

IE_BEGIN(channels_needed_for_mobiles_1_and_2)

BF(2,0,ACT_CHECK,second_channel,"spare, any channel")

BF(2,0,ACT_CHECK, first_channel,"spare, any channel")

IE_END(channels_needed_for_mobiles_1_and_2)

IE_BEGIN(cc_capabilities)

BF(8,1,ACT_CHECK, length,       SILENT)

BF(8,1,ACT_CHECK, dtmf_support, SILENT)

IE_END(cc_capabilities)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,1),ACT_CHECK, channel_type,"SDCCH/SACCH 4(1) ")

BF( 3,            0,ACT_CHECK, time_slot_number,"zero")

BF( 3,          BCC,ACT_CHECK, training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK, hopping,"No")

BF( 2,            0,ACT_CHECK, spare,SILENT)

BF(10,   ARFCN_BCCH,ACT_CHECK, arfcn,"ARFCN of the BCCH")

IE_END(channel_description)

IE_BEGIN(channel_description_tch)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,       channel_type,"TCH Full Rate")

BF( 3,            0,ACT_CHECK,       time_slot_number,"zero     ")

BF( 3,          BCC,ACT_CHECK,       tsc     ,"same as BCCH     ")

BF( 1,            0,ACT_CHECK,       hopping,"No                ")

BF( 2,            0,ACT_CHECK,       spare,SILENT                )

BF(10,   ARFCN_BCCH,ACT_CHECK,       arfcn,"ARFCN of the BCCH   ")

IE_END(channel_description_tch)

IE_BEGIN(channel_mode_speech)

BF( 8,            1,ACT_CHECK,      mode,"Speech full rate")

IE_END(channel_mode_speech)

IE_BEGIN(cm_service_type_ss)

BF(4,M4(1,0,0,0),ACT_CHECK,service_type,"supplementary services")

IE_END(cm_service_type_ss)

IE_BEGIN(cm_service_type_moc)

BF(4,M4(0,0,0,1),ACT_CHECK,service_type,"mobile originated call")

IE_END(cm_service_type_moc)

IE_BEGIN(ciphering_key_sequence_number)

BF(1,        0,ACT_CHECK,       spare,SILENT)

BF(3,M3(0,1,1),ACT_CHECK,key_sequence,"from SIM card (3)")

IE_END(ciphering_key_sequence_number)

IE_BEGIN(ciphering_key_sequence_number_2)

BF(1,        0,ACT_CHECK,       spare,SILENT)

BF(3,M3(0,1,0),ACT_CHECK,key_sequence,"sent BS->MS")

IE_END(ciphering_key_sequence_number_2)

IE_BEGIN(ciphering_mode_setting)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1           ")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(ciphering_mode_setting)

IE_BEGIN(cipher_response)

BF(3,0,ACT_CHECK,          spare,SILENT                        )

BF(1,0,ACT_CHECK,cipher_response,"IMEISV shall not be included")

IE_END(cipher_response)

IE_BEGIN(description_of_the_first_channel_after_time)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,      channel_type,"TCH ")

BF( 3,            3,ACT_CHECK,      time_slot_number,"three")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,      hopping,"No")

BF( 2,            0,ACT_CHECK,      spare,SILENT)

BF(10,   ARFCN_BCCH,ACT_CHECK,      arfcn,"ARFCN of the BCCH")

IE_END(description_of_the_first_channel_after_time)

IE_BEGIN(channel_description_ho)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,   channel_type,"TCH ")

BF( 3,            2,ACT_CHECK,   time_slot_number,"two")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,   hopping,"No")

BF( 2,            0,ACT_CHECK,   spare,SILENT)

BF(10,   ARFCN_BCCH,ACT_CHECK,   arfcn,"ARFCN of the BCCH")

IE_END(channel_description_ho)

IE_BEGIN(cell_description)

BF( 2,ARFCN_BCCH>>8    ,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,                 1,ACT_CHECK,ncc,     "network colour code")

BF( 3,                 5,ACT_CHECK,bcc,     "base station colour code")

BF( 8,ARFCN_BCCH        ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(sync_ind_synch)

BF( 1,   0,ACT_CHECK,nci,"ignore out of range timing advance")

BF( 1,   0,ACT_CHECK,rot,"shall not be included")

BF( 2,   1,ACT_CHECK,si ,"synchronized handover")

IE_END(sync_ind_synch)

IE_BEGIN(ia_rest_octets)   /* maximum length (11), no hop, no start time */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(ia_rest_octets)

IE_BEGIN(iei_D)

BF(4,0x0D,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_D)

IE_BEGIN(iei_08)

BF(8,0x08,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_08)

IE_BEGIN(iei_1C)

BF(8,0x1C,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_1C)

IE_BEGIN(iei_7E)

BF(8,0x7E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_7E)

IE_BEGIN(iei_7F)

BF(8,0x7F,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_7F)

IE_BEGIN(iei_1E)

BF(8,0x1E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_1E)

IE_BEGIN(iei_63)

BF(8,0x63,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_63)

IE_BEGIN(iei_5E)

BF(8,0x5E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_5E)

IE_BEGIN(iei_15)

BF(8,0x15,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_15)

IE_BEGIN(iei_04)

BF(8,0x04,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_04)

IE_BEGIN(iei_34)

BF(8,0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_34)

IE_BEGIN(iei_2C)

BF(8,0x2C,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_2C)

IE_BEGIN(iei_13)

BF(8,0x13,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_13)

IE_BEGIN(iei_5C)

BF(8,0x5C,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_5C)

IE_BEGIN(keypad_facility_0)

BF(8, 0x30,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_0)

IE_BEGIN(keypad_facility_1)

BF(8, 0x31,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_1)

IE_BEGIN(keypad_facility_2)

BF(8, 0x32,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_2)

IE_BEGIN(keypad_facility_3)

BF(8, 0x33,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_3)

IE_BEGIN(keypad_facility_4)

BF(8, 0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_4)

IE_BEGIN(keypad_facility_5)

BF(8, 0x35,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_5)

IE_BEGIN(keypad_facility_6)

BF(8, 0x36,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_6)

IE_BEGIN(keypad_facility_7)

BF(8, 0x37,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_7)

IE_BEGIN(keypad_facility_8)

BF(8, 0x38,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_8)

IE_BEGIN(keypad_facility_9)

BF(8, 0x39,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_9)

IE_BEGIN(keypad_facility_A)

BF(8, 0x41,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_A)

IE_BEGIN(keypad_facility_B)

BF(8, 0x42,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_B)

IE_BEGIN(keypad_facility_C)

BF(8, 0x43,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_C)

IE_BEGIN(keypad_facility_D)

BF(8, 0x44,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_D)

IE_BEGIN(keypad_facility_star)

BF(8, 0x2A,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_star)

IE_BEGIN(keypad_facility_hash)

BF(8, 0x23,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(keypad_facility_hash)

IE_BEGIN(l2_pseudo_length_11)  /* pag req type 1 with TMSI (one mobile)*/

BF(8, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_11)

IE_BEGIN(mobile_allocation)

BF(8,0,ACT_CHECK,length,"length 0 due to hopping disabled")

IE_END(mobile_allocation)

IE_BEGIN(mobile_identity)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_4,SILENT)

BF(4,          0,ACT_CHECK, digit_3,SILENT)

BF(4,          7,ACT_CHECK, digit_6,SILENT)

BF(4,          4,ACT_CHECK, digit_5,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          1,ACT_CHECK, digit_7,SILENT)

BF(4,          9,ACT_CHECK, digit_10,SILENT)

BF(4,          4,ACT_CHECK, digit_9,SILENT)

BF(4,          2,ACT_CHECK, digit_12,SILENT)

BF(4,          1,ACT_CHECK, digit_11,SILENT)

IE_END(mobile_identity)

IE_BEGIN(mobility_management_protocol_discriminator)

BF(4, 5,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobility_management_protocol_discriminator)

IE_BEGIN(identity_type)

BF(4,1,ACT_CHECK, type_of_identity,"IMSI")

IE_END(identity_type)

IE_BEGIN(mode_of_the_first_channel)

BF(8,1,ACT_CHECK,mode,"Speech full rate")

IE_END(mode_of_the_first_channel)

IE_BEGIN(mode_of_the_first_channel_efr)

BF(8,0x21,ACT_CHECK,mode,"Enhanced full rate")

IE_END(mode_of_the_first_channel_efr)

IE_BEGIN(ms_classmark)

BF(8,        3,ACT_CHECK, length,SILENT)

BF(1,        0,ACT_CHECK, spare,SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,"phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,"with 'Contr. Early Classmark Send.'")

BF(1,        0,ACT_CHECK, a5_1,"encryption algorithm A5/1 available")

BF(3,M3(0,0,1),ACT_CHECK, rf_power_capability,"class 2")

BF(1,        0,ACT_CHECK, spare2,SILENT)

BF(1,        1,ACT_CHECK, ps_capability,SILENT)

BF(2,  M2(1,1),ACT_CHECK, ss_screening_indicator,SILENT)

BF(1,        1,ACT_CHECK, sm_capability,"point to point SMS")

BF(1,        0,ACT_CHECK, vbs,"no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,"no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"no extention band G1")

BF(1,        0,ACT_CHECK, classmark_3,"add. MS cap. information")

BF(5,        0,ACT_CHECK, spare3,SILENT)

BF(1,        0,ACT_CHECK, a5_3,"A5/3 not available")

BF(1,        0,ACT_CHECK, a5_2,"A5/2 not available")

IE_END(ms_classmark)

IE_BEGIN(p1_rest_octets)

/* pag. req. type1 : 22 - 11 (L2 pseud. len) =  11 bytes */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(p1_rest_octets)

IE_BEGIN(p2_rest_octets)

/* pag. req. type1 : 22 - 11 (L2 pseud. len) =  11 bytes */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(p2_rest_octets)

IE_BEGIN(p3_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

IE_END(p3_rest_octets)

IE_BEGIN(page_mode)

BF(2,0,ACT_CHECK,spare,"two spare bits  ")

BF(2,0,ACT_CHECK,pm   ,"Normal Paging")

IE_END(page_mode)

IE_BEGIN(power_command)

BF(8,10,ACT_CHECK,power,SILENT)

IE_END(power_command)

IE_BEGIN(rach_111)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"SS")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_111)

IE_BEGIN(rach)

BF(3,M3(1,0,0),ACT_CHECK ,establishment_cause,"paging ind. any chan'")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

IE_BEGIN(rach_reest_neci_0)

BF(3,M3(1,1,0),ACT_CHECK ,establishment_cause,"Reest, NECI=0, TCH/F")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_reest_neci_0)

IE_BEGIN(request_reference)

BF(3,    M3(1,0,0),ACT_NOP,random_access_info,"As in CHAN REQ")

BF(5,M5(1,1,1,1,1),ACT_NOP,  random_reference,SILENT)

BF(5,            0,ACT_NOP,               t1_,SILENT)

BF(6,            0,ACT_NOP,               t3 ,SILENT)

BF(5,            0,ACT_NOP,               t2 ,SILENT)

IE_END(request_reference)

IE_BEGIN(rr_cause)

BF(8,0,ACT_CHECK,rr_cause,"normal event")

IE_END(rr_cause)

IE_BEGIN(rr_cause_1)

BF(8,1,ACT_CHECK,rr_cause,"abnormal event, unspecified")

IE_END(rr_cause_1)

IE_BEGIN(rr_cause_6F)

BF(8,0x6F,ACT_CHECK,rr_cause,"protocol error unspecified")

IE_END(rr_cause_6F)

IE_BEGIN(reject_cause)

BF(8,0x20,ACT_CHECK,reject_cause,"service not available")

IE_END(reject_cause)

IE_BEGIN(progress_indicator_32)

BF(8,  2,ACT_CHECK, length,"two octets")

BF(1,  1,ACT_CHECK, ext_1,SILENT)

BF(2,  3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,  0,ACT_SHOW,  spare,SILENT)

BF(4,  0,ACT_CHECK, location,"User")

BF(1,  1,ACT_CHECK, ext_2,SILENT)

BF(7, 32,ACT_CHECK, progress_description,"call is end-to-end PLMN/ISDN")

IE_END(progress_indicator_32)

IE_BEGIN(progress_indicator_8)

BF(8,  2,ACT_CHECK, length,"two octets")

BF(1,  1,ACT_CHECK, ext_1,SILENT)

BF(2,  3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,  0,ACT_SHOW,  spare,SILENT)

BF(4,  0,ACT_CHECK, location,"User")

BF(1,  1,ACT_CHECK, ext_2,SILENT)

BF(7,  8,ACT_CHECK, progress_description,"in band tones available")

IE_END(progress_indicator_8)

IE_BEGIN(ss_version)

BF(8,    1,ACT_CHECK,length,SILENT)

BF(8,    0,ACT_SHOW, version,SILENT)

IE_END(ss_version)

IE_BEGIN(cause_03)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,          3,ACT_CHECK, cause,"no route to destination")

IE_END(cause_03)

IE_BEGIN(cause_17)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         17,ACT_CHECK, cause,"user busy")

IE_END(cause_17)

IE_BEGIN(cause_31)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         31,ACT_CHECK, cause,"normal, unspecified")

IE_END(cause_31)

IE_BEGIN(cause_57)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         57,ACT_CHECK, cause,"bearer cap not authorized")

IE_END(cause_57)

IE_BEGIN(cause_81)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         81,ACT_CHECK, cause,"invalid transaction identifier")

IE_END(cause_81)

IE_BEGIN(cause_88)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         88,ACT_CHECK, cause,"incompatible destination")

IE_END(cause_88)

IE_BEGIN(cause_102_303)

BF(8,          5,ACT_CHECK, length,"five octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,        102,ACT_CHECK, cause,"recovery on timer expiry")

BF(8,       0x33,ACT_CHECK, diag_0,"3")

BF(8,       0x30,ACT_CHECK, diag_1,"0")

BF(8,       0x33,ACT_CHECK, diag_2,"3")

IE_END(cause_102_303)

IE_BEGIN(cause_102_310)

BF(8,          5,ACT_CHECK, length,"five octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,        102,ACT_CHECK, cause,"recovery on timer expiry")

BF(8,       0x33,ACT_CHECK, diag_0,"3")

BF(8,       0x31,ACT_CHECK, diag_1,"1")

BF(8,       0x30,ACT_CHECK, diag_2,"0")

IE_END(cause_102_310)

IE_BEGIN(cause_102_313)

BF(8,          5,ACT_CHECK, length,"five octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,        102,ACT_CHECK, cause,"recovery on timer expiry")

BF(8,       0x33,ACT_CHECK, diag_0,"3")

BF(8,       0x31,ACT_CHECK, diag_1,"1")

BF(8,       0x33,ACT_CHECK, diag_2,"3")

IE_END(cause_102_313)

IE_BEGIN(cause_97_20)

BF(8,          3,ACT_CHECK, length,"three octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         97,ACT_CHECK, cause,"message not implemented")

BF(8,       0x20,ACT_CHECK, diag_0,"message type 0x20")

IE_END(cause_97_20)

IE_BEGIN(cause_16)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         16,ACT_CHECK, cause,"normal clearing")

IE_END(cause_16)

IE_BEGIN(cause_30)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         30,ACT_CHECK, cause,"response to enquiry")

IE_END(cause_30)

IE_BEGIN(call_state_1)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          1,ACT_CHECK, call_state_value,"U1 Call Initiated")

IE_END(call_state_1)

IE_BEGIN(call_state_3)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          3,ACT_CHECK, call_state_value,"U3 Call Proceeding")

IE_END(call_state_3)

IE_BEGIN(call_state_4)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          4,ACT_CHECK, call_state_value,"U4 Call Delivered")

IE_END(call_state_4)

IE_BEGIN(call_state_7)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          7,ACT_CHECK, call_state_value,"U7 Call Receiving")

IE_END(call_state_7)

IE_BEGIN(call_state_8)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          8,ACT_CHECK, call_state_value,"U8 Connect Request")

IE_END(call_state_8)

IE_BEGIN(call_state_9)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          9,ACT_CHECK, call_state_value,"U9 Call Confirmed")

IE_END(call_state_9)

IE_BEGIN(call_state_10)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,         10,ACT_CHECK, call_state_value,"U10 Call Active")

IE_END(call_state_10)

IE_BEGIN(call_state_11)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,         11,ACT_CHECK, call_state_value,"U11 Disconnect Request")

IE_END(call_state_11)

IE_BEGIN(call_state_12)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,         12,ACT_CHECK, call_state_value,"U12 Disconnect Ind")

IE_END(call_state_12)

IE_BEGIN(call_state_19)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,         19,ACT_CHECK, call_state_value,"U19 Release Request")

IE_END(call_state_19)

IE_BEGIN(signal_call_waiting)

BF(8,M8(0,0,0,0,0,1,1,1),ACT_CHECK,signal_value,"(Any non-res. value)")

IE_END(signal_call_waiting)

IE_BEGIN(spare_half_octet)

BF(4, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(spare_half_octet)

IE_BEGIN(facility_unstructured_data)

BF(8,   35  ,ACT_CHECK,length_of_fac_ie_content,SILENT)

BF(8,   0xA1,ACT_CHECK,component_type_tag,"INVOKE")

BF(8,   0x21,ACT_CHECK,component_length, SILENT)

BF(8,   0x02,ACT_CHECK,invoke_id_tag, SILENT)

BF(8,   0x01,ACT_CHECK,invoke_id_length, SILENT)

BF(8,   0x00,ACT_CHECK,invoke_id, SILENT)

BF(8,   0x02,ACT_CHECK,operation_code_tag, SILENT)

BF(8,   0x01,ACT_CHECK,operation_code_length, SILENT)

BF(8,   19  ,ACT_CHECK,operation_code, "Process unstructured SS data")

BF(8,   0x30,ACT_CHECK,Process_SS_Arg, SILENT)

BF(8,   25  ,ACT_CHECK,Process_SS_Arg_length, SILENT)

BF(8,   0xF0,ACT_CHECK,dcs, SILENT)

BF(8,   0x2A,ACT_CHECK,dig_1, "*")

BF(8,   0x30,ACT_CHECK,dig_2, "0")

BF(8,   0x30,ACT_CHECK,dig_3, "0")

BF(8,   0x2A,ACT_CHECK,dig_4, "*")

BF(8,   0x30,ACT_CHECK,dig_5, "0")

BF(8,   0x31,ACT_CHECK,dig_6, "1")

BF(8,   0x32,ACT_CHECK,dig_7, "2")

BF(8,   0x33,ACT_CHECK,dig_8, "3")

BF(8,   0x34,ACT_CHECK,dig_9, "4")

BF(8,   0x35,ACT_CHECK,dig_10,"5")

BF(8,   0x36,ACT_CHECK,dig_11,"6")

BF(8,   0x37,ACT_CHECK,dig_12,"7")

BF(8,   0x38,ACT_CHECK,dig_13,"8")

BF(8,   0x39,ACT_CHECK,dig_14,"9")

BF(8,   0x30,ACT_CHECK,dig_15,"0")

BF(8,   0x31,ACT_CHECK,dig_16,"1")

BF(8,   0x32,ACT_CHECK,dig_17,"2")

BF(8,   0x33,ACT_CHECK,dig_18,"3")

BF(8,   0x34,ACT_CHECK,dig_19,"4")

BF(8,   0x35,ACT_CHECK,dig_20,"5")

BF(8,   0x36,ACT_CHECK,dig_21,"6")

BF(8,   0x37,ACT_CHECK,dig_22,"7")

BF(8,   0x38,ACT_CHECK,dig_23,"8")

BF(8,   0x23,ACT_CHECK,dig_24,"#")

IE_END(facility_unstructured_data)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,         spare,SILENT)

BF(6,0,ACT_CHECK,timing_advance,"0"   )

IE_END(timing_advance)

IE_BEGIN(transaction_identifier_source)

BF(4,M4(0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source)

IE_BEGIN(transaction_identifier_source_1)

BF(4,M4(0,0,0,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_1)

IE_BEGIN(transaction_identifier_source_2)

BF(4,M4(0,0,1,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_2)

IE_BEGIN(transaction_identifier_source_3)

BF(4,M4(0,0,1,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_3)

IE_BEGIN(transaction_identifier_source_4)

BF(4,M4(0,1,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_4)

IE_BEGIN(transaction_identifier_source_5)

BF(4,M4(0,1,0,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_5)

IE_BEGIN(transaction_identifier_source_6)

BF(4,M4(0,1,1,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_6)

IE_BEGIN(transaction_identifier_dest)

BF(4,M4(1,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest)

IE_BEGIN(transaction_identifier_dest_1)

BF(4,M4(1,0,0,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_1)

IE_BEGIN(transaction_identifier_dest_2)

BF(4,M4(1,0,1,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_2)

IE_BEGIN(transaction_identifier_dest_3)

BF(4,M4(1,0,1,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_3)

IE_BEGIN(transaction_identifier_dest_4)

BF(4,M4(1,1,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_4)

IE_BEGIN(transaction_identifier_dest_5)

BF(4,M4(1,1,0,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_5)

IE_BEGIN(transaction_identifier_dest_6)

BF(4,M4(1,1,1,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_6)

IE_BEGIN(authentication_request_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x12,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(authentication_request_message_type)

IE_BEGIN(identity_request_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x18,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(identity_request_message_type)

IE_BEGIN(identity_response_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x19,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(identity_response_message_type)

IE_BEGIN(tmsi_reallocation_command_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x1A,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(tmsi_reallocation_command_message_type)

IE_BEGIN(tmsi_reallocation_complete_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x1B,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(tmsi_reallocation_complete_message_type)

IE_BEGIN(cm_reestablish_request_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x28,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cm_reestablish_request_message_type)

IE_BEGIN(authentication_response_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x14,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(authentication_response_message_type)

IE_BEGIN(ss_register_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x3B,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(ss_register_message_type)

IE_BEGIN(setup_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x05,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(setup_message_type)

IE_BEGIN(call_confirmed_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x08,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_confirmed_message_type)

IE_BEGIN(connect_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x07,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(connect_message_type)

IE_BEGIN(connect_acknowledge_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x0F,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(connect_acknowledge_message_type)

IE_BEGIN(alerting_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x01,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(alerting_message_type)

IE_BEGIN(progress_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x03,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(progress_message_type)

IE_BEGIN(status_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x3D,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(status_message_type)

IE_BEGIN(status_enquiry_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(status_enquiry_message_type)

IE_BEGIN(notify_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x3E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(notify_message_type)

IE_BEGIN(unknown_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x20,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(unknown_message_type)

IE_BEGIN(disconnect_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x25,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(disconnect_message_type)

IE_BEGIN(release_complete_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x2A,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(release_complete_message_type)

IE_BEGIN(release_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x2D,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(release_message_type)

IE_BEGIN(channel_release_message_type)

BF(8, 0x0D,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(channel_release_message_type)

IE_BEGIN(start_dtmf_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x35,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(start_dtmf_message_type)

IE_BEGIN(start_dtmf_acknowledge_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x36,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(start_dtmf_acknowledge_message_type)

IE_BEGIN(stop_dtmf_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x31,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(stop_dtmf_message_type)

IE_BEGIN(stop_dtmf_acknowledge_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x32,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(stop_dtmf_acknowledge_message_type)

IE_BEGIN(start_dtmf_reject_message_type)

BF(1,    0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1,    0,ACT_SHOW, ANONYMOUS,SILENT)

BF(6, 0x37,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(start_dtmf_reject_message_type)

IE_BEGIN(user_user_disconnect)

BF(8,    5,ACT_CHECK,length,SILENT)

BF(8,    1,ACT_CHECK,pd,    SILENT)

BF(8, 0x30,ACT_CHECK,octet1,SILENT)

BF(8, 0x34,ACT_CHECK,octet2,SILENT)

BF(8, 0x35,ACT_CHECK,octet3,SILENT)

BF(8, 0x36,ACT_CHECK,octet4,SILENT)

IE_END(user_user_disconnect)

IE_BEGIN(user_user_setup)

BF(8,    5,ACT_CHECK,length,SILENT)

BF(8,    1,ACT_CHECK,pd,    SILENT)

BF(8, 0x30,ACT_CHECK,octet1,SILENT)

BF(8, 0x31,ACT_CHECK,octet2,SILENT)

BF(8, 0x32,ACT_CHECK,octet3,SILENT)

BF(8, 0x33,ACT_CHECK,octet4,SILENT)

IE_END(user_user_setup)

IE_BEGIN(notify_user_resumed)

BF(8,    1,ACT_CHECK,notific,SILENT)

IE_END(notify_user_resumed)

MSG3_BEGIN(paging_request_type_1)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(paging_request_type_1_message_type)

IE(channels_needed_for_mobiles_1_and_2)

IE(page_mode)

IE(mobile_identity)

IE(p1_rest_octets)

MSG3_END(paging_request_type_1)

MSG3_BEGIN(paging_request_type_1_tmsi)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(paging_request_type_1_message_type)

IE(channels_needed_for_mobiles_1_and_2)

IE(page_mode)

IE(mobile_identity_tmsi)

IE(p1_rest_octets)

MSG3_END(paging_request_type_1_tmsi)

MSG3_BEGIN(paging_request_type_2_tmsi)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(paging_request_type_2_message_type)

IE(channels_needed_for_mobiles_1_and_2)

IE(page_mode)

IE(tmsi)

IE(tmsi_2)

IE(p2_rest_octets)

MSG3_END(paging_request_type_2_tmsi)

MSG3_BEGIN(paging_request_type_3_tmsi)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(paging_request_type_3_message_type)

IE(channels_needed_for_mobiles_1_and_2)

IE(page_mode)

IE(tmsi)

IE(tmsi_2)

IE(tmsi_2)

IE(tmsi_2)

IE(p3_rest_octets)

MSG3_END(paging_request_type_3_tmsi)

MSG3_BEGIN(channel_request)

IE(rach)

MSG3_END(channel_request)

MSG3_BEGIN(channel_request_ss)

IE(rach_111)

MSG3_END(channel_request_ss)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

MSG3_BEGIN(channel_request_reest_neci_0)

IE(rach_reest_neci_0)

MSG3_END(channel_request_reest_neci_0)

MSG3_BEGIN(immediate_assignment)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment)

MSG3_BEGIN(immediate_assignment_tch)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_tch)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment_tch)

MSG3_BEGIN(paging_response)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(paging_response_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(ms_classmark)

IE(mobile_identity)

MSG3_END(paging_response)

MSG3_BEGIN(paging_response_tmsi)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(paging_response_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(ms_classmark)

IE(mobile_identity_tmsi)

MSG3_END(paging_response_tmsi)

MSG3_BEGIN(paging_response_cksn_2)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(paging_response_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number_2)

IE(ms_classmark)

IE(mobile_identity)

MSG3_END(paging_response_cksn_2)

MSG3_BEGIN(authentication_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number_2)

IE(authentication_parameter_rand)

MSG3_END(authentication_request)

MSG3_BEGIN(authentication_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_response_message_type)

IE(authentication_parameter_sres)

MSG3_END(authentication_response)

MSG3_BEGIN(identity_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(identity_request_message_type)

IE(spare_half_octet)

IE(identity_type)

MSG3_END(identity_request)

MSG3_BEGIN(identity_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(identity_response_message_type)

IE(mobile_identity)

MSG3_END(identity_response)

MSG3_BEGIN(cm_service_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_request_message_type)

IE(ciphering_key_sequence_number)

IE(cm_service_type_moc)

IE(ms_classmark)

IE(mobile_identity)

MSG3_END(cm_service_request)

MSG3_BEGIN(cm_service_request_ss)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_request_message_type)

IE(ciphering_key_sequence_number)

IE(cm_service_type_ss)

IE(ms_classmark)

IE(mobile_identity)

MSG3_END(cm_service_request_ss)

MSG3_BEGIN(cm_reestablish_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_reestablish_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number_2)

IE(ms_classmark)

IE(mobile_identity_tmsi)

IE(iei_13)

IE(location_area_identification)

MSG3_END(cm_reestablish_request)

MSG3_BEGIN(cm_service_abort)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_abort_message_type)

MSG3_END(cm_service_abort)

MSG3_BEGIN(cm_service_reject)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_reject_message_type)

IE(reject_cause)

MSG3_END(cm_service_reject)

MSG3_BEGIN(cm_service_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_accept_message_type)

MSG3_END(cm_service_accept)

MSG3_BEGIN(ciphering_mode_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(cipher_response)

IE(ciphering_mode_setting)

MSG3_END(ciphering_mode_command)

MSG3_BEGIN(ciphering_mode_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

MSG3_END(ciphering_mode_complete)

MSG3_BEGIN(channel_mode_modify)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_mode_modify_message_type)

IE(channel_description_tch)

IE(channel_mode_speech)

MSG3_END(channel_mode_modify)

MSG3_BEGIN(channel_mode_modify_acknowledge)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_mode_modify_acknowledge_message_type)

IE(channel_description_tch)

IE(channel_mode_speech)

MSG3_END(channel_mode_modify_acknowledge)

MSG3_BEGIN(setup)            /* contains 'signal' but no 'Bearer Cap' */

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

IE(iei_5C)

IE(calling_party_bcd_number)

MSG3_END(setup)

MSG3_BEGIN(setup_user_to_user)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

IE(iei_7E)

IE(user_user_setup)

MSG3_END(setup_user_to_user)

MSG3_BEGIN(setup_no_signal)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

MSG3_END(setup_no_signal)

MSG3_BEGIN(setup_moc)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_04)

IE(bearer_capability)

IE(iei_5E)

IE(called_party_bcd_number)

MSG3_END(setup_moc)

MSG3_BEGIN(setup_moc_efr)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_04)

IE(bearer_capability_efr)

IE(iei_5E)

IE(called_party_bcd_number)

MSG3_END(setup_moc_efr)

MSG3_BEGIN(setup_data)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_04)

IE(bearer_capability_data)

IE(iei_5E)

IE(called_party_bcd_number)

MSG3_END(setup_data)

MSG3_BEGIN(call_confirmed)           /* contains bearer capability */

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(call_confirmed_message_type)

IE(iei_04)

IE(bearer_capability)

IE(iei_15)

IE(cc_capabilities)

MSG3_END(call_confirmed)

MSG3_BEGIN(call_confirmed_user_busy) /* contains cause user busy */

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(call_confirmed_message_type)

IE(iei_04)

IE(bearer_capability)

IE(iei_08)

IE(cause_17)

IE(iei_15)

IE(cc_capabilities)

MSG3_END(call_confirmed_user_busy)

MSG3_BEGIN(call_proceeding)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(call_proceeding_message_type)

MSG3_END(call_proceeding)

MSG3_BEGIN(connect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(connect_message_type)

MSG3_END(connect)

MSG3_BEGIN(unknown_message)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(unknown_message_type)

MSG3_END(unknown_message)

MSG3_BEGIN(unknown_message_mtc)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(unknown_message_type)

MSG3_END(unknown_message_mtc)

MSG3_BEGIN(alerting)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(alerting_message_type)

MSG3_END(alerting)

MSG3_BEGIN(progress)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(progress_message_type)

IE(progress_indicator_32)

MSG3_END(progress)

MSG3_BEGIN(progress_1)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(progress_message_type)

IE(progress_indicator_8)

MSG3_END(progress_1)

MSG3_BEGIN(assignment_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(assignment_command)

MSG3_BEGIN(assignment_command_efr)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel_efr)

MSG3_END(assignment_command_efr)

MSG3_BEGIN(assignment_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_complete_message_type)

IE(rr_cause)

MSG3_END(assignment_complete)

MSG3_BEGIN(handover_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_complete_message_type)

IE(rr_cause)

MSG3_END(handover_complete)

MSG3_BEGIN(connect_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(connect_acknowledge_message_type)

MSG3_END(connect_acknowledge)

MSG3_BEGIN(status_enquiry_0)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_0)

MSG3_BEGIN(status_enquiry_1)

IE(transaction_identifier_dest_1)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_1)

MSG3_BEGIN(status_enquiry_2)

IE(transaction_identifier_dest_2)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_2)

MSG3_BEGIN(status_enquiry_3)

IE(transaction_identifier_dest_3)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_3)

MSG3_BEGIN(status_enquiry_4)

IE(transaction_identifier_dest_4)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_4)

MSG3_BEGIN(status_enquiry_5)

IE(transaction_identifier_dest_5)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_5)

MSG3_BEGIN(status_enquiry_6)

IE(transaction_identifier_dest_6)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_6)

MSG3_BEGIN(status_enquiry_0_mtc)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_0_mtc)

MSG3_BEGIN(status_enquiry_1_mtc)

IE(transaction_identifier_source_1)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_1_mtc)

MSG3_BEGIN(status_enquiry_2_mtc)

IE(transaction_identifier_source_2)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_2_mtc)

MSG3_BEGIN(status_enquiry_3_mtc)

IE(transaction_identifier_source_3)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_3_mtc)

MSG3_BEGIN(status_enquiry_4_mtc)

IE(transaction_identifier_source_4)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_4_mtc)

MSG3_BEGIN(status_enquiry_5_mtc)

IE(transaction_identifier_source_5)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_5_mtc)

MSG3_BEGIN(status_enquiry_6_mtc)

IE(transaction_identifier_source_6)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_6_mtc)

MSG3_BEGIN(notify)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(notify_message_type)

IE(notify_user_resumed)

MSG3_END(notify)

MSG3_BEGIN(disconnect_t303)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_102_303)

MSG3_END(disconnect_t303)

MSG3_BEGIN(disconnect_t310)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_102_310)

MSG3_END(disconnect_t310)

MSG3_BEGIN(disconnect_t313)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_102_313)

MSG3_END(disconnect_t313)

MSG3_BEGIN(disconnect_8)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_16)

IE(iei_1E)

IE(progress_indicator_8)

MSG3_END(disconnect_8)

MSG3_BEGIN(disconnect_8_mtc)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_16)

IE(iei_1E)

IE(progress_indicator_8)

MSG3_END(disconnect_8_mtc)

MSG3_BEGIN(disconnect_ms)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_16)

MSG3_END(disconnect_ms)

MSG3_BEGIN(disconnect_user_to_user)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_16)

IE(iei_7E)

IE(user_user_disconnect)

MSG3_END(disconnect_user_to_user)

MSG3_BEGIN(disconnect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_16)

MSG3_END(disconnect)

MSG3_BEGIN(release_complete)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_03)

MSG3_END(release_complete)

MSG3_BEGIN(release_complete_empty)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

MSG3_END(release_complete_empty)

MSG3_BEGIN(release)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(release_message_type)

MSG3_END(release)

MSG3_BEGIN(release_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_message_type)

MSG3_END(release_mtc)

MSG3_BEGIN(release_t305)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(release_message_type)

IE(iei_08)

IE(cause_16)

MSG3_END(release_t305)

MSG3_BEGIN(release_bs)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_message_type)

IE(iei_08)

IE(cause_31)

MSG3_END(release_bs)

MSG3_BEGIN(release_complete_88)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_88)

MSG3_END(release_complete_88)

MSG3_BEGIN(release_complete_bs)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

MSG3_END(release_complete_bs)

MSG3_BEGIN(release_complete_0)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_0)

MSG3_BEGIN(release_complete_1)

IE(transaction_identifier_source_1)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_1)

MSG3_BEGIN(release_complete_2)

IE(transaction_identifier_source_2)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_2)

MSG3_BEGIN(release_complete_3)

IE(transaction_identifier_source_3)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_3)

MSG3_BEGIN(release_complete_4)

IE(transaction_identifier_source_4)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_4)

MSG3_BEGIN(release_complete_5)

IE(transaction_identifier_source_5)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_5)

MSG3_BEGIN(release_complete_6)

IE(transaction_identifier_source_6)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_6)

MSG3_BEGIN(release_complete_0_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_0_mtc)

MSG3_BEGIN(release_complete_1_mtc)

IE(transaction_identifier_dest_1)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_1_mtc)

MSG3_BEGIN(release_complete_2_mtc)

IE(transaction_identifier_dest_2)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_2_mtc)

MSG3_BEGIN(release_complete_3_mtc)

IE(transaction_identifier_dest_3)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_3_mtc)

MSG3_BEGIN(release_complete_4_mtc)

IE(transaction_identifier_dest_4)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_4_mtc)

MSG3_BEGIN(release_complete_5_mtc)

IE(transaction_identifier_dest_5)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_5_mtc)

MSG3_BEGIN(release_complete_6_mtc)

IE(transaction_identifier_dest_6)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_6_mtc)

MSG3_BEGIN(channel_release)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_release_message_type)

IE(rr_cause)

MSG3_END(channel_release)

MSG3_BEGIN(status_30_1)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_1)

MSG3_END(status_30_1)

MSG3_BEGIN(status_30_3)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_3)

MSG3_END(status_30_3)

MSG3_BEGIN(status_30_4)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_4)

MSG3_END(status_30_4)

MSG3_BEGIN(status_30_7)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_7)

MSG3_END(status_30_7)

MSG3_BEGIN(status_30_8)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_8)

MSG3_END(status_30_8)

MSG3_BEGIN(status_30_9)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_9)

MSG3_END(status_30_9)

MSG3_BEGIN(status_30_10)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_10)

MSG3_END(status_30_10)

MSG3_BEGIN(status_30_10_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_10)

MSG3_END(status_30_10_mtc)

MSG3_BEGIN(status_30_11)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_11)

MSG3_END(status_30_11)

MSG3_BEGIN(status_30_11_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_11)

MSG3_END(status_30_11_mtc)

MSG3_BEGIN(status_30_12)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_12)

MSG3_END(status_30_12)

MSG3_BEGIN(status_30_12_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_12)

MSG3_END(status_30_12_mtc)

MSG3_BEGIN(status_30_19)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_19)

MSG3_END(status_30_19)

MSG3_BEGIN(status_30_19_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_19)

MSG3_END(status_30_19_mtc)

MSG3_BEGIN(status_97_1)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_1)

MSG3_END(status_97_1)

MSG3_BEGIN(status_97_3)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_3)

MSG3_END(status_97_3)

MSG3_BEGIN(status_97_4)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_4)

MSG3_END(status_97_4)

MSG3_BEGIN(status_97_7)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_7)

MSG3_END(status_97_7)

MSG3_BEGIN(status_97_8)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_8)

MSG3_END(status_97_8)

MSG3_BEGIN(status_97_9)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_9)

MSG3_END(status_97_9)

MSG3_BEGIN(status_97_11)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_11)

MSG3_END(status_97_11)

MSG3_BEGIN(status_97_12)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_12)

MSG3_END(status_97_12)

MSG3_BEGIN(stop_dtmf)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(stop_dtmf_message_type)

MSG3_END(stop_dtmf)

MSG3_BEGIN(stop_dtmf_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(stop_dtmf_acknowledge_message_type)

MSG3_END(stop_dtmf_acknowledge)

MSG3_BEGIN(start_dtmf_reject)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_reject_message_type)

IE(cause_03)

MSG3_END(start_dtmf_reject)

MSG3_BEGIN(start_dtmf_0)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_0)

MSG3_END(start_dtmf_0)

MSG3_BEGIN(start_dtmf_1)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_1)

MSG3_END(start_dtmf_1)

MSG3_BEGIN(start_dtmf_2)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_2)

MSG3_END(start_dtmf_2)

MSG3_BEGIN(start_dtmf_3)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_3)

MSG3_END(start_dtmf_3)

MSG3_BEGIN(start_dtmf_4)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_4)

MSG3_END(start_dtmf_4)

MSG3_BEGIN(start_dtmf_5)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_5)

MSG3_END(start_dtmf_5)

MSG3_BEGIN(start_dtmf_6)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_6)

MSG3_END(start_dtmf_6)

MSG3_BEGIN(start_dtmf_7)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_7)

MSG3_END(start_dtmf_7)

MSG3_BEGIN(start_dtmf_8)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_8)

MSG3_END(start_dtmf_8)

MSG3_BEGIN(start_dtmf_9)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_9)

MSG3_END(start_dtmf_9)

MSG3_BEGIN(start_dtmf_A)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_A)

MSG3_END(start_dtmf_A)

MSG3_BEGIN(start_dtmf_B)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_B)

MSG3_END(start_dtmf_B)

MSG3_BEGIN(start_dtmf_C)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_C)

MSG3_END(start_dtmf_C)

MSG3_BEGIN(start_dtmf_D)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_D)

MSG3_END(start_dtmf_D)

MSG3_BEGIN(start_dtmf_star)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_star)

MSG3_END(start_dtmf_star)

MSG3_BEGIN(start_dtmf_hash)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_hash)

MSG3_END(start_dtmf_hash)

MSG3_BEGIN(start_dtmf_acknowledge_0)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_0)

MSG3_END(start_dtmf_acknowledge_0)

MSG3_BEGIN(start_dtmf_acknowledge_1)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_1)

MSG3_END(start_dtmf_acknowledge_1)

MSG3_BEGIN(start_dtmf_acknowledge_2)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_2)

MSG3_END(start_dtmf_acknowledge_2)

MSG3_BEGIN(start_dtmf_acknowledge_3)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_3)

MSG3_END(start_dtmf_acknowledge_3)

MSG3_BEGIN(start_dtmf_acknowledge_4)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_4)

MSG3_END(start_dtmf_acknowledge_4)

MSG3_BEGIN(start_dtmf_acknowledge_5)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_5)

MSG3_END(start_dtmf_acknowledge_5)

MSG3_BEGIN(start_dtmf_acknowledge_6)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_6)

MSG3_END(start_dtmf_acknowledge_6)

MSG3_BEGIN(start_dtmf_acknowledge_7)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_7)

MSG3_END(start_dtmf_acknowledge_7)

MSG3_BEGIN(start_dtmf_acknowledge_8)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_8)

MSG3_END(start_dtmf_acknowledge_8)

MSG3_BEGIN(start_dtmf_acknowledge_9)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_9)

MSG3_END(start_dtmf_acknowledge_9)

MSG3_BEGIN(start_dtmf_acknowledge_A)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_A)

MSG3_END(start_dtmf_acknowledge_A)

MSG3_BEGIN(start_dtmf_acknowledge_B)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_B)

MSG3_END(start_dtmf_acknowledge_B)

MSG3_BEGIN(start_dtmf_acknowledge_C)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_C)

MSG3_END(start_dtmf_acknowledge_C)

MSG3_BEGIN(start_dtmf_acknowledge_D)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_D)

MSG3_END(start_dtmf_acknowledge_D)

MSG3_BEGIN(start_dtmf_acknowledge_star)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_star)

MSG3_END(start_dtmf_acknowledge_star)

MSG3_BEGIN(start_dtmf_acknowledge_hash)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_hash)

MSG3_END(start_dtmf_acknowledge_hash)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_ho)

IE(handover_reference)

IE(power_command)

IE(iei_D)

IE(sync_ind_synch)

MSG3_END(handover_command)

MSG3_BEGIN(handover_access)

IE(handover_reference)

MSG3_END(handover_access)

MSG3_BEGIN(handover_failure)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_failure_message_type)

IE(rr_cause_1)

MSG3_END(handover_failure)

MSG3_BEGIN(assignment_failure)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_failure_message_type)

IE(rr_cause_6F)

MSG3_END(assignment_failure)

MSG3_BEGIN(modify_data)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(modify_message_type)

IE(bearer_capability_data)

MSG3_END(modify_data)

MSG3_BEGIN(modify_reject)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(modify_reject_message_type)

IE(bearer_capability)

IE(cause_57)

MSG3_END(modify_reject)

MSG3_BEGIN(system_information_type_1_A)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter_reest)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_A)

MSG3_BEGIN(system_information_type_2_A)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(bcch_frequency_list)

IE(ncc_permitted)

IE(rach_control_parameter_reest)

MSG3_END(system_information_type_2_A)

MSG3_BEGIN(system_information_type_3_A)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options)

IE(cell_selection_parameter)

IE(rach_control_parameter_reest)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_A)

MSG3_BEGIN(system_information_type_4_A)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter_reest)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_A)

MSG3_BEGIN(system_information_type_1_B)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter_reest)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_B)

MSG3_BEGIN(system_information_type_2_B)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(bcch_frequency_list)

IE(ncc_permitted)

IE(rach_control_parameter_reest)

MSG3_END(system_information_type_2_B)

MSG3_BEGIN(system_information_type_3_B)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_B)

IE(location_area_identification_B)

IE(control_channel_description)

IE(cell_options)

IE(cell_selection_parameter)

IE(rach_control_parameter_reest)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_B)

MSG3_BEGIN(system_information_type_4_B)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification_B)

IE(cell_selection_parameter)

IE(rach_control_parameter_reest)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_B)

MSG3_BEGIN(system_information_type_5_B)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(bcch_frequency_list)

MSG3_END(system_information_type_5_B)

MSG3_BEGIN(system_information_type_6_B)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_6_message_type)

IE(cell_identity)

IE(location_area_identification_B)

IE(cell_options)

IE(ncc_permitted)

MSG3_END(system_information_type_6_B)

MSG3_BEGIN( tmsi_reallocation_command)
	IE( skip_indicator )
	IE( mobility_management_protocol_discriminator )
	IE( tmsi_reallocation_command_message_type )
	IE( location_area_identification)
	IE( mobile_identity_tmsi)
MSG3_END( tmsi_reallocation_command)

MSG3_BEGIN( tmsi_reallocation_complete)
	IE( skip_indicator )
	IE( mobility_management_protocol_discriminator )
	IE( tmsi_reallocation_complete_message_type )
MSG3_END( tmsi_reallocation_complete)

MSG3_BEGIN(ss_register_unstructured_data)

IE(transaction_identifier_source)

IE(supplementary_services_protocol_discriminator)

IE(ss_register_message_type)

IE(iei_1C)

IE(facility_unstructured_data)

IE(iei_7F)

IE(ss_version)

MSG3_END(ss_register_unstructured_data)

# TEST CASES

## Preambles

### ATI001: Power On

Description:	This test describes the initialization of environment and activation of the Mobile Station.

Preamble:	None

Script:

ISS_INIT (4);

BS_SET_SYS_INFO ( 0 , system_information_type_1 );

BS_SET_SYS_INFO ( 0 , system_information_type_2 );

BS_SET_SYS_INFO ( 0 , system_information_type_3 );

BS_SET_SYS_INFO ( 0 , system_information_type_4 );

BS_SET_SYS_INFO_SACCH ( 0 , system_information_type_5 );

BS_SET_SYS_INFO_SACCH ( 0 , system_information_type_6 );

BS_SET_SCH ( 0 , BSIC , RFN );

BS_SET_ARFCN ( 0 , ARFCN_BCCH );

BS_SET_POWER ( 0 , -50 );

BS_ON_OFF  ( 0 , TRUE );

AT_SEND ("AT+CFUN=1\r\n",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	AK	Initial

### ATI010: Request Subscriber Number

Description:	This command requests the subscriber numbers.

Preamble:	ATI001

Script:

AT_SEND ("AT+CNUM\r\n",SILENT);

AT_RECEIVE("+CNUM: \"Condat\",\"030390940\",129",SILENT);

AT_RECEIVE("+CNUM: \"Mathias\",\"017165432\",129,,4",SILENT);

AT_RECEIVE("+CNUM: \"ANDREAS\",\"01234567890123456789\",129,65,1,1",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	SAB	Initial

### ATI011: Request Subscriber Number

Description:	This command requests the subscriber numbers.

Preamble:	ATI010

Script:

AT_SEND ("AT+CNUM\r\n",SILENT);

AT_RECEIVE("+CNUM: \"Condat\",\"030390940\",129",SILENT);

AT_RECEIVE("+CNUM: \"Mathias\",\"017165432\",129,,4",SILENT);

AT_RECEIVE("+CNUM: \"ANDREAS\",\"01234567890123456789\",129,65,1,1",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	SAB	Initial

### ATI110: Storing of SMS Profile

Description:	This procedure stores a SMS profile.

Preamble:	ATI001

Script:

AT_SEND ("AT+CSCA=\"+49171123456789\"\r\n",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSMP=25,\"94/05/06,22:10:00-08\",10,20\r\n",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSCB=1,\"1-2,3-4,5-6,7,8,9,10-11,12,13,14\",\"1,3,5\"\r\n",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSAS=2\r\n",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	SAB	Initial

### ATI111: Changing of SMS Profile

Description:	This procedure changes a SMS profile.

Preamble:	ATI110

Script:

AT_SEND ("AT+CSCA=\"0172987654321\"\r\n",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSMP=17,2,3,4\r\n",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSCB=0,\"\",\"\"\r\n",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSCA?\r\n",SILENT);

AT_RECEIVE ("+CSCA: \"0172987654321\",129",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSMP?\r\n",SILENT);

AT_RECEIVE ("+CSMP: 17,2,3,4",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSCB?\r\n",SILENT);

AT_RECEIVE ("+CSCB: 0,\"\",\"\"",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	SAB	Initial

### ATI112: Restoring of SMS Profile

Description:	This procedure restores a SMS profile.

Preamble:	ATI111

Script:

AT_SEND ("AT+CRES=2\r\n",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSCA?\r\n",SILENT);

AT_RECEIVE ("+CSCA: \"49171123456789\",145",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSMP?\r\n",SILENT);

AT_RECEIVE ("+CSMP: 25,\"94/05/06,22:10:00-08\",10,20",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CSCB?\r\n",SILENT);

AT_RECEIVE ("+CSCB: 1,\"1-2,3-4,5-6,7,8,9,10-11,12,13,14\",\"1,3,5\"",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	SAB	Initial

### ATI113: Selecting ME Memory

Description:	This procedure is used to test the selection of ME memory for storing short messages.

Preamble:	ATI001

Script:

AT_SEND ("AT+CPMS=\"ME\",\"SM\",\"ME\"\r\n",SILENT);

AT_RECEIVE("+CPMS: 0,1,0,3,0,1",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CPMS=\"ME\",\"ME\",\"ME\"\r\n",SILENT);

AT_RECEIVE("+CPMS: 0,1,0,1,0,1",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	SAB	Initial

### ATI114: Write a Short Message to ME Memory

Description:	This procedure is used to test whether a short message is written to ME memory successfully.

Preamble:	ATI113

Script:

AT_SEND ("AT+CMGW=\"01711124\"\r\n",SILENT);

AT_RECEIVE("\r\n> ",SILENT);

AT_SEND ("This is the short message\r\n",SILENT);

AT_RECEIVE ("+CMGW: 1",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CMGR=1\r\n",SILENT);

AT_RECEIVE ("+CMGR: \"STO UNSENT\",\"01711124\",\"ANDREAS\",129,17,0,0,167,,,25",SILENT);

AT_RECEIVE ("\r\nThis is the short message",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	SAB	Initial

### ATI020: Request Preferred PLMN List

Description:	This command requests the list of preferred PLMN's.

Preamble:	ATI001

Script:

AT_SEND ("AT+CPOL?\r\n",SILENT);

AT_RECEIVE("+CPOL: 1,2,\"23410\"",SILENT);

AT_RECEIVE("+CPOL: 2,2,\"23455\"",SILENT);

AT_RECEIVE("+CPOL: 3,2,\"23450\"",SILENT);

AT_RECEIVE("+CPOL: 4,2,\"23458\"",SILENT);

AT_RECEIVE("+CPOL: 5,2,\"23430\"",SILENT);

AT_RECEIVE("+CPOL: 6,2,\"23433\"",SILENT);

AT_RECEIVE("+CPOL: 7,2,\"23415\"",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("AT+CPOL?\r\n",SILENT);

AT_RECEIVE("+CPOL: 1,2,\"23410\"",SILENT);

AT_RECEIVE("+CPOL: 2,2,\"23455\"",SILENT);

AT_RECEIVE("+CPOL: 3,2,\"23450\"",SILENT);

AT_RECEIVE("+CPOL: 4,2,\"23458\"",SILENT);

AT_RECEIVE("+CPOL: 5,2,\"23430\"",SILENT);

AT_RECEIVE("+CPOL: 6,2,\"23433\"",SILENT);

AT_RECEIVE("+CPOL: 7,2,\"23415\"",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	SAB	Initial

### ATI030: Read SIM binary field

Description:	This command requests the binary field Efecc from SIM.

Preamble:	ATI001

Script:

AT_SEND ("AT+CRSM=176,28464,0,0,30\r\n",SILENT);

AT_RECEIVE("+CRSM: 144,0,32F40132F45532F40532F48532F40332F43332F451FFFFFFFFFFFFFFFFFF",SILENT);

AT_RECEIVE("OK",SILENT);

History:	13.01.00	AK	Initial

### ATI310: Testing of AT Command Extension Mechanism

Description:

Preamble:	ATI001

Script:

AT_SEND ("AT%H\r\n",SILENT);

AT_RECEIVE("<unsolicited result code>",SILENT);

AT_RECEIVE("Hello",SILENT);

AT_RECEIVE("",SILENT);

AT_RECEIVE("World",SILENT);

AT_RECEIVE("OK",SILENT);

History:	01.02.99	SAB	Initial

### ATI320: Testing of AT Command Extension Mechanism

Description:

Preamble:	ATI001

Script:

AT_SEND ("ATI0\r\n",SILENT);

AT_RECEIVE("<manufacturer1>",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("ATI1\r\n",SILENT);

AT_RECEIVE("<manufacturer2>",SILENT);

AT_RECEIVE("OK",SILENT);

AT_SEND ("ATI9\r\n",SILENT);

AT_RECEIVE("<manufacturer2>",SILENT);

AT_RECEIVE("OK",SILENT);

History:	25.09.00	ENZ	Initial

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |