Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM Protocol Stack

Test Specification

BMISAT

(Edit via File/Properties/Summary!)

| Document Number: | xxxx.yyy.00.100 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2000-Oct-31 |
| Last changed: | 2003-May-12 by XGUTTEFE |
| File Name: | bmisat.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2000-Oct-31 | VO |  | 0.1 |  | 1 |
| 2003-May-12 | XGUTTEFE |  | 0.2 | Draft |  |

Notes:

- Initial version

Table of Contents

1.1	Abbreviations	4

1.2	Terms	5

2	Overview	6

3	Parameters	8

4	TEST CASES	12

4.1	Routing (internal)	12

4.1.1	BMISAT001:  Setup the Routing and the PCO view for the MMI test	12

4.2	MMI Initialisation	13

4.2.1	BMISAT002: Power On Sequence: phonebook read ready indication (only ADN)	13

4.2.2	BMISAT003: Power On Sequence: phonebook read ready indication (FDN enable)	17

4.2.3	BMISAT004:  Continue Power On Sequence: read SMS list and memory status (only ADN)	19

4.2.4	BMISAT005:  Continue Power On Sequence: read SMS list and memory status (FDN enable)	20

4.3	SAT Proactive Command REFRESH	22

4.3.1	BMISAT101: SIM Reset Indication	22

4.3.2	BMISAT102: SIM Activation after SIM Reset Indication.	22

4.3.3	BMISAT201: SAT File Change Notification: no Update Required	23

4.3.4	BMISAT202: SAT File Change Notification: Update Required for SPN, But This Field is not allocated	24

4.3.5	BMISAT203: SAT File Change Notification: Update Required SIM Service Table	24

4.3.6	BMISAT204: SAT File Change Notification: Update Required for GID1	26

4.3.7	BMISAT205: SAT File Change Notification: Update Required for GID2	27

4.3.8	BMISAT206: SAT File Change Notification: Update Required for SPN, SIM Service Table, GID2	28

4.3.9	BMISAT207: SAT File Change Notification: Update Required for CPHS Infomation, SIM Service Table, SMS	29

4.3.10	BMISAT208: SAT File Change Notification: Update Required for FDN	31

4.3.11	BMISAT209: SAT File Change Notification: Update SIM Service Table for Phonebook	33

4.3.12	BMISAT210: SAT File Change Notification: Update Required for FDN and GID1	37

4.3.13	BMISAT211: SAT File Change Notification: Update SIM Service Table for Phonebook with Error Reading of SIM Service Table	39

4.3.14	BMISAT212: SAT File Change Notification: Update SIM Service Table for Phonebook with Error Reading of first ADN Record	40

4.3.15	BMISAT300: Supplementary Service: Closed User Group - Set State/Get State	41

Appendices	43

A.	Acronyms	43

B.	Glossary	43

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

Error! Objects cannot be created from editing field codes.

Figure 1: Mobile-station protocol architecture

This document describes the tests for the whole protocol stack using the MMI Framework.

# Parameters

DECLARATION (PLMN_262_01)
DECLARATION (MCC_262)
DECLARATION (MNC_01)

DECLARATION (EF_CPHS_CINF)

DECLARATION (SAT_FU_IND_EF_KC)

DECLARATION (SAT_FU_IND_EF_SPN)
DECLARATION (SAT_FU_IND_EF_GID1)
DECLARATION (SAT_FU_IND_EF_GID2)
DECLARATION (SAT_FU_IND_EF_SST)
DECLARATION (SAT_FU_IND_EF_SPN_SST_GID2)
DECLARATION (SAT_FU_IND_EF_SST_CINF_SMS)
DECLARATION (SAT_FU_IND_EF_FDN)
DECLARATION (SAT_FU_IND_EF_FDN_SPN_SST)
DECLARATION (SAT_FU_IND_EF_FDN_GID1)

BYTE DUMMY 0

BYTE NUM_0 0
BYTE NUM_1 1
BYTE NUM_2 2
BYTE NUM_3 3
BYTE NUM_4 4
BYTE NUM_5 5
BYTE NUM_10 10
BYTE NUM_17 17
BYTE NUM_255 255

BYTE MAX_SIM_SMS_10 10
BYTE USED_SIM_SMS_0 0

BYTE V_PLMN_PRES 1

/* sim record */

BYTE RECORD_1 1
BYTE RECORD_2 2
BYTE RECORD_3 3
BYTE RECORD_4 4
BYTE RECORD_5 5
BYTE RECORD_6 6

- BYTE LENGTH_ADN 52
BYTE LENGTH_FDN 52

/* SIM IMSI */

FIELD (IMSI) 0x29, 0x26, 0x10, 0x74, 0x11, 0x94, 0x21, 0xFF
ENDFIELD (IMSI, 8)

FIELD (EC_CODES) 0x11, 0xF2, 0xFF, 
		 0x99, 0xF9, 0xFF,
		 0xFF, 0xFF,  0xFF, 
		 0xFF, 0xFF,  0xFF, 
		 0xFF, 0xFF,  0xFF
ENDFIELD (EC_CODES, 15)

/* SIM service table */

FIELD (F_SIM_SRV) 0x0C, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (F_SIM_SRV, 10)

FIELD (F_SIM_SRV_FDN_ENABLED) 0x3C, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (F_SIM_SRV_FDN_ENABLED, 10)

FIELD (F_SIM_SRV_UPDATE) 0x00, 0x00, 0x00, 0x00, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (F_SIM_SRV_UPDATE, 10)

FIELD (F_SIM_SRV_UPDATE_ADN_FDN) 0x3C, 0x00, 0x00, 0x00, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (F_SIM_SRV_UPDATE_ADN_FDN, 10)

/* ADN phonebook records */

FIELD (ADN_RECORD_1) //JIE

0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45, 0x45,

0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45, 0x45, 0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45,	0x45, 0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45, 0x45, 0x4A, 0x49, 0x45, 0x45, 0x45, 0xFF,	0x06, 0x81,

0x30, 0x30, 0x09, 0x49, 0x01, 0xF3, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (ADN_RECORD_1, 52)

FIELD (ADN_RECORD_2)  //ERWIN

0x45, 0x52, 0x57, 0x49, 0x4E, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x06, 0x81,

0x10, 0x27, 0x83, 0x83, 0x99, 0xF9, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (ADN_RECORD_2, 52) //ANDREAS

FIELD (ADN_RECORD_3)

0x41, 0x4E, 0x44, 0x52, 0x45, 0x41, 0x53, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x04, 0x81,

0x10, 0x17, 0x11, 0x42, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (ADN_RECORD_3, 52)

FIELD (ADN_RECORD_4) //BERND

0x42, 0x45, 0x52, 0x4E, 0x44, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x05, 0x81,

0x94, 0x93, 0x90, 0x14, 0x61, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (ADN_RECORD_4, 52)//JVJ

FIELD (ADN_RECORD_5)

0x4A, 0x56, 0x4A, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x05, 0x81,

0x93, 0x48, 0x68, 0x57, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (ADN_RECORD_5, 52)

FIELD (ADN_RECORD_6) //STEFAN

0x53, 0x74, 0x65, 0x66, 0x61, 0x6E, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x06, 0x81,

0x30, 0x30, 0x09, 0x49, 0x11, 0xF7, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (ADN_RECORD_6, 52)

/* ADN phonebook records */

FIELD (FDN_RECORD_1) //JIE

0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45, 0x45,

0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45, 0x45, 0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45,	0x45, 0x4A, 0x49, 0x45, 0x45, 0x45, 0x45, 0x45, 0x45, 0x4A, 0x49, 0x45, 0x45, 0x45, 0xFF,	0x06, 0x81,

0x30, 0x30, 0x09, 0x49, 0x01, 0xF3, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (FDN_RECORD_1, 52)

FIELD (FDN_RECORD_2)  //ERWIN

0x45, 0x52, 0x57, 0x49, 0x4E, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x06, 0x81,

0x10, 0x27, 0x83, 0x83, 0x99, 0xF9, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (FDN_RECORD_2, 52)

FIELD (FDN_RECORD_3)

0x41, 0x4E, 0x44, 0x52, 0x45, 0x41, 0x53, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x04, 0x81,

0x10, 0x17, 0x11, 0x42, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (FDN_RECORD_3, 52)

FIELD (FDN_RECORD_4) //BERND

0x42, 0x45, 0x52, 0x4E, 0x44, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x05, 0x81,

0x94, 0x93, 0x90, 0x14, 0x61, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (FDN_RECORD_4, 52)//JVJ

FIELD (FDN_UPDATE_RECORD_1) //JIE

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (FDN_UPDATE_RECORD_1, 52)

FIELD (FDN_UPDATE_RECORD_2)  //CONDAT

0x43, 0x4F, 0x4E, 0x44, 0x41, 0x54, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x06, 0x81,

0x10, 0x27, 0x83, 0x83, 0x99, 0xF9, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (FDN_UPDATE_RECORD_2, 52)

FIELD (FDN_UPDATE_RECORD_3)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (FDN_UPDATE_RECORD_3, 52)

FIELD (FDN_UPDATE_RECORD_4)

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (FDN_UPDATE_RECORD_4, 52)

FIELD (LND_RECORD_1) //STEFAN

0x53, 0x74, 0x65, 0x66, 0x61, 0x6E, 0xFF, 0xFF,

0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF,	0x06, 0x81,

0x30, 0x30, 0x09, 0x49, 0x11, 0xF7, 0xFF, 0xFF, 0xFF, 0xFF,

0xFF, 0xFF

ENDFIELD (LND_RECORD_1, 52)

FIELD (CPHS_CINF)

0x00, 0x00, 0x00

ENDFIELD (CPHS_CINF, 3)

FIELD (CPHS_CINF_UPDATE)

0xFF, 0xFF, 0xFF

ENDFIELD (CPHS_CINF_UPDATE, 3)

FIELD (SPN_CONDAT)

0x00,

0x43, 0x4F, 0x4E, 0x44, 0x41, 0x54, 0xFF, 0xFF, 
0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF

ENDFIELD (SPN_CONDAT, 17)

FIELD (GID1_11111)

0x01, 0x01, 0x01, 0x01, 0x01

ENDFIELD (GID1_11111, 5)

FIELD (GID2_22222)

0x02, 0x02, 0x02, 0x02, 0x02

ENDFIELD (GID2_22222, 5)

FIELD (GID2_33333)

0x03, 0x03, 0x03, 0x03, 0x03

ENDFIELD (GID2_33333, 5)

/* --- Primitive Parameters --- */

FIELD (SIM_STATUS_DEF) /*128,*/ 0, 0, 0, 0, 0, 0, 0, 0, 0, 0
ENDFIELD (SIM_STATUS_DEF, 10)

FIELD (ME_STATUS_DEF) /*128,*/ 0, 0, 0, 0, 0, 0, 0, 0, 0, 0
ENDFIELD (ME_STATUS_DEF, 10)

FIELD (SIM_STATUS_2REC_UNREAD) /*128,*/ 0x33, 0, 0, 0, 0, 0, 0, 0, 0, 0
ENDFIELD (SIM_STATUS_2REC_UNREAD, 10)

FIELD (SIM_STATUS_3REC) /*128,*/ 0x33, 0x03, 0, 0, 0, 0, 0, 0, 0, 0
ENDFIELD (SIM_STATUS_3REC, 10)

BYTE L_SM7_AA 0x02

BEGINARRAY (MCC_262, 3) 0x02, 0x06, 0x02 ENDARRAY 
BEGINARRAY (MNC_01, 2) 0x00, 0x01 ENDARRAY

/* list of file identifier */

BEGINARRAY (SAT_FU_IND_EF_KC, 2) SIM_KC, (SIM_KC>>8) ENDARRAY

BEGINARRAY (SAT_FU_IND_EF_SPN, 2) SIM_SPN, (SIM_KC>>8) ENDARRAY

BEGINARRAY (SAT_FU_IND_EF_GID1, 2) SIM_GID1, (SIM_KC>>8) ENDARRAY

BEGINARRAY (SAT_FU_IND_EF_GID2, 2) SIM_GID2, (SIM_KC>>8) ENDARRAY

BEGINARRAY (SAT_FU_IND_EF_SST, 2) SIM_SST, (SIM_KC>>8) ENDARRAY

BEGINARRAY (SAT_FU_IND_EF_SPN_SST_GID2, 8) SIM_SPN, (SIM_KC>>8), SIM_KC, (SIM_KC>>8), SIM_SST, (SIM_KC>>8), SIM_GID2, (SIM_KC>>8) ENDARRAY

BEGINARRAY (SAT_FU_IND_EF_SST_CINF_SMS, 6) SIM_SST, (SIM_KC>>8), SIM_CPHS_CINF, (SIM_KC>>8), SIM_SMS, (SIM_KC>>8) ENDARRAY

BEGINARRAY (SAT_FU_IND_EF_FDN, 2) SIM_FDN, (SIM_KC>>8) ENDARRAY

BEGINARRAY (SAT_FU_IND_EF_FDN_SPN_SST, 6) SIM_FDN, (SIM_KC>>8), SIM_SPN, (SIM_KC>>8), SIM_SST, (SIM_KC>>8) ENDARRAY

BEGINARRAY (SAT_FU_IND_EF_FDN_GID1, 4) SIM_FDN, (SIM_KC>>8), SIM_GID1, (SIM_KC>>8) ENDARRAY

BEGIN_PSTRUCT ("plmn", PLMN_262_01)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_262)
	SET_COMP ("mnc",	MNC_01)
ENDSTRUCT

# TEST CASES

## Routing (internal)

### BMISAT001:  Setup the Routing and the PCO view for the MMI test

Description:

Routings for the ACI tests are set.

Preamble:

None

APL		ACI		PS

|		|		|

COMMAND (TAP RESET)
COMMAND (CC RESET)

COMMAND (MM RESET)
COMMAND (SIM RESET)
COMMAND (SS RESET)
COMMAND (MMI RESET)
COMMAND (SMS RESET)
COMMAND (RA RESET)
COMMAND (T30 RESET)
	 |		|		|
COMMAND (TAP REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (RA REDIRECT CLEAR)
COMMAND (T30 REDIRECT CLEAR)
	 |		|		|
COMMAND (MMI REDIRECT CC TAP)
COMMAND (MMI REDIRECT MM TAP)
COMMAND (MMI REDIRECT SIM TAP)
COMMAND (MMI REDIRECT SS TAP)
COMMAND (MMI REDIRECT MMI TAP)
COMMAND (MMI REDIRECT SMS TAP)
COMMAND (MMI REDIRECT RA TAP)
COMMAND (MMI REDIRECT T30 TAP)
	|		|		|

COMMAND (TAP REDIRECT TAP MMI)
COMMAND (MMI REDIRECT MMI TAP)
COMMAND (CST REDIRECT L1 TAP)
	|		|		|

|		|		|

Parametrization:

Primitive	Parameter	Value

History:	14.12.98	AK	Initial

## MMI Initialisation

### BMISAT002: Power On Sequence: phonebook read ready indication (only ADN)

Description:

Preamble:	BMISAT001

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=POWER)

(1)	|		| 	SIM_ACTIVATE_REQ	|

|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|

|		*	<==========================	*

(3)	|		| 	SIM_MMI_INSERT_IND	|

|		*	<==========================	*

(4)	|		| 	SIM_READ_REQ	|

|		*	==========================>	*

(5)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(6)	|		| 	SIM_READ_REQ	|

|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|

|		*	<==========================	*

(8)	|		|	MMR_PLMN_MODE_REQ	|
	|			*==========================>*
(9)	|		|	MMR_REG_REQ	|
	|			*==========================>*
(10)	|		|	MMR_REG_CNF	|
	|			*<==========================*

(11)	|		|	MMR_PLMN_MODE_REQ	|
	|			*==========================>*

(12)	|		|	MNSMS_REPORT_IND	|

|		*	<==========================	*

(13)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(14)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*
(15)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(16)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(17)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(18)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(19)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(20)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(21)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(22)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(23)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(24)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ

- proc	SIM_INITIALISATION

- mmi_pro_file	NOT_USED

- stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF

- error	SIM_INIT_NO_ERROR

- pin_cnt	NUM_3

- puk_cnt	NUM_10

- pin2_cnt	NUM_3

- puk2_cnt	NUM_10

- ec_code	EC_CODES

- pref_lang	NOT_USED

- SIM_MMI_INSERT_IND

- func	SIM_ADN_ENABLED

- sim_serv	F_SIM_SRV

- imsi_field	NOT_USED

- pref_plmn	NOT_USED

- phase	PHASE_2_SIM

- access_acm	NOT_USED

- access_acmmax	NOT_USED

- access_puct	NOT_USED

- SIM_READ_REQ	
	source	SRC_MMI
	offset	NOT_USED
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF	
	datafield	SIM_CPHS_CINF
	error	SIM_NO_ERROR
	length	NUM_3
	trans_data	CPHS_CINF

- SIM_READ_REQ	
	source	SRC_MMI
	offset	NOT_USED
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NOT_USED

- SIM_READ_CNF	
	datafield	SIM_ECC
	error	SIM_NO_ERROR
	length	NOT_USED
	trans_data	NOT_USED

- MMR_PLMN_MODE_REQ
	mode		MODE_AUTO

- MMR_REG_REQ
	service_mode	SERVICE_MODE_FULL

- MMR_REG_CNF
	plmn	PLMN_262_01

- MMR_PLMN_MODE_REQ

- mode	MODE_AUTO

- MNSMS_REPORT_IND

- cause	CS_SMS_READY

- msg_ref	NOT_PRESENT_8BIT

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_1

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_1

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_1

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_2

- length	LENGTH_ADN

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_2

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_2

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_3

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_3

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_3

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_4

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_4

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_4

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_5

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_5

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_5

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_6

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_6

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_6

History:	31.10.00	MFW	Initial

### BMISAT003: Power On Sequence: phonebook read ready indication (FDN enable)

Description:

Preamble:	BMISAT001

APL		ACI		PS

|		|		|

COMMAND (MMI CONFIG KEY_SEQUENCE=POWER)

(1)	|		| 	SIM_ACTIVATE_REQ	|
	|		*	==========================>	*

(2)	|		| 	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

(3)	|		| 	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

(4)	|		| 	SIM_READ_REQ	|
	|		*	==========================>	*

(5)	|		| 	SIM_READ_CNF	|
	|		*	<==========================	*

(6)	|		| 	SIM_READ_REQ	|
	|		*	==========================>	*

(7)	|		| 	SIM_READ_CNF	|
	|		*	<==========================	*

(8)	|		|	MMR_PLMN_MODE_REQ	|
	|			*==========================>*

(9)	|		|	MMR_REG_REQ	|
	|			*==========================>*

(10)	|		|	MMR_REG_CNF	|
	|			*<==========================*

(11)	|		|	MMR_PLMN_MODE_REQ	|
	|			*==========================>*

(12)	|		|	MNSMS_REPORT_IND	|
	|		*	<==========================	*

(13)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*

(14)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(15)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*

(16)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(17)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(18)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(19)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(20)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_REQ	proc	SIM_INITIALISATION

- mmi_pro_file	NOT_USED

- stk_pro_file	NOT_USED

- SIM_ACTIVATE_CNF	error	SIM_INIT_NO_ERROR

- pin_cnt	NUM_3

- puk_cnt	NUM_10

- pin2_cnt	NUM_3

- puk2_cnt	NUM_10

- ec_code	EC_CODES

- pref_lang	NOT_USED

- SIM_MMI_INSERT_IND	func	SIM_FDN_ENABLED

- sim_serv	F_SIM_SRV_FDN_ENABLED

- imsi_field	NOT_USED

- pref_plmn	NOT_USED

- phase	PHASE_2_SIM

- access_acm	NOT_USED

- access_acmmax	NOT_USED

- access_puct	NOT_USED

- SIM_READ_REQ	
	source	SRC_MMI
	offset	NOT_USED
	datafield	SIM_CPHS_CINF
	length	NOT_PRESENT_8BIT
	max_length	NUM_3

- SIM_READ_CNF	
	datafield	SIM_CPHS_CINF
	error	SIM_NO_ERROR
	length	NUM_3
	trans_data	CPHS_CINF

- SIM_READ_REQ	
	source	SRC_MMI
	offset	NOT_USED
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NOT_USED

- SIM_READ_CNF	
	datafield	SIM_ECC
	error	SIM_NO_ERROR
	length	NOT_USED
	trans_data	NOT_USED

- MMR_PLMN_MODE_REQ
	mode	MODE_AUTO

- MMR_REG_REQ
	service_mode	SERVICE_MODE_FULL

- MMR_REG_CNF
	plmn	PLMN_262_01

- MMR_PLMN_MODE_REQ
	mode	MODE_AUTO

- MNSMS_REPORT_IND
	cause	CS_SMS_READY

- msg_ref		NOT_PRESENT_8BIT

- SIM_READ_RECORD_REQ
	source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_1

- length	NOT_USED

- SIM_READ_RECORD_CNF
	datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_1

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_RECORD_1

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_2

- length	LENGTH_FDN

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_2

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_RECORD_2

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_3

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_3

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_RECORD_3

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_4

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_4

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_RECORD_4

History:	31.10.00	MFW	Initial

### BMISAT004:  Continue Power On Sequence: read SMS list and memory status (only ADN)

Description:

Preamble:

BMISAT002

APL		ACI		PS

|		|		|

(1)	|		|	MNSMS_INFO_REQ	|

|		*	==========================>	*

(2)	|		|	MNSMS_INFO_CNF	|

|		*	<==========================	*

(3)	|		|	MNSMS_INFO_REQ	|

|		*	==========================>	*

(4)	|		|	MNSMS_INFO_CNF	|

|		*	<==========================	*

(5)	|		|	MNSMS_CONFIGURE_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_INFO_REQ

- param	DUMMY

- MNSMS_INFO_CNF

- total_sim	MAX_SIM_SMS_10

- used_sim	USED_SIM_SMS_0

- status_sim	NOT_USED

- total_me	NOT_USED

- used_me	NOT_USED

- status_me	NOT_USED

- MNSMS_INFO_REQ

- param	DUMMY

- MNSMS_INFO_CNF

- total_sim	MAX_SIM_SMS_10

- used_sim	USED_SIM_SMS_0

- status_sim	NOT_USED

- total_me	NOT_USED

- used_me	NOT_USED

- status_me	NOT_USED

- MNSMS_CONFIGURE_REQ

- pref_mem_3	NOT_USED

- mt	MT2

- ds	DS1

- mhc	SMS_MHC_PH2

History:	1.11.00	MFW	Initial

### BMISAT005:  Continue Power On Sequence: read SMS list and memory status (FDN enable)

Description:

Preamble:

BMISAT003

APL		ACI		PS

|		|		|

(1)	|		|	MNSMS_INFO_REQ	|

|		*	==========================>	*

(2)	|		|	MNSMS_INFO_CNF	|

|		*	<==========================	*

(3)	|		|	MNSMS_INFO_REQ	|

|		*	==========================>	*

(4)	|		|	MNSMS_INFO_CNF	|

|		*	<==========================	*

(5)	|		|	MNSMS_CONFIGURE_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSMS_INFO_REQ

- param	DUMMY

- MNSMS_INFO_CNF

- total_sim	MAX_SIM_SMS_10

- used_sim	USED_SIM_SMS_0

- status_sim	NOT_USED

- total_me	NOT_USED

- used_me	NOT_USED

- status_me	NOT_USED

- MNSMS_INFO_REQ

- param	DUMMY

- MNSMS_INFO_CNF

- total_sim	MAX_SIM_SMS_10

- used_sim	USED_SIM_SMS_0

- status_sim	NOT_USED

- total_me	NOT_USED

- used_me	NOT_USED

- status_me	NOT_USED

- MNSMS_CONFIGURE_REQ

- pref_mem_3	NOT_USED

- mt	MT2

- ds	DS1

- mhc	SMS_MHC_PH2

History:	1.11.00	MFW	Initial

## SAT Proactive Command REFRESH

### BMISAT101: SIM Reset Indication

Description:

After power-on and SIM activation a primitive SIM_REMOVE_IND is received. If the error code is SIM_NO_ERROR, then it it a SAT command REFRESH (SIM Reset). The result simulated with an unsolicited AT response.

Variants:

<A>....<B>

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
(1)	|		|	SIM_REMOVE_IND	|
	|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_REMOVE_IND
<A>	error	SIM_NO_ERROR
<B>	error	SIM_FATAL_ERROR

History:	31.10.2000	MFW	Initial

### BMISAT102: SIM Activation after SIM Reset Indication.

Description:

After a SAT command REFRESH (SIM Reset) issued by the preamble a primitive SIM_ACTIVATE_IND is received. The error code is evaluated. The result is simulated with an unsolicited AT response.

Variants:

<A>....<C>

Preamble:

<A>	BMISAT101A
<B>	BMISAT101A
<C>	BMISAT101A

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		|	SIM_ACTIVATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_ACTIVATE_IND
<A>	error	SIM_NO_ERROR
<B>	error	SIM_INVALID_PIN_1
<C>	error	SIM_INVALID_PUK_1
<A>	pin_cnt	NUM_3
<B>	pin_cnt	NUM_3
<C>	pin_cnt	NUM_0
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10
	ec_code	EC_CODES
	pref_lang	NOT_USED

- SIM_MMI_INSERT_IND
		func	SIM_ADN_ENABLED
		sim_serv	F_SIM_SRV_UPDATE
		imsi_field	NOT_USED
		pref_plmn	NOT_USED
		phase	PHASE_2_SIM
		access_acm	NOT_USED
		access_acmmax	NOT_USED
		access_puct	NOT_USED

History:	31.10.2000	MFW	Initial

### BMISAT201: SAT File Change Notification: no Update Required

Description:

ACI receives the primitive SIM_FILE_UPDATE_IND which includes various file identifiers of changed EFs. None of the EFs is buffered by the ACI, therefore the result primitive SIM_FILE_UPDATE_RESPONSE is returned without doing anything.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
	val_nr	NUM_1
	file_id	SAT_FU_IND_EF_KC

- SIM_FILE_UPDATE_RES
	source	SRC_MMI
	fu_rsc	SIM_FU_SUCCESS

History:	31.10.2000	MFW	Initial

### BMISAT202: SAT File Change Notification: Update Required for SPN, But This Field is not allocated

Description:

ACI receives the primitive SIM_FILE_UPDATE_IND which includes various file identifiers of changed EFs. Any EF(SPN), EF(GID1), EF(GID2) or EF(SST) in the list triggers rereading of these EFs by the ACI, then the result primitive SIM_FILE_UPDATE_RESPONSE is returned.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
	val_nr	NUM_1
	file_id	SAT_FU_IND_EF_SPN

- SIM_FILE_UPDATE_RES
	source	SRC_MMI
	fu_rsc	SIM_FU_SUCCESS

History:	31.10.2000	MFW	Initial

### BMISAT203: SAT File Change Notification: Update Required SIM Service Table

Description:

ACI receives the primitive SIM_FILE_UPDATE_IND which includes various file identifiers of changed EFs. Any EF(SPN), EF(GID1), EF(GID2) or EF(SST) in the list triggers rereading of these EFs by the ACI, then the result primitive SIM_FILE_UPDATE_RESPONSE is returned.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(5)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(6)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_1
		file_id	SAT_FU_IND_EF_SST

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NUM_0

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_NO_ERROR
		length	NUM_10
		trans_data	F_SIM_SRV_UPDATE

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NOT_USED

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_NO_ERROR
		length	NUM_10
		trans_data		F_SIM_SRV_UPDATE

- SIM_FILE_UPDATE_RES
		source	SRC_MMI
		fu_rsc	SIM_FU_SUCC_ADD

History:	31.10.2000	MFW	Initial

### BMISAT204: SAT File Change Notification: Update Required for GID1

Description:

ACI receives the primitive SIM_FILE_UPDATE_IND which includes various file identifiers of changed EFs. Any EF(SPN), EF(GID1), EF(GID2) or EF(SST) in the list triggers rereading of these EFs by the ACI, then the result primitive SIM_FILE_UPDATE_RESPONSE is returned.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(4)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_1
		file_id	SAT_FU_IND_EF_GID1

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_GID1
		length	NOT_PRESENT_8BIT
		max_length	NUM_5

- SIM_READ_CNF
		datafield	SIM_GID1
		error	SIM_NO_ERROR
		length	NUM_5
		trans_data	GID1_11111

- SIM_FILE_UPDATE_RES
		source	SRC_MMI
		fu_rsc	SIM_FU_SUCCESS

History:	31.10.2000	MFW	Initial

### BMISAT205: SAT File Change Notification: Update Required for GID2

Description:

ACI receives the primitive SIM_FILE_UPDATE_IND which includes various file identifiers of changed EFs. Any EF(SPN), EF(GID1), EF(GID2) or EF(SST) in the list triggers rereading of these EFs by the ACI, then the result primitive SIM_FILE_UPDATE_RESPONSE is returned.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(4)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_1
		file_id	SAT_FU_IND_EF_GID2

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_GID2
		length	NOT_PRESENT_8BIT
		max_length	NUM_5

- SIM_READ_CNF
		datafield	SIM_GID2
		error	SIM_NO_ERROR
		length	NUM_5
		trans_data	GID2_22222

- SIM_FILE_UPDATE_RES
		source	SRC_MMI
		fu_rsc	SIM_FU_SUCCESS

History:	31.10.2000	MFW	Initial

### BMISAT206: SAT File Change Notification: Update Required for SPN, SIM Service Table, GID2

Description:

ACI receives the primitive SIM_FILE_UPDATE_IND which includes various file identifiers of changed EFs. Any EF(SPN), EF(GID1), EF(GID2) or EF(SST) in the list triggers rereading of these EFs by the ACI, then the result primitive SIM_FILE_UPDATE_RESPONSE is returned.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(5)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(6)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(7)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(8)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(9)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(10)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_4
		file_id	SAT_FU_IND_EF_SPN_SST_GID2

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NUM_0

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_NO_ERROR
		length	NUM_10
		trans_data	F_SIM_SRV_UPDATE

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NOT_USED

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_NO_ERROR
		length	NUM_10
		trans_data		F_SIM_SRV_UPDATE

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SPN
		length	NOT_PRESENT_8BIT
		max_length	NUM_17

- SIM_READ_CNF
		datafield	SIM_SPN
		error	SIM_NO_ERROR
		length	NUM_17
		trans_data	SPN_CONDAT

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_GID2
		length	NOT_PRESENT_8BIT
		max_length	NUM_5

- SIM_READ_CNF
		datafield	SIM_GID2
		error	SIM_NO_ERROR
		length	NUM_5
		trans_data	GID2_33333

- SIM_FILE_UPDATE_RES
		source	SRC_MMI
		fu_rsc	SIM_FU_SUCC_ADD

History:	31.10.2000	MFW	Initial

### BMISAT207: SAT File Change Notification: Update Required for CPHS Infomation, SIM Service Table, SMS

Description:

ACI receives the primitive SIM_FILE_UPDATE_IND which includes various file identifiers of changed EFs. Any EF(SPN), EF(GID1), EF(GID2) or EF(SST) in the list triggers rereading of these EFs by the ACI, then the result primitive SIM_FILE_UPDATE_RESPONSE is returned.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(5)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(6)	|		|	MNSMS_INFO_REQ	|

|		*	==========================>	*

(7)	|		|	MNSMS_INFO_CNF	|

|		*	<==========================	*

(8)	|		|	MNSMS_INFO_REQ	|

|		*	==========================>	*

(9)	|		|	MNSMS_INFO_CNF	|

|		*	<==========================	*

(10)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_3
		file_id	SAT_FU_IND_EF_SST_CINF_SMS

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NUM_0

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_NO_ERROR
		length	NUM_10
		trans_data	F_SIM_SRV_UPDATE

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NOT_USED

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_NO_ERROR
		length	NUM_10
		trans_data		F_SIM_SRV_UPDATE

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_CPHS_CINF
		length	NOT_PRESENT_8BIT
		max_length	NUM_3

- SIM_READ_CNF
		datafield	SIM_CPHS_CINF
		error	SIM_NO_ERROR
		length	NUM_3
		trans_data	CPHS_CINF_UPDATE

- MNSMS_INFO_REQ

- param	DUMMY

- MNSMS_INFO_CNF

- total_sim	MAX_SIM_SMS_10

- used_sim	USED_SIM_SMS_0

- status_sim	NOT_USED

- total_me	NOT_USED

- used_me	NOT_USED

- status_me	NOT_USED

- MNSMS_INFO_REQ

- param	DUMMY

- MNSMS_INFO_CNF

- total_sim	MAX_SIM_SMS_10

- used_sim	USED_SIM_SMS_0

- status_sim	NOT_USED

- total_me	NOT_USED

- used_me	NOT_USED

- status_me	NOT_USED

- SIM_FILE_UPDATE_RES
		source	SRC_MMI
		fu_rsc	SIM_FU_SUCC_ADD

History:	31.10.2000	MFW	Initial

### BMISAT208: SAT File Change Notification: Update Required for FDN

Description:

ACI receives the primitive SIM_FILE_UPDATE_IND which includes various file identifiers of changed EFs. Any EF(SPN), EF(GID1), EF(GID2) or EF(SST) in the list triggers rereading of these EFs by the ACI, then the result primitive SIM_FILE_UPDATE_RESPONSE is returned.

Preamble:

BMISAT005

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*

(3)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(4)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*

(5)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(6)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(7)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(8)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(9)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(10)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_1
		file_id	SAT_FU_IND_EF_FDN

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_1

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_1

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_UPDATE_RECORD_1

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_2

- length	LENGTH_FDN

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_2

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_UPDATE_RECORD_2

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_3

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_3

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_UPDATE_RECORD_3

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_4

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_4

- max_record	RECORD_4

- length	LENGTH_FDN
linear_data	FDN_UPDATE_RECORD_4

- SIM_FILE_UPDATE_RES
	source	SRC_MMI
	fu_rsc	SIM_FU_SUCC_ADD

History:	7.11.2000	MFW	Initial

### BMISAT209: SAT File Change Notification: Update SIM Service Table for Phonebook

Description:

SIM service table (for ADN and FDN) is changed. ADN and FDN wll be updated.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(4)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(5)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*
(6)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(7)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(8)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(9)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(10)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(11)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(12)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(13)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(14)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(15)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(16)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*

(17)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(18)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*

(19)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(20)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(21)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(22)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(23)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(24)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(25)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(26)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_1
		file_id	SAT_FU_IND_EF_SST

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NUM_0

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_NO_ERROR
		length	NUM_10
		trans_data		F_SIM_SRV_UPDATE_ADN_FDN

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_1

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_1

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_1

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_2

- length	LENGTH_ADN

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_2

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_2

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_3

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_3

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_3

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_4

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_4

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_4

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_5

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN

- error	SIM_NO_ERROR

- record	RECORD_5

- max_record	RECORD_6

- length	LENGTH_ADN

- linear_data	ADN_RECORD_5

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_ADN

- record	RECORD_6

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_ADN
		error	SIM_NO_ERROR
		record	RECORD_6
		max_record	RECORD_6
		length	LENGTH_ADN
		linear_data	ADN_RECORD_6

- SIM_READ_RECORD_REQ

- source	SRC_MMI
		datafield	SIM_FDN
		record	RECORD_1
		length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN
		error	SIM_NO_ERROR
		record	RECORD_1
		max_record	RECORD_4
		length	LENGTH_FDN
		linear_data	FDN_UPDATE_RECORD_1

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_2

- length	LENGTH_FDN

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_2

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_UPDATE_RECORD_2

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_3

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_3

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_UPDATE_RECORD_3

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_4

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_4

- max_record	RECORD_4

- length	LENGTH_FDN
linear_data	FDN_UPDATE_RECORD_4

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NOT_USED

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_NO_ERROR
		length	NUM_10
		trans_data		F_SIM_SRV_UPDATE_ADN_FDN

- SIM_FILE_UPDATE_RES
		source	SRC_MMI
		fu_rsc	SIM_FU_SUCC_ADD

History:	09.11.2000	MFW	Initial

### BMISAT210: SAT File Change Notification: Update Required for FDN and GID1

Description:

ACI receives the primitive SIM_FILE_UPDATE_IND which includes various file identifiers of changed EFs. Any EF(SPN), EF(GID1), EF(GID2) or EF(SST) in the list triggers rereading of these EFs by the ACI, then the result primitive SIM_FILE_UPDATE_RESPONSE is returned.

Preamble:

BMISAT005

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*

(3)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(4)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*

(5)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(6)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(7)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(8)	|		|	SIM_READ_RECORD_REQ	|
	|			*==========================>*
(9)	|		|	SIM_READ_RECORD_CNF	|
	|			*<==========================*

(10)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(11)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(12)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_2
		file_id	SAT_FU_IND_EF_FDN_GID1

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_1

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_1

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_UPDATE_RECORD_1

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_2

- length	LENGTH_FDN

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_2

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_UPDATE_RECORD_2

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_3

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_3

- max_record	RECORD_4

- length	LENGTH_FDN

- linear_data	FDN_UPDATE_RECORD_3

- SIM_READ_RECORD_REQ

- source	SRC_MMI

- datafield	SIM_FDN

- record	RECORD_4

- length	NOT_USED

- SIM_READ_RECORD_CNF

- datafield	SIM_FDN

- error	SIM_NO_ERROR

- record	RECORD_4

- max_record	RECORD_4

- length	LENGTH_FDN
linear_data	FDN_UPDATE_RECORD_4

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_GID1
		length	NOT_PRESENT_8BIT
		max_length	NUM_5

- SIM_READ_CNF
		datafield	SIM_GID1
		error	SIM_NO_ERROR
		length	NUM_5
		trans_data	GID1_11111

- SIM_FILE_UPDATE_RES
	source	SRC_MMI
	fu_rsc	SIM_FU_SUCC_ADD

History:	09.11.2000	MFW	Initial

### BMISAT211: SAT File Change Notification: Update SIM Service Table for Phonebook with Error Reading of SIM Service Table

Description:

SIM service table (for ADN and FDN) is changed. ADN and FDN wll be updated.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(5)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(6)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_1
		file_id	SAT_FU_IND_EF_SST

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NUM_0

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_FATAL_ERROR
		length	NUM_10
		trans_data	F_SIM_SRV_UPDATE_ADN_FDN

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NOT_USED

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_FATAL_ERROR
		length	NUM_10
		trans_data	F_SIM_SRV_UPDATE_ADN_FDN

- SIM_FILE_UPDATE_RES
		source	SRC_MMI
		fu_rsc	SIM_FU_SUCC_ADD

History:	09.11.2000	MFW	Initial

### BMISAT212: SAT File Change Notification: Update SIM Service Table for Phonebook with Error Reading of first ADN Record

Description:

SIM service table (for ADN and FDN) is changed. ADN and FDN wll be updated.

Preamble:

BMISAT004

APL		ACI		PS
	|		|		|
TIMEOUT(1000)
(1)	|		| 	SIM_FILE_UPDATE_IND	|
	|		*	<==========================	*

(2)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(3)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(4)	|		|	SIM_READ_REQ	|
	|		*	==========================>	*

(5)	|		|	SIM_READ_CNF	|
	|		*	<==========================	*

(6)	|		| 	SIM_FILE_UPDATE_RES	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	NUM_1
		file_id	SAT_FU_IND_EF_SST

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NUM_0

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_FATAL_ERROR
		length	NUM_10
		trans_data	F_SIM_SRV_UPDATE_ADN_FDN

- SIM_READ_REQ
		source	SRC_MMI
		offset	NUM_0
		datafield	SIM_SST
		length	NOT_PRESENT_8BIT
		max_length	NOT_USED

- SIM_READ_CNF
		datafield	SIM_SST
		error	SIM_NO_ERROR
		length	NUM_10
		trans_data	F_SIM_SRV_UPDATE_ADN_FDN

- SIM_FILE_UPDATE_RES
		source	SRC_MMI
		fu_rsc	SIM_FU_SUCC_ADD

History:	09.11.2000	MFW	Initial

### BMISAT300: Supplementary Service: Closed User Group - Set State/Get State

Description:

Set parameter for cug: cug_index, cug_mode, cug_info MFW to ACI,

Get parameter for cug: cug_index, cug_mode, cug_info ACI to MFW

Preamble:

BMISAT002

APL		ACI		PS

|		|		|

TIMEOUT_WAIT (2000)

COMMAND (MMI CONFIG KEY_SEQUENCE= LEFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= LEFT)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= DOWN)

TIMEOUT_WAIT (1000)

COMMAND (MMI CONFIG KEY_SEQUENCE= LEFT)

TIMEOUT_WAIT (4000)

COMMAND (MMI CONFIG KEY_SEQUENCE= RIGHT)

|		|		|

Parametrization:

Primitive	Parameter	Value

History:	07.11.2000	MFW	Initial

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |