- GSM Protocol Stack

- Test Specification

CC

Author:	TI Berlin AG
	Alt-Moabit 90a
	D-10559 Berlin
	Germany





Date:	11-December-2002
Document No.:	6147.400.97.116
File:	CC.DOC

Table of Contents

0.1	Document History	10

0.2	Abbreviations	14

0.3	Terms	16

1	Overview	16

2	Parameters	19

3	TEST CASES	114

3.1	Routing (internal)	114

3.1.1	CC000: Setup the routing and PCO view for the CC test	114

3.2	Basic Preambles	115

3.2.1	CC001: Configuration of connection element	115

3.3	Call Establishment: Mobile Originated Call	115

3.3.1	CC011: Establish Mobile Originated Call - Data (U0)	115

3.3.2	CC012: Establish Mobile Originated Call - Speech (U0)	117

3.3.3	CC013: Establish Mobile Originated Call - Speech (U0)	118

3.3.4	CC021: MM connection established for basic call - data (U0.1)	119

3.3.5	CC022: MM connection established - basic call - speech (U0.1)	120

3.3.6	CC023: MM connection established - basic call - speech line 2(U0.1)	122

3.3.7	CC024: MM connection established - emergency speech call (U0.1)	123

3.3.8	CC025: PROGRESS message received (U1)	124

3.3.9	CC026: CALL PROCEEDING message received - speech (U1)	125

3.3.10	CC054: CALL PROCEEDING message received - data (U1)	126

3.3.11	CC027: ALERT message received (U1)	127

3.3.12	CC028: (GSM )TCH assignment from network during call establishment (U1)	128

3.3.13	CC029: Facility information element sent to  network (U1)	129

3.3.14	CC030: Facility information element received from network (U1)	130

3.3.15	CC031: STATUS message with different call state received from network (U1)	131

3.3.16	CC032: CONNECT message received - speech (U1)	133

3.3.17	CC033: PROGRESS message received (U3)	135

3.3.18	CC034: CALL PROCEEDING message received (U3)	136

3.3.19	CC035: ALERT message received (U3)	138

3.3.20	CC036: TCH assignment from network during call establishment (U3)	139

3.3.21	CC037: Facility information element sent to  network (U3)	140

3.3.22	CC038: Facility information element received from network (U3)	141

3.3.23	CC040: STATUS message with different call state received from network (U3)	142

3.3.24	CC041: CONNECT message received - speech (U3)	144

3.3.25	CC043: TCH assignment from network during call establishment (U3), data	146

3.3.26	CC044: PROGRESS message received (U4)	147

3.3.27	CC045: CALL PROCEEDING message received (U4)	148

3.3.28	CC046: ALERT message received (U4)	149

3.3.29	CC047: TCH assignment from network during call establishment (U4)	150

3.3.30	CC048: Facility information element sent to network (U4)	151

3.3.31	CC049: Facility information element received from network (U4)	152

3.3.32	CC051: STATUS message with different call state received from network (U4)	153

3.3.33	CC053: CONNECT message received - data (U4)	155

3.3.34	CC060: CC receives MMCC_PROMPT_IND	157

3.3.35	CC061: CC receives MNCC_PROMPT_RSP	158

3.3.36	CC062: CC receives MNCC_PROMPT_REJ	159

3.3.37	CC063: CC receives CC ESTABLISHMENT message, FR mobile only	160

3.3.38	CC064: CC receives CC ESTABLISHMENT message, EFR_FR_HR mobile	161

3.3.39	CC065: CC receives RECALL	162

3.3.40	CC066: CC receives RECALL, user wants connection	163

3.3.41	CC067: CC receives RECALL, user refuses RECALL	165

3.3.42	CC068: CC receives RECALL, no answer from user	166

3.3.43	CC069: Timeout of T332	167

3.3.44	CC070: CC receives CC ESTABLISHMENT message, no RECALL from net	168

3.4	Call Establishment: Mobile Terminated Call	170

3.4.1	CC081: SETUP message received from network (U0)	170

3.4.2	CC082: SETUP message received from network (U0)	172

3.4.3	CC083: STATUS message with different call state received from network (U0)	174

3.4.4	CC084: CALL PROCEEDING message received from network (U0)	175

3.4.5	CC085: TI flag set: incoming SETUP message ignored (U0)	177

3.4.6	CC086: STATUS ENQUIRY message received from network (U0)	178

3.4.7	CC087: SETUP message received from network (U0), CTM Support by MS	179

3.4.8	CC092: Test of incoming SETUP message successful (U6)	181

3.4.9	CC093: TI flag set: incoming SETUP message rejected (U6)	183

3.4.10	CC094: TI is a reserved value: incoming SETUP message rejected (U6)	184

3.4.11	CC097: All transactions in use: incoming SETUP message rejected (U0)	185

3.4.12	CC098: Compatibility check failed: incoming SETUP message rejected (U0)	190

3.4.13	CC100: Facility information element sent to network (U9)	192

3.4.14	CC101: Facility information element received from network (U9)	193

3.4.15	CC102: STATUS message with different call state received from network (U9)	194

3.4.16	CC110: TCH assignment received from network during call establishment (U9)	196

3.4.17	CC112: Alerting in course of call establishment (U9)	197

3.4.18	CC113: Confirmation from MMI (U9)	198

3.4.19	CC115: TCH assignment from network during call establishment (U7)	199

3.4.20	CC116: Facility information element sent to network (U7)	200

3.4.21	CC117: Facility information element received from network (U7)	201

3.4.22	CC118: STATUS message with different call state received from network (U7)	202

3.4.23	CC119: Confirmation from MMI (U7)	204

3.4.24	CC123: Facility information element sent to network (U8)	205

3.4.25	CC124: Facility information element received from network (U8)	206

3.4.26	CC125: STATUS message with different call state received from network (U8)	207

3.4.27	CC126: No response from the network (U8)	209

3.4.28	CC130: CONNECT ACKNOWLEDGE message received (U8)	210

3.5	Call Active (state = U10)	211

3.5.1	CC221: Notification received from network	211

3.5.2	CC222: Notification sent to  network	212

3.5.3	CC223: Facility information element received from network	213

3.5.4	CC224: Facility information element sent to  network	214

3.5.5	CC225: Start Dual Tone Multi Frequency (DTMF)	215

3.5.6	CC226: Stop Dual Tone Multi Frequency (DTMF)	217

3.5.7	CC228: Connection interrupted - reestablishment successful	218

3.5.8	CC229: Connection interrupted - reestablishment unsuccessful	219

3.5.9	CC230: STATUS ENQUIRY message received from network	220

3.5.10	CC231: STATUS message received from network: state compatible	221

3.5.11	CC232: STATUS message received from network: state incompatible	223

3.5.12	CC243: HOLD requested by mobile station	225

3.5.13	CC244: Network acknowledges HOLD request by mobile station	226

3.5.14	CC245: Network acknowledges HOLD request with FACILITY message	227

3.5.15	CC246: Network rejects HOLD request by mobile station	228

3.5.16	CC247: RETRIEVE requested by mobile station	229

3.5.17	CC248: Network acknowledges RETRIEVE request by mobile station	230

3.5.18	CC249: Network acknowledges RETRIEVE request with FACILITY message	231

3.5.19	CC250: Network rejects RETRIEVE request by mobile station	232

3.5.20	CC251: HOLD requested by mobile station followed by Call Establishment	233

3.5.21	CC261: In-call modification started by MMI (Mobile Originated)	235

3.5.22	CC262: In-call modification successful (Mobile Originated)	236

3.5.23	CC263: In-call modification unsuccessful (Mobile Originated)	237

3.5.24	CC264: No response to in-call modification request (Mobile Originated)	238

3.5.25	CC267: Successful reestablishment during modification (Mobile Originated)	239

3.5.26	CC265: In-call modification accepted by MS (Mobile Terminated)	241

3.5.27	CC266: In-call modification rejected by MS (Mobile Terminated)	243

3.5.28	CC280: In-call modification started by MMI (Mobile Terminated)	244

3.5.29	CC281: In-call modification successful (Mobile Terminated)	245

3.5.30	CC271: Send user data - USER USER message (Mobile Originated)	246

3.5.31	CC272: Send user data - CONGESTION message (Mobile Originated)	247

3.5.32	CC273: Receive user data - USER USER message (Mobile Terminated)	248

3.5.33	CC274: Receive user data - CONGESTION message (Mobile Terminated Call)	249

3.6	Call Clearing	250

3.6.1	CC301: STATUS message with different call state received from network (U0)	250

3.6.2	CC302: STATUS message with same call state received from network (U0)	252

3.6.3	CC303: STATUS ENQUIRY message received from network (U0)	253

3.6.4	CC304: RELEASE COMPLETE message received from network (U0)	254

3.6.5	CC305: CALL PROCEEDING message received from network (U0)	255

3.6.6	CC306: PROGRESS message received (U0)	257

3.6.7	CC307: Facility information element received from network (U0)	259

3.6.8	CC308: SETUP message received with the TI incorrectly set (U0)	260

3.6.9	CC312: MM connection rejected (U0.1)	262

3.6.10	CC321: No response from BS - timer expiry (U0.1)	263

3.6.11	CC322: Mobile originated call release requested (U1)	264

3.6.12	CC323: SETUP rejected: RELEASE COMPLETE message received (U1)	265

3.6.13	CC324: Mobile originated call release requested (U1)	266

3.6.14	CC325: No response from BS - timer expiry (U1)	267

3.6.15	CC326: Error from network while awaiting call establishment (U1)	268

3.6.16	CC331: Mobile originated call release requested (U3)	269

3.6.17	CC332: No response from BS - timer  expiry (U3)	270

3.6.18	CC333: DISCONNECT message received from network (U3)	271

3.6.19	CC334: RELEASE message received (U3)	273

3.6.20	CC335: Error from network during call establishment (U3)	275

3.6.21	CC341: Mobile originated call release requested (U4)	276

3.6.22	CC342: DISCONNECT message received from network (U4)	277

3.6.23	CC343: RELEASE message received (U4)	279

3.6.24	CC344: Error from network during call establishment (U4)	281

3.6.25	CC361: Mobile originated call release requested (U9)	282

3.6.26	CC362: DISCONNECT message received from network (U9)	283

3.6.27	CC363: RELEASE message received (U9)	285

3.6.28	CC364: Error from network during call establishment (U9)	287

3.6.29	CC371: Mobile originated call release requested (U7)	288

3.6.30	CC372: DISCONNECT message received from network (U7)	289

3.6.31	CC373: RELEASE message received (U7)	291

3.6.32	CC374: Error from network during call establishment (U7)	293

3.6.33	CC381: Mobile originated call release requested (U8)	294

3.6.34	CC382: No response from BS - timer  expiry (U8)	295

3.6.35	CC383: DISCONNECT message received from network (U8)	296

3.6.36	CC384: RELEASE message received (U8)	298

3.6.37	CC385: Error from network during call establishment (U8)	300

3.6.38	CC402: Mobile originated call release requested (U10)	301

3.6.39	CC407: DISCONNECT message received from network (U10)	302

3.6.40	CC408: DISCONNECT message received from network (U10)	303

3.6.41	CC418: RELEASE COMPLETE message received from network (U10)	304

3.6.42	CC404: RELEASE message received - mobile originated call release (U11)	305

3.6.43	CC405: No response to DISCONNECT message (U11)	307

3.6.44	CC415: No response to RELEASE message (U11)	309

3.6.45	CC409: No in-band tones - release by MS - mobile originated call release (U12)	311

3.6.46	CC412: In-band tones - release follows hook-on (U12)	313

3.6.47	CC420: DISCONNECT received (U3), CCBS, in-band tones	315

3.6.48	CC421: DISCONNECT received (U3), CCBS, no in-band tones	316

3.6.49	CC422: RELEASE sent with CCBS activation	317

3.7	Bearer Capability Handling	319

3.7.1	CC500: MOC, Speech, Fullrate only	319

3.7.2	CC501: MOC, Speech, Fullrate and Halfrate	321

3.7.3	CC502: MOC, Speech, Fullrate, Halfrate and Enhanced Fullrate	323

3.7.4	CC503: MOC, Speech, Fullrate and Enhanced Fullrate	325

3.7.5	CC900: MOC, EC-Speech, Fullrate, Halfrate and Enhanced Fullrate	327

3.7.6	CC901: MTC, Speech, accepted, with EFR	328

3.7.7	CC902: MOC, Speech, Fullrate, Halfrate Enhanced Fullrate, AMR Half- & Fullrate	330

3.7.8	CC903: MTC, Speech, accepted, with AMR_FR	332

3.7.9	CC504: MTC, Speech, accepted	334

3.7.10	CC505: MTC, no Bearer Capabilities, accepted	336

3.7.11	CC506: MOC, BS21, 3.1 kHz Audio, Transparent, 0.3 kBit/s (I)	338

3.7.12	CC507: MOC, BS21, 3.1 kHz Audio, Transparent, 0.3 kBit/s (II)	340

3.7.13	CC508: MOC, BS21, 3.1 kHz Audio, Transparent, 0.3 kBit/s (III)	342

3.7.14	CC509: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 0.3 kBit/s (I)	343

3.7.15	CC510: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 0.3 kBit/s (II)	345

3.7.16	CC511: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 0.3 kBit/s (III)	347

3.7.17	CC512: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 0.3 kBit/s (IV)	349

3.7.18	CC513: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s accepted	350

3.7.19	CC514: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s negotiated (I)	352

3.7.20	CC515: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s negotiated (II)	354

3.7.21	CC516: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s negotiated (III)	356

3.7.22	CC517: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s rejected	358

3.7.23	CC518: MTC, BS21, UDI Transparent 0.3 kBit/s accepted	360

3.7.24	CC519: MTC, BS21, UDI Transparent 0.3 kBit/s negotiated (I)	362

3.7.25	CC520: MTC, BS21, UDI Transparent 0.3 kBit/s negotiated (II)	364

3.7.26	CC521: MTC, BS21, UDI Transparent 0.3 kBit/s negotiated (III)	366

3.7.27	CC522: MTC, BS21, UDI 0.3 kBit/s rejected	368

3.7.28	CC523: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s accepted	370

3.7.29	CC524: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s negotiated (I)	372

3.7.30	CC525: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s negotiated (II)	374

3.7.31	CC526: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s negotiated (III)	376

3.7.32	CC527: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s rejected	378

3.7.33	CC528: MTC, BS21, UDI Non-Transparent 0.3 kBit/s accepted	380

3.7.34	CC529: MTC, BS21, UDI Non-Transparent 0.3 kBit/s negotiated (I)	382

3.7.35	CC530: MTC, BS21, UDI Non-Transparent 0.3 kBit/s negotiated (II)	384

3.7.36	CC531: MTC, BS21, UDI Non-Transparent 0.3 kBit/s negotiated (III)	386

3.7.37	CC532: MTC, BS21, UDI 0.3 kBit/s rejected	388

3.7.38	CC533: MOC, BS22, 3.1 kHz Audio, Transparent, 1.2 kBit/s	390

3.7.39	CC534: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 1.2 kBit/s	392

3.7.40	CC535: MTC, BS21, 3.1 kHz Audio Transparent 1.2 kBit/s accepted	394

3.7.41	CC536: MTC, BS21, UDI Transparent 1.2 kBit/s accepted	396

3.7.42	CC537: MTC, BS22, 3.1 kHz Audio Non-Transparent 1.2 kBit/s accepted	398

3.7.43	CC538: MTC, BS22, UDI Non-Transparent 1.2 kBit/s accepted	400

3.7.44	CC539: MOC, BS24, 3.1 kHz Audio, Transparent, 2.4 kBit/s	402

3.7.45	CC540: MOC, BS24, 3.1 kHz Audio, Non-Transparent, 2.4 kBit/s	404

3.7.46	CC541: MTC, BS24, 3.1 kHz Audio Transparent 2.4 kBit/s accepted	406

3.7.47	CC542: MTC, BS24, UDI Transparent 2.4 kBit/s accepted	408

3.7.48	CC543: MTC, BS24, 3.1 kHz Audio Non-Transparent 2.4 kBit/s accepted	410

3.7.49	CC544: MTC, BS24, UDI Non-Transparent 2.4 kBit/s accepted	412

3.7.50	CC545: MOC, BS25, 3.1 kHz Audio, Transparent, 4.8 kBit/s	414

3.7.51	CC546: MOC, BS25, 3.1 kHz Audio, Non-Transparent, 4.8 kBit/s	416

3.7.52	CC547: MTC, BS25, 3.1 kHz Audio Transparent 4.8 kBit/s accepted	418

3.7.53	CC548: MTC, BS25, UDI Transparent 4.8 kBit/s accepted	420

3.7.54	CC549: MTC, BS25, 3.1 kHz Audio Non-Transparent 4.8 kBit/s accepted	422

3.7.55	CC550: MTC, BS25, UDI Non-Transparent 4.8 kBit/s accepted	424

3.7.56	CC551: MOC, BS26, 3.1 kHz Audio, Transparent, 9.6 kBit/s	426

3.7.57	CC552: MOC, BS26, 3.1 kHz Audio, Non-Transparent, 9.6 kBit/s	428

3.7.58	CC553: MTC, BS26, 3.1 kHz Audio Transparent 9.6 kBit/s accepted	430

3.7.59	CC554: MTC, BS26, UDI Transparent 9.6 kBit/s accepted	432

3.7.60	CC555: MTC, BS26, 3.1 kHz Audio Non-Transparent 9.6 kBit/s accepted	434

3.7.61	CC556: MTC, BS26, UDI Non-Transparent 9.6 kBit/s accepted	436

3.7.62	CC571: MTC, BS26, 3.1 kHz Audio Non-Transparent 9.6 kBit/s negotiated	438

3.7.63	CC557: MOC, BS23, 3.1 kHz Audio, Transparent, 1200 / 75 Bit/s	440

3.7.64	CC558: MOC, BS23, 3.1 kHz Audio, Non-Transparent, 1200 / 75 Bit/s	442

3.7.65	CC559: MOC, BS61, Alternate Speech / Transparent Async	444

3.7.66	CC560: MOC, BS61, Alternate Speech / Non-Transparent Async	446

3.7.67	CC561: MOC, BS81, Sequential Speech / Transparent Async	448

3.7.68	CC562: MOC, BS81, Sequential Speech / Non-Transparent Async	450

3.7.69	CC563: MOC, TS61, Alternate Speech / Transparent Fax	452

3.7.70	CC564: MOC, TS62, Automatic Fax	454

3.7.71	CC565: MTC, BS61, Alternate Speech / Transparent Asynchronous	456

3.7.72	CC566: MTC, BS61, Alternate Speech / Non-Transparent Asynchronous	458

3.7.73	CC567: MTC, BS81, Sequential Speech / Transparent Asynchronous	460

3.7.74	CC568: MTC, BS81, Sequential Speech / Non-Transparent Asynchronous	462

3.7.75	CC569: MTC, TS61, Alternate Speech / transparent Fax	464

3.7.76	CC570: MTC, TS62, Automatic transparent Fax	466

3.7.77	CC572: MTC, Non-Transparent 14.4 kBit/s accepted	468

3.7.78	CC573: MTC, Non-Transparent 14.4 kBit/s negotiated (I)	470

3.7.79	CC574: MTC Transparent 14.4 kBit/s negotiated (II)	472

3.7.80	CC575: MTC, Non-Transparent 14.4 kBit/s rejected	474

3.7.81	CC576: MOC, Transparent, 14.4 kBit/s	476

3.7.82	CC577: MOC, Transparent, 14.4 kBit/s, rejected	478

3.7.83	CC578: MOC, Non-Transparent, 14.4 kBit/s	479

3.7.84	CC579: Negotiation of Intermediate Rate, 29.2.3.5	481

3.7.85	CC580: MTC, TS61, Alternate Speech / transparent Fax, SNS=5, with BC	483

3.7.86	CC581: MTC, TS61, Alternate Speech / transparent Fax, SNS=5, no BC	485

3.7.87	CC582: MTC, no Bearer Capabilities, accepted, SNS=0	487

3.7.88	CC583: MTC, TS61, Alternate Speech / transparent Fax, SNS=1, no BC	489

3.7.89	CC584: MTC, TS61, Alternate Speech / transparent Fax, SNS=1, with BC	491

3.7.90	CC585: MTC, TS62, Automatic transparent Fax, SNS=2, no BCs	493

3.7.91	CC586: MTC, BS61, Alternate Speech / Transparent Asynchronous, SNS=3, no BC	495

3.7.92	CC587: MTC, BS61, Alternate Speech / Transparent Asynch, SNS=3, with BC	497

3.7.93	CC588: MTC, BS61, Alternate Speech / Non-Transparent Asynch, SNS=3, no BCs	499

3.7.94	CC589: MTC, BS61, Alternate Speech / Non-Transp Asynch, SNS=3, with BCs	501

3.7.95	CC590: MTC, BS24, 3.1 kHz Audio Transparent 2.4 kBit/s,SNS=4, no BCs	503

3.7.96	CC591: MTC, BS61, Alternate Speech / Transparent Asynchronous, SNS=6, no BC	505

3.7.97	CC592: MTC, BS61, Alternate Speech / Transparent Asynch, SNS=6, with BC	507

3.7.98	CC593: MTC, BS61, Alternate Speech / Non-Transparent Asynch, SNS=6, no BCs	509

3.7.99	CC594: MTC, BS61, Alternate Speech / Non-Transp Asynch, SNS=6, with BCs	511

3.7.100	CC595: Sequential Speech / Transparent Asynchronous, SNS=7, no BC	513

3.7.101	CC596: MTC Transparent 14.4 kBit/s negotiated (III)	515

3.7.102	CC600: Bearer capability coding	516

3.7.103	CC601: Bearer capability decoding	517

3.7.104	CC602: Bearer capability decoding, successfull	518

3.8	DTMF Handling	520

3.8.1	CC610: Custumer reported problem with DTMF with manual DTMF start/stop	520

3.8.2	CC611: Custumer reported problem with DTMF in far east country	525

3.8.3	CC612: DTMF start attempt in state U3	529

3.9	Diconnect with poor network	530

3.9.1	CC620: DISCONNECT in state U1 with poor network	530

3.9.2	CC621: DISCONNECT in state U1 with poor network, bad release sequence	533

3.9.3	CC622: DISCONNECT in state U3 with poor network	535

3.10	MOC Bearer Capability Negotiation	537

3.10.1	CC650: MOC, BS26, 3.1 kHz Audio, CE both	537

3.10.2	CC655: MOC, BS61, Alternate Speech/Data, CE both, RI CIRCULAR	540

3.10.3	CC660: MOC, BS81, Speech followed by Data, CE both, RI SEQUENTIAL	542

3.10.4	CC700: MOC, TS61, Accepted by the network	544

3.10.5	CC705: MOC, TS61/62, Negotiation FAX first	545

3.10.6	CC710: MOC, TS61/62, Negotiation Speach first	547

3.10.7	CC720: MTC, BS26, 3.1 kHz Audio Non-Transparent 9.6 kBit/s negotiated TEST	548

3.11	Engineering Mode	549

3.11.1	CC750: MM connection established for basic call - data (U0.1)	549

3.11.2	CC751: CONNECT message received - speech (U3)	551

Document Control

ã Copyright Condat AG, 2002.

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat AG reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat AG. Condat AG accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a licence agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat AG.

- Condat AG

Alt Moabit 90a

10559 Berlin

Germany

- Telephone:	+49.30.3983-0

Fax:	+49.30.3983-1300

Internet:	http://www.condat.de

E-mail:	gsm@condat.de

## Document History

Document Id.	Date	Author	Remarks

6147.400.97.100	25-Apr-97	LE et al	Initial
6147.400.97.101	01-Aug-01	JHU, SBK	Converted to TAP2, used partially partial ARRAYs
6147.400.97.102	22-Aug-01	SBK	Modified due to consideration of +CBST in MTCs
6147.400.97.103	24-Aug-01	SBK	Adapt to outband flow control for MOCs;
			improved due to some tdscheck results
6147.400.97.104	14-Dec-01	SBK	Finalized conversion to TAP2: use MUTE
6147.400.97.105	18-Feb-02	OT	New Testcases for AMR implementation.
6147.400.97.106	15-Mar-02	HM	Changes due to LND implementaion.
6147.400.97.107	18-Mar-02	HM	Changed bearer caps in MNCC_CONFIGURE_REQ to
			bearer caps parameters.
6147.400.97.108	19-Mar-02	HM	Reworked CC422, CC066, call confirmed now in
			WIN32 as in target without CC capabilities.
6147.400.97.109	10-Apr-02	HM	Changed document for CD.
	17-July-02	JK	MOC: for transaparent mode no octet 6f in BC sent ,
			MTC: if UIMI or WAIUR in BC rcvd not present,then
			no BC octets 6d, 6e, 6f in confirmation massege.
			(CC-ENH-2378)
6147.400.97.110	12-Jul-02	HM	CC_CAUSE_USER_BUSY if no slot left.
6147.400.97.111	15-Jul-02	HM	ESTCS_EMERGE for emergency call.
6147.400.97.112	30-Jul-02	SBK	Adapted at cause concept realization
6147.400.97.113	27-Sep-02	SBK	Added CC267 (issue 4992).
	23-Sep-02	MSB	Merge from gprs_1.3.3 to g23m.
	23-Sep-02	SBK	Corrected merge, remove obsolete CC598
6147.400.97.114	07-Nov-02	SBK	Finalized Cause Concept, rebase with g23m
6147.400.97.115	02-Dec-2002	FK	Added CTM support to existing test cases
6147.400.97.116	11-Dec-02	SBK/HM	CC230 revised, CC233 new after PS change

References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat AG

[39]	Service Access Point MNCC
	6147.101.96.100; Condat AG

[40]	Service Access Point MNSS
	6147.102.96.100; Condat AG

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat AG

[42]	Service Access Point MMCC
	6147.104.97.100; Condat AG

[43]	Service Access Point MMSS
	6147.105.97.100; Condat AG

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat AG

[45]	Service Access Point RR
	6147.107.97.100; Condat AG

[46]	Service Access Point SIM
	6147.108.97.100; Condat AG

[47]	Service Access Point MPH
	6147.109.96.100; Condat AG

[48]	Service Access Point DL
	6147.110.96.100; Condat AG

[49]	Service Access Point MDL
	6147.111.96.100; Condat AG

[50]	Service Access Point PH
	6147.112.97.100; Condat AG

[51]	Service Access Point MMI
	6147.113.96.100; Condat AG

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat AG

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat AG

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat AG

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat AG

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat AG

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat AG

[58]	Users Guide
	6147.300.96.100; Condat AG

[59]	Test Specification CC
	6147.400.97.100; Condat AG

[60]	Test Specification SS
	6147.401.97.100; Condat AG

[61]	Test Specification SMS
	6147.402.97.100; Condat AG

[62]	Test Specification MM
	6147.403.97.100; Condat AG

[63]	Test Specification RR
	6147.404.97.100; Condat AG

[64]	Test Specification DL
	6147.405.97.100; Condat AG

[65]	Test Specification CCD
	6147.406.97.100; Condat AG

[66]	SDL Specification CC
	6147.500.97.100; Condat AG

[67]	SDL Specification SS
	6147.501.97.100; Condat AG

[68]	SDL Specification SMS
	6147.502.97.100; Condat AG

[69]	SDL Specification MM
	6147.503.97.100; Condat AG

[70]	SDL Specification RR
	6147.504.97.100; Condat AG

[71]	SDL Specification DL
	6147.505.97.100; Condat AG

[72]	Message Specification CC
	6147.600.97.100; Condat AG

[73]	Message Specification SS
	6147.601.97.100; Condat AG

[74]	Message Specification SMS
	6147.602.97.100; Condat AG

[75]	Message Specification MM
	6147.603.97.100; Condat AG

[76]	Message Specification RR
	6147.604.97.100; Condat AG

[77]	Message Specification DL
	6147.605.97.100; Condat AG

[78]	Technical Documentation CC
	6147.700.97.100; Condat AG

[79]	Technical Documentation SS
	6147.701.97.100; Condat AG

[80]	Technical Documentation SMS
	6147.702.97.100; Condat AG

[81]	Technical Documentation MM
	6147.703.97.100; Condat AG

[82]	Technical Documentation RR
	6147.704.97.100; Condat AG

[83]	Technical Documentation DL
	6147.705.97.100; Condat AG

[84]	Technical Documentation CCD
	6147.706.97.100; Condat AG

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

- Error! Objects cannot be created from editing field codes.

- Figure 1: Mobile-station protocol architecture

This document describes the tests for Call Control.

# Parameters

/* DECLARATIONS; structs */

DECLARATION (BC_SPEECH_FR)
DECLARATION (BC_AUX_SPEECH_FR)
DECLARATION (BC_SPEECH_FR_HR)
DECLARATION (BC_AUX_SPEECH_FR_HR)
DECLARATION (BC_SPEECH_AMR_EFR_HR_FR)
DECLARATION (BC_SPEECH_FR_HR_EFR)
DECLARATION (BC_SPEECH_FR_EFR)
DECLARATION (BC_SPEECH_FR_CTM)
DECLARATION (BC_AUX_SPEECH_FR_CTM)
DECLARATION (BC_SPEECH_FR_HR_CTM)
DECLARATION (BC_SPEECH_AMR_EFR_HR_FR_CTM)
DECLARATION (BC_FAX_T_2400)
DECLARATION (BC_FAX_T_4800)
DECLARATION (BC_FAX_T_9600)
DECLARATION (BC_BS21_T_300)
DECLARATION (BC_BS20_T_14400)
DECLARATION (BC_BS20_T_14400_1)
DECLARATION (BC_BS20_T_14400_2)
DECLARATION (BC_BS20_T_14400_3)
DECLARATION (BC_BS22_T_1200)
DECLARATION (BC_BS24_T_2400)
DECLARATION (BC_BS25_T_4800)
DECLARATION (BC_BS26_T_9600)
DECLARATION (BC_BS23_T)
DECLARATION (BC_BS21_T_300_UDI)
DECLARATION (BC_BS22_T_1200_UDI)
DECLARATION (BC_BS24_T_2400_UDI)
DECLARATION (BC_BS25_T_4800_UDI)
DECLARATION (BC_BS26_T_9600_UDI)
DECLARATION (BC_BS21_T_300_FR)
DECLARATION (BC_BS21_300_BTP)
DECLARATION (BC_BS20_14400_BTP)
DECLARATION (BC_BS20_14400_BTP_1)
DECLARATION (BC_BS20_14400_BTP_2)
DECLARATION (BC_BS20_14400_BTP_3)
DECLARATION (BC_BS21_300_BNTP)
DECLARATION (BC_BS20_14400_BNTP)
DECLARATION (BC_BS21_300_BTP_UDI)
DECLARATION (BC_BS21_300_BNTP_UDI)
DECLARATION (BC_BS21_NT_300_1)
DECLARATION (BC_BS21_NT_300_1_OUTBAND)
DECLARATION (BC_BS21_NT_300_1_RSP)
DECLARATION (BC_BS21_NT_300_1_UDI)
DECLARATION (BC_BS20_NT_14400_1)
DECLARATION (BC_BS20_NT_14400_2)
DECLARATION (BC_BS22_NT_1200)
DECLARATION (BC_BS22_NT_1200_OUTBAND)
DECLARATION (BC_BS22_NT_1200_RSP)
DECLARATION (BC_BS22_NT_1200_UDI)
DECLARATION (BC_BS24_NT_2400)
DECLARATION (BC_BS24_NT_2400_V26TER)
DECLARATION (BC_BS24_NT_2400_V26TER_OUTBAND)
DECLARATION (BC_BS24_NT_2400_RSP)
DECLARATION (BC_BS24_NT_2400_UDI)
DECLARATION (BC_BS25_NT_4800)
DECLARATION (BC_BS25_NT_4800_OUTBAND)
DECLARATION (BC_BS25_NT_4800_RSP)
DECLARATION (BC_BS25_NT_4800_A)
DECLARATION (BC_BS25_NT_4800_A2)
DECLARATION (BC_BS25_NT_4800_B)
DECLARATION (BC_BS25_NT_4800_C)
DECLARATION (BC_BS25_NT_4800_D)
DECLARATION (BC_BS25_NT_4800_UDI)
DECLARATION (BC_BS26_NT_9600)
DECLARATION (BC_BS26_NT_9600A)
DECLARATION (BC_BS26_NT_9600_OUTBAND)
DECLARATION (BC_BS26_NT_9600_1)
DECLARATION (BC_BS26_NT_9600_UDI)
DECLARATION (BC_BS26_9600_BNTP)
DECLARATION (BC_BS26_9600_BNTPT)
DECLARATION (BC_BS26_9600_TPT)
DECLARATION (BC_BS26_BOTH_T_9600)
DECLARATION (BC_BS26_BOTH_NT_9600)
DECLARATION (BC_BS23_NT)
DECLARATION (BC_BS23_NT_OUTBAND)
DECLARATION (BC_BS21_NT_300_2)
DECLARATION (BC_BS21_NT_300_3)
DECLARATION (BC_BS21_NT_300_4)
DECLARATION (BC_BS21_NT_300_4B)
DECLARATION (BC_BS20_NT_14400_4)
DECLARATION (BC_BS20_NT_14400_5)
DECLARATION (BC_BS21_NT_300_4_UDI)
DECLARATION (BC_BS21_NT_300_5)
DECLARATION (BC_BS21_NT_300_5_UDI)
DECLARATION (CALL_CTRL_CAP_1)
DECLARATION (CALL_STATE_CS_0)
DECLARATION (CALL_STATE_CS_3)
DECLARATION (CALL_STATE_CS_4)
DECLARATION (CALL_STATE_CS_9)
DECLARATION (CALL_STATE_CS_10)
DECLARATION (CALL_STATE_CS_26)
DECLARATION (CC_CAUSE_BEARER_CAP_AUTH)
DECLARATION (CC_CAUSE_USER_BUSY)
DECLARATION (CC_CAUSE_CALL_REJECT)
DECLARATION (ALLOWED_ACTIONS_CCBS_ON)
DECLARATION (RECALL_TYPE_CCBS)
DECLARATION (CC_CAUSE_INCOMPAT)
DECLARATION (CC_CAUSE_CALL_CLEAR)
DECLARATION (CC_DIAG_CAUSE_MT)
DECLARATION (CC_CAUSE_UNSPECIFIED)
DECLARATION (CC_CAUSE_TEMP_FAIL)
DECLARATION (CC_CAUSE_MSG_B_STATUS)
DECLARATION (CC_CAUSE_MSG_B_STATUS_2)
DECLARATION (CC_PROCEED_CAUSE_MT)
DECLARATION (CC_CAUSE_MSG_D_CALL_PROCEED)
DECLARATION (CC_ALERT_CAUSE_MT)
DECLARATION (CC_CAUSE_MSG_D_ALERT)
DECLARATION (CC_CAUSE_INVALID_TI)
DECLARATION (CC_TIMER_303)
DECLARATION (CC_CAUSE_TIMER_303)
DECLARATION (CC_TIMER_310)
DECLARATION (CC_CAUSE_TIMER_310)
DECLARATION (CC_TIMER_313)
DECLARATION (CC_CAUSE_TIMER_313)
DECLARATION (CC_TIMER_323)
DECLARATION (CC_CAUSE_TIMER_323)
DECLARATION (CC_TIMER_332)
DECLARATION (CC_CAUSE_TIMER_332)
DECLARATION (CC_TIMER_335)
DECLARATION (CC_CAUSE_TIMER_335)
DECLARATION (CC_CAUSE_CCBS_TIMER_4)
DECLARATION (CC_CAUSE_CHAN_UNACCEPT)
DECLARATION (CC_CAUSE_STATUS_ENQ)
DECLARATION (RCS_IE_PRESENT)
DECLARATION (KEY_FACILITY_9)
DECLARATION (SS_VERS_3)
DECLARATION (CONNECT_NUM_654321)
DECLARATION (UL_CLED_NUM_654321)
DECLARATION (UL_CLED_NUM_7654321)
DECLARATION (DL_CLED_NUM_654321)
DECLARATION (DL_CLED_NUM_7654321)
DECLARATION (CLNG_NUM_654321)
DECLARATION (CLNG_NUM_7654321)
DECLARATION (CLED_PARTY_654321)
DECLARATION (CLED_PARTY_7654321)
DECLARATION (CLNG_PARTY_654321)
DECLARATION (CLNG_PARTY_7654321)
DECLARATION (REDIR_PARTY_NONE)
DECLARATION (CONNECTED_NUMBER_654321)
DECLARATION (CLED_PARTY_SUB_NONE)
DECLARATION (REDIR_PARTY_SUB_NONE)
DECLARATION (PROG_1)
DECLARATION (NOTIFIC_USER_SUSPEND)
DECLARATION (NOTIFIC_USED_RESUME)
DECLARATION (NOTIFIC_BEARER_CHANGE)
DECLARATION (PROGRESS_INBAND_AVAIL)
DECLARATION (SETUP_CONT_SPEECH_FR) 
DECLARATION (SETUP_CONT_SPEECH_FR_HR_EFR) 

DECLARATION (BC_PARA_BS21_NT_300_1)
DECLARATION (BC_PARA_BS21_NT_300_1_UDI)
DECLARATION (BC_PARA_BS21_NT_300_2)
DECLARATION (BC_PARA_BS21_NT_300_3)
DECLARATION (BC_PARA_BS21_NT_300_4)
DECLARATION (BC_PARA_BS21_NT_300_4_UDI)
DECLARATION (BC_PARA_BS21_NT_300_5)
DECLARATION (BC_PARA_BS21_NT_300_5_UDI)
DECLARATION (BC_PARA_BS21_T_300)
DECLARATION (BC_PARA_BS21_BTP_300)
DECLARATION (BC_PARA_BS21_T_300_UDI)
DECLARATION (BC_PARA_BS22_T_1200)
DECLARATION (BC_PARA_BS22_T_1200_UDI)
DECLARATION (BC_PARA_BS22_NT_1200)
DECLARATION (BC_PARA_BS22_NT_1200_UDI)
DECLARATION (BC_PARA_BS24_T_2400)
DECLARATION (BC_PARA_BS24_T_2400_UDI)
DECLARATION (BC_PARA_BS24_NT_2400)
DECLARATION (BC_PARA_BS24_NT_2400_V22BIS)
DECLARATION (BC_PARA_BS24_NT_2400_UDI)
DECLARATION (BC_PARA_BS25_T_4800)
DECLARATION (BC_PARA_BS25_T_4800_UDI)
DECLARATION (BC_PARA_BS25_NT_4800)
DECLARATION (BC_PARA_BS25_NT_4800_UDI)
DECLARATION (BC_PARA_BS26_T_9600)
DECLARATION (BC_PARA_BS26_T_9600A)
DECLARATION (BC_PARA_BS26_T_9600B)
DECLARATION (BC_PARA_BS26_T_9600C)
DECLARATION (BC_PARA_BS26_T_9600D)
DECLARATION (BC_PARA_BS26_T_9600_UDI)
DECLARATION (BC_PARA_BS26_NT_9600)
DECLARATION (BC_PARA_BS26_NT_9600A)
DECLARATION (BC_PARA_BS26_NT_9600_UDI)
DECLARATION (BC_PARA_BS26_NT_9600_1)
DECLARATION (BC_PARA_BS26_BOTH_T_9600)
DECLARATION (BC_PARA_BS26_BOTH_NT_9600)
DECLARATION (BC_PARA_BS23_T)
DECLARATION (BC_PARA_BS23_NT)
DECLARATION (BC_PARA_BS20_T_14400)
DECLARATION (BC_PARA_BS20_NT_14400_1)
DECLARATION (BC_PARA_BS20_NT_14400_4)
DECLARATION (BC_PARA_FAX_T_2400)
DECLARATION (BC_PARA_FAX_T_4800)
DECLARATION (BC_PARA_FAX_T_9600)
DECLARATION (FACILITY_A)
DECLARATION (FACILITY_A_FAC)
DECLARATION (FACILITY_NONE)
DECLARATION (USER_A)
DECLARATION (USER_A_USER)
DECLARATION (CH_DATA_9_6)
DECLARATION (CH_DATA_4_8)
DECLARATION (BC_CONF_NO_SERVICE)
DECLARATION (BC_CONF_SPEECH_FR_HR_EFR)
DECLARATION (BC_CONF_BS21_T_300)
DECLARATION (BC_PARA_UNSUPP)
DECLARATION (BC_CONF_PACKET_UNSUPP)

DECLARATION (CH_SPEECH_FULL)
DECLARATION (CH_SIG_ONLY)
DECLARATION (BC_PARA_SPEECH)
DECLARATION (BC_PARA_SPEECH_LINE_2)
DECLARATION (BC_PARA_NO_SERVICE)
DECLARATION (BC_PARA_SPEECH_CTM)
DECLARATION (BC_PARA_SPEECH_LINE_2_CTM)

/* DECLARATIONS; arrays */

DECLARATION (NUM_654321)
DECLARATION (NUM_7654321)
DECLARATION (NUM_UNUSED)
DECLARATION (FACILITY_A_6)
DECLARATION (SETUP_CONT_SPEECH_FR_HR_EFR_15)
DECLARATION (SETUP_CONT_SPEECH_FR_13)
DECLARATION (USER_A_4)
DECLARATION (BC_CONF_SPEECH_FR_HR_EFR_ARRAY)
DECLARATION (BC_CONF_BS21_T_300_ARRAY)
DECLARATION (BC_CONF_PACKET_UNSUPP_ARRAY)
DECLARATION (SS_VERS_3_ARRAY)

/* DEFINES */

BYTE	ZERO_VALUE	0x00
BYTE 	L_FAC_48	48
BYTE	PD_5	0x05
BYTE	C_INFO_4	0x04
BYTE	RATE_1	0x01
BYTE	RATE_2	0x02
BYTE	RATE_3	0x03
BYTE	RATE_4	0x04
BYTE	RATE_5	0x05
BYTE	RATE_7	0x07
/* 
  * note that RATE_8 is used instead of UR_14_4_KBIT which is used at the MNCC SAP; note also
  * that UR_14_4_KBIT / RATE_8 cannot be used over the air interface as there it is (currently) a 
  * resevered value
  */
BYTE	RATE_8	0x08
BYTE	BEARER_SERV_1	0x01
BYTE	BEARER_SERV_4	0x04
BYTE	BEARER_SERV_15	0x0F
BYTE	CONN_ELEM_1	0x01
BYTE	CONN_ELEM_B_T	0x02
BYTE	CONN_ELEM_B_NT	0x03
BYTE	STOP_BITS_1	0x01
BYTE	DATA_BITS_1	0x01
BYTE	FLOW_CONTROL_8	0x08
BYTE	FLOW_CONTROL_12	0x0C
BYTE	FLOW_CONTROL_255	0xFF
BYTE	MODEM_TYPE_1	0x01
BYTE	MODEM_TYPE_2	0x02
BYTE	MODEM_TYPE_3	0x03
BYTE	MODEM_TYPE_4	0x04
BYTE	MODEM_TYPE_5	0x05
BYTE	MODEM_TYPE_6	0x06
BYTE	PARITY_2	0x02
BYTE	PARITY_3	0x03
BYTE	PARITY_4	0x04
 /* ti (Transaction identifier) */
BYTE	TI_MO_5	0x05
BYTE	TI_MO_5_RESP	0x0D
BYTE	TI_MO_6	0x06
/* ms originated transaction */
BYTE	TI_MO_7	0x07
BYTE	TI_MO_7_RESP	0x0F
/* reserved */
BYTE	TI_MT_7	0x07
BYTE	TI_MT_7_RESP	0x0F
/* reserved */
BYTE	TI_MT_1	0x01
BYTE	TI_MT_1_RESP	0x09

BYTE	TI_MT_2	0x02
BYTE	TI_MT_2_RESP	0x0A

BYTE	TI_MT_3	0x03
BYTE	TI_MT_3_RESP	0x0B

BYTE	TI_MT_4	0x04
BYTE	TI_MT_4_RESP	0x0C

BYTE	TI_MT_5	0x05
BYTE	TI_MT_5_RESP	0x0D

/* Request ID for MNCC_BEAER_CAP primitives */
BYTE	REQ_ID_5	0x05

/* length for bearer capabilities */
BYTE	BC_LEN_0	0x00
BYTE	BC_LEN_4	0x04
BYTE	BC_LEN_7	0x07
BYTE	BC_LEN_8	0x08

/* cause values originated by higher layers / ACI; ACI uses CC as originating entity */
SHORT	ACI_CAUSE_CALL_REJECT	MNCC_CAUSE_CALL_REJECT
SHORT	ACI_CAUSE_CALL_CLEAR	MNCC_CAUSE_CALL_CLEAR

/* cause values originated by lower layers / RR */

SHORT	RRCS_NORM	0x0300
SHORT	RRCS_ABORT_CEL_SEL_FAIL	0xC310
SHORT	RRCS_ABORT_RAD_LNK_FAIL	0xC311
SHORT	RRCS_DL_EST_FAIL	0xC323

/* Engineering Mode */
BYTE	EM_ENTITY	0x05
LONG	Bitm_L_1	0x0002
LONG	Bitm_H_1	0x0000
LONG	Bitm_L_2	0x0200
LONG	Bitm_H_2	0x0000

/* ARRAYS */

/* SBK2001-08-01: need partial arrays for called / calling party BCD number / subaddress as e.g. T_calling_name is defined as having a component num with fixed size in the MNCC SAP / .h file */

BEGINARRAY_PART (NUM_654321, 6) 0x06, 0x05, 0x04, 0x03, 0x02, 0x01 ENDARRAY
BEGINARRAY_PART (NUM_7654321, 7) 0x07, 0x06, 0x05, 0x04, 0x03, 0x02, 0x01 ENDARRAY
BEGINARRAY_PART (NUM_UNUSED, 1) 0xFF ENDARRAY

BEGINARRAY_PART (FACILITY_A_6, 6) 0x00, 0x01, 0x02, 0x03, 0x04, 0x05 ENDARRAY
BEGINARRAY_PART (SS_VERS_3_ARRAY, 1) 
	0x01 /* prefer to have the absolute value as used over the air  ere instead of SS_VERSION_3 */
ENDARRAY
BEGINARRAY (SETUP_CONT_SPEECH_FR_HR_EFR_15, 15)
	0x04, 0x04, 0x60, 0x02, 0x00, 0x81, 0x5E, 0x04, 0xA8, 0x56, 0x34, 0x12, 0x15, 0x01, 0x03
ENDARRAY
BEGINARRAY (SETUP_CONT_SPEECH_FR_13, 13)
	0x04, 0x02, 0x20, 0x80, 0x5E, 0x04, 0xA8, 0x56, 0x34, 0x12, 0x15, 0x01, 0x03
ENDARRAY
BEGINARRAY_PART (USER_A_4, 4) 0x02, 0x03, 0x04, 0x05 ENDARRAY
BEGINARRAY_PART (BC_CONF_SPEECH_FR_HR_EFR_ARRAY, 4) 
	0x60, /* octet 3: dual rate, HR preferred, GSM, circuit, speech ITC */
	0x02, /* octet 3a: EFR */
	0x00, /* octet 3b: FR */
	0x81  /* octet 3c: HR */ 
ENDARRAY
BEGINARRAY_PART (BC_CONF_BS21_T_300_ARRAY, 7) 0xA2, 0xB8, 0x81, 0x21, 0x51, 0x43, 0x81 ENDARRAY
BEGINARRAY_PART (BC_CONF_PACKET_UNSUPP_ARRAY, 8) 0xA1, 0x88, 0x96, 0x20, 0x13, 0x63, 0xA0, 0xC6 ENDARRAY

/* PSTRUCTS */

/* BC_PARA_BS21_NT_300_1 */

BEGIN_PSTRUCT ("bcpara2", BC_PARA_BS21_NT_300_1)
SET_COMP ("rate", UR_0_3_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V21)
ENDSTRUCT

/* BC_PARA_BS21_NT_300_1_UDI */

BEGIN_PSTRUCT ("bcpara2", BC_PARA_BS21_NT_300_1_UDI)
SET_COMP ("rate", UR_0_3_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS21_NT_300_2 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS21_NT_300_2)
SET_COMP ("rate", UR_0_3_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_EVEN)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V21)
ENDSTRUCT

/* BC_PARA_BS21_NT_300_3 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS21_NT_300_3)
SET_COMP ("rate", UR_0_3_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_FORCED_TO_0)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V21)
ENDSTRUCT

/* BC_PARA_BS21_NT_300_4 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS21_NT_300_4)
SET_COMP ("rate", UR_0_3_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V21)
ENDSTRUCT

/* BC_PARA_BS21_NT_300_4_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS21_NT_300_4_UDI)
SET_COMP ("rate", UR_0_3_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS21_NT_300_5 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS21_NT_300_5)
SET_COMP ("rate", UR_0_3_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V21)
ENDSTRUCT

/* BC_PARA_BS21_NT_300_5_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS21_NT_300_5_UDI)
SET_COMP ("rate", UR_0_3_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS21_T_300_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS21_T_300_UDI)
SET_COMP ("rate", UR_0_3_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS22_T_1200 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS22_T_1200)
SET_COMP ("rate", UR_1_2_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V22)
ENDSTRUCT

/* BC_PARA_BS22_T_1200_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS22_T_1200_UDI)
SET_COMP ("rate", UR_1_2_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS22_NT_1200 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS22_NT_1200)
SET_COMP ("rate", UR_1_2_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V22)
ENDSTRUCT

/* BC_PARA_BS22_NT_1200_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS22_NT_1200_UDI)
SET_COMP ("rate", UR_1_2_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS24_T_2400 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS24_T_2400)
SET_COMP ("rate", UR_2_4_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V22_BIS)
ENDSTRUCT

/* BC_PARA_BS24_T_2400_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS24_T_2400_UDI)
SET_COMP ("rate", UR_2_4_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS24_NT_2400 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS24_NT_2400)
SET_COMP ("rate", UR_2_4_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem",CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V26_TER)
ENDSTRUCT

/* BC_PARA_BS24_NT_2400_V22BIS */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS24_NT_2400_V22BIS)
SET_COMP ("rate", UR_2_4_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem",CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V22_BIS)
ENDSTRUCT

/* BC_PARA_BS24_NT_2400_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS24_NT_2400_UDI)
SET_COMP ("rate", UR_2_4_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem",CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS25_T_4800 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS25_T_4800)
SET_COMP ("rate", UR_4_8_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS25_T_4800_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS25_T_4800_UDI)
SET_COMP ("rate", UR_4_8_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS25_NT_4800 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS25_NT_4800)
SET_COMP ("rate", UR_4_8_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS25_NT_4800_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS25_NT_4800_UDI)
SET_COMP ("rate", UR_4_8_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS26_T_9600 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_T_9600)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS26_T_9600A */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_T_9600A)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS26_T_9600B */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_T_9600B)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS26_T_9600C */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_T_9600C)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_AUTOBAUD)
ENDSTRUCT

/* BC_PARA_BS26_T_9600D */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_T_9600D)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_AUTOBAUD)
ENDSTRUCT

/* BC_PARA_BS26_T_9600_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_T_9600_UDI)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS26_NT_9600 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_NT_9600)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS26_NT_9600A */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_NT_9600A)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS26_NT_9600_UDI */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_NT_9600_UDI)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_BS26_NT_9600_1 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_NT_9600_1)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS26_BOTH_T_9600 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_BOTH_T_9600)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS_PREF)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS26_BOTH_NT_9600 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS26_BOTH_NT_9600)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS_PREF)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V32)
ENDSTRUCT

/* BC_PARA_BS23_T */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS23_T)
SET_COMP ("rate", UR_1_2_KBIT_V23)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V23)
ENDSTRUCT

/* BC_PARA_BS23_NT */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS23_NT)
SET_COMP ("rate", UR_1_2_KBIT_V23)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_V23)
ENDSTRUCT

/* BC_PARA_BS20_T_14400 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS20_T_14400)
SET_COMP ("rate", UR_14_4_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_AUTOBAUD)
ENDSTRUCT

/* BC_PARA_BS20_NT_14400_1 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS20_NT_14400_1)
SET_COMP ("rate", UR_14_4_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_7_BIT)
SET_COMP ("parity", PARITY_ODD)
SET_COMP ("flow_control", INBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_AUTOBAUD)
ENDSTRUCT

/* BC_PARA_BS20_NT_14400_4 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS20_NT_14400_4)
SET_COMP ("rate", UR_14_4_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
SET_COMP ("conn_elem", CONN_ELEM_NON_TRANS)
SET_COMP ("stop_bits", STOP_2_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", NO_FLOW_CONTROL)
SET_COMP ("modem_type", MT_AUTOBAUD)
ENDSTRUCT

/* BC_PARA_FAX_T_2400 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_FAX_T_2400)
SET_COMP ("rate", UR_2_4_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_FAX)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_FAX_T_4800 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_FAX_T_4800)
SET_COMP ("rate", UR_4_8_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_FAX)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* BC_PARA_FAX_T_9600 */

BEGIN_PSTRUCT ("bcpara", BC_PARA_FAX_T_9600)
SET_COMP ("rate", UR_9_6_KBIT)
SET_COMP ("bearer_serv", BEARER_SERV_FAX)
SET_COMP ("conn_elem", CONN_ELEM_TRANS)
SET_COMP ("stop_bits", STOP_1_BIT)
SET_COMP ("data_bits", DATA_8_BIT)
SET_COMP ("parity", PARITY_NONE)
SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
SET_COMP ("modem_type", MT_NONE)
ENDSTRUCT

/* USER_A_USER */

BEGIN_PSTRUCT ("user", USER_A_USER)
SET_COMP ("info_context", USER_IN_USER)
SET_COMP ("pd", PD_5)
SET_COMP ("c_info", C_INFO_4)
SET_COMP ("info", USER_A_4)
ENDSTRUCT

/* CH_DATA_9_6 */

BEGIN_PSTRUCT ("chm", CH_DATA_9_6)
SET_COMP ("ch_type", CH_TCH_F)
SET_COMP ("ch_mode", CHM_DATA_9_6)
ENDSTRUCT

/* CH_DATA_4_8 */

BEGIN_PSTRUCT ("chm", CH_DATA_4_8)
SET_COMP ("ch_type", CH_TCH_F)
SET_COMP ("ch_mode", CHM_DATA_4_8)
ENDSTRUCT

/* BC_CONF_NO_SERVICE */

BEGIN_PSTRUCT ("bcconf", BC_CONF_NO_SERVICE)
SET_COMP ("bc_len", BC_LEN_0)
SKIP_COMP ("bc") /* SAP indicates that if bc_len=0 then the bc component is not valid */
ENDSTRUCT

/* BC_CONF_SPEECH_FR_HR_EFR */

BEGIN_PSTRUCT ("bcconf", BC_CONF_SPEECH_FR_HR_EFR)
SET_COMP ("bc_len", BC_LEN_4)
SET_COMP ("bc", BC_CONF_SPEECH_FR_HR_EFR_ARRAY)
ENDSTRUCT

/* BC_CONF_BS21_T_300 */

BEGIN_PSTRUCT ("bcconf", BC_CONF_BS21_T_300)
SET_COMP ("bc_len", BC_LEN_7)
SET_COMP ("bc", BC_CONF_BS21_T_300_ARRAY)
ENDSTRUCT

/* BC_PARA_UNSUPP */

BEGIN_PSTRUCT ("bcpara", BC_PARA_UNSUPP)
SET_COMP ("rate", ZERO_VALUE)
SET_COMP ("bearer_serv", BEARER_SERV_15)
SET_COMP ("conn_elem", ZERO_VALUE)
SET_COMP ("stop_bits", ZERO_VALUE)
SET_COMP ("data_bits", ZERO_VALUE)
SET_COMP ("parity", ZERO_VALUE)
SET_COMP ("flow_control", ZERO_VALUE)
SET_COMP ("modem_type", ZERO_VALUE)
ENDSTRUCT

/* BC_CONF_PACKET_UNSUPP */

BEGIN_PSTRUCT ("bcconf", BC_CONF_PACKET_UNSUPP)
SET_COMP ("bc_len", BC_LEN_8)
SET_COMP ("bc", BC_CONF_PACKET_UNSUPP_ARRAY)
ENDSTRUCT

/* FACILITY_A_FAC */

BEGIN_PSTRUCT ("fac_inf", FACILITY_A_FAC)
SET_COMP ("l_fac", L_FAC_48)
SET_COMP ("o_fac", ZERO_VALUE)
SET_COMP ("fac", FACILITY_A_6)
ENDSTRUCT

/* FACILITY_NONE */

BEGIN_PSTRUCT ("fac_inf", FACILITY_NONE)
SET_COMP ("l_fac", 0)
SKIP_COMP ("o_fac")
SKIP_COMP ("fac")
ENDSTRUCT

/* CH_SPEECH_FULL */

BEGIN_PSTRUCT ("chm", CH_SPEECH_FULL)
SET_COMP ("ch_type", CH_TCH_F)
SET_COMP ("ch_mode", CHM_SPEECH)
ENDSTRUCT

/* CH_SIG_ONLY */

BEGIN_PSTRUCT ("chm", CH_SIG_ONLY)
SET_COMP ("ch_type", CH_SDCCH)
SET_COMP ("ch_mode", CHM_SIG_ONLY)
ENDSTRUCT

/* BC_PARA_SPEECH

| 0 | User rate |
| --- | --- |
| 5 | Bearer serv speech |
| 0 | Connection element |
| 0 | Stop bits |
| 0 | Data bits |
| 0 | Parity |
| 0 | Flow control |
| 0 | Modem type |

*/

BEGIN_PSTRUCT ("bcpara", BC_PARA_SPEECH)
	SET_COMP ("rate", ZERO_VALUE)
	SET_COMP ("bearer_serv", BEARER_SERV_SPEECH)
	SET_COMP ("conn_elem", ZERO_VALUE)
	SET_COMP ("stop_bits",	ZERO_VALUE)
	SET_COMP ("data_bits",	ZERO_VALUE)
	SET_COMP ("parity", ZERO_VALUE)
	SET_COMP ("flow_control", ZERO_VALUE)
	SET_COMP ("modem_type", ZERO_VALUE)
ENDSTRUCT

/* BC_PARA_SPEECH_LINE_2

| 0 | User rate |
| --- | --- |
| 6 | Bearer serv aux speech |
| 0 | Connection element |
| 0 | Stop bits |
| 0 | Data bits |
| 0 | Parity |
| 0 | Flow control |
| 0 | Modem type |

*/
BEGIN_PSTRUCT ("bcpara", BC_PARA_SPEECH_LINE_2)
	SET_COMP ("rate", ZERO_VALUE)
	SET_COMP ("bearer_serv", BEARER_SERV_AUX_SPEECH)
	SET_COMP ("conn_elem", ZERO_VALUE)
	SET_COMP ("stop_bits", ZERO_VALUE)
	SET_COMP ("data_bits", ZERO_VALUE)
	SET_COMP ("parity", ZERO_VALUE)
	SET_COMP ("flow_control", ZERO_VALUE)
	SET_COMP ("modem_type", ZERO_VALUE) 
ENDSTRUCT

/* BC_PARA_SPEECH with CTM

| 0 | User rate |
| --- | --- |
| 5 | Bearer serv speech |
| 0 | Connection element |
| 0 | Stop bits |
| 0 | Data bits |
| 0 | Parity |
| 0 | Flow control |
| 0 | Modem type |

*/

BEGIN_PSTRUCT ("bcpara", BC_PARA_SPEECH_CTM)
	SET_COMP ("rate", ZERO_VALUE)
	SET_COMP ("bearer_serv", BEARER_SERV_SPEECH_CTM)
	SET_COMP ("conn_elem", ZERO_VALUE)
	SET_COMP ("stop_bits",	ZERO_VALUE)
	SET_COMP ("data_bits",	ZERO_VALUE)
	SET_COMP ("parity", ZERO_VALUE)
	SET_COMP ("flow_control", ZERO_VALUE)
	SET_COMP ("modem_type", ZERO_VALUE)
ENDSTRUCT

/* BC_PARA_SPEECH_LINE_2 with CTM

| 0 | User rate |
| --- | --- |
| 6 | Bearer serv aux speech |
| 0 | Connection element |
| 0 | Stop bits |
| 0 | Data bits |
| 0 | Parity |
| 0 | Flow control |
| 0 | Modem type |

*/
BEGIN_PSTRUCT ("bcpara", BC_PARA_SPEECH_LINE_2_CTM)
	SET_COMP ("rate", ZERO_VALUE)
	SET_COMP ("bearer_serv", BEARER_SERV_AUX_SPEECH_CTM)
	SET_COMP ("conn_elem", ZERO_VALUE)
	SET_COMP ("stop_bits", ZERO_VALUE)
	SET_COMP ("data_bits", ZERO_VALUE)
	SET_COMP ("parity", ZERO_VALUE)
	SET_COMP ("flow_control", ZERO_VALUE)
	SET_COMP ("modem_type", ZERO_VALUE) 
ENDSTRUCT

/*

| 1 | User rate 300 |
| --- | --- |
| 1 | Bearer serv async |
| 0 | Connection element transparent |
| 1 | Stop bits (2) |
| 1 | Data bits (8) |
| 3 | Parity (none) |
| 255 | Flow control (outband) |
| 1 | Modem type (V.21) |

*/

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS21_T_300)
	SET_COMP ("rate", UR_0_3_KBIT)
	SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
	SET_COMP ("conn_elem", CONN_ELEM_TRANS)
	SET_COMP ("stop_bits", STOP_2_BIT)
	SET_COMP ("data_bits", DATA_8_BIT)
	SET_COMP ("parity", PARITY_NONE)
	SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
	SET_COMP ("modem_type", MT_V21) 
ENDSTRUCT

/*

| 1 | User rate 300 |
| --- | --- |
| 1 | Bearer serv async |
| 2 | Connection element both, transparent preferred |
| 1 | Stop bits (2) |
| 1 | Data bits (8) |
| 3 | Parity (none) |
| 255 | Flow control (outband) |
| 1 | Modem type (V.21) |

*/

BEGIN_PSTRUCT ("bcpara", BC_PARA_BS21_BTP_300)
	SET_COMP ("rate", UR_0_3_KBIT)
	SET_COMP ("bearer_serv", BEARER_SERV_ASYNC)
	SET_COMP ("conn_elem", CONN_ELEM_TRANS_PREF)
	SET_COMP ("stop_bits", STOP_2_BIT)
	SET_COMP ("data_bits", DATA_8_BIT)
	SET_COMP ("parity", PARITY_NONE)
	SET_COMP ("flow_control", OUTBAND_FLOW_CONTROL)
	SET_COMP ("modem_type", MT_V21) 
ENDSTRUCT

/* BC_PARA_NO_SERVICE

| 0 | User rate |
| --- | --- |
| 255 | Bearer serv none |
| 0 | Connection element |
| 0 | stop bits |
| 0 | data bits |
| 0 | Parity |
| 0 | flow control |
| 0 | Modem type |

*/
BEGIN_PSTRUCT ("bcpara", BC_PARA_NO_SERVICE)
	SET_COMP ("rate", ZERO_VALUE)
	SET_COMP ("bearer_serv", BEARER_SERV_NOT_PRES)
	SET_COMP ("conn_elem",  ZERO_VALUE)
	SET_COMP ("stop_bits", ZERO_VALUE)
	SET_COMP ("data_bits", ZERO_VALUE)
	SET_COMP ("parity", ZERO_VALUE)
	SET_COMP ("flow_control", ZERO_VALUE)
	SET_COMP ("modem_type", ZERO_VALUE) 
ENDSTRUCT

/* called_party (Called party BCD number) */
BEGIN_PSTRUCT ("called_party", CLED_PARTY_654321)
	SET_COMP ("ton", TON_NATIONAL)
	SET_COMP ("npi", NPI_NATIONAL)
	SET_COMP ("c_called_num", 6)
	SET_COMP ("called_num", NUM_654321)
ENDSTRUCT
BEGIN_PSTRUCT ("called_party", CLED_PARTY_7654321)
	SET_COMP ("ton", TON_NATIONAL)
	SET_COMP ("npi", NPI_NATIONAL)
	SET_COMP ("c_called_num", 7)
	SET_COMP ("called_num", NUM_7654321)
ENDSTRUCT

/* redir_party (Redirecting party BCD number) */
BEGIN_PSTRUCT ("redirecting_party", REDIR_PARTY_NONE)
	SKIP_COMP ("ton")
	SKIP_COMP ("npi")
	SKIP_COMP ("present")
	SKIP_COMP ("screen")
	SET_COMP ("c_redir_num", 0)
	SKIP_COMP ("redir_num")
ENDSTRUCT

/* called_party_sub (Called party subaddress) */
BEGIN_PSTRUCT ("called_party_sub", CLED_PARTY_SUB_NONE)
	SET_COMP ("tos", TOS_NOT_PRES)
	SKIP_COMP ("odd_even")
	SET_COMP ("c_subaddr", ZERO_VALUE)
	SKIP_COMP ("subaddr")
ENDSTRUCT

/* redir_party_sub (Redirecting party subaddress) */
BEGIN_PSTRUCT ("redirecting_party_sub", REDIR_PARTY_SUB_NONE)
	SET_COMP ("tos", TOS_NOT_PRES)
	SKIP_COMP ("odd_even")
	SET_COMP ("c_subaddr", ZERO_VALUE)
	SKIP_COMP ("subaddr")
ENDSTRUCT

/* MSTRUCTS */

/* reverse_call */
BEGIN_MSTRUCT ("reverse_call", RCS_IE_PRESENT)
ENDSTRUCT

/* key_facility (Keypad facility) */
BEGIN_MSTRUCT ("key_facility", KEY_FACILITY_9)
	SET_COMP ("key", DIG_9)
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_CALL_CLEAR)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_CALL_CLEAR)
	SKIP_COMP ("diag")
ENDSTRUCT

/* progress (Progress indicator) */
BEGIN_MSTRUCT ("progress", PROG_1)
	SET_COMP ("cs", CS_GSM_PLMN)
	SET_COMP ("loc", LOC_PRIV_NET_LOCAL_USER)
	SET_COMP ("progress_desc", PROG_END_TO_END_PLMN)
ENDSTRUCT

BEGIN_MSTRUCT ("call_ctrl_cap", CALL_CTRL_CAP_1)
	SET_COMP ("pcp", PCP_YES)
	SET_COMP ("dtmf", DTMF_CURRENT)
ENDSTRUCT

/* connect_num  */
BEGIN_MSTRUCT ("connect_num", CONNECT_NUM_654321)
	SET_COMP ("ton",	TON_NATIONAL)
	SET_COMP ("npi",	NPI_NATIONAL)
	SET_COMP ("present",	PRES_ALLOWED)
	SET_COMP ("screen",	SCR_USER_UNSCREENED)
	SET_COMP ("num",	NUM_654321)
ENDSTRUCT

/* connected_number */
BEGIN_PSTRUCT ("connected_number", CONNECTED_NUMBER_654321)
	SET_COMP ("ton",	TON_NATIONAL)
	SET_COMP ("npi",	NPI_NATIONAL)
	SET_COMP ("present",	PRES_ALLOWED)
	SET_COMP ("screen",	SCR_USER_UNSCREENED)
	SET_COMP ("c_num",	6)
	SET_COMP ("num",	NUM_654321)
ENDSTRUCT

/* called_num (Called party BCD number) */
BEGIN_MSTRUCT ("ul_called_num", UL_CLED_NUM_654321)
	SET_COMP ("ton", TON_NATIONAL)
	SET_COMP ("npi", NPI_NATIONAL)
	SET_COMP ("num", NUM_654321)
ENDSTRUCT
BEGIN_MSTRUCT ("ul_called_num", UL_CLED_NUM_7654321)
	SET_COMP ("ton",	TON_NATIONAL)
	SET_COMP ("npi",	NPI_NATIONAL)
	SET_COMP ("num",	NUM_7654321)
ENDSTRUCT

BEGIN_MSTRUCT ("dl_called_num", DL_CLED_NUM_654321)
	SET_COMP ("ton", TON_NATIONAL)
	SET_COMP ("npi", NPI_NATIONAL)
	SET_COMP ("num", NUM_654321)
ENDSTRUCT
BEGIN_MSTRUCT ("dl_called_num", DL_CLED_NUM_7654321)
	SET_COMP ("ton",	TON_NATIONAL)
	SET_COMP ("npi",	NPI_NATIONAL)
	SET_COMP ("num",	NUM_7654321)
ENDSTRUCT

/* calling_num (Calling party BCD number) */
BEGIN_MSTRUCT ("calling_num", CLNG_NUM_654321)
	SET_COMP ("ton", TON_NATIONAL)
	SET_COMP ("npi", NPI_NATIONAL)
	SET_COMP ("present", PRES_ALLOWED)
	SET_COMP ("screen", SCR_USER_UNSCREENED)
	SET_COMP ("num", NUM_654321)
ENDSTRUCT
BEGIN_MSTRUCT ("calling_num", CLNG_NUM_7654321)
	SET_COMP ("ton",	TON_NATIONAL)
	SET_COMP ("npi",	NPI_NATIONAL)
	SET_COMP ("present",	PRES_ALLOWED)
	SET_COMP ("screen",	SCR_USER_UNSCREENED)
	SET_COMP ("num",	NUM_7654321)
ENDSTRUCT

/* calling_party (Calling party address) */
BEGIN_PSTRUCT ("calling_party", CLNG_PARTY_654321)
	SET_COMP ("ton",	TON_NATIONAL)
	SET_COMP ("npi",	NPI_NATIONAL)
	SET_COMP ("present",	PRES_ALLOWED)
	SET_COMP ("screen",	SCR_USER_UNSCREENED)
	SET_COMP ("c_num",	6)
	SET_COMP ("num",	NUM_654321)
ENDSTRUCT

BEGIN_PSTRUCT ("calling_party", CLNG_PARTY_7654321)
	SET_COMP ("ton",	TON_NATIONAL)
	SET_COMP ("npi",	NPI_NATIONAL)
	SET_COMP ("present",	PRES_ALLOWED)
	SET_COMP ("screen",	SCR_USER_UNSCREENED)
	SET_COMP ("c_num",	7)
	SET_COMP ("num",	NUM_7654321)
ENDSTRUCT

/* bearer_cap (Bearer capibility) */
BEGIN_MSTRUCT ("bearer_cap", BC_SPEECH_FR)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_SPEECH)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_AUX_SPEECH_FR)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUXILIARY_SPEECH)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SKIP_COMP ("compress")
	SKIP_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_SPEECH_FR_HR)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_SPEECH)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_SPEECH_AMR_EFR_HR_FR)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_SPEECH)
	SET_COMP ("coding_bc3x1",	CODING_BC3X_SPEECH)
	SET_COMP ("ctm",	CTM_NO)
	SET_COMP ("speech_vers1",	SPEECH_VERS_AMR_FR)
	SET_COMP ("coding_bc3x2",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers2",	SPEECH_VERS_AMR_HR)
	SET_COMP ("coding_bc3x3",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers3",	SPEECH_VERS_EFR)
	SET_COMP ("coding_bc3x4",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers4",	SPEECH_VERS_FR)
	SET_COMP ("coding_bc3x5",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers5",	SPEECH_VERS_HR)
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_SPEECH_FR_CTM)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_SPEECH)
	SET_COMP ("coding_bc3x1",	CODING_BC3X_SPEECH)
	SET_COMP ("ctm",	CTM_YES)
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_AUX_SPEECH_FR_CTM)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUXILIARY_SPEECH)
	SET_COMP ("coding_bc3x1",	CODING_BC3X_SPEECH)
	SET_COMP ("ctm",	CTM_YES)
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_SPEECH_FR_HR_CTM)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_SPEECH)
	SET_COMP ("coding_bc3x1",	CODING_BC3X_SPEECH)
	SET_COMP ("ctm",	CTM_YES)
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_SPEECH_AMR_EFR_HR_FR_CTM)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_SPEECH)
	SET_COMP ("coding_bc3x1",	CODING_BC3X_SPEECH)
	SET_COMP ("ctm",	CTM_YES)
	SET_COMP ("speech_vers1",	SPEECH_VERS_AMR_FR)
	SET_COMP ("coding_bc3x2",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers2",	SPEECH_VERS_AMR_HR)
	SET_COMP ("coding_bc3x3",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers3",	SPEECH_VERS_EFR)
	SET_COMP ("coding_bc3x4",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers4",	SPEECH_VERS_FR)
	SET_COMP ("coding_bc3x5",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers5",	SPEECH_VERS_HR)
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_AUX_SPEECH_FR_HR)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUXILIARY_SPEECH)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_SPEECH_FR_HR_EFR)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_SPEECH)
	SET_COMP ("coding_bc3x1",	CODING_BC3X_SPEECH)
	SET_COMP ("ctm",	CTM_NO)
	SET_COMP ("speech_vers1",	SPEECH_VERS_EFR)
	SET_COMP ("coding_bc3x2",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers2",	SPEECH_VERS_FR)
	SET_COMP ("coding_bc3x3",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers3",	SPEECH_VERS_HR)
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_SPEECH_FR_EFR)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_SPEECH)
	SET_COMP ("coding_bc3x1",	CODING_BC3X_SPEECH)
	SET_COMP ("ctm",	CTM_NO)
	SET_COMP ("speech_vers1",	SPEECH_VERS_EFR)
	SET_COMP ("coding_bc3x2",	CODING_BC3X_SPEECH)
	SET_COMP ("speech_vers2",	SPEECH_VERS_FR)
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	FORBID_COMP ("compress")
	FORBID_COMP ("structure")
	FORBID_COMP ("duplex")
	FORBID_COMP ("config")
	FORBID_COMP ("nirr")
	FORBID_COMP ("establish")
	FORBID_COMP ("access_ident")
	FORBID_COMP ("rate_adapt")
	FORBID_COMP ("sig_access_prot")
	FORBID_COMP ("l1_ident")
	FORBID_COMP ("user_inf_l1_prot")
	FORBID_COMP ("sync_async")
	FORBID_COMP ("num_stop")
	FORBID_COMP ("negotiate")
	FORBID_COMP ("num_data")
	FORBID_COMP ("user_rate")
	FORBID_COMP ("intermed_rate")
	FORBID_COMP ("nic_tx")
	FORBID_COMP ("nic_rx")
	FORBID_COMP ("parity")
	FORBID_COMP ("conn_elem")
	FORBID_COMP ("modem_type")
	FORBID_COMP ("modem_type_2")
	FORBID_COMP ("fnur")
	FORBID_COMP ("acc")
	FORBID_COMP ("mTch")
	FORBID_COMP ("uimi")
	FORBID_COMP ("waiur")
	FORBID_COMP ("l2_ident")
	FORBID_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_FAX_T_2400)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_FAX_GROUP_3)
	FORBID_COMP ("coding_bc3x1")
	FORBID_COMP ("ctm")
	FORBID_COMP ("speech_vers1")
	FORBID_COMP ("coding_bc3x2")
	FORBID_COMP ("speech_vers2")
	FORBID_COMP ("coding_bc3x3")
	FORBID_COMP ("speech_vers3")
	FORBID_COMP ("coding_bc3x4")
	FORBID_COMP ("speech_vers4")
	FORBID_COMP ("coding_bc3x5")
	FORBID_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	SYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_2_4_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_FAX_T_4800)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_FAX_GROUP_3)
	FORBID_COMP ("coding_bc3x1")
	FORBID_COMP ("ctm")
	FORBID_COMP ("speech_vers1")
	FORBID_COMP ("coding_bc3x2")
	FORBID_COMP ("speech_vers2")
	FORBID_COMP ("coding_bc3x3")
	FORBID_COMP ("speech_vers3")
	FORBID_COMP ("coding_bc3x4")
	FORBID_COMP ("speech_vers4")
	FORBID_COMP ("coding_bc3x5")
	FORBID_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	SYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_FAX_T_9600)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_FAX_GROUP_3)
	FORBID_COMP ("coding_bc3x1")
	FORBID_COMP ("ctm")
	FORBID_COMP ("speech_vers1")
	FORBID_COMP ("coding_bc3x2")
	FORBID_COMP ("speech_vers2")
	FORBID_COMP ("coding_bc3x3")
	FORBID_COMP ("speech_vers3")
	FORBID_COMP ("coding_bc3x4")
	FORBID_COMP ("speech_vers4")
	FORBID_COMP ("coding_bc3x5")
	FORBID_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	SYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_T_300)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	FORBID_COMP ("coding_bc3x1")
	FORBID_COMP ("ctm")
	FORBID_COMP ("speech_vers1")
	FORBID_COMP ("coding_bc3x2")
	FORBID_COMP ("speech_vers2")
	FORBID_COMP ("coding_bc3x3")
	FORBID_COMP ("speech_vers3")
	FORBID_COMP ("coding_bc3x4")
	FORBID_COMP ("speech_vers4")
	FORBID_COMP ("coding_bc3x5")
	FORBID_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

/* note that all bearer caps for 14.4kbps use UR_9_6_KBIT over the air interface */
BEGIN_MSTRUCT ("bearer_cap", BC_BS20_T_14400)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V32)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_V34)
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_ALL)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_14400)
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_T_14400_1)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_AUTO)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_NONE)
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_ALL)
	SET_COMP ("mTch",	MAX_TCH_1)
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_T_14400_2)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V34)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_V34)
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_ALL)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_14400)
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_T_14400_3)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS22_T_1200)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_1_2_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V22)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS24_T_2400)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_2_4_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V22BIS)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_T_4800)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_T_9600)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS23_T)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_1_2_KBIT_V23)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V23)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_T_300_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS22_T_1200_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_1_2_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS24_T_2400_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_2_4_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_T_4800_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_T_9600_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_T_300_FR)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_300_BTP)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA_PREF)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_14400_BTP)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA_PREF)
	SET_COMP ("modem_type",	MT_AUTO)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_NONE)
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_EMPTY)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_NOT_APPLICABLE)
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_14400_BTP_1)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA_PREF)
	SET_COMP ("modem_type",	MT_AUTO)
	SKIP_COMP ("modem_type_2")
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_EMPTY)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_NOT_APPLICABLE)
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_14400_BTP_2)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA_PREF)
	SET_COMP ("modem_type",	MT_AUTO)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_NONE)
	SKIP_COMP ("fnur")
	SET_COMP ("acc",	ACC_EMPTY)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_NOT_APPLICABLE)
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_14400_BTP_3)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA_PREF)
	SET_COMP ("modem_type",	MT_AUTO)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SET_COMP ("acc",	ACC_EMPTY)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_NOT_APPLICABLE)
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_300_BNTP)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP_PREF)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_14400_BNTP)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP_PREF)
	SET_COMP ("modem_type",	MT_AUTO)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_NONE)
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_EMPTY)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_NOT_APPLICABLE)
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_300_BTP_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA_PREF)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_300_BNTP_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP_PREF)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_1)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

/* derived from BC_BS21_NT_300_1 by setting outband flow control; needed for MOCs */
BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_1_OUTBAND)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident") 	/* outband flow control */
	SKIP_COMP ("user_inf_l2_prot")	/* outband flow control */
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_1_RSP)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_1_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_NT_14400_1)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_AUTO)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_NONE)
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_EMPTY)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_NOT_APPLICABLE)
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_NT_14400_2)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_AUTO)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_NONE)
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_ALL)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_14400)
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS22_NT_1200)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_1_2_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V22)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS22_NT_1200_OUTBAND)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_1_2_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V22)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS22_NT_1200_RSP)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_1_2_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V22)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS22_NT_1200_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_1_2_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS24_NT_2400)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_2_4_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V22BIS)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS24_NT_2400_V26TER)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_2_4_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V26_TER)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS24_NT_2400_V26TER_OUTBAND)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_2_4_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V26_TER)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS24_NT_2400_RSP)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_2_4_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V22_BIS)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS24_NT_2400_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_2_4_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_NT_4800)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_NT_4800_OUTBAND)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_NT_4800_RSP)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_NT_4800_A)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_NT_4800_A2)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_NT_4800_B)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP_PREF)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_NT_4800_C)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_NT_4800_D)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_8_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP_PREF)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS25_NT_4800_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_4_8_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_NT_9600)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_NT_9600A)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_NT_9600_OUTBAND)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_NT_9600_1)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_NT_9600_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_9600_BNTP)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP_PREF)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_9600_BNTPT)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CSTD_GSM)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_YES)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP_PREF)
	SET_COMP ("modem_type",	MT_AUTO)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_9600_TPT)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CSTD_GSM)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_UNSTRUCTURED)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS23_NT)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_1_2_KBIT_V23)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V23)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS23_NT_OUTBAND)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_1_2_KBIT_V23)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_ODD)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V23)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_2)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_EVEN)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_3)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_1)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_7)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_0)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_4)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_DATA)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_ISO6429)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_4B)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_NT_14400_4)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_AUTO)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_V34)
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_ALL)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_14400)
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS20_NT_14400_5)
	SET_COMP ("rad_chan_req",	RCR_FULL_PREF)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V21)
	SET_COMP ("modem_type_2",	OTHER_MODEM_TYPE_NONE)
	SET_COMP ("fnur",	FNUR_14400)
	SET_COMP ("acc",	ACC_EMPTY)
	SET_COMP ("mTch",	MAX_TCH_1)
	SET_COMP ("uimi",	UIMI_NOT_ALLOWED)
	SET_COMP ("waiur",	WAIUR_NOT_APPLICABLE)
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_4_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SET_COMP ("l2_ident",	L2_OCT_ID)
	SET_COMP ("user_inf_l2_prot",	L2_COPFC)
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_5)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_V21)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS21_NT_300_5_UDI)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_DIGITAL_UNRESTRICTED)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_V110)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_0_3_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP)
	SET_COMP ("modem_type",	MT_NONE)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_BOTH_T_9600)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_TRANSPA_PREF)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("bearer_cap", BC_BS26_BOTH_NT_9600)
	SET_COMP ("rad_chan_req",	RCR_FULL_ONLY)
	SET_COMP ("code",	CS_Q931)
	SET_COMP ("trans_mode",	TM_CIRCUIT)
	SET_COMP ("trans_cap",	ITC_AUDIO)
	SKIP_COMP ("coding_bc3x1")
	SKIP_COMP ("ctm")
	SKIP_COMP ("speech_vers1")
	SKIP_COMP ("coding_bc3x2")
	SKIP_COMP ("speech_vers2")
	SKIP_COMP ("coding_bc3x3")
	SKIP_COMP ("speech_vers3")
	SKIP_COMP ("coding_bc3x4")
	SKIP_COMP ("speech_vers4")
	SKIP_COMP ("coding_bc3x5")
	SKIP_COMP ("speech_vers5")
	SET_COMP ("compress",	COMP_NO)
	SET_COMP ("structure",	STRC_SERV_DATA_INTEG)
	SET_COMP ("duplex",	DUPLEX_FULL)
	SET_COMP ("config",	CONF_POINT_TO_POINT)
	SET_COMP ("nirr",	NIRR_NO_MEANING)
	SET_COMP ("establish",	ESTAB_DEMAND)
	SET_COMP ("access_ident",	AI_OCT_ID)
	SET_COMP ("rate_adapt",	RA_NONE)
	SET_COMP ("sig_access_prot",	SIAP_I440)
	SET_COMP ("l1_ident",	L1_OCT_ID)
	SET_COMP ("user_inf_l1_prot",	L1_DEFAULT)
	SET_COMP ("sync_async",	ASYNCHRONOUS)
	SET_COMP ("num_stop",	NSTOP_2)
	SET_COMP ("negotiate",	NEGOTIATE_NO)
	SET_COMP ("num_data",	NDATA_8)
	SET_COMP ("user_rate",	UR_9_6_KBIT)
	SET_COMP ("intermed_rate",	IR_16_KBIT)
	SET_COMP ("nic_tx",	NIC_TX_NO)
	SET_COMP ("nic_rx",	NIC_RX_NO)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("conn_elem",	CE_RLP_PREF)
	SET_COMP ("modem_type",	MT_V32)
	SKIP_COMP ("modem_type_2")
	SKIP_COMP ("fnur")
	SKIP_COMP ("acc")
	SKIP_COMP ("mTch")
	SKIP_COMP ("uimi")
	SKIP_COMP ("waiur")
	SKIP_COMP ("l2_ident")
	SKIP_COMP ("user_inf_l2_prot")
ENDSTRUCT

BEGIN_MSTRUCT ("facility", FACILITY_A)
SET_COMP ("fac", FACILITY_A_6)
ENDSTRUCT

BEGIN_MSTRUCT ("ss_version", 	SS_VERS_3)
	SET_COMP ("ver", 	SS_VERS_3_ARRAY)
ENDSTRUCT

/* cc_cause (CC cause) */

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_BEARER_CAP_AUTH)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_BEARER_CAP_AUTHORIZ)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_UNSPECIFIED)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_UNSPECIFIED)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_TEMP_FAIL)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_PUB_NET_LOCAL_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_TEMP_FAIL)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGINARRAY (CC_DIAG_CAUSE_MT, 1) B_STATUS ENDARRAY

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_MSG_B_STATUS)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_MESSAGE_TYPE_INCOMPAT)
	SET_COMP ("diag",	CC_DIAG_CAUSE_MT)
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_MSG_B_STATUS_2)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_MESSAGE_INCOMPAT)
	SET_COMP ("diag",	CC_DIAG_CAUSE_MT)
ENDSTRUCT

BEGINARRAY (CC_PROCEED_CAUSE_MT, 1) D_CALL_PROCEED ENDARRAY

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_MSG_D_CALL_PROCEED)
	SET_COMP ("cs",	CS_GSM_PLMN) 
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_MESSAGE_TYPE_INCOMPAT)
	SET_COMP ("diag",	CC_PROCEED_CAUSE_MT)
ENDSTRUCT

BEGINARRAY (CC_ALERT_CAUSE_MT, 1) D_ALERT ENDARRAY

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_MSG_D_ALERT)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_MESSAGE_TYPE_INCOMPAT)
	SET_COMP ("diag",	CC_ALERT_CAUSE_MT)
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_INVALID_TI)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_INVALID_TI)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_USER_BUSY)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_USER_BUSY)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_CALL_REJECT)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_CALL_REJECT)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_INCOMPAT)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_INCOMPAT_DEST)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGINARRAY (CC_TIMER_303, 3) 0x33, 0x30, 0x33 ENDARRAY

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_TIMER_303)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_TIMER)
	SET_COMP ("diag",	CC_TIMER_303)
ENDSTRUCT

BEGINARRAY (CC_TIMER_310, 3) 0x33, 0x31, 0x30 ENDARRAY

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_TIMER_310)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_TIMER)
	SET_COMP ("diag",	CC_TIMER_310)
ENDSTRUCT

BEGINARRAY (CC_TIMER_313, 3) 0x33, 0x31, 0x33 ENDARRAY

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_TIMER_313)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_TIMER)
	SET_COMP ("diag",	CC_TIMER_313)
ENDSTRUCT

BEGINARRAY (CC_TIMER_323, 3) 0x33, 0x32, 0x33 ENDARRAY

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_TIMER_323)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")	
	SET_COMP ("cause",	CAUSE_TIMER)
	SET_COMP ("diag",	CC_TIMER_323)
ENDSTRUCT

BEGINARRAY (CC_TIMER_332, 3) 0x33, 0x33, 0x32 ENDARRAY

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_TIMER_332)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")	
	SET_COMP ("cause",	CAUSE_TIMER)
	SET_COMP ("diag",	CC_TIMER_332)
ENDSTRUCT

BEGINARRAY (CC_TIMER_335, 3) 0x33, 0x33, 0x35 ENDARRAY

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_TIMER_335)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")	
	SET_COMP ("cause",	CAUSE_TIMER)
	SET_COMP ("diag",	CC_TIMER_335)
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_CCBS_TIMER_4)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")	
	SET_COMP ("cause",	CAUSE_TIMER)
	SET_COMP ("diag",	CC_TIMER_335)
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_CHAN_UNACCEPT)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_CHAN_UNACCEPT)
	SKIP_COMP ("diag")
ENDSTRUCT

BEGIN_MSTRUCT ("cc_cause",	CC_CAUSE_STATUS_ENQ)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("loc",	LOC_USER)
	SKIP_COMP ("rec")
	SET_COMP ("cause",	CAUSE_STATUS_ENQUIRY)
	SKIP_COMP ("diag")
ENDSTRUCT

/* call_state (Call state) */

BEGIN_MSTRUCT ("call_state", 	CALL_STATE_CS_0)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("state",	CS_0)
ENDSTRUCT

/* call_state (Call state)  (VK, 24-june-1997) */

BEGIN_MSTRUCT ("call_state", 	CALL_STATE_CS_3)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("state",	CS_3)
ENDSTRUCT

BEGIN_MSTRUCT ("call_state", 	CALL_STATE_CS_4)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("state",	CS_4)
ENDSTRUCT

BEGIN_MSTRUCT ("call_state",	CALL_STATE_CS_9)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("state",	CS_9)
ENDSTRUCT

BEGIN_MSTRUCT ("call_state",	CALL_STATE_CS_10)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("state",	CS_10)
ENDSTRUCT

BEGIN_MSTRUCT ("call_state",	CALL_STATE_CS_26)
	SET_COMP ("cs",	CS_GSM_PLMN)
	SET_COMP ("state",	CS_26)
ENDSTRUCT

/* call_ctrl_cap (Call Control capabilities) */

BEGIN_MSTRUCT ("allowed_actions", ALLOWED_ACTIONS_CCBS_ON)
	SET_COMP ("ccbs_act",	CCBS_YES)
ENDSTRUCT

BEGIN_MSTRUCT ("recall_type", RECALL_TYPE_CCBS)
	SET_COMP ("rcl_type",	CCBS)
ENDSTRUCT

BEGIN_MSTRUCT ("notific", NOTIFIC_USER_SUSPEND)
	SET_COMP ("nd",	ND_USER_SUSPEND)
ENDSTRUCT

BEGIN_MSTRUCT ("notific", NOTIFIC_USED_RESUME)
	SET_COMP ("nd",	ND_USED_RESUME)
ENDSTRUCT

BEGIN_MSTRUCT ("notific", NOTIFIC_BEARER_CHANGE)
	SET_COMP ("nd",	ND_BEARER_CHANGE)
ENDSTRUCT

/* progress (Progress indicator) */
BEGIN_MSTRUCT ("progress", PROGRESS_INBAND_AVAIL)	/* VK 29-JUL-97 */
	SET_COMP ("cs",	CS_GSM_PLMN)	/* VK 29-JUL-97 */
	SET_COMP ("loc",	LOC_USER)	/* VK 29-JUL-97 */
	SET_COMP ("progress_desc",	PROG_INBAND_AVAIL)	/* VK 29-JUL-97 */
ENDSTRUCT

BEGIN_MSTRUCT ("setup_cont", SETUP_CONT_SPEECH_FR_HR_EFR)
	SET_COMP ("setup_msg", SETUP_CONT_SPEECH_FR_HR_EFR_15)
ENDSTRUCT
BEGIN_MSTRUCT ("setup_cont", SETUP_CONT_SPEECH_FR)
	SET_COMP ("setup_msg", SETUP_CONT_SPEECH_FR_13)
ENDSTRUCT

/* USER_A */

BEGIN_MSTRUCT ("user_user", USER_A)
	SET_COMP ("pd", PD_5)
	SET_COMP ("info", USER_A_4)
ENDSTRUCT

# TEST CASES

## Routing (internal)

### CC000: Setup the routing and PCO view for the CC test

Description:	Routings for the CC tests are set.

Preamble:	None

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (CC RESET)
COMMAND (MM RESET)
COMMAND (PL RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
	 |		  |		  |
COMMAND (MMI REDIRECT CC NULL)
	 |		  |		  |
COMMAND (CC REDIRECT MMI TAP)
COMMAND (CC REDIRECT MM TAP)
	 |		  |		  |
	 |		  |		  |
COMMAND (TAP REDIRECT TAP CC)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	04-Jul-97	PZ	Initial
	06 March 2001	MLP	Copied from GSM ETS
	06 July 2001	JHU	Converted to TAP2
	22-Aug-01	SBK	Removed obsolete commands to speed up

## Basic Preambles

### CC001: Configuration of connection element

Description:	CC is configured for the connection element for MTCs to be transparant (Variant A) or both, transparent preffered.

Variants:	<A>..<B>

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_T_300
<B>		bcpara	BC_PARA_BS21_BTP_300
<A>		sns_mode	SNS_MODE_DATA
<B>		sns_mode	SNS_MODE_DATA
		ctm_ena	CTM_DISABLED

History:	22-Aug-2001	SBK	Initial
	18-Mar-2002	HM	Bearer cap parameter instead bearer caps
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

## Call Establishment: Mobile Originated Call

### CC011: Establish Mobile Originated Call - Data (U0)

Description:	Upon receiving a MNCC-SETUP request primitive from MMI CC forwards a MMCC-ESTABLISH request primitive to MM, starts the T303 timer and enters the state MO CONNECTION PENDING (U0.1). (Ref. [1] 5.2.1.2)

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

History:	25-Apr-97	DL	Initial
	23-Jun-97	VK	MNCC_SETUP_REQ parameter revised
	27-Jul-98	LE	adapted to PCS 1900
	27-Jun-00	HM	establish cause not indicating HR support
	05-Jul-01	JHU	Converted to TAP2

### CC012: Establish Mobile Originated Call - Speech (U0)

Description:	Upon receiving a MNCC-SETUP request primitive from MMI CC forwards a MMCC-ESTABLISH request primitive to MM, starts the T303 timer and enters the state MO CONNECTION PENDING (U0.1). (Ref. [1] 5.2.1.2). Speech Codec support for Enhanced Fullrate.

Variants:	<A>....<F>

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
	<A>	prio	PRIO_NORM_CALL
	<B>	prio	PRIO_EMERG_CALL
	<C>	prio	PRIO_NORM_CALL
	<D>	prio	PRIO_NORM_CALL
	<E>	prio	PRIO_EMERG_CALL
	<F>	prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
	<A>	bcpara	BC_PARA_SPEECH
	<B>	bcpara	BC_PARA_SPEECH
	<C>	bcpara	BC_PARA_SPEECH_LINE_2
	<D>	bcpara	BC_PARA_SPEECH_CTM
	<E>	bcpara	BC_PARA_SPEECH_CTM
	<F>	bcpara	BC_PARA_SPEECH_LINE_2_CTM
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
	<A>	estcs	ESTCS_MOB_ORIG_SPCH
	<B>	estcs	ESTCS_EMERGE
	<C>	estcs	ESTCS_MOB_ORIG_SPCH
	<D>	estcs	ESTCS_MOB_ORIG_SPCH
	<E>	estcs	ESTCS_EMERGE
	<F>	estcs	ESTCS_MOB_ORIG_SPCH

History:	25-Apr-97	DL	Initial
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	12-Jul-02	HM	ESTCS_EMERGE for emergency call
	05-Dec-2002	FK	Variants for CTM added

### CC013: Establish Mobile Originated Call - Speech (U0)

Description:	Upon receiving a MNCC-SETUP request primitive from MMI CC forwards a MMCC-ESTABLISH request primitive to MM, starts the T303 timer and enters the state MO CONNECTION PENDING (U0.1). (Ref. [1] 5.2.1.2). Speech Codec support for AMR.

Variants:	<A>....<B>

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=30)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
	<A>	bcpara	BC_PARA_SPEECH
	<B>	bcpara	BC_PARA_SPEECH_CTM
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_SPCH

History:	25-Apr-97	DL	Initial
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	11-Jun-02	HM	std=30 instead of std=8
	05-Dec-2002	FK	Variants for CTM added

### CC021: MM connection established for basic call - data (U0.1)

Description:	MM signals CC in the form of a MMCC-ESTABLISH confirmation primitive that a connection has been successfully established. CC issues a MMCC-DATA request primitive and enters the state CALL INIT (U1). (Ref. [1] 5.n.n.n)

Preamble:	CC011

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC022: MM connection established - basic call - speech (U0.1)

Description:	MM signals CC in the form of a MMCC-ESTABLISH confirmation primitive that a connection has been successfully established. CC issues a MMCC-DATA request primitive and enters the state CALL INIT (U1). (Ref. [1] 5.n.n.n)

Variants:	<A>....<D>

Preamble:	<A>	CC012A
	<B>	CC013A
	<C>	CC012D
	<D>	CC013B

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	FORBID
	<A>	bearer_cap	BC_SPEECH_FR
	<B>	bearer_cap	BC_SPEECH_AMR_EFR_HR_FR
	<C>	bearer_cap	BC_SPEECH_FR_CTM
	<D>	bearer_cap	BC_SPEECH_AMR_EFR_HR_FR_CTM
		bearer_cap_2	FORBID
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax and data
	06 March 2001	MLP	Copied from GSM ETS
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant <A>…<B> introduced, second
				one for AMR
	05-Dec-2002	FK	Variants for CTM added

### CC023: MM connection established - basic call - speech line 2(U0.1)

Description:	MM signals CC in the form of a MMCC-ESTABLISH confirmation primitive that a connection has been successfully established. CC issues a MMCC-DATA request primitive and enters the state CALL INIT (U1). (Ref. [1] 5.n.n.n)

Variants:	<A>....<B>

Preamble:
	<A>	CC012C
	<B>	CC012F

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	FORBID
	<A>	bearer_cap	BC_AUX_SPEECH_FR
	<B>	bearer_cap	BC_AUX_SPEECH_FR_CTM
		bearer_cap_2	FORBID
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax and data
	05-Jul-01	JHU	Converted to TAP2
	05-Dec-2002	FK	Variants for CTM added

### CC024: MM connection established - emergency speech call (U0.1)

Description:	MM signals CC in the form of a MMCC-ESTABLISH confirmation primitive that a connection has been successfully established. CC issues a MMCC-DATA request primitive and enters the state CALL INIT (U1). (Ref. [1] 5.n.n.n)

Variants:	<A>....<B>

Preamble:
	<A>	CC012B
	<B>	CC012E

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (EMERGENCY SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_EMERGE_SETUP
		ti	TI_MO_5
	<A>	bearer_cap	NOT_USED
	<B>	bearer_cap	BC_SPEECH_FR_CTM
		}

History:	01-Apr-97	DL	Initial
	27-Jun-97	VK	bearer_cap NOT_USED
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06 March 2001	MLP	Copied from GSM ETS
	05-Jul-01	JHU	Converted to TAP2
	05-Dec-2002	FK	Variants for CTM added

### CC025: PROGRESS message received (U1)

Description:	While in the state CALL INIT (U1) CC receives a PROGRESS message in the form of a MNCC-DATA request primitive. CC sgends a MNCC-PROGRESS indication primitive to MMI. (Ref. [1] 5.4.3.2)

Preamble:	CC024A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (PROGRESS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_PROGRESS_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_PROGRESS
		ti	TI_MO_5_RESP
		progress	PROG_1
		user_user	NOT_USED
		}

- MNCC_PROGRESS_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN

History:	25-Apr-97	DL	Initial
	30-Jun-97	VK	new is: PROG_1in MMCC_DATA_IND
	30-Jun-97	VK	Preamble is CC024 (was CC011A)
	01-Jul-97	VK	PROGRESS_IND params revised
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	05-Dec-2002	HM	Number of preamble changed

### CC026: CALL PROCEEDING message received - speech (U1)

Description:	While in the state CALL INIT (U1) CC receives a CALL PROCEEDING message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then issues a MNCC-CALL-PROCEED indication primitive and enters the state MO CALL PROCEEDING (U3). (Ref. [1] 5.2.1.3)

Variants:	<A>....<B>

Preamble:	<A>CC022A
		<B>CC022B

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(2)	 |	MNCC_CALL_PROCEED_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MNCC_CALL_PROCEED_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE

History:	25-Apr-97	DL	Initial
	01-Jul-97	VK	Preamble is CC022 (was CC026 !!!)
	01-Jul-97	VK	‘repeat’ is NOT_USED
	01-Jul-97	VK	‘progress’ renamed to ‘progress_desc’
	01-Jul-97	VK	‘fac_inf’ introduced
	01-Jul-97	VK	‘serv1’ assigned SERV_SPEECH
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06 March 2001	MLP	Copied from GSM ETS
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant <A>…<B> introduced, second 				one for AMR.

### CC054: CALL PROCEEDING message received - data (U1)

Description:	While in the state CALL INIT (U1) CC receives a CALL PROCEEDING message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then issues a MNCC-CALL-PROCEED indication primitive and enters the state MO CALL PROCEEDING (U3). (Ref. [1] 5.2.1.3)

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(2)	 |	MNCC_CALL_PROCEED_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MNCC_CALL_PROCEED_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300
		bcpara2	BC_PARA_NO_SERVICE

History:	27-Jul-98	LE	initial
	05-Jul-01	JHU	Converted to TAP2

### CC027: ALERT message received (U1)

Description:	CC receives an ALERT message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then stops the T303 timer, enters the state CALL DELIVERED (U4) and issues a MNCC-ALERT indication primitive. (Ref. [1] 5.2.1.5)

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(ALERT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_ALERT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_ALERT
		ti	TI_MO_5_RESP
		facility	NOT_USED
		progress	PROG_1
		user_user	NOT_USED
		}

- MNCC_ALERT_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN

History:	25-Apr-97	DL	Initial
	01-Jul-97	VK	ALERT_IND params revised
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC028: (GSM )TCH assignment from network during call establishment (U1)

Description:	While in the state CALL INIT (U1) CC receives a traffic channel (TCH) assignment in the form of a MMCC-SYNC indication primitive. CC sends a MNCC-SYNC indication primitive to MMI. (Ref. [1] 5.5.2.2.7)

SHOULD PREABLE BE CC021 ?

NOT SUPPORTED FOR UMTS

Preamble:	CC022A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_SYNC_IND
		ti	TI_MO_5
		chm	CH_SIG_ONLY

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CHANNEL_SYNC
		chm	CH_SIG_ONLY

History:	25-Apr-97	DL	Initial
	01-Jul-97	VK	‘cause’ and ‘mode’ introduced
	01-Jul-97	VK	use MNCC.DOC constants
	27-Jul-98	LE	adapted to fax & data
	19-Jul-99	LE	CH_SIG_ONLY used
	06 March 2001	MLP	Copied from GSM ETS
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC022A instead of CC022

### CC029: Facility information element sent to  network (U1)

Description:	CC receives a MNCC-FACILITY request primitive from MMI. CC builds a FACILITY message and sends this as part of a MNCC-DATA request primitive to MM. (Ref. [1] 5.?.?)

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_FACILITY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (FACILITY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_FACILITY_REQ
		ti	TI_MO_5
		fac_inf	FACILITY_A_FAC
		ss_version	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_FACILITY
		ti	TI_MO_5
		facility	FACILITY_A
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC030: Facility information element received from network (U1)

Description:	CC receives a FACILITY message from the network as part of a MMCC-DATA indication primitive. CC informs MMI by issuing a MNCC-FACILITY indication primitive.

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (FACILITY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MO_5_RESP
		facility	FACILITY_A
		}

- MNCC_FACILITY_IND
		ti	TI_MO_5
		fac_context	FAC_IN_FACILITY
		fac_inf	FACILITY_A_FAC

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC031: STATUS message with different call state received from network (U1)

Description:	CC receives a STATUS message from the network in the form of a MMCC-ESTABLISH indication primitive while in the state CALL INIT (U1). The call state in the message received is not identical to the current call state of  CC, whereby CC immediately issues a message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_9
		aux_states	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_MESSAGE_INCOMPAT

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	CC_CAUSE_MSG_B_STATUS_2
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	06-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC032: CONNECT message received - speech (U1)

Description:	While in the state CALL INIT (U1) CC receives a CONNECT message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then stops the T303 timer, enters the state ACTIVE (U10) and issues a MNCC-SETUP confirmation primitive to MMI and a CONNECT ACKNOWLEDGE message as part of a MMCC-DATA request primitive to MM. (Ref. [1] 5.2.1.6)

Preamble:	CC022A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CONNECT)	  |
	 |		  *<==============================*
(4)	 |	MNCC_SETUP_CNF	  |		  |
	 *<==============================*		  |
(5)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CONNECT ACKNOWLEDGE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_SYNC_IND
		ti	TI_MO_5
		chm	CH_SPEECH_FULL

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CHANNEL_SYNC
		chm	CH_SPEECH_FULL

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CONNECT
		ti	TI_MO_5_RESP
		facility	NOT_USED
		progress	PROG_1
		connect_num	CONNECT_NUM_654321
		connect_subaddr	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_SUCCESS
		progress_desc	PROG_END_TO_END_PLMN
		connected_number	CONNECTED_NUMBER_654321
		connected_number_sub	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_CONNECT_ACK
		ti	TI_MO_5
		}

History:	25-Apr-97	DL	Initial
	03-Jul-97	VK	MNCC_SETUP_CNF revised
	04.-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	12-Oct-00	HM	TCH assignment before CONNECT
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC022A instead of CC022

### CC033: PROGRESS message received (U3)

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a PROGRESS message in the form of a MNCC-DATA request primitive. CC sends a MNCC-PROGRESS indication primitive to MMI. (Ref. [1] 5.4.3.2)

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (PROGRESS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_PROGRESS_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_PROGRESS
		ti	TI_MO_5_RESP
		progress	PROG_1
		user_user	NOT_USED
		}

- MNCC_PROGRESS_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN

History:	25-Apr-97	DL	Initial
	03-Jul-97	VK	‘progress’ in D_PROGRESS introduced
	03-Jul-97	VK	PROGRESS_IND params. revised
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC026A instead of CC026

### CC034: CALL PROCEEDING message received (U3)

Description:	CC receives a CALL PROCEEDING message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then builds a status message with cause set to CC_CAUSE_MSG_INCOMPAT and sends this as part of  a MNCC-DATA request primitive. (Ref. [1] 5.2.1.3)

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STATUS)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MO_5
		cc_cause	CC_CAUSE_MSG_D_CALL_PROCEED
		call_state	CALL_STATE_CS_3
		aux_states	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	03-Jul-97	VK	repeat set to NOT_USED
	03-Jul-97	VK	Preamble is CC026 (was CC021)
	03-Jul-97	VK	cc_cause revised
	04-Jul-97	VK	aux_states revised
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC026A instead of CC026

### CC035: ALERT message received (U3)

Description:	CC receives an ALERT message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then stops the T303 timer, enters the state CALL DELIVERED (U4) and issues a MNCC-ALERT indication primitive. (Ref. [1] 5.2.1.5)

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(ALERT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_ALERT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_ALERT
		ti	TI_MO_5_RESP
		facility	NOT_USED
		progress	PROG_1
		user_user	NOT_USED
		}

- MNCC_ALERT_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN

History:	25-Apr-97	DL	Initial
	03-Jul-97	VK	MNCC_ALERT_IND params revised
	04-Aug97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC026A instead of CC026

### CC036: TCH assignment from network during call establishment (U3)

Description:	While in the state CALL MO CALL PROCEEDING (U3) CC receives a traffic channel (TCH) assignment in the form of a MMCC-SYNC indication primitive. CC sends a MNCC-SYNC indication primitive to MMI. (Ref. [1] 5.5.2.2.7)

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_SYNC_IND
		ti	TI_MO_5
		chm	CH_SPEECH_FULL

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CHANNEL_SYNC
		chm	CH_SPEECH_FULL

History:	25-Apr-97	DL	Initial
	03-Jul-97	VK	MNCC_SYNC_IND params revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC026A instead of CC026

### CC037: Facility information element sent to  network (U3)

Description:	CC receives a MNCC-FACILITY request primitive from MMI. CC builds a FACILITY message and sends this as part of a MNCC-DATA request primitive to MM. (Ref. [1] 5.?.?)

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_FACILITY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (FACILITY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_FACILITY_REQ
		ti	TI_MO_5
		fac_inf	FACILITY_A_FAC
		ss_version	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_FACILITY
		ti	TI_MO_5
		facility	FACILITY_A
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC026A instead of CC026

### CC038: Facility information element received from network (U3)

Description:	CC receives a FACILITY message from the network as part of a MMCC-DATA indication primitive. CC informs MMI by issuing a MNCC-FACILITY indication primitive.

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (FACILITY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MO_5_RESP
		facility	FACILITY_A
		}

- MNCC_FACILITY_IND
		ti	TI_MO_5
		fac_context	FAC_IN_FACILITY
		fac_inf	FACILITY_A_FAC

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC026A instead of CC026

### CC040: STATUS message with different call state received from network (U3)

Description:	CC receives a STATUS message from the network in the form of a MMCC-DATA indication primitive while in the state CALL PROCEEDING (U3). The call state in the message received is not identical to the current call state of  CC, whereby CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_9
		aux_states	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_MESSAGE_INCOMPAT

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	CC_CAUSE_MSG_B_STATUS_2
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	05-Nov-97	LE	Revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC026A instead of CC026

### CC041: CONNECT message received - speech (U3)

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a CONNECT message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then stops the T303 timer, enters the state ACTIVE (U10) and issues a MNCC-SETUP confirmation primitive to MMI and a CONNECT ACKNOWLEDGE message as part of a MMCC-DATA request primitive to MM. (Ref. [1] 5.2.1.6)

Variants:	<A>....<B>

Preamble:	<A>CC026A

<B>CC026B

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_CNF	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CONNECT ACKNOWLEDGE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CONNECT
		ti	TI_MO_5_RESP
		facility	NOT_USED
		progress	PROG_1
		connect_num	CONNECT_NUM_654321
		connect_subaddr	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_SUCCESS
		progress_desc	PROG_END_TO_END_PLMN
		connected_number	CONNECTED_NUMBER_654321
		connected_number_sub	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_CONNECT_ACK
		ti	TI_MO_5
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant <A>…<B> introduced, second 				one for AMR.

### CC043: TCH assignment from network during call establishment (U3), data

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a traffic channel (TCH) assignment in the form of a MMCC-SYNC indication primitive. CC sends a MNCC-SYNC indication primitive to MMI. (Ref. [1] 5.5.2.2.7)

Preamble:	CC054

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_SYNC_IND
		ti	TI_MO_5
		chm	CH_DATA_9_6

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CHANNEL_SYNC
		chm	CH_DATA_9_6

History:	25-Apr-97	DL	Initial
	04-Jul-97	VK	MNCC_SYNC_IND params revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC044: PROGRESS message received (U4)

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a PROGRESS message in the form of a MNCC-DATA request primitive. CC sends a MNCC-PROGRESS indication primitive to MMI. (Ref. [1] 5.4.3.2)

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (PROGRESS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_PROGRESS_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_PROGRESS
		ti	TI_MO_5_RESP
		progress	PROG_1
		user_user	NOT_USED
		}

- MNCC_PROGRESS_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN

History:	25-Apr-97	DL	Initial
	04-Jul-97	VK	PROG_1 introduced
	04-Jul-97	VK	PROGRESS_IND params revised
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant CC026A instead of CC026

### CC045: CALL PROCEEDING message received (U4)

Description:	CC receives a CALL PROCEEDING message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then builds a status message with cause set to CC_CAUSE_MSG_INCOMPAT and sends this as part of  a MNCC-DATA request primitive. (Ref. [1] 5.2.1.3)

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STATUS)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MO_5
		cc_cause	CC_CAUSE_MSG_D_CALL_PROCEED
		call_state	CALL_STATE_CS_4
		aux_states	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Jul-97	VK	‘repeat’ takes on ‘NOT_USED’
	04-Jul-97	VK	...REQ cc_cause parameter revised
	04-Aug-97	VK	MNCC_DATA_REQ renamed to ...
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC046: ALERT message received (U4)

Description:	CC receives an ALERT message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then builds a status message with cause set to CC_CAUSE_MSG_INCOMPAT and sends this as part of  a MNCC-DATA request primitive.. (Ref. [1] 5.2.1.5)

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(ALERT)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STATUS)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_ALERT
		ti	TI_MO_5_RESP
		facility	NOT_USED
		progress	PROG_1
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MO_5
		cc_cause	CC_CAUSE_MSG_D_ALERT
		call_state	CALL_STATE_CS_4
		aux_states	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Jul-97	VK	new : CC_CAUSE_MSG_D_ALERT
	04-Jul-97	VK	‘aux_states’ assigned ‘NOT_USED’
	04-Aug-97	VK	MNCC_DATA_REQ renamed to ...
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC047: TCH assignment from network during call establishment (U4)

Description:	While in the state CALL DELIVERED (U4) CC receives a traffic channel (TCH) assignment in the form of a MMCC-SYNC indication primitive. CC sends a MNCC-SYNC indication primitive to MMI. (Ref. [1] 5.5.2.2.7)

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_SYNC_IND
		ti	TI_MO_5
		chm	CH_DATA_4_8

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CHANNEL_SYNC
		chm	CH_DATA_4_8

History:	25-Apr-97	DL	Initial
	04-Jul-97	VK	MNCC_SYNC_IND params revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC048: Facility information element sent to network (U4)

Description:	CC receives a MNCC-FACILITY request primitive from MMI. CC builds a FACILITY message and sends this as part of a MNCC-DATA request primitive to MM.

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_FACILITY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (FACILITY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_FACILITY_REQ
		ti	TI_MO_5
		fac_inf	FACILITY_A_FAC
		ss_version	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_FACILITY
		ti	TI_MO_5
		facility	FACILITY_A
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC049: Facility information element received from network (U4)

Description:	CC receives a FACILITY message from the network as part of a MMCC-DATA indication primitive. CC informs MMI by issuing a MNCC-FACILITY indication primitive.

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (FACILITY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MO_5_RESP
		facility	FACILITY_A
		}

- MNCC_FACILITY_IND
		ti	TI_MO_5
		fac_context	FAC_IN_FACILITY
		fac_inf	FACILITY_A_FAC

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC051: STATUS message with different call state received from network (U4)

Description:	CC receives a STATUS message from the network in the form of a MMCC-DATA indication primitive while in the state CALL INIT (U4). The call state in the message received is not identical to the current call state of  CC, whereby CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_9
		aux_states	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_MESSAGE_INCOMPAT

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	CC_CAUSE_MSG_B_STATUS_2
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	05-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC053: CONNECT message received - data (U4)

Description:	While in the state CALL DELIVERED (U4) CC receives a CONNECT message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then stops the T303 timer, enters the state ACTIVE (U10) and issues a MNCC-SETUP confirmation primitive to MMI and a CONNECT ACKNOWLEDGE message as part of a MMCC-DATA request primitive to MM. (Ref. [1] 5.2.1.6)

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_CNF	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CONNECT ACKNOWLEDGE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CONNECT
		ti	TI_MO_5_RESP
		facility	NOT_USED
		progress	PROG_1
		connect_num	CONNECT_NUM_654321
		connect_subaddr	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_SUCCESS
		progress_desc	PROG_END_TO_END_PLMN
		connected_number	CONNECTED_NUMBER_654321
		connected_number_sub	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_CONNECT_ACK
		ti	TI_MO_5
		}

History:	25-Apr-97	DL	Initial
	07-Jul-97	VK	MNCC_SETUP_CNF params revised
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC060: CC receives MMCC_PROMPT_IND

Description:	CC receives a MMCC_PROMPT_IND from the network. This primitive is forwarded transparently to the MMI without further CC action.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_PROMPT_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_PROMPT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_PROMPT_IND

- MNCC_PROMPT_IND

History:	28-Jan-00	HM	Initial
05-Jul-01	JHU	Converted to TAP2

### CC061: CC receives MNCC_PROMPT_RSP

Description:	CC receives MNCC_PROMPT_RSP from the MMI. A mobile originated transaction identifier is assigned, MM is notified by MMCC_PROMPT_RSP. A START CC message is sent to the network with the primitive MMCC_DATA_REQ. CC waits in state U0.3 for further network information.

Preamble:	CC060

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_PROMPT_RSP	  |		  |
	 |	(ti)	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_PROMPT_RSP	  |
	 |		  |	(ti)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(START CC)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_PROMPT_RSP
		ti	TI_MO_5

- MMCC_PROMPT_RSP
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_START_CC
		ti	TI_MO_5
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	28-Jan-00	HM	Initial
05-Jul-01	JHU	Converted to TAP2

### CC062: CC receives MNCC_PROMPT_REJ

Description:	CC receives MNCC_PROMPT_REJ. This primitive is transparently forwarded to MM. By this primitive the MMI requests the rejection of the Network Initiated MO CM request e.g. due to a temporary lack of tranaction identifiers.

Preamble:	CC060

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_PROMPT_REJ	  |		  |
	 *==============================>*		  |
 (2)	 |		  |	MMCC_PROMPT_REJ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_PROMPT_REJ

- MMCC_PROMPT_REJ

History:	31-Jan-00	HM	Initial
05-Jul-01	JHU	Converted to TAP2

### CC063: CC receives CC ESTABLISHMENT message, FR mobile only

Description:	CC receives the CC ESTABLISHMENT message in state U.03. This causes emission of CC ESTABLISHMENT CONFIRMED message to the network and state transition to state U0.5. As the SETUP message provided by the SETUP container contains bearer capabilities for enhanced full rate, full rate and half rate codecs and the mobile only provides full rate speech version 1, the bearer caps are changed to the mobile's capabilities.

Preamble:	CC061

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(CC ESTABLISHMENT)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(CC EST. CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CC_ESTABLISHMENT
		ti	TI_MO_5_RESP
		setup_cont	SETUP_CONT_SPEECH_FR_HR_EFR
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CC_EST_CONF
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	03-Feb-00	HM	Initial
05-Jul-01	JHU	Converted to TAP2
04-Apr-02	HM	Basic servive group alignment fixed.

### CC064: CC receives CC ESTABLISHMENT message, EFR_FR_HR mobile

Description:	CC receives the CC ESTABLISHMENT message in state U.03. This causes emission of CC ESTABLISHMENT CONFIRMED message to the network and state transition to state U0.5. In case the SETUP message provided by the SETUP container contains bearer capabilities for full rate speech code version 1 only, but the mobile is able of using enhanced full rate, full rate and half rate codecs, the bearer caps are changed to the mobile's capabilities.

Preamble:	CC061

Variants:	<A>..<B>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(CC ESTABLISHMENT)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(CC EST. CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CC_ESTABLISHMENT
		ti	TI_MO_5_RESP
<A>		setup_cont	SETUP_CONT_SPEECH_FR
<B>		setup_cont	SETUP_CONT_SPEECH_FR_HR_EFR
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CC_EST_CONF
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR_HR_EFR
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	04-Apr-02	HM	Initial

### CC065: CC receives RECALL

Description:	CC sent  CC ESTABLISHMENT CONFIRMED message and receives RECALL message, the relevant informations received with the CC ESTABLISHMENT (setup container) and RECALL are forwarded by MNCC_RECALL_IND to upper layers.

Preamble:	CC063

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(RECALL)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RECALL_IND	  |		  |
	 *<==============================*		  |
(3)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RECALL
		ti	TI_MO_5_RESP
		recall_type	RECALL_TYPE_CCBS
		facility	FACILITY_A
		}

- MNCC_RECALL_IND 
		ti	TI_MO_5
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		rcl_type	RCL_CCBS

- MNCC_FACILITY_IND	ti	TI_MO_5
		fac_context	FAC_IN_RECALL
		fac_inf	FACILITY_A_FAC

History:	04-Feb-00	HM	Initial
	21-Mar-00	HM	Facility now transferred separately
	05-Jul-01	JHU	Converted to TAP2

### CC066: CC receives RECALL, user wants connection

Description:	CC sent  CC ESTABLISHMENT CONFIRMED message and received RECALL message.  The user wants a connection. A mobile originated call is following.

Preamble:	CC065

MMI		 CC		 MMI
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(SETUP)	  |
	 |		  *===============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	NOT_USED
		bcpara2	NOT_USED
		called_party	NOT_USED
		called_party_sub	NOT_USED
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		fac_adv	NOT_USED
		}

History:	07-Feb-00	HM	Initial
05-Jul-01	JHU	Converted to TAP2
03-Apr-02	HM	PRIO_CCBS_CALL => PRIO_NORM_CALL,
		basic service group alignment fixed

### CC067: CC receives RECALL, user refuses RECALL

Description:	CC received the RECALL message. The user doesn't want a connection. The CCBS recall is rejected by sending of a RELEASE COMPLETE message with CAUSE_CALL_REJECT to the network. If a RELEASE COMPLETE with this cause is received by the network the CCBS entry is cancelled by the network.

Preamble:	CC065

MMI		 CC		 MMI
	 |		  |		  |
(1)	 |	MNCC_REJECT_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_REJECT_REQ
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_REJECT

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_REJECT
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	07-Feb-00	HM	Initial
05-Jul-01	JHU	Converted to TAP2

### CC068: CC receives RECALL, no answer from user

Description:	CC sent  CC ESTABLISHMENT CONFIRMED message and received RECALL message, which is forwarded as facility indication to upper layers. The user doesn't answer the RECALL attempt,.network sends RELEASE COMPLETE.

Preamble:	CC065

MMI		 CC		 MMI
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(RELEASE COMPLETE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE_COMP
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CCBS_TIMER_4
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_TIMER

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	07-Feb-00	HM	Initial
05-Jul-01	JHU	Converted to TAP2

### CC069: Timeout of T332

Description:	CC sent START CC message and is awaiting CC ESTABLISHMENT from network. The message from network is not received. A timeout of T332 will occur.

Preamble:	CC061

MMI		 CC		 MM
	 |		  |		  |
MUTE (25000)
(1)	 |	MNCC_RELEASE_IND	  |		  |
 	 *<==============================*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_TIMER

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	CC_CAUSE_TIMER_332
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	31-Jan-00	HM	Initial
05-Jul-01	JHU	Converted to TAP2

### CC070: CC receives CC ESTABLISHMENT message, no RECALL from net

Description:	CC receives the CC ESTABLISHMENT message in state U.03. This causes emission of CC ESTABLISHMENT CONFIRMED message to the network and state transition to state U0.5. Timer T335 expires, the call is released with RELEASE COMPLETE message.

Preamble:	CC061

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(CC ESTABLISHMENT)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(CC EST. CONFIRMED)	  |
	 |		  *==============================>*
MUTE (25000)
(3)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(RELEASE COMPLETE)	  |
	 |		  *==============================>*
(5)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CC_ESTABLISHMENT
		ti	TI_MO_5_RESP
		setup_cont	SETUP_CONT_SPEECH_FR_HR_EFR
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CC_EST_CONF
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_TIMER

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	CC_CAUSE_TIMER_335
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	03-Feb-00	HM	Initial
05-Jul-01	JHU	Converted to TAP2

## Call Establishment: Mobile Terminated Call

### CC081: SETUP message received from network (U0)

Description:	CC receives a SETUP message from the network in the form of a MMCC-ESTABLISH indication primitive. CC enters the state CALL PRESENT (U6) and starts compatibility checks. (Ref. [1] 5.2.2.1)

Variants:	<A>..<F>

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
		bcpara	BC_PARA_BS21_T_300
<A>		sns_mode	SNS_MODE_DATA
<B>		sns_mode	NOT_PRESENT_8BIT
<C>		sns_mode	SNS_MODE_DATA
<D>		sns_mode	SNS_MODE_DATA
<E>		sns_mode	NOT_PRESENT_8BIT
<F>		sns_mode	NOT_PRESENT_8BIT
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
<A>		repeat	NOT_USED
<B>		repeat	NOT_USED
<C>		repeat	REPEAT_SEQUENTIAL
<D>		repeat	REPEAT_SEQUENTIAL
<E>		repeat	NOT_USED
<F>		repeat	NOT_USED
<A>		bearer_cap	BC_BS21_T_300
<B>		bearer_cap	BC_SPEECH_FR
<C>		bearer_cap	BC_BS21_T_300
<D>		bearer_cap	BC_SPEECH_FR
<E>		bearer_cap	BC_AUX_SPEECH_FR
<F>		bearer_cap	BC_SPEECH_AMR_EFR_HR_FR
<A>		bearer_cap_2	NOT_USED
<B>		bearer_cap_2	NOT_USED
<C>		bearer_cap_2	BC_SPEECH_FR
<D>		bearer_cap_2	BC_BS21_T_300
<E>		bearer_cap_2	NOT_USED
<F>		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	06-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	04-Sep-00	LE	testcase for ALS added
	05-Jul-01	JHU	Converted to TAP2
	21-Aug-01	SBK	Adapted to NT default config
	26-Nov-01	OT	Variant F for AMR added.
	18-Mar-02	HM	Bearer cap parameter instead bearer caps
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC082: SETUP message received from network (U0)

Description:	CC receives a SETUP message from the network in the form of a MMCC-ESTABLISH indication primitive. CC enters the state CALL PRESENT (U6) and starts compatibility checks (Ref. [1] 5.2.2.1). One time an even number of digits is present in the message, the other time its an odd number.

Variants:	<A>..<B>

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
<A>		calling_num	CLNG_NUM_654321
<B>		calling_num	CLNG_NUM_7654321
		calling_subaddr	NOT_USED
<A>		dl_called_num	DL_CLED_NUM_654321
<B>		dl_called_num	DL_CLED_NUM_7654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	NOT_USED
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
<A>		calling_party	CLNG_PARTY_654321
<B>		calling_party	CLNG_PARTY_7654321
		calling_party_sub	NOT_USED
<A>		called_party	CLED_PARTY_654321
<B>		called_party	CLED_PARTY_7654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

History:	28-Apr-00	HM	Initial
	05-Jul-01	JHU	Converted to TAP2

### CC083: STATUS message with different call state received from network (U0)

Description:	CC receives a STATUS message from the network in the form of a MMCC-ESTABLISH indication primitive while in the state NULL (U0). The call state in the message received is not identical to the current call state of  CC, whereby CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MT_1
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_9
		aux_states	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_MSG_B_STATUS_2
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	05-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC084: CALL PROCEEDING message received from network (U0)

Description:	CC receives a CALL PROCEEDING message from the network in the form of a MMCC-ESTABLISH indication primitive while in the state NULL (U0). CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INVALID_TI
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	06-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC085: TI flag set: incoming SETUP message ignored (U0)

Description:	The network starts call establishment by sending a SETUP message with the transaction identifier flag set. CC ignores this message and issues a MMCC-RELEASE request primitive to MM. (Ref. [1] 5.2.2.2)

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MO_5_RESP
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	06-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC086: STATUS ENQUIRY message received from network (U0)

Description:	CC receives a STATUS ENQUIRY message from the network in the form of a MMCC-ESTABLISH indication primitive while in the state NULL (U0). CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (STATUS ENQUIRY)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS_ENQ
		ti	TI_MT_1
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INVALID_TI
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	06-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC087: SETUP message received from network (U0), CTM Support by MS

Description:	CC receives a SETUP message from the network in the form of a MMCC-ESTABLISH indication primitive. CC enters the state CALL PRESENT (U6) and starts compatibility checks. (Ref. [1] 5.2.2.1). Indication of the received call is given to MMI and the call is answered with indicating CTM support in the call confirmed message.

Variants:	<A>..<C>

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
		bcpara	BC_PARA_NO_SERVICE
		sns_mode	SNS_MODE_VOICE
		ctm_ena	CTM_ENABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
	<A>	bearer_cap	BC_SPEECH_FR
	<B>	bearer_cap	BC_AUX_SPEECH_FR
	<C>	bearer_cap	BC_SPEECH_AMR_EFR_HR_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
	<A>	bcpara	BC_PARA_SPEECH_CTM
	<B>	bcpara	BC_PARA_SPEECH_LINE_2_CTM
	<C>	bcpara	BC_PARA_SPEECH_CTM
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	FORBID
	<A>	bearer_cap	BC_SPEECH_FR_CTM
	<B>	bearer_cap	BC_AUX_SPEECH_FR_CTM
	<C>	bearer_cap	BC_SPEECH_FR_CTM
		bearer_cap_2	FORBID
		cc_cause	NOT_USED
		}

History:	06-Dec-2002	FK	Initial

### CC092: Test of incoming SETUP message successful (U6)

Description:	The compatibility checks have been evaluated successfully. CC sends a MNCC-SETUP indication primitive to MMI, builds a CALL CONFIRMED message and sends this in the form of a MNCC-DATA request primitive to MM, and enters the state MT CALL CONFIRMED (U9). (Ref. [1] 5.2.2.3.1)

Variants:	<A>..<F>

Preamble:	<A>CC081A
		<B>CC081B
		<C>CC081C
		<D>CC081D
		<E>CC081E
		<F>CC081F

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
<A>		ri	NOT_PRESENT_8BIT
<B>		ri	NOT_PRESENT_8BIT
<C>		ri	REPEAT_SEQUENTIAL
<D>		ri	REPEAT_SEQUENTIAL
<E>		ri	NOT_PRESENT_8BIT
<F>		ri	NOT_PRESENT_8BIT
<A>		bcpara	BC_PARA_BS21_T_300
<B>		bcpara	BC_PARA_SPEECH
<C>		bcpara	BC_PARA_BS21_T_300
<D>		bcpara	BC_PARA_SPEECH
<E>		bcpara	BC_PARA_SPEECH_LINE_2
<F>		bcpara	BC_PARA_SPEECH
<A>		bcpara2	NOT_USED
<B>		bcpara2	NOT_USED
<C>		bcpara2	BC_PARA_SPEECH
<D>		bcpara2	BC_PARA_BS21_T_300
<E>		bcpara2	NOT_USED
<F>		bcpara2	NOT_USED
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	variants <A>..<C>
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	04-Sep-00	LE	testcase for ALS added
	05-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant <F> for AMR added.
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC093: TI flag set: incoming SETUP message rejected (U6)

Description:	One of the compatibility checks has established that the transaction identifier (TI) flag is set. CC therefore ignores the SETUP message and issues a MMCC-RELEASE request primitive to MM. (Ref. [1] 5.2.2.2)

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1

History:	25-Apr-97	DL	Initial
	09-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC094: TI is a reserved value: incoming SETUP message rejected (U6)

Description:	One of the compatibility checks has established that the transaction identifier (TI) flag is a reserved value. CC ignores the SETUP message, builds a STATUS message and sends this in the form of a MMCC-DATA request primitive to MM. CC then sends a MMCC-RELEASE request primitive to MM. (Ref. [1] 5.2.2.2)

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_7
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_7_RESP

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	10-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

### CC097: All transactions in use: incoming SETUP message rejected (U0)

Description:	All transactions are currently in use. CC ignores the SETUP message, builds a RELEASE COMPLETE message and sends this in the form of a MMCC-DATA request primitive to MM. CC then sends a MMCC-RELEASE request primitive to MM. (Ref. [1] 5.2.2.2)

Note: A maximum number of 4 is only used in the PC/simulation environment. In the target, the maximum number of calls supported by CC is 7.

Preamble:	CC092A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |	 |		  *==============================>*
(4)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(5)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
(7)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(8)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(9)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
(10)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(11)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(12)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_2
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	NOT_USED
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_2_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	NOT_USED
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_2_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_3
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	NOT_USED
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_3_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	NOT_USED
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_3_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_4
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	NOT_USED
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_4_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	NOT_USED
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_4_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	NOT_USED
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_5_RESP
		cc_cause	CC_CAUSE_USER_BUSY
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_5_RESP

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	12-Jul-02	HM	CC_CAUSE_USER_BUSY if no slot left

### CC098: Compatibility check failed: incoming SETUP message rejected (U0)

Description:	The compatibility checks have failed. CC ignores the SETUP message, builds a RELEASE COMPLETE message and sends this in the form of a MMCC-DATA request primitive to MM. CC then sends a MMCC-RELEASE request primitive to MM. (Ref. [1] 5.2.2.2)

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INCOMPAT
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC100: Facility information element sent to network (U9)

Description:	CC receives a MNCC-FACILITY request primitive from MMI. CC builds a FACILITY message and sends this as part of a MNCC-DATA request primitive to MM.

Preamble:	CC092B

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_FACILITY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (FACILITY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_FACILITY_REQ
		ti	TI_MT_1_RESP
		fac_inf	FACILITY_A_FAC
		ss_version	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_FACILITY
		ti	TI_MT_1_RESP
		facility	FACILITY_A
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	10-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC101: Facility information element received from network (U9)

Description:	CC receives a FACILITY message from the network as part of a MMCC-DATA indication primitive. CC informs MMI by issuing a MNCC-FACILITY indication primitive.

Preamble:	CC092B

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (FACILITY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MT_1
		facility	FACILITY_A
		}

- MNCC_FACILITY_IND
		ti	TI_MT_1_RESP
		fac_context	FAC_IN_FACILITY
		fac_inf	FACILITY_A_FAC

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	10-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC102: STATUS message with different call state received from network (U9)

Description:	CC receives a STATUS message from the network in the form of a MMCC-DATA indication. The call state in the message received is not identical to the current call state of  CC, whereby CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC092B

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MT_1
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_4
		aux_states	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_MS_MESSAGE_INCOMPAT

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_MSG_B_STATUS_2
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	05-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC110: TCH assignment received from network during call establishment (U9)

Description:	While in the state MT CALL CONFIRMED (U9) CC receives a traffic channel (TCH) assignment in the form of a MMCC-SYNC indication primitive. CC sends a MNCC-SYNC indication primitive to MMI. (Ref. [1] 5.2.2.7)

Preamble:	CC092B

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_SYNC_IND
		ti	TI_MT_1_RESP
		chm	CH_SIG_ONLY

- MNCC_SYNC_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_CHANNEL_SYNC
		chm	CH_SIG_ONLY

History:	25-Apr-97	DL	Initial
	15-Jul-97	VK	MNCC_SYNC_IND revised
	27-Jul-98	LE	adapted to fax & data
	19-Jul-99	LE	CH_SIG_ONLY used
	06-Jul-01	JHU	Converted to TAP2

### CC112: Alerting in course of call establishment (U9)

Description:	While in the state MT CALL CONFIRMED (U9) CC receives a MNCC-ALERT request primitive from MMI to indicate that alerting has been switched on. CC builds an ALERTING message, sends this in the form of a MMCC-DATA request primitive to MM and enters the state CALL RECEIVED (U7).

Preamble:	CC092B

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_ALERT_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (ALERTING)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_ALERT_REQ
		ti	TI_MT_1_RESP

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_ALERT
		ti	TI_MT_1_RESP
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	15-Jul-97	VK	direction UPLINK (was DOWNLINK)
	15-Jul-97	VK	U_ALERT instaed of U_CONNECT
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC113: Confirmation from MMI (U9)

Description:	While in the state MT CALL CONFIRMED (U9) CC receives a MNCC-SETUP response primitive from MMI to indicate that the call has been accepted. CC builds a CONNECT message, sends this in the form of a MMCC-DATA request primitive to MM, starts the T313 timer and enters the state CONNECT REQUEST (U8). (Ref. [1] 5.2.2.5)

Variants:	<A>..<E>

Preamble:	<A>CC092A
		<B>CC092B
		<C>CC092C
		<D>CC092D
		<E>CC092F

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_SETUP_RES	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_RES
		ti	TI_MT_1_RESP

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CONNECT
		ti	TI_MT_1_RESP
		facility	NOT_USED
		connect_subaddr	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	15-Jul-97	VK	direction UPLINK (was DOWNLINK)
	31-Jul-97	VK	preamble is cc092a
	31-Jul-97	VK	variants <A>..<B>
	04-Jul-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant <E> for AMR added.

### CC115: TCH assignment from network during call establishment (U7)

Description:	While in the state CALL RECEIVED (U7) CC receives a traffic channel (TCH) assignment in the form of a MMCC-SYNC indication primitive. CC sends a MNCC-SYNC indication primitive to MMI. (Ref. [1] 5.5.2.2.7)

Preamble:	CC112

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_SYNC_IND
		ti	TI_MT_1_RESP
		chm	CH_SIG_ONLY

- MNCC_SYNC_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_CHANNEL_SYNC
		chm	CH_SIG_ONLY

History:	25-Apr-97	DL	Initial
	15-Jul-97	VK	MNCC_SYNC_IND params revised
	27-Jul-98	LE	adapted to fax & data
	19-Jul-99	LE	CH_SIG_ONLY inserted
	06-Jul-01	JHU	Converted to TAP2

### CC116: Facility information element sent to network (U7)

Description:	CC receives a MNCC-FACILITY request primitive from MMI. CC builds a FACILITY message and sends this as part of a MNCC-DATA request primitive to MM.

Preamble:	CC112

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_FACILITY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (FACILITY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_FACILITY_REQ
		ti	TI_MT_1_RESP
		fac_inf	FACILITY_A_FAC
		ss_version	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_FACILITY
		ti	TI_MT_1_RESP
		facility	FACILITY_A
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	15-Jul-97	VK	params revised
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC117: Facility information element received from network (U7)

Description:	CC receives a FACILITY message from the network as part of a MMCC-DATA indication primitive. CC informs MMI by issuing a MNCC-FACILITY indication primitive.

Preamble:	CC112

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (FACILITY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MT_1
		facility	FACILITY_A
		}

- MNCC_FACILITY_IND
		ti	TI_MT_1_RESP
		fac_context	FAC_IN_FACILITY
		fac_inf	FACILITY_A_FAC

History:	25-Apr-97	DL	Initial
	15-Jul-97	VK	params revised
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC118: STATUS message with different call state received from network (U7)

Description:	CC receives a STATUS message from the network in the form of a MMCC-DATA indication primitive while in the state U7. The call state in the message received is not identical to the current call state of  CC, whereby CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC112

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MT_1
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_9
		aux_states	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_MS_MESSAGE_INCOMPAT

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_MSG_B_STATUS_2
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	15-Jul-97	VK	revised (ref CC102)
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC119: Confirmation from MMI (U7)

Description:	While in the state CALL RECEIVED (U7) CC receives a MNCC-SETUP response primitive from MMI to indicate that the call has been accepted. CC builds a CONNECT message, sends this in the form of a MMCC-DATA request primitive to MM, starts the T313 timer and enters the state CONNECT REQUEST (U8). (Ref. [1] 5.2.2.5)

Preamble:	CC112

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_SETUP_RES	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_RES
		ti	TI_MT_1_RESP

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CONNECT
		ti	TI_MT_1_RESP
		facility	NOT_USED
		connect_subaddr	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	15-Jul-97	VK	direction is UPLINK (was DOWNLINK)
	15-Jul-97	VK	connect_subaddr is new
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC123: Facility information element sent to network (U8)

Description:	CC receives a MNCC-FACILITY request primitive from MMI. CC builds a FACILITY message and sends this as part of a MNCC-DATA request primitive to MM.

Preamble:	CC113A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_FACILITY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (FACILITY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_FACILITY_REQ
		ti	TI_MT_1_RESP
		fac_inf	FACILITY_A_FAC
		ss_version	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_FACILITY
		ti	TI_MT_1_RESP
		facility	FACILITY_A
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC124: Facility information element received from network (U8)

Description:	CC receives a FACILITY message from the network as part of a MMCC-DATA indication primitive. CC informs MMI by issuing a MNCC-FACILITY indication primitive.

Preamble:	CC113A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (FACILITY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MT_1
		facility	FACILITY_A
		}

- MNCC_FACILITY_IND
		ti	TI_MT_1_RESP
		fac_context	FAC_IN_FACILITY
		fac_inf	FACILITY_A_FAC

History:	25-Apr-97	DL	Initial
	15-Jul-97	VK	revised
	31-Jul-97	VK	preamble is cc113a
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC125: STATUS message with different call state received from network (U8)

Description:	CC receives a STATUS message from the network in the form of a MMCC-DATA indication primitive while in the state U8. The call state in the message received is not identical to the current call state of  CC, whereby CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC113A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MT_1
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_9
		aux_states	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_MS_MESSAGE_INCOMPAT

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_MSG_B_STATUS_2
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc113a
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC126: No response from the network (U8)

Description:	While in the state CONNECT REQUEST (U8) CC receives no response from the network before the T313 timer expires. CC builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive to MM. CC then sends a MNCC-SETUP-COMPL indication primitive to MMI and enters the state DISCONNECT REQUEST (U11). (Ref. [1] 5.2.2.6)

Preamble:	CC113A

MMI		 CC		 MM
	 |		  |		  |
MUTE (25000)
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
(2)	 |	MNCC_SETUP_COMPL_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_TIMER_313
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MNCC_SETUP_COMPL_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_MS_TIMER

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2

### CC130: CONNECT ACKNOWLEDGE message received (U8)

Description:	While in the state CONNECT REQUEST (U8) CC receives a CONNECT ACKNOWLEDGE message as part of a MMCC-DATA indication primitive to confirm that a connection has been established. CC then stops the T313 timer, enters the state ACTIVE (U10) and issues a MNCC-SETUP-COMPL indication primitive to MMI. (Ref. [1] 5.2.2.6)

Variants:	<A>..<E>

Preamble:	<A>CC113A
		<B>CC113B
		<C>CC113C
		<D>CC113D
		<E>CC113E

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CONNECT ACKNOWLEDGE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_COMPL_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_CONNECT_ACK
		ti	TI_MT_1
		}

- MNCC_SETUP_COMPL_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_SUCCESS

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc113a
	31-Jul-97	VK	variants <A> .. <C>
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	06-Jul-01	JHU	Converted to TAP2
	26-Nov-01	OT	Variant <E> for AMR added.

## Call Active (state = U10)

### CC221: Notification received from network

Description:	CC receives a NOTIFY message from the network as part of a MMCC-DATA indication primitive. CC informs MMI by issuing a MNCC-NOTIFY indication primitive. (Ref. [1] 5.3.1)

Preamble:	CC053

Variants:	<A>....<C>

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (NOTIFY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_NOTIFY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_NOTIFY
		ti	TI_MO_5_RESP
<A>		notific	NOTIFIC_USER_SUSPEND
<B>		notific	NOTIFIC_USED_RESUME
<C>		notific	NOTIFIC_BEARER_CHANGE
		}

- MNCC_NOTIFY_IND
		ti	TI_MO_5
<A>		nd	ND_USER_SUSPEND
<B>		nd	ND_USED_RESUME
<C>		nd	ND_BEARER_CHANGE

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	use TI_MO_5 instead of TI_MT_9
	16-Jul-97	VK	introduce NOTIFIC_* names
	16-Jul-97	VK	revise MNCC_NOTIFY_IND params
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC222: Notification sent to  network

Description:	CC receives a MNCC-NOTIFY request primitive from MMI. CC builds a NOTIFY message and sends this as part of a MNCC-DATA request primitive to MM. (Ref. [1] 5.3.1)

Preamble:	CC053

Variants:	<A>....<C>

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_NOTIFY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (NOTIFY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_NOTIFY_REQ
		ti	TI_MO_5
<A>		nd	ND_USER_SUSPEND
<B>		nd	ND_USED_RESUME
<C>		nd	ND_BEARER_CHANGE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_NOTIFY
		ti	TI_MO_5
<A>		notific	NOTIFIC_USER_SUSPEND
<B>		notific	NOTIFIC_USED_RESUME
<C>		notific	NOTIFIC_BEARER_CHANGE
		}

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	use TI_MO_5 instead of TI_MT_9
	16-Jul-97	VK	revise prarms
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC223: Facility information element received from network

Description:	CC receives a FACILITY message from the network as part of a MMCC-DATA indication primitive. CC informs MMI by issuing a MNCC-FACILITY indication primitive.

Preamble:	CC053

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (FACILITY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MO_5_RESP
		facility	FACILITY_A
		}

- MNCC_FACILITY_IND
		ti	TI_MO_5
		fac_context	FAC_IN_FACILITY
		fac_inf	FACILITY_A_FAC

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC224: Facility information element sent to  network

Description:	CC receives a MNCC-FACILITY request primitive from MMI. CC builds a FACILITY message and sends this as part of a MNCC-DATA request primitive to MM.

Preamble:	CC053

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_FACILITY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (FACILITY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_FACILITY_REQ
		ti	TI_MO_5
		fac_inf	FACILITY_A_FAC
		ss_version	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_FACILITY
		ti	TI_MO_5
		facility	FACILITY_A
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC225: Start Dual Tone Multi Frequency (DTMF)

Description:	CC receives a MNCC-START-DTMF request primitive from MMI, requesting that DTMF tones be sent. CC builds a START DTMF message and sends this in the form of a MMCC-DATA request primitive to MM. On receipt of a START DTMF ACK message in the form of a MMCC-DATA indication primitive from MM, CC issues a MNCC-START-DTMF indication primitive. (Ref. [1] 5.5.7.1-2)

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_START_DTMF_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (START DTMF)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (START DTMF ACK)	  |
	 |		  *<==============================*
(4)	 |	MNCC_START_DTMF_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_AUTO

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_START_DTMF
		ti	TI_MO_5
		key_facility	KEY_FACILITY_9
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_START_DTMF_ACK
		ti	TI_MO_5_RESP
		key_facility	KEY_FACILITY_9
		}

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_DTMF_START_SUCCESS
		dtmf_mod	DTMF_MOD_AUTO

History:	25-Apr-97	DL	Initial
	30-Jul-97	VK	revised
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	25-Jan-00	HM	added dtmf_mod
	09-Jul-01	JHU	Converted to TAP2

### CC226: Stop Dual Tone Multi Frequency (DTMF)

Description:	Following a break in the sending of keystroke sequences from MMI, CC builds a STOP DTMF message and sends this in the form of a MMCC-DATA request primitive to MM. The network then stops sending the DTMF tone and issues a with a STOP DTMF ACK message (MMCC-DATA indication primitive). (Ref. [1] 5.5.7.1-2)

Preamble:	CC225

MMI		 CC		 MM
	 |		  |		  |
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STOP DTMF)	  |
	 |		  *==============================>*
(2)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STOP DTMF ACK)	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_STOP_DTMF
		ti	TI_MO_5
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_STOP_DTMF_ACK
		ti	TI_MO_5_RESP
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC228: Connection interrupted - reestablishment successful

Description:	CC receives a MNCC-ERROR indication primitive from MM indicating that the connection has been interrupted but that reestablishment is permitted. CC issues a MNCC-SYNC indication primitive to MMI and a MMCC-REESTABLISH request primitive to MM. CC receives  a MMCC-REESTABLISH confirmation primitive, indication that reestablishment has been successful, whereupon it issues a MNCC-SYNC indication primitive and enters the state Active  Pending (U10.1). (Ref. [1] 5.5.4)

Preamble:	CC053

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ERROR_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_REESTABLISH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_REESTABLISH_CNF	  |
	 |		  *<==============================*
(5)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ERROR_IND
		ti	TI_MO_5
		cause	RRCS_ABORT_RAD_LNK_FAIL

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_REEST_STARTED
		chm	NOT_USED

- MMCC_REESTABLISH_REQ
		ti	TI_MO_5

- MMCC_REESTABLISH_CNF
		ti	TI_MO_5

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_REEST_FINISHED
		chm	NOT_USED

History:	25-Apr-97	DL	Initial
	30-Jul-97	VK	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC229: Connection interrupted - reestablishment unsuccessful

Description:	CC receives a MNCC-ERROR indication primitive from MM indicating that the connection has been interrupted but that reestablishment is permitted. CC issues a MNCC-SYNC indication primitive to MMI and a MMCC-REESTABLISH request primitive to MM. CC receives a MMCC-RELEASE indication primitive, indicating that reestablishment has not been successful, whereupon it issues a MNCC-REJECT indication primitive to MMI and enters the state Null (U0). (Ref. [1] 5.5.4)

Preamble:	CC053

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ERROR_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_REESTABLISH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |	MNCC_REJECT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ERROR_IND
		ti	TI_MO_5
		cause	RRCS_ABORT_RAD_LNK_FAIL

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_REEST_STARTED
		chm	NOT_USED

- MMCC_REESTABLISH_REQ
		ti	TI_MO_5

- MMCC_RELEASE_IND
		ti	TI_MO_5
		cause	RRCS_DL_EST_FAIL

- MNCC_REJECT_IND
		ti	TI_MO_5
		cause	RRCS_DL_EST_FAIL

History:	25-Apr-97	DL	Initial
	30-Jul-97	VK	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC230: STATUS ENQUIRY message received from network

Description:	CC receives a STATUS ENQUIRY message from the network as part of a MMCC-DATA indication primitive. It responds by building a STATUS message and sending this as part of a MMCC-DATA request primitive. (Ref. [1] 5.5.3.1)

Preamble:	CC053

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS ENQUIRY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_STATUS_IND	  |		  |
	 *<==============================*		  |
MUTE (500)
(3)	 |	MNCC_STATUS_RES	  |		  |
	 *==============================>*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STATUS)	  |
	 |		  *==============================>*
MUTE (500)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS_ENQ
		ti	TI_MO_5_RESP
		}

- MNCC_STATUS_IND
		ti	TI_MO_5

- MNCC_STATUS_RES
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_STATUS
		ti	TI_MO_5
		cc_cause	CC_CAUSE_STATUS_ENQ
		call_state	CALL_STATE_CS_10
		aux_states	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	30-Jul-97	VK	cc_cause revised
	30-Jul-97	VK	state is CS_10 with CC053
	30-Jul-97	VK	aux_states   NOT_USED
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2
	28-Nov-02	HM	Revised

### CC231: STATUS message received from network: state compatible

Description:	CC receives a STATUS message from the network as part of a MMCC-DATA indication primitive. The state is compatible, whereupon CC responds by issuing a MNCC-DISCONNECT indication primitive to MMI. It then builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive to MM. (Ref. [1] 5.5.3.2)

Preamble:	CC053

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_MSG_B_STATUS
		call_state	CALL_STATE_CS_10
		aux_states	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MESSAGE_TYPE_INCOMPAT
		diagnostic	NOT_USED
		progress_desc	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	30-Jul-97	VK	state is CS_10 with CC053
	30-Jul-97	VK	use CC_CAUSE_MSG_INCOMPAT
	30-Jul-97	VK	MNCC_DISCONNECT_IND revised
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC232: STATUS message received from network: state incompatible

Description:	CC receives a STATUS message from the network as part of a MMCC-DATA indication primitive. The state is incompatible, whereupon CC responds by issuing a MNCC-RELEASE indication primitive to MMI. It then builds a RELEASE COMPLETE message and sends this as part of a MMCC-DATA request primitive to MM. (Ref. [1] 5.5.3.2)

Preamble:	CC053

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_0
		aux_states	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_MESSAGE_INCOMPAT

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	CC_CAUSE_MSG_B_STATUS_2
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	30-Jul-97	VK	U_RELEASE_COM renamed to ...
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC233: STATUS ENQUIRY message received from network, non-active

Description:	CC receives a STATUS ENQUIRY message from the network as part of a MMCC-DATA indication primitive. It responds by building a STATUS message and sending this as part of a MMCC-DATA request primitive. (Ref. [1] 5.5.3.1)

Preamble:	CC033

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS ENQUIRY)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STATUS)	  |
	 |		  *==============================>*
MUTE (500)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS_ENQ
		ti	TI_MO_5_RESP
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_STATUS
		ti	TI_MO_5
		cc_cause	CC_CAUSE_STATUS_ENQ
		call_state	CALL_STATE_CS_3
		aux_states	NOT_USED
		}

History:		28-Nov-02	HM	Initial

### CC243: HOLD requested by mobile station

Description:	MMI requests the current call to be held by issuing a MNCC-HOLD request primitive. CC builds a HOLD message and forwards this to MM in the form of a MMCC-DATA request primitive.

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_HOLD_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (HOLD)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_HOLD_REQ
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_HOLD
		ti	TI_MO_5
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC244: Network acknowledges HOLD request by mobile station

Description:	CC receives a HOLD ACKNOWLEDGE message in the form of a MMCC-DATA indication primitive, indicating that the connection is being held as requested, whereby CC then informs MMI by issuing a MNCC-HOLD confirmation primitive.

Preamble:	CC243

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (HOLD ACKNOWLEDGE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_HOLD_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_HOLD_ACK
		ti	TI_MO_5_RESP
		}

- MNCC_HOLD_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_HOLD_SUCCESS

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC245: Network acknowledges HOLD request with FACILITY message

Description:	CC receives a FACILITY message in the form of a MMCC-DATA indication primitive, indicating that the connection is being held as requested, whereby CC then informs MMI by issuing a MNCC-FACILILTY confirmation primitive.

Preamble:	CC243

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (FACILITY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MO_5_RESP
		facility	FACILITY_A
		}

- MNCC_FACILITY_IND
		ti	TI_MO_5
		fac_context	FAC_IN_FACILITY
		fac_inf	FACILITY_A_FAC

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC246: Network rejects HOLD request by mobile station

Description:	CC receives a HOLD ACKNOWLEDGE message in the form of a MMCC-DATA indication primitive, indicating that the connection cannot be held, whereby CC then informs MMI by issuing a MNCC-HOLD confirmation primitive.

Preamble:	CC243

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (HOLD REJECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_HOLD_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_HOLD_REJ
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_BEARER_CAP_AUTH
		}

- MNCC_HOLD_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_BEARER_CAP_AUTHORIZ

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC247: RETRIEVE requested by mobile station

Description:	MMI requests the currently call to be retrieved by issuing a MNCC-RETRIEVE request primitive. CC builds a RETRIEVE message and forwards this to MM in the form of a MMCC-DATA request primitive.

Preamble:	CC244

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_RETRIEVE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RETRIEVE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_RETRIEVE_REQ
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RETRIEVE
		ti	TI_MO_5
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC248: Network acknowledges RETRIEVE request by mobile station

Description:	CC receives a RETRIEVE ACKNOWLEDGE message in the form of a MMCC-DATA indication primitive, indicating that the held connection is being retrieved as requested, whereby CC then informs MMI by issuing a MNCC-RETRIEVE confirmation primitive.

Preamble:	CC247

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RETRIEVE ACKNOWLEDGE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RETRIEVE_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RETRIEVE_ACK
		ti	TI_MO_5_RESP
		}

- MNCC_RETRIEVE_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_RETRIEVE_SUCCESS

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC249: Network acknowledges RETRIEVE request with FACILITY message

Description:	CC receives a FACILITY message in the form of a MMCC-DATA indication primitive, indicating that the held connection is being retrieved as requested, whereby CC then informs MMI by issuing a MNCC-FACILILTY confirmation primitive.

Preamble:	CC247

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (FACILITY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MO_5_RESP
		facility	FACILITY_A
		}

- MNCC_FACILITY_IND
		ti	TI_MO_5
		fac_context	FAC_IN_FACILITY
		fac_inf	FACILITY_A_FAC

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11-Nov-97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC250: Network rejects RETRIEVE request by mobile station

Description:	CC receives a RETRIEVE REJECT message in the form of a MMCC-DATA indication primitive, indicating that the held connection cannot be retrieved, whereby CC then informs MMI by issuing a MNCC-RETRIEVE confirmation primitive.

Preamble:	CC247

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RETRIEVE REJECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RETRIEVE_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RETRIEVE_REJ
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_UNSPECIFIED
		}

- MNCC_RETRIEVE_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_UNSPECIFIED

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC251: HOLD requested by mobile station followed by Call Establishment

Description:	MMI requests the current call to be held by issuing a MNCC-HOLD request primitive. CC builds a HOLD message and forwards this to MM in the form of a MMCC-DATA request primitive. Then the MMI requests call establishment for a different transaction identifier.

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_HOLD_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (HOLD)	  |
	 |		  *==============================>*
(3)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
(7)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (HOLD ACKNOWLEDGE)	  |
	 |		  *<==============================*
(8)	 |	MNCC_HOLD_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_HOLD_REQ
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_HOLD
		ti	TI_MO_5
		}

- MNCC_SETUP_REQ
		ti	TI_MO_6
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_6
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
		ti	TI_MO_6

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_6
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_HOLD_ACK
		ti	TI_MO_5_RESP
		}

- MNCC_HOLD_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_HOLD_SUCCESS

History:	02-Feb-99	LE	Initial
	09-Jul-01	JHU	Converted to TAP2

### CC261: In-call modification started by MMI (Mobile Originated)

Description:	MMI initiates in-call modification by issuing a MNCC-g request primitive. CC checks that no hold, retrieve or DTMF procedure is currently active before building a MODIFY message and forwarding this to MM in the form of a MMCC-DATA request primitive. The T323 timer is started and CC enters the state MO MODIFY (U26). (Ref. [1] 5.3.4.3.1)

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_MODIFY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (MODIFY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_MODIFY_REQ
		ti	TI_MO_5
		serv	SERV_DATA

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_MODIFY
		ti	TI_MO_5
		bearer_cap	NOT_USED
		low_layer_comp	NOT_USED
		high_layer_comp	NOT_USED
		reverse_call	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	1st prim was MNCC_RETRIEVE_REQ
	31-Jul-97	VK	1st prim is MNCC_MODIFY_REQ
	31-Jul-97	VK	reverse_call...NOT_USED
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC262: In-call modification successful (Mobile Originated)

Description:	CC receives a MODIFY COMPLETE message as part of a MMCC-DATA indication primitive, indicating that in-call modification has been successful. CC issues a MNCC-MODIFY confirmation primitive, stops the T323 timer reenters the state ACTIVE (U10). (Ref. [1] 5.3.4.3.2)

Preamble:	CC261

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (MODIFY COMPLETE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_MODIFY_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_MODIFY_COMP
		ti	TI_MO_5_RESP
		bearer_cap	BC_SPEECH_FR
		low_layer_comp	NOT_USED
		high_layer_comp	NOT_USED
		reverse_call	NOT_USED
		}

- MNCC_MODIFY_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_MODIFY_SUCCESS

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC263: In-call modification unsuccessful (Mobile Originated)

Description:	CC receives a MODIFY REJECT message as part of a MMCC-DATA indication primitive, indicating that in-call modification has not succeeded. CC issues a MNCC-MODIFY confirmation primitive, stops the T323 timer reenters the state ACTIVE (U10). (Ref. [1] 5.3.4.3.4)

Preamble:	CC261

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (MODIFY REJECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_MODIFY_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_MODIFY_REJ
		ti	TI_MO_5_RESP
		bearer_cap	NOT_USED
		cc_cause	CC_CAUSE_UNSPECIFIED
		low_layer_comp	NOT_USED
		high_layer_comp	NOT_USED
		}

- MNCC_MODIFY_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_UNSPECIFIED

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC264: No response to in-call modification request (Mobile Originated)

Description:	The T323 timer expires without the network having responded. CC releases the call, sends a DISCONNECT message to the network as part of a MMCC-DATA request primitive and issues a MNCC-MODIFY confirmation primitive. (Ref. [1] 5.3.4.3.4)

Preamble:	CC261

MMI		 CC		 MM
	 |		  |		  |
MUTE (25000)
(1)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
(2)	 |	MNCC_MODIFY_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_TIMER_323
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MNCC_MODIFY_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_TIMER

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	‘TIMEOUT (5)’   removed
	31-Jul-97	VK	use CC_CAUSE_TIMER_323
	04-Aug-97	VK	d1,d2 added
	04-Aug-97	VK	use MNCC_MODIFY_IND
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC267: Successful reestablishment during modification (Mobile Originated)

Description:	CC receives after initiation of the mobile originated in-call modification an MNCC-ERROR indication primitive from MM indicating that the connection has been interrupted but that reestablishment is permitted. CC issues an MNCC-SYNC indication primitive to MMI and an MMCC-REESTABLISH request primitive to MM. CC receives  an MMCC-REESTABLISH confirmation primitive, indication that reestablishment has been successful, whereupon it issues an MNCC-SYNC indication primitive and (re-)enters the state Mobile Originating Modify (U26). The status enquiry procedure is used to verify this.

Preamble:	CC261

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ERROR_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_REESTABLISH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_REESTABLISH_CNF	  |
	 |		  *<==============================*
(5)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STATUS ENQUIRY)	  |
	 |		  *<==============================*
(7)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STATUS)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ERROR_IND
		ti	TI_MO_5
		cause	RRCS_ABORT_RAD_LNK_FAIL

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_REEST_STARTED
		chm	NOT_USED

- MMCC_REESTABLISH_REQ
		ti	TI_MO_5

- MMCC_REESTABLISH_CNF
		ti	TI_MO_5

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_REEST_FINISHED
		chm	NOT_USED

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS_ENQ
		ti	TI_MO_5_RESP
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_STATUS
		ti	TI_MO_5
		cc_cause	CC_CAUSE_STATUS_ENQ
		call_state	CALL_STATE_CS_26
		aux_states	NOT_USED
		}

History:	27-Sep-02	SBK	Initial (issue 4992)

### CC265: In-call modification accepted by MS (Mobile Terminated)

Description:	CC receives a MODIFY message as part of a MMCC-DATA indication primitive, indicating that the network wishes to start in-call modification. The mobile is able to accept the modification, whereby CC issues a MNCC-MODIFY indication primitive to MMI and sends a MODIFY_COMPLETE message as part of a MNCC-DATA indication primitive to MM. (Ref. [1] 5.3.4.3.2)

Preamble:	CC130C

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (MODIFY)	  |
	 |		  *<==============================*
(2)	 |	MNCC_MODIFY_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (MODIFY COMPLETE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_MODIFY
		ti	TI_MT_1
		bearer_cap	BC_SPEECH_FR
		low_layer_comp	NOT_USED
		high_layer_comp	NOT_USED
		reverse_call	NOT_USED
		}

- MNCC_MODIFY_IND
		ti	TI_MT_1_RESP
		serv	SERV_SPEECH

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_MODIFY_COMP
		ti	TI_MT_1_RESP
		bearer_cap	BC_SPEECH_FR_HR_EFR
		low_layer_comp	NOT_USED
		high_layer_comp	NOT_USED
		reverse_call	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	11.10.97	LE	revised
	28-Jul-98	LE	adapted to fax & data
	16-Mar-00	HM	MODIFY_COMP with full capabilities
	09-Jul-01	JHU	Converted to TAP2

### CC266: In-call modification rejected by MS (Mobile Terminated)

Description:	CC receives a MODIFY message as part of a MMCC-DATA indication primitive, indicating that the network wishes to start in-call modification. The mobile is unable to accept the modification: CC sends a MODIFY_COMPLETE message as part of a MNCC-DATA indication primitive to MM. (Ref. [1] 5.3.4.3.4)

Preamble:	CC130A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (MODIFY)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (MODIFY REJECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_MODIFY
		ti	TI_MT_1
		bearer_cap	BC_SPEECH_FR
		low_layer_comp	NOT_USED
		high_layer_comp	NOT_USED
		reverse_call	RCS_IE_PRESENT
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_MODIFY_REJ
		ti	TI_MT_1_RESP
		bearer_cap	NOT_USED
		cc_cause	CC_CAUSE_BEARER_CAP_AUTH
		low_layer_comp	NOT_USED
		high_layer_comp	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	new : bearer_cap in MODIFY_REJ
	31-Jul-97	VK	preamble is cc130a
	01-Aug-97	VK	CAUSE_BEARER_CAP_AUTHORIZ
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2
	24-Aug-01	SBK	reverse_call: VALID_FALSE renamed

### CC280: In-call modification started by MMI (Mobile Terminated)

Description:	MMI initiates in-call modification by issuing a MNCC-g request primitive. CC checks that no hold, retrieve or DTMF procedure is currently active before building a MODIFY message and forwarding this to MM in the form of a MMCC-DATA request primitive. The T323 timer is started and CC enters the state MO MODIFY (U26). (Ref. [1] 5.3.4.3.1)

Preamble:	CC130D

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_MODIFY_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (MODIFY)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_MODIFY_REQ
		ti	TI_MT_1_RESP
		serv	SERV_DATA

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_MODIFY
		ti	TI_MT_1_RESP
		bearer_cap	BC_BS21_T_300
		low_layer_comp	NOT_USED
		high_layer_comp	NOT_USED
		reverse_call	NOT_USED
		}

History:	29-Jan-99	LE	Initial
	09-Jul-01	JHU	Converted to TAP2

### CC281: In-call modification successful (Mobile Terminated)

Description:	CC receives a MODIFY COMPLETE message as part of a MMCC-DATA indication primitive, indicating that in-call modification has been successful. CC issues a MNCC-MODIFY confirmation primitive, stops the T323 timer reenters the state ACTIVE (U10). (Ref. [1] 5.3.4.3.2)

Preamble:	CC280

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (MODIFY COMPLETE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_MODIFY_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_MODIFY_COMP
		ti	TI_MT_1
		bearer_cap	BC_BS21_T_300
		low_layer_comp	NOT_USED
		high_layer_comp	NOT_USED
		reverse_call	RCS_IE_PRESENT
		}

- MNCC_MODIFY_CNF
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_MODIFY_SUCCESS

History:	29-Jan-99	LE	Initial
	09-Jul-01	JHU	Converted to TAP2
	24-Aug-01	SBK	Renamed TRUE of RCS IE, tdscheck

### CC271: Send user data - USER USER message (Mobile Originated)

Description:	MMI sends user-to-user data to CC in the form of a MNCC-USER request primitive. Congestion level is set to READY, therefore, MM builds a USER USER message and forwards this to MM as part of a MNCC-DATA request primitive.

Preamble:	CC053

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_USER_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (USER USER)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_USER_REQ
		ti	TI_MO_5
		congest_lev	NOT_PRESENT_8BIT
		more_data	MD_NO_MORE_DATA
		user	USER_A_USER

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_USER_INFO
		ti	TI_MO_5
		user_user	USER_A
		more_data	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	B_USERINFO   ->  B_USER_INFO
	31-Jul-97	VK	congestion   ->   congest_lev
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC272: Send user data - CONGESTION message (Mobile Originated)

Description:	MMI sends user-to-user data to CC in the form of a MNCC-USER request primitive. Congestion level is set to NOT READY, therefore, MM builds a CONGESTION message and forwards this to MM as part of a MNCC-DATA request primitive.

Preamble:	CC053

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_USER_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CONGESTION)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_USER_REQ
		ti	TI_MO_5
		congest_lev	CL_RECEIVER_NOT_READY
		more_data	MD_NO_MORE_DATA
		user	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_CONGEST_CTRL
		ti	TI_MO_5
		congest_lev	CL_RECEIVER_NOT_READY
		cc_cause	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	use B_CONGEST_CTRL
	31-Jul-97	VK	congestion   ->   congest_lev
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC273: Receive user data - USER USER message (Mobile Terminated)

Description:	CC receives a USER USER message as part of a MMCC-DATA indication primitive. CC forwards the information to MMI as part of a MNCC-USER indication primitive.

Preamble:	CC130A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (USER USER)	  |
	 |		  *<==============================*
(2)	 |	MNCC_USER_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_USER_INFO
		ti	TI_MT_1
		user_user	USER_A
		more_data	NOT_USED
		}

- MNCC_USER_IND
		ti	TI_MT_1_RESP
		congest_lev	NOT_PRESENT_8BIT
		more_data	MD_NO_MORE_DATA
		user	USER_A_USER

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	B_USERINFO   ->  B_USER_INFO
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC274: Receive user data - CONGESTION message (Mobile Terminated Call)

Description:	CC receives a USER USER message as part of a MMCC-DATA indication primitive. CC forwards the information to MMI as part of a MNCC-USER indication primitive.

Preamble:	CC130A

Variants:	<A>...<B>

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CONGESTION)	  |
	 |		  *<==============================*
(2)	 |	MNCC_USER_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_CONGEST_CTRL
		ti	TI_MT_1
		congest_lev	CL_RECEIVER_NOT_READY
<A>		cc_cause	CC_CAUSE_BEARER_CAP_AUTH
<B>		cc_cause	NOT_USED
		}

- MNCC_USER_IND
		ti	TI_MT_1_RESP
		congest_lev	CL_RECEIVER_NOT_READY
		more_data	NOT_PRESENT_8BIT
		user	NOT_USED

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	use B_CONGEST_CTRL
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

## Call Clearing

### CC301: STATUS message with different call state received from network (U0)

Description:	CC receives a STATUS message from the network in the form of a MMCC-ESTABLISH indication primitive while in the state NULL (U0). The call state in the message received is not identical to the current call state of  CC, whereby CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MT_1
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_9
		aux_states	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_MSG_B_STATUS_2
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	use CC_CAUSE_UNSPECIFIED
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC302: STATUS message with same call state received from network (U0)

Description:	CC receives a STATUS message from the network in the form of a MMCC-ESTABLISH indication primitive while in the state NULL (U0). The call state in the message received is identical to the current call state of  CC, whereby CC issues a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (STATUS)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS
		ti	TI_MT_1
		cc_cause	CC_CAUSE_UNSPECIFIED
		call_state	CALL_STATE_CS_0
		aux_states	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	use CC_CAUSE_UNSPECIFIED
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC303: STATUS ENQUIRY message received from network (U0)

Description:	CC receives a STATUS ENQUIRY message from the network in the form of a MMCC-ESTABLISH indication primitive while in the state NULL (U0), whereby CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (STATUS ENQUIRY)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	B_STATUS_ENQ
		ti	TI_MT_1
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INVALID_TI
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	use CC_CAUSE_INVALID_TI
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC304: RELEASE COMPLETE message received from network (U0)

Description:	CC receives a RELEASE COMPLETE message from the network in the form of a MMCC-ESTABLISH indication primitive while in the state NULL (U0), whereby CC issues a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE_COMP
		ti	TI_MT_1
		cc_cause	CC_CAUSE_CHAN_UNACCEPT
		facility	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	..._INACCEPT renamed into ..._UNACCEPT
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC305: CALL PROCEEDING message received from network (U0)

Description:	CC receives a CALL PROCEEDING message from the network in the form of a MMCC-ESTABLISH indication primitive while in the state NULL (U0), whereby CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INVALID_TI
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	repeat   NOT_PRESET
	16-Jul-97	VK	use CC_CAUSE_INVALID_TI
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC306: PROGRESS message received (U0)

Description:	While in the state NULL (U0) CC receives a PROGRESS message in the form of a MNCC-DATA request primitive. CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (PROGRESS)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_PROGRESS
		ti	TI_MO_5_RESP
		progress	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	CC_CAUSE_INVALID_TI
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	use CC_CAUSE_INVALID_TI
	16-Jul-97	VK	introduce progress
	16-Jul-97	VK	use TI_MO_5
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC307: Facility information element received from network (U0)

Description:	While in the state NULL (U0) CC receives a FACILITY message in the form of a MNCC-DATA request primitive. CC immediately issues a RELEASE COMPLETE message as part of a MMCC-DATA request primitive followed by a MMCC-RELEASE request primitive.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (PROGRESS)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_FACILITY
		ti	TI_MO_5_RESP
		facility	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	CC_CAUSE_INVALID_TI
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	D_FACILITY and facility introduced
	16-Jul-97	VK	use TI_MO_5
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC308: SETUP message received with the TI incorrectly set (U0)

Description:	CC receives a SETUP message from the network in the form of a MMCC-ESTABLISH indication primitive. The TI flag is set, or the value of TI is a reserved value, whereupon CC issues a MMCC-RELEASE request primitive.

Preamble:	CC000

Variants:	<A>....<B>

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
<A>		ti	TI_MO_5_RESP
<B>		ti	TI_MO_7_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
<A>		ti	TI_MO_5
<B>		ti	TI_MO_7

History:	25-Apr-97	DL	Initial
	11-Jul-97	VK	use CLNG_NUM_654321
	16-Jul-97	VK	repeat   NOT_USED
	16-Jul-97	VK	rename TI_M0_5 to TI_MO_5
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC312: MM connection rejected (U0.1)

Description:	MM signals CC in the form of a MMCC-RELEASE indication primitive that a connection cannot be established. CC stops the T303 timer, issues a MNCC-REJ indication primitive and enters the state NULL (U0).

Preamble:	CC011

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_REJECT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_RELEASE_IND
		ti	TI_MO_5
		cause	RRCS_DL_EST_FAIL

- MNCC_REJECT_IND
		ti	TI_MO_5
		cause	RRCS_DL_EST_FAIL

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	use ... instead of REJ_NO_TI
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC321: No response from BS - timer expiry (U0.1)

Description:	The T303 timer expires before a MM connection can be established. CC issues a MNCC-REJ indication primitive, enters the state NULL (U0) and requests MM to abort the establishment in the form of a MMCC-RELEASE request primitive.

Preamble:	CC011

MMI		 CC		 MM
MUTE (25000)
	 |		  |		  |
(1)	 |	MNCC_REJECT_IND	  |		  |
	 *<==============================*		  |
(2)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_REJECT_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_TIMER

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	use REJ_T303 instead of REJ_NO_TI
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC322: Mobile originated call release requested (U1)

Description:	While in the state CAL INIT (U1) CC receives a MNCC-DISCONNECT request primitive. CC builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive. CC then starts the T305 timer and enters the state DISCONNECT REQUEST (U11). (Ref. [1] 5.4.3.1)

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_DISCONNECT_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	FACILITY_NONE
		ss_version	SS_VER_NOT_PRES

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	remove cs and loc in DISCONNECT
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC323: SETUP rejected: RELEASE COMPLETE message received (U1)

Description:	While in the state CC receives a RELEASE COMPLETE message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then stops the T303 timer, enters the state NULL (U0) and issues a MNCC-RELEASE indication primitive to MMI followed by a MMCC-RELEASE request primitive to MM. (Ref. [1] 5.2.1.7)

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(RELEASE COMPLETE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE_COMP
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CHAN_UNACCEPT
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CHAN_UNACCEPT

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	CAUSE_CHAN_UNACCEPT
	16-Jul-97	VK	RELEASE_IND params revised
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC324: Mobile originated call release requested (U1)

Description:	While in the state CALL INIT (U1) CC receives a MNCC-DISCONNECT request primitive. CC builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive. CC then starts the T305 timer and enters the state DISCONNECT REQUEST (U11). (Ref. [1] 5.4.3.1)

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_DISCONNECT_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	FACILITY_NONE
		ss_version	SS_VER_NOT_PRES

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	cs and loc removed
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC325: No response from BS - timer expiry (U1)

Description:	The T303 timer expires before a response to the SETUP message is received. CC issues a DISCONNECT message as part of a MNCC-DATA request primitive followed by a MNCC-REJECT indication primitive and enters the state DISCONNECT REQUEST (U11).

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
MUTE (25000)
(1)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
(2)	 |	MNCC_SETUP_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_TIMER_303
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MNCC_SETUP_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_TIMER
		progress_desc	NOT_USED
		connected_number	NOT_USED
		connected_number_sub	NOT_USED

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	use   CC_CAUSE_TIMER_303
	16-Jul-97	VK	SETUP_CNF params revised
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC326: Error from network while awaiting call establishment (U1)

Description:	While in the state CALL INIT (U1) CC receives a MMCC-ERROR indication primitive from MM. CC issues a MNCC-REJ indication primitive to MMI followed by a MMCC-RELEASE request primitive to MM and enters the state NULL (U0).

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ERROR_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ERROR_IND
		ti	TI_MO_5
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	16-Jul-97	VK	introduce ERRCS_CELL_SELECT_FAIL
	16-Jul-97	VK	remove MNCC_REJECT_IND
	16-Jul-97	VK	introduce MNCC_RELEASE_IND
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC331: Mobile originated call release requested (U3)

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a MNCC-DISCONNECT request primitive. CC builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive. CC then starts the T305 timer and enters the state DISCONNECT REQUEST (U11). (Ref. [1] 5.4.3.1)

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_DISCONNECT_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	FACILITY_NONE
		ss_version	SS_VER_NOT_PRES

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	17-Jul-97	VK	remove cs and loc in DISCONNECT
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

26-Nov-01	OT	Variant CC026A instead of CC026

### CC332: No response from BS - timer  expiry (U3)

Description:	The T310 timer expires before an ALERTING or CONNECT message is received. CC issues a DISCONNECT message as part of a MNCC-DATA request primitive followed by a MNCC-SETUP confirmation primitive indicating the timeout and enters the state DISCONNECT REQUEST (U11).

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
MUTE (25000)
(1)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
(2)	 |	MNCC_SETUP_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_TIMER_310
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MNCC_SETUP_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_TIMER
		progress_desc	NOT_USED
		connected_number	NOT_USED
		connected_number_sub	NOT_USED

History:	25-Apr-97	DL	Initial
	17-Jul-97	VK	params revised
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

26-Nov-01	OT	Variant CC026A instead of CC026

### CC333: DISCONNECT message received from network (U3)

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-DISCONNECT indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive. CC then enters the state RELEASE REQUEST (U19).

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_UNSPECIFIED
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_UNSPECIFIED
		diagnostic	NOT_USED
		progress_desc	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	NOT_USED
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

26-Nov-01	OT	Variant CC026A instead of CC026

### CC334: RELEASE message received (U3)

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-RELEASE indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive and a MMCC-RELEASE request primitive to MM. CC then enters the state NULL (U0).

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE 
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	17-Jul-97	VK	params revised
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

26-Nov-01	OT	Variant CC026A instead of CC026

### CC335: Error from network during call establishment (U3)

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a MMCC-ERROR indication primitive from MM. CC issues a MNCC-REJ indication primitive to MMI followed by a MMCC-RELEASE request primitive to MM and enters the state NULL (U0).

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ERROR_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ERROR_IND
		ti	TI_MO_5
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	17-Jul-97	VK	REJECT_IND params revised
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

26-Nov-01	OT	Variant CC026A instead of CC026

### CC341: Mobile originated call release requested (U4)

Description:	While in the state CALL DELIVERED (U4) CC receives a MNCC-DISCONNECT request primitive. CC builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive. CC then starts the T305 timer and enters the state DISCONNECT REQUEST (U11). (Ref. [1] 5.4.3.1)

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_DISCONNECT_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	FACILITY_NONE
		ss_version	SS_VER_NOT_PRES

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC342: DISCONNECT message received from network (U4)

Description:	While in the state CALL DELIVERED (U4) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-DISCONNECT indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive. CC then enters the state RELEASE REQUEST (U19).

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_UNSPECIFIED
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_UNSPECIFIED
		diagnostic	NOT_USED
		progress_desc	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	NOT_USED
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC343: RELEASE message received (U4)

Description:	While in the state CALL DELIVERED (U4) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-RELEASE indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive and a MMCC-RELEASE request primitive to MM. CC then enters the state NULL (U0).

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE 
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC344: Error from network during call establishment (U4)

Description:	While in the state CALL DELIVERED (U4) CC receives a MMCC-ERROR indication primitive from MM. CC issues a MNCC-REJ indication primitive to MMI followed by a MMCC-RELEASE request primitive to MM and enters the state NULL (U0).

Preamble:	CC027

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ERROR_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ERROR_IND
		ti	TI_MO_5
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC361: Mobile originated call release requested (U9)

Description:	While in the state MT CALL CONFIRMED (U9) CC receives a MNCC-DISCONNECT request primitive. CC builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive. CC then starts the T305 timer and enters the state DISCONNECT REQUEST (U11). (Ref. [1] 5.4.3.1)

Preamble:	CC092A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_DISCONNECT_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
		ti	TI_MT_1_RESP
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	FACILITY_NONE
		ss_version	SS_VER_NOT_PRES

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc092a
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC362: DISCONNECT message received from network (U9)

Description:	While in the state MT CALL CONFIRMED (U9) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-DISCONNECT indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive. CC then enters the state RELEASE REQUEST (U19).

Preamble:	CC092A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MT_1
		cc_cause	CC_CAUSE_UNSPECIFIED
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_UNSPECIFIED
		diagnostic	NOT_USED
		progress_desc	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MT_1_RESP
		cc_cause	NOT_USED
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc092a
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC363: RELEASE message received (U9)

Description:	While in the state MT CALL CONFIRMED (U9) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-RELEASE indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive and a MMCC-RELEASE request primitive to MM. CC then enters the state NULL (U0).

Preamble:	CC092A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE 
		ti	TI_MT_1
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc092a
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC364: Error from network during call establishment (U9)

Description:	While in the state MT CALL CONFIRMED (U9) CC receives a MMCC-ERROR indication primitive from MM. CC issues a MNCC-REJ indication primitive to MMI followed by a MMCC-RELEASE request primitive to MM and enters the state NULL (U0).

Preamble:	CC092A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ERROR_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ERROR_IND
		ti	TI_MT_1_RESP
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MNCC_RELEASE_IND
		ti	TI_MT_1_RESP
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc092a
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC371: Mobile originated call release requested (U7)

Description:	While in the state CALL RECEIVED (U7) CC receives a MNCC-DISCONNECT request primitive. CC builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive. CC then starts the T305 timer and enters the state DISCONNECT REQUEST (U11). (Ref. [1] 5.4.3.1)

Preamble:	CC112

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_DISCONNECT_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
		ti	TI_MT_1_RESP
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	FACILITY_NONE
		ss_version	SS_VER_NOT_PRES

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC372: DISCONNECT message received from network (U7)

Description:	While in the state CALL RECEIVED (U7) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-DISCONNECT indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive. CC then enters the state RELEASE REQUEST (U19).

Preamble:	CC112

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MT_1
		cc_cause	CC_CAUSE_UNSPECIFIED
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_UNSPECIFIED
		diagnostic	NOT_USED
		progress_desc	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MT_1_RESP
		cc_cause	NOT_USED
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC373: RELEASE message received (U7)

Description:	While in the state CALL RECEIVED (U7) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-RELEASE indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive and a MMCC-RELEASE request primitive to MM. CC then enters the state NULL (U0).

Preamble:	CC112

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE 
		ti	TI_MT_1
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC374: Error from network during call establishment (U7)

Description:	While in the state CALL RECEIVED (U7) CC receives a MMCC-ERROR indication primitive from MM. CC issues a MNCC-REJ indication primitive to MMI followed by a MMCC-RELEASE request primitive to MM and enters the state NULL (U0).

Preamble:	CC112

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ERROR_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ERROR_IND
		ti	TI_MT_1_RESP
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MNCC_RELEASE_IND
		ti	TI_MT_1_RESP
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC381: Mobile originated call release requested (U8)

Description:	While in the state CALL CONNECT (U8) CC receives a MNCC-DISCONNECT request primitive. CC builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive. CC then starts the T305 timer and enters the state DISCONNECT REQUEST (U11). (Ref. [1] 5.4.3.1)

Preamble:	CC113A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_DISCONNECT_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
		ti	TI_MT_1_RESP
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	FACILITY_NONE
		ss_version	SS_VER_NOT_PRES

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc113a
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC382: No response from BS - timer  expiry (U8)

Description:	The T313 timer expires before a CONNECT ACKNOWLEDGE message is received. CC issues a DISCONNECT message as part of a MNCC-DATA request primitive followed by a MNCC-SETUP-COMPL indication primitive and enters the state DISCONNECT REQUEST (U11).

Preamble:	CC113A

MMI		 CC		 MM
	 |		  |		  |
MUTE (25000)
(1)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
(2)	 |	MNCC_SETUP_COMPL_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_TIMER_313
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MNCC_SETUP_COMPL_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_MS_TIMER

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc113a
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC383: DISCONNECT message received from network (U8)

Description:	While in the state CALL CONNECT (U8) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-DISCONNECT indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive. CC then enters the state RELEASE REQUEST (U19).

Preamble:	CC113A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MT_1
		cc_cause	CC_CAUSE_UNSPECIFIED
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_UNSPECIFIED
		diagnostic	NOT_USED
		progress_desc	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MT_1_RESP
		cc_cause	NOT_USED
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc113a
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC384: RELEASE message received (U8)

Description:	While in the state CALL CONNECT (U8) CC receives a DISCONNECT message as part of a MNCC-DATA request primitive. CC sends a MNCC-RELEASE indication primitive to MMI; followed by RELEASE COMPLETE message as part of a MMCC-DATA request primitive and a MMCC-RELEASE request primitive to MM. CC then enters the state NULL (U0).

Preamble:	CC113A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE 
		ti	TI_MT_1
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MT_1_RESP
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc113a
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC385: Error from network during call establishment (U8)

Description:	While in the state CALL CONNECT (U8) CC receives a MMCC-ERROR indication primitive from MM. CC issues a MNCC-REJ indication primitive to MMI followed by a MMCC-RELEASE request primitive to MM and enters the state NULL (U0).

Preamble:	CC113A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ERROR_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ERROR_IND
		ti	TI_MT_1_RESP
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MNCC_RELEASE_IND
		ti	TI_MT_1_RESP
		cause	RRCS_ABORT_CEL_SEL_FAIL

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	25-Apr-97	DL	Initial
	31-Jul-97	VK	preamble is cc113a
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC402: Mobile originated call release requested (U10)

Description:	While in the state ACTIVE (U10) CC receives a MNCC-DISCONNECT request primitive. CC builds a DISCONNECT message and sends this as part of a MMCC-DATA request primitive. CC then starts the T305 timer and enters the state DISCONNECT REQUEST (U11). (Ref. [1] 5.4.3.1)

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_DISCONNECT_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_DISCONNECT_REQ
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	FACILITY_NONE
		ss_version	SS_VER_NOT_PRES

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC407: DISCONNECT message received from network (U10)

Description:	While in the state ACTIVE (U10) CC receives a DISCONNECT message as part of a MMCC-DATA indication primitive. CC issues a MNCC- DISCONNECT indication primitive to MMI and enters the state DISCONNECT INDICATION (U12). (Ref. [1] 5.4.4.1)

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_UNSPECIFIED
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_UNSPECIFIED
		diagnostic	NOT_USED
		progress_desc	NOT_USED

History:	25-Apr-97	DL	Initial
	29-jul-97	VK	next state is U19
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC408: DISCONNECT message received from network (U10)

Description:	While in the state ACTIVE (U10) CC receives a DISCONNECT message as part of a MMCC-DATA indication primitive. The message contains the information in-band tones available.

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(4)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_SYNC_IND
		ti	TI_MO_5
		chm	CH_SPEECH_FULL

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CHANNEL_SYNC
		chm	CH_SPEECH_FULL

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_UNSPECIFIED
		facility	NOT_USED
		progress	PROGRESS_INBAND_AVAIL
		user_user	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_UNSPECIFIED
		diagnostic	NOT_USED
		progress_desc	PROG_INBAND_AVAIL

History:	29-Jul-97	VK	Initial (preamble for CC412)
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC418: RELEASE COMPLETE message received from network (U10)

Description:	While in the state ACTIVE (U10) CC receives a RELEASE COMPLETE message as part of a MMCC-DATA indication primitive. CC issues a MNCC RELEASE CNF indication primitive to MMI, releases the connection and enterst the state NULL (U0).

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(RELEASE COMPLETE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE_COMP
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC404: RELEASE message received - mobile originated call release (U11)

Description:	While in the state DISCONNECT REQUEST (U11) CC receives a RELEASE message in the form of a MMCC-DATA indication primitive. CC builds a RELEASE COMPLETE message and sends this in the form of a MMCC-DATA indication primitive to MM. CC then issues a MNCC-RELEASE indication primitive to MMI and a MMCC-RELEASE indication primitive to MM and then enters the state NULL (U0). (Ref. [1] 5.4.3.3)

Preamble:	CC402

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE)	  |
	 |		  *<==============================*
(2)	 |	MNCC_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(RELEASE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE 
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MO_5
		cc_cause	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	09-Jul-01	JHU	Converted to TAP2

### CC405: No response to DISCONNECT message (U11)

Description:	While in the state DISCONNECT REQUEST (U11) the T308 timer expires before the network has responded to the previous DISCONNECT message. CC builds a RELEASE message and sends this in the form of a MMCC-DATA indication primitive to MM followed by a MNCC-DISCONNECT indication primitive to MMI. On receipt of a RELEASE COMPLETE mesage as part of a MMCC-DATA indication primitive CC issues a MNCC-RELEASE confirmation primitive to MMI followed by a MMCC-RELEASE request primitive to MM and then enters the state NULL (U0). (Ref. [1] 5.4.3.5)

Preamble:	CC402

MMI		 CC		 MM
	 |		  |		  |
MUTE (25000)
(1)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *<==============================*
(4)	 |	MNCC_RELEASE_CNF	  |		  |
	 *<==============================*		  |
(5)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_CLEAR
		diagnostic	NOT_USED
		progress_desc	NOT_USED

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE_COMP
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	29-Jul-97	VK	timer is T305 (ref [1], 5.4.3.5)
	29-Jul-97	VK	RELEASE_CNF param revised
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	25-Jan-00	HM	empty facility added
	21-Mar-00	HM	and empty facility removed
	09-Jul-01	JHU	Converted to TAP2

### CC415: No response to RELEASE message (U11)

Description:	CC is in the state DISCONNECT REQUEST (U11). It transmits a RELEASE message as part of a MMCC-DATA request primitive to MM. There is no response from the network, therefore; following expiry of the T308 timer, CC restarts the timer retransmits the RELEASE message.  When T308 timer expiry occurs a second time CC issues a MNCC-RELEASE confirmation primitive to MMI followed by a MMCC-RELEASE request primitive to MM and then enters the state NULL (U0). (Ref. [1] 5.4.3.5)

Preamble:	CC402

MMI		 CC		 MM
	 |		  |		  |
MUTE (25000)
(1)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
MUTE (25000)
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
MUTE (25000)
(4)	 |	MNCC_RELEASE_CNF	  |		  |
	 *<==============================*		  |
(5)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MNCC_DISCONNECT_IND
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_CLEAR
		diagnostic	NOT_USED
		progress_desc	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MNCC_RELEASE_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_TIMER

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	29-Jul-97	VK	remove ‘timeout’ comments
	29-Jul-97	VK	cc_cause in U_RELEASE : NOT_USED
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	25-Jan-99	HM	empty facilities added
	21-Mar-00	HM	and empty facility removed
	09-Jul-01	JHU	Converted to TAP2

### CC409: No in-band tones - release by MS - mobile originated call release (U12)

Description:	CC is in the state DISCONNECT INDICATION (U12). As no in-band tones are provided CC starts release immediately by issuing a RELEASE message as part of a MMCC-DATA indication primitive to MM and a MNCC-RELEASE confirmation primitive to MMI. CC then issues a RELEASE COMPLETE message to MM as part of a MMCC-DATA indication primitive and enters the state RELEASE REQUEST (U19). (Ref. [1] 5.4.4.1)

Preamble:	CC407

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
(2)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *<==============================*
(3)	 |	MNCC_RELEASE_CNF	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	NOT_USED
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	D_RELEASE_COMP
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	29-Jul-97	VK	cc_cause in U_RELEASE : NOT_USED
	29-Jul-97	VK	RELEASE_CNF param revised
	04-Aug-97	VK	use MMCC_DATA_IND
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	25-Jan-00	HM	empty facilities added
	21-Mar-00	HM	and empty facility removed
	09-Jul-01	JHU	Converted to TAP2

### CC412: In-band tones - release follows hook-on (U12)

Description:	CC is in the state DISCONNECT INDICATION (U12). In-band tones are provided, so CC awaits a MNCC-RELEASE request primitive from MMI, indicating that hook-on is complete. CC then issues a RELEASE message as part of a MMCC-DATA indication primitive to MM and a MNCC-RELEASE confirmation primitive to MMI. This is followed by a RELEASE COMPLETE message to MM as part of a MMCC-DATA indication primitive and CC then enters the state RELEASE REQUEST (U19). (Ref. [1] 5.4.4.1)

Preamble:	CC408

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_RELEASE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *<==============================*
(4)	 |	MNCC_RELEASE_CNF	  |		  |
	 *<==============================*		  |
(5)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_RELEASE_REQ
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	NOT_USED
		ss_version	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	D_RELEASE_COMP
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	NOT_USED
		user_user	NOT_USED
		}

- MNCC_RELEASE_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_CALL_CLEAR

- MMCC_RELEASE_REQ
		ti	TI_MO_5

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	28-Jul-98	LE	adapted to fax & data
	24-Jan-00	HM	empty facilities added
	21-Mar-00	HM	and empty facilty removed
	09-Jul-01	JHU	Converted to TAP2

### CC420: DISCONNECT received (U3), CCBS, in-band tones

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a DISCONNECT  message as part of a MNCC-DATA request primitive. CC sends a MNCC-DISCONNECT indication primitive to MMI and await user decision about CCBS. The connection must't be released witout further user interaction, CC has to stay in state DISCONNECT INDICATION (U12).

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_USER_BUSY
		facility	NOT_USED
		progress	PROGRESS_INBAND_AVAIL
		user_user	NOT_USED
		allowed_actions	ALLOWED_ACTIONS_CCBS_ON
		}

- MNCC_DISCONNECT_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_USER_BUSY
		diagnostic	DIAG_CCBS_POSSIBLE
		progress_desc	PROG_INBAND_AVAIL

History:	21-Jan-00	HM	Initial
09-Jul-01	JHU	Converted to TAP2
26-Nov-01	OT	Variant CC026A instead of CC026

### CC421: DISCONNECT received (U3), CCBS, no in-band tones

Description:	CC received a DISCONNECT message, CCBS activation is possible. No in-band tones are provided by the network. The connection mustn't be released, CC has to stay in state U12.

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |	MNCC_DISCONNECT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_USER_BUSY
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		allowed_actions	ALLOWED_ACTIONS_CCBS_ON
		}

- MNCC_DISCONNECT_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_USER_BUSY
		diagnostic	DIAG_CCBS_POSSIBLE
		progress_desc	NOT_USED

History:	21-Jan-00	HM	Initial
09-Jul-01	JHU	Converted to TAP2
26-Nov-01	OT	Variant CC026A instead of CC026

### CC422: RELEASE sent with CCBS activation

Description:	The user wants to invoke CCBS, a RELEASE message with facility is sent to the network.

Preamble:	CC421

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_RELEASE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	(RELEASE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	(RELEASE COMPLETE)	  |
	 |		  *<==============================*
(4)	 |	MNCC_FACILITY_IND	  |		  |
	 *<==============================*		  |
(5)	 |	MNCC_RELEASE_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_RELEASE_REQ
		ti	TI_MO_5
		cause	ACI_CAUSE_CALL_CLEAR
		fac_inf	FACILITY_A_FAC
		ss_version	SS_VERSION_3

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	CC_CAUSE_CALL_CLEAR
		cc_cause_2	NOT_USED
		facility	FACILITY_A
		user_user	NOT_USED
		ss_version	SS_VERS_3
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE_COMP
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_CALL_CLEAR
		facility	FACILITY_A
		user_user	NOT_USED
		}

- MNCC_FACILITY_IND	ti	TI_MO_5
		fac_context	FAC_IN_RELEASE_COMP
		fac_inf	FACILITY_A_FAC

- MNCC_RELEASE_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_CALL_CLEAR

History:	21-Jan-00	HM	Initial
21-Mar-00	HM	Enhanced
19-Mar-02	HM	Changed order of primitives

## Bearer Capability Handling

### CC500: MOC, Speech, Fullrate only

Description:	A mobile originated call is started for speech in the GSM 900 range. The PC version has only fullrate enabled for GSM 900.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC501: MOC, Speech, Fullrate and Halfrate

Description:	A mobile originated call is started for speech in the DCS 1800 range. The PC version has enabled fullrate and halfrate for DCS 1800. It is expected that the bearer capability indicates dual rate, full rate preferred.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=4)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR_HR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC502: MOC, Speech, Fullrate, Halfrate and Enhanced Fullrate

Description:	A mobile originated call is started for speech in the PCS 1900 range. The PC version has enabled fullrate, halfrate and enhanced fullrate for PCS 1900. It is expected that the bearer capability indicates dual rate, full rate preferred and three extensions of octet 3 for fullrate, enhanced fullrate and halfrate.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR_HR_EFR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC503: MOC, Speech, Fullrate and Enhanced Fullrate

Description:	A mobile originated call is started for speech in the Dualband range. The PC version has enabled fullrate and enhanced fullrate for Dualband. It is expected that the bearer capability indicates full rate support only mobile stations and two extensions of octet 3 for fullrate and enhanced fullrate.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR_EFR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC900: MOC, EC-Speech, Fullrate, Halfrate and Enhanced Fullrate

Description:	A mobile originated call is started for speech in the PCS 1900 range. The PC version has enabled fullrate, halfrate and enhanced fullrate for PCS 1900. It is expected that the bearer capability indicates dual rate, full rate preferred and three extensions of octet 3 for fullrate, enhanced fullrate and halfrate.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (EMERGENCY SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_EMERG_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_EMERGE

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_EMERGE_SETUP
		ti	TI_MO_5
		bearer_cap	BC_SPEECH_FR_HR_EFR
		}

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	12-Jul-02	HM	ESTCS_EMERGE for emergency call

### CC901: MTC, Speech, accepted, with EFR

Description:	CC receives a SETUP message from the network in the form of a MMCC-ESTABLISH indication primitive. CC enters the state CALL PRESENT (U6) and starts compatibility checks. (Ref. [1] 5.2.2.1)

The compatibility checks have been evaluated successfully. CC sends a MNCC-SETUP indication primitive to MMI, builds a CALL CONFIRMED message and sends this in the form of a MNCC-DATA request primitive to MM, and enters the state MT CALL CONFIRMED (U9). (Ref. [1] 5.2.2.3.1)

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR_HR_EFR
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	24-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC902: MOC, Speech, Fullrate, Halfrate Enhanced Fullrate, AMR Half- & Fullrate

Description:	A mobile originated call is started for speech in the PCS 1900 range. The PC version has enabled fullrate, halfrate, enhanced fullrate and amr half- & fullrate for PCS 1900. It is expected that the bearer capability indicates dual rate, full rate preferred and five extensions of octet 3 for amr_fr, amr_hr, efr, hr, fr.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=30)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_AMR_EFR_HR_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Nov-01	OT	Initial
	11-Jun-02	HM	std=30 instead of std=8

### CC903: MTC, Speech, accepted, with AMR_FR

Description:	CC receives a SETUP message from the network in the form of a MMCC-ESTABLISH indication primitive. CC enters the state CALL PRESENT (U6) and starts compatibility checks. (Ref. [1] 5.2.2.1)

The compatibility checks have been evaluated successfully. CC sends a MNCC-SETUP indication primitive to MMI, builds a CALL CONFIRMED message and sends this in the form of a MNCC-DATA request primitive to MM, and enters the state MT CALL CONFIRMED (U9). (Ref. [1] 5.2.2.3.1)

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=30)
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_AMR_EFR_HR_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	NOT_USED
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	23-Nov-01	OT	Initial
	05-Apr-02	HM	call_ctrl_cap removed as in target
	11-Jun-02	HM	std=30 instead of std=8

### CC504: MTC, Speech, accepted

Description:	CC receives a SETUP message from the network in the form of a MMCC-ESTABLISH indication primitive. CC enters the state CALL PRESENT (U6) and starts compatibility checks. (Ref. [1] 5.2.2.1)

The compatibility checks have been evaluated successfully. CC sends a MNCC-SETUP indication primitive to MMI, builds a CALL CONFIRMED message and sends this in the form of a MNCC-DATA request primitive to MM, and enters the state MT CALL CONFIRMED (U9). (Ref. [1] 5.2.2.3.1)

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	24-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC505: MTC, no Bearer Capabilities, accepted

Description:	CC receives a SETUP message from the network in the form of a MMCC-ESTABLISH indication primitive. CC enters the state CALL PRESENT (U6) and starts compatibility checks. (Ref. [1] 5.2.2.1)

The compatibility checks have been evaluated successfully. CC sends a MNCC-SETUP indication primitive to MMI, builds a CALL CONFIRMED message and sends this in the form of a MNCC-DATA request primitive to MM, and enters the state MT CALL CONFIRMED (U9). (Ref. [1] 5.2.2.3.1)

The incoming SETUP message contains no bearer capabilities. CC accepts this and answers in the CALL CONFIRMED message with a speech bearer capability.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	24-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC506: MOC, BS21, 3.1 kHz Audio, Transparent, 0.3 kBit/s (I)

Description:	A mobile originated call is started for transparent asynchronous data with 0.3 kBit/s. This is bearer service 21. The data service is supported in PCS 1900 range by the PC version.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC507: MOC, BS21, 3.1 kHz Audio, Transparent, 0.3 kBit/s (II)

Description:	A mobile originated call is started for transparent asynchronous data with 0.3 kBit/s. This is bearer service 21. The data service is supported in Dualband range by the PC version. This range supports no halfrate. So a different establishment cause is expected.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300_FR
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC508: MOC, BS21, 3.1 kHz Audio, Transparent, 0.3 kBit/s (III)

Description:	A mobile originated call is started for transparent asynchronous data with 0.3 kBit/s. This is bearer service 21. The data service is not supported in GSM 900 range by the PC version. A release is indicated to the upper layer.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MNCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_BEARER_NOT_IMPLEM

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC509: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 0.3 kBit/s (I)

Description:	A mobile originated call is started for non-transparent asynchronous data with 0.3 kBit/s. This is bearer service 21. The data service is supported in PCS 1900 range by the PC version. The indicated flow control is inband flow control.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_1
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_1_OUTBAND
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC510: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 0.3 kBit/s (II)

Description:	A mobile originated call is started for non-transparent asynchronous data with 0.3 kBit/s. This is bearer service 21. The data service is supported in PCS range by the PC version. The indicated flow control is outband flow control.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_2
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_2
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC511: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 0.3 kBit/s (III)

Description:	A mobile originated call is started for non-transparent asynchronous data with 0.3 kBit/s. This is bearer service 21. The data service is supported in PCS range by the PC version. The indicated flow control is no flow control.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_3
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_3
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC512: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 0.3 kBit/s (IV)

Description:	A mobile originated call is started for non-transparent asynchronous data with 0.3 kBit/s. This is bearer service 21. The data service is not supported in GSM 900 range by the PC version. A release is indicated to the upper layer.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MNCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_1
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_BEARER_NOT_IMPLEM

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC513: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s accepted

Description:	A mobile terminated call is started for BS21 transparent (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001B

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC514: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s negotiated (I)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, transparent preferred. So it is exspected, that the mobile selects transparent.

Preamble:	CC001A

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BTP
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC515: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s negotiated (II)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, non-transparent preferred. As no explicit configuration of CC via MNCC_CONFIGURE_REQ takes place in this case, the default setting to non-transparent leads to a passing compatibilty check and the mobile selects non-transparent.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BNTP
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_4
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_4B
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC516: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s negotiated (III)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the Dualband range. The PC version supports this data service for Dualband. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, non-transparent preferred. So it is exspected, that the mobile selects transparent, because in dualband (STD=5) only transparent services are configured as supported.

Preamble:	CC001B

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BNTP
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC517: MTC, BS21, 3.1 kHz Audio Transparent 0.3 kBit/s rejected

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the GSM 900 range. The PC version doesn´t support data service for the GSM 900 only variant. It is expected that the call attempt is rejected.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BNTP
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INCOMPAT
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC518: MTC, BS21, UDI Transparent 0.3 kBit/s accepted

Description:	A mobile terminated call is started for BS21 transparent (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001A

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC519: MTC, BS21, UDI Transparent 0.3 kBit/s negotiated (I)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, transparent preferred. So it is exspected, that the mobile selects transparent.

Preamble:	CC001B

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BTP_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300_UDI
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC520: MTC, BS21, UDI Transparent 0.3 kBit/s negotiated (II)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, non-transparent preferred. As no explicit configuration of CC via MNCC_CONFIGURE_REQ takes place in this case, the default setting to non-transparent leads to a passing compatibilty check and the mobile selects non-transparent.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BNTP_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_4_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_4_UDI
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC521: MTC, BS21, UDI Transparent 0.3 kBit/s negotiated (III)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the Dualband range. The PC version supports this data service for Dualband only for transparent services. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, non-transparent preferred. So it is exspected, that the mobile selects transparent, because in dualband (STD=5) only transparent services are configured as supported.

Preamble:	CC001A

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BNTP_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300_UDI
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC522: MTC, BS21, UDI 0.3 kBit/s rejected

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the GSM 900 range. The PC version doesn´t support data service for GSM 900. It is expected that the call attempt is rejected.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BNTP_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INCOMPAT
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC523: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s accepted

Description:	A mobile terminated call is started for BS21 non-transparent (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation. The incoming bearer capability indicates as connection element of non-transparent. As no explicit configuration of CC via MNCC_CONFIGURE_REQ takes place in this case, the default setting to non-transparent leads to a passing compatibilty check.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_1
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_1
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC524: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s negotiated (I)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, non-transparent preferred. As no explicit configuration of CC via MNCC_CONFIGURE_REQ takes place in this case, the default setting to non-transparent leads to a passing compatibilty check and the mobile selects non-transparent.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BNTP
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_4
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_4B
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC525: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s negotiated (II)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, transparent preferred.

Preamble:	CC001B

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BTP
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC526: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s negotiated (III)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the DCS 1800 range. The PC version supports this data service for DCS 1800. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, transparent preferred. So it is exspected, that the mobile selects non-transparent, because in DCS 1800 only (STD=4) only non-transparent services are configured as supported.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=4)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BTP
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_5
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_5
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC527: MTC, BS21, 3.1 kHz Audio Non-Transparent 0.3 kBit/s rejected

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the GSM 900 range. The PC version doesn´t support data service for GSM 900. It is expected that the call attempt is rejected.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_4
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INCOMPAT
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC528: MTC, BS21, UDI Non-Transparent 0.3 kBit/s accepted

Description:	A mobile terminated call is started for BS21 non-transparent (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_1_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_1_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC529: MTC, BS21, UDI Non-Transparent 0.3 kBit/s negotiated (I)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, non-transparent preferred. So it is exspected, that the mobile selects non-transparent.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BNTP_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_4_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_4_UDI
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC530: MTC, BS21, UDI Non-Transparent 0.3 kBit/s negotiated (II)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, transparent preferred. So it is exspected, that the mobile selects transparent.

Preamble:	CC001A

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BTP_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_T_300_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300_UDI
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC531: MTC, BS21, UDI Non-Transparent 0.3 kBit/s negotiated (III)

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the DCS 1800 range. The PC version supports this data service for DCS 1800. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, transparent preferred. So it is exspected, that the mobile selects non-transparent, because in DCS 1800 only non-transparent services are configured as supported.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=4)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BTP_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS21_NT_300_5_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS21_NT_300_5_UDI
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC532: MTC, BS21, UDI 0.3 kBit/s rejected

Description:	A mobile terminated call is started for BS21 (300 bit/s) in the GSM 900 range. The PC version doesn´t support data service for GSM 900. It is expected that the call attempt is rejected.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS21_300_BTP_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INCOMPAT
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC533: MOC, BS22, 3.1 kHz Audio, Transparent, 1.2 kBit/s

Description:	A mobile originated call is started for transparent asynchronous data with 1.2 kBit/s. This is bearer service 22. The data service is supported in PCS 1900 range by the PC version.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS22_T_1200
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS22_T_1200
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	29-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC534: MOC, BS21, 3.1 kHz Audio, Non-Transparent, 1.2 kBit/s

Description:	A mobile originated call is started for non-transparent asynchronous data with 1.2 kBit/s. This is bearer service 22. The data service is supported in PCS 1900 range by the PC version. The indicated flow control is inband flow control.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS22_NT_1200
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS22_NT_1200_OUTBAND
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	29-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC535: MTC, BS21, 3.1 kHz Audio Transparent 1.2 kBit/s accepted

Description:	A mobile terminated call is started for BS22 transparent (1200 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001B

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS22_T_1200
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS22_T_1200
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC536: MTC, BS21, UDI Transparent 1.2 kBit/s accepted

Description:	A mobile terminated call is started for BS22 transparent (1200 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001A

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS22_T_1200_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS22_T_1200_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC537: MTC, BS22, 3.1 kHz Audio Non-Transparent 1.2 kBit/s accepted

Description:	A mobile terminated call is started for BS22 non-transparent (1200 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation. The incoming bearer capability indicates as connection element non-transparent. As no explicit configuration of CC via MNCC_CONFIGURE_REQ takes place in this case, the default setting to non-transparent leads to a passing compatibilty check.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS22_NT_1200
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS22_NT_1200
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC538: MTC, BS22, UDI Non-Transparent 1.2 kBit/s accepted

Description:	A mobile terminated call is started for BS22 non-transparent (1200 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation. The incoming bearer capability indicates as connection element non-transparent preferred. As no explicit configuration of CC via MNCC_CONFIGURE_REQ takes place in this case, the default setting to non-transparent leads to a passing compatibilty check.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS22_NT_1200_UDI
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS22_NT_1200_UDI
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC539: MOC, BS24, 3.1 kHz Audio, Transparent, 2.4 kBit/s

Description:	A mobile originated call is started for transparent asynchronous data with 2.4 kBit/s. This is bearer service 24. The data service is supported in PCS 1900 range by the PC version.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS24_T_2400
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS24_T_2400
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	29-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC540: MOC, BS24, 3.1 kHz Audio, Non-Transparent, 2.4 kBit/s

Description:	A mobile originated call is started for non-transparent asynchronous data with 2.4 kBit/s. This is bearer service 24. The data service is supported in PCS 1900 range by the PC version. The indicated flow control is inband flow control.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS24_NT_2400
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	NOT_USED
	bearer_cap	BC_BS24_NT_2400_V26TER_OUTBAND
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC541: MTC, BS24, 3.1 kHz Audio Transparent 2.4 kBit/s accepted

Description:	A mobile terminated call is started for BS24 transparent (2400 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001B

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS24_T_2400
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS24_T_2400
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC542: MTC, BS24, UDI Transparent 2.4 kBit/s accepted

Description:	A mobile terminated call is started for BS24 transparent (2400 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001A

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS24_T_2400_UDI
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS24_T_2400_UDI
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC543: MTC, BS24, 3.1 kHz Audio Non-Transparent 2.4 kBit/s accepted

Description:	A mobile terminated call is started for BS24 non-transparent (2400 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS24_NT_2400
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS24_NT_2400_V22BIS
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC544: MTC, BS24, UDI Non-Transparent 2.4 kBit/s accepted

Description:	A mobile terminated call is started for BS24 non-transparent (2400 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS24_NT_2400_UDI
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS24_NT_2400_UDI
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC545: MOC, BS25, 3.1 kHz Audio, Transparent, 4.8 kBit/s

Description:	A mobile originated call is started for transparent asynchronous data with 4.8 kBit/s. This is bearer service 25. The data service is supported in PCS 1900 range by the PC version.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS25_T_4800
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	NOT_USED
	bearer_cap	BC_BS25_T_4800
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC546: MOC, BS25, 3.1 kHz Audio, Non-Transparent, 4.8 kBit/s

Description:	A mobile originated call is started for non-transparent asynchronous data with 4.8 kBit/s. This is bearer service 25. The data service is supported in PCS 1900 range by the PC version. The indicated flow control is inband flow control.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS25_NT_4800
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	NOT_USED
	bearer_cap	BC_BS25_NT_4800_OUTBAND
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC547: MTC, BS25, 3.1 kHz Audio Transparent 4.8 kBit/s accepted

Description:	A mobile terminated call is started for BS25 transparent (4800 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001B

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS25_T_4800
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS25_T_4800
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC548: MTC, BS25, UDI Transparent 4.8 kBit/s accepted

Description:	A mobile terminated call is started for BS25 transparent (4800 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001A

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS25_T_4800_UDI
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS25_T_4800_UDI
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC549: MTC, BS25, 3.1 kHz Audio Non-Transparent 4.8 kBit/s accepted

Description:	A mobile terminated call is started for BS25 non-transparent (4800 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS25_NT_4800
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS25_NT_4800
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC550: MTC, BS25, UDI Non-Transparent 4.8 kBit/s accepted

Description:	A mobile terminated call is started for BS25 non-transparent (4800 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS25_NT_4800_UDI
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS25_NT_4800_UDI
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC551: MOC, BS26, 3.1 kHz Audio, Transparent, 9.6 kBit/s

Description:	A mobile originated call is started for transparent asynchronous data with 9.6 kBit/s. This is bearer service 26. The data service is supported in PCS 1900 range by the PC version.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS26_T_9600
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	NOT_USED
	bearer_cap	BC_BS26_T_9600
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC552: MOC, BS26, 3.1 kHz Audio, Non-Transparent, 9.6 kBit/s

Description:	A mobile originated call is started for non-transparent asynchronous data with 9.6 kBit/s. This is bearer service 26. The data service is supported in PCS 1900 range by the PC version. The indicated flow control is inband flow control.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS26_NT_9600
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	NOT_USED
	bearer_cap	BC_BS26_NT_9600_OUTBAND
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC553: MTC, BS26, 3.1 kHz Audio Transparent 9.6 kBit/s accepted

Description:	A mobile terminated call is started for BS26 transparent (9600 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001B

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS26_T_9600
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS26_T_9600
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC554: MTC, BS26, UDI Transparent 9.6 kBit/s accepted

Description:	A mobile terminated call is started for BS26 transparent (9600 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC001A

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS26_T_9600_UDI
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS26_T_9600_UDI
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC555: MTC, BS26, 3.1 kHz Audio Non-Transparent 9.6 kBit/s accepted

Description:	A mobile terminated call is started for BS26 non-transparent (9600 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS26_NT_9600
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS26_NT_9600
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC556: MTC, BS26, UDI Non-Transparent 9.6 kBit/s accepted

Description:	A mobile terminated call is started for BS26 non-transparent (9600 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted without negotiation.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS26_NT_9600_UDI
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS26_NT_9600_UDI
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC571: MTC, BS26, 3.1 kHz Audio Non-Transparent 9.6 kBit/s negotiated

Description:	A mobile terminated call is started for BS26 (9600 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, non-transparent preferred. So it is exspected, that the mobile selects non-transparent.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
	bearer_cap	BC_BS26_9600_BNTP
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS26_NT_9600_1
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	BC_BS26_NT_9600_1
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	28-Jan-99	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC557: MOC, BS23, 3.1 kHz Audio, Transparent, 1200 / 75 Bit/s

Description:	A mobile originated call is started for transparent asynchronous data with 1200 / 75 Bit/s. This is bearer service 23. The data service is supported in PCS 1900 range by the PC version.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS23_T
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	NOT_USED
	bearer_cap	BC_BS23_T
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC558: MOC, BS23, 3.1 kHz Audio, Non-Transparent, 1200 / 75 Bit/s

Description:	A mobile originated call is started for non-transparent asynchronous data with 1200 / 75 Bit/s. This is bearer service 23. The data service is supported in PCS 1900 range by the PC version. The indicated flow control is inband flow control.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS23_NT
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	NOT_USED
	bearer_cap	BC_BS23_NT_OUTBAND
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC559: MOC, BS61, Alternate Speech / Transparent Async

Description:	A mobile originated call is started for alternate speech and transparent asynchronous data service. This is bearer service 61. The data service is supported in PCS 1900 range by the PC version.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s
F:	1200 / 75 bit/s

Preamble:	CC000

Variants:	<A> ... <F>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	REPEAT_CIRCULAR
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_BS21_T_300
<B>	bcpara2	BC_PARA_BS22_T_1200
<C>	bcpara2	BC_PARA_BS24_T_2400
<D>	bcpara2	BC_PARA_BS25_T_4800
<E>	bcpara2	BC_PARA_BS26_T_9600
<F>	bcpara2	BC_PARA_BS23_T
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	REPEAT_CIRCULAR
	bearer_cap	BC_SPEECH_FR_HR_EFR
<A>	bearer_cap_2	BC_BS21_T_300
<B>	bearer_cap_2	BC_BS22_T_1200
<C>	bearer_cap_2	BC_BS24_T_2400
<D>	bearer_cap_2	BC_BS25_T_4800
<E>	bearer_cap_2	BC_BS26_T_9600
<F>	bearer_cap_2	BC_BS23_T
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC560: MOC, BS61, Alternate Speech / Non-Transparent Async

Description:	A mobile originated call is started for alternate speech and non-transparent asynchronous data service. This is bearer service 61. The data service is supported in PCS 1900 range by the PC version.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s
F:	1200 / 75 bit/s

Preamble:	CC000

Variants:	<A> ... <F>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	REPEAT_CIRCULAR
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_BS21_NT_300_1
<B>	bcpara2	BC_PARA_BS22_NT_1200
<C>	bcpara2	BC_PARA_BS24_NT_2400
<D>	bcpara2	BC_PARA_BS25_NT_4800
<E>	bcpara2	BC_PARA_BS26_NT_9600
<F>	bcpara2	BC_PARA_BS23_NT
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	REPEAT_CIRCULAR
	bearer_cap	BC_SPEECH_FR_HR_EFR
<A>	bearer_cap_2	BC_BS21_NT_300_1_OUTBAND
<B>	bearer_cap_2	BC_BS22_NT_1200_OUTBAND
<C>	bearer_cap_2	BC_BS24_NT_2400_V26TER_OUTBAND
<D>	bearer_cap_2	BC_BS25_NT_4800_OUTBAND
<E>	bearer_cap_2	BC_BS26_NT_9600_OUTBAND
<F>	bearer_cap_2	BC_BS23_NT_OUTBAND
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC561: MOC, BS81, Sequential Speech / Transparent Async

Description:	A mobile originated call is started for speech followed by transparent asynchronous data service. This is bearer service 81. The data service is supported in PCS 1900 range by the PC version.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s
F:	1200 / 75 bit/s

Preamble:	CC000

Variants:	<A> ... <F>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	REPEAT_SEQUENTIAL
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_BS21_T_300
<B>	bcpara2	BC_PARA_BS22_T_1200
<C>	bcpara2	BC_PARA_BS24_T_2400
<D>	bcpara2	BC_PARA_BS25_T_4800
<E>	bcpara2	BC_PARA_BS26_T_9600
<F>	bcpara2	BC_PARA_BS23_T
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	REPEAT_SEQUENTIAL
	bearer_cap	BC_SPEECH_FR_HR_EFR
<A>	bearer_cap_2	BC_BS21_T_300
<B>	bearer_cap_2	BC_BS22_T_1200
<C>	bearer_cap_2	BC_BS24_T_2400
<D>	bearer_cap_2	BC_BS25_T_4800
<E>	bearer_cap_2	BC_BS26_T_9600
<F>	bearer_cap_2	BC_BS23_T
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC562: MOC, BS81, Sequential Speech / Non-Transparent Async

Description:	A mobile originated call is started for speech followed by non-transparent asynchronous data service. This is bearer service 81. The data service is supported in PCS 1900 range by the PC version.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s
F:	1200 / 75 bit/s

Preamble:	CC000

Variants:	<A> ... <F>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	REPEAT_SEQUENTIAL
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_BS21_NT_300_1
<B>	bcpara2	BC_PARA_BS22_NT_1200
<C>	bcpara2	BC_PARA_BS24_NT_2400
<D>	bcpara2	BC_PARA_BS25_NT_4800
<E>	bcpara2	BC_PARA_BS26_NT_9600
<F>	bcpara2	BC_PARA_BS23_NT
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	REPEAT_SEQUENTIAL
	bearer_cap	BC_SPEECH_FR_HR_EFR
<A>	bearer_cap_2	BC_BS21_NT_300_1_OUTBAND
<B>	bearer_cap_2	BC_BS22_NT_1200_OUTBAND
<C>	bearer_cap_2	BC_BS24_NT_2400_V26TER_OUTBAND
<D>	bearer_cap_2	BC_BS25_NT_4800_OUTBAND
<E>	bearer_cap_2	BC_BS26_NT_9600_OUTBAND
<F>	bearer_cap_2	BC_BS23_NT_OUTBAND
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC563: MOC, TS61, Alternate Speech / Transparent Fax

Description:	A mobile originated call is started for alternate speech and transparent fax. This is tele service 61. The data service is supported in PCS 1900 range by the PC version.

A:	2400 bit / s
B:	4800 bit / s
C:	9600 bit / s

Preamble:	CC000

Variants:	<A>..<C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	REPEAT_CIRCULAR
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_FAX_T_2400
<B>	bcpara2	BC_PARA_FAX_T_4800
<C>	bcpara2	BC_PARA_FAX_T_9600
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	REPEAT_CIRCULAR
	bearer_cap	BC_SPEECH_FR_HR_EFR
<A>	bearer_cap_2	BC_FAX_T_2400
<B>	bearer_cap_2	BC_FAX_T_4800
<C>	bearer_cap_2	BC_FAX_T_9600
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC564: MOC, TS62, Automatic Fax

Description:	A mobile originated call is started for automatic fax. This is tele service 62. The data service is supported in PCS 1900 range by the PC version.

A:	2400 bit / s
B:	4800 bit / s
C:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
	ti	TI_MO_5
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
<A>	bcpara	BC_PARA_FAX_T_2400
<B>	bcpara	BC_PARA_FAX_T_4800
<C>	bcpara	BC_PARA_FAX_T_9600
	bcpara2	BC_PARA_NO_SERVICE
	called_party	CLED_PARTY_654321
	called_party_sub	CLED_PARTY_SUB_NONE
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
	ti	TI_MO_5
	estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
	ti	TI_MO_5

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_SETUP
	ti	TI_MO_5
	repeat	NOT_USED
<A>	bearer_cap	BC_FAX_T_2400
<B>	bearer_cap	BC_FAX_T_4800
<C>	bearer_cap	BC_FAX_T_9600
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	calling_subaddr	NOT_USED
	ul_called_num	UL_CLED_NUM_654321
	called_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	ss_version	NOT_USED
	clir_suppr	NOT_USED
	clir_invoc	NOT_USED
	call_ctrl_cap	CALL_CTRL_CAP_1
	}

History:	29-Jul-98	LE	Initial
	27-Jun-00	HM	establish cause not indicating HR support
	10-Jul-01	JHU	Converted to TAP2

### CC565: MTC, BS61, Alternate Speech / Transparent Asynchronous

Description:	A mobile terminated call is started for alternate speech and transparent asynchronous data service. This is bearer service 61. The data service is supported in PCS 1900 range by the PC version.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC001B

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	REPEAT_CIRCULAR
	bearer_cap	BC_SPEECH_FR
<A>	bearer_cap_2	BC_BS21_T_300
<B>	bearer_cap_2	BC_BS22_T_1200
<C>	bearer_cap_2	BC_BS24_T_2400
<D>	bearer_cap_2	BC_BS25_T_4800
<E>	bearer_cap_2	BC_BS26_T_9600
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	REPEAT_CIRCULAR
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_BS21_T_300
<B>	bcpara2	BC_PARA_BS22_T_1200
<C>	bcpara2	BC_PARA_BS24_T_2400
<D>	bcpara2	BC_PARA_BS25_T_4800
<E>	bcpara2	BC_PARA_BS26_T_9600
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC566: MTC, BS61, Alternate Speech / Non-Transparent Asynchronous

Description:	A mobile terminated call is started for alternate speech and non-transparent asynchronous data service. This is bearer service 61. The data service is supported in PCS 1900 range by the PC version. As no explicit configuration of CC via MNCC_CONFIGURE_REQ takes place in this case, the default setting to non-transparent leads to a passing compatibilty check.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	REPEAT_CIRCULAR
	bearer_cap	BC_SPEECH_FR
<A>	bearer_cap_2	BC_BS21_NT_300_1
<B>	bearer_cap_2	BC_BS22_NT_1200
<C>	bearer_cap_2	BC_BS24_NT_2400_V26TER
<D>	bearer_cap_2	BC_BS25_NT_4800
<E>	bearer_cap_2	BC_BS26_NT_9600
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	REPEAT_CIRCULAR
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_BS21_NT_300_1
<B>	bcpara2	BC_PARA_BS22_NT_1200
<C>	bcpara2	BC_PARA_BS24_NT_2400
<D>	bcpara2	BC_PARA_BS25_NT_4800
<E>	bcpara2	BC_PARA_BS26_NT_9600
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC567: MTC, BS81, Sequential Speech / Transparent Asynchronous

Description:	A mobile terminated call is started for sequential speech and transparent asynchronous data service. This is bearer service 81. The data service is supported in PCS 1900 range by the PC version.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC001B

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	REPEAT_SEQUENTIAL
	bearer_cap	BC_SPEECH_FR
<A>	bearer_cap_2	BC_BS21_T_300
<B>	bearer_cap_2	BC_BS22_T_1200
<C>	bearer_cap_2	BC_BS24_T_2400
<D>	bearer_cap_2	BC_BS25_T_4800
<E>	bearer_cap_2	BC_BS26_T_9600
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	REPEAT_SEQUENTIAL
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_BS21_T_300
<B>	bcpara2	BC_PARA_BS22_T_1200
<C>	bcpara2	BC_PARA_BS24_T_2400
<D>	bcpara2	BC_PARA_BS25_T_4800
<E>	bcpara2	BC_PARA_BS26_T_9600
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC568: MTC, BS81, Sequential Speech / Non-Transparent Asynchronous

Description:	A mobile terminated call is started for sequential speech and non-transparent asynchronous data service. This is bearer service 81. The data service is supported in PCS 1900 range by the PC version.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	REPEAT_SEQUENTIAL
	bearer_cap	BC_SPEECH_FR
<A>	bearer_cap_2	BC_BS21_NT_300_1
<B>	bearer_cap_2	BC_BS22_NT_1200
<C>	bearer_cap_2	BC_BS24_NT_2400_V26TER
<D>	bearer_cap_2	BC_BS25_NT_4800
<E>	bearer_cap_2	BC_BS26_NT_9600
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	REPEAT_SEQUENTIAL
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_BS21_NT_300_1
<B>	bcpara2	BC_PARA_BS22_NT_1200
<C>	bcpara2	BC_PARA_BS24_NT_2400
<D>	bcpara2	BC_PARA_BS25_NT_4800
<E>	bcpara2	BC_PARA_BS26_NT_9600
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC569: MTC, TS61, Alternate Speech / transparent Fax

Description:	A mobile terminated call is started for sequential speech and transparent fax service. This is tele service 61. The data service is supported in PCS 1900 range by the PC version.

A:	2400 bit / s
B:	4800 bit / s
C:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	REPEAT_CIRCULAR
	bearer_cap	BC_SPEECH_FR
<A>	bearer_cap_2	BC_FAX_T_2400
<B>	bearer_cap_2	BC_FAX_T_4800
<C>	bearer_cap_2	BC_FAX_T_9600
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	REPEAT_CIRCULAR
	bcpara	BC_PARA_SPEECH
<A>	bcpara2	BC_PARA_FAX_T_2400
<B>	bcpara2	BC_PARA_FAX_T_4800
<C>	bcpara2	BC_PARA_FAX_T_9600
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC570: MTC, TS62, Automatic transparent Fax

Description:	A mobile terminated call is started for automatic transparent fax service. This is tele service 62. The data service is supported in PCS 1900 range by the PC version.

A:	2400 bit / s
B:	4800 bit / s
C:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	NOT_USED
<A>	bearer_cap	BC_FAX_T_2400
<B>	bearer_cap	BC_FAX_T_4800
<C>	bearer_cap	BC_FAX_T_9600
	bearer_cap_2	NOT_USED
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num	DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_USED
<A>	bcpara	BC_PARA_FAX_T_2400
<B>	bcpara	BC_PARA_FAX_T_4800
<C>	bcpara	BC_PARA_FAX_T_9600
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
	bearer_cap	NOT_USED
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC572: MTC, Non-Transparent 14.4 kBit/s accepted

Description:	A mobile terminated call is started for non-transparent (14400 bit/s) in the Dualband Extended range. The PC version supports this in this frequency range. It is expected that the bearer capability is accepted without negotiation. As no explicit configuration of CC via MNCC_CONFIGURE_REQ takes place in this case, the default setting to non-transparent leads to a passing compatibilty check.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS20_NT_14400_1
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS20_NT_14400_1
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC573: MTC, Non-Transparent 14.4 kBit/s negotiated (I)

Description:	A mobile terminated call is started for 14400 bit/s in the Dualband Extended range. The PC version supports this in this frequency range. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, non-transparent preferred. So it is exspected, that the mobile selects non-transparent.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS20_14400_BNTP
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS20_NT_14400_4
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS20_NT_14400_4
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC574: MTC Transparent 14.4 kBit/s negotiated (II)

Description:	A mobile terminated call is started for 14400 bit/s in the Dualband Extended range. The PC version supports this in this frequency range. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element both, transparent preferred. The mobile has been set to transparent, so it is exspected, that it selects transparent.

Preamble:	CC001B

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS20_14400_BTP
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS20_T_14400
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS20_T_14400
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC575: MTC, Non-Transparent 14.4 kBit/s rejected

Description:	A mobile terminated call is started for 14.400 bit/s in the GSM 900 range. The PC version doesn´t support 14.400 for GSM 900. It is expected that the call attempt is rejected.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS20_NT_14400_5
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE_COMP
		ti	TI_MT_1_RESP
		cc_cause	CC_CAUSE_INCOMPAT
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MT_1_RESP

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC576: MOC, Transparent, 14.4 kBit/s

Description:	A mobile originated call is started for transparent asynchronous data with 14.4 kBit/s. The data service is supported in Dualband Extended range by the PC version. No UIMI or WAIUR parameter are to be sent to IWF in case of transparent mode .

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS20_T_14400
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS20_T_14400_1
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	04-Nov-02	SBK	Editorial improvements

### CC577: MOC, Transparent, 14.4 kBit/s, rejected

Description:	A mobile originated call is started for transparent asynchronous data with 14.4 kBit/s. The data service is not supported in GSM 900 range by the PC version. A release is indicated to the upper layer.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MNCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS20_T_14400
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MNCC_RELEASE_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_BEARER_NOT_IMPLEM

History:	23-Jul-98	LE	Initial

### CC578: MOC, Non-Transparent, 14.4 kBit/s

Description:	A mobile originated call is started for non-transparent asynchronous data with 14.4 kBit/s. The data service is supported in Dualband Extended frequency range by the PC version. The indicated flow control is inband flow control.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS20_NT_14400_1
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS20_NT_14400_2
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	23-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC579: Negotiation of Intermediate Rate, 29.2.3.5

Description:	To verify that the MS responds correctly to a request for a negotiation of the intermediate rate parameter in a mobile terminated setup and negotiates according to its capabilities and to the service requested. A mobile terminated call is started for BS25 non-transparent (4800 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. The following different cases are defined
A connection element = non-transparent, NIRR = no meaning, 
B connection element = both, NT preferred, NIRR = no meaning
C connection element = non-transparent, NIRR = 6 kBit/s, 
D connection element = both, NT preferred, NIRR = 6 kBit/s

Preamble:	CC000

Variants:	<A>....<D>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
<A>		bearer_cap	BC_BS25_NT_4800_A
<B>		bearer_cap	BC_BS25_NT_4800_B
<C>		bearer_cap	BC_BS25_NT_4800_C
<D>		bearer_cap	BC_BS25_NT_4800_D
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
	ti	TI_MT_1_RESP
	ri	NOT_PRESENT_8BIT
	bcpara	BC_PARA_BS25_NT_4800
	bcpara2	BC_PARA_NO_SERVICE
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	UPLINK
	pd	U_CALL_CONF
	ti	TI_MT_1_RESP
	repeat	NOT_USED
<A>	bearer_cap	NOT_USED
<B>	bearer_cap	BC_BS25_NT_4800_A2
<C>	bearer_cap	NOT_USED
<D>	bearer_cap	BC_BS25_NT_4800_C
	bearer_cap_2	NOT_USED
	cc_cause	NOT_USED
	}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target

### CC580: MTC, TS61, Alternate Speech / transparent Fax, SNS=5, with BC

Description:	A mobile terminated call is started for sequential speech and transparent fax service. This is tele service 61. The data service is supported in PCS 1900 range by the PC version. The Single numbering scheme is configured for mode 5, that means alternating voice / fax, fax first. It is expected that CC swaps the two bearer capabilities.

A:	2400 bit / s
B:	4800 bit / s
C:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
	called_party_sub	NOT_USED
<A>	bcpara	BC_PARA_FAX_T_2400
<B>	bcpara	BC_PARA_FAX_T_4800
<C>	bcpara	BC_PARA_FAX_T_9600
	sns_mode	SNS_MODE_VAF_FAX
	ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
	d1	NOT_USED
	d2	NOT_USED
	sdu
	{
	component	CC
	direction	DOWNLINK
	pd	D_SETUP
	ti	TI_MT_1
	repeat	REPEAT_CIRCULAR
	bearer_cap	BC_SPEECH_FR
<A>	bearer_cap_2	BC_FAX_T_2400
<B>	bearer_cap_2	BC_FAX_T_4800
<C>	bearer_cap_2	BC_FAX_T_9600
	facility	NOT_USED
	progress	PROG_1
	signal	SIGNAL_DIAL_ON
	calling_num	CLNG_NUM_654321
	calling_subaddr	NOT_USED
	dl_called_num DL_CLED_NUM_654321
	called_subaddr	NOT_USED
	redirecting_num	NOT_USED
	redirecting_subaddr	NOT_USED
	repeat_2	NOT_USED
	low_layer_comp	NOT_USED
	low_layer_comp_2	NOT_USED
	repeat_3	NOT_USED
	high_layer_comp	NOT_USED
	high_layer_comp_2	NOT_USED
	user_user	NOT_USED
	}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
<A>	bcpara	BC_PARA_FAX_T_2400
<B>	bcpara	BC_PARA_FAX_T_4800
<C>	bcpara	BC_PARA_FAX_T_9600
	bcpara2	BC_PARA_SPEECH
	progress_desc	PROG_END_TO_END_PLMN
	sig	SIGNAL_DIAL_ON
	calling_party	CLNG_PARTY_654321
	calling_party_sub	NOT_USED
	called_party	CLED_PARTY_654321
	called_party_sub	NOT_USED
	redirecting_party	REDIR_PARTY_NONE
	redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
<A>		bearer_cap	BC_FAX_T_2400
<B>		bearer_cap	BC_FAX_T_4800
<C>		bearer_cap	BC_FAX_T_9600
		bearer_cap_2	BC_SPEECH_FR_HR_EFR
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	25-Jan-00	HM	BC_SPEECH_FR_HR_EFR
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC581: MTC, TS61, Alternate Speech / transparent Fax, SNS=5, no BC

Description:	A mobile terminated call is started without bearer capabilitiesThe Single numbering scheme is configured for mode 5, that means alternating voice / fax, fax first. It is expected that CC inidicates alternating speech / fax, fax first.

A:	2400 bit / s
B:	4800 bit / s
C:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_FAX_T_2400
<B>		bcpara	BC_PARA_FAX_T_4800
<C>		bcpara	BC_PARA_FAX_T_9600
		sns_mode	SNS_MODE_VAF_FAX
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
<A>		bcpara	BC_PARA_FAX_T_2400
<B>		bcpara	BC_PARA_FAX_T_4800
<C>		bcpara	BC_PARA_FAX_T_9600
		bcpara2	BC_PARA_SPEECH
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
<A>		bearer_cap	BC_FAX_T_2400
<B>		bearer_cap	BC_FAX_T_4800
<C>		bearer_cap	BC_FAX_T_9600
		bearer_cap_2	BC_SPEECH_FR_HR_EFR
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC582: MTC, no Bearer Capabilities, accepted, SNS=0

Description:	The single numbering scheme mode 0 is configured. A setup message receives without bearer capabilities. It is expected that call control accepts the call and responds with a voice bearer capability in the call proceeding message.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
		bcpara	BC_PARA_SPEECH
		sns_mode	SNS_MODE_VOICE
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_SPEECH
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_SPEECH_FR
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	24-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC583: MTC, TS61, Alternate Speech / transparent Fax, SNS=1, no BC

Description:	A mobile terminated call is started without bearer caps. The pre-defined single numbering scheme is alternating speech and transparent fax service, voice first.

A:	2400 bit / s
B:	4800 bit / s
C:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_FAX_T_2400
<B>		bcpara	BC_PARA_FAX_T_4800
<C>		bcpara	BC_PARA_FAX_T_9600
		sns_mode	SNS_MODE_VAF_VOICE
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
		bcpara	BC_PARA_SPEECH
<A>		bcpara2	BC_PARA_FAX_T_2400
<B>		bcpara2	BC_PARA_FAX_T_4800
<C>		bcpara2	BC_PARA_FAX_T_9600
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
		bearer_cap	BC_SPEECH_FR_HR_EFR
<A>		bearer_cap_2	BC_FAX_T_2400
<B>		bearer_cap_2	BC_FAX_T_4800
<C>		bearer_cap_2	BC_FAX_T_9600
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC584: MTC, TS61, Alternate Speech / transparent Fax, SNS=1, with BC

Description:	A mobile terminated call is started with alternating speech / transparent fax, fax first. The pre-defined single numbering scheme is alternating speech and transparent fax service, voice first. It is expected that call control swaps the bearer capabilities.

A:	2400 bit / s
B:	4800 bit / s
C:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_FAX_T_2400
<B>		bcpara	BC_PARA_FAX_T_4800
<C>		bcpara	BC_PARA_FAX_T_9600
		sns_mode	SNS_MODE_VAF_VOICE
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	REPEAT_CIRCULAR
<A>		bearer_cap	BC_FAX_T_2400
<B>		bearer_cap	BC_FAX_T_4800
<C>		bearer_cap	BC_FAX_T_9600
		bearer_cap_2	BC_SPEECH_FR
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
		bcpara	BC_PARA_SPEECH
<A>		bcpara2	BC_PARA_FAX_T_2400
<B>		bcpara2	BC_PARA_FAX_T_4800
<C>		bcpara2	BC_PARA_FAX_T_9600
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
		bearer_cap	BC_SPEECH_FR_HR_EFR
<A>		bearer_cap_2	BC_FAX_T_2400
<B>		bearer_cap_2	BC_FAX_T_4800
<C>		bearer_cap_2	BC_FAX_T_9600
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	16-Mar-00	HM	CALL CONF with all capabilities
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC585: MTC, TS62, Automatic transparent Fax, SNS=2, no BCs

Description:	A mobile terminated call is started for automatic transparent fax service. This is tele service 62. The data service is supported in PCS 1900 range by the PC version.

A:	2400 bit / s
B:	4800 bit / s
C:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_FAX_T_2400
<B>		bcpara	BC_PARA_FAX_T_4800
<C>		bcpara	BC_PARA_FAX_T_9600
		sns_mode	SNS_MODE_FAX
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_USED
<A>		bcpara	BC_PARA_FAX_T_2400
<B>		bcpara	BC_PARA_FAX_T_4800
<C>		bcpara	BC_PARA_FAX_T_9600
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
<A>		bearer_cap	BC_FAX_T_2400
<B>		bearer_cap	BC_FAX_T_4800
<C>		bearer_cap	BC_FAX_T_9600
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC586: MTC, BS61, Alternate Speech / Transparent Asynchronous, SNS=3, no BC

Description:	The predefined single numbering is alternating voice / data, voice first. A mobile terminated call is started without bearer capabilities.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_T_300
<B>		bcpara	BC_PARA_BS22_T_1200
<C>		bcpara	BC_PARA_BS24_T_2400
<D>		bcpara	BC_PARA_BS25_T_4800
<E>		bcpara	BC_PARA_BS26_T_9600
		sns_mode	SNS_MODE_VAD_VOICE
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
		bcpara	BC_PARA_SPEECH
<A>		bcpara2	BC_PARA_BS21_T_300
<B>		bcpara2	BC_PARA_BS22_T_1200
<C>		bcpara2	BC_PARA_BS24_T_2400
<D>		bcpara2	BC_PARA_BS25_T_4800
<E>		bcpara2	BC_PARA_BS26_T_9600
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
		bearer_cap	BC_SPEECH_FR_HR_EFR
<A>		bearer_cap_2	BC_BS21_T_300
<B>		bearer_cap_2	BC_BS22_T_1200
<C>		bearer_cap_2	BC_BS24_T_2400
<D>		bearer_cap_2	BC_BS25_T_4800
<E>		bearer_cap_2	BC_BS26_T_9600
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC587: MTC, BS61, Alternate Speech / Transparent Asynch, SNS=3, with BC

Description:	The predefined single numbering is alternating voice / data, voice first. A mobile terminated call is started with bearer capabilities. It is expected that call control swaps the incoming bearer capabilities.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_T_300
<B>		bcpara	BC_PARA_BS22_T_1200
<C>		bcpara	BC_PARA_BS24_T_2400
<D>		bcpara	BC_PARA_BS25_T_4800
<E>		bcpara	BC_PARA_BS26_T_9600
		sns_mode	SNS_MODE_VAD_VOICE
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	REPEAT_CIRCULAR
<A>		bearer_cap	BC_BS21_T_300
<B>		bearer_cap	BC_BS22_T_1200
<C>		bearer_cap	BC_BS24_T_2400
<D>		bearer_cap	BC_BS25_T_4800
<E>		bearer_cap	BC_BS26_T_9600
		bearer_cap_2	BC_SPEECH_FR
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
		bcpara	BC_PARA_SPEECH
<A>		bcpara2	BC_PARA_BS21_T_300
<B>		bcpara2	BC_PARA_BS22_T_1200
<C>		bcpara2	BC_PARA_BS24_T_2400
<D>		bcpara2	BC_PARA_BS25_T_4800
<E>		bcpara2	BC_PARA_BS26_T_9600
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
		bearer_cap	BC_SPEECH_FR_HR_EFR
<A>		bearer_cap_2	BC_BS21_T_300
<B>		bearer_cap_2	BC_BS22_T_1200
<C>		bearer_cap_2	BC_BS24_T_2400
<D>		bearer_cap_2	BC_BS25_T_4800
<E>		bearer_cap_2	BC_BS26_T_9600
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	16-Oct-00	HM	Changed BC in CALL CONFIRMED
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC588: MTC, BS61, Alternate Speech / Non-Transparent Asynch, SNS=3, no BCs

Description:	The predefined single numbering is alternating voice / data, voice first. A mobile terminated call is started without bearer capabilities.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_NT_300_1
<B>		bcpara	BC_PARA_BS22_NT_1200
<C>		bcpara	BC_PARA_BS24_NT_2400
<D>		bcpara	BC_PARA_BS25_NT_4800
<E>		bcpara	BC_PARA_BS26_NT_9600
		sns_mode	SNS_MODE_VAD_VOICE
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
		bcpara	BC_PARA_SPEECH
<A>		bcpara2	BC_PARA_BS21_NT_300_1
<B>		bcpara2	BC_PARA_BS22_NT_1200
<C>		bcpara2	BC_PARA_BS24_NT_2400_V22BIS
<D>		bcpara2	BC_PARA_BS25_NT_4800
<E>		bcpara2	BC_PARA_BS26_NT_9600
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
		bearer_cap	BC_SPEECH_FR_HR_EFR
<A>		bearer_cap_2	BC_BS21_NT_300_1_RSP
<B>		bearer_cap_2	BC_BS22_NT_1200_RSP
<C>		bearer_cap_2	BC_BS24_NT_2400_RSP
<D>		bearer_cap_2	BC_BS25_NT_4800_RSP
<E>		bearer_cap_2	BC_BS26_NT_9600_OUTBAND
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC589: MTC, BS61, Alternate Speech / Non-Transp Asynch, SNS=3, with BCs

Description:	The predefined single numbering is alternating voice / data, voice first. A mobile terminated call is started with bearer capabilities. It is expected that call control swaps the incoming bearer capabilities.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_NT_300_1
<B>		bcpara	BC_PARA_BS22_NT_1200
<C>		bcpara	BC_PARA_BS24_NT_2400
<D>		bcpara	BC_PARA_BS25_NT_4800
<E>		bcpara	BC_PARA_BS26_NT_9600
		sns_mode	SNS_MODE_VAD_VOICE
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	REPEAT_CIRCULAR
<A>	bearer_cap	BC_BS21_NT_300_1
<B>	bearer_cap	BC_BS22_NT_1200
<C>	bearer_cap	BC_BS24_NT_2400_V26TER
<D>	bearer_cap	BC_BS25_NT_4800
<E>	bearer_cap	BC_BS26_NT_9600
		bearer_cap_2	BC_SPEECH_FR
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
		bcpara	BC_PARA_SPEECH
<A>		bcpara2	BC_PARA_BS21_NT_300_1
<B>		bcpara2	BC_PARA_BS22_NT_1200
<C>		bcpara2	BC_PARA_BS24_NT_2400
<D>		bcpara2	BC_PARA_BS25_NT_4800
<E>		bcpara2	BC_PARA_BS26_NT_9600
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
		bearer_cap	BC_SPEECH_FR_HR
<A>		bearer_cap_2	BC_BS21_NT_300_1
<B>		bearer_cap_2	BC_BS22_NT_1200
<C>		bearer_cap_2	BC_BS24_NT_2400_V26TER
<D>		bearer_cap_2	BC_BS25_NT_4800
<E>		bearer_cap_2	BC_BS26_NT_9600
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC590: MTC, BS24, 3.1 kHz Audio Transparent 2.4 kBit/s,SNS=4, no BCs

Description:	The predefined single numbering is data. A mobile terminated call is started without bearer capabilities.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
		bcpara	BC_PARA_BS24_T_2400
		sns_mode	SNS_MODE_DATA
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS24_T_2400
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS24_T_2400
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC591: MTC, BS61, Alternate Speech / Transparent Asynchronous, SNS=6, no BC

Description:	The predefined single numbering is alternating voice / data, data first. A mobile terminated call is started without bearer capabilities.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_T_300
<B>		bcpara	BC_PARA_BS22_T_1200
<C>		bcpara	BC_PARA_BS24_T_2400
<D>		bcpara	BC_PARA_BS25_T_4800
<E>		bcpara	BC_PARA_BS26_T_9600
		sns_mode	SNS_MODE_VAD_DATA
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
<A>		bcpara	BC_PARA_BS21_T_300
<B>		bcpara	BC_PARA_BS22_T_1200
<C>		bcpara	BC_PARA_BS24_T_2400
<D>		bcpara	BC_PARA_BS25_T_4800
<E>		bcpara	BC_PARA_BS26_T_9600
		bcpara2	BC_PARA_SPEECH
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
<A>		bearer_cap	BC_BS21_T_300
<B>		bearer_cap	BC_BS22_T_1200
<C>		bearer_cap	BC_BS24_T_2400
<D>		bearer_cap	BC_BS25_T_4800
<E>		bearer_cap	BC_BS26_T_9600
		bearer_cap_2	BC_SPEECH_FR_HR_EFR
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC592: MTC, BS61, Alternate Speech / Transparent Asynch, SNS=6, with BC

Description:	The predefined single numbering is alternating voice / data, data first. A mobile terminated call is started with bearer capabilities. It is expected that call control swaps the incoming bearer capabilities.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_T_300
<B>		bcpara	BC_PARA_BS22_T_1200
<C>		bcpara	BC_PARA_BS24_T_2400
<D>		bcpara	BC_PARA_BS25_T_4800
<E>		bcpara	BC_PARA_BS26_T_9600
		sns_mode	SNS_MODE_VAD_DATA
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	REPEAT_CIRCULAR
		bearer_cap	BC_SPEECH_FR
<A>		bearer_cap_2	BC_BS21_T_300
<B>		bearer_cap_2	BC_BS22_T_1200
<C>		bearer_cap_2	BC_BS24_T_2400
<D>		bearer_cap_2	BC_BS25_T_4800
<E>		bearer_cap_2	BC_BS26_T_9600
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
<A>		bcpara	BC_PARA_BS21_T_300
<B>		bcpara	BC_PARA_BS22_T_1200
<C>		bcpara	BC_PARA_BS24_T_2400
<D>		bcpara	BC_PARA_BS25_T_4800
<E>		bcpara	BC_PARA_BS26_T_9600
		bcpara2	BC_PARA_SPEECH
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
<A>		bearer_cap	BC_BS21_T_300
<B>		bearer_cap	BC_BS22_T_1200
<C>		bearer_cap	BC_BS24_T_2400
<D>		bearer_cap	BC_BS25_T_4800
<E>		bearer_cap	BC_BS26_T_9600
		bearer_cap_2	BC_SPEECH_FR_HR_EFR
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	25-Jan-00	HM	BC_SPEECH_FR_HR_EFR
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC593: MTC, BS61, Alternate Speech / Non-Transparent Asynch, SNS=6, no BCs

Description:	The predefined single numbering is alternating voice / data, data first. A mobile terminated call is started without bearer capabilities.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_NT_300_1
<B>		bcpara	BC_PARA_BS22_NT_1200
<C>		bcpara	BC_PARA_BS24_NT_2400
<D>		bcpara	BC_PARA_BS25_NT_4800
<E>		bcpara	BC_PARA_BS26_NT_9600
		sns_mode	SNS_MODE_VAD_DATA
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
<A>		bcpara	BC_PARA_BS21_NT_300_1
<B>		bcpara	BC_PARA_BS22_NT_1200
<C>		bcpara	BC_PARA_BS24_NT_2400_V22BIS
<D>		bcpara	BC_PARA_BS25_NT_4800
<E>		bcpara	BC_PARA_BS26_NT_9600
		bcpara2	BC_PARA_SPEECH
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
<A>		bearer_cap	BC_BS21_NT_300_1_RSP
<B>		bearer_cap	BC_BS22_NT_1200_RSP
<C>		bearer_cap	BC_BS24_NT_2400_RSP
<D>		bearer_cap	BC_BS25_NT_4800_RSP
<E>		bearer_cap	BC_BS26_NT_9600_OUTBAND
		bearer_cap_2	BC_SPEECH_FR_HR_EFR
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC594: MTC, BS61, Alternate Speech / Non-Transp Asynch, SNS=6, with BCs

Description:	The predefined single numbering is alternating voice / data, data first. A mobile terminated call is started with bearer capabilities. It is expected that call control swaps the incoming bearer capabilities.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_NT_300_1
<B>		bcpara	BC_PARA_BS22_NT_1200
<C>		bcpara	BC_PARA_BS24_NT_2400
<D>		bcpara	BC_PARA_BS25_NT_4800
<E>		bcpara	BC_PARA_BS26_NT_9600
		sns_mode	SNS_MODE_VAD_DATA
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	REPEAT_CIRCULAR
		bearer_cap	BC_SPEECH_FR
<A>		bearer_cap_2	BC_BS21_NT_300_1
<B>		bearer_cap_2	BC_BS22_NT_1200
<C>		bearer_cap_2	BC_BS24_NT_2400_V26TER
<D>		bearer_cap_2	BC_BS25_NT_4800
<E>		bearer_cap_2	BC_BS26_NT_9600
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_CIRCULAR
<A>		bcpara	BC_PARA_BS21_NT_300_1
<B>		bcpara	BC_PARA_BS22_NT_1200
<C>		bcpara	BC_PARA_BS24_NT_2400
<D>		bcpara	BC_PARA_BS25_NT_4800
<E>		bcpara	BC_PARA_BS26_NT_9600
		bcpara2	BC_PARA_SPEECH
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_CIRCULAR
<A>		bearer_cap	BC_BS21_NT_300_1
<B>		bearer_cap	BC_BS22_NT_1200
<C>		bearer_cap	BC_BS24_NT_2400_V26TER
<D>		bearer_cap	BC_BS25_NT_4800
<E>		bearer_cap	BC_BS26_NT_9600
		bearer_cap_2	BC_SPEECH_FR_HR_EFR
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	25-Jan-00	HM	BC_SPEECH_FR_HR_EFR
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC595: Sequential Speech / Transparent Asynchronous, SNS=7, no BC

Description:	The predefined single numbering is sequential voice / data. A mobile terminated call is started without bearer capabilities.

A:	300 bit / s
B:	1200 bit / s
C:	2400 bit / s
D:	4800 bit / s
E:	9600 bit / s

Preamble:	CC000

Variants:	<A> ... <E>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
<A>		bcpara	BC_PARA_BS21_T_300
<B>		bcpara	BC_PARA_BS22_T_1200
<C>		bcpara	BC_PARA_BS24_T_2400
<D>		bcpara	BC_PARA_BS25_T_4800
<E>		bcpara	BC_PARA_BS26_T_9600
		sns_mode	SNS_MODE_VFD
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	REPEAT_SEQUENTIAL
		bcpara	BC_PARA_SPEECH
<A>		bcpara2	BC_PARA_BS21_T_300
<B>		bcpara2	BC_PARA_BS22_T_1200
<C>		bcpara2	BC_PARA_BS24_T_2400
<D>		bcpara2	BC_PARA_BS25_T_4800
<E>		bcpara2	BC_PARA_BS26_T_9600
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	REPEAT_SEQUENTIAL
		bearer_cap	BC_SPEECH_FR_HR_EFR
<A>		bearer_cap_2	BC_BS21_T_300
<B>		bearer_cap_2	BC_BS22_T_1200
<C>		bearer_cap_2	BC_BS24_T_2400
<D>		bearer_cap_2	BC_BS25_T_4800
<E>		bearer_cap_2	BC_BS26_T_9600
		cc_cause	NOT_USED
		}

History:	29-Jul-98	LE	Initial
	10-Jul-01	JHU	Converted to TAP2
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

### CC596: MTC Transparent 14.4 kBit/s negotiated (III)

Description:	A mobile terminated call is started for 14400 bit/s in the Dualband Extended range. The PC version supports this in this frequency range. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element both, transparent preferred. The mobile has been set to transparent, so it is exspected, that it selects transparent. Three cases are created where eather ‘other_modem_type_2’ or ‘fnur’ or both paramters are lacking in BC received from MM. Hence CC includes no 6d, 6e, 6f octets in the BCs sent in the CALL CORFIRMED massage to MM in return.

Preamble:	CC001B

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(2)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
<A>	bearer_cap	BC_BS20_14400_BTP_1
<B>	bearer_cap	BC_BS20_14400_BTP_2
<C>	bearer_cap	BC_BS20_14400_BTP_3
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS26_T_9600D
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS20_T_14400_3
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		call_ctrl_cap	NOT_USED
		}

History:	13-July-02	JK	Initial

### CC600: Bearer capability coding

Description:	The ACI requests coding of bearer capabilities. This may be done in any state. The mobile station is configurated for HR, FR and EFR support.

A:	One valid bearer capability
B:	Two valid bearer capabilities
C:	Second bearer capability invalid

Preamble:	CC000

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_BEARER_CAP_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	MNCC_BEARER_CAP_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_BEARER_CAP_REQ
		req_id	REQ_ID_5
		bc_mod	BC_MOD_CODE
<A>		bcconf	NOT_USED
<A>		bcpara	BC_PARA_SPEECH
<A>		bcconf2	BC_CONF_NO_SERVICE
<A>		bcpara2	BC_PARA_NO_SERVICE
<B>		bcconf	NOT_USED
<B>		bcpara	BC_PARA_SPEECH
<B>		bcconf2	NOT_USED
<B>		bcpara2	BC_PARA_BS21_T_300
<C>		bcconf	NOT_USED
<C>		bcpara	BC_PARA_SPEECH
<C>		bcconf2	NOT_USED
<C>		bcpara2	BC_PARA_UNSUPP

- MNCC_BEARER_CAP_CNF
		req_id	REQ_ID_5
		bc_mod	BC_MOD_CODE
<A>		bcconf	BC_CONF_SPEECH_FR_HR_EFR
<A>		bcpara	BC_PARA_SPEECH
<A>		bcconf2	BC_CONF_NO_SERVICE
<A>		bcpara2	BC_PARA_NO_SERVICE
<A>		cause	MNCC_CAUSE_SUCCESS
<B>		bcconf	BC_CONF_SPEECH_FR_HR_EFR
<B>		bcpara	BC_PARA_SPEECH
<B>		bcconf2	BC_CONF_BS21_T_300
<B>		bcpara2	BC_PARA_BS21_T_300
<B>		cause	MNCC_CAUSE_SUCCESS
<C>		bcconf	BC_CONF_SPEECH_FR_HR_EFR
<C>		bcpara	BC_PARA_SPEECH
<C>		bcconf2	BC_CONF_NO_SERVICE
<C>		bcpara2	BC_PARA_UNSUPP
<C>		cause	MNCC_CAUSE_MS_INCOMPAT_DEST

History:	14-Mar-00	HM	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC601: Bearer capability decoding

Description:	The ACI requests decoding of bearer capabilities. This may be done in any state. The mobile station is configurated for HR, FR and EFR support.

A:	One valid bearer capability parameter
B:	Two valid bearer capability parameters
C:	Second bearer capability invalid (not supported yet)

Preamble:	CC001B

Variants:	<A> ... <C>

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_BEARER_CAP_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	MNCC_BEARER_CAP_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_BEARER_CAP_REQ
		req_id	REQ_ID_5
		bc_mod	BC_MOD_DECODE
<A>		bcconf	BC_CONF_SPEECH_FR_HR_EFR
<A>		bcpara	NOT_USED
<A>		bcconf2	BC_CONF_NO_SERVICE
<A>		bcpara2	NOT_USED
<B>		bcconf	BC_CONF_SPEECH_FR_HR_EFR
<B>		bcpara	NOT_USED
<B>		bcconf2	BC_CONF_BS21_T_300
<B>		bcpara2	NOT_USED
<C>		bcconf	BC_CONF_SPEECH_FR_HR_EFR
<C>		bcpara	NOT_USED
<C>		bcconf2	BC_CONF_PACKET_UNSUPP
<C>		bcpara2	NOT_USED

- MNCC_BEARER_CAP_CNF
		req_id	REQ_ID_5
		bc_mod	BC_MOD_DECODE
<A>		bcconf	BC_CONF_SPEECH_FR_HR_EFR
<A>		bcpara	BC_PARA_SPEECH
<A>		bcconf2	BC_CONF_NO_SERVICE
<A>		bcpara2	BC_PARA_NO_SERVICE
<A>		cause	MNCC_CAUSE_SUCCESS
<B>		bcconf	BC_CONF_SPEECH_FR_HR_EFR
<B>		bcpara	BC_PARA_SPEECH
<B>		bcconf2	BC_CONF_BS21_T_300
<B>		bcpara2	BC_PARA_BS21_T_300
<B>		cause	MNCC_CAUSE_SUCCESS
<C>		bcconf	BC_CONF_SPEECH_FR_HR_EFR
<C>		bcpara	BC_PARA_SPEECH
<C>		bcconf2	BC_CONF_PACKET_UNSUPP
<C>		bcpara2	BC_PARA_NO_SERVICE
<C>		cause	MNCC_CAUSE_MS_INCOMPAT_DEST

History:	14-Mar-00	HM	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC602: Bearer capability decoding, successfull

Description:	The ACI requests decoding of bearer capabilities. This may be done in any state. The bearer capabilities are indicating HR, FR and EFR. The mobile station is configurated for FR support only. Successfull conversion is expected, the call will be downgraded.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_BEARER_CAP_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	MNCC_BEARER_CAP_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_BEARER_CAP_REQ
		req_id	REQ_ID_5
		bc_mod	BC_MOD_DECODE
		bcconf	BC_CONF_SPEECH_FR_HR_EFR
		bcpara	NOT_USED
		bcconf2	BC_CONF_NO_SERVICE
		bcpara2	NOT_USED

- MNCC_BEARER_CAP_CNF
		req_id	REQ_ID_5
		bc_mod	BC_MOD_DECODE
		bcconf	BC_CONF_SPEECH_FR_HR_EFR
		bcpara	BC_PARA_SPEECH
		bcconf2	BC_CONF_NO_SERVICE
		bcpara2	BC_PARA_NO_SERVICE
		cause	MNCC_CAUSE_SUCCESS

History:	14-Mar-00	HM	Initial
	10-Jul-01	JHU	Converted to TAP2

## DTMF Handling

### CC610: Custumer reported problem with DTMF with manual DTMF start/stop

Description:	The ACI requests sending of DTMF tones in a very fast manner. The buffer in CC has a buffer overflow. This caused drop of a STOP DTMF request and thus sending of two START DTMF requests in sequence. This is incorrectly and caused some networks not to place the call on hold. The code was changed in a way that it is ensured that no START DTMF is written to the buffer if it cannot be ensured that a STOP DTMF can be written. Also it is ensured that no two STOP DTMF requests can be written in sequence. To test this, DTMF_BUF_SIZE has to be decreased to 3, which means there will be room for 2 elements in the buffer. Otherwise the test will fail and this is perfectly ok. All dropped primitives are marked by a "*" in the paramter.

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_START)	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (START DTMF)	  |
	 |		  *==============================>*
(3)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP)	  |		  |
	 *==============================>*		  |
(4)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_START *)	  |		  |
	 *==============================>*		  |
(5)	 |	MNCC_START_DTMF_CNF	  |		  |
	 |  (cause: DTMF_BUFFER_FULL)	  |		  |
	 *<==============================*		  |
(6)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP *)	  |		  |
	 *==============================>*		  |
(7)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (START DTMF ACK)	  |
	 |		  *<==============================*
(8)	 |	MNCC_START_DTMF_CNF	  |		  |
	 *<==============================*		  |
(9)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STOP DTMF)	  |
	 |		  *==============================>*
(10)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_START)	  |		  |
	 *==============================>*		  |
(11)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP)	  |		  |
	 *==============================>*		  |
(12)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP *)	  |		  |
	 *==============================>*		  |
(13)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STOP DTMF ACK)	  |
	 |		  *<==============================*
(14)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (START DTMF)	  |
	 |		  *==============================>*
(15)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (START DTMF ACK)	  |
	 |		  *<==============================*
(16)	 |	MNCC_START_DTMF_CNF	  |		  |
	 *<==============================*		  |
(17)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STOP DTMF)	  |
	 |		  *==============================>*
(18)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STOP DTMF ACK)	  |
	 |		  *<==============================*
MUTE (3000)
(19)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_START)	  |		  |
	 *==============================>*		  |
(20)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (START DTMF)	  |
	 |		  *==============================>*
(21)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (START DTMF ACK)	  |
	 |		  *<==============================*
(22)	 |	MNCC_START_DTMF_CNF	  |		  |
	 *<==============================*		  |
MUTE (3000)
(23)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP)	  |		  |
	 *==============================>*		  |
(24)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STOP DTMF)	  |
	 |		  *==============================>*
(25)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STOP DTMF ACK)	  |
	 |		  *<==============================*
MUTE (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_START

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_START_DTMF
		ti	TI_MO_5
		key_facility	KEY_FACILITY_9
		}

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_START

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_DTMF_BUFFER_FULL
		dtmf_mod	DTMF_MOD_MAN_START

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_START_DTMF_ACK
		ti	TI_MO_5_RESP
		key_facility	KEY_FACILITY_9
		}

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_DTMF_START_SUCCESS
		dtmf_mod	DTMF_MOD_MAN_START

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_STOP_DTMF
		ti	TI_MO_5
		}

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_START

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_STOP_DTMF_ACK
		ti	TI_MO_5_RESP
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_START_DTMF
		ti	TI_MO_5
		key_facility	KEY_FACILITY_9
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_START_DTMF_ACK
		ti	TI_MO_5_RESP
		key_facility	KEY_FACILITY_9
		}

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_DTMF_START_SUCCESS
		dtmf_mod	DTMF_MOD_MAN_START

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_STOP_DTMF
		ti	TI_MO_5
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_STOP_DTMF_ACK
		ti	TI_MO_5_RESP
		}

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_START

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_START_DTMF
		ti	TI_MO_5
		key_facility	KEY_FACILITY_9
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_START_DTMF_ACK
		ti	TI_MO_5_RESP
		key_facility	KEY_FACILITY_9
		}

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_DTMF_START_SUCCESS
		dtmf_mod	DTMF_MOD_MAN_START

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_STOP_DTMF
		ti	TI_MO_5
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_STOP_DTMF_ACK
		ti	TI_MO_5_RESP
		}

History:	05-Apr-00	HM	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC611: Custumer reported problem with DTMF in far east country

Description:	The ACI requests sending of DTMF tones. The network doesn't send acknowledgements sometimes. After timeout of either T336 or T337 the request is forgotten and a new request can be started.

Preamble:	CC032

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_START)	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (START DTMF)	  |
	 |		  *==============================>*
(3)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP)	  |		  |
	 *==============================>*		  |
(4)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_START *)	  |		  |
	 *==============================>*		  |
(5)	 |	MNCC_START_DTMF_CNF	  |		  |
	 |  (cause: DTMF_BUFFER_FULL)	  |		  |
	 *<==============================*		  |
(6)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP *)	  |		  |
	 *==============================>*		  |
MUTE (9000)
(7)	 |	MNCC_START_DTMF_CNF	  |		  |
	 *<==============================*		  |
(8)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_START)	  |		  |
	 *==============================>*		  |
(9)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP)	  |		  |
	 *==============================>*		  |
(10)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP *)	  |		  |
	 *==============================>*		  |
(11)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (START DTMF)	  |
	 |		  *==============================>*
(12)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (START DTMF ACK)	  |
	 |		  *<==============================*
(13)	 |	MNCC_START_DTMF_CNF	  |		  |
	 *<==============================*		  |
(14)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STOP DTMF)	  |
	 |		  *==============================>*
MUTE (9000)
(15)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_START)	  |		  |
	 *==============================>*		  |
(16)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (START DTMF)	  |
	 |		  *==============================>*
(17)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (START DTMF ACK)	  |
	 |		  *<==============================*
(18)	 |	MNCC_START_DTMF_CNF	  |		  |
	 *<==============================*		  |
MUTE (3000)
(19)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_STOP)	  |		  |
	 *==============================>*		  |
(20)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (STOP DTMF)	  |
	 |		  *==============================>*
(21)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (STOP DTMF ACK)	  |
	 |		  *<==============================*
MUTE (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_START

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_START_DTMF
		ti	TI_MO_5
		key_facility	KEY_FACILITY_9
		}

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_START

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_DTMF_BUFFER_FULL

		dtmf_mod	DTMF_MOD_MAN_START

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_MS_TIMER
		dtmf_mod	DTMF_MOD_MAN_START

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_START

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_START_DTMF
		ti	TI_MO_5
		key_facility	KEY_FACILITY_9
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_START_DTMF_ACK
		ti	TI_MO_5_RESP
		key_facility	KEY_FACILITY_9
		}

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_DTMF_START_SUCCESS
		dtmf_mod	DTMF_MOD_MAN_START

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_STOP_DTMF
		ti	TI_MO_5
		}

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_START

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_START_DTMF
		ti	TI_MO_5
		key_facility	KEY_FACILITY_9
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_START_DTMF_ACK
		ti	TI_MO_5_RESP
		key_facility	KEY_FACILITY_9
		}

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_DTMF_START_SUCCESS
		dtmf_mod	DTMF_MOD_MAN_START

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_STOP

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_STOP_DTMF
		ti	TI_MO_5
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_STOP_DTMF_ACK
		ti	TI_MO_5_RESP
		}

History:	27-Jun-00	HM	Initial
	10-Jul-01	JHU	Converted to TAP2
	14-Dec-01	SBK	Substituted timeout by mute properly.

### CC612: DTMF start attempt in state U3

Description:	The ACI requests sending of DTMF tones in state U3. It is a speech call, the TCH is assigned and the progress indicator IE has been received in the CALL PROCEEDING message. It is assumend DTMF is allowed in this state as the conditions of GSM 04.08 subclause 5.5.7 are met. Timer T310 is running, after DTMF START REQUEST sent also T336 in parallel. As no stop conditons for either timer will be met, both timers will timeout. This testcase tests two things: First, whether DTMF works in state U3 as desired, and second whether the parallel operation of timers in the call control entity is working at all.

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MNCC_SYNC_IND	  |		  |
	 *<==============================*		  |
(3)	 |	MNCC_START_DTMF_REQ	  |		  |
	 |	(DTMF_MODE_MAN_START)	  |		  |
	 *==============================>*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (START DTMF)	  |
	 |		  *==============================>*
MUTE (9000)
(5)	 |	MNCC_START_DTMF_CNF	  |		  |
	 |	(cause: CAUSE_MS_TIMER)	  |		  |
	 *<==============================*		  |
MUTE (18000)
(6)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *==============================>*
(7)	 |	MNCC_SETUP_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_SYNC_IND
		ti	TI_MO_5
		chm	CH_SPEECH_FULL

- MNCC_SYNC_IND
		ti	TI_MO_5
		cause	MNCC_CAUSE_CHANNEL_SYNC
		chm	CH_SPEECH_FULL

- MNCC_START_DTMF_REQ
		ti	TI_MO_5
		key	DIG_9
		dtmf_mod	DTMF_MOD_MAN_START

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_START_DTMF
		ti	TI_MO_5
		key_facility	KEY_FACILITY_9
		}

- MNCC_START_DTMF_CNF
		ti	TI_MO_5
		key	DIG_9
		cause	MNCC_CAUSE_MS_TIMER
		dtmf_mod	DTMF_MOD_MAN_START

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_DISCONNECT
		ti	TI_MO_5
		cc_cause	CC_CAUSE_TIMER_310
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MNCC_SETUP_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_MS_TIMER
		progress_desc	NOT_USED
		connected_number	NOT_USED
		connected_number_sub	NOT_USED

History:	12-Oct-00	HM	Initial
	10-Jul-01	JHU	Converted to TAP2
	14-Dec-01	SBK	Substituted timeout by mute properly.
	26-Nov-01	OT	Variant CC026A instead of CC026

## Diconnect with poor network

### CC620: DISCONNECT in state U1 with poor network

Description:	While in the state CALL INIT (U1) CC receives a DISCONNECT message with certain causes. The selected network is well known as of poor quality. The stack tries to establish the MO connection for a second time.

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_TEMP_FAIL
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	NOT_USED
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE_COMP
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_TEMP_FAIL
		facility	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	27-Jun-00	HM	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC621: DISCONNECT in state U1 with poor network, bad release sequence

Description:	While in the state CALL INIT (U1) CC receives a DISCONNECT message with certain causes. The selected network is well known as of poor quality. The stack tries to establish the MO connection for a second time.

Preamble:	CC021

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_TEMP_FAIL
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	NOT_USED
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_RELEASE_IND
		ti	TI_MO_5
		cause	RRCS_NORM

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	27-Jun-00	HM	Initial
	10-Jul-01	JHU	Converted to TAP2

### CC622: DISCONNECT in state U3 with poor network

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a DISCONNECT message with certain causes. The selected network is well known as of poor quality. The stack tries to establish the MO connection for a second time. There

Preamble:	CC054

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (DISCONNECT)	  |
	 |		  *<==============================*
(2)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (RELEASE)	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (RELEASE COMPLETE)	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_DISCONNECT
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_TEMP_FAIL
		facility	NOT_USED
		progress	NOT_USED
		user_user	NOT_USED
		}

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_RELEASE
		ti	TI_MO_5
		cc_cause	NOT_USED
		cc_cause_2	NOT_USED
		facility	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_RELEASE_COMP
		ti	TI_MO_5_RESP
		cc_cause	CC_CAUSE_TEMP_FAIL
		facility	NOT_USED
		user_user	NOT_USED
		}

- MMCC_RELEASE_REQ
		ti	TI_MO_5

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

History:	27-Jun-00	HM	Initial
	10-Jul-01	JHU	Converted to TAP2

## MOC Bearer Capability Negotiation

### CC650: MOC, BS26, 3.1 kHz Audio, CE both

Description:	A mobile originated call is started for both transparent preferred asynchronous data with 9.6 kBit/s. This is bearer service 26. The data service is supported in PCS 1900 range by the PC version. A CE both is negatiated to transparent or non transparent by the network.

Variants:	<A>..<D>

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
(5)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(6)	 |	MNCC_CALL_PROCEED_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	NOT_PRESENT_8BIT
<A>		bcpara	BC_PARA_BS26_BOTH_T_9600
<B>		bcpara	BC_PARA_BS26_BOTH_T_9600
<C>		bcpara	BC_PARA_BS26_BOTH_NT_9600
<D>		bcpara	BC_PARA_BS26_BOTH_NT_9600
		bcpara2	BC_PARA_NO_SERVICE
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
<A>		bearer_cap	BC_BS26_BOTH_T_9600
<B>		bearer_cap	BC_BS26_BOTH_T_9600
<C>		bearer_cap	BC_BS26_BOTH_NT_9600
<D>		bearer_cap	BC_BS26_BOTH_NT_9600
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	NOT_USED
<A>		bearer_cap	BC_BS26_T_9600
<B>		bearer_cap	BC_BS26_NT_9600A
<C>		bearer_cap	BC_BS26_NT_9600A
<D>		bearer_cap	BC_BS26_T_9600
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MNCC_CALL_PROCEED_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN
		ri	NOT_PRESENT_8BIT
<A>		bcpara	BC_PARA_BS26_T_9600
<B>		bcpara	BC_PARA_BS26_NT_9600A
<C>		bcpara	BC_PARA_BS26_NT_9600A
<D>		bcpara	BC_PARA_BS26_T_9600
		bcpara2	BC_PARA_NO_SERVICE

History:	10-Sept-01	JK	Initial

### CC655: MOC, BS61, Alternate Speech/Data, CE both, RI CIRCULAR

Description:	A mobile originated call is started for both transparent preferred asynchronous data with 9.6 kBit/s. This is bearer service 26. The data service is supported in PCS 1900 range by the PC version. A CE both is negatiated to transparent or non transparent by the network.

Variants:	<A>..<B>

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
(5)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(6)	 |	MNCC_CALL_PROCEED_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	RI_CIRCULAR
<A>		bcpara	BC_PARA_BS26_BOTH_T_9600
<B>		bcpara	BC_PARA_SPEECH
<A>		bcpara2	BC_PARA_SPEECH
<B>		bcpara2	BC_PARA_BS26_BOTH_T_9600
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
<A>		estcs	ESTCS_MOB_ORIG_DATA
<B>		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	RI_CIRCULAR
<A>		bearer_cap	BC_BS26_BOTH_T_9600
<B>		bearer_cap	BC_SPEECH_FR_HR
<A>		bearer_cap_2	BC_SPEECH_FR_HR
<B>		bearer_cap_2	BC_BS26_BOTH_T_9600
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	RI_CIRCULAR
<A>		bearer_cap	BC_BS26_T_9600
<B>		bearer_cap	BC_SPEECH_FR_HR
<A>		bearer_cap_2	BC_SPEECH_FR_HR
<B>		bearer_cap_2	BC_BS26_T_9600
		facility	NOT_USED
		progress	PROG_1
		}

- MNCC_CALL_PROCEED_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN
		ri	RI_CIRCULAR
<A>		bcpara	BC_PARA_BS26_T_9600
<B>		bcpara	BC_PARA_SPEECH
<A>		bcpara2	BC_PARA_SPEECH
<B>		bcpara2	BC_PARA_BS26_T_9600

History:	14-Sept-01	JK	Initial

### CC660: MOC, BS81, Speech followed by Data, CE both, RI SEQUENTIAL

Description:	A mobile originated call is started for both transparent preferred asynchronous data with 9.6 kBit/s. This is bearer service 26. The data service is supported in PCS 1900 range by the PC version. A CE both is negatiated to transparent or non transparent by the network.

Variants:	<A>..<B>

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
(5)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(6)	 |	MNCC_CALL_PROCEED_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	RI_SEQUENTIAL
<A>		bcpara	BC_PARA_SPEECH
<B>		bcpara	BC_PARA_BS26_BOTH_NT_9600
<A>		bcpara2	BC_PARA_BS26_BOTH_NT_9600
<B>		bcpara2	BC_PARA_SPEECH
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
<A>		estcs	ESTCS_MOB_ORIG_SPCH
<B>		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	RI_SEQUENTIAL
<A>		bearer_cap	BC_SPEECH_FR_HR
<B>		bearer_cap	BC_BS26_BOTH_NT_9600
<A>		bearer_cap_2	BC_BS26_BOTH_NT_9600
<B>		bearer_cap_2	BC_SPEECH_FR_HR
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	RI_SEQUENTIAL
<A>		bearer_cap	BC_SPEECH_FR_HR
<B>		bearer_cap	BC_BS26_NT_9600A
<A>		bearer_cap_2	BC_BS26_NT_9600A
<B>		bearer_cap_2	BC_SPEECH_FR_HR
		facility	NOT_USED
		progress	PROG_1
		}

- MNCC_CALL_PROCEED_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN
		ri	RI_SEQUENTIAL
<A>		bcpara	BC_PARA_SPEECH
<B>		bcpara	BC_PARA_BS26_NT_9600A
<A>		bcpara2	BC_PARA_BS26_NT_9600A
<B>		bcpara2	BC_PARA_SPEECH

History:	14-Sept-01	JK	Initial

### CC700: MOC, TS61, Accepted by the network

Description:	A mobile originated call is started for alternate speech and transparent fax. This is tele service 61. A speech BC was sent first (in preamble Test case CC563). The request is expected to be negotiated with the network.

Preamble:	CC563C

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(2)	 |	MNCC_CALL_PROCEED_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	NOT_USED
		bearer_cap	BC_FAX_T_9600
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MNCC_CALL_PROCEED_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_FAX_T_9600
		bcpara2	NOT_USED

History:	17-Sept-01	JK	Initial

### CC705: MOC, TS61/62, Negotiation FAX first

Description:	A mobile originated call is started for alternate speech and transparent fax. This is tele service 61/62. A fax BC was sent first. The request is expected to be negotiated with the network.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
(1)	 |	MNCC_SETUP_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
(5)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(6)	 |	MNCC_CALL_PROCEED_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_SETUP_REQ
		ti	TI_MO_5
		prio	PRIO_NORM_CALL
		ri	RI_CIRCULAR
		bcpara	BC_PARA_FAX_T_9600
		bcpara2	BC_PARA_SPEECH 
		called_party	CLED_PARTY_654321
		called_party_sub	CLED_PARTY_SUB_NONE
		clir_sup	NOT_PRESENT_8BIT
		fac_inf	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_MO_5
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	RI_CIRCULAR
		bearer_cap	BC_FAX_T_9600 
		bearer_cap_2	BC_SPEECH_FR_HR_EFR
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	NOT_USED
		bearer_cap	BC_FAX_T_9600
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MNCC_CALL_PROCEED_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_FAX_T_9600
		bcpara2	BC_PARA_NO_SERVICE

History: 	17-Sept-01	JK	Initial

### CC710: MOC, TS61/62, Negotiation Speach first

Description:	A mobile originated call is started for alternate speech and transparent fax. This is tele service 61/62. A speech BC was sent first (in preamble Test case CC563). The request is expected to be confirmed by the network.

Preamble:	CC563C

MMI		 CC		 MM
	 |		  |		  |
(1)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CALL PROCEEDING)	  |
	 |		  *<==============================*
(2)	 |	MNCC_CALL_PROCEED_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CALL_PROCEED
		ti	TI_MO_5_RESP
		repeat	NOT_USED
		bearer_cap	NOT_USED
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		}

- MNCC_CALL_PROCEED_IND
		ti	TI_MO_5
		progress_desc	PROG_END_TO_END_PLMN
		ri	NOT_USED
		bcpara	NOT_USED
		bcpara2	NOT_USED

History: 	17-Sept-01	JK	Initial

### CC720: MTC, BS26, 3.1 kHz Audio Non-Transparent 9.6 kBit/s negotiated TEST

Description:	A mobile terminated call is started for BS26 (9600 bit/s) in the PCS 1900 range. The PC version supports this data service for PCS 1900. It is expected that the bearer capability is accepted with negotiation. The incoming bearer capability indicates as connection element Both, non-transparent preferred. MS is set to ‘transparent’, so it is exspected, that it selects transparent.

Preamble:	CC000

MMI		 CC		 MM
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |
	MMI		 CC		 MM
	 |		  |		  |
(1) 	 |	MNCC_CONFIGURE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_IND	  |
	 |		  |	  (SETUP)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CALL CONFIRMED)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MNCC_CONFIGURE_REQ
		called_party_sub	NOT_USED
		bcpara	BC_PARA_BS26_T_9600B
		sns_mode	SNS_MODE_DATA
		ctm_ena	CTM_DISABLED

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_SETUP
		ti	TI_MT_1
		repeat	NOT_USED
		bearer_cap	BC_BS26_9600_BNTPT
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		progress	PROG_1
		signal	SIGNAL_DIAL_ON
		calling_num	CLNG_NUM_654321
		calling_subaddr	NOT_USED
		dl_called_num	DL_CLED_NUM_654321
		called_subaddr	NOT_USED
		redirecting_num	NOT_USED
		redirecting_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_IND
		ti	TI_MT_1_RESP
		ri	NOT_PRESENT_8BIT
		bcpara	BC_PARA_BS26_T_9600C
		bcpara2	BC_PARA_NO_SERVICE
		progress_desc	PROG_END_TO_END_PLMN
		sig	SIGNAL_DIAL_ON
		calling_party	CLNG_PARTY_654321
		calling_party_sub	NOT_USED
		called_party	CLED_PARTY_654321
		called_party_sub	NOT_USED
		redirecting_party	REDIR_PARTY_NONE
		redirecting_party_sub	REDIR_PARTY_SUB_NONE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_CALL_CONF
		ti	TI_MT_1_RESP
		repeat	NOT_USED
		bearer_cap	BC_BS26_9600_TPT
		bearer_cap_2	NOT_USED
		cc_cause	NOT_USED
		}

History: 	25-Sept-01	JK	Initial
	05-Apr-02	HM	call_ctrl_cap removed as in target
	02-Dec-2002	FK	Parameter ‘ctm_ena’ added

## Engineering Mode

### CC750: MM connection established for basic call - data (U0.1)

Description:	MM signals CC in the form of a MMCC-ESTABLISH confirmation primitive that a connection has been successfully established. CC issues a MMCC-DATA request primitive and enters the state CALL INIT (U1). (Ref. [1] 5.n.n.n)

Preamble:	CC011

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	EM_CC_EVENT_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_ESTABLISH_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (SETUP)	  |
	 |		  *==============================>*
(4)	 |	EM_DATA_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- EM_CC_EVENT_REQ
		bitmask_cc_h	Bitm_H_1
		bitmask_cc_l	Bitm_L_1

- MMCC_ESTABLISH_CNF
		ti	TI_MO_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	U_SETUP
		ti	TI_MO_5
		repeat	NOT_USED
		bearer_cap	BC_BS21_T_300
		bearer_cap_2	NOT_USED
		facility	NOT_USED
		calling_subaddr	NOT_USED
		ul_called_num	UL_CLED_NUM_654321
		called_subaddr	NOT_USED
		repeat_2	NOT_USED
		low_layer_comp	NOT_USED
		low_layer_comp_2	NOT_USED
		repeat_3	NOT_USED
		high_layer_comp	NOT_USED
		high_layer_comp_2	NOT_USED
		user_user	NOT_USED
		ss_version	NOT_USED
		clir_suppr	NOT_USED
		clir_invoc	NOT_USED
		call_ctrl_cap	CALL_CTRL_CAP_1
		}

- EM_DATA_IND
		entity	EM_ENTITY

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

23-Oct-01	OT	Adaptations for EM

### CC751: CONNECT message received - speech (U3)

Description:	While in the state MO CALL PROCEEDING (U3) CC receives a CONNECT message as part of a MMCC-DATA indication primitive in response to a SETUP message. CC then stops the T303 timer, enters the state ACTIVE (U10) and issues a MNCC-SETUP confirmation primitive to MMI and a CONNECT ACKNOWLEDGE message as part of a MMCC-DATA request primitive to MM. (Ref. [1] 5.2.1.6)

Preamble:	CC026A

MMI		 CC		 MM
	 |		  |		  |
(1)	 |	EM_CC_EVENT_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MMCC_DATA_IND	  |
	 |		  |	  (CONNECT)	  |
	 |		  *<==============================*
(3)	 |	MNCC_SETUP_CNF	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MMCC_DATA_REQ	  |
	 |		  |	  (CONNECT ACKNOWLEDGE)	  |
	 |		  *==============================>*
(5)	 |	EM_DATA_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- EM_CC_EVENT_REQ
		bitmask_cc_h	Bitm_H_2
		bitmask_cc_l	Bitm_L_2

- MMCC_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	DOWNLINK
		pd	D_CONNECT
		ti	TI_MO_5_RESP
		facility	NOT_USED
		progress	PROG_1
		connect_num	CONNECT_NUM_654321
		connect_subaddr	NOT_USED
		user_user	NOT_USED
		}

- MNCC_SETUP_CNF
		ti	TI_MO_5
		cause	MNCC_CAUSE_SUCCESS
		progress_desc	PROG_END_TO_END_PLMN
		connected_number	CONNECTED_NUMBER_654321
		connected_number_sub	NOT_USED

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	CC
		direction	UPLINK
		pd	B_CONNECT_ACK
		ti	TI_MO_5
		}

- EM_DATA_IND	entity	EM_ENTITY

History:	25-Apr-97	DL	Initial
	04-Aug-97	VK	d1,d2 added
	27-Jul-98	LE	adapted to fax & data
	05-Jul-01	JHU	Converted to TAP2

23-Oct-01	OT	Adaptations for EMproperly.
	26-Nov-01	OT	Variant CC026A instead of CC026