Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document – Confidential

GSM Protocol Stack

Test Specification

MM

(Edit via File/Properties/Summary!)

| Document Number: | 6147.403.97.100 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.5 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1997-Jul-09 |
| Last changed: | 2003-May-19 by XGUTTEFE |
| File Name: | mm.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1997-Jul-09 | Stefan Lemke et al |  | 0.1 |  | 1 |
| 2001-Mar-14 | HM |  | 0.2 |  | 2 |
| 2001-Apr-18 | HM |  | 0.3 |  | 3 |
| 2001-Apr-02 | HM |  | 0.4 |  | 4 |
| 2003-May-19 | XGUTTEFE |  | 0.5 | Draft |  |

Notes:

- Initial version

- Revised

- Revised during AT+CFUN=4 implentation

- Revised after protocol change

Table of Contents

1.1	References	11

1.2	Abbreviations	14

1.3	Terms	16

2	Overview	16

3	Parameters	18

4	TEST CASES	42

4.1	Internal Routing	42

4.1.1	MM001: Configure internal routing and PCO view	42

4.2	Registration	44

4.2.1	MM021: Registration without SIM card	44

4.2.2	MM022: Mobile station is synchronous to a Cell	45

4.2.3	MM023: SIM inserted - initiate cell selection	46

4.2.4	MM024: Successful conclusion of cell selection - cell with same LAI	47

4.2.5	MM025: SIM inserted - Search for specific network (no LUP)	48

4.2.6	MM026: SIM inserted - Search for specific network (with LUP)	50

4.2.7	MM027: Mobile station is synchronous to a Cell (reestablish allowed in cell)	53

4.3	Connection Establishment	54

4.3.1	MM041: Establish request prior to existing RR Connection	54

4.3.2	MM042: Mobile terminated MM Connection	55

4.3.3	MM043: Acknowledgement of MM Connection	56

4.3.4	MM044: Identity Request in State 5	57

4.3.5	MM045: RR-MM Synchronization	59

4.3.6	MM047: Data Transfer from Mobile Station in State 6	60

4.3.7	MM048: Data Transfer in both directions in State 6	61

4.3.8	MM049: Random Access Failed, internal auto redial	62

4.3.9	MM050: Initiation of CCBS call back	64

4.3.10	MM051: Initiation of CCBS call back, out of ti	65

4.3.11	MM055: RR_ESTABLISH_IND in state MM_WAIT_FOR_RR_CONN_MM	66

4.3.12	MM056: RR_RELEASE_IND in state MM_WAIT_FOR_RR_CONN_MM	67

4.4	Emergency Call	68

4.4.1	MM061: Emergency Call in Idle State 19.2	68

4.4.2	MM062: RR Release followed by Call Establishment Request	70

4.4.3	MM063: Emergency Call in Idle State 19.4	71

4.4.4	MM064: Reestablishment during EC (Emergency Call)	73

4.4.5	MM065: Emergency Call in State 19.3	75

4.4.6	MM066: Calls after power cycle without SIM Remove	77

4.5	Connection Release	80

4.5.1	MM081: Release of Connection via MS	80

4.5.2	MM082: MMCC_REL_REQ in State 14	81

4.5.3	MM083: Release of Connection via BS	82

4.6	Location Updating	83

4.6.1	MM101: MS in new Location Area	83

4.6.2	MM102: New LAI with Periodic Updating	84

4.6.3	MM103: Access classes barred in State 13	85

4.6.4	MM104: Random access delayed in State 13	86

4.6.5	MM105: Successful Location Updating	87

4.6.6	MM106: Location Accept without Mobile Identity	89

4.6.7	MM606: Location Accept with empty Mobile Identity	91

4.6.8	MM907: Location Updating triggered by Normal Call in Idle State 19.2	93

4.6.9	MM908: Location Updating triggered by Normal Call in Idle State 19.2	95

4.6.10	MM107: Connection request stored until Location Updating complete	97

4.6.11	MM108: Location Updating Reject (PLMN not allowed)	100

4.6.12	MM109: Change of Area in Update Status U3	102

4.6.13	MM110: Location Updating Reject (LAI roaming not allowed)	105

4.6.14	MM111: New Location Area Code in Update Status U3	107

4.6.15	MM112: Normal Location Updating rejected four times	110

4.6.16	MM113: Re-attempt after time-out Location updating timer	115

4.6.17	MM114: Re-attempt after fieldstrength jump	118

4.6.18	MM115: MS in new Location Area (TEST SIM inserted)	121

4.6.19	MM116: Location Updating Reject (PLMN not allowed), not HPLMN	122

4.6.20	MM117: Location Updating Reject (PLMN not allowed), mode AUTO	125

4.6.21	MM118: Location Updating Reject (IMSI unknown in HLR), mode AUTO	131

4.7	Identity Request	135

4.7.1	MM121: Identity Request during Location Updating	135

4.7.2	MM122: Identity Request during MTC	136

4.8	Authentication	137

4.8.1	MM141: Authentication Request	137

4.8.2	MM142: Authentication Response	138

4.8.3	MM143: Authentication Request in State 6	139

4.8.4	MM144: Authentication Reject in State 3	141

4.8.5	MM145: Authentication Reject and SIM Removal in State 3	143

4.8.6	MM146: Authentication Request in State 5	145

4.8.7	MM147: Response to Authentication Request in State 5	147

4.8.8	MM148: Authentication Reject in State 5	148

4.8.9	MM149: Authentication Reject during request for a second connection	150

4.8.10	MM150: Authentication in State 9	153

4.8.11	MM151: Response to Authentication Request in State 9	154

4.8.12	MM152: Authentication Reject in State 9	155

4.8.13	MM153: Registration request following authorization failure	156

4.8.14	MM154: LUP Rejection following AUTHENTICATION RESPONSE in State 3	158

4.8.15	MM155: LUP with MM INFORMATION	159

4.9	Net Request	161

4.9.1	MM161: Net Request in State 19.4	161

4.9.2	MM162: RR_ABORT_IND in State 19	162

4.10	TMSI Reallocation	163

4.10.1	MM181: TMSI Reallocation in State 5	163

4.11	Deregistration	165

4.11.1	MM201: MMR_NREG_REQ in State 3	165

4.11.2	MM202: Power Off in Upate Status U1	167

4.11.3	MM203: Authentication Reject and Power off in State 6	168

4.11.4	MM204: Power Off in State 10	169

4.11.5	MM205: Power Off in State 14	171

4.11.6	MM206: Power Off in State 18	172

4.11.7	MM207: Power Off Remove in State 19	173

4.11.8	MM208: Switch on after power off, IMSI ATTACH needed	174

4.12	Registration (REG_MS_OFF, no SIM Card)	176

4.12.1	MM300: Registration	176

4.12.2	MM301: Deregistration	177

4.12.3	MM302: SIM Insertion	178

4.12.4	MM352: PLMN Available Request	179

4.13	Registration (REG_NO_SERVICE, no SIM Card)	180

4.13.1	MM303: Registration	180

4.13.2	MM304: Deregistration	181

4.13.3	MM305: SIM Insertion	182

4.13.4	MM306: RR failure (No Service)	183

4.13.5	MM307: MM Success ( Limited Service)	184

4.13.6	MM353: PLMN Available Request	184

4.14	Registration (REG_LIMITED_SERVICE, no SIM Card)	186

4.14.1	MM308: Registration	186

4.14.2	MM309: Deregistration	187

4.14.3	MM310: SIM Insertion	188

4.14.4	MM311: RR failure (No Service)	189

4.14.5	MM312: MM Success ( Limited Service)	190

4.14.6	MM354: PLMN Available Request	191

4.15	Registration (REG_MS_OFF, with SIM card)	192

4.15.1	MM313: Registration	192

4.15.2	MM314: PLMN Mode Change	194

4.15.3	MM315: Deregistration (Power Off)	195

4.15.4	MM316: Deregistration (SIM invalid)	196

4.15.5	MM317: SIM Removal	197

4.15.6	MM351: PLMN Available Request	198

4.16	Registration (REG_NO_SERVICE, with SIM card, automatic mode)	199

4.16.1	MM318: Registration	199

4.16.2	MM319: PLMN Mode Change	201

4.16.3	MM320: Deregistration (Power Off)	203

4.16.4	MM321: Deregistration (SIM invalid)	204

4.16.5	MM322: SIM Removal	205

4.16.6	MM323: RR failure (No Service)	206

4.16.7	MM324: RR failure (Limited Service, no further PLMNs)	207

4.16.8	MM325: RR failure (Limited Service, further PLMNs available)	208

4.16.9	MM326: MM Success	209

4.16.10	MM327: MM Failure	210

4.16.11	MM328: MM Authentication Failure	216

4.16.12	MM355: PLMN Available Request	219

4.16.13	MM358: PLMN Selection successful	222

4.16.14	MM359: PLMN Selection unsuccessful	223

4.16.15	MM364: PLMN Available List to MMI	225

4.16.16	MM365: PLMN Selection successful (manual Mode)	227

4.16.17	MM366: PLMN Selection unsuccessful (Manual Mode)	228

4.17	Registration (REG_LIMITED_SERVICE, with SIM card, automatic mode)	230

4.17.1	MM329: Registration	230

4.17.2	MM330: PLMN Mode Change	232

4.17.3	MM331: Deregistration (Power Off)	234

4.17.4	MM332: Deregistration (SIM invalid)	235

4.17.5	MM333: SIM Removal	236

4.17.6	MM334: RR failure (No Service)	237

4.17.7	MM335: RR failure (Limited Service, no further PLMNs)	238

4.17.8	MM336: RR failure (Limited Service, further PLMNs available)	239

4.17.9	MM337: MM Success	240

4.17.10	MM338: MM Failure	241

4.17.11	MM339: MM Authentication Failure	247

4.17.12	MM356: PLMN Available Request	250

4.17.13	MM360: PLMN Selection successful	253

4.17.14	MM361: PLMN Selection unsuccessful	254

4.18	Registration (REG_FULL_SERVICE, with SIM card, automatic mode)	256

4.18.1	MM340: Registration	256

4.18.2	MM341: PLMN Mode Change	257

4.18.3	MM342: Deregistration (Power Off)	259

4.18.4	MM343: Deregistration (SIM invalid)	260

4.18.5	MM344: SIM Removal	261

4.18.6	MM345: RR failure (No Service)	262

4.18.7	MM346: RR failure (Limited Service, no further PLMNs)	263

4.18.8	MM347: RR failure (Limited Service, further PLMNs available)	264

4.18.9	MM348: MM Success	265

4.18.10	MM349: MM Failure	267

4.18.11	MM350: MM Authentication Failure	273

4.18.12	MM357: PLMN Available Request	276

4.18.13	MM362: PLMN Selection successful	278

4.18.14	MM363: PLMN Selection unsuccessful	279

4.19	MM Idle Mode Behaviour (Normal Service)	281

4.19.1	MM400: Normal Service, new cell, same location area	281

4.19.2	MM401: Normal Service, new cell, new location area	282

4.19.3	MM402: Normal Service, new cell, new PLMN identification	283

4.19.4	MM403: Updated, IMSI Attach	284

4.19.5	MM404: Normal Service, Timeout T3211, LUP Reject Cause #17	286

4.19.6	MM405: Normal Service, Timeout T3211, LUP Reject Cause, 2.-4. attempt	288

4.19.7	MM406: Updated, Periodic LUP	291

4.19.8	MM407: Normal Service, Timeout T3211, LUP Reject Cause #17	293

4.19.9	MM408: Normal Service, Timeout T3211, LUP Reject Cause, 2.-4. attempt	295

4.19.10	MM409: Normal Service, Timeout T3211, RR Release before end of proc	299

4.19.11	MM410: Normal Service, T3211, RR Release before end of proc, 2.-4. attempt	300

4.19.12	MM411: Normal Service, T3211, RR Release before end of proc	303

4.19.13	MM412: Normal Service, T3211, RR Release before end of proc, 2.-4. attempt	304

4.19.14	MM413: Normal Service, T3211, Timeout T3210	307

4.19.15	MM414: Normal Service, T3211, Timeout T3210, 2.-4. attempt	309

4.19.16	MM415: Normal Service, T3211, Timeout T3210	312

4.19.17	MM416: Normal Service, T3211, Timeout T3210, 2.-4. attempt	314

4.19.18	MM417: Normal Service, T3211, RR connection failure	317

4.19.19	MM418: Normal Service, T3211, RR Connection Failure, 2.-4. attempt	319

4.19.20	MM419: Normal Service, T3211, RR Connection Failure	322

4.19.21	MM420: Normal Service, T3211, RR Connection Failure, 2.-4. attempt	324

4.19.22	MM421: Normal Service, T3211, Random Access Failure	327

4.19.23	MM422: Normal Service, T3211, Random Access Failure, 2.-4. attempt	329

4.19.24	MM423: Normal Service, T3211, Random Access Failure	333

4.19.25	MM424: Normal Service, T3211, Random Access Failure, 2.-4. attempt	335

4.19.26	MM425: Random access delayed	339

4.19.27	MM426: Random access barred	340

4.19.28	MM427: Not Updated in Current LA, Start Normal Location Updating	341

4.19.29	MM600: Normal Service, T3211, Anite Behaviour	343

4.20	MM Idle Mode Behaviour (Attempt to Update)	347

4.20.1	MM428: Attempt to Update, Timeout T3211, LUP Reject Cause #17	347

4.20.2	MM429: Attempt to update, Timeout T3211, LUP Reject Cause, 2.-4. attempt	349

4.20.3	MM430: Not Updated, Periodic LUP	353

4.20.4	MM431: Periodic not updated, Timeout T3211, LUP Reject Cause #17	354

4.20.5	MM432: Attempt to Udpate, Normal, T3211, RR Release before end of proc	360

4.20.6	MM433: Periodic, Attempt to Update, T3211, RR Release before end of proc	364

4.20.7	MM434: Attempt to Udpate, Normal, T3211, Timeout T3210	369

4.20.8	MM435: Periodic, Attempt to Update, T3211, Timeout T3210	373

4.20.9	MM436: Attempt to Update, Normal, T3211, RR connection failure	378

4.20.10	MM437: Periodic, Attempt to Update, T3211, RR Connection Failure	382

4.20.11	MM438: Attempt to Update, Normal, T3211, RR connection failure	387

4.20.12	MM439: Periodic, Attempt to Update, T3211, Random Access Failure	392

4.20.13	MM440: Random access delayed	399

4.20.14	MM441: Random access barred	400

4.20.15	MM442: Normal Service, IMSI Detach, Power OFF, Est Cnf	401

4.20.16	MM443: Normal Service, IMSI Detach, Power OFF, Rel Ind	403

4.20.17	MM444: Normal Service, IMSI Detach, Power OFF, Radio Link Failure	405

4.20.18	MM446: Normal Service, IMSI Detach, SIM Remove by SIM, Est Cnf	407

4.20.19	MM447: Normal Service, IMSI Detach, SIM Remove by SIM, Rel Ind	409

4.20.20	MM448: Normal Service, IMSI Detach, SIM Remove by SIM, Radio Link Fail	411

4.20.21	MM450: Normal Service, IMSI Detach, SIM Remove by MMI, Est Cnf	413

4.20.22	MM451: Normal Service, IMSI Detach, SIM Remove by MMI, Rel Ind	415

4.20.23	MM452: Normal Service, IMSI Detach, SIM Remove by MMI, Radio Link Fail	417

4.20.24	MM454: IMSI Attach and CM Connection Establishment	419

4.20.25	MM455: Connection Active, IMSI Detach, Power OFF, Timeout T3220	422

4.20.26	MM456: Connection Active, IMSI Detach, Power OFF, Rel Ind	424

4.20.27	MM457: Connection Active, IMSI Detach, Power OFF, Radio Link Failure	426

4.20.28	MM458: Active, IMSI Detach, SIM Remove by SIM, Rel Ind	428

4.20.29	MM459: Active, IMSI Detach, SIM Remove by SIM, Radio Link Fail	430

4.20.30	MM460: Active, IMSI Detach, SIM Remove by SIM, T3220 Timeout	432

4.20.31	MM461: Active, IMSI Detach, SIM Remove by MMI, Rel Ind	434

4.20.32	MM462: Active, IMSI Detach, SIM Remove by MMI, Radio Link Fail	436

4.20.33	MM463: Active, IMSI Detach, SIM Remove by MMI, T3220 Timeout	438

4.20.34	MM464: Attempt to update, change of location area identification	440

4.20.35	MM465: Attempt to update, change of cell, random access failure	441

4.20.36	MM466: Attempt to update, change of cell, RR connection failure	442

4.20.37	MM467: Attempt to update, change of cell, Release before end of proc	443

4.20.38	MM468: Attempt to update, change of cell, Release before end of proc	444

4.20.39	MM469: Attempt to update, change of cell, Location Updating Request	448

4.20.40	MM470: Attempt to update, change of cell, Location updating reject	449

4.20.41	MM471: Attempt to Update, IMSI Detach, Power OFF	454

4.20.42	MM472: Attempt to Update, IMSI Detach, SIM Remove by SIM	455

4.20.43	MM473: Attempt to Update, IMSI Detach, SIM Remove by MMI	456

4.20.44	MM474: Attempt to update, Emergency Call	457

4.20.45	MM475: Attempt to update, CM connection request (I)	459

4.20.46	MM476: Attempt to update, CM connection request (II)	462

4.20.47	MM477: Attempt to update, CM connection request (III)	467

4.20.48	MM478: Attempt to update, CM connection request (IV)	470

4.21	MM Idle Mode Behaviour (Limited Service)	473

4.21.1	MM479: Limited Service State	473

4.21.2	MM480: Limited, not perform Periodic LUP	475

4.21.3	MM481: Limited, IMSI Detach, Power OFF	476

4.21.4	MM482: Limited, IMSI Detach, SIM Remove by SIM	477

4.21.5	MM483: Limited, IMSI Detach, SIM Remove by MMI	478

4.21.6	MM484: Limited, Call attempts, Emergency Calls	479

4.21.7	MM485: Limited, new cell, new LA, but only limited service	483

4.21.8	MM487: Limited, new location area, full service	484

4.21.9	MM488: Limited Service, LUP Reject Cause #17	485

4.21.10	MM489: Limited Service, Location updating accept	487

4.21.11	MM490: Limited Service, Mobile terminated Connection	490

4.22	MM Idle Mode Behaviour (No IMSI Service)	491

4.22.1	MM491: No IMSI State	491

4.22.2	MM492: No Imsi, not perform Periodic LUP	492

4.22.3	MM493: No IMSI, IMSI Detach, Power OFF	493

4.22.4	MM494: No IMSI, Call attempts, Emergency Calls	494

4.22.5	MM495: No IMSI, new cell, new LA, but only limited service	498

4.23	MM Idle Mode Behaviour (No Cell available)	499

4.23.1	MM497: No Cell available indication from Normal Service	499

4.23.2	MM498: No Cell available, same cell	500

4.23.3	MM499: No Cell available, same location area	501

4.23.4	MM500: No IMSI, IMSI Detach, Power OFF	502

4.23.5	MM501: No Cell available, no IMSI Detach, SIM Remove by SIM	503

4.23.6	MM502: No Cell available, IMSI Detach, SIM Remove by MMI	504

4.23.7	MM503: No cell available, Call attempts by upper layer	505

4.23.8	MM504: No cell available, new cell, new LA, but only limited service	507

4.23.9	MM506: No Cell available, new cell, new location area, full service	508

4.24	Additional registration testcases	509

4.24.1	MM520: MM needs IMSI ATTACH after switch on. Cell temporary barred	509

4.24.2	MM521: MM needs IMSI ATTACH after switch on. Cell temporary barred	512

4.24.3	MM522: MM doesn't find LPLMN, needs NORMAL UPDATE, cell barred	515

4.24.4	MM523: MM doesn't find LPLMN, needs NORMAL UPDATE, cell barred	518

4.24.5	MM524: MM needs IMSI ATTACH after switch on. Cell temporary barred	522

4.24.6	MM525: MM needs IMSI ATTACH after switch on in tunnel	525

4.24.7	MM526: MM IDLE updated, T3212 running, manual network search I	527

4.24.8	MM527: MM IDLE updated, T3212 running, manual network search II	530

4.25	Network search	533

4.25.1	MM530: Aborted manual NW search in MM_IDLE_ATTEMPT_TO_UPDATE	533

4.25.2	MM531: MM_IDLE_ATTEMPT_TO_UPDATE, MO call	535

4.25.3	MM532: Manual NW search in MM_IDLE_ATTEMPT_TO_UPDATE	538

4.26	Behaviour due to SAT activity	541

4.26.1	MM601: SIM insert indication by SAT .- IMSI change, detach/attach, automatic	541

4.26.2	MM602: SIM insert indication by SAT - IMSI change, detach/attach, manual	546

4.26.3	MM603: SIM insert indication by SAT - IMSI change, automatic mode	553

4.26.4	MM604: SIM insert indication by SAT - IMSI change, manual mode	556

4.26.5	MM605: SIM insert indication by SAT - no IMSI change, RR notification	559

4.26.6	MM606: SIM inserted - File update by SAT	560

4.26.7	MM607: SIM inserted - File update by SAT	561

4.26.8	MM608: File update by SAT, RR notification	562

4.27	Switching service modes back and forth	564

4.27.1	MM620: SIM inserted - Deregistered -> Deregistered	564

4.27.2	MM621: SIM inserted - Deregistered -> Limited	565

4.27.3	MM622: SIM inserted - Deregistered -> Full (automatic mode)	567

4.27.4	MM623: SIM inserted - Deregistered -> Full (manual mode)	568

4.27.5	MM624: SIM inserted - Limited -> Deregistered	569

4.27.6	MM625: SIM inserted - Limited -> Limited	570

4.27.7	MM626: SIM inserted - Limited -> Full (automatic mode)	571

4.27.8	MM627: SIM inserted - Limited -> Full (manual mode)	573

4.27.9	MM628: SIM inserted - Full (automatic mode) -> Deregistered	575

4.27.10	MM629: SIM inserted - Full (automatic mode) -> Limited	576

4.27.11	MM630: SIM inserted - Full (automatic mode) -> Full (automatic mode)	578

4.27.12	MM631: SIM inserted - Full (automatic mode) -> Full (manual mode)	580

4.27.13	MM632: SIM inserted - Full (manual mode) -> Deregistered	582

4.27.14	MM633: SIM inserted - Full (manual mode) -> Limited	583

4.27.15	MM634: SIM inserted - Full (manual mode) -> Full (automatic mode)	585

4.27.16	MM635: SIM inserted - Full (manual mode) -> Full (manual mode)	587

4.28	Engineering mode	588

4.28.1	MM650: Identity Request during Location Updating	588

4.28.2	MM651: Authentication Request	589

Appendices	591

A.	Acronyms	591

B.	Glossary	591

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat AG

[39]	Service Access Point MNCC
	6147.101.96.100; Condat AG

[40]	Service Access Point MNSS
	6147.102.96.100; Condat AG

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat AG

[42]	Service Access Point MMCC
	6147.104.97.100; Condat AG

[43]	Service Access Point MMSS
	6147.105.97.100; Condat AG

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat AG

[45]	Service Access Point RR
	6147.107.97.100; Condat AG

[46]	Service Access Point SIM
	6147.108.97.100; Condat AG

[47]	Service Access Point MPH
	6147.109.96.100; Condat AG

[48]	Service Access Point DL
	6147.110.96.100; Condat AG

[49]	Service Access Point MDL
	6147.111.96.100; Condat AG

[50]	Service Access Point PH
	6147.112.97.100; Condat AG

[51]	Service Access Point MMI
	6147.113.96.100; Condat AG

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat AG

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat AG

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat AG

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat AG

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat AG

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat AG

[58]	Users Guide
	6147.300.96.100; Condat AG

[59]	Test Specification CC
	6147.400.97.100; Condat AG

[60]	Test Specification SS
	6147.401.97.100; Condat AG

[61]	Test Specification SMS
	6147.402.97.100; Condat AG

[62]	Test Specification MM
	6147.403.97.100; Condat AG

[63]	Test Specification RR
	6147.404.97.100; Condat AG

[64]	Test Specification DL
	6147.405.97.100; Condat AG

[65]	Test Specification CCD
	6147.406.97.100; Condat AG

[66]	SDL Specification CC
	6147.500.97.100; Condat AG

[67]	SDL Specification SS
	6147.501.97.100; Condat AG

[68]	SDL Specification SMS
	6147.502.97.100; Condat AG

[69]	SDL Specification MM
	6147.503.97.100; Condat AG

[70]	SDL Specification RR
	6147.504.97.100; Condat AG

[71]	SDL Specification DL
	6147.505.97.100; Condat AG

[72]	Message Specification CC
	6147.600.97.100; Condat AG

[73]	Message Specification SS
	6147.601.97.100; Condat AG

[74]	Message Specification SMS
	6147.602.97.100; Condat AG

[75]	Message Specification MM
	6147.603.97.100; Condat AG

[76]	Message Specification RR
	6147.604.97.100; Condat AG

[77]	Message Specification DL
	6147.605.97.100; Condat AG

[78]	Technical Documentation CC
	6147.700.97.100; Condat AG

[79]	Technical Documentation SS
	6147.701.97.100; Condat AG

[80]	Technical Documentation SMS
	6147.702.97.100; Condat AG

[81]	Technical Documentation MM
	6147.703.97.100; Condat AG

[82]	Technical Documentation RR
	6147.704.97.100; Condat AG

[83]	Technical Documentation DL
	6147.705.97.100; Condat AG

[84]	Technical Documentation CCD
	6147.706.97.100; Condat AG

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

- Fehler! Es ist nicht möglich, durch die Bearbeitung von Feldfunktionen Objekte zu erstellen.

- Figure 1: Mobile-station protocol architecture

- This document describes the tests for Mobility Management.

# Parameters

- FIELD (RXLEVEL_20)

0x20
ENDFIELD (RXLEVEL_20, 1)

- FIELD (RXLEVEL_20_A)

12, /* Number of supported networks by SAPI*/
	0x20
ENDFIELD (RXLEVEL_20_A, 1)

- FIELD (RXLEVEL_20_18)

0x20, 0x18
ENDFIELD (RXLEVEL_20_18, 2)

- FIELD (RXLEVEL_20_18_A)

12, /* Number of supported networks by SAPI*/
	0x20, 0x18
ENDFIELD (RXLEVEL_20_18_A, 3)

FIELD (PLMN_LIST_FORB)
	V_PLMN_PRES,
	1,2,3,
	3,2
ENDFIELD (PLMN_LIST_FORB, 6)

FIELD (PLMN_123_32_A)
	V_PLMN_PRES,
	1,2,3,
	3,2
ENDFIELD (PLMN_123_32_A, 6)

FIELD (PLMN_LIST_LPLMN)
	V_PLMN_PRES,
	1,2,3,
	3,1
ENDFIELD (PLMN_LIST_LPLMN, 6)

FIELD (PLMN_LIST_2_PLMN)
	V_PLMN_PRES,
	1,2,3,
	3,3,
	V_PLMN_PRES,
	1,2,3,
	3,1,
ENDFIELD (PLMN_LIST_2_PLMN, 12)

FIELD (PLMN_LIST_2_PLMN_1_NEW_FORB)
	V_PLMN_PRES,
	1,2,3,
	4,4,
	V_PLMN_PRES,
	1,2,3,
	3,1,
ENDFIELD (PLMN_LIST_2_PLMN_1_NEW_FORB, 12)

FIELD (PLMN_LIST_2_PLMN_A)
	12, /* Number of supported networks by SAPI*/
	V_PLMN_PRES,
	1,2,3,
	3,3,
	V_PLMN_PRES,
	1,2,3,
	3,1,
ENDFIELD (PLMN_LIST_2_PLMN_A, 13)

FIELD (PLMN_LIST_2_PLMN_F)	V_PLMN_PRES,
	1,2,3,
	3,3,
	V_PLMN_PRES,
	1,2,3,
	3,2,
ENDFIELD (PLMN_LIST_2_PLMN_F, 12)

FIELD (PLMN_LIST_2_PLMN_AF)
	12, /* Number of supported networks by SAPI*/
	V_PLMN_PRES,
	1,2,3,
	3,3,
	V_PLMN_PRES,
	1,2,3,
	3,2,
ENDFIELD (PLMN_LIST_2_PLMN_AF, 13)

FIELD (PLMN_LIST_1_PLMN_A)
	12, /* Number of supported networks by SAPI*/
	V_PLMN_PRES,
	1,2,3,
	3,2,
ENDFIELD (PLMN_LIST_1_PLMN_A, 7)

FIELD (FORB_PLMN_ID)
		12, /* Number of supported networks by SAPI*/
		0,0
ENDFIELD (FORB_PLMN_ID, 3)

FIELD (FORB_PLMN_ID_F)
		12, /* Number of supported networks by SAPI*/
		0,1
ENDFIELD (FORB_PLMN_ID_F, 3)

FIELD (FORB_PLMN_ID_1F)
		12, /* Number of supported networks by SAPI*/
		0
ENDFIELD (FORB_PLMN_ID_1F, 2)

FIELD (CHM_NOT_PRESENT)	0xFF, 0xFF
ENDFIELD (CHM_NOT_PRESENT, 2)

FIELD (LOC_UPD_ACCEPT_2)	0x70, 0x00,
	0x00, 0x00,
	0x05, 0x02, 0x21, 0xf3, 0x33, 0x00, 0x02,
	0x17, 0x05, 0xf0, 0x12, 0x34, 0x56, 0x78
ENDFIELD (LOC_UPD_ACCEPT_2, 18)

FIELD (LOC_UPD_ACCEPT_3)	0x78, 0x00,
	0x00, 0x00,
	0x05, 0x02, 0x21, 0xf3, 0x33, 0x00, 0x02,
	0x17, 0x05, 0xf0, 0x12, 0x34, 0x56, 0x78,
	0xA1
ENDFIELD (LOC_UPD_ACCEPT_3, 19)

FIELD (LOC_UPD_ACCEPT_4)	0x50, 0x00,
	0x00, 0x00,
	0x05, 0x02, 0x21, 0xf3, 0x33, 0x00, 0x02,
	0x17, 0x00, 0xA1
ENDFIELD (LOC_UPD_ACCEPT_4, 14)

FIELD (LOC_UPD_ACCEPT_5)	0x58, 0x00,
	0x00, 0x00,
	0x05, 0x02, 0x21, 0xf3, 0x33, 0x00, 0x02,
	0x17, 0x01, 0xF0,0xA1
ENDFIELD (LOC_UPD_ACCEPT_5, 15)

FIELD (LOC_UPD_ACCEPT_6)	0x40, 0x00,
	0x00, 0x00,
	0x05, 0x02, 0x21, 0xf3, 0x33, 0x00, 0x02,
	0xA1
ENDFIELD (LOC_UPD_ACCEPT_6, 12)

FIELD (LOC_UPD_ACCEPT_3_B)	0x70, 0x00,
	0x00, 0x00,
	0x05, 0x02, 0x21, 0xf3, 0x33, 0x00, 0x02,
	0x17, 0x05, 0xf0, 0x12, 0x34, 0x56, 0x78
ENDFIELD (LOC_UPD_ACCEPT_3_B, 18)

FIELD (LOC_UPD_ACCEPT_4_B)	0x48, 0x00,
	0x00, 0x00,
	0x05, 0x02, 0x21, 0xf3, 0x33, 0x00, 0x02,
	0x17, 0x00
ENDFIELD (LOC_UPD_ACCEPT_4_B, 13)

FIELD (LOC_UPD_ACCEPT_5_B)	0x50, 0x00,
	0x00, 0x00,
	0x05, 0x02, 0x21, 0xf3, 0x33, 0x00, 0x02,
	0x17, 0x01, 0xF0
ENDFIELD (LOC_UPD_ACCEPT_5_B, 14)

FIELD (LOC_UPD_ACCEPT_6_B)	0x38, 0x00,
	0x00, 0x00,
	0x05, 0x02, 0x21, 0xf3, 0x33, 0x00, 0x02
ENDFIELD (LOC_UPD_ACCEPT_6_B, 11)

DECLARATION (BCCH_INF_2)
DECLARATION (EMPTY_IMSI)
DECLARATION (IMEI_DIGITS)
DECLARATION (IMEI_DIGITS_BCD)
DECLARATION (IMEISV_DIGITS)
DECLARATION (IMSI_1233347114912)

DECLARATION (INT_IMSI_001010123456789)
DECLARATION (PRI_IMSI_001010123456789)
DECLARATION (SIM_IMSI_001010123456789)
DECLARATION (MSG_IMSI_001010123456789)
DECLARATION (MIMSI_001010123456789)
DECLARATION (INT_IMSI_001010011223344)
DECLARATION (PRI_IMSI_001010011223344)
DECLARATION (SIM_IMSI_001010011223344)
DECLARATION (MSG_IMSI_001010011223344)
DECLARATION (MIMSI_001010011223344)
DECLARATION (INT_IMSI_001019876543210)
DECLARATION (PRI_IMSI_001019876543210)
DECLARATION (SIM_IMSI_001019876543210)
DECLARATION (MSG_IMSI_001019876543210)
DECLARATION (MIMSI_001019876543210)

DECLARATION (IE_FOLLOW_PROCEED)

DECLARATION (IMEI_DIGITS_CODED)
DECLARATION (KC_DELETED)
DECLARATION (KC_DELETED_SIM)
DECLARATION (KC_EMPTY)
DECLARATION (KC_VALUE_EMPTY)
DECLARATION (KC_11223344)
DECLARATION (KC_11223344_SIM)
DECLARATION (LOC_DELETED)
DECLARATION (LOC_INFO_123_33_FEFF)
DECLARATION (LOC_INFO_LAC_NOT_ALLOWED)
DECLARATION (LOC_INFO_PLMN_NOT_ALLOW)
DECLARATION (MCC_123)
DECLARATION (MCC_NONE)
DECLARATION (MIMSI_1233347114912)
DECLARATION (SRES_1)
DECLARATION (SRES_1_CODED)
DECLARATION (MNC_31)
DECLARATION (MNC_32)
DECLARATION (MNC_33)
DECLARATION (MNC_44)
DECLARATION (MNC_NONE)
DECLARATION (RAND_1)
DECLARATION (RAND_1_P)
DECLARATION (TMSI_34125708)

- DECLARATION (AUTH_RAND_1)
DECLARATION (CC_MESSAGE)
DECLARATION (CIPH_KEY_NUM_01)
DECLARATION (CIPH_KEY_NUM_04)
DECLARATION (CIPH_KEY_NUM_RES)
DECLARATION (IDENT_TYPE_IMEI)
DECLARATION (IDENT_TYPE_IMEISV)
DECLARATION (IDENT_TYPE_IMSI)
DECLARATION (IDENT_TYPE_TMSI)
DECLARATION (LOC_AREA_ID_123_31_0002)
DECLARATION (LOC_AREA_ID_123_31_2147)
DECLARATION (LOC_AREA_ID_123_33_0001)
DECLARATION (LOC_AREA_ID_123_33_0002)
DECLARATION (LOC_AREA_ID_123_33_2147)
DECLARATION (LOC_AREA_ID_123_33_FEFF)
DECLARATION (LOC_AREA_ID_123_44_0002)
DECLARATION (LOC_UPD_TYPE_NORMAL)
DECLARATION (LOC_UPD_TYPE_NORMAL_FOL)
DECLARATION (LOC_UPD_TYPE_ATTACH)
DECLARATION (LOC_UPD_TYPE_PERIODIC)
DECLARATION (MOB_CLASS_1)
DECLARATION (MOB_CLASS_2)
DECLARATION (MOB_IDENT_IMEI)
DECLARATION (MOB_IDENT_IMEISV)
DECLARATION (MOB_IDENT_IMSI)
DECLARATION (MOB_IDENT_TMSI)
DECLARATION (MOB_IDENT_NEW_TMSI)

DECLARATION (ACC_CTRL_1)
DECLARATION (BCCH_INF_1)
DECLARATION (BCCH_INFO_1)
DECLARATION (BCCH_EMPTY_CHANNEL_LIST)
DECLARATION (BCCH_INFO_ECL)
DECLARATION (FORB_PLMN_NONE)
DECLARATION (FORB_PLMN_2)
DECLARATION (IMSI_FIELD_1)
DECLARATION (KCV_11223344)
DECLARATION (KCV_EMPTY)
DECLARATION (LOC_INFO_UPDATED_1)
DECLARATION (LOC_INFO_UPDATED_2)
DECLARATION (LOC_INFO_UPDATED_3)
DECLARATION (LOC_INFO_UPDATED_31_0002)
DECLARATION (LOC_INFO_UPDATED_31_2147)
DECLARATION (LOC_INFO_UPDATED_44)
DECLARATION (LOC_INFO_UPDATED_123_44_0002)
DECLARATION (LOC_INFO_UPDATED_4)
DECLARATION (LOC_INFO_UPDATED_5)
DECLARATION (LOC_INFO_DEL_SIM)
DECLARATION (LOC_INFO_DELETED)
DECLARATION (LOC_INFO_UPDATED_LPLMN)
DECLARATION (LOC_INFO_NOT_UPD_LPLMN)
DECLARATION (MM_INFO)
DECLARATION (MM_INFO_2)
DECLARATION (MM_INFO_ATT)
DECLARATION (MM_INFO_PER)
DECLARATION (MOB_ID_IMEI)
DECLARATION (MOB_ID_IMSI)
DECLARATION (MOB_ID_NEW_TMSI)
DECLARATION (MOB_ID_NO_ID)
DECLARATION (MS_CLASS_2)
DECLARATION (MS_CLASS_3_NOT_SET)
DECLARATION (OP_MODE_NO_SIM_NO_SERV)
DECLARATION (OP_MODE_NO_SIM_NO_SERV_M)
DECLARATION (OP_MODE_NO_SIM_LIM_SERV)
DECLARATION (OP_MODE_SIM_NO_SERV)
DECLARATION (OP_MODE_SIM_NO_SERV1)
DECLARATION (OP_MODE_SIM_NO_SERV4)
DECLARATION (OP_MODE_SIM_LIM_SERV4)
DECLARATION (OP_MODE_SIM_NO_SERV_M)
DECLARATION (OP_MODE_SIM_NO_SERV_M1)
DECLARATION (OP_MODE_SIM_NO_SERV_M2)
DECLARATION (OP_MODE_SIM_NO_SERV_A)
DECLARATION (OP_MODE_SIM_LIM_SERV_A)
DECLARATION (OP_MODE_SIM_SERV_A)
DECLARATION (OP_MODE_SIM_LIM_SERV)
DECLARATION (OP_MODE_SIM)
DECLARATION (OP_MODE_SIM_A)
DECLARATION (OP_MODE_SIM_M)
DECLARATION (OP_MODE_TEST_SIM)
DECLARATION (OP_MODE_TEST_SIM_NO_SERV)
DECLARATION (OP_MODE_NET_REQUEST)
DECLARATION (OP_MODE_SIM_1)
DECLARATION (OP_MODE_SIM_2)
DECLARATION (OP_MODE_SIM_3)
DECLARATION (OP_MODE_SIM_4)
DECLARATION (OP_SIM_MAN_MMI_SRCH_FS)
DECLARATION (OP_SIM_AUTO_MMI_SRCH_NS)
DECLARATION (OP_SIM_MAN_MMI_SRCH_NS)
DECLARATION (PLMN_123_33)
DECLARATION (PLMN_123_32)
DECLARATION (PLMN_123_31)
DECLARATION (PLMN_123_44)
DECLARATION (PLMN_NO_ID)
DECLARATION (PLMN_44)
DECLARATION (PREF_PLMN_NONE)
DECLARATION (PD_CC_AND_SAPI_0)
DECLARATION (FULL_NET_NAME)
DECLARATION (FULL_NAME)
DECLARATION (SHORT_NET_NAME)
DECLARATION (SHORT_NAME)
DECLARATION (NET_TEXT_A)
DECLARATION (NET_TZ)
DECLARATION (NTZ_MET)
DECLARATION (NET_TZ_AND_TIME)
DECLARATION (TIME)
DECLARATION (PLMN_SEL_NLPTT_PROX)
DECLARATION (EF_MSISDN)
DECLARATION (EF_PLMN_SEL)
DECLARATION (EF_PLMN_SEL_THPLMN)
DECLARATION (SIM_THPLMN_FF)

BYTE	NO_PLMN_FOUND	0
BYTE	ONE_PLMN_FOUND	1
BYTE	TWO_PLMN_FOUND	2

- /*
 * accc (Access control)
 */
SHORT	ACC_2143	0x2143
SHORT	ACC_CLASS_0000	0x0000

- /*
 *  bcc (base station colour code)
 */
BYTE	BCC_0	0x00

- /* 
 * c_loc
 */
BYTE C_LOC_LEN_11	0x0B

- /*
 * cid (Cell identity)
 */
SHORT	CELL_ID_1122	0x1122
SHORT	CELL_ID_1123	0x1123
SHORT	CELL_ID_0045	0x0045

- /*
 * cksn
 */
BYTE	CKSN_NO_KEY	0x07
BYTE	CKSN_00	0x00
BYTE	CKSN_01	0x01
BYTE	CKSN_04	0x04

- /*
 * lac (Location area code)
 */
SHORT	LAC_0001	0x0001
SHORT	LAC_0002	0x0002
SHORT	LAC_2147	0x2147
SHORT	LAC_FEFF	0xFFFE

- /*
 * c_loc (length of loc_info)
 */
BYTE	LOC_INFO_LEN_11	0x0B

- /*
 * lut (Location updating type)
 */
BYTE	LUT_NORMAL	0x00
BYTE	LUT_ATTACH	0x02
BYTE	LUT_PERIODIC	0x01

- /* 
 * ncc (National colour code)
 */
BYTE	NCC_3	0x03

- /*
 * nreg_cs
 */
SHORT	NREG_AUTH_FAIL 	0x8439

- /*
 * Reject cause
 */
BYTE	RC_RETRY_UPON_NEW_CELL	0x32
BYTE	MMR_RC_RETRY_UPON_NEW_CELL	0x30 /* 0x30 - 0x3f => 0x30 */

- /*
 * prio (priority)
 */
BYTE	PRIO_NORM_CALL	0x0

/*
 *  thplmn (HPLMN time))
 */
BYTE	THPLMN_01	0x01

BYTE	THPLMN_FF	0xFF
BEGINARRAY (SIM_THPLMN_FF, 1)
	0xFF
ENDARRAY
BYTE	LENGTH_THPLMN	0x01

/*
 * ss (SS screening indicator)
 */
BYTE	SS_IND_3	0x03

- /*
 * ti (Transaction identifier)
 */
BYTE	TI_0	0x00
BYTE	TI_2	0x02
BYTE	TI_3	0x03
BYTE	TI_4	0x04
BYTE	TI_5	0x05
BYTE	TI_6	0x06

- /*
 * ti (Transaction identifier)
 */
BYTE	TMSI_34125708_ULONG	0x34125708

- /*
 * T3212 Periodic Time
 */
BYTE	T3212_6_MIN	1

/* 
 * Definitions for MM INFORMATION
 */
BYTE	YEAR_00	0x00
BYTE	MONTH_02	0x02
BYTE	DAY_08	0x08
BYTE	HOUR_15	0x15
BYTE	MINUTE_55	0x55
BYTE	SECOND_10	0x10
BYTE	TZ_MET	0x01
BYTE	CS_UCS2	0x00
BYTE	NUM_SPARE_0	0x00

/*
 * Definitions for EM
 */
LONG	Bitm_1			0x1000

LONG	Bitm_2			0x2000

BYTE	EM_ENTITY		0x04

/*
 * Definitions for SAT
 */
SHORT	FILE_CHG_1	0x01	/* Number of changed files */
SHORT	FILE_CHG_2	0x02
BYTE	OFFSET_0	0x0000	/* Offset of data field */
BYTE	MAX_LEN_PREF_PLMN	(96)
BYTE	PLMN_SEL_LENGTH	0x06
BEGINARRAY (PLMN_SEL_NLPTT_PROX, 6)
	0x02, 0xF4, 0x80,
	0x02, 0xF6, 0x10
ENDARRAY

BEGINARRAY (EF_MSISDN, 2)
	0x40, 0x6f
ENDARRAY

BEGINARRAY (EF_PLMN_SEL, 2)
	0x30, 0x6f
ENDARRAY

BEGINARRAY (EF_PLMN_SEL_THPLMN, 4)
	0x30, 0x6f,
	0x31, 0x6f
ENDARRAY

- /* 
 * empty IMSI
 */
BEGINARRAY (EMPTY_IMSI, 16)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

- /* 
 * imei (IMEI BCD)
 */
BEGINARRAY (IMEI_DIGITS_BCD, 16) 0x01, 0x03, 0x05, 0x07, 0x09, 0x00, 0x02, 0x04,
	0x06, 0x08, 0x01, 0x01, 0x02, 0x02, 0xFF, 0xFF ENDARRAY
BEGINARRAY (IMEI_DIGITS_CODED, 15) 0x0E, 0x01, 0x03, 0x05, 0x07, 0x09, 0x00, 0x02,
	0x04, 0x06, 0x08, 0x01, 0x01, 0x02, 0x02 ENDARRAY

- /*
 * imei (IMEI - Mobile identity)
 */
BEGINARRAY (IMEISV_DIGITS, 16)
	0x01, 0x03, 0x05, 0x07, 0x09, 0x00, 0x02, 0x04,
	0x06, 0x08, 0x01, 0x01, 0x02, 0x02, 0x07, 0x08
ENDARRAY

- BEGINARRAY (IMEI_DIGITS, 15)
	0x01, 0x03, 0x05, 0x07, 0x09, 0x00, 0x02, 0x04,
	0x06, 0x08, 0x01, 0x01, 0x02, 0x02, 0x00
ENDARRAY

- /*
 * imsi 
 */
BEGINARRAY (IMSI_1233347114912, 16) 0x01, 0x02, 0x03, 0x03, 0x03, 0x04, 0x07, 0x01,
	0x01, 0x04, 0x09, 0x01, 0x02, 0x0F, 0x0F, 0xFF ENDARRAY

BEGINARRAY (LOC_DELETED, 11) 0xFF, 0xFF, 0xFF, 0xFF, 0x21, 0xF3, 0x33,
	0x00, 0x02, 0x00, 0x00 ENDARRAY

- BEGINARRAY (MIMSI_1233347114912, 13) 0x01, 0x02, 0x03, 0x03, 0x03, 0x04, 0x07, 0x01,
	0x01, 0x04, 0x09, 0x01, 0x02 ENDARRAY

- /* 
 * imsi "001010123456789" as of GSM 11.11 subclause 27.22.4.7.4.2
 */
BEGINARRAY (SIM_IMSI_001010123456789, 9)
	0x08, 0x09, 0x10, 0x10, 0x10, 0x32, 0x54, 0x76, 0x98
ENDARRAY
BEGINARRAY (INT_IMSI_001010123456789, 16)
	0x00, 0x00, 0x01, 0x00, 0x01,
	0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0xFF
ENDARRAY
BEGIN_PSTRUCT("imsi", PRI_IMSI_001010123456789)
	SET_COMP ("v_mid",	V_MID_PRES)
	SET_COMP ("id_type",	TYPE_IMSI)
	SET_COMP ("id",	INT_IMSI_001010123456789)
	SKIP_COMP ("tmsi_dig")
ENDSTRUCT
BEGINARRAY (MIMSI_001010123456789, 15)
	0x00, 0x00, 0x01, 0x00, 0x01,
	0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09
ENDARRAY
BEGIN_MSTRUCT("mob_id", MSG_IMSI_001010123456789)
	SET_COMP ("ident_type",	ID_TYPE_IMSI)
	SET_COMP ("odd_even",	ODD)
	SET_COMP ("ident_dig",	MIMSI_001010123456789)
	SKIP_COMP ("tmsi")
	SKIP_COMP ("dmy")
ENDSTRUCT

- BEGIN_MSTRUCT("follow_proceed", IE_FOLLOW_PROCEED)
ENDSTRUCT

- /*
 * imsi "001 01 00 11223344" as of GSM 11.11 subclause 27.22.4.7.4.2
 */
BEGINARRAY (SIM_IMSI_001010011223344, 9) 
	0x08, 0x09, 0x10, 0x10, 0x00, 0x11, 0x22, 0x33, 0x44
ENDARRAY
BEGINARRAY (INT_IMSI_001010011223344, 16) 
	0x00, 0x00, 0x01, 0x00, 0x01,
	0x00, 0x00, 0x01, 0x01, 0x02, 0x02, 0x03, 0x03, 0x04, 0x04, 0xFF
ENDARRAY
BEGIN_PSTRUCT("imsi", PRI_IMSI_001010011223344)
	SET_COMP ("v_mid",	V_MID_PRES)
	SET_COMP ("id_type",	TYPE_IMSI)
	SET_COMP ("id",	INT_IMSI_001010011223344)
	SKIP_COMP ("tmsi_dig")
ENDSTRUCT
BEGINARRAY (MIMSI_001010011223344, 15)
	0x00, 0x00, 0x01, 0x00, 0x01,
	0x00, 0x00, 0x01, 0x01, 0x02, 0x02, 0x03, 0x03, 0x04, 0x04
ENDARRAY
BEGIN_MSTRUCT("mob_id", MSG_IMSI_001010011223344)
	SET_COMP ("ident_type",	ID_TYPE_IMSI)
	SET_COMP ("odd_even",	ODD)
	SET_COMP ("ident_dig",	MIMSI_001010011223344)
	SKIP_COMP ("tmsi")
	SKIP_COMP ("dmy")
ENDSTRUCT

- /*
 * imsi "001 01 9876543210" as of GSM 11.11 subclause 27.22.4.7.4.2 
 */
BEGINARRAY (SIM_IMSI_001019876543210, 9)
	0x08, 0x09, 0x10, 0x10, 0x89, 0x67, 0x45, 0x23, 0x01
ENDARRAY
BEGINARRAY (INT_IMSI_001019876543210, 16)
	0x00, 0x00, 0x01, 0x00, 0x01,
	0x09, 0x08, 0x07, 0x06, 0x05, 0x04, 0x03, 0x02, 0x01, 0x00, 0xFF
ENDARRAY
BEGIN_PSTRUCT("imsi", PRI_IMSI_001019876543210)
	SET_COMP ("v_mid",	V_MID_PRES)
	SET_COMP ("id_type",	TYPE_IMSI)
	SET_COMP ("id",	INT_IMSI_001019876543210)
	SKIP_COMP ("tmsi_dig")
ENDSTRUCT
BEGINARRAY (MIMSI_001019876543210, 15)
	0x00, 0x00, 0x01, 0x00, 0x01,
	0x09, 0x08, 0x07, 0x06, 0x05, 0x04, 0x03, 0x02, 0x01, 0x00, 0xFF
ENDARRAY
BEGIN_MSTRUCT("mob_id", MSG_IMSI_001019876543210)
	SET_COMP ("ident_type",	ID_TYPE_IMSI)
	SET_COMP ("odd_even",	ODD)
	SET_COMP ("ident_dig",	MIMSI_001019876543210)
	SKIP_COMP ("tmsi")
	SKIP_COMP ("dmy")
ENDSTRUCT

- /*
 * sres (authentication parameter sres)
 */
BEGINARRAY (SRES_1, 4) 0x01, 0x02, 0x03, 0x04 ENDARRAY
BEGINARRAY (SRES_1_CODED, 8) 0x20, 0x00, 0x00, 0x00, 0x01, 0x02, 0x03, 0x04 ENDARRAY

- /*
 *  tmsi 
 */
BEGINARRAY (TMSI_34125708, 8) 0x20, 0x00, 0x00, 0x00, 0x34, 0x12, 0x57, 0x08 ENDARRAY

- /*
 * kc (KC value)
 */
BEGINARRAY (KC_DELETED, 8)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY
BEGINARRAY (KC_DELETED_SIM, 9)
	0x08, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF
ENDARRAY
BEGINARRAY (KC_11223344, 8)
	0x01, 0x01, 0x02, 0x02, 0x03, 0x03, 0x04, 0x04
ENDARRAY
BEGINARRAY (KC_11223344_SIM, 9)
	0x08, 0x01, 0x01, 0x02, 0x02, 0x03, 0x03, 0x04, 0x04
ENDARRAY

/*
 *  mcc (Mobile country code)
 */
BEGINARRAY (MCC_NONE, 3)	0x0F, 0x0F, 0x0F ENDARRAY 
BEGINARRAY (MCC_123, 3) 0x01, 0x02, 0x03 ENDARRAY

/*
 * mnc (Mobile network code) 
 */
BEGINARRAY (MNC_NONE, 3) 0x0F, 0x0F, 0x0F ENDARRAY
BEGINARRAY (MNC_33, 2) 0x03, 0x03 ENDARRAY
BEGINARRAY (MNC_32, 2) 0x02, 0x03 ENDARRAY
BEGINARRAY (MNC_31, 2) 0x03, 0x01 ENDARRAY
BEGINARRAY (MNC_44, 2) 0x04, 0x04 ENDARRAY

/*
 * imsi_field (International mobile subscriber identity - GSM 11.11)
 */
BEGINARRAY (IMSI_FIELD_1, 9)
	0x08,	/* Length */
	0x19, 0x32, 0x33,	/* MCC=123, MNC=33 is HPLMN*/
	0x74, 0x11, 0x94, 0x21, 0xFF
ENDARRAY

- /* 
 * loc_info (Location Info - GSM 11.11)
 */

- BEGINARRAY (LOC_INFO_UPDATED_1, 12)
	0x0B,	/* Length */
	0xFF, 0xFF, 0xFF, 0xFF,	/* TMSI */
	0x21, 0xF3, 0x33,0x21, 0x47,	/* LAI: MCC=123, MNC=33, LAI=2147 */
	0x00,	/* TMSI TIME */
	0x00	/* Location update status */
ENDARRAY
BEGINARRAY (LOC_INFO_UPDATED_2, 12)
	0x0B,
	0xFF, 0xFF, 0xFF, 0xFF,
	0x21, 0xF3, 0x33, 0x00, 0x02,	/* LAI: MCC=123, MNC=33, LAI=0002 */
	0x00,
	0x00
ENDARRAY
BEGINARRAY (LOC_INFO_UPDATED_3, 12)
	0x0B,
	0x34, 0x12, 0x57, 0x08,
	0x21, 0xF3, 0x33, 0x00, 0x02,	/* LAI: MCC=123, MNC=33, LAI=0002 */
	0x00,
	0x00
ENDARRAY
BEGINARRAY (LOC_INFO_UPDATED_31_2147, 12)
	0x0B,
	0x34, 0x12, 0x57, 0x08,
	0x21, 0xF3, 0x13, 0x21, 0x47,	/* LAI: MCC= 123, MNC=31, LAI=2147 */
	0x00,
	0x00
ENDARRAY
BEGINARRAY (LOC_INFO_UPDATED_31_0002, 12)
	0x0B,
	0x34, 0x12, 0x57, 0x08,
	0x21, 0xF3, 0x13, 0x00, 0x02,	/* LAI: MCC= 123, MNC=31, LAI=0002 */
	0x00,
	0x00
ENDARRAY
BEGINARRAY (LOC_INFO_UPDATED_44, 12)
	0x0B,
	0x34, 0x12, 0x57, 0x08,
	0x21, 0xF3, 0x44, 0x00, 0x02,	/* LAI: MCC=123, MNC=44, LAI=0002 */
	0x00,
	0x00
ENDARRAY
BEGINARRAY (LOC_INFO_UPDATED_123_44_0002, 12)
	0x0B,
	0xFF, 0xFF, 0xFF, 0xFF,
	0x21, 0xF3, 0x44, 0x00, 0x02,	/* LAI: MCC=123, MNC=44, LAI=0002 */
	0x00,
	0x00
ENDARRAY
BEGINARRAY (LOC_INFO_UPDATED_4, 12)
	0x0B,
	0x34, 0x12, 0x57, 0x08,
	0x21, 0xF3, 0x33, 0x00, 0x01,	/* LAI: MCC=123, MNC=33, LAI=0001 */
	0x00,
	0x00
ENDARRAY
BEGINARRAY (LOC_INFO_UPDATED_5, 12)
	0x0B,
	0x34, 0x12, 0x57, 0x08,
	0x21, 0xF3, 0x33, 0x21, 0x47,	/* LAI: MCC=123, MNC=33, LAI=2147 */
	0x00,
	0x00
ENDARRAY
BEGINARRAY (LOC_INFO_DELETED, 12)
	0x0B,
	0xFF, 0xFF, 0xFF, 0xFF,
	0x21, 0xF3, 0x33,0x00, 0x02,	/* LAI: MCC=123, MNC=33, LAI=0002 */
	0x00,
	0x00
ENDARRAY
BEGINARRAY (LOC_INFO_123_33_FEFF, 12)
	0x0B,
	0xFF, 0xFF, 0xFF, 0xFF,
	0x21, 0xF3, 0x33, 0xFF, 0xFE,	/* LAI: MCC=123, MNC=33, LAI=deleted */
	0x00,
	0x01
ENDARRAY
BEGINARRAY (LOC_INFO_LAC_NOT_ALLOWED, 12)
	0x0B,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF, 0xFE,	/* LAI deleted */
	0x00,
	0x03
ENDARRAY
BEGINARRAY (LOC_INFO_PLMN_NOT_ALLOW, 12)
	0x0B,
	0xFF, 0xFF, 0xFF, 0xFF,
	0x21, 0xF3, 0x33, 0xFF, 0xFE,	/* LAI: MCC=123, MNC=33, LAI=deleted */
	0x00,
	0x03
ENDARRAY
BEGINARRAY (LOC_INFO_UPDATED_LPLMN, 12)
	0x0B,	 /* Length */
	0xFF, 0xFF, 0xFF, 0xFF,	/* TMSI */
	0x21, 0xF3, 0x13, 0x21, 0x47,	/* LAI: MNC=123, MCC=31, LAI=2147 */
	0x00,	/* TMSI TIME */
	0x00	/* Location update status */
ENDARRAY
BEGINARRAY (LOC_INFO_NOT_UPD_LPLMN, 12)
	0x0B,
	0xFF, 0xFF, 0xFF, 0xFF,
	0x21, 0xF3, 0x32, 0x00, 0x02,	/* LAI: MNC=123, MCC=32, LAI=0002 */
	0x00,
	0x01
ENDARRAY

/*
 * pref_plmn (Preferred plmns)
 */
BEGINARRAY (PREF_PLMN_NONE, 25)
	0x18,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF
ENDARRAY

/*
 * forb_plmn (Forbidden plmns)
 */
BEGINARRAY (FORB_PLMN_NONE, 13)
	0x0C,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF
ENDARRAY
BEGINARRAY (FORB_PLMN_2, 13)
	0x0C,
	0x21, 0xF3,0x23,
	0xFF, 0xFF, 0xFF,
	0xFF,0xFF, 0xFF,
	0xFF, 0xFF, 0xFF
ENDARRAY

/*
 * acc_ctrl (Access control)
 */
BEGINARRAY (ACC_CTRL_1, 3) 0x02, 0x21, 0x43 ENDARRAY

- /*
 * bcch_inf (BCCH information from SIM card)
 */
BEGINARRAY (BCCH_INF_1, 17) 0x10, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00 ENDARRAY

- BEGINARRAY (BCCH_INF_2, 17) 0x10, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00 ENDARRAY

- /*
 * rand (Authentication parameter RAND)
 */
BEGINARRAY (RAND_1, 16) 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08,
	0x09, 0x0A, 0x0B, 0x0C, 0x0D, 0x0E, 0x0F, 0x10 ENDARRAY
BEGINARRAY (RAND_1_P, 17) 0x10, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08,
	0x09, 0x0A, 0x0B, 0x0C, 0x0D, 0x0E, 0x0F, 0x10 ENDARRAY

/*
 * CC Message
 *
 *	0x10, 0x00,	length in bits
 *	0x18, 0x00,	offset in bits
 *	0x00, 0x00, 0x00,	not used
 *	0x23,	ti = 2, pd = CC
 *	0x01	msg_type = CC_ALERTING
 */

- BEGINARRAY (CC_MESSAGE, 9)	0x10, 0x00,
		0x18, 0x00,
		0x00, 0x00, 0x00,
		0x23,
		0x01
		ENDARRAY

BEGINARRAY (NET_TEXT_A, 20)
	0x00, 0x01,
	0x00, 0x02,
	0x00, 0x03,
	0x00, 0x04,
	0x00, 0x05,
	0x00, 0x06,
	0x00, 0x07,
	0x00, 0x08,
	0x00, 0x09,
	0x00, 0x0a
ENDARRAY
BYTE	NET_TEXT_LEN_A	20

/*
 *  auth_rand (Ciphering key sequence number)
 */
BEGIN_MSTRUCT ("auth_rand", AUTH_RAND_1)
	SET_COMP ("rand",	RAND_1)
ENDSTRUCT

/*
 * ciph_key_num (Ciphering key sequence number)
 */
BEGIN_MSTRUCT ("ciph_key_num", CIPH_KEY_NUM_RES)
	SET_COMP ("key_seq",	CKSN_RES)
ENDSTRUCT
 BEGIN_MSTRUCT ("ciph_key_num", CIPH_KEY_NUM_01)
	SET_COMP ("key_seq",	CKSN_01)
ENDSTRUCT

BEGIN_MSTRUCT ("ciph_key_num", CIPH_KEY_NUM_04)
	SET_COMP ("key_seq",	CKSN_04)
ENDSTRUCT

/*
 * ident (identity type)
 */
BEGIN_MSTRUCT("ident", IDENT_TYPE_IMEI)
	SET_COMP ("ident_type",	ID_TYPE_IMEI)
ENDSTRUCT

BEGIN_MSTRUCT("ident", IDENT_TYPE_IMEISV)
	SET_COMP ("ident_type",	ID_TYPE_IMEISV)
ENDSTRUCT

BEGIN_MSTRUCT("ident", IDENT_TYPE_IMSI)
	SET_COMP ("ident_type",	ID_TYPE_IMSI)
ENDSTRUCT

BEGIN_MSTRUCT("ident", IDENT_TYPE_TMSI)
	SET_COMP ("ident_type",	ID_TYPE_TMSI)
ENDSTRUCT

/*
 * loc_upd_type(Location updating type)
 */
BEGIN_MSTRUCT ("loc_upd_type", LOC_UPD_TYPE_NORMAL)
	SET_COMP ("follow",	FOR_PENDING_NO)
	SET_COMP ("lut",	LUT_NORMAL)
ENDSTRUCT
BEGIN_MSTRUCT ("loc_upd_type", LOC_UPD_TYPE_NORMAL_FOL)
	SET_COMP ("follow",	FOR_PENDING_YES)
	SET_COMP ("lut",	LUT_NORMAL)
ENDSTRUCT
BEGIN_MSTRUCT ("loc_upd_type", LOC_UPD_TYPE_ATTACH)
	SET_COMP ("follow",	FOR_PENDING_NO)
	SET_COMP ("lut",	LUT_ATTACH)
ENDSTRUCT
BEGIN_MSTRUCT ("loc_upd_type", LOC_UPD_TYPE_PERIODIC)
	SET_COMP ("follow",	FOR_PENDING_NO)
	SET_COMP ("lut",	LUT_PERIODIC)
ENDSTRUCT
/*
 * loc_area_ident (Location area identification)
 */
BEGIN_MSTRUCT ("loc_area_ident", LOC_AREA_ID_123_33_0001)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_33)
	SET_COMP ("lac",	LAC_0001)
ENDSTRUCT
BEGIN_MSTRUCT ("loc_area_ident", LOC_AREA_ID_123_33_0002)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_33)
	SET_COMP ("lac",	LAC_0002)
ENDSTRUCT
BEGIN_MSTRUCT ("loc_area_ident", LOC_AREA_ID_123_31_0002)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_31)
	SET_COMP ("lac",	LAC_0002)
ENDSTRUCT
BEGIN_MSTRUCT ("loc_area_ident", LOC_AREA_ID_123_31_2147)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_31)
	SET_COMP ("lac",	LAC_2147)
ENDSTRUCT
BEGIN_MSTRUCT ("loc_area_ident", LOC_AREA_ID_123_33_2147)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_33)
	SET_COMP ("lac",	LAC_2147)
ENDSTRUCT
BEGIN_MSTRUCT ("loc_area_ident", LOC_AREA_ID_123_33_FEFF)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_33)
	SET_COMP ("lac",	LAC_FEFF)
ENDSTRUCT
BEGIN_MSTRUCT ("loc_area_ident", LOC_AREA_ID_123_44_0002)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_44)
	SET_COMP ("lac",	LAC_0002)
ENDSTRUCT

/*
 * mob_ident (Mobile identity)
 */
BEGIN_MSTRUCT("mob_id", MOB_IDENT_IMSI)
	SET_COMP ("ident_type",	ID_TYPE_IMSI)
	SET_COMP ("odd_even",	ODD)
	SET_COMP ("ident_dig",	MIMSI_1233347114912)
	SKIP_COMP ("tmsi")
	SKIP_COMP ("dmy")
ENDSTRUCT
BEGIN_MSTRUCT("mob_id", MOB_IDENT_TMSI)
	SET_COMP ("ident_type",	ID_TYPE_TMSI)
	SET_COMP ("odd_even",	EVEN)
	SKIP_COMP ("ident_dig")
	SET_COMP ("tmsi", TMSI_34125708)
	SKIP_COMP ("dmy")
ENDSTRUCT
BEGIN_MSTRUCT("mob_id", MOB_IDENT_IMEI)
	SET_COMP ("ident_type",	ID_TYPE_IMEI)
	SET_COMP ("odd_even",	ODD)
	SET_COMP ("ident_dig",	IMEI_DIGITS)
	SKIP_COMP ("tmsi")
	SKIP_COMP ("dmy")
ENDSTRUCT

BEGIN_MSTRUCT("mob_id", MOB_IDENT_IMEISV)
	SET_COMP ("ident_type",	ID_TYPE_IMEISV)
	SET_COMP ("odd_even",	EVEN)
	SET_COMP ("ident_dig",	IMEISV_DIGITS)
	SKIP_COMP ("tmsi")
	SKIP_COMP ("dmy")
ENDSTRUCT

/*
 * mob_class_1 (Mobile station Classmark 1)
 */
BEGIN_MSTRUCT ("mob_class_1", MOB_CLASS_1)
	SET_COMP ("rev_lev",	PHASE_2)
	SET_COMP ("es_ind",	ES_IMPL_YES)
	SET_COMP ("a5_1",	A5_1_YES)
	SET_COMP ("rf_pow_cap",	RF_CLASS_2)
ENDSTRUCT

/*
 * mob_class_2 (Mobile station Classmark 2)
 */
BEGIN_MSTRUCT ("mob_class_2", MOB_CLASS_2)
	SET_COMP ("rev_lev",	PHASE_2)
	SET_COMP ("es_ind",	ES_IMPL_YES)
	SET_COMP ("a5_1",	A5_1_YES)
	SET_COMP ("rf_pow_cap",	RF_CLASS_2)
	SET_COMP ("ps_cap",	PS_CAP_YES)
	SET_COMP ("ss_screen",	SS_IND_3)
	SET_COMP ("sm_cap",	SM_CAP_YES)
	SET_COMP ("freq_cap",	FREQ_CAP_NO)
	SET_COMP ("class_3",	MC2_CLASS_3_NO)
	SET_COMP("cmsp",	CMSP_SUPPORTED_YES)
	SET_COMP ("a5_3",	A5_3_NO)
	SET_COMP ("a5_2",	A5_2_NO)
ENDSTRUCT

BEGIN_MSTRUCT("pd_and_sapi", PD_CC_AND_SAPI_0)
	SET_COMP("sapi",	SAPI_0)
	SET_COMP("pd",	PD_CC)
ENDSTRUCT

/*
 * mob_id (Mobile identity)
 */
BEGIN_MSTRUCT("mob_id", MOB_IDENT_NEW_TMSI)
	SET_COMP ("ident_type",	ID_TYPE_TMSI)
	SKIP_COMP ("odd_even")
	SKIP_COMP ("ident_dig")
	SET_COMP ("tmsi",	TMSI_34125708)
	SKIP_COMP ("dmy")
ENDSTRUCT

BEGIN_MSTRUCT("full_net_name", FULL_NET_NAME)
	SET_COMP ("cs",	CS_UCS2)
	SET_COMP ("add_ci",	ADD_CI_NO)
	SET_COMP ("num_spare",	NUM_SPARE_0)
	SET_COMP ("text",	NET_TEXT_A)
ENDSTRUCT

BEGIN_MSTRUCT("short_net_name", SHORT_NET_NAME)
	SET_COMP ("cs",	CS_UCS2)
	SET_COMP ("add_ci",	ADD_CI_NO)
	SET_COMP ("num_spare",	NUM_SPARE_0)
	SET_COMP ("text",	NET_TEXT_A)
ENDSTRUCT

BEGIN_MSTRUCT("net_tz", NET_TZ)
	SET_COMP ("tz",	TZ_MET)
ENDSTRUCT

BEGIN_MSTRUCT ("net_tz_and_time", NET_TZ_AND_TIME)
	SET_COMP("year",	YEAR_00)
	SET_COMP("month",	MONTH_02)
	SET_COMP("day",	DAY_08)
	SET_COMP("hour",	HOUR_15)
	SET_COMP("minute",	MINUTE_55)
	SET_COMP("second",	SECOND_10)
	SET_COMP ("tz",	TZ_MET)
ENDSTRUCT

/*
 * bcch_info(BCCH information - RR)
 */
BEGIN_PSTRUCT("bcch_info", BCCH_INFO_1)
	SET_COMP ("v_bcch",	V_BCCH_NOT_PRES)
	SKIP_COMP ("bcch")
ENDSTRUCT

BEGINARRAY (BCCH_EMPTY_CHANNEL_LIST, 16)
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL,
	BCCH_CH_NOT_INCL
ENDARRAY

BEGIN_PSTRUCT("bcch_info", BCCH_INFO_ECL)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	BCCH_EMPTY_CHANNEL_LIST)
ENDSTRUCT

/*
 * imei/imsi/tmsi (Mobile identity)
 */
BEGIN_PSTRUCT("imsi", MOB_ID_NO_ID)
	SET_COMP ("v_mid",	V_MID_NOT_PRES)
	SET_COMP ("id_type",	TYPE_NO_ID)
	SKIP_COMP("id")
	SKIP_COMP ("tmsi_dig")
ENDSTRUCT

BEGIN_PSTRUCT("imsi", MOB_ID_IMSI)
	SET_COMP ("v_mid",	V_MID_PRES)
	SET_COMP ("id_type",	TYPE_IMSI)
	SET_COMP ("id",	IMSI_1233347114912)
	SKIP_COMP ("tmsi_dig")
ENDSTRUCT

BEGIN_PSTRUCT("tmsi", MOB_ID_NEW_TMSI)
	SET_COMP ("v_mid",	V_MID_PRES)
	SET_COMP ("id_type",	TYPE_TMSI)
	SKIP_COMP ("id")
	SET_COMP ("tmsi_dig",	TMSI_34125708_ULONG)
ENDSTRUCT

/*
 * kcv (Authentication key)
 */
BEGINARRAY (KC_EMPTY, 10)
	9, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0x07
ENDARRAY

BEGINARRAY (KC_VALUE_EMPTY, 8)
	0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF
ENDARRAY

/* 
 * kcv (Authentication key)
 */
BEGIN_PSTRUCT("kcv", KCV_11223344)
	SET_COMP ("v_kc",	V_KC_PRES)
	SET_COMP ("kc",	KC_11223344)
ENDSTRUCT

BEGIN_PSTRUCT("kcv", KCV_EMPTY)
	SET_COMP ("v_kc",	V_KC_PRES)
	SET_COMP("kc",	KC_VALUE_EMPTY)
ENDSTRUCT

/*
 * loc_info (Location info)
 */
BEGIN_PSTRUCT("loc_info", LOC_INFO_DEL_SIM)
	SET_COMP ("c_loc",	LOC_INFO_LEN_11)
	SET_COMP ("loc",	LOC_DELETED)
ENDSTRUCT

/*
 * mm_info (MM information)
 */
BEGIN_PSTRUCT("mm_info", MM_INFO)
	SET_COMP ("valid",	MM_INFO_PRES)
	SET_COMP ("la",	LA_NOT_IN_FRBD_LST_INCL)
	SET_COMP ("att",	ATT_NOT_ALLOW)
	SET_COMP ("re",	RE_NOT_ALLOW)
	SKIP_COMP ("band")
	SET_COMP ("ncc",	NCC_3)
	SET_COMP ("bcc",	BCC_0)
	SET_COMP ("t3212",	T3212_NO_PRD_UPDAT)
ENDSTRUCT
BEGIN_PSTRUCT("mm_info", MM_INFO_2)
	SET_COMP ("valid",	MM_INFO_PRES)
	SET_COMP ("la",	LA_NOT_IN_FRBD_LST_INCL)
	SET_COMP ("att",	ATT_NOT_ALLOW)
	SET_COMP ("re",	RE_ALLOW)
	SKIP_COMP ("band")
	SET_COMP ("ncc",	NCC_3)
	SET_COMP ("bcc",	BCC_0)
	SET_COMP ("t3212",	T3212_NO_PRD_UPDAT)
ENDSTRUCT

BEGIN_PSTRUCT("mm_info", MM_INFO_ATT)
	SET_COMP ("valid",	MM_INFO_PRES)
	SET_COMP ("la",	LA_NOT_IN_FRBD_LST_INCL)
	SET_COMP ("att",	ATT_ALLOW)
	SET_COMP ("re",	RE_NOT_ALLOW)
	SKIP_COMP ("band")
	SET_COMP ("ncc",	NCC_3)
	SET_COMP ("bcc",	BCC_0)
	SET_COMP ("t3212",	T3212_NO_PRD_UPDAT)
ENDSTRUCT
BEGIN_PSTRUCT("mm_info", MM_INFO_PER)
	SET_COMP ("valid",	MM_INFO_PRES)
	SET_COMP ("la",	LA_NOT_IN_FRBD_LST_INCL)
	SET_COMP ("att",	ATT_NOT_ALLOW)
	SET_COMP ("re",	RE_NOT_ALLOW)
	SKIP_COMP ("band")
	SET_COMP ("ncc",	NCC_3)
	SET_COMP ("bcc",	BCC_0)
	SET_COMP ("t3212",	T3212_6_MIN)
ENDSTRUCT
/*
 * op (Operation mode)
 */
BEGIN_PSTRUCT ("op", OP_MODE_SIM)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	FULL_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_A)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_M)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	FULL_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_SERV_A)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	FULL_SERVICE)
ENDSTRUCT

BYTE	NORMAL_SIM_INS	0x00
/* The following byte is documented in SIM.DOC as 0x01, to be corrected sometime */
BYTE	TEST_SIM_INS	0x81

BEGIN_PSTRUCT ("op", OP_MODE_SIM_NO_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_NO_SERV1)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SKIP_COMP ("func")
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_NO_SERV4)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_LIM_SERV4)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	LIMITED_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_NO_SERV_M)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_NO_SERV_M1)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SKIP_COMP ("func")
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_NO_SERV_M2)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_NO_SERV_A)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_LIM_SERV_A)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	LIMITED_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_LIM_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	LIMITED_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_TEST_SIM)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	FULL_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_TEST_SIM_NO_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_NO_SIM_NO_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_NO_INSRT)
	SET_COMP ("func",	FUNC_LIM_SERV_ST_SRCH)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_NO_SIM_NO_SERV_M)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_NO_INSRT)
	SET_COMP ("func",	FUNC_LIM_SERV_ST_SRCH)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_NO_SIM_LIM_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_NO_INSRT)
	SET_COMP ("func",	FUNC_LIM_SERV_ST_SRCH)
	SET_COMP ("service",	LIMITED_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_NET_REQUEST)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op", OP_MODE_SIM_1)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_2)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_3)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	FULL_SERVICE)
ENDSTRUCT
BEGIN_PSTRUCT ("op", OP_MODE_SIM_4)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	LIMITED_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op", OP_SIM_AUTO_MMI_SRCH_NS)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op", OP_SIM_MAN_MMI_SRCH_NS)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op", OP_SIM_MAN_MMI_SRCH_FS)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_MAN)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	FULL_SERVICE)
ENDSTRUCT

/*
 * plmn (PLMN identification - RR)
 */
BEGIN_PSTRUCT ("plmn", PLMN_123_33)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_33)
ENDSTRUCT
BEGIN_PSTRUCT ("plmn", PLMN_123_32)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_32)
ENDSTRUCT
BEGIN_PSTRUCT ("plmn", PLMN_123_31)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_31)
ENDSTRUCT
BEGIN_PSTRUCT ("plmn", PLMN_123_44)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_44)
ENDSTRUCT
BEGIN_PSTRUCT ("plmn", PLMN_NO_ID)
	SET_COMP ("v_plmn",	V_PLMN_NOT_PRES)
	SET_COMP ("mcc",	MCC_NONE)
	SET_COMP ("mnc",	MNC_NONE)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn", PLMN_44)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_44)
ENDSTRUCT

BEGIN_PSTRUCT ("full_name", FULL_NAME)
	SET_COMP ("v_name",	TRUE)
	SET_COMP ("cs",	CS_UCS2)
	SET_COMP("add_ci",	ADD_CI_NO)
	SET_COMP("num_spare",	NUM_SPARE_0)
	SET_COMP("c_text",	NET_TEXT_LEN_A)
	SET_COMP("text",	NET_TEXT_A)
ENDSTRUCT

BEGIN_PSTRUCT ("short_name", SHORT_NAME)
	SET_COMP ("v_name",	TRUE)
	SET_COMP ("cs",	CS_UCS2)
	SET_COMP("add_ci",	ADD_CI_NO)
	SET_COMP("num_spare",	NUM_SPARE_0)
	SET_COMP("c_text",	NET_TEXT_LEN_A)
	SET_COMP("text",	NET_TEXT_A)
ENDSTRUCT

BEGIN_PSTRUCT ("ntz", NTZ_MET)
	SET_COMP ("v_tz",	TRUE)
	SET_COMP ("tz",	TZ_MET)
ENDSTRUCT

BEGIN_PSTRUCT ("time", TIME)
	SET_COMP ("v_time",	TRUE)
	SET_COMP ("year",	YEAR_00)
	SET_COMP ("month",	MONTH_02)
	SET_COMP ("day",	DAY_08)
	SET_COMP ("hour",	HOUR_15)
	SET_COMP ("minute",	MINUTE_55)
	SET_COMP ("second",	SECOND_10)
ENDSTRUCT

# TEST CASES

## Internal Routing

### MM001: Configure internal routing and PCO view

Description:	Internal routing is configured and the duplication of primitives for performing the component tests with TAP and PCO view is carried out

Preamble:	None

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (CC RESET)
COMMAND (SS RESET)
COMMAND (SMS RESET)
COMMAND (MM RESET)
COMMAND (RR RESET)
COMMAND (DL RESET)
COMMAND (SIM RESET)
COMMAND (PL RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (RR REDIRECT CLEAR)
COMMAND (DL REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)
	 |		  |		  |
COMMAND (MMI REDIRECT MM NULL)
COMMAND (MMI REDIRECT CC NULL)
COMMAND (MMI REDIRECT SS NULL)
COMMAND (MMI REDIRECT SMS NULL)
COMMAND (MMI REDIRECT PL NULL)
	 |		  |		  |
COMMAND (CC REDIRECT MMI NULL)
COMMAND (CC REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SS REDIRECT MMI NULL)
COMMAND (SS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SMS REDIRECT MMI NULL)
COMMAND (SMS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (MM REDIRECT MMI TAP)
COMMAND (MM REDIRECT CC TAP)
COMMAND (MM REDIRECT SS TAP)
COMMAND (MM REDIRECT SMS TAP)
COMMAND (MM REDIRECT SIM TAP)
COMMAND (MM REDIRECT RR TAP)
COMMAND (MM REDIRECT DL TAP)
	 |		  |		  |
COMMAND (RR REDIRECT MM NULL)
COMMAND (RR REDIRECT DL NULL)
COMMAND (RR REDIRECT PL NULL)
COMMAND (RR CONFIG NO_SYS_TIME)
	 |		  |		  |
COMMAND (DL REDIRECT RR NULL)
COMMAND (DL REDIRECT MM NULL)
COMMAND (DL REDIRECT PL NULL)
	 |		  |		  |
COMMAND (PL REDIRECT RR NULL)
COMMAND (PL REDIRECT DL NULL)
COMMAND (PL REDIRECT MMI NULL)
	 |		  |		  |
COMMAND (SIM REDIRECT MM NULL)
	 |		  |		  |
COMMAND (TAP REDIRECT TAP MM)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	04.07.97	PZ	Initial Registration

## Registration

### MM021: Registration without SIM card

Description:	The reaction of MM to a request for registration with unplugged SIM card is tested. MM receives a MMR-REG request primitive and responds by issuing a RR-ACTIVATE request primitive in which the op field is set to 'limited service, no SIM'.

Preamble:	MM001

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM022: Mobile station is synchronous to a Cell

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell. The network identification if forwarded to MMI as part of a MMR-NREG indication primitive.

Preamble:	MM021

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_NOSIM

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)

### MM023: SIM inserted - initiate cell selection

Description:	MM receives a SIM-INSERT indication primitive and initiates cell selection by issuing a RR-ACTIVATE request primitive.

Preamble:	MM022

Variants:	<A>....<B>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
<A>		op_mode	NORMAL_SIM_INS
<B>		op_mode	TEST_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
<A>		op	OP_MODE_SIM_NO_SERV
<B>		op	OP_MODE_TEST_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
<A>		bcch_info	BCCH_INFO_ECL
<B>		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	24.02.00	HM	Revised
	12.04.00	HM	Revised

### MM024: Successful conclusion of cell selection - cell with same LAI

Description:	Successful conclusion of cell selection is signalled by the receipt of a RR-ACTIVATE confirmation primitive. MM forwards the PLMN identification to MMI in the form of a MMR-REG confirmation primitive.

Preamble:	MM023A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_2
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	09.07.97	HK	Initial

### MM025: SIM inserted - Search for specific network (no LUP)

Description:	MM receives a SIM-INSERT indication primitive and initiates cell selection by issuing a RR-ACTIVATE request primitive after receiving a start trigger of MMI.

Preamble:	MM022

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(5)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(7)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_NO_SERV_M1
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM_M
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	23.04.99	LE	Initial

### MM026: SIM inserted - Search for specific network (with LUP)

Description:	MM receives a SIM-INSERT indication primitive and initiates cell selection by issuing a RR-ACTIVATE request primitive after receiving a start trigger of MMI. A location updating is necessary

Preamble:	MM022

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(5)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(10)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(11)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(14)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_NO_SERV_M1
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_PLMN_RES
		plmn	PLMN_123_44

- RR_ACTIVATE_REQ
		plmn	PLMN_123_44
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_44_0002
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_44
		lac	LAC_0002
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_44

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_44
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

History:	23.04.99	LE	Initial

### MM027: Mobile station is synchronous to a Cell (reestablish allowed in cell)

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell. The network identification if forwarded to MMI as part of a MMR-NREG indication primitive. This testcase is equivalent to testcase MM022 with the difference that call reestablishment is allowed in the cell (MM_INFO -> MM_INFO_2).

Preamble:	MM021

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO_2
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_NOSIM

History:	16.02.00	HM	Initial
	02.03.00	HM	Revised (search_running)

## Connection Establishment

### MM041: Establish request prior to existing RR Connection

Description:	MM receives a MMCC-ESTABLISH request primitive from CC and requests a RR connection by sending a CM SERVICE REQUEST message to RR as part of a RR-ESTABLISH request primitive.

Preamble:	MM024

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_2
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

History:	08.07.97	HK	Initial

### MM042: Mobile terminated MM Connection

Description:	MM is alerted of a mobile-terminated call by the receipt of a RR- ESTABLISH indication primitive followed by a CC message as part of a RR-DATA indication primitive. MM enters State 6 (MM Connection Active) and issues a MMCC-ESTABLISH indication primitive.

Preamble:	MM024

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CC message)	  |
	 |		  *<==============================*
(3)	 |	MMCC_ESTABLISH_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

History:	09.07.97	HK	Initial
	17.09.97	DL	revised

### MM043: Acknowledgement of MM Connection

Description:	The MM connection is confirmed by the network in the form of a RR-ESTABLISH primitive followed by with a CM SERVICE ACCEPT message. MM changes to State 6 (MM Connection Active) and informs CC by issuing a MMCC-ESTABLISH confirmation primitive.

Preamble:	MM041

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	(CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(3)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_2

History:	08.07.97	HK	Initial

### MM044: Identity Request in State 5

Description:	MM changes to State 5 (Wait for Outgoing MM Connection) on receipt of a RR-ESTABLISH confirmation primitive. An IDENTITY REQUEST message is received from the network, to which MM responds by issuing an IDENTITY RESPONSE message.

Preamble:	MM041

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (IDENTITY REQUEST IMSI)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (IDENTITY RESPONSE)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (IDENTITY REQUEST IMEI)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (IDENTITY RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (ID REQUEST IMEISV)	  |
	 |		  *<==============================*
(7)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (IDENTITY RESPONSE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMSI
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MOB_IDENT_IMSI
		}

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMEI
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MOB_IDENT_IMEI
		}

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMEISV
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MOB_IDENT_IMEISV
		}

History:	08.07.97	HK	Initial
	07.11.00	HM	Revised

### MM045: RR-MM Synchronization

Description:	Synchronization by RR is signalled in the form of a RR-SYNC indication primitive. MM issues a MNCC-ESTABLISH confirmation primitive to CC.

Preamble:	MM044

Variants:	<A>....<B>

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(2)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_SYNC_IND
<A>	ciph	CIPH_ON
<B>	ciph	CIPH_OFF
		mm_info	MM_INFO_2
		bcch_info	BCCH_INFO_1
		synccs	NOT_USED
		chm	CHM_NOT_PRESENT

- MMCC_ESTABLISH_CNF
		ti	TI_2

History:	09.07.97	HK	Initial

### MM047: Data Transfer from Mobile Station in State 6

Description:	In State 6 (MM Connection Active) MM receives a CC message and forwards this to the network as part of a RR-DATA request primitive.

Preamble:	MM043

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_DATA_REQ	  | 		  |
	 |	  (CC message)	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (CC message)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

History:	09.07.97	HK	Initial

### MM048: Data Transfer in both directions in State 6

Description:	In State 6 (MM Connection Active) MM receives a CC message both from CC and the network. MM forwards the message from the network to CC as part of a MMCC-DATA indication primitive and to the network as part of a RR-DATA request primitive.

Preamble:	MM043

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CC message)	  |
	 |		  *<==============================*
(2)	 |	MMCC_ESTABLISH_IND	  | 		  |
	 |	  (CC message)	  |		  |
	 *<==============================*		  |
(3)	 |	MMCC_DATA_REQ	  | 		  |
	 |	  (CC message)	  |		  |
	 *==============================>*		  |
(4)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (CC message)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

History:	09.07.97	HK	Initial

### MM049: Random Access Failed, internal auto redial

Description:	RR responses with RR-RELEASE-IND to MM indicating a random access failure. MM starts one internal redialling.

Preamble:	MM041

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_RND_ACC_FAIL

History:	08.07.97	HK	Initial

### MM050: Initiation of CCBS call back

Description:	Network sends CM SERVICE PROMPT for CM entity CC and SAPI = 0. MM notifies CC by MMCC_PROMPT_IND and enters state MM_PROCESS_PROMPT. CC assigns a transaction identifier for the new MM connection by MMCC_PROMPT_RSP, from now CC is allowed to send CM messages. The first CM message for CCBS is a START CC message, this will be forwarded by MM to RR.

Preamble:	MM024

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	(CM SERVICE PROMPT)	  |
	 |		  *<==============================*
(3)	 |	MMCC_PROMPT_IND	  |		  |
	 *<==============================*		  |
(4)	 |	MMCC_PROMPT_RSP	  |		  |
	 |	(newly assigned ti)	  |		  |
	 *==============================>*		  |
(5)	 |	MMCC_DATA_REQ	  |		  |
	 |	(START CC)	  |		  |
	 *==============================>*		  |
(6)	 |		  |	RR_DATA_REQ	  |
	 |		  |	(START CC)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND	d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERVICE_PROMPT
		pd_and_sapi	PD_CC_AND_SAPI_0
		}

- MMCC_PROMPT_IND

- MMCC_PROMPT_RSP
		ti	TI_5

- MMCC_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

History:	15.02.00	HM	Initial

### MM051: Initiation of CCBS call back, out of ti

Description:	Network sends CM SERVICE PROMPT for CM entity CC and SAPI = 0. MM notifies CC by MMCC_PROMPT_IND and enters state MM_PROCESS_PROMPT. CC doesn't assign a transaction identifier for the new MM connection e.g. due to a lack of transaction identifiers. MM shall return to state MM_WAIT_FOR_NW_CMD and release the connection after timeout if no further messages are received by the network.

Preamble:	MM024

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	(CM SERVICE PROMPT)	  |
	 |		  *<==============================*
(3)	 |	MMCC_PROMPT_IND	  |		  |
	 *<==============================*		  |
(4)	 |	MMCC_PROMPT_REJ	  |		  |
	 *==============================>*		  |
(5)	 |		  |	RR_DATA_REQ	  |
	 |		  |	(MM_STATUS)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND	d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERVICE_PROMPT
		pd_and_sapi	PD_CC_AND_SAPI_0
		}

- MMCC_PROMPT_IND

- MMCC_PROMPT_REJ

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	B_MM_STATUS
		rej_cause	RC_SERVICE_ORDER
		}

History:	15.02.00	HM	Initial

### MM055: RR_ESTABLISH_IND in state MM_WAIT_FOR_RR_CONN_MM

Description:	MM receives a RR_ESTABLISH_IND in state MM_WAIT_FOR_RR_CONN_MM. This is a clash case. The MT connection has precedence, a MNCC_RELEASE_IND is sent for the MO connection and the MT connection is going to be established.

Preamble:	MM041

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CC message)	  |
	 |		  *<==============================*
(4)	 |	MMCC_ESTABLISH_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_IND
		param	NOT_USED

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_PREEMPT

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

History:	19.04.00	HM	Initial

### MM056: RR_RELEASE_IND in state MM_WAIT_FOR_RR_CONN_MM

Description:	MM receives a RR_RELEASE_IND in state MM_WAIT_FOR_RR_CONN_MM. This is a clash case. There is an incoming MT call already in RR, the MT connection has precedence, RR sends a RR_RELEASE_IND for the MO connection and then the MT connection is going to be established.

Preamble:	MM041

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(3)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CC message)	  |
	 |		  *<==============================*
(5)	 |	MMCC_ESTABLISH_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_MO_MT_COLL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- RR_ESTABLISH_IND
		param	NOT_USED

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_MO_MT_COLL

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

History:	19.04.00	HM	Initial

## Emergency Call

### MM061: Emergency Call in Idle State 19.2

Description:	MM receives an establishment request from CC while in State 19.2 (Idle Attempting to Update). It then sends a CM SERVICE REQUEST message to the network. On receipt of a RR-ESTABLISH confirmation primitive from the network followed by a CM SERVICE ACCEPT message, MM issues as MMCC-ESTABLISH confirmation primitive.

Preamble:	MM103

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(5)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

History:	09.07.97	HK	Initial

### MM062: RR Release followed by Call Establishment Request

Description:	Release of the RR connection is followed by an establishment request for an emergency call.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_ACCESS_BARRED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_ESTABLISH_REQ
		ti	TI_6
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_EMRG_CAL

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial

### MM063: Emergency Call in Idle State 19.4

Description:	MM receives an establishment request from CC while in State 19.4 (Idle No IMSI). It then sends a CM SERVICE REQUEST message to the network. On receipt of a RR-ESTABLISH confirmation primitive from the network followed by a CM SERVICE ACCEPT message, MM issues as MMCC-ESTABLISH confirmation primitive.

Preamble:	MM022

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(5)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_EMRG_CAL

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMEI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

History:	09.07.97	HK	Initial
	19.09.97	DL	revised

### MM064: Reestablishment during EC (Emergency Call)

Description:	MM receives an establishment request from CC while in State 19.2 (Idle Attempting to Update). It then sends a CM SERVICE REQUEST message to the network. On receipt of a RR-ESTABLISH confirmation primitive from the network followed by a CM SERVICE ACCEPT message, MM issues as MMCC-ESTABLISH confirmation. On receipt of a RR-ABORT indication primitive MDL_RELEASE_REQ MMCC_ERROR_IND.

Preamble:	MM027

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(5)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(8)	 |	MMCC_ERROR_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMEI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_ERROR_IND
		ti	TI_3
		reest	REEST_ALLOW
		errcs	ERRCS_RADIO_LINK_FAIL

History:	21.07.97	HK	Initial
	19.09.97	DL	revised
	16.02.00	HM	revised

### MM065: Emergency Call in State 19.3

Description:	An emergency call is started following an establishment request in State 19.3 (Idle, Limited Service).

Preamble:	MM108

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(5)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
(6)	 |	MMCC_RELEASE_REQ	  | 		  |
	 *==============================>*		  |
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_EMRG_CAL

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

- MMCC_RELEASE_REQ
		ti	TI_3

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	09.07.97	HK	Initial

### MM066: Calls after power cycle without SIM Remove

Description:	Following a power off the mobile station the mobile station is reactivated in a cell where the mobile is updated, an IMSI ATTACH is not allowed by the network, a call is started.
<A>	Emergency call
<B>	Normal call

Preamble:	MM202

Variants:	<A>....<B>

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(5)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(9)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
(10)	 |	MMCC_RELEASE_REQ	  | 		  |
	 *==============================>*		  |
(11)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_2
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- MMCC_ESTABLISH_REQ
		ti	TI_3
<A>		prio	PRIO_EMERG_CALL
<A>		estcs	ESTCS_EMRG_CAL
<B>		prio	PRIO_NORM_CALL
<B>		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC

- RR_ESTABLISH_REQ
<A>		estcs	ESTCS_EMRG_CAL
<B>		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
<A>		cm_serv_type	ST_EMERGENCY
<B>		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	NOT_USED
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

- MMCC_RELEASE_REQ
		ti	TI_3

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	22.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	11.04.01	HM	Revised, now 2 subcases after CFUN=4

## Connection Release

### MM081: Release of Connection via MS

Description:	Following the release of a connection M issues a MDL-RELEASE request primitive and returns to State 1 Idle Updated.

Preamble:	MM043

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_RELEASE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_RELEASE_REQ
		ti	TI_2

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	09.07.97	HK	Initial

### MM082: MMCC_REL_REQ in State 14

Description:	MM receives a MMCC-RELEASE request primitive from CC before a RR connection has been made. MM issues a RR-ABORT request.

Preamble:	MM041

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_RELEASE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_RELEASE_REQ
		ti	TI_2

- RR_ABORT_REQ
		abcs	ABCS_NORM

History:	10.07.97	HK	Initial

### MM083: Release of Connection via BS

Description:	MM receives a RR-RELEASE request primitive from RR. MM issues a a MDL-RELEASE request primitive to DL followed by a MMCC-RELEASE request primitive.

Preamble:	MM043

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_NORM

History:	09.07.97	HK	Initial

## Location Updating

### MM101: MS in new Location Area

Description:	A new location area is signalled to MM in the form of a RR-ACTIVATE confirmation primitive. MM starts normal Location Updating by issuing a LOCATION UPDATING REQ message as part of a RR-ESTABLISH request primitive and enters State 13 (Wait for RR-Connection - Location Updating).

Preamble:	MM023A

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	07.07.97	HK	Initial

### MM102: New LAI with Periodic Updating

Description:	A new location area is signalled to MM in the form of a RR-ACTIVATE confirmation primitive. MM starts normal Location Updating by issuing a LOCATION UPDATING REQ message as part of a RR-ESTABLISH request primitive and enters State 13 (Wait for RR-Connection - Location Updating).

Preamble:	MM023A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  |		  |
	 *<==============================*		  |
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCTION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0001
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ	estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	08.07.97	HK	Initial
	04.08.97	DL	Revised

### MM103: Access classes barred in State 13

Description:	MM receives a RR-RELEASE indication primitive from RR; normal Location Updating cannot be carried out because the access classes in the chosen cell are barred. MM issues a MDL-RELEASE request primitive and changes to State 19.2 (Idle Attempting to Update).

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
	relcs	RELCS_ACCESS_BARRED
	sapi	SAPI_0
	gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	09.07.97	HK	Initial

### MM104: Random access delayed in State 13

Description:	MM receives a RR-RELEASE indication primitive from RR; Normal Location Updating cannot be carried out because of random access delay. MM issues a MDL-RELEASE request primitive.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
	relcs	RELCS_RND_ACC_DELAY
	sapi	SAPI_0
	gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	09.07.97	HK	Initial

### MM105: Successful Location Updating

Description:	Successful registration of the mobile station is signalled by the base station in the form of a RR-ESTABLISH confirmation primitive and the receipt of a LOCTION UPDATING ACC message. MM changes to State 9 (Wait for Network Command) and issues a TMSI REALLOC COMPLETE message followed by a RR-SYNC request, a MMR-REG confirmation and a SIM-MM-UPDATE request primitive.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_0002
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_0002
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_3
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	08.07.97	HK	Initial
	31.07.97	DL	Revised
	12.08.97	HK	Revised
	30.05.00	HM	Revised

### MM106: Location Accept without Mobile Identity

Description:	Successful registration of the mobile station is signalled by the base station in the form of a RR-ESTABLISH confirmation primitive. MM changes to State 9 (Wait for Network Command) and issues a LOCTION UPDATING ACC message (without Mobile Identity ) followed by a RR-SYNC request, a MMR-REG confirmation and a SIM-MM-UPDATE request primitive. Following receipt of a RR-RELEASE indication primitive, MM issues a MDL-RELEASE request primitive.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(5)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |	MDL_RELEASE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_0002
		mob_id	NOT_USED
		follow_proceed	NOT_USED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_0002
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_2
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	10.07.97	HK	Initial

### MM606: Location Accept with empty Mobile Identity

Description:	Same testcase as 106, but uses special coding of Anite-System, which shall be accepted by MM.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(5)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |	MDL_RELEASE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	LOC_UPD_ACCEPT_2

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_0002
		synccs	SYNCCS_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_2
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	10.07.97	HK	Initial

### MM907: Location Updating triggered by Normal Call in Idle State 19.2

Description:	MM receives an establishment request from CC while in State 19.2 (Idle Attempting to Update). It starts a location updating. The location updating accept does contain the follow on proceed indication and the call attempt is started.

Preamble:	MM112

Variants:	<A>...<D>

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(7)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(8)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (CM SERVICE REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL_FOL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
<A>		sdu	LOC_UPD_ACCEPT_3
<B>		sdu	LOC_UPD_ACCEPT_4
<C>		sdu	LOC_UPD_ACCEPT_5
<D>		sdu	LOC_UPD_ACCEPT_6

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_0002
		synccs	SYNCCS_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_2
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial

### MM908: Location Updating triggered by Normal Call in Idle State 19.2

Description:	MM receives an establishment request from CC while in State 19.2 (Idle Attempting to Update). It starts a location updating. The location updating accept does not contain the follow on proceed indication and the call attempt is released.

Preamble:	MM112

Variants:	<A>...<D>

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(7)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(8)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(9)	 |	MDL_RELEASE_REQ	  | 		  |
	 *<==============================*		  |
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL_FOL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
<A>		sdu	LOC_UPD_ACCEPT_3_B
<B>		sdu	LOC_UPD_ACCEPT_4_B
<C>		sdu	LOC_UPD_ACCEPT_5_B
<D>		sdu	LOC_UPD_ACCEPT_6_B

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_0002
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_2
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_DATA
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial

### MM107: Connection request stored until Location Updating complete

Description:	MM receives a MMCC-ESTABLISH request primitive from MM and a RR-ESTABLISH confirmation primitive from RR. MM stores the connection request until Location Updating has been completed. The mobile-terminated call is then processed (RR-SYNC request, MMR-REG confirmation primitive). Following completion of the MOT in the form of a RR-RELEASE indication primitive, RR releases the requested connection. After having completely released the RR connection, this is reestablished immediately to satisfy the pending connection request. For this test, a TEST SIM is inserted into the mobile.

Preamble:	MM115

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING ACC)	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(8)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(9)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_0002
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_0002
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_3
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_UNSPECIFIED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_TMSI
		}

History:	15.09.97	DL	Initial
	16.02.00	HM	Revised

### MM108: Location Updating Reject (PLMN not allowed)

Description:	A request for a RR connection for Location Updating is confirmed in the form of a RR-ESTABLISH confirmation primitive. MM changes to State 3 (Location Updating initiated). MM is then notified in the form of a LOCATION-UPDATING-REJECT message that PLMN roaming is not allowed for the mobile station. MM releases the RR connection by issuing a MDL-RELEASE request primitive and changes to State 11 (Idle Roaming not allowed). The PLMN for which the LOCATION UPDATING REJECT with cause PLMN NOT ALLOWED received is the HPLMN, thus it will *not* be added to the forbidden list.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(7)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_PLMN_NOT_ALLOWED
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_PLMN_NOT_ALLOWED

History:	09.07.97	HK	Initial
	04.08.97	DL	Revised
	02.03.00	HM	Revised (search_running)
	22.05.00	HM	Revised

### MM109: Change of Area in Update Status U3

Description:	MM receives a RR-ACTIVATE indication primitive indicating a change of location area while in Update Status U3 (Roaming not allowed). MM starts Location Updating by sending a LOCATION UPDATING REQ message. After completion of the Location Updating, MM is in State 1 (Idle updated).

Preamble:	MM108

MMI/CM/SIM	 MM		 RR/<DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING ACC)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(9)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_44_0002
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	LAC_0002
		synccs	SYNCCS_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_44

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_44
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	09.07.97	HK	Initial
	05.08.97	DL	Revised

### MM110: Location Updating Reject (LAI roaming not allowed)

Description:	While in Update Status U3 (Roaming not allowed) MM receives a RR-ESTABLISH confirmation primitive followed by a LOCATION UPDATING REJECT message indicating that LAI roaming is not permitted for the mobile station. MM releases the RR connection and then issues a SIM-MM-UPDATE request primitive.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(8)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_LA_NOT_ALLOWED
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_LAI_NOT_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_LA_NOT_ALLOWED

History:	09.07.97	HK	Initial
	05.08.97	DL	Revised
	02.03.00	HM	Revised (search_running)

### MM111: New Location Area Code in Update Status U3

Description:	In Update Status U3 (Roaming not allowed) MM is . A new Location Updating is started by a change of cell. After completion of Location Updating MM is once more in State U1 Idle Updated.

Preamble:	MM110

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING ACC)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(9)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |	MDL_RELEASE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0001
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_0001
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_0001
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_4
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	09.07.97	HK	Initial

### MM112: Normal Location Updating rejected four times

Description:	A Normal Location Updating has been started in Update Status 2 (Idle not updated). A LOCATION UPDATING REJ message is received and MM releases the RR connection, initiates synchronization with RR and request SIM for an update . This sequence is carried out four times; each attempt is rejected (LOCATION UPDATING REJ message). Following the fourth attempt MM remains in Update Status 2 (Idle not Updated) and issues a MMR-NREG indication primitive.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(14)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(15)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(16)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(17)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(20)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(21)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(22)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(23)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(24)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(25)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(26)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(27)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(28)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_UNSPECIFIED

History:	09.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM113: Re-attempt after time-out Location updating timer

Description:	MM enters state 19.2 (Idle Attempting to Update). The registration timer is started. After expiry, a normal location update attempt is started.

Preamble:	MM409

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
COMMAND (MM CONFIG TIMER_SET=<T_REG, 25000>)
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(14)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (30000)
(15)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial
	26.01.01	HM	Revised

### MM114: Re-attempt after fieldstrength jump

Description:	MM enters state 19.2 (Idle Attempting to Update). RR indicates a fieldstrength jump and a re-attempt is started, if a SIM card, but no Test-SIM card is inserted.

Preamble:	MM409

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
COMMAND (MM CONFIG TIMER_SET=<T_REG, 25000>)
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(14)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (1000)
(15)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(16)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_LUP_RETRY
		chm	CHM_NOT_PRESENT

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial
	26.01.01	HM	Revised

### MM115: MS in new Location Area (TEST SIM inserted)

Description:	A new location area is signalled to MM in the form of a RR-ACTIVATE confirmation primitive. MM starts normal Location Updating by issuing a LOCATION UPDATING REQ message as part of a RR-ESTABLISH request primitive and enters State 13 (Wait for RR-Connection - Location Updating).

Preamble:	MM023B

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	15.02.00	HM	Initial

### MM116: Location Updating Reject (PLMN not allowed), not HPLMN

Description:	A request for a RR connection for Location Updating is confirmed in the form of a RR-ESTABLISH confirmation primitive. MM changes to State 3 (Location Updating initiated). MM is then notified in the form of a LOCATION-UPDATING-REJECT message that PLMN roaming is not allowed for the mobile station. MM releases the RR connection by issuing a MDL-RELEASE request primitive and changes to State 11 (Idle Roaming not allowed). The PLMN for which the LOCATION UPDATING REJECT with cause PLMN NOT ALLOWED received is *not* the HPLMN, thus it *will* be added to the forbidden list.

Preamble:	MM105

MMI/CM/SIM	 MM		 RR/DL
TIMEOUT (20000)
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(9)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(10)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_0002
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_PLMN_NOT_ALLOWED
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_LIMITED_SERVICE
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_123_44
		limited_cause	RC_PLMN_NOT_ALLOWED

History:		30.05.00	HM	Initial

### MM117: Location Updating Reject (PLMN not allowed), mode AUTO

Description:	The mobile station is requested to register in automatic mode. The HPLMN is not present. The first PLMN the mobile station tries to register is forbidden and newly entered into the forbidden PLMN list. Then the second found PLMN is tried. It is expected that the registration attempt into the second found PLMN works and we enter full service state.
PLMN_123_44	First found PLMN, forbidden
PLMN_123_31	Second found PLMN, allowed

Preamble:	MM001

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(7)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(12)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(16)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(17)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(18)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(20)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(21)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(22)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING ACC)	  |
	 |		  *<==============================*
(23)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(24)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(25)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(26)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(27)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(28)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(29)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	TWO_PLMN_FOUND
		plmn	PLMN_LIST_2_PLMN_1_NEW_FORB
		rxlevel	RXLEVEL_20_18
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_REQ
		plmn	PLMN_123_44
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_PLMN_NOT_ALLOWED
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_LIMITED_SERVICE
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_RUNNING
		new_forb_plmn	PLMN_123_44
		limited_cause	RC_PLMN_NOT_ALLOWED

- RR_ACTIVATE_REQ
		plmn	PLMN_123_31
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_31
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_31_0002
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_31
		lac	LAC_0002
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_31

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_31_0002
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:		30.05.00	HM	Initial
		07.01.01	HM	Revised, updating now into selected cell

### MM118: Location Updating Reject (IMSI unknown in HLR), mode AUTO

Description:	The mobile station is requested to register in automatic mode. The HPLMN is not present. The first PLMN the mobile station tries to register sends LOCATION UPDATING REJECT with cause #2 (IMSI unknown in HLR). It is expected that no further registration attempt will be tried into the second found PLMN, even if an RR_ACTIVATE_IND with op_mode OP_MODE_SIM will be received.
PLMN_123_44	First found PLMN, sends reject with cause #2
PLMN_123_31	Second found PLMN

Preamble:	MM001

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(7)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(12)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(15)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(16)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (5000)
(17)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(18)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (5000)
(19)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(20)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(21)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(22)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	TWO_PLMN_FOUND
		plmn	PLMN_LIST_2_PLMN_1_NEW_FORB
		rxlevel	RXLEVEL_20_18
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_REQ
		plmn	PLMN_123_44
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_IMSI_IN_HLR
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_IMSI_IN_HLR

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0001
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_IMSI_IN_HLR

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:		04.01.01	HM	Initial

## Identity Request

### MM121: Identity Request during Location Updating

Description:	Connection is confirmed in the form of a RR-ESTABLISH confirmation primitive, whereupon MM changes to State 3 (Location Updating Initiated). In the course of Location Updating the network requests the identity of the MS. MM responds by issuing an Identity Response message as part of a RR-DATA indication primitive.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (IDENTITY REQUEST)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (IDENTITY RESPONSE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMSI
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial

### MM122: Identity Request during MTC

Description:	The mobile station receives an IDENTITY REQUEST message for the IMSI or IMEI from the network and responds by issuing a IDENTITY RESPONSE message containing the IMEI or IMSI, accordingly as requested.

Preamble:	MM043

Variants:	<A>....<B>

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (IDENTITY REQUEST)	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (IDENTITY RESPONSE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
<A>		ident	IDENT_TYPE_IMSI
<B>		ident	IDENT_TYPE_IMEI
		}

- RR_DATA_REQ		
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
<A>		mob_id	MOB_IDENT_IMSI
<B>		mob_id	MOB_IDENT_IMEI
		}

History:	07.07.97	HK	Initial
	31.07.97	DL	Revised
	18.04.01	HM	Revised

## Authentication

### MM141: Authentication Request

Description:	The network confirms the request from the mobile station for a RR Connection in the form of a RR-ESTABLISH confirmation primitive. MM changes to State 3 ((Location Updating initiated) and issues an AUTHENTICATION REQ message and a SIM-AUTHENTICATION request primitive, initiating the authentication procedure with SIM.

Preamble:	MM101

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REQ)	  |
	 |		  *<==============================*
(3)	 |	SIM_AUTHENTICATION_REQ	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REQ
		ti	TI_0
		ciph_key_num	CIPH_KEY_NUM_01
		auth_rand	AUTH_RAND_1
		}

- SIM_AUTHENTICATION_REQ
		source	SRC_MM
		rand	RAND_1_P
		cksn	CKSN_01

History:	07.07.97	HK	Initial
	31.07.97	DL	Revised

### MM142: Authentication Response

Description:	The authentication parameters are received from SIM in the form of a SIM-AUTHENTICATION confirmation primitive and are forwarded to the network as part of a AUTHENTICATION RES message. MM then commences synchronization with RR by issuing a RR-SYNC request primitive.

Preamble:	MM141

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_AUTHENTICATION_CNF	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (AUTHENTICATION RES)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_AUTHENTICATION_CNF
		sres	SRES_1
		kc	KC_11223344

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_AUTH_RES
		ti	TI_0
		auth_sres	SRES_1_CODED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_01
		kcv	KCV_11223344
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

History:	07.07.97	HK	Initial
	31.07.97	DL	Revised

### MM143: Authentication Request in State 6

Description:	Authentication is requested by the network in the form of a AUTHENTICATION REQ message while the mobile station is in State 6 (MM Connection Active). Authentication is then sought and received form SIM (SIM-AUTHENTICATION request and confirmation primitives). MM sends this data to the network as part of a AUTHENTICATION RES message.

Preamble:	MM043

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REQ)	  |
	 |		  *<==============================*
(2)	 |	SIM_AUTHENTICATION_REQ	  | 		  |
	 *<==============================*		  |
(3)	 |	SIM_AUTHENTICATION_CNF	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (AUTHENTICATION RES)	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(7)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(8)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(9)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REQ
		ti	TI_0
		ciph_key_num	CIPH_KEY_NUM_01
		auth_rand	AUTH_RAND_1
		}

- SIM_AUTHENTICATION_REQ
		source	SRC_MM
		rand	AUTH_RAND_1
		cksn	CKSN_01

- SIM_AUTHENTICATION_CNF
		sres	SRES_1
		kc	KC_11223344

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_AUTH_RES
		ti	TI_0
		auth_sres	SRES_1_CODED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_01
		kcv	KCV_11223344
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_NO_REGISTRATION

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi 	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

History:	08.07.97	HK	Initial
	26.01.01	HM	Revised
	27.04.01	HM	Revised

### MM144: Authentication Reject in State 3

Description:	Authentication is rejected by the network in the form of a AUTHENTICATION REJ message followed by a RR-ABORT indication primitive while the mobile station is in State 3 (Location Updating Initiated). MM releases the RR connection and signals failure to MMI in the form of a MMR-NREG indication primitive with the negative registration cause set to 'authentication failure'.

Preamble:	MM142

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi 	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_INT_PROT_ERR
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_AUTHREJ

History:	09.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	27.04.01	HM	Changed order of primitives

### MM145: Authentication Reject and SIM Removal in State 3

Description:	Authentication is rejected by the network in the form of a AUTHENTICATION REJ message while the mobile station is in State 3 (Location Updating Initiated). This is followed by the receipt of a SIM-REMOVE indication primitive. MM releases the RR connection and signals failure to MMI in the form of a MMR-NREG indication primitive with the negative registration cause set to 'authentication failure'.

Preamble:	MM142

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(4)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (11000)
(5)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- SIM_REMOVE_IND
		error	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	08.07.97	HK	Initial
	03.03.00	HM	Revised (search_running)
	29.08.00	HM	Revised
	07.01.01	HM	Adaption caused by GPRS integration
	27.04.01	HM	Revised

### MM146: Authentication Request in State 5

Description:	In State 5 (Wait for Outgoing MM Connection) MM receives an establishment request from CC and sends CM SERVICE REQUEST message as part of a RR-ESTABLISH request primitive. The network commences the Authentication procedure in by sending a AUTHENTICATION REQ message and MM issues a SIM-AUTHENTICATION request primitive.

Preamble:	MM024

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REQ)	  |
	 |		  *<==============================*
(5)	 |	SIM_AUTHENTICATION_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_5
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REQ
		ti	TI_0
		ciph_key_num	CIPH_KEY_NUM_04
		auth_rand	AUTH_RAND_1
		}

- SIM_AUTHENTICATION_REQ
		source	NOT_USED
		rand	AUTH_RAND_1
		cksn	CKSN_04

History:	08.07.97	HK	Initial

### MM147: Response to Authentication Request in State 5

Description:	In State 5 (Wait for outgoing MM Connection to the Network) MM receives a SIM-AUTHENTICATION confirmation primitive and sends the authentication parameters to the network in the form of an AUTHENTICATION RES message.

Preamble:	MM146

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_AUTHENTICATION_CNF	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (AUTHENTICATION RES)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_AUTHENTICATION_CNF
		sres	SRES_1
		kc	KC_11223344

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_AUTH_RES
		ti	TI_0
		auth_sres	SRES_1_CODED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_04
		kcv	KCV_11223344
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

History:	08.07.97	HK	Initial

### MM148: Authentication Reject in State 5

Description:	Authentication is rejected by the network in the form of a AUTHENTICATION REJ message while the mobile station is in State 5 (Wait for outgoing MM Connection). This is followed by the receipt of a SIM-REMOVE indication primitive. MM releases the RR connection and signals failure to MMI in the form of a MMR-NREG indication primitive with the negative registration cause set to 'authentication failure'.

Preamble:	MM147

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(5)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- MMCC_RELEASE_IND
		ti	TI_5
		relcs	RELCS_NO_REGISTRATION

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_AUTHREJ

History:	08.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	26.01.01	HM	Revised
	27.04.01	HM	Changed ordering of primitives

### MM149: Authentication Reject during request for a second connection

Description:	MM receives a second connection request from CC and sends a CM SERVICE REQUEST message to the network. This is followed by a AUTHENTICATION REQ message from the network; MM receives authentication confirmation for the previous authentication request (SIM-AUTHENTICATION confirmation primitive) and then seeks authentication for the second request. The data for the first authentication confirmation is forwarded to the network in the form of a AUTHENTICATION RES message. Authentication is rejected by the network in the form of a AUTHENTICATION REJ message. MM releases the RR connection and signals failure to MMI in the form of a MMR-NREG indication primitive with the negative registration cause set to 'authentication failure'.	The reaction of MM to an Authentication failure during the Establishment of a second Connection is tested.

Preamble:	MM045A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REQ)	  |
	 |		  *<==============================*
(4)	 |	SIM_AUTHENTICATION_REQ	  | 		  |
	 *<==============================*		  |
(5)	 |	SIM_AUTHENTICATION_CNF	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (AUTHENTICATION RES)	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(9)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(10)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(11)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(13)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_5
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REQ
		ti	TI_0
		ciph_key_num	CIPH_KEY_NUM_04
		auth_rand	AUTH_RAND_1
		}

- SIM_AUTHENTICATION_REQ
		source	SRC_MM
		rand	AUTH_RAND_1
		cksn	CKSN_04

- SIM_AUTHENTICATION_CNF
		sres	SRES_1
		kc	KC_11223344

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_AUTH_RES
		ti	TI_0
		auth_sres	SRES_1_CODED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_04
		kcv	KCV_11223344
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_NO_REGISTRATION

- MMCC_RELEASE_IND
		ti	TI_5
		relcs	RELCS_NO_REGISTRATION

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_AUTHREJ

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	26.01.01	HM	Revised
	27.04.01	HM	Changed ordering of primitives

### MM150: Authentication in State 9

Description:	In State 9 (Wait for Network Command) MM receives RR-ESTABLISH indication primitive and an AUTHENTICATION REQ message from the network. MM forwards the authentication request to SIM in the form of a SIM-AUTHENTICATION request primitive containing the ciphering key data received from the network.

Preamble:	MM024

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REQ)	  |
	 |		  *<==============================*
(3)	 |	SIM_AUTHENTICATION_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REQ
		ti	TI_0
		ciph_key_num	CIPH_KEY_NUM_01
		auth_rand	AUTH_RAND_1
		}

- SIM_AUTHENTICATION_REQ
		source	NOT_USED
		rand	AUTH_RAND_1
		cksn	CKSN_01

History:	10.07.97	HK	Initial

### MM151: Response to Authentication Request in State 9

Description:	In State 9 (Wait for Network Command) MM receives a SIM-AUTHENTICATION confirmation primitive and sends the authentication parameters to the network in the form of an AUTHENTICATION RES message.

Preamble:	MM150

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_AUTHENTICATION_CNF	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (AUTHENTICATION RES)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_AUTHENTICATION_CNF
		sres	SRES_1
		kc	KC_11223344

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_AUTH_RES
		ti	TI_0
		auth_sres	SRES_1_CODED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_01
		kcv	KCV_11223344
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

History:	10.07.97	HK	Initial

### MM152: Authentication Reject in State 9

Description:	In State 9 Wait for Network command MM receives RR_DATA_IND with an Authentication Reject message. MM sets the flag "Authentication fail", invalidates the SIM in RR, informs the SIM entity about loss of registration, considers the SIM as not present in MM and stays in State 9 WAIT FOR NETWORK COMMAND until the RR connection is released.

Preamble:	MM151

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi 	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

History:	10.07.97	HK	Initial
	30.04.01	HM	Revised

### MM153: Registration request following authorization failure

Description:	Authentication is rejected by the network in the form of a AUTHENTICATION REJ message followed by a RR-ABORT indication primitive while the mobile station is in State 3 (Location Updating Initiated). MM releases the RR connection and signals failure to MMI in the form of a MMR-NREG indication primitive with the negative registration cause set to 'authentication failure'.

Preamble:	MM142

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(4)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(5)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_AUTHREJ

History:	21.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	27.04.01	HM	Revised

### MM154: LUP Rejection following AUTHENTICATION RESPONSE in State 3

Description:	In State 3 (Location Updating Initiated) MM receives a LOCATION UPDATING REJ message. MM releases the RR connection and signals failure to MMI in the form of a MMR-NREG indication primitive. The PLMN for which the LOCATION UPDATING REJECT with cause PLMN NOT ALLOWED received is the HPLMN, thus it will *not* be added to the forbidden list.

Preamble:	MM142

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(6)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_PLMN_NOT_ALLOWED
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_PLMN_NOT_ALLOWED

History:	21.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	22.05.00	HM	Revised

### MM155: LUP with MM INFORMATION

Description:	The SIM card needs some time to process the SRES. This time may be used by the network to transfer some information to the mobile station, which is forawrded transparently to upper layers. The authentication parameters are received from SIM in the form of a SIM-AUTHENTICATION confirmation primitive and are forwarded to the network as part of a AUTHENTICATION RES message. MM then commences synchronization with RR by issuing a RR-SYNC request primitive. The purpose of this testcase is to test that all parameters of the MM INFORMATION message are correctly transmitted to the ACI. Derived from testcase MM142.

Preamble:	MM141

Variants:	<A>....<B>

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_DATA_IND	  |
	 |		  |	(MM INFORMATION)	  |
	 |		  *<==============================*
(2)	 |	MMR_INFO_IND	  |		  |
	 *<==============================*		  |
(3)	 |	SIM_AUTHENTICATION_CNF	  |		  |
	 *==============================>*		  |
(4)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (AUTHENTICATION RES)	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_MM_INFORMATION
		full_net_name	FULL_NET_NAME
		short_net_name	SHORT_NET_NAME
<A>		net_tz	NET_TZ
<A>		net_tz_and_time	NOT_USED
<B>		net_tz	NOT_USED
<B>		net_tz_and_time	NET_TZ_AND_TIME
		}

- MMR_INFO_IND
		plmn	PLMN_123_33
		full_name	FULL_NAME
		short_name	SHORT_NAME
		ntz	NTZ_MET
<A>		time	NOT_USED
<B>		time	TIME

- SIM_AUTHENTICATION_CNF
		sres	SRES_1
		kc	KC_11223344

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_AUTH_RES
		ti	TI_0
		auth_sres	SRES_1_CODED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_01
		kcv	KCV_11223344
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

History:	08.03.00	HM	Initial

## Net Request

### MM161: Net Request in State 19.4

Description:	MM receives a MMR-NET request primitive in State 19.4 (Idle No IMSI). A RR-ACTIVATE request with an empty IMSI is issued and MM enters State 19.8 (Idle, PLMN Search, Normal Service).

Preamble:	MM024

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NET_REQ
		param	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NET_REQUEST
		cksn	CKSN_NO_KEY
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	EMPTY_IMSI
		tmsi	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	10.07.97	HK	Initial
	17.09.97	DL	revised

### MM162: RR_ABORT_IND in State 19

Description:	MM receives a RR-ABORT indication primitive in State 19 (MM Idle). MM enters the limited service state.

Preamble:	MM022

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	NOT_USED
		plmn	PLMN_123_44
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	10.07.97	HK	Initial
	17.02.00	HM	Revised

## TMSI Reallocation

### MM181: TMSI Reallocation in State 5

Description:	RR connection is confirmed in the form of a RR-ESTABLISH confirmation primitive. This is followed by a TMSI REALLOCATION COMMAND message. MM issues a TMSI REALLOC COMPLETE message and begins synchronization with RR (RR-SYNC request primitive). On receiving a CM SERVICE ACCEPT message MM issues a MMCC-ESTABLISH confirmation primitive.

Preamble:	MM041

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (TMSI REALLOC COMMAND)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(5)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(7)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_TMSI_REALLOC_CMD
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_0001
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_2

History:	09.07.97	HK	Initial

## Deregistration

### MM201: MMR_NREG_REQ in State 3

Description:	MM receives a RR-ESTABLISH confirmation primitive in State 3 (Location updating initiated). This is followed by a MMR-NREG request primitive. MM releases the RR connection and issues a MMR-NREG confirmation primitive with cause set to 'Power off'.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(5)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(9)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- MMR_NREG_REQ
		cs	CS_POW_OFF

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	09.07.97	HK	Initial
	15.09.97	DL	revised
	11.04.01	HM	Revised
	25.04.01	HM	Revised

### MM202: Power Off in Upate Status U1

Description:	MM receives a a MMR-NREG request primitive. MM issues a RR-DEACTIVATE request primitive and a MMR-NREG confirmation primitive with cause set to 'Power off' and changes to changes to State 0 (Null).

Preamble:	MM023A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	08.07.97	HK	Initial

### MM203: Authentication Reject and Power off in State 6

Description:	MM receives a MMR-NREG request primitive in State6 (MM Connection Active) and commences a shutdown.

Preamble:	MM143

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	09.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	26.01.01	HM	Revised
	30.04.01	HM	Revised

### MM204: Power Off in State 10

Description:	MM receives a MMR-NREG request primitive in State 10 (Location Update Rejected) and commences a shutdown.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(10)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- MMR_NREG_REQ
		cs	CS_POW_OFF

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	10.07.97	HK	Initial
	16.09.97	DL	revised
	11.04.01	HM	Revised
	25.04.01	HM	Revised

### MM205: Power Off in State 14

Description:	MM receives a MMR-NREG request primitive in State 14(Wait for RR Connection) and commences a shutdown.

Preamble:	MM041

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(4)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(5)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_UNSPECIFIED

- MMCC_RELEASE_IND
		ti	TI_3
		relcs	RELCS_UNSPECIFIED

- RR_ABORT_REQ
		abcs	ABCS_NORM

History:	10.07.97	HK	Initial
	16.09.97	DL	revised

### MM206: Power Off in State 18

Description:	MM receives a a MMR-NREG request primitive in State 18 (Wait for RR Active) and issues a RR-DEACTIVATE request primitive.

Preamble:	MM023A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

History:	10.07.97	HK	Initial

### MM207: Power Off Remove in State 19

Description:	MM receives a a MMR-NREG request primitive in State 19 (MM Idle) and issues a RR-DEACTIVATE request primitive.

Preamble:	MM161

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

History:	10.07.97	HK	Initial

### MM208: Switch on after power off, IMSI ATTACH needed

Description:	The mobile was switched off logically, but the power was not removed physically. This is the behaviour expected with the AT command AT+CFUN=4. The SIM data should have been preserved. After reactivation in the same location area, an IMSI ATTACH is needed.

Preamble:	MM202

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(5)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	11.04.01	HM	Initial

## Registration (REG_MS_OFF, no SIM Card)

### MM300: Registration

Description:	The reaction of MM to a request for registration with unplugged SIM card is tested. MM receives a MMR-REG request primitive and responds by issuing a RR-ACTIVATE request primitive in which the op field is set to 'limited service, no SIM'.

Preamble:	MM001

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM301: Deregistration

Description:	MM receives a MMR-NREG request primitive. MM is in NULL state, no lower layers are active. MM issues a MMR-NREG confirmation primitive with cause set to 'Power off'.

Preamble:	MM001

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	09.07.97	HK	Initial
	15.09.97	DL	revised
	31.03.00	HM	Revised

### MM302: SIM Insertion

Description:	MM receives a SIM-INSERT indication primitive. It stores the parameters and waits for registration start by MMI.

Preamble:	MM001

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM352: PLMN Available Request

Description:	MMI requests a PLMN available list. There is no SIM inserted. This is not allowed.

Preamble:	MM001

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NET_REQ
	param	NOT_USED

- MMR_PLMN_IND
	res	RES_REJ_NO_SIM_AVAIL
	plmn	NOT_USED
	forb_ind	NOT_USED
	rxlevel	NOT_USED

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

## Registration (REG_NO_SERVICE, no SIM Card)

### MM303: Registration

Description:	The reaction of MM to a request for registration with unplugged SIM card is tested. MM receives a MMR-REG request primitive and responds by issuing a RR-ACTIVATE request primitive in which the op field is set to 'limited service, no SIM'.

Preamble:	MM300

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM304: Deregistration

Description:	MM receives a a MMR-NREG request primitive. MM issues a RR-DEACTIVATE request primitive and a MMR-NREG confirmation primitive with cause set to 'Power off' and changes to changes to State 0 (Null).

Preamble:	MM300

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	08.07.97	HK	Initial

### MM305: SIM Insertion

Description:	MM receives a SIM-INSERT indication primitive. It stores the parameters and waits for registration start by MMI.

Preamble:	MM300

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM306: RR failure (No Service)

Description:	MM receives a RR-ABORT indication primitive indicating No Service.

Preamble:	MM300

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_NO_SIM_NO_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_FOUND
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_CELL_SELECTION_FAILED
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_NOSIM

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM307: MM Success ( Limited Service)

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell. No SIM is inserted.

Preamble:	MM300

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_NOSIM

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)

### MM353: PLMN Available Request

Description:	MMI requests a PLMN available list. There is no SIM inserted. This is not allowed.

Preamble:	MM300

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NET_REQ
		param	NOT_USED

- MMR_PLMN_IND
		res	RES_REJ_NO_SIM_AVAIL
		plmn	NOT_USED
		forb_ind	NOT_USED
		rxlevel	NOT_USED

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

## Registration (REG_LIMITED_SERVICE, no SIM Card)

### MM308: Registration

Description:	The reaction of MM to a request for registration with unplugged SIM card is tested. MM receives a MMR-REG request primitive and responds by issuing a RR-ACTIVATE request primitive in which the op field is set to 'limited service, no SIM'.

Preamble:	MM307

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM309: Deregistration

Description:	MM receives a a MMR-NREG request primitive. MM issues a RR-DEACTIVATE request primitive and a MMR-NREG confirmation primitive with cause set to 'Power off' and changes to changes to State 0 (Null).

Preamble:	MM307

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	08.07.97	HK	Initial

### MM310: SIM Insertion

Description:	MM receives a SIM-INSERT indication primitive. It stores the parameters and waits for registration start by MMI.

Preamble:	MM307

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM311: RR failure (No Service)

Description:	MM receives a RR-ABORT indication primitive indicating No Service. No SIM is inserted.

Preamble:	MM307

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_NO_SIM_NO_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_FOUND
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_CELL_SELECTION_FAILED
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_NOSIM

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM312: MM Success ( Limited Service)

Description:	MM is informed by means of a RR-ACTIVATE indication primitive that the mobile station is synchronous to a cell. No SIM is inserted.

Preamble:	MM307

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO
		cid	CELL_ID_0045
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_NOSIM

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)

### MM354: PLMN Available Request

Description:	MMI requests a PLMN available list. There is no SIM inserted. This is not allowed.

Preamble:	MM307

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NET_REQ
	param	NOT_USED

- MMR_PLMN_IND
	res	RES_REJ_NO_SIM_AVAIL
	plmn	NOT_USED
	forb_ind	NOT_USED
	rxlevel	NOT_USED

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

## Registration (REG_MS_OFF, with SIM card)

### MM313: Registration

Description:	MM receives SIM information followed by the registration start of MMI. The cell selection is started.
Variant A:	not updated	start with HPLMN
Variant B:	updated, HPLMN	start with HPLMN
Variant C:	updated, not HPLMN	start with LPLMN

Preamble:	MM001

Variants:	<A>....<C>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
	op_mode	OP_NORMAL_SIM
	imsi_field	IMSI_FIELD_1
<A>	loc_info	LOC_INFO_NOT_UPD_LPLMN
<B>	loc_info	LOC_INFO_UPDATED_1
<C>	loc_info	LOC_INFO_UPDATED_LPLMN
	acc_ctrl	ACC_CTRL_1
	bcch_inf	BCCH_INF_1
	kc_n	KC_EMPTY
	pref_plmn	PREF_PLMN_NONE
<A>	forb_plmn	FORB_PLMN_NONE
<B>	forb_plmn	FORB_PLMN_2
<C>	forb_plmn	FORB_PLMN_NONE
	phase	PHASE_2_SIM
	hplmn	THPLMN_01

- MMR_REG_REQ
	service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
<A>	plmn	PLMN_123_33
<B>	plmn	PLMN_123_33
<C>	plmn	PLMN_123_31
	op	OP_MODE_SIM_NO_SERV
	cksn	CKSN_RES
	kcv	KCV_EMPTY
	accc	ACC_2143
	imsi	MOB_ID_IMSI
	tmsi	MOB_ID_NO_ID
	thplmn	THPLMN_01
<A>	bcch_info	BCCH_INFO_1
<B>	bcch_info	BCCH_INFO_ECL
<C>	bcch_info	BCCH_INFO_ECL
	cell_test	CELL_TEST_DISABLE
	gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	24.02.00	HM	Revised

### MM314: PLMN Mode Change

Description:	MMI changes the PLMN mode from automatic to manual. MM receives SIM information followed by the registration start of MMI. The cell selection is started.

Preamble:	MM001

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
(3)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_NOT_UPD_LPLMN
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_M
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM315: Deregistration (Power Off)

Description:	MM receives a MMR-NREG request primitive. issues a MMR-NREG confirmation primitive with cause set to 'Power off'.

Preamble:	MM302

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	09.07.97	HK	Initial
	15.09.97	DL	revised

### MM316: Deregistration (SIM invalid)

Description:	MM receives a MMR-NREG request primitive. issues a MMR-NREG confirmation primitive with cause set to 'SIM Remove'. This doesn't erase SIM data in MM, but MM now considers the SIM as invalid until MMR_REG_REQ with SERVICE_MODE_FULL is received.

Preamble:	MM302

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
(3)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

- MMR_REG_REQ
		service_mode	SERVICE_MODE_LIMITED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	09.07.97	HK	Initial
	15.09.97	DL	Revised
	26.04.00	HM	Revided

### MM317: SIM Removal

Description:	The SIM card is removed before starting with registration. MM is in NULL state, no lower layers are active in this state.

Preamble:	MM302

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(3)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- MMR_NREG_IND
		nreg_cs	NREG_CELL_SELECTION_FAILED
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	08.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	31.08.00	HM	Revised

### MM351: PLMN Available Request

Description:	MMI requests a PLMN available list before starting registration. This is not allowed.

Preamble:	MM302

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NET_REQ
	param	NOT_USED

- MMR_PLMN_IND
	res	RES_REJ_NOT_IDL_MOD
	plmn	NOT_USED
	forb_ind	NOT_USED
	rxlevel	NOT_USED

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

## Registration (REG_NO_SERVICE, with SIM card, automatic mode)

### MM318: Registration

Description:	MM receives SIM information followed by the registration start of MMI. The cell selection is started.
Variant A:	not updated	start with HPLMN
Variant B:	updated, HPLMN	start with HPLMN
Variant C:	updated, not HPLMN	start with LPLMN

Preamble:	MM300

Variants:	<A>....<C>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
<A>		loc_info	LOC_INFO_NOT_UPD_LPLMN
<B>		loc_info	LOC_INFO_UPDATED_1
<C>		loc_info	LOC_INFO_UPDATED_LPLMN
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
<A>		forb_plmn	FORB_PLMN_NONE
<B>		forb_plmn	FORB_PLMN_2
<C>		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
<A>		plmn	PLMN_123_33
<B>		plmn	PLMN_123_33
<C>		plmn	PLMN_123_31
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
<A>		bcch_info	BCCH_INFO_1
<B>		bcch_info	BCCH_INFO_ECL
<C>		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	24.02.00	HM	Revised

### MM319: PLMN Mode Change

Description:	MMI changes the PLMN mode from automatic to manual. MM receives SIM information followed by the registration start of MMI. The cell selection is started.

Preamble:	MM300

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
(3)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_NO_SIM_NO_SERV_M
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_NOT_UPD_LPLMN
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_M
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM320: Deregistration (Power Off)

Description:	MM receives a a MMR-NREG request primitive. MM issues a RR-DEACTIVATE request primitive and a MMR-NREG confirmation primitive with cause set to 'Power off' and changes to changes to State 0 (Null).

Preamble:	MM313A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	08.07.97	HK	Initial

### MM321: Deregistration (SIM invalid)

Description:	MM receives a MMR-NREG request primitive. issues a MMR-NREG confirmation primitive with cause set to 'SIM Remove'.

Preamble:	MM313A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
(4)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(5)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	09.07.97	HK	Initial
	15.09.97	DL	revised
	30.08.00	HM	Revised

### MM322: SIM Removal

Description:	The SIM card is removed before starting with registration.

Preamble:	MM313A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(4)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(5)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_NO_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	08.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption caused by GPRS integration

### MM323: RR failure (No Service)

Description:	MM receives a RR-ABORT indication primitive indicating No Service.

Preamble:	MM313A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_SIM_NO_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_FOUND
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_CELL_SELECTION_FAILED
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM324: RR failure (Limited Service, no further PLMNs)

Description:	MM receives a RR-ABORT indication primitive indicating Limited Service. The one PLMN which was found is in the forbidden PLMN list and cannot be used.

Preamble:	MM313B

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_FORB
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	10.07.97	HK	Initial
	03.03.00	HM	Revised (search_running)

### MM325: RR failure (Limited Service, further PLMNs available)

Description:	MM receives a RR-ABORT indication primitive indicating Limited Service. The one PLMN which was found is not in the forbidden PLMN list and can be used.

Preamble:	MM313A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
	op	OP_MODE_SIM_LIM_SERV
	abcs	ABCS_CEL_SEL_FAIL
	plmn_avail	ONE_PLMN_FOUND
	plmn	PLMN_LIST_LPLMN
	rxlevel	RXLEVEL_20
	power	RF_CLASS_2

- MDL_RELEASE_REQ
	ch_type	NOT_PRESENT_8BIT
	sapi	SAPI_0

- MMR_NREG_IND
	nreg_cs	NREG_LIMITED_SERVICE
	search_running	SEARCH_RUNNING
	new_forb_plmn	PLMN_NO_ID
	limited_cause	MMR_RC_NONE

- RR_ACTIVATE_REQ
		plmn	PLMN_123_31
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM326: MM Success

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell.

Preamble:	MM313B

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
	op	OP_MODE_SIM
	mm_info	MM_INFO
	cid	CELL_ID_1122
	plmn	PLMN_123_33
	lac	LAC_2147
	power	RF_CLASS_2
	gprs_indic	GPRS_NO

- MMR_REG_CNF
	plmn	PLMN_123_33

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM327: MM Failure

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell. A location updating is started. After four rejections limitied service is indicated to the user.

Preamble:	MM313B

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(6)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(9)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(11)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(13)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(16)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(17)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(18)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(19)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(20)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(21)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(23)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(24)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(25)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(26)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(27)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(28)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(29)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(30)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(31)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_UNSPECIFIED

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)

### MM328: MM Authentication Failure

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell. A location updating is started. After authentication failure only limited service shall be allowed.

Preamble:	MM313B

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REQ)	  |
	 |		  *<==============================*
(6)	 |	SIM_AUTHENTICATION_REQ	  |		  |
	 *<==============================*		  |
(7)	 |	SIM_AUTHENTICATION_CNF	  |		  |
	 *==============================>*		  |
(8)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (AUTHENTICATION RES)	  |
	 |		  *==============================>*
(9)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(11)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(13)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REQ
		ti	TI_0
		ciph_key_num	CIPH_KEY_NUM_01
		auth_rand	AUTH_RAND_1
		}

- SIM_AUTHENTICATION_REQ
		source	SRC_MM
		rand	RAND_1_P
		cksn	CKSN_01

- SIM_AUTHENTICATION_CNF
		sres	SRES_1
		kc	KC_11223344

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_AUTH_RES
		ti	TI_0
		auth_sres	SRES_1_CODED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_01
		kcv	KCV_11223344
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi 	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_AUTHREJ

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)
	27.04.01	HM	Changed ordering of primitives

### MM355: PLMN Available Request

Description:	MM is in no service condition. MMI requests a PLMN available request and starts a PLMN search in RR. The result is forwarded to MMI.
[MMR_NREG_IND (cell selection failed) could cause display "NO SERVICE" in step 11. This may not be correct ...]

Preamble:	MM300

Variants:	<A>...<C>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(7)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(8)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(10)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
(11)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
	op_mode	OP_NORMAL_SIM
	imsi_field	IMSI_FIELD_1
<A>	loc_info	LOC_INFO_NOT_UPD_LPLMN
<B>	loc_info	LOC_INFO_UPDATED_1
<C>	loc_info	LOC_INFO_UPDATED_LPLMN
	acc_ctrl	ACC_CTRL_1
	bcch_inf	BCCH_INF_1
	kc_n	KC_EMPTY
	pref_plmn	PREF_PLMN_NONE
<A>	forb_plmn	FORB_PLMN_NONE
<B>	forb_plmn	FORB_PLMN_NONE
<C>	forb_plmn	FORB_PLMN_2
	phase	PHASE_2_SIM
	hplmn	THPLMN_01

- MMR_REG_REQ
	service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
<A>	plmn	PLMN_123_33
<B>	plmn	PLMN_123_33
<C>	plmn	PLMN_123_31
	op	OP_MODE_SIM_NO_SERV
	cksn	CKSN_RES
	kcv	KCV_EMPTY
	accc	ACC_2143
	imsi	MOB_ID_IMSI
	tmsi	MOB_ID_NO_ID
	thplmn	THPLMN_01
<A>	bcch_info	BCCH_INFO_1
<B>	bcch_info	BCCH_INFO_ECL
<C>	bcch_info	BCCH_INFO_ECL
	cell_test	CELL_TEST_DISABLE
	gprs_indic	GPRS_NO

- RR_ABORT_IND
	op	OP_MODE_SIM_NO_SERV
	abcs	ABCS_CEL_SEL_FAIL
	plmn_avail	NO_PLMN_FOUND
	plmn	NOT_USED
	rxlevel	NOT_USED
	power	RF_CLASS_2

- MDL_RELEASE_REQ
	ch_type	NOT_PRESENT_8BIT
	sapi	SAPI_0

- MMR_NREG_IND
	nreg_cs	NREG_CELL_SELECTION_FAILED
	search_running	SEARCH_NOT_RUNNING
	new_forb_plmn	PLMN_NO_ID
'	limited_cause	MMR_RC_NONE

- MMR_NET_REQ
	param	NOT_USED

- RR_ACTIVATE_REQ
	plmn	PLMN_NO_ID
	op	OP_MODE_SIM_NO_SERV_A
	cksn	CKSN_NO_KEY
	kcv	KC_DELETED
	accc	ACC_CLASS_0000
	imsi	EMPTY_IMSI
	tmsi	NOT_USED
	thplmn	NOT_USED
	bcch_info	NOT_USED
	cell_test	CELL_TEST_DISABLE
	gprs_indic	GPRS_NO

- RR_ABORT_IND
	op	OP_MODE_SIM_NO_SERV4
	abcs	ABCS_CEL_SEL_FAIL
<A>	plmn_avail	NO_PLMN_FOUND
<B>	plmn_avail	TWO_PLMN_FOUND
<C>	plmn_avail	TWO_PLMN_FOUND
<A>	plmn	NOT_USED
<B>	plmn	PLMN_LIST_2_PLMN
<C>	plmn	PLMN_LIST_2_PLMN_F
<A>	rxlevel	NOT_USED
<B>	rxlevel	RXLEVEL_20_18
<C>	rxlevel	RXLEVEL_20_18
	power	RF_CLASS_2

- MMR_PLMN_IND
<A>	res	RES_OK_NO_PLMN_FND
<B>	res	RES_OK_PLMN_LST_AVAIL
<C>	res	RES_OK_PLMN_LST_AVAIL
<A>	plmn	NOT_USED
<B>	plmn	PLMN_LIST_2_PLMN_A
<C>	plmn	PLMN_LIST_2_PLMN_AF
<A>	forb_ind	NOT_USED
<B>	forb_ind	FORB_PLMN_ID
<C>	forb_ind	FORB_PLMN_ID_F
<A>	rxlevel	NOT_USED
<B>	rxlevel	RXLEVEL_20_18_A
<C>	rxlevel	RXLEVEL_20_18_A

- MMR_NREG_IND
	nreg_cs	NREG_CELL_SELECTION_FAILED
	search_running	SEARCH_NOT_RUNNING
	new_forb_plmn	PLMN_NO_ID
	limited_cause	MMR_RC_NONE

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	24.02.00	HM	Revised
	03.03.00	HM	Revised (search_running)

### MM358: PLMN Selection successful

Description:	MMI has requested a PLMN available list. It now selects a PLMN. The selection is successfull.

Preamble:	MM355B

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_RES
	plmn	PLMN_123_33

- RR_ACTIVATE_REQ
	plmn	PLMN_123_33
	op	OP_MODE_SIM_NO_SERV
	cksn	CKSN_RES
	kcv	KCV_EMPTY
	accc	ACC_2143
	imsi	MOB_ID_IMSI
	tmsi	MOB_ID_NO_ID
	thplmn	THPLMN_01
	bcch_info	BCCH_INFO_1
	cell_test	CELL_TEST_DISABLE
	gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
	op	OP_MODE_SIM
	mm_info	MM_INFO
	cid	CELL_ID_1122
	plmn	PLMN_123_33
	lac	LAC_2147
	power	RF_CLASS_2
	gprs_indic	GPRS_NO

- MMR_REG_CNF
	plmn	PLMN_123_33

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM359: PLMN Selection unsuccessful

Description:	MMI has requested a PLMN available list. It now selects a PLMN. The selection is unsuccessfull.

Preamble:	MM355B

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_RES
	plmn	PLMN_123_33

- RR_ACTIVATE_REQ
	plmn	PLMN_123_33
	op	OP_MODE_SIM_NO_SERV
	cksn	CKSN_RES
	kcv	KCV_EMPTY
	accc	ACC_2143
	imsi	MOB_ID_IMSI
	tmsi	MOB_ID_NO_ID
	thplmn	THPLMN_01
	bcch_info	BCCH_INFO_1
	cell_test	CELL_TEST_DISABLE
	gprs_indic	GPRS_NO

- RR_ABORT_IND
	op	OP_MODE_SIM_LIM_SERV
	abcs	ABCS_CEL_SEL_FAIL
	plmn_avail	ONE_PLMN_FOUND
	plmn	PLMN_LIST_FORB
	rxlevel	RXLEVEL_20
	power	RF_CLASS_2

- MDL_RELEASE_REQ
	ch_type	NOT_PRESENT_8BIT
	sapi	SAPI_0

- MMR_NREG_IND
	nreg_cs	NREG_LIMITED_SERVICE
	search_running	SEARCH_RUNNING
	new_forb_plmn	PLMN_NO_ID
	limited_cause	MMR_RC_NONE

- RR_ACTIVATE_REQ
	plmn	PLMN_123_32_A
	op	OP_MODE_SIM_NO_SERV
	cksn	CKSN_RES
	kcv	KCV_EMPTY
	accc	ACC_2143
	imsi	MOB_ID_IMSI
	tmsi	MOB_ID_NO_ID
	thplmn	THPLMN_01
	bcch_info	BCCH_INFO_1
	cell_test	CELL_TEST_DISABLE
	gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	03.03.00	HM	Revised (search_running)

### MM364: PLMN Available List to MMI

Description:	MM has requested the list of available PLMNs. This list is forwarded to MMI..
[What about the last MMR_NREG_IND? Is this really correct here? ...]

Preamble:	MM001

Variants:	<A>...<C>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
(3)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(6)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
(7)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
<A>		forb_plmn	FORB_PLMN_NONE
<B>		forb_plmn	FORB_PLMN_NONE
<C>		forb_plmn	FORB_PLMN_2
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_M
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM_1
		abcs	ABCS_CEL_SEL_FAIL
<A>		plmn_avail	NO_PLMN_FOUND
<B>		plmn_avail	TWO_PLMN_FOUND
<C>		plmn_avail	TWO_PLMN_FOUND
<A>		plmn	NOT_USED
<B>		plmn	PLMN_LIST_2_PLMN
<C>		plmn	PLMN_LIST_2_PLMN_F
<A>		rxlevel	NOT_USED
<B>		rxlevel	RXLEVEL_20_18
<C>		rxlevel	RXLEVEL_20_18
		power	RF_CLASS_2

- MMR_PLMN_IND
<A>		res	RES_OK_NO_PLMN_FND
<B>		res	RES_OK_USR_MST_SEL_PLMN
<C>		res	RES_OK_USR_MST_SEL_PLMN
<A>		plmn	NOT_USED
<B>		plmn	PLMN_LIST_2_PLMN_A
<C>		plmn	PLMN_LIST_2_PLMN_AF
<A>		forb_ind	NOT_USED
<B>		forb_ind	FORB_PLMN_ID
<C>		forb_ind	FORB_PLMN_ID_F
<A>		rxlevel	NOT_USED
<B>		rxlevel	RXLEVEL_20_18_A
<C>		rxlevel	RXLEVEL_20_18_A

- MMR_NREG_IND
		nreg_cs	NREG_CELL_SELECTION_FAILED
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)

### MM365: PLMN Selection successful (manual Mode)

Description:	MMI has requested a PLMN available list. It now selects a PLMN. The selection is successfull.

Preamble:	MM364B

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_RES
	plmn	PLMN_123_33

- RR_ACTIVATE_REQ
	plmn	PLMN_123_33
	op	OP_MODE_SIM_2
	cksn	CKSN_RES
	kcv	KCV_EMPTY
	accc	ACC_2143
	imsi	MOB_ID_IMSI
	tmsi	MOB_ID_NO_ID
	thplmn	THPLMN_01
	bcch_info	BCCH_INFO_1
	cell_test	CELL_TEST_DISABLE
	gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
	op	OP_MODE_SIM_3
	mm_info	MM_INFO
	cid	CELL_ID_1122
	plmn	PLMN_123_33
	lac	LAC_2147
	power	RF_CLASS_2
	gprs_indic	GPRS_NO

- MMR_REG_CNF
	plmn	PLMN_123_33

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM366: PLMN Selection unsuccessful (Manual Mode)

Description:	MMI has requested a PLMN available list. It now selects a PLMN. The selection is unsuccessfull.

Preamble:	MM364B

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(6)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM_4
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_FORB
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- MMR_PLMN_IND
		res	RES_OK_USR_MST_SEL_PLMN
		plmn	PLMN_LIST_1_PLMN_A
		forb_ind	FORB_PLMN_ID_1F
		rxlevel	RXLEVEL_20_A

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)

## Registration (REG_LIMITED_SERVICE, with SIM card, automatic mode)

### MM329: Registration

Description:	MM receives SIM information followed by the registration start of MMI. The cell selection is started.
Variant A:	not updated	start with HPLMN
Variant B:	updated, HPLMN	start with HPLMN
Variant C:	updated, not HPLMN	start with LPLMN

Preamble:	MM307

Variants:	<A>....<C>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
<A>		loc_info	LOC_INFO_NOT_UPD_LPLMN
<B>		loc_info	LOC_INFO_UPDATED_1
<C>		loc_info	LOC_INFO_UPDATED_LPLMN
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
<A>		forb_plmn	FORB_PLMN_NONE
<B>		forb_plmn	FORB_PLMN_2
<C>		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
<A>		plmn	PLMN_123_33
<B>		plmn	PLMN_123_33
<C>		plmn	PLMN_123_31
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
<A>		bcch_info	BCCH_INFO_1
<B>		bcch_info	BCCH_INFO_ECL
<C>		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	24.02.00	HM	Revised

### MM330: PLMN Mode Change

Description:	MMI changes the PLMN mode from automatic to manual. MM receives SIM information followed by the registration start of MMI. The cell selection is started.

Preamble:	MM307

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
TIMEOUT (3000)
(4)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(5)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_NO_SIM_NO_SERV_M
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_NOT_UPD_LPLMN
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_M
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM331: Deregistration (Power Off)

Description:	MM receives a a MMR-NREG request primitive. MM issues a RR-DEACTIVATE request primitive and a MMR-NREG confirmation primitive with cause set to 'Power off' and changes to changes to State 0 (Null).

Preamble:	MM329A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	08.07.97	HK	Initial

### MM332: Deregistration (SIM invalid)

Description:	MM receives a MMR-NREG request primitive. issues a MMR-NREG confirmation primitive with cause set to 'SIM Remove'. SIM data will not be deleted in MM, only a flag will be set that most of the data shall not be used anymore. If receiving a MMR_REG_REQ primitive with SERVICE_MODE_FULL indication, the SIM data will be revalidated.

Preamble:	MM329A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
(4)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(5)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	09.07.97	HK	Initial
	15.09.97	DL	revised
	30.08.00	HM	Revised

### MM333: SIM Removal

Description:	The SIM card is removed before starting with registration.

Preamble:	MM329A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(4)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(5)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_NO_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	08.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption caused by GPRS integration

### MM334: RR failure (No Service)

Description:	MM receives a RR-ABORT indication primitive indicating No Service.

Preamble:	MM329A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_SIM_NO_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_FOUND
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_CELL_SELECTION_FAILED
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM335: RR failure (Limited Service, no further PLMNs)

Description:	MM receives a RR-ABORT indication primitive indicating Limited Service. The one PLMN which was found is in the forbidden PLMN list and cannot be used.

Preamble:	MM329B

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_FORB
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM336: RR failure (Limited Service, further PLMNs available)

Description:	MM receives a RR-ABORT indication primitive indicating Limited Service. The one PLMN which was found is not in the forbidden PLMN list and can be used. The MMI is informed about the fact that the stack currently is in limited service, but that the search for a network delivering full service still is going on.

Preamble:	MM329A

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_LPLMN
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_REQ
		plmn	PLMN_123_31
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM337: MM Success

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell.

Preamble:	MM329B

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM338: MM Failure

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell. A location updating is started. After four rejections limited service is indicated to the user.

Preamble:	MM329B

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(6)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(9)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(11)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(13)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(16)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(17)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(18)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(19)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(20)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(21)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(23)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(24)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(25)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(26)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(27)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(28)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(29)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(30)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(31)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_UNSPECIFIED

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)

### MM339: MM Authentication Failure

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell. A location updating is started. After authentication failure only limited service shall be allowed.

Preamble:	MM329B

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REQ)	  |
	 |		  *<==============================*
(6)	 |	SIM_AUTHENTICATION_REQ	  |		  |
	 *<==============================*		  |
(7)	 |	SIM_AUTHENTICATION_CNF	  |		  |
	 *==============================>*		  |
(8)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (AUTHENTICATION RES)	  |
	 |		  *==============================>*
(9)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(11)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(13)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REQ
		ti	TI_0
		ciph_key_num	CIPH_KEY_NUM_01
		auth_rand	AUTH_RAND_1
		}

- SIM_AUTHENTICATION_REQ
		source	SRC_MM
		rand	RAND_1_P
		cksn	CKSN_01

- SIM_AUTHENTICATION_CNF
		sres	SRES_1
		kc	KC_11223344

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_AUTH_RES
		ti	TI_0
		auth_sres	SRES_1_CODED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_01
		kcv	KCV_11223344
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi 	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_AUTHREJ

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)
	27.04.01	HM	Changed ordering of primitives

### MM356: PLMN Available Request

Description:	MM is in limited service condition. MMI requests a PLMN available request and starts a PLMN search in RR. The result is forwarded to MMI. Even if no PLMN is present in the list, the service delivered from RR decides whether it is limited or no service.

Preamble:	MM300

Variants:	<A>...<C>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(7)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(8)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(10)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
(11)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	OP_NORMAL_SIM
		imsi_field	IMSI_FIELD_1
<A>		loc_info	LOC_INFO_NOT_UPD_LPLMN
<B>		loc_info	LOC_INFO_UPDATED_1
<C>		loc_info	LOC_INFO_UPDATED_LPLMN
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
<A>		forb_plmn	FORB_PLMN_NONE
<B>		forb_plmn	FORB_PLMN_NONE
<C>		forb_plmn	FORB_PLMN_2
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
	service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
<A>		plmn	PLMN_123_33
<B>		plmn	PLMN_123_33
<C>		plmn	PLMN_123_31
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
<A>		bcch_info	BCCH_INFO_1
<B>		bcch_info	BCCH_INFO_ECL
<C>		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
<A>		plmn_avail	ONE_PLMN_FOUND
<B>		plmn_avail	ONE_PLMN_FOUND
<C>		plmn_avail	NO_PLMN_FOUND
<A>		plmn	PLMN_123_33
<B>		plmn	PLMN_123_33
<C>		plmn	NOT_USED
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- MMR_NET_REQ
		param	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_A
		cksn	CKSN_NO_KEY
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	EMPTY_IMSI
		tmsi	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV4
		abcs	ABCS_CEL_SEL_FAIL
<A>		plmn_avail	NO_PLMN_FOUND
<B>		plmn_avail	TWO_PLMN_FOUND
<C>		plmn_avail	TWO_PLMN_FOUND
<A>		plmn	NOT_USED
<B>		plmn	PLMN_LIST_2_PLMN
<C>		plmn	PLMN_LIST_2_PLMN_F
<A>		rxlevel	NOT_USED
<B>		rxlevel	RXLEVEL_20_18
<C>		rxlevel	RXLEVEL_20_18
		power	RF_CLASS_2

- MMR_PLMN_IND
<A>		res	RES_OK_NO_PLMN_FND
<B>		res	RES_OK_PLMN_LST_AVAIL
<C>		res	RES_OK_PLMN_LST_AVAIL
<A>		plmn	NOT_USED
<B>		plmn	PLMN_LIST_2_PLMN_A
<C>		plmn	PLMN_LIST_2_PLMN_AF
<A>		forb_ind	NOT_USED
<B>		forb_ind	FORB_PLMN_ID
<C>		forb_ind	FORB_PLMN_ID_F
<A>		rxlevel	NOT_USED
<B>		rxlevel	RXLEVEL_20_18_A
<C>		rxlevel	RXLEVEL_20_18_A

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	24.02.00	HM	Revised
	02.03.00	HM	Revised (search_running)
	15.03.01	HM	Revised

### MM360: PLMN Selection successful

Description:	MMI has requested a PLMN available list. It now selects a PLMN. The selection is successfull.

Preamble:	MM356B

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM361: PLMN Selection unsuccessful

Description:	MMI has requested a PLMN available list. It now selects a PLMN. The selection is unsuccessfull.

Preamble:	MM356B

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_FORB
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_REQ
		plmn	PLMN_123_32_A
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised

## Registration (REG_FULL_SERVICE, with SIM card, automatic mode)

### MM340: Registration

Description:	MMI restarts registration. This is not allowed in full service. Instead a PLMN list must be requested by MMI and a selection must be done using MMR_PLMN_RES.

Preamble:	MM337

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_REG_CNF	  |		  |
	 *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.05.01	HM	Revised

### MM341: PLMN Mode Change

Description:	MMI changes the PLMN mode from automatic to manual. MM receives SIM information followed by the registration start of MMI. The cell selection is started.

Preamble:	MM337

Variants:	<A>...<B>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(6)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_2
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_NET_REQ
		param	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_M
		cksn	CKSN_NO_KEY
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	EMPTY_IMSI
		tmsi	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_SIM_MAN_MMI_SRCH_FS
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	TWO_PLMN_FOUND
<A>		plmn	PLMN_LIST_2_PLMN
<B>		plmn	PLMN_LIST_2_PLMN_F
		rxlevel	RXLEVEL_20_18
		power	RF_CLASS_2

- MMR_PLMN_IND
		res	RES_OK_USR_MST_SEL_PLMN
<A>		plmn	PLMN_LIST_2_PLMN_A
<B>		plmn	PLMN_LIST_2_PLMN_AF
<A>		forb_ind	FORB_PLMN_ID
<B>		forb_ind	FORB_PLMN_ID_F
		rxlevel	RXLEVEL_20_18_A

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM342: Deregistration (Power Off)

Description:	MM receives a a MMR-NREG request primitive. MM issues a RR-DEACTIVATE request primitive and a MMR-NREG confirmation primitive with cause set to 'Power off' and changes to changes to State 0 (Null).

Preamble:	MM337

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	08.07.97	HK	Initial

### MM343: Deregistration (SIM invalid)

Description:	MM receives a MMR-NREG request primitive and issues a MMR-NREG confirmation primitive with cause set to 'SIM Remove'.SIM data in MM will not be deleted, but only be considered invalid until a MMR_REG_REQ primitive with parameter SERVICE_MODE_FULL will be received. Compare this testcase wih MM332.

Preamble:	MM337

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
(5)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	09.07.97	HK	Initial
	15.09.97	DL	revised
	30.08.00	HM	Revised

### MM344: SIM Removal

Description:	The SIM card is removed before starting with registration.

Preamble:	MM337

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(5)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	08.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption caused by GPRS integration

### MM345: RR failure (No Service)

Description:	MM receives a RR-ABORT indication primitive indicating No Service.

Preamble:	MM337

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_SIM_NO_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_FOUND
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_CELL_SELECTION_FAILED
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM346: RR failure (Limited Service, no further PLMNs)

Description:	MM receives a RR-ABORT indication primitive indicating Limited Service. The one PLMN which was found is in the forbidden PLMN list. This indication is forwarded to MMI.

Preamble:	MM337

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_FORB
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)

### MM347: RR failure (Limited Service, further PLMNs available)

Description:	MM receives a RR-ABORT indication primitive indicating Limited Service. This indication is forwarded to MMI.

Preamble:	MM337

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_LPLMN
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_REQ
		plmn	PLMN_123_31
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

History:	10.07.97	HK	Initial
	02.03.00	HM	Revised (search_running)
	14.03.01	HM	Revised

### MM348: MM Success

Description:	MM is informed about a cell change.

Preamble:	MM337

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_31
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_31_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_31
		lac	LAC_2147
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_31_2147
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM349: MM Failure

Description:	MM is informed about a cell change. A location updating is started. After four rejections MM enters the state ATTEMPTING TO UPDATE and no further reactions are expected.

Preamble:	MM337

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(12)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(15)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(16)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(17)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(18)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(19)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(20)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(21)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(22)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(23)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(24)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(25)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(26)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(27)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(28)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(29)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(30)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_31
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_UNSPECIFIED
		}

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	BCCH_INF_2
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_UNSPECIFIED

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	15.01.01	HM	Revised (last NREG_IND added)

### MM350: MM Authentication Failure

Description:	MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell. A location updating is started. After authentication failure only limited service shall be allowed.

Preamble:	MM329B

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  |		  |
	 *<==============================*		  |
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REQ)	  |
	 |		  *<==============================*
(6)	 |	SIM_AUTHENTICATION_REQ	  |		  |
	 *<==============================*		  |
(7)	 |	SIM_AUTHENTICATION_CNF	  |		  |
	 *==============================>*		  |
(8)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (AUTHENTICATION RES)	  |
	 |		  *==============================>*
(9)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REJ)	  |
	 |		  *<==============================*
(11)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(13)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REQ
		ti	TI_0
		ciph_key_num	CIPH_KEY_NUM_01
		auth_rand	AUTH_RAND_1
		}

- SIM_AUTHENTICATION_REQ
		source	SRC_MM
		rand	RAND_1_P
		cksn	CKSN_01

- SIM_AUTHENTICATION_CNF
		sres	SRES_1
		kc	KC_11223344

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_AUTH_RES
		ti	TI_0
		auth_sres	SRES_1_CODED
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_01
		kcv	KCV_11223344
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REJ
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi 	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL_NO_PAG
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_PLMN_NOT_ALLOW
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_NO_KEY
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_AUTHREJ

History:	09.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)
	27.04.01	HM	Changed ordering of primitives

### MM357: PLMN Available Request

Description:	MM is in full service condition. MMI requests a PLMN available request and starts a PLMN search in RR. The result is forwarded to MMI.

Preamble:	MM329B

Variants:	<A>...<C>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(3)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(6)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- MMR_NET_REQ
	param	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_A
		cksn	CKSN_NO_KEY
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	EMPTY_IMSI
		tmsi	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM_SERV_A
		abcs	ABCS_CEL_SEL_FAIL
<A>		plmn_avail	NO_PLMN_FOUND
<B>		plmn_avail	TWO_PLMN_FOUND
<C>		plmn_avail	TWO_PLMN_FOUND
<A>		plmn	NOT_USED
<B>		plmn	PLMN_LIST_2_PLMN
<C>		plmn	PLMN_LIST_2_PLMN_F
<A>		rxlevel	NOT_USED
<B>		rxlevel	RXLEVEL_20_18
<C>		rxlevel	RXLEVEL_20_18
		power	RF_CLASS_2

- MMR_PLMN_IND
<A>		res	RES_OK_NO_PLMN_FND
<B>		res	RES_OK_PLMN_LST_AVAIL
<C>		res	RES_OK_PLMN_LST_AVAIL
<A>		plmn	NOT_USED
<B>		plmn	PLMN_LIST_2_PLMN_A
<C>		plmn	PLMN_LIST_2_PLMN_AF
<A>		forb_ind	NOT_USED
<B>		forb_ind	FORB_PLMN_ID
<C>		forb_ind	FORB_PLMN_ID_F
<A>		rxlevel	NOT_USED
<B>		rxlevel	RXLEVEL_20_18_A
<C>		rxlevel	RXLEVEL_20_18_A

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised

### MM362: PLMN Selection successful

Description:	MMI has requested a PLMN available list. It now selects a PLMN. The selection is successfull.

Preamble:	MM357B

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	24.03.00	HM	Revised

### MM363: PLMN Selection unsuccessful

Description:	MMI has requested a PLMN available list. It now selects a PLMN. The selection is unsuccessfull.

Preamble:	MM357B

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_FORB
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	07.07.97	HK	Initial
	06.08.97	DL	Revised
	12.08.97	HK	Revised
	02.03.00	HM	Revised (search_running)

## MM Idle Mode Behaviour (Normal Service)

### MM400: Normal Service, new cell, same location area

Description:	MM is in service state NORMAL SERVICE. A new cell is entered in the same location area. The new cell indicates IMSI attach. It is assumed that no location updating is started.

Preamble:	MM024

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
TIMEOUT (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1123
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

History:	03.05.99	LE	Initial

### MM401: Normal Service, new cell, new location area

Description:	MM is in service state NORMAL SERVICE. A new cell is entered in a new location area. A normal location updating is started.

Preamble:	MM024

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	07.07.97	HK	Initial

### MM402: Normal Service, new cell, new PLMN identification

Description:	MM is in service state NORMAL SERVICE. A new cell is entered in a new PLMN. A normal location updating is started.

Preamble:	MM024

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	07.07.97	HK	Initial

### MM403: Updated, IMSI Attach

Description:	MM receives a SIM-INSERT indication primitive and initiates cell selection by issuing a RR-ACTIVATE request primitive. Successful conclusion of cell selection is signalled by the receipt of a RR-ACTIVATE confirmation primitive. MM forwards the PLMN identification to MMI in the form of a MMR-REG confirmation primitive. An IMSI attach is started.

Preamble:	MM022

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial
	24.02.00	HM	Revised

### MM404: Normal Service, Timeout T3211, LUP Reject Cause #17

Description:	The location updating is finished with a location updating reject message and the cause #17 network failure. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(5)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM405: Normal Service, Timeout T3211, LUP Reject Cause, 2.-4. attempt

Description:	The location updating is finished with a location updating reject message and the cause #17 network failure. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM404

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(5)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(6)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(8)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(11)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(13)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(16)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

History:	04.05.99	LE	Initial

### MM406: Updated, Periodic LUP

Description:	MM receives a SIM-INSERT indication primitive and initiates cell selection by issuing a RR-ACTIVATE request primitive. Successful conclusion of cell selection is signalled by the receipt of a RR-ACTIVATE confirmation primitive. MM forwards the PLMN identification to MMI in the form of a MMR-REG confirmation primitive. The periodic location updating timer is started.

Preamble:	MM022

MMI/CM		 MM		 RR/DL
	 |		  |		  |
COMMAND (MM CONFIG T3212_CNT=5)
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
TIMEOUT (55000)
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_PER
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial
	24.02.00	HM	Revised

### MM407: Normal Service, Timeout T3211, LUP Reject Cause #17

Description:	The location updating is finished with a location updating reject message and the cause #17 network failure. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM406

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(5)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM408: Normal Service, Timeout T3211, LUP Reject Cause, 2.-4. attempt

Description:	The location updating is finished with a location updating reject message and the cause #17 network failure. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM407

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(5)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(6)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(8)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(11)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(13)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(16)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(17)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_NETWORK_FAILURE

History:	04.05.99	LE	Initial
	15.01.01	HM	Revised (last NREG_IND added)

### MM409: Normal Service, Timeout T3211, RR Release before end of proc

Description:	The RR connection is established and released before receiving an answer by the network. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM410: Normal Service, T3211, RR Release before end of proc, 2.-4. attempt

Description:	The RR connection is established and released before receiving an answer by the network. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM409

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(14)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	25.01.01	HM	Revised (last MMR_NREG_IND added)

### MM411: Normal Service, T3211, RR Release before end of proc

Description:	The RR connection is established and released before receiving an answer by the network. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM406

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM412: Normal Service, T3211, RR Release before end of proc, 2.-4. attempt

Description:	The RR connection is established and released before receiving an answer by the network. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM411

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(14)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	25.01.01	HM	Revised (last MMR_NREG_IND added)

### MM413: Normal Service, T3211, Timeout T3210

Description:	The RR connection is established and no answer receives from the network. After timeout of T3210 the RR connection is aborted. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(5)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial
	27.04.01	HM	Changed abort behaviour

### MM414: Normal Service, T3211, Timeout T3210, 2.-4. attempt

Description:	The RR connection is established and no answer of the network receives. After timeout of T3210 the connection is aborted. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM413

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(5)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(6)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(7)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(8)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(11)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(12)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(13)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(16)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(17)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	27.04.01	HM	Changed abort behaviour

### MM415: Normal Service, T3211, Timeout T3210

Description:	The RR connection is established and there is no answer by the network. After timeout of T3210 a connection abort is processed. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM406

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(5)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial
	27.04.01	HM	Changed abort behaviour

### MM416: Normal Service, T3211, Timeout T3210, 2.-4. attempt

Description:	The RR connection is established and there is no answer by the network. After timeout T3210 the connection is aborted. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM415

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(5)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(6)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(7)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(8)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(11)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(12)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(13)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(16)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(17)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	27.04.01	HM	Changed abort behaviour

### MM417: Normal Service, T3211, RR connection failure

Description:	The RR connection is established and released before receiving an answer by the network due to a radio link failure. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM418: Normal Service, T3211, RR Connection Failure, 2.-4. attempt

Description:	The RR connection is established and released before receiving an answer by the network due to a radio link failure. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM417

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

History:	04.05.99	LE	Initial

### MM419: Normal Service, T3211, RR Connection Failure

Description:	The RR connection is established and released before receiving an answer by the network due to a radio link failure. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM406

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM420: Normal Service, T3211, RR Connection Failure, 2.-4. attempt

Description:	The RR connection is established and released before receiving an answer by the network due to a radio link failure. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM419

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

History:	04.05.99	LE	Initial

### MM421: Normal Service, T3211, Random Access Failure

Description:	An Random Access Failure occurs. After timeout T3213 the next attempt is started which fails with the same cause. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM422: Normal Service, T3211, Random Access Failure, 2.-4. attempt

Description:	A random access failure occurs. T3213 is started. The next attempt fails due to the same cause. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM421

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(12)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(13)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(15)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(16)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(18)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(19)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

History:	04.05.99	LE	Initial

### MM423: Normal Service, T3211, Random Access Failure

Description:	A random access failure occurs. T3213 is started. After timeout a new attempt is started. The attempt fails due to the same cause. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM406

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM424: Normal Service, T3211, Random Access Failure, 2.-4. attempt

Description:	A random access failure occurs. T3213 is started. The next attempt fails due to the same cause. The update status is UPDATED and the stored LAI is equal to the one received on the BCCH from the current serving cell and the attempt counter is smaller than 4. The MS shall keep the update status to UPDATED, the MM IDLE substate after the RR connection release is NORMAL SERVICE. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered.

Preamble:	MM423

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(12)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(13)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(15)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(16)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(18)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(19)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

History:	04.05.99	LE	Initial

### MM425: Random access delayed

Description:	MM receives a RR-RELEASE indication primitive from RR; Normal Location Updating cannot be carried out because of random access delay. MM issues a MDL-RELEASE request primitive. After T3122 timeout in RR a new attempt is started.

Preamble:	MM401

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (30000)
(3)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_DELAY
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_T3122_TIM_OUT
		chm	CHM_NOT_PRESENT

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial

### MM426: Random access barred

Description:	MM receives a RR-RELEASE indication primitive from RR; Normal Location Updating cannot be carried out because of access barred. MM issues a MDL-RELEASE request primitive. After access control class changes in RR a new attempt is started.

Preamble:	MM401

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (30000)
(3)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_ACCESS_BARRED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_ACC_CLS_CHA
		chm	CHM_NOT_PRESENT

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial

### MM427: Not Updated in Current LA, Start Normal Location Updating

Description:	MM receives a SIM-INSERT indication primitive and initiates cell selection by issuing a RR-ACTIVATE request primitive. Successful conclusion of cell selection is signalled by the receipt of a RR-ACTIVATE confirmation primitive. MM forwards the PLMN identification to MMI in the form of a MMR-REG confirmation primitive. A normal location updating is started.

Preamble:	MM022

MMI/CM		 MM		 RR/DL
	 |		  |		  |
COMMAND (MM CONFIG T3212_CNT=5)
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_2
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_PER
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_0002
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial
	24.02.00	HM	Revised

### MM600: Normal Service, T3211, Anite Behaviour

Description:	The radio link failure in the preamble is realized by the Anite by simply switching off the BCCH carrier. This leads to a cell reselection problem in RR and a cell selection failure is indicated to MM. RR retries the synchronisation and forwards a RR ACTIVATE IND to MM indicating that it is back to full service.

Preamble:	MM406

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (20000)
(8)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(13)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(14)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(15)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(17)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(18)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(19)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(20)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(21)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(22)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(23)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ABORT_IND
		op	OP_MODE_SIM_NO_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_FOUND
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_NO_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info 	MM_INFO_PER
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	18.06.01	HM	Revised

## MM Idle Mode Behaviour (Attempt to Update)

### MM428: Attempt to Update, Timeout T3211, LUP Reject Cause #17

Description:	The location updating is finished with a location updating reject message and the cause #17 network failure. The update status is NOT UPDATED. The MS shall delete the location information and enter the MM IDLE substate ATTEMPTING TO UPDATE after the RR connection release. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again.

Preamble:	MM427

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM429: Attempt to update, Timeout T3211, LUP Reject Cause, 2.-4. attempt

Description:	The location updating is finished with a location updating reject message and the cause #17 network failure. The update status is NOT UPDATED. The MS is in MM IDLE substate ATTEMPTING TO UPDATE after the RR connection release. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again. After the 4. Attempt the MM IDLE substate ATTEMPTING TO UPDATE is entered and no new location updating is triggered.

Preamble:	MM428

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(14)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(15)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(16)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(17)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(20)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(21)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (20000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_NETWORK_FAILURE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM430: Not Updated, Periodic LUP

Description:	MM has processed four unsuccessful location updatings and stays in the MM idle sub-state ATTEMPTING TO UPDATE. After timeout of the periodic location updating timer a new set of normal location updatings is started.

Preamble:	MM429

MMI/CM		 MM		 RR/DL
	 |		  |		  |
TIMEOUT (55000)
(1)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM431: Periodic not updated, Timeout T3211, LUP Reject Cause #17

Description:	The MS is not updated. After timeout of T3212 a new attempt is made. The MS remains in ATTEMPTING TO UPDATE state. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again

Preamble:	MM430

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(14)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(15)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(16)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(17)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(20)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(21)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(22)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(23)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(24)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(25)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(26)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(27)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(28)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (20000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	RC_NETWORK_FAILURE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM432: Attempt to Udpate, Normal, T3211, RR Release before end of proc

Description:	The RR connection is established and released before receiving an answer by the network. The MS shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again.

Preamble:	MM428

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(7)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(12)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(13)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(14)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(17)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(18)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM433: Periodic, Attempt to Update, T3211, RR Release before end of proc

Description:	After timeout of the periodic location updating timer a new set of four attempts for normal location updating is started.

Preamble:	MM432

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
TIMEOUT (55000)
(1)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(2)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(13)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(14)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(15)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(17)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(18)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(19)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(20)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(21)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(22)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(24)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(25)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM434: Attempt to Udpate, Normal, T3211, Timeout T3210

Description:	The RR connection is established and no answer receives from the network. After timeout of T3210 the RR connection is aborted. The MS shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again.

Preamble:	MM428

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(2)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(3)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(9)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(10)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(14)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(15)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(16)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(17)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(20)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(21)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	27.04.01	HM	Change of abort behaviour

### MM435: Periodic, Attempt to Update, T3211, Timeout T3210

Description:	After timeout of the periodic location updating timer a new set of four attempts for normal location updating is started.

Preamble:	MM434

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
TIMEOUT (55000)
(1)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(2)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(3)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(4)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(10)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(11)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(14)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(15)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(16)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(17)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(18)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(20)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(21)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(22)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(23)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
TIMEOUT (17000)
(24)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(25)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(26)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(27)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(28)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(29)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	27.04.01	HM	Changed abort behaviour

### MM436: Attempt to Update, Normal, T3211, RR connection failure

Description:	The RR connection is established and a radio link failure is detected. The MS shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again.

Preamble:	MM428

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(7)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(12)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(13)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(14)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(17)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(18)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM437: Periodic, Attempt to Update, T3211, RR Connection Failure

Description:	After timeout of the periodic location updating timer a new set of four attempts for normal location updating is started.

Preamble:	MM436

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
TIMEOUT (55000)
(1)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(2)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(13)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(14)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(15)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(17)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(18)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(19)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(20)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(21)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(22)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(24)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(25)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM438: Attempt to Update, Normal, T3211, RR connection failure

Description:	The RR connection establishment fails due to random access failure. After timeout of T3213 a second attempt is started. This attempt fails due to the same failure. The MS shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again.

Preamble:	MM428

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(3)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(4)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(11)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(12)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(15)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(16)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(17)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(19)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(20)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(21)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(23)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(24)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM439: Periodic, Attempt to Update, T3211, Random Access Failure

Description:	After timeout of the periodic location updating timer a new set of four attempts for normal location updating is started.

Preamble:	MM438

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
TIMEOUT (55000)
(1)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(2)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(12)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(13)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(16)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(17)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(18)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(20)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(21)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(22)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(24)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(25)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(26)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(27)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (2000)
(28)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(29)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(30)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(31)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(32)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(33)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM440: Random access delayed

Description:	MM receives a RR-RELEASE indication primitive from RR; Normal Location Updating cannot be carried out because of random access delay. MM issues a MDL-RELEASE request primitive. After T3122 timeout in RR a new attempt is started.

Preamble:	MM427

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (30000)
(3)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_RND_ACC_DELAY
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_T3122_TIM_OUT
		chm	CHM_NOT_PRESENT

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_0002
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial

### MM441: Random access barred

Description:	MM receives a RR-RELEASE indication primitive from RR; Normal Location Updating cannot be carried out because of access barred. MM issues a MDL-RELEASE request primitive. After access control class changes in RR a new attempt is started.

Preamble:	MM427

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (30000)
(3)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(4)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_RELEASE_IND
		relcs	RELCS_ACCESS_BARRED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_ACC_CLS_CHA
		chm	CHM_NOT_PRESENT

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_0002
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	09.07.97	HK	Initial

### MM442: Normal Service, IMSI Detach, Power OFF, Est Cnf

Description:	MM is in IDLE Normal Service state. It is switched off. An IMSI Detach followed by the deactivation of the lower layer is started.
[=MM445]

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(12)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_REQ
		cs	CS_POW_OFF

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	05.05.99	LE	Initial
	30.08.00	HM	Revised
	27.04.01	HM	Changed T3220 behaviour

### MM443: Normal Service, IMSI Detach, Power OFF, Rel Ind

Description:	MM is in IDLE Normal Service state. It is switched off. An IMSI Detach followed by the deactivation of the lower layer is started.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(10)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_REQ
		cs	CS_POW_OFF

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	05.05.99	LE	Initial
	30.08.00	HM	Revised

### MM444: Normal Service, IMSI Detach, Power OFF, Radio Link Failure

Description:	MM is in IDLE Normal Service state. It is switched off. An IMSI Detach followed by the deactivation of the lower layer is started.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(10)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_REQ
		cs	CS_POW_OFF

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	05.05.99	LE	Initial
	30.08.00	HM	Revised

### MM446: Normal Service, IMSI Detach, SIM Remove by SIM, Est Cnf

Description:	MM is in IDLE Normal Service state. The SIM manager has detected a SIM Remove. An IMSI Detach is processed. After timeout T3220 in state IMSI DETACH INITIATED; the MS enters the IDLE NO IMSI state.
[=MM449]

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(12)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- SIM_REMOVE_IND
		error	NOT_USED

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	05.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption caused by GPRS integration
	18.04.01	HM	Revised (nreg_cs)
	27.04.01	HM	Changed behaviour of T3220

### MM447: Normal Service, IMSI Detach, SIM Remove by SIM, Rel Ind

Description:	MM is in IDLE Normal Service state. The SIM manager has detected a SIM Remove. An IMSI Detach is processed. The RR connection establishment fails and the MS enters the IDLE NO IMSI state.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(10)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- SIM_REMOVE_IND
		error	NOT_USED

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	05.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption caused by GPRS integration
	18.04.01	HM	Revised (nreg_cs)

### MM448: Normal Service, IMSI Detach, SIM Remove by SIM, Radio Link Fail

Description:	MM is in IDLE Normal Service state. The SIM manager has detected a SIM Remove. An IMSI Detach is processed. After radio link failure, the MS enters the IDLE NO IMSI state.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(10)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- SIM_REMOVE_IND
		error	NOT_USED

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	05.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption caused by GPRS integration
	18.04.01	HM	Revised (nreg_cs)

### MM450: Normal Service, IMSI Detach, SIM Remove by MMI, Est Cnf

Description:	MM is in IDLE Normal Service state. The MMI requests limited service. An IMSI Detach is processed. After timeout T3220 in state IMSI DETACH INITIATED; the MS enters the IDLE NO IMSI state.
[=MM453]

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(9)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(12)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_REQ
		cs	CS_SIM_REM

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

History:	05.05.99	LE	Initial
	30.08.00	HM	Revised
	30.04.01	HM	Revised

### MM451: Normal Service, IMSI Detach, SIM Remove by MMI, Rel Ind

Description:	MM is in IDLE Normal Service state. MMI requests limited service. An IMSI Detach is processed. The RR connection establishment is released by the network and the MS enters the IDLE NO IMSI state.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(11)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_REQ
		cs	CS_SIM_REM

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

History:	05.05.99	LE	Initial
	30.08.00	HM	Revised

### MM452: Normal Service, IMSI Detach, SIM Remove by MMI, Radio Link Fail

Description:	MM is in IDLE Normal Service state. MMI has requested limited service. An IMSI Detach is processed. After radio link failure, the MS enters the IDLE NO IMSI state.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(11)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_REQ
		cs	CS_SIM_REM

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

History:	05.05.99	LE	Initial
	30.08.00	HM	Revised

### MM454: IMSI Attach and CM Connection Establishment

Description:	MM is switched on, processes an IMSI attach and starts a connection establishment.

Preamble:	MM403

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(9)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(11)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	RR_DATA_IND	  |
	 |		  |	(CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(13)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_2147
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_ESTABLISH_REQ
		ti	TI_2
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_2

History:	23.04.99	LE	Initial

### MM455: Connection Active, IMSI Detach, Power OFF, Timeout T3220

Description:	MM is in Connection Active state. It is switched off. An IMSI Detach is started. After timeout of T3220 deactivation of the lower layer is started.

Preamble:	MM454

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  *==============================>*
TIMEOUT(2000)
(4)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	05.05.99	LE	Initial
	30.08.00	HM	Revised
	27.04.01	HM	Changed T3220 behaviour

### MM456: Connection Active, IMSI Detach, Power OFF, Rel Ind

Description:	MM is in Connection Active state. It is switched off. An IMSI Detach is started. The network releases the RR-Connection .Deactivation of the lower layer is started.

Preamble:	MM454

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(7)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	05.05.99	LE	Initial
	30.08.00	HM	Revised

### MM457: Connection Active, IMSI Detach, Power OFF, Radio Link Failure

Description:	MM is in Connection Active state. It is switched off. An IMSI Detach is started. A radio link failure occurs .Deactivation of the lower layer is started.

Preamble:	MM454

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(7)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	05.05.99	LE	Initial
	30.08.00	HM	Revised

### MM458: Active, IMSI Detach, SIM Remove by SIM, Rel Ind

Description:	MM is in active state. The SIM manager has detected a SIM Remove. All CM connections are released. An IMSI Detach is processed. The network releases the RR connection and the MS enters the IDLE NO IMSI state.

Preamble:	MM454

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(7)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	05.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption after GPRS integration

### MM459: Active, IMSI Detach, SIM Remove by SIM, Radio Link Fail

Description:	MM is in active state. The SIM manager has detected a SIM Remove. All CM connections are released. An IMSI Detach is processed. After radio link failure, the MS enters the IDLE NO IMSI state.

Preamble:	MM454

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(7)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	05.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption after GPRS integration

### MM460: Active, IMSI Detach, SIM Remove by SIM, T3220 Timeout

Description:	MM is in active state. The SIM manager has detected a SIM Remove. All CM connections are released. An IMSI Detach is processed. After timeout T3220, the MS enters the IDLE NO IMSI state.

Preamble:	MM454

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  *==============================>*
TIMEOUT(2000)
(4)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	05.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption after GPRS integration
	27.04.01	HM	Changed T3220 behaviour

### MM461: Active, IMSI Detach, SIM Remove by MMI, Rel Ind

Description:	MM is in active state. MMI requests limited service. All CM connections are released. An IMSI Detach is processed. The RR connection is released by the network and the MS enters the IDLE NO IMSI state.

Preamble:	MM454

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(7)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

History:	05.05.99	LE	Initial
	31.08.00	HM	Revised

### MM462: Active, IMSI Detach, SIM Remove by MMI, Radio Link Fail

Description:	MM is in active state. MMI has requested limited service. All CM connections are released. An IMSI Detach is processed. After radio link failure, the MS enters the IDLE NO IMSI state.

Preamble:	MM454

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(7)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		abcs	ABCS_RAD_LNK_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

History:	05.05.99	LE	Initial
	31.08.00	HM	Revised

### MM463: Active, IMSI Detach, SIM Remove by MMI, T3220 Timeout

Description:	MM is in Active state. MMI has requested limited service. All CM connections are released. An IMSI Detach is processed. After timeout T3220, the MS enters the IDLE NO IMSI state.

Preamble:	MM454

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(5)	 |		  | 	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	NOT_USED

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

History:	05.05.99	LE	Initial
	31.08.00	HM	Revised
	30.04.01	HM	Revised

### MM464: Attempt to update, change of location area identification

Description:	The MS is in attempting to update state. RR indicates a change of location area identification. A normal location updating is started.

Preamble:	MM431

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM465: Attempt to update, change of cell, random access failure

Description:	The MS is in attempting to update state. The state was entered due to random access failure. RR indicates a change of cell. A normal location updating is started.

Preamble:	MM439

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1123
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM466: Attempt to update, change of cell, RR connection failure

Description:	The MS is in attempting to update state. The state was entered due to RR connection failure. RR indicates a change of cell. A normal location updating is started.

Preamble:	MM438

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1123
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM467: Attempt to update, change of cell, Release before end of proc

Description:	The MS is in attempting to update state. The state was entered due to release before end of procedure with cause different to abnormal release, unspecified. RR indicates a change of cell. A normal location updating is started.

Preamble:	MM432

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1123
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial

### MM468: Attempt to update, change of cell, Release before end of proc

Description:	The MS is in attempting to update state. The state was entered due to release before end of procedure with cause to abnormal release, unspecified. RR indicates a change of cell. A normal location updating is started.

Preamble:	MM428

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(7)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(12)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(13)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(14)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(17)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(18)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
TIMEOUT (10000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1123
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

History:	04.05.99	LE	Initial

### MM469: Attempt to update, change of cell, Location Updating Request

Description:	The MS is in attempting to update state. The state was entered due to location updating request. with cause different from retry upon entry into a new cell. RR indicates a change of cell. No normal location updating shall be started.

Preamble:	MM429

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1123
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

History:	04.05.99	LE	Initial

### MM470: Attempt to update, change of cell, Location updating reject

Description:	The MS is in attempting to update state. The state was entered due to location updating reject with the cause retry upon entry into a new cell. RR indicates a change of cell. A normal location updating is started.

Preamble:	MM428

(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(14)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(15)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(16)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(17)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(20)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(21)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (20000)
(22)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(23)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_RETRY_UPON_NEW_CELL
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_RETRY_UPON_NEW_CELL

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1123
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM471: Attempt to Update, IMSI Detach, Power OFF

Description:	MM is in IDLE Attempting to update state. It is switched off. MM shall not perform IMSI Detach. It shall only deactivate the lower layer.

Preamble:	MM429

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	05.05.99	LE	Initial

### MM472: Attempt to Update, IMSI Detach, SIM Remove by SIM

Description:	MM is in Attempt to update state. The SIM manager has detected a SIM Remove. MM shall not process IMSI Detach. MM enters the IDLE NO IMSI state.

Preamble:	MM429

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	05.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption after GPRS integration
	18.04.01	HM	Revised (nreg_cs)

### MM473: Attempt to Update, IMSI Detach, SIM Remove by MMI

Description:	MM is in Attempt to Update state. The MMI requests limited service. An IMSI Detach shall not be processed. The MS enters the IDLE NO IMSI state.

Preamble:	MM429

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

History:	05.05.99	LE	Initial
	31.08.00	HM	Revised

### MM474: Attempt to update, Emergency Call

Description:	MM receives an establishment request from CC while in State 19.2 (Idle Attempting to Update). It then sends a CM SERVICE REQUEST message to the network. On receipt of a RR-ESTABLISH confirmation primitive from the network followed by a CM SERVICE ACCEPT message, MM issues as MMCC-ESTABLISH confirmation primitive.

Preamble:	MM429

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(5)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

History:	09.07.97	HK	Initial

### MM475: Attempt to update, CM connection request (I)

Description:	MM receives a MMCC-ESTABLISH request primitive from CC in attempting to update state. This is a trigger to start location updating (with follow on request). MM stores the connection request until Location Updating has been completed. The location updating fails. After timeout in CC the connection attempt is cleared. In the next attempt the follow on request bit shall be cleared.

Preamble:	MM429

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(9)	 |	MMCC_RELEASE_REQ	  |		  |
	 *==============================>*		  |
TIMEOUT (10000)
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL_FOL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- MMCC_RELEASE_REQ
		ti	TI_4

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	15.09.97	DL	Initial

### MM476: Attempt to update, CM connection request (II)

Description:	MM receives a MMCC-ESTABLISH request primitive from CC in attempting to update state. This is a trigger to start location updating (with follow on request). MM stores the connection request until Location Updating has been completed. The location updating fails. After timeout a second attempt is started. The follow on request bit shall be set. The location updating is successful indicating no follow on proceed. MM sends the CM SERVICE REQUEST message after establishment a new RR-connection after the release of the old one.

Preamble:	MM429

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING ACC)	  |
	 |		  *<==============================*
(12)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(13)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(15)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(16)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(17)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(20)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(21)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(22)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL_FOL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL_FOL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_2147
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_4

History:	15.09.97	DL	Initial

### MM477: Attempt to update, CM connection request (III)

Description:	MM receives a MMCC-ESTABLISH request primitive from CC in attempting to update state. This is a trigger to start location updating (with follow on request). MM stores the connection request until Location Updating has been completed. The follow on request bit shall be set. The location updating is successful indicating no follow on proceed. MM doesn't release the connection request, after release of RR and layer 2 connection a RR_ESTABLISH_REQ is sent to establish the call.

Preamble:	MM429

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING ACC)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(9)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL_FOL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_2147
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_TMSI
		}

History:	15.09.97	DL	Initial
	16.02.00	HM	Revised

### MM478: Attempt to update, CM connection request (IV)

Description:	MM receives a MMCC-ESTABLISH request primitive from CC in attempting to update state. This is a trigger to start location updating (with follow on request). MM stores the connection request until Location Updating has been completed. The follow on request bit shall be set. The location updating is successful indicating follow on proceed. MM sends the CM SERVICE REQUEST on the existing RR-connection.

Preamble:	MM429

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING ACC)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(9)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(10)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(11)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(12)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL_FOL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	TRUE
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_2147
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_4

- History:	15.09.97	DL	Initial

## MM Idle Mode Behaviour (Limited Service)

### MM479: Limited Service State

Description:	MM receives a SIM-INSERT indication primitive and initiates cell selection by issuing a RR-ACTIVATE request primitive. RR indicates only limited service. The periodic location updating timer shall not be started.

Preamble:	MM022

MMI/CM		 MM		 RR/DL
	 |		  |		  |
COMMAND (MM CONFIG T3212_CNT=6)
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM_LIM_SERV
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	04.05.99	LE	Initial
	24.02.00	HM	Revised
	02.03.00	HM	Revised (search_running)

### MM480: Limited, not perform Periodic LUP

Description:	The testcase waits 70 seconds. If the periodic LUP timer is started in the preamble, the testcase will fail, else MM has the expected behaviour. T3212 is decreased to 60 seconds by a dynamic config.

Preamble:	MM479

MMI/CM		 MM		 RR/DL
	 |		  |		  |
TIMEOUT (70000)
	 |		  |		  |

- Parametrization

- Primitive	Parameter	Value

History:	09.06.99	LE	Initial

### MM481: Limited, IMSI Detach, Power OFF

Description:	MM is in IDLE limited service state. It is switched off. MM shall not perform IMSI Detach. It shall only deactivate the lower layer.

Preamble:	MM479

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	09.06.99	LE	Initial

### MM482: Limited, IMSI Detach, SIM Remove by SIM

Description:	MM is in Limited service state. The SIM manager has detected a SIM Remove. MM shall not process IMSI Detach. MM enters the IDLE NO IMSI state.

Preamble:	MM479

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	09.06.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption after GPRS integration

### MM483: Limited, IMSI Detach, SIM Remove by MMI

Description:	MM is in Limited service  state. The MMI requests limited service. An IMSI Detach shall not be processed. The MS enters the IDLE NO IMSI state.

Preamble:	MM479

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

History:	09.06.99	LE	Initial
	31.08.00	HM	Revised

### MM484: Limited, Call attempts, Emergency Calls

Description:	MM is in idle limited service state. It shall reject all requests from CM entities except emergency calls. First the rejects are tested, then an emergency call. Then the whole sequence is tested again to check coming back in limited service state.

Preamble:	MM479

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |	MMSS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |	MMSS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(5)	 |	MMSMS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |	MMSMS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(7)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(9)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(11)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
(12)	 |	MMCC_RELEASE_REQ	  | 		  |
	 *==============================>*		  |
TIMEOUT (5000)
(13)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(16)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(17)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(18)	 |	MMSS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(19)	 |	MMSS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(20)	 |	MMSMS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(21)	 |	MMSMS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(22)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(23)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(24)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(25)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(26)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
(27)	 |	MMCC_RELEASE_REQ	  | 		  |
	 *==============================>*		  |
TIMEOUT (5000)
(28)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(29)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(30)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_RELEASE_IND
		ti	TI_4
		relcs	RELCS_NO_REGISTRATION

- MMSS_ESTABLISH_REQ
		ti	TI_3

- MMSS_RELEASE_IND
		ti	TI_3
		relcs	RELCS_NO_REGISTRATION

- MMSMS_ESTABLISH_REQ
		ti	TI_5

- MMSMS_RELEASE_IND
		ti	TI_5
		relcs	RELCS_NO_REGISTRATION

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

- MMCC_RELEASE_REQ
		ti	TI_3

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_RELEASE_IND
		ti	TI_4
		relcs	RELCS_NO_REGISTRATION

- MMSS_ESTABLISH_REQ
		ti	TI_3

- MMSS_RELEASE_IND
		ti	TI_3
		relcs	RELCS_NO_REGISTRATION

- MMSMS_ESTABLISH_REQ
		ti	TI_5

- MMSMS_RELEASE_IND
		ti	TI_5
		relcs	RELCS_NO_REGISTRATION

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

- MMCC_RELEASE_REQ
		ti	TI_3

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	09.06.99	LE	Initial
	26.01.01	HM	Revised

### MM485: Limited, new cell, new LA, but only limited service

Description:	MM is in service state Idle limited service. A.) A new cell is entered. B.) A new location area is entered. RR indicates only limited service. No reaction of MM is exspected.

Preamble:	MM479

Variants:	<A>....<B>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |	MMR_NREG_IND	  |		  |
	 *<==============================*		  |
TIMEOUT (10000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM_LIM_SERV
		mm_info	MM_INFO
<A>		cid	CELL_ID_1123
<B>		cid	CELL_ID_1122
		plmn	PLMN_123_44
<A>		lac	LAC_0002
<B>		lac	LAC_0001
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	09.06.99	LE	Initial
	02.03.00	HM	Revised (search_running, variants)

### MM487: Limited, new location area, full service

Description:	MM is in service state Idle limited service. A new location area is entered. RR indicates full service. MM shall start normal location updating.

Preamble:	MM479

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	09.06.99	LE	Initial

### MM488: Limited Service, LUP Reject Cause #17

Description:	The location updating is finished with a location updating reject message and the cause #17 network failure. The update status is NOT UPDATED and the the MM IDLE substate after the RR connection release is idle attempting to update. The MS shall memorize the location updating type used in the location updating procedure. It shall start timer T3211 when the RR connection is released. When timer T3211 expires the location updating procedure is triggered again with the memorized location updating type.

Preamble:	MM487

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCATION UPDATING REJ)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_REJ
		ti	TI_0
		rej_cause	RC_NETWORK_FAILURE
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	synccs_TMSI_CKSN_KC_INVAL
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_123_33_FEFF
		bcch_inf	NOT_USED
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	09.06.99	LE	Initial

### MM489: Limited Service, Location updating accept

Description:	MM is in idle limited service. After cell reselection a location updating is started. A connection establishment is started after successful location updating.

Preamble:	MM487

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(7)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(8)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(10)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(11)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(12)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(13)	 |		  |	RR_DATA_IND	  |
	 |		  |	(CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(14)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_0002
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_0002
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_3
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_ESTABLISH_REQ
		ti	TI_2
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_NEW_TMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_2

History:	09.06.99	LE	Initial

### MM490: Limited Service, Mobile terminated Connection

Description:	MM is in limited service state. RR will be paged. It is checked whether MM starts connection establishment.

Preamble:	MM479

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CC message)	  |
	 |		  *<==============================*
(3)	 |	MMCC_ESTABLISH_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

- MMCC_ESTABLISH_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	CC_MESSAGE

History:	09.06.99	LE	Initial

## MM Idle Mode Behaviour (No IMSI Service)

### MM491: No IMSI State

Description:	MM is in state idle no imsi and initiates cell selection by issuing a RR-ACTIVATE request primitive. RR indicates only limited service. The periodic location updating timer shall not be started.

Preamble:	MM022

MMI/CM		 MM		 RR/DL
	 |		  |		  |
COMMAND (MM CONFIG T3212_CNT=6)
(1)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO_PER
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_NOSIM

History:	04.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM492: No Imsi, not perform Periodic LUP

Description:	The testcase waits 70 seconds. If the periodic LUP timer is started in the preamble, the testcase will fail, else MM has the expected behaviour. T3212 is decreased to 60 seconds by a dynamic config.

Preamble:	MM491

MMI/CM		 MM		 RR/DL
	 |		  |		  |
TIMEOUT (70000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	09.06.99	LE	Initial

### MM493: No IMSI, IMSI Detach, Power OFF

Description:	MM is in IDLE No IMSI service state. It is switched off. MM shall not perform IMSI Detach. It shall only deactivate the lower layer.

Preamble:	MM491

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	09.06.99	LE	Initial

### MM494: No IMSI, Call attempts, Emergency Calls

Description:	MM is in idle idle no IMSI service state. It shall reject all requests from CM entities except emergency calls. First the rejects are tested, then an emergency call. Then the whole sequence is tested again to check coming back in limited service state.

Preamble:	MM491

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |	MMSS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |	MMSS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(5)	 |	MMSMS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |	MMSMS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(7)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(9)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(11)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
(12)	 |	MMCC_RELEASE_REQ	  | 		  |
	 *==============================>*		  |
TIMEOUT (5000)
(13)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(16)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(17)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(18)	 |	MMSS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(19)	 |	MMSS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(20)	 |	MMSMS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(21)	 |	MMSMS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(22)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(23)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(24)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(25)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(26)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
(27)	 |	MMCC_RELEASE_REQ	  | 		  |
	 *==============================>*		  |
TIMEOUT (5000)
(28)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(29)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(30)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_RELEASE_IND
		ti	TI_4
		relcs	RELCS_NO_REGISTRATION

- MMSS_ESTABLISH_REQ
		ti	TI_3

- MMSS_RELEASE_IND
		ti	TI_3
		relcs	RELCS_NO_REGISTRATION

- MMSMS_ESTABLISH_REQ
		ti	TI_5

- MMSMS_RELEASE_IND
		ti	TI_5
		relcs	RELCS_NO_REGISTRATION

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMEI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

- MMCC_RELEASE_REQ
		ti	TI_3

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_RELEASE_IND
		ti	TI_4
		relcs	RELCS_NO_REGISTRATION

- MMSS_ESTABLISH_REQ
		ti	TI_3

- MMSS_RELEASE_IND
		ti	TI_3
		relcs	RELCS_NO_REGISTRATION

- MMSMS_ESTABLISH_REQ
		ti	TI_5

- MMSMS_RELEASE_IND
		ti	TI_5
		relcs	RELCS_NO_REGISTRATION

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMEI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

- MMCC_RELEASE_REQ
		ti	TI_3

- RR_ABORT_REQ
		abcs	ABCS_NORM

- RR_RELEASE_IND
		relcs	RELCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	09.06.99	LE	Initial
	26.01.01	HM	Revised

### MM495: No IMSI, new cell, new LA, but only limited service

Description:	MM is in service state Idle no IMSI. A.) A new cell is entered. B.) A new LA is entered. RR indicates only limited service. No reaction of MM is exspected.

Preamble:	MM491

Variants:	<A>....<B>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |	MMR_NREG_IND	  |		  |
	 *<==============================*		  |
TIMEOUT (10000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO
<A>		cid	CELL_ID_1123
<B>		cid	CELL_ID_1122
		plmn	PLMN_123_44
<A>		lac	LAC_0002
<B>		lac	LAC_0001
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_NOSIM

History:	09.06.99	LE	Initial
	02.03.00	HM	Revised (search_running, variants)

## MM Idle Mode Behaviour (No Cell available)

### MM497: No Cell available indication from Normal Service

Description:	MM is in state Idle Normal Service. RR indicates that no cell is available. MM enters the no cell available state.

Preamble:	MM024

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM_NO_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	NOT_USED
		plmn	NOT_USED
		rxlevel	NOT_USED
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_CELL_SELECTION_FAILED
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	09.06.99	LE	Initial
	02.03.00	HM	Revised (search_running)

### MM498: No Cell available, same cell

Description:	MM is in service state No Cell available. RR indicates the availability of full service. It is the same cell as before. It is assumed that no location updating is started.

Preamble:	MM497

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	03.05.99	LE	Initial

### MM499: No Cell available, same location area

Description:	MM is in service state No Cell available. RR indicates the availability of full service. It is a different cell but the same location area. It is assumed that no location updating is started.

Preamble:	MM497

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1123
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	03.05.99	LE	Initial

### MM500: No IMSI, IMSI Detach, Power OFF

Description:	MM is in IDLE No cell available service state. It is switched off. MM shall not perform IMSI Detach. It shall only deactivate the lower layer.

Preamble:	MM497

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	09.06.99	LE	Initial

### MM501: No Cell available, no IMSI Detach, SIM Remove by SIM

Description:	MM is in no cell available state. The SIM manager has detected a SIM Remove. MM shall not process IMSI Detach. MM remains in the same state.

Preamble:	MM497

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_REMOVE_IND	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_REMOVE_IND
		error	NOT_USED

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_IND
		nreg_cs	NREG_NO_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_REMOVED

History:	05.05.99	LE	Initial
	02.03.00	HM	Revised (search_running)
	07.01.01	HM	Adaption after GPRS integration

### MM502: No Cell available, IMSI Detach, SIM Remove by MMI

Description:	MM is in no cell available state. The MMI requests limited service. An IMSI Detach shall not be processed. The MS remains in the same state.

Preamble:	MM497

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_SIM_REM

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- MMR_NREG_CNF
		cs	CS_SIM_REM

History:	05.05.99	LE	Initial
	31.08.00	HM	Revised

### MM503: No cell available, Call attempts by upper layer

Description:	MM is in idle no cell available state. It shall reject all requests from CM entities including emergency calls.

Preamble:	MM497

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(3)	 |	MMSS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |	MMSS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(5)	 |	MMSMS_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |	MMSMS_RELEASE_IND	  | 		  |
	 *<==============================*		  |
(7)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(8)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_4
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_RELEASE_IND
		ti	TI_4
		relcs	RELCS_NO_REGISTRATION

- MMSS_ESTABLISH_REQ
		ti	TI_3

- MMSS_RELEASE_IND
		ti	TI_3
		relcs	RELCS_NO_REGISTRATION

- MMSMS_ESTABLISH_REQ
		ti	TI_5

- MMSMS_RELEASE_IND
		ti	TI_5
		relcs	RELCS_NO_REGISTRATION

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- MMCC_RELEASE_IND
		ti	TI_3
		relcs	RELCS_NO_REGISTRATION

History:	09.06.99	LE	Initial

### MM504: No cell available, new cell, new LA, but only limited service

Description:	MM is in service state Idle no cell available. A.) A new cell is entered. B.) A new LA is entered. RR indicates only limited service. No reaction of MM is exspected.

Preamble:	MM497

Variants:	<A>....<B>

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |	MMR_NREG_IND	  |		  |
	 *<==============================*		  |
TIMEOUT (10000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM_LIM_SERV
		mm_info	MM_INFO
<A>		cid	CELL_ID_1123
<B>		cid	CELL_ID_1122
		plmn	PLMN_123_44
<A>		lac	LAC_0002
<B>		lac	LAC_0001
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

History:	09.06.99	LE	Initial
	02.03.00	HM	Revised (search_running, variants)

### MM506: No Cell available, new cell, new location area, full service

Description:	MM is in service state No cell available. A new cell is entered in a new location area. A normal location updating is started.

Preamble:	MM497

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	09.06.99	LE	Initial

## Additional registration testcases

### MM520: MM needs IMSI ATTACH after switch on. Cell temporary barred

Description:	The mobile is switched on. A SIM is inserted. The mobile station is updated on the cell, but an IMSI ATTACH is needed. The cell is barred. After a short period of time, an access class change is indicated. A second attempt is expected. The difference between this and MM520 is that here no registration attempt is tried before the SIM is inserted.
<A> Previous registration to limited service without SIM
<B> No previous registration to limited service

Variants:	<A>....<B>

Preamble:	<A>	MM022
		<B>	MM001

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(9)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_ACCESS_BARRED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_ACC_CLS_CHA
		chm	CHM_NOT_PRESENT

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	19.02.01	HM	Initial

### MM521: MM needs IMSI ATTACH after switch on. Cell temporary barred

Description:	The mobile is switched on. A SIM is inserted. The mobile station is updated on the cell, but an IMSI ATTACH is needed. The cell is barred. A call attempt is made and rejected due to the fact that the cell is barred. After a short period of time, an access class change is indicated. A second attempt is expected.
<A> Previous registration to limited service without SIM
<B> No previous registration to limited service

Variants:	<A>....<B>

Preamble:	<A>	MM022
		<B>	MM001

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(9)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(11)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(13)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(14)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(15)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_ACCESS_BARRED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_ESTABLISH_REQ
		ti	TI_2
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_ACCESS_BARRED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_ACCESS_BARRED

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_ACC_CLS_CHA
		chm	CHM_NOT_PRESENT

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	19.02.01	HM	Initial

### MM522: MM doesn't find LPLMN, needs NORMAL UPDATE, cell barred

Description:	The mobile is switched on. A SIM is inserted. The mobile station doesn't find the requested PLMN, but another PLMN is available. A location updating attempt is performed, but the cell is barred. After some time the change of the access class is indicated by RR; the location updating attempt is started immediately.

Preamble:	MM001

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(7)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
TIMEOUT (10000)
(12)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(13)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_LPLMN
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_REQ
		plmn	PLMN_123_31
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_31
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_ACCESS_BARRED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_ACC_CLS_CHA
		chm	CHM_NOT_PRESENT

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	19.02.01	HM	Initial

### MM523: MM doesn't find LPLMN, needs NORMAL UPDATE, cell barred

Description:	The mobile is switched on. A SIM is inserted. The mobile station doesn't find the requested PLMN, but another PLMN is available. A location updating attempt is performed, but the cell is barred. A call attempt is made and rejected due to the fact that the cell is barred. After some time the change of the access class is indicated by RR; the location updating attempt is started immediately.

Preamble:	MM001

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(7)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(10)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(12)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(13)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(14)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(16)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
TIMEOUT (10000)
(17)	 |		  |	RR_SYNC_IND	  |
	 |		  *<==============================*
(18)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_LPLMN
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_REQ
		plmn	PLMN_123_31
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_31
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_ACCESS_BARRED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_ESTABLISH_REQ
		ti	TI_2
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_SPCH_CAL_BY_CC
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_ACCESS_BARRED
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_ACCESS_BARRED

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_ACC_CLS_CHA
		chm	CHM_NOT_PRESENT

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	19.02.01	HM	Initial

### MM524: MM needs IMSI ATTACH after switch on. Cell temporary barred

Description:	The mobile is switched on. A SIM is inserted. The mobile station is updated on the cell, but an IMSI ATTACH is needed. The cell is barred. After a short period of time, the coverage is lost. After some time, coverage is gained again on the first cell. An IMSI ATTACH is expected.
<A> No service, access barred
<B> Limited service, access barred
<C> No service, random access failure
<D> Limited service, random access failure
[The newly introduced testcase currently doesn't run, the reason for this is a minor implementation problem in the protocol stack.]

Variants:	<A>....<D>

Preamble:	MM001

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(11)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(12)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(13)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_2
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
<A>		relcs	RELCS_ACCESS_BARRED
<B>		relcs	RELCS_ACCESS_BARRED
<C>		relcs	RELCS_RND_ACC_FAIL
<D>		relcs	RELCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_IND
<A>		op	OP_MODE_SIM_NO_SERV
<B>		op	OP_MODE_SIM_LIM_SERV
<C>		op	OP_MODE_SIM_NO_SERV
<D>		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
<A>		plmn_avail	NO_PLMN_FOUND
<B>		plmn_avail	ONE_PLMN_FOUND
<C>		plmn_avail	NO_PLMN_FOUND
<D>		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_FORB
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
<A>		nreg_cs	NREG_CELL_SELECTION_FAILED
<B>		nreg_cs	NREG_LIMITED_SERVICE
<C>		nreg_cs	NREG_CELL_SELECTION_FAILED
<D>		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info 	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	19.02.01	HM	Initial

### MM525: MM needs IMSI ATTACH after switch on in tunnel

Description:	The mobile is switched on. A SIM was inserted which has an update state in the area. At switch on, there is no service as the switch on was in a tunnel. After some time, the mobile phone leaves the tunnel and MM receives an RR_ACTIVATE_IND which indicates a cell selection in the updated location area, but the IMSI ATTACH is of course still needed.
<A> No service after switch on
<B> Only limited service after switch on

Variants:	<A>....<B>

Preamble:	MM001

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(7)	 |		  |	RR_ACTIVATE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_2
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
<A>		op	OP_MODE_SIM_NO_SERV
<B>		op	OP_MODE_SIM_LIM_SERV
		abcs	ABCS_CEL_SEL_FAIL
<A>		plmn_avail	NO_PLMN_FOUND
<B>		plmn_avail	ONE_PLMN_FOUND
		plmn	PLMN_LIST_FORB
		rxlevel	RXLEVEL_20
		power	RF_CLASS_2

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- MMR_NREG_IND
<A>		nreg_cs	NREG_CELL_SELECTION_FAILED
<B>		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_RC_NONE

- RR_ACTIVATE_IND
		op	OP_MODE_SIM
		mm_info 	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	22.02.01	HM	Initial

### MM526: MM IDLE updated, T3212 running, manual network search I

Description:	The mobile is switched on. A SIM was inserted. An IMSI ATTACH update is performed. After a while the user starts a manual network search. It is checked that this does not restart T3212.

Preamble:	MM022

MMI/CM		 MM		 RR/DL
	 |		  |		  |
COMMAND (MM CONFIG T3212_CNT=6)
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(6)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(9)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(11)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
(12)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(13)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
TIMEOUT (55000)
(14)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_PER
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_2
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_NET_REQ
		param	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_M
		cksn	CKSN_NO_KEY
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	EMPTY_IMSI
		tmsi	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_SIM_MAN_MMI_SRCH_FS
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	TWO_PLMN_FOUND
		plmn	PLMN_LIST_2_PLMN
		rxlevel	RXLEVEL_20_18
		power	RF_CLASS_2

- MMR_PLMN_IND
		res	RES_OK_USR_MST_SEL_PLMN
		plmn	PLMN_LIST_2_PLMN_A
		forb_ind	FORB_PLMN_ID
		rxlevel	RXLEVEL_20_18_A

- MMR_PLMN_MODE_REQ
		mode	MODE_AUTO

- RR_SYNC_REQ
		op	OP_SIM_AUTO_MMI_SRCH_NS
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	28.02.01	HM	Initial

### MM527: MM IDLE updated, T3212 running, manual network search II

Description:	The mobile is switched on. A SIM was inserted. An IMSI ATTACH update is performed. After a while the user starts a manual network search. It is checked that this does not restart T3212.

Preamble:	MM022

MMI/CM		 MM		 RR/DL
	 |		  |		  |
COMMAND (MM CONFIG T3212_CNT=6)
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
TIMEOUT (35000)
(6)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(9)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(11)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
(12)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(13)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
TIMEOUT (20000)
(14)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_PER
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_2
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_NET_REQ
		param	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_M
		cksn	CKSN_NO_KEY
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	EMPTY_IMSI
		tmsi	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_SIM_MAN_MMI_SRCH_FS
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	TWO_PLMN_FOUND
		plmn	PLMN_LIST_2_PLMN
		rxlevel	RXLEVEL_20_18
		power	RF_CLASS_2

- MMR_PLMN_IND
		res	RES_OK_USR_MST_SEL_PLMN
		plmn	PLMN_LIST_2_PLMN_A
		forb_ind	FORB_PLMN_ID
		rxlevel	RXLEVEL_20_18_A

- MMR_PLMN_MODE_REQ
		mode	MODE_AUTO

- RR_SYNC_REQ
		op	OP_SIM_AUTO_MMI_SRCH_NS
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_PERIODIC
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

History:	28.02.01	HM	Initial

## Network search

### MM530: Aborted manual NW search in MM_IDLE_ATTEMPT_TO_UPDATE

Description:	The mobile is in MM_IDLE_ATTEMPT_TO_UPDATE state, manual network mode selected. The user starts a manual PLMN search. While this is running, the user aborts the operation. It is not expected that MM really stops searching PLMNs, but if a call attempt will be made in the next testcase, MM has to remember that it came from MM_IDLE_ATTEMPT_TO_UPDATE state.

Preamble:	MM410

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_2
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_NET_REQ
		param	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_M
		cksn	CKSN_NO_KEY
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	EMPTY_IMSI
		tmsi	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_SIM_MAN_MMI_SRCH_NS
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

History:	28.02.01	HM	Initial

### MM531: MM_IDLE_ATTEMPT_TO_UPDATE, MO call

Description:	The mobile is in MM_IDLE_ATTEMPT_TO_UPDATE or MM_PLMN_SEARCH_NORMAL_SERVICE state. The user makes a call attempt. As the mobile station was in MM_IDLE_ATTEMPT_TO_UPDATE state if it was not searching and the searching is now really stopped by the call attempt, it is expected that before the actual call attempt is made first a location updating procedure is performed.

Variants:	<A>....<B>

Preamble:	<A>	MM410
		<B>	MM530

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(5)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(9)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(10)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (CM SERVICE REQ)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL_FOL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_FEFF
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	IE_FOLLOW_PROCEED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_2147
		synccs	SYNCCS_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_MOC
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_NEW_TMSI
		}

History:	28.02.01	HM	Initial

### MM532: Manual NW search in MM_IDLE_ATTEMPT_TO_UPDATE

Description:	The mobile is in MM_IDLE_ATTEMPT_TO_UPDATE state, manual network mode selected. The user starts a manual PLMN search. After the search is finished, the user tries an emergency call.

Preamble:	MM410

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_NET_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ABORT_IND	  |
	 |		  *<==============================*
(6)	 |	MMR_PLMN_IND	  | 		  |
	 *<==============================*		  |
(7)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(8)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(9)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (CM SERVICE REQUEST)	  |
	 |		  *==============================>*
(11)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (CM SERVICE ACCEPT)	  |
	 |		  *<==============================*
(13)	 |	MMCC_ESTABLISH_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_2
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_NET_REQ
		param	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_SIM_NO_SERV_M
		cksn	CKSN_NO_KEY
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	EMPTY_IMSI
		tmsi	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ABORT_IND
		op	OP_SIM_MAN_MMI_SRCH_FS
		abcs	ABCS_CEL_SEL_FAIL
		plmn_avail	TWO_PLMN_FOUND
		plmn	PLMN_LIST_2_PLMN
		rxlevel	RXLEVEL_20_18
		power	RF_CLASS_2

- MMR_PLMN_IND
		res	RES_OK_USR_MST_SEL_PLMN
		plmn	PLMN_LIST_2_PLMN_A
		forb_ind	FORB_PLMN_ID
		rxlevel	RXLEVEL_20_18_A

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_SIM_MAN_MMI_SRCH_NS
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMCC_ESTABLISH_REQ
		ti	TI_3
		prio	PRIO_EMERG_CALL
		estcs	ESTCS_MOB_ORIG_DATA

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_CM_SERV_REQ
		ti	TI_0
		cm_serv_type	ST_EMERGENCY
		ciph_key_num	CIPH_KEY_NUM_RES
		mob_class_2	MOB_CLASS_2
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_CM_SERV_ACCEPT
		ti	TI_0
		}

- MMCC_ESTABLISH_CNF
		ti	TI_3

History:	28.02.01	HM	Initial

## Behaviour due to SAT activity

### MM601: SIM insert indication by SAT .- IMSI change, detach/attach, automatic

Description:	MM receives a SIM_MM_INSERT_IND with IMSI change. This shall cause a MM Restart procedure. Automatic regstration mode version.

Preamble:	MM403

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	(LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(7)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(9)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(10)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(17)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(18)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(19)	 |		  |	RR_DATA_IND	  |
	 |		  |	(LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(20)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(21)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(23)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(24)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(25)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(26)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_2147
		synccs	SYNCCS_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	SIM_IMSI_001010123456789
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_TMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	PRI_IMSI_001010123456789
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MSG_IMSI_001010123456789
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_2147
		synccs	SYNCCS_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	10.04.00	HM	Initial
	14.02.01	HM	Revised

### MM602: SIM insert indication by SAT - IMSI change, detach/attach, manual

Description:	MM receives a SIM_MM_INSERT_IND with IMSI change. This shall cause a MM Restart procedure. In this testcase, no detach is required by cell. Manual mode. The mobile is updated by the SIM in another location area that the one the mobile is manual required to register to. A normal location updating on the manually selected PLMN is expected.

Preamble:	MM022

MMI/CM		 MM		 RR/DL
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(5)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(8)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(10)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(11)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(14)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(15)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(16)	 |		  |	RR_DATA_IND	  |
	 |		  |	(Identity request)	  |
	 |		  *<==============================*
(17)	 |		  |	RR_DATA_REQ	  |
	 |		  |	(Identity response)	  |
	 |		  *==============================>*
(18)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(20)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(21)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(23)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(24)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(25)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(26)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(27)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(28)	 |		  |	RR_ESTABLISH_REQ	  |
	 |		  |	  (LOCATION UPDATING REQ)	  |
	 |		  *==============================>*
(29)	 |		  |	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(30)	 |		  |	RR_DATA_IND	  |
	 |		  |	(LOCTION UPDATING ACC)	  |
	 |		  *<==============================*
(31)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (TMSI REALLOC COMPLETE)	  |
	 |		  *==============================>*
(32)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(33)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(34)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
(35)	 |	SIM_MM_UPDATE_REQ	  | 		  |
	 *<==============================*		  |
(36)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(37)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_NO_SERV_M1
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_ATTACH
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_IMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_2147
		synccs	synccs_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMSI
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	SIM_IMSI_001010123456789
		loc_info	LOC_INFO_UPDATED_123_44_0002
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- RR_ESTABLISH_REQ
		estcs	ESTCS_MOB_ORIG_CAL_BY_SS_SMS
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IMSI_DETACH_IND
		ti	TI_0
		mob_class_1	MOB_CLASS_1
		mob_id	MOB_IDENT_TMSI
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	PRI_IMSI_001010123456789
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM_M
		mm_info	MM_INFO_ATT
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_LOC_UPD_REQ
		ti	TI_0
		loc_upd_type	LOC_UPD_TYPE_NORMAL
		ciph_key_num	CIPH_KEY_NUM_RES
		loc_area_ident	LOC_AREA_ID_123_44_0002
		mob_class_1	MOB_CLASS_1
		mob_id	MSG_IMSI_001010123456789
		}

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_LOC_UPD_ACCEPT
		ti	TI_0
		loc_area_ident	LOC_AREA_ID_123_33_2147
		mob_id	MOB_IDENT_NEW_TMSI
		follow_proceed	NOT_USED
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{	
		component	MM
		direction	UPLINK
		pd	U_TMSI_REALLOC_COMP
		ti	TI_0
		}

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	MOB_ID_NEW_TMSI
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_USED
		accc	NOT_USED
		thplmn	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	PLMN_123_33
		lac	LAC_2147
		synccs	SYNCCS_LAI_ALLOW
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_CNF
		plmn	PLMN_123_33

- SIM_MM_UPDATE_REQ
		loc_info	LOC_INFO_UPDATED_5
		bcch_inf	BCCH_INF_1
		forb_plmn	NOT_USED
		cksn	CKSN_RES
		kc	KC_DELETED_SIM
		cell_identity	CELL_ID_1122

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	30.03.00	HM	Initial
	14.02.01	HM	Revised

### MM603: SIM insert indication by SAT - IMSI change, automatic mode

Description:	MM receives a SIM_MM_INSERT_IND with IMSI change. This shall cause a MM Restart procedure. In this testcase, no detach is required by cell and the mobile is in automatic mode. As the same cell is selected after the IMSI ATTACH, there is no need to indicate the full service condition to the MMI as there was never something else than full service indicated.

Preamble:	MM024

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	(Identity request)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  |	(Identity response)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(12)	 |		  |	RR_DATA_IND	  |
	 |		  |	(Identity request)	  |
	 |		  *<==============================*
(13)	 |		  |	RR_DATA_REQ	  |
	 |		  |	(Identity response)	  |
	 |		  *==============================>*
(14)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMSI
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	SIM_IMSI_001010123456789
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	PRI_IMSI_001010123456789
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM_M
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMSI
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MSG_IMSI_001010123456789
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	14.02.01	HM	Initial

### MM604: SIM insert indication by SAT - IMSI change, manual mode

Description:	MM receives a SIM_MM_INSERT_IND with IMSI change. This shall cause a MM Restart procedure. In this testcase, no detach/attach is required by cell. As the same cell is selected after the IMSI ATTACH, there is no need to indicate the full service condition to the MMI as there was never something else than full service indicated.

Preamble:	MM025

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(2)	 |		  |	RR_DATA_IND	  |
	 |		  |	(Identity request)	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_REQ	  |
	 |		  |	(Identity response)	  |
	 |		  *==============================>*
(4)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(6)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(7)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	RR_ABORT_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	RR_ESTABLISH_IND	  |
	 |		  *<==============================*
(12)	 |		  |	RR_DATA_IND	  |
	 |		  |	(Identity request)	  |
	 |		  *<==============================*
(13)	 |		  |	RR_DATA_REQ	  |
	 |		  |	(Identity response)	  |
	 |		  *==============================>*
(14)	 |		  |	RR_RELEASE_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMSI
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MOB_IDENT_IMSI
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	SIM_IMSI_001010123456789
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_01

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_ABORT_REQ
		abcs	ABCS_SIM_REM

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	PRI_IMSI_001010123456789
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM_M
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- RR_ESTABLISH_IND
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMSI
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MSG_IMSI_001010123456789
		}

- RR_RELEASE_IND
		relcs	RELCS_NORM
		sapi	SAPI_0
		gprs_resumption	GPRS_RESUMPTION_NOT_ACK

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

History:	30.03.00	HM	Initial
	14.02.01	HM	Revised

### MM605: SIM insert indication by SAT - no IMSI change, RR notification

Description:	MM receives a SIM_MM_INSERT_IND without IMSI change. This shall not cause a MM Restart procedure, but there were fields changed which cause RR notification.

Preamble:	MM025

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_MM_INSERT_IND	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_MM_INSERT_IND
		op_mode	NORMAL_SIM_INS
		imsi_field	IMSI_FIELD_1
		loc_info	LOC_INFO_UPDATED_1
		acc_ctrl	ACC_CTRL_1
		bcch_inf	BCCH_INF_1
		kc_n	KC_EMPTY
		pref_plmn	PREF_PLMN_NONE
		forb_plmn	FORB_PLMN_NONE
		phase	PHASE_2_SIM
		hplmn	THPLMN_FF

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_ACCC_THPLMN
		accc	ACC_2143
		thplmn	THPLMN_FF

History:	04.04.00	HM	Initial

### MM606: SIM inserted - File update by SAT

Description:	MM receives a SIM_FILE_UPDATE_IND for some not interesting crap. MM does nothing special and confirms the request.

Preamble:	MM025

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_FILE_UPDATE_IND	  |		  |
	 *==============================>*		  |
(2)	 |	SIM_FILE_UPDATE_RES	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	FILE_CHG_1
		file_id	EF_MSISDN

- SIM_FILE_UPDATE_RES
		source	SRC_MM
		fu_rsc	SIM_FU_SUCCESS

History:	14.02.01	HM	Initial

### MM607: SIM inserted - File update by SAT

Description:	MM receives a SIM_FILE_UPDATE_IND for the preferred PLMN list

Preamble:	MM025

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_FILE_UPDATE_IND	  |		  |
	 *==============================>*		  |
(2)	 |	SIM_READ_REQ	  |		  |
	 *<==============================*		  |
(3)	 |	SIM_READ_CNF	  |		  |
	 *==============================>*		  |
(4)	 |	SIM_FILE_UPDATE_RES	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	FILE_CHG_1
		file_id	EF_PLMN_SEL

- SIM_READ_REQ
		source	SRC_MM
		offset	OFFSET_0
		datafield	SIM_PLMNSEL
		length	NOT_PRESENT_8BIT
		max_length	MAX_LEN_PREF_PLMN

- SIM_READ_CNF
		datafield	SIM_PLMNSEL
		error	SIM_NO_ERROR
		length	PLMN_SEL_LENGTH
		trans_data	PLMN_SEL_NLPTT_PROX

- SIM_FILE_UPDATE_RES
		source	SRC_MM
		fu_rsc	SIM_FU_SUCCESS

History:	30.03.00	HM	Initial
	16.10.00	HM	Modified SIM_FILE_UPDATE_RES

### MM608: File update by SAT, RR notification

Description:	MM receives a SIM_FILE_UPDATE_IND for the HPLMN search period. RR has to be notified about the change

Preamble:	MM025

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	SIM_FILE_UPDATE_IND	  |		  |
	 *==============================>*		  |
(2)	 |	SIM_READ_REQ	  |		  |
	 *<==============================*		  |
(3)	 |	SIM_READ_CNF	  |		  |
	 *==============================>*		  |
(4)	 |	SIM_READ_REQ	  |		  |
	 *<==============================*		  |
(5)	 |	SIM_READ_CNF	  |		  |
	 *==============================>*		  |
(6)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |	SIM_FILE_UPDATE_RES	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- SIM_FILE_UPDATE_IND
		val_nr	FILE_CHG_2
		file_id	EF_PLMN_SEL_THPLMN

- SIM_READ_REQ
		source	SRC_MM
		offset	OFFSET_0
		datafield	SIM_HPLMN
		length	NOT_PRESENT_8BIT
		max_length	LENGTH_THPLMN

- SIM_READ_CNF
		datafield	SIM_HPLMN
		error	SIM_NO_ERROR
		length	LENGTH_THPLMN
		trans_data	SIM_THPLMN_FF

- SIM_READ_REQ
		source	SRC_MM
		offset	OFFSET_0
		datafield	SIM_PLMNSEL
		length	NOT_PRESENT_8BIT
		max_length	MAX_LEN_PREF_PLMN

- SIM_READ_CNF
		datafield	SIM_PLMNSEL
		error	SIM_NO_ERROR
		length	PLMN_SEL_LENGTH
		trans_data	PLMN_SEL_NLPTT_PROX

- RR_SYNC_REQ
		op	NOT_USED
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_ACCC_THPLMN
		accc	ACC_2143
		thplmn	THPLMN_FF

- SIM_FILE_UPDATE_RES
		source	SRC_MM
		fu_rsc	SIM_FU_SUCCESS

History:	30.03.00	HM	Initial
	16.10.00	HM	Modified SIM_FILE_UPDATE_RES

## Switching service modes back and forth

### MM620: SIM inserted - Deregistered -> Deregistered

Description:	MM receives a SIM-INSERT indication primitive. Deregistering is required by the MMI.

Preamble:	MM315

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	25.04.00	HM	Initial

### MM621: SIM inserted - Deregistered -> Limited

Description:	A limited service search is required by the MMI. To prove MM is in a state not allowing normal calls, an establish attempt is tried. This will be refused by MM.

Preamble:	MM620

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 |	(SERVICE_MODE_LIMITED)	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(5)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_LIMITED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_MMIREQ

- MMCC_ESTABLISH_REQ
		ti	TI_2
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_NO_REGISTRATION

History:	25.04.00	HM	Initial

### MM622: SIM inserted - Deregistered -> Full (automatic mode)

Description:	A full service search in automatic mode is required by the MMI.

Preamble:	MM620

MMI/CM		 MM		 RR/DL
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_AUTO

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_2
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	20.04.00	HM	Initial

### MM623: SIM inserted - Deregistered -> Full (manual mode)

Description:	MM receives a request to manual register in deregitered mode.

Preamble:	MM620

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM_M
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	25.04.00	HM	Initial

### MM624: SIM inserted - Limited -> Deregistered

Description:	MM receives a request to deregister in limited service state.

Preamble:	MM621

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	25.04.00	HM	Initial

### MM625: SIM inserted - Limited -> Limited

Description:	MM receives a request to register to limited service in limited service state. This is done by MMR_REG_REQ with parameter SERVICE_MODE_LIMITED here.

Preamble:	MM621

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(3)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(5)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_LIMITED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_MMIREQ

History:	27.04.00	HM	Initial

### MM626: SIM inserted - Limited -> Full (automatic mode)

Description:	MM receives a request to register to full service in automatic mode. Currently MM is in limited service state.

Preamble:	MM621

MMI/CM		 MM		 RR/DL
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(6)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_AUTO

- RR_SYNC_REQ
		op	OP_MODE_SIM_NO_SERV1
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_2
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	25.04.00	HM	Initial

### MM627: SIM inserted - Limited -> Full (manual mode)

Description:	MM receives a request to manual register in limited service state.

Preamble:	MM621

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(6)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_NO_SERV_M1
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM_M
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	25.04.00	HM	Initial

### MM628: SIM inserted - Full (automatic mode) -> Deregistered

Description:	MM receives a request to deregister in full service state.

Preamble:	MM622

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	25.04.00	HM	Initial

### MM629: SIM inserted - Full (automatic mode) -> Limited

Description:	MM receives a request to register to limited service in full service state. This is done by MMR_REG_REQ with parameter SERVICE_MODE_LIMITED here.

Preamble:	MM622

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 |	(SERVICE_MODE_LIMITED)	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(5)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_LIMITED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_MMIREQ

- MMCC_ESTABLISH_REQ
		ti	TI_2
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_NO_REGISTRATION

History:	27.04.00	HM	Initial

### MM630: SIM inserted - Full (automatic mode) -> Full (automatic mode)

Description:	MM receives a request to register to full service in automatic mode. Currently MM already is in full service state (automatic mode).

Preamble:	MM622

MMI/CM		 MM		 RR/DL
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(6)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_AUTO

- RR_SYNC_REQ
		op	OP_MODE_SIM_NO_SERV
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_2
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	27.04.00	HM	Initial
	02.05.01	HM	Revised

### MM631: SIM inserted - Full (automatic mode) -> Full (manual mode)

Description:	MM receives a request to register to full service in manual mode. Currently MM is in full service state (automatic mode).

Preamble:	MM622

(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(6)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM_M
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	27.04.00	HM	Initial

### MM632: SIM inserted - Full (manual mode) -> Deregistered

Description:	MM receives a request to deregister in full service state.

Preamble:	MM623

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_NREG_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MDL_RELEASE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_DEACTIVATE_REQ	  |
	 |		  *==============================>*
(4)	 |	MMR_NREG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_NREG_REQ
		cs	CS_POW_OFF

- MDL_RELEASE_REQ
		ch_type	NOT_PRESENT_8BIT
		sapi	SAPI_0

- RR_DEACTIVATE_REQ
		param	NOT_USED

- MMR_NREG_CNF
		cs	CS_POW_OFF

History:	27.04.00	HM	Initial

### MM633: SIM inserted - Full (manual mode) -> Limited

Description:	MM receives a request to register to limited service in full service state. This is done by MMR_REG_REQ with parameter SERVICE_MODE_LIMITED here.

Preamble:	MM623

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	MMR_REG_REQ	  | 		  |
	 |	(SERVICE_MODE_LIMITED)	  |		  |
	 *==============================>*		  |
(2)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(4)	 |	MMR_NREG_IND	  | 		  |
	 *<==============================*		  |
(5)	 |	MMCC_ESTABLISH_REQ	  | 		  |
	 *==============================>*		  |
(6)	 |	MMCC_RELEASE_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_REG_REQ
		service_mode	SERVICE_MODE_LIMITED

- RR_ACTIVATE_REQ
		plmn	PLMN_NO_ID
		op	OP_MODE_NO_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KC_DELETED
		accc	ACC_CLASS_0000
		imsi	MOB_ID_NO_ID
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_FF
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_NO_SIM_LIM_SERV
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_44
		lac	LAC_0002
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_NREG_IND
		nreg_cs	NREG_LIMITED_SERVICE
		search_running	SEARCH_NOT_RUNNING
		new_forb_plmn	PLMN_NO_ID
		limited_cause	MMR_SIM_INVAL_MMIREQ

- MMCC_ESTABLISH_REQ
		ti	TI_2
		prio	PRIO_NORM_CALL
		estcs	ESTCS_MOB_ORIG_SPCH

- MMCC_RELEASE_IND
		ti	TI_2
		relcs	RELCS_NO_REGISTRATION

History:	27.04.00	HM	Initial

### MM634: SIM inserted - Full (manual mode) -> Full (automatic mode)

Description:	MM receives a request to register to full service in automatic mode. Currently MM already is in full service state (manual mode).

Preamble:	MM623

MMI/CM		 MM		 RR/DL
(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_REG_REQ	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(6)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_AUTO

- RR_SYNC_REQ
		op	OP_MODE_SIM_NO_SERV
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_REG_REQ
		service_mode	SERVICE_MODE_FULL

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_ECL
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM
		mm_info 	MM_INFO_2
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	27.04.00	HM	Initial
	02.05.01	HM	Revised

### MM635: SIM inserted - Full (manual mode) -> Full (manual mode)

Description:	MM receives a request to register to full service in manual mode. Currently MM is in full service state (manual mode).

Preamble:	MM623

(1)	 |	MMR_PLMN_MODE_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	RR_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |	MMR_PLMN_RES	  | 		  |
	 *==============================>*		  |
(4)	 |		  |	RR_ACTIVATE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	RR_ACTIVATE_CNF	  |
	 |		  *<==============================*
(6)	 |	MMR_REG_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MMR_PLMN_MODE_REQ
		mode	MODE_MAN

- RR_SYNC_REQ
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	NOT_USED
		kcv	NOT_USED
		tmsi	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	NOT_USED
		thplmn	NOT_USED

- MMR_PLMN_RES
		plmn	PLMN_123_33

- RR_ACTIVATE_REQ
		plmn	PLMN_123_33
		op	OP_MODE_SIM_NO_SERV_M2
		cksn	CKSN_RES
		kcv	KCV_EMPTY
		accc	ACC_2143
		imsi	MOB_ID_IMSI
		tmsi	MOB_ID_NO_ID
		thplmn	THPLMN_01
		bcch_info	BCCH_INFO_1
		cell_test	CELL_TEST_DISABLE
		gprs_indic	GPRS_NO

- RR_ACTIVATE_CNF
		op	OP_MODE_SIM_M
		mm_info	MM_INFO
		cid	CELL_ID_1122
		plmn	PLMN_123_33
		lac	LAC_2147
		power	RF_CLASS_2
		gprs_indic	GPRS_NO

- MMR_REG_CNF
		plmn	PLMN_123_33

History:	27.04.00	HM	Initial

## Engineering mode

### MM650: Identity Request during Location Updating

Description:	Connection is confirmed in the form of a RR-ESTABLISH confirmation primitive, whereupon MM changes to State 3 (Location Updating Initiated). In the course of Location Updating the network requests the identity of the MS. MM responds by issuing an Identity Response message as part of a RR-DATA indication primitive.

Preamble:	MM101

MMI/CM/SIM	 MM		 RR/DL
	 |		  |		  |
(1)	 |	EM_MM_EVENT_REQ	  | 		  |
	 *==============================>* 		  |
(2)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (IDENTITY REQUEST)	  |
	 |		  *<==============================*
(4)	 |		  |	RR_DATA_REQ	  |
	 |		  |	  (IDENTITY RESPONSE)	  |
	 |		  *==============================>*
(5)	 |	EM_DATA_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- EM_MM_EVENT_REQ
	bitmask_mm	Bitm_1

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_IDENT_REQ
		ti	TI_0
		ident	IDENT_TYPE_IMSI
		}

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	UPLINK
		pd	U_IDENT_RES
		ti	TI_0
		mob_id	MOB_IDENT_IMSI
		}

- EM_DATA_IND
	entity	EM_ENTITY

History:	09.07.97	HK	Initial

23-Oct-01	OT	EM adaptations

### MM651: Authentication Request

Description:	The network confirms the request from the mobile station for a RR Connection in the form of a RR-ESTABLISH confirmation primitive. MM changes to State 3 ((Location Updating initiated) and issues an AUTHENTICATION REQ message and a SIM-AUTHENTICATION request primitive, initiating the authentication procedure with SIM.

Preamble:	MM101

MMI/CM		 MM		 RR/DL
	 |		  |		  |
(1)	 |	EM_MM_EVENT_REQ	  | 		  |
	 *==============================>* 		  |
(2)	 |		  | 	RR_ESTABLISH_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	RR_DATA_IND	  |
	 |		  |	  (AUTHENTICATION REQ)	  |
	 |		  *<==============================*
(4)	 |	SIM_AUTHENTICATION_REQ	  |		  |
	 *<==============================*		  |
(5)	 |	EM_DATA_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- EM_MM_EVENT_REQ
	bitmask_mm	Bitm_2

- RR_ESTABLISH_CNF
		param	NOT_USED

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu
		{
		component	MM
		direction	DOWNLINK
		pd	D_AUTH_REQ
		ti	TI_0
		ciph_key_num	CIPH_KEY_NUM_01
		auth_rand	AUTH_RAND_1
		}

- SIM_AUTHENTICATION_REQ
		source	SRC_MM
		rand	RAND_1_P
		cksn	CKSN_01

- EM_DATA_IND
	entity	EM_ENTITY

History:	07.07.97	HK	Initial
	31.07.97	DL	Revised

23-Oct-01	OT	EM adaptations

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |