Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document – Confidential

GSM Protocol Stack

Multilayer Test Specification

RR

(Edit via File/Properties/Summary!)

| Document Number: | 6147.403.02.107 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.9 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1997-Jul-09 |
| Last changed: | 2003-Aug-05 by Manuel Palmowski |
| File Name: | mrr.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1997-Jul-09 | Stefan Lemke et al |  | 0.1 |  | 1 |
| 1999-Jan-25 | Stefan Lemke et al |  | 0.2 |  | 2 |
| 2000-Feb-03 | OSE |  | 0.3 |  | 3 |
| 2001-Mar-26 | VK |  | 0.4 |  | 4 |
| 2001-Nov-16 | VK |  | 0.5 |  | 5 |
| 2002-May-29 | MSB |  | 0.6 |  | 6 |
| 2002-Jun-19 | MSB |  | 0.7 |  | 7 |
| 2002-Nov-19 | MSB |  | 0.8 |  | 8 |
| 2003-Aug-05 | XGUTTEFE |  | 0.9 | Draft |  |
| 2003-Aug-05 | MPA |  | 1.0 | Draft | 9 |

Notes:

- Initial version

- Testcases updated

- New Template

- Introduce test case MRR410

- Add IE CLIR Suppression

- Classmarks adapted to new multiband code

- Fit Classmarks to new rfcap file

- Add Classmark interrogation tests for further frequency bands

- Added some redirections/duplicates

Table of Contents

1	Overview	6

2	Parameters	6

3	TEST CASES	49

3.1	Initialization of the IUT	49

3.1.1	MRR000: Initialization (One cell)	49

3.1.2	MRR001: Initialization (Two cells)	50

3.2	Immediate assignment	51

3.2.1	MRR100: Immediate assignment / SDCCH or TCH assignment (26.6.1.1)	51

3.2.2	MRR101: Immediate assignment / extended assignment (26.6.1.2)	52

3.2.3	MRR102: Immediate assignment / assignment rejection (26.6.1.3)	54

3.2.4	MRR103: Immediate assignment / ignore assignment (26.6.1.4, Step 1-2)	56

3.2.5	MRR104: Immediate assignment / ignore assignment (26.6.1.4, Step 3)	57

3.2.6	MRR105: Immediate assignment / ignore assignment (26.6.1.4, Step 4-8)	58

3.2.7	MRR106: Immediate assignment / ignore assignment (26.6.1.4, Step 9)	59

3.2.8	MRR107: Immediate assignment / ignore assignment (26.6.1.4, Step 10-11)	60

3.2.9	MRR108: Immediate assignment / ignore assignment (26.6.1.4)	61

3.2.10	MRR109: Immediate assignment after immediate assignment reject (26.6.1.5)	63

3.3	Paging	64

3.3.1	MRR130: Paging /normal /type 1 (26.6.2.1.1)	64

3.3.2	MRR131: Paging /normal /type 2 (26.6.2.1.2)	67

3.3.3	MRR132: Paging /normal /type 3 (26.6.2.1.3)	70

3.3.4	MRR133: Paging / extended (26.6.2.2)	73

3.3.5	MRR134: Paging / reorganization (26.6.2.3)	76

3.3.6	MRR135: Call Setup Procedure for Mobile Terminated Calls (10.1)	78

3.4	Measurements	79

3.4.1	MRR150: Measurement / no neighbours (26.6.3.1)	79

3.4.2	MRR151: Measurement / all neighbours present (26.6.3.2)	88

3.4.3	MRR152: Multiple Call Setup Procedure for Mobile Terminated Calls	97

3.4.4	MRR153: Measurement / barred cells and non-permitted NCCs (26.6.3.3)	98

3.4.5	MRR154: Measurement / barred cells and non-permitted NCCs (26.6.3.3, K=1+2)	107

3.4.6	MRR155: Measurement / DTX (26.6.3.4)	116

3.4.7	MRR156: Measurement / Frequency Formats (26.6.3.5)	126

3.5	Channel Assignment	135

3.5.1	MRR180: Dedicated assignment / successful case (26.6.4.1)	135

3.5.2	MRR181: Dedicated assignment / failure during active state (26.6.4.2.1)	138

3.5.3	MRR182: Dedicated assignment / failure / general case (26.6.4.2.2)	139

3.6	Handover	141

3.6.1	MRR200: Handover / Preamble (26.6.5.1)	141

3.6.2	MRR201: Handover / successful / active call / non-sync  / M=1 (26.6.5.1)	143

3.6.3	MRR202: Handover / successful / active call / non-sync  / M=2 (26.6.5.1)	145

3.6.4	MRR203: Handover / successful / active call / non-sync  / M=3 (26.6.5.1)	147

3.6.5	MRR210: Handover / Preamble (26.6.5.2)	148

3.6.6	MRR211: Handover / successful / call estab. / non-sync / M=1 (26.6.5.2)	150

3.6.7	MRR212: Handover / successful / call estab. / non-sync / M=3 (26.6.5.2)	152

3.6.8	MRR213: Handover / successful / call estab. / non-sync / M=7 (26.6.5.2)	154

3.6.9	MRR220: Handover / successful / active call / finely sync / M=1 (26.6.5.3)	156

3.6.10	MRR240: Handover / successful / active call / pre-sync / no TA IE (26.6.5.5.1)	158

3.6.11	MRR241: Handover / successful / active call / pre-sync / TA IE / ROT (26.6.5.5.2)	160

3.6.12	MRR242: Handover / successful / active call / pre-sync / TA IE / ROT (26.6.5.5.2)	161

3.6.13	MRR243: Handover / successful / active call / pre-sync / TA IE / ROT (26.6.5.5.2)	164

3.6.14	MRR250: Handover / successful / active call / pseudo-sync (26.6.5.6)	166

3.6.15	MRR251: Handover / failure / active call / pseudo-sync (not supported)	168

3.6.16	MRR270: Handover / layer 3 failure (26.6.5.8)	170

3.7	Frequency Redefinition	172

3.7.1	MRR280: Frequency redefinition  (26.6.6.1)	172

3.8	Channel Mode Modify	175

3.8.1	MRR290: Channel Mode Modify / full rate (26.6.7.1)	175

3.9	Cipher Mode Setting	176

3.9.1	MRR300: Ciphering mode / start ciphering (26.6.8.1)	176

3.9.2	MRR301: Ciphering mode / no ciphering (26.6.8.2)	178

3.9.3	MRR302: Ciphering mode / no ciphering (26.6.8.3)	180

3.9.4	MRR303: Ciphering mode setting (26.6.8.4)	181

3.9.5	MRR304: Ciphering mode / IMEISV request (26.6.8.5)	185

3.10	Classmark Interrogation	186

3.10.1	MRR350: Classmark interrogation (GSM 900, Single Band) (26.6.11.2)	186

3.10.2	MRR351: Classmark interrogation (DCS 1800, Single Band) (26.6.11.2)	188

3.10.3	MRR352: Classmark interrogation (GSM 900, Dual Band) (26.6.11.2)	190

3.10.4	MRR353: Classmark interrogation (DCS 1800, Dual Band) (26.6.11.2)	192

3.10.5	MRR354: Classmark interrogation (E-GSM, Dual Extended Band) (26.6.11.2)	193

3.10.6	MRR355: Classmark interrogation (PCS 1900, Single Band) (26.6.11.2)	196

3.10.7	MRR356: Classmark interrogation (GSM 850, Single Band) (26.6.11.2)	197

3.10.8	MRR357: Classmark interrogation (GSM 850+PCS 1900, DUAL US-Band) (26.6.11.2)	199

3.11	Channel Release	201

3.11.1	MRR401: Channel release / SDCCH (26.6.12.1)	201

3.11.2	MRR402: TCH Assignment with Hopping for DCS 1800	201

3.12	Test of Starting Time	202

3.12.1	MRR410: Imm Ass with starting time / successful / time not elapsed / (26.6.13.9)	202

3.13	Extended GSM	205

3.13.1	MRR450: Immediate Assignment with Frequency Hopping (26.10.2.2 K=2)	205

3.14	Additional Testcases	206

3.14.1	MRR500: Frequency Hopping in Assignment Command	206

3.14.2	MRR501: CBCH Problem with Frequency Hopping in Turkey	207

Appendices	208

A.	Acronyms	208

B.	Glossary	208

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

# Overview

This document describes the mulilayer tests ... as set out in GSM 11.10-1, §26.2.

# Parameters

#include <tcsl.h>

#include <tcsl_gsm.h>

/*------------------------------------------------------------------*\
| Default settings for cell A and B (GSM 11.10, 26.6.14)
\*------------------------------------------------------------------*/

#define  DOWNLINK_INPUT_LEVEL_DBM   -73   /* 63 bBmuV/m=(63-136.5) dBm=-73.5 dBm (05.05, 5) */
#define  DOWNLINK_INPUT_LEVEL        38   /* -73+111 (GSM 05.08, 8.1.4) */
#define  ARFCN_BCCH_GSM              20
#define  ARFCN_TCH_GSM               30
#define  ARFCN_BCCH_DCS             540
#define  ARFCN_TCH_DCS              700
#define  ARFCN_BCCH_EGSM            999
#define  ARFCN_TCH_EGSM            1000
#define  ARFCN_BCCH_850             200
#define  ARFCN_TCH_850              220
#define  DOWNLINK_INPUT_LEVEL_DBM_CELL_B  -83   /* 53 bBmuV/m=(63-136.5) dBm=-83.5 dBm */
#define  DOWNLINK_INPUT_LEVEL_CELL_B       28   /* -83+111 (GSM 05.08, 8.1.4) */
#define  ARFCN_BCCH_GSM_CELL_B             10
#define  RXLEV_60                   -60
#define  RXLEV_85                   -85
#define  RXLEV_80                   -80
#define  RXLEV_75                   -75
#define  RXLEV_55                   -55
#define  RXLEV_50                   -50
#define  RXLEV_45                   -45
#define  RXLEV_40                   -40

/* contains value of std=6 */
#define  ARFCN_BCCH_GSM_MB_CELL_1        ((6<<11)|(ARFCN_BCCH_GSM))
#define  ARFCN_BCCH_GSM_MB_CELL_2        ((6<<11)|(ARFCN_BCCH_GSM_CELL_B))

/*------------------------------------------------------------------*\
| other system settings
\*------------------------------------------------------------------*/

#define  NCC	0x1  /* note : ncc_permitted (GSM 04.08, 10.5.2.27) */
#define  BCC	0x5  /* note : GSM 11.10, 26.6.14, IMMEDIATE ASSIGNMENT */
#define  BSIC	((NCC<<3)|(BCC))
#define  RFN	0
#define  CKSN	3   /* ciphering key sequence number */
#define  TIMESLOT_TCH	3

#define BSIC_1_1	((1<<3)|(1))
#define BSIC_1_3	((1<<3)|(3))
#define BSIC_1_5	((1<<3)|(5))
#define BSIC_1_7	((1<<3)|(7))
#define ARFCN_000	0
#define ARFCN_002	2
#define ARFCN_026	26
#define ARFCN_038	38
#define ARFCN_990	990
#define ARFCN_1003	1003
#define ARFCN_1005	1005
#define ARFCN_1020	1020

/*------------------------------------------------------------------*\
| System Information IEs (GSM 11.10, 26.6.14 and 26.1.1)
\*------------------------------------------------------------------*/

/*
** cell_channel_description
** definition : GSM 04.08, 10.5.2.1b
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_1 (cell A)
*/
IE_BEGIN(cell_channel_description)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0000000,ACT_CHECK,chan_124_thru_097,"")
   BF(32,0x00000020,ACT_CHECK,chan_096_thru_065,"               70")
   BF(32,0x00020000,ACT_CHECK,chan_064_thru_033,"        50")
   BF(32,0x20080000,ACT_CHECK,chan_032_thru_001,"  30    20")
IE_END(cell_channel_description)

IE_BEGIN(cell_channel_description_turkey)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0004002,ACT_CHECK,chan_124_thru_097,"")
   BF(32,0x00000000,ACT_CHECK,chan_096_thru_065,"")
   BF(32,0x00000000,ACT_CHECK,chan_064_thru_033,"")
   BF(32,0x00000000,ACT_CHECK,chan_032_thru_001,"")
IE_END(cell_channel_description_turkey)

IE_BEGIN(cell_channel_description_300)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28,0x08000000,ACT_CHECK,chan_124_thru_097,"")
   BF(32,0x00080000,ACT_CHECK,chan_096_thru_065,"               70")
   BF(32,0x00000004,ACT_CHECK,chan_064_thru_033,"        50")
   BF(32,0x00020000,ACT_CHECK,chan_032_thru_001,"  30    20")
IE_END(cell_channel_description_300)

IE_BEGIN(cell_channel_description_SDCCH8_HOP_S1)
   BF( 2,         2,ACT_CHECK,format_id        ,"range 1024")
   BF( 1,         0,ACT_CHECK,ext_ind          ,"")
   BF( 1,         0,ACT_CHECK,spare            ,"")
   BF( 1,         0,ACT_CHECK,format_id2       ,"")
   BF( 1,         1,ACT_CHECK,f0               ,"")
   BF( 2,         0,ACT_CHECK,w1               ,"")
   BF( 8,        30,ACT_CHECK,w2               ,"")
   BF( 8,       234,ACT_CHECK,w3               ,"")
   BF( 8,       137,ACT_CHECK,w4               ,"")
   BF( 8,        62,ACT_CHECK,w5               ,"")
   BF( 8,       249,ACT_CHECK,w6               ,"")
   BF( 8,       129,ACT_CHECK,w7               ,"")
   BF( 8,        67,ACT_CHECK,w8               ,"")
   BF( 8,       128,ACT_CHECK,w9               ,"")
   BF( 8,         0,ACT_CHECK,w10              ,"")
   BF( 8,         0,ACT_CHECK,w11              ,"")
   BF( 8,         0,ACT_CHECK,w12              ,"")
   BF( 8,         0,ACT_CHECK,w13              ,"")
   BF( 8,         0,ACT_CHECK,w14              ,"")
   BF( 8,         0,ACT_CHECK,w15              ,"")
   BF( 8,         0,ACT_CHECK,w16              ,"")
IE_END(cell_channel_description_SDCCH8_HOP_S1)

/*
** cell_channel_description_26_6_13
** definition : GSM 04.08, 10.5.2.1b
** values     : GSM 11.10, 26.6.13
** usage      : system_information_type_1_cell_a_gsm900_26_6_13
*/
IE_BEGIN(cell_channel_description_cell_a_gsm900_26_6_13)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0020800,ACT_CHECK,chan_124_thru_097,"            114 108")
   BF(32,0x00000f02,ACT_CHECK,chan_096_thru_065,"     76 75 74 73 66")
   BF(32,0x04083202,ACT_CHECK,chan_064_thru_033,"  59 52 46 45 42 34")
   BF(32,0x02090200,ACT_CHECK,chan_032_thru_001," 26   20  17  10")
IE_END(cell_channel_description_cell_a_gsm900_26_6_13)

/*
** cell_channel_description_cell_b
** definition : GSM 04.08, 10.5.2.1b
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_1 (cell B)
*/
IE_BEGIN(cell_channel_description_cell_b)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0000000,ACT_CHECK,chan_124_thru_097,"")
   BF(32,0x00000000,ACT_CHECK,chan_096_thru_065,"")
   BF(32,0x00000000,ACT_CHECK,chan_064_thru_033,"")
   BF(32,0x00000200,ACT_CHECK,chan_032_thru_001,"channel number 10")
IE_END(cell_channel_description_cell_b)

/*
** cell_identity
** definition : GSM 04.08, 10.5.1.1
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_3 (cell A)
** usage      : system_information_type_6 (cell A)
*/
IE_BEGIN(cell_identity)
   BF(16,0x0001,ACT_CHECK,ci,"0001H")
IE_END(cell_identity)

IE_BEGIN(cell_identity_1)
   BF(16,0x0001,ACT_CHECK,ci,"0001H")
IE_END(cell_identity_1)

IE_BEGIN(cell_identity_2)
   BF(16,0x0002,ACT_CHECK,ci,"0002H")
IE_END(cell_identity_2)

IE_BEGIN(cell_identity_3)
   BF(16,0x0003,ACT_CHECK,ci,"0003H")
IE_END(cell_identity_3)

IE_BEGIN(cell_identity_4)
   BF(16,0x0004,ACT_CHECK,ci,"0004H")
IE_END(cell_identity_4)

IE_BEGIN(cell_identity_5)
   BF(16,0x0005,ACT_CHECK,ci,"0005H")
IE_END(cell_identity_5)

IE_BEGIN(cell_identity_6)
   BF(16,0x0006,ACT_CHECK,ci,"0006H")
IE_END(cell_identity_6)

IE_BEGIN(cell_identity_7)
   BF(16,0x0007,ACT_CHECK,ci,"0007H")
IE_END(cell_identity_7)

IE_BEGIN(cell_identity_8)
   BF(16,0x0008,ACT_CHECK,ci,"0008H")
IE_END(cell_identity_8)

IE_BEGIN( identity_type_imsi)		/* Ref.: [1], §10.5.3.4 */
	BF( 1,	0,	ACT_CHECK,	ANONYMOUS,	"spare")
	BF( 3,	TYPE_IMSI,	ACT_CHECK,	type,	"IMSI")
IE_END( identity_type_imsi)

/*
** cell_identity_cell_b
** definition : GSM 04.08, 10.5.1.1
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_3 (cell B)
** usage      : system_information_type_6 (cell B)
*/
IE_BEGIN(cell_identity_cell_b)
   BF(16,0x0002,ACT_CHECK,ANONYMOUS,"0002H")
IE_END(cell_identity_cell_b)

/*
** cell_options_bcch
** definition : GSM 04.08, 10.5.2.3
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_3
*/
IE_BEGIN(cell_options_bcch)
   BF(1,0,ACT_CHECK,ANONYMOUS          ,"spare")
   BF(1,0,ACT_CHECK,pwrc               ,"power control indicator is not set")
   BF(2,2,ACT_CHECK,dtx                ,"MS shall not use DTX")
   BF(4,1,ACT_CHECK,radio_link_timeout ,"8 SACCH blocks")
IE_END(cell_options_bcch)

/*
** cell_options_sacch
** definition : GSM 04.08, 10.5.2.3a
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_3
*/
IE_BEGIN(cell_options_sacch)
   BF(1,0,ACT_CHECK,dtx0               ,"MS shall not use DTX")
   BF(1,0,ACT_CHECK,pwrc               ,"power control indicator is not set")
   BF(2,2,ACT_CHECK,dtx                ,"MS shall not use DTX")
   BF(4,1,ACT_CHECK,radio_link_timeout ,"8 SACCH blocks")
IE_END(cell_options_sacch)

/*
** cell_selection_parameter
** definition : GSM 04.08, 10.5.2.4
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_3
** usage      : system_information_type_4
*/
IE_BEGIN(cell_selection_parameter)
   BF(3,M3(1,1,0),ACT_CHECK,cell_reselect_hysteresis,"12 dB")
   BF(5,       31,ACT_CHECK,ms_txpwr_max_cch        ,"minimum level (5 dBm)")
   BF(1,        0,ACT_CHECK,acs                     ,"no add cell params (SI4), 0 in SI3")
   BF(1,        0,ACT_CHECK,neci                    ,"New establishment cause not supported")
   BF(6,        0,ACT_CHECK,rxlev_access_min        ,"minimum level (less than -111 dBm)")
IE_END(cell_selection_parameter)

IE_BEGIN(cipher_mode_setting_a5_1)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(cipher_mode_setting_a5_1)

IE_BEGIN(cipher_mode_setting_a5_2)

BF(3,M3(0,0,1),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(cipher_mode_setting_a5_2)

IE_BEGIN(cipher_mode_setting_none)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        0,ACT_CHECK,     start_ciphering,"not start ciphering")

IE_END(cipher_mode_setting_none)

/*
** control_channel_description
** definition : GSM 04.08, 10.5.2.11
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_3
*/
IE_BEGIN(control_channel_description)

BF(1,        0,ACT_CHECK,ANONYMOUS     ,"spare")
   BF(1,        0,ACT_CHECK,att           ,"no attach/detach")
   BF(3,        0,ACT_CHECK,bs_ag_blks_res,"0 blocks reserved for access grant")
   BF(3,M3(0,0,1),ACT_CHECK,ccch_conf     ,"1 basic physical channel,combined with SDCCHs")
   BF(5,        0,ACT_CHECK,ANONYMOUS     ,"spare")
   BF(3,        3,ACT_CHECK,bs_pa_mfrms   ,"5 multiframes periods for transmission" 
                                      "   of paging messages")
   BF(8,        0,ACT_CHECK,t3212         ,"Infinite")
IE_END(control_channel_description)

/*
** l2_pseudo_length_9
** definition : GSM 04.08, 10.5.2.16
** values     : GSM 04.08, 10.5.2.16
** usage      : paging_request_type_1
*/
IE_BEGIN(l2_pseudo_length_9)
   BF(6,9,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1,0,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1,1,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(l2_pseudo_length_9)

/*
** l2_pseudo_length_11
** definition : GSM 04.08, 10.5.2.16
** values     : GSM 04.08, 10.5.2.16
** usage      : immediate_assignment
*/
IE_BEGIN(l2_pseudo_length_11)
   BF(6,11,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(l2_pseudo_length_11)

/*
** l2_pseudo_length_12
** definition : GSM 04.08, 10.5.2.19
** values     : GSM 04.08, 10.5.2.19
** usage      : system_information_type_4
*/
IE_BEGIN(l2_pseudo_length_12)
   BF(6,12,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(l2_pseudo_length_12)

/*
** l2_pseudo_length_14
** definition : GSM 04.08, 10.5.2.19
** values     : GSM 04.08, 10.5.2.19
** usage      : 26.6.5.2, M=7
*/
IE_BEGIN(l2_pseudo_length_14)
   BF(6,14,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(l2_pseudo_length_14)

/*
** l2_pseudo_length_18
** definition : GSM 04.08, 10.5.2.19
** values     : GSM 04.08, 10.5.2.19
** usage      : system_information_type_3
** usage      : immediate_assignment_extended
*/
IE_BEGIN(l2_pseudo_length_18)
   BF(6,18,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(l2_pseudo_length_18)

/*
** l2_pseudo_length_19
** definition : GSM 04.08, 10.5.2.19
** values     : GSM 04.08, 10.5.2.19
** usage      : immediate_assignment_reject
** usage      : paging_type_3
*/
IE_BEGIN(l2_pseudo_length_19)
   BF(6,19,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(l2_pseudo_length_19)

/*
** l2_pseudo_length_21
** definition : GSM 04.08, 10.5.2.19
** values     : GSM 04.08, 10.5.2.19
+** usage      : system_information_type_1
** usage      : immediate_assignment
*/
IE_BEGIN(l2_pseudo_length_21)
   BF(6,21,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(l2_pseudo_length_21)

/*
** l2_pseudo_length_22
** definition : GSM 04.08, 10.5.2.19
** values     : GSM 04.08, 10.5.2.19
** usage      : system_information_type_2
*/
IE_BEGIN(l2_pseudo_length_22)
   BF(6,22,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)
   BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(l2_pseudo_length_22)

/*
** location_area_identification
** definition : GSM 04.08, 10.5.1.3
** values     : GSM 04.08, 26.6.14
** usage      : system_information_type_4
** usage      : system_information_type_6
** usage      : location_updating_request
*/
IE_BEGIN(location_area_identification)
   BF( 4,     6,ACT_CHECK,mcc_dig_2,"digit 2 of mobile country code")
   BF( 4,     2,ACT_CHECK,mcc_dig_1,"digit 1 of mobile country code")
   BF( 4,   0xF,ACT_CHECK,ANONYMOUS,"end of MCC")
   BF( 4,     2,ACT_CHECK,mcc_dig_3,"digit 3 of mobile country code")
   BF( 4,     1,ACT_CHECK,mnc_dig_2,"digit 2 of mobile network code")
   BF( 4,     0,ACT_CHECK,mnc_dig_1,"digit 1 of mobile network code")
   BF(16,0x0001,ACT_CHECK,lac      ,"Location area code")
IE_END(location_area_identification)

/*
** ncc_permitted
** definition : GSM 04.08, 10.5.2.27
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_2
*/
IE_BEGIN(ncc_permitted)
   BF(8,M8(0,0,0,0,0,0,1,0),ACT_CHECK,ncc_permit,"0000 0010 (NCC==1 only)")
IE_END(ncc_permitted)

/*
** neigbour_cell_description
** definition : GSM 04.08, 10.5.2.22
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_2
** usage      : system_information_type_5
*/
IE_BEGIN(neighbour_cell_description)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"BCCH allocation sequence")
   BF(28, 0x0802008,ACT_CHECK,ba_124_097,"channel numbers:  120  110  100")
   BF(32,0x02008000,ACT_CHECK,ba_096_065,"channel numbers:   90     80")
   BF(32,0x00000080,ACT_CHECK,ba_064_033,"channel numbers:             40")
   BF(32,0x00080200,ACT_CHECK,ba_032_001,"channel numbers:        20  10")
IE_END(neighbour_cell_description)

IE_BEGIN(neighbour_cell_description_E)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"BCCH allocation sequence")
   BF(28, 0x0802008,ACT_CHECK,ba_124_097,"channel numbers:  120  110  100")
   BF(32,0x02008000,ACT_CHECK,ba_096_065,"channel numbers:   90     80")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"channel numbers:")
   BF(32,0x00080200,ACT_CHECK,ba_032_001,"channel numbers:        20  10")
IE_END(neighbour_cell_description_E)

IE_BEGIN(neighbour_cell_description_E_N1)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"band allocation indication")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         0,ACT_CHECK,f0       ,"frequency 0 indicator")
   BF(10,       990,ACT_CHECK,w1       ,"channel 990 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_E_N1)

IE_BEGIN(neighbour_cell_description_E_N2)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"band allocation indication")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         0,ACT_CHECK,f0       ,"frequency 0 indicator")
   BF(10,      1005,ACT_CHECK,w1       ,"channel 1005 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_E_N2)

IE_BEGIN(neighbour_cell_description_E_N3)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"band allocation indication")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         1,ACT_CHECK,f0       ,"frequency 0 indicator")
   BF(10,         0,ACT_CHECK,w1       ,"no further channels")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_E_N3)

IE_BEGIN(neighbour_cell_description_E_N4)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"band allocation indication")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         0,ACT_CHECK,f0       ,"frequency 0 indicator")
   BF(10,        26,ACT_CHECK,w1       ,"channel 26 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_E_N4)

IE_BEGIN(neighbour_cell_description_E_N5)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"band allocation indication")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         0,ACT_CHECK,f0       ,"frequency 0 indicator")
   BF(10,      1020,ACT_CHECK,w1       ,"channel 1020 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_E_N5)

IE_BEGIN(neighbour_cell_description_E_N6)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"band allocation indication")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         0,ACT_CHECK,f0       ,"frequency 0 indicator")
   BF(10,        38,ACT_CHECK,w1       ,"channel 38 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_E_N6)

IE_BEGIN(neighbour_cell_description_E_N7)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"band allocation indication")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         0,ACT_CHECK,f0       ,"frequency 0 indicator")
   BF(10,      1003,ACT_CHECK,w1       ,"channel 1003 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_E_N7)

/*
** rach_control_parameter
** definition : GSM 04.08, 10.5.2.29
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_1
** usage      : system_information_type_2
** usage      : system_information_type_3
** usage      : system_information_type_4
*/
IE_BEGIN(rach_control_parameter)
   BF( 2,          0,ACT_CHECK,max_retrans                ,"max 1 retrans")
   BF( 4,M4(0,0,1,0),ACT_CHECK,tx_integer                 ,"5 slots used")
   BF( 1,          0,ACT_CHECK,cell_bar_access            ,"cell is not barred")
   BF( 1,          1,ACT_CHECK,call_re_establishment      ,"not Allowed")
   BF( 5,          0,ACT_CHECK,access_control_class_15_11 ,"access is not barred")
   BF( 1,          0,ACT_CHECK,emergency_call             ,"allowed")
   BF(10,          0,ACT_CHECK,access_control_class_09_00 ,"access is not barred")
IE_END(rach_control_parameter)

/*
** si_1_rest_octets
** definition : GSM 04.08, 10.5.2.32
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_1
*/
IE_BEGIN(si_1_rest_octets)
   BF(8,REST_OCTET,ACT_CHECK,spare,"not used (all bits are set to spare)")
IE_END(si_1_rest_octets)

/*
** si_3_rest_octets
** definition : GSM 04.08, 10.5.2.34
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_3
*/
IE_BEGIN(si_3_rest_octets)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(si_3_rest_octets)

/*
** si_4_rest_octets
** definition : GSM 04.08, 10.5.2.35
** values     : GSM 11.10, 26.6.14
** usage      : system_information_type_4
*/
IE_BEGIN(si_4_rest_octets)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(si_4_rest_octets)

IE_BEGIN(si_6_rest_octets)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(si_6_rest_octets)

IE_BEGIN(mobile_identity_imsi)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_5,SILENT)

BF(4,          0,ACT_CHECK, digit_4,SILENT)

BF(4,          7,ACT_CHECK, digit_7,SILENT)

BF(4,          4,ACT_CHECK, digit_6,SILENT)

BF(4,          1,ACT_CHECK, digit_9,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          9,ACT_CHECK, digit_11,SILENT)

BF(4,          4,ACT_CHECK, digit_10,SILENT)

BF(4,          2,ACT_CHECK, digit_13,SILENT)

BF(4,          1,ACT_CHECK, digit_12,SILENT)

IE_END(mobile_identity_imsi)

IE_BEGIN(si_4_rest_octets_turkey)
   BF(8,0x80,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,0x00,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(si_4_rest_octets_turkey)

IE_BEGIN(cbch_channel_description_turkey)
   BF(8,0x64,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,0x52,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,0x70,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,0x36,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(cbch_channel_description_turkey)

IE_BEGIN(cbch_mobile_allocation_turkey)
   BF(8,0x72,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,0x01,ACT_CHECK,ANONYMOUS,SILENT)
   BF(8,0x03,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(cbch_mobile_allocation_turkey)

MSG3_BEGIN( identity_request)	/* Ref.: [1], §9.2.10 */
	IE( skip_indicator)
	IE( mobility_management_protocol_discriminator)
	IE( identity_request_message_type)
	IE( spare_half_octet)
	IE( identity_type_imsi)
MSG3_END( identity_request)

MSG3_BEGIN( identity_response)	/* Ref.: [1], §9.2.11 */
	IE( skip_indicator)
	IE( mobility_management_protocol_discriminator)
	IE( identity_response_message_type)
	IE( mobile_identity_imsi)
MSG3_END( identity_response)

/*------------------------------------------------------------------*\
| System Information Messages (GSM 11.10, 26.6.14 and 26.1.1)
\*------------------------------------------------------------------*/

/*
** system_information_type_1
** definition : GSM 04.08, 9.1.31
** values     : GSM 11.10, 26.6.14
*/
MSG3_BEGIN(system_information_type_1)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1)

MSG3_BEGIN(system_information_type_1_cell_a_gsm900_26_6_13)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description_cell_a_gsm900_26_6_13)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_cell_a_gsm900_26_6_13)

MSG3_BEGIN(system_information_type_1_turkey)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description_turkey)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_turkey)

/*
** system_information_type_1_cell_b
** definition : GSM 04.08, 9.1.31
** values     : GSM 11.10, 26.6.14
*/
MSG3_BEGIN(system_information_type_1_cell_b)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description_cell_b)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_cell_b)

/*
** system_information_type_2
** definition : GSM 04.08, 9.1.32
** values     : GSM 11.10, 26.6.14
*/
MSG3_BEGIN(system_information_type_2)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2)

/*
** system_information_type_3
** definition : GSM 04.08, 9.1.35
** values     : GSM 11.10, 26.6.14
*/
MSG3_BEGIN(system_information_type_3)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3)

/*
** system_information_type_3_cell_b
** definition : GSM 04.08, 9.1.35
** values     : GSM 11.10, 26.6.14
*/
MSG3_BEGIN(system_information_type_3_cell_b)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_cell_b)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_cell_b)

/*
** system_information_type_4
** definition : GSM 04.08, 9.1.36
** values     : GSM 11.10, 26.6.14
*/
MSG3_BEGIN(system_information_type_4)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4)

MSG3_BEGIN(system_information_type_4_turkey)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(cbch_channel_description_turkey)

IE(cbch_mobile_allocation_turkey)

IE(si_4_rest_octets_turkey)

MSG3_END(system_information_type_4_turkey)

/*
** system_information_type_5
** definition : GSM 04.08, 9.1.37
** values     : GSM 11.10, 26.6.14
*/
MSG3_BEGIN(system_information_type_5)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(neighbour_cell_description)

MSG3_END(system_information_type_5)

/*
** system_information_type_6
** definition : GSM 04.08, 9.1.40
** values     : GSM 11.10, 26.6.14
*/
MSG3_BEGIN(system_information_type_6)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_6_message_type)

IE(cell_identity)

IE(location_area_identification)

IE(cell_options_sacch)

IE(ncc_permitted)

IE(si_6_rest_octets)

MSG3_END(system_information_type_6)

/*
** system_information_type_6_cell_b
** definition : GSM 04.08, 9.1.40
** values     : GSM 11.10, 26.6.14
*/
MSG3_BEGIN(system_information_type_6_cell_b)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_6_message_type)

IE(cell_identity_cell_b)

IE(location_area_identification)

IE(cell_options_sacch)

IE(ncc_permitted)

IE(si_6_rest_octets)

MSG3_END(system_information_type_6_cell_b)

/*------------------------------------------------------------------*\
| Information Elements (GSM 11.10, 26.6.14)
\*------------------------------------------------------------------*/

IE_BEGIN(mobile_identity_tmsi)             /* has 8 octets */

BF(8,          5,ACT_CHECK,  length,"seven octets to come")

BF(4,       0x0F,ACT_CHECK, digit_1,SILENT)

BF(1,          0,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(1,0,0),ACT_CHECK,    type,"TMSI")

BF(8,       0x12,ACT_CHECK, digit_3,SILENT)

BF(8,       0x34,ACT_CHECK, digit_2,SILENT)

BF(8,       0x56,ACT_CHECK, digit_5,SILENT)

BF(8,       0x78,ACT_CHECK, digit_4,SILENT)

IE_END(mobile_identity_tmsi)

MSG3_BEGIN( tmsi_reallocation_command_tmsi)	/* Ref.: [1], §9.2.17 */
	IE( skip_indicator)
	IE( mobility_management_protocol_discriminator)
	IE( tmsi_reallocation_command_message_type)
	IE( location_area_identification)
	IE( mobile_identity_tmsi)
MSG3_END( tmsi_reallocation_command_tmsi)
MSG3_BEGIN( tmsi_reallocation_complete)	/* Ref.: [1], §9.2.18 */
	IE( skip_indicator)
	IE( mobility_management_protocol_discriminator)
	IE( tmsi_reallocation_complete_message_type)
MSG3_END( tmsi_reallocation_complete)
/*
** authentication_parameter_rand
** definition : GSM 04.08, 10.5.3.1 (MM)
** values     : GSM 11.10, 26.6.14
** usage      : authentication_request
*/
IE_BEGIN(authentication_parameter_rand)
   BF(32,0x80000000,ACT_CHECK,rand_127_096,"chosen")
   BF(32,0x00000012,ACT_CHECK,rand_095_064,"arbitrarily")
   BF(32,0x34000000,ACT_CHECK,rand_063_032,"by the")
   BF(32,0x0000000F,ACT_CHECK,rand_031_000,"test house")
IE_END(authentication_parameter_rand)

/*
** authentication_parameter_sres
** definition : GSM 04.08, 10.5.3.2 (MM)
** values     : GSM 11.10, 26.6.14
** usage      : authentication_response
*/
IE_BEGIN(authentication_parameter_sres)

BF(32,0x0000000F,ACT_SHOW,sres_031_000,"not checked")

IE_END(authentication_parameter_sres)

/*
** bearer_capability
** definition : GSM 04.08, 10.5.4.5 (CC)
** values     : GSM 11.10, 26.6.14
** usage      : setup
*/
IE_BEGIN(bearer_capability)

BF(8,4,ACT_CHECK,length,                   SILENT)

BF(1,0,ACT_CHECK,ext3,                     SILENT)

BF(2,1,ACT_CHECK,radio_channel_requirement,"full rate channel")

BF(1,0,ACT_CHECK,coding_standard,       "   GSM standardized coding")

BF(1,0,ACT_CHECK,transfer_mode,         "   circuit mode")

BF(3,0,ACT_CHECK,info_transfer_capability," speech")

BF(1,0,ACT_CHECK,ext3a,                      SILENT)

BF(7,4,ACT_CHECK,speech_version_indication_1,SILENT)

BF(1,0,ACT_CHECK,ext3b,                      SILENT)

BF(7,5,ACT_CHECK,speech_version_indication_2,SILENT)

BF(1,1,ACT_CHECK,ext3c,                      SILENT)

BF(7,0,ACT_CHECK,speech_version_indication_3,SILENT)

IE_END(bearer_capability)

IE_BEGIN(bearer_capability_1800_DUAL)

BF(8,0x03,ACT_CHECK,length,  SILENT)

BF(8,0x20,ACT_CHECK,rcr,     SILENT)

BF(8,0x02,ACT_CHECK,hr_fr,   SILENT)

BF(8,0x80,ACT_CHECK,efr,     SILENT)

IE_END(bearer_capability_1800_DUAL)

/*
** called_party_bcd_number
** definition : GSM 04.08, 10.5.4.7 (CC)
** values     : GSM 11.10, 26.6.14
** usage      : setup
*/
IE_BEGIN(called_party_bcd_number)

BF(8,9,ACT_CHECK,length,         SILENT)

BF(1,1,ACT_CHECK,ext1,           SILENT)

BF(3,0,ACT_CHECK,type_of_number,"unknown")

BF(4,1,ACT_CHECK,numbering_plan,"unknown")

BF(16,0x2143,ACT_CHECK,ANONYMOUS,"digits")

BF(16,0x6587,ACT_CHECK,ANONYMOUS,SILENT)

BF(16,0x0921,ACT_CHECK,ANONYMOUS,SILENT)

BF(16,0x4365,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(called_party_bcd_number)

/*
** cell_description
** definition : GSM 04.08, 10.5.2.2 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : handover_command
*/
IE_BEGIN(cell_description)

BF( 2,ARFCN_BCCH_GSM_CELL_B>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,                   1,ACT_CHECK,ncc,  "   network colour code")

BF( 3,                   5,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_BCCH_GSM_CELL_B   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(cell_description_20)

BF( 2,                   0,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,                   1,ACT_CHECK,ncc,  "   network colour code")

BF( 3,                   5,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,                  20,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description_20)

/*
** channel_description
** definition : GSM 04.08, 10.5.2.5 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : assignment_command
** usage      : channel_mode_modify
** usage      : handover_command
** usage      : immediate_assignment_extended
*/
IE_BEGIN(channel_description_GSM)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,  channel_type,"Bm + ACCHs (TCH/F+ACCHs)")

BF( 3, TIMESLOT_TCH,ACT_CHECK,  time_slot_number,"chosen arbitrarily by the test house")

BF( 3,          BCC,ACT_CHECK,  training_sequence_code,"")

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,    ARFCN_TCH_GSM,ACT_CHECK,  arfcn,"GSM channel number")

IE_END(channel_description_GSM)

IE_BEGIN(channel_description_DCS)

BF( 5,  M5(0,0,0,0,1),ACT_CHECK,  channel_type,"Bm + ACCHs (TCH/F+ACCHs)")

BF( 3,   TIMESLOT_TCH,ACT_CHECK,  time_slot_number,"chosen arbitrarily by the test house")

BF( 3,            BCC,ACT_CHECK,  training_sequence_code,"")

BF( 1,              0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,              0,ACT_CHECK,  spare,SILENT)

BF(10,  ARFCN_TCH_DCS,ACT_CHECK,  arfcn,"DCS channel number")

IE_END(channel_description_DCS)

IE_BEGIN(channel_description_EGSM)

BF( 5,  M5(0,0,0,0,1),ACT_CHECK,  channel_type,"Bm + ACCHs (TCH/F+ACCHs)")

BF( 3,   TIMESLOT_TCH,ACT_CHECK,  time_slot_number,"chosen arbitrarily by the test house")

BF( 3,            BCC,ACT_CHECK,  training_sequence_code,"")

BF( 1,              0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,              0,ACT_CHECK,  spare,SILENT)

BF(10, ARFCN_TCH_EGSM,ACT_CHECK,  arfcn,"E-GSM channel number")

IE_END(channel_description_EGSM)

IE_BEGIN(channel_description_850)

BF( 5,  M5(0,0,0,0,1),ACT_CHECK,  channel_type,"Bm + ACCHs (TCH/F+ACCHs)")

BF( 3,   TIMESLOT_TCH,ACT_CHECK,  time_slot_number,"chosen arbitrarily by the test house")

BF( 3,            BCC,ACT_CHECK,  training_sequence_code,"")

BF( 1,              0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,              0,ACT_CHECK,  spare,SILENT)

BF(10,  ARFCN_TCH_850,ACT_CHECK,  arfcn,"GSM 850 channel number")

IE_END(channel_description_850)

IE_BEGIN(channel_description_HOP_TCHF4)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            4,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            7,ACT_CHECK,training_sequence_code,SILENT)

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            1,ACT_CHECK,                  maio,SILENT)

BF( 6,           34,ACT_CHECK,                   hsn,SILENT)

IE_END(channel_description_HOP_TCHF4)

IE_BEGIN(channel_description_HOP_TCHH4)

BF( 5,M5(0,0,0,1,1),ACT_CHECK,          channel_type,"TCH/H")

BF( 3,            4,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            7,ACT_CHECK,training_sequence_code,SILENT)

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            1,ACT_CHECK,                  maio,SILENT)

BF( 6,           34,ACT_CHECK,                   hsn,SILENT)

IE_END(channel_description_HOP_TCHH4)

IE_BEGIN(channel_description_HOP_TCHF1)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            1,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            0,ACT_CHECK,training_sequence_code,SILENT)

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            1,ACT_CHECK,                  maio,SILENT)

BF( 6,            0,ACT_CHECK,                   hsn,SILENT)

IE_END(channel_description_HOP_TCHF1)

IE_BEGIN(channel_description_HOP_TCHF6)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            6,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            6,ACT_CHECK,training_sequence_code,SILENT)

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            1,ACT_CHECK,                  maio,SILENT)

BF( 6,            0,ACT_CHECK,                   hsn,SILENT)

IE_END(channel_description_HOP_TCHF6)

IE_BEGIN(channel_description_HOP_SDCCH8)

BF( 5,M5(0,1,0,0,0),ACT_CHECK,          channel_type,"SDCCH/8")

BF( 3,            2,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            5,ACT_CHECK,training_sequence_code,SILENT)

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            0,ACT_CHECK,                  maio,SILENT)

BF( 6,            1,ACT_CHECK,                   hsn,SILENT)

IE_END(channel_description_HOP_SDCCH8)

/*
** channel_description_non_comb
** definition : GSM 04.08, 10.5.2.5 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment
*/
IE_BEGIN(channel_description_non_comb)

BF( 5,M5(0,1,0,0,0),ACT_CHECK,  channel_type,"SDCCH/8")

BF( 3, TIMESLOT_TCH,ACT_CHECK,  time_slot_number,"chosen arbitrarily by the test house")

BF( 3,          BCC,ACT_CHECK,  training_sequence_code,"")

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,    ARFCN_TCH_GSM,ACT_CHECK,  arfcn,"channel number 30")

IE_END(channel_description_non_comb)

/*
** channel_description_comb
** definition : GSM 04.08, 10.5.2.5 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment
*/
IE_BEGIN(channel_description_comb)

BF( 5,M5(0,0,1,0,0),ACT_CHECK,  channel_type,"SDCCH/4")

BF( 3,            0,ACT_CHECK,  time_slot_number,"for combind CCCH/SDCCH timeslot 0")

BF( 3,          BCC,ACT_CHECK,  training_sequence_code,"TSC=5")

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,  arfcn,"channel number 20")

IE_END(channel_description_comb)

/*
** channel_mode
** definition : GSM 04.08, 10.5.2.6 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : channel_mode_modify
*/
IE_BEGIN(channel_mode)

BF(8,0,ACT_CHECK,mode_value,"depending on test")

IE_END(channel_mode)

/*
** channel_needed
** definition : GSM 04.08, 10.5.2.8 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : paging_request_type_1
*/
IE_BEGIN(channel_needed)
   BF(2,0,ACT_CHECK,second_channel,"spare, any channel")
   BF(2,0,ACT_CHECK, first_channel,"spare, any channel")
IE_END(channel_needed)

/*
** ciphering_mode_setting
** definition : GSM 04.08, 10.5.2.9 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : ciphering_mode_command
*/
IE_BEGIN(ciphering_mode_setting)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(ciphering_mode_setting)

/*
** cipher_response
** definition : GSM 04.08, 10.5.2.10 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : ciphering_mode_command
*/
IE_BEGIN(cipher_response)

BF(3,0,ACT_CHECK,          spare,SILENT)

BF(1,0,ACT_CHECK,cipher_response,"IMEISV shall not be included")

IE_END(cipher_response)

/*
** ciphering_key_sequence_number
** definition : GSM 04.08, 10.5.1.2 (common)
** values     : GSM 11.10, 26.6.14
** usage      : authentication_request
** usage      : cm_service_request
*/
IE_BEGIN(ciphering_key_sequence_number)

BF(1,   0,ACT_CHECK,       spare,SILENT)

BF(3,CKSN,ACT_CHECK,key_sequence,"chosen arbitrarily by the test house")

IE_END(ciphering_key_sequence_number)

/*
** cm_service_type
** definition : GSM 04.08, 10.5.3.3 (MM)
** values     : GSM 11.10, 26.6.14
** usage      : authentication_request
** usage      : cm_service_request
*/
IE_BEGIN(cm_service_type)

BF(4, 1,ACT_CHECK,type_value,"mobile originated call")

IE_END(cm_service_type)

/*
 * frequency list
 */
IE_BEGIN(frequency_list_1800_H)
   BF( 8,      0x04,ACT_CHECK,length,"length of information element")
   BF( 8,      0x89,ACT_CHECK,format,"format 512")
   BF( 8,      0x12,ACT_CHECK,orig_arfcn,"ARFCN = 548")
   BF( 8,      0x01,ACT_CHECK,w1," w1 = 5")
   BF( 8,      0x40,ACT_CHECK,w2," w2 = 0")
IE_END(frequency_list_1800_H)

/*
** handover_reference
** definition : GSM 04.08, 10.5.2.15 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : handover_access
** usage      : handover_command
*/
IE_BEGIN(handover_reference)

BF(8,98,ACT_CHECK,reference_value,"equal in handover_command and handover_access")

IE_END(handover_reference)

/*
** ia_rest_octets
** definition : GSM 04.08, 10.5.2.16 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment
*/
IE_BEGIN(ia_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(ia_rest_octets)

/*
** iar_rest_octets
** definition : GSM 04.08, 10.5.2.17 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_reject
*/
IE_BEGIN(iar_rest_octets) 
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */
IE_END(iar_rest_octets)

/*
** iax_rest_octets
** definition : GSM 04.08, 10.5.2.18 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_extended
*/
IE_BEGIN(iax_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

IE_END(iax_rest_octets)

IE_BEGIN(iei_04)
   BF(8,0x04,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(iei_04)

IE_BEGIN(iei_05)
   BF(8,0x05,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(iei_05)

IE_BEGIN(iei_09)
   BF(4,0x09,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(iei_09)

IE_BEGIN(iei_17)
   BF(8,0x17,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(iei_17)

IE_BEGIN(iei_5e)
   BF(8,0x5e,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(iei_5e)

IE_BEGIN(iei_62)

BF(8,0x62,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_62)

IE_BEGIN(iei_63)

BF(8,0x63,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_63)

IE_BEGIN(iei_69)

BF(8,0x69,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_69)

IE_BEGIN(iei_72)
   BF(8,0x72,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(iei_72)
IE_BEGIN(iei_77)
   BF(8,0x77,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(iei_77)
IE_BEGIN(iei_7c)

BF(8,0x7c,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_7c)

IE_BEGIN(iei_7b)

BF(8,0x7b,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_7b)

IE_BEGIN(iei_7d)

BF(8,0x7d,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_7d)

IE_BEGIN(iei_a1)

BF(8,0xa1,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_a1)

/*
** location_updating_type
** definition : GSM 04.08, 10.5.3.5 (MM)
** values     : GSM 11.10, 26.6.14
** usage      : location_updating_request
*/
IE_BEGIN(location_updating_type)

BF(1,0,ACT_CHECK,for  ,"no follow-on request pending")

BF(1,0,ACT_CHECK,spare,SILENT)

BF(2,0,ACT_CHECK,lut  ,"normal location updating")

IE_END(location_updating_type)

IE_BEGIN(location_updating_type_att)

BF(1,0,ACT_CHECK,for  ,"no follow-on request pending")

BF(1,0,ACT_CHECK,spare,SILENT)

BF(2,2,ACT_CHECK,lut  ,"IMSI attach")

IE_END(location_updating_type_att)

/*
** mobile_allocation
** definition : GSM 04.08, 10.5.2.21 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment
*/
IE_BEGIN(mobile_allocation)

BF(8,0,ACT_CHECK,length,"length 0 due to hopping disabled")

IE_END(mobile_allocation)

IE_BEGIN(mobile_allocation_28)

BF(8,1,ACT_CHECK,length,SILENT)

BF(8,M8(0,0,0,0,0,0,0,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobile_allocation_28)

IE_BEGIN(mobile_allocation_300)

BF(8,1,ACT_CHECK,length,SILENT)

BF(8,M8(0,0,0,0,1,1,1,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobile_allocation_300)

IE_BEGIN(mobile_allocation_E)

BF(8,1,ACT_CHECK,length,SILENT)

BF(8,184,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobile_allocation_E)

/*
** mobile_identity
** definition : GSM 04.08, 10.5.1.4 (common)
** values     : GSM 11.10, 26.6.14
** length     : 6 octets
** usage      : classmark_change
** usage      : cm_service_request
** usage      : paging_request_type_1
*/
IE_BEGIN(mobile_identity_fill)
   BF(8,          1,ACT_CHECK,length,"   one octet to come")
   BF(4,M4(0,0,0,0),ACT_CHECK,ANONYMOUS,"bits 5-8 of octet 3 ='0000'")
   BF(1,          0,ACT_CHECK,odd_even,"")
   BF(3,  M3(0,0,0),ACT_CHECK,type,  "   no identity")
IE_END(mobile_identity_fill)

/*
** mobile_identity_tmsi
** definition : GSM 04.08, 10.5.2.42 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : paging_request_type_2
*/
IE_BEGIN(tmsi)
   BF(8,       0x12,ACT_CHECK,ANONYMOUS, SILENT)
   BF(8,       0x34,ACT_CHECK,ANONYMOUS, SILENT)
   BF(8,       0x56,ACT_CHECK,ANONYMOUS, SILENT)
   BF(8,       0x78,ACT_CHECK,ANONYMOUS, SILENT)
IE_END(tmsi)

/*
** mobile_identity_tmsi_2
** definition : GSM 04.08, 10.5.2.42 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : paging_request_type_2
*/
IE_BEGIN(tmsi_2)
   BF(8,       0xAA,ACT_CHECK,ANONYMOUS, SILENT)
   BF(8,       0xBB,ACT_CHECK,ANONYMOUS, SILENT)
   BF(8,       0xCC,ACT_CHECK,ANONYMOUS, SILENT)
   BF(8,       0xDD,ACT_CHECK,ANONYMOUS, SILENT)
IE_END(tmsi_2)

IE_BEGIN(mobile_identity_imeisv)             /* has 8 octets */

BF(8,          9,ACT_CHECK,  length,"nine octets to come")

BF(4,          1,ACT_CHECK , digit_1,SILENT)

BF(1,          0,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,1,1),ACT_CHECK,    type,"IMEISV")

BF(4,          5,ACT_CHECK , digit_3,SILENT)

BF(4,          3,ACT_CHECK , digit_2,SILENT)

BF(4,          9,ACT_CHECK , digit_5,SILENT)

BF(4,          7,ACT_CHECK , digit_4,SILENT)

BF(4,          2,ACT_CHECK , digit_7,SILENT)

BF(4,          0,ACT_CHECK , digit_6,SILENT)

BF(4,          6,ACT_CHECK , digit_9,SILENT)

BF(4,          4,ACT_CHECK , digit_8,SILENT)

BF(4,          1,ACT_CHECK , digit_11,SILENT)

BF(4,          8,ACT_CHECK , digit_10,SILENT)

BF(4,          2,ACT_CHECK , digit_13,SILENT)

BF(4,          1,ACT_CHECK , digit_12,SILENT)

BF(4,          7,ACT_CHECK , digit_15,SILENT)

BF(4,          2,ACT_CHECK , digit_14,SILENT)

BF(4,        0xF,ACT_CHECK, eoi     ,SILENT)

BF(4,          8,ACT_CHECK , digit_16,SILENT)

IE_END(mobile_identity_imeisv)

/*
** mobile_station_classmark_1
** definition : GSM 04.08, 10.5.1.5 (common)
** values     : GSM 11.10, 26.6.14 (??? values of this IE : see PICS/PIXIT)
** usage      : classmark_change
** usage      : cm_service_request
*/
IE_BEGIN(mobile_station_classmark_1_GSM)

BF(1,        0,ACT_CHECK, spare,                  SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,     "   phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,             "   Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,               "   encryption algorithm A5/1 available")

BF(3,M3(0,1,1),ACT_CHECK, rf_power_capability,"   class 4")

IE_END(mobile_station_classmark_1_GSM)

IE_BEGIN(mobile_station_classmark_1_DCS)

BF(1,        0,ACT_CHECK, spare,                  SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,     "   phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,             "   Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,               "   encryption algorithm A5/1 available")

BF(3,M3(0,0,0),ACT_CHECK, rf_power_capability,"   class 1")

IE_END(mobile_station_classmark_1_DCS)

/*
** mobile_station_classmark_2
** definition : GSM 04.08, 10.5.1.6 (common)
** values     : GSM 11.10, 26.6.14 (??? values of this IE : see PICS/PIXIT)
** usage      : classmark_change
** usage      : cm_service_request
*/
IE_BEGIN(mobile_station_classmark_2_GSM)

BF(8,        3,ACT_CHECK, length,                 SILENT)

BF(1,        0,ACT_CHECK, spare,                  SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,     "   phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,             "   Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,               "   encryption algorithm A5/1 available")

BF(3,M3(0,1,1),ACT_CHECK, rf_power_capability,"   class 4")

BF(1,        0,ACT_CHECK, spare2,                 SILENT)

BF(1,        0,ACT_CHECK, ps_capability,      "   no pseudo-synch capability")

BF(2,  M2(0,1),ACT_CHECK, ss_screening_indicator,"phase 2 handling")

BF(1,        1,ACT_CHECK, sm_capability,      "   point to point SMS")

BF(1,        0,ACT_CHECK, vbs,                "   no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,               "   no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"  no extention band G1")

BF(1,        1,ACT_CHECK, class3,             "   add. MS cap. information")

BF(1,	 0,ACT_CHECK, spare,                  SILENT)

BF(1,        0,ACT_CHECK, lcsva,              "   LCS value added location")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 encoded")

BF(1,        0,ACT_CHECK, solsa,              "   SoLSA")

BF(1,        1,ACT_CHECK, cmsp,               "   CM service prompt")

BF(1,        0,ACT_CHECK, a5_3,               "   A5/3 not available")

BF(1,        1,ACT_CHECK, a5_2,               "   A5/2 available")

IE_END(mobile_station_classmark_2_GSM)

IE_BEGIN(mobile_station_classmark_2_GSM_pseudo_sync)

BF(8,        3,ACT_CHECK, length,                 SILENT)

BF(1,        0,ACT_CHECK, spare,                  SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,     "   phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,             "   Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,               "   encryption algorithm A5/1 available")

BF(3,M3(0,1,1),ACT_CHECK, rf_power_capability,"   class 4")

BF(1,        0,ACT_CHECK, spare2,                 SILENT)

BF(1,        1,ACT_CHECK, ps_capability,      "   supports pseudo-sync capability")

BF(2,  M2(0,1),ACT_CHECK, ss_screening_indicator,"phase 2 handling")

BF(1,        1,ACT_CHECK, sm_capability,      "   point to point SMS")

BF(1,        0,ACT_CHECK, vbs,                "   no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,               "   no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"  no extention band G1")

BF(1,        1,ACT_CHECK, class3,             "   add. MS cap. information")

BF(1,	 0,ACT_CHECK, spare,                  SILENT)

BF(1,        0,ACT_CHECK, lcsva,              "   LCS value added location")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 encoded")

BF(1,        0,ACT_CHECK, solsa,              "   SoLSA")

BF(1,        1,ACT_CHECK, cmsp,               "   CM service prompt")

BF(1,        0,ACT_CHECK, a5_3,               "   A5/3 not available")

BF(1,        1,ACT_CHECK, a5_2,               "   A5/2 available")

IE_END(mobile_station_classmark_2_GSM_pseudo_sync)

IE_BEGIN(mobile_station_classmark_2_EGSM)

BF(8,        3,ACT_CHECK, length,                 SILENT)

BF(1,        0,ACT_CHECK, spare,                  SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,     "   phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,             "   Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,               "   encryption algorithm A5/1 available")

BF(3,M3(0,1,1),ACT_CHECK, rf_power_capability,"   class 4")

BF(1,        0,ACT_CHECK, spare2,                 SILENT)

BF(1,        0,ACT_CHECK, ps_capability,      "   no pseudo-synch capability")

BF(2,  M2(0,1),ACT_CHECK, ss_screening_indicator,"phase 2 handling")

BF(1,        1,ACT_CHECK, sm_capability,      "   point to point SMS")

BF(1,        0,ACT_CHECK, vbs,                "   no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,               "   no VGCS cap. or no notific. wanted")

BF(1,        1,ACT_CHECK, frequency_capability,"  extention band G1")

BF(1,        1,ACT_CHECK, class3,             "   add. MS cap. information")

BF(1,	 0,ACT_CHECK, spare,                  SILENT)

BF(1,        0,ACT_CHECK, lcsva,              "   LCS value added location")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 encoded")

BF(1,        0,ACT_CHECK, solsa,              "   SoLSA")

BF(1,        1,ACT_CHECK, cmsp,               "   CM service prompt")

BF(1,        0,ACT_CHECK, a5_3,               "   A5/3 not available")

BF(1,        1,ACT_CHECK, a5_2,               "   A5/2 available")

IE_END(mobile_station_classmark_2_EGSM)

IE_BEGIN(mobile_station_classmark_2_DCS)

BF(8,        3,ACT_CHECK, length,                 SILENT)

BF(1,        0,ACT_CHECK, spare,                  SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,     "   phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,             "   Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,               "   encryption algorithm A5/1 available")

BF(3,M3(0,0,0),ACT_CHECK, rf_power_capability,"   class 1")

BF(1,        0,ACT_CHECK, spare2,                 SILENT)

BF(1,        0,ACT_CHECK, ps_capability,      "   no pseudo-synch capability")

BF(2,  M2(0,1),ACT_CHECK, ss_screening_indicator,"phase 2 handling")

BF(1,        1,ACT_CHECK, sm_capability,      "   point to point SMS")

BF(1,        0,ACT_CHECK, vbs,                "   no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,               "   no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"  no extention band G1")

BF(1,        1,ACT_CHECK, class3,             "   add. MS cap. information")

BF(1,	 0,ACT_CHECK, spare,                  SILENT)

BF(1,        0,ACT_CHECK, lcsva,              "   LCS value added location")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 encoded")

BF(1,        0,ACT_CHECK, solsa,              "   SoLSA")

BF(1,        1,ACT_CHECK, cmsp,               "   CM service prompt")

BF(1,        0,ACT_CHECK, a5_3,               "   A5/3 not available")

BF(1,        1,ACT_CHECK, a5_2,               "   A5/2 available")

IE_END(mobile_station_classmark_2_DCS)

/*
** mobile_station_classmark_3
** definition : GSM 04.08, 10.5.1.7 (common)
** usage      : classmark_change
** usage      : cm_service_request
*/
IE_BEGIN(mobile_station_classmark_3_900)

BF(8,     0x20,ACT_CHECK, iei,                "   information element identifier")

BF(8,        4,ACT_CHECK, length,                SILENT)

BF(1,        0,ACT_CHECK, e_bit,              "   no extension")

BF(1,        0,ACT_CHECK, band_3,             "   no DCS 1800 support")

BF(1,        0,ACT_CHECK, band_2,             "   no E-GSM support")

BF(1,        1,ACT_CHECK, band_1,             "   GSM 900 support")

BF(1,        0,ACT_CHECK, a5_7,               "   A5/7 not available")

BF(1,        0,ACT_CHECK, a5_6,               "   A5/6 not available")

BF(1,        0,ACT_CHECK, a5_5,               "   A5/5 not available")

BF(1,        0,ACT_CHECK, a5_4,               "   A5/4 not available")

BF(4,        0,ACT_CHECK, spare_padding,      "   no class")

BF(4,        4,ACT_CHECK, rf_power_capability_1,"   class 4 for gsm 900")

BF(1,        0,ACT_CHECK, r_support,          "   no support of R-GSM")

BF(1,        0,ACT_CHECK, multi_slot_cap,     "   no multislot capability of GSM")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 treatment")

BF(1,        0,ACT_CHECK, ext_measurement,    "   no extended measurement capability")

BF(1,        0,ACT_CHECK, measurement,        "   no switch times")

BF(1,        0,ACT_CHECK, positioning,        "   no positioning capabilities")

BF(1,        0,ACT_CHECK, edge_multislot,     "   no support of EDGE")

BF(1,        0,ACT_CHECK, edge_struct,        "   no support of EDGE")

BF(1,        0,ACT_CHECK, gsm400,             "   no support of GSM 450 or 480")

BF(1,        0,ACT_CHECK, gsm850,             "   no support of GSM 850")

BF(1,        0,ACT_CHECK, pcs1900,            "   no support of PCS 1900")

BF(1,        0,ACT_CHECK, umts_fdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, umts_tdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, cdma,               "   no support of CDMA 2000")

BF(1,        0,ACT_CHECK, dtm,                "   no support of DTM")

BF(1,        0,ACT_CHECK, single_band,        "   no support of UMTS")

IE_END(mobile_station_classmark_3_900)

IE_BEGIN(mobile_station_classmark_3_1800)

BF(8,     0x20,ACT_CHECK, iei,                "   information element identifier")

BF(8,        4,ACT_CHECK, length,                SILENT)

BF(1,        0,ACT_CHECK, e_bit,              "   no extension")

BF(1,        1,ACT_CHECK, band_3,             "   DCS 1800 support")

BF(1,        0,ACT_CHECK, band_2,             "   no E-GSM support")

BF(1,        0,ACT_CHECK, band_1,             "   no GSM 900 support")

BF(1,        0,ACT_CHECK, a5_7,               "   A5/7 not available")

BF(1,        0,ACT_CHECK, a5_6,               "   A5/6 not available")

BF(1,        0,ACT_CHECK, a5_5,               "   A5/5 not available")

BF(1,        0,ACT_CHECK, a5_4,               "   A5/4 not available")

BF(4,        0,ACT_CHECK, spare_padding,      "   no class")

BF(4,        1,ACT_CHECK, rf_power_capability_1,"   class 1 for 1800")

BF(1,        0,ACT_CHECK, r_support,          "   no support of R-GSM")

BF(1,        0,ACT_CHECK, multi_slot_cap,     "   no multislot capability of GSM")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 treatment")

BF(1,        0,ACT_CHECK, ext_measurement,    "   no extended measurement capability")

BF(1,        0,ACT_CHECK, measurement,        "   no switch times")

BF(1,        0,ACT_CHECK, positioning,        "   no positioning capabilities")

BF(1,        0,ACT_CHECK, edge_multislot,     "   no support of EDGE")

BF(1,        0,ACT_CHECK, edge_struct,        "   no support of EDGE")

BF(1,        0,ACT_CHECK, gsm400,             "   no support of GSM 450 or 480")

BF(1,        0,ACT_CHECK, gsm850,             "   no support of GSM 850")

BF(1,        0,ACT_CHECK, pcs1900,            "   no support of PCS 1900")

BF(1,        0,ACT_CHECK, umts_fdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, umts_tdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, cdma,               "   no support of CDMA 2000")

BF(1,        0,ACT_CHECK, dtm,                "   no support of DTM")

BF(1,        0,ACT_CHECK, single_band,        "   no support of UMTS")

IE_END(mobile_station_classmark_3_1800)

IE_BEGIN(mobile_station_classmark_3_DUAL)

BF(8,     0x20,ACT_CHECK, iei,                "   information element identifier")

BF(8,        4,ACT_CHECK, length,                SILENT)

BF(1,        0,ACT_CHECK, e_bit,              "   no extension")

BF(1,        1,ACT_CHECK, band_3,             "   DCS 1800 support")

BF(1,        0,ACT_CHECK, band_2,             "   no E-GSM support")

BF(1,        1,ACT_CHECK, band_1,             "   GSM 900 support")

BF(1,        0,ACT_CHECK, a5_7,               "   A5/7 not available")

BF(1,        0,ACT_CHECK, a5_6,               "   A5/6 not available")

BF(1,        0,ACT_CHECK, a5_5,               "   A5/5 not available")

BF(1,        0,ACT_CHECK, a5_4,               "   A5/4 not available")

BF(4,        1,ACT_CHECK, rf_power_capability_2,"   class 1 for dcs 1800")

BF(4,        4,ACT_CHECK, rf_power_capability_1,"   class 4 for gsm 900")

BF(1,        0,ACT_CHECK, r_support,          "   no support of R-GSM")

BF(1,        0,ACT_CHECK, multi_slot_cap,     "   no multislot capability of GSM")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 treatment")

BF(1,        0,ACT_CHECK, ext_measurement,    "   no extended measurement capability")

BF(1,        0,ACT_CHECK, measurement,        "   no switch times")

BF(1,        0,ACT_CHECK, positioning,        "   no positioning capabilities")

BF(1,        0,ACT_CHECK, edge_multislot,     "   no support of EDGE")

BF(1,        0,ACT_CHECK, edge_struct,        "   no support of EDGE")

BF(1,        0,ACT_CHECK, gsm400,             "   no support of GSM 450 or 480")

BF(1,        0,ACT_CHECK, gsm850,             "   no support of GSM 850")

BF(1,        0,ACT_CHECK, pcs1900,            "   no support of PCS 1900")

BF(1,        0,ACT_CHECK, umts_fdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, umts_tdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, cdma,               "   no support of CDMA 2000")

BF(1,        0,ACT_CHECK, dtm,                "   no support of DTM")

BF(1,        0,ACT_CHECK, single_band,        "   no support of UMTS")

IE_END(mobile_station_classmark_3_DUAL)

IE_BEGIN(mobile_station_classmark_3_DUAL_EXT)

BF(8,     0x20,ACT_CHECK, iei,                "   information element identifier")

BF(8,        4,ACT_CHECK, length,                SILENT)

BF(1,        0,ACT_CHECK, e_bit,              "   no extension")

BF(1,        1,ACT_CHECK, band_3,             "   DCS 1800 support")

BF(1,        1,ACT_CHECK, band_2,             "   E-GSM support and GSM 900 support")

BF(1,        0,ACT_CHECK, band_1,             "   (band_2 and band_3 are mutually exclusive)")

BF(1,        0,ACT_CHECK, a5_7,               "   A5/7 not available")

BF(1,        0,ACT_CHECK, a5_6,               "   A5/6 not available")

BF(1,        0,ACT_CHECK, a5_5,               "   A5/5 not available")

BF(1,        0,ACT_CHECK, a5_4,               "   A5/4 not available")

BF(4,        1,ACT_CHECK, rf_power_capability_2,"   class 1 for dcs 1800")

BF(4,        4,ACT_CHECK, rf_power_capability_1,"   class 4 for gsm 900")

BF(1,        0,ACT_CHECK, r_support,          "   no support of R-GSM")

BF(1,        0,ACT_CHECK, multi_slot_cap,     "   no multislot capability of GSM")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 treatment")

BF(1,        0,ACT_CHECK, ext_measurement,    "   no extended measurement capability")

BF(1,        0,ACT_CHECK, measurement,        "   no switch times")

BF(1,        0,ACT_CHECK, positioning,        "   no positioning capabilities")

BF(1,        0,ACT_CHECK, edge_multislot,     "   no support of EDGE")

BF(1,        0,ACT_CHECK, edge_struct,        "   no support of EDGE")

BF(1,        0,ACT_CHECK, gsm400,             "   no support of GSM 450 or 480")

BF(1,        0,ACT_CHECK, gsm850,             "   no support of GSM 850")

BF(1,        0,ACT_CHECK, pcs1900,            "   no support of PCS 1900")

BF(1,        0,ACT_CHECK, umts_fdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, umts_tdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, cdma,               "   no support of CDMA 2000")

BF(1,        0,ACT_CHECK, dtm,                "   no support of DTM")

BF(1,        0,ACT_CHECK, single_band,        "   no support of UMTS")

IE_END(mobile_station_classmark_3_DUAL_EXT)

IE_BEGIN(mobile_station_classmark_3_1900)

BF(8,     0x20,ACT_CHECK, iei,                "   information element identifier")

BF(8,        4,ACT_CHECK, length,                SILENT)

BF(1,        0,ACT_CHECK, e_bit,              "   no extension")

BF(1,        0,ACT_CHECK, band_3,             "   no DCS 1800 support")

BF(1,        0,ACT_CHECK, band_2,             "   no E-GSM support")

BF(1,        0,ACT_CHECK, band_1,             "   no GSM 900 support")

BF(1,        0,ACT_CHECK, a5_7,               "   A5/7 not available")

BF(1,        0,ACT_CHECK, a5_6,               "   A5/6 not available")

BF(1,        0,ACT_CHECK, a5_5,               "   A5/5 not available")

BF(1,        0,ACT_CHECK, a5_4,               "   A5/4 not available")

BF(1,        0,ACT_CHECK, r_support,          "   no support of R-GSM")

BF(1,        0,ACT_CHECK, multi_slot_cap,     "   no multislot capability of GSM")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 treatment")

BF(1,        0,ACT_CHECK, ext_measurement,    "   no extended measurement capability")

BF(1,        0,ACT_CHECK, measurement,        "   no switch times")

BF(1,        0,ACT_CHECK, positioning,        "   no positioning capabilities")

BF(1,        0,ACT_CHECK, edge_multislot,     "   no support of EDGE")

BF(1,        0,ACT_CHECK, edge_struct,        "   no support of EDGE")

BF(1,        0,ACT_CHECK, gsm400,             "   no support of GSM 450 or 480")

BF(1,        0,ACT_CHECK, gsm850,             "   no support of GSM 850")

BF(1,        1,ACT_CHECK, pcs1900,            "   support of PCS 1900")

BF(4,        1,ACT_CHECK, pcs1900_cap,        "   class 1 for 1900")

BF(1,        0,ACT_CHECK, umts_fdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, umts_tdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, cdma,               "   no support of CDMA 2000")

BF(1,        0,ACT_CHECK, dtm,                "   no support of DTM")

BF(1,        0,ACT_CHECK, single_band,        "   no support of UMTS")

IE_END(mobile_station_classmark_3_1900)

IE_BEGIN(mobile_station_classmark_3_850)

BF(8,     0x20,ACT_CHECK, iei,                "   information element identifier")

BF(8,        4,ACT_CHECK, length,                SILENT)

BF(1,        0,ACT_CHECK, e_bit,              "   no extension")

BF(1,        0,ACT_CHECK, band_3,             "   no DCS 1800 support")

BF(1,        0,ACT_CHECK, band_2,             "   no E-GSM support")

BF(1,        0,ACT_CHECK, band_1,             "   no GSM 900 support")

BF(1,        0,ACT_CHECK, a5_7,               "   A5/7 not available")

BF(1,        0,ACT_CHECK, a5_6,               "   A5/6 not available")

BF(1,        0,ACT_CHECK, a5_5,               "   A5/5 not available")

BF(1,        0,ACT_CHECK, a5_4,               "   A5/4 not available")

BF(1,        0,ACT_CHECK, r_support,          "   no support of R-GSM")

BF(1,        0,ACT_CHECK, multi_slot_cap,     "   no multislot capability of GSM")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 treatment")

BF(1,        0,ACT_CHECK, ext_measurement,    "   no extended measurement capability")

BF(1,        0,ACT_CHECK, measurement,        "   no switch times")

BF(1,        0,ACT_CHECK, positioning,        "   no positioning capabilities")

BF(1,        0,ACT_CHECK, edge_multislot,     "   no support of EDGE")

BF(1,        0,ACT_CHECK, edge_struct,        "   no support of EDGE")

BF(1,        0,ACT_CHECK, gsm400,             "   no support of GSM 450 or 480")

BF(1,        1,ACT_CHECK, gsm850,             "   support of GSM 850")

BF(4,        4,ACT_CHECK, gsm850_cap,         "   class 4 for 850")

BF(1,        0,ACT_CHECK, pcs1900,            "   no support of PCS 1900")

BF(1,        0,ACT_CHECK, umts_fdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, umts_tdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, cdma,               "   no support of CDMA 2000")

BF(1,        0,ACT_CHECK, dtm,                "   no support of DTM")

BF(1,        0,ACT_CHECK, single_band,        "   no support of UMTS")

IE_END(mobile_station_classmark_3_850)

IE_BEGIN(mobile_station_classmark_3_DUAL_US)

BF(8,     0x20,ACT_CHECK, iei,                "   information element identifier")

BF(8,        4,ACT_CHECK, length,                SILENT)

BF(1,        0,ACT_CHECK, e_bit,              "   no extension")

BF(1,        0,ACT_CHECK, band_3,             "   no DCS 1800 support")

BF(1,        0,ACT_CHECK, band_2,             "   no E-GSM support")

BF(1,        0,ACT_CHECK, band_1,             "   no GSM 900 support")

BF(1,        0,ACT_CHECK, a5_7,               "   A5/7 not available")

BF(1,        0,ACT_CHECK, a5_6,               "   A5/6 not available")

BF(1,        0,ACT_CHECK, a5_5,               "   A5/5 not available")

BF(1,        0,ACT_CHECK, a5_4,               "   A5/4 not available")

BF(1,        0,ACT_CHECK, r_support,          "   no support of R-GSM")

BF(1,        0,ACT_CHECK, multi_slot_cap,     "   no multislot capability of GSM")

BF(1,        1,ACT_CHECK, ucs2_treat,         "   UCS2 treatment")

BF(1,        0,ACT_CHECK, ext_measurement,    "   no extended measurement capability")

BF(1,        0,ACT_CHECK, measurement,        "   no switch times")

BF(1,        0,ACT_CHECK, positioning,        "   no positioning capabilities")

BF(1,        0,ACT_CHECK, edge_multislot,     "   no support of EDGE")

BF(1,        0,ACT_CHECK, edge_struct,        "   no support of EDGE")

BF(1,        0,ACT_CHECK, gsm400,             "   no support of GSM 450 or 480")

BF(1,        1,ACT_CHECK, gsm850,             "   support of GSM 850")

BF(4,        4,ACT_CHECK, gsm850_cap,         "   class 4 for 850")

BF(1,        1,ACT_CHECK, pcs1900,            "   support of PCS 1900")

BF(4,        1,ACT_CHECK, pcs1900_cap,        "   class 1 for 1900")

BF(1,        0,ACT_CHECK, umts_fdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, umts_tdd,           "   no support of UMTS")

BF(1,        0,ACT_CHECK, cdma,               "   no support of CDMA 2000")

BF(1,        0,ACT_CHECK, dtm,                "   no support of DTM")

BF(1,        0,ACT_CHECK, single_band,        "   no support of UMTS")

IE_END(mobile_station_classmark_3_DUAL_US)

/*
** mode_of_the_first_channel
** definition : GSM 04.08, 10.5.2.6 (RR)
** usage      : MRR135, MRR203
*/
IE_BEGIN(mode_of_the_first_channel)

BF(8,1,ACT_CHECK,mode,"Speech full rate")

IE_END(mode_of_the_first_channel)

/*
** p1_rest_octets
** definition : GSM 04.08, 10.5.2.23 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : paging_request_type_1
*/
IE_BEGIN(p1_rest_octets)
                 /* pag. req. type1 : 22 - 9 (L2 pseud. len) = 13 bytes */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 11 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 12 */
IE_END(p1_rest_octets)

IE_BEGIN(p1_rest_octets_11)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */
IE_END(p1_rest_octets_11)

IE_BEGIN(p1_rest_octets_fill)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 11 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 12 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 13 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 14 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 15 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 16 */
IE_END(p1_rest_octets_fill)

/*
** p2_rest_octets
** definition : GSM 04.08, 10.5.2.24 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : paging_request_type_2
*/
IE_BEGIN(p2_rest_octets)
                 /* pag. req. type2 : 22 - 11  (L2 pseud. len) = 11 bytes */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */
IE_END(p2_rest_octets)

IE_BEGIN(p2_rest_octets_2)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
IE_END(p2_rest_octets_2)

/*
** p3_rest_octets
** definition : GSM 04.08, 10.5.2.25 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : paging_request_type_3
*/
IE_BEGIN(p3_rest_octets)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */
IE_END(p3_rest_octets)

/*
** page_mode
** definition : GSM 04.08, 10.5.2.26 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment
** usage      : immediate_assignment_extended
** usage      : immediate_assignment_reject
** usage      : paging_request_type_1
** usage      : paging_request_type_2
** usage      : paging_request_type_3
*/
IE_BEGIN(page_mode)

BF(2,0,ACT_CHECK,spare,"two spare bits")

BF(2,0,ACT_CHECK,pm   ,"Normal Paging")

IE_END(page_mode)

/*
** power_command
** definition : GSM 04.08, 10.5.2.28 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : assignment_command
** usage      : handover_command
*/
IE_BEGIN(power_command)

BF(3,                   0,ACT_CHECK,spare,SILENT)

BF(5,DOWNLINK_INPUT_LEVEL,ACT_CHECK,power,SILENT) /* ??? is value OK ? */

IE_END(power_command)

IE_BEGIN(power_command_28)

BF(3,0,ACT_CHECK,spare,SILENT)

BF(5,23,ACT_CHECK,power,SILENT)

IE_END(power_command_28)

/*
** rach
** definition : GSM 04.08, 9.1.8
** values     : GSM 11.10, 26.6.14
** usage      : assignment_complete
** usage      : channel_request
*/
IE_BEGIN(rach)
   BF(3,M3(1,0,0),ACT_CHECK ,establishment_cause,"paging ind. any channel")
   BF(5,        0,ACT_SHOW  ,ref                ,"not checked")
IE_END(rach)

/*
** request_reference
** definition : GSM 04.08, 10.5.2.30
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment
*/
IE_BEGIN(request_reference)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,"As in CHAN REQ")

BF(5,M5(1,1,1,1,1),ACT_SHOW,ref,SILENT)

BF(5,            0,ACT_SHOW,t1_,SILENT)

BF(6,            0,ACT_SHOW,t3 ,SILENT)

BF(5,            0,ACT_SHOW,t2 ,SILENT)

IE_END(request_reference)

/*
** request_reference_2
** definition : GSM 04.08, 10.5.2.30
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_extended
** usage      : immediate_assignment_reject
*/
IE_BEGIN(request_reference_2)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,SILENT)

BF(5,M5(1,1,1,1,1),ACT_SHOW,ref,"as in non of the CHAN REQ sent by the MS!")

BF(5,            0,ACT_SHOW,t1_,SILENT)

BF(6,            0,ACT_SHOW,t3 ,SILENT)

BF(5,            0,ACT_SHOW,t2 ,SILENT)

IE_END(request_reference_2)

/*
** request_reference_3
** definition : GSM 04.08, 10.5.2.30
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_reject
*/
IE_BEGIN(request_reference_3)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,SILENT)

BF(5,M5(1,1,1,1,1),ACT_SHOW,ref,SILENT)

BF(5,            0,ACT_SHOW,t1_,SILENT)

BF(6,            0,ACT_SHOW,t3 ,SILENT)

BF(5,            0,ACT_SHOW,t2 ,SILENT)

IE_END(request_reference_3)

/*
** request_reference_4
** definition : GSM 04.08, 10.5.2.30
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_reject
*/
IE_BEGIN(request_reference_4)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,SILENT)

BF(5,M5(1,1,1,1,1),ACT_SHOW,ref,SILENT)

BF(5,            0,ACT_SHOW,t1_,SILENT)

BF(6,            0,ACT_SHOW,t3 ,SILENT)

BF(5,            0,ACT_SHOW,t2 ,SILENT)

IE_END(request_reference_4)

/*
** rr_cause
** definition : GSM 04.08, 10.5.2.31 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : assignment_complete
*/
IE_BEGIN(rr_cause)

BF(8,M8(0,0,0,0,0,0,0,0),ACT_CHECK,cause,"normal event")

IE_END(rr_cause)

IE_BEGIN(rr_cause_abnormal_release)

BF(8,M8(0,0,0,0,0,0,0,1),ACT_CHECK,cause,"abnormal release")

IE_END(rr_cause_abnormal_release)

IE_BEGIN(rr_cause_incorrect_msg)

BF(8,M8(0,1,0,1,1,1,1,1),ACT_CHECK,cause," Semantically incorrect message")

IE_END(rr_cause_incorrect_msg)

/*
** rr_cause_failure
** definition : GSM 04.08, 10.5.2.31 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : assignment_failure
*/
IE_BEGIN(rr_cause_failure)

BF(8,M8(0,0,0,0,0,0,0,1),ACT_CHECK,cause,"depending on test")

IE_END(rr_cause_failure)

/*
** timing_advance
** definition : GSM 04.08, 10.5.2.40 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment
*/
IE_BEGIN(timing_advance)

BF(2, 0,ACT_CHECK,         spare,SILENT)

BF(6,30,ACT_CHECK,timing_advance,"30 bit periods")

IE_END(timing_advance)

/*
** timing_advance_20
** definition : GSM 04.08, 10.5.2.40 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : physical_information
*/
IE_BEGIN(timing_advance_20)

BF(2, 0,ACT_CHECK,         spare,SILENT)

BF(6,20,ACT_CHECK,timing_advance,"20 bit periods")

IE_END(timing_advance_20)

/*
** timing_advance_0
** definition : GSM 04.08, 10.5.2.40 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_extended
*/
IE_BEGIN(timing_advance_0)

BF(2,0,ACT_CHECK,         spare,SILENT)

BF(6,0,ACT_CHECK,timing_advance,"chosen arbitrarily by the test house")

IE_END(timing_advance_0)

/*
** transaction_identifier_dest
** definition : GSM 04.08, 10.3.2 (very common)
** values     : GSM 11.10, 26.6.14
** usage      : alerting
** usage      : connect
*/
IE_BEGIN(transaction_identifier_dest)

BF(1,    M1(1),ACT_CHECK,ti_flag ,"1 (destination side)")

BF(3,M3(0,0,0),ACT_CHECK,ti_value,"as used in the setup message")

IE_END(transaction_identifier_dest)

/*
** transaction_identifier_source
** definition : GSM 04.08, 10.3.2 (very common)
** values     : GSM 11.10, 26.6.14
** usage      : connect_acknowledge
*/
IE_BEGIN(transaction_identifier_source)

BF(1,    M1(0),ACT_CHECK,ti_flag ,"0 (source side)")

BF(3,M3(0,0,0),ACT_CHECK,ti_value,"as used in the setup message")

IE_END(transaction_identifier_source)

/*
** wait_indication
** definition : GSM 04.08, 10.5.2.43 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_reject
*/
IE_BEGIN(wait_indication)
   BF(8,0,ACT_CHECK,t3122,"0 seconds")
IE_END(wait_indication)
/*
** wait_indication_2
** definition : GSM 04.08, 10.5.2.43 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_reject
*/
IE_BEGIN(wait_indication_2)
   BF(8,0,ACT_CHECK,t3122,"0 seconds")
IE_END(wait_indication_2)
/*
** wait_indication_3
** definition : GSM 04.08, 10.5.2.43 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_reject
*/
IE_BEGIN(wait_indication_3)
   BF(8,0,ACT_CHECK,t3122,"0 seconds")
IE_END(wait_indication_3)
/*
** wait_indication_4
** definition : GSM 04.08, 10.5.2.43 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : immediate_assignment_reject
*/
IE_BEGIN(wait_indication_4)
   BF(8,0,ACT_CHECK,t3122,"0 seconds")
IE_END(wait_indication_4)
IE_BEGIN(channels_needed_for_mobiles_1_and_2)
   BF(2,0,ACT_CHECK,second_channel,"spare, any channel")
   BF(2,0,ACT_CHECK, first_channel,"spare, any channel")
IE_END(channels_needed_for_mobiles_1_and_2)

IE_BEGIN(cc_capabilities)

BF(8,             1,ACT_CHECK,                length,SILENT)

BF(6,             0,ACT_CHECK,                 spare,SILENT)

BF(1,             1,ACT_CHECK,                   pcp,"same value as classmark2:cmsp")

BF(1,             1,ACT_CHECK,                  dtmf,"MS supports DTMF")

IE_END(cc_capabilities)

IE_BEGIN(description_of_the_first_channel_after_time)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            3,ACT_CHECK,      time_slot_number,"three")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(description_of_the_first_channel_after_time)

IE_BEGIN(iei_5E)

BF(8,0x5E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_5E)

IE_BEGIN(iei_15)

BF(8,0x15,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_15)

IE_BEGIN(iei_34)

BF(8,0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_34)

IE_BEGIN(mobile_identity)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_4,SILENT)

BF(4,          0,ACT_CHECK, digit_3,SILENT)

BF(4,          7,ACT_CHECK, digit_6,SILENT)

BF(4,          4,ACT_CHECK, digit_5,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          1,ACT_CHECK, digit_7,SILENT)

BF(4,          9,ACT_CHECK, digit_10,SILENT)

BF(4,          4,ACT_CHECK, digit_9,SILENT)

BF(4,          2,ACT_CHECK, digit_12,SILENT)

BF(4,          1,ACT_CHECK, digit_11,SILENT)

IE_END(mobile_identity)

IE_BEGIN(ms_classmark)

BF(8,        3,ACT_CHECK, length,SILENT)

BF(1,        0,ACT_CHECK, spare,SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,"phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,"Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,"encryption algorithm A5/1 available")

BF(3,M3(0,0,1),ACT_CHECK, rf_power_capability,"class 2")

BF(1,        0,ACT_CHECK, spare2,SILENT)

BF(1,        0,ACT_CHECK, ps_capability,"no pseudo-synch capability")

BF(2,  M2(0,0),ACT_CHECK, ss_screening_indicator,"default phase 1")

BF(1,        0,ACT_CHECK, sm_capability,"no point to point SMS")

BF(1,        0,ACT_CHECK, vbs,"no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,"no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"no extention band G1")

BF(1,        0,ACT_CHECK, classmark_3,"no add. MS cap. information")

BF(5,        1,ACT_CHECK, ccbs,SILENT)

BF(1,        0,ACT_CHECK, a5_3,"A5/3 not available")

BF(1,        0,ACT_CHECK, a5_2,"A5/2 not available")

IE_END(ms_classmark)

IE_BEGIN(neighbour_cell_description_26_6_3_1_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_032_001,"no neigbours")
IE_END(neighbour_cell_description_26_6_3_1_3)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

IE_BEGIN(signal_call_waiting)

BF(8,M8(0,0,0,0,0,1,1,1),ACT_CHECK,signal_value,"(Any non-res. value)")

IE_END(signal_call_waiting)

/*------------------------------------------------------------------*\
| Messages
\*------------------------------------------------------------------*/

/*
** alerting (SS to MS)
** definition : GSM 04.08, 9.3.1.1 (CC)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(alerting)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(alerting_message_type)

MSG3_END(alerting)

/*
** assignment_command
** definition : GSM 04.08, 9.1.2 (RR) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(assignment_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_GSM)

IE(power_command)

MSG3_END(assignment_command)

MSG3_BEGIN(assignment_command_no_ciph)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_GSM)

IE(power_command)

IE(iei_09)

IE(cipher_mode_setting_none)

MSG3_END(assignment_command_no_ciph)

MSG3_BEGIN(assignment_command_ciph)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_GSM)

IE(power_command)

IE(iei_09)

IE(cipher_mode_setting_a5_1)

MSG3_END(assignment_command_ciph)

MSG3_BEGIN(assignment_command_ciph_a2)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_GSM)

IE(power_command)

IE(iei_09)

IE(cipher_mode_setting_a5_2)

MSG3_END(assignment_command_ciph_a2)

MSG3_BEGIN(assignment_command_19)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(assignment_command_19)

MSG3_BEGIN(assignment_command_28)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_HOP_TCHF4)

IE(power_command_28)

IE(iei_72)

IE(mobile_allocation_28)

MSG3_END(assignment_command_28)

MSG3_BEGIN(assignment_command_28_H)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_HOP_TCHH4)

IE(power_command_28)

IE(iei_72)

IE(mobile_allocation_28)

MSG3_END(assignment_command_28_H)

MSG3_BEGIN(assignment_command_300)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_HOP_TCHF1)

IE(power_command)

IE(iei_62)

IE(cell_channel_description_300)

IE(iei_63)

IE(mode_of_the_first_channel)

IE(iei_72)

IE(mobile_allocation_300)

MSG3_END(assignment_command_300)

MSG3_BEGIN(assignment_command_1800_H)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_HOP_TCHF6)

IE(power_command_28)

IE(iei_05)

IE(frequency_list_1800_H)

MSG3_END(assignment_command_1800_H)

/*
** assignment_complete
** definition : GSM 04.08, 9.1.3 (RR) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(assignment_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_complete_message_type)

IE(rr_cause)

MSG3_END(assignment_complete)

/*
** assignment_failure
** definition : GSM 04.08, 9.1.3 (RR) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(assignment_failure)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_failure_message_type)

IE(rr_cause_failure)

MSG3_END(assignment_failure)

/*
** authentication_request
** definition : GSM 04.08, 9.2.2 (MM) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(authentication_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(authentication_parameter_rand)

MSG3_END(authentication_request)

/*
** authentication_response
** definition : GSM 04.08, 9.2.3 (MM) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(authentication_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_response_message_type)

IE(authentication_parameter_sres)

MSG3_END(authentication_response)

/*
** call_proceeding
** definition : GSM 04.08, 9.2.3 (CC)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(call_proceeding)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(call_proceeding_message_type)

MSG3_END(call_proceeding)

/*
** channel_mode_modify
** definition : GSM 04.08, 9.1.5 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(channel_mode_modify)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_mode_modify_message_type)

IE(channel_description_GSM)

IE(channel_mode)

MSG3_END(channel_mode_modify)

/*
** channel_mode_modify_acknowledge
** definition : GSM 04.08, 9.1.6 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(channel_mode_modify_acknowledge)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_mode_modify_acknowledge_message_type)

IE(channel_description_GSM) /* should equal to what have been sent in channel_mode_modify */

IE(channel_mode)        /* should equal to what have been sent in channel_mode_modify */

MSG3_END(channel_mode_modify_acknowledge)

/*
** channel_release
** definition : GSM 04.08, 9.1.7 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(channel_release)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_release_message_type)

IE(rr_cause)

MSG3_END(channel_release)

/*
** channel_request
** definition : GSM 04.08, 9.1.8 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(channel_request)
   IE(rach)
MSG3_END(channel_request)

/*
** ciphering_mode_command
** definition : GSM 04.08, 9.1.9 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(ciphering_mode_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(cipher_response)

IE(ciphering_mode_setting)

MSG3_END(ciphering_mode_command)

MSG3_BEGIN(ciphering_mode_command_no_ciph)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(cipher_response)

IE(cipher_mode_setting_none)

MSG3_END(ciphering_mode_command_no_ciph)

/*
** ciphering_mode_complete
** definition : GSM 04.08, 9.1.10 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(ciphering_mode_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

MSG3_END(ciphering_mode_complete)

/*
** classmark_change
** definition : GSM 04.08, 9.1.11 (RR)
** values     : GSM 11.10, 26.6.14 (??? classmark_3 may be nec. if required by PICS/PIXIT)
** usage      : mrr*
*/
MSG3_BEGIN(classmark_change)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_change_message_type)

IE(mobile_station_classmark_2_GSM)

IE(mobile_station_classmark_3_900)

MSG3_END(classmark_change)

MSG3_BEGIN(classmark_change_1800)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_change_message_type)

IE(mobile_station_classmark_2_DCS)

IE(mobile_station_classmark_3_1800)

MSG3_END(classmark_change_1800)

MSG3_BEGIN(classmark_change_GSM_DUAL)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_change_message_type)

IE(mobile_station_classmark_2_GSM)

IE(mobile_station_classmark_3_DUAL)

MSG3_END(classmark_change_GSM_DUAL)

MSG3_BEGIN(classmark_change_GSM_DUAL_EXT)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_change_message_type)

IE(mobile_station_classmark_2_EGSM)

IE(mobile_station_classmark_3_DUAL_EXT)

MSG3_END(classmark_change_GSM_DUAL_EXT)

MSG3_BEGIN(classmark_change_DCS_DUAL)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_change_message_type)

IE(mobile_station_classmark_2_DCS)

IE(mobile_station_classmark_3_DUAL)

MSG3_END(classmark_change_DCS_DUAL)

MSG3_BEGIN(classmark_change_1900)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_change_message_type)

IE(mobile_station_classmark_2_DCS)

IE(mobile_station_classmark_3_1900)

MSG3_END(classmark_change_1900)

MSG3_BEGIN(classmark_change_850)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_change_message_type)

IE(mobile_station_classmark_2_GSM)

IE(mobile_station_classmark_3_850)

MSG3_END(classmark_change_850)

MSG3_BEGIN(classmark_change_DUAL_US)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_change_message_type)

IE(mobile_station_classmark_2_DCS)

IE(mobile_station_classmark_3_DUAL_US)

MSG3_END(classmark_change_DUAL_US)

/*
** cm_service_accept
** definition : GSM 04.08, 9.2.5 (MM) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(cm_service_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_accept_message_type)

MSG3_END(cm_service_accept)

/*
** cm_service_request
** definition : GSM 04.08, 9.2.9 (MM) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(cm_service_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_request_message_type)

IE(ciphering_key_sequence_number)

IE(cm_service_type)

IE(mobile_station_classmark_2_GSM)

IE(mobile_identity_imsi)

MSG3_END(cm_service_request)

MSG3_BEGIN(cm_service_request_pseudo_sync)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_request_message_type)

IE(ciphering_key_sequence_number)

IE(cm_service_type)

IE(mobile_station_classmark_2_GSM_pseudo_sync)

IE(mobile_identity_imsi)

MSG3_END(cm_service_request_pseudo_sync)

/*
** connect (SS to MS)
** definition : GSM 04.08, 9.3.5.1 (CC)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(connect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(connect_message_type)

MSG3_END(connect)

/*
** connect_acknowledge
** definition : GSM 04.08, 9.3.6 (CC)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(connect_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(connect_acknowledge_message_type)

MSG3_END(connect_acknowledge)

/*
** handover_access
** definition : GSM 04.08, 9.1.14 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(handover_access)

IE(handover_reference)

MSG3_END(handover_access)

/*
** handover_command
** definition : GSM 04.08, 9.1.15 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_GSM)

IE(handover_reference)

IE(power_command)

MSG3_END(handover_command)

MSG3_BEGIN(handover_command_ciph)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description_20)

IE(channel_description_GSM)

IE(handover_reference)

IE(power_command)

IE(iei_09)

IE(cipher_mode_setting_a5_1)

MSG3_END(handover_command_ciph)

MSG3_BEGIN(handover_command_ciph_a2)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description_20)

IE(channel_description_GSM)

IE(handover_reference)

IE(power_command)

IE(iei_09)

IE(cipher_mode_setting_a5_2)

MSG3_END(handover_command_ciph_a2)

MSG3_BEGIN(handover_command_no_ciph)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description_20)

IE(channel_description_GSM)

IE(handover_reference)

IE(power_command)

IE(iei_09)

IE(cipher_mode_setting_none)

MSG3_END(handover_command_no_ciph)

/*
** handover_complete
** definition : GSM 04.08, 9.1.16 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(handover_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_complete_message_type)

IE(rr_cause)

MSG3_END(handover_complete)

/*
** handover_failure
** definition : GSM 04.08, 9.1.17 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(handover_failure_incorrect_msg)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_failure_message_type)

IE(rr_cause_incorrect_msg)

MSG3_END(handover_failure_incorrect_msg)

/*
** handover_failure
** definition : GSM 04.08, 9.1.17 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(handover_failure_abnormal_release)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_failure_message_type)

IE(rr_cause_abnormal_release)

MSG3_END(handover_failure_abnormal_release)

/*
** immediate_assignment
** definition : GSM 04.08, 9.1.18 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(immediate_assignment_GSM)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_GSM)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment_GSM)

MSG3_BEGIN(immediate_assignment_DCS)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_DCS)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment_DCS)

MSG3_BEGIN(immediate_assignment_EGSM)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_EGSM)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment_EGSM)

MSG3_BEGIN(immediate_assignment_850)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_850)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment_850)

MSG3_BEGIN(immediate_assignment_HOP_SDCCH8)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_HOP_SDCCH8)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation_E)

IE(ia_rest_octets)

MSG3_END(immediate_assignment_HOP_SDCCH8)

/*
** immediate_assignment_extended
** definition : GSM 04.08, 9.1.19 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(immediate_assignment_extended)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_extended_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_GSM)

IE(request_reference)

IE(timing_advance_0)

IE(channel_description_GSM) /* not filled out yet !!! */

IE(request_reference_2)

IE(timing_advance_0)

IE(mobile_allocation)

IE(iax_rest_octets)

MSG3_END(immediate_assignment_extended)

/*
** immediate_assignment_reject
** definition : GSM 04.08, 9.1.19 (RR)
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(immediate_assignment_reject)
   IE(l2_pseudo_length_19)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(immediate_assignment_reject_message_type)
   IE(page_mode)
   IE(spare_half_octet)
   IE(request_reference)
   IE(wait_indication)
   IE(request_reference_2)
   IE(wait_indication_2)
   IE(request_reference_3)
   IE(wait_indication_3)
   IE(request_reference_4)
   IE(wait_indication_4)
   IE(iar_rest_octets)
MSG3_END(immediate_assignment_reject)

/*
** location_updating_request
** definition : GSM 04.08, 9.2.15 (MM) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(location_updating_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_request_message_type)
   IE(ciphering_key_sequence_number)

IE(location_updating_type)

IE(location_area_identification)

IE(mobile_station_classmark_1_GSM)

IE(mobile_identity_imsi)

MSG3_END(location_updating_request)

/*
** paging_request_type_1
** definition : GSM 04.08, 9.1.22 (RR) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(paging_request_type_1)
   IE(l2_pseudo_length_9)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_1_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(mobile_identity_imsi)
   IE(p1_rest_octets)
MSG3_END(paging_request_type_1)

MSG3_BEGIN(paging_request_type_1_imsi_none)
   IE(l2_pseudo_length_9)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_1_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(mobile_identity_imsi)
   IE(p1_rest_octets_11)
MSG3_END(paging_request_type_1_imsi_none)

MSG3_BEGIN(paging_request_type_1_fill)
   IE(l2_pseudo_length_9)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_1_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(mobile_identity_fill)
   IE(p1_rest_octets_fill)
MSG3_END(paging_request_type_1_fill)

/*
** paging_request_type_2
** definition : GSM 04.08, 9.1.23 (RR) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(paging_request_type_2)
   IE(l2_pseudo_length_11)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_2_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi)
   IE(tmsi_2)
   IE(p2_rest_octets)
MSG3_END(paging_request_type_2)

MSG3_BEGIN(paging_request_type_2_other_other_imsi)
   IE(l2_pseudo_length_11)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_2_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(iei_17)
   IE(mobile_identity_imsi)
   IE(p2_rest_octets_2)
MSG3_END(paging_request_type_2_other_other_imsi)

/*
** paging_request_type_3
** definition : GSM 04.08, 9.1.24 (RR) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(paging_request_type_3)
   IE(l2_pseudo_length_19)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_3_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(p3_rest_octets)
MSG3_END(paging_request_type_3)

/*
** paging_response
** definition : GSM 04.08, 9.1.25 (RR) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(paging_response_imsi)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_response_message_type)
   IE(spare_half_octet)
   IE(ciphering_key_sequence_number)
   IE(mobile_station_classmark_2_GSM)
   IE(mobile_identity_imsi)
MSG3_END(paging_response_imsi)

MSG3_BEGIN(paging_response_imsi_egsm)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_response_message_type)
   IE(spare_half_octet)
   IE(ciphering_key_sequence_number)
   IE(mobile_station_classmark_2_EGSM)
   IE(mobile_identity_imsi)
MSG3_END(paging_response_imsi_egsm)

MSG3_BEGIN(paging_response_imsi_1800)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_response_message_type)
   IE(spare_half_octet)
   IE(ciphering_key_sequence_number)
   IE(mobile_station_classmark_2_DCS)
   IE(mobile_identity_imsi)
MSG3_END(paging_response_imsi_1800)

MSG3_BEGIN(paging_response_imsi_900_dual)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_response_message_type)
   IE(spare_half_octet)
   IE(ciphering_key_sequence_number)
   IE(mobile_station_classmark_2_GSM)
   IE(mobile_identity_imsi)
MSG3_END(paging_response_imsi_900_dual)

MSG3_BEGIN(paging_response_imsi_dual)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_response_message_type)
   IE(spare_half_octet)
   IE(ciphering_key_sequence_number)
   IE(mobile_station_classmark_2_DCS)
   IE(mobile_identity_imsi)
MSG3_END(paging_response_imsi_dual)

MSG3_BEGIN(paging_response_tmsi)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_response_message_type)
   IE(spare_half_octet)
   IE(ciphering_key_sequence_number)
   IE(mobile_station_classmark_2_GSM)
   IE(mobile_identity_tmsi)
MSG3_END(paging_response_tmsi)

/*
** physical_information
** definition : GSM 04.08, 9.1.25 (RR) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(physical_information)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(physical_information_message_type)

IE(timing_advance_20)
MSG3_END(physical_information)

/*
** setup (MS to SS)
** definition : GSM 04.08, 9.3.23.1 (CC) 
** values     : GSM 11.10, 26.6.14
** usage      : mrr*
*/
MSG3_BEGIN(setup)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_04)

IE(bearer_capability)

IE(iei_5e)

IE(called_party_bcd_number)

IE(iei_15)

IE(cc_capabilities)

MSG3_END(setup)

MSG3_BEGIN(setup_19)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

MSG3_END(setup_19)

MSG3_BEGIN(call_confirmed)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(call_confirmed_message_type)

IE(iei_04)

IE(bearer_capability)

MSG3_END(call_confirmed)

MSG3_BEGIN(call_confirmed_1800_DUAL)           /* contains bearer capability */

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(call_confirmed_message_type)

IE(iei_04)

IE(bearer_capability_1800_DUAL)

MSG3_END(call_confirmed_1800_DUAL)

/* ----- Extended GSM ------- */

MSG3_BEGIN(system_information_type_1_E_S1)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description_SDCCH8_HOP_S1)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_E_S1)

MSG3_BEGIN(system_information_type_1_E_N1)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_E_N1)

MSG3_BEGIN(system_information_type_1_E_N2)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_E_N2)

MSG3_BEGIN(system_information_type_1_E_N3)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_E_N3)

MSG3_BEGIN(system_information_type_1_E_N4)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_E_N4)

MSG3_BEGIN(system_information_type_1_E_N5)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_E_N5)

MSG3_BEGIN(system_information_type_1_E_N6)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_E_N6)

MSG3_BEGIN(system_information_type_1_E_N7)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_E_N7)

MSG3_BEGIN(system_information_type_2_E_S1)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description_E)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2_E_S1)

MSG3_BEGIN(system_information_type_2_E_N1)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description_E_N1)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2_E_N1)

MSG3_BEGIN(system_information_type_2_E_N2)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description_E_N2)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2_E_N2)

MSG3_BEGIN(system_information_type_2_E_N3)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description_E_N3)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2_E_N3)

MSG3_BEGIN(system_information_type_2_E_N4)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description_E_N4)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2_E_N4)

MSG3_BEGIN(system_information_type_2_E_N5)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description_E_N5)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2_E_N5)

MSG3_BEGIN(system_information_type_2_E_N6)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description_E_N6)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2_E_N6)

MSG3_BEGIN(system_information_type_2_E_N7)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description_E_N7)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2_E_N7)

MSG3_BEGIN(system_information_type_3_E_S1)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_1)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_E_S1)

MSG3_BEGIN(system_information_type_3_E_N1)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_2)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_E_N1)

MSG3_BEGIN(system_information_type_3_E_N2)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_3)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_E_N2)

MSG3_BEGIN(system_information_type_3_E_N3)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_4)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_E_N3)

MSG3_BEGIN(system_information_type_3_E_N4)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_5)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_E_N4)

MSG3_BEGIN(system_information_type_3_E_N5)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_6)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_E_N5)

MSG3_BEGIN(system_information_type_3_E_N6)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_6)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_E_N6)

MSG3_BEGIN(system_information_type_3_E_N7)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity_8)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options_bcch)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3_E_N7)

MSG3_BEGIN(system_information_type_4_E_S1)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_E_S1)

MSG3_BEGIN(system_information_type_4_E_N1)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_E_N1)

MSG3_BEGIN(system_information_type_4_E_N2)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_E_N2)

MSG3_BEGIN(system_information_type_4_E_N3)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_E_N3)

MSG3_BEGIN(system_information_type_4_E_N4)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_E_N4)

MSG3_BEGIN(system_information_type_4_E_N5)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_E_N5)

MSG3_BEGIN(system_information_type_4_E_N6)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_E_N6)

MSG3_BEGIN(system_information_type_4_E_N7)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_E_N7)

# TEST CASES

## Initialization of the IUT

### MRR000: Initialization (One cell)

Description:	(Ref.: GSM 11.10-1, §26.6)

Preamble:	None

Script:

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

History:	15.12.97	VK	Initial

### MRR001: Initialization (Two cells)

Description:	(Ref.: GSM 11.10-1, §26.6)

Preamble:	None

Script:

ISS_INIT	( 2)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SYS_INFO	( 1, system_information_type_1_cell_b)

BS_SET_SYS_INFO	( 1, system_information_type_2)

BS_SET_SYS_INFO	( 1, system_information_type_3_cell_b)

BS_SET_SYS_INFO	( 1, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 1, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 1, system_information_type_6_cell_b)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

BS_SET_SCH	( 1,BSIC , RFN)

BS_SET_ARFCN	( 1,ARFCN_BCCH_GSM_CELL_B               )

BS_SET_POWER	( 1,DOWNLINK_INPUT_LEVEL_DBM_CELL_B)

BS_ON_OFF	( 1,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cells */

History:	07.01.98	VK	Initial

## Immediate assignment

### MRR100: Immediate assignment / SDCCH or TCH assignment (26.6.1.1)

Description:	(Ref.: GSM 11.10-1, §26.6.1.1)

Preamble:	MRR000

Script:

{

MSG3_VAR ( msg)

SET_TIMEOUT	( 30000)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, msg,"  2:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,"  3: SDCCH test: Channel type = SDCCH/8")

BF_SET_VAL       (ref,MSG3_BF_GET_VAL(msg,ref),"")

BF_SET_VAL       (t1_,MSG3_T1_(msg),SILENT)

BF_SET_VAL       ( t3,MSG3_T3 (msg),SILENT)

BF_SET_VAL       ( t2,MSG3_T2 (msg),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 4:")

BS_MSG3_SEND        ( 0, channel_release," 5:")

MSG3_COPY_DESTROY	( msg)

}

History:	15.12.97	VK	Initial

### MRR101: Immediate assignment / extended assignment (26.6.1.2)

Description:	(Ref.: GSM 11.10-1, §26.6.1.2)
Differences to Test Specification :
The number of CHANNEL REQUESTS in step 1, 9, and 18 are not arbitrarily chosen. Instead these items take on fixed values (8).

Preamble:	None

Script:

{

MSG3_VAR ( page_msg0)

MSG3_VAR ( page_msg1)

MSG3_VAR ( page_msg2)

MSG3_VAR ( page_msg3)

MSG3_VAR ( page_msg4)

MSG3_VAR ( page_msg5)

MSG3_VAR ( page_msg6)

MSG3_VAR ( page_msg7)

IE_BF_SET_VAL       ( rach_control_parameter, max_retrans, 3," Max-retrans is 7 (coded:3)")

#include            <mrr000.tds>

SET_TIMEOUT         ( 30000)

/* PART 1 of 3 : successul paging */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2: 1st answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 2: 2nd answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg2," 2: 3rd answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg3," 2: 4th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg4," 2: 5th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg5," 2: 6th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg6," 2: 7th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg7," 3: 8th answer to paging")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_extended,"  4:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg5,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg5),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg5),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg5),SILENT)

BF_SET_VAL       (request_reference_2.random_access_info,M3(0,1,0),"different Req Ref")

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 5:")

BS_MSG3_SEND        ( 0, channel_release," 6:")

ISS_DELAY           ( 12000) /* wait until cell reselection (step 7) */

/* PART 2 of 3 : only the last three 'channel requests' are handled */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 8:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 8: 1st answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 8: 2nd answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg2," 8: 3rd answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg3," 8: 4th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg4," 8: 5th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg5," 8: 6th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg6," 8: 7th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg7," 10: 8th answer to paging")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_extended,"  11:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg4,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg4),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg4),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg4),SILENT)

BF_SET_VAL       (request_reference_2.random_access_info,M3(0,1,0),"different Req Ref")

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

TIMEOUT             ( 3000) /* step 14 */

IE_BF_SET_VAL       ( control_channel_description, ccch_conf, 0," 15:not combined")

BS_SET_SYS_INFO	( 0, system_information_type_3)

ISS_DELAY           ( 40000) /* wait until cell reselection (step 16) */

/* PART 3 of 3 : extented immediate assigment via 'second valid Request Reference' */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 17:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 18: 1st answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 18: 2nd answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg2," 18: 3rd answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg3," 18: 4th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg4," 18: 5th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg5," 18: 6th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg6," 18: 7th answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg7," 19: 8th answer to paging")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_extended,"  20:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg5,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg5),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg5),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg5),SILENT)

BF_SET_VAL       (request_reference.random_access_info,M3(0,1,0),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 5:")

BS_MSG3_SEND        ( 0, channel_release," 6:")

MSG3_COPY_DESTROY	( page_msg7)

MSG3_COPY_DESTROY	( page_msg6)

MSG3_COPY_DESTROY	( page_msg5)

MSG3_COPY_DESTROY	( page_msg4)

MSG3_COPY_DESTROY	( page_msg3)

MSG3_COPY_DESTROY	( page_msg2)

MSG3_COPY_DESTROY	( page_msg1)

MSG3_COPY_DESTROY	( page_msg0)

}

History:	15.12.97	VK	Initial

### MRR102: Immediate assignment / assignment rejection (26.6.1.3)

Description:	(Ref.: GSM 11.10-1, §26.6.1.3)
Differences to Test Specification : 
After the reception of IMMEDIATE ASSIGNMENT REJECT, the MS shall not transmit during the time indicated in the" Wait Indication" field of the IMMEDIATE ASSIGNMENT REJECT message. This conformance requirement cannot be checked in this test suite.

Preamble:	None

Script:

{

MSG3_VAR ( page_msg)

IE_BF_SET_VAL       ( rach_control_parameter, max_retrans, 3," Max-retrans is 7 (coded:3)")

#include            <mrr001.tds>

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 1:")

REPEAT_BEGIN        ( COUNT, 8)

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg," 1+n: answer to paging")

REPEAT_END(COUNT)

BS_RACH_EXPECT_TIMEOUT( 0, 1000)

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  2+n:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg),SILENT)

BF_SET_VAL       (wait_indication_2.t3122, 15,"wait 15 seconds")

BF_SET_VAL       (request_reference.random_access_info  ,M3(0,1,0),"different Req Ref")

BF_SET_VAL       (request_reference_3.random_access_info,M3(0,1,0),"different Req Ref")

BF_SET_VAL       (request_reference_4.random_access_info,M3(0,1,0),"different Req Ref")

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

REPEAT_BEGIN        ( COUNT, 10)

BS_MSG3_SEND     ( 0, paging_request_type_1," 3+n:")

BS_RACH_EXPECT_TIMEOUT( 0, 1000)

REPEAT_END(COUNT)

ISS_DELAY           ( 6000) /* wait another xx seconds */

/* T3122 should now have been elapsed */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," k:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg," k+1:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  k+2:")

BF_SET_VAL       (request_reference_3.ref,MSG3_BF_GET_VAL(page_msg,ref),"")

BF_SET_VAL       (request_reference_3.t1_,MSG3_T1_(page_msg),SILENT)

BF_SET_VAL       (request_reference_3.t3 ,MSG3_T3 (page_msg),SILENT)

BF_SET_VAL       (request_reference_3.t2 ,MSG3_T2 (page_msg),SILENT)

BF_SET_VAL       (wait_indication_3.t3122,255,"wait 255 seconds")

BF_SET_VAL       (request_reference.random_access_info  ,M3(0,1,0),"different Req Ref")

BF_SET_VAL       (request_reference_2.random_access_info,M3(0,1,0),"different Req Ref")

BF_SET_VAL       (request_reference_4.random_access_info,M3(0,1,0),"different Req Ref")

BS_MSG3_SEND_END()

BS_SET_POWER	( 0,-9999)

BS_SET_POWER	( 1,-10)

ISS_DELAY	( 20000) /* wait until the MS has detected the new cell */

BS_CONFIG_CHANNEL   ( 1, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 1, paging_request_type_1," k+4:")

BS_RACH_AWAIT_COPY  ( 1, channel_request, page_msg," k+5:")

BS_CONFIG_CHANNEL   ( 1, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 1, immediate_assignment_reject,"  k+6:")

BF_SET_VAL       (request_reference_3.ref,MSG3_BF_GET_VAL(page_msg,ref),"")

BF_SET_VAL       (request_reference_3.t1_,MSG3_T1_(page_msg),SILENT)

BF_SET_VAL       (request_reference_3.t3 ,MSG3_T3 (page_msg),SILENT)

BF_SET_VAL       (request_reference_3.t2 ,MSG3_T2 (page_msg),SILENT)

BF_SET_VAL       (wait_indication_3.t3122,255,"wait 255 seconds")

BF_SET_VAL       (request_reference.random_access_info  ,M3(0,1,0),"different Req Ref")

BF_SET_VAL       (request_reference_2.random_access_info,M3(0,1,0),"different Req Ref")

BF_SET_VAL       (request_reference_4.random_access_info,M3(0,1,0),"different Req Ref")

BS_MSG3_SEND_END()

MSG3_COPY_DESTROY	( page_msg)

}

History:	08.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR103: Immediate assignment / ignore assignment (26.6.1.4, Step 1-2)

Description:	(Ref.: GSM 11.10-1, §26.6.1.4, Step 1, 2)

Preamble:	None

Script:

{

MSG3_VAR ( page_msg)

IE_BF_SET_VAL       ( rach_control_parameter, max_retrans, 3," Max-retrans is 7 (coded:3)")

#include            <mrr000.tds>

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg," 2: answer to paging")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,"  3:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg)  ,SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg)+2,"2 too high")

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg)  ,SILENT)

BS_MSG3_SEND_END()

MSG3_COPY_DESTROY	( page_msg)

}

History:	08.01.97	VK	Initial

### MRR104: Immediate assignment / ignore assignment (26.6.1.4, Step 3)

Description:	(Ref.: GSM 11.10-1, §26.6.1.4, Step 3)

Preamble:	MRR103

Script:

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_EXPECT_TIMEOUT( 0, 2000)

BS_MSG3_EXPECT_TIMEOUT( 0, 10000) /* wait another 10 seconds !!! */

History:	08.01.97	VK	Initial

### MRR105: Immediate assignment / ignore assignment (26.6.1.4, Step 4-8)

Description:	(Ref.: GSM 11.10-1, §26.6.1.4, Step 4, 5, 6, 7, 8)

Preamble:	MRR103

Script:

{

MSG3_VAR ( page_msg)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg," 4: answer to paging")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  5:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg),SILENT)

BS_MSG3_SEND_END()

ISS_DELAY           ( 6000) /* SS waits for 6 seconds (step 6:) */

ISS_DELAY           ( 4000) /* if PCO is used, wait another 4 seconds */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 7:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg," 8: answer to paging")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,"  9:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg,ref)^~0,"inv Req Ref")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg),SILENT)

BS_MSG3_SEND_END()

MSG3_COPY_DESTROY	( page_msg)

}

History:	08.01.97	VK	Initial

### MRR106: Immediate assignment / ignore assignment (26.6.1.4, Step 9)

Description:	(Ref.: GSM 11.10-1, §26.6.1.4, Step 9)

Preamble:	MRR105

Script:

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_EXPECT_TIMEOUT( 0, 2000)

BS_MSG3_EXPECT_TIMEOUT( 0, 10000) /* wait another 10 seconds !!! */

History:	08.01.97	VK	Initial

### MRR107: Immediate assignment / ignore assignment (26.6.1.4, Step 10-11)

Description:	(Ref.: GSM 11.10-1, §26.6.1.4, Step 10,11)

Preamble:	MRR105

Script:

{

MSG3_VAR ( page_msg)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg," 10: answer to paging")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  11:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg),SILENT)

BS_MSG3_SEND_END()

MSG3_COPY_DESTROY	( page_msg)

}

History:	08.01.97	VK	Initial

### MRR108: Immediate assignment / ignore assignment (26.6.1.4)

Description:	(Ref.: GSM 11.10-1, §26.6.1.4)
Note : Step 3 and step 4 are swapped. Step 8 and step 9 are swapped.

Preamble:	MRR000

Script:

{

MSG3_VAR ( page_msg0)

MSG3_VAR ( page_msg1)

SET_TIMEOUT         ( 30000)

REPEAT              (COUNT,2)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2: answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 4: answer to paging")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,"  3:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg0,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg0)  ,SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg0)+2,"2 too high")

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg0)  ,SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_EXPECT_TIMEOUT( 0, 2000)

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  5:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

ISS_DELAY           ( 6000) /* SS waits for 6 seconds (step 6:) */

ISS_DELAY           ( 4000) /* if PCO is used, wait another 4 seconds */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 7:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 8: answer to paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 10: answer to paging")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,"  9:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg0,ref)^~0,"inv Req Ref")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg0),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg0),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg0),SILENT)

BS_MSG3_SEND_END()

BS_MSG3_EXPECT_TIMEOUT( 0, 2000)

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  11:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

ISS_DELAY           ( 6000) /* SS waits for 6 seconds */

ISS_DELAY           ( 4000) /* if PCO is used, wait another 4 seconds */

REPEAT_END(COUNT)

MSG3_COPY_DESTROY	( page_msg0)

MSG3_COPY_DESTROY	( page_msg1)

}

History:	08.01.97	VK	Initial

### MRR109: Immediate assignment after immediate assignment reject (26.6.1.5)

Description:	(Ref.: GSM 11.10-1, §26.6.1.5)
Note : in step 7 it is not verified, that the paging response message is sent on the correct channel. These is no delay between step 5 and 6.

Preamble:	None

Script:

{

MSG3_VAR ( page_msg0)

MSG3_VAR ( page_msg1)

MSG3_VAR ( page_msg2)

IE_BF_SET_VAL       ( control_channel_description, ccch_conf, M3(0,0,0)," 1 chan, not comb.");

IE_BF_SET_VAL       ( rach_control_parameter, max_retrans, 3," Max-retrans is 7 (coded:3)")

IE_BF_SET_VAL       ( rach_control_parameter, tx_integer, M4(0,1,0,0)," 7 slots");

#include            <mrr000.tds>

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1,  "   1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2: first  request")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 3: second request")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg2," 4: third  request")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  5:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg0,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg0),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg0),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg0),SILENT)

BF_SET_VAL       (wait_indication.t3122,6,"wait 6 seconds")

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,"  6:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 7:")

BS_MSG3_SEND        ( 0, channel_release," 8:")

MSG3_COPY_DESTROY	( page_msg0)

MSG3_COPY_DESTROY	( page_msg1)

MSG3_COPY_DESTROY	( page_msg2)

}

History:	12.01.97	VK	Initial

## Paging

### MRR130: Paging /normal /type 1 (26.6.2.1.1)

Description:	(Ref.: GSM 11.10-1, §26.6.2.1.1)
Note CCCH_CONF, BS_AG_BLKS_RES and BS_PA_MFRMS take on fixed values.

Preamble:	None

Script:

{

MSG3_VAR ( page_msg0)

MSG3_VAR ( page_msg1)

IE_BEGIN(p1_rest_octets_6)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */
IE_END(p1_rest_octets_6)

IE_BEGIN(p1_rest_octets_4)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */
IE_END(p1_rest_octets_4)

IE_BEGIN(mobile_identity_tmsi_other)
   BF(8,          5,ACT_CHECK,length,"   five octets to come")
   BF(4,M4(1,1,1,1),ACT_CHECK,ANONYMOUS,"bits 5-8 of octet 3 ='1111'")
   BF(1,          0,ACT_CHECK,odd_even," as applicable for TMSI")
   BF(3,  M3(1,0,0),ACT_CHECK,type,  "   TMSI")
   BF(8,       0x78,ACT_CHECK,ANONYMOUS, SILENT)
   BF(8,       0x56,ACT_CHECK,ANONYMOUS, SILENT)
   BF(8,       0x34,ACT_CHECK,ANONYMOUS, SILENT)
   BF(8,       0x12,ACT_CHECK,ANONYMOUS, SILENT)
IE_END(mobile_identity_tmsi_other)

MSG3_BEGIN(paging_request_type_1_tmsi_other)
   IE(l2_pseudo_length_9)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_1_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(mobile_identity_tmsi)
   IE(iei_17)
   IE(mobile_identity_tmsi_other)
   IE(p1_rest_octets_6)
MSG3_END(paging_request_type_1_tmsi_other)

MSG3_BEGIN(paging_request_type_1_other_imsi)
   IE(l2_pseudo_length_9)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_1_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(mobile_identity_tmsi_other)
   IE(iei_17)
   IE(mobile_identity_imsi)
   IE(p1_rest_octets_4)
MSG3_END(paging_request_type_1_other_imsi)

MSG3_BEGIN(paging_request_type_1_other_tmsi)
   IE(l2_pseudo_length_9)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_1_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(mobile_identity_tmsi_other)
   IE(iei_17)
   IE(mobile_identity_tmsi)
   IE(p1_rest_octets_6)
MSG3_END(paging_request_type_1_other_tmsi)

IE_BF_SET_VAL       ( rach_control_parameter, max_retrans, M2(0,1)," Max-retrans is 2")

#include            <mrr000.tds>

SET_TIMEOUT         ( 30000)

/* part I : 1st Mobile Identity : IMSI of IUT, 2nd Mobile Identity : not present */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1_imsi_none,"1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 3:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   4:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 5:")

BS_MSG3_SEND        ( 0, tmsi_reallocation_command_tmsi , SILENT)
BS_MSG3_AWAIT       ( 0, tmsi_reallocation_complete , SILENT)
BS_MSG3_SEND        ( 0, channel_release," 6:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part II : 1st Mobile Identity : TMSI of IUT, 2nd Mobile Identity : other TMSI */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1_tmsi_other,"7:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg0," 8:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg1," 9:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   10:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),SILENT)

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_tmsi," 11: Mobile Ident: TMSI.")

BS_MSG3_SEND        ( 0, channel_release," 12:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part III : 1st Mobile Identity : TMSI of other MS, 2nd Mobile Identity : IMSI of IUT*/

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1_other_imsi,"13:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg0," 14:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg1," 15:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,     "   16:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),SILENT)

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 17: Mobile Ident: IMSI.")

BS_MSG3_SEND        ( 0, channel_release,   "   18:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part IV : 1st Mobile Identity : TMSI of other MS, 2nd Mobile Identity : TMSI of IUT */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1_other_tmsi,"19:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg0," 20:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg1," 21:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,     "   22:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),SILENT)

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_tmsi," 23: Mobile Ident: TMSI.")

BS_MSG3_SEND        ( 0, channel_release," 24:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part V : 1st Mobile Identity : TMSI of IUT (wrong type), 2nd Mobile Identity : none */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, paging_request_type_1,"25:")

BF_SET_VAL       ( mobile_identity_imsi.type, M3(0,0,0)," no Identity")

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0) /* step 26: no signalling for 1 second */

BS_MSG3_EXPECT_TIMEOUT( 0, 1000)

MSG3_COPY_DESTROY	( page_msg0)

MSG3_COPY_DESTROY	( page_msg1)

}

History:	12.01.97	VK	Initial

### MRR131: Paging /normal /type 2 (26.6.2.1.2)

Description:	(Ref.: GSM 11.10-1, §26.6.2.1.2)
Note CCCH_CONF, BS_AG_BLKS_RES and BS_PA_MFRMS take on fixed values.

Preamble:	None

Script:

{

MSG3_VAR ( page_msg0)

MSG3_VAR ( page_msg1)

IE_BEGIN(iei_17)
   BF(8,0x17,ACT_CHECK,ANONYMOUS,SILENT)
IE_END(iei_17)

IE_BEGIN(p2_rest_octets_4)
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */
   BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */
IE_END(p2_rest_octets_4)

MSG3_BEGIN(paging_request_type_2_tmsi_other_none)
   IE(l2_pseudo_length_11)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_2_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi)
   IE(tmsi_2)
   IE(p2_rest_octets)
MSG3_END(paging_request_type_2_tmsi_other_none)

MSG3_BEGIN(paging_request_type_2_other_tmsi_none)
   IE(l2_pseudo_length_11)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_2_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi_2)
   IE(tmsi)
   IE(p2_rest_octets)
MSG3_END(paging_request_type_2_other_tmsi_none)

MSG3_BEGIN(paging_request_type_2_other_other_tmsi)
   IE(l2_pseudo_length_11)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_2_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(iei_17)
   IE(mobile_identity_tmsi)
   IE(p2_rest_octets_4)
MSG3_END(paging_request_type_2_other_other_tmsi)

IE_BF_SET_VAL       ( rach_control_parameter, max_retrans, M2(0,1)," Max-retrans is 2")

#include            <mrr000.tds>

ISS_DELAY	( 5000) /* wait until the MS has detected the cells */

SET_TIMEOUT         ( 30000)

/* preamble: allocation of TMSI */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1_imsi_none,"1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 3:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   4:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 5:")

BS_MSG3_SEND        ( 0, tmsi_reallocation_command_tmsi , SILENT)
BS_MSG3_AWAIT       ( 0, tmsi_reallocation_complete , SILENT)
BS_MSG3_SEND        ( 0, channel_release," 6:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part I : 1st TMSI : TMSI of IUT, 2nd TMSI : another TMSI, Mob ID not present*/

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_2_tmsi_other_none,"1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 3:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   4:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_tmsi," 5: Mobile Ident : TMSI")

BS_MSG3_SEND        ( 0, channel_release," 6:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part II : 1st TMSI : another , 2nd TMSI : TMSI of IUT, Mob ID not present*/

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_2_other_tmsi_none,"7:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg0," 8:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg1," 9:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   10:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),SILENT)

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_tmsi," 11: Mobile Ident: TMSI.")

BS_MSG3_SEND        ( 0, channel_release," 12:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part III : 1st TMSI : another , 2nd TMSI : another, Mob ID : TMSI of IUT */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_2_other_other_tmsi,"13:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg0," 14:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg1," 15:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,     "   16:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),SILENT)

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_tmsi," 17: Mobile Ident: TMSI.")

BS_MSG3_SEND        ( 0, channel_release,   "   18:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part IV : 1st TMSI : another , 2nd TMSI : another, Mob ID : IMSI of IUT */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_2_other_other_imsi,"19:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg0," 20:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg1," 21:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,     "   22:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),SILENT)

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 23: Mobile Ident: IMSI.")

BS_MSG3_SEND        ( 0, channel_release," 24:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part V : 1st TMSI : another , 2nd TMSI : another, Mob ID : TMSI of IUT (wrong type)*/

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, paging_request_type_2_other_other_tmsi,"25:")

BF_SET_VAL       ( mobile_identity_tmsi.type, M3(0,0,0)," no Identity")

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0) /* step 26: no signalling for 1 second */

BS_MSG3_EXPECT_TIMEOUT( 0, 1000)

MSG3_COPY_DESTROY	( page_msg0)

MSG3_COPY_DESTROY	( page_msg1)

}

History:	13.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR132: Paging /normal /type 3 (26.6.2.1.3)

Description:	(Ref.: GSM 11.10-1, §26.6.2.1.3)
Note CCCH_CONF, BS_AG_BLKS_RES and BS_PA_MFRMS take on fixed values.

Preamble:	None

Script:

{

MSG3_VAR ( page_msg0)

MSG3_VAR ( page_msg1)

MSG3_BEGIN(paging_request_type_3_tmsi_1)
   IE(l2_pseudo_length_19)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_3_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(p3_rest_octets)
MSG3_END(paging_request_type_3_tmsi_1)

MSG3_BEGIN(paging_request_type_3_tmsi_2)
   IE(l2_pseudo_length_19)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_3_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi_2)
   IE(tmsi)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(p3_rest_octets)
MSG3_END(paging_request_type_3_tmsi_2)

MSG3_BEGIN(paging_request_type_3_tmsi_3)
   IE(l2_pseudo_length_19)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_3_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(tmsi)
   IE(tmsi_2)
   IE(p3_rest_octets)
MSG3_END(paging_request_type_3_tmsi_3)

MSG3_BEGIN(paging_request_type_3_tmsi_4)
   IE(l2_pseudo_length_19)
   IE(skip_indicator)
   IE(rr_management_protocol_discriminator)
   IE(paging_request_type_3_message_type)
   IE(channel_needed)
   IE(page_mode)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(tmsi_2)
   IE(tmsi)
   IE(p3_rest_octets)
MSG3_END(paging_request_type_3_tmsi_4)

IE_BF_SET_VAL       ( rach_control_parameter, max_retrans, M2(0,1)," Max-retrans is 2")

#include            <mrr000.tds>

SET_TIMEOUT         ( 30000)

/* preamble: allocation of TMSI */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1_imsi_none,"1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 3:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   4:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 5:")

BS_MSG3_SEND        ( 0, tmsi_reallocation_command_tmsi , SILENT)
BS_MSG3_AWAIT       ( 0, tmsi_reallocation_complete , SILENT)
BS_MSG3_SEND        ( 0, channel_release," 6:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part I : 1st TMSI : TMSI of IUT */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_3_tmsi_1,"1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 3:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   4:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_tmsi," 5: Mobile Ident : TMSI")

BS_MSG3_SEND        ( 0, channel_release," 6:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part II : 2nd TMSI : TMSI of IUT */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_3_tmsi_2,"7:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg0," 8:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg1," 9:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   10:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),SILENT)

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_tmsi," 11: Mobile Ident: TMSI.")

BS_MSG3_SEND        ( 0, channel_release," 12:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part III : 3rd TMSI : TMSI of IUT */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_3_tmsi_3,"13:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg0," 14:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg1," 15:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,     "   16:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),SILENT)

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_tmsi," 17: Mobile Ident: TMSI.")

BS_MSG3_SEND        ( 0, channel_release,   "   18:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part IV : 4th TMSI : TMSI of IUT */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_3_tmsi_4,"19:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg0," 20:")

BS_RACH_AWAIT_COPY  ( 0,  channel_request, page_msg1," 21:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,     "   22:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),SILENT)

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_tmsi," 23: Mobile Ident: TMSI.")

BS_MSG3_SEND        ( 0, channel_release," 24:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

MSG3_COPY_DESTROY	( page_msg0)

MSG3_COPY_DESTROY	( page_msg1)

}

History:	13.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR133: Paging / extended (26.6.2.2)

Description:	(Ref.: GSM 11.10-1, §26.6.2.2)
Note CCCH_CONF, BS_AG_BLKS_RES and BS_PA_MFRMS take on fixed values.

Preamble:	None

Script:

{

MSG3_VAR ( page_msg0)

MSG3_VAR ( page_msg1)

IE_BF_SET_VAL       ( rach_control_parameter, max_retrans, M2(0,1)," Max-retrans is 2")

#include            <mrr000.tds>

SET_TIMEOUT         ( 30000)

/* preamble: allocation of TMSI */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1_imsi_none,"1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 3:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   4:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 5:")

BS_MSG3_SEND        ( 0, tmsi_reallocation_command_tmsi , SILENT)
BS_MSG3_AWAIT       ( 0, tmsi_reallocation_complete , SILENT)
BS_MSG3_SEND        ( 0, channel_release," 6:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part I : */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, paging_request_type_1,"1:")

BF_SET_VAL       (page_mode.pm,M2(0,1),"extended paging")

BS_MSG3_SEND_END()

BS_MSG3_SEND        ( 0, paging_request_type_1,"2:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 3:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 4:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  5:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg1),SILENT)

BF_SET_VAL       (wait_indication_2.t3122,5,"wait 5 seconds")

BS_MSG3_SEND_END()

ISS_DELAY ( 5000) /* step 6: */

ISS_DELAY ( 3000) /* wait another 3 seconds */

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   7:")

BF_SET_VAL       (page_mode.pm,M2(0,1),"extended paging")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref)^(-1),"different")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

/* part II : */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1,"8:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 9:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 10:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  11:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg1),SILENT)

BF_SET_VAL       (wait_indication_2.t3122,5,"wait 5 seconds")

BS_MSG3_SEND_END()

ISS_DELAY ( 5000) /* step 12: */

ISS_DELAY ( 3000) /* wait another 3 seconds */

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_extended," 13:")

BF_SET_VAL       (page_mode.pm,M2(0,1),"extended paging")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref)^(-1),"different")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

/* part III : */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_3,"14:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 15:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 16:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  17:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg1),SILENT)

BF_SET_VAL       (wait_indication_2.t3122,5,"wait 5 seconds")

BS_MSG3_SEND_END()

ISS_DELAY ( 5000) /* step 18: */

ISS_DELAY ( 3000) /* wait another 3 seconds */

/* part IV : */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, paging_request_type_3,"19:")

BF_SET_VAL       (page_mode.pm,M2(0,1),"extended paging")

BS_MSG3_SEND_END()

BS_MSG3_SEND        ( 0, paging_request_type_2_other_other_imsi,"20:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 21:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 22:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  23:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg1),SILENT)

BF_SET_VAL       (wait_indication_2.t3122,5,"wait 5 seconds")

BS_MSG3_SEND_END()

ISS_DELAY ( 5000) /* step 24: */

ISS_DELAY ( 3000) /* wait another 3 seconds */

/* part V : */

BS_MSG3_SEND_BEGIN  ( 0, paging_request_type_2,"25:")

BF_SET_VAL       (page_mode.pm,M2(0,1),"extended paging")

BS_MSG3_SEND_END()

BS_MSG3_SEND        ( 0, paging_request_type_1_imsi_none,"26:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 27:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 28:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject,"  29:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg1),SILENT)

BF_SET_VAL       (wait_indication_2.t3122,5,"wait 5 seconds")

BS_MSG3_SEND_END()

MSG3_COPY_DESTROY	( page_msg0)

MSG3_COPY_DESTROY	( page_msg1)

}

History:	13.01.97	VK	Initial

### MRR134: Paging / reorganization (26.6.2.3)

Description:	(Ref.: GSM 11.10-1, §26.6.2.3)
Note CCCH_CONF, BS_AG_BLKS_RES and BS_PA_MFRMS take on fixed values.

Preamble:	None

Script:

{

MSG3_VAR ( page_msg0)

MSG3_VAR ( page_msg1)

IE_BF_SET_VAL       ( rach_control_parameter, max_retrans, M2(0,1)," Max-retrans is 2")

IE_BF_SET_VAL       ( control_channel_description, ccch_conf, M3(0,0,0)," 1 chan, uncomb.")

#include            <mrr000.tds>

SET_TIMEOUT         ( 30000)

/* preamble: allocation of TMSI */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1_imsi_none,"1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 2:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 3:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,    "   4:")

BF_SET_VAL       (request_reference.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference.t2 ,MSG3_T2 (page_msg1),SILENT)

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 5:")

BS_MSG3_SEND        ( 0, tmsi_reallocation_command_tmsi , SILENT)
BS_MSG3_AWAIT       ( 0, tmsi_reallocation_complete , SILENT)
BS_MSG3_SEND        ( 0, channel_release," 6:")

ISS_DELAY ( 12000)  /*" The SS waits 12 seconds to perform cell reselection." */

/* part I : */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_extended," 1:")

BF_SET_VAL       (page_mode.pm,M2(1,0),"reorganization")

BS_MSG3_SEND_END()

ISS_DELAY           ( 3000)        /* wait for the MS to accept the new page mode */

BS_SET_PAGE_MODE    ( 0, M2(1,0), 1, -1) /* set 'paging reorganization' */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_2,"2: normal paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 3:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 4:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject," 5:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg1),SILENT)

BF_SET_VAL       (wait_indication_2.t3122,0,"wait 0 seconds")

BS_MSG3_SEND_END()

ISS_DELAY ( 8000) /* step 6: */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, paging_request_type_2," 7: paging reorganization")

BF_SET_VAL       (page_mode.pm,M2(1,0),"reorganization")

BS_MSG3_SEND_END()

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 8:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 9:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject," 10:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg1),SILENT)

BF_SET_VAL       (wait_indication_2.t3122,0,"wait 0 seconds")

BS_MSG3_SEND_END()

IE_BF_SET_VAL       ( control_channel_description, ccch_conf, M3(0,0,1)," 1 chan, comb.")

IE_BF_SET_VAL       ( control_channel_description, bs_ag_blks_res,2,"");

IE_BF_SET_VAL       ( control_channel_description, bs_pa_mfrms,7,"9 multiframes");

BS_SET_SYS_INFO     ( 0, system_information_type_3)

BS_SET_PAGE_MODE    ( 0, M2(0,0), 1, -1) /* set 'normal paging' */

ISS_DELAY ( 3000) /* step 14: */

ISS_DELAY ( 20000) /* wait another 20 seconds */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1,"15: normal paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 16:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 17:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject," 18:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg1),SILENT)

BF_SET_VAL       (wait_indication_2.t3122,0,"wait 0 seconds")

BS_MSG3_SEND_END()

ISS_DELAY ( 5000) /* step 14: */

ISS_DELAY ( 3000) /* wait another 3 seconds: */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_2,"19: normal paging")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg0," 20:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, page_msg1," 21:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_reject," 22:")

BF_SET_VAL       (request_reference_2.ref,MSG3_BF_GET_VAL(page_msg1,ref),"")

BF_SET_VAL       (request_reference_2.t1_,MSG3_T1_(page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t3 ,MSG3_T3 (page_msg1),SILENT)

BF_SET_VAL       (request_reference_2.t2 ,MSG3_T2 (page_msg1),SILENT)

BF_SET_VAL       (wait_indication_2.t3122,0,"wait 0 seconds")

BS_MSG3_SEND_END()

MSG3_COPY_DESTROY   ( page_msg0)

MSG3_COPY_DESTROY   ( page_msg1)

}

History:	13.01.97	VK	Initial

### MRR135: Call Setup Procedure for Mobile Terminated Calls (10.1)

Description:	This test describes the generic mobile terminated call setup procedure.
System information messages must have been sent.

Preamble:	MRR000

Script:

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup_19,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command_19,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

History:	22.01.97	VK	Initial

## Measurements

### MRR150: Measurement / no neighbours (26.6.3.1)

Description:	(Ref.: GSM 11.10-1, §26.6.3.1)

Preamble:	None

Script:

IE_BEGIN(measurement_results)

BF(1,1,ACT_CHECK  ,ba_ind                  ,SILENT)

BF(1,0,ACT_CHECK,dtx                     ,"DTX was not used")

BF(6,0x32,ACT_CHECK  ,rxlev_full_serving_cell ,SILENT)

BF(1,0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(1,0,ACT_CHECK,meas_valid              ,"valid within the 2nd SACCH"

"   block at the latest")

BF(6,0x32,ACT_CHECK  ,rxlev_sub_serving_cell  ,SILENT)

BF(1,0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(3,0,ACT_CHECK  ,rxqual_full_serving_cell,SILENT)

BF(3,0,ACT_CHECK  ,rxqual_sub_serving_cell ,SILENT)

BF(3,0,ACT_CHECK ,num_ncell_m             ,"no neighbor cell (0) or :"
                                          "   ncell info not avail (7)")

BF(6,0,ACT_CHECK,    rxlev_ncell_1       ,SILENT)

BF(5,0,ACT_CHECK,bcch_freq_ncell_1       ,SILENT)

BF(6,0,ACT_CHECK,     bsic_ncell_1       ,SILENT)

BF(6,0,ACT_CHECK,    rxlev_ncell_2       ,SILENT)

BF(5,0,ACT_CHECK,bcch_freq_ncell_2       ,SILENT)

BF(6,0,ACT_CHECK,     bsic_ncell_2       ,SILENT)

BF(6,0,ACT_CHECK,    rxlev_ncell_3       ,SILENT)

BF(5,0,ACT_CHECK,bcch_freq_ncell_3       ,SILENT)

BF(6,0,ACT_CHECK,     bsic_ncell_3       ,SILENT)

BF(6,0,ACT_CHECK,    rxlev_ncell_4       ,SILENT)

BF(5,0,ACT_CHECK,bcch_freq_ncell_4       ,SILENT)

BF(6,0,ACT_CHECK,     bsic_ncell_4       ,SILENT)

BF(6,0,ACT_CHECK,    rxlev_ncell_5       ,SILENT)

BF(5,0,ACT_CHECK,bcch_freq_ncell_5       ,SILENT)

BF(6,0,ACT_CHECK,     bsic_ncell_5       ,SILENT)

BF(6,0,ACT_CHECK,    rxlev_ncell_6       ,SILENT)

BF(5,0,ACT_CHECK,bcch_freq_ncell_6       ,SILENT)

BF(6,0,ACT_CHECK,     bsic_ncell_6       ,SILENT)

IE_END(measurement_results)

IE_BEGIN(neighbour_cell_description_26_6_3_1_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000001,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_032_001,"no neigbours")
IE_END(neighbour_cell_description_26_6_3_1_3)

IE_BEGIN(neighbour_cell_description_5bis_26_6_3_1_3)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         1,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         0,ACT_CHECK,f0       ,"frequency 0 indicator")
   BF(10,       500,ACT_CHECK,w1       ,"channel 500 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_5bis_26_6_3_1_3)

MSG3_BEGIN(measurement_report)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(measurement_report_message_type)

IE(measurement_results)

MSG3_END(measurement_report)

MSG3_BEGIN(system_information_type_5_26_6_3_1_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(neighbour_cell_description_26_6_3_1_3)

MSG3_END(system_information_type_5_26_6_3_1_3)

MSG3_BEGIN(system_information_type_5bis_26_6_3_1_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5bis_message_type)

IE(neighbour_cell_description_5bis_26_6_3_1_3)

MSG3_END(system_information_type_5bis_26_6_3_1_3)

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

IE_BEGIN(authentication_parameter_rand)

BF(32,0x80000000,ACT_CHECK,rand_127_096,SILENT)

BF(32,0x00000012,ACT_CHECK,rand_095_064,SILENT)

BF(32,0x34000000,ACT_CHECK,rand_063_032,SILENT)

BF(32,0x0000000F,ACT_CHECK,rand_031_000,SILENT)

IE_END(authentication_parameter_rand)

IE_BEGIN(authentication_parameter_sres)

BF(32,0x0000000F,ACT_SHOW,sres_031_000,SILENT)

IE_END(authentication_parameter_sres)

IE_BEGIN(call_control_protocol_discriminator)

BF(4, 3,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_control_protocol_discriminator)

IE_BEGIN(called_party_bcd_number)

BF( 8,7,ACT_CHECK,length                     ,"length of IE")

BF( 1,1,ACT_CHECK,ext                        ,"Extension Bit")

BF( 3,0,ACT_CHECK,type_of_number             ,"Unknown")

BF( 4,1,ACT_CHECK,numbering_plan             ,"ISDN/telephony")

BF( 4,3,ACT_CHECK,digit_2                    ,"Digit 2")

BF( 4,0,ACT_CHECK,digit_1                    ,"Digit 1")

BF( 4,3,ACT_CHECK,digit_4                    ,"Digit 4")

BF( 4,0,ACT_CHECK,digit_3                    ,"Digit 3")

BF( 4,0,ACT_CHECK,digit_6                    ,"Digit 6")

BF( 4,9,ACT_CHECK,digit_5                    ,"Digit 5")

BF( 4,4,ACT_CHECK,digit_8                    ,"Digit 8")

BF( 4,9,ACT_CHECK,digit_7                    ,"Digit 7")

BF( 4,1,ACT_CHECK,digit_10                   ,"Digit 10")

BF( 4,1,ACT_CHECK,digit_9                   ,"Digit 9")

BF( 4,7,ACT_CHECK,digit_12                  ,"Digit 12")

BF( 4,0xF,ACT_CHECK,digit_11                ,"Digit 11")

IE_END(called_party_bcd_number)

IE_BEGIN(channels_needed_for_mobiles_1_and_2)

BF(2,0,ACT_CHECK,second_channel,"spare, any channel")

BF(2,0,ACT_CHECK, first_channel,"spare, any channel")

IE_END(channels_needed_for_mobiles_1_and_2)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,1),ACT_CHECK,          channel_type,"SDCCH/SACCH 4(1)")

BF( 3,            0,ACT_CHECK,      time_slot_number,"zero")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(channel_description)

IE_BEGIN(ciphering_key_sequence_number_2)

BF(1,        0,ACT_CHECK,       spare,SILENT)

BF(3,M3(1,0,1),ACT_CHECK,key_sequence,"sent BS->MS")

IE_END(ciphering_key_sequence_number_2)

IE_BEGIN(ciphering_mode_setting)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(ciphering_mode_setting)

IE_BEGIN(cipher_response)

BF(3,0,ACT_CHECK,          spare,SILENT)

BF(1,0,ACT_CHECK,cipher_response,"IMEISV shall not be included")

IE_END(cipher_response)

IE_BEGIN(description_of_the_first_channel_after_time)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            3,ACT_CHECK,      time_slot_number,"three")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(description_of_the_first_channel_after_time)

IE_BEGIN(ia_rest_octets)   /* maximum length (11), no hop, no start time */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(ia_rest_octets)

IE_BEGIN(iei_5E)

BF(8,0x5E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_5E)

IE_BEGIN(iei_15)

BF(8,0x15,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_15)

IE_BEGIN(iei_04)

BF(8,0x04,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_04)

IE_BEGIN(iei_34)

BF(8,0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_34)

IE_BEGIN(l2_pseudo_length_11)  /* pag req type 1 with TMSI (one mobile)*/

BF(8, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_11)

IE_BEGIN(mobile_allocation)

BF(8,0,ACT_CHECK,length,"length 0 due to hopping disabled")

IE_END(mobile_allocation)

IE_BEGIN(mobile_identity)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_4,SILENT)

BF(4,          0,ACT_CHECK, digit_3,SILENT)

BF(4,          7,ACT_CHECK, digit_6,SILENT)

BF(4,          4,ACT_CHECK, digit_5,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          1,ACT_CHECK, digit_7,SILENT)

BF(4,          9,ACT_CHECK, digit_10,SILENT)

BF(4,          4,ACT_CHECK, digit_9,SILENT)

BF(4,          2,ACT_CHECK, digit_12,SILENT)

BF(4,          1,ACT_CHECK, digit_11,SILENT)

IE_END(mobile_identity)

IE_BEGIN(mobility_management_protocol_discriminator)

BF(4, 5,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobility_management_protocol_discriminator)

IE_BEGIN(ms_classmark)

BF(8,        3,ACT_CHECK, length,SILENT)

BF(1,        0,ACT_CHECK, spare,SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,"phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,"Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,"encryption algorithm A5/1 available")

BF(3,M3(0,0,1),ACT_CHECK, rf_power_capability,"class 2")

BF(1,        0,ACT_CHECK, spare2,SILENT)

BF(1,        0,ACT_CHECK, ps_capability,"no pseudo-synch capability")

BF(2,  M2(0,0),ACT_CHECK, ss_screening_indicator,"default phase 1")

BF(1,        0,ACT_CHECK, sm_capability,"no point to point SMS")

BF(1,        0,ACT_CHECK, vbs,"no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,"no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"no extention band G1")

BF(1,        0,ACT_CHECK, classmark_3,"no add. MS cap. information")

BF(5,        1,ACT_CHECK, ccbs,SILENT)

BF(1,        0,ACT_CHECK, a5_3,"A5/3 not available")

BF(1,        0,ACT_CHECK, a5_2,"A5/2 not available")

IE_END(ms_classmark)

IE_BEGIN(p1_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(p1_rest_octets)

IE_BEGIN(page_mode)

BF(2,0,ACT_CHECK,spare,"two spare bits")

BF(2,0,ACT_CHECK,pm   ,"Normal Paging")

IE_END(page_mode)

IE_BEGIN(power_command)

BF(8,10,ACT_CHECK,power,SILENT)

IE_END(power_command)

IE_BEGIN(rach)

BF(3,M3(1,0,0),ACT_CHECK ,establishment_cause,"paging ind. any chan'")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

IE_BEGIN(request_reference)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,"As in CHAN REQ")

BF(5,M5(1,1,1,1,1),ACT_SHOW,  random_reference,SILENT)

BF(5,            0,ACT_SHOW,               t1_,SILENT)

BF(6,            0,ACT_SHOW,               t3 ,SILENT)

BF(5,            0,ACT_SHOW,               t2 ,SILENT)

IE_END(request_reference)

IE_BEGIN(rr_cause)

BF(8,0,ACT_CHECK,rr_cause,"normal event")

IE_END(rr_cause)

IE_BEGIN(signal_call_waiting)

BF(8,M8(0,0,0,0,0,1,1,1),ACT_CHECK,signal_value,"(Any non-res. value)")

IE_END(signal_call_waiting)

IE_BEGIN(spare_half_octet)

BF(4, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(spare_half_octet)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,         spare,SILENT)

BF(6,0,ACT_CHECK,timing_advance,"0")

IE_END(timing_advance)

IE_BEGIN(transaction_identifier_source)

BF(4,M4(0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source)

IE_BEGIN(transaction_identifier_dest)

BF(4,M4(1,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest)

/*------------------------------------------------------------------*\

| Messages

\*------------------------------------------------------------------*/

MSG3_BEGIN(alerting)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(alerting_message_type)

MSG3_END(alerting)

MSG3_BEGIN(assignment_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(assignment_command)

MSG3_BEGIN(assignment_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_complete_message_type)

IE(rr_cause)

MSG3_END(assignment_complete)

MSG3_BEGIN(authentication_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(authentication_parameter_rand)

MSG3_END(authentication_request)

MSG3_BEGIN(authentication_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_response_message_type)

IE(authentication_parameter_sres)

MSG3_END(authentication_response)

MSG3_BEGIN(ciphering_mode_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(cipher_response)

IE(ciphering_mode_setting)

MSG3_END(ciphering_mode_command)

MSG3_BEGIN(ciphering_mode_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

MSG3_END(ciphering_mode_complete)

MSG3_BEGIN(connect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(connect_message_type)

MSG3_END(connect)

MSG3_BEGIN(connect_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(connect_acknowledge_message_type)

MSG3_END(connect_acknowledge)

MSG3_BEGIN(setup)            /* contains 'signal' but no 'Bearer Cap' */

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

MSG3_END(setup)

/* ------------- start testcase ---------- */

REPEAT(K,2)

ISS_INIT	( 8)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

REPEAT(N,8)

IE_BF_SET_VAL         ( cell_identity, ci, N," ci used for SI 3 and SI 6")

IE_BF_SET_VAL         ( neighbour_cell_description, ba_ind,  0," BA for BCCH")

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 0, SILENT)

BS_SET_SYS_INFO       ( N, system_information_type_1)

BS_SET_SYS_INFO       ( N, system_information_type_2)

BS_SET_SYS_INFO       ( N, system_information_type_3)

BS_SET_SYS_INFO       ( N, system_information_type_4)

IE_BF_SET_VAL         ( neighbour_cell_description,  ba_ind, 1," BA for SACCH")

if ( K == 1)

{

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 1,"")

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5bis_26_6_3_1_3)

}

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5_26_6_3_1_3)

BS_SET_SYS_INFO_SACCH ( N, system_information_type_6)

REPEAT_END(N)

#define              NCC_0    1

#define              NCC_1    1

#define              NCC_2    1

#define              NCC_3    1

#define              NCC_4    1

#define              NCC_5    1

#define              NCC_6    1

#define              NCC_7    1

#define              BCC_0    3

#define              BCC_1    5

#define              BCC_2    7

#define              BCC_3    1

#define              BCC_4    3

#define              BCC_5    5

#define              BCC_6    7

#define              BCC_7    1

#define              BSIC_0   ((NCC_0<<3)|(BCC_0))

#define              BSIC_1   ((NCC_1<<3)|(BCC_1))

#define              BSIC_2   ((NCC_2<<3)|(BCC_2))

#define              BSIC_3   ((NCC_3<<3)|(BCC_3))

#define              BSIC_4   ((NCC_4<<3)|(BCC_4))

#define              BSIC_5   ((NCC_5<<3)|(BCC_5))

#define              BSIC_6   ((NCC_6<<3)|(BCC_6))

#define              BSIC_7   ((NCC_7<<3)|(BCC_7))

#define              ARFCN_0    2

#define              ARFCN_1    8

#define              ARFCN_2   14

#define              ARFCN_3   20

#define              ARFCN_4   26

#define              ARFCN_5   32

#define              ARFCN_6   38

#define              ARFCN_7   44

#define              PWRLEV_0  -60

#define              PWRLEV_1  -85

#define              PWRLEV_2  -80

#define              PWRLEV_3  -75

#define              PWRLEV_4  -55

#define              PWRLEV_5  -50

#define              PWRLEV_6  -45

#define              PWRLEV_7  -40

BS_SET_SCH           ( 0,  BSIC_0,RFN)

BS_SET_ARFCN         ( 0, ARFCN_0)

BS_SET_POWER         ( 0,PWRLEV_0)

BS_SET_SCH           ( 1,  BSIC_1,RFN)

BS_SET_ARFCN         ( 1, ARFCN_1)

BS_SET_POWER         ( 1,PWRLEV_1)

BS_SET_SCH           ( 2,  BSIC_2,RFN)

BS_SET_ARFCN         ( 2, ARFCN_2)

BS_SET_POWER         ( 2,PWRLEV_2)

BS_SET_SCH           ( 3,  BSIC_3,RFN)

BS_SET_ARFCN         ( 3, ARFCN_3)

BS_SET_POWER         ( 3,PWRLEV_3)

BS_SET_SCH           ( 4,  BSIC_4,RFN)

BS_SET_ARFCN         ( 4, ARFCN_4)

BS_SET_POWER         ( 4,PWRLEV_4)

BS_SET_SCH           ( 5,  BSIC_5,RFN)

BS_SET_ARFCN         ( 5, ARFCN_5)

BS_SET_POWER         ( 5,PWRLEV_5)

BS_SET_SCH           ( 6,  BSIC_6,RFN)

BS_SET_ARFCN         ( 6, ARFCN_6)

BS_SET_POWER         ( 6,PWRLEV_6)

BS_SET_SCH           ( 7,  BSIC_7,RFN)

BS_SET_ARFCN         ( 7, ARFCN_7)

BS_SET_POWER         ( 7,PWRLEV_7)

BS_ON_OFF	( 0,TRUE)

BS_ON_OFF	( 1,FALSE)

BS_ON_OFF	( 2,FALSE)

BS_ON_OFF	( 3,FALSE)

BS_ON_OFF	( 4,FALSE)

BS_ON_OFF	( 5,FALSE)

BS_ON_OFF	( 6,FALSE)

BS_ON_OFF	( 7,FALSE)

/*------------------------------------------------------------------*\

| Switch on IUT

\*------------------------------------------------------------------*/

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

SET_TIMEOUT         ( 30000)

/*------------------------------------------------------------------*\

| Message exchange

\*------------------------------------------------------------------*/

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (4000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

/* ---- end of preamble ---- */

BS_CONFIG_CHANNEL (0, SACCH, UNACK, SAPI_0)

/*

* not tested

* BS_MSG3_AWAIT     (0, measurement_report,"allow one unchecked report")

*/

ISS_DELAY (5000)

BS_MSG3_AWAIT_BEGIN (0, measurement_report,SILENT)

BF_SET_VAL       (ba_ind     ,1,"BA INDEX SET TO 1")

BF_SET_VAL       (num_ncell_m,0,"should be 0 OR 7 (IUT uses 0)")

BS_MSG3_AWAIT_END()

AT_SEND ("AT+CFUN=0"," Power Off");

BS_ON_OFF ( 0,FALSE)

ISS_DELAY (4000)

IE_BF_SET_VAL       ( neighbour_cell_description_26_6_3_1_3, ext_ind, 1," part of BA")

REPEAT_END(K)

History:	16.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR151: Measurement / all neighbours present (26.6.3.2)

Description:	(Ref.: GSM 11.10-1, §26.6.3.2)

Preamble:	None

Script:

IE_BEGIN(measurement_results)

BF(1,   1,ACT_CHECK ,ba_ind                  ,SILENT)

BF(1,   0,ACT_CHECK ,dtx                     ,"DTX was not used")

BF(6,0x3f,ACT_CHECK ,rxlev_full_serving_cell ,SILENT)

BF(1,   0,ACT_CHECK ,ANONYMOUS               ,"spare")

BF(1,   0,ACT_CHECK ,meas_valid              ,"valid within the 2nd SACCH"

"   block at the latest")

BF(6,0x3f,ACT_CHECK ,rxlev_sub_serving_cell  ,SILENT)

BF(1,   0,ACT_CHECK ,ANONYMOUS               ,"spare")

BF(3,   0,ACT_CHECK ,rxqual_full_serving_cell,SILENT)

BF(3,   0,ACT_CHECK ,rxqual_sub_serving_cell ,SILENT)

BF(3,   6,ACT_CHECK ,num_ncell_m             ,"num of neighbor cells")

BF(6,0x3F,ACT_CHECK ,    rxlev_ncell_1       ,SILENT)

BF(5,0x00,ACT_CHECK ,bcch_freq_ncell_1       ,SILENT)

BF(6,0x0b,ACT_CHECK ,     bsic_ncell_1       ,SILENT)

BF(6,0x3f,ACT_CHECK ,    rxlev_ncell_2       ,SILENT)

BF(5,0x1f,ACT_CHECK ,bcch_freq_ncell_2       ,SILENT)

BF(6,0x09,ACT_CHECK ,     bsic_ncell_2       ,SILENT)

BF(6,0x3f,ACT_CHECK ,    rxlev_ncell_3       ,SILENT)

BF(5,0x1d,ACT_CHECK ,bcch_freq_ncell_3       ,SILENT)

BF(6,0x0f,ACT_CHECK ,     bsic_ncell_3       ,SILENT)

BF(6,0x3c,ACT_CHECK ,    rxlev_ncell_4       ,SILENT)

BF(5,0x19,ACT_CHECK ,bcch_freq_ncell_4       ,SILENT)

BF(6,0x0d,ACT_CHECK ,     bsic_ncell_4       ,SILENT)

BF(6,0x37,ACT_CHECK ,    rxlev_ncell_5       ,SILENT)

BF(5,0x15,ACT_CHECK ,bcch_freq_ncell_5       ,SILENT)

BF(6,0x0b,ACT_CHECK ,     bsic_ncell_5       ,SILENT)

BF(6,0x23,ACT_CHECK ,    rxlev_ncell_6       ,SILENT)

BF(5,0x10,ACT_CHECK ,bcch_freq_ncell_6       ,SILENT)

BF(6,0x09,ACT_CHECK ,     bsic_ncell_6       ,SILENT)

IE_END(measurement_results)

IE_BEGIN(neighbour_cell_description_26_6_3_2_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x000008ae,ACT_CHECK,ba_064_033,"ARFCN 34,35,36,38,40,44")
   BF(32,0xbaffbffa,ACT_CHECK,ba_032_001,"all except : 31,27,25,15,3,1")

IE_END(neighbour_cell_description_26_6_3_2_3)

IE_BEGIN(neighbour_cell_description_5bis_26_6_3_2_3)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         1,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         1,ACT_CHECK,f0       ,"frequency 0 indicator (F0 included)")
   BF(10,       800,ACT_CHECK,w1       ,"channel 800 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_5bis_26_6_3_2_3)

MSG3_BEGIN(measurement_report)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(measurement_report_message_type)

IE(measurement_results)

MSG3_END(measurement_report)

MSG3_BEGIN(system_information_type_5_26_6_3_2_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(neighbour_cell_description_26_6_3_2_3)

MSG3_END(system_information_type_5_26_6_3_2_3)

MSG3_BEGIN(system_information_type_5bis_26_6_3_2_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5bis_message_type)

IE(neighbour_cell_description_5bis_26_6_3_2_3)

MSG3_END(system_information_type_5bis_26_6_3_2_3)

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

IE_BEGIN(authentication_parameter_rand)

BF(32,0x80000000,ACT_CHECK,rand_127_096,SILENT)

BF(32,0x00000012,ACT_CHECK,rand_095_064,SILENT)

BF(32,0x34000000,ACT_CHECK,rand_063_032,SILENT)

BF(32,0x0000000F,ACT_CHECK,rand_031_000,SILENT)

IE_END(authentication_parameter_rand)

IE_BEGIN(authentication_parameter_sres)

BF(32,0x0000000F,ACT_SHOW,sres_031_000,SILENT)

IE_END(authentication_parameter_sres)

IE_BEGIN(call_control_protocol_discriminator)

BF(4, 3,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_control_protocol_discriminator)

IE_BEGIN(called_party_bcd_number)

BF( 8,7,ACT_CHECK,length                     ,"length of IE")

BF( 1,1,ACT_CHECK,ext                        ,"Extension Bit")

BF( 3,0,ACT_CHECK,type_of_number             ,"Unknown")

BF( 4,1,ACT_CHECK,numbering_plan             ,"ISDN/telephony")

BF( 4,3,ACT_CHECK,digit_2                    ,"Digit 2")

BF( 4,0,ACT_CHECK,digit_1                    ,"Digit 1")

BF( 4,3,ACT_CHECK,digit_4                    ,"Digit 4")

BF( 4,0,ACT_CHECK,digit_3                    ,"Digit 3")

BF( 4,0,ACT_CHECK,digit_6                    ,"Digit 6")

BF( 4,9,ACT_CHECK,digit_5                    ,"Digit 5")

BF( 4,4,ACT_CHECK,digit_8                    ,"Digit 8")

BF( 4,9,ACT_CHECK,digit_7                    ,"Digit 7")

BF( 4,1,ACT_CHECK,digit_10                   ,"Digit 10")

BF( 4,1,ACT_CHECK,digit_9                   ,"Digit 9")

BF( 4,7,ACT_CHECK,digit_12                  ,"Digit 12")

BF( 4,0xF,ACT_CHECK,digit_11                ,"Digit 11")

IE_END(called_party_bcd_number)

IE_BEGIN(channels_needed_for_mobiles_1_and_2)

BF(2,0,ACT_CHECK,second_channel,"spare, any channel")

BF(2,0,ACT_CHECK, first_channel,"spare, any channel")

IE_END(channels_needed_for_mobiles_1_and_2)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,1),ACT_CHECK,          channel_type,"SDCCH/SACCH 4(1)")

BF( 3,            0,ACT_CHECK,      time_slot_number,"zero")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(channel_description)

IE_BEGIN(ciphering_key_sequence_number_2)

BF(1,        0,ACT_CHECK,       spare,SILENT)

BF(3,M3(1,0,1),ACT_CHECK,key_sequence,"sent BS->MS")

IE_END(ciphering_key_sequence_number_2)

IE_BEGIN(ciphering_mode_setting)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(ciphering_mode_setting)

IE_BEGIN(cipher_mode_setting_a5_1)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(cipher_mode_setting_a5_1)

IE_BEGIN(cipher_mode_setting_a5_2)

BF(3,M3(0,0,1),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(cipher_mode_setting_a5_2)

IE_BEGIN(cipher_mode_setting_none)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        0,ACT_CHECK,     start_ciphering,"not start ciphering")

IE_END(cipher_mode_setting_none)

IE_BEGIN(cipher_response)

BF(3,0,ACT_CHECK,          spare,SILENT)

BF(1,0,ACT_CHECK,cipher_response,"IMEISV shall not be included")

IE_END(cipher_response)

IE_BEGIN(description_of_the_first_channel_after_time)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            3,ACT_CHECK,      time_slot_number,"three")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(description_of_the_first_channel_after_time)

IE_BEGIN(ia_rest_octets)   /* maximum length (11), no hop, no start time */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(ia_rest_octets)

IE_BEGIN(iei_5E)

BF(8,0x5E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_5E)

IE_BEGIN(iei_15)

BF(8,0x15,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_15)

IE_BEGIN(iei_04)

BF(8,0x04,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_04)

IE_BEGIN(iei_34)

BF(8,0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_34)

IE_BEGIN(l2_pseudo_length_11)  /* pag req type 1 with TMSI (one mobile)*/

BF(8, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_11)

IE_BEGIN(mobile_allocation)

BF(8,0,ACT_CHECK,length,"length 0 due to hopping disabled")

IE_END(mobile_allocation)

IE_BEGIN(mobile_identity)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_4,SILENT)

BF(4,          0,ACT_CHECK, digit_3,SILENT)

BF(4,          7,ACT_CHECK, digit_6,SILENT)

BF(4,          4,ACT_CHECK, digit_5,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          1,ACT_CHECK, digit_7,SILENT)

BF(4,          9,ACT_CHECK, digit_10,SILENT)

BF(4,          4,ACT_CHECK, digit_9,SILENT)

BF(4,          2,ACT_CHECK, digit_12,SILENT)

BF(4,          1,ACT_CHECK, digit_11,SILENT)

IE_END(mobile_identity)

IE_BEGIN(mobility_management_protocol_discriminator)

BF(4, 5,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobility_management_protocol_discriminator)

IE_BEGIN(ms_classmark)

BF(8,        3,ACT_CHECK, length,SILENT)

BF(1,        0,ACT_CHECK, spare,SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,"phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,"Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,"encryption algorithm A5/1 available")

BF(3,M3(0,0,1),ACT_CHECK, rf_power_capability,"class 2")

BF(1,        0,ACT_CHECK, spare2,SILENT)

BF(1,        0,ACT_CHECK, ps_capability,"no pseudo-synch capability")

BF(2,  M2(0,0),ACT_CHECK, ss_screening_indicator,"default phase 1")

BF(1,        0,ACT_CHECK, sm_capability,"no point to point SMS")

BF(1,        0,ACT_CHECK, vbs,"no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,"no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"no extention band G1")

BF(1,        0,ACT_CHECK, classmark_3,"no add. MS cap. information")

BF(5,        1,ACT_CHECK, ccbs,SILENT)

BF(1,        0,ACT_CHECK, a5_3,"A5/3 not available")

BF(1,        0,ACT_CHECK, a5_2,"A5/2 not available")

IE_END(ms_classmark)

IE_BEGIN(p1_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(p1_rest_octets)

IE_BEGIN(page_mode)

BF(2,0,ACT_CHECK,spare,"two spare bits")

BF(2,0,ACT_CHECK,pm   ,"Normal Paging")

IE_END(page_mode)

IE_BEGIN(power_command)

BF(8,10,ACT_CHECK,power,SILENT)

IE_END(power_command)

IE_BEGIN(rach)

BF(3,M3(1,0,0),ACT_CHECK ,establishment_cause,"paging ind. any chan'")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

IE_BEGIN(request_reference)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,"As in CHAN REQ")

BF(5,M5(1,1,1,1,1),ACT_SHOW,  random_reference,SILENT)

BF(5,            0,ACT_SHOW,               t1_,SILENT)

BF(6,            0,ACT_SHOW,               t3 ,SILENT)

BF(5,            0,ACT_SHOW,               t2 ,SILENT)

IE_END(request_reference)

IE_BEGIN(rr_cause)

BF(8,0,ACT_CHECK,rr_cause,"normal event")

IE_END(rr_cause)

IE_BEGIN(signal_call_waiting)

BF(8,M8(0,0,0,0,0,1,1,1),ACT_CHECK,signal_value,"(Any non-res. value)")

IE_END(signal_call_waiting)

IE_BEGIN(spare_half_octet)

BF(4, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(spare_half_octet)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,         spare,SILENT)

BF(6,0,ACT_CHECK,timing_advance,"0")

IE_END(timing_advance)

IE_BEGIN(transaction_identifier_source)

BF(4,M4(0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source)

IE_BEGIN(transaction_identifier_dest)

BF(4,M4(1,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest)

/*------------------------------------------------------------------*\

| Messages

\*------------------------------------------------------------------*/

MSG3_BEGIN(alerting)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(alerting_message_type)

MSG3_END(alerting)

MSG3_BEGIN(assignment_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(assignment_command)

MSG3_BEGIN(assignment_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_complete_message_type)

IE(rr_cause)

MSG3_END(assignment_complete)

MSG3_BEGIN(authentication_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(authentication_parameter_rand)

MSG3_END(authentication_request)

MSG3_BEGIN(authentication_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_response_message_type)

IE(authentication_parameter_sres)

MSG3_END(authentication_response)

MSG3_BEGIN(ciphering_mode_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(cipher_response)

IE(ciphering_mode_setting)

MSG3_END(ciphering_mode_command)

MSG3_BEGIN(ciphering_mode_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

MSG3_END(ciphering_mode_complete)

MSG3_BEGIN(connect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(connect_message_type)

MSG3_END(connect)

MSG3_BEGIN(connect_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(connect_acknowledge_message_type)

MSG3_END(connect_acknowledge)

MSG3_BEGIN(setup)            /* contains 'signal' but no 'Bearer Cap' */

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

MSG3_END(setup)

/* ---- start testcase -------- */

REPEAT(K,2)

ISS_INIT	( 8)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

REPEAT(N,8)

IE_BF_SET_VAL         ( cell_identity, ci, N," ci used for SI 3 and SI 6")

IE_BF_SET_VAL         ( neighbour_cell_description, ba_ind,  0," BA for BCCH")

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 0, SILENT)

BS_SET_SYS_INFO       ( N, system_information_type_1)

BS_SET_SYS_INFO       ( N, system_information_type_2)

BS_SET_SYS_INFO       ( N, system_information_type_3)

BS_SET_SYS_INFO       ( N, system_information_type_4)

IE_BF_SET_VAL         ( neighbour_cell_description,  ba_ind, 1," BA for SACCH")

if ( K == 1)

{

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 1,"")

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5bis_26_6_3_2_3)

}

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5_26_6_3_2_3)

BS_SET_SYS_INFO_SACCH ( N, system_information_type_6)

REPEAT_END(N)

#define              NCC_0    1

#define              NCC_1    1

#define              NCC_2    1

#define              NCC_3    1

#define              NCC_4    1

#define              NCC_5    1

#define              NCC_6    1

#define              NCC_7    1

#define              BCC_0    3

#define              BCC_1    5

#define              BCC_2    7

#define              BCC_3    1

#define              BCC_4    3

#define              BCC_5    5

#define              BCC_6    7

#define              BCC_7    1

#define              BSIC_0   ((NCC_0<<3)|(BCC_0))

#define              BSIC_1   ((NCC_1<<3)|(BCC_1))

#define              BSIC_2   ((NCC_2<<3)|(BCC_2))

#define              BSIC_3   ((NCC_3<<3)|(BCC_3))

#define              BSIC_4   ((NCC_4<<3)|(BCC_4))

#define              BSIC_5   ((NCC_5<<3)|(BCC_5))

#define              BSIC_6   ((NCC_6<<3)|(BCC_6))

#define              BSIC_7   ((NCC_7<<3)|(BCC_7))

#define              ARFCN_0    2

#define              ARFCN_1    8

#define              ARFCN_2   14

#define              ARFCN_3   20

#define              ARFCN_4   26

#define              ARFCN_5   32

#define              ARFCN_6   38

#define              ARFCN_7   44

#define              PWRLEV_0  -35	/* force to be serving cell (was -60) */

#define              PWRLEV_1  -85

#define              PWRLEV_2  -80

#define              PWRLEV_3  -75

#define              PWRLEV_4  -55

#define              PWRLEV_5  -50

#define              PWRLEV_6  -45

#define              PWRLEV_7  -40

BS_SET_SCH           ( 0,  BSIC_0,RFN)

BS_SET_ARFCN         ( 0, ARFCN_0)

BS_SET_POWER         ( 0,PWRLEV_0)

BS_SET_SCH           ( 1,  BSIC_1,RFN)

BS_SET_ARFCN         ( 1, ARFCN_1)

BS_SET_POWER         ( 1,PWRLEV_1)

BS_SET_SCH           ( 2,  BSIC_2,RFN)

BS_SET_ARFCN         ( 2, ARFCN_2)

BS_SET_POWER         ( 2,PWRLEV_2)

BS_SET_SCH           ( 3,  BSIC_3,RFN)

BS_SET_ARFCN         ( 3, ARFCN_3)

BS_SET_POWER         ( 3,PWRLEV_3)

BS_SET_SCH           ( 4,  BSIC_4,RFN)

BS_SET_ARFCN         ( 4, ARFCN_4)

BS_SET_POWER         ( 4,PWRLEV_4)

BS_SET_SCH           ( 5,  BSIC_5,RFN)

BS_SET_ARFCN         ( 5, ARFCN_5)

BS_SET_POWER         ( 5,PWRLEV_5)

BS_SET_SCH           ( 6,  BSIC_6,RFN)

BS_SET_ARFCN         ( 6, ARFCN_6)

BS_SET_POWER         ( 6,PWRLEV_6)

BS_SET_SCH           ( 7,  BSIC_7,RFN)

BS_SET_ARFCN         ( 7, ARFCN_7)

BS_SET_POWER         ( 7,PWRLEV_7)

BS_ON_OFF	( 0,TRUE)

BS_ON_OFF	( 1,TRUE)

BS_ON_OFF	( 2,TRUE)

BS_ON_OFF	( 3,TRUE)

BS_ON_OFF	( 4,TRUE)

BS_ON_OFF	( 5,TRUE)

BS_ON_OFF	( 6,TRUE)

BS_ON_OFF	( 7,TRUE)

/*------------------------------------------------------------------*\

| Switch on IUT

\*------------------------------------------------------------------*/

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

SET_TIMEOUT         ( 30000)

/*------------------------------------------------------------------*\

| Message exchange

\*------------------------------------------------------------------*/

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

/* ---- end of preamble ---- */

ISS_DELAY (10000)

BS_CONFIG_CHANNEL (0, SACCH, UNACK, SAPI_0)

BS_MSG3_AWAIT     (0, measurement_report,"allow one unchecked report")

REPEAT(COUNT,10)

BS_MSG3_AWAIT_BEGIN (0, measurement_report,SILENT)

BF_SET_VAL       (dtx              ,0   ,"DTX was not used")

BF_SET_VAL       (meas_valid       ,0   ,"measurement vaild (0)")

BF_SET_VAL       (ba_ind           ,1   ,"BA INDEX SET TO 1")

BF_SET_VAL       (num_ncell_m      ,6   ,"all neighbour present")

BF_SET_VAL       (bcch_freq_ncell_1,0x00,"refers to ARFCN 2")

BF_SET_VAL       (bcch_freq_ncell_2,0x1f,"refers to ARFCN 44")

BF_SET_VAL       (bcch_freq_ncell_3,0x1d,"refers to ARFCN 38")

BF_SET_VAL       (bcch_freq_ncell_4,0x19,"refers to ARFCN 32")

BF_SET_VAL       (bcch_freq_ncell_5,0x15,"refers to ARFCN 26")

BF_SET_VAL       (bcch_freq_ncell_6,0x10,"refers to ARFCN 20")

BS_MSG3_AWAIT_END()

REPEAT_END(COUNT)

AT_SEND ("AT+CFUN=0"," Power Off");

BS_ON_OFF	( 0,FALSE)

BS_ON_OFF	( 1,FALSE)

BS_ON_OFF	( 2,FALSE)

BS_ON_OFF	( 3,FALSE)

BS_ON_OFF	( 4,FALSE)

BS_ON_OFF	( 5,FALSE)

BS_ON_OFF	( 6,FALSE)

BS_ON_OFF	( 7,FALSE)

ISS_DELAY (10000);

IE_BF_SET_VAL       ( neighbour_cell_description_26_6_3_2_3, ext_ind, 1," part of BA")

REPEAT_END(K)

History:	22.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR152: Multiple Call Setup Procedure for Mobile Terminated Calls

Description:	Do the MTC several times.

Preamble:	None

Script:

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

BS_SET_SYS_INFO       ( 0, system_information_type_1)

BS_SET_SYS_INFO       ( 0, system_information_type_2)

BS_SET_SYS_INFO       ( 0, system_information_type_3)

BS_SET_SYS_INFO       ( 0, system_information_type_4)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

REPEAT(COUNT,3)

BS_ON_OFF       ( 0,TRUE)

#include        <MRR135.tds>

ISS_DELAY       (10000)

AT_SEND ("AT+CFUN=0"," Power Off");

ISS_DELAY       (10000)

BS_ON_OFF       ( 0,FALSE)

ISS_DELAY       (4000)

COMMAND         ("MMI MEMCHECK")

REPEAT_END(COUNT)

History:	26.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR153: Measurement / barred cells and non-permitted NCCs (26.6.3.3)

Description:	(Ref.: GSM 11.10-1, §26.6.3.3)

Preamble:	None

Script:

IE_BEGIN(measurement_results)

BF(1,   1,ACT_CHECK  ,ba_ind                  ,SILENT)

BF(1,   0,ACT_CHECK  ,dtx                     ,"DTX was not used")

BF(6,0x3f,ACT_CHECK  ,rxlev_full_serving_cell ,SILENT)

BF(1,   0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(1,   0,ACT_CHECK  ,meas_valid              ,"valid within the 2nd SACCH"

"   block at the latest")

BF(6,0x3f,ACT_CHECK  ,rxlev_sub_serving_cell  ,SILENT)

BF(1,   0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(3,   0,ACT_CHECK  ,rxqual_full_serving_cell,SILENT)

BF(3,   0,ACT_CHECK  ,rxqual_sub_serving_cell ,SILENT)

BF(3,   4,ACT_CHECK ,num_ncell_m             ,"num of neighbor cells")

BF(6,0x3f,ACT_CHECK ,    rxlev_ncell_1       ,SILENT)

BF(5,   0,ACT_CHECK ,bcch_freq_ncell_1       ,SILENT)

BF(6,0x0b,ACT_CHECK ,     bsic_ncell_1       ,SILENT)

BF(6,0x3f,ACT_CHECK ,    rxlev_ncell_2       ,SILENT)

BF(5,   4,ACT_CHECK ,bcch_freq_ncell_2       ,SILENT)

BF(6,0x09,ACT_CHECK ,     bsic_ncell_2       ,SILENT)

BF(6,0x3f,ACT_CHECK ,    rxlev_ncell_3       ,SILENT)

BF(5,0x03,ACT_CHECK ,bcch_freq_ncell_3       ,SILENT)

BF(6,0x0F,ACT_CHECK ,     bsic_ncell_3       ,SILENT)

BF(6,0x1E,ACT_CHECK ,    rxlev_ncell_4       ,SILENT)

BF(5,0x01,ACT_CHECK ,bcch_freq_ncell_4       ,SILENT)

BF(6,0x0F,ACT_CHECK ,     bsic_ncell_4       ,SILENT)

BF(6,   0,ACT_CHECK ,    rxlev_ncell_5       ,SILENT)

BF(5,   0,ACT_CHECK ,bcch_freq_ncell_5       ,SILENT)

BF(6,   0,ACT_CHECK ,     bsic_ncell_5       ,SILENT)

BF(6,   0,ACT_CHECK ,    rxlev_ncell_6       ,SILENT)

BF(5,   0,ACT_CHECK ,bcch_freq_ncell_6       ,SILENT)

BF(6,   0,ACT_CHECK ,     bsic_ncell_6       ,SILENT)

IE_END(measurement_results)

IE_BEGIN(neighbour_cell_description_26_6_3_3_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000820,ACT_CHECK,ba_064_033,"ARFCN 44, 38")
   BF(32,0x00082002,ACT_CHECK,ba_032_001,"ARFCN 20, 14,  2")

IE_END(neighbour_cell_description_26_6_3_3_3)

IE_BEGIN(neighbour_cell_description_5bis_26_6_3_3_3)  /* same as 26_6_3_2_3 */
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         1,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         1,ACT_CHECK,f0       ,"frequency 0 indicator (F0 included)")
   BF(10,       800,ACT_CHECK,w1       ,"channel 800 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_5bis_26_6_3_3_3)

MSG3_BEGIN(measurement_report)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(measurement_report_message_type)

IE(measurement_results)

MSG3_END(measurement_report)

MSG3_BEGIN(system_information_type_5_26_6_3_3_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(neighbour_cell_description_26_6_3_3_3)

MSG3_END(system_information_type_5_26_6_3_3_3)

MSG3_BEGIN(system_information_type_5bis_26_6_3_3_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5bis_message_type)

IE(neighbour_cell_description_5bis_26_6_3_3_3)

MSG3_END(system_information_type_5bis_26_6_3_3_3)

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

IE_BEGIN(authentication_parameter_rand)

BF(32,0x80000000,ACT_CHECK,rand_127_096,SILENT)

BF(32,0x00000012,ACT_CHECK,rand_095_064,SILENT)

BF(32,0x34000000,ACT_CHECK,rand_063_032,SILENT)

BF(32,0x0000000F,ACT_CHECK,rand_031_000,SILENT)

IE_END(authentication_parameter_rand)

IE_BEGIN(authentication_parameter_sres)

BF(32,0x0000000F,ACT_SHOW,sres_031_000,SILENT)

IE_END(authentication_parameter_sres)

IE_BEGIN(call_control_protocol_discriminator)

BF(4, 3,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_control_protocol_discriminator)

IE_BEGIN(called_party_bcd_number)

BF( 8,7,ACT_CHECK,length                     ,"length of IE")

BF( 1,1,ACT_CHECK,ext                        ,"Extension Bit")

BF( 3,0,ACT_CHECK,type_of_number             ,"Unknown")

BF( 4,1,ACT_CHECK,numbering_plan             ,"ISDN/telephony")

BF( 4,3,ACT_CHECK,digit_2                    ,"Digit 2")

BF( 4,0,ACT_CHECK,digit_1                    ,"Digit 1")

BF( 4,3,ACT_CHECK,digit_4                    ,"Digit 4")

BF( 4,0,ACT_CHECK,digit_3                    ,"Digit 3")

BF( 4,0,ACT_CHECK,digit_6                    ,"Digit 6")

BF( 4,9,ACT_CHECK,digit_5                    ,"Digit 5")

BF( 4,4,ACT_CHECK,digit_8                    ,"Digit 8")

BF( 4,9,ACT_CHECK,digit_7                    ,"Digit 7")

BF( 4,1,ACT_CHECK,digit_10                   ,"Digit 10")

BF( 4,1,ACT_CHECK,digit_9                   ,"Digit 9")

BF( 4,7,ACT_CHECK,digit_12                  ,"Digit 12")

BF( 4,0xF,ACT_CHECK,digit_11                ,"Digit 11")

IE_END(called_party_bcd_number)

IE_BEGIN(channels_needed_for_mobiles_1_and_2)

BF(2,0,ACT_CHECK,second_channel,"spare, any channel")

BF(2,0,ACT_CHECK, first_channel,"spare, any channel")

IE_END(channels_needed_for_mobiles_1_and_2)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,1),ACT_CHECK,          channel_type,"SDCCH/SACCH 4(1)")

BF( 3,            0,ACT_CHECK,      time_slot_number,"zero")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(channel_description)

IE_BEGIN(ciphering_mode_setting)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(ciphering_mode_setting)

IE_BEGIN(cipher_response)

BF(3,0,ACT_CHECK,          spare,SILENT)

BF(1,0,ACT_CHECK,cipher_response,"IMEISV shall not be included")

IE_END(cipher_response)

IE_BEGIN(description_of_the_first_channel_after_time)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            3,ACT_CHECK,      time_slot_number,"three")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(description_of_the_first_channel_after_time)

IE_BEGIN(ia_rest_octets)   /* maximum length (11), no hop, no start time */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(ia_rest_octets)

IE_BEGIN(iei_5E)

BF(8,0x5E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_5E)

IE_BEGIN(iei_15)

BF(8,0x15,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_15)

IE_BEGIN(iei_04)

BF(8,0x04,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_04)

IE_BEGIN(iei_34)

BF(8,0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_34)

IE_BEGIN(l2_pseudo_length_11)  /* pag req type 1 with TMSI (one mobile)*/

BF(8, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_11)

IE_BEGIN(mobile_allocation)

BF(8,0,ACT_CHECK,length,"length 0 due to hopping disabled")

IE_END(mobile_allocation)

IE_BEGIN(mobile_identity)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_4,SILENT)

BF(4,          0,ACT_CHECK, digit_3,SILENT)

BF(4,          7,ACT_CHECK, digit_6,SILENT)

BF(4,          4,ACT_CHECK, digit_5,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          1,ACT_CHECK, digit_7,SILENT)

BF(4,          9,ACT_CHECK, digit_10,SILENT)

BF(4,          4,ACT_CHECK, digit_9,SILENT)

BF(4,          2,ACT_CHECK, digit_12,SILENT)

BF(4,          1,ACT_CHECK, digit_11,SILENT)

IE_END(mobile_identity)

IE_BEGIN(mobility_management_protocol_discriminator)

BF(4, 5,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobility_management_protocol_discriminator)

IE_BEGIN(ms_classmark)

BF(8,        3,ACT_CHECK, length,SILENT)

BF(1,        0,ACT_CHECK, spare,SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,"phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,"Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,"encryption algorithm A5/1 available")

BF(3,M3(0,0,1),ACT_CHECK, rf_power_capability,"class 2")

BF(1,        0,ACT_CHECK, spare2,SILENT)

BF(1,        0,ACT_CHECK, ps_capability,"no pseudo-synch capability")

BF(2,  M2(0,0),ACT_CHECK, ss_screening_indicator,"default phase 1")

BF(1,        0,ACT_CHECK, sm_capability,"no point to point SMS")

BF(1,        0,ACT_CHECK, vbs,"no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,"no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"no extention band G1")

BF(1,        0,ACT_CHECK, classmark_3,"no add. MS cap. information")

BF(5,        1,ACT_CHECK, ccbs,SILENT)

BF(1,        0,ACT_CHECK, a5_3,"A5/3 not available")

BF(1,        0,ACT_CHECK, a5_2,"A5/2 not available")

IE_END(ms_classmark)

IE_BEGIN(neighbour_cell_description_26_6_3_1_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_032_001,"no neigbours")
IE_END(neighbour_cell_description_26_6_3_1_3)

IE_BEGIN(p1_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(p1_rest_octets)

IE_BEGIN(page_mode)

BF(2,0,ACT_CHECK,spare,"two spare bits")

BF(2,0,ACT_CHECK,pm   ,"Normal Paging")

IE_END(page_mode)

IE_BEGIN(power_command)

BF(8,10,ACT_CHECK,power,SILENT)

IE_END(power_command)

IE_BEGIN(rach)

BF(3,M3(1,0,0),ACT_CHECK ,establishment_cause,"paging ind. any chan'")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

IE_BEGIN(request_reference)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,"As in CHAN REQ")

BF(5,M5(1,1,1,1,1),ACT_SHOW,  random_reference,SILENT)

BF(5,            0,ACT_SHOW,               t1_,SILENT)

BF(6,            0,ACT_SHOW,               t3 ,SILENT)

BF(5,            0,ACT_SHOW,               t2 ,SILENT)

IE_END(request_reference)

IE_BEGIN(rr_cause)

BF(8,0,ACT_CHECK,rr_cause,"normal event")

IE_END(rr_cause)

IE_BEGIN(signal_call_waiting)

BF(8,M8(0,0,0,0,0,1,1,1),ACT_CHECK,signal_value,"(Any non-res. value)")

IE_END(signal_call_waiting)

IE_BEGIN(spare_half_octet)

BF(4, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(spare_half_octet)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,         spare,SILENT)

BF(6,0,ACT_CHECK,timing_advance,"0")

IE_END(timing_advance)

IE_BEGIN(transaction_identifier_source)

BF(4,M4(0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source)

IE_BEGIN(transaction_identifier_dest)

BF(4,M4(1,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest)

/*------------------------------------------------------------------*\

| Messages

\*------------------------------------------------------------------*/

MSG3_BEGIN(alerting)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(alerting_message_type)

MSG3_END(alerting)

MSG3_BEGIN(assignment_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(assignment_command)

MSG3_BEGIN(assignment_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_complete_message_type)

IE(rr_cause)

MSG3_END(assignment_complete)

MSG3_BEGIN(authentication_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(authentication_parameter_rand)

MSG3_END(authentication_request)

MSG3_BEGIN(authentication_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_response_message_type)

IE(authentication_parameter_sres)

MSG3_END(authentication_response)

MSG3_BEGIN(ciphering_mode_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(cipher_response)

IE(ciphering_mode_setting)

MSG3_END(ciphering_mode_command)

MSG3_BEGIN(ciphering_mode_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

MSG3_END(ciphering_mode_complete)

MSG3_BEGIN(connect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(connect_message_type)

MSG3_END(connect)

MSG3_BEGIN(connect_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(connect_acknowledge_message_type)

MSG3_END(connect_acknowledge)

MSG3_BEGIN(setup)            /* contains 'signal' but no 'Bearer Cap' */

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

MSG3_END(setup)

/* ------ Start of Testcase ----- */

REPEAT(K,2)

ISS_INIT	( 8)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

REPEAT(N,8)

IE_BF_SET_VAL         ( cell_identity, ci, N," ci used for SI 3 and SI 6")

IE_BF_SET_VAL         ( neighbour_cell_description, ba_ind,  0," BA for BCCH")

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 0, SILENT)

BS_SET_SYS_INFO       ( N, system_information_type_1)

BS_SET_SYS_INFO       ( N, system_information_type_2)

BS_SET_SYS_INFO       ( N, system_information_type_3)

BS_SET_SYS_INFO       ( N, system_information_type_4)

IE_BF_SET_VAL         ( neighbour_cell_description,  ba_ind, 1," BA for SACCH")

if ( K == 1)

{

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 1,"")

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5bis_26_6_3_3_3)

}

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( N, system_information_type_6)

REPEAT_END(N)

#define              NCC_0    1

#define              NCC_1    1

#define              NCC_2    1

#define              NCC_3    2

#define              NCC_4    3

#define              NCC_5    4

#define              NCC_6    1

#define              NCC_7    1

#define              BCC_0    3

#define              BCC_1    5

#define              BCC_2    7

#define              BCC_3    1

#define              BCC_4    3

#define              BCC_5    5

#define              BCC_6    7

#define              BCC_7    1

#define              BSIC_0   ((NCC_0<<3)|(BCC_0))

#define              BSIC_1   ((NCC_1<<3)|(BCC_1))

#define              BSIC_2   ((NCC_2<<3)|(BCC_2))

#define              BSIC_3   ((NCC_3<<3)|(BCC_3))

#define              BSIC_4   ((NCC_4<<3)|(BCC_4))

#define              BSIC_5   ((NCC_5<<3)|(BCC_5))

#define              BSIC_6   ((NCC_6<<3)|(BCC_6))

#define              BSIC_7   ((NCC_7<<3)|(BCC_7))

#define              ARFCN_0    2

#define              ARFCN_1    8

#define              ARFCN_2   14

#define              ARFCN_3   20

#define              ARFCN_4   26

#define              ARFCN_5   32

#define              ARFCN_6   38

#define              ARFCN_7   44

#define              PWRLEV_0  -35	/* force to be serving cell (was -60) */

#define              PWRLEV_1  -85

#define              PWRLEV_2  -80

#define              PWRLEV_3  -75

#define              PWRLEV_4  -55

#define              PWRLEV_5  -50

#define              PWRLEV_6  -45

#define              PWRLEV_7  -40

BS_SET_SCH           ( 0,  BSIC_0,RFN)

BS_SET_ARFCN         ( 0, ARFCN_0)

BS_SET_POWER         ( 0,PWRLEV_0)

BS_SET_SCH           ( 1,  BSIC_1,RFN)

BS_SET_ARFCN         ( 1, ARFCN_1)

BS_SET_POWER         ( 1,PWRLEV_1)

BS_SET_SCH           ( 2,  BSIC_2,RFN)

BS_SET_ARFCN         ( 2, ARFCN_2)

BS_SET_POWER         ( 2,PWRLEV_2)

BS_SET_SCH           ( 3,  BSIC_3,RFN)

BS_SET_ARFCN         ( 3, ARFCN_3)

BS_SET_POWER         ( 3,PWRLEV_3)

BS_SET_SCH           ( 4,  BSIC_4,RFN)

BS_SET_ARFCN         ( 4, ARFCN_4)

BS_SET_POWER         ( 4,PWRLEV_4)

BS_SET_SCH           ( 5,  BSIC_5,RFN)

BS_SET_ARFCN         ( 5, ARFCN_5)

BS_SET_POWER         ( 5,PWRLEV_5)

BS_SET_SCH           ( 6,  BSIC_6,RFN)

BS_SET_ARFCN         ( 6, ARFCN_6)

BS_SET_POWER         ( 6,PWRLEV_6)

BS_SET_SCH           ( 7,  BSIC_7,RFN)

BS_SET_ARFCN         ( 7, ARFCN_7)

BS_SET_POWER         ( 7,PWRLEV_7)

BS_ON_OFF	( 0,TRUE)

BS_ON_OFF	( 1,TRUE)

BS_ON_OFF	( 2,TRUE)

BS_ON_OFF	( 3,TRUE)

BS_ON_OFF	( 4,TRUE)

BS_ON_OFF	( 5,TRUE)

BS_ON_OFF	( 6,TRUE)

BS_ON_OFF	( 7,TRUE)

/*------------------------------------------------------------------*\

| Switch on IUT

\*------------------------------------------------------------------*/

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

SET_TIMEOUT         ( 30000)

/*------------------------------------------------------------------*\

| Message exchange

\*------------------------------------------------------------------*/

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

/* ---- end of preamble ---- */

/*

* BS_CONFIG_CHANNEL (0, SACCH, UNACK, SAPI_0)

* BS_MSG3_AWAIT     (0, measurement_report,"allow one unchecked report")

*

* not checked

*/

ISS_DELAY (10000)

REPEAT(COUNT,10)

BS_CONFIG_CHANNEL (0, SACCH, UNACK, SAPI_0)

BS_MSG3_AWAIT_BEGIN (0, measurement_report,SILENT)

BF_SET_VAL       (dtx              ,0   ,"DTX was not used")

BF_SET_VAL       (meas_valid       ,0   ,"measurement vaild (0)")

BF_SET_VAL       (ba_ind           ,1   ,"BA INDEX SET TO 1")

BF_SET_VAL       (num_ncell_m      ,4   ,"number of neighbours measured")

BF_SET_VAL       (bcch_freq_ncell_1,0x00,"refers to ARFCN 2")

BF_SET_VAL       (bcch_freq_ncell_2,0x04,"refers to ARFCN 44")

BF_SET_VAL       (bcch_freq_ncell_3,0x03,"refers to ARFCN 38")

BF_SET_VAL       (bcch_freq_ncell_4,0x01,"refers to ARFCN 14")

BS_MSG3_AWAIT_END()

REPEAT_END(COUNT)

AT_SEND ("AT+CFUN=0"," Power Off");

BS_ON_OFF	( 0,FALSE)

BS_ON_OFF	( 1,FALSE)

BS_ON_OFF	( 2,FALSE)

BS_ON_OFF	( 3,FALSE)

BS_ON_OFF	( 4,FALSE)

BS_ON_OFF	( 5,FALSE)

BS_ON_OFF	( 6,FALSE)

BS_ON_OFF	( 7,FALSE)

ISS_DELAY (10000);

IE_BF_SET_VAL       ( neighbour_cell_description_26_6_3_3_3, ext_ind, 1," part of BA")

REPEAT_END(K)

History:	27.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR154: Measurement / barred cells and non-permitted NCCs (26.6.3.3, K=1+2)

Description:	(Ref.: GSM 11.10-1, §26.6.3.3)

Preamble:	None

Script:

IE_BEGIN(measurement_results)

BF(1,   1,ACT_CHECK  ,ba_ind                  ,SILENT)

BF(1,   0,ACT_CHECK  ,dtx                     ,"DTX was not used")

BF(6,0x3f,ACT_CHECK  ,rxlev_full_serving_cell ,SILENT)

BF(1,   0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(1,   0,ACT_CHECK  ,meas_valid              ,"valid within the 2nd SACCH"

"   block at the latest")

BF(6,0x3f,ACT_CHECK  ,rxlev_sub_serving_cell  ,SILENT)

BF(1,   0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(3,   0,ACT_CHECK  ,rxqual_full_serving_cell,SILENT)

BF(3,   0,ACT_CHECK  ,rxqual_sub_serving_cell ,SILENT)

BF(3,   4,ACT_CHECK ,num_ncell_m             ,"num of neighbor cells")

BF(6,0x3f,ACT_CHECK ,    rxlev_ncell_1       ,SILENT)

BF(5,   0,ACT_CHECK ,bcch_freq_ncell_1       ,SILENT)

BF(6,0x0b,ACT_CHECK ,     bsic_ncell_1       ,SILENT)

BF(6,0x3f,ACT_CHECK ,    rxlev_ncell_2       ,SILENT)

BF(5,   4,ACT_CHECK ,bcch_freq_ncell_2       ,SILENT)

BF(6,0x09,ACT_CHECK ,     bsic_ncell_2       ,SILENT)

BF(6,0x3f,ACT_CHECK ,    rxlev_ncell_3       ,SILENT)

BF(5,0x03,ACT_CHECK ,bcch_freq_ncell_3       ,SILENT)

BF(6,0x0F,ACT_CHECK ,     bsic_ncell_3       ,SILENT)

BF(6,0x1E,ACT_CHECK ,    rxlev_ncell_4       ,SILENT)

BF(5,0x01,ACT_CHECK ,bcch_freq_ncell_4       ,SILENT)

BF(6,0x0F,ACT_CHECK ,     bsic_ncell_4       ,SILENT)

BF(6,   0,ACT_CHECK ,    rxlev_ncell_5       ,SILENT)

BF(5,   0,ACT_CHECK ,bcch_freq_ncell_5       ,SILENT)

BF(6,   0,ACT_CHECK ,     bsic_ncell_5       ,SILENT)

BF(6,   0,ACT_CHECK ,    rxlev_ncell_6       ,SILENT)

BF(5,   0,ACT_CHECK ,bcch_freq_ncell_6       ,SILENT)

BF(6,   0,ACT_CHECK ,     bsic_ncell_6       ,SILENT)

IE_END(measurement_results)

IE_BEGIN(neighbour_cell_description_26_6_3_3_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000820,ACT_CHECK,ba_064_033,"ARFCN 44, 38")
   BF(32,0x00082002,ACT_CHECK,ba_032_001,"ARFCN 20, 14,  2")

IE_END(neighbour_cell_description_26_6_3_3_3)

IE_BEGIN(neighbour_cell_description_26_6_3_3_3_k2)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         1,ACT_CHECK,ext_ind  ,"this IE carries the partBA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000820,ACT_CHECK,ba_064_033,"ARFCN 44, 38")
   BF(32,0x00082002,ACT_CHECK,ba_032_001,"ARFCN 20, 14,  2")

IE_END(neighbour_cell_description_26_6_3_3_3_k2)

IE_BEGIN(neighbour_cell_description_5bis_26_6_3_3_3)  /* same as 26_6_3_2_3 */
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         1,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         1,ACT_CHECK,f0       ,"frequency 0 indicator (F0 included)")
   BF(10,       800,ACT_CHECK,w1       ,"channel 800 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_5bis_26_6_3_3_3)

MSG3_BEGIN(measurement_report)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(measurement_report_message_type)

IE(measurement_results)

MSG3_END(measurement_report)

MSG3_BEGIN(system_information_type_5_26_6_3_3_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(neighbour_cell_description_26_6_3_3_3)

MSG3_END(system_information_type_5_26_6_3_3_3)

MSG3_BEGIN(system_information_type_5_26_6_3_3_3_k2)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(neighbour_cell_description_26_6_3_3_3_k2)

MSG3_END(system_information_type_5_26_6_3_3_3_k2)

MSG3_BEGIN(system_information_type_5bis_26_6_3_3_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5bis_message_type)

IE(neighbour_cell_description_5bis_26_6_3_3_3)

MSG3_END(system_information_type_5bis_26_6_3_3_3)

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

IE_BEGIN(authentication_parameter_rand)

BF(32,0x80000000,ACT_CHECK,rand_127_096,SILENT)

BF(32,0x00000012,ACT_CHECK,rand_095_064,SILENT)

BF(32,0x34000000,ACT_CHECK,rand_063_032,SILENT)

BF(32,0x0000000F,ACT_CHECK,rand_031_000,SILENT)

IE_END(authentication_parameter_rand)

IE_BEGIN(authentication_parameter_sres)

BF(32,0x0000000F,ACT_SHOW,sres_031_000,SILENT)

IE_END(authentication_parameter_sres)

IE_BEGIN(call_control_protocol_discriminator)

BF(4, 3,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_control_protocol_discriminator)

IE_BEGIN(called_party_bcd_number)

BF( 8,7,ACT_CHECK,length                     ,"length of IE")

BF( 1,1,ACT_CHECK,ext                        ,"Extension Bit")

BF( 3,0,ACT_CHECK,type_of_number             ,"Unknown")

BF( 4,1,ACT_CHECK,numbering_plan             ,"ISDN/telephony")

BF( 4,3,ACT_CHECK,digit_2                    ,"Digit 2")

BF( 4,0,ACT_CHECK,digit_1                    ,"Digit 1")

BF( 4,3,ACT_CHECK,digit_4                    ,"Digit 4")

BF( 4,0,ACT_CHECK,digit_3                    ,"Digit 3")

BF( 4,0,ACT_CHECK,digit_6                    ,"Digit 6")

BF( 4,9,ACT_CHECK,digit_5                    ,"Digit 5")

BF( 4,4,ACT_CHECK,digit_8                    ,"Digit 8")

BF( 4,9,ACT_CHECK,digit_7                    ,"Digit 7")

BF( 4,1,ACT_CHECK,digit_10                   ,"Digit 10")

BF( 4,1,ACT_CHECK,digit_9                   ,"Digit 9")

BF( 4,7,ACT_CHECK,digit_12                  ,"Digit 12")

BF( 4,0xF,ACT_CHECK,digit_11                ,"Digit 11")

IE_END(called_party_bcd_number)

IE_BEGIN(channels_needed_for_mobiles_1_and_2)

BF(2,0,ACT_CHECK,second_channel,"spare, any channel")

BF(2,0,ACT_CHECK, first_channel,"spare, any channel")

IE_END(channels_needed_for_mobiles_1_and_2)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,1),ACT_CHECK,          channel_type,"SDCCH/SACCH 4(1)")

BF( 3,            0,ACT_CHECK,      time_slot_number,"zero")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(channel_description)

IE_BEGIN(ciphering_mode_setting)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(ciphering_mode_setting)

IE_BEGIN(cipher_response)

BF(3,0,ACT_CHECK,          spare,SILENT)

BF(1,0,ACT_CHECK,cipher_response,"IMEISV shall not be included")

IE_END(cipher_response)

IE_BEGIN(description_of_the_first_channel_after_time)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            3,ACT_CHECK,      time_slot_number,"three")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(description_of_the_first_channel_after_time)

IE_BEGIN(ia_rest_octets)   /* maximum length (11), no hop, no start time */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(ia_rest_octets)

IE_BEGIN(iei_5E)

BF(8,0x5E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_5E)

IE_BEGIN(iei_15)

BF(8,0x15,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_15)

IE_BEGIN(iei_04)

BF(8,0x04,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_04)

IE_BEGIN(iei_34)

BF(8,0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_34)

IE_BEGIN(l2_pseudo_length_11)  /* pag req type 1 with TMSI (one mobile)*/

BF(8, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_11)

IE_BEGIN(mobile_allocation)

BF(8,0,ACT_CHECK,length,"length 0 due to hopping disabled")

IE_END(mobile_allocation)

IE_BEGIN(mobile_identity)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_4,SILENT)

BF(4,          0,ACT_CHECK, digit_3,SILENT)

BF(4,          7,ACT_CHECK, digit_6,SILENT)

BF(4,          4,ACT_CHECK, digit_5,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          1,ACT_CHECK, digit_7,SILENT)

BF(4,          9,ACT_CHECK, digit_10,SILENT)

BF(4,          4,ACT_CHECK, digit_9,SILENT)

BF(4,          2,ACT_CHECK, digit_12,SILENT)

BF(4,          1,ACT_CHECK, digit_11,SILENT)

IE_END(mobile_identity)

IE_BEGIN(mobility_management_protocol_discriminator)

BF(4, 5,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobility_management_protocol_discriminator)

IE_BEGIN(ms_classmark)

BF(8,        3,ACT_CHECK, length,SILENT)

BF(1,        0,ACT_CHECK, spare,SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,"phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,"Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,"encryption algorithm A5/1 available")

BF(3,M3(0,0,1),ACT_CHECK, rf_power_capability,"class 2")

BF(1,        0,ACT_CHECK, spare2,SILENT)

BF(1,        0,ACT_CHECK, ps_capability,"no pseudo-synch capability")

BF(2,  M2(0,0),ACT_CHECK, ss_screening_indicator,"default phase 1")

BF(1,        0,ACT_CHECK, sm_capability,"no point to point SMS")

BF(1,        0,ACT_CHECK, vbs,"no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,"no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"no extention band G1")

BF(1,        0,ACT_CHECK, classmark_3,"no add. MS cap. information")

BF(5,        1,ACT_CHECK, ccbs,SILENT)

BF(1,        0,ACT_CHECK, a5_3,"A5/3 not available")

BF(1,        0,ACT_CHECK, a5_2,"A5/2 not available")

IE_END(ms_classmark)

IE_BEGIN(neighbour_cell_description_26_6_3_1_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_032_001,"no neigbours")
IE_END(neighbour_cell_description_26_6_3_1_3)

IE_BEGIN(p1_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(p1_rest_octets)

IE_BEGIN(page_mode)

BF(2,0,ACT_CHECK,spare,"two spare bits")

BF(2,0,ACT_CHECK,pm   ,"Normal Paging")

IE_END(page_mode)

IE_BEGIN(power_command)

BF(8,10,ACT_CHECK,power,SILENT)

IE_END(power_command)

IE_BEGIN(rach)

BF(3,M3(1,0,0),ACT_CHECK ,establishment_cause,"paging ind. any chan'")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

IE_BEGIN(request_reference)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,"As in CHAN REQ")

BF(5,M5(1,1,1,1,1),ACT_SHOW,  random_reference,SILENT)

BF(5,            0,ACT_SHOW,               t1_,SILENT)

BF(6,            0,ACT_SHOW,               t3 ,SILENT)

BF(5,            0,ACT_SHOW,               t2 ,SILENT)

IE_END(request_reference)

IE_BEGIN(rr_cause)

BF(8,0,ACT_CHECK,rr_cause,"normal event")

IE_END(rr_cause)

IE_BEGIN(signal_call_waiting)

BF(8,M8(0,0,0,0,0,1,1,1),ACT_CHECK,signal_value,"(Any non-res. value)")

IE_END(signal_call_waiting)

IE_BEGIN(spare_half_octet)

BF(4, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(spare_half_octet)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,         spare,SILENT)

BF(6,0,ACT_CHECK,timing_advance,"0")

IE_END(timing_advance)

IE_BEGIN(transaction_identifier_source)

BF(4,M4(0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source)

IE_BEGIN(transaction_identifier_dest)

BF(4,M4(1,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest)

/*------------------------------------------------------------------*\

| Messages

\*------------------------------------------------------------------*/

MSG3_BEGIN(alerting)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(alerting_message_type)

MSG3_END(alerting)

MSG3_BEGIN(assignment_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(assignment_command)

MSG3_BEGIN(assignment_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_complete_message_type)

IE(rr_cause)

MSG3_END(assignment_complete)

MSG3_BEGIN(authentication_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(authentication_parameter_rand)

MSG3_END(authentication_request)

MSG3_BEGIN(authentication_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_response_message_type)

IE(authentication_parameter_sres)

MSG3_END(authentication_response)

MSG3_BEGIN(ciphering_mode_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(cipher_response)

IE(ciphering_mode_setting)

MSG3_END(ciphering_mode_command)

MSG3_BEGIN(ciphering_mode_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

MSG3_END(ciphering_mode_complete)

MSG3_BEGIN(connect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(connect_message_type)

MSG3_END(connect)

MSG3_BEGIN(connect_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(connect_acknowledge_message_type)

MSG3_END(connect_acknowledge)

MSG3_BEGIN(setup)            /* contains 'signal' but no 'Bearer Cap' */

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

MSG3_END(setup)

/* ------ Start of Testcase ----- */

ISS_INIT	( 8)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

REPEAT(N,8)

IE_BF_SET_VAL         ( cell_identity, ci, N," ci used for SI 3 and SI 6")

IE_BF_SET_VAL         ( neighbour_cell_description, ba_ind,  0," BA for BCCH")

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 0, SILENT)

BS_SET_SYS_INFO       ( N, system_information_type_1)

BS_SET_SYS_INFO       ( N, system_information_type_2)

BS_SET_SYS_INFO       ( N, system_information_type_3)

BS_SET_SYS_INFO       ( N, system_information_type_4)

IE_BF_SET_VAL         ( neighbour_cell_description,  ba_ind, 1," BA for SACCH")

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( N, system_information_type_6)

REPEAT_END(N)

#define              NCC_0    1

#define              NCC_1    1

#define              NCC_2    1

#define              NCC_3    2

#define              NCC_4    3

#define              NCC_5    4

#define              NCC_6    1

#define              NCC_7    1

#define              BCC_0    3

#define              BCC_1    5

#define              BCC_2    7

#define              BCC_3    1

#define              BCC_4    3

#define              BCC_5    5

#define              BCC_6    7

#define              BCC_7    1

#define              BSIC_0   ((NCC_0<<3)|(BCC_0))

#define              BSIC_1   ((NCC_1<<3)|(BCC_1))

#define              BSIC_2   ((NCC_2<<3)|(BCC_2))

#define              BSIC_3   ((NCC_3<<3)|(BCC_3))

#define              BSIC_4   ((NCC_4<<3)|(BCC_4))

#define              BSIC_5   ((NCC_5<<3)|(BCC_5))

#define              BSIC_6   ((NCC_6<<3)|(BCC_6))

#define              BSIC_7   ((NCC_7<<3)|(BCC_7))

#define              ARFCN_0    2

#define              ARFCN_1    8

#define              ARFCN_2   14

#define              ARFCN_3   20

#define              ARFCN_4   26

#define              ARFCN_5   32

#define              ARFCN_6   38

#define              ARFCN_7   44

#define              PWRLEV_0  -35	/* force to be serving cell (was -60) */

#define              PWRLEV_1  -85

#define              PWRLEV_2  -80

#define              PWRLEV_3  -75

#define              PWRLEV_4  -55

#define              PWRLEV_5  -50

#define              PWRLEV_6  -45

#define              PWRLEV_7  -40

BS_SET_SCH           ( 0,  BSIC_0,RFN)

BS_SET_ARFCN         ( 0, ARFCN_0)

BS_SET_POWER         ( 0,PWRLEV_0)

BS_SET_SCH           ( 1,  BSIC_1,RFN)

BS_SET_ARFCN         ( 1, ARFCN_1)

BS_SET_POWER         ( 1,PWRLEV_1)

BS_SET_SCH           ( 2,  BSIC_2,RFN)

BS_SET_ARFCN         ( 2, ARFCN_2)

BS_SET_POWER         ( 2,PWRLEV_2)

BS_SET_SCH           ( 3,  BSIC_3,RFN)

BS_SET_ARFCN         ( 3, ARFCN_3)

BS_SET_POWER         ( 3,PWRLEV_3)

BS_SET_SCH           ( 4,  BSIC_4,RFN)

BS_SET_ARFCN         ( 4, ARFCN_4)

BS_SET_POWER         ( 4,PWRLEV_4)

BS_SET_SCH           ( 5,  BSIC_5,RFN)

BS_SET_ARFCN         ( 5, ARFCN_5)

BS_SET_POWER         ( 5,PWRLEV_5)

BS_SET_SCH           ( 6,  BSIC_6,RFN)

BS_SET_ARFCN         ( 6, ARFCN_6)

BS_SET_POWER         ( 6,PWRLEV_6)

BS_SET_SCH           ( 7,  BSIC_7,RFN)

BS_SET_ARFCN         ( 7, ARFCN_7)

BS_SET_POWER         ( 7,PWRLEV_7)

BS_ON_OFF	( 0,TRUE)

BS_ON_OFF	( 1,TRUE)

BS_ON_OFF	( 2,TRUE)

BS_ON_OFF	( 3,TRUE)

BS_ON_OFF	( 4,TRUE)

BS_ON_OFF	( 5,TRUE)

BS_ON_OFF	( 6,TRUE)

BS_ON_OFF	( 7,TRUE)

/*------------------------------------------------------------------*\

| Switch on IUT

\*------------------------------------------------------------------*/

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

REPEAT(K,2)

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

SET_TIMEOUT         ( 30000)

/*------------------------------------------------------------------*\

| Message exchange

\*------------------------------------------------------------------*/

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

/* ---- end of preamble ---- */

/*

* BS_CONFIG_CHANNEL (0, SACCH, UNACK, SAPI_0)

* BS_MSG3_AWAIT     (0, measurement_report,"allow one unchecked report")

*

* not checked

*/

ISS_DELAY (10000)

REPEAT(COUNT,10)

BS_CONFIG_CHANNEL (0, SACCH, UNACK, SAPI_0)

BS_MSG3_AWAIT_BEGIN (0, measurement_report,SILENT)

BF_SET_VAL       (dtx              ,0   ,"DTX was not used")

BF_SET_VAL       (meas_valid       ,0   ,"measurement vaild (0)")

BF_SET_VAL       (ba_ind           ,1   ,"BA INDEX SET TO 1")

BF_SET_VAL       (num_ncell_m      ,4   ,"number of neighbours measured")

BF_SET_VAL       (bcch_freq_ncell_1,0x00,"refers to ARFCN 2")

BF_SET_VAL       (bcch_freq_ncell_2,0x04,"refers to ARFCN 44")

BF_SET_VAL       (bcch_freq_ncell_3,0x03,"refers to ARFCN 38")

BF_SET_VAL       (bcch_freq_ncell_4,0x01,"refers to ARFCN 14")

BS_MSG3_AWAIT_END()

REPEAT_END(COUNT)

BS_CONFIG_CHANNEL   (0, FACCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, channel_release," 16:")

ISS_DELAY (10000);

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 1,"")

BS_SET_SYS_INFO_SACCH ( 0, system_information_type_5bis_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( 1, system_information_type_5bis_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( 2, system_information_type_5bis_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( 3, system_information_type_5bis_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( 4, system_information_type_5bis_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( 5, system_information_type_5bis_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( 6, system_information_type_5bis_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( 7, system_information_type_5bis_26_6_3_3_3)

BS_SET_SYS_INFO_SACCH ( 0, system_information_type_5_26_6_3_3_3_k2)

BS_SET_SYS_INFO_SACCH ( 1, system_information_type_5_26_6_3_3_3_k2)

BS_SET_SYS_INFO_SACCH ( 2, system_information_type_5_26_6_3_3_3_k2)

BS_SET_SYS_INFO_SACCH ( 3, system_information_type_5_26_6_3_3_3_k2)

BS_SET_SYS_INFO_SACCH ( 4, system_information_type_5_26_6_3_3_3_k2)

BS_SET_SYS_INFO_SACCH ( 5, system_information_type_5_26_6_3_3_3_k2)

BS_SET_SYS_INFO_SACCH ( 6, system_information_type_5_26_6_3_3_3_k2)

BS_SET_SYS_INFO_SACCH ( 7, system_information_type_5_26_6_3_3_3_k2)

IE_BF_SET_VAL       ( neighbour_cell_description_26_6_3_3_3, ext_ind, 1," part of BA")

REPEAT_END(K)

History:	27.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR155: Measurement / DTX (26.6.3.4)

Description:	(Ref.: GSM 11.10-1, §26.6.3.4)

Preamble:	None

Script:

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

IE_BEGIN(authentication_parameter_rand)

BF(32,0x80000000,ACT_CHECK,rand_127_096,SILENT)

BF(32,0x00000012,ACT_CHECK,rand_095_064,SILENT)

BF(32,0x34000000,ACT_CHECK,rand_063_032,SILENT)

BF(32,0x0000000F,ACT_CHECK,rand_031_000,SILENT)

IE_END(authentication_parameter_rand)

IE_BEGIN(authentication_parameter_sres)

BF(32,0x0000000F,ACT_SHOW,sres_031_000,SILENT)

IE_END(authentication_parameter_sres)

IE_BEGIN(call_control_protocol_discriminator)

BF(4, 3,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_control_protocol_discriminator)

IE_BEGIN(called_party_bcd_number)

BF( 8,7,ACT_CHECK,length                     ,"length of IE")

BF( 1,1,ACT_CHECK,ext                        ,"Extension Bit")

BF( 3,0,ACT_CHECK,type_of_number             ,"Unknown")

BF( 4,1,ACT_CHECK,numbering_plan             ,"ISDN/telephony")

BF( 4,3,ACT_CHECK,digit_2                    ,"Digit 2")

BF( 4,0,ACT_CHECK,digit_1                    ,"Digit 1")

BF( 4,3,ACT_CHECK,digit_4                    ,"Digit 4")

BF( 4,0,ACT_CHECK,digit_3                    ,"Digit 3")

BF( 4,0,ACT_CHECK,digit_6                    ,"Digit 6")

BF( 4,9,ACT_CHECK,digit_5                    ,"Digit 5")

BF( 4,4,ACT_CHECK,digit_8                    ,"Digit 8")

BF( 4,9,ACT_CHECK,digit_7                    ,"Digit 7")

BF( 4,1,ACT_CHECK,digit_10                   ,"Digit 10")

BF( 4,1,ACT_CHECK,digit_9                   ,"Digit 9")

BF( 4,7,ACT_CHECK,digit_12                  ,"Digit 12")

BF( 4,0xF,ACT_CHECK,digit_11                ,"Digit 11")

IE_END(called_party_bcd_number)

IE_BEGIN(channels_needed_for_mobiles_1_and_2)

BF(2,0,ACT_CHECK,second_channel,"spare, any channel")

BF(2,0,ACT_CHECK, first_channel,"spare, any channel")

IE_END(channels_needed_for_mobiles_1_and_2)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,1),ACT_CHECK,          channel_type,"SDCCH/SACCH 4(1)")

BF( 3,            0,ACT_CHECK,      time_slot_number,"zero")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(channel_description)

IE_BEGIN(ciphering_mode_setting)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(ciphering_mode_setting)

IE_BEGIN(cipher_response)

BF(3,0,ACT_CHECK,          spare,SILENT)

BF(1,0,ACT_CHECK,cipher_response,"IMEISV shall not be included")

IE_END(cipher_response)

IE_BEGIN(description_of_the_first_channel_after_time)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            3,ACT_CHECK,      time_slot_number,"three")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(description_of_the_first_channel_after_time)

IE_BEGIN(ia_rest_octets)   /* maximum length (11), no hop, no start time */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(ia_rest_octets)

IE_BEGIN(iei_5E)

BF(8,0x5E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_5E)

IE_BEGIN(iei_15)

BF(8,0x15,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_15)

IE_BEGIN(iei_04)

BF(8,0x04,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_04)

IE_BEGIN(iei_34)

BF(8,0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_34)

IE_BEGIN(l2_pseudo_length_11)  /* pag req type 1 with TMSI (one mobile)*/

BF(8, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_11)

IE_BEGIN(mobile_allocation)

BF(8,0,ACT_CHECK,length,"length 0 due to hopping disabled")

IE_END(mobile_allocation)

IE_BEGIN(mobile_identity)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_4,SILENT)

BF(4,          0,ACT_CHECK, digit_3,SILENT)

BF(4,          7,ACT_CHECK, digit_6,SILENT)

BF(4,          4,ACT_CHECK, digit_5,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          1,ACT_CHECK, digit_7,SILENT)

BF(4,          9,ACT_CHECK, digit_10,SILENT)

BF(4,          4,ACT_CHECK, digit_9,SILENT)

BF(4,          2,ACT_CHECK, digit_12,SILENT)

BF(4,          1,ACT_CHECK, digit_11,SILENT)

IE_END(mobile_identity)

IE_BEGIN(mobility_management_protocol_discriminator)

BF(4, 5,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobility_management_protocol_discriminator)

IE_BEGIN(ms_classmark)

BF(8,        3,ACT_CHECK, length,SILENT)

BF(1,        0,ACT_CHECK, spare,SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,"phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,"Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,"encryption algorithm A5/1 available")

BF(3,M3(0,0,1),ACT_CHECK, rf_power_capability,"class 2")

BF(1,        0,ACT_CHECK, spare2,SILENT)

BF(1,        0,ACT_CHECK, ps_capability,"no pseudo-synch capability")

BF(2,  M2(0,0),ACT_CHECK, ss_screening_indicator,"default phase 1")

BF(1,        0,ACT_CHECK, sm_capability,"no point to point SMS")

BF(1,        0,ACT_CHECK, vbs,"no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,"no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"no extention band G1")

BF(1,        0,ACT_CHECK, classmark_3,"no add. MS cap. information")

BF(5,        1,ACT_CHECK, ccbs,SILENT)

BF(1,        0,ACT_CHECK, a5_3,"A5/3 not available")

BF(1,        0,ACT_CHECK, a5_2,"A5/2 not available")

IE_END(ms_classmark)

IE_BEGIN(neighbour_cell_description_26_6_3_1_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_032_001,"no neigbours")
IE_END(neighbour_cell_description_26_6_3_1_3)

IE_BEGIN(p1_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(p1_rest_octets)

IE_BEGIN(page_mode)

BF(2,0,ACT_CHECK,spare,"two spare bits")

BF(2,0,ACT_CHECK,pm   ,"Normal Paging")

IE_END(page_mode)

IE_BEGIN(power_command)

BF(8,10,ACT_CHECK,power,SILENT)

IE_END(power_command)

IE_BEGIN(rach)

BF(3,M3(1,0,0),ACT_CHECK ,establishment_cause,"paging ind. any chan'")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

IE_BEGIN(request_reference)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,"As in CHAN REQ")

BF(5,M5(1,1,1,1,1),ACT_SHOW,  random_reference,SILENT)

BF(5,            0,ACT_SHOW,               t1_,SILENT)

BF(6,            0,ACT_SHOW,               t3 ,SILENT)

BF(5,            0,ACT_SHOW,               t2 ,SILENT)

IE_END(request_reference)

IE_BEGIN(rr_cause)

BF(8,0,ACT_CHECK,rr_cause,"normal event")

IE_END(rr_cause)

IE_BEGIN(signal_call_waiting)

BF(8,M8(0,0,0,0,0,1,1,1),ACT_CHECK,signal_value,"(Any non-res. value)")

IE_END(signal_call_waiting)

IE_BEGIN(spare_half_octet)

BF(4, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(spare_half_octet)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,         spare,SILENT)

BF(6,0,ACT_CHECK,timing_advance,"0")

IE_END(timing_advance)

IE_BEGIN(transaction_identifier_source)

BF(4,M4(0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source)

IE_BEGIN(transaction_identifier_dest)

BF(4,M4(1,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest)

/*------------------------------------------------------------------*\

| Messages

\*------------------------------------------------------------------*/

MSG3_BEGIN(alerting)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(alerting_message_type)

MSG3_END(alerting)

MSG3_BEGIN(assignment_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(assignment_command)

MSG3_BEGIN(assignment_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_complete_message_type)

IE(rr_cause)

MSG3_END(assignment_complete)

MSG3_BEGIN(authentication_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(authentication_parameter_rand)

MSG3_END(authentication_request)

MSG3_BEGIN(authentication_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_response_message_type)

IE(authentication_parameter_sres)

MSG3_END(authentication_response)

MSG3_BEGIN(ciphering_mode_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(cipher_response)

IE(ciphering_mode_setting)

MSG3_END(ciphering_mode_command)

MSG3_BEGIN(ciphering_mode_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

MSG3_END(ciphering_mode_complete)

MSG3_BEGIN(connect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(connect_message_type)

MSG3_END(connect)

MSG3_BEGIN(connect_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(connect_acknowledge_message_type)

MSG3_END(connect_acknowledge)

MSG3_BEGIN(setup)            /* contains 'signal' but no 'Bearer Cap' */

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

MSG3_END(setup)

#define              NCC_0    1

#define              NCC_1    1

#define              NCC_2    1

#define              NCC_3    1

#define              NCC_4    1

#define              NCC_5    1

#define              NCC_6    1

#define              NCC_7    1

#define              BCC_0    3

#define              BCC_1    5

#define              BCC_2    7

#define              BCC_3    1

#define              BCC_4    3

#define              BCC_5    5

#define              BCC_6    7

#define              BCC_7    1

#define              BSIC_0   ((NCC_0<<3)|(BCC_0))

#define              BSIC_1   ((NCC_1<<3)|(BCC_1))

#define              BSIC_2   ((NCC_2<<3)|(BCC_2))

#define              BSIC_3   ((NCC_3<<3)|(BCC_3))

#define              BSIC_4   ((NCC_4<<3)|(BCC_4))

#define              BSIC_5   ((NCC_5<<3)|(BCC_5))

#define              BSIC_6   ((NCC_6<<3)|(BCC_6))

#define              BSIC_7   ((NCC_7<<3)|(BCC_7))

#define              ARFCN_0    2

#define              ARFCN_1    8

#define              ARFCN_2   14

#define              ARFCN_3   20

#define              ARFCN_4   26

#define              ARFCN_5   32

#define              ARFCN_6   38

#define              ARFCN_7   44

#define              PWRLEV_0  -60

#define              PWRLEV_1  -85

#define              PWRLEV_2  -80

#define              PWRLEV_3  -75

#define              PWRLEV_4  -55

#define              PWRLEV_5  -50

#define              PWRLEV_6  -45

#define              PWRLEV_7  -40

IE_BEGIN(cell_options_sacch_use_dtx)
   BF(1,0,ACT_CHECK,dtx0               ,"MS shall use DTX")
   BF(1,0,ACT_CHECK,pwrc               ,"power control indicator is not set")
   BF(2,1,ACT_CHECK,dtx                ,"MS shall use DTX")
   BF(4,1,ACT_CHECK,radio_link_timeout ,"8 SACCH blocks")
IE_END(cell_options_sacch_use_dtx)

IE_BEGIN(cell_description_ho)

BF( 2,ARFCN_0>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,     NCC_0,ACT_CHECK,ncc,  "   network colour code")

BF( 3,     BCC_0,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_0   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description_ho)

IE_BEGIN(synchronization_indication)

BF( 4, 0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,   0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,   0, ACT_CHECK,   rot,"observed time diff not included")

BF( 2,   0, ACT_CHECK,    si,"synchronization indication")

IE_END(synchronization_indication)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description_ho)

IE(channel_description)

IE(handover_reference)

IE(power_command)

IE(synchronization_indication)

MSG3_END(handover_command)

IE_BEGIN(measurement_results)

BF(1,0x01,ACT_CHECK  ,ba_ind                  ,SILENT)

BF(1,   1,ACT_CHECK  ,dtx                     ,"DTX was used")

BF(6,0x32,ACT_CHECK  ,rxlev_full_serving_cell ,SILENT)

BF(1,   0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(1,   0,ACT_CHECK  ,meas_valid              ,"valid within the 2nd SACCH"

"   block at the latest")

BF(6,0x32,ACT_CHECK  ,rxlev_sub_serving_cell  ,SILENT)

BF(1,   0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(3,   0,ACT_CHECK  ,rxqual_full_serving_cell,SILENT)

BF(3,   0,ACT_CHECK  ,rxqual_sub_serving_cell ,SILENT)

BF(3,   6,ACT_CHECK , num_ncell_m             ,"num of neighbor cells")

BF(6,0x3f,ACT_CHECK , rxlev_ncell_1           ,SILENT)

BF(5,   7,ACT_CHECK , bcch_freq_ncell_1       ,SILENT)

BF(6,0x09,ACT_CHECK , bsic_ncell_1            ,SILENT)

BF(6,0x3f,ACT_CHECK , rxlev_ncell_2           ,SILENT)

BF(5,   6,ACT_CHECK , bcch_freq_ncell_2       ,SILENT)

BF(6,0x0F,ACT_CHECK , bsic_ncell_2            ,SILENT)

BF(6,0x3c,ACT_CHECK , rxlev_ncell_3           ,SILENT)

BF(5,   5,ACT_CHECK , bcch_freq_ncell_3       ,SILENT)

BF(6,0x0d,ACT_CHECK , bsic_ncell_3            ,SILENT)

BF(6,0x37,ACT_CHECK , rxlev_ncell_4           ,SILENT)

BF(5,   4,ACT_CHECK , bcch_freq_ncell_4       ,SILENT)

BF(6,0x0b,ACT_CHECK , bsic_ncell_4            ,SILENT)

BF(6,0x32,ACT_CHECK , rxlev_ncell_5           ,SILENT)

BF(5,   0,ACT_CHECK , bcch_freq_ncell_5       ,SILENT)

BF(6,0x0B,ACT_CHECK , bsic_ncell_5            ,SILENT)

BF(6,0x23,ACT_CHECK , rxlev_ncell_6           ,SILENT)

BF(5,   3,ACT_CHECK , bcch_freq_ncell_6       ,SILENT)

BF(6,0x09,ACT_CHECK , bsic_ncell_6            ,SILENT)

IE_END(measurement_results)

IE_BEGIN(neighbour_cell_description_bcch_26_6_3_4_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         0,ACT_CHECK,ba_ind   ,"BCCH allocation sequence")
   BF(28,0x00000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"no neigbours")
   BF(32,0x00000002,ACT_CHECK,ba_032_001,"ARFCN 2")

IE_END(neighbour_cell_description_bcch_26_6_3_4_3)

IE_BEGIN(neighbour_cell_description_sacch_26_6_3_4_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28,0x00000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000820,ACT_CHECK,ba_064_033,"ARFCN 44, 38")
   BF(32,0x82082082,ACT_CHECK,ba_032_001,"ARFCN 32, 26, 20, 14, 8, 2")

IE_END(neighbour_cell_description_sacch_26_6_3_4_3)

IE_BEGIN(neighbour_cell_description_sacch)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28,0x00000000,ACT_CHECK,ba_124_097,"no neighbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neighbours")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"no neighbours")
   BF(32,0x82082082,ACT_CHECK,ba_032_001,"ARFCN 32, 26, 20, 14, 8, 2")

IE_END(neighbour_cell_description_sacch)

IE_BEGIN(neighbour_cell_description_5bis_26_6_3_4_3)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         1,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         0,ACT_CHECK,f0       ,"frequency 0 indicator (F0 not included)")
   BF(10,       500,ACT_CHECK,w1       ,"channel 500 belongs to the BA")
   BF( 9,         0,ACT_CHECK,w2       ,SILENT)
   BF( 9,         0,ACT_CHECK,w3       ,SILENT)
   BF( 8,         0,ACT_CHECK,w4       ,SILENT)
   BF( 8,         0,ACT_CHECK,w5       ,SILENT)
   BF( 8,         0,ACT_CHECK,w6       ,SILENT)
   BF( 8,         0,ACT_CHECK,w7       ,SILENT)
   BF( 7,         0,ACT_CHECK,w8       ,SILENT)
   BF( 7,         0,ACT_CHECK,w9       ,SILENT)
   BF( 7,         0,ACT_CHECK,w10      ,SILENT)
   BF( 7,         0,ACT_CHECK,w11      ,SILENT)
   BF( 7,         0,ACT_CHECK,w12      ,SILENT)
   BF( 7,         0,ACT_CHECK,w13      ,SILENT)
   BF( 7,         0,ACT_CHECK,w14      ,SILENT)
   BF( 7,         0,ACT_CHECK,w15      ,SILENT)
   BF( 6,         0,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_5bis_26_6_3_4_3)

MSG3_BEGIN(measurement_report)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(measurement_report_message_type)

IE(measurement_results)

MSG3_END(measurement_report)

MSG3_BEGIN(system_information_type_2_26_6_3_4_3)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(neighbour_cell_description_bcch_26_6_3_4_3)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2_26_6_3_4_3)

MSG3_BEGIN(system_information_type_5)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(neighbour_cell_description_sacch)

MSG3_END(system_information_type_5)

MSG3_BEGIN(system_information_type_5_26_6_3_4_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(neighbour_cell_description_sacch_26_6_3_4_3)

MSG3_END(system_information_type_5_26_6_3_4_3)

MSG3_BEGIN(system_information_type_5bis_26_6_3_4_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5bis_message_type)

IE(neighbour_cell_description_5bis_26_6_3_4_3)

MSG3_END(system_information_type_5bis_26_6_3_4_3)

MSG3_BEGIN(system_information_type_6_26_6_3_4_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_6_message_type)

IE(cell_identity)

IE(location_area_identification)

IE(cell_options_sacch_use_dtx)

IE(ncc_permitted)

IE(si_6_rest_octets)

MSG3_END(system_information_type_6_26_6_3_4_3)

REPEAT(K,2)

ISS_INIT	( 8)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

BS_SET_SCH           ( 7,  BSIC_0,RFN)

BS_SET_ARFCN         ( 7, ARFCN_0)

BS_SET_POWER         ( 7,PWRLEV_0)

#define           S1   7

BS_SET_SCH           ( 1,  BSIC_1,RFN)

BS_SET_ARFCN         ( 1, ARFCN_1)

BS_SET_POWER         ( 1,PWRLEV_1)

#define           N1   1

BS_SET_SCH           ( 2,  BSIC_2,RFN)

BS_SET_ARFCN         ( 2, ARFCN_2)

BS_SET_POWER         ( 2,PWRLEV_2)

#define           N2   2

BS_SET_SCH           ( 3,  BSIC_3,RFN)

BS_SET_ARFCN         ( 3, ARFCN_3)

BS_SET_POWER         ( 3,PWRLEV_3)

#define           N3   3

BS_SET_SCH           ( 4,  BSIC_4,RFN)

BS_SET_ARFCN         ( 4, ARFCN_4)

BS_SET_POWER         ( 4,PWRLEV_4)

#define           N4   4

BS_SET_SCH           ( 5,  BSIC_5,RFN)

BS_SET_ARFCN         ( 5, ARFCN_5)

BS_SET_POWER         ( 5,PWRLEV_5)

#define           N5   5

BS_SET_SCH           ( 6,  BSIC_6,RFN)

BS_SET_ARFCN         ( 6, ARFCN_6)

BS_SET_POWER         ( 6,PWRLEV_6)

#define           N6   6

BS_SET_SCH           ( 0,  BSIC_7,RFN)

BS_SET_ARFCN         ( 0, ARFCN_7)

BS_SET_POWER         ( 0,PWRLEV_7)

#define           N7   0

REPEAT(N,8)

IE_BF_SET_VAL         ( cell_options_sacch, dtx, 2," MS shall not use DTX (default)")

IE_BF_SET_VAL         ( cell_identity, ci, N," ci used for SI 3 and SI 6")

BS_SET_SYS_INFO       ( N, system_information_type_1)

BS_SET_SYS_INFO       ( N, system_information_type_2_26_6_3_4_3)

BS_SET_SYS_INFO       ( N, system_information_type_3)

BS_SET_SYS_INFO       ( N, system_information_type_4)

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5)

BS_SET_SYS_INFO_SACCH ( N, system_information_type_6)

REPEAT_END(N)

IE_BF_SET_VAL         ( cell_identity, ci, S1," ci used for SI 3 and SI 6")

BS_SET_SYS_INFO       ( S1, system_information_type_1)

BS_SET_SYS_INFO       ( S1, system_information_type_2_26_6_3_4_3)

BS_SET_SYS_INFO       ( S1, system_information_type_3)

BS_SET_SYS_INFO       ( S1, system_information_type_4)

BS_ON_OFF	( 0,TRUE)

BS_ON_OFF	( 1,TRUE)

BS_ON_OFF	( 2,TRUE)

BS_ON_OFF	( 3,TRUE)

BS_ON_OFF	( 4,TRUE)

BS_ON_OFF	( 5,TRUE)

BS_ON_OFF	( 6,TRUE)

BS_ON_OFF	( 7,TRUE)

ISS_DELAY (10000);

/* MS will select N7 */

/*------------------------------------------------------------------*\

| Switch on IUT

\*------------------------------------------------------------------*/

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

COMMAND	(" CST CONFIG AUDIO_MUTE")

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

SET_TIMEOUT         ( 30000)

/*------------------------------------------------------------------*\

| Message exchange

\*------------------------------------------------------------------*/

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

/* ---- end of preamble ---- */

ISS_DELAY (5000)

BS_CONFIG_CHANNEL   ( N7, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( N7, handover_command," 1:")

BS_CONFIG_CHANNEL   ( S1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( S1, handover_complete," 6:")

if ( K == 1)

{

IE_BF_SET_VAL         ( neighbour_cell_description_sacch_26_6_3_4_3, ext_ind, 1,"")

BS_SET_SYS_INFO_SACCH ( S1, system_information_type_5bis_26_6_3_4_3)

}

BS_SET_SYS_INFO_SACCH ( S1, system_information_type_5_26_6_3_4_3)

BS_SET_SYS_INFO_SACCH ( S1, system_information_type_6_26_6_3_4_3)

ISS_DELAY 		( 20000)

/*

* BS_CONFIG_CHANNEL (S1, SACCH, UNACK, SAPI_0)

* BS_MSG3_AWAIT     (S1, measurement_report,"allow one unchecked report")

*

* not checked

*/

REPEAT(COUNT,10)

BS_CONFIG_CHANNEL (S1, SACCH, UNACK, SAPI_0)

BS_MSG3_AWAIT (S1, measurement_report,SILENT)

REPEAT_END(COUNT)

AT_SEND ("AT+CFUN=0"," Power Off");

BS_ON_OFF	( 0,FALSE)

BS_ON_OFF	( 1,FALSE)

BS_ON_OFF	( 2,FALSE)

BS_ON_OFF	( 3,FALSE)

BS_ON_OFF	( 4,FALSE)

BS_ON_OFF	( 5,FALSE)

BS_ON_OFF	( 6,FALSE)

BS_ON_OFF	( 7,FALSE)

ISS_DELAY (10000);

IE_BF_SET_VAL       ( neighbour_cell_description_sacch_26_6_3_4_3, ext_ind, 1," part of BA")

REPEAT_END(K)

History:	28.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR156: Measurement / Frequency Formats (26.6.3.5)

Description:	(Ref.: GSM 11.10-1, §26.6.3.5)
This test has been modified. The following W(i) are used yielding F(i):
w=1,510,3,255,254,6,7,126,120,125,11,12,13,14,15,16
f=1,1022,4,1021,2,517,11,1019,505,514,149,778,271,531,26,971
Of those frequencies only 1,4,2,11, and 26 are handled by the IUT.

Preamble:	None

Script:

IE_BEGIN(measurement_results)

BF(1,   1,ACT_CHECK  ,ba_ind                  ,SILENT)

BF(1,   0,ACT_CHECK  ,dtx                     ,"DTX was not used")

BF(6,0x32,ACT_CHECK  ,rxlev_full_serving_cell ,SILENT)

BF(1,   0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(1,   0,ACT_CHECK  ,meas_valid              ,"valid within the 2nd SACCH"

"   block at the latest")

BF(6,0x32,ACT_CHECK  ,rxlev_sub_serving_cell  ,SILENT)

BF(1,   0,ACT_CHECK  ,ANONYMOUS               ,"spare")

BF(3,   0,ACT_CHECK  ,rxqual_full_serving_cell,SILENT)

BF(3,   0,ACT_CHECK  ,rxqual_sub_serving_cell ,SILENT)

BF(3,   5,ACT_CHECK , num_ncell_m             ,"num of neighbor cells")

BF(6,0x3f,ACT_CHECK , rxlev_ncell_1           ,SILENT)

BF(5,   4,ACT_CHECK , bcch_freq_ncell_1       ,"refers to ARFCN 26")

BF(6,0x0F,ACT_CHECK , bsic_ncell_1            ,SILENT)

BF(6,0x32,ACT_CHECK , rxlev_ncell_2           ,SILENT)

BF(5,   0,ACT_CHECK , bcch_freq_ncell_2       ,"refers to ARFCN 1")

BF(6,0x0B,ACT_CHECK , bsic_ncell_2            ,SILENT)

BF(6,0x23,ACT_CHECK , rxlev_ncell_3           ,SILENT)

BF(5,   3,ACT_CHECK , bcch_freq_ncell_3       ,"refers to ARFCN 11")

BF(6,0x09,ACT_CHECK , bsic_ncell_3            ,SILENT)

BF(6,0x1E,ACT_CHECK , rxlev_ncell_4           ,SILENT)

BF(5,   2,ACT_CHECK , bcch_freq_ncell_4       ,"refers to ARFCN 4")

BF(6,0x0F,ACT_CHECK , bsic_ncell_4            ,SILENT)

BF(6,0x19,ACT_CHECK , rxlev_ncell_5           ,SILENT)

BF(5,   1,ACT_CHECK , bcch_freq_ncell_5       ,"refers to ARFCN 2")

BF(6,0x0D,ACT_CHECK , bsic_ncell_5            ,SILENT)

BF(6,   0,ACT_CHECK , rxlev_ncell_6           ,SILENT)

BF(5,   0,ACT_CHECK , bcch_freq_ncell_6       ,SILENT)

BF(6,   0,ACT_CHECK , bsic_ncell_6            ,SILENT)

IE_END(measurement_results)

IE_BEGIN(neighbour_cell_description_26_6_3_5_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         1,ACT_CHECK,ext_ind  ,"this IE carries a part of the BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28,0x00000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"no neigbours")
   BF(32,0x00000002,ACT_CHECK,ba_032_001,"ARFCN")

IE_END(neighbour_cell_description_26_6_3_5_3)

IE_BEGIN(neighbour_cell_description_5bis_26_6_3_5_3)
   BF( 2,   M2(1,0),ACT_CHECK,id_8_7   ,"format 1024 ID, part I")
   BF( 1,         1,ACT_CHECK,ext_ind  ,"extention indication")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF( 1,         0,ACT_CHECK,id_4     ,"format 1024 ID, part II")
   BF( 1,         1,ACT_CHECK,f0       ,"frequency 0 indicator (F0 included)")
   BF(10,         1,ACT_CHECK,w1       ,"channel 500 belongs to the BA")
   BF( 9,       510,ACT_CHECK,w2       ,SILENT)
   BF( 9,         3,ACT_CHECK,w3       ,SILENT)
   BF( 8,       255,ACT_CHECK,w4       ,SILENT)
   BF( 8,       254,ACT_CHECK,w5       ,SILENT)
   BF( 8,         6,ACT_CHECK,w6       ,SILENT)
   BF( 8,         7,ACT_CHECK,w7       ,SILENT)
   BF( 7,       126,ACT_CHECK,w8       ,SILENT)
   BF( 7,       120,ACT_CHECK,w9       ,SILENT)
   BF( 7,       125,ACT_CHECK,w10      ,SILENT)
   BF( 7,        11,ACT_CHECK,w11      ,SILENT)
   BF( 7,        12,ACT_CHECK,w12      ,SILENT)
   BF( 7,        13,ACT_CHECK,w13      ,SILENT)
   BF( 7,        14,ACT_CHECK,w14      ,SILENT)
   BF( 7,        15,ACT_CHECK,w15      ,SILENT)
   BF( 6,        16,ACT_CHECK,w16      ,SILENT)
IE_END(neighbour_cell_description_5bis_26_6_3_5_3)

MSG3_BEGIN(measurement_report)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(measurement_report_message_type)

IE(measurement_results)

MSG3_END(measurement_report)

MSG3_BEGIN(system_information_type_5_26_6_3_5_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(neighbour_cell_description_26_6_3_5_3)

MSG3_END(system_information_type_5_26_6_3_5_3)

MSG3_BEGIN(system_information_type_5bis_26_6_3_5_3)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5bis_message_type)

IE(neighbour_cell_description_5bis_26_6_3_5_3)

MSG3_END(system_information_type_5bis_26_6_3_5_3)

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

IE_BEGIN(authentication_parameter_rand)

BF(32,0x80000000,ACT_CHECK,rand_127_096,SILENT)

BF(32,0x00000012,ACT_CHECK,rand_095_064,SILENT)

BF(32,0x34000000,ACT_CHECK,rand_063_032,SILENT)

BF(32,0x0000000F,ACT_CHECK,rand_031_000,SILENT)

IE_END(authentication_parameter_rand)

IE_BEGIN(authentication_parameter_sres)

BF(32,0x0000000F,ACT_SHOW,sres_031_000,SILENT)

IE_END(authentication_parameter_sres)

IE_BEGIN(call_control_protocol_discriminator)

BF(4, 3,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_control_protocol_discriminator)

IE_BEGIN(called_party_bcd_number)

BF( 8,7,ACT_CHECK,length                     ,"length of IE")

BF( 1,1,ACT_CHECK,ext                        ,"Extension Bit")

BF( 3,0,ACT_CHECK,type_of_number             ,"Unknown")

BF( 4,1,ACT_CHECK,numbering_plan             ,"ISDN/telephony")

BF( 4,3,ACT_CHECK,digit_2                    ,"Digit 2")

BF( 4,0,ACT_CHECK,digit_1                    ,"Digit 1")

BF( 4,3,ACT_CHECK,digit_4                    ,"Digit 4")

BF( 4,0,ACT_CHECK,digit_3                    ,"Digit 3")

BF( 4,0,ACT_CHECK,digit_6                    ,"Digit 6")

BF( 4,9,ACT_CHECK,digit_5                    ,"Digit 5")

BF( 4,4,ACT_CHECK,digit_8                    ,"Digit 8")

BF( 4,9,ACT_CHECK,digit_7                    ,"Digit 7")

BF( 4,1,ACT_CHECK,digit_10                   ,"Digit 10")

BF( 4,1,ACT_CHECK,digit_9                   ,"Digit 9")

BF( 4,7,ACT_CHECK,digit_12                  ,"Digit 12")

BF( 4,0xF,ACT_CHECK,digit_11                ,"Digit 11")

IE_END(called_party_bcd_number)

IE_BEGIN(channels_needed_for_mobiles_1_and_2)

BF(2,0,ACT_CHECK,second_channel,"spare, any channel")

BF(2,0,ACT_CHECK, first_channel,"spare, any channel")

IE_END(channels_needed_for_mobiles_1_and_2)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,1),ACT_CHECK,          channel_type,"SDCCH/SACCH 4(1)")

BF( 3,            0,ACT_CHECK,      time_slot_number,"zero")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(channel_description)

IE_BEGIN(ciphering_mode_setting)

BF(3,M3(0,0,0),ACT_CHECK,algorithm_identifier,"A5/1")

BF(1,        1,ACT_CHECK,     start_ciphering,"Start ciphering")

IE_END(ciphering_mode_setting)

IE_BEGIN(cipher_response)

BF(3,0,ACT_CHECK,          spare,SILENT)

BF(1,0,ACT_CHECK,cipher_response,"IMEISV shall not be included")

IE_END(cipher_response)

IE_BEGIN(description_of_the_first_channel_after_time)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,            3,ACT_CHECK,      time_slot_number,"three")

BF( 3,          BCC,ACT_CHECK,training_sequence_code,"same as BCCH")

BF( 1,            0,ACT_CHECK,               hopping,"No")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(description_of_the_first_channel_after_time)

IE_BEGIN(ia_rest_octets)   /* maximum length (11), no hop, no start time */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(ia_rest_octets)

IE_BEGIN(iei_5E)

BF(8,0x5E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_5E)

IE_BEGIN(iei_15)

BF(8,0x15,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_15)

IE_BEGIN(iei_04)

BF(8,0x04,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_04)

IE_BEGIN(iei_34)

BF(8,0x34,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_34)

IE_BEGIN(l2_pseudo_length_11)  /* pag req type 1 with TMSI (one mobile)*/

BF(8, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_11)

IE_BEGIN(mobile_allocation)

BF(8,0,ACT_CHECK,length,"length 0 due to hopping disabled")

IE_END(mobile_allocation)

IE_BEGIN(mobile_identity)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_4,SILENT)

BF(4,          0,ACT_CHECK, digit_3,SILENT)

BF(4,          7,ACT_CHECK, digit_6,SILENT)

BF(4,          4,ACT_CHECK, digit_5,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          1,ACT_CHECK, digit_7,SILENT)

BF(4,          9,ACT_CHECK, digit_10,SILENT)

BF(4,          4,ACT_CHECK, digit_9,SILENT)

BF(4,          2,ACT_CHECK, digit_12,SILENT)

BF(4,          1,ACT_CHECK, digit_11,SILENT)

IE_END(mobile_identity)

IE_BEGIN(mobility_management_protocol_discriminator)

BF(4, 5,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobility_management_protocol_discriminator)

IE_BEGIN(ms_classmark)

BF(8,        3,ACT_CHECK, length,SILENT)

BF(1,        0,ACT_CHECK, spare,SILENT)

BF(2,  M2(0,1),ACT_CHECK, revision_level,"phase 2 MS")

BF(1,        1,ACT_CHECK, es_ind,"Contr. Early Classmark Send.")

BF(1,        0,ACT_CHECK, a5_1,"encryption algorithm A5/1 available")

BF(3,M3(0,0,1),ACT_CHECK, rf_power_capability,"class 2")

BF(1,        0,ACT_CHECK, spare2,SILENT)

BF(1,        0,ACT_CHECK, ps_capability,"no pseudo-synch capability")

BF(2,  M2(0,0),ACT_CHECK, ss_screening_indicator,"default phase 1")

BF(1,        0,ACT_CHECK, sm_capability,"no point to point SMS")

BF(1,        0,ACT_CHECK, vbs,"no VBS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, vgcs,"no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_CHECK, frequency_capability,"no extention band G1")

BF(1,        0,ACT_CHECK, classmark_3,"no add. MS cap. information")

BF(5,        1,ACT_CHECK, ccbs,SILENT)

BF(1,        0,ACT_CHECK, a5_3,"A5/3 not available")

BF(1,        0,ACT_CHECK, a5_2,"A5/2 not available")

IE_END(ms_classmark)

IE_BEGIN(neighbour_cell_description_26_6_3_1_3)
   BF( 2,         0,ACT_CHECK,format_id,"bit map 0")
   BF( 1,         0,ACT_CHECK,ext_ind  ,"this IE carries the complete BA")
   BF( 1,         1,ACT_CHECK,ba_ind   ,"SACCH allocation sequence")
   BF(28, 0x0000000,ACT_CHECK,ba_124_097,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_096_065,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_064_033,"no neigbours")
   BF(32,0x00000000,ACT_CHECK,ba_032_001,"no neigbours")
IE_END(neighbour_cell_description_26_6_3_1_3)

IE_BEGIN(p1_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /* 10 */

IE_END(p1_rest_octets)

IE_BEGIN(page_mode)

BF(2,0,ACT_CHECK,spare,"two spare bits")

BF(2,0,ACT_CHECK,pm   ,"Normal Paging")

IE_END(page_mode)

IE_BEGIN(power_command)

BF(8,10,ACT_CHECK,power,SILENT)

IE_END(power_command)

IE_BEGIN(rach)

BF(3,M3(1,0,0),ACT_CHECK ,establishment_cause,"paging ind. any chan'")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F")

BF(5,M5(1,1,1,1,1),ACT_SHOW,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

IE_BEGIN(request_reference)

BF(3,    M3(1,0,0),ACT_SHOW,random_access_info,"As in CHAN REQ")

BF(5,M5(1,1,1,1,1),ACT_SHOW,  random_reference,SILENT)

BF(5,            0,ACT_SHOW,               t1_,SILENT)

BF(6,            0,ACT_SHOW,               t3 ,SILENT)

BF(5,            0,ACT_SHOW,               t2 ,SILENT)

IE_END(request_reference)

IE_BEGIN(rr_cause)

BF(8,0,ACT_CHECK,rr_cause,"normal event")

IE_END(rr_cause)

IE_BEGIN(signal_call_waiting)

BF(8,M8(0,0,0,0,0,1,1,1),ACT_CHECK,signal_value,"(Any non-res. value)")

IE_END(signal_call_waiting)

IE_BEGIN(spare_half_octet)

BF(4, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(spare_half_octet)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,         spare,SILENT)

BF(6,0,ACT_CHECK,timing_advance,"0")

IE_END(timing_advance)

IE_BEGIN(transaction_identifier_source)

BF(4,M4(0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source)

IE_BEGIN(transaction_identifier_dest)

BF(4,M4(1,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest)

/*------------------------------------------------------------------*\

| Messages

\*------------------------------------------------------------------*/

MSG3_BEGIN(alerting)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(alerting_message_type)

MSG3_END(alerting)

MSG3_BEGIN(assignment_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(assignment_command)

MSG3_BEGIN(assignment_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_complete_message_type)

IE(rr_cause)

MSG3_END(assignment_complete)

MSG3_BEGIN(authentication_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(authentication_parameter_rand)

MSG3_END(authentication_request)

MSG3_BEGIN(authentication_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_response_message_type)

IE(authentication_parameter_sres)

MSG3_END(authentication_response)

MSG3_BEGIN(ciphering_mode_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(cipher_response)

IE(ciphering_mode_setting)

MSG3_END(ciphering_mode_command)

MSG3_BEGIN(ciphering_mode_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

MSG3_END(ciphering_mode_complete)

MSG3_BEGIN(connect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(connect_message_type)

MSG3_END(connect)

MSG3_BEGIN(connect_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(connect_acknowledge_message_type)

MSG3_END(connect_acknowledge)

MSG3_BEGIN(setup)            /* contains 'signal' but no 'Bearer Cap' */

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

MSG3_END(setup)

/* ------- start of testcase --------- */

ISS_INIT	( 8)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

REPEAT(N,8)

IE_BF_SET_VAL         ( cell_identity, ci, N," ci used for SI 3 and SI 6")

IE_BF_SET_VAL         ( neighbour_cell_description, ba_ind,  0," BA for BCCH")

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 0, SILENT)

BS_SET_SYS_INFO       ( N, system_information_type_1)

BS_SET_SYS_INFO       ( N, system_information_type_2)

BS_SET_SYS_INFO       ( N, system_information_type_3)

BS_SET_SYS_INFO       ( N, system_information_type_4)

IE_BF_SET_VAL         ( neighbour_cell_description,  ba_ind, 1," BA for SACCH")

IE_BF_SET_VAL         ( neighbour_cell_description, ext_ind, 1,"")

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5_26_6_3_5_3)

BS_SET_SYS_INFO_SACCH ( N, system_information_type_5bis_26_6_3_5_3)

BS_SET_SYS_INFO_SACCH ( N, system_information_type_6)

REPEAT_END(N)

#define              NCC_0    1

#define              NCC_1    1

#define              NCC_2    1

#define              NCC_3    1

#define              NCC_4    1

#define              NCC_5    1

#define              NCC_6    1

#define              NCC_7    1

#define              BCC_0    3

#define              BCC_1    5

#define              BCC_2    7

#define              BCC_3    1

#define              BCC_4    3

#define              BCC_5    5

#define              BCC_6    7

#define              BCC_7    1

#define              BSIC_0   ((NCC_0<<3)|(BCC_0))

#define              BSIC_1   ((NCC_1<<3)|(BCC_1))

#define              BSIC_2   ((NCC_2<<3)|(BCC_2))

#define              BSIC_3   ((NCC_3<<3)|(BCC_3))

#define              BSIC_4   ((NCC_4<<3)|(BCC_4))

#define              BSIC_5   ((NCC_5<<3)|(BCC_5))

#define              BSIC_6   ((NCC_6<<3)|(BCC_6))

#define              BSIC_7   ((NCC_7<<3)|(BCC_7))

#define              ARFCN_0    1

#define              ARFCN_1    2

#define              ARFCN_2    4

#define              ARFCN_3   11

#define              ARFCN_4   15

#define              ARFCN_5   21

#define              ARFCN_6   26

#define              ARFCN_7   44

#define              PWRLEV_0  -60

#define              PWRLEV_1  -85

#define              PWRLEV_2  -80

#define              PWRLEV_3  -75

#define              PWRLEV_4  -55

#define              PWRLEV_5  -50

#define              PWRLEV_6  -45

#define              PWRLEV_7  -40

BS_SET_SCH           ( 0,  BSIC_0,RFN)

BS_SET_ARFCN         ( 0, ARFCN_0)

BS_SET_POWER         ( 0,PWRLEV_0)

BS_SET_SCH           ( 1,  BSIC_1,RFN)

BS_SET_ARFCN         ( 1, ARFCN_1)

BS_SET_POWER         ( 1,PWRLEV_1)

BS_SET_SCH           ( 2,  BSIC_2,RFN)

BS_SET_ARFCN         ( 2, ARFCN_2)

BS_SET_POWER         ( 2,PWRLEV_2)

BS_SET_SCH           ( 3,  BSIC_3,RFN)

BS_SET_ARFCN         ( 3, ARFCN_3)

BS_SET_POWER         ( 3,PWRLEV_3)

BS_SET_SCH           ( 4,  BSIC_4,RFN)

BS_SET_ARFCN         ( 4, ARFCN_4)

BS_SET_POWER         ( 4,PWRLEV_4)

BS_SET_SCH           ( 5,  BSIC_5,RFN)

BS_SET_ARFCN         ( 5, ARFCN_5)

BS_SET_POWER         ( 5,PWRLEV_5)

BS_SET_SCH           ( 6,  BSIC_6,RFN)

BS_SET_ARFCN         ( 6, ARFCN_6)

BS_SET_POWER         ( 6,PWRLEV_6)

BS_SET_SCH           ( 7,  BSIC_7,RFN)

BS_SET_ARFCN         ( 7, ARFCN_7)

BS_SET_POWER         ( 7,PWRLEV_7)

BS_ON_OFF	( 0,TRUE)

/*------------------------------------------------------------------*\

| Switch on IUT

\*------------------------------------------------------------------*/

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

SET_TIMEOUT         ( 30000)

/*------------------------------------------------------------------*\

| Message exchange

\*------------------------------------------------------------------*/

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

/*

* BS_CONFIG_CHANNEL (0, SACCH, UNACK, SAPI_0)

* BS_MSG3_AWAIT     (0, measurement_report,"allow one unchecked report")

*

* not checked by testcase

*/

BS_ON_OFF	( 1,TRUE)

BS_ON_OFF	( 2,TRUE)

BS_ON_OFF	( 3,TRUE)

BS_ON_OFF	( 4,TRUE)

BS_ON_OFF	( 5,TRUE)

BS_ON_OFF	( 6,TRUE)

BS_ON_OFF	( 7,TRUE)

ISS_DELAY (20000)

REPEAT(COUNT,10)

BS_CONFIG_CHANNEL (0, SACCH, UNACK, SAPI_0)

BS_MSG3_AWAIT     (0, measurement_report,SILENT)

REPEAT_END(COUNT)

AT_SEND ("AT+CFUN=0"," Power Off");

BS_ON_OFF	( 0,FALSE)

BS_ON_OFF	( 1,FALSE)

BS_ON_OFF	( 2,FALSE)

BS_ON_OFF	( 3,FALSE)

BS_ON_OFF	( 4,FALSE)

BS_ON_OFF	( 5,FALSE)

BS_ON_OFF	( 6,FALSE)

BS_ON_OFF	( 7,FALSE)

ISS_DELAY (10000);

History:	28.01.98	VK	Initial

25.01.99	LE	Testcase updated

## Channel Assignment

### MRR180: Dedicated assignment / successful case (26.6.4.1)

Description:	(Ref.: GSM 11.10-1, §26.6.4.1)

Preamble:	MRR000

Script:

{

#define   N   5

IE_BEGIN(cell_channel_description_step_8)

BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28,0x00020800,ACT_CHECK,chan_124_thru_097,"         114  108")
   BF(32,0x00000f02,ACT_CHECK,chan_096_thru_065,"       76..73  66")
   BF(32,0x04083000,ACT_CHECK,chan_064_thru_033,"  59    52 46 45")
   BF(32,0x00000000,ACT_CHECK,chan_032_thru_001,"")
IE_END(cell_channel_description_step_8)

IE_BEGIN(channel_description_step_5)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,  ((N+1) % 8),ACT_CHECK,      time_slot_number,"N+1 mod 8")

BF( 3,            2,ACT_CHECK,training_sequence_code,"chosen arbitrarily")

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            2,ACT_CHECK,                  maio,"0..N-1")

BF( 6,           25,ACT_CHECK,                   hsn,"chosen arbitrarily")

IE_END(channel_description_step_5)

IE_BEGIN(channel_description_step_8)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,  ((N+3) % 8),ACT_CHECK,      time_slot_number," N+3  mod 8")

BF( 3,            7,ACT_CHECK,training_sequence_code,"chosen arbitrarily")

BF( 1,            0,ACT_CHECK,               hopping,"no")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,   ARFCN_BCCH_GSM,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(channel_description_step_8)

IE_BEGIN(channel_description_step_12)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,  ((N+4) % 8),ACT_CHECK,      time_slot_number," N+4  mod 8")

BF( 3,            1,ACT_CHECK,training_sequence_code,"chosen arbitrarily")

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            1,ACT_CHECK,                  maio,"0..N-1")

BF( 6,           34,ACT_CHECK,                   hsn,"chosen arbitrarily")

IE_END(channel_description_step_12)

IE_BEGIN(channel_description_step_15)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH")

BF( 3,  ((N+5) % 8),ACT_CHECK,      time_slot_number," N+5  mod 8")

BF( 3,            2,ACT_CHECK,training_sequence_code,"chosen arbitrarily")

BF( 1,            0,ACT_CHECK,               hopping,"no")

BF( 2,            0,ACT_CHECK,                 spare,SILENT)

BF(10,           10,ACT_CHECK,                 arfcn,"ARFCN of the BCCH")

IE_END(channel_description_step_15)

IE_BEGIN(channel_mode_step_5)

BF(8,0,ACT_CHECK,mode,"SIGNALLING")

IE_END(channel_mode_step_5)

IE_BEGIN(channel_mode_step_8)

BF(8,1,ACT_CHECK,mode,"speech full, version 1")

IE_END(channel_mode_step_8)

IE_BEGIN(channel_mode_step_15)

BF(8,0,ACT_CHECK,mode,"SIGNALLING")

IE_END(channel_mode_step_15)

IE_BEGIN(mobile_allocation_step_5)

BF(8,1,ACT_CHECK,length,"ARFCN [70,50,30,20] need 4 bits - one octet")

BF(8,M8(0,0,0,0,1,1,1,0),ACT_CHECK,ANONYMOUS,"20 is the BCCH [ARFCN exclued]")

IE_END(mobile_allocation_step_5)

IE_BEGIN(mobile_allocation_step_12)

BF(8,2,ACT_CHECK,length,"CA has 11 ARFCN -> 2 octets")

BF(8,M8(0,0,0,0,0,1,1,1),ACT_CHECK,ANONYMOUS,"114,108,76")

BF(8,M8(1,1,1,0,0,0,1,1),ACT_CHECK,ANONYMOUS,"75,74,73,[excluded:66,59,52],46,45")

IE_END(mobile_allocation_step_12)

IE_BEGIN(power_command_step_5)

BF(3,0,ACT_CHECK,spare,SILENT)

BF(5,5,ACT_CHECK,power,"arbitrarily, but with a changed value")

IE_END(power_command_step_5)

IE_BEGIN(power_command_step_8)

BF(3,0,ACT_CHECK,spare,SILENT)

BF(5,8,ACT_CHECK,power,"arbitrarily, but with a changed value")

IE_END(power_command_step_8)

IE_BEGIN(power_command_step_12)

BF(3,0,ACT_CHECK,spare,SILENT)

BF(5,12,ACT_CHECK,power,"arbitrarily, but with a changed value")

IE_END(power_command_step_12)

IE_BEGIN(power_command_step_15)

BF(3,0,ACT_CHECK,spare,SILENT)

BF(5,15,ACT_CHECK,power,"arbitrarily, but with a changed value")

IE_END(power_command_step_15)

IE_BEGIN(starting_time_step_15)

BF(5,0,ACT_CHECK,_t1,SILENT)

BF(6,0,ACT_CHECK, t3,SILENT)

BF(5,0,ACT_CHECK, t2,SILENT)

IE_END(starting_time_step_15)

MSG3_BEGIN(assignment_command_step_5)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_step_5)

IE(power_command_step_5)

IE(iei_63)

IE(channel_mode_step_5)

IE(iei_72)

IE(mobile_allocation_step_5)

MSG3_END(assignment_command_step_5)

MSG3_BEGIN(assignment_command_step_8)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_step_8)

IE(power_command_step_8)

IE(iei_62)

IE(cell_channel_description_step_8)

IE(iei_63)

IE(channel_mode_step_8)

MSG3_END(assignment_command_step_8)

MSG3_BEGIN(assignment_command_step_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_step_12)

IE(power_command_step_12)

IE(iei_72)

IE(mobile_allocation_step_12)

MSG3_END(assignment_command_step_12)

MSG3_BEGIN(assignment_command_step_15)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(channel_description_step_15)

IE(power_command_step_15)

IE(iei_63)

IE(channel_mode_step_15)

IE(iei_7c)

IE(starting_time_step_15)

MSG3_END(assignment_command_step_15)

MSG3_VAR ( msg)

SET_TIMEOUT	( 30000)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 1:")

BS_RACH_AWAIT_COPY  ( 0, channel_request, msg,"  2:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_MSG3_SEND_BEGIN  ( 0, immediate_assignment_GSM,"  3:")

BF_SET_VAL       (ref,MSG3_BF_GET_VAL(msg,ref),"")

BF_SET_VAL       (t1_,MSG3_T1_(msg),SILENT)

BF_SET_VAL       ( t3,MSG3_T3 (msg),SILENT)

BF_SET_VAL       ( t2,MSG3_T2 (msg),SILENT)

BF_SET_VAL       ( channel_type, M5(0,1,0,0,0)," SDCCH/8, subchannel 0")

BF_SET_VAL       ( time_slot_number, N," chosen arbitrarily")

BF_SET_VAL       ( training_sequence_code, 0," chosen arbitrarily")

BF_SET_VAL       ( arfcn, ARFCN_BCCH_GSM," the ARFCN of the BCCH carrier")

BS_MSG3_SEND_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 4:")

BS_MSG3_SEND        ( 0, assignment_command_step_5," 5:")

BS_MSG3_AWAIT       ( 0, assignment_complete," 6:")

ISS_DELAY ( 5000) /* 7: check power level */

BS_MSG3_SEND        ( 0, assignment_command_step_8," 8:")

BS_MSG3_AWAIT       ( 0, assignment_complete," 9:")

BS_MSG3_SEND        ( 0, authentication_request,"10:")

BS_MSG3_AWAIT       ( 0, authentication_response,"11: not L2 acknowledged")

BS_MSG3_SEND        ( 0, assignment_command_step_12," 12:")

BS_MSG3_AWAIT       ( 0, assignment_complete," 13:")

/* performed by DL

BS_MSG3_AWAIT       ( 0, authentication_response,"14: same as in step 11")

*/

BS_MSG3_SEND        ( 0, assignment_command_step_15," 12:")

BS_MSG3_AWAIT       ( 0, assignment_complete," 18:")

MSG3_COPY_DESTROY	( msg)

}

History:	15.12.97	VK	Initial

### MRR181: Dedicated assignment / failure during active state (26.6.4.2.1)

Description:	(Ref.: GSM 11.10-1, §26.6.4.2.1)

Preamble:	MRR135

Script:

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_SET_ERROR        ( 0, 3) /* step 2: generate DL_RELEASE_IND on receipt of DEDI_REQ! */

BS_MSG3_SEND        ( 0, assignment_command," 1:")

BS_MSG3_AWAIT_BEGIN ( 0, assignment_failure," 3:")

BF_SET_VAL       ( cause, M8(0,1,1,0,1,1,1,1)," GSM 04.08, 10.5.2.31")

BS_MSG3_AWAIT_END()

History:	30.01.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR182: Dedicated assignment / failure / general case (26.6.4.2.2)

Description:	(Ref.: GSM 11.10-1, §26.6.4.2.2)

Preamble:	None

Script:

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_DCS)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

COMMAND	("RR CONFIG PCM=<MSCAP,0x33,0,0,0,0,0>")

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 1:")

BS_RACH_AWAIT       ( 0, channel_request," 2:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS(0, 0)

BS_MSG3_SEND        ( 0, immediate_assignment_GSM,"  3: SDCCH test: Channel type = SDCCH/8")

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 4:")

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_SET_ERROR        ( 0, 3)

BS_MSG3_SEND        ( 0, assignment_command_28," 5:")

BS_MSG3_AWAIT_BEGIN ( 0, assignment_failure," 7:")

BF_SET_VAL       ( cause, M8(0,1,1,0,1,1,1,1)," GSM 04.08, 10.5.2.31")

BS_MSG3_AWAIT_END()

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_SET_ERROR        ( 0, 3)

BS_MSG3_SEND        ( 0, assignment_command_28_H," 8:")

BS_MSG3_AWAIT_BEGIN ( 0, assignment_failure," 10:")

BF_SET_VAL       ( cause, M8(0,1,1,0,1,1,1,1)," GSM 04.08, 10.5.2.31")

BS_MSG3_AWAIT_END()

BS_MSG3_SEND        ( 0, assignment_command_28," 11:")

BS_CONFIG_CHANNEL   ( 0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, assignment_complete," 12:")

BS_SET_ERROR        ( 0, 3)

BS_MSG3_SEND        ( 0, assignment_command_28_H," 13:")

BS_MSG3_AWAIT_BEGIN ( 0, assignment_failure," 15:")

BF_SET_VAL       ( cause, M8(0,1,1,0,1,1,1,1)," GSM 04.08, 10.5.2.31")

BS_MSG3_AWAIT_END()

BS_MSG3_SEND        ( 0, channel_release," 16:")

History:	30.01.98	VK	Initial

## Handover

### MRR200: Handover / Preamble (26.6.5.1)

Description:	(Ref.: GSM 11.10-1, §26.6.5.1)

Preamble:	None

Script:

IE_BEGIN(cell_channel_description)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0020800,ACT_CHECK,chan_124_thru_097," 114  108")
   BF(32,0x00000f02,ACT_CHECK,chan_096_thru_065,"   76 75 74 73   66")
   BF(32,0x04083202,ACT_CHECK,chan_064_thru_033," 59 52 46 45 42 34")
   BF(32,0x02090200,ACT_CHECK,chan_032_thru_001,"   26  20  17  10")
IE_END(cell_channel_description)

IE_BEGIN(cell_channel_description_cell_b)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0020800,ACT_CHECK,chan_124_thru_097," 114  108")
   BF(32,0x00000f02,ACT_CHECK,chan_096_thru_065,"   76 75 74 73   66")
   BF(32,0x080000a0,ACT_CHECK,chan_064_thru_033,"  60         40  38")
   BF(32,0x60a22000,ACT_CHECK,chan_032_thru_001,"31 30 24 22 18 14")
IE_END(cell_channel_description_cell_b)

MSG3_BEGIN(system_information_type_1)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1)

MSG3_BEGIN(system_information_type_1_cell_b)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description_cell_b)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_cell_b)

ISS_INIT	( 2)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SYS_INFO	( 1, system_information_type_1_cell_b)

BS_SET_SYS_INFO	( 1, system_information_type_2)

BS_SET_SYS_INFO	( 1, system_information_type_3_cell_b)

BS_SET_SYS_INFO	( 1, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 1, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 1, system_information_type_6_cell_b)

#define MAKE_T1(FN)  ((ULONG)(((FN)/(26*51))     & ((1<<11)-1)))

#define MAKE_T2(FN)  ((ULONG)(((FN)%26)          & ((1<< 5)-1)))

#define MAKE_T3(FN)  ((ULONG)(((((FN)%51)-1)/10) & ((1<< 3)-1)))

#define MAKE_RFN(FN) ((MAKE_T1((FN))<<(5+3)) | (MAKE_T2((FN))<<3) | MAKE_T1(FN))

#define ARFCN_A      20

#define ARFCN_B      40

BS_SET_SCH	( 0,BSIC , 0)

BS_SET_ARFCN	( 0,ARFCN_A)

BS_ON_OFF	( 0,TRUE)

BS_SET_SCH	( 1,BSIC , MAKE_RFN(100))

BS_SET_ARFCN	( 1,ARFCN_B)

BS_ON_OFF	( 1,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

History:	30.01.98	VK	Initial

### MRR201: Handover / successful / active call / non-sync  / M=1 (26.6.5.1)

Description:	(Ref.: GSM 11.10-1, §26.6.5.1)

Preamble:	MRR200

Script:

{

#define   ARFCN_B   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_B>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,         1,ACT_CHECK,ncc,  "   network colour code")

BF( 3,         5,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_B   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(synchronization_indication)

BF( 4, 0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,   0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,   0, ACT_CHECK,   rot,"observed time diff not included")

BF( 2,   0, ACT_CHECK,    si,"synchronization indication")

IE_END(synchronization_indication)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_GSM)

IE(handover_reference)

IE(power_command)

IE(synchronization_indication)

MSG3_END(handover_command)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_SET_POWER	( 1,DOWNLINK_INPUT_LEVEL_DBM_CELL_B)

ISS_DELAY	( 5000) /* wait until the MS has detected the cells */

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup_19,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command_19,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

ISS_DELAY	( 5000) /* wait another 5 sec */

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 1:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 2:")

/*

* not sent in simulation

* BS_MSG3_SEND        ( 1, physical_information," 3:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 6:")

ISS_DELAY	( 5000)

}

History:	30.01.98	VK	Initial

### MRR202: Handover / successful / active call / non-sync  / M=2 (26.6.5.1)

Description:	(Ref.: GSM 11.10-1, §26.6.5.1)

Preamble:	MRR201

Script:

{

#define   ARFCN_HO   20

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          1,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          5,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH/F+ACCHs")

BF( 3,            5,ACT_CHECK,      time_slot_number,"N+1 mod 8")

BF( 3,            4,ACT_CHECK,training_sequence_code,"chosen arbitrarily")

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,           11,ACT_CHECK,                  maio,"0..# in Freq Chan Seq - 1")

BF( 6,           63,ACT_CHECK,                   hsn,"chosen arbitrarily")

IE_END(channel_description)

IE_BEGIN(frequency_channel_sequence)

BF(1, 0,ACT_CHECK,             spare,SILENT)

BF(7,10,ACT_CHECK,      lowest_arfcn," 10")

BF(4, 7,ACT_CHECK, inc_skip_arfcn_01," 17= 10+ 7")

BF(4, 3,ACT_CHECK, inc_skip_arfcn_02," 20= 17+ 3")

BF(4, 6,ACT_CHECK, inc_skip_arfcn_03," 26= 20+ 6")

BF(4, 0,ACT_CHECK, inc_skip_arfcn_04," 41= 26+15 (unused)")

BF(4, 0,ACT_CHECK, inc_skip_arfcn_05," 56= 41+15 (unused)")

BF(4, 3,ACT_CHECK, inc_skip_arfcn_06," 59= 56+ 3")

BF(4, 7,ACT_CHECK, inc_skip_arfcn_07," 66= 59+ 7")

BF(4, 7,ACT_CHECK, inc_skip_arfcn_08," 73= 66+ 7")

BF(4, 1,ACT_CHECK, inc_skip_arfcn_09," 74= 73+ 1")

BF(4, 1,ACT_CHECK, inc_skip_arfcn_10," 75= 74+ 1")

BF(4, 1,ACT_CHECK, inc_skip_arfcn_11," 76= 75+ 1")

BF(4, 0,ACT_CHECK, inc_skip_arfcn_12," 91= 76+15 (unused)")

BF(4, 0,ACT_CHECK, inc_skip_arfcn_13,"106= 91+15 (unused)")

BF(4, 2,ACT_CHECK, inc_skip_arfcn_14,"108=106+ 2")

BF(4, 6,ACT_CHECK, inc_skip_arfcn_15,"114=108+ 6")

BF(4, 0,ACT_CHECK, inc_skip_arfcn_16,"unused")

IE_END(frequency_channel_sequence)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description)

IE(handover_reference)

IE(power_command)
   IE(iei_69)

IE(frequency_channel_sequence)

MSG3_END(handover_command)

ISS_DELAY	( 5000) /* wait 5 sec */

BS_CONFIG_CHANNEL   ( 1, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 1, handover_command," 1:")

BS_CONFIG_CHANNEL   ( 0, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 0, handover_access," 2:")

/*

* not send in simulation

*

* BS_MSG3_SEND        ( 0, physical_information," 3:")

*/

BS_CONFIG_CHANNEL   ( 0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, handover_complete," 6:")

ISS_DELAY	( 5000)

}

History:	04.02.98	VK	Initial

### MRR203: Handover / successful / active call / non-sync  / M=3 (26.6.5.1)

Description:	(Ref.: GSM 11.10-1, §26.6.5.1)

Preamble:	MRR202

Script:

{

#define   ARFCN_HO2   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO2>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,           1,ACT_CHECK,ncc,  "   network colour code")

BF( 3,           5,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO2   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(starting_time)

BF(5,10,ACT_CHECK,_t1,SILENT)

BF(6,30,ACT_CHECK, t3,SILENT)

BF(5,20,ACT_CHECK, t2,SILENT)

IE_END(starting_time)

IE_BEGIN(synchronization_indication)

BF( 4, 0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,   1, ACT_CHECK,   nci,"out of range TAV shall trigger HO failure")

BF( 1,   0, ACT_CHECK,   rot,"observed time diff not included")

BF( 2,   0, ACT_CHECK,    si,"Non-synchronized")

IE_END(synchronization_indication)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_GSM)

IE(handover_reference)

IE(power_command)
   IE(synchronization_indication)

IE(iei_63)

IE(mode_of_the_first_channel)

IE(iei_7c)

IE(starting_time)

MSG3_END(handover_command)

ISS_DELAY	( 5000) /* wait 5 sec */

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 1:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 2:")

/*

* not send in simulation

*

* BS_MSG3_SEND        ( 1, physical_information," 3:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 6:")

ISS_DELAY	( 5000)

}

History:	05.02.98	VK	Initial

### MRR210: Handover / Preamble (26.6.5.2)

Description:	(Ref.: GSM 11.10-1, §26.6.5.2)

Preamble:	None

Script:

IE_BEGIN(cell_channel_description)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0020800,ACT_CHECK,chan_124_thru_097," 114  108")
   BF(32,0x00000f02,ACT_CHECK,chan_096_thru_065,"   76 75 74 73   66")
   BF(32,0x04083202,ACT_CHECK,chan_064_thru_033," 59 52 46 45 42 34")
   BF(32,0x02090200,ACT_CHECK,chan_032_thru_001,"   26  20  17  10")
IE_END(cell_channel_description)

IE_BEGIN(cell_channel_description_cell_b)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0020800,ACT_CHECK,chan_124_thru_097," 114  108")
   BF(32,0x00000f02,ACT_CHECK,chan_096_thru_065,"   76 75 74 73   66")
   BF(32,0x080000a0,ACT_CHECK,chan_064_thru_033,"  60         40  38")
   BF(32,0x60a22000,ACT_CHECK,chan_032_thru_001,"31 30 24 22 18 14")
IE_END(cell_channel_description_cell_b)

MSG3_BEGIN(system_information_type_1)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1)

MSG3_BEGIN(system_information_type_1_cell_b)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description_cell_b)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1_cell_b)

ISS_INIT	( 2)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

/* the following setup of mcc and mnc do not conform with the GSM 11.10 */

IE_BF_SET_VAL	( location_area_identification, mcc_dig_1, 2, SILENT)

IE_BF_SET_VAL	( location_area_identification, mcc_dig_2, 6, SILENT)

IE_BF_SET_VAL	( location_area_identification, mcc_dig_3, 2, SILENT)

IE_BF_SET_VAL	( location_area_identification, mnc_dig_1, 0, SILENT)

IE_BF_SET_VAL	( location_area_identification, mnc_dig_2, 1, SILENT)

IE_BF_SET_VAL	( ncc_permitted, ncc_permit, M8(0,0,0,0,1,0,1,0)," Allow NCC=3, NCC=1")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

IE_BF_SET_VAL	( ncc_permitted, ncc_permit, M8(0,0,0,0,0,0,1,0)," Allow NCC=1")

BS_SET_SYS_INFO	( 1, system_information_type_1_cell_b)

BS_SET_SYS_INFO	( 1, system_information_type_2)

BS_SET_SYS_INFO	( 1, system_information_type_3_cell_b)

BS_SET_SYS_INFO	( 1, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 1, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 1, system_information_type_6_cell_b)

#define ARFCN_A      20

#define ARFCN_B      40

#define  NCC_B	0x3  /* note : ncc_permitted (GSM 04.08, 10.5.2.27) */
#define  BCC_B	0x0  /* note : GSM 11.10, 26.6.14, IMMEDIATE ASSIGNMENT */
#define  BSIC_B	((NCC_B<<3)|(BCC_B))

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_A)

BS_ON_OFF	( 0,TRUE)

BS_SET_POWER	( 1,DOWNLINK_INPUT_LEVEL_DBM_CELL_B)

BS_SET_SCH	( 1,BSIC_B, RFN)

BS_SET_ARFCN	( 1,ARFCN_B)

BS_ON_OFF	( 1,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 20000)

History:	09.02.98	VK	Initial

### MRR211: Handover / successful / call estab. / non-sync / M=1 (26.6.5.2)

Description:	(Ref.: GSM 11.10-1, §26.6.5.2)

Preamble:	MRR210

Script:

{

#define   ARFCN_HO   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          3,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          0,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH/F+ACCHs")

BF( 3,            0,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            4,ACT_CHECK,training_sequence_code,"chosen arbitrarily")

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,           11,ACT_CHECK,                  maio,"0..# in Freq Chan Seq - 1")

BF( 6,           63,ACT_CHECK,                   hsn,"chosen arbitrarily")

IE_END(channel_description)

IE_BEGIN(frequency_channel_sequence)

BF(1, 0,ACT_CHECK,             spare,SILENT)

BF(7,14,ACT_CHECK,      lowest_arfcn," 14")

BF(4, 4,ACT_CHECK, inc_skip_arfcn_01," 18= 14+ 4")

BF(4, 4,ACT_CHECK, inc_skip_arfcn_02," 22= 18+ 4")

BF(4, 2,ACT_CHECK, inc_skip_arfcn_03," 24= 22+ 2")

BF(4, 6,ACT_CHECK, inc_skip_arfcn_04," 30= 24+ 6")

BF(4, 1,ACT_CHECK, inc_skip_arfcn_05," 31= 30+ 1")

BF(4, 7,ACT_CHECK, inc_skip_arfcn_06," 38= 31+ 7")

BF(4,15,ACT_CHECK, inc_skip_arfcn_07," 53= 38+15")

BF(4,13,ACT_CHECK, inc_skip_arfcn_08," 66= 53+13")

BF(4, 7,ACT_CHECK, inc_skip_arfcn_09," 73= 66+ 7")

BF(4, 1,ACT_CHECK, inc_skip_arfcn_10," 74= 73+ 1")

BF(4, 1,ACT_CHECK, inc_skip_arfcn_11," 75= 74+ 1")

BF(4, 1,ACT_CHECK, inc_skip_arfcn_12," 76= 75+ 1")

BF(4, 0,ACT_CHECK, inc_skip_arfcn_13," 91= 76+15 (not used)")

BF(4, 0,ACT_CHECK, inc_skip_arfcn_14,"106= 91+15 (not used)")

BF(4, 2,ACT_CHECK, inc_skip_arfcn_15,"108=106+ 2")

BF(4, 6,ACT_CHECK, inc_skip_arfcn_16,"114=108+ 6")

IE_END(frequency_channel_sequence)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description)

IE(handover_reference)

IE(power_command)
   IE(iei_63)

IE(mode_of_the_first_channel)

IE(iei_69)

IE(frequency_channel_sequence)

MSG3_END(handover_command)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

AT_SEND ("ATD1234567890123456;"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"2:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"4:")

BS_MSG3_SEND         (0,ciphering_mode_command,"5:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"6:")

BS_MSG3_AWAIT        (0,setup,"7:")

ISS_DELAY ( 15000)   /* wait for measurement result -> sync to neighbour cell */

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 8:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 9:")

/*

* not send in simulation

*

* BS_MSG3_SEND        ( 1, physical_information," 3:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 11:")

BS_MSG3_SEND        ( 1, channel_release," 15:")

}

History:	09.02.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR212: Handover / successful / call estab. / non-sync / M=3 (26.6.5.2)

Description:	(Ref.: GSM 11.10-1, §26.6.5.2)

Preamble:	MRR210

Script:

{

#define   ARFCN_HO   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          3,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          0,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description)

BF( 5,M5(0,1,0,0,0),ACT_CHECK,          channel_type,"SDCCH/8+SACCH/C8")

BF( 3,            0,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            4,ACT_CHECK,training_sequence_code,"chosen arbitrarily")

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            7,ACT_CHECK,                  maio,"0..# in Freq Chan Seq - 1")

BF( 6,           21,ACT_CHECK,                   hsn,"chosen arbitrarily")

IE_END(channel_description)

IE_BEGIN(frequency_list)

BF( 8,        16,ACT_CHECK,length           ,SILENT)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,SILENT)
   BF(28, 0x0020800,ACT_CHECK,chan_124_thru_097," 114  108")
   BF(32,0x00000f02,ACT_CHECK,chan_096_thru_065,"   76 75 74 73   66")
   BF(32,0x08000020,ACT_CHECK,chan_064_thru_033,"  60            38")
   BF(32,0x60a22000,ACT_CHECK,chan_032_thru_001,"31 30 24 22 18 14")
IE_END(frequency_list)

IE_BEGIN(mode_of_the_first_channel)

BF(8,0,ACT_CHECK,mode,"Signalling only")

IE_END(mode_of_the_first_channel)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description)

IE(handover_reference)

IE(power_command)
   IE(iei_05)

IE(frequency_list)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(handover_command)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

AT_SEND ("ATD1234567890123456;"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"2:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"4:")

BS_MSG3_SEND         (0,ciphering_mode_command,"5:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"6:")

BS_MSG3_AWAIT        (0,setup,"7:")

ISS_DELAY (8000)

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 8:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 9:")

/*

* not send in simulation

*

* BS_MSG3_SEND        ( 1, physical_information," 3:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 11:")

BS_MSG3_SEND        ( 1, channel_release," 15:")

}

History:	10.02.98	VK	Initial.

25.01.99	LE	Testcase updated

### MRR213: Handover / successful / call estab. / non-sync / M=7 (26.6.5.2)

Description:	(Ref.: GSM 11.10-1, §26.6.5.2)

Preamble:	MRR210

Script:

{

#define   ARFCN_HO   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          3,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          0,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description_ho)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH/F + ACCHs")

BF( 3,            0,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            4,ACT_CHECK,training_sequence_code,"chosen arbitrarily")

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            7,ACT_CHECK,                  maio,"0..# in Freq Chan Seq - 1")

BF( 6,           21,ACT_CHECK,                   hsn,"chosen arbitrarily")

IE_END(channel_description_ho)

IE_BEGIN(channel_description_imm_ass)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,          channel_type,"TCH/F + ACCHs")

BF( 3,            0,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            4,ACT_CHECK,training_sequence_code,"chosen arbitrarily")

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            2,ACT_CHECK,                  maio,"0..# in Freq Chan Seq - 1")

BF( 6,            0,ACT_CHECK,                   hsn,"chosen arbitrarily")

IE_END(channel_description_imm_ass)

IE_BEGIN(frequency_list)

BF( 8,        16,ACT_CHECK,length           ,SILENT)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,SILENT)
   BF(28, 0x0020800,ACT_CHECK,chan_124_thru_097," 114  108")
   BF(32,0x00000f02,ACT_CHECK,chan_096_thru_065,"   76 75 74 73   66")
   BF(32,0x080000A0,ACT_CHECK,chan_064_thru_033,"  60        40  38")
   BF(32,0x60a22000,ACT_CHECK,chan_032_thru_001,"31 30 24 22 18 14")
IE_END(frequency_list)

IE_BEGIN(ia_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

IE_END(ia_rest_octets)

IE_BEGIN(mobile_allocation)

BF(8,3,ACT_CHECK,length  ,"3 octets required by test case")

BF(8,M8(0,0,0,0,0,0,0,1),ACT_CHECK,ANONYMOUS,SILENT)

BF(8,M8(0,0,0,0,0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

BF(8,M8(0,0,0,0,0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobile_allocation)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_ho)

IE(handover_reference)

IE(power_command)
   IE(iei_05)

IE(frequency_list)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(handover_command)

MSG3_BEGIN(immediate_assignment_213)

IE(l2_pseudo_length_14)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_imm_ass)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment_213)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

AT_SEND ("ATD1234567890123456;"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"2:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment_213,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"4:")

BS_MSG3_SEND         (0,ciphering_mode_command,"5:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"6:")

BS_MSG3_AWAIT        (0,setup,"7:")

ISS_DELAY (8000);

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 8:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 9:")

/*

* not send in simulation

*

* BS_MSG3_SEND        ( 1, physical_information," 3:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 11:")

BS_MSG3_SEND        ( 1, channel_release," 15:")

}

History:	10.02.98	VK	Initial

25.01.99	LE	Testcase updated

### MRR220: Handover / successful / active call / finely sync / M=1 (26.6.5.3)

Description:	(Ref.: GSM 11.10-1, §26.6.5.3)

Preamble:	MRR200

Script:

{

#define   ARFCN_B   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_B>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,         1,ACT_CHECK,ncc,  "   network colour code")

BF( 3,         5,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_B   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,  channel_type,"Bm + ACCHs (TCH/F+ACCHs)")

BF( 3,            5,ACT_CHECK,  time_slot_number,"Arbitaray value, but not zero")

BF( 3,            4,ACT_CHECK,  training_sequence_code, SILENT)

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,      ARFCN_B,ACT_CHECK,  arfcn,"channel number 40")

IE_END(channel_description)

IE_BEGIN(synchronization_indication)

BF( 4, 0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,   0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,   0, ACT_CHECK,   rot,"observed time diff not included")

BF( 2,   1, ACT_CHECK,    si,"synchronization indication")

IE_END(synchronization_indication)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description)

IE(handover_reference)

IE(power_command)

IE(synchronization_indication)

MSG3_END(handover_command)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_SET_POWER	( 1,DOWNLINK_INPUT_LEVEL_DBM_CELL_B)

ISS_DELAY	( 5000) /* wait until the MS has detected the cells */

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup_19,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command_19,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

ISS_DELAY (15000);

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 1:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 2:")

/*

* only one rach message in simulation

*

* BS_RACH_AWAIT       ( 1, handover_access," 3:")

* BS_RACH_AWAIT       ( 1, handover_access," 4:")

* BS_RACH_AWAIT       ( 1, handover_access," 5:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 6:")

ISS_DELAY	( 5000)

}

History:	10.02.98	VK	Initial

### MRR240: Handover / successful / active call / pre-sync / no TA IE (26.6.5.5.1)

Description:	(Ref.: GSM 11.10-1, §26.6.5.5.1)
Timing Advance shall be set to 1.

Preamble:	MRR200

Script:

{

#define   ARFCN_HO   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          1,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          5,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,  channel_type,"Bm + ACCHs (TCH/F+ACCHs)")

BF( 3,            5,ACT_CHECK,  time_slot_number,"Arbitaray value, but not zero")

BF( 3,            4,ACT_CHECK,  training_sequence_code, SILENT)

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,     ARFCN_HO,ACT_CHECK,  arfcn,"channel number 40")

IE_END(channel_description)

IE_BEGIN(synchronization_indication)

BF( 4,     0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,       0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,       0, ACT_CHECK,   rot,"observed time diff not included")

BF( 2, M2(1,0), ACT_CHECK,    si,"pre-synchronized")

IE_END(synchronization_indication)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description)

IE(handover_reference)

IE(power_command)

IE(synchronization_indication)

MSG3_END(handover_command)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_SET_POWER	( 1,DOWNLINK_INPUT_LEVEL_DBM_CELL_B)

ISS_DELAY	( 5000) /* wait until the MS has detected the cells */

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup_19,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command_19,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

ISS_DELAY (15000);

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 1:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 2:")

/*

* only one rach message in simulation

*

* BS_RACH_AWAIT       ( 1, handover_access," 3:")

* BS_RACH_AWAIT       ( 1, handover_access," 4:")

* BS_RACH_AWAIT       ( 1, handover_access," 5:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 8:")

ISS_DELAY	( 5000)

}

History:	10.02.98	VK	Initial.

25.01.99	LE	Testcase updated

### MRR241: Handover / successful / active call / pre-sync / TA IE / ROT (26.6.5.5.2)

Description:	(Ref.: GSM 11.10-1, §26.6.5.5.2)

Preamble:	MRR210

Script:

{

#define   ARFCN_HO   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          3,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          0,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description_ho)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,  channel_type,"TCH/F + ACCHs")

BF( 3,            0,ACT_CHECK,  time_slot_number,SILENT)

BF( 3,            4,ACT_CHECK,  training_sequence_code,"chosen arbitrarily")

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,     ARFCN_HO,ACT_CHECK,  arfcn,SILENT)

IE_END(channel_description_ho)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,0),ACT_CHECK,  channel_type,"to an SDCCH/4")

BF( 3, TIMESLOT_TCH,ACT_CHECK,  time_slot_number, SILENT)

BF( 3,          BCC,ACT_CHECK,  training_sequence_code, SILENT)

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,    ARFCN_TCH_GSM,ACT_CHECK,  arfcn,"channel number 30")

IE_END(channel_description)

#define   K   5   /* The BCCH of cell A is sent k bit periods before the BCCH of cell B */

#define   Y   10  /* The MS is commanded to use a timing advance of y bit periods on cell A*/

IE_BEGIN(mobile_time_difference)

BF( 8,     3,ACT_CHECK,length,"3 octets to come")

BF(21, 2*K+Y,ACT_CHECK,   mtd, SILENT)

BF( 3,     0,ACT_CHECK, spare, SILENT)

IE_END(mobile_time_difference)

IE_BEGIN(synchronization_indication)

BF( 4,     0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,       0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,       1, ACT_CHECK,   rot,"observed time diff not included")

BF( 2, M2(1,0), ACT_CHECK,    si,"pre-synchronized")

IE_END(synchronization_indication)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,spare,SILENT)

BF(6,7,ACT_CHECK,tav  ,"9 bit periods")

IE_END(timing_advance)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_ho)

IE(handover_reference)

IE(power_command)
   IE(synchronization_indication)

IE(iei_7d)

IE(timing_advance)

MSG3_END(handover_command)

MSG3_BEGIN(handover_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_complete_message_type)

IE(rr_cause)

IE(iei_77)

IE(mobile_time_difference)

MSG3_END(handover_complete)

MSG3_BEGIN(immediate_assignment)

IE(l2_pseudo_length_14)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

BS_SET_TIMING_ADVANCE(0,Y)

BS_SET_TIMING_ADVANCE(1,K)  /* BCCH of cell A is k bit before BCCH of cell B      */

AT_SEND ("ATD1234567890123456;"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"2:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"4:")

BS_MSG3_SEND         (0,ciphering_mode_command,"5:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"6:")

BS_MSG3_AWAIT        (0,setup,"7:")

ISS_DELAY (15000);

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 8:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 9:")

/*

* only one rach message in simulation

*

* BS_RACH_AWAIT       ( 1, handover_access," 10:")

* BS_RACH_AWAIT       ( 1, handover_access," 11:")

* BS_RACH_AWAIT       ( 1, handover_access," 12:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 15:")

BS_MSG3_SEND        ( 1, channel_release," 18:")

}

History:	11.02.98	VK	Initial.

25.01.99	LE	Testcase updated

### MRR242: Handover / successful / active call / pre-sync / TA IE / ROT (26.6.5.5.2)

Description:	(Ref.: GSM 11.10-1, §26.6.5.5.2). Same as MRR241, but with different parameters.

Preamble:	MRR210

Script:

{

#define   ARFCN_HO   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          3,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          0,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description_ho)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,  channel_type,"TCH/F + ACCHs")

BF( 3,            0,ACT_CHECK,  time_slot_number,SILENT)

BF( 3,            4,ACT_CHECK,  training_sequence_code,"chosen arbitrarily")

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,     ARFCN_HO,ACT_CHECK,  arfcn,SILENT)

IE_END(channel_description_ho)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,0),ACT_CHECK,  channel_type,"to an SDCCH/4")

BF( 3, TIMESLOT_TCH,ACT_CHECK,  time_slot_number, SILENT)

BF( 3,          BCC,ACT_CHECK,  training_sequence_code, SILENT)

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,    ARFCN_TCH_GSM,ACT_CHECK,  arfcn,"channel number 30")

IE_END(channel_description)

#define   K   11   /* The BCCH of cell A is sent k bit periods before the BCCH of cell B */

#define   Y   0    /* The MS is commanded to use a timing advance of y bit periods on cell A*/

IE_BEGIN(mobile_time_difference)

BF( 8,     3,ACT_CHECK,length,"3 octets to come")

BF(21, 2*K+Y,ACT_CHECK,   mtd, SILENT)

BF( 3,     0,ACT_CHECK, spare, SILENT)

IE_END(mobile_time_difference)

IE_BEGIN(synchronization_indication)

BF( 4,     0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,       0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,       1, ACT_CHECK,   rot,"observed time diff not included")

BF( 2, M2(1,0), ACT_CHECK,    si,"pre-synchronized")

IE_END(synchronization_indication)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,spare,SILENT)

BF(6,7,ACT_CHECK,tav  ,"9 bit periods")

IE_END(timing_advance)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_ho)

IE(handover_reference)

IE(power_command)
   IE(synchronization_indication)

IE(iei_7d)

IE(timing_advance)

MSG3_END(handover_command)

MSG3_BEGIN(handover_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_complete_message_type)

IE(rr_cause)

IE(iei_77)

IE(mobile_time_difference)

MSG3_END(handover_complete)

MSG3_BEGIN(immediate_assignment)

IE(l2_pseudo_length_14)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

BS_SET_TIMING_ADVANCE(0,Y)

BS_SET_TIMING_ADVANCE(1,K)  /* BCCH of cell A is k bit before BCCH of cell B      */

AT_SEND ("ATD1234567890123456;"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"2:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"4:")

BS_MSG3_SEND         (0,ciphering_mode_command,"5:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"6:")

BS_MSG3_AWAIT        (0,setup,"7:")

ISS_DELAY (15000);

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 8:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 9:")

/*

* only one rach message in simulation

*

* BS_RACH_AWAIT       ( 1, handover_access," 10:")

* BS_RACH_AWAIT       ( 1, handover_access," 11:")

* BS_RACH_AWAIT       ( 1, handover_access," 12:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 15:")

BS_MSG3_SEND        ( 1, channel_release," 18:")

}

History:	11.02.98	VK	Initial.

25.01.99	LE	Testcase updated

### MRR243: Handover / successful / active call / pre-sync / TA IE / ROT (26.6.5.5.2)

Description:	(Ref.: GSM 11.10-1, §26.6.5.5.2). Same as MRR241, but with different parameters.

Preamble:	MRR210

Script:

{

#define   ARFCN_HO   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          3,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          0,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description_ho)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,  channel_type,"TCH/F + ACCHs")

BF( 3,            0,ACT_CHECK,  time_slot_number,SILENT)

BF( 3,            4,ACT_CHECK,  training_sequence_code,"chosen arbitrarily")

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,     ARFCN_HO,ACT_CHECK,  arfcn,SILENT)

IE_END(channel_description_ho)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,0),ACT_CHECK,  channel_type,"to an SDCCH/4")

BF( 3, TIMESLOT_TCH,ACT_CHECK,  time_slot_number, SILENT)

BF( 3,          BCC,ACT_CHECK,  training_sequence_code, SILENT)

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,    ARFCN_TCH_GSM,ACT_CHECK,  arfcn,"channel number 30")

IE_END(channel_description)

#define   K   35   /* The BCCH of cell A is sent k bit periods before the BCCH of cell B */

#define   Y   40   /* The MS is commanded to use a timing advance of y bit periods on cell A*/

IE_BEGIN(mobile_time_difference)

BF( 8,     3,ACT_CHECK,length,"3 octets to come")

BF(21, 2*K+Y,ACT_CHECK,   mtd, SILENT)

BF( 3,     0,ACT_CHECK, spare, SILENT)

IE_END(mobile_time_difference)

IE_BEGIN(synchronization_indication)

BF( 4,     0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,       0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,       1, ACT_CHECK,   rot,"observed time diff not included")

BF( 2, M2(1,0), ACT_CHECK,    si,"pre-synchronized")

IE_END(synchronization_indication)

IE_BEGIN(timing_advance)

BF(2,0,ACT_CHECK,spare,SILENT)

BF(6,7,ACT_CHECK,tav  ,"9 bit periods")

IE_END(timing_advance)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_ho)

IE(handover_reference)

IE(power_command)
   IE(synchronization_indication)

IE(iei_7d)

IE(timing_advance)

MSG3_END(handover_command)

MSG3_BEGIN(handover_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_complete_message_type)

IE(rr_cause)

IE(iei_77)

IE(mobile_time_difference)

MSG3_END(handover_complete)

MSG3_BEGIN(immediate_assignment)

IE(l2_pseudo_length_14)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

BS_SET_TIMING_ADVANCE(0,Y)

BS_SET_TIMING_ADVANCE(1,K)  /* BCCH of cell A is k bit before BCCH of cell B      */

AT_SEND ("ATD1234567890123456;"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"2:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"4:")

BS_MSG3_SEND         (0,ciphering_mode_command,"5:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"6:")

BS_MSG3_AWAIT        (0,setup,"7:")

ISS_DELAY (15000);

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 8:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 9:")

/*

* only one rach message in simulation

*

* BS_RACH_AWAIT       ( 1, handover_access," 10:")

* BS_RACH_AWAIT       ( 1, handover_access," 11:")

* BS_RACH_AWAIT       ( 1, handover_access," 12:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 15:")

BS_MSG3_SEND        ( 1, channel_release," 18:")

}

History:	11.02.98	VK	Initial.

25.01.99	LE	Testcase updated

### MRR250: Handover / successful / active call / pseudo-sync (26.6.5.6)

Description:	(Ref.: GSM 11.10-1, §26.6.5.6)

Preamble:	MRR210

Script:

{

#define   ARFCN_HO   40

#define   K   5   /* The BCCH of cell A is sent k bit periods before the BCCH of cell B */

#define   Y   10  /* The MS is commanded to use a timing advance of y bit periods on cell A*/

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          3,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          0,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description_ho)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,  channel_type,"TCH/F + ACCHs")

BF( 3,            0,ACT_CHECK,  time_slot_number,SILENT)

BF( 3,            4,ACT_CHECK,  training_sequence_code," chosen arbitrarily")

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,     ARFCN_HO,ACT_CHECK,  arfcn,SILENT)

IE_END(channel_description_ho)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,0),ACT_CHECK,  channel_type,"to an SDCCH/4")

BF( 3, TIMESLOT_TCH,ACT_CHECK,  time_slot_number,"chosen arbitrarily by the test house")

BF( 3,          BCC,ACT_CHECK,  training_sequence_code, SILENT)

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,    ARFCN_TCH_GSM,ACT_CHECK,  arfcn,"channel number 30")

IE_END(channel_description)

IE_BEGIN(time_difference)

BF( 8,     1,ACT_CHECK,length,"3 octets to come")

BF( 8,     Y,ACT_CHECK,   tdv, SILENT)

IE_END(time_difference)

IE_BEGIN(mobile_time_difference)

BF( 8,     3,ACT_CHECK,length,"3 octets to come")

BF( 8,     0,ACT_CHECK,mtd_1,SILENT)

BF( 8,     0,ACT_CHECK,mtd_2,SILENT)

BF( 8,  0x50,ACT_CHECK,mtd_3,SILENT)

IE_END(mobile_time_difference)

IE_BEGIN(synchronization_indication)

BF( 4,     0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,       0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,       1, ACT_CHECK,   rot,"observed time diff not included")

BF( 2, M2(1,1), ACT_CHECK,    si,"pseudo-synchronized")

IE_END(synchronization_indication)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_ho)

IE(handover_reference)

IE(power_command)
   IE(synchronization_indication)

IE(iei_7b)

IE(time_difference)

MSG3_END(handover_command)

MSG3_BEGIN(handover_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_complete_message_type)

IE(rr_cause)

IE(iei_77)

IE(mobile_time_difference)

MSG3_END(handover_complete)

MSG3_BEGIN(immediate_assignment)

IE(l2_pseudo_length_14)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

COMMAND (" RR CONFIG PSEUDO_SYNC_HO=1")	/* switch on ps capability */

BS_SET_TIMING_ADVANCE(0,0)

BS_SET_TIMING_ADVANCE(1,K)  /* BCCH of cell A is k bit before BCCH of cell B      */

AT_SEND ("ATD1234567890123456;"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"2:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request_pseudo_sync,"4:")	/* use ps capability */

BS_MSG3_SEND         (0,ciphering_mode_command,"5:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"6:")

BS_MSG3_AWAIT        (0,setup,"7:")

ISS_DELAY (15000);

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 8:")

BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

BS_RACH_AWAIT       ( 1, handover_access," 9:")

/*

* only one rach message in simulation

*

* BS_RACH_AWAIT       ( 1, handover_access," 10:")

* BS_RACH_AWAIT       ( 1, handover_access," 11:")

* BS_RACH_AWAIT       ( 1, handover_access," 12:")

*/

BS_CONFIG_CHANNEL   ( 1, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 1, handover_complete," 13:")

BS_MSG3_SEND        ( 1, channel_release," 14:")

}

History:	11.02.98	VK	Initial.

25.01.99	LE	Testcase updated

24.06.02	MSB	configure the capability of pseudo-sync HO

### MRR251: Handover / failure / active call / pseudo-sync (not supported)

Description:

Preamble:	MRR210

Script:

{

#define   ARFCN_HO   40

#define   K   5   /* The BCCH of cell A is sent k bit periods before the BCCH of cell B */

#define   Y   10  /* The MS is commanded to use a timing advance of y bit periods on cell A*/

IE_BEGIN(cell_description)

BF( 2,ARFCN_HO>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,          3,ACT_CHECK,ncc,  "   network colour code")

BF( 3,          0,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_HO   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(channel_description_ho)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,  channel_type,"TCH/F + ACCHs")

BF( 3,            0,ACT_CHECK,  time_slot_number,SILENT)

BF( 3,            4,ACT_CHECK,  training_sequence_code," chosen arbitrarily")

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,     ARFCN_HO,ACT_CHECK,  arfcn,SILENT)

IE_END(channel_description_ho)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,0),ACT_CHECK,  channel_type,"to an SDCCH/4")

BF( 3, TIMESLOT_TCH,ACT_CHECK,  time_slot_number,"chosen arbitrarily by the test house")

BF( 3,          BCC,ACT_CHECK,  training_sequence_code, SILENT)

BF( 1,            0,ACT_CHECK,  hopping,"single RF channel")

BF( 2,            0,ACT_CHECK,  spare,SILENT)

BF(10,    ARFCN_TCH_GSM,ACT_CHECK,  arfcn,"channel number 30")

IE_END(channel_description)

IE_BEGIN(time_difference)

BF( 8,     1,ACT_CHECK,length,"3 octets to come")

BF( 8,     Y,ACT_CHECK,   tdv, SILENT)

IE_END(time_difference)

IE_BEGIN(mobile_time_difference)

BF( 8,     3,ACT_CHECK,length,"3 octets to come")

BF( 8,     0,ACT_CHECK,mtd_1,SILENT)

BF( 8,     0,ACT_CHECK,mtd_2,SILENT)

BF( 8,  0x50,ACT_CHECK,mtd_3,SILENT)

IE_END(mobile_time_difference)

IE_BEGIN(synchronization_indication)

BF( 4,     0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,       0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,       1, ACT_CHECK,   rot,"observed time diff not included")

BF( 2, M2(1,1), ACT_CHECK,    si,"pseudo-synchronized")

IE_END(synchronization_indication)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_ho)

IE(handover_reference)

IE(power_command)
   IE(synchronization_indication)

IE(iei_7b)

IE(time_difference)

MSG3_END(handover_command)

MSG3_BEGIN(handover_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_complete_message_type)

IE(rr_cause)

IE(iei_77)

IE(mobile_time_difference)

MSG3_END(handover_complete)

MSG3_BEGIN(immediate_assignment_251)

IE(l2_pseudo_length_14)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment_251)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

BS_SET_TIMING_ADVANCE(0,0)

BS_SET_TIMING_ADVANCE(1,K)  /* BCCH of cell A is k bit before BCCH of cell B      */

AT_SEND ("ATD1234567890123456;"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"2:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment_251,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"4:")

BS_MSG3_SEND         (0,ciphering_mode_command,"5:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"6:")

BS_MSG3_AWAIT        (0,setup,"7:")

ISS_DELAY (15000);

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 7:")

/* no support of pseudo-synchronized HO */

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, handover_failure_incorrect_msg," 8:")

BS_MSG3_SEND        ( 1, channel_release," 9:")

}

History:	11.02.98	VK	Initial.

25.01.99	LE	Testcase updated

24.06.02	MSB	Test of not supported pseudo-sync HO

### MRR270: Handover / layer 3 failure (26.6.5.8)

Description:	(Ref.: GSM 11.10-1, §26.6.5.8)

Preamble:	MRR200

Script:

{

#define   ARFCN_B   40

IE_BEGIN(cell_description)

BF( 2,ARFCN_B>>8,ACT_CHECK,arfcn_hi,"BCCH ARFCN (high part)")

BF( 3,         1,ACT_CHECK,ncc,  "   network colour code")

BF( 3,         5,ACT_CHECK,bcc,  "   base station colour code")

BF( 8,ARFCN_B   ,ACT_CHECK,arfcn_lo,"BCCH ARFCN (low part)")

IE_END(cell_description)

IE_BEGIN(power_command)

BF(3,0,ACT_CHECK,spare,SILENT)

BF(5,8,ACT_CHECK,power,SILENT)

IE_END(power_command)

IE_BEGIN(synchronization_indication)

BF( 4, 0xD, ACT_CHECK, iei_d,"IEI of sync ind")

BF( 1,   0, ACT_CHECK,   nci,"out of range TAV is ignored")

BF( 1,   0, ACT_CHECK,   rot,"observed time diff not included")

BF( 2,   0, ACT_CHECK,    si,"non-synchronized")

IE_END(synchronization_indication)

MSG3_BEGIN(handover_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(handover_command_message_type)

IE(cell_description)

IE(channel_description_GSM)

IE(handover_reference)

IE(power_command)

IE(synchronization_indication)

MSG3_END(handover_command)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_SET_POWER	( 1,DOWNLINK_INPUT_LEVEL_DBM_CELL_B)

ISS_DELAY	( 5000) /* wait until the MS has detected the cells */

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup_19,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command_19,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

ISS_DELAY	( 5000) /* wait another 5 sec */

BS_SET_ERROR (0, 5)       /* T3124 Timeout */

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND        ( 0, handover_command," 1:")

/*

* send by layer 1, but not from the simulation

* BS_CONFIG_CHANNEL   ( 1, FACCH, UNACK, SAPI_0)

*  BS_RACH_AWAIT       ( 1, handover_access," 2: sent once to L1, repeated there")

*/

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, handover_failure_abnormal_release," 3:")

ISS_DELAY	( 5000)

}

History:	12.02.98	VK	Initial.

25.01.99	LE	Testcase updated

## Frequency Redefinition

### MRR280: Frequency redefinition  (26.6.6.1)

Description:	(Ref.: GSM 11.10-1, §26.6.6.1)

Preamble:	MRR000

Script:

{

IE_BEGIN(cell_channel_description_0) /* actually not used (equals to default) */
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0000000,ACT_CHECK,chan_124_thru_097,"")
   BF(32,0x00000020,ACT_CHECK,chan_096_thru_065,"               70")
   BF(32,0x00020000,ACT_CHECK,chan_064_thru_033,"        50")
   BF(32,0x20080000,ACT_CHECK,chan_032_thru_001,"  30    20")
IE_END(cell_channel_description_0)

IE_BEGIN(cell_channel_description_1)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0000000,ACT_CHECK,chan_124_thru_097,"")
   BF(32,0x00000000,ACT_CHECK,chan_096_thru_065,"")
   BF(32,0x0000000F,ACT_CHECK,chan_064_thru_033,"         36 35 34 33")
   BF(32,0x00000000,ACT_CHECK,chan_032_thru_001,"")
IE_END(cell_channel_description_1)

IE_BEGIN(cell_channel_description_2)
   BF( 2,         0,ACT_CHECK,format_id        ,"bit map 0")
   BF( 2,         0,ACT_CHECK,spare            ,"")
   BF(28, 0x0000000,ACT_CHECK,chan_124_thru_097,"")
   BF(32,0x00000000,ACT_CHECK,chan_096_thru_065,"")
   BF(32,0x00000000,ACT_CHECK,chan_064_thru_033,"")
   BF(32,0xF0000000,ACT_CHECK,chan_032_thru_001,"32 31 30 29")
IE_END(cell_channel_description_2)

IE_BEGIN(channel_description_0)

BF( 5,M5(0,1,0,0,0),ACT_CHECK,          channel_type,"SDCCH/8 offset 0")

BF( 3,            0,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            0,ACT_CHECK,training_sequence_code,SILENT)

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            1,ACT_CHECK,                  maio,"")

BF( 6,            0,ACT_CHECK,                   hsn,"")

IE_END(channel_description_0)

IE_BEGIN(channel_description_1)

BF( 5,M5(0,1,0,0,0),ACT_CHECK,          channel_type,"SDCCH/8 offset 0")

BF( 3,            0,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            0,ACT_CHECK,training_sequence_code,SILENT)

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            1,ACT_CHECK,                  maio,"")

BF( 6,            0,ACT_CHECK,                   hsn,"")

IE_END(channel_description_1)

IE_BEGIN(channel_description_2)

BF( 5,M5(0,1,0,0,0),ACT_CHECK,          channel_type,"SDCCH/8 offset 0")

BF( 3,            0,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            0,ACT_CHECK,training_sequence_code,SILENT)

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            0,ACT_CHECK,                  maio,"")

BF( 6,            0,ACT_CHECK,                   hsn,"")

IE_END(channel_description_2)

IE_BEGIN(ia_rest_octets_0)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT) /*  9 */

IE_END(ia_rest_octets_0)

IE_BEGIN(mobile_allocation_0)

BF(8,1,ACT_CHECK,length,"ARFCN [70,50,30,20] need 4 bits - one octet")

BF(8,M8(0,0,0,0,1,1,1,0),ACT_CHECK,ANONYMOUS,"70, 50, and 30 selected")

IE_END(mobile_allocation_0)

IE_BEGIN(mobile_allocation_1)

BF(8,1,ACT_CHECK,length,"ARFCN [36,35,34,33] need 4 bits - one octet")

BF(8,M8(0,0,0,0,1,1,0,0),ACT_CHECK,ANONYMOUS,"36 and 35 selected")

IE_END(mobile_allocation_1)

IE_BEGIN(mobile_allocation_2)

BF(8,1,ACT_CHECK,length,"ARFCN [32,31,30,29] need 4 bits - one octet")

BF(8,M8(0,0,0,0,0,0,1,1),ACT_CHECK,ANONYMOUS,"30 and 29 selected")

IE_END(mobile_allocation_2)

IE_BEGIN(starting_time)

BF(5,19,ACT_CHECK,_t1,SILENT)

BF(6,39,ACT_CHECK, t3,SILENT)

BF(5,12,ACT_CHECK, t2,SILENT)

IE_END(starting_time)

MSG3_BEGIN(immediate_assignment)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_0)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation_0)

IE(ia_rest_octets_0)

MSG3_END(immediate_assignment)

MSG3_BEGIN(frequency_redefinition_1)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(frequency_redefinition_message_type)

IE(channel_description_0)

IE(mobile_allocation_1)

IE(starting_time)

IE(iei_62)

IE(cell_channel_description_1)

MSG3_END(frequency_redefinition_1)

MSG3_BEGIN(frequency_redefinition_2)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(frequency_redefinition_message_type)

IE(channel_description_0)

IE(mobile_allocation_2)

IE(starting_time)

IE(iei_62)

IE(cell_channel_description_2)

MSG3_END(frequency_redefinition_2)

SET_TIMEOUT	( 30000)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 1:")

BS_RACH_AWAIT       ( 0, channel_request,"  2:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS(0,0)

BS_MSG3_SEND        ( 0, immediate_assignment,"  3: SDCCH test: Channel type = SDCCH/8")

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," 4:")

ISS_DELAY( 5000)

BS_MSG3_SEND        ( 0, frequency_redefinition_1," 6:")

ISS_DELAY( 5000)

BS_MSG3_SEND        ( 0, frequency_redefinition_2," 8:")

ISS_DELAY( 5000)

BS_MSG3_SEND        ( 0, channel_release," 10:")

ISS_DELAY( 5000)

}

History:	12.02.98	VK	Initial.

25.01.99	LE	Testcase updated

## Channel Mode Modify

### MRR290: Channel Mode Modify / full rate (26.6.7.1)

Description:	(Ref.: GSM 11.10-1, §26.6.7.1)

Preamble:	MRR000

Script:

{

MSG3_BEGIN(channel_mode_modify)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_mode_modify_message_type)

IE(channel_description_GSM)

IE(channel_mode)

MSG3_END(channel_mode_modify)

SET_TIMEOUT	( 30000)

BS_CONFIG_CHANNEL   ( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND        ( 0, paging_request_type_1," 1:")

BS_RACH_AWAIT       ( 0, channel_request,"  2:")

BS_CONFIG_CHANNEL   ( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS(0,0)

BS_MSG3_SEND        ( 0, immediate_assignment_GSM,"  3:")

BS_CONFIG_CHANNEL   ( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT       ( 0, paging_response_imsi," ?:")

BS_MSG3_SEND_BEGIN  ( 0, channel_mode_modify," 4:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,0,0,0,0,1)," speech full rate")

BS_MSG3_SEND_END()

BS_MSG3_AWAIT_BEGIN ( 0, channel_mode_modify_acknowledge," 5:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,0,0,0,0,1)," speech full rate")

BS_MSG3_AWAIT_END()

BS_MSG3_SEND_BEGIN  ( 0, channel_mode_modify," 6:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,0,0,0,1,1)," data, 12 kbit radio")

BS_MSG3_SEND_END()

BS_MSG3_AWAIT_BEGIN ( 0, channel_mode_modify_acknowledge," 7:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,0,0,0,0,1)," speech full rate")

BS_MSG3_AWAIT_END()

BS_MSG3_SEND_BEGIN  ( 0, channel_mode_modify," 8:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,0,1,0,1,1)," data, 6.0 kbit radio")

BS_MSG3_SEND_END()

BS_MSG3_AWAIT_BEGIN ( 0, channel_mode_modify_acknowledge," 9:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,0,0,0,0,1)," speech full rate")

BS_MSG3_AWAIT_END()

BS_MSG3_SEND_BEGIN  ( 0, channel_mode_modify," 10:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,1,0,0,1,1)," data, 3.6 kbit radio")

BS_MSG3_SEND_END()

BS_MSG3_AWAIT_BEGIN ( 0, channel_mode_modify_acknowledge," 11:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,0,0,0,0,1)," speech full rate")

BS_MSG3_AWAIT_END()

BS_MSG3_SEND_BEGIN  ( 0, channel_mode_modify," 12:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,0,0,0,0,0)," signalling only")

BS_MSG3_SEND_END()

BS_MSG3_AWAIT_BEGIN ( 0, channel_mode_modify_acknowledge," 13:")

BF_SET_VAL       ( channel_mode.mode_value, M8(0,0,0,0,0,0,0,0)," signalling only")

BS_MSG3_AWAIT_END()

BS_MSG3_SEND        ( 0, channel_release," 14:")

}

History:	13.02.98	VK	Initial.

25.01.99	LE	Testcase updated

## Cipher Mode Setting

### MRR300: Ciphering mode / start ciphering (26.6.8.1)

Description:	(Ref.: GSM 11.10-1, §26.6.8.1)

Preamble:	None

Script:

{

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

/* the following setup of mcc and mnc do not conform with the GSM 11.10 */

IE_BF_SET_VAL	( location_area_identification, mcc_dig_1, 2, SILENT)

IE_BF_SET_VAL	( location_area_identification, mcc_dig_2, 6, SILENT)

IE_BF_SET_VAL	( location_area_identification, mcc_dig_3, 2, SILENT)

IE_BF_SET_VAL	( location_area_identification, mnc_dig_1, 0, SILENT)

IE_BF_SET_VAL	( location_area_identification, mnc_dig_2, 1, SILENT)

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

AT_SEND ("ATD1234567890123456;\r\n"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"1:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"2:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"3:")

BS_MSG3_SEND         (0,authentication_request,"4:")

BS_MSG3_AWAIT        (0,authentication_response,"5:")

BS_MSG3_SEND         (0,ciphering_mode_command,"6:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"7:")

BS_MSG3_AWAIT        (0,setup,"8:")

BS_MSG3_SEND         (0,authentication_request,"9:")

BS_MSG3_AWAIT        (0,authentication_response,"10:")

BS_MSG3_SEND        ( 0, channel_release," 11:")

}

History:	13.02.98	VK	Initial.

### MRR301: Ciphering mode / no ciphering (26.6.8.2)

Description:	(Ref.: GSM 11.10-1, §26.6.8.2)

Preamble:	None

Script:

{

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

/* the following setup of mcc and mnc do not conform with the GSM 11.10 */

IE_BF_SET_VAL	( location_area_identification, mcc_dig_1, 2, SILENT)

IE_BF_SET_VAL	( location_area_identification, mcc_dig_2, 6, SILENT)

IE_BF_SET_VAL	( location_area_identification, mcc_dig_3, 2, SILENT)

IE_BF_SET_VAL	( location_area_identification, mnc_dig_1, 0, SILENT)

IE_BF_SET_VAL	( location_area_identification, mnc_dig_2, 1, SILENT)

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

AT_SEND ("ATD1234567890123456;\r\n"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"1:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"2:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"3:")

BS_MSG3_SEND         (0,authentication_request,"4:")

BS_MSG3_AWAIT        (0,authentication_response,"5:")

BS_MSG3_SEND_BEGIN   (0,ciphering_mode_command,"6:")

BF_SET_VAL        (start_ciphering,0,"no ciphering")

BS_MSG3_SEND_END()

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"7:")

BS_MSG3_AWAIT        (0,setup,"8:")

BS_MSG3_SEND        ( 0, channel_release," 11:")

}

History:	16.02.98	VK	Initial.

### MRR302: Ciphering mode / no ciphering (26.6.8.3)

Description:	(Ref.: GSM 11.10-1, §26.6.8.3)

Preamble:	MRR301

Script:

{

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F, NECI not set to 1")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

ISS_DELAY	( 10000) /* wait after the release from MRR301 */

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 20000) /* wait until the MS has detected the cell */

AT_SEND ("ATD1234567890123456;\r\n"," Dial");

BS_RACH_AWAIT        (0,channel_request_moc,"1:")

BS_CONFIG_CHANNEL    (0,AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0,0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"2:")

BS_CONFIG_CHANNEL    (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT        (0,cm_service_request,"3:")

BS_MSG3_SEND         (0,ciphering_mode_command,"4:")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"5:")

BS_MSG3_AWAIT        (0,setup,"6:")

BS_MSG3_SEND         (0, channel_release," 7:")

}

History:	16.02.98	VK	Initial.

### MRR303: Ciphering mode setting (26.6.8.4)

Description:	This test is related to 26.6.8.4.

Preamble:	None

Script:

{

IE_BEGIN(kc_3)
   BF( 8,  7,ACT_CHECK, kc_value_0 ,SILENT)
   BF( 8,  6,ACT_CHECK, kc_value_1 ,SILENT)
   BF( 8,  5,ACT_CHECK, kc_value_2 ,SILENT)
   BF( 8,  4,ACT_CHECK, kc_value_3 ,SILENT)
   BF( 8,  3,ACT_CHECK, kc_value_4 ,SILENT)
   BF( 8,  2,ACT_CHECK, kc_value_5 ,SILENT)
   BF( 8,  1,ACT_CHECK, kc_value_6 ,SILENT)
   BF( 8,  0,ACT_CHECK, kc_value_7 ,SILENT)
IE_END(kc_3)

IE_BEGIN(kc_4)
   BF( 8,  8,ACT_CHECK, kc_value_0 ,SILENT)
   BF( 8,  7,ACT_CHECK, kc_value_1 ,SILENT)
   BF( 8,  6,ACT_CHECK, kc_value_2 ,SILENT)
   BF( 8,  5,ACT_CHECK, kc_value_3 ,SILENT)
   BF( 8,  4,ACT_CHECK, kc_value_4 ,SILENT)
   BF( 8,  3,ACT_CHECK, kc_value_5 ,SILENT)
   BF( 8,  2,ACT_CHECK, kc_value_6 ,SILENT)
   BF( 8,  1,ACT_CHECK, kc_value_7 ,SILENT)
IE_END(kc_4)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

BS_SET_SYS_INFO       ( 0, system_information_type_1)

BS_SET_SYS_INFO       ( 0, system_information_type_2)

BS_SET_SYS_INFO       ( 0, system_information_type_3)

BS_SET_SYS_INFO       ( 0, system_information_type_4)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 5")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 6")

/*

* Check Cipher Parameter

* Start Ciphering

* Algorithm A5/1

* KC = KC of CKSN = 3

*/

MS_CHECK_CIPH (1, 0, kc_3);

BS_MSG3_SEND         (0,authentication_request,"Step 8")

BS_MSG3_AWAIT        (0,authentication_response,"Step 9")

BS_MSG3_SEND         (0,handover_command_no_ciph,"Step 10")

/*

* handover access not seen in simulation

*/

BS_MSG3_AWAIT        (0,handover_complete,"Step 16")

BS_MSG3_SEND         (0,assignment_command_ciph,"Step 17")

BS_MSG3_AWAIT        (0,assignment_complete,"Step 19")

/*

* Check Cipher Parameter

* Start Ciphering

* Algorithm A5/1

* KC = KC of CKSN = 3

*/

MS_CHECK_CIPH (1, 0, kc_3);

BS_MSG3_SEND         (0,ciphering_mode_command_no_ciph,"Step 20")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 21")

BS_MSG3_SEND         (0,handover_command_ciph,"Step 23")

/*

* handover access not seen in simulation

*/

BS_MSG3_AWAIT        (0,handover_complete,"Step 29")

/*

* Check Cipher Parameter

* Start Ciphering

* Algorithm A5/1

* KC = KC of CKSN = 4

*/

MS_CHECK_CIPH (1, 0, kc_4);

BS_MSG3_SEND         (0,identity_request,"Step 30")

BS_MSG3_AWAIT        (0,identity_response,"Step 31")

BS_MSG3_SEND         (0,handover_command_ciph,"Step 32")

/*

* handover access not seen in simulation

*/

BS_MSG3_AWAIT        (0,handover_complete,"Step 38")

/*

* Check Cipher Parameter

* Start Ciphering

* Algorithm A5/1

* KC = KC of CKSN = 4

*/

MS_CHECK_CIPH (1, 0, kc_4);

BS_MSG3_SEND         (0,assignment_command_no_ciph,"Step 39")

BS_MSG3_AWAIT        (0,assignment_complete,"Step 41")

BS_MSG3_SEND         (0,ciphering_mode_command_no_ciph,"Step 42")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 43")

BS_MSG3_SEND         (0,authentication_request,"Step 44")

BS_MSG3_AWAIT        (0,authentication_response,"Step 45")

BS_MSG3_SEND         (0,handover_command_ciph,"Step 46")

/*

* handover access not seen in simulation

*/

BS_MSG3_AWAIT        (0,handover_complete,"Step 52")

/*

* Check Cipher Parameter

* Start Ciphering

* Algorithm A5/1

* KC = KC of CKSN = 4

*/

MS_CHECK_CIPH (1, 0, kc_4);

BS_SET_ERROR         ( 0, 3) /* step 2: generate DL_RELEASE_IND on receipt of DEDI_REQ! */

BS_MSG3_SEND         (0,handover_command_no_ciph,"Step 53")

BS_MSG3_AWAIT        (0,handover_failure_abnormal_release,"Step 55")

/*

* Check Cipher Parameter

* Start Ciphering

* Algorithm A5/1

* KC = KC of CKSN = 4

*/

MS_CHECK_CIPH (1, 0, kc_4);

BS_MSG3_SEND         (0,identity_request,"Step 56")

BS_MSG3_AWAIT        (0,identity_response,"Step 57")

BS_SET_ERROR         ( 0, 3) /* step 2: generate DL_RELEASE_IND on receipt of DEDI_REQ! */

BS_MSG3_SEND         (0,assignment_command_no_ciph,"Step 58")

BS_MSG3_AWAIT_BEGIN ( 0, assignment_failure," Step 60")

BF_SET_VAL       ( cause, M8(0,1,1,0,1,1,1,1),"")

BS_MSG3_AWAIT_END()

/*

* Check Cipher Parameter

* Start Ciphering

* Algorithm A5/1

* KC = KC of CKSN = 4

*/

MS_CHECK_CIPH (1, 0, kc_4);

BS_MSG3_SEND         (0,assignment_command_ciph_a2,"Step 61B")

BS_MSG3_AWAIT        (0,assignment_complete,"Step 63B")

/*

* Check Cipher Parameter

* Start Ciphering

* Algorithm A5/2

* KC = KC of CKSN = 4

*/

MS_CHECK_CIPH (1, 1, kc_4);

BS_MSG3_SEND         (0,handover_command_ciph,"Step 64B")

/*

* handover access not seen in simulation

*/

BS_MSG3_AWAIT        (0,handover_complete,"Step 70B")

/*

* Check Cipher Parameter

* Start Ciphering

* Algorithm A5/1

* KC = KC of CKSN = 4

*/

MS_CHECK_CIPH (1, 0, kc_4);

BS_MSG3_SEND         (0,channel_release,"Step 71B")

}

History:	29.04.98	VK	Initial

### MRR304: Ciphering mode / IMEISV request (26.6.8.5)

Description:	(Ref.: GSM 11.10-1, §26.6.8.5)

Preamble:	MRR000

Script:

{

MSG3_BEGIN(ciphering_mode_complete_mob_ident)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

IE(iei_17)

IE(mobile_identity_imeisv)
MSG3_END(ciphering_mode_complete_mob_ident)

IE_BF_SET_VAL ( ciphering_mode_setting, start_ciphering, 0," no ciphering")

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"1:")

BS_RACH_AWAIT        (0,channel_request,SILENT,"2:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"4:")

BS_MSG3_SEND_BEGIN   (0,ciphering_mode_command,"5:")

BF_SET_VAL        (cipher_response,0,"IMEISV shall not be included")

BS_MSG3_SEND_END()

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"6:")

BS_MSG3_SEND_BEGIN   (0,ciphering_mode_command,"7:")

BF_SET_VAL        (cipher_response,1,"IMEISV shall be included")

BS_MSG3_SEND_END()

BS_MSG3_AWAIT        (0,ciphering_mode_complete_mob_ident,"8:")

BS_MSG3_SEND         (0, channel_release," 9:")

}

History:	17.02.98	VK	Initial.

## Classmark Interrogation

### MRR350: Classmark interrogation (GSM 900, Single Band) (26.6.11.2)

Description:	(Ref.: GSM 11.10-1, §26.6.11.2)

Preamble:	None

Script:

{

IE_BEGIN(rach)
   BF(3,M3(0,0,0),ACT_CHECK ,establishment_cause,"location update (NECI=0)")
   BF(5,        0,ACT_SHOW  ,ref                ,"not checked")
IE_END(rach)

MSG3_BEGIN(channel_request)
   IE(rach)
MSG3_END(channel_request)

MSG3_BEGIN(classmark_enquiry)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_enquiry_message_type)

MSG3_END(classmark_enquiry)

MSG3_BEGIN(location_updating_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_request_message_type)
   IE(ciphering_key_sequence_number)

IE(location_updating_type_att)

IE(location_area_identification)

IE(mobile_station_classmark_1_GSM)

IE(mobile_identity_imsi)

MSG3_END(location_updating_request)

MSG3_BEGIN(location_updating_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_accept_message_type)
   IE(location_area_identification)

MSG3_END(location_updating_accept)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

IE_BF_SET_VAL ( control_channel_description, att, 1," IMSI attach-detach flag changed")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

SET_TIMEOUT (20000);

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT        (0,channel_request,SILENT,"4:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"5:")

BS_MSG3_AWAIT        (0,location_updating_request,"6:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND         (0,classmark_enquiry,"7:")

BS_MSG3_AWAIT        (0,classmark_change,"8:")

BS_MSG3_SEND         (0,location_updating_accept,"9:")

BS_MSG3_SEND         (0, channel_release," 10:")

}

History:	17.02.98	VK	Initial.

21.02.98	VK	establishment_cause changed

21.01.99	LE	testcase updated

### MRR351: Classmark interrogation (DCS 1800, Single Band) (26.6.11.2)

Description:	(Ref.: GSM 11.10-1, §26.6.11.2)

Preamble:	None

Script:

{

IE_BEGIN(rach)
   BF(3,M3(0,0,0),ACT_CHECK ,establishment_cause,"location update (NECI=0)")
   BF(5,        0,ACT_SHOW  ,ref                ,"not checked")
IE_END(rach)

MSG3_BEGIN(channel_request)
   IE(rach)
MSG3_END(channel_request)

MSG3_BEGIN(classmark_enquiry)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_enquiry_message_type)

MSG3_END(classmark_enquiry)

MSG3_BEGIN(location_updating_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_request_message_type)
   IE(ciphering_key_sequence_number)

IE(location_updating_type_att)

IE(location_area_identification)

IE(mobile_station_classmark_1_DCS)

IE(mobile_identity_imsi)

MSG3_END(location_updating_request)

MSG3_BEGIN(location_updating_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_accept_message_type)
   IE(location_area_identification)

MSG3_END(location_updating_accept)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

IE_BF_SET_VAL ( control_channel_description, att, 1," IMSI attach-detach flag changed")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_DCS)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

COMMAND	(" PL CONFIG STD=4")

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

SET_TIMEOUT (20000);

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT        (0,channel_request,SILENT,"4:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_DCS,"5:")

BS_MSG3_AWAIT        (0,location_updating_request,"6:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND         (0,classmark_enquiry,"7:")

BS_MSG3_AWAIT        (0,classmark_change_1800,"8:")

BS_MSG3_SEND         (0,location_updating_accept,"9:")

BS_MSG3_SEND         (0, channel_release," 10:")

}

History:	17.02.98	VK	Initial.

21.02.98	VK	establishment_cause changed

21.01.99	LE	testcase updated

### MRR352: Classmark interrogation (GSM 900, Dual Band) (26.6.11.2)

Description:	(Ref.: GSM 11.10-1, §26.6.11.2)

Preamble:	None

Script:

{

IE_BEGIN(rach)
   BF(3,M3(0,0,0),ACT_CHECK ,establishment_cause,"location update (NECI=0)")
   BF(5,        0,ACT_SHOW  ,ref                ,"not checked")
IE_END(rach)

MSG3_BEGIN(channel_request)
   IE(rach)
MSG3_END(channel_request)

MSG3_BEGIN(classmark_enquiry)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_enquiry_message_type)

MSG3_END(classmark_enquiry)

MSG3_BEGIN(location_updating_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_request_message_type)
   IE(ciphering_key_sequence_number)

IE(location_updating_type_att)

IE(location_area_identification)

IE(mobile_station_classmark_1_GSM)

IE(mobile_identity_imsi)

MSG3_END(location_updating_request)

MSG3_BEGIN(location_updating_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_accept_message_type)
   IE(location_area_identification)

MSG3_END(location_updating_accept)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

IE_BF_SET_VAL ( control_channel_description, att, 1," IMSI attach-detach flag changed")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

COMMAND	(" PL CONFIG STD=5")

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

SET_TIMEOUT (20000);

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT        (0,channel_request,SILENT,"4:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"5:")

BS_MSG3_AWAIT        (0,location_updating_request,"6:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND         (0,classmark_enquiry,"7:")

BS_MSG3_AWAIT        (0,classmark_change_GSM_DUAL,"8:")

BS_MSG3_SEND         (0,location_updating_accept,"9:")

BS_MSG3_SEND         (0, channel_release," 10:")

}

History:	17.02.98	VK	Initial.

21.02.98	VK	establishment_cause changed

21.01.99	LE	testcase updated

### MRR353: Classmark interrogation (DCS 1800, Dual Band) (26.6.11.2)

Description:	(Ref.: GSM 11.10-1, §26.6.11.2)

Preamble:	None

Script:

{

IE_BEGIN(rach)
   BF(3,M3(0,0,0),ACT_CHECK ,establishment_cause,"location update (NECI=0)")
   BF(5,        0,ACT_SHOW  ,ref                ,"not checked")
IE_END(rach)

MSG3_BEGIN(channel_request)
   IE(rach)
MSG3_END(channel_request)

MSG3_BEGIN(classmark_enquiry)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_enquiry_message_type)

MSG3_END(classmark_enquiry)

MSG3_BEGIN(location_updating_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_request_message_type)
   IE(ciphering_key_sequence_number)

IE(location_updating_type_att)

IE(location_area_identification)

IE(mobile_station_classmark_1_DCS)

IE(mobile_identity_imsi)

MSG3_END(location_updating_request)

MSG3_BEGIN(location_updating_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_accept_message_type)
   IE(location_area_identification)

MSG3_END(location_updating_accept)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

IE_BF_SET_VAL ( control_channel_description, att, 1," IMSI attach-detach flag changed")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_DCS)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

COMMAND	(" PL CONFIG STD=5")

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

SET_TIMEOUT (20000);

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT        (0,channel_request,SILENT,"4:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_DCS,"5:")

BS_MSG3_AWAIT        (0,location_updating_request,"6:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND         (0,classmark_enquiry,"7:")

BS_MSG3_AWAIT        (0,classmark_change_DCS_DUAL,"8:")

BS_MSG3_SEND         (0,location_updating_accept,"9:")

BS_MSG3_SEND         (0, channel_release," 10:")

}

History:	17.02.98	VK	Initial.

21.02.98	VK	establishment_cause changed

21.01.99	LE	testcase updated

### MRR354: Classmark interrogation (E-GSM, Dual Extended Band) (26.6.11.2)

Description:	(Ref.: GSM 11.10-1, §26.6.11.2)

Preamble:	None

Script:

{

IE_BEGIN(rach)
   BF(3,M3(0,0,0),ACT_CHECK ,establishment_cause,"location update (NECI=0)")
   BF(5,        0,ACT_SHOW  ,ref                ,"not checked")
IE_END(rach)

MSG3_BEGIN(channel_request)
   IE(rach)
MSG3_END(channel_request)

MSG3_BEGIN(classmark_enquiry)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_enquiry_message_type)

MSG3_END(classmark_enquiry)

MSG3_BEGIN(location_updating_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_request_message_type)
   IE(ciphering_key_sequence_number)

IE(location_updating_type_att)

IE(location_area_identification)

IE(mobile_station_classmark_1_GSM)

IE(mobile_identity_imsi)

MSG3_END(location_updating_request)

MSG3_BEGIN(location_updating_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_accept_message_type)
   IE(location_area_identification)

MSG3_END(location_updating_accept)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

IE_BF_SET_VAL ( control_channel_description, att, 1," IMSI attach-detach flag changed")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_EGSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

COMMAND	(" PL CONFIG STD=6")

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

SET_TIMEOUT (20000);

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT        (0,channel_request,SILENT,"4:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_DCS,"5:")

BS_MSG3_AWAIT        (0,location_updating_request,"6:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND         (0,classmark_enquiry,"7:")

BS_MSG3_AWAIT        (0,classmark_change_GSM_DUAL_EXT,"8:")

BS_MSG3_SEND         (0,location_updating_accept,"9:")

BS_MSG3_SEND         (0, channel_release," 10:")

}

History:	19.11.02	MSB	Initial (copy from MRR353).

### MRR355: Classmark interrogation (PCS 1900, Single Band) (26.6.11.2)

Description:	(Ref.: GSM 11.10-1, §26.6.11.2)

Preamble:	None

Script:

{

IE_BEGIN(rach)
   BF(3,M3(0,0,0),ACT_CHECK ,establishment_cause,"location update (NECI=0)")
   BF(5,        0,ACT_SHOW  ,ref                ,"not checked")
IE_END(rach)

MSG3_BEGIN(channel_request)
   IE(rach)
MSG3_END(channel_request)

MSG3_BEGIN(classmark_enquiry)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_enquiry_message_type)

MSG3_END(classmark_enquiry)

MSG3_BEGIN(location_updating_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_request_message_type)
   IE(ciphering_key_sequence_number)

IE(location_updating_type_att)

IE(location_area_identification)

IE(mobile_station_classmark_1_DCS)

IE(mobile_identity_imsi)

MSG3_END(location_updating_request)

MSG3_BEGIN(location_updating_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_accept_message_type)
   IE(location_area_identification)

MSG3_END(location_updating_accept)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

IE_BF_SET_VAL ( control_channel_description, att, 1," IMSI attach-detach flag changed")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_DCS)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

COMMAND	("PL CONFIG STD=3")

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

SET_TIMEOUT (20000);

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT        (0,channel_request,SILENT,"4:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_DCS,"5:")

BS_MSG3_AWAIT        (0,location_updating_request,"6:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND         (0,classmark_enquiry,"7:")

BS_MSG3_AWAIT        (0,classmark_change_1900,"8:")

BS_MSG3_SEND         (0,location_updating_accept,"9:")

BS_MSG3_SEND         (0, channel_release," 10:")

}

History:	19.11.02	MSB	Initial (copy from MRR353).

### MRR356: Classmark interrogation (GSM 850, Single Band) (26.6.11.2)

Description:	(Ref.: GSM 11.10-1, §26.6.11.2)

Preamble:	None

Script:

{

IE_BEGIN(rach)
   BF(3,M3(0,0,0),ACT_CHECK ,establishment_cause,"location update (NECI=0)")
   BF(5,        0,ACT_SHOW  ,ref                ,"not checked")
IE_END(rach)

MSG3_BEGIN(channel_request)
   IE(rach)
MSG3_END(channel_request)

MSG3_BEGIN(classmark_enquiry)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_enquiry_message_type)

MSG3_END(classmark_enquiry)

MSG3_BEGIN(location_updating_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_request_message_type)
   IE(ciphering_key_sequence_number)

IE(location_updating_type_att)

IE(location_area_identification)

IE(mobile_station_classmark_1_GSM)

IE(mobile_identity_imsi)

MSG3_END(location_updating_request)

MSG3_BEGIN(location_updating_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_accept_message_type)
   IE(location_area_identification)

MSG3_END(location_updating_accept)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

IE_BF_SET_VAL ( control_channel_description, att, 1," IMSI attach-detach flag changed")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_850)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

COMMAND	("PL CONFIG STD=7")

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

SET_TIMEOUT (20000);

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT        (0,channel_request,SILENT,"4:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_850,"5:")

BS_MSG3_AWAIT        (0,location_updating_request,"6:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND         (0,classmark_enquiry,"7:")

BS_MSG3_AWAIT        (0,classmark_change_850,"8:")

BS_MSG3_SEND         (0,location_updating_accept,"9:")

BS_MSG3_SEND         (0, channel_release," 10:")

}

History:	19.11.02	MSB	Initial (copy from MRR353).

### MRR357: Classmark interrogation (GSM 850+PCS 1900, DUAL US-Band) (26.6.11.2)

Description:	(Ref.: GSM 11.10-1, §26.6.11.2)

Preamble:	None

Script:

{

IE_BEGIN(rach)
   BF(3,M3(0,0,0),ACT_CHECK ,establishment_cause,"location update (NECI=0)")
   BF(5,        0,ACT_SHOW  ,ref                ,"not checked")
IE_END(rach)

MSG3_BEGIN(channel_request)
   IE(rach)
MSG3_END(channel_request)

MSG3_BEGIN(classmark_enquiry)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(classmark_enquiry_message_type)

MSG3_END(classmark_enquiry)

MSG3_BEGIN(location_updating_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_request_message_type)
   IE(ciphering_key_sequence_number)

IE(location_updating_type_att)

IE(location_area_identification)

IE(mobile_station_classmark_1_DCS)

IE(mobile_identity_imsi)

MSG3_END(location_updating_request)

MSG3_BEGIN(location_updating_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(location_updating_accept_message_type)
   IE(location_area_identification)

MSG3_END(location_updating_accept)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

IE_BF_SET_VAL ( control_channel_description, att, 1," IMSI attach-detach flag changed")

BS_SET_SYS_INFO	( 0, system_information_type_1)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_DCS)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

COMMAND	("PL CONFIG STD=8")

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

SET_TIMEOUT (20000);

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_RACH_AWAIT        (0,channel_request,SILENT,"4:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_DCS,"5:")

BS_MSG3_AWAIT        (0,location_updating_request,"6:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_SEND         (0,classmark_enquiry,"7:")

BS_MSG3_AWAIT        (0,classmark_change_DUAL_US,"8:")

BS_MSG3_SEND         (0,location_updating_accept,"9:")

BS_MSG3_SEND         (0, channel_release," 10:")

}

History:	19.11.02	MSB	Initial (copy from MRR353).

## Channel Release

### MRR401: Channel release / SDCCH (26.6.12.1)

Description:	(Ref.: GSM 11.10-1, §26.6.12.1)

Preamble:	MRR000

Script:

{

REPEAT_BEGIN(COUNT,2)

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"1:")

BS_RACH_AWAIT        (0,channel_request,SILENT,"2:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"3:")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"4:")

BS_MSG3_SEND         (0, channel_release," 5:")

ISS_DELAY( 12000) /* step 7: */

ISS_DELAY(  6000) /* wait another 6 seconds */

REPEAT_END(COUNT)

}

History:	17.02.98	VK	Initial.

### MRR402: TCH Assignment with Hopping for DCS 1800

Description:	A mobile terminated call establishment is started. A channel assignment to TCH with frequency hopping is started in the DCS 1800 frequency range.

Preamble:	MRR353

Script:

ISS_DELAY (10000)

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_DCS,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi_dual,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup_19,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed_1800_DUAL,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command_1800_H,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

History:	22.01.97	VK	Initial

## Test of Starting Time

### MRR410: Imm Ass with starting time / successful / time not elapsed / (26.6.13.9)

Description:	Beside the actual test of Starting Time this test case checks that the IA rest octets are correctly decoded.
(Ref.: GSM 11.10-1, §26.6.13.9)

Preamble:	None

Script:

{

IE_BEGIN(channel_description_26_6_13_9)

BF( 5,M5(0,1,0,0,0),ACT_CHECK,          channel_type,"SDCCH/8")

BF( 3,            2,ACT_CHECK,      time_slot_number,SILENT)

BF( 3,            5,ACT_CHECK,training_sequence_code,SILENT)

BF( 1,            1,ACT_CHECK,               hopping,"yes")

BF( 6,            0,ACT_CHECK,                  maio,SILENT)

BF( 6,            1,ACT_CHECK,                   hsn,SILENT)

IE_END(channel_description_26_6_13_9)

IE_BEGIN(mobile_allocation_ch114)

BF(8,3,ACT_CHECK,length,SILENT)

BF(8,M8(0,0,0,0,0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

BF(8,M8(0,0,0,0,0,0,0,0),ACT_CHECK,ANONYMOUS,SILENT)

BF(8,M8(0,0,0,0,0,0,0,1),ACT_CHECK,ANONYMOUS,SILENT)

IE_END(mobile_allocation_ch114)

IE_BEGIN(starting_time)

BF(5,10,ACT_CHECK,_t1,SILENT)

BF(6,30,ACT_CHECK, t3,SILENT)

BF(5,20,ACT_CHECK, t2,SILENT)

IE_END(starting_time)

IE_BEGIN(ia_rest_octets_26_6_13_9)

BF(2,2,ACT_CHECK,freq_ind,"HL: Frequency Parameter, before time")

BF(6,2,ACT_CHECK,freq_len,"Length of Frequency Parameter, before time")

BF(2,0,ACT_CHECK,ANONYMOUS,"spare bits")

BF(6,47,ACT_CHECK,ANONYMOUS,"MAIO")

BF(8,1,ACT_CHECK,ANONYMOUS,"Mobile Allocation Bitfield") /* selects first cell ARFCN */

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(ia_rest_octets_26_6_13_9)

MSG3_BEGIN(immediate_assignment_26_6_13_9)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_26_6_13_9)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation_ch114)
   IE(iei_7c)

IE(starting_time)

IE(ia_rest_octets_26_6_13_9)

MSG3_END(immediate_assignment_26_6_13_9)

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

BS_SET_SYS_INFO	( 0, system_information_type_1_cell_a_gsm900_26_6_13)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

AT_SEND	("AT+CFUN=1\r\n"," Power On")

AT_SEND	("AT+COPS=0\r\n"," Start Registration")

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"1:")

BS_RACH_AWAIT        (0,channel_request,SILENT,"2:")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_26_6_13_9,"3: Mob Alloc in rest octets")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"4:")

BS_MSG3_SEND         (0, channel_release," 5:")

ISS_DELAY( 6000)

}

History:	26.03.2001	VK	Initial.

## Extended GSM

### MRR450: Immediate Assignment with Frequency Hopping (26.10.2.2 K=2)

Description:	A mobile terminated call is started for E-GSM with frequency hopping. Channel 0 is used in the hopping list.

Preamble:	None

Script:

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

COMMAND	(" PL CONFIG STD=6")

BS_SET_SYS_INFO	( 0, system_information_type_1_E_S1)

BS_SET_SYS_INFO	( 0, system_information_type_2_E_S1)

BS_SET_SYS_INFO	( 0, system_information_type_3_E_S1)

BS_SET_SYS_INFO	( 0, system_information_type_4_E_S1)

BS_SET_SCH	( 0, BSIC_1_3 , RFN)

BS_SET_ARFCN	( 0, ARFCN_002)

BS_SET_POWER	( 0, RXLEV_60)

BS_ON_OFF	( 0, TRUE)

SET_TIMEOUT (20000);

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

COMMAND	(" PL CONFIG STD=6")

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_HOP_SDCCH8,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi_egsm,"Step 4")

History:	26.01.99	LE	Initial

## Additional Testcases

### MRR500: Frequency Hopping in Assignment Command

Description:	A call attempt with frequency hopping during assignment command.

Preamble:	MRR000

Script:

SET_TIMEOUT         ( 30000)

BS_CONFIG_CHANNEL    (0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND         (0,paging_request_type_1,"Step 1")

BS_RACH_AWAIT        (0,channel_request,"Step 2")

BS_CONFIG_CHANNEL    (0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0)

BS_MSG3_SEND         (0,immediate_assignment_GSM,"Step 3")

BS_CONFIG_CHANNEL    (0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,paging_response_imsi,"Step 4")

BS_MSG3_SEND         (0,authentication_request,"Step 5")

BS_MSG3_AWAIT        (0,authentication_response,"Step 6")

BS_MSG3_SEND         (0,ciphering_mode_command,"Step 7")

BS_MSG3_AWAIT        (0,ciphering_mode_complete,"Step 8")

BS_MSG3_SEND         (0,setup_19,"Step 1")

BS_MSG3_AWAIT        (0,call_confirmed,"Step 9")

BS_MSG3_AWAIT        (0,alerting,"Step 10")

ISS_DELAY (10000);

AT_SEND ("ATA\r\n"," Hook off");

BS_MSG3_AWAIT        (0,connect,"Step 11")

BS_MSG3_SEND         (0,assignment_command_300,"Step 12")

BS_CONFIG_CHANNEL    (0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT        (0,assignment_complete,"Step 13")

BS_MSG3_SEND         (0,connect_acknowledge,"Step 14")

History:	22.01.97	VK	Initial

### MRR501: CBCH Problem with Frequency Hopping in Turkey

Description:	The testcase simulates an environment found in Turkey which has problems with CBCH reception if the CBCH channel uses frequency hopping.

Preamble:	None

Script:

ISS_INIT	( 1)

COMMAND ("TAP REDIRECT CLEAR")

COMMAND ("RR REDIRECT CLEAR")

COMMAND ("MM REDIRECT CLEAR")

COMMAND ("MMI REDIRECT CLEAR")

COMMAND ("TAP DUPLICATE CLEAR")

COMMAND ("RR DUPLICATE CLEAR")

COMMAND ("MM DUPLICATE CLEAR")

COMMAND ("MMI DUPLICATE CLEAR")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000100 PCO")

COMMAND ("TAP DUPLICATE TAP 0*010111******** PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP DUPLICATE TAP 10000000********0*00000000000011 PCO")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000100 RR")

COMMAND ("TAP REDIRECT TAP 0*010111******** RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 10000000********0*00000000000011 RR")

COMMAND ("TAP REDIRECT TAP 0*010101******** MMI")

COMMAND ("RR DUPLICATE ALL PCO") /* duplicate RR primitives */

COMMAND ("DL REDIRECT RR NULL") /* duplicate RR primitives */

COMMAND ("GRR DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MM DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("LC DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RRLP DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("MMI DUPLICATE RR PCO") /* duplicate RR primitives */

COMMAND ("RR DUPLICATE PL TAP")

COMMAND ("RR DUPLICATE DL TAP")

COMMAND ("MM REDIRECT DL TAP")

COMMAND ("MMI REDIRECT MMI TAP")

BS_SET_SYS_INFO	( 0, system_information_type_1_turkey)

BS_SET_SYS_INFO	( 0, system_information_type_2)

BS_SET_SYS_INFO	( 0, system_information_type_3)

BS_SET_SYS_INFO	( 0, system_information_type_4_turkey)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6)

BS_SET_SCH	( 0,BSIC , RFN)

BS_SET_ARFCN	( 0,ARFCN_BCCH_GSM)

BS_SET_POWER	( 0,DOWNLINK_INPUT_LEVEL_DBM)

BS_ON_OFF	( 0,TRUE)

AT_SEND ("AT+CFUN=1\r\n"," Power On");

AT_SEND ("AT+COPS=0\r\n"," Start Registration");

ISS_DELAY	( 5000) /* wait until the MS has detected the cell */

History:	22.12.00	LE	Initial

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |