Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document – Confidential

GSM Protocol Stack

Multilayer Test Specification

Short Message Services

(Edit via File/Properties/Summary!)

| Document Number: | 6147.411.97.100 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1998-Jan-26 |
| Last changed: | 2003-May-21 by XGUTTEFE |
| File Name: | msms.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1998-Jan-26 | LE, MS |  | 0.1 |  | 1 |
| 2003-May-21 | XGUTTEFE |  | 0.2 | Draft |  |

Notes:

- Initial version

Table of Contents

1.1	References	4

1.2	Abbreviations	7

1.3	Terms	9

2	Overview	10

3	Parameters	12

4	TEST CASES	56

4.1	Preambles	56

4.1.1	MSMS000: Initialization (One cell)	56

4.2	Short message service point to point	57

4.2.1	MSMS001: SMS mobile terminated (34.2.1)	57

4.2.2	MSMS002: SMS mobile originated (34.2.2)	64

4.2.3	MSMS003: Test of memory full condition and memory available notification (34.2.3)	68

4.2.4	MSMS004: Test of the status report capabilities and of SMS-COMMAND (34.2.4)	73

4.3	Test of message class 0 to 3	75

4.3.1	MSMS005: Short message class 0 (34.2.5.1)	75

4.3.2	MSMS006: Test of class 1 short messages (34.2.5.2)	76

4.3.3	MSMS007: Test of class 2 short messages (34.2.5.3)	77

4.4	Short message service point to point	80

4.4.1	MSMS008: Test of the replace mechanism for SM type 1-7 (34.2.7)	80

4.4.2	MSMS009: Test of the reply path scheme (34.2.8)	82

4.4.3	MSMS010: Cell Broadcast (34.3)	84

4.5	Additional SMS Testcases	85

4.5.1	MSMS500: Write and Read of a mobile originated SMS	85

Appendices	86

A.	Acronyms	86

B.	Glossary	86

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH

[50]	Service Access Point PH
	6147.112.97.100; Condat GmbH

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH

[58]	Users Guide
	6147.300.96.100; Condat GmbH

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

Fehler! Es ist nicht möglich, durch die Bearbeitung von Feldfunktionen Objekte zu erstellen.

Figure 1: Mobile-station protocol architecture

This document describes the tests for the whole protocol stack according to GSM 11.10 chapter 26.8.

# Parameters

#include "P_DL.H"

#define REST_OCTET 0x2b

#define  ARFCN_BCCH	 122

#define  BCC	0x6

/*------------------------------------------------------------------*\

| GSM 11.10

| 10 Generic call set up procedure

| 10.1 Generic call setup-up procedure for mobile terminating speech calls

| 10.1.2 Definition of system information messages

\*------------------------------------------------------------------*/

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

/*---------------------------------------------------------------*\

| BCCH Frequency list:

| Indicates seven surrounding cells on any ARFCN of the supported

| band, excluding ARFCNs in or immediately adjacent to those

| specified in section 6.2 (GSM 11.10).

| From GSM 11.10, section6.2 the following ARFCN are given :

|     10, 14, 17, 18, 22, 24, 26, 30, 31, 34, 38, 42, 45, 46, 50,

|     52, 54, 58, 59, 62, 66, 70, 73, 74, 78, 80, 82, 86, 87, 90,

|     94, 98, 101, 102, 106, 108, 110, 114

| The following 7 cells are chosen :

|              121,117,       76,      48,   12,7,1

| Thus BA is : 01100000  00000800 00008000 00000841

\*---------------------------------------------------------------*/

IE_BEGIN(bcch_frequency_list)

BF(32,0x01100000,ACT_CHECK,ANONYMOUS,"bit 128 thru  97")

BF(32,0x00000800,ACT_CHECK,ANONYMOUS,"bit  96 thru  65")

BF(32,0x00008000,ACT_CHECK,ANONYMOUS,"bit  64 thru  33")

BF(32,0x00000841,ACT_CHECK,ANONYMOUS,"bit  32 thru   1")

IE_END(bcch_frequency_list)

IE_BEGIN(cell_channel_description)

BF(32,0x00000000,ACT_CHECK,ANONYMOUS,"Includes the       ")

BF(32,0x00000000,ACT_CHECK,ANONYMOUS,"hopping sequence   ")

BF(32,0x00000000,ACT_CHECK,ANONYMOUS,"ARFCNs, if hopping ")

BF(32,0x00000000,ACT_CHECK,ANONYMOUS,"is used.           ")

IE_END(cell_channel_description)

IE_BEGIN(cell_identity)

BF(16,0x0001,ACT_CHECK,ANONYMOUS,"CI VALUE 0001 hex (not relevant)")

IE_END(cell_identity)

IE_BEGIN(channel_description)

BF( 5,M5(0,0,1,0,1),ACT_NOP,          channel_type,"SDCCH/SACCH 4(1) ")

BF( 3,            0,ACT_NOP,      time_slot_number,"zero             ")

BF( 3,          BCC,ACT_NOP,training_sequence_code,"same as BCCH     ")

BF( 1,            0,ACT_NOP,               hopping,"No               ")

BF( 2,            0,ACT_NOP,                 spare,SILENT             )

BF(10,   ARFCN_BCCH,ACT_NOP,                 arfcn,"ARFCN of the BCCH")

IE_END(channel_description)

IE_BEGIN(mobile_allocation)

BF(8,0,ACT_NOP,length,"length 0 due to hopping disabled")

IE_END(mobile_allocation)

IE_BEGIN(cell_options)

BF(1,0,ACT_CHECK,ANONYMOUS            ,"spare")

BF(1,0,ACT_CHECK,pwrc                        ,"power control not set")

BF(2,2,ACT_CHECK,dtx                          ,"MS must not use DTX")

BF(4,1,ACT_CHECK,radio_link_time_out ,"8")

IE_END(cell_options)

IE_BEGIN(cell_selection_parameter)

BF(3, 0,ACT_CHECK,cell_reselect_hysteresis,"0 dB")

BF(5, 0,ACT_CHECK,ms_txpwr_max_cch        ,"Max. output power of MS")

BF(1, 0,ACT_CHECK,acs                               ,"no additional cell params")

BF(1, 0,ACT_CHECK,neci                             ,"New estab. cause not supp.")

BF(6,-90+111,ACT_NOP,rxlev_access_min    ,"-90 dBm")

IE_END(cell_selection_parameter)

IE_BEGIN(control_channel_description)

BF(1,0,ACT_CHECK,ANONYMOUS    ,"spare")

BF(1,0,ACT_CHECK,att                  ,"MS shall not apply (not relevant)")

BF(3,0,ACT_CHECK,bs_ag_blks_res,"0 blocks reserved (not relevant)")

BF(3,1,ACT_CHECK,ccch_conf        ,"Combined CCCH/SDCCH (not relevant)")

BF(5,0,ACT_CHECK,ANONYMOUS    ,"spare")

BF(3,3,ACT_CHECK,bs_pa_mfrms   ,"5 multiframes (not relevant)")

BF(8,0,ACT_CHECK,t3212              ,"Infinite")

IE_END(control_channel_description)

IE_BEGIN(l2_pseudo_length_12)

BF(6,12,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_12)

IE_BEGIN(l2_pseudo_length_18)

BF(6,18,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_18)

IE_BEGIN(l2_pseudo_length_21)

BF(6,21,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_21)

IE_BEGIN(l2_pseudo_length_22)

BF(6,22,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 1,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_22)

#define  MCC   0x262    /* 262  decimal (not relevant)   */

#define  MNC   1        /* 01   decimal (not relevant)   */

#define  LAC   0x0001   /* 0001 hex (not relevant)       */

IE_BEGIN(location_area_identification)

BF( 4,6     ,ACT_CHECK,mcc_dig_2,"digit 2 of mobile country code")

BF( 4,2     ,ACT_CHECK,mcc_dig_1,"digit 1 of mobile country code")

BF( 4,   0xF,ACT_CHECK,ANONYMOUS   ,"end of MCC                    ")

BF( 4,2     ,ACT_CHECK,mcc_dig_3,"digit 3 of mobile country code")

BF( 4,1     ,ACT_CHECK,mnc_dig_2,"digit 2 of mobile network code")

BF( 4,0     ,ACT_CHECK,mnc_dig_1,"digit 1 of mobile network code")

BF(16,   LAC,ACT_CHECK,lac      ,"Location area code            ")

IE_END(location_area_identification)

IE_BEGIN(ncc_permitted)

BF(8,0xFF,ACT_CHECK,ncc_permit,"e.g. all NCCs permitted")

IE_END(ncc_permitted)

IE_BEGIN(rach_control_parameter)

BF( 2,0,ACT_CHECK,max_retrans,"Any Value  ")

BF( 4,0,ACT_CHECK,tx_integer,"Any Value  ")

BF( 1,0,ACT_CHECK,cell_bar_access            ,"Not barred ")

BF( 1,1,ACT_CHECK,call_re_establishment      ,"Not Allowed")

BF( 5,0,ACT_CHECK,access_control_class_15_11 ,"None Barred")

BF( 1,0,ACT_CHECK,emergency_call             ,"Allowed    ")

BF(10,0,ACT_CHECK,access_control_class_09_00 ,"None Barred")

IE_END(rach_control_parameter)

IE_BEGIN(rr_management_protocol_discriminator)

BF(4, 6,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(rr_management_protocol_discriminator)

IE_BEGIN(skip_indicator)

BF(4, 0,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(skip_indicator)

IE_BEGIN(si_1_rest_octets)

BF(8,REST_OCTET,ACT_CHECK,ANONYMOUS,"Spare Octets")

IE_END(si_1_rest_octets)

IE_BEGIN(si_3_rest_octets) /* optionally contains cell (re)select params */

BF(1, 0,                          ACT_CHECK,p1,"C2 parameters not present")

BF(7, REST_OCTET & 0x7F, ACT_CHECK,ANONYMOUS,SILENT)

BF(8, REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT)

BF(8, REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT)

BF(8, REST_OCTET       ,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(si_3_rest_octets)

IE_BEGIN(si_4_rest_octets) /* optionally contains cell (re)select params */

BF(1, 0,                         ACT_CHECK, p1             ,"C2 parameters not present")

BF(7,REST_OCTET & 0x7F,ACT_CHECK,ANONYMOUS,SILENT)

BF(8,REST_OCTET,           ACT_CHECK,ANONYMOUS,SILENT)

BF(8,REST_OCTET,           ACT_CHECK,ANONYMOUS,SILENT)

BF(8,REST_OCTET,           ACT_CHECK,ANONYMOUS,SILENT)

BF(8,REST_OCTET,           ACT_NOP,ANONYMOUS,SILENT)

BF(8,REST_OCTET,           ACT_NOP,ANONYMOUS,SILENT)

BF(8,REST_OCTET,           ACT_NOP,ANONYMOUS,SILENT)

BF(8,REST_OCTET,           ACT_NOP,ANONYMOUS,SILENT)

BF(8,REST_OCTET,           ACT_NOP,ANONYMOUS,SILENT)

BF(8,REST_OCTET,           ACT_NOP,ANONYMOUS,SILENT)

IE_END(si_4_rest_octets)

IE_BEGIN(iei_64)

BF(8,0x64,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_64)

IE_BEGIN(iei_72)

BF(8,0x72,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_72)

IE_BEGIN(cp_ack_message_type)

BF(1, 0, ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0, ACT_NOP,ANONYMOUS,SILENT)

BF(6, 4, ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cp_ack_message_type)

IE_BEGIN(cp_data_message_type)

BF(1, 0, ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0, ACT_NOP,ANONYMOUS,SILENT)

BF(6, 1, ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cp_data_message_type)

IE_BEGIN(cp_error_message_type)

BF(1, 0, ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0, ACT_NOP,ANONYMOUS,SILENT)

BF(6, 32, ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cp_error_message_type)

IE_BEGIN(system_information_type_1_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x19,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_1_message_type)

IE_BEGIN(system_information_type_2_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x1A,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_2_message_type)

IE_BEGIN(system_information_type_3_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x1B,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_3_message_type)

IE_BEGIN(system_information_type_4_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x1C,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_4_message_type)

IE_BEGIN(system_information_type_5_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x1D,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_5_message_type)

IE_BEGIN(system_information_type_6_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x1E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(system_information_type_6_message_type)

IE_BEGIN(paging_request_type_1_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x21,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(paging_request_type_1_message_type)

IE_BEGIN(immediate_assignment_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x3F,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(immediate_assignment_message_type)

IE_BEGIN(paging_response_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x27,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(paging_response_message_type)

IE_BEGIN(ciphering_mode_command_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x35,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(ciphering_mode_command_message_type)

IE_BEGIN(ciphering_mode_complete_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x32,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(ciphering_mode_complete_message_type)

IE_BEGIN(channel_mode_modify_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x10,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(channel_mode_modify_message_type)

IE_BEGIN(channel_mode_modify_acknowledge_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x17,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(channel_mode_modify_acknowledge_message_type)

IE_BEGIN(assignment_command_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x2E,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(assignment_command_message_type)

IE_BEGIN(assignment_complete_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x29,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(assignment_complete_message_type)

IE_BEGIN(cm_service_request_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x24,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cm_service_request_message_type)

IE_BEGIN(cm_service_abort_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x23,ACT_NOP,ANONYMOUS,SILENT)

IE_END(cm_service_abort_message_type)

IE_BEGIN(cm_service_reject_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x22,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cm_service_reject_message_type)

IE_BEGIN(cm_service_accept_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x21,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(cm_service_accept_message_type)

IE_BEGIN(call_proceeding_message_type)

BF(1, 0,     ACT_CHECK,ANONYMOUS,SILENT)

BF(1, 0,     ACT_NOP,ANONYMOUS,SILENT)

BF(6, 0x02,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(call_proceeding_message_type)

/*------------------------------------------------------------------*\

| Messages

\*------------------------------------------------------------------*/

MSG3_BEGIN(system_information_type_1)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_1_message_type)

IE(cell_channel_description)

IE(rach_control_parameter)

IE(si_1_rest_octets)

MSG3_END(system_information_type_1)

MSG3_BEGIN(system_information_type_2)

IE(l2_pseudo_length_22)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_2_message_type)

IE(bcch_frequency_list)

IE(ncc_permitted)

IE(rach_control_parameter)

MSG3_END(system_information_type_2)

MSG3_BEGIN(system_information_type_3)

IE(l2_pseudo_length_18)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_3_message_type)

IE(cell_identity)

IE(location_area_identification)

IE(control_channel_description)

IE(cell_options)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_3_rest_octets)

MSG3_END(system_information_type_3)

MSG3_BEGIN(system_information_type_4)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4)

MSG3_BEGIN(system_information_type_4_CB)

IE(l2_pseudo_length_12)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_4_message_type)

IE(location_area_identification)

IE(cell_selection_parameter)

IE(rach_control_parameter)
   IE(iei_64)
   IE(channel_description)
   IE(iei_72)
   IE(mobile_allocation)

IE(si_4_rest_octets)

MSG3_END(system_information_type_4_CB)

MSG3_BEGIN(system_information_type_5)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_5_message_type)

IE(bcch_frequency_list)

MSG3_END(system_information_type_5)

MSG3_BEGIN(system_information_type_6)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(system_information_type_6_message_type)

IE(cell_identity)

IE(location_area_identification)

IE(cell_options)

IE(ncc_permitted)

MSG3_END(system_information_type_6)

/*------------------------------------------------------------------*\

| GSM 11.10

| 10 Generic call set up procedure

| 10.1 Generic call setup-up procedure for mobile terminating speech calls

| 10.1.4 Specific message contents

\*------------------------------------------------------------------*/

/*------------------------------------------------------------------*\

| Information Elements

\*------------------------------------------------------------------*/

#define  NCC	0x5

#define  BSIC	((NCC<<3)|(BCC))

#define  RFN	0

IE_BEGIN(authentication_parameter_rand)

BF(32,0x80000000,ACT_NOP,rand_127_096,SILENT)

BF(32,0x00000012,ACT_NOP,rand_095_064,SILENT)

BF(32,0x34000000,ACT_NOP,rand_063_032,SILENT)

BF(32,0x0000000F,ACT_NOP,rand_031_000,SILENT)

IE_END(authentication_parameter_rand)

IE_BEGIN(authentication_parameter_sres)

BF(32,0x0000000F,ACT_NOP,sres_031_000,SILENT)

IE_END(authentication_parameter_sres)

IE_BEGIN(bearer_capability)

BF(8,1,ACT_CHECK,   length,                   SILENT)

BF(1,1,ACT_CHECK,   ext3,                     SILENT)

BF(2,1,ACT_CHECK,   radio_channel_requirement,SILENT)

BF(1,0,ACT_CHECK,   coding_standard,          SILENT)

BF(1,0,ACT_CHECK,   transfer_mode,            SILENT)

BF(3,0,ACT_CHECK,   info_transfer_capability, SILENT)

IE_END(bearer_capability)

IE_BEGIN(bearer_capability_data)

BF(8,7,ACT_CHECK,   length,                   SILENT)

BF(1,1,ACT_CHECK,   ext3,                     SILENT)

BF(2,1,ACT_CHECK,   radio_channel_requirement,SILENT)

BF(1,0,ACT_CHECK,   coding_standard,          SILENT)

BF(1,0,ACT_CHECK,   transfer_mode,            SILENT)

BF(3,2,ACT_CHECK,   info_transfer_capability, SILENT)

BF(1,1,ACT_CHECK,   ext4,                     SILENT)

BF(1,1,ACT_CHECK,   compression,              SILENT)

BF(2,0,ACT_CHECK,   structure,                SILENT)

BF(1,1,ACT_CHECK,   duplex_mode,              SILENT)

BF(1,0,ACT_CHECK,   configuration,            SILENT)

BF(1,0,ACT_CHECK,   NIRR,                     SILENT)

BF(1,0,ACT_CHECK,   establishment,            SILENT)

BF(1,1,ACT_CHECK,   ext5,                     SILENT)

BF(2,1,ACT_CHECK,   access_identifier,        SILENT)

BF(2,0,ACT_CHECK,   rate_adaption,            SILENT)

BF(3,1,ACT_CHECK,   access_protocol,          SILENT)

BF(1,0,ACT_CHECK,   ext6,                     SILENT)

BF(2,1,ACT_CHECK,   layer_1,                  SILENT)

BF(4,0,ACT_CHECK,   default_layer_1,          SILENT)

BF(1,1,ACT_CHECK,   sync_async,               SILENT)

BF(1,0,ACT_CHECK,   ext6a,                    SILENT)

BF(1,0,ACT_CHECK,   stop_bits,                SILENT)

BF(1,0,ACT_CHECK,   negotiation,              SILENT)

BF(1,1,ACT_CHECK,   data_bits,                SILENT)

BF(4,3,ACT_CHECK,   user_rate,                SILENT)

BF(1,0,ACT_CHECK,   ext6b,                    SILENT)

BF(2,3,ACT_CHECK,   intermediate_rate,        SILENT)

BF(1,0,ACT_CHECK,   NIC_TX,                   SILENT)

BF(1,0,ACT_CHECK,   NIC_RX,                   SILENT)

BF(3,3,ACT_CHECK,   parity,                   SILENT)

BF(1,1,ACT_CHECK,   ext6c,                    SILENT)

BF(2,1,ACT_CHECK,   connection_element,       SILENT)

BF(5,3,ACT_CHECK,   modem_type,               SILENT)

IE_END(bearer_capability_data)

IE_BEGIN(call_control_protocol_discriminator)

BF(4, 3,ACT_NOP,ANONYMOUS,SILENT)

IE_END(call_control_protocol_discriminator)

IE_BEGIN(sms_protocol_discriminator)

BF(4, 9,ACT_NOP,ANONYMOUS,SILENT)

IE_END(sms_protocol_discriminator)

IE_BEGIN(called_party_bcd_number)

BF( 8,7,ACT_NOP,length                     ,"length of IE")

BF( 1,1,ACT_NOP,ext                        ,"Extension Bit")

BF( 3,0,ACT_NOP,type_of_number             ,"Unknown")

BF( 4,1,ACT_NOP,numbering_plan             ,"ISDN/telephony")

BF( 4,3,ACT_NOP,digit_2                    ,"Digit 2")

BF( 4,0,ACT_NOP,digit_1                    ,"Digit 1")

BF( 4,3,ACT_NOP,digit_4                    ,"Digit 4")

BF( 4,0,ACT_NOP,digit_3                    ,"Digit 3")

BF( 4,0,ACT_NOP,digit_6                    ,"Digit 6")

BF( 4,9,ACT_NOP,digit_5                    ,"Digit 5")

BF( 4,4,ACT_NOP,digit_8                    ,"Digit 8")

BF( 4,9,ACT_NOP,digit_7                    ,"Digit 7")

BF( 4,1,ACT_NOP,digit_10                   ,"Digit 10")

BF( 4,1,ACT_NOP,digit_9                   ,"Digit 9")

BF( 4,7,ACT_NOP,digit_12                  ,"Digit 12")

BF( 4,0xF,ACT_NOP,digit_11                ,"Digit 11")

IE_END(called_party_bcd_number)

IE_BEGIN(channels_needed_for_mobiles_1_and_2)

BF(2,0,ACT_NOP,second_channel,"spare, any channel")

BF(2,0,ACT_NOP, first_channel,"spare, any channel")

IE_END(channels_needed_for_mobiles_1_and_2)

IE_BEGIN(cc_capabilities)

BF(8,1,ACT_CHECK, length,       SILENT)

BF(8,1,ACT_CHECK, dtmf_support, SILENT)

IE_END(cc_capabilities)

IE_BEGIN(channel_description_tch)

BF( 5,M5(0,0,0,0,1),ACT_CHECK,       channel_type,"TCH Full Rate")

BF( 3,            0,ACT_CHECK,       time_slot_number,"zero     ")

BF( 3,          BCC,ACT_CHECK,       tsc     ,"same as BCCH     ")

BF( 1,            0,ACT_CHECK,       hopping,"No                ")

BF( 2,            0,ACT_CHECK,       spare,SILENT                )

BF(10,   ARFCN_BCCH,ACT_CHECK,       arfcn,"ARFCN of the BCCH   ")

IE_END(channel_description_tch)

IE_BEGIN(channel_mode_speech)

BF( 8,            1,ACT_CHECK,      mode,"Speech full rate")

IE_END(channel_mode_speech)

IE_BEGIN(cm_service_type_moc)

BF(4,M4(0,0,0,1),ACT_NOP,service_type,"mobile originated call")

IE_END(cm_service_type_moc)

IE_BEGIN(ciphering_key_sequence_number)

BF(1,        0,ACT_NOP,       spare,SILENT)

BF(3,M3(1,1,1),ACT_NOP,key_sequence,"no key is available (MS->BS)")

IE_END(ciphering_key_sequence_number)

IE_BEGIN(ciphering_key_sequence_number_2)

BF(1,        0,ACT_NOP,       spare,SILENT)

BF(3,M3(1,0,1),ACT_NOP,key_sequence,"sent BS->MS")

IE_END(ciphering_key_sequence_number_2)

IE_BEGIN(ciphering_mode_setting)

BF(3,M3(0,0,0),ACT_NOP,algorithm_identifier,"A5/1           ")

BF(1,        1,ACT_NOP,     start_ciphering,"Start ciphering")

IE_END(ciphering_mode_setting)

IE_BEGIN(cipher_response)

BF(3,0,ACT_NOP,          spare,SILENT                        )

BF(1,0,ACT_NOP,cipher_response,"IMEISV shall not be included")

IE_END(cipher_response)

IE_BEGIN(description_of_the_first_channel_after_time)

BF( 5,M5(0,0,0,0,1),ACT_NOP,          channel_type,"TCH ")

BF( 3,            3,ACT_NOP,      time_slot_number,"three            ")

BF( 3,          BCC,ACT_NOP,training_sequence_code,"same as BCCH     ")

BF( 1,            0,ACT_NOP,               hopping,"No               ")

BF( 2,            0,ACT_NOP,                 spare,SILENT             )

BF(10,   ARFCN_BCCH,ACT_NOP,                 arfcn,"ARFCN of the BCCH")

IE_END(description_of_the_first_channel_after_time)

IE_BEGIN(ia_rest_octets)   /* maximum length (11), no hop, no start time */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /* 10 */

IE_END(ia_rest_octets)

IE_BEGIN(iei_08)

BF(8,0x08,ACT_NOP,ANONYMOUS,SILENT)

IE_END(iei_08)

IE_BEGIN(iei_1E)

BF(8,0x1E,ACT_NOP,ANONYMOUS,SILENT)

IE_END(iei_1E)

IE_BEGIN(iei_63)

BF(8,0x63,ACT_NOP,ANONYMOUS,SILENT)

IE_END(iei_63)

IE_BEGIN(iei_5E)

BF(8,0x5E,ACT_NOP,ANONYMOUS,SILENT)

IE_END(iei_5E)

IE_BEGIN(iei_15)

BF(8,0x15,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_15)

IE_BEGIN(iei_04)

BF(8,0x04,ACT_CHECK,ANONYMOUS,SILENT)

IE_END(iei_04)

IE_BEGIN(iei_34)

BF(8,0x34,ACT_NOP,ANONYMOUS,SILENT)

IE_END(iei_34)

IE_BEGIN(iei_2C)

BF(8,0x2C,ACT_NOP,ANONYMOUS,SILENT)

IE_END(iei_2C)

IE_BEGIN(keypad_facility_0)

BF(8, 0x30,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_0)

IE_BEGIN(keypad_facility_1)

BF(8, 0x31,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_1)

IE_BEGIN(keypad_facility_2)

BF(8, 0x32,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_2)

IE_BEGIN(keypad_facility_3)

BF(8, 0x33,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_3)

IE_BEGIN(keypad_facility_4)

BF(8, 0x34,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_4)

IE_BEGIN(keypad_facility_5)

BF(8, 0x35,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_5)

IE_BEGIN(keypad_facility_6)

BF(8, 0x36,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_6)

IE_BEGIN(keypad_facility_7)

BF(8, 0x37,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_7)

IE_BEGIN(keypad_facility_8)

BF(8, 0x38,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_8)

IE_BEGIN(keypad_facility_9)

BF(8, 0x39,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_9)

IE_BEGIN(keypad_facility_A)

BF(8, 0x41,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_A)

IE_BEGIN(keypad_facility_B)

BF(8, 0x42,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_B)

IE_BEGIN(keypad_facility_C)

BF(8, 0x43,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_C)

IE_BEGIN(keypad_facility_D)

BF(8, 0x44,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_D)

IE_BEGIN(keypad_facility_star)

BF(8, 0x2A,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_star)

IE_BEGIN(keypad_facility_hash)

BF(8, 0x23,ACT_NOP,ANONYMOUS,SILENT)

IE_END(keypad_facility_hash)

IE_BEGIN(l2_pseudo_length_11)  /* pag req type 1 with TMSI (one mobile)*/

BF(8, 0,ACT_NOP,ANONYMOUS,SILENT)

IE_END(l2_pseudo_length_11)

IE_BEGIN(mobile_identity)             /* has 8 octets */

BF(8,          7,ACT_CHECK,  length,"seven octets to come")

BF(4,          2,ACT_CHECK, digit_1,SILENT)

BF(1,          1,ACT_CHECK,odd_even,SILENT)

BF(3,  M3(0,0,1),ACT_CHECK,    type,"IMSI")

BF(4,          2,ACT_CHECK, digit_3,SILENT)

BF(4,          6,ACT_CHECK, digit_2,SILENT)

BF(4,          1,ACT_CHECK, digit_4,SILENT)

BF(4,          0,ACT_CHECK, digit_3,SILENT)

BF(4,          7,ACT_CHECK, digit_6,SILENT)

BF(4,          4,ACT_CHECK, digit_5,SILENT)

BF(4,          1,ACT_CHECK, digit_8,SILENT)

BF(4,          1,ACT_CHECK, digit_7,SILENT)

BF(4,          9,ACT_CHECK, digit_10,SILENT)

BF(4,          4,ACT_CHECK, digit_9,SILENT)

BF(4,          2,ACT_CHECK, digit_12,SILENT)

BF(4,          1,ACT_CHECK, digit_11,SILENT)

IE_END(mobile_identity)

IE_BEGIN(mobility_management_protocol_discriminator)

BF(4, 5,ACT_NOP,ANONYMOUS,SILENT)

IE_END(mobility_management_protocol_discriminator)

IE_BEGIN(identity_type)

BF(4,1,ACT_CHECK, type_of_identity,"IMSI")

IE_END(identity_type)

IE_BEGIN(mode_of_the_first_channel)

BF(8,1,ACT_NOP,mode,"Speech full rate")

IE_END(mode_of_the_first_channel)

IE_BEGIN(ms_classmark)

BF(8,        3,ACT_NOP, length,SILENT)

BF(1,        0,ACT_NOP, spare,SILENT)

BF(2,  M2(0,1),ACT_NOP, revision_level,"phase 2 MS")

BF(1,        0,ACT_NOP, es_ind,"No 'Contr. Early Classmark Send.'")

BF(1,        0,ACT_NOP, a5_1,"encryption algorithm A5/1 available")

BF(3,M3(0,0,1),ACT_NOP, rf_power_capability,"class 2")

BF(1,        0,ACT_NOP, spare2,SILENT)

BF(1,        0,ACT_NOP, ps_capability,"no pseudo-synch capability")

BF(2,  M2(0,0),ACT_NOP, ss_screening_indicator,"default phase 1")

BF(1,        0,ACT_NOP, sm_capability,"no point to point SMS")

BF(1,        0,ACT_NOP, vbs,"no VBS cap. or no notific. wanted")

BF(1,        0,ACT_NOP, vgcs,"no VGCS cap. or no notific. wanted")

BF(1,        0,ACT_NOP, frequency_capability,"no extention band G1")

BF(1,        0,ACT_NOP, classmark_3,"no add. MS cap. information")

BF(5,        0,ACT_NOP, spare3,SILENT)

BF(1,        0,ACT_NOP, a5_3,"A5/3 not available")

BF(1,        0,ACT_NOP, a5_2,"A5/2 not available")

IE_END(ms_classmark)

IE_BEGIN(p1_rest_octets)

/* pag. req. type1 : 22 - 11 (L2 pseud. len) =  11 bytes */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  0 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  1 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  2 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  3 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  4 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  5 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  6 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  7 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  8 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /*  9 */

BF(8,REST_OCTET,ACT_NOP,ANONYMOUS,SILENT) /* 10 */

IE_END(p1_rest_octets)

IE_BEGIN(page_mode)

BF(2,0,ACT_NOP,spare,"two spare bits  ")

BF(2,0,ACT_NOP,pm   ,"Normal Paging")

IE_END(page_mode)

IE_BEGIN(power_command)

BF(8,10,ACT_NOP,power,SILENT)

IE_END(power_command)

IE_BEGIN(rach)

BF(3,M3(1,0,0),ACT_CHECK ,establishment_cause,"paging ind. any chan'")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach)

IE_BEGIN(rach_moc)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"MOC & TCH/F")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_moc)

IE_BEGIN(rach_111)

BF(3,M3(1,1,1),ACT_CHECK ,establishment_cause,"SDCCH Assignment")

BF(5,M5(1,1,1,1,1),ACT_NOP,   random_reference,"ignore Random Ref.")

IE_END(rach_111)

IE_BEGIN(request_reference)

BF(3,    M3(1,0,0),ACT_NOP,random_access_info,"As in CHAN REQ")

BF(5,M5(1,1,1,1,1),ACT_NOP,  random_reference,SILENT)

BF(5,            0,ACT_NOP,               t1_,SILENT)

BF(6,            0,ACT_NOP,               t3 ,SILENT)

BF(5,            0,ACT_NOP,               t2 ,SILENT)

IE_END(request_reference)

IE_BEGIN(rr_cause)

BF(8,0,ACT_CHECK,rr_cause,"normal event")

IE_END(rr_cause)

IE_BEGIN(reject_cause)

BF(8,0x20,ACT_CHECK,reject_cause,"service not available")

IE_END(reject_cause)

IE_BEGIN(progress_indicator_32)

BF(8,  2,ACT_CHECK, length,"two octets")

BF(1,  1,ACT_CHECK, ext_1,SILENT)

BF(2,  3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,  0,ACT_SHOW,  spare,SILENT)

BF(4,  0,ACT_CHECK, location,"User")

BF(1,  1,ACT_CHECK, ext_2,SILENT)

BF(7, 32,ACT_CHECK, progress_description,"call is end-to-end PLMN/ISDN")

IE_END(progress_indicator_32)

IE_BEGIN(progress_indicator_8)

BF(8,  2,ACT_CHECK, length,"two octets")

BF(1,  1,ACT_CHECK, ext_1,SILENT)

BF(2,  3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,  0,ACT_SHOW,  spare,SILENT)

BF(4,  0,ACT_CHECK, location,"User")

BF(1,  1,ACT_CHECK, ext_2,SILENT)

BF(7,  8,ACT_CHECK, progress_description,"in band tones available")

IE_END(progress_indicator_8)

IE_BEGIN(cause_03)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,          3,ACT_CHECK, cause,"no route to destination")

IE_END(cause_03)

IE_BEGIN(cause_17)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         17,ACT_CHECK, cause,"user busy")

IE_END(cause_17)

IE_BEGIN(cause_31)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         31,ACT_CHECK, cause,"normal, unspecified")

IE_END(cause_31)

IE_BEGIN(cause_81)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         81,ACT_CHECK, cause,"invalid transaction identifier")

IE_END(cause_81)

IE_BEGIN(cause_88)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         88,ACT_CHECK, cause,"incompatible destination")

IE_END(cause_88)

IE_BEGIN(cause_102_303)

BF(8,          5,ACT_CHECK, length,"five octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,        102,ACT_CHECK, cause,"recovery on timer expiry")

BF(8,       0x33,ACT_CHECK, diag_0,"3")

BF(8,       0x30,ACT_CHECK, diag_1,"0")

BF(8,       0x33,ACT_CHECK, diag_2,"3")

IE_END(cause_102_303)

IE_BEGIN(cause_102_310)

BF(8,          5,ACT_CHECK, length,"five octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,        102,ACT_CHECK, cause,"recovery on timer expiry")

BF(8,       0x33,ACT_CHECK, diag_0,"3")

BF(8,       0x31,ACT_CHECK, diag_1,"1")

BF(8,       0x30,ACT_CHECK, diag_2,"0")

IE_END(cause_102_310)

IE_BEGIN(cause_102_313)

BF(8,          5,ACT_CHECK, length,"five octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,        102,ACT_CHECK, cause,"recovery on timer expiry")

BF(8,       0x33,ACT_CHECK, diag_0,"3")

BF(8,       0x31,ACT_CHECK, diag_1,"1")

BF(8,       0x33,ACT_CHECK, diag_2,"3")

IE_END(cause_102_313)

IE_BEGIN(cause_97_20)

BF(8,          3,ACT_CHECK, length,"three octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         97,ACT_CHECK, cause,"message not implemented")

BF(8,       0x20,ACT_CHECK, diag_0,"message type 0x20")

IE_END(cause_97_20)

IE_BEGIN(cause_16)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         16,ACT_CHECK, cause,"normal clearing")

IE_END(cause_16)

IE_BEGIN(cause_30)

BF(8,          2,ACT_CHECK, length,"two octets")

BF(1,          1,ACT_CHECK, ext_1,SILENT)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(1,          0,ACT_SHOW,  spare,SILENT)

BF(4,          0,ACT_CHECK, location,"User")

BF(1,          1,ACT_CHECK, ext_2,SILENT)

BF(7,         30,ACT_CHECK, cause,"response to enquiry")

IE_END(cause_30)

IE_BEGIN(call_state_1)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          1,ACT_CHECK, call_state_value,"U1 Call Initiated")

IE_END(call_state_1)

IE_BEGIN(call_state_3)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          3,ACT_CHECK, call_state_value,"U3 Call Proceeding")

IE_END(call_state_3)

IE_BEGIN(call_state_4)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          4,ACT_CHECK, call_state_value,"U4 Call Delivered")

IE_END(call_state_4)

IE_BEGIN(call_state_7)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          7,ACT_CHECK, call_state_value,"U7 Call Receiving")

IE_END(call_state_7)

IE_BEGIN(call_state_8)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          8,ACT_CHECK, call_state_value,"U8 Connect Request")

IE_END(call_state_8)

IE_BEGIN(call_state_9)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,          9,ACT_CHECK, call_state_value,"U9 Call Confirmed")

IE_END(call_state_9)

IE_BEGIN(call_state_10)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,         10,ACT_CHECK, call_state_value,"U10 Call Active")

IE_END(call_state_10)

IE_BEGIN(call_state_11)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,         11,ACT_CHECK, call_state_value,"U11 Disconnect Request")

IE_END(call_state_11)

IE_BEGIN(call_state_12)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,         12,ACT_CHECK, call_state_value,"U12 Disconnect Ind")

IE_END(call_state_12)

IE_BEGIN(call_state_19)

BF(2,          3,ACT_CHECK, coding_standard, "GSM Standard")

BF(6,         19,ACT_CHECK, call_state_value,"U19 Release Request")

IE_END(call_state_19)

IE_BEGIN(signal_call_waiting)

BF(8,M8(0,0,0,0,0,1,1,1),ACT_NOP,signal_value,"(Any non-res. value)")

IE_END(signal_call_waiting)

IE_BEGIN(spare_half_octet)

BF(4, 0,ACT_NOP,ANONYMOUS,SILENT)

IE_END(spare_half_octet)

IE_BEGIN(timing_advance)

BF(2,0,ACT_NOP,         spare,SILENT)

BF(6,0,ACT_NOP,timing_advance,"0"   )

IE_END(timing_advance)

IE_BEGIN(transaction_identifier_source)

BF(4,M4(0,0,0,0),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source)

IE_BEGIN(transaction_identifier_source_1)

BF(4,M4(0,0,0,1),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_1)

IE_BEGIN(transaction_identifier_source_2)

BF(4,M4(0,0,1,0),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_2)

IE_BEGIN(transaction_identifier_source_3)

BF(4,M4(0,0,1,1),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_3)

IE_BEGIN(transaction_identifier_source_4)

BF(4,M4(0,1,0,0),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_4)

IE_BEGIN(transaction_identifier_source_5)

BF(4,M4(0,1,0,1),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_5)

IE_BEGIN(transaction_identifier_source_6)

BF(4,M4(0,1,1,0),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_source_6)

IE_BEGIN(transaction_identifier_dest)

BF(4,M4(1,0,0,0),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest)

IE_BEGIN(transaction_identifier_dest_1)

BF(4,M4(1,0,0,1),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_1)

IE_BEGIN(transaction_identifier_dest_2)

BF(4,M4(1,0,1,0),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_2)

IE_BEGIN(transaction_identifier_dest_3)

BF(4,M4(1,0,1,1),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_3)

IE_BEGIN(transaction_identifier_dest_4)

BF(4,M4(1,1,0,0),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_4)

IE_BEGIN(transaction_identifier_dest_5)

BF(4,M4(1,1,0,1),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_5)

IE_BEGIN(transaction_identifier_dest_6)

BF(4,M4(1,1,1,0),ACT_NOP,ANONYMOUS,SILENT)

IE_END(transaction_identifier_dest_6)

IE_BEGIN(authentication_request_message_type)

BF(8, 0x12,ACT_NOP,ANONYMOUS,SILENT)

IE_END(authentication_request_message_type)

IE_BEGIN(identity_request_message_type)

BF(8, 0x18,ACT_NOP,ANONYMOUS,SILENT)

IE_END(identity_request_message_type)

IE_BEGIN(identity_response_message_type)

BF(8, 0x19,ACT_NOP,ANONYMOUS,SILENT)

IE_END(identity_response_message_type)

IE_BEGIN(authentication_response_message_type)

BF(8, 0x14,ACT_NOP,ANONYMOUS,SILENT)

IE_END(authentication_response_message_type)

IE_BEGIN(setup_message_type)

BF(8, 0x05,ACT_NOP,ANONYMOUS,SILENT)

IE_END(setup_message_type)

IE_BEGIN(call_confirmed_message_type)

BF(8, 0x08,ACT_NOP,ANONYMOUS,SILENT)

IE_END(call_confirmed_message_type)

IE_BEGIN(connect_message_type)

BF(8, 0x07,ACT_NOP,ANONYMOUS,SILENT)

IE_END(connect_message_type)

IE_BEGIN(connect_acknowledge_message_type)

BF(8, 0x0F,ACT_NOP,ANONYMOUS,SILENT)

IE_END(connect_acknowledge_message_type)

IE_BEGIN(alerting_message_type)

BF(8, 0x01,ACT_NOP,ANONYMOUS,SILENT)

IE_END(alerting_message_type)

IE_BEGIN(progress_message_type)

BF(8, 0x03,ACT_NOP,ANONYMOUS,SILENT)

IE_END(progress_message_type)

IE_BEGIN(status_message_type)

BF(8, 0x3D,ACT_NOP,ANONYMOUS,SILENT)

IE_END(status_message_type)

IE_BEGIN(status_enquiry_message_type)

BF(8, 0x34,ACT_NOP,ANONYMOUS,SILENT)

IE_END(status_enquiry_message_type)

IE_BEGIN(unknown_message_type)

BF(8, 0x20,ACT_NOP,ANONYMOUS,SILENT)

IE_END(unknown_message_type)

IE_BEGIN(disconnect_message_type)

BF(8, 0x25,ACT_NOP,ANONYMOUS,SILENT)

IE_END(disconnect_message_type)

IE_BEGIN(release_complete_message_type)

BF(8, 0x2A,ACT_NOP,ANONYMOUS,SILENT)

IE_END(release_complete_message_type)

IE_BEGIN(release_message_type)

BF(8, 0x2D,ACT_NOP,ANONYMOUS,SILENT)

IE_END(release_message_type)

IE_BEGIN(channel_release_message_type)

BF(8, 0x0D,ACT_NOP,ANONYMOUS,SILENT)

IE_END(channel_release_message_type)

IE_BEGIN(start_dtmf_message_type)

BF(8, 0x35,ACT_NOP,ANONYMOUS,SILENT)

IE_END(start_dtmf_message_type)

IE_BEGIN(start_dtmf_acknowledge_message_type)

BF(8, 0x36,ACT_NOP,ANONYMOUS,SILENT)

IE_END(start_dtmf_acknowledge_message_type)

IE_BEGIN(stop_dtmf_message_type)

BF(8, 0x31,ACT_NOP,ANONYMOUS,SILENT)

IE_END(stop_dtmf_message_type)

IE_BEGIN(stop_dtmf_acknowledge_message_type)

BF(8, 0x32,ACT_NOP,ANONYMOUS,SILENT)

IE_END(stop_dtmf_acknowledge_message_type)

IE_BEGIN(start_dtmf_reject_message_type)

BF(8, 0x37,ACT_NOP,ANONYMOUS,SILENT)

IE_END(start_dtmf_reject_message_type)

IE_BEGIN(cp_user_data_sms_submit)

BF(8,0x1d, ACT_CHECK, len, "length of cp user data")

BF(5, 0,   ACT_CHECK, spare, "spare bits")

BF(3, 0,   ACT_CHECK, mti, "rp data mobile to mobile")

BF(8,0,    ACT_CHECK, msg_ref, "message reference")

BF(8, 0,   ACT_CHECK, rp_orig_len, "rp originator address")

BF(8, 5,   ACT_CHECK, rp_dest_len, "rp destination address")

BF(1, 1,   ACT_CHECK, rp_dest_ext, "Extension Bit")

BF(3, 1,   ACT_CHECK, rp_dest_type_of_number, "Unknown")

BF(4, 1,   ACT_CHECK, rp_dest_numbering_plan, "ISDN/telephony")

BF(4,2,    ACT_CHECK, rp_dest_digit_2, "Digit 2")

BF(4,1,    ACT_CHECK, rp_dest_digit_1, "Digit 1")

BF(4,4,    ACT_CHECK, rp_dest_digit_4, "Digit 4")

BF(4,3,    ACT_CHECK, rp_dest_digit_3, "Digit 3")

BF(4,6,    ACT_CHECK, rp_dest_digit_6, "Digit 6")

BF(4,5,    ACT_CHECK, rp_dest_digit_5, "Digit 5")

BF(4,8,    ACT_CHECK, rp_dest_digit_8, "Digit 8")

BF(4,7,    ACT_CHECK, rp_dest_digit_7, "Digit 7")

BF(8 ,19, ACT_CHECK, rp_user_data_len, "rp user data len")

BF(1, 0,   ACT_CHECK, tp_rp, "reply path")

BF(1, 0,   ACT_CHECK, tp_udhi, "no user data header indicator")

BF(1, 0,   ACT_CHECK, tp_srr, "no status report request")

BF(2, 2,   ACT_CHECK, tp_vpf, "relative validity period format")

BF(1, 0,   ACT_CHECK, tp_rd, "no reject duplicates")

BF(2, 1,   ACT_CHECK, tp_mti, "sms submit")

BF(8, 0,   ACT_CHECK, tp_msg_ref, "tp_message reference")

BF(8, 4,   ACT_CHECK, tp_da_len, "tp_da with four digits")

BF(1, 1,   ACT_CHECK, tp_da_ext, "Extension Bit")

BF(3, 1,   ACT_CHECK, tp_da_type_of_number, "Unknown")

BF(4, 1,   ACT_CHECK, tp_da_numbering_plan, "ISDN/telephony")

BF(4,6,    ACT_CHECK, tp_da_digit_2, "Digit 2")

BF(4,5,    ACT_CHECK, tp_da_digit_1, "Digit 1")

BF(4,8,    ACT_CHECK, tp_da_digit_4, "Digit 4")

BF(4,7,    ACT_CHECK, tp_da_digit_3, "Digit 3")

BF(8, 0,   ACT_CHECK, tp_pd, "protocol identifier")

BF(8, 0,   ACT_CHECK, tp_dcs, "data coding scheme")

BF(8, 3,   ACT_CHECK, tp_vp, "relative time 03")

BF(8, 0x0A, ACT_CHECK, tp_ud_len, "characters in sms")

BF(8, 0xC8, ACT_CHECK, tp_ud_1, "SMS Halli Hallo")

BF(8, 0x20, ACT_CHECK, tp_ud_2, SILENT)

BF(8, 0x93, ACT_CHECK, tp_ud_3, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_4, SILENT)

BF(8, 0x44, ACT_CHECK, tp_ud_5, SILENT)

BF(8, 0x06, ACT_CHECK, tp_ud_6, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_7, SILENT)

BF(8, 0xCC, ACT_CHECK, tp_ud_8, SILENT)

BF(8, 0x27, ACT_CHECK, tp_ud_9, SILENT)

IE_END(cp_user_data_sms_submit)

IE_BEGIN(cp_user_data_sms_deliver)

BF(8,38, ACT_CHECK, len, "length of cp user data")

BF(5, 0,   ACT_CHECK, spare, "spare bits")

BF(3, 1,   ACT_CHECK, mti, "rp data network to mobile")

BF(8, 22, ACT_CHECK, msg_ref, "message reference")

BF(8, 7,   ACT_CHECK, rp_orig_len, "rp originator address")

BF(1, 1,   ACT_CHECK, rp_orig_ext, "Extension Bit")

BF(3, 0,   ACT_CHECK, rp_orig_type_of_number, "Unknown")

BF(4, 1,   ACT_CHECK, rp_orig_numbering_plan, "ISDN/telephony")

BF(4,3,    ACT_CHECK, rp_orig_digit_2, "Digit 2")

BF(4,0,    ACT_CHECK, rp_orig_digit_1, "Digit 1")

BF(4,3,    ACT_CHECK, rp_orig_digit_4, "Digit 4")

BF(4,0,    ACT_CHECK, rp_orig_digit_3, "Digit 3")

BF(4,0,    ACT_CHECK, rp_orig_digit_6, "Digit 6")

BF(4,9,    ACT_CHECK, rp_orig_digit_5, "Digit 5")

BF(4,4,    ACT_CHECK, rp_orig_digit_8, "Digit 8")

BF(4,9,    ACT_CHECK, rp_orig_digit_7, "Digit 7")

BF(4,1,    ACT_CHECK, rp_orig_digit_10, "Digit 10")

BF(4,1,    ACT_CHECK, rp_orig_digit_9, "Digit 9")

BF(4,0xF, ACT_CHECK, rp_orig_digit_12, "Digit 12")

BF(4,0x7, ACT_CHECK, rp_orig_digit_11, "Digit 11")

BF(8, 0,   ACT_CHECK, rp_dest_len, "rp destination address")

BF(8 ,26, ACT_CHECK, rp_user_data_len, "rp user data len")

BF(1, 1,   ACT_CHECK, tp_rp, "reply path")

BF(1, 0,   ACT_CHECK, tp_udhi, "no user data header indicator")

BF(1, 0,   ACT_CHECK, tp_sri, "no status report indication")

BF(2, 0,   ACT_CHECK, tp_spare, "spare bits")

BF(1, 0,   ACT_CHECK, tp_mms, "no more messages to send")

BF(2, 0,   ACT_CHECK, tp_mti, "sms deliver")

BF(8, 8,   ACT_CHECK, tp_oa_len, "tp oa with eight digits")

BF(1, 1,   ACT_CHECK, tp_oa_ext, "Extension Bit")

BF(3, 0,   ACT_CHECK, tp_oa_type_of_number, "Unknown")

BF(4, 1,   ACT_CHECK, tp_oa_numbering_plan, "ISDN/telephony")

BF(4,3,    ACT_CHECK, tp_oa_digit_2, "Digit 2")

BF(4,0,    ACT_CHECK, tp_oa_digit_1, "Digit 1")

BF(4,3,    ACT_CHECK, tp_oa_digit_4, "Digit 4")

BF(4,0,    ACT_CHECK, tp_oa_digit_3, "Digit 3")

BF(4,0,    ACT_CHECK, tp_oa_digit_6, "Digit 6")

BF(4,9,    ACT_CHECK, tp_oa_digit_5, "Digit 5")

BF(4,4,    ACT_CHECK, tp_oa_digit_8, "Digit 8")

BF(4,9,    ACT_CHECK, tp_oa_digit_7, "Digit 7")

BF(8,0x1F,   ACT_CHECK, tp_pd, "protocol identifier")

BF(8, 0xF2, ACT_CHECK, tp_dcs, "data coding scheme")

BF(8, 0x89, ACT_CHECK, tp_scts_year, "year 98")

BF(8, 0x40, ACT_CHECK, tp_scts_month, "month 04")

BF(8, 0x61, ACT_CHECK, tp_scts_day, "day 16")

BF(8, 0x01, ACT_CHECK, tp_scts_hour, "hour 10")

BF(8, 0x51, ACT_CHECK, tp_scts_minute, "minute 15")

BF(8, 0x43, ACT_CHECK, tp_scts_second, "second 34")

BF(8, 0x00, ACT_CHECK, tp_scts_timezone, "zone 00")

BF(8, 0x0A, ACT_CHECK, tp_ud_len, "characters in sms")

BF(8, 0xC8, ACT_CHECK, tp_ud_1, "SMS Halli Hallo")

BF(8, 0x20, ACT_CHECK, tp_ud_2, SILENT)

BF(8, 0x93, ACT_CHECK, tp_ud_3, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_4, SILENT)

BF(8, 0x44, ACT_CHECK, tp_ud_5, SILENT)

BF(8, 0x06, ACT_CHECK, tp_ud_6, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_7, SILENT)

BF(8, 0xCC, ACT_CHECK, tp_ud_8, SILENT)

BF(8, 0x27, ACT_CHECK, tp_ud_9, SILENT)

IE_END(cp_user_data_sms_deliver)

IE_BEGIN(cp_user_data_sms_deliver_class1)

BF(8,38, ACT_CHECK, len, "length of cp user data")

BF(5, 0,   ACT_CHECK, spare, "spare bits")

BF(3, 1,   ACT_CHECK, mti, "rp data network to mobile")

BF(8, 22, ACT_CHECK, msg_ref, "message reference")

BF(8, 7,   ACT_CHECK, rp_orig_len, "rp originator address")

BF(1, 1,   ACT_CHECK, rp_orig_ext, "Extension Bit")

BF(3, 0,   ACT_CHECK, rp_orig_type_of_number, "Unknown")

BF(4, 1,   ACT_CHECK, rp_orig_numbering_plan, "ISDN/telephony")

BF(4,3,    ACT_CHECK, rp_orig_digit_2, "Digit 2")

BF(4,0,    ACT_CHECK, rp_orig_digit_1, "Digit 1")

BF(4,3,    ACT_CHECK, rp_orig_digit_4, "Digit 4")

BF(4,0,    ACT_CHECK, rp_orig_digit_3, "Digit 3")

BF(4,0,    ACT_CHECK, rp_orig_digit_6, "Digit 6")

BF(4,9,    ACT_CHECK, rp_orig_digit_5, "Digit 5")

BF(4,4,    ACT_CHECK, rp_orig_digit_8, "Digit 8")

BF(4,9,    ACT_CHECK, rp_orig_digit_7, "Digit 7")

BF(4,1,    ACT_CHECK, rp_orig_digit_10, "Digit 10")

BF(4,1,    ACT_CHECK, rp_orig_digit_9, "Digit 9")

BF(4,0xF, ACT_CHECK, rp_orig_digit_12, "Digit 12")

BF(4,0x7, ACT_CHECK, rp_orig_digit_11, "Digit 11")

BF(8, 0,   ACT_CHECK, rp_dest_len, "rp destination address")

BF(8 ,26, ACT_CHECK, rp_user_data_len, "rp user data len")

BF(1, 1,   ACT_CHECK, tp_rp, "reply path")

BF(1, 0,   ACT_CHECK, tp_udhi, "no user data header indicator")

BF(1, 0,   ACT_CHECK, tp_sri, "no status report indication")

BF(2, 0,   ACT_CHECK, tp_spare, "spare bits")

BF(1, 0,   ACT_CHECK, tp_mms, "no more messages to send")

BF(2, 0,   ACT_CHECK, tp_mti, "sms deliver")

BF(8, 8,   ACT_CHECK, tp_oa_len, "tp oa with eight digits")

BF(1, 1,   ACT_CHECK, tp_oa_ext, "Extension Bit")

BF(3, 0,   ACT_CHECK, tp_oa_type_of_number, "Unknown")

BF(4, 1,   ACT_CHECK, tp_oa_numbering_plan, "ISDN/telephony")

BF(4,3,    ACT_CHECK, tp_oa_digit_2, "Digit 2")

BF(4,0,    ACT_CHECK, tp_oa_digit_1, "Digit 1")

BF(4,3,    ACT_CHECK, tp_oa_digit_4, "Digit 4")

BF(4,0,    ACT_CHECK, tp_oa_digit_3, "Digit 3")

BF(4,0,    ACT_CHECK, tp_oa_digit_6, "Digit 6")

BF(4,9,    ACT_CHECK, tp_oa_digit_5, "Digit 5")

BF(4,4,    ACT_CHECK, tp_oa_digit_8, "Digit 8")

BF(4,9,    ACT_CHECK, tp_oa_digit_7, "Digit 7")

BF(8,0x1F,   ACT_CHECK, tp_pd, "protocol identifier")

BF(8, 0xF1, ACT_CHECK, tp_dcs, "data coding scheme")

BF(8, 0x89, ACT_CHECK, tp_scts_year, "year 98")

BF(8, 0x40, ACT_CHECK, tp_scts_month, "month 04")

BF(8, 0x61, ACT_CHECK, tp_scts_day, "day 16")

BF(8, 0x01, ACT_CHECK, tp_scts_hour, "hour 10")

BF(8, 0x51, ACT_CHECK, tp_scts_minute, "minute 15")

BF(8, 0x43, ACT_CHECK, tp_scts_second, "second 34")

BF(8, 0x00, ACT_CHECK, tp_scts_timezone, "zone 00")

BF(8, 0x0A, ACT_CHECK, tp_ud_len, "characters in sms")

BF(8, 0xC8, ACT_CHECK, tp_ud_1, "SMS Halli Hallo")

BF(8, 0x20, ACT_CHECK, tp_ud_2, SILENT)

BF(8, 0x93, ACT_CHECK, tp_ud_3, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_4, SILENT)

BF(8, 0x44, ACT_CHECK, tp_ud_5, SILENT)

BF(8, 0x06, ACT_CHECK, tp_ud_6, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_7, SILENT)

BF(8, 0xCC, ACT_CHECK, tp_ud_8, SILENT)

BF(8, 0x27, ACT_CHECK, tp_ud_9, SILENT)

IE_END(cp_user_data_sms_deliver_class1)

IE_BEGIN(cp_user_data_sms_deliver_def)

BF(8,38, ACT_CHECK, len, "length of cp user data")

BF(5, 0,   ACT_CHECK, spare, "spare bits")

BF(3, 1,   ACT_CHECK, mti, "rp data network to mobile")

BF(8, 22, ACT_CHECK, msg_ref, "message reference")

BF(8, 7,   ACT_CHECK, rp_orig_len, "rp originator address")

BF(1, 1,   ACT_CHECK, rp_orig_ext, "Extension Bit")

BF(3, 0,   ACT_CHECK, rp_orig_type_of_number, "Unknown")

BF(4, 1,   ACT_CHECK, rp_orig_numbering_plan, "ISDN/telephony")

BF(4,3,    ACT_CHECK, rp_orig_digit_2, "Digit 2")

BF(4,0,    ACT_CHECK, rp_orig_digit_1, "Digit 1")

BF(4,3,    ACT_CHECK, rp_orig_digit_4, "Digit 4")

BF(4,0,    ACT_CHECK, rp_orig_digit_3, "Digit 3")

BF(4,0,    ACT_CHECK, rp_orig_digit_6, "Digit 6")

BF(4,9,    ACT_CHECK, rp_orig_digit_5, "Digit 5")

BF(4,4,    ACT_CHECK, rp_orig_digit_8, "Digit 8")

BF(4,9,    ACT_CHECK, rp_orig_digit_7, "Digit 7")

BF(4,1,    ACT_CHECK, rp_orig_digit_10, "Digit 10")

BF(4,1,    ACT_CHECK, rp_orig_digit_9, "Digit 9")

BF(4,0xF, ACT_CHECK, rp_orig_digit_12, "Digit 12")

BF(4,0x7, ACT_CHECK, rp_orig_digit_11, "Digit 11")

BF(8, 0,   ACT_CHECK, rp_dest_len, "rp destination address")

BF(8 ,26, ACT_CHECK, rp_user_data_len, "rp user data len")

BF(1, 1,   ACT_CHECK, tp_rp, "reply path")

BF(1, 0,   ACT_CHECK, tp_udhi, "no user data header indicator")

BF(1, 0,   ACT_CHECK, tp_sri, "no status report indication")

BF(2, 0,   ACT_CHECK, tp_spare, "spare bits")

BF(1, 0,   ACT_CHECK, tp_mms, "no more messages to send")

BF(2, 0,   ACT_CHECK, tp_mti, "sms deliver")

BF(8, 8,   ACT_CHECK, tp_oa_len, "tp oa with eight digits")

BF(1, 1,   ACT_CHECK, tp_oa_ext, "Extension Bit")

BF(3, 0,   ACT_CHECK, tp_oa_type_of_number, "Unknown")

BF(4, 1,   ACT_CHECK, tp_oa_numbering_plan, "ISDN/telephony")

BF(4,3,    ACT_CHECK, tp_oa_digit_2, "Digit 2")

BF(4,0,    ACT_CHECK, tp_oa_digit_1, "Digit 1")

BF(4,3,    ACT_CHECK, tp_oa_digit_4, "Digit 4")

BF(4,0,    ACT_CHECK, tp_oa_digit_3, "Digit 3")

BF(4,0,    ACT_CHECK, tp_oa_digit_6, "Digit 6")

BF(4,9,    ACT_CHECK, tp_oa_digit_5, "Digit 5")

BF(4,4,    ACT_CHECK, tp_oa_digit_8, "Digit 8")

BF(4,9,    ACT_CHECK, tp_oa_digit_7, "Digit 7")

BF(8,0x1F,   ACT_CHECK, tp_pd, "protocol identifier")

BF(8, 0x00, ACT_CHECK, tp_dcs, "data coding scheme")

BF(8, 0x89, ACT_CHECK, tp_scts_year, "year 98")

BF(8, 0x40, ACT_CHECK, tp_scts_month, "month 04")

BF(8, 0x61, ACT_CHECK, tp_scts_day, "day 16")

BF(8, 0x01, ACT_CHECK, tp_scts_hour, "hour 10")

BF(8, 0x51, ACT_CHECK, tp_scts_minute, "minute 15")

BF(8, 0x43, ACT_CHECK, tp_scts_second, "second 34")

BF(8, 0x00, ACT_CHECK, tp_scts_timezone, "zone 00")

BF(8, 0x0A, ACT_CHECK, tp_ud_len, "characters in sms")

BF(8, 0xC8, ACT_CHECK, tp_ud_1, "SMS Halli Hallo")

BF(8, 0x20, ACT_CHECK, tp_ud_2, SILENT)

BF(8, 0x93, ACT_CHECK, tp_ud_3, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_4, SILENT)

BF(8, 0x44, ACT_CHECK, tp_ud_5, SILENT)

BF(8, 0x06, ACT_CHECK, tp_ud_6, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_7, SILENT)

BF(8, 0xCC, ACT_CHECK, tp_ud_8, SILENT)

BF(8, 0x27, ACT_CHECK, tp_ud_9, SILENT)

IE_END(cp_user_data_sms_deliver_def)

IE_BEGIN(cp_user_data_rp_ack_to_bs)

BF(8, 2, ACT_CHECK, len, "length of cp user data")

BF(5, 0,   ACT_CHECK, spare, "spare bits")

BF(3, 2,   ACT_CHECK, mti, "rp ack mobile to network")

BF(8, 22, ACT_CHECK, msg_ref, "message reference")

IE_END(cp_user_data_rp_ack_to_bs)

IE_BEGIN(cp_user_data_rp_ack_to_ms)

BF(8, 2, ACT_CHECK, len, "length of cp user data")

BF(5, 0,   ACT_CHECK, spare, "spare bits")

BF(3, 2,   ACT_CHECK, mti, "rp ack network to mobile")

BF(8, 22, ACT_CHECK, msg_ref, "message reference")

IE_END(cp_user_data_rp_ack_to_ms)

IE_BEGIN(cp_user_data_rp_error_111_to_bs)

BF(8, 4, ACT_CHECK, len, "length of cp user data")

BF(5, 0,   ACT_CHECK, spare, "spare bits")

BF(3, 4,   ACT_CHECK, mti, "rp error mobile to network")

BF(8, 22, ACT_CHECK, msg_ref, "message reference")

BF(8, 1, ACT_CHECK, cause_len, "rp error cause len")

BF(8,111, ACT_CHECK, cause, "rp error cause")

IE_END(cp_user_data_rp_error_111_to_bs)

IE_BEGIN(cp_user_data_rp_error_22_to_bs)

BF(8, 4, ACT_CHECK, len, "length of cp user data")

BF(5, 0,   ACT_CHECK, spare, "spare bits")

BF(3, 4,   ACT_CHECK, mti, "rp error mobile to network")

BF(8, 22, ACT_CHECK, msg_ref, "message reference")

BF(8, 1, ACT_CHECK, cause_len, "rp error cause len")

BF(8,22, ACT_CHECK, cause, "rp error cause")

IE_END(cp_user_data_rp_error_22_to_bs)

IE_BEGIN(cp_user_data_rp_smma_to_bs)

BF(8, 2, ACT_CHECK, len, "length of cp user data")

BF(5, 0,   ACT_CHECK, spare, "spare bits")

BF(3, 6,   ACT_CHECK, mti, "rp smma mobile to network")

BF(8, 0, ACT_CHECK, msg_ref, "message reference")

IE_END(cp_user_data_rp_smma_to_bs)

IE_BEGIN(cbch_message)

BF(16 ,0x0001, ACT_CHECK, serial_number, "serial number")

BF(16, 0x0005, ACT_CHECK, mid, "message identifier")

BF(8, 0x00,      ACT_CHECK, dcs, "data coding scheme")

BF(8, 22,         ACT_CHECK, page, "page parameter")

BF(8, 0xC8, ACT_CHECK, tp_ud_1, "SMS Halli Hallo")

BF(8, 0x20, ACT_CHECK, tp_ud_2, SILENT)

BF(8, 0x93, ACT_CHECK, tp_ud_3, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_4, SILENT)

BF(8, 0x44, ACT_CHECK, tp_ud_5, SILENT)

BF(8, 0x06, ACT_CHECK, tp_ud_6, SILENT)

BF(8, 0x99, ACT_CHECK, tp_ud_7, SILENT)

BF(8, 0xCC, ACT_CHECK, tp_ud_8, SILENT)

BF(8, 0x27, ACT_CHECK, tp_ud_9, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(32, 0, ACT_CHECK, ANONYMOUS, SILENT)

BF(24, 0, ACT_CHECK, ANONYMOUS, SILENT)

IE_END(cbch_message)

/*------------------------------------------------------------------*\

| Messages

\*------------------------------------------------------------------*/

MSG3_BEGIN(paging_request_type_1)

IE(l2_pseudo_length_11)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(paging_request_type_1_message_type)

IE(channels_needed_for_mobiles_1_and_2)

IE(page_mode)

IE(mobile_identity)

IE(p1_rest_octets)

MSG3_END(paging_request_type_1)

MSG3_BEGIN(channel_request)

IE(rach)

MSG3_END(channel_request)

MSG3_BEGIN(channel_request_moc)

IE(rach_moc)

MSG3_END(channel_request_moc)

MSG3_BEGIN(channel_request_111)

IE(rach_111)

MSG3_END(channel_request_111)

MSG3_BEGIN(immediate_assignment)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment)

MSG3_BEGIN(immediate_assignment_tch)

IE(l2_pseudo_length_21)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(immediate_assignment_message_type)

IE(spare_half_octet)

IE(page_mode)

IE(channel_description_tch)

IE(request_reference)

IE(timing_advance)

IE(mobile_allocation)

IE(ia_rest_octets)

MSG3_END(immediate_assignment_tch)

MSG3_BEGIN(paging_response)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(paging_response_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number)

IE(ms_classmark)

IE(mobile_identity)

MSG3_END(paging_response)

MSG3_BEGIN(authentication_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_request_message_type)

IE(spare_half_octet)

IE(ciphering_key_sequence_number_2)

IE(authentication_parameter_rand)

MSG3_END(authentication_request)

MSG3_BEGIN(authentication_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(authentication_response_message_type)

IE(authentication_parameter_sres)

MSG3_END(authentication_response)

MSG3_BEGIN(identity_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(identity_request_message_type)

IE(spare_half_octet)

IE(identity_type)

MSG3_END(identity_request)

MSG3_BEGIN(identity_response)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(identity_response_message_type)

IE(mobile_identity)

MSG3_END(identity_response)

MSG3_BEGIN(cm_service_request)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_request_message_type)

IE(ciphering_key_sequence_number)

IE(cm_service_type_moc)

IE(ms_classmark)

IE(mobile_identity)

MSG3_END(cm_service_request)

MSG3_BEGIN(cm_service_abort)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_abort_message_type)

MSG3_END(cm_service_abort)

MSG3_BEGIN(cm_service_reject)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_reject_message_type)

IE(reject_cause)

MSG3_END(cm_service_reject)

MSG3_BEGIN(cm_service_accept)

IE(skip_indicator)

IE(mobility_management_protocol_discriminator)

IE(cm_service_accept_message_type)

MSG3_END(cm_service_accept)

MSG3_BEGIN(ciphering_mode_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_command_message_type)

IE(ciphering_mode_setting)

IE(cipher_response)

MSG3_END(ciphering_mode_command)

MSG3_BEGIN(ciphering_mode_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(ciphering_mode_complete_message_type)

MSG3_END(ciphering_mode_complete)

MSG3_BEGIN(channel_mode_modify)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_mode_modify_message_type)

IE(channel_description_tch)

IE(channel_mode_speech)

MSG3_END(channel_mode_modify)

MSG3_BEGIN(channel_mode_modify_acknowledge)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_mode_modify_acknowledge_message_type)

IE(channel_description_tch)

IE(channel_mode_speech)

MSG3_END(channel_mode_modify_acknowledge)

MSG3_BEGIN(setup)            /* contains 'signal' but no 'Bearer Cap' */

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_34)

IE(signal_call_waiting)

MSG3_END(setup)

MSG3_BEGIN(setup_no_signal)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

MSG3_END(setup_no_signal)

MSG3_BEGIN(setup_moc)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_04)

IE(bearer_capability)

IE(iei_5E)

IE(called_party_bcd_number)

MSG3_END(setup_moc)

MSG3_BEGIN(setup_data)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(setup_message_type)

IE(iei_04)

IE(bearer_capability_data)

IE(iei_5E)

IE(called_party_bcd_number)

MSG3_END(setup_data)

MSG3_BEGIN(call_confirmed)           /* contains bearer capability */

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(call_confirmed_message_type)

IE(iei_04)

IE(bearer_capability)

IE(iei_15)

IE(cc_capabilities)

MSG3_END(call_confirmed)

MSG3_BEGIN(call_confirmed_user_busy) /* contains cause user busy */

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(call_confirmed_message_type)

IE(iei_04)

IE(bearer_capability)

IE(iei_08)

IE(cause_17)

IE(iei_15)

IE(cc_capabilities)

MSG3_END(call_confirmed_user_busy)

MSG3_BEGIN(call_proceeding)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(call_proceeding_message_type)

MSG3_END(call_proceeding)

MSG3_BEGIN(connect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(connect_message_type)

MSG3_END(connect)

MSG3_BEGIN(unknown_message)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(unknown_message_type)

MSG3_END(unknown_message)

MSG3_BEGIN(unknown_message_mtc)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(unknown_message_type)

MSG3_END(unknown_message_mtc)

MSG3_BEGIN(alerting)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(alerting_message_type)

MSG3_END(alerting)

MSG3_BEGIN(progress)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(progress_message_type)

IE(progress_indicator_32)

MSG3_END(progress)

MSG3_BEGIN(progress_1)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(progress_message_type)

IE(progress_indicator_8)

MSG3_END(progress_1)

MSG3_BEGIN(assignment_command)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_command_message_type)

IE(description_of_the_first_channel_after_time)

IE(power_command)

IE(iei_63)

IE(mode_of_the_first_channel)

MSG3_END(assignment_command)

MSG3_BEGIN(assignment_complete)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(assignment_complete_message_type)

IE(rr_cause)

MSG3_END(assignment_complete)

MSG3_BEGIN(connect_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(connect_acknowledge_message_type)

MSG3_END(connect_acknowledge)

MSG3_BEGIN(status_enquiry_0)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_0)

MSG3_BEGIN(status_enquiry_1)

IE(transaction_identifier_dest_1)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_1)

MSG3_BEGIN(status_enquiry_2)

IE(transaction_identifier_dest_2)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_2)

MSG3_BEGIN(status_enquiry_3)

IE(transaction_identifier_dest_3)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_3)

MSG3_BEGIN(status_enquiry_4)

IE(transaction_identifier_dest_4)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_4)

MSG3_BEGIN(status_enquiry_5)

IE(transaction_identifier_dest_5)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_5)

MSG3_BEGIN(status_enquiry_6)

IE(transaction_identifier_dest_6)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_6)

MSG3_BEGIN(status_enquiry_0_mtc)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_0_mtc)

MSG3_BEGIN(status_enquiry_1_mtc)

IE(transaction_identifier_source_1)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_1_mtc)

MSG3_BEGIN(status_enquiry_2_mtc)

IE(transaction_identifier_source_2)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_2_mtc)

MSG3_BEGIN(status_enquiry_3_mtc)

IE(transaction_identifier_source_3)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_3_mtc)

MSG3_BEGIN(status_enquiry_4_mtc)

IE(transaction_identifier_source_4)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_4_mtc)

MSG3_BEGIN(status_enquiry_5_mtc)

IE(transaction_identifier_source_5)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_5_mtc)

MSG3_BEGIN(status_enquiry_6_mtc)

IE(transaction_identifier_source_6)

IE(call_control_protocol_discriminator)

IE(status_enquiry_message_type)

MSG3_END(status_enquiry_6_mtc)

MSG3_BEGIN(disconnect_t303)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_102_303)

MSG3_END(disconnect_t303)

MSG3_BEGIN(disconnect_t310)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_102_310)

MSG3_END(disconnect_t310)

MSG3_BEGIN(disconnect_t313)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_102_313)

MSG3_END(disconnect_t313)

MSG3_BEGIN(disconnect_8)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_16)

IE(iei_1E)

IE(progress_indicator_8)

MSG3_END(disconnect_8)

MSG3_BEGIN(disconnect_8_mtc)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_16)

IE(iei_1E)

IE(progress_indicator_8)

MSG3_END(disconnect_8_mtc)

MSG3_BEGIN(disconnect_ms)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_16)

MSG3_END(disconnect_ms)

MSG3_BEGIN(disconnect)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(disconnect_message_type)

IE(cause_16)

MSG3_END(disconnect)

MSG3_BEGIN(release_complete)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_03)

MSG3_END(release_complete)

MSG3_BEGIN(release_complete_empty)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

MSG3_END(release_complete_empty)

MSG3_BEGIN(release_complete_ms)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

MSG3_END(release_complete_ms)

MSG3_BEGIN(release)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(release_message_type)

MSG3_END(release)

MSG3_BEGIN(release_ms)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_message_type)

MSG3_END(release_ms)

MSG3_BEGIN(release_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_message_type)

MSG3_END(release_mtc)

MSG3_BEGIN(release_t305)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(release_message_type)

IE(iei_08)

IE(cause_16)

MSG3_END(release_t305)

MSG3_BEGIN(release_bs)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_message_type)

IE(iei_08)

IE(cause_31)

MSG3_END(release_bs)

MSG3_BEGIN(release_complete_88)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_88)

MSG3_END(release_complete_88)

MSG3_BEGIN(release_complete_bs)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

MSG3_END(release_complete_bs)

MSG3_BEGIN(release_complete_0)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_0)

MSG3_BEGIN(release_complete_1)

IE(transaction_identifier_source_1)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_1)

MSG3_BEGIN(release_complete_2)

IE(transaction_identifier_source_2)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_2)

MSG3_BEGIN(release_complete_3)

IE(transaction_identifier_source_3)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_3)

MSG3_BEGIN(release_complete_4)

IE(transaction_identifier_source_4)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_4)

MSG3_BEGIN(release_complete_5)

IE(transaction_identifier_source_5)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_5)

MSG3_BEGIN(release_complete_6)

IE(transaction_identifier_source_6)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_6)

MSG3_BEGIN(release_complete_0_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_0_mtc)

MSG3_BEGIN(release_complete_1_mtc)

IE(transaction_identifier_dest_1)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_1_mtc)

MSG3_BEGIN(release_complete_2_mtc)

IE(transaction_identifier_dest_2)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_2_mtc)

MSG3_BEGIN(release_complete_3_mtc)

IE(transaction_identifier_dest_3)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_3_mtc)

MSG3_BEGIN(release_complete_4_mtc)

IE(transaction_identifier_dest_4)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_4_mtc)

MSG3_BEGIN(release_complete_5_mtc)

IE(transaction_identifier_dest_5)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_5_mtc)

MSG3_BEGIN(release_complete_6_mtc)

IE(transaction_identifier_dest_6)

IE(call_control_protocol_discriminator)

IE(release_complete_message_type)

IE(iei_08)

IE(cause_81)

MSG3_END(release_complete_6_mtc)

MSG3_BEGIN(channel_release)

IE(skip_indicator)

IE(rr_management_protocol_discriminator)

IE(channel_release_message_type)

IE(rr_cause)

MSG3_END(channel_release)

MSG3_BEGIN(status_30_1)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_1)

MSG3_END(status_30_1)

MSG3_BEGIN(status_30_3)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_3)

MSG3_END(status_30_3)

MSG3_BEGIN(status_30_4)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_4)

MSG3_END(status_30_4)

MSG3_BEGIN(status_30_7)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_7)

MSG3_END(status_30_7)

MSG3_BEGIN(status_30_8)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_8)

MSG3_END(status_30_8)

MSG3_BEGIN(status_30_9)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_9)

MSG3_END(status_30_9)

MSG3_BEGIN(status_30_10)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_10)

MSG3_END(status_30_10)

MSG3_BEGIN(status_30_10_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_10)

MSG3_END(status_30_10_mtc)

MSG3_BEGIN(status_30_11)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_11)

MSG3_END(status_30_11)

MSG3_BEGIN(status_30_11_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_11)

MSG3_END(status_30_11_mtc)

MSG3_BEGIN(status_30_12)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_12)

MSG3_END(status_30_12)

MSG3_BEGIN(status_30_12_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_12)

MSG3_END(status_30_12_mtc)

MSG3_BEGIN(status_30_19)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_19)

MSG3_END(status_30_19)

MSG3_BEGIN(status_30_19_mtc)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_30)

IE(call_state_19)

MSG3_END(status_30_19_mtc)

MSG3_BEGIN(status_97_1)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_1)

MSG3_END(status_97_1)

MSG3_BEGIN(status_97_3)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_3)

MSG3_END(status_97_3)

MSG3_BEGIN(status_97_4)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_4)

MSG3_END(status_97_4)

MSG3_BEGIN(status_97_7)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_7)

MSG3_END(status_97_7)

MSG3_BEGIN(status_97_8)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_8)

MSG3_END(status_97_8)

MSG3_BEGIN(status_97_9)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_9)

MSG3_END(status_97_9)

MSG3_BEGIN(status_97_11)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_11)

MSG3_END(status_97_11)

MSG3_BEGIN(status_97_12)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(status_message_type)

IE(cause_97_20)

IE(call_state_12)

MSG3_END(status_97_12)

MSG3_BEGIN(stop_dtmf)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(stop_dtmf_message_type)

MSG3_END(stop_dtmf)

MSG3_BEGIN(stop_dtmf_acknowledge)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(stop_dtmf_acknowledge_message_type)

MSG3_END(stop_dtmf_acknowledge)

MSG3_BEGIN(start_dtmf_reject)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_reject_message_type)

IE(cause_03)

MSG3_END(start_dtmf_reject)

MSG3_BEGIN(start_dtmf_0)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_0)

MSG3_END(start_dtmf_0)

MSG3_BEGIN(start_dtmf_1)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_1)

MSG3_END(start_dtmf_1)

MSG3_BEGIN(start_dtmf_2)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_2)

MSG3_END(start_dtmf_2)

MSG3_BEGIN(start_dtmf_3)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_3)

MSG3_END(start_dtmf_3)

MSG3_BEGIN(start_dtmf_4)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_4)

MSG3_END(start_dtmf_4)

MSG3_BEGIN(start_dtmf_5)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_5)

MSG3_END(start_dtmf_5)

MSG3_BEGIN(start_dtmf_6)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_6)

MSG3_END(start_dtmf_6)

MSG3_BEGIN(start_dtmf_7)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_7)

MSG3_END(start_dtmf_7)

MSG3_BEGIN(start_dtmf_8)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_8)

MSG3_END(start_dtmf_8)

MSG3_BEGIN(start_dtmf_9)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_9)

MSG3_END(start_dtmf_9)

MSG3_BEGIN(start_dtmf_A)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_A)

MSG3_END(start_dtmf_A)

MSG3_BEGIN(start_dtmf_B)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_B)

MSG3_END(start_dtmf_B)

MSG3_BEGIN(start_dtmf_C)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_C)

MSG3_END(start_dtmf_C)

MSG3_BEGIN(start_dtmf_D)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_D)

MSG3_END(start_dtmf_D)

MSG3_BEGIN(start_dtmf_star)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_star)

MSG3_END(start_dtmf_star)

MSG3_BEGIN(start_dtmf_hash)

IE(transaction_identifier_dest)

IE(call_control_protocol_discriminator)

IE(start_dtmf_message_type)

IE(iei_2C)

IE(keypad_facility_hash)

MSG3_END(start_dtmf_hash)

MSG3_BEGIN(start_dtmf_acknowledge_0)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_0)

MSG3_END(start_dtmf_acknowledge_0)

MSG3_BEGIN(start_dtmf_acknowledge_1)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_1)

MSG3_END(start_dtmf_acknowledge_1)

MSG3_BEGIN(start_dtmf_acknowledge_2)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_2)

MSG3_END(start_dtmf_acknowledge_2)

MSG3_BEGIN(start_dtmf_acknowledge_3)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_3)

MSG3_END(start_dtmf_acknowledge_3)

MSG3_BEGIN(start_dtmf_acknowledge_4)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_4)

MSG3_END(start_dtmf_acknowledge_4)

MSG3_BEGIN(start_dtmf_acknowledge_5)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_5)

MSG3_END(start_dtmf_acknowledge_5)

MSG3_BEGIN(start_dtmf_acknowledge_6)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_6)

MSG3_END(start_dtmf_acknowledge_6)

MSG3_BEGIN(start_dtmf_acknowledge_7)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_7)

MSG3_END(start_dtmf_acknowledge_7)

MSG3_BEGIN(start_dtmf_acknowledge_8)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_8)

MSG3_END(start_dtmf_acknowledge_8)

MSG3_BEGIN(start_dtmf_acknowledge_9)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_9)

MSG3_END(start_dtmf_acknowledge_9)

MSG3_BEGIN(start_dtmf_acknowledge_A)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_A)

MSG3_END(start_dtmf_acknowledge_A)

MSG3_BEGIN(start_dtmf_acknowledge_B)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_B)

MSG3_END(start_dtmf_acknowledge_B)

MSG3_BEGIN(start_dtmf_acknowledge_C)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_C)

MSG3_END(start_dtmf_acknowledge_C)

MSG3_BEGIN(start_dtmf_acknowledge_D)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_D)

MSG3_END(start_dtmf_acknowledge_D)

MSG3_BEGIN(start_dtmf_acknowledge_star)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_star)

MSG3_END(start_dtmf_acknowledge_star)

MSG3_BEGIN(start_dtmf_acknowledge_hash)

IE(transaction_identifier_source)

IE(call_control_protocol_discriminator)

IE(start_dtmf_acknowledge_message_type)

IE(iei_2C)

IE(keypad_facility_hash)

MSG3_END(start_dtmf_acknowledge_hash)

MSG3_BEGIN(cp_data_sms_submit)

IE(transaction_identifier_source)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_sms_submit)

MSG3_END(cp_data_sms_submit)

MSG3_BEGIN(cp_data_sms_deliver)

IE(transaction_identifier_source)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_sms_deliver)

MSG3_END(cp_data_sms_deliver)

MSG3_BEGIN(cp_data_sms_deliver_class1)

IE(transaction_identifier_source)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_sms_deliver_class1)

MSG3_END(cp_data_sms_deliver_class1)

MSG3_BEGIN(cp_data_sms_deliver_def)

IE(transaction_identifier_source)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_sms_deliver_def)

MSG3_END(cp_data_sms_deliver_def)

MSG3_BEGIN(cp_data_rp_ack_to_bs)

IE(transaction_identifier_dest)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_rp_ack_to_bs)

MSG3_END(cp_data_rp_ack_to_bs)

MSG3_BEGIN(cp_data_rp_error_111_to_bs)

IE(transaction_identifier_dest)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_rp_error_111_to_bs)

MSG3_END(cp_data_rp_error_111_to_bs)

MSG3_BEGIN(cp_data_rp_error_22_to_bs)

IE(transaction_identifier_dest)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_rp_error_22_to_bs)

MSG3_END(cp_data_rp_error_22_to_bs)

MSG3_BEGIN(cp_data_rp_smma_to_bs)

IE(transaction_identifier_dest)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_rp_smma_to_bs)

MSG3_END(cp_data_rp_smma_to_bs)

MSG3_BEGIN(cp_ack_to_bs)

IE(transaction_identifier_dest)

IE(sms_protocol_discriminator)

IE(cp_ack_message_type)

MSG3_END(cp_ack_to_bs)

MSG3_BEGIN(cp_ack_to_bs_mo)

IE(transaction_identifier_source)

IE(sms_protocol_discriminator)

IE(cp_ack_message_type)

MSG3_END(cp_ack_to_bs_mo)

MSG3_BEGIN(cp_ack_to_ms)

IE(transaction_identifier_source)

IE(sms_protocol_discriminator)

IE(cp_ack_message_type)

MSG3_END(cp_ack_to_ms)

MSG3_BEGIN(cp_ack_to_ms_mo)

IE(transaction_identifier_dest)

IE(sms_protocol_discriminator)

IE(cp_ack_message_type)

MSG3_END(cp_ack_to_ms_mo)

MSG3_BEGIN(cp_data_rp_ack_to_bs_mo)

IE(transaction_identifier_source)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_rp_ack_to_bs)

MSG3_END(cp_data_rp_ack_to_bs_mo)

MSG3_BEGIN(cp_data_rp_ack_to_ms_mo)

IE(transaction_identifier_dest)

IE(sms_protocol_discriminator)

IE(cp_data_message_type)

IE(cp_user_data_rp_ack_to_ms)

MSG3_END(cp_data_rp_ack_to_ms_mo)

MSG3_BEGIN(cp_error_to_ms_mo)

IE(transaction_identifier_dest)

IE(sms_protocol_discriminator)

IE(cp_error_message_type)

MSG3_END(cp_error_to_ms_mo)

MSG3_BEGIN(sms_cbch_message)

IE(cbch_message)

MSG3_END(sms_cbch_message)

# TEST CASES

## Preambles

### MSMS000: Initialization (One cell)

Description:

Preamble:	None

Script:

ISS_INIT	( 1 )

BS_SET_SYS_INFO	( 0, system_information_type_1 )

BS_SET_SYS_INFO	( 0, system_information_type_2 )

BS_SET_SYS_INFO	( 0, system_information_type_3 )

BS_SET_SYS_INFO	( 0, system_information_type_4 )

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5 )

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6 )

BS_SET_SCH	( 0,BSIC , RFN               )

BS_SET_ARFCN	( 0,ARFCN_BCCH               )

BS_SET_POWER	( 0,-73                      )

BS_ON_OFF	( 0,TRUE                     )

COMMAND	( "MMI CONFIG KEY_SEQUENCE=<#*91*1#>" )

ISS_DELAY	( 20000 ) /* wait until the MS has detected the cell */

SET_TIMEOUT (40000)

History:	15.12.97	VK	Initial

## Short message service point to point

### MSMS001: SMS mobile terminated (34.2.1)

Description:	To verify the ability of a MS to receive and decode the SMS where provided for the point to point service.

Preamble:	MSMS000

Script:

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11: MS shall respond to SABM in step 10" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"13: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"14" )

NOT_IMPLEMENTED	(	"15: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"16: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"17: Within TC1M after step 16, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"18: The main signalling link is released." )

NOT_IMPLEMENTED	(	"19: The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed" )

COMMAND	( "MMI CONFIG AT+CMGR=1" )

ISS_DELAY (5000)

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY (2000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"20" )

BS_RACH_AWAIT	( 0, channel_request,	"21: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"22: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"23: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"24" )

BS_MSG3_AWAIT	( 0, authentication_response,	"25: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"26: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"27: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"28: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"29: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"30: The MS shall respond to the SABM" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"31: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"32: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"33" )

NOT_IMPLEMENTED	(	"34: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"35: First CP-DATA from MS, contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"36: First CP-DATA message not acknowledged by SS" )

ISS_DELAY (5000)

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"37: Retransmitted CP-DATA from MS within twice TC1M, after step 35, contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"38: Second CP-DATA message is acknowledged within TC1M after step 37, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"39: The main signalling link is released." )

NOT_IMPLEMENTED	(	"40: The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed" )

ISS_DELAY (5000)

COMMAND	( "MMI CONFIG AT+CMGR=1" )

ISS_DELAY (5000)

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY (2000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK, SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"41" )

BS_RACH_AWAIT	( 0, channel_request,	"42: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0)

BS_MSG3_SEND	( 0, immediate_assignment,	"43: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"44: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"45" )

BS_MSG3_AWAIT	( 0, authentication_response,	"46: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"47: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"48: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"49: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"50: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"51: The MS shall respond to the SABM" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"52: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"53: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"54" )

NOT_IMPLEMENTED	(	"55: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"56: Contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"57: First CP-DATA message not acknowledged by SS" )

ISS_DELAY (5000)

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"58: Retransmitted CP-DATA from MS within twice TC1M after step 56, contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"59: Retransmitted CP-DATA message not acknowledged by SS" )

NOT_IMPLEMENTED	(	"60: Depending upon the maximum number of CP-DATA retransmissions implemented, step 58 and 59 may be repeated." )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"61: The main signalling link is released after a duration of TC1M + 5 seconds after the last CP-DATA retransmission.." )

ISS_DELAY (5000)

NOT_IMPLEMENTED	(	"62: The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed" )

COMMAND	( "MMI CONFIG AT+CMGR=1" )

ISS_DELAY (5000)

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY (2000)

COMMAND ("MMI CONFIG KEY_SEQUENCE=03039094117")

BS_RACH_AWAIT	( 0,channel_request_moc,	SILENT)

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0);

BS_MSG3_SEND	( 0,immediate_assignment,	SILENT)

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, cm_service_request,	SILENT)

BS_MSG3_SEND	( 0, ciphering_mode_command,	SILENT)

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	SILENT)

BS_MSG3_AWAIT	( 0, setup_moc,	SILENT)

BS_MSG3_SEND	( 0, call_proceeding,	SILENT)

BS_MSG3_SEND	( 0, alerting,	SILENT)

BS_MSG3_SEND	( 0, assignment_command,	SILENT)

BS_CONFIG_CHANNEL	( 0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, assignment_complete,	SILENT)

BS_MSG3_SEND	( 0, connect,	SILENT)

BS_MSG3_AWAIT	( 0, connect_acknowledge,	SILENT)

NOT_IMPLEMENTED	(	"63: A data or speech call is established on a TCH and the state U10 of call control is entered." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"64: Sent on SACCH associated with TCH" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"65: The MS shall respond to the SABM" )

*/

BS_CONFIG_CHANNEL	( 0, SACCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"66: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"67: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"68" )

NOT_IMPLEMENTED	(	"69: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"70: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"71: Within TC1M after step 70, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, FACCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"72: The main signalling link is released." )

ISS_DELAY (5000)

NOT_IMPLEMENTED	(	"73: The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed" )

NOT_IMPLEMENTED	(	"74: Clear the SMS message store" )

COMMAND	( "MMI CONFIG AT+CMGR=1" )

ISS_DELAY (5000)

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY (2000)

COMMAND ("MMI CONFIG KEY_SEQUENCE=03039094117")

BS_RACH_AWAIT	( 0,channel_request_moc,	SILENT)

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0);

BS_MSG3_SEND	( 0,immediate_assignment,	SILENT)

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, cm_service_request,	SILENT)

BS_MSG3_SEND	( 0, ciphering_mode_command,	SILENT)

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	SILENT)

BS_MSG3_AWAIT	( 0, setup_moc,	SILENT)

BS_MSG3_SEND	( 0, call_proceeding,	SILENT)

BS_MSG3_SEND	( 0, alerting,	SILENT)

BS_MSG3_SEND	( 0, assignment_command,	SILENT)

BS_CONFIG_CHANNEL	( 0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, assignment_complete,	SILENT)

BS_MSG3_SEND	( 0, connect,	SILENT)

BS_MSG3_AWAIT	( 0, connect_acknowledge,	SILENT)

NOT_IMPLEMENTED	(	"75: A data or speech call is established on a TCH and the state U10 of call control is entered." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"76: Sent on SACCH associated with TCH" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"77: The MS shall respond to the SABM" )

*/

BS_CONFIG_CHANNEL	( 0, SACCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"78: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"79: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"80" )

NOT_IMPLEMENTED	(	"81: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"82: First CP-DATA from MS, contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"83: First CP-DATA message not acknowledged by SS" )

ISS_DELAY (5000)

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"84: Retransmitted CP-DATA message within twice TC1M after step 82, contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"85: Second CP-DATA message is acknowledged within TC1M after step 84, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, FACCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"86: The main signalling link is released." )

ISS_DELAY (5000)

NOT_IMPLEMENTED	(	"87: The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed" )

NOT_IMPLEMENTED	(	"88: Clear the SMS message store" )

COMMAND	( "MMI CONFIG AT+CMGR=1" )

ISS_DELAY (5000)

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY (2000)

COMMAND ("MMI CONFIG KEY_SEQUENCE=03039094117")

BS_RACH_AWAIT	( 0,channel_request_moc,	SILENT)

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0);

BS_MSG3_SEND	( 0,immediate_assignment,	SILENT)

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, cm_service_request,	SILENT)

BS_MSG3_SEND	( 0, ciphering_mode_command,	SILENT)

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	SILENT)

BS_MSG3_AWAIT	( 0, setup_moc,	SILENT)

BS_MSG3_SEND	( 0, call_proceeding,	SILENT)

BS_MSG3_SEND	( 0, alerting,	SILENT)

BS_MSG3_SEND	( 0, assignment_command,	SILENT)

BS_CONFIG_CHANNEL	( 0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, assignment_complete,	SILENT)

BS_MSG3_SEND	( 0, connect,	SILENT)

BS_MSG3_AWAIT	( 0, connect_acknowledge,	SILENT)

NOT_IMPLEMENTED	(	"89: A data or speech call is established on a TCH and the state U10 of call control is entered." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"90: Sent on SACCH associated with TCH" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"91: The MS shall respond to the SABM" )

*/

BS_CONFIG_CHANNEL	( 0, SACCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"92: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"93: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"94" )

NOT_IMPLEMENTED	(	"95: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"96: First CP-DATA from MS, contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"97: First CP-DATA message not acknowledged by SS" )

ISS_DELAY (5000)

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"98: Transmitted CP-DATA message within twice TC1M after step 96, contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"99: Retransmitted CP-DATA message not acknowledged by SS" )

NOT_IMPLEMENTED	(	"100: Depending on the maximum number of CP-DATA retransmissions implemented, step 98-99 may be repeated. The maximum number of retransmissions may however not exceed three." )

BS_CONFIG_CHANNEL	( 0, FACCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"101: The main signalling link is released after a duration of TC1M + 15 seconds after the last CP-DATA retransmission." )

ISS_DELAY (5000)

NOT_IMPLEMENTED	(	"102: The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed" )

NOT_IMPLEMENTED	(	"103: Clear the SMS message store" )

COMMAND	( "MMI CONFIG AT+CMGR=1" )

ISS_DELAY (5000)

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY (2000)

COMMAND ("MMI CONFIG KEY_SEQUENCE=03039094117")

BS_RACH_AWAIT	( 0,channel_request_moc,	SILENT)

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0);

BS_MSG3_SEND	( 0,immediate_assignment,	SILENT)

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, cm_service_request,	SILENT)

BS_MSG3_SEND	( 0, ciphering_mode_command,	SILENT)

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	SILENT)

BS_MSG3_AWAIT	( 0, setup_moc,	SILENT)

BS_MSG3_SEND	( 0, call_proceeding,	SILENT)

BS_MSG3_SEND	( 0, alerting,	SILENT)

BS_MSG3_SEND	( 0, assignment_command,	SILENT)

BS_CONFIG_CHANNEL	( 0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, assignment_complete,	SILENT)

BS_MSG3_SEND	( 0, connect,	SILENT)

BS_MSG3_AWAIT	( 0, connect_acknowledge,	SILENT)

NOT_IMPLEMENTED	(	"104: A data or speech call is established on a TCH and the state U10 of call control is entered." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"105: Sent on SACCH associated with TCH" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"106: The MS shall respond to the SABM" )

*/

BS_CONFIG_CHANNEL	( 0, FACCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, disconnect,	"107: The speech call is cleared by the SS. The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS." )

BS_MSG3_AWAIT	( 0, release,	SILENT)

BS_CONFIG_CHANNEL	( 0, SACCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"108: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"109: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"110" )

NOT_IMPLEMENTED	(	"111: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"112: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"113: Within TC1M after step 112, no further CP-DATA" )

BS_CONFIG_CHANNEL	( 0, FACCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"114: The main signalling link is released." )

ISS_DELAY (5000)

NOT_IMPLEMENTED	(	"115: The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed" )

NOT_IMPLEMENTED	(	"116: Clear the SMS message store" )

COMMAND	( "MMI CONFIG AT+CMGR=1" )

ISS_DELAY (5000)

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY (2000)

COMMAND ("MMI CONFIG KEY_SEQUENCE=03039094117")

BS_RACH_AWAIT	( 0,channel_request_moc,	SILENT)

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0);

BS_MSG3_SEND	( 0,immediate_assignment,	SILENT)

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, cm_service_request,	SILENT)

BS_MSG3_SEND	( 0, ciphering_mode_command,	SILENT)

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	SILENT)

BS_MSG3_AWAIT	( 0, setup_moc,	SILENT)

BS_MSG3_SEND	( 0, call_proceeding,	SILENT)

BS_MSG3_SEND	( 0, alerting,	SILENT)

BS_MSG3_SEND	( 0, assignment_command,	SILENT)

BS_CONFIG_CHANNEL	( 0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, assignment_complete,	SILENT)

BS_MSG3_SEND	( 0, connect,	SILENT)

BS_MSG3_AWAIT	( 0, connect_acknowledge,	SILENT)

NOT_IMPLEMENTED	(	"117: A data or speech call is established on a TCH and the state U10 of call control is entered." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"118: Sent on SACCH associated with TCH" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"119: The MS shall respond to the SABM" )

*/

NOT_IMPLEMENTED	(	"120: Waits 15 seconds" )

COMMAND ("MMI CONFIG KEY_SEQUENCE=<#*43*1#>");          /* Hook On */

BS_MSG3_AWAIT	( 0, disconnect_ms,	SILENT);

NOT_IMPLEMENTED	(	"121: During the SS wait time in step 120 the speech call shall be cleared from the MS. The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS." )

BS_MSG3_SEND	( 0, release_ms,	SILENT)

BS_MSG3_AWAIT	( 0, release_complete_ms,	SILENT)

BS_CONFIG_CHANNEL	( 0, SACCH, ACK, SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"122: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"123: Waits max 25 seconds for CP-ACK" )

BS_CONFIG_CHANNEL	( 0, SACCH, ACK, SAPI_3)

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"124" )

NOT_IMPLEMENTED	(	"125: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"126: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"127: Within TC1M after step 126, no further CP-DATA" )

BS_CONFIG_CHANNEL	( 0, FACCH, ACK, SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"128: The main signalling link is released." )

NOT_IMPLEMENTED	(	"129: The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed" )

NOT_IMPLEMENTED	(	"130: Clear the SMS message store" )

COMMAND	( "MMI CONFIG AT+CMGR=1" )

ISS_DELAY (5000)

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY (2000)

History:	26.01.98	MS	Initial

### MSMS002: SMS mobile originated (34.2.2)

Description:	To verify that the MS is able to correctly send a short message where the SMS is provided for the point to point service. The test also verifies that the MS is capable of simultaneously receive a network originated SM whilst sending a mobile originated SM

Preamble:	MSMS000

Script:

COMMAND ("MMI CONFIG AT+CSCA=12345678*145")

COMMAND ("MMI CONFIG AT+CSMP=18*03*0*0")

COMMAND ("MMI CONFIG AT+CMGS=5678*145*HALLIHALLO")

BS_RACH_AWAIT	( 0, channel_request_111,	"1: Establishment cause is Other procedures which can be completed with an SDDCH” NECI = 0" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"2: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0);

BS_MSG3_AWAIT	( 0, cm_service_request,	"3: Message is contained in SABM on SAPI 0. CM service type set to Short message transfer" )

BS_MSG3_SEND	( 0, authentication_request,	"4" )

BS_MSG3_AWAIT	( 0, authentication_response,	"5: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"6: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"7: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"8: SS starts ciphering." )

NOT_IMPLEMENTED	(	"9: MS establishes SAPI 3" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_3);

BS_MSG3_AWAIT	( 0, cp_data_sms_submit,	"11: Contains RP-DATA RPDU (SMS SUBMIT TPDU)" )

BS_MSG3_SEND	( 0, cp_ack_to_ms_mo,	"12: Sent within TC1M after step 11" )

BS_MSG3_SEND	( 0, cp_data_rp_ack_to_ms_mo,	"13: Contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"14: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs_mo,	"15" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0);

BS_MSG3_SEND	( 0, channel_release,	"16: The main signalling link is released." )

ISS_DELAY (10000)

COMMAND ("MMI CONFIG AT+CMGS=5678*145*HALLIHALLO")

BS_RACH_AWAIT	( 0, channel_request_111,	"18: Establishment cause is Other procedures which can be completed with an SDDCH” (NECI=0)" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0);

BS_STORE_RACH_PARAMS (0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"19: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0);

BS_MSG3_AWAIT	( 0, cm_service_request,	"20: Message is contained in SABM on SAPI 0." )

BS_MSG3_SEND	( 0, authentication_request,	"21" )

BS_MSG3_AWAIT	( 0, authentication_response,	"22: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"23: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"24: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"25: SS starts ciphering." )

NOT_IMPLEMENTED	(	"26: MS establishes SAPI 3" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_3);

BS_MSG3_AWAIT_BEGIN	( 0, cp_data_sms_submit,	"28: Contains RP-DATA RPDU (SMS SUBMIT TPDU)" )

BF_SET_VAL	(msg_ref, 1, SILENT)

BF_SET_VAL	(tp_msg_ref,1 , SILENT)

BS_MSG3_AWAIT_END	()

NOT_IMPLEMENTED	(	"29: SS configured not to send CP-ACK" )

BS_MSG3_AWAIT_BEGIN	( 0, cp_data_sms_submit,	"30: Retransmitted CP-DATA message within twice TC1M after step 28" )

BF_SET_VAL	(msg_ref, 1, SILENT)

BF_SET_VAL	(tp_msg_ref, 1, SILENT)

BS_MSG3_AWAIT_END	()

NOT_IMPLEMENTED	(	"31: Depending on the maximum number of CP-DATA retransmissions implemented, step 30 may be repeated. The maximum number of retransmissions may however not exceed three." )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0);

BS_MSG3_SEND	( 0, channel_release,	"32: The main signalling link is released after a duration of TC1M + 5 seconds after the last CP-DATA retransmission." )

ISS_DELAY (10000)

COMMAND ("MMI CONFIG AT+CMGS=5678*145*HALLIHALLO")

BS_RACH_AWAIT	( 0, channel_request_111,	"33: Establishment cause is Other procedures which can be completed with an SDDCH” (NECI=0)" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0);

BS_STORE_RACH_PARAMS (0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"34: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0);

BS_MSG3_AWAIT	( 0, cm_service_request,	"35: Message is contained in SABM. CM service type set to short message transfer" )

BS_MSG3_SEND	( 0, authentication_request,	"36" )

BS_MSG3_AWAIT	( 0, authentication_response,	"37: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"38: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"39: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	" 40	SS starts ciphering." )

NOT_IMPLEMENTED	(	"41: MS establishes SAPI 3" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_3);

BS_MSG3_AWAIT_BEGIN	( 0, cp_data_sms_submit,	"43: Contains RP-DATA RPDU (SMS SUBMIT TPDU)" )

BF_SET_VAL	(msg_ref, 2, SILENT)

BF_SET_VAL	(tp_msg_ref,2 , SILENT)

BS_MSG3_AWAIT_END	()

BS_MSG3_SEND	( 0, cp_error_to_ms_mo,	"44: Sent within TC1M containing Network Failure cause." )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0);

BS_MSG3_SEND	( 0, channel_release,	"45: The main signalling link is released." )

ISS_DELAY (10000);

COMMAND ("MMI CONFIG KEY_SEQUENCE=03039094117")

BS_RACH_AWAIT	( 0,channel_request_moc,	SILENT)

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS (0, 0);

BS_MSG3_SEND	( 0,immediate_assignment,	SILENT)

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, cm_service_request,	SILENT)

BS_MSG3_SEND	( 0, ciphering_mode_command,	SILENT)

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	SILENT)

BS_MSG3_AWAIT	( 0, setup_moc,	SILENT)

BS_MSG3_SEND	( 0, call_proceeding,	SILENT)

BS_MSG3_SEND	( 0, alerting,	SILENT)

BS_MSG3_SEND	( 0, assignment_command,	SILENT)

BS_CONFIG_CHANNEL	( 0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, assignment_complete,	SILENT)

BS_MSG3_SEND	( 0, connect,	SILENT)

BS_MSG3_AWAIT	( 0, connect_acknowledge,	SILENT)

NOT_IMPLEMENTED	(	"46: A data or speech call is established on a TCH and the state U10 of call control is entered." )

NOT_IMPLEMENTED	(	"47: The MS is set up to send an SM" )

COMMAND ("MMI CONFIG AT+CMGS=5678*145*HALLIHALLO")

BS_MSG3_AWAIT	( 0, cm_service_request,	"48: Sent in a layer 2 frame on the FACCH. CM service type set to short message transfer" )

BS_MSG3_SEND	( 0, cm_service_accept,	"49" )

NOT_IMPLEMENTED	(	"50: MS establishes SAPI 3" )

BS_CONFIG_CHANNEL (0, FACCH, ACK, SAPI_3);

BS_MSG3_AWAIT_BEGIN	( 0, cp_data_sms_submit,	"52: Contains RP-DATA RPDU (SMS SUBMIT TPDU)" )

BF_SET_VAL	(msg_ref, 3, SILENT)

BF_SET_VAL	(tp_msg_ref,3 , SILENT)

BS_MSG3_AWAIT_END	()

BS_MSG3_SEND	( 0, cp_ack_to_ms_mo,	"53: Sent within TC1M after step 52" )

BS_MSG3_SEND	( 0, cp_data_rp_ack_to_ms_mo,	"54: Contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"55: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs_mo,	"56" )

BS_CONFIG_CHANNEL (0, FACCH, ACK, SAPI_0);

BS_MSG3_SEND	( 0, channel_release,	"57: The main signalling link is released." )

ISS_DELAY (10000);

COMMAND ("MMI CONFIG KEY_SEQUENCE=03039094117")

BS_RACH_AWAIT	( 0, channel_request_moc,	SILENT)

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK, SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0)

BS_MSG3_SEND	( 0, immediate_assignment,	SILENT)

BS_CONFIG_CHANNEL (0, SDCCH, 1, SAPI_0)

BS_MSG3_AWAIT	( 0, cm_service_request,	SILENT)

BS_MSG3_SEND	( 0, ciphering_mode_command,	SILENT)

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	SILENT)

BS_MSG3_AWAIT	( 0, setup_moc,	SILENT)

BS_MSG3_SEND	( 0, call_proceeding,	SILENT)

BS_MSG3_SEND	( 0, alerting,	SILENT)

BS_MSG3_SEND	( 0, assignment_command,	SILENT)

BS_CONFIG_CHANNEL ( 0, FACCH, ACK, SAPI_0)

BS_MSG3_AWAIT	( 0, assignment_complete,	SILENT)

BS_MSG3_SEND	( 0, connect,	SILENT)

BS_MSG3_AWAIT	( 0, connect_acknowledge,	SILENT)

NOT_IMPLEMENTED	(	"59: A data or speech call is established on a TCH and the state U10 of call control is entered." )

COMMAND ("MMI CONFIG AT+CMGS=5678*145*HALLIHALLO")

BS_MSG3_AWAIT	( 0, cm_service_request,	"60: Sent in a layer 2 frame on the FACCH. CM service type set to short message transfer" )

BS_MSG3_SEND	( 0, cm_service_accept,	"61" )

NOT_IMPLEMENTED	(	"62: MS establishes SAPI 3" )

BS_CONFIG_CHANNEL (0, FACCH, ACK, SAPI_3);

BS_MSG3_AWAIT_BEGIN	( 0, cp_data_sms_submit,	"64: Contains RP-DATA RPDU (SMS SUBMIT TPDU)" )

BF_SET_VAL	(msg_ref, 4, SILENT)

BF_SET_VAL	(tp_msg_ref,4 , SILENT)

BS_MSG3_AWAIT_END	()

NOT_IMPLEMENTED	(	"65: SS configured not to send CP-ACK" )

BS_MSG3_AWAIT_BEGIN	( 0, cp_data_sms_submit,	"66: Transmitted CP-DATA message within twice TC1M after step 64" )

BF_SET_VAL	(msg_ref, 4, SILENT)

BF_SET_VAL	(tp_msg_ref,4 , SILENT)

BS_MSG3_AWAIT_END	()

NOT_IMPLEMENTED	(	"67: Depending on the maximum number of CP-DATA retransmissions implemented, step 66 may be repeated. The maximum number of retransmissions may however not exceed three." )

BS_CONFIG_CHANNEL (0, FACCH, ACK, SAPI_0);

BS_MSG3_SEND	( 0, channel_release,	"68: The main signalling link is released after a duration of TC1m + 15 seconds after the last CP-DATA retransmission." )

ISS_DELAY (10000)

COMMAND ("MMI CONFIG AT+CMGS=5678*145*HALLIHALLO")

BS_RACH_AWAIT	( 0, channel_request_111,	"70: Establishment cause is Other procedures which can be completed with an SDDCH” (NECI=0)" )

BS_STORE_RACH_PARAMS ( 0, 0)

BS_CONFIG_CHANNEL (0, AGCH, UNACK, SAPI_0);

BS_MSG3_SEND	( 0, immediate_assignment,	"71: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL (0, SDCCH, ACK, SAPI_0);

BS_MSG3_AWAIT	( 0, cm_service_request,	"72: Message is contained in SABM. CM service type set to short message transfer" )

BS_MSG3_SEND	( 0, authentication_request,	"73" )

BS_MSG3_AWAIT	( 0, authentication_response,	"74: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"75: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"76: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"77: SS starts ciphering." )

NOT_IMPLEMENTED	(	"62: MS establishes SAPI 3" )

BS_CONFIG_CHANNEL (0, SDCCH, ACK, SAPI_3);

BS_MSG3_AWAIT_BEGIN	( 0, cp_data_sms_submit,	"80: Contains RP-DATA RPDU (SMS SUBMIT TPDU)" )

BF_SET_VAL	(msg_ref, 5, SILENT)

BF_SET_VAL	(tp_msg_ref,5 , SILENT)

BS_MSG3_AWAIT_END	()

NOT_IMPLEMENTED	(	"81: The SS sends an SM to the MS triggered by the end of the CP-DATA message from the MS" )

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"82: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"83: !!!!!!!! The MS shall correctly receive the SM and indicate that a message has arrived." )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	SILENT)

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	SILENT)

BS_MSG3_SEND	( 0, cp_ack_to_ms_mo,	SILENT)

BS_MSG3_SEND	( 0, cp_ack_to_ms,	SILENT)

BS_CONFIG_CHANNEL (0, SDCCH, ACK, SAPI_0);

BS_MSG3_SEND	( 0, channel_release,	SILENT)

ISS_DELAY (10000)

COMMAND ("MMI CONFIG AT+CMGS=5678*145*HALLIHALLO")

BS_RACH_AWAIT	( 0, channel_request_111,	"84: Establishment cause is Other procedures which can be completed with an SDDCH” (NECI=0)" )

BS_STORE_RACH_PARAMS ( 0, 0)

BS_CONFIG_CHANNEL (0, AGCH, UNACK, SAPI_0);

BS_MSG3_SEND	( 0, immediate_assignment,	"85: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL (0, SDCCH, ACK, SAPI_0);

BS_MSG3_AWAIT	( 0, cm_service_request,	"86: Message is contained in SABM. CM service type set to short message transfer" )

BS_MSG3_SEND	( 0, cm_service_reject,	"87: Reject cause set to Service Option not supported or Service Option temporarily out of order" )

NOT_IMPLEMENTED	(	"88: The MS shall not establish SAPI-3" )

BS_MSG3_SEND	( 0, channel_release,	"89: Sent 5 seconds after CM SERVICE REJ" )

History:	26.01.98	MS	Initial

### MSMS003: Test of memory full condition and memory available notification (34.2.3)

Description:	1.  To verify that the MS sends the correct acknowledgement when its memory in the SIM becomes full.  2.  To verify that the MS sends the correct acknowledgement when its memory in the ME and the SIM becomes full, and sets the "memory exceeded" notification flag in the SIM.  3.  To verify that the MS performs the "memory available" procedure when its message store becomes available for receiving short messages, and only at this moment.

Preamble:	MSMS000

Script:

COMMAND	( "MMI CONFIG AT+CPMS=1" )

COMMAND	( "MMI CONFIG AT+CNMI=1" )

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11: MS shall respond to SABM in step 10" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"13: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"14" )

NOT_IMPLEMENTED	(	"15: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"16: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"17: Within TC1M after step 16, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"18: The main signalling link is released." )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11: MS shall respond to SABM in step 10" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"13: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"14" )

NOT_IMPLEMENTED	(	"15: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"16: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"17: Within TC1M after step 16, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"18: The main signalling link is released." )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11: MS shall respond to SABM in step 10" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"13: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"14" )

NOT_IMPLEMENTED	(	"15: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"16: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"17: Within TC1M after step 16, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"18: The main signalling link is released." )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11: MS shall respond to SABM in step 10" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"13: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"14" )

NOT_IMPLEMENTED	(	"15: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_error_111_to_bs,	"16: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"17: Within TC1M after step 16, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"18: The main signalling link is released." )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"19" )

BS_RACH_AWAIT	( 0, channel_request,	"20: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"21: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"22: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"23" )

BS_MSG3_AWAIT	( 0, authentication_response,	"24: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"25: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"26: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"27: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"28: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"29: MS shall respond to SABM in step 10" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver_class1,	"30: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"31: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"32" )

NOT_IMPLEMENTED	(	"33: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"34: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"35: Within TC1M after step 16, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"36: The main signalling link is released." )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"19" )

BS_RACH_AWAIT	( 0, channel_request,	"20: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"21: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"22: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"23" )

BS_MSG3_AWAIT	( 0, authentication_response,	"24: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"25: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"26: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"27: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"28: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"29: MS shall respond to SABM in step 10" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver_class1,	"30: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"31: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"32" )

NOT_IMPLEMENTED	(	"33: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"34: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"35: Within TC1M after step 16, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"36: The main signalling link is released." )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"19" )

BS_RACH_AWAIT	( 0, channel_request,	"20: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"21: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"22: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"23" )

BS_MSG3_AWAIT	( 0, authentication_response,	"24: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"25: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"26: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"27: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"28: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"29: MS shall respond to SABM in step 10" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver_class1,	"30: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"31: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"32" )

NOT_IMPLEMENTED	(	"33: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_error_22_to_bs,	"34: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"35: Within TC1M after step 16, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"36: The main signalling link is released." )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"37" )

BS_RACH_AWAIT	( 0, channel_request,	"38: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"39: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"40: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"41" )

BS_MSG3_AWAIT	( 0, authentication_response,	"42: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"43: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"44: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"45: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"46: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"47: MS shall respond to SABM in step 10" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver_def,	"48: Contains RP-DATA RPDU (SMS DELIVER TPDU)" )

NOT_IMPLEMENTED	(	"49: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"50" )

NOT_IMPLEMENTED	(	"51: Waits max 60 seconds for RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_data_rp_error_22_to_bs,	"52: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"53: Within TC1M after step 16, no further CP-DATA messages" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"54: The main signalling link is released." )

ISS_DELAY (10000)

NOT_IMPLEMENTED	(	"55: Delete one message from SIM")

COMMAND	( "MMI CONFIG AT+CPMS=3" )

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY (2000)

BS_RACH_AWAIT	( 0, channel_request_111,	"57: Establishment cause is Other procedures which can be completed with an SDDCH” NECI = 0" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"58: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0);

BS_MSG3_AWAIT	( 0, cm_service_request,	"59: Message is contained in SABM on SAPI 0. CM service type set to Short message transfer" )

BS_MSG3_SEND	( 0, cm_service_accept,	"60" )

NOT_IMPLEMENTED	(	"61: MS establishes SAPI 3" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_3);

BS_MSG3_AWAIT	( 0, cp_data_rp_smma_to_bs,	"63: Contains RP-DATA RPDU (SMS SUBMIT TPDU)" )

BS_MSG3_SEND	( 0, cp_ack_to_ms_mo,	"64: Sent within TC1M after step 11" )

BS_MSG3_SEND	( 0, cp_data_rp_ack_to_ms_mo,	"65: Contains RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs_mo,	"66" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK, SAPI_0);

BS_MSG3_SEND	( 0, channel_release,	"67: The main signalling link is released." )

ISS_DELAY (10000)

NOT_IMPLEMENTED	(	"68: Delete one message from SIM")

COMMAND	( "MMI CONFIG AT+CMGD=1" )

BS_RACH_EXPECT_TIMEOUT (1, 10000)

History:	26.01.98	MS	Initial

### MSMS004: Test of the status report capabilities and of SMS-COMMAND (34.2.4)

Description:	1)To verify that the MS is able to accept a SMS-STATUS-REPORT TPDU. 2)	To verify that the MS is able to use the SMS-COMMAND functionality correctly and sends an SMS-COMMAND TPDU with the correct TP-Message-Reference.

Preamble:	MSMS000

Script:

BS_RACH_AWAIT	( 0, channel_request,	"1: Establishment cause is Other procedures which can be completed with an SDCCH  (NECI=0)" )

BS_MSG3_SEND	( 0, immediate_assignment,	"2: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, cm_service_request,	"3: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"4" )

BS_MSG3_AWAIT	( 0, authentication_response,	"5: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"6: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"7: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"8: SS starts ciphering." )

BS_MSG3_AWAIT	( 0, sabm_(sapi=3),	"9: MS establishes SAPI 3" )

BS_MSG3_SEND	( 0, ua_(sapi=3),	"10" )

BS_MSG3_AWAIT	( 0, cp-data,	"11: Contains RP-DATA RPDU (SMS SUBMIT TPDU)" )

BS_MSG3_SEND	( 0, cp-ack,	"12: Sent within TC1M after step 11" )

BS_MSG3_SEND	( 0, cp-data,	"13: Contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"14: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp-ack,	"15" )

BS_MSG3_SEND	( 0, channel_release,	"16: The main signalling link is released." )

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"17" )

BS_RACH_AWAIT	( 0, channel_request,	"18: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"19: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"20: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"22" )

BS_MSG3_AWAIT	( 0, authentication_response,	"23: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"24: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"25: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"26: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"27: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"28" )

BS_MSG3_SEND	( 0, cp-data,	"29: Contains RP-DATA RPDU (SMS-STATUS-REPORT TPDU)" )

BS_MSG3_AWAIT	( 0, cp-ack,	"30" )

BS_MSG3_AWAIT	( 0, cp-data,	"31: Contains RP-ACK RPDU" )

BS_MSG3_SEND	( 0, cp-ack,	"32" )

BS_MSG3_SEND	( 0, channel_release,	"33" )

NOT_IMPLEMENTED	(	"34: The MS is made to send an SMS-COMMAND message enquiring about the previously submitted SM" )

BS_RACH_AWAIT	( 0, channel_request,	"35: Establishment cause Other services which can be completed with an SDCCH." )

BS_MSG3_SEND	( 0, immediate_assignment,	"36: 	SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, cm_service_request,	"37: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"38" )

BS_MSG3_AWAIT	( 0, authentication_response,	"39: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"40: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"41: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"42: SS starts ciphering." )

BS_MSG3_AWAIT	( 0, sabm_(sapi=3),	"43: MS establishes SAPI 3" )

BS_MSG3_SEND	( 0, ua_(sapi=3),	"44" )

BS_MSG3_AWAIT	( 0, cp-data,	"45: Contains RP-DATA RPDU (SMS-COMMAND TPDU) which shall contain the correct TP-MR" )

BS_MSG3_SEND	( 0, cp-ack,	"46" )

BS_MSG3_SEND	( 0, cp-data,	"47: Contains RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp-ack,	"48" )

BS_MSG3_SEND	( 0, channel_release,	"49" )

NOT_IMPLEMENTED	( 0, the_ms_is_made_to_send_an_sms-command	"50:  message requiring to delete the previously submitted SM." )

BS_RACH_AWAIT	( 0, channel_request,	"51: Establishment cause Other services which can be completed with an SDCCH." )

BS_MSG3_SEND	( 0, immediate_assignment,	"52: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, cm_service_request,	"53: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"54" )

BS_MSG3_AWAIT	( 0, authentication_response,	"55: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"56: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"57: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"58: SS starts ciphering." )

BS_MSG3_AWAIT	( 0, sabm_(sapi=3),	"59: MS establishes SAPI 3" )

BS_MSG3_SEND	( 0, ua_(sapi=3),	"60" )

BS_MSG3_AWAIT	( 0, cp-data,	"61: Contains RP-DATA RPDU (SMS-COMMAND TPDU) which shall contain the correct TP-MR" )

BS_MSG3_SEND	( 0, cp-ack,	"62" )

BS_MSG3_SEND	( 0, cp-data,	"63: Contains RP-ACK RPDU" )

BS_MSG3_AWAIT	( 0, cp-ack,	"64" )

BS_MSG3_SEND	( 0, channel_release,	"65" )

History:	26.01.98	MS	Initial

## Test of message class 0 to 3

### MSMS005: Short message class 0 (34.2.5.1)

Description:	To verify that the MS will accept and display but not store a class 0 message, and that it will accept and display a class 0 message if its message store is full. NOTE: failure of this test in a mobile could cause it to reject a class 0 message when its SMS memory becomes full. This could lead to unwanted repetitions between the MS and the service centre.

Preamble:	MSMS000

Script:

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	28: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11" )

BS_MSG3_SEND	( 0, cp-data,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 0 Short Message" )

BS_MSG3_AWAIT	( 0, cp-ack,	"13" )

BS_MSG3_AWAIT	( 0, cp-data,	"14: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"15" )

BS_MSG3_SEND	( 0, channel_release,	"16" )

NOT_IMPLEMENTED	(	"17: The content of the short message shall be displayed by the ME. The MS shall not store the message. This can be checked by verifying that it is impossible to retrieve any short messages from the MS message store." )

NOT_IMPLEMENTED	(	"18: The MS message store shall be filled (for example by using the method of 34.2.3) with Class 1 SMS-DELIVER TPDU." )

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"19" )

BS_RACH_AWAIT	( 0, channel_request,	"20: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"21: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"22: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"23" )

BS_MSG3_AWAIT	( 0, authentication_response,	"24: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"25: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"26: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"27: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"28: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"29" )

BS_MSG3_SEND	( 0, cp-data,	"30: Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 0 Short Message" )

BS_MSG3_AWAIT	( 0, cp-ack,	"31" )

BS_MSG3_AWAIT	( 0, cp-data,	"32: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"33" )

BS_MSG3_SEND	( 0, channel_release,	"34" )

NOT_IMPLEMENTED	(	"35: The content of the short message shall be displayed by the ME." )

History:	26.01.98	MS	Initial

### MSMS006: Test of class 1 short messages (34.2.5.2)

Description:	This procedure verifies that the MS acts correctly on receiving a class 1 message, i.e. that it stores the message in the ME or SIM and sends an acknowledgement (at RP and CP-Layer).

Preamble:	MSMS

Script:

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11" )

BS_MSG3_SEND	( 0, cp-data,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 1 Short Message" )

BS_MSG3_AWAIT	( 0, cp-ack,	"13" )

BS_MSG3_AWAIT	( 0, cp-data,	"14: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"15" )

BS_MSG3_SEND	( 0, channel_release,	"16" )

NOT_IMPLEMENTED	(	"17: The short message shall be recalled and displayed at the MS." )

History:	26.01.98	MS	Initial

### MSMS007: Test of class 2 short messages (34.2.5.3)

Description:	This procedure verifies that the MS acts correctly on receiving a class 2 message, i.e. that it stores the message correctly in the SIM, and if this is not possible, returns a protocol error message, with the correct error cause, to the network. There are 2 cases: 1) If the MS supports storing of short messages in the SIM and in the ME, and storage in the ME is not full, and the short message cannot be stored in the SIM, the error cause shall be "protocol error, unspecified". 2) If the MS supports storing of short messages in the SIM and not in the ME, and storage in the ME is not full, and the short message cannot be stored in the SIM, the error cause shall be "memory capacity exceeded". NOTE:	If the MS supports storing of short messages in the SIM and the ME, and storage in the ME is full, and the short message cannot be stored in the SIM, the error cause shall be "memory capacity exceeded". This case is not tested in this test.

Preamble:	MSMS000

Script:

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 2 Short Message" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"13" )

/*NOT_IMPLEMENTED	(	"14: The ME shall correctly store the short message in a free record of EFSMS in the SIM, i.e. -	the ME shall use a free record -	the first byte of the record shall indicate message received by MS from network -	the TS-Service-Centre-Address shall be correctly stored -	the TPDU shall be identical to that sent by the SS -	bytes following the TPDU shall be set to FF" )

NOT_IMPLEMENTED	(	"15: The SIM simulator returns the status response OK (90 00). The SIM simulator shall indicate if an attempt was made by the ME to store the short message in the SIM." )*/

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"16: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"16A" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"17" )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 2 Short Message" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"13" )

/*NOT_IMPLEMENTED	(	"14: The ME shall correctly store the short message in a free record of EFSMS in the SIM, i.e. -	the ME shall use a free record -	the first byte of the record shall indicate message received by MS from network -	the TS-Service-Centre-Address shall be correctly stored -	the TPDU shall be identical to that sent by the SS -	bytes following the TPDU shall be set to FF" )

NOT_IMPLEMENTED	(	"15: The SIM simulator returns the status response OK (90 00). The SIM simulator shall indicate if an attempt was made by the ME to store the short message in the SIM." )*/

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"16: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"16A" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"17" )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 2 Short Message" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"13" )

/*NOT_IMPLEMENTED	(	"14: The ME shall correctly store the short message in a free record of EFSMS in the SIM, i.e. -	the ME shall use a free record -	the first byte of the record shall indicate message received by MS from network -	the TS-Service-Centre-Address shall be correctly stored -	the TPDU shall be identical to that sent by the SS -	bytes following the TPDU shall be set to FF" )

NOT_IMPLEMENTED	(	"15: The SIM simulator returns the status response OK (90 00). The SIM simulator shall indicate if an attempt was made by the ME to store the short message in the SIM." )*/

BS_MSG3_AWAIT	( 0, cp_data_rp_ack_to_bs,	"16: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"16A" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"17" )

ISS_DELAY (10000)

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"18" )

BS_RACH_AWAIT	( 0, channel_request,	"19: Establishment cause is Answer to paging" )

BS_CONFIG_CHANNEL	( 0, AGCH, UNACK,SAPI_0)

BS_STORE_RACH_PARAMS ( 0, 0);

BS_MSG3_SEND	( 0, immediate_assignment,	"20: SS assigns an SDCCH" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_AWAIT	( 0, paging_response,	"21: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"22" )

BS_MSG3_AWAIT	( 0, authentication_response,	"23: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"24: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"25: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"26: SS starts ciphering." )

/*

not implemented

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"27: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"28" )

*/

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_3)

BS_MSG3_SEND	( 0, cp_data_sms_deliver,	"29: Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 2 Short Message" )

BS_MSG3_AWAIT	( 0, cp_ack_to_bs,	"30" )

NOT_IMPLEMENTED	(	"31: The ME shall attempt to store the short message in a free record of EFSMS in the SIM." )

/*NOT_IMPLEMENTED	(	"32: The SIM simulator returns the status response memory problem (92 40). The SIM simulator shall indicate if an attempt was made by the ME to store the short message in the SIM." )*/

BS_MSG3_AWAIT	( 0, cp_data_rp_error_111_to_bs,	"33: Contains RP-ERROR RPDU with error cause protocol error, unspecified")

BS_MSG3_SEND	( 0, cp_ack_to_ms,	"33A" )

BS_CONFIG_CHANNEL	( 0, SDCCH, ACK,SAPI_0)

BS_MSG3_SEND	( 0, channel_release,	"34" )

History:	26.01.98	MS	Initial

## Short message service point to point

### MSMS008: Test of the replace mechanism for SM type 1-7 (34.2.7)

Description:	This procedure verifies the correct implementation of the replace mechanism for Replace Short Messages.

Preamble:	MSMS000

Script:

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11" )

BS_MSG3_SEND	( 0, cp-data,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is Replace Short Message Type n, TP-OA is TPOA1 and RP-OA is RPOA1" )

BS_MSG3_AWAIT	( 0, cp-ack,	"13" )

BS_MSG3_AWAIT	( 0, cp-data,	"14: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"14A" )

BS_MSG3_SEND	( 0, channel_release,	"15" )

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"16" )

BS_RACH_AWAIT	( 0, channel_request,	"17: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"18: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"19: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"20" )

BS_MSG3_AWAIT	( 0, authentication_response,	"21: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"22: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"23: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"24: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"25: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"26" )

BS_MSG3_SEND	( 0, cp-data,	"27: Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is Replace Short Message Type n, TP-OA is TPOA2 and RP-OA is RPOA1, TP-UD different from step 12" )

BS_MSG3_AWAIT	( 0, cp-ack,	"28" )

BS_MSG3_AWAIT	( 0, cp-data,	"29: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"29A" )

BS_MSG3_SEND	( 0, channel_release,	"30" )

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"31" )

BS_RACH_AWAIT	( 0, channel_request,	"32: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"33: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"34: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"35" )

BS_MSG3_AWAIT	( 0, authentication_response,	"36: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"37: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"38: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"39: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"40: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"41" )

BS_MSG3_SEND	( 0, cp-data,	"42: Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is Replace Short Message Type n, TP-OA is TPOA2 and RP-OA is RPOA2, TP-UD different from step 12 and 27" )

BS_MSG3_AWAIT	( 0, cp-ack,	"43" )

BS_MSG3_AWAIT	( 0, cp-data,	"44: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"44A" )

BS_MSG3_SEND	( 0, channel_release,	"45" )

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"46" )

BS_RACH_AWAIT	( 0, channel_request,	"47: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"48: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"49: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"50" )

BS_MSG3_AWAIT	( 0, authentication_response,	"51: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"52: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"53: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"54: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"55: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"56" )

BS_MSG3_SEND	( 0, cp-data,	"57: Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is Replace Short Message Type m, TP-OA is TPOA2 and RP-OA is RPOA2, TP-UD different from step 12, 27 and 42" )

BS_MSG3_AWAIT	( 0, cp-ack,	"58" )

BS_MSG3_AWAIT	( 0, cp-data,	"59: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"59A" )

BS_MSG3_SEND	( 0, channel_release,	"60" )

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"61" )

BS_RACH_AWAIT	( 0, channel_request,	"62: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"63: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"64: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"65" )

BS_MSG3_AWAIT	( 0, authentication_response,	"66: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"67: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"68: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"69: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"70: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"71" )

BS_MSG3_SEND	( 0, cp-data,	"72: Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is Replace Short Message Type m, TP-OA is TPOA2 and RP-OA is RPOA2, TP-UD different from step 57" )

BS_MSG3_AWAIT	( 0, cp-ack,	"73" )

BS_MSG3_AWAIT	( 0, cp-data,	"74: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"74A" )

BS_MSG3_SEND	( 0, channel_release,	"75" )

NOT_IMPLEMENTED	(	"76: Prompts the operator to display the Short Messages stored in the MS. Only the Short Messages delivered in step 12, 27, 42 and 72 shall be retrievable and displayed" )

History:	26.01.98	MS	Initial

### MSMS009: Test of the reply path scheme (34.2.8)

Description:	This procedure verifies that the MS is able to send a Reply Short Message back to the correct originating SME even if in the meantime it receives another Short Message.

Preamble:	MSMS000

Script:

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"1" )

BS_RACH_AWAIT	( 0, channel_request,	"2: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"3: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"4: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"5" )

BS_MSG3_AWAIT	( 0, authentication_response,	"6: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"7: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"8: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"9: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"10: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"11" )

BS_MSG3_SEND	( 0, cp-data,	"12: Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-RP set to 1" )

BS_MSG3_AWAIT	( 0, cp-ack,	"13: Sent within TC1M after step 12" )

BS_MSG3_AWAIT	( 0, cp-data,	"14: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"14A" )

BS_MSG3_SEND	( 0, channel_release,	"15" )

BS_CONFIG_CHANNEL	( 0, PCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, paging_request_type_1,	"16" )

BS_RACH_AWAIT	( 0, channel_request,	"17: Establishment cause is Answer to paging" )

BS_MSG3_SEND	( 0, immediate_assignment,	"18: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, paging_response,	"19: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"20" )

BS_MSG3_AWAIT	( 0, authentication_response,	"21: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"22: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"23: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"24: SS starts ciphering." )

BS_MSG3_SEND	( 0, sabm_(sapi=3),	"25: SS establishes SAPI 3" )

BS_MSG3_AWAIT	( 0, ua_(sapi=3),	"26" )

BS_MSG3_SEND	( 0, cp-data,	"27: Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-OA, RP-OA and TP-UD different from step 12" )

BS_MSG3_AWAIT	( 0, cp-ack,	"28: Sent within TC1M after step 12" )

BS_MSG3_AWAIT	( 0, cp-data,	"29: Contains RP-ACK RPDU." )

BS_MSG3_SEND	( 0, cp-ack,	"29A" )

BS_MSG3_SEND	( 0, channel_release,	"30" )

NOT_IMPLEMENTED	(	"31: One of the two Short Messages is displayed and the Reply Short Message is submitted." )

BS_RACH_AWAIT	( 0, channel_request,	"32" )

BS_MSG3_SEND	( 0, immediate_assignment,	"33: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, cm_service_request,	"34: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"35" )

BS_MSG3_AWAIT	( 0, authentication_response,	"36: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"37: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete,	"38: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"39: SS starts ciphering." )

BS_MSG3_AWAIT	( 0, sabm_(sapi=3),	"40: MS establishes SAPI 3" )

BS_MSG3_SEND	( 0, ua_(sapi=3),	"41" )

BS_MSG3_AWAIT	( 0, cp-data,	"42: Contains RP-DATA RPDU (SMS SUBMIT TPDU) RP-DA = RP-OA corresponding to the message displayed TP-DA = TP-OA corresponding to the message displayed" )

BS_MSG3_SEND	( 0, cp-ack,	"43: Sent within TC1M after step 42" )

BS_MSG3_SEND	( 0, cp-data,	"44: Contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"45: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp-ack,	"46" )

BS_MSG3_SEND	( 0, channel_release,	"47: The main signalling link is released.  " )

NOT_IMPLEMENTED	(	"48: The other Short Message is displayed and the Reply Short Message is submitted." )

BS_RACH_AWAIT	( 0, channel_request,	"49" )

BS_MSG3_SEND	( 0, immediate_assignment,	"50: SS assigns an SDCCH" )

BS_MSG3_AWAIT	( 0, cm_service_request,	"51: Message is contained in SABM." )

BS_MSG3_SEND	( 0, authentication_request,	"52" )

BS_MSG3_AWAIT	( 0, authentication_response,	"53: SRES specifies correct value." )

BS_MSG3_SEND	( 0, ciphering_mode_command,	"54: SS starts deciphering after sending the message." )

BS_MSG3_AWAIT	( 0, ciphering_mode_complete, "55: Shall be sent enciphered. All following messages shall be sent enciphered." )

NOT_IMPLEMENTED	(	"56: SS starts ciphering." )

BS_MSG3_AWAIT	( 0, sabm_(sapi=3),	"57: MS establishes SAPI 3" )

BS_MSG3_SEND	( 0, ua_(sapi=3),	"58" )

BS_MSG3_AWAIT	( 0, cp-data,	"59: Contains RP-DATA RPDU (SMS SUBMIT TPDU) RP-DA = RP-OA corresponding to the Message displayed TP-DA = TP-OA corresponding to the message displayed" )

BS_MSG3_SEND	( 0, cp-ack,	"60: Sent within TC1M after step 59" )

BS_MSG3_SEND	( 0, cp-data,	"61: Contains RP-ACK RPDU" )

NOT_IMPLEMENTED	(	"62: Waits max 25 seconds for CP-ACK" )

BS_MSG3_AWAIT	( 0, cp-ack,	"63" )

BS_MSG3_SEND	( 0, channel_release,	"64: The main signalling link is released." )

History:	26.01.98	MS	Initial

### MSMS010: Cell Broadcast (34.3)

Description:	This procedure checks SMS cell broadcast.

Preamble:	none

Script:

ISS_INIT	( 1 )

BS_SET_SYS_INFO	( 0, system_information_type_1 )

BS_SET_SYS_INFO	( 0, system_information_type_2 )

BS_SET_SYS_INFO	( 0, system_information_type_3 )

BS_SET_SYS_INFO	( 0, system_information_type_4_CB )

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5 )

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6 )

BS_SET_SCH	( 0,BSIC , RFN               )

BS_SET_ARFCN	( 0,ARFCN_BCCH               )

BS_SET_POWER	( 0,-73                      )

BS_ON_OFF	( 0,TRUE                     )

COMMAND	( "MMI CONFIG KEY_SEQUENCE=<#*91*1#>" )

ISS_DELAY	( 20000 ) /* wait until the MS has detected the cell */

BS_CONFIG_CHANNEL	( 0, CBCH, UNACK,SAPI_0)

BS_MSG3_SEND	( 0, sms_cbch_message,	"1: SS sends a CBCH message" )

ISS_DELAY	( 10000 )

COMMAND	( "MMI CONFIG AT+CPMS=0" )

COMMAND	( "MMI CONFIG AT+CMGR=1" )

ISS_DELAY	( 5000 )

COMMAND	( "MMI CONFIG AT+CMGD=1" )

ISS_DELAY	( 5000 )

History:	26.01.98	MS	Initial

## Additional SMS Testcases

### MSMS500: Write and Read of a mobile originated SMS

Description:	A mobile originated SMS is stored on the SIM card and read again.

Preamble:	none

Script:

ISS_INIT	( 1 )

BS_SET_SYS_INFO	( 0, system_information_type_1 )

BS_SET_SYS_INFO	( 0, system_information_type_2 )

BS_SET_SYS_INFO	( 0, system_information_type_3 )

BS_SET_SYS_INFO	( 0, system_information_type_4)

BS_SET_SYS_INFO_SACCH( 0, system_information_type_5 )

BS_SET_SYS_INFO_SACCH( 0, system_information_type_6 )

BS_SET_SCH	( 0,BSIC , RFN               )

BS_SET_ARFCN	( 0,ARFCN_BCCH               )

BS_SET_POWER	( 0,-73                      )

BS_ON_OFF	( 0,TRUE                     )

COMMAND	( "MMI CONFIG KEY_SEQUENCE=<#*91*1#>" )

ISS_DELAY	( 20000 ) /* wait until the MS has detected the cell */

AT_SEND ("AT+CMGW=\"654321\",,\r\n",SILENT);    /* start entering MO-SMS */

AT_RECEIVE ("\n\r>", SILENT);

AT_SEND ("ABCDEFGHIJKLMNOPQRSTUVWXYZ\r\n",SILENT);

AT_RECEIVE ("+CMGW: 1", SILENT);

AT_SEND ("AT+CMGR=1", SILENT);    /* read MO-SMS */

AT_RECEIVE ("+CMGR: STO UNSENT", SILENT);

History:	26.01.98	MS	Initial

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |