| Date: | March 06, 2003 |
| --- | --- |
| ID: | 6147.404.97.107 |
| Status: | Planned |

| Author: | Condat AG Alt Moabit 90a 10559 Berlin Germany |
| --- | --- |

| Condat Proprietary Information NDA - Confidential Do Not Copy |  |
| --- | --- |

G23-GSM Protocol Stack

RR

Entity Test Specification

GSM

Table of Contents

0	Document Control	6

0.1	Document History	6

0.2	References, Abbreviations, Terms	6

1	Introduction	7

2	Parameters	8

2.1	Declarations	45

2.2	Basic Types	54

2.3	Bit Buffers	59

2.4	Arrays	59

2.4.1	Arrays of Bytes	59

2.4.2	Arrays of Shorts	63

2.4.3	Arrays of Longs	67

2.5	Primitive Structures	68

2.6	Arrays of Structures	89

2.7	Message Structures	95

3	TEST CASES	111

3.1	Routing (internal)	111

3.1.1	RR000: Setup the routing and PCO view for the RR test	111

3.2	Cell Selection	113

3.2.1	RR001: Start cell selection (std = 1)	113

3.2.2	RR002: Start cell selection with or without BCCH information (std 6)	115

3.2.3	RR003: Start cell selection with or without BCCH information (std 5)	117

3.2.4	RR004: Start cell selection without BCCH information (std 4)	118

3.2.5	RR010 : Network Sync Timeout	122

3.2.6	RR011: Listen to SYS INFO (without SIM)	123

3.2.7	RR012: Listen to SYS INFOs (with SIM)	126

3.2.8	RR013: Listen to SYS INFOs (with SIM)	129

3.2.9	RR021: BCCH carrier not suitable (cell barred, without SIM)	133

3.2.10	RR022: BCCH carrier not suitable (C1 <= 0, without SIM)	134

3.2.11	RR023: BCCH carrier not suitable (cell barred, with SIM, second cell)	135

3.2.12	RR024: BCCH carrier not suitable (C1 <= 0, with cell, second cell)	136

3.2.13	RR025: BCCH carrier not suitable (incorrect network, with SIM, second cell)	137

3.2.14	RR027: BCCH carrier not suitable (cell barred, with SIM)	138

3.2.15	RR028: BCCH carrier not suitable (C1 <= 0, with SIM)	139

3.2.16	RR029: BCCH carrier not suitable (incorrect network, with SIM)	140

3.2.17	RR030: Suitable BCCH carrier found (with SIM)	141

3.2.18	RR033: Suitable BCCH carrier found (without SIM)	143

3.2.19	RR034: Suitable BCCH carrier found (low priority cell, without SIM)	145

3.2.20	RR035: Suitable BCCH carrier found (with SIM, second cell)	147

3.2.21	RR036: Suitable BCCH carrier found - NCell description in 128-range format	149

3.2.22	RR037: Suitable BCCH carrier found - NCell description in 256-range format	151

3.2.23	RR038: Suitable BCCH carrier found - NCell description in 512-range format	153

3.2.24	RR039: Suitable BCCH carrier found - NCell description in 1024-range format	155

3.2.25	RR040: Suitable BCCH carrier found - NCell description in variable-bit format	157

3.2.26	RR041: Listen to SYS INFOs, SYS INFO 1 not required	159

3.2.27	RR070: No BCCH carrier available	163

3.2.28	RR071: No BCCH carrier available (limited service possible)	165

3.2.29	RR072: Message Cell Broadcast	169

3.2.30	RR080: Cell Selection for Dualband	171

3.2.31	RR045: No SIM card, suitable if using C1 Offset	175

3.2.32	RR046: With normal SIM Card, suitable if using C1 Offset	179

3.2.33	RR047: With Test SIM Card, unsuitable although using C1 Offset	182

3.2.34	RR048: Cell Selection E-GSM	185

3.2.35	RR049: Cell Selection for Testcase 13.1	188

3.3	Idle Mode Neighbour Cell Procedures	191

3.3.1	RR132: Listening to Neighbour Cell	191

3.3.2	RR133: Listening to Neighbour Cell and Network is lost, Limited service only possible	194

3.4	Immediate Assignment	200

3.4.1	RR136: RR-Connection Establishment by the MS	200

3.4.2	RR153: Paging for MS (IMSI)	201

3.4.3	RR154: Imm Ass for the MS, T3122 is not running	202

3.4.4	RR152: Imm Ass for the MS, spare bits unequal 0	205

3.4.5	RR155: Imm Ass Extended for the MS, T3122 is not running	206

3.4.6	RR156: Imm Ass Reject for the MS, T3122 is not running	210

3.4.7	RR157: Immediate Assignment E-GSM	212

3.4.8	RR158: Immediate Assignment TC 13.1	215

3.4.9	RR159: Immediate Assignment (Multiband)	218

3.4.10	RR160: T3126 Timeout, T3122 is not running, Mobile Originated Call	220

3.4.11	RR161: DLF indiciation crosses connection establishment	220

3.4.12	RR162: Imm Ass Extended for the MS fails	221

3.4.13	RR400: Paging for MS, TMSI	223

3.4.14	RR403: Imm Ass Extended for the MS	225

3.4.15	RR404: Indicate Fieldstrength Jump	227

3.4.16	RR405: Paging for MS (IMSI) after connection	228

3.5	Channel Assignment	229

3.5.1	RR204: start of intracell handover without change in channel mode	229

3.5.2	RR209: start of intracell handover with change of channel mode	233

3.5.3	RR700: start of intracell handover with change of channel mode-AMR	235

3.5.4	RR211: start of intracell handover using starting time	238

3.5.5	RR212: start of intracell handover with E-GSM channel 0	241

3.5.6	RR214: start of intracell handover for testcase 13.1	244

3.5.7	RR205: successful end of intracell handover, no change in channel mode	246

3.5.8	RR210: successful end of intracell handover with change in channel mode	247

3.5.9	RR213: unsuccessful intracell handover due to "frequency not implemented" (1800)	248

3.5.10	RR701: unsuccessful intracell handover without multirate conf IE-AMR	250

3.5.11	RR702: unsuccessful intracell handover with inconsistent multirate conf IE-AMR	252

3.5.12	RR206: new configuration failed, back to old configuration	254

3.5.13	RR207: reconnection of old configuration successful	255

3.5.14	RR208: reconnection of old configuration failed	256

3.6	Handover Command	260

3.6.1	RR215: Handover Command	260

3.6.2	RR216: Handover Command	261

3.6.3	RR703: Handover Command - AMR	263

3.6.4	RR706: Handover Command - AMR - no Channel Mode element	267

3.6.5	RR704: Handover cmd with inconsistent multirate conf IE - AMR	269

3.6.6	RR217: Handover Command with mandatory errors	270

3.6.7	RR218: Assignment Command with mandatory errors	272

3.7	Channel Release	273

3.7.1	RR227: Timeout T3110, same cell	273

3.7.2	RR229: Release by DL_RELEASE_IND, same cell	275

3.8	Layer 3 Messages	277

3.8.1	RR230: Downlink Messages	277

3.8.2	RR231: Uplink Messages	278

3.9	Measurement Report	279

3.9.1	RR185: Send Measurement Reports on SACCH	279

3.9.2	RR186: Send Measurement Reports on SACCH (including 5ter)	281

3.9.3	RR187: Send Measurement Reports on SACCH (Multiband)	283

3.9.4	RR188: Send Measurement Reports on SACCH (E-GSM)	285

3.9.5	RR189: Send Measurement Reports on SACCH (E-GSM) (II)	288

3.9.6	RR190: Send Measurement Reports on SACCH (E-GSM) (III)	291

3.9.7	RR191: Send Measurement Reports on SACCH (E-GSM) (IV)	294

3.9.8	RR192: Send Measurement Reports on SACCH (E-GSM) (V)	297

3.9.9	RR193: Send Measurement Reports on SACCH / 1900	300

3.10	Extended Measurement Report on SACCH (EMO)	302

3.10.1	RR250: EMO, Normal case including NC reconfig	302

3.10.2	RR251: EMO, 10 sec expiry	303

3.10.3	RR252: EMO, before first neighbour cell configuration	305

3.10.4	RR253: EMO, successful Channel Change during EMO	306

3.10.5	RR254: EMO, failed Channel Change with successful reconnection during EMO	309

3.10.6	RR255: EMO, new system info 5* (new NC config) during EMO	312

3.10.7	RR256: EMO, frequencies out of band in EMO request	314

3.10.8	RR257: EMO, Frequency List contains frequency ARFCN=0	316

3.10.9	RR258: EMO, too many (more than 21) frequencies	317

3.10.10	RR259: EMO, Multiple EMO requests, same sequence number	319

3.10.11	RR260: EMO, Multiple EMO requests, different sequence number	321

3.11	Channel Mode Modify	324

3.11.1	RR225: Channel Mode Modify Procedure	324

3.11.2	RR705: Channel Mode Modify Procedure - AMR	325

3.11.3	RR705A: Channel Mode Modify Procedure – AMR (with inconsistent Multirate IE)	326

3.12	Cipher Mode Setting	327

3.12.1	RR226: Cipher Mode Setting Procedure	327

3.13	Test Functions	331

3.13.1	RR300: Close TCH Loop	331

3.13.2	RR301: Open TCH Loop (Loop unequal Type C)	332

3.13.3	RR302: Open TCH Loop (Loop equal Type C)	333

3.13.4	RR303: Test Interface	334

3.14	Error Handling	336

3.14.1	RR500: Frequency not implemented	336

3.14.2	RR501: Channel mode unacceptable	338

3.15	Registration (Limited Service)	340

3.15.1	RR600: Cell with unsufficient SYS INFO	340

3.15.2	RR601: Cell with unreadable SYS INFO	342

3.15.3	RR602: Second cell is okay	345

3.15.4	RR603: No further BCCH detected, enter no service condition	348

3.15.5	RR604: Second Cell with unsufficient SYS INFO	349

3.15.6	RR605: Second Cell with unreadable SYS INFO	351

3.15.7	RR606: Restart by MM for limited service, successful	353

3.15.8	RR607: Restart by MM for limited service fails, no BSIC	357

3.15.9	RR608: Restart by MM for limited service fails, insufficient SYS INFOS	359

3.15.10	RR609: Restart by MM for limited service fails, unreadable SYS INFOS	362

3.15.11	RR618: Restart by MM for full service, successful, with bcch info	365

3.15.12	RR619: Restart by MM for full service, successful, without bcch info	369

3.15.13	RR620: Restart by MM for full service fails, no BSIC	373

3.15.14	RR621: Restart by MM for full service fails, insufficient SYS INFOS	375

3.15.15	RR622: Restart by MM for full service fails , unreadable SYS INFOS	378

3.15.16	RR623: Restart by MM for full service fails, limited service available	382

3.15.17	RR624: Restart by RR for full service is successful	387

3.15.18	RR625: Restart by RR for full Service fails, no BSIC	392

3.15.19	RR617: Restart by RR for full Service fails, SC is read again	394

3.15.20	RR626: Restart by RR for full Service fails, insufficient SYS INFOS	396

3.15.21	RR627: Restart by RR for full Service fails, unreadable SYS INFOS	398

3.16	Registration (No Service)	400

3.16.1	RR610: Restart by RR for limited Service, successful	400

3.16.2	RR611: Restart by RR for limited Service fails, no BSIC	404

3.16.3	RR612: Restart by RR for limited Service fails, insufficient SYS INFOS	405

3.16.4	RR613: Restart by RR for limited Service fails, unreadable SYS INFOS	407

3.16.5	RR615: Restart by MM for limited service fails, no BSIC	410

3.16.6	RR628: Restart by MM for full service, successful, with bcch info	412

3.16.7	RR629: Restart by MM for full service, successful, without bcch info	416

3.16.8	RR630: Restart by MM for full service fails, no BSIC	420

3.16.9	RR631: Restart by MM for full service fails, insufficient SYS INFOS	422

3.16.10	RR632: Restart by MM for full service fails, unreadable SYS INFOS	425

3.16.11	RR633: Restart by RR for full service is successful	427

3.16.12	RR634: Restart by RR for full Service fails, no BSIC	431

3.16.13	RR635: Restart by RR for full Service fails, insufficient SYS INFOS	432

3.16.14	RR636: Restart by RR for full Service fails, unreadable SYS INFOS	434

3.17	Registration (Full Service)	437

3.17.1	RR637: Start by MM for full service, successful, with bcch info info (used as preamble only)	437

3.17.2	RR638: Restart by MM for full service, successful, without bcch info	441

3.17.3	RR639: Restart by MM for full service fails, no BSIC	445

3.17.4	RR640: Restart by MM for full service fails, insufficient SYS INFOS	448

3.17.5	RR641: Restart by MM for full service fails, unreadable SYS INFOS	455

3.17.6	RR642: Restart by MM for full service, successful, with bcch info	458

3.17.7	RR643: Restart by MM for full service, successful, without bcch info	462

3.17.8	RR644: Restart by MM for full service fails, no BSIC	466

3.17.9	RR645: Restart by MM for full service fails, insufficient SYS INFOS	468

3.17.10	RR646: Restart by MM for full service fails, unreadable SYS INFOS	471

3.17.11	RR647: Restart by MM for limited service, successful	474

3.17.12	RR648: Restart by MM for limited service fails, no BSIC	478

3.17.13	RR649: Restart by MM for limited service fails, insufficient SYS INFOS	480

3.17.14	RR650: Restart by MM for limited service fails, unreadable SYS INFOS	483

3.18	Registration (Network Search)	485

3.18.1	RR657: No Service, Network Search requested by MMI, no PLMNs	485

3.18.2	RR658: No Service, Network Search requested by MMI, one PLMN	490

3.18.3	RR661: Limited Service, Network Search requested by MMI, no PLMNs	496

3.18.4	RR662: Limited Service, Network Search requested by MMI, one PLMN	501

3.18.5	RR667: Full Service, Network Search requested by MMI, select PLMN	507

3.18.6	RR670: Network Search, interrupted by cell reselection	515

3.18.7	RR671: Network Search, interrupted by downlink signalling failure	521

3.18.8	RR672: Network Search, interrupted by downlink signalling failure (II)	527

3.18.9	RR673: Network Search, interrupted by downlink signalling failure (III)	534

3.18.10	RR674: Network Search requested by MMI, five PLMNs	540

3.19	Miscellaneous	550

3.19.1	RR680: Overload primitive store / Start cell selection without BCCH information (std 6)	550

3.19.2	RR681: Overload primitive store  / RR-Connection Establishment by the MS	553

3.19.3	RR682: Overload primitive store  / Cell Selection	554

3.19.4	RR683: Overload primitive store / Listen to SYS INFOs (with SIM)	557

3.19.5	RR684: Engineering Mode – Assignment Command received & complete (Event number 19 + 22)	558

3.20	E-OTD	561

3.20.1	RR800: Successful E-OTD	561

3.20.2	RR802: E-OTD Request rejected by ALR	562

3.20.3	RR804: E-OTD Request rejected by RR – Handover in progress	563

3.20.4	RR806: E-OTD Operation interrupted due to start of Handover	564

3.20.5	RR820: sending APDU, no segmentation	566

3.20.6	RR821: sending APDU, with segmentation	567

3.20.7	RR822: sending APDU, three segments	568

3.20.8	RR823: receiving APDU, no segmentation	569

3.20.9	RR824: receiving APDU, three segments	569

3.20.10	RR825: receiving APDU, three segments, timeout	570

3.20.11	RR826: receiving APDU, abnormal, other layer 3 message	571

3.20.12	RR827: receiving APDU, abnormal, "First or Only Segment"	573

3.20.13	RR828: receiving APDU, abnormal, "First or Only Segment"	574

4	Suites	576

# Document Control

ã Copyright Condat AG, 2002

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat AG reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat AG. Condat AG accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat AG.

Condat AG

Alt Moabit 90a

10559 Berlin

Germany

Telephone:	+49.30.39 49 0

Fax:		+49.30.39 49 1300

Internet:	www.condat.de

## Document History

ID	Author	Date	Status
6147.404.97.001	MPA	16 June, 2002	Planned

6147.404.97.107	VK	July 30, 2002	Re-activate RR667

6147.404.97.108	LG	March 06, 2003	insert lac_list, more unique formatting, timeout review

## References, Abbreviations, Terms

[C_7010.801]	7010.801, References and Vocabulary, Condat AG

# Introduction

This document is the test document for the entity RR.

Status of Document: On baseline g23m_S180 all tests passed.

# Parameters

FIELD(AMR_HO_ALC)
0x68, 0x00,

0x18, 0x00,

0x00, 0x00, 0x00, 0x06, 0x2B, 0x00, 0x56, 0x0E, 0x00, 0x5A, 0x0C, 0x05, 0x03, 0x02, 0x20, 0x80, 0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDFIELD(AMR_HO_ALC,25)

/*

| 31 | length of list |
| --- | --- |
| 43,0 | channel 0x43 |
| 0xFF, 0xFF | end of list |

*/

FIELD (ACT_DESC_0043)	31,
	0x43,0,
	0xFF,0xFF
ENDFIELD (ACT_DESC_0043, 5)

/*

| 31 | length of list |
| --- | --- |
| 3,0 | Channel 3 |
| 24, 0 | Channel 24 |
| 99,0 | Channel 99 |
| 0xFF, 0xFF | end of list |

*/

FIELD (ACT_DESC_1)	31,
	3,0,
	24,0,
	99,0,
	0xFF,0xFF
ENDFIELD (ACT_DESC_1, 9)

/*

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x00, 0x04, 0x00, 0x00 | 115 |
| 0x00, 0x02, 0x00, 0x01 | 82 65 |
| 0x00, 0x08, 0x00, 0x00 | 52 |
| 0x81, 0x00, 0x00, 0x00 | 32 25 |

*/

FIELD (CELL_CHAN_DESC_1)	0x80, 0x00,
	0x00, 0x00,
	0x00, 0x04, 0x00, 0x00,
	0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x00,
	0x81, 0x00, 0x00, 0x00
ENDFIELD (CELL_CHAN_DESC_1, 20)

FIELD (CELL_CHAN_DESC_EGSM)	0x80, 0x00,
	0x00, 0x00,
	0x84, 0x18, 0xF5, 0x92,
	0xC0, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (CELL_CHAN_DESC_EGSM, 20)

FIELD (CELL_CHAN_DESC_512_15)	0x80, 0x00,
	0x00, 0x00,
	0x8F, 0x00, 0x70, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (CELL_CHAN_DESC_512_15, 20)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x00, 0x04, 0x00, 0x01 | 115 97 |
| 0x00, 0x02, 0x00, 0x01 | 82 65 |
| 0x00, 0x08, 0x00, 0x01 | 52 33 |
| 0x81, 0x00, 0x00, 0x0F | 32 25 4,3,2,1 |

*/

FIELD (CELL_CHAN_DESC_2)	0x80, 0x00,
	0x00, 0x00,
	0x00, 0x04, 0x00, 0x01,
	0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x01,
	0x81, 0x00, 0x00, 0x0F
ENDFIELD (CELL_CHAN_DESC_2, 20)

/*

| 0, 0x37, ... | time in TDMA frames between random bursts |
| --- | --- |

*/

FIELD (DELTA_TWO_BURSTS)	0, 0x37,0,0,0,0,0,0
ENDFIELD (DELTA_TWO_BURSTS, 8)

FIELD (DELTA_EIGHT_BURSTS)	0, 0x56,0x56,0x56,0x56,0x56,0x56,0x56
ENDFIELD (DELTA_EIGHT_BURSTS, 8)

/*

|  | Frequency Channel Sequence (1,25,50,65,111,112,113,114) |
| --- | --- |
| 0x01 | Lowest ARFCN = 1 |
| 0x0 | +15 = 16 not used |
| 9 | +9 = 25 |
| 0x0 | +15 = 40 not used |
| a | +10 = 50 |
| 0xf | +15 = 65 |
| 0 | +15 = 80 not used |
| 0x0 | +15 = 95 not used |
| 0 | +15 = 110 not used |
| 0x1 | +1 = 111 |
| 1 | +1 = 112 |
| 0x1 | +1 = 113 |
| 1 | +1 = 114 |
| 0x0 | not used |
| 0 | not used |
| 0x0 | not used |
| 0 | not used |

*/

FIELD (FREQ_CHAN_SEQ_1_INC_SKIP)	0x0,0x9,0x0,0xa,0xf,0x0,0x0,0x0,0x1,0x1,0x1,0x1,0x0,0x0,0x0,0x0
ENDFIELD (FREQ_CHAN_SEQ_1_INC_SKIP,16)

/*

|  | Frequency list describing the same frequencies as hopping_2 (52, 82) |
| --- | --- |

*/

FIELD (FREQ_LIST_AFTER_2)	16,
0x00, 0x00, 0x00, 0x00,
0x00, 0x02, 0x00, 0x00,
0x00, 0x08, 0x00, 0x00,
0x00, 0x00, 0x00, 0x00
ENDFIELD (FREQ_LIST_AFTER_2, 17)

/*

| 1..32, 93..124 | Frequency list |
| --- | --- |

*/

FIELD (FREQ_LIST_BEFORE_1)	16,
	0x0F, 0xFF, 0xFF, 0xFF,
	0xF0, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0xFF, 0xFF, 0xFF, 0xFF

ENDFIELD (FREQ_LIST_BEFORE_1, 17)

/*

| 0x00, ... | digits |
| --- | --- |
| 0x0F | end of digits |

*/

FIELD (IMSI_08158912)	0,8,1,5,8,9,1,2,
	0x0F
ENDFIELD (IMSI_08158912, 9)

/*

| 0x01, ... | digits |
| --- | --- |
| 0x0F | end of digits |

*/

FIELD (IMSI_1233247114912)	0x01, 0x02, 0x03, 0x03,
	0x02, 0x04, 0x07, 0x01,
	0x01, 0x04, 0x09, 0x01,
	0x02,
	0x0F
ENDFIELD (IMSI_1233247114912, 14)

FIELD (IMSI_0010147114912)	0x00, 0x00, 0x01, 0x00,
	0x01, 0x04, 0x07, 0x01,
	0x01, 0x04, 0x09, 0x01,
	0x02,
	0x0F
ENDFIELD (IMSI_0010147114912, 14)
/*

| 0x00, ... | Kc values |
| --- | --- |

*/

- FIELD (KC_00112233)	0x00, 0x00, 0x01, 0x01,
	0x02, 0x02, 0x03, 0x03
ENDFIELD (KC_00112233, 8)

/*

| 0x01, ... | Kc values |
| --- | --- |

*/

FIELD (KC_12345678)	0x01, 0x02, 0x03, 0x04,
	0x05, 0x06, 0x07, 0x08
ENDFIELD (KC_12345678, 8)

/*

| 0x01, ... | IMEI digits (in messages) |
| --- | --- |

*/

FIELD (M_IMEI)	1, 3, 5, 7, 9, 0, 2, 4,
	6, 8, 1, 1, 2, 2, 7, 8
ENDFIELD (M_IMEI, 16)

/*

| 0x00, ... | IMSI digits (in messages) |
| --- | --- |

*/

FIELD (M_IMSI_08158912)	0,8,1,5,8,9,1,2
ENDFIELD (M_IMSI_08158912, 8)

/*

| 0x01, ... | IMSI digits (in messages) |
| --- | --- |

*/

- FIELD (M_IMSI_1233247114912)	0x01, 0x02, 0x03, 0x03,
	0x02, 0x04, 0x07, 0x01,
	0x01, 0x04, 0x09, 0x01,
	0x02
ENDFIELD (M_IMSI_1233247114912, 13)

- FIELD (M_IMSI_0010147114912)	0x00, 0x00, 0x01, 0x00,
	0x01, 0x04, 0x07, 0x01,
	0x01, 0x04, 0x09, 0x01,
	0x02
ENDFIELD (M_IMSI_0010147114912, 13)

/*

| 0x01, ... | mobile country code 122 |
| --- | --- |

*/

FIELD (MCC_122)	0x01, 0x02, 0x02
ENDFIELD (MCC_122, 3)

FIELD (MCC_001)	0x00, 0x00, 0x01
ENDFIELD (MCC_001, 3)

/*

| 0x01, ... | Mobile country code 123 |
| --- | --- |

*/

FIELD (MCC_123)	0x01, 0x02, 0x03
ENDFIELD (MCC_123, 3)

FIELD (HANDOVER_CMD_ERROR)	0x60, 0x00, 
	0x18, 0x00,
	0x00, 0x00, 0x00, 
	0x06,       /* PD + TI */

0x2b, /* HANDOVER command */

0x0b, 0x14, /* Cell description */

0x09, 0xa0, 0x14, /* Channel description after */

0x01, /* Handover Reference */

0x08, /* Power Command and Access Type */

0x00, 0x01, 0x00  /* Unknown Tag : Comprehension Required */
ENDFIELD (HANDOVER_CMD_ERROR, 19)

FIELD (ASSIGNMENT_CMD_ERROR)	0x60, 0x00, 
	0x18, 0x00,
	0x00, 0x00, 0x00, 
	0x06,       /* PD + TI */

0x2e, /* ASSIGNMENT command */

0x09, 0xa0, 0x14, /* Channel description */

0x08, /* Power Command and Access Type */

0x00, 0x01, 0x00  /* Unknown Tag : Comprehension Required */
ENDFIELD (ASSIGNMENT_CMD_ERROR, 16)

/*

| 3, 3 | mobile network code 33 |
| --- | --- |

*/

FIELD (MNC_36)	0x03, 0x06, 0x0F
ENDFIELD (MNC_36, 3)

FIELD (MNC_35)	0x03, 0x05, 0x0F
ENDFIELD (MNC_35, 3)

FIELD (MNC_34)	0x03, 0x04, 0x0F
ENDFIELD (MNC_34, 3)

FIELD (MNC_33)	0x03, 0x03, 0x0F
ENDFIELD (MNC_33, 3)

FIELD (MNC_32)	0x03, 0x02, 0x0F
ENDFIELD (MNC_32, 3)

/*
 * MM hasn't the ability to distinguish the 3rd MNC digit from an artefact,
 * but this doesn't matter as RR uses the comparision routines described in
 * GSM 03.22 Annex A (normative) and can distinguish therefore.
 * The '4' is an artefact from the SIM here, the MNC is "32".
 */
FIELD (MNC_32X)	0x03, 0x02, 0x04
ENDFIELD (MNC_32X, 3)

FIELD (MNC_01)	0x00, 0x01, 0x0F
ENDFIELD (MNC_01, 3)

FIELD (MSG_MNC_36)	0x03, 0x06
ENDFIELD (MSG_MNC_36, 2)

FIELD (MSG_MNC_35)	0x03, 0x05
ENDFIELD (MSG_MNC_35, 2)

FIELD (MSG_MNC_34)	0x03, 0x04
ENDFIELD (MSG_MNC_34, 2)

FIELD (MSG_MNC_33)	0x03, 0x03
ENDFIELD (MSG_MNC_33, 2)

FIELD (MSG_MNC_32)	0x03, 0x02
ENDFIELD (MSG_MNC_32, 2)

FIELD (MSG_MNC_01)	0x00, 0x01
ENDFIELD (MSG_MNC_01, 2)

/*

| 0x16 | mobile allocation |
| --- | --- |

*/

FIELD (MOB_ALLOC_1)	0x16
ENDFIELD (MOB_ALLOC_1, 1)

FIELD (MOB_ALLOC_EGSM)	0x1F
ENDFIELD (MOB_ALLOC_EGSM, 1)

/*

| 0x14 | mobile allocation |
| --- | --- |

*/

FIELD (MOB_ALLOC_2)	0x14
ENDFIELD (MOB_ALLOC_2, 1)

/*

| 0x3F | mobile allocation |
| --- | --- |

*/

FIELD (MOB_ALLOC_3)	0x3F
ENDFIELD (MOB_ALLOC_3, 1)

/*

|  | mobile allocation |
| --- | --- |

*/

FIELD (MOB_ALLOC_4)	0x0F, 0xFF
ENDFIELD (MOB_ALLOC_4, 2)

/*

| 33 | size of list |
| --- | --- |
| 3, 0 | channel 3 |
| 32, 0 | channel 24 |
| 99, 0 | channel 99 |
| 124, 0 | channel 124 |
| 0xFF, 0xFF | end of list |

*/

FIELD (MPH_NCELL_1)	33,
	3,0,
	32,0,
	99,0,
	124,0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_1, 11)

FIELD (MPH_NCELL_5TER_FTA)	33,
	14,0,
	20,0,
	32,0,
	44,0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5TER_FTA, 11)

FIELD (MPH_NCELL_5TER_5_FTA)	33,
	8,2,
	0x0C,3,
	0x70,3,
	14,0,
	20,0,
	32,0,
	44,0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5TER_5_FTA, 17)

FIELD (MPH_NCELL_5TER)	33,
	3,0,
	32,0,
	99,0,
	124,0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5TER, 11)

FIELD (MPH_NCELL_5_FTA_A)	33,
	0xde, 3,
	0xeb, 3,
	0xed, 3,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_FTA_A, 9)

FIELD (MPH_NCELL_5_FTA_B)	33,
	26, 0,
	38, 0,
	0, 0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_FTA_B, 9)

FIELD (MPH_NCELL_5_FTA_C)	33,
	8, 2,
	0xde, 3,
	0xfc, 3,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_FTA_C, 9)

FIELD (MPH_NCELL_5_FTA_D)	33,
	26, 0,
	38, 0,
	0xde, 3,
	0xED, 3,
	0xFC, 3,
	0x0, 0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_FTA_D, 15)

FIELD (MPH_NCELL_5_FTA_E)	33,
	38, 0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_FTA_E, 5)

FIELD (MPH_NCELL_5_5TER_FTA_C)	33,
	8,2,
	0xde, 3,
	0xfc, 3,
	26, 0,
	0xeb, 3,
	0xed, 3,
	0x00, 0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_5TER_FTA_C, 17)

FIELD (MPH_NCELL_5_5TER_FTA_D)	33,
	26, 0,
	38, 0,
	0xde, 3,
	0xED, 3,
	0xFC, 3,
	0x0, 0,
	0xE8, 3,
	0xEB, 3,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_5TER_FTA_D, 19)

FIELD (MPH_NCELL_5_5TER_FTA_E)	33,
	38, 0,
	26, 0,
	0xeb, 3,
	0xFC, 3,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_5TER_FTA_E, 11)

FIELD (MPH_NCELL_5_5BIS_FTA_A)	33,
	0x1A, 0,
	0x26, 0,
	0xde, 3,
	0xeb, 3,
	0xed, 3,
	0x00, 0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_5BIS_FTA_A, 15)

FIELD (MPH_NCELL_5_5BIS_FTA_B)	33,
	26, 0,
	38, 0,
	0x74, 3,
	0xd4, 3,
	0xe1, 3,
	0xe3, 3,
	0x0, 0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_5_5BIS_FTA_B, 17)

FIELD (MPH_NCELL_1B)	33,
	3,0,
	24,0,
	99,0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_1B, 9)

FIELD (MPH_NCELL_2)	33,
	3,0,
	32,0,
	99,0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_2, 9)

FIELD (MPH_NCELL_DG_186)	33,
	115,02,
	123,02,
	131,02,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_DG_186, 9)

FIELD (MPH_NCELL_528_31)	33,
	0x10,0x02,
	0x11,0x02,
	0x12,0x02,
	0x13,0x02,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_528_31, 11)

FIELD (MPH_NCELL_EMPTY)	33,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_EMPTY, 3)

/*

| 33 | size of list |
| --- | --- |
| 32, 0 | channel 32 |
| 67, 0 | channel 67 |
| 124, 0 | channel 124 |
| 0xFF, 0xFF | end of list |

*/

FIELD (MPH_NCELL_1_LIM)	33,
	0x20, 0,
	0x43, 0,
	124, 0,
	0xFF,0xFF
ENDFIELD (MPH_NCELL_1_LIM, 9)

/*

| 33 | size of list |
| --- | --- |
| 2, 0 | channel 2 |
| 9, 0 | channel 9 |
| 10, 0 | channel 10 |
| 80,0 | channel 80 |
| 90, 0 | channel 90 |
| 100, 0 | channel 100 |
| 110, 0 | channel 110 |
| 120, 0 | channel 120 |
| 123, 0 | channel 113 |
| 124, 0 | channel 124 |
| 0xFF, 0xFF | end of list |

*/

FIELD (MPH_NCELL_8)	33,
	2,0,
	9,0,
	10,0,
	80,0,
	90,0,
	100,0,
	110,0,
	120,0,
	123,0,
	124,0,
	0xFF, 0xFF
ENDFIELD (MPH_NCELL_8, 23)

/*

| 15 | Length IMSI |
| --- | --- |
| 0,0,... | IMSI (max 15 digits) |
| 1 | TMSI valid flag |
| 142, ... | TMSI (4 bytes) |

*/

FIELD (MS_ID_IMSI_HPLMN_TMSI)	13,
	1,2,3,3,2,4,7,1,1,4,9,1,2,0,0,
	1,
	0x42,1,0,0
ENDFIELD (MS_ID_IMSI_HPLMN_TMSI, 21)

FIELD (MS_ID_IMSI_TEST_TMSI)	13,
	0,0,1,0,1,4,7,1,1,4,9,1,2,0,0,
	1,
	0x42,1,0,0
ENDFIELD (MS_ID_IMSI_TEST_TMSI, 21)

FIELD (MS_ID_NO_IMSI_NO_TMSI)	0,
	0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,
	0,
	0,0,0,0
ENDFIELD (MS_ID_NO_IMSI_NO_TMSI, 21)

FIELD (MS_ID_IMSI_HPLMN_NO_TMSI)	13,
	1,2,3,3,2,4,7,1,1,4,9,1,2,0,0,
	0,
	0,0,0,0
ENDFIELD (MS_ID_IMSI_HPLMN_NO_TMSI, 21)

/*

* sys info 3 with rest octets

*/

FIELD (SYS_INFO_3_A)	0xB8, 0x00,
	0x18, 0x00,
	0x00, 0x00, 0x00,
	0x06, 0x1B, 0x37, 0x48, 0x21, 0xF3, 0x23, 0x21, 0x47,
	0x68, 0x02, 0x06, 0x5F, 0x42, 0x56, 0x00, 0x00, 0x40,
	0x2b, 0x2b, 0x2B, 0x2B, 0x2B, 0x2b
ENDFIELD (SYS_INFO_3_A, 31)

/*
  *     system information messages for testcase 15 (FTA)
  */

FIELD (SYS_INFO_1_15)	0xB0, 0x00,
	0x18, 0x00,
	0x00, 0x00, 0x00,
	0x06, 0x19, 
	0x08, 0x00, 0x00, 0x00, 
	0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00,

0x00, 0x00, 0x00, 0x00,
	0xCD, 0x00, 0x00, 0x2B
ENDFIELD (SYS_INFO_1_15, 29)

FIELD (SYS_INFO_2_15)	0xB0, 0x00,
	0x18, 0x00,
	0x00, 0x00, 0x00,
	0x06, 0x1A, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x02, 0xCD, 0x00, 0x00
ENDFIELD (SYS_INFO_2_15, 29)

FIELD (SYS_INFO_3_15)	0xB0, 0x00,
	0x18, 0x00,
	0x00, 0x00, 0x00,
	0x06, 0x1B, 0x00, 0x01, 0x00, 0xF1, 0x10, 0x00, 0x01,
	0x01, 0x03, 0x00, 0x21, 0xCD, 0x00, 0xCD, 0x00, 0x00,
	0x4B, 0x2b, 0x2b, 0x2B
ENDFIELD (SYS_INFO_3_15, 29)

FIELD (SYS_INFO_4_15)	0xB0, 0x00,
	0x18, 0x00,
	0x00, 0x00, 0x00,
	0x06, 0x1C, 0x00, 0xF1, 0x10, 0x00, 0x01, 0xCD, 0x00,
	0xCD, 0x00, 0x00, 0x2B, 0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2b, 0x2b, 0x2B, 0x2B
ENDFIELD (SYS_INFO_4_15, 29)

/*
  *   Assignment command for 13.1

FIELD (ASSIGN_CMD)	0x50, 0x01,
	0x18, 0x00,
	0x00, 0x00, 0x00,
	0x06, 0x2E, 0x81, 0xB0, 0x00, 0x07, 0x05, 0x20, 0x80,
	17, 240, 7,61,60,67,131,125,250,251,232,64,129,97,61,251,
	223,188,243,207,65,8,16,129,16,64,185,206,115,
	221,224,0x63,0x01
ENDFIELD (ASSIGN_CMD, 49)

*/

/*

| 2 | number of neighbour cells |
| --- | --- |
| 3, 0 | Channel 3 |
| 18, 0 | Channel 18 |
| 0, ... | other channels (not used) |
| 0x20 | rxlevel channel 3 |
| 0x2A | rxlevel channel 18 |
| 0, ... | other rxlevel (not used) |
| 6 | bsic channel 3 |
| 5 | bsic channel 18 |
| 0, ... | other bsic not used |
| 2,0,…. | timing advance channel 3 |
| 4,0,…. | timing advance channel 18 |
| 0, ... | other timing advance (not used) |

*/

FIELD (NCELLS_6_FTA)	6,
	44, 0,
	0x70,3,
	32, 0,
	0x0c, 3,
	20, 0,
	14, 0,
	63,
	63,
	60,
	55,
	35,
	31,
	9,
	15,
	13,
	11,
	9,
	15,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0
ENDFIELD (NCELLS_6_FTA, 73)

FIELD (NCELLS_4)	1,
	0x43, 0,
	00, 0,
	0, 0, 0, 0, 0, 0, 0, 0,
	45,
	0,
	0, 0, 0, 0,
	6,
	0,
	0, 0, 0, 0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0
ENDFIELD (NCELLS_4, 73)

FIELD (NCELLS_3)	2,
	0x03, 0,
	0x20, 0,
	0, 0, 0, 0, 0, 0, 0, 0,
	0x20,
	0x2A,
	0, 0, 0, 0,
	6,
	5,
	0, 0, 0, 0,
	2,0,0,0,0,0,0,0,
	4,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0
ENDFIELD (NCELLS_3, 73)

FIELD (NCELLS_0)	0,
	0, 0,
	0, 0,
	0, 0, 0, 0, 0, 0, 0, 0,
	0,
	0,
	0, 0, 0, 0,
	0,
	0,
	0, 0, 0, 0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0
ENDFIELD (NCELLS_0, 73)

/*

| 0x20 | rxlevel channel 3 |
| --- | --- |
| 0 | index channel 3 in neighbour cell list |
| 6 | bsic channel 3 |
| 0 | Alignment |
| 0x2A | rxlevel channel 32 |
| 1 | index channel 32 in neighbour cell list |
| 5 | bsic channel 32 |
| 0 | Alignment |
| 0, ... | other channels (not used) |

*/

FIELD (NCELLS_3_SACCH)	0x20,
	0,
	6,
	0,
	0x2A,
	1,
	5,
	0,
	0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0
ENDFIELD (NCELLS_3_SACCH, 24)

FIELD (NCELLS_3_SACCH_DG_186)	0x20,
	0,
	6,
	0,
	0x2A,
	0,
	5,
	0,
	0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0
ENDFIELD (NCELLS_3_SACCH_DG_186, 24)

FIELD (NCELLS_6_SACCH)	63,
	6,
	9,
	0,
	63,
	2,
	15,
	0,
	60,
	5,
	13,
	0,
	55,
	1,
	11,
	0,
	35,
	4,
	9,
	0,
	31,
	3,
	15,
	0,
ENDFIELD (NCELLS_6_SACCH, 24)

FIELD (MPH_NCELLS_FTA_B)	6,
	0xe1, 3,
	38, 0,
	26,0,
	0, 0,
	0xe3, 3,
	0xd4, 3,
	63,
	63,
	57,
	38,
	32,
	28,
	9,
	15,
	11,
	9,
	15,
	13,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0
ENDFIELD (MPH_NCELLS_FTA_B, 73)

FIELD (NCELLS_FTA_B)	63,
	4,
	9,
	0,
	63,
	1,
	15,
	0,
	57,
	0,
	11,
	0,
	38,
	6,
	9,
	0,
	32,
	5,
	15,
	0,
	28,
	3,
	13,
	0,
ENDFIELD (NCELLS_FTA_B, 24)

FIELD (MPH_NCELLS_FTA_C)	6,
	0xeb, 3,
	0xfc, 3,
	26,0,
	0, 0,
	0xed, 3,
	0xde, 3,
	63,
	63,
	57,
	38,
	32,
	28,
	9,
	13,
	11,
	9,
	15,
	13,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0
ENDFIELD (MPH_NCELLS_FTA_C, 73)

FIELD (NCELLS_FTA_C)	63,
	4,
	9,
	0,
	63,
	2,
	13,
	0,
	57,
	3,
	11,
	0,
	38,
	6,
	9,
	0,
	32,
	5,
	15,
	0,
	28,
	1,
	13,
	0,
ENDFIELD (NCELLS_FTA_C, 24)

FIELD (MPH_NCELLS_FTA_D)	6,
	0xeb, 3,
	38, 0,
	0xfc,3,
	26,0,
	0, 0,
	0xed, 3,
	63,
	60,
	59,
	51,
	34,
	31,
	9,
	15,
	13,
	11,
	9,
	15,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0
ENDFIELD (MPH_NCELLS_FTA_D, 73)

FIELD (NCELLS_FTA_D)	63,
	7,
	9,
	0,
	60,
	1,
	15,
	0,
	59,
	4,
	13,
	0,
	51,
	0,
	11,
	0,
	34,
	5,
	9,
	0,
	31,
	3,
	15,
	0,
ENDFIELD (NCELLS_FTA_D, 24)

FIELD (MPH_NCELLS_FTA_E)	4,
	0xeb, 3,
	38, 0,
	0xfc,3,
	26,0,
	0, 0,
	0, 0,
	63,
	63,
	63,
	57,
	0,
	0,
	9,
	15,
	13,
	11,
	0,
	0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0
ENDFIELD (MPH_NCELLS_FTA_E, 73)

FIELD (NCELLS_FTA_E)	63,
	2,
	9,
	0,
	63,
	0,
	15,
	0,
	63,
	3,
	13,
	0,
	57,
	1,
	11,
	0,
	0,
	0,
	0,
	0,
	0,
	0,
	0,
	0,
ENDFIELD (NCELLS_FTA_E, 24)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x00, ... | content (16 byte) |
| format | 0 |
| neighbour cells |  |

*/

FIELD (NCELL_DESC_1)	0x80, 0x00,
	0x00, 0x00,
	0x08, 0x00, 0x00, 0x04,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x80, 0x00, 0x00, 0x04
ENDFIELD (NCELL_DESC_1, 20)

FIELD (NCELL_DESC_1_EMO)	0x80, 0x00,
	0x00, 0x00,
	0x08, 0x00, 0x00, 0x04,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x01,
	0x80, 0x00, 0x00, 0x04
ENDFIELD (NCELL_DESC_1_EMO, 20)

FIELD (NCELL_DESC_1_V)	0x80, 0x00,
	0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x0F, 0x00, 0x00
ENDFIELD (NCELL_DESC_1_V, 20)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x00, ... | Content (16 byte) |

*/

FIELD (NCELL_DESC_5TER)	0x80, 0x00,
	0x00, 0x00,
	0xDF, 0x39, 0x80, 0x80,
	0x80, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5TER, 20)

/*26.11.2.3 bitmap 0 GSM 900

14,20,32,44*/

FIELD (NCELL_DESC_5_FTA)	0x80, 0x00,
	0x00, 0x00,
	0x10, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x08, 0x00,
	0x80, 0x08, 0x20, 0x00
ENDFIELD (NCELL_DESC_5_FTA, 20)

/*26.11.2.3 1800 in 512 format, multiband_reporting = 3

520,780,880 */

FIELD (NCELL_DESC_5TER_FTA)	0x80, 0x00,
	0x00, 0x00,
	0xF9, 0x04, 0x5A, 0x27,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5TER_FTA, 20)

FIELD (NCELL_DESC_5_FTA_A)	0x80, 0x00,
	0x00, 0x00,
	0xBB, 0xEF, 0x07, 0xFE,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5_FTA_A, 20)

FIELD (NCELL_DESC_5BIS_FTA_A)	0x80, 0x00,
	0x00, 0x00,
	0xBC, 0x00, 0x26, 0xD0,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5BIS_FTA_A, 20)

FIELD (NCELL_DESC_5_FTA_B)	0x80, 0x00,
	0x00, 0x00,
	0xBE, 0x00, 0x00, 0x00,
	0x00, 0x20, 0x02, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5_FTA_B, 20)

FIELD (NCELL_DESC_5BIS_FTA_B)	0x80, 0x00,
	0x00, 0x00,
	0xBF, 0xBA, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x80, 0x05
ENDFIELD (NCELL_DESC_5BIS_FTA_B, 20)

FIELD (NCELL_DESC_5_FTA_C)	0x80, 0x00,
	0x00, 0x00,
	0x99, 0x04, 0x7d, 0x38,
	0x80, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5_FTA_C, 20)

FIELD (NCELL_DESC_5TER_FTA_C)	0x80, 0x00,
	0x00, 0x00,
	0x97, 0xED, 0xFF, 0x0B,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5TER_FTA_C, 20)

FIELD (NCELL_DESC_5_FTA_D)	0x80, 0x00,
	0x00, 0x00,
	0x97, 0xFC, 0xF8, 0x8a,
	0x7c, 0x7d, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5_FTA_D, 20)

FIELD (NCELL_DESC_5TER_FTA_D)	0x80, 0x00,
	0x00, 0x00,
	0x9F, 0xF4, 0x10, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5TER_FTA_D, 20)

FIELD (NCELL_DESC_5_FTA_E)	0x80, 0x00,
	0x00, 0x00,
	0x9A, 0x13, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5_FTA_E, 20)

FIELD (NCELL_DESC_5TER_FTA_E)	0x80, 0x00,
	0x00, 0x00,
	0x93, 0xFC, 0xF7, 0x87,
	0x40, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_5TER_FTA_E, 20)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x00, ... | content (16 byte) |

*/

FIELD (NCELL_DESC_2)	0x80, 0x00,
	0x00, 0x00,
	0x00, 0x00, 0x00, 0x04,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x80, 0x00, 0x00, 0x04
ENDFIELD (NCELL_DESC_2, 20)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x0C, ... | content (16 byte) |

*/

FIELD (NCELL_DESC_8)	0x80, 0x00,
	0x00, 0x00,
	0x0C, 0x80, 0x20, 0x08,
	0x02, 0x00, 0x80, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x03, 0x02
ENDFIELD (NCELL_DESC_8, 20)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x8C, ... | content (16 byte) |

*/

FIELD (NCELL_DESC_128)	0x80, 0x00,
	0x00, 0x00,
	0x8C, 0x01, 0x95, 0x30,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_128, 20)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x8A, ... | content (16 byte) |

*/

FIELD (NCELL_DESC_256)	0x80, 0x00,
	0x00, 0x00,
	0x8A, 0x01, 0xB0, 0x35,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_256, 20)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x88, ... | content (16 byte) |

*/

FIELD (NCELL_DESC_512)	0x80, 0x00,
	0x00, 0x00,
	0x88, 0x01, 0x98, 0x2D,
	0x40, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_512, 20)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x80, ... | content (16 byte) |

*/

FIELD (NCELL_DESC_1024)	0x80, 0x00,
	0x00, 0x00,
	0x80, 0x18, 0xF5, 0x92,
	0xC0, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_1024, 20)

/*

| 0x80, 0x00 | length in bits |
| --- | --- |
| 0x00, 0x00 | offset in bits |
| 0x80, .. | content (16 byte) |
| Cells | 3/18/63 |

*/

FIELD (NCELL_DESC_VAR)	0x80, 0x00,
	0x00, 0x00,
	0x8E, 0x01, 0x80, 0x00,
	0x04, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x80, 0x00
ENDFIELD (NCELL_DESC_VAR, 20)

FIELD (NCELL_DESC_528_31)	0x80, 0x00,
	0x00, 0x00,
	0x8F, 0x08, 0x70, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_528_31, 20)

/*

| 0x0c | size of list |
| --- | --- |
| 0x01 | present flag |
| 1,2,2 | mobile country code 122 |
| 3, 3 | mobile network code 33 |

*/

FIELD (PLMN_122_FOUND)	0x0c,
	V_PLMN_PRES,
	1,2,2,
	3,3
ENDFIELD (PLMN_122_FOUND, 7)

FIELD (PLMN_123_FOUND)	0x0c,
	V_PLMN_PRES,
	1,2,3,
	3,3
ENDFIELD (PLMN_123_FOUND, 7)

FIELD (PLMN_123_123V_FOUND)	0x0c,
	V_PLMN_PRES,
	1,2,3,
	3,3,
	V_PLMN_PRES,
	1,2,3,
	3,2
ENDFIELD (PLMN_123_123V_FOUND, 13)

FIELD (PLMN_123_32_FOUND)	0x0c,
	V_PLMN_PRES,
	1,2,3,
	3,2
ENDFIELD (PLMN_123_32_FOUND, 7)

/*

| 0x20, 0 | channel 32 |
| --- | --- |
| 0x34, 0 | channel 52 |
| 0x52, 0 | channel 82 |
| 0xFF, 0xFF | end of hopping list |
| 0x00, ... | rest of empty list |

*/

FIELD (PRR_HOPPING_1)	0x20, 0x00,
	0x34, 0x00,
	0x52, 0x00,
	0xFF, 0xFF,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (PRR_HOPPING_1, 128)

/*

| 0x34, 0 | channel 52 |
| --- | --- |
| 0x52, 0 | channel 82 |
| 0xFF, 0xFF | end of hopping list |
| 0x00, ... | rest of empty list |

*/

FIELD (PRR_HOPPING_2)	0x34, 0x00,
	0x52, 0x00,
	0xFF, 0xFF,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00
ENDFIELD (PRR_HOPPING_2, 128)

FIELD (PRR_HOPPING_EGSM)	0x00, 0x00,
	0x03, 0x00,
	0x18, 0x00,
	0x63, 0x00,
	0xFF, 0xFF,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (PRR_HOPPING_EGSM, 128)

/*

| 0x19, 0 | channel 25 |
| --- | --- |
| 0x20, 0 | channel 32 |
| 0x34, 0 | channel 52 |
| 0x41, 0 | channel 65 |
| 0x52, 0 | channel 82 |
| 0x73, 0 | channel 115 |
| 0xFF, 0xFF | end of hopping list |
| 0x00, ... | rest of empty list |

*/

FIELD (PRR_HOPPING_3)	0x19, 0,
	0x20, 0,
	0x34, 0,
	0x41, 0,
	0x52, 0,
	0x73, 0,
	0xFF, 0xFF,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00
ENDFIELD (PRR_HOPPING_3, 128)

/*

| 0x01, 0 | channel 1 |
| --- | --- |
| 0x02, 0 | channel 2 |
| 0x03, 0 | channel 3 |
| 0x04, 0 | channel 4 |
| 0x19, 0 | channel 25 |
| 0x20, 0 | channel 32 |
| 0x21,0 | channel 33 |
| 0x34, 0 | channel 52 |
| 0x41, 0 | channel 65 |
| 0x52, 0 | channel 82 |
| 0x61, 0 | channel 97 |
| 0x73, 0 | channel 115 |
| 0xFF, 0xFF | end of hopping list |
| 0x00, ... | rest of empty list |

*/

FIELD (PRR_HOPPING_4)	0x01, 0,
	0x02, 0,
	0x03, 0,
	0x04, 0,
	0x19, 0,
	0x20, 0,
	0x21, 0,
	0x34, 0,
	0x41, 0,
	0x52, 0,
	0x61, 0,
	0x73, 0,
	0xFF, 0xFF,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (PRR_HOPPING_4, 128)

/*

| 0x01, 0 | channel 1 |
| --- | --- |
| 0x02, 0 | channel 2 |
| 0x03, 0 | channel 3 |
| 0x04, 0 | channel 4 |
| 0x19, 0 | channel 25 |
| 0x20, 0 | channel 32 |
| 0xFF, 0xFF | end of hopping list |
| 0x00, ... | rest of empty list |

*/

FIELD (PRR_HOPPING_5)	0x01, 0,
	0x02, 0,
	0x03, 0,
	0x04, 0,
	0x19, 0,
	0x20, 0,
	0xFF, 0xFF,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00
ENDFIELD (PRR_HOPPING_5, 128)

/*

| 0x01, 0 | channel 1 |
| --- | --- |
| ... | thru |
| 0x20, 0 | channel 32 |
| 0x5D, 0 | channel 93 |
| ... | thru |
| 0x7C, 0 | channel 124 |
| 0xFF, 0xFF | end of hopping list |

*/

FIELD (PRR_HOPPING_6)
	0x01, 0x00, 0x02, 0x00, 0x03, 0x00, 0x04, 0x00, 
	0x05, 0x00, 0x06, 0x00, 0x07, 0x00, 0x08, 0x00, 
	0x09, 0x00, 0x0A, 0x00, 0x0B, 0x00, 0x0C, 0x00, 
	0x0D, 0x00, 0x0E, 0x00, 0x0F, 0x00, 0x10, 0x00, 
	0x11, 0x00, 0x12, 0x00, 0x13, 0x00, 0x14, 0x00, 
	0x15, 0x00, 0x16, 0x00, 0x17, 0x00, 0x18, 0x00, 
	0x19, 0x00, 0x1A, 0x00, 0x1B, 0x00, 0x1C, 0x00, 
	0x1D, 0x00, 0x1E, 0x00, 0x1F, 0x00, 0x20, 0x00, 
	0x5D, 0x00, 0x5E, 0x00, 0x5F, 0x00, 0x60, 0x00, 
	0x61, 0x00, 0x62, 0x00, 0x63, 0x00, 0x64, 0x00, 
	0x65, 0x00, 0x66, 0x00, 0x67, 0x00, 0x68, 0x00, 
	0x69, 0x00, 0x6A, 0x00, 0x6B, 0x00, 0x6C, 0x00, 
	0x6D, 0x00, 0x6E, 0x00, 0x6F, 0x00, 0x70, 0x00, 
	0x71, 0x00, 0x72, 0x00, 0x73, 0x00, 0x74, 0x00, 
	0x75, 0x00, 0x76, 0x00, 0x77, 0x00, 0x78, 0x00, 
	0x79, 0x00, 0x7A, 0x00, 0x7B, 0x00, 0x7C, 0x00
ENDFIELD (PRR_HOPPING_6, 128)

/*

|  | channel 1,25,50,65,111,112,113,114 |
| --- | --- |

*/

FIELD (PRR_HOPPING_7)
	1, 0, 25, 0, 50, 0, 65, 0, 111, 0, 112, 0, 113, 0, 114, 0,
	0xFF, 0xFF,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDFIELD (PRR_HOPPING_7, 128)

/*

| 0x0B, ... | content channel bursts |
| --- | --- |
| 0, ... | free not used |

*/

FIELD (RACH_2_BURSTS)	0x0B,0x01,
	0,0,0,0,0,0
ENDFIELD (RACH_2_BURSTS, 8)

FIELD (RACH_8_BURSTS)	0x9b,0x91,
	0x86,0x8a,0x8f,0x82,0x97,0x9d
ENDFIELD (RACH_8_BURSTS, 8)

/*

| 0x9B, ... | content channel bursts |
| --- | --- |
| 0, ... | free not used |

*/

FIELD (RACH_2_BURSTS2)	0x9B,0x91,
	0,0,0,0,0,0
ENDFIELD (RACH_2_BURSTS2, 8)

/*

| 0x20, 0x00 | length of content |
| --- | --- |
| 0x00, 0x00. | offset of content |
| 0x42, ... | TMSI = 0x00000142 |

*/

FIELD (TMSI_1)	0x20, 0x00,
	0x00, 0x00,
	0x00, 0x00, 0x01, 0x42
ENDFIELD (TMSI_1, 8)

/*

| 0x20, 0x00 | length of content |
| --- | --- |
| 0x00, 0x00. | offset of content |
| 0xAA, ... | TMSI = 0xDDCCBBAA |

*/

FIELD (TMSI_2)	0x20, 0x00,
	0x00, 0x00,
	0xAA, 0xBB, 0xCC, 0xDD
ENDFIELD (TMSI_2, 8)

/*

| 24, 0 | 24 bit length |
| --- | --- |
| 0, 0 | 0 bit offset |
| 0x82 | P = 2, length = 2 |
| 0x10 | maio |
| 0x14 | mac |

*/

FIELD (IA_REST_1)	24, 0,
	0, 0,
	0x82,
	0x10,
	0x14
ENDFIELD (IA_REST_1, 7)

/*

| 16, 0 | 16 bit length |
| --- | --- |
| 0, 0 | 0 bit offset |
| 0xC0 | PI = 1, CBQ=1, Cell Reselect Off = 0 |
| 0x1F | temp off = 0, penalty = 31 |

*/

FIELD (SI1_REST_4_LOW)	16, 0,
	0, 0,
	0xC0,
	0x1F
ENDFIELD (SI1_REST_4_LOW, 6)

/*

| 24, 0 | 24 bit length |
| --- | --- |
| 24, 0 | 24 bit offset |
| 0,0,0 | layer 2 header |
| 0x0F | ti = 0, pd = test |
| 0x14 | message = test interface |
| 0x00 | tested device = 0 |

*/

FIELD (TEST_INTERFACE_0)	24, 0,
	24, 0,
	0,0,0,
	0x0F,
	0x14,
	0x00
ENDFIELD (TEST_INTERFACE_0, 10)

/*

| 24, 0 | 24 bit length |
| --- | --- |
| 24, 0 | 24 bit offset |
| 0,0,0 | layer 2 header |
| 0x0F | ti = 0, pd = test |
| 0x14 | message = test interface |
| 0x01 | tested device = 1 |

*/

FIELD (TEST_INTERFACE_1)	24, 0,
	24, 0,
	0,0,0,
	0x0F,
	0x14,
	0x01
ENDFIELD (TEST_INTERFACE_1, 10)

/*

| 24, 0 | 24 bit length |
| --- | --- |
| 24, 0 | 24 bit offset |
| 0,0,0 | layer 2 header |
| 0x0F | ti = 0, pd = test |
| 0x14 | message = test interface |
| 0x02 | tested device = 2 |

*/

FIELD (TEST_INTERFACE_2)	24, 0,
	24, 0,
	0,0,0,
	0x0F,
	0x14,
	0x02
ENDFIELD (TEST_INTERFACE_2, 10)

/*

| 24, 0 | 24 bit length |
| --- | --- |
| 24, 0 | 24 bit offset |
| 0,0,0 | layer 2 header |
| 0x0F | ti = 0, pd = test |
| 0x14 | message = test interface |
| 0x04 | tested device = 4 |

*/

FIELD (TEST_INTERFACE_4)	24, 0,
	24, 0,
	0,0,0,
	0x0F,
	0x14,
	0x04
ENDFIELD (TEST_INTERFACE_4, 10)

/*

| 24, 0 | 24 bit length |
| --- | --- |
| 24, 0 | 24 bit offset |
| 0,0,0 | layer 2 header |
| 0x0F | ti = 0, pd = test |
| 0x00 | message = close tch loop cmd |
| 0x01 | subchannel = 1 |

*/

FIELD (CLOSE_TCH_LOOP_A)	24, 0,
	24, 0,
	0,0,0,
	0x0F,
	0x00,
	0x01
ENDFIELD (CLOSE_TCH_LOOP_A, 10)

/*

| 24, 0 | 24 bit length |
| --- | --- |
| 24, 0 | 24 bit offset |
| 0,0,0 | layer 2 header |
| 0x0F | ti = 0, pd = test |
| 0x00 | message = close tch loop cmd |
| 0x06 | subchannel = 6 |

*/

FIELD (CLOSE_TCH_LOOP_C)	24, 0,
	24, 0,
	0,0,0,
	0x0F,
	0x00,
	0x06
ENDFIELD (CLOSE_TCH_LOOP_C, 10)

/*

| 16, 0 | 16 bit length |
| --- | --- |
| 24, 0 | 24 bit offset |
| 0,0,0 | layer 2 header |
| 0x0F | ti = 0, pd = test |
| 0x01 | message = close tch loop ack |

*/

FIELD (CLOSE_TCH_LOOP_ACK)	16, 0,
	24, 0,
	0,0,0,
	0x0F,
	0x01
ENDFIELD (CLOSE_TCH_LOOP_ACK, 9)

/*

| 16, 0 | 24 bit length |
| --- | --- |
| 24, 0 | 24 bit offset |
| 0,0,0 | layer 2 header |
| 0x0F | ti = 0, pd = test |
| 0x06 | message = open tch loop cmd |

*/

FIELD (OPEN_TCH_LOOP_CMD)	16, 0,
	24, 0,
	0,0,0,
	0x0F,
	0x06
ENDFIELD (OPEN_TCH_LOOP_CMD, 9)

/*

| 24, 0 | 24 bit length |
| --- | --- |
| 24, 0 | 24 bit offset |
| 0,0,0 | layer 2 header |
| 0x0F | ti = 0, pd = test |
| 0x06 | message = close tch loop ack |
| 0x81 | acknowledge |

*/

FIELD (OPEN_TCH_LOOP_ACK)	24, 0,
	24, 0,
	0,0,0,
	0x0F,
	0x06,
	0x81
ENDFIELD (OPEN_TCH_LOOP_ACK, 10)

/*

| 32, 0 | l_buf |
| --- | --- |
| 0, 0 | o_buf |
| 0xc0 | c2par = 1 cbq = 1 |
| 0x00 | temp off & penalty time |
| 0x2b | rest octets |
| 0x2b | rest octest |

*/

FIELD (SI4_REST_LOW)	32, 0,
	0, 0,
	0xc0,
	0x00,
	0x2b,
	0x2b
ENDFIELD (SI4_REST_LOW, 8)

FIELD (SI3_REST_LOW)	32, 0,
	0, 0,
	0xc0,
	0x00,
	0x2b,
	0x2b
ENDFIELD (SI3_REST_LOW, 8)

FIELD (SI3_REST_NO_2TER)	0x08,
	0x00,
	0x00,
	0x00,
	0x20,
	0x00,
	0x00,
	0x00
ENDFIELD (SI3_REST_NO_2TER, 8)

FIELD (SI3_REST_DEF)	0x2b,
	0x2b,
	0x2b,
	0x2b,
	0x2b,
	0x2b,
	0x2b,
	0x2b
ENDFIELD (SI3_REST_DEF, 8)

FIELD (SI3_REST_OPT_SEL_PAR)	0x80,
	0x00,
	0x2b,
	0x2b,
	0x2b,
	0x2b,
	0x2b,
	0x2b
ENDFIELD (SI3_REST_OPT_SEL_PAR, 8)

FIELD (SI3_REST_EMPTY)	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00,
	0x00
ENDFIELD (SI3_REST_EMPTY, 18)

/*

* CHM

*/

FIELD (CHM_SPEECH_FULL)

1,1
ENDFIELD (CHM_SPEECH_FULL, 2)

FIELD (CHM_DG_629)

0,0xFF
ENDFIELD (CHM_DG_629, 2)

FIELD (NCELL_DESC_EMPTY)	0x80, 0x00,
	0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDFIELD (NCELL_DESC_EMPTY, 20)

/*

* power cnf

*/

FIELD (ARFCN_24_527)

24, 0,
	15, 2,
ENDFIELD (ARFCN_24_527, 4)

FIELD (ARFCN_67_32_124)

67, 0,
	32, 0,
	124, 0
ENDFIELD (ARFCN_67_32_124, 6)

FIELD (ARFCN_67_64_32_26_124)

67, 0,
	64, 0,
	32, 0,
	26, 0,
	124, 0
ENDFIELD (ARFCN_67_64_32_26_124, 10)

FIELD (ARFCN_64_DCS832_PCS640MSB)

64, 0,
	0x40, 0x03,
	0x80, 0x82,
ENDFIELD (ARFCN_64_DCS832_PCS640MSB, 6)

FIELD (ARFCN_DCS832_PCS640MSB_64)

0x40, 0x03,
	0x80, 0x82,
	0x40, 0
ENDFIELD (ARFCN_DCS832_PCS640MSB_64, 6)

FIELD (ARFCN_PCS640MSB_64_DCS832)

0x80, 0x82,
	0x40, 0,
	0x40, 0x03
ENDFIELD (ARFCN_PCS640MSB_64_DCS832, 6)

FIELD (RXLEV_22_21)

0x22,0x21
ENDFIELD (RXLEV_22_21, 2)

FIELD(MM_MESSAGE)	24,0,
	24,0,
	0,0,0,
	3,4,5
ENDFIELD(MM_MESSAGE,10)

FIELD (RXLEV_22_21_20)

0x22,0x21,0x20
ENDFIELD (RXLEV_22_21_20, 3)

FIELD (RXLEV_24_23_22_13_10)

0x24,0x23,0x22,0x13,0x10
ENDFIELD (RXLEV_24_23_22_13_10, 5)

FIELD (RXLEV_10_8_4)

0x10,8,4
ENDFIELD (RXLEV_10_8_4, 3)

FIELD (RXLEVEL_22)

0x0c, 0x22
ENDFIELD (RXLEVEL_22, 2)

FIELD (RXLEVEL_21)

0x0c, 0x21
ENDFIELD (RXLEVEL_21, 2)

FIELD (RXLEVEL_20)

0x0c, 0x20
ENDFIELD (RXLEVEL_20, 2)

FIELD (RXLEVEL_22_21)

0x0c, 0x22, 0x21
ENDFIELD (RXLEVEL_22_21, 3)

- FIELD (EXT_MEAS_FREQ_000)

- 0x80,0x00,

- 0x00,0x00,

0x18,0x00,0x00,0x00,

0x00,0x00,0x00,0x00,

0x00,0x00,0x60,0x00,

0x00,0x00,0x00,0x01

- ENDFIELD (EXT_MEAS_FREQ_000,20)

- FIELD (EXT_MEAS_FREQ_001)

- 0x80,0x00,

- 0x00,0x00,

0x83,0x00,0xFF,0xC0,

0x00,0x00,0x00,0x00,

0x00,0x00,0x00,0x00,

0x00,0x00,0x00,0x00

- ENDFIELD (EXT_MEAS_FREQ_001,20)

- FIELD (EXT_MEAS_FREQ_002)

- 0x80,0x00,

- 0x00,0x00,

0x87,0x00,0xFF,0xC0,

0x00,0x00,0x00,0x00,

0x00,0x00,0x00,0x00,

0x00,0x00,0x00,0x00

- ENDFIELD (EXT_MEAS_FREQ_002,20)

- FIELD (EXT_MEAS_FREQ_003)

- 0x80,0x00,

- 0x00,0x00,

0x1F,0xFF,0xFF,0xFF,

0xFF,0xFF,0xFF,0xFF,

0xFF,0xFF,0xFF,0xFF,

0xFF,0xFF,0xFF,0xFF

- ENDFIELD (EXT_MEAS_FREQ_003,20)

- FIELD (EXT_MEAS_FREQ_004)

- 0x80,0x00,

- 0x00,0x00,

0x08,0x00,0x00,0x00,

0x00,0x00,0x00,0x00,

0x00,0x00,0x60,0x00,

0x00,0x00,0x00,0x01

- ENDFIELD (EXT_MEAS_FREQ_004,20)

/*

* APDU with 247 Bytes, 1976 Bits, 0x7B8 bits

*/

FIELD (APDU_MSG_RRRLP_820)

0xB8,0x07,

0x00,0x00,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88

ENDFIELD (APDU_MSG_RRRLP_820, 252)

/*

* Layer 2 Frame with 251 Bytes, 2008 Bits, 0x7D8 bits

* PD=0x06, msg type=0x038, flags: c/r=1 first=!0 last=!0 -> 0x40, len=0xF7

*/

FIELD (APDU_MSG_DL_820)

0xD8,0x07,

0x00,0x00,

0x06, 0x38, 0x40, 0xF7,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88

ENDFIELD (APDU_MSG_DL_820, 255)

/*

* APDU with 248 Bytes, 1984 Bits, 0x7C0 bits

*/

FIELD (APDU_MSG_RRRLP_821)

0xC0,0x07,

0x00,0x00,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88, 0x99

ENDFIELD (APDU_MSG_RRRLP_821, 253)

/*

* Layer 2 Frame with 251 Bytes, 2008 Bits, 0x7D8 bits

* PD=0x06, msg type=0x038, flags: c/r=0 first=!0 last= !1 ->0x10, len=0xF7

*/

FIELD (APDU_MSG_DL_821a)

0xD8,0x07,

0x00,0x00,

0x06, 0x38, 0x10, 0xF7,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88

ENDFIELD (APDU_MSG_DL_821a, 255)

/*

* Layer 2 Frame with 5 Byte, 40 Bits, 0x28 bits

* PD=0x06, msg type=0x038, flags: c/r=1 first=!1 last=!0 -> 0x60, len=0x01

*/

FIELD (APDU_MSG_DL_821b)

0x28,0x00,

0x00,0x00,

0x06, 0x38, 0x60, 0x01,

0x99

ENDFIELD (APDU_MSG_DL_821b, 9)

/*

* APDU with 741 Bytes, 5928 Bits, 0x1728 bits

*/

FIELD (APDU_MSG_RRRLP_822)

0x28,0x17,

0x00,0x00,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88,

ENDFIELD (APDU_MSG_RRRLP_822, 745)

/*

* First of 3 Layer 2 Frames:

* Layer 2 Frame with 251 Bytes, 2008 Bits, 0x7D8 bits

* PD=0x06, msg type=0x038, flags: c/r=0 first=!0 last= !1 ->0x10, len=0xF7

*/

FIELD (APDU_MSG_DL_822a)

0xD8,0x07,

0x00,0x00,

0x06, 0x38, 0x10, 0xF7,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88

ENDFIELD (APDU_MSG_DL_822a, 255)

/*

* Second of 3 Layer 2 Frames:

* Layer 2 Frame with 251 Bytes, 2008 Bits, 0x7D8 bits

* PD=0x06, msg type=0x038, flags: c/r=0 first=!1 last= !1 ->0x30, len=0xF7

*/

FIELD (APDU_MSG_DL_822b)

0xD8,0x07,

0x00,0x00,

0x06, 0x38, 0x30, 0xF7,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88

ENDFIELD (APDU_MSG_DL_822b, 255)

/*

* Second of 3 Layer 2 Frames:

* Layer 2 Frame with 251 Bytes, 2008 Bits, 0x7D8 bits

* PD=0x06, msg type=0x038, flags: c/r=1 first=!1 last= !0 ->0x60, len=0xF7

*/

FIELD (APDU_MSG_DL_822c)

0xD8,0x07,

0x00,0x00,

0x06, 0x38, 0x60, 0xF7,

0xAA, 0xBB, 0xCC, 0xDD,

0x00,0x01,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,0x0A,0x0B,0x0C,0x0D,0x0E,0x0F,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0x50,0x51,0x52,0x53,0x54,0x55,0x56,0x57,0x58,0x59,0x5A,0x5B,0x5C,0x5D,0x5E,0x5F,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0x90,0x91,0x92,0x93,0x94,0x95,0x96,0x97,0x98,0x99,0x9A,0x9B,0x9C,0x9D,0x9E,0x9F,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xD0,0xD1,0xD2,0xD3,0xD4,0xD5,0xD6,0xD7,0xD8,0xD9,0xDA,0xDB,0xDC,0xDD,0xDE,0xDF,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0x66, 0x77, 0x88

ENDFIELD (APDU_MSG_DL_822c, 255)

/*

* First frame containing a segmented APDU (copied and modified from APDU_MSG_DL_822a)

* Layer 2 Frame with 251 Bytes, 2008 Bits, 0x7D8 bits

* PD=0x06, msg type=0x038, flags: c/r=0 first=!0 last= !1 ->0x10, len=0xF7

*/

FIELD (APDU_MSG_DL_827)

0xD8,0x07,

0x00,0x00,

0x06, 0x38, 0x10, 0xF7,

0x11, 0x22, 0x33, 0x44,

0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,

0x10,0x11,0x12,0x13,0x14,0x15,0x16,0x17,0x18,0x19,0x1A,0x1B,0x1C,0x1D,0x1E,0x1F,

0x20,0x21,0x22,0x23,0x24,0x25,0x26,0x27,0x28,0x29,0x2A,0x2B,0x2C,0x2D,0x2E,0x2F,

0x30,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x38,0x39,0x3A,0x3B,0x3C,0x3D,0x3E,0x3F,

0x40,0x41,0x42,0x43,0x44,0x45,0x46,0x47,0x48,0x49,0x4A,0x4B,0x4C,0x4D,0x4E,0x4F,

0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,

0x60,0x61,0x62,0x63,0x64,0x65,0x66,0x67,0x68,0x69,0x6A,0x6B,0x6C,0x6D,0x6E,0x6F,

0x70,0x71,0x72,0x73,0x74,0x75,0x76,0x77,0x78,0x79,0x7A,0x7B,0x7C,0x7D,0x7E,0x7F,

0x80,0x81,0x82,0x83,0x84,0x85,0x86,0x87,0x88,0x89,0x8A,0x8B,0x8C,0x8D,0x8E,0x8F,

0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,

0xA0,0xA1,0xA2,0xA3,0xA4,0xA5,0xA6,0xA7,0xA8,0xA9,0xAA,0xAB,0xAC,0xAD,0xAE,0xAF,

0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9,0xBA,0xBB,0xBC,0xBD,0xBE,0xBF,

0xC0,0xC1,0xC2,0xC3,0xC4,0xC5,0xC6,0xC7,0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,

0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,0xaa,

0xE0,0xE1,0xE2,0xE3,0xE4,0xE5,0xE6,0xE7,0xE8,0xE9,0xEA,0xEB,0xEC,0xED,0xEE,0xEF,

0xAA, 0xBB, 0xCC

ENDFIELD (APDU_MSG_DL_827, 255)

/*

* Layer 2 Frame with 11 Bytes, 88 Bits, 0x58 bits

* PD=0x06, msg type=0x038, flags: c/r=0 first=!1 last= !1 ->0x30, len=0x07

*/

FIELD (APDU_MSG_DL_828a)

0x58,0x00,

0x00,0x00,

0x06, 0x38, 0x30, 0x07,

0xAA, 0xBB, 0xCC, 0xDD,

0x66, 0x77, 0x88

ENDFIELD (APDU_MSG_DL_828a, 15)

/*

* Layer 2 Frame with 11 Bytes, 88 Bits, 0x58 bits ( same like APDU_MSG_DL_828b with diff. c/r )

* PD=0x06, msg type=0x038, flags: c/r=0 first=!1 last= !0 ->0x20, len=0x07

*/

FIELD (APDU_MSG_DL_828b)

0x58,0x00,

0x00,0x00,

0x06, 0x38, 0x30, 0x07,

0xAA, 0xBB, 0xCC, 0xDD,

0x66, 0x77, 0x99

ENDFIELD (APDU_MSG_DL_828b, 15)

FIELD (IMM_ASS_152)
	0xB0, 0x00, 0x18, 0x00,
	0x00, 0x00, 0x00, 
	0x06, 0x3F, 0x00, 0x2C, 0x40, 0x73, 0x91, 0x21,
	0x06, 0x1B, 0x01, 0x16, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDFIELD (IMM_ASS_152, 29)

FIELD (IMM_ASS_152_SPARE_900)
	0xB0, 0x00, 0x18, 0x00,
	0x00, 0x00, 0x00, 
	0x06, 0x3F, 0x8C, 0x2C, 0x4C, 0x73, 0x91, 0x21,
	0x06, 0xDB, 0x00, 0x29, 0xAB, 0xAB, 0xAB, 0xAB,
	0xAB, 0xAB, 0xAB, 0xAB, 0xAB, 0xAB
ENDFIELD (IMM_ASS_152_SPARE_900,29)

FIELD (IMM_ASS_152_SPARE_1900_CONQUEST_8479)
	0xB0, 0x00, 0x18, 0x00,
	0x00, 0x00, 0x00, 
	0x06, 0x3F, 0x84, 0x20, 0xA6, 0x4E, 0x91, 0x21,
	0x06, 0x40, 0x00, 0x29, 0xAB, 0xAB, 0xAB, 0xAB,
	0xAB, 0xAB, 0xAB, 0xAB, 0xAB, 0xAB

ENDFIELD (IMM_ASS_152_SPARE_1900_CONQUEST_8479,29)

/*

## Declarations

*/

DECLARATION (A_MPH_NCELL_1F)
DECLARATION (A_MAC_IA)
DECLARATION (A_MPH_NCELL_1E)

DECLARATION (S_COD_PROP_9_ALC)

DECLARATION (S_COD_PROP_8_ALC)

DECLARATION (S_COD_PROP_7_ALC)

DECLARATION (S_COD_PROP_6_ALC)

DECLARATION (S_COD_PROP_5_ER)

DECLARATION (S_COD_PROP_4_ER)

DECLARATION (SA_cod_prop_M2_ALC)

DECLARATION (SA_cod_prop_M4_ALC)

DECLARATION (SA_cod_prop_M4_ER)

DECLARATION (SA_cod_prop_M3_ER)
DECLARATION (SA_cod_prop_M2_ALC_ALR)

DECLARATION (SA_cod_prop_M4_ALC_ALR)

DECLARATION (SA_cod_prop_M4_ER_ALR)

DECLARATION (SA_cod_prop_M3_ER_ALR)
DECLARATION (S_multirate_conf_ER_AHS)
DECLARATION (S_multirate_conf_ER_AfS)

DECLARATION (S_multirate_conf_ALC_AHS)

DECLARATION (S_multirate_conf_ALC_AFS)
DECLARATION (S_AMR_conf_5_ER_AFS)
DECLARATION (S_AMR_conf_6_ALC_AFS)
DECLARATION (S_AMR_conf_6_ALC_AFS2)

DECLARATION (S_AMR_conf_7_ER_AHS)

DECLARATION (S_AMR_conf_8_ALC_AHS)

DECLARATION (CHANNEL_DESC_FACCH3_HALF)

DECLARATION (PRR_CHANNEL_TYPE_HALF)
DECLARATION (PRR_CHANNEL_TYPE_HOP_HALF)

DECLARATION (PRR_CHANNEL_TYPE_HOP_HALF2)
DECLARATION (CHANNEL_DESC_FACCH4_HALF)

DECLARATION (MOBILE_ALLOCATION_1800_FAIL)
DECLARATION (A_RXLEVEL_20)
DECLARATION (MOB_ALLOC_ONE_CHAN)
DECLARATION (MOBILE_ALLOCATION_ONE_CHAN)
DECLARATION (CHANNEL_DESC_SDCCH2_1900_1)
DECLARATION (PRR_CHANNEL_TYPE_3_1900)

DECLARATION(A_BCCH_INFO_DG_629a)

DECLARATION(S_BCCH_INFO_DG_629a)

DECLARATION (S_NCELL_63_8_1)
DECLARATION (S_NCELL_63_3_7)
DECLARATION (S_NCELL_61_6_5)
DECLARATION (S_NCELL_56_7_3)
DECLARATION (S_NCELL_36_5_1)
DECLARATION (S_NCELL_32_1_3)
DECLARATION (S_NCELL_63_3_9)
DECLARATION (S_NCELL_63_6_15)
DECLARATION (S_NCELL_61_5_11)
DECLARATION (S_NCELL_45_1_9)
DECLARATION (S_NCELL_45_1_913)
DECLARATION (S_NCELL_35_4_13)
DECLARATION (A_ARFCN_NCELLS_1900)
DECLARATION (A_RXLEV_NCELLS_1900)
DECLARATION (A_BSIC_NCELLS_1900)
DECLARATION (AS_NCELLS_3_SACCH_1900)
DECLARATION (S_NCELLS_6)
DECLARATION (MEAS_RESULT_NCELL_1900)
DECLARATION(A_MPH_NCELL_1900)
DECLARATION(A_MPH_NCELL_1900a)
DECLARATION(BCCH_FREQ_LIST_1900)
DECLARATION(BCCH_FREQ_LIST_1900_bis)
DECLARATION(MOB_ALLOC_1800_FAIL)
DECLARATION(MOB_CLASS_1800)
DECLARATION(MOB_CLASS_1900)
DECLARATION(PRR_CHANNEL_TYPE_7_1800)
DECLARATION(PRR_CHANNEL_TYPE_6_1800)
DECLARATION(PRR_HOPPING_2_1800)
DECLARATION(PRR_CHANNEL_TYPE_3_CB_1800)
DECLARATION(PRR_CHANNEL_TYPE_3_1800)
DECLARATION(CHANNEL_DESC_SDCCH2_1800_1)
DECLARATION (CELL_DESC_1)
DECLARATION (POW_CMD_7)
DECLARATION (MOBILE_ALLOCATION_1800)
DECLARATION (MOB_ALLOC_1800)
DECLARATION (CHAN_DESC_13_1)
DECLARATION (CHAN_LIST_13_1)
DECLARATION (FREQ_LIST_AFTER_13_1)
DECLARATION (CELL_DESC_BAD_BCCH)
DECLARATION (CLASS_MS)
DECLARATION (CLASS_MS_DUALBAND)
DECLARATION (CLASS_MS_DG_081C)
DECLARATION (CLASS_MS_1800)
DECLARATION (CIPHERING_2)
DECLARATION (CIPHERING_3)
DECLARATION (FRAME_NUMBER_1)
DECLARATION (HO_PARAM_0)
DECLARATION (HO_PARAM_1)
DECLARATION (KCV_EMPTY)
DECLARATION (KCV_00112233)
DECLARATION (KCV_12345678)
DECLARATION (MM_INFO_1)
DECLARATION (MM_INFO_2)
DECLARATION (MM_INFO_3)
DECLARATION (MM_INFO_15)
DECLARATION(CHANNEL_DESC_SDCCH2_1800)
DECLARATION (MM_INFO_4_DG_629)
DECLARATION (MOBILE_ID_IMSI_HPLMN)
DECLARATION (MOBILE_ID_NOT_SET)
DECLARATION (MOBILE_ID_TMSI)
DECLARATION (MOBILE_ID_IMSI_TEST)
DECLARATION (NO_CIPHERING)
DECLARATION (NO_STARTING_TIME)
DECLARATION (PAGING_NORMAL)
DECLARATION (PLMN_ID_EMPTY)
DECLARATION (PLMN_ID_123)
DECLARATION (PLMN_ID_123X)
DECLARATION (PLMN_ID_TEST)
DECLARATION (PLMN_ID_123_V)
DECLARATION (PLMN_ID_122)
DECLARATION (PRR_CHANNEL_TYPE_0)
DECLARATION (PRR_CHANNEL_TYPE_2)
DECLARATION (PRR_CHANNEL_TYPE_3)
DECLARATION (PRR_CHANNEL_TYPE_3_CB)
DECLARATION (PRR_CHANNEL_TYPE_4)
DECLARATION (PRR_CHANNEL_TYPE_5)
DECLARATION (PRR_CHANNEL_TYPE_6)
DECLARATION (PRR_CHANNEL_TYPE_7)
DECLARATION (PRR_CHANNEL_TYPE_8)
DECLARATION (PRR_CHANNEL_TYPE_9)
DECLARATION (PRR_CHANNEL_TYPE_10)
DECLARATION (PRR_CHANNEL_TYPE_11)
DECLARATION (PRR_CHANNEL_TYPE_EGSM)
DECLARATION (PRR_TR_PARA_2)
DECLARATION (PRR_TR_PARA_2B)
DECLARATION (PRR_TR_PARA_3)
DECLARATION (PRR_TR_PARA_13)
DECLARATION (PRR_TR_PARA_HO)
DECLARATION (PRR_TR_PARA_ASY_HO)
DECLARATION (CELL_ALLOC_1024)
DECLARATION (PRR_TR_PARA_AMR)
DECLARATION (PRR_TR_PARA_AMR_1)
DECLARATION (SEND_MODE_2_BURSTS)
DECLARATION (SEND_MODE_2_BURSTS2)
DECLARATION (SEND_MODE_8_BURSTS)
DECLARATION (SEND_MODE_NO_BURSTS)
DECLARATION (STARTING_TIME_1)
DECLARATION (OP_MODE_EMPTY)
DECLARATION (OP_MODE_EMPTY_NO_SERV)
DECLARATION (OP_MODE_NORMAL)
DECLARATION (OP_MODE_NORMAL_NO_SERV)
DECLARATION (OP_MODE_TEST_SIM)
DECLARATION (OP_MODE_TEST_SIM_NO_SERV)
DECLARATION (OP_MODE_TEST_SIM_LIM_SERV)
DECLARATION (OP_MODE_NET_SRCH_MMI_NO_SRV)
DECLARATION (OP_MODE_NET_SRCH_MMI_LIM_SRV)
DECLARATION (OP_MODE_NET_SRCH_MMI)
DECLARATION (OP_MODE_NO_SIM_LIM_SERV)
DECLARATION (CELL_OPT_BCCH_1)
DECLARATION (CELL_SELECT_1)
DECLARATION (CELL_SELECT_2)
DECLARATION (CELL_SELECT_3)
DECLARATION (CHANNEL_DESC_SDCCH)
DECLARATION (CHANNEL_DESC_SDCCH2)
DECLARATION (CHANNEL_DESC_BAD)
DECLARATION (CHANNEL_DESC_FACCH2)
DECLARATION (CHANNEL_DESC_HALFRATE)
DECLARATION (CHANNEL_DESC_FACCH3)
DECLARATION (CHANNEL_DESC_FACCH4)
DECLARATION (CHANNEL_NEEDED_1)
DECLARATION (CIPH_MODE_ON)
DECLARATION (CIPH_MODE_OFF)
DECLARATION (CIPH_RESP_NO_IMEI)
DECLARATION (CIPH_RESP_WITH_IMEI)
DECLARATION (CKSN_RESERVED)
DECLARATION (CTRL_CHAN_DESC_1)
DECLARATION (FREQ_CHAN_SEQ_1)
DECLARATION (LOC_AREA_IDENT_122_36_2147)
DECLARATION (LOC_AREA_IDENT_122_35_2147)
DECLARATION (LOC_AREA_IDENT_122_34_2147)
DECLARATION (LOC_AREA_IDENT_122_2147)
DECLARATION (LOC_AREA_IDENT_122_2147_V)
DECLARATION (LOC_AREA_IDENT_123_2147)
DECLARATION (LOC_AREA_IDENT_123_2147_V)
DECLARATION (LOC_AREA_IDENT_123_2148)
DECLARATION (MEAS_RESULT_NCELL_0)
DECLARATION (MEAS_RESULT_NCELL_3)
DECLARATION (MEAS_RESULT_NCELL_3_DG_186)
DECLARATION (MEAS_RESULT_NCELL_6_FTA)
DECLARATION (MEAS_RESULT_NCELL_FTA_B)
DECLARATION (MEAS_RESULT_NCELL_FTA_C)
DECLARATION (MEAS_RESULT_NCELL_FTA_D)
DECLARATION (MEAS_RESULT_NCELL_FTA_E)
DECLARATION (MOB_CLASS2_NORMAL)
DECLARATION (MOB_CLASS2_900)
DECLARATION (MOB_CLASS2_EGSM)
DECLARATION (MOB_CLASS2_212)
DECLARATION (MOBILE_ALLOCATION_1)
DECLARATION (MOBILE_ALLOCATION_2)
DECLARATION (MOBILE_ALLOCATION_3)
DECLARATION (MOBILE_ALLOCATION_4)
DECLARATION (MOBILE_ALLOCATION_EGSM)
DECLARATION (MOBILE_IDENTITY_TMSI)
DECLARATION (MOBILE_IDENTITY_IMEISV)
DECLARATION (MOBILE_IDENTITY_IMSI2)
DECLARATION (MOBILE_IDENTITY_IMSI_HPLMN)
DECLARATION (MOBILE_IDENTITY_IMSI_TEST)
DECLARATION (POW_05_HO)
DECLARATION (POWER_COMMAND_05)
DECLARATION (RACH_CTRL_1)
DECLARATION (RACH_CTRL_2)
DECLARATION (REQUEST_REFERENCE_1)
DECLARATION (REQUEST_REFERENCE_2)
DECLARATION (REQUEST_REFERENCE_3)
DECLARATION (START_TIME_1)
DECLARATION (SYNCH_IND_0)
DECLARATION (SYNCH_IND_1)
DECLARATION (SYNCH_IND_2)
DECLARATION (TIMING_ADVANCE_10)
DECLARATION (TIMING_ADVANCE_27)

- /* new declarations */
/* using old names +  a prefix */
/* S_ to denote the new element as a struct */
/* A_ to denote the new element as an array */
/* SA_ to denote the new element as an array of structs */
DECLARATION (S_BCCH_INFO_EMPTY)
 DECLARATION (A_BCCH_INFO_EMPTY)
DECLARATION (S_BCCH_INFO_CLEAR)
DECLARATION (S_BCCH_INFO_32)
DECLARATION (A_BCCH_INFO_32)
DECLARATION (S_BCCH_INFO_NO_24)
DECLARATION (S_BCCH_INFO_24)
DECLARATION (A_BCCH_INFO_24)
DECLARATION (S_BCCH_INFO_DG_081)
DECLARATION (S_BCCH_INFO_DG_091)
DECLARATION (S_BCCH_INFO_DG_629)
DECLARATION (A_BCCH_INFO_DG_081)
DECLARATION (A_BCCH_INFO_DG_629)
DECLARATION (S_BCCH_INFO_NCELL_DESC_1)
DECLARATION (A_BCCH_INFO_NCELL_DESC_1)
DECLARATION (S_SI3_REST_EMPTY)
DECLARATION (S_SI4_REST_EMPTY)
DECLARATION (S_SI3_REST_LOW)
DECLARATION (S_SI4_REST_LOW)
DECLARATION (S_SI34_OPT_SEL_PAR)
DECLARATION (A_LAC_LIST1)
DECLARATION (A_LAC_LIST1A)
DECLARATION (A_LAC_LIST5)
DECLARATION (A_MPH_NCELL_1)
DECLARATION (A_MPH_NCELL_1_V)
DECLARATION (A_MPH_NCELL_1B)
DECLARATION (A_MPH_NCELL_1C)
DECLARATION (A_MPH_NCELL_1D)
DECLARATION (A_MPH_NCELL_1D_EMO)
DECLARATION (A_MPH_NCELL_2)
DECLARATION (A_MPH_NCELL_8)
DECLARATION (S_MS_ID_IMSI_HPLMN_TMSI)
DECLARATION (S_MS_ID_NO_IMSI_NO_TMSI)
DECLARATION (S_MS_ID_IMSI_HPLMN_NO_TMSI)
DECLARATION (S_MS_ID_IMSI_TEST_TMSI)
DECLARATION (A_IMSI_CONTENT)
DECLARATION (S_BCCH_INFO_1_PLUS_43_V)
DECLARATION (A_BCCH_INFO_1_PLUS_43_V)
DECLARATION (S_PLMN_122_FOUND)
DECLARATION (S_PLMN_EMPTY)
DECLARATION (AS_PLMN)
DECLARATION (A_RXLEVEL_22)
DECLARATION (S_CHM_DG_629)
DECLARATION (A_MPH_NCELL_528_31)
DECLARATION (S_NCELLS_3)
DECLARATION (S_NCELLS_667_0)
DECLARATION (S_NCELLS_4)

- DECLARATION (A_ARFCN_NCELLS_3)
DECLARATION (A_RX_NCELLS_3)
DECLARATION (A_BSIC_NCELLS_3)
DECLARATION (A_TIME_NCELLS_3)
DECLARATION (A_FRAME_NCELLS_3)

- DECLARATION (A_ARFCN_NCELLS_667_0)
DECLARATION (A_RX_NCELLS_667_0)
DECLARATION (A_BSIC_NCELLS_667_0)
DECLARATION (A_TIME_NCELLS_667_0)
DECLARATION (A_FRAME_NCELLS_667_0)

- DECLARATION (A_ARFCN_NCELLS_6_FTA)
DECLARATION (A_RX_NCELLS_6_FTA)
DECLARATION (A_BSIC_NCELLS_6_FTA)
DECLARATION (A_TIME_NCELLS_6_FTA)
DECLARATION (S_IA_REST_1)
DECLARATION (S_IA_REST_OCT_PAR)
DECLARATION (S_IA_FREQ_PAR)
DECLARATION (AS_NCELLS_3_SACCH)
DECLARATION (AS_NCELLS_6_SACCH)
DECLARATION (A_MPH_NCELL_DG_186)
DECLARATION (AS_NCELLS_3_SACCH_DG_186)
DECLARATION (A_MPH_NCELL_5TER_FTA)
DECLARATION (A_MPH_NCELL_5TER_5_FTA)
DECLARATION (S_NCELLS_6_FTA)
DECLARATION (A_MPH_NCELL_5_FTA)
DECLARATION (A_MPH_NCELL_5_FTA_A)
DECLARATION (A_MPH_NCELL_5_5BIS_FTA_A)
DECLARATION (A_MPH_NCELL_5_FTA_B)
DECLARATION (A_MPH_NCELL_5_5BIS_FTA_B)
DECLARATION (A_MPH_NCELL_5_FTA_C)
DECLARATION (A_MPH_NCELL_5_5TER_FTA_C)
DECLARATION (A_MPH_NCELL_5_FTA_D)
DECLARATION (A_MPH_NCELL_5_5TER_FTA_D)
DECLARATION (A_MPH_NCELL_5_FTA_E)
DECLARATION (A_MPH_NCELL_5_5TER_FTA_E)
DECLARATION (S_MPH_NCELLS_FTA_B)
DECLARATION (S_MPH_NCELLS_FTA_D)
DECLARATION (S_MPH_NCELLS_FTA_E)
DECLARATION (A_ARFCN_MPH_NCELLS_FTA_B)
DECLARATION (A_RX_MPH_NCELLS_FTA_B)
DECLARATION (A_BSIC_MPH_NCELLS_FTA_B)
DECLARATION (A_TIME_MPH_NCELLS_FTA_B)
DECLARATION (AS_NCELLS_FTA_B)
DECLARATION (S_NCELL_63_2_9)
DECLARATION (S_NCELL_63_0_15)
DECLARATION (S_NCELL_63_3_13)
DECLARATION (S_NCELL_63_6_9)
DECLARATION (S_NCELL_63_2_15)
DECLARATION (S_NCELL_63_4_9)
DECLARATION (S_NCELL_63_1_15)
DECLARATION (S_NCELL_63_2_13)
DECLARATION (S_NCELL_63_7_9)
DECLARATION (S_NCELL_60_5_13)
DECLARATION (S_NCELL_60_1_15)
DECLARATION (S_NCELL_59_4_13)
DECLARATION (S_NCELL_57_1_11)
DECLARATION (S_NCELL_57_0_11)
DECLARATION (S_NCELL_57_3_11)
DECLARATION (S_NCELL_55_1_11)
DECLARATION (S_NCELL_51_0_11)
DECLARATION (S_NCELL_42_1_5)
DECLARATION (S_NCELL_42_0_5)
DECLARATION (S_NCELL_38_6_9)
DECLARATION (S_NCELL_35_4_9)
DECLARATION (S_NCELL_34_5_9)
DECLARATION (S_NCELL_32_5_15)
DECLARATION (S_NCELL_32_0_6)
DECLARATION (S_NCELL_31_3_15)
DECLARATION (S_NCELL_28_1_13)
DECLARATION (S_NCELL_28_3_13)
DECLARATION (S_NCELL_0_0_0)
DECLARATION (S_MPH_NCELLS_FTA_C)
DECLARATION (A_ARFCN_MPH_NCELLS_FTA_C)
DECLARATION (A_BSIC_MPH_NCELLS_FTA_C)
DECLARATION (A_ARFCN_MPH_NCELLS_FTA_D)
DECLARATION (A_RX_MPH_NCELLS_FTA_D)
DECLARATION (A_BSIC_MPH_NCELLS_FTA_D)
DECLARATION (A_ARFCN_MPH_NCELLS_FTA_E)
DECLARATION (A_RX_MPH_NCELLS_FTA_E)
DECLARATION (A_BSIC_MPH_NCELLS_FTA_E)
DECLARATION (AS_NCELLS_FTA_C)
DECLARATION (AS_NCELLS_FTA_D)
DECLARATION (AS_NCELLS_FTA_E)
DECLARATION (S_NCELLS_0)
DECLARATION (A_ARFCN_NCELLS_0)
DECLARATION (A_RX_NCELLS_0)
DECLARATION (A_BSIC_NCELLS_0)
DECLARATION (A_ARFCN_NCELLS_4)
DECLARATION (A_RX_NCELLS_4)
DECLARATION (A_BSIC_NCELLS_4)
DECLARATION (S_CHM_SPEECH_FULL)
DECLARATION (S_CHM_SPEECH_AMR_FULL)
 DECLARATION (S_BCCH_INFO_DG_212)
DECLARATION (S_BCCH_INFO_DG_213)
DECLARATION (A_BCCH_INFO_DG_212)
DECLARATION (A_BCCH_INFO_DG_213)
DECLARATION (A_FREQ_LIST_AFTER_2)
DECLARATION (S_FREQ_LIST_AFTER_2)
DECLARATION (A_FREQ_LIST_BEFORE_1)
DECLARATION (S_FREQ_LIST_BEFORE_1)
DECLARATION (A_MPH_NCELL_EMPTY)
DECLARATION (A_IMSI_CONTENT_TEST_TMSI)
DECLARATION (A_RXLEVEL_21)
DECLARATION (AS_PLMN_123_32_FOUND)
DECLARATION (S_PLMN_123_32_FOUND)
DECLARATION (SA_COD_PROP_M1)
DECLARATION (SA_COD_PROP_M2)
DECLARATION (SA_COD_PROP_M3)
DECLARATION (SA_COD_PROP_M4)
DECLARATION (SA_COD_PROP_1)
DECLARATION (SA_COD_PROP_2)
DECLARATION (SA_COD_PROP_3)
DECLARATION (SA_COD_PROP_4)
DECLARATION (SA_COD_PROP_702D)
DECLARATION (SA_COD_PROP_702E)
DECLARATION (S_multirate_conf_1)
DECLARATION (S_multirate_conf_2)
DECLARATION (S_multirate_conf_3)
DECLARATION (S_multirate_conf_4)
DECLARATION (S_multirate_conf_4_ICMI)
DECLARATION (S_multirate_conf_4_702A)
DECLARATION (S_multirate_conf_4_702B)
DECLARATION (S_multirate_conf_4_702C)
DECLARATION (S_multirate_conf_4_702D)
DECLARATION (S_multirate_conf_4_702E)
DECLARATION (S_AMR_conf_0)
DECLARATION (S_AMR_conf_1)
DECLARATION (S_AMR_conf_2)
DECLARATION (S_AMR_conf_3)
DECLARATION (S_AMR_conf_4)
DECLARATION (S_AMR_conf_4_ICMI)
DECLARATION (S_COD_PROP_0)
DECLARATION (S_COD_PROP_1)
DECLARATION (S_COD_PROP_2)
DECLARATION (S_COD_PROP_3)
DECLARATION (A_ARFCN_527_601)

/* DL/26.09.02: EOTD */

/* DECLARATIONS: arrays */

DECLARATION (eotd_crosscor_12)

/* DECLARATIONS: structs – primitives */

DECLARATION (ARFCN_14_EOTD)

DECLARATION (ARFCN_124_EOTD)

DECLARATION (ARFCN_1_EOTD)

DECLARATION (ARFCN_32_EOTD)

DECLARATION (ARFCN_26_EOTD)

DECLARATION (ARFCN_64_EOTD)

DECLARATION (eotd_sc_res_ok)

DECLARATION (eotd_sc_res1_ok)

DECLARATION (eotd_nc_res_14)

DECLARATION (eotd_nc_res_124)

DECLARATION (eotd_nc_res_1)

DECLARATION (eotd_nc_res_32)

DECLARATION (eotd_nc_res_26)

DECLARATION (eotd_nc_res_64)

/* DECLARATIONS: struct arrays */

DECLARATION (NCELL_EOTD_6)

DECLARATION (EOTD_RESULT_6)

- DECLARATION (S_PLMN_SEARCH_RES)
DECLARATION (S_PLMN_SEARCH_RES_123_32)
DECLARATION (S_PLMN_SEARCH_RES_123_33)
DECLARATION (S_PLMN_SEARCH_RES_VOID)
DECLARATION (A_SEARCH_RES_MCC_123)
DECLARATION (A_SEARCH_RES_MNC_32)
DECLARATION (A_SEARCH_RES_MNC_33)
DECLARATION (A_SEARCH_RES_RXLEV)

DECLARATION (A_ARFCN_EMO_000)
DECLARATION (A_ARFCN_EMO_001)
DECLARATION (A_ARFCN_EMO_002)
DECLARATION (A_ARFCN_EMO_003)
DECLARATION (S_EMO_MEAS_RES_000)
DECLARATION (S_EMO_MEAS_RES_001)
DECLARATION (S_EMO_MEAS_RES_002)
DECLARATION (S_EMO_MEAS_RES_003)
DECLARATION (S_EMO_MEAS_RES_000_000)
DECLARATION (S_EMO_MEAS_RES_001_000)
DECLARATION (S_EMO_MEAS_RES_002_000)
DECLARATION (S_EMO_MEAS_RES_002_001)
DECLARATION (S_EMO_MEAS_RES_000_001)
DECLARATION (S_EMO_MEAS_RES_000_002)
DECLARATION (S_EMO_MEAS_RES_000_003)
DECLARATION (S_EMO_MEAS_RES_003_000)
DECLARATION (S_EMO_MEAS_RES_003_001)
DECLARATION (S_EMO_MEAS_RES_003_002)
DECLARATION (S_EMO_MEAS_RES_003_003)
DECLARATION (S_EMO_MEAS_RES_003_004)
DECLARATION (S_EMO_MEAS_RES_003_005)
DECLARATION (S_EMO_MEAS_RES_003_006)
DECLARATION (S_EMO_MEAS_RES_003_007)
DECLARATION (S_EMO_MEAS_RES_003_008)
DECLARATION (S_EMO_MEAS_RES_003_009)
DECLARATION (S_EMO_MEAS_RES_003_010)
DECLARATION (S_EMO_MEAS_RES_003_011)
DECLARATION (S_EMO_MEAS_RES_003_012)
DECLARATION (S_EMO_MEAS_RES_003_013)
DECLARATION (S_EMO_MEAS_RES_003_014)
DECLARATION (S_EMO_MEAS_RES_003_015)
DECLARATION (S_EMO_MEAS_RES_003_016)
DECLARATION (S_EMO_MEAS_RES_003_017)
DECLARATION (S_EMO_MEAS_RES_003_018)
DECLARATION (S_EMO_MEAS_RES_003_019)
DECLARATION (S_EMO_MEAS_RES_003_020)
DECLARATION (S_EXT_MEAS_RES_000)
DECLARATION (S_EXT_MEAS_RES_001)
DECLARATION (S_EXT_MEAS_RES_002)
DECLARATION (S_EXT_MEAS_RES_003)
DECLARATION (A_EXT_MEAS_RES_000_LEV)
DECLARATION (A_EXT_MEAS_RES_001_LEV)
DECLARATION (A_EXT_MEAS_RES_002_LEV)
DECLARATION (A_EXT_MEAS_RES_003_LEV)
/*DECLARATION(EXT_MEAS_FREQ_000)*/

- /*

## Basic Types

- */

- SHORT	ACC_CTRL_CLASS_0000	0x0000
SHORT	ACC_CTRL_CLASS_0008	0x0008
SHORT	ACC_CTRL_CLASS_4000	0x4000
SHORT	ACC_0005	0x0040
BYTE	ALGO_0	0
SHORT	ARFCN_3	3
SHORT	ARFCN_24	24
SHORT	ARFCN_26	26
SHORT	ARFCN_32	32
SHORT	ARFCN_64	64
SHORT	ARFCN_67	67
SHORT	ARFCN_115	115
SHORT	ARFCN_124	124
SHORT	ARFCN_527	527
SHORT	ARFCN_612	612
SHORT	ARFCN_811	811
SHORT	ARFCN_812	812
SHORT	ARFCN_800	800
SHORT	ARFCN_DCS832	832
SHORT	ARFCN_PCS640	640
SHORT	ARFCN_PCS640MSB	0x8280
BYTE	ATC_1	1
BYTE	BA_0	0
BYTE	BA_1	1
BYTE	BCC_0_DG_629	0
BYTE	BCC_5	5
BYTE	BCC_6	6
BYTE	BCCH_ARFCN_CELL_DESC_HI	0
BYTE	BCCH_ARFCN_CELL_DESC_LO	0x20
BYTE	BCCH_ARFCN_CELL_DESC_HI_BAD	3
BYTE	BCCH_ARFCN_CELL_DESC_LO_BAD	0x20
BYTE	BS_AG_BLKS_RES_0	0x00
BYTE	BS_AG_BLKS_RES_5	0x05
BYTE	BND_VAL_DG_629	0x00
BYTE	BS_PA_MFRMS_2	0x02
BYTE	BS_PA_MFRMS_3	0x03
BYTE	BS_PA_MFRMS_7	0x07
BYTE	BSIC_0	0x00
BYTE	BSIC_5	0x05
BYTE	BSIC_6	0x06
BYTE	BSIC_CELL_DESC_NCC	0x0
BYTE	BSIC_CELL_DESC_BCC	0x5
BYTE	CBQ_1	0x01
SHORT	CELL_IDENT_3748	0x3748
SHORT	CELL_IDENT_0001	0x0001
BYTE	CELL_RESEL_OFF_0	0x00
BYTE	CHANNEL_MODE_SPEECH	1
BYTE	CHANNEL_MODE_AMR	0x41
BYTE	CHANNEL_MODE_UNDEF	26
BYTE	CHANNELS_2	2
BYTE	CHANNELS_3	3
BYTE	CHANNELS_5	5
BYTE	CKSN_6	6
BYTE	CKSN_7	7
BYTE	CONST_0	0
BYTE	CONST_1	1
BYTE	CONST_0	0
BYTE	CONST_1	1
BYTE	CONST_2	2
BYTE	CONST_3	3
BYTE	CONST_4	4
BYTE	CONST_5	5
BYTE	CONST_6	6
BYTE	CONST_7	7
BYTE	CONST_8	8
BYTE	CONST_9	9
BYTE	CONST_10	10
BYTE	CONST_11	11
BYTE	CONST_12	12
BYTE	CONST_13	13
BYTE	CONST_14	14
BYTE	CONST_15	15
BYTE	CONST_16	16
BYTE	CONST_17	17
BYTE	CONST_18	18
BYTE	CONST_19	19
BYTE	CONST_20	20
BYTE	CONST_21	21
BYTE	CONST_22	22
BYTE	CONST_23	23
BYTE	CONST_24	24
BYTE	CONST_25	25
BYTE	CONST_26	26
BYTE	CONST_27	27
BYTE	CONST_28	28
BYTE	CONST_29	29
BYTE	CONST_30	30
BYTE	CONST_31	31
BYTE	CONST_32	32
BYTE	CONST_46	46
BYTE	CONST_47	47
BYTE	CONST_53	53
BYTE	CONST_75	75
BYTE	CONST_124	124
SHORT	CONST_129	129
SHORT	CONST_130	130
BYTE	DLT_10	10
BYTE	DLT_18	18
BYTE	DLT_23	23
BYTE	DLT_LIMITED	10
SHORT	FN_OFFSET_1_SEC	1000
BYTE	FREQ_CHAN_SEQ_1_LOW_ARFCN	1
BYTE	HANDOV_REF_1	0xA5
BYTE	HSN_4	0x04
BYTE	HSN_14	0x14
BYTE	HSN_0	0x0
BYTE	HSN_1	0x1
SHORT	LAC_0001	0x0001
SHORT	LAC_2147	0x2147
SHORT	LAC_2148	0x2148
BYTE	LEN_2	2
BYTE	LOOP_A	1
BYTE	LOOP_C	6
BYTE	MAIO_0	0
BYTE	MAIO_2	0x02
BYTE	MAIO_12	0x12
BYTE	MAIO_10	0x10
BYTE	MS_TXPWR_MAX_CCH_02	0x02
BYTE	MS_TXPWR_MAX_CCH_0D	0x0D
BYTE	NCI_1	1
BYTE	NCI_0	0
BYTE	NCC_0	0
BYTE	NCC_A_DG_629	0x0A
BYTE	NCC_PERMITTED_1	0x01
BYTE	NCC_PERMITTED_2	0x02
BYTE	NCC_PERMITTED_FF	255
BYTE	NO_PLMN_AVAILABLE	0
BYTE	NO_BURSTS	0
SHORT	NOT_PRESENT_16BIT_MIN1	0xFFFE
BYTE	ONE_PLMN_AVAILABLE	1
BYTE	TWO_PLMN_AVAILABLE	2
BYTE	THREE_PLMN_AVAILABLE	3
BYTE	FOUR_PLMN_AVAILABLE	4
BYTE	FIVE_PLMN_AVAILABLE	5
BYTE	P2	2
BYTE	PAGE_NORMAL	0
BYTE	PENALTY_31	31
BYTE	PG_0	0
BYTE	PG_12	12
BYTE	PI_1	0x01
BYTE	POI_0	0x00
BYTE	POW_01_DG_629	1
BYTE	POW_05	5
BYTE	POW_05_HO_ATC	1
BYTE	POW_7	7
BYTE	POW_05_HO_POW	5
BYTE	RA_1	0x91
BYTE	RA_2	0x10
BYTE	RA_3	0x0B
BYTE	RF_CLASS_0	0
BYTE	RLT_64	0x0F
BYTE	ROT_0	0
BYTE	ROT_1	1
BYTE	RR_CAUSE_0	0x00
BYTE	RR_CAUSE_1	0x01
BYTE	RR_CAUSE_2	0x02
BYTE	RR_CAUSE_09	0x09
BYTE	RR_CAUSE_0A	0x0A
BYTE	RR_CAUSE_6F	0x6F
BYTE	RR_CAUSE_96	96
BYTE	RX_LEV_0	0x0
BYTE	RX_LEV_10	0x10
BYTE	RX_LEV_20	0x20
BYTE	RX_LEV_35	0x2D
BYTE	RX_LEV_ACCESS_MIN_16	0x16
BYTE	RX_LEV_ACCESS_MIN_22	0x22
BYTE	RX_LEV_ACCESS_MIN_24	0x24
BYTE	RX_QUAL_1	1
BYTE	SS_SCREEN_1	0x01
BYTE	T1	4
BYTE	T2	6
BYTE	T3	8
BYTE	T3212_0_MIN	0x00
BYTE	T3212_30_MIN_DG_629	0x05
BYTE	T3212_36_MIN	0x06
BYTE	TEMP_OFF_0	0x00
BYTE	TAV_1	1
BYTE	TI_0	0x00
BYTE	TIME_ADV_27	27
BYTE	TIME_ADV_10	10
BYTE	TIME_HPLMN_EMPTY	0x00
BYTE	TIME_HPLMN_VALID	0x05
SHORT	TMSI_142	0x0142
BYTE	TN_0	0
BYTE	TN_1	1
BYTE	TN_3	3
BYTE	TN_4	4
BYTE	TSC_2	2
BYTE	TSC_5	5
BYTE	TWO_BURSTS	2
BYTE	EIGHT_BURSTS	8
BYTE	TWO_NCELLS	2
BYTE	TWO_NCELLS_7	7
BYTE	FOUR_NCELLS	4
BYTE	SIX_NCELLS	6
BYTE	NO_NCELLS	0
BYTE	NO_NCELLS_7	7
BYTE	V_START_NOT_PRES	0
BYTE	V_START_PRES	1
BYTE	WAIT_INDICATION_0A	10
BYTE	TLV_LEN_1	0x02
BYTE	TLV_LEN_2	0x04
BYTE	TLV_LEN_3	0x05
BYTE	TLV_LEN_4	0x06
BYTE	MR_VERS	0x00
BYTE	NSCB	0x00
BYTE	ICMI	0x00
BYTE	ICMI_S	0x01
BYTE	ST_MODE_0	0x00
BYTE	ST_MODE_1	0x01
BYTE	ST_MODE_4	0x04
BYTE	SET_AMR_0	0x00
BYTE	SET_AMR_1	0x01
BYTE	SET_AMR_2	0x03
BYTE	SET_AMR_3	0x07
BYTE	SET_AMR_4	0x0F
BYTE	SET_AMR_5	0x1F
BYTE	ACS_0	0x00
BYTE	ACS_1	0x01
BYTE	ACS_2	0x03
BYTE	ACS_3	0x07
BYTE	ACS_4	0x0F
BYTE	V_COD_PROP_NV	0x00
BYTE	V_COD_PROP	0x01
BYTE	C_COD_PROP_1	0x00
BYTE	C_COD_PROP_2	0x01
BYTE	C_COD_PROP_3	0x02
BYTE	C_COD_PROP_4	0x03
LONG	Bitm_L	0x240000
SHORT	Bitm_H	0x0000
BYTE	EM_ENTITY	0x03
/* DL/26.09.02: EOTD */

BYTE	TAV_EOTD	0x20

BYTE	mfrm_offset_44	0x2C

BYTE	NO_NCELLS_6	0x06

BYTE	SB_TRUE	0x01

SHORT	REQ_ID_0	0x0000

SHORT	ARFCN_14	0x000E

SHORT	ARFCN_1	0x0001

SHORT	OTD_666	0x029A

SHORT	ARFCN_23	0x0017

LONG	time_alignment_1000	0x000003E8

LONG	RSSI_61440	0x0000F000

LONG	nom_pos_1000	0x000003E8

- /*

## Bit Buffers

*/

/*

Let us take the following subset of 16 elements of the set [0..1023] : [13, 71, 122, 191, 251, 321, 402, 476, 
	521,575, 635, 701, 765, 831, 906, 981]

Encoding is then: 122 on 10 bits, 
	then 2 and 69 on 9 bits each, 
	then 204, 75,  66 and 60 on 8 bits each, 
	then 70, 83, 3, 24, 67, 54, 64 and 70 on 7 bits each,
	 and finally 9 on 6 bits.

*/

SET_BITBUF("cell_chan_desc", CELL_ALLOC_1024,128)
	0x80, 0x7A, 0x01, 0x11, 0x73, 0x12, 0xD0, 0x8F, 0x23, 0x53, 0x06, 0x62, 0x1B, 0x68,0x11,0x89
ENDBITBUF

/*

- SET_BITBUF ("ext_meas_freq", EXT_MEAS_FREQ_000, 128)

0x18,0x00,0x00,0x00,

0x00,0x00,0x00,0x00,

0x00,0x00,0x60,0x00,

0x00,0x00,0x00,0x01

- ENDBITBUF

*/

/*

Neighbour cells description for SYTEM INFORMATION 5 for PCS 1900 measurement testing.
	BA list = {549,602,665, 686, 810}. Format is range 1024. IE carries only part of the BA.

*/

SET_BITBUF("neigh_cell_desc", BCCH_FREQ_LIST_1900, 16*8)
	0xB2, 0x99, 0xE0, 0xA4, 0x72, 0xE1, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDBITBUF

/*

Neighbour cells description for SYTEM INFORMATION 5bis for PCS 1900 measurement testing.
	BA list = {20,514,530,549,762}. Format is range 1024. IE carries only part of the BA.

*/

SET_BITBUF("neigh_cell_desc", BCCH_FREQ_LIST_1900_bis, 16*8)
	0xB0, 0x14, 0x73, 0xFF, 0x8A, 0xFC, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDBITBUF

/*

## Arrays

- */

/*

### Arrays of Bytes

- */

BEGINARRAY (A_BCCH_INFO_1_PLUS_43_V, 16)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x04,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x0F, 0x00, 0x00
ENDARRAY

BEGINARRAY(CHAN_LIST_13_1, 32)
	0x80,
	17, 240, 7,61,60,
	67,131,125,250,251,
	232,64,129,97,61,
	251,223,188,243,207,
	65,8,16,129,16,
	64,185,206,115,221,
	224
ENDARRAY

BEGINARRAY_PART(A_MAC_IA,1)
	0x14

ENDARRAY

/*

from the cell channel description the following channels are allocated to the MS:

521, 575, 635

*/

BEGINARRAY_PART(MOB_ALLOC_1800,2)
	0x00, 0x38
ENDARRAY

BEGINARRAY_PART(MOB_ALLOC_1800_FAIL,2)
	0x00, 0x3C
ENDARRAY

BEGINARRAY_PART(MOB_ALLOC_ONE_CHAN,1)

0x1

ENDARRAY

BEGINARRAY (A_BCCH_INFO_EMPTY, 16)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_BCCH_INFO_32, 16)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_BCCH_INFO_24, 16)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_BCCH_INFO_DG_081, 16)
	0x08, 0x00, 0x00, 0x04, 0x00, 0x00, 0x00, 0x00,
	0x80, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x04
ENDARRAY

BEGINARRAY (A_BCCH_INFO_DG_212, 16)
	0x00, 0x00, 0x00, 0x04, 0x00, 0x00, 0x00, 0x04,
	0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x04
ENDARRAY

BEGINARRAY (A_BCCH_INFO_DG_629, 16)
	0x08, 0x00, 0x00, 0x04, 0x00, 0x00, 0x00, 0x04,
	0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x04
ENDARRAY

BEGINARRAY (A_BCCH_INFO_DG_629a, 16)
	0x08, 0x00, 0x00, 0x04, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x04
ENDARRAY

BEGINARRAY (A_BCCH_INFO_DG_213, 16)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x04,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_BCCH_INFO_NCELL_DESC_1, 16)
	0x08, 0x00, 0x00, 0x04, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x00, 0x04
ENDARRAY

BEGINARRAY (A_IMSI_CONTENT_TEST_TMSI, 15)
	0x00, 0x00, 0x01, 0x00, 0x01, 0x04, 0x07, 0x01,
	0x01, 0x04, 0x09, 0x01, 0x02, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_IMSI_CONTENT, 15)
	0x01, 0x02, 0x03, 0x03, 0x02, 0x04, 0x07, 0x01,
	0x01, 0x04, 0x09, 0x01, 0x02, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_RXLEVEL_22, 12)
	0x22, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_RX_NCELLS_0, 6)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_BSIC_NCELLS_0, 6)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_RX_NCELLS_3, 6)
	0x20, 0x2A, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_RX_NCELLS_667_0, 6)
	0x5, 0x8, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_BSIC_NCELLS_3, 6)
	0x06, 0x05, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_BSIC_NCELLS_667_0, 6)
	0x06, 0x05, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_RX_NCELLS_4, 6)
	0x2D, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_BSIC_NCELLS_4, 6)
	0x06, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY(A_RXLEV_NCELLS_1900,6)
	63, 63, 61, 56, 36, 0x20
ENDARRAY
BEGINARRAY(A_BSIC_NCELLS_1900,6)
	1,7,5,3,1,3
ENDARRAY

BEGINARRAY (A_RX_NCELLS_6_FTA, 6)
	63, 63, 63, 61, 45, 35
ENDARRAY

BEGINARRAY (A_BSIC_NCELLS_6_FTA, 6)
	9, 15, 13, 11, 9, 13
ENDARRAY

BEGIN_LONG_ARRAY (A_TIME_NCELLS_6_FTA, 6)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_RX_MPH_NCELLS_FTA_B, 6)
	0x3F, 0x3F, 0x39, 0x26, 0x20, 0x1C
ENDARRAY

BEGINARRAY (A_BSIC_MPH_NCELLS_FTA_B, 6)
	0x09, 0x0F, 0x0B, 0x09, 0x0F, 0x0D
ENDARRAY

BEGINARRAY (A_BSIC_MPH_NCELLS_FTA_C, 6)
	0x09, 0x0D, 0x0B, 0x09, 0x0F, 0x0D
ENDARRAY

BEGINARRAY (A_RX_MPH_NCELLS_FTA_D, 6)
	0x3F, 0x3C, 0x3B, 0x33, 0x22, 0x1F
ENDARRAY

BEGINARRAY (A_BSIC_MPH_NCELLS_FTA_D, 6)
	0x09, 0x0F, 0x0D, 0x0B, 0x09, 0x0F
ENDARRAY

BEGINARRAY (A_RX_MPH_NCELLS_FTA_E, 6)
	0x3F, 0x3F, 0x3F, 0x39, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_BSIC_MPH_NCELLS_FTA_E, 6)
	0x09, 0x0F, 0x0D, 0x0B, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_FREQ_LIST_AFTER_2, 16)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x02, 0x00, 0x00,
	0x00, 0x08, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_FREQ_LIST_BEFORE_1, 16)
	0x0F, 0xFF, 0xFF, 0xFF, 0xF0, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0xFF, 0xFF, 0xFF, 0xFF
ENDARRAY

BEGINARRAY (A_RXLEVEL_21, 12)
	0x21, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDARRAY

/* DL/26.09.02: EOTD */

BEGIN_SHORT_ARRAY (eotd_crosscor_12, 18)
	0x000F, 0x001F, 0x003F, 0x007F, 0x00FF, 0x01FF, 0x03FF, 0x07FF, 
	0x0FFF, 0x1FFF, 0x3FFF, 0x7FFF, 0,0,0,0,0,0
ENDARRAY

BEGINARRAY (A_SEARCH_RES_MCC_123, 3)
	1,2,3

ENDARRAY

BEGINARRAY (A_SEARCH_RES_MNC_32, 3)
	3,2,0xF

ENDARRAY

BEGINARRAY (A_SEARCH_RES_MNC_33, 3)
	3,3,0xF

ENDARRAY

BEGINARRAY (A_SEARCH_RES_RXLEV, 12)
	0x2D,0x21,0,0,0,0,0,0,0,0,0,0

ENDARRAY

BEGINARRAY (A_RXLEVEL_20, 12)
	0x20, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (A_EXT_MEAS_RES_000_LEV , 21)

CONST_30,
	CONST_6,
	CONST_53,
	CONST_13,
	0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0
ENDARRAY

BEGINARRAY (A_EXT_MEAS_RES_001_LEV , 21)

CONST_53,
	0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0
ENDARRAY

BEGINARRAY (A_EXT_MEAS_RES_002_LEV , 21)

CONST_46,0,0,CONST_53,
	0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0
ENDARRAY

BEGINARRAY (A_EXT_MEAS_RES_003_LEV , 21)

1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21
ENDARRAY

/*

### Arrays of Shorts

- */

BEGIN_SHORT_ARRAY_PART(A_ARFCN_527_601, 2)
	527,601

ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_EMPTY, 33)
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_1, 33)
	0x7C, 0x20, 0x63, 0x03, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_1_V, 33)
	0x11, 0x12, 0x13, 0x14, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_1B, 33)
	0x03, 0x18, 0x63, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_1F, 33)
	32, 3, 99, 124, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY_PART (PRR_HOPPING_2_1800,4)
	521,575,635,0xFFFF
ENDARRAY

/*

from SI5 and SI5bis the BA list is 
	BA list = {20, 514, 530,549,602,665, 686, 762, 810}
	but the MS is configured as 1900 only so the 20 is removed

*/

BEGIN_SHORT_ARRAY_PART(A_MPH_NCELL_1900, 8)
	514, 530, 549, 602, 665, 686, 762, 810
ENDARRAY

BEGIN_SHORT_ARRAY_PART(A_MPH_NCELL_1900a,5)
	549, 602, 665, 686, 810
ENDARRAY

BEGIN_SHORT_ARRAY(A_ARFCN_NCELLS_1900,6)
	810, 549, 686, 762, 665, 514
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_1C, 33)
	0x20, 0x7C, 0x63, 0x03, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_1D, 33)
	0x03, 0x20, 0x63, 0x7C, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_1D_EMO, 33)
	0x03, 0x20, 0x21, 0x63, 0x7C, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_1E, 33)
	0x7C, 0x20, 0x63, 0x03, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_2, 33)
	0x20, 0x03, 0x63, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_8, 33)
	0x7C, 0x09, 0x0A, 0x50, 0x5A, 0x64, 0x6E, 0x78,
	0x7B, 0x02, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_DG_186, 33)
	0x0273, 0x027B, 0x0283, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_528_31, 33)
	0x0210, 0x0211, 0x0212, 0x0213, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_FTA, 33)
	0x000E, 0x0014, 0x0020, 0x002C, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5TER_5_FTA, 33)
	0x000E, 0x0014, 0x0020, 0x002C, 0x0208, 0x030C, 0x0370, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_FTA_A, 33)
	0x03DE, 0x03EB, 0x03ED, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_5BIS_FTA_A, 33)
	0x001A, 0x0026, 0x03DE, 0x03EB, 0x03ED, 0x0000, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_FTA_B, 33)
	0x001A, 0x0026, 0x000, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_5BIS_FTA_B, 33)
	0x001A, 0x0026, 0x0374, 0x03D4, 0x03E1, 0x03E3, 0x0000, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_FTA_C, 33)
	0x0208, 0x03DE, 0x03FC, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_5TER_FTA_C, 33)
	0x0208, 0x03DE, 0x03FC, 0x001A, 0x03EB, 0x03ED, 0x0000, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_FTA_D, 33)
	0x001A, 0x0026, 0x03DE, 0x03ED, 0x03FC, 0x0000, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_5TER_FTA_D, 33)
	0x001A, 0x0026, 0x03DE, 0x03ED, 0x03FC, 0x0000, 0x03E8, 0x03EB,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_FTA_E, 33)
	0x0026, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_MPH_NCELL_5_5TER_FTA_E, 33)
	0x0026, 0x001A, 0x03EB, 0x03FC, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF, 0xFFFF,
	0xFFFF
ENDARRAY

BEGIN_SHORT_ARRAY (A_ARFCN_MPH_NCELLS_FTA_C, 6)
	0x03EB, 0x03FC, 0x1A, 0x00, 0x03ED, 0x03DE
ENDARRAY

BEGIN_SHORT_ARRAY (A_ARFCN_MPH_NCELLS_FTA_D, 6)
	0x03EB, 0x26, 0x03FC, 0x1A, 0x0000, 0x03ED
ENDARRAY

BEGIN_SHORT_ARRAY (A_ARFCN_MPH_NCELLS_FTA_E, 6)
	0x03EB, 0x26, 0x03FC, 0x1A, 0x00, 0x00
ENDARRAY

BEGIN_SHORT_ARRAY (A_ARFCN_MPH_NCELLS_FTA_B, 6)
	0x03E1, 0x26, 0x1A, 0x00, 0x03E3, 0x03D4
ENDARRAY

BEGIN_SHORT_ARRAY (A_ARFCN_NCELLS_0, 6)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_SHORT_ARRAY (A_ARFCN_NCELLS_3, 6)
	0x03, 0x20, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_SHORT_ARRAY (A_ARFCN_NCELLS_667_0, 6)
	0x14, 0x11, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_SHORT_ARRAY (A_ARFCN_NCELLS_4, 6)
	0x43, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_SHORT_ARRAY (A_ARFCN_NCELLS_6_FTA, 6)
	44, 880, 32, 780, 20, 520
ENDARRAY

- BEGIN_SHORT_ARRAY (A_ARFCN_EMO_000,4)

1,46,47,124
ENDARRAY

- BEGIN_SHORT_ARRAY (A_ARFCN_EMO_001,1)

1
ENDARRAY

- BEGIN_SHORT_ARRAY (A_ARFCN_EMO_002,2)

1,0
ENDARRAY

- BEGIN_SHORT_ARRAY (A_ARFCN_EMO_003,21)

1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21

ENDARRAY

BEGIN_SHORT_ARRAY_PART (A_LAC_LIST1, 1)
	LAC_2147

ENDARRAY

BEGIN_SHORT_ARRAY_PART (A_LAC_LIST1A, 1)
	LAC_2148

ENDARRAY

BEGIN_SHORT_ARRAY_PART (A_LAC_LIST5, 5)
	LAC_2147, LAC_2147, LAC_2147, LAC_2147, LAC_2147

ENDARRAY

/*

### Arrays of Longs

- */

BEGIN_LONG_ARRAY (A_TIME_NCELLS_3, 6)
	0x02, 0x04, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_LONG_ARRAY (A_TIME_NCELLS_667_0, 6)
	0x02, 0x04, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_LONG_ARRAY (A_FRAME_NCELLS_3, 6)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_LONG_ARRAY (A_FRAME_NCELLS_667_0, 6)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_LONG_ARRAY (A_TIME_MPH_NCELLS_FTA_B, 6)
	0x00, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

/*

## Primitive Structures

- */

BEGIN_PSTRUCT ("cod_prop",	S_COD_PROP_4_ER)
	SET_COMP ("codec_thr",	0x12)
	SET_COMP ("codec_hyst",	0x02)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop",	S_COD_PROP_5_ER)
	SET_COMP ("codec_thr",	0x21)
	SET_COMP ("codec_hyst",	0x03)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop",	S_COD_PROP_6_ALC)
	SET_COMP ("codec_thr",	0x22)
	SET_COMP ("codec_hyst",	0x04)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop",	S_COD_PROP_7_ALC)
	SET_COMP ("codec_thr",	0x27)
	SET_COMP ("codec_hyst",	0x04)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop",	S_COD_PROP_8_ALC)
	SET_COMP ("codec_thr",	0x32)
	SET_COMP ("codec_hyst",	0x04)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop",	S_COD_PROP_9_ALC)
	SET_COMP ("codec_thr",	0x00)
	SET_COMP ("codec_hyst",	0x00)
ENDSTRUCT

BEGIN_PSTRUCT ("classmark",	 CLASS_MS_DG_081C)
	SET_COMP ("pclass",	CLASS_4)
	SKIP_COMP ("pclass2")
ENDSTRUCT

BEGIN_PSTRUCT ("classmark",	 CLASS_MS)
	SET_COMP ("pclass",	CLASS_4)
	SKIP_COMP ("pclass2")
ENDSTRUCT

BEGIN_PSTRUCT ("classmark",	 CLASS_MS_1800)
	SET_COMP ("pclass",	CLASS_1)
	SKIP_COMP ("pclass2")
ENDSTRUCT

BEGIN_PSTRUCT ("classmark",	 CLASS_MS_DUALBAND)
	SET_COMP ("pclass",	CLASS_4)
	SET_COMP ("pclass2",	CLASS_1)
ENDSTRUCT

BEGIN_PSTRUCT ("frame_no",	 FRAME_NUMBER_1)
	SET_COMP ("t1",	T1)
	SET_COMP ("t2",	T2)
	SET_COMP ("t3",	T3)
ENDSTRUCT

BEGIN_PSTRUCT ("kcv",	 KCV_00112233)
	SET_COMP ("v_kc",	V_KC_PRES)
	SET_COMP ("kc",	KC_00112233)
ENDSTRUCT

BEGIN_PSTRUCT ("kcv",	 KCV_12345678)
	SET_COMP ("v_kc",	V_KC_PRES)
	SET_COMP ("kc",	KC_12345678)
ENDSTRUCT

BEGIN_PSTRUCT ("kcv",	 KCV_EMPTY)
	SET_COMP ("v_kc",	V_KC_NOT_PRES)
	SKIP_COMP ("kc")
ENDSTRUCT

BEGIN_PSTRUCT ("mm_info",	 MM_INFO_1)
	SET_COMP ("valid",	MM_INFO_PRES)
	SET_COMP ("la",	LA_NOT_IN_FRBD_LST_INCL)
	SET_COMP ("att",	ATT_ALLOW)
	SET_COMP ("re",	REESTAB_YES)
	SET_COMP ("band",	BND_DMY_VAL)
	SET_COMP ("ncc",	NCC_0)
	SET_COMP ("bcc",	BCC_6)
	SET_COMP ("t3212",	T3212_36_MIN)
ENDSTRUCT

BEGIN_PSTRUCT ("mm_info",	 MM_INFO_2)
	SET_COMP ("valid",	MM_INFO_PRES)
	SET_COMP ("la",	LA_NOT_IN_FRBD_LST_INCL)
	SET_COMP ("att",	ATT_ALLOW)
	SET_COMP ("re",	REESTAB_YES)
	SET_COMP ("band",	BND_DMY_VAL)
	SET_COMP ("ncc",	NCC_0)
	SET_COMP ("bcc",	BCC_5)
	SET_COMP ("t3212",	T3212_36_MIN)
ENDSTRUCT

BEGIN_PSTRUCT ("mm_info",	 MM_INFO_15)
	SET_COMP ("valid",	MM_INFO_PRES)
	SET_COMP ("la",	LA_NOT_IN_FRBD_LST_INCL)
	SET_COMP ("att",	ATT_NOT_ALLOW)
	SET_COMP ("re",	REESTAB_NO)
	SET_COMP ("band",	BND_DMY_VAL)
	SET_COMP ("ncc",	NCC_0)
	SET_COMP ("bcc",	BCC_5)
	SET_COMP ("t3212",	T3212_0_MIN)
ENDSTRUCT

BEGIN_PSTRUCT ("mm_info",	 MM_INFO_3)
	SET_COMP ("valid",	MM_INFO_PRES)
	SET_COMP ("la",	LA_NOT_IN_FRBD_LST_INCL)
	SET_COMP ("att",	ATT_ALLOW)
	SET_COMP ("re",	REESTAB_YES)
	SET_COMP ("band",	BND_DMY_VAL)
	SET_COMP ("ncc",	NCC_0)
	SET_COMP ("bcc",	BCC_6)
	SET_COMP ("t3212",	T3212_36_MIN)
ENDSTRUCT

BEGIN_PSTRUCT ("mm_info",	 MM_INFO_4_DG_629)
	SET_COMP ("valid",	MM_INFO_NOT_PRES)
	SET_COMP ("la",	LA_NOT_IN_FRBD_LST_INCL)
	SET_COMP ("att",	ATT_NOT_ALLOW)
	SET_COMP ("re",	REESTAB_YES)
	SET_COMP ("band",	BND_VAL_DG_629)
	SET_COMP ("ncc",	NCC_A_DG_629)
	SET_COMP ("bcc",	BCC_0_DG_629)
	SET_COMP ("t3212",	T3212_30_MIN_DG_629)
ENDSTRUCT

BEGIN_PSTRUCT ("imsi_struct",	 MOBILE_ID_NOT_SET)
	SET_COMP ("v_mid",	V_MID_PRES)
	SET_COMP ("id_type",	NOT_PRESENT_8BIT)
	SKIP_COMP ("id")
	SKIP_COMP ("tmsi_dig")
ENDSTRUCT

BEGIN_PSTRUCT ("imsi_struct",	 MOBILE_ID_IMSI_HPLMN)
	SET_COMP ("v_mid",	V_MID_PRES)
	SET_COMP ("id_type",	TYPE_IMSI)
	SET_COMP ("id",	IMSI_1233247114912)
	SKIP_COMP ("tmsi_dig")
ENDSTRUCT

BEGIN_PSTRUCT ("imsi_struct",	 MOBILE_ID_IMSI_TEST)
	SET_COMP ("v_mid",	V_MID_PRES)
	SET_COMP ("id_type",	TYPE_IMSI)
	SET_COMP ("id",	IMSI_0010147114912)
	SKIP_COMP ("tmsi_dig")
ENDSTRUCT

BEGIN_PSTRUCT ("tmsi_struct",	 MOBILE_ID_TMSI)
	SET_COMP ("v_mid",	V_MID_PRES)
	SET_COMP ("id_type",	TYPE_TMSI)
	SKIP_COMP ("id")
	SET_COMP ("tmsi_dig",	TMSI_142)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop",	 S_COD_PROP_0)
	SET_COMP ("codec_thr",	0x00)
	SET_COMP ("codec_hyst",	0x00)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop",	 S_COD_PROP_1)
	SET_COMP ("codec_thr",	0x01)
	SET_COMP ("codec_hyst",	0x01)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop",	 S_COD_PROP_2)
	SET_COMP ("codec_thr",	0x03)
	SET_COMP ("codec_hyst",	0x03)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop",	 S_COD_PROP_3)
	SET_COMP ("codec_thr",	0x05)
	SET_COMP ("codec_hyst",	0x05)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	 S_AMR_conf_0)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_0)
	SET_COMP ("acs",	ACS_0)
	SET_COMP ("v_cod_prop",	V_COD_PROP_NV)
	SET_COMP ("c_cod_prop",	C_COD_PROP_1)
	SET_COMP ("cod_prop",	SA_cod_prop_1)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	 S_AMR_conf_1)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	ACS_1)
	SET_COMP ("v_cod_prop",	V_COD_PROP_NV)
	SET_COMP ("c_cod_prop",	C_COD_PROP_1)
	SET_COMP ("cod_prop",	SA_cod_prop_1)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	 S_AMR_conf_2)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	ACS_2)
	SET_COMP ("v_cod_prop",	V_COD_PROP)
	SET_COMP ("c_cod_prop",	C_COD_PROP_2)
	SET_COMP ("cod_prop",	SA_cod_prop_2)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	 S_AMR_conf_3)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	ACS_3)
	SET_COMP ("v_cod_prop",	V_COD_PROP)
	SET_COMP ("c_cod_prop",	C_COD_PROP_3)
	SET_COMP ("cod_prop",	SA_cod_prop_3)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	 S_AMR_conf_4)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	ACS_4)
	SET_COMP ("v_cod_prop",	V_COD_PROP)
	SET_COMP ("c_cod_prop",	C_COD_PROP_4)
	SET_COMP ("cod_prop",	SA_cod_prop_4)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	 S_AMR_conf_4_ICMI)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI_S)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	ACS_4)
	SET_COMP ("v_cod_prop",	V_COD_PROP)
	SET_COMP ("c_cod_prop",	C_COD_PROP_4)
	SET_COMP ("cod_prop",	SA_cod_prop_4)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	S_AMR_conf_5_ER_AFS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	0x4D)
	SET_COMP ("v_cod_prop",	V_COD_PROP)
	SET_COMP ("c_cod_prop",	C_COD_PROP_4)
	SET_COMP ("cod_prop",	SA_COD_PROP_M4_ER_ALR)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	S_AMR_conf_6_ALC_AFS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	0xE1)
	SET_COMP ("v_cod_prop",	V_COD_PROP)
	SET_COMP ("c_cod_prop",	C_COD_PROP_4)
	SET_COMP ("cod_prop",	SA_COD_PROP_M4_ALC_ALR)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	S_AMR_conf_6_ALC_AFS2)
	SET_COMP ("nscb",	0)
	SET_COMP ("icmi",	0)
	SET_COMP ("st_mode",	0)
	SET_COMP ("acs",	0x80)
	SET_COMP ("v_cod_prop",	0)
	SKIP_COMP ("c_cod_prop")
	SKIP_COMP ("cod_prop")
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	S_AMR_conf_7_ER_AHS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	0x25)
	SET_COMP ("v_cod_prop",	V_COD_PROP)
	SET_COMP ("c_cod_prop",	C_COD_PROP_3)
	SET_COMP ("cod_prop",	SA_COD_PROP_M3_ER_ALR)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf",	S_AMR_conf_8_ALC_AHS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	0x18)
	SET_COMP ("v_cod_prop",	V_COD_PROP)
	SET_COMP ("c_cod_prop",	C_COD_PROP_2)
	SET_COMP ("cod_prop",	SA_COD_PROP_M2_ALC_ALR)
ENDSTRUCT

BEGIN_PSTRUCT ("ciph",	 NO_CIPHERING)
	SET_COMP ("stat",	STAT_CIPH_OFF)
	SKIP_COMP ("algo")
	SKIP_COMP ("kc")
ENDSTRUCT

BEGIN_PSTRUCT ("ciph",	 CIPHERING_2)
	SET_COMP ("stat",	STAT_CIPH_ON)
	SET_COMP ("algo",	ALGO_0)
	SET_COMP ("kc",	KC_12345678)
ENDSTRUCT

BEGIN_PSTRUCT ("ciph",	 CIPHERING_3)
	SET_COMP ("stat",	STAT_CIPH_ON)
	SET_COMP ("algo",	ALGO_0)
	SET_COMP ("kc",	KC_00112233)
ENDSTRUCT

BEGIN_PSTRUCT ("start",	 NO_STARTING_TIME)
	SET_COMP ("v_start",	V_START_NOT_PRES)
	SKIP_COMP ("t1")
	SKIP_COMP ("t2")
	SKIP_COMP ("t3")
ENDSTRUCT

BEGIN_PSTRUCT ("start",	 STARTING_TIME_1)
	SET_COMP ("v_start",	V_START_PRES)
	SET_COMP ("t1",	T1)
	SET_COMP ("t2",	T2)
	SET_COMP ("t3",	T3)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	 PLMN_ID_EMPTY)
	SET_COMP ("v_plmn",	V_PLMN_NOT_PRES)
	SKIP_COMP ("mcc")
	SKIP_COMP ("mnc")
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	 PLMN_ID_123)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_32)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	 PLMN_ID_123X)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_32X)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	 PLMN_ID_123_V)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_33)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	 PLMN_ID_122)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_122)
	SET_COMP ("mnc",	MNC_32)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	 PLMN_ID_TEST)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_001)
	SET_COMP ("mnc",	MNC_01)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_EMPTY)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_NO_INSRT)
	SET_COMP ("func",	FUNC_LIM_SERV_ST_SRCH)
	SET_COMP ("service",	LIMITED_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_EMPTY_NO_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_NO_INSRT)
	SET_COMP ("func",	FUNC_LIM_SERV_ST_SRCH)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_NORMAL)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	FULL_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_NORMAL_NO_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_NET_SRCH_MMI_NO_SRV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_NET_SRCH_MMI_LIM_SRV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	LIMITED_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_NET_SRCH_MMI)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_NET_SRCH_BY_MMI)
	SET_COMP ("service",	FULL_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_TEST_SIM)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	FULL_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_TEST_SIM_NO_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	NO_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_TEST_SIM_LIM_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_INSRT)
	SET_COMP ("func",	FUNC_PLMN_SRCH)
	SET_COMP ("service",	LIMITED_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("op",	 OP_MODE_NO_SIM_LIM_SERV)
	SET_COMP ("v_op",	V_OP_PRES)
	SET_COMP ("ts",	TS_NO_AVAIL)
	SET_COMP ("m",	M_AUTO)
	SET_COMP ("sim_ins",	SIM_NO_INSRT)
	SET_COMP ("func",	FUNC_LIM_SERV_ST_SRCH)
	SET_COMP ("service",	LIMITED_SERVICE)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_0)
	SKIP_COMP ("ch")
	SKIP_COMP ("tn")
	SKIP_COMP ("tsc")
	SKIP_COMP ("h")
	SKIP_COMP ("arfcn")
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
	SKIP_COMP ("ma")
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_2)
	SET_COMP ("ch",	CH_SDCCH_4_1)
	SET_COMP ("tn",	TN_3)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_12)
	SET_COMP ("hsn",	HSN_14)
	SET_COMP ("ma",	PRR_HOPPING_1)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_3)
	SET_COMP ("ch",	CH_SDCCH_4_1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_115)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
	SKIP_COMP ("ma")
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	PRR_CHANNEL_TYPE_3_1800)
	SET_COMP ("ch",	CH_SDCCH_4_1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_812)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
	SKIP_COMP ("ma")
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	PRR_CHANNEL_TYPE_3_1900)
	SET_COMP ("ch",	CH_SDCCH_4_1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_612)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
	SKIP_COMP ("ma")
ENDSTRUCT

BEGIN_PSTRUCT ("cbch",	 PRR_CHANNEL_TYPE_3_CB)
	SET_COMP ("stat",	TRUE)
	SET_COMP ("ch",	CH_SDCCH_4_1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_115)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
	SKIP_COMP ("ma")
ENDSTRUCT

BEGIN_PSTRUCT ("cbch",	 PRR_CHANNEL_TYPE_3_CB_1800)
	SET_COMP ("stat",	TRUE)
	SET_COMP ("ch",	CH_SDCCH_4_1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_811)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
	SKIP_COMP ("ma")
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_4)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_115)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
	SKIP_COMP ("ma")
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_5)
	SKIP_COMP ("ch")
	SKIP_COMP ("tn")
	SKIP_COMP ("tsc")
	SKIP_COMP ("h")
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_10)
	SKIP_COMP ("hsn")
	SET_COMP ("ma",	PRR_HOPPING_2)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	PRR_CHANNEL_TYPE_HALF)
	SET_COMP ("ch",	CH_TCH_H_2)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_115)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
	SKIP_COMP ("ma")
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	PRR_CHANNEL_TYPE_HOP_HALF)
	SET_COMP ("ch",	CH_TCH_H_2)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio", 	MAIO_2)
	SET_COMP ("hsn",	HSN_4)
	SHOW_COMP ("ma")
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_HOP_HALF2)
	SET_COMP ("ch",	CH_TCH_H_2)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_12)
	SET_COMP ("hsn",	HSN_14)
	SET_COMP ("ma",	PRR_HOPPING_2)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_6)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_12)
	SET_COMP ("hsn",	HSN_14)
	SET_COMP ("ma",	PRR_HOPPING_2)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	PRR_CHANNEL_TYPE_6_1800)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_12)
	SET_COMP ("hsn",	HSN_14)
	SET_COMP ("ma",	PRR_HOPPING_2_1800)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_7)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_2)
	SET_COMP ("hsn",	HSN_4)
	SET_COMP ("ma",	PRR_HOPPING_3)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	PRR_CHANNEL_TYPE_7_1800)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_2)
	SET_COMP ("hsn",	HSN_4)
	SET_COMP ("ma",	PRR_HOPPING_2_1800)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_8)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_12)
	SET_COMP ("hsn",	HSN_14)
	SET_COMP ("ma",	PRR_HOPPING_4)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_9)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_2)
	SET_COMP ("hsn",	HSN_4)
	SET_COMP ("ma",	PRR_HOPPING_5)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_10)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_2)
	SET_COMP ("hsn",	HSN_4)
	SET_COMP ("ma",	PRR_HOPPING_6)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_11)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_2)
	SET_COMP ("hsn",	HSN_4)
	SET_COMP ("ma",	PRR_HOPPING_7)
ENDSTRUCT

BEGIN_PSTRUCT ("ch_type",	 PRR_CHANNEL_TYPE_EGSM)
	SET_COMP ("ch",	CH_TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("h",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_12)
	SET_COMP ("hsn",	HSN_14)
	SET_COMP ("ma",	PRR_HOPPING_EGSM)
ENDSTRUCT

BEGIN_PSTRUCT ("ho_param",	 HO_PARAM_1)
	SET_COMP ("ho_ref",	HANDOV_REF_1)
	SET_COMP ("ho_pow",	POW_05)
	SET_COMP ("ho_acc_type",	ATC_1)
	SET_COMP ("ho_nci",	NCI_1)
ENDSTRUCT

BEGIN_PSTRUCT ("ho_param",	 HO_PARAM_0)
	SET_COMP ("ho_ref",	HANDOV_REF_1)
	SET_COMP ("ho_pow",	POW_05)
	SET_COMP ("ho_acc_type",	ATC_1)
	SET_COMP ("ho_nci",	NCI_0)
ENDSTRUCT

BEGIN_PSTRUCT ("tr_para",	 PRR_TR_PARA_2)
	SET_COMP ("power",	MS_TXPWR_MAX_CCH_02)
	SET_COMP ("dtx",	DTX_USED)
	SET_COMP ("rlt",	RLT_64)
	SET_COMP ("tav",	TIME_ADV_27)
	SET_COMP ("pwrc",	POW_CTRL_YES)
	SET_COMP ("mode",	CHM_SIG_ONLY)
ENDSTRUCT

BEGIN_PSTRUCT ("tr_para",	 PRR_TR_PARA_2B)
	SET_COMP ("power",	MS_TXPWR_MAX_CCH_02)
	SET_COMP ("dtx",	DTX_USED)
	SET_COMP ("rlt",	RLT_64)
	SET_COMP ("tav",	TIME_ADV_10)
	SET_COMP ("pwrc",	POW_CTRL_YES)
	SET_COMP ("mode",	CHM_SIG_ONLY)
ENDSTRUCT

BEGIN_PSTRUCT ("tr_para",	 PRR_TR_PARA_3)
	SET_COMP ("power",	POW_05)
	SET_COMP ("dtx",	DTX_USED)
	SET_COMP ("rlt",	RLT_64)
	SKIP_COMP ("tav")
	SET_COMP ("pwrc",	POW_CTRL_YES)
	SET_COMP ("mode",	CHM_SIG_ONLY)
ENDSTRUCT

BEGIN_PSTRUCT ("tr_para",	 PRR_TR_PARA_13)
	SET_COMP ("power",	POW_05)
	SET_COMP ("dtx",	DTX_USED)
	SET_COMP ("rlt",	RLT_64)
	SKIP_COMP ("tav")
	SET_COMP ("pwrc",	POW_CTRL_YES)
	SET_COMP ("mode",	CHM_SPEECH)
ENDSTRUCT

BEGIN_PSTRUCT ("tr_para",	 PRR_TR_PARA_HO)
	SKIP_COMP ("power")
	SKIP_COMP ("dtx")
	SKIP_COMP ("rlt")
	SET_COMP ("tav",	TAV_1)
	SKIP_COMP ("pwrc")
	SET_COMP ("mode",	CHM_SPEECH)
ENDSTRUCT

BEGIN_PSTRUCT ("tr_para",	 PRR_TR_PARA_ASY_HO)
	SKIP_COMP ("power")
	SKIP_COMP ("dtx")
	SKIP_COMP ("rlt")
	SKIP_COMP ("tav")
	SKIP_COMP ("pwrc")
	SET_COMP ("mode",	CHM_SPEECH)
ENDSTRUCT

BEGIN_PSTRUCT ("tr_para",	 PRR_TR_PARA_AMR)
	SET_COMP ("power",	POW_05)
	SET_COMP ("dtx",	DTX_USED)
	SET_COMP ("rlt",	RLT_64)
	SKIP_COMP ("tav")
	SET_COMP ("pwrc",	POW_CTRL_YES)
	SET_COMP ("mode",	CHANNEL_MODE_AMR)
ENDSTRUCT

BEGIN_PSTRUCT ("tr_para",	 PRR_TR_PARA_AMR_1)
	SKIP_COMP ("power")
	SKIP_COMP ("dtx")
	SKIP_COMP ("rlt")
	SKIP_COMP ("tav")
	SKIP_COMP ("pwrc")
	SET_COMP ("mode",	CHANNEL_MODE_AMR)
ENDSTRUCT

BEGIN_PSTRUCT ("send_mode",	 SEND_MODE_2_BURSTS)
	SKIP_COMP ("c")
	SET_COMP ("no",	TWO_BURSTS)
	SET_COMP ("delta",	DELTA_TWO_BURSTS)
	SET_COMP ("rach",	RACH_2_BURSTS)
ENDSTRUCT

BEGIN_PSTRUCT ("send_mode",	 SEND_MODE_2_BURSTS2)
	SKIP_COMP ("c")
	SET_COMP ("no",	TWO_BURSTS)
	SET_COMP ("delta",	DELTA_TWO_BURSTS)
	SET_COMP ("rach",	RACH_2_BURSTS2)
ENDSTRUCT

BEGIN_PSTRUCT ("send_mode",	 SEND_MODE_8_BURSTS)
	SKIP_COMP ("c")
	SET_COMP ("no",	EIGHT_BURSTS)
	SET_COMP ("delta",	DELTA_EIGHT_BURSTS)
	SET_COMP ("rach",	RACH_8_BURSTS)
ENDSTRUCT

BEGIN_PSTRUCT ("send_mode",	 SEND_MODE_NO_BURSTS)
	SKIP_COMP ("c")
	SET_COMP ("no",	NO_BURSTS)
	SKIP_COMP ("delta")
	SKIP_COMP ("rach")
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_EMPTY)
	SET_COMP ("v_bcch",	V_BCCH_NOT_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_EMPTY)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_CLEAR)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_EMPTY)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_32)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_32)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_NO_24)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_32)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_24)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_24)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_DG_081)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_DG_081)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_DG_091)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_EMPTY)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_DG_212)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_DG_212)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_DG_629)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_DG_629)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	S_BCCH_INFO_DG_629a)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_DG_629a)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	 S_BCCH_INFO_DG_213)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_DG_213)
ENDSTRUCT

BEGIN_PSTRUCT ("bcch_info",	S_BCCH_INFO_NCELL_DESC_1)
	SET_COMP ("v_bcch",	V_BCCH_PRES)
	SET_COMP ("bcch",	A_BCCH_INFO_NCELL_DESC_1)

ENDSTRUCT
BEGIN_PSTRUCT ("mid",	 S_MS_ID_NO_IMSI_NO_TMSI)
	SET_COMP ("len_imsi",	0x00)
	SKIP_COMP ("imsi")
	SET_COMP ("v_tmsi",	0)
	SKIP_COMP ("tmsi")
	SET_COMP ("v_ptmsi",	0)
	SKIP_COMP ("ptmsi")
	SET_COMP ("v_ptmsi2",	0)
	SKIP_COMP ("ptmsi2")
ENDSTRUCT

BEGIN_PSTRUCT ("mid",	 S_MS_ID_IMSI_TEST_TMSI)
	SET_COMP ("len_imsi",	0x0D)
	SET_COMP ("imsi",	A_IMSI_CONTENT_TEST_TMSI)
	SET_COMP ("v_tmsi",	1)
	SET_COMP ("tmsi",	0x142)
	SET_COMP ("v_ptmsi",	0)
	SKIP_COMP ("ptmsi")
	SET_COMP ("v_ptmsi2",	0)
	SKIP_COMP ("ptmsi2")
ENDSTRUCT

BEGIN_PSTRUCT ("mid",	 S_MS_ID_IMSI_HPLMN_NO_TMSI)
	SET_COMP ("len_imsi",	0x0D)
	SET_COMP ("imsi",	A_IMSI_CONTENT)
	SET_COMP ("v_tmsi",	0)
	SKIP_COMP ("tmsi")
	SET_COMP ("v_ptmsi",	0)
	SKIP_COMP ("ptmsi")
	SET_COMP ("v_ptmsi2",	0)
	SKIP_COMP ("ptmsi2")
ENDSTRUCT

BEGIN_PSTRUCT ("mid",	 S_MS_ID_IMSI_HPLMN_TMSI)
	SET_COMP ("len_imsi",	0x0D)
	SET_COMP ("imsi",	A_IMSI_CONTENT)
	SET_COMP ("v_tmsi",	1)
	SET_COMP ("tmsi",	0x142)
	SET_COMP ("v_ptmsi",	0)
	SKIP_COMP ("ptmsi")
	SET_COMP ("v_ptmsi2",	0)
	SKIP_COMP ("ptmsi2")
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	 S_PLMN_122_FOUND)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_122)
	SET_COMP ("mnc",	MNC_33)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	 S_PLMN_123_32_FOUND)
	SET_COMP ("v_plmn",	V_PLMN_PRES)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MNC_32)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	 S_PLMN_EMPTY)
	SET_COMP ("v_plmn",	V_PLMN_NOT_PRES)
	SKIP_COMP ("mcc"	)
	SKIP_COMP ("mnc"	)
ENDSTRUCT

BEGIN_PSTRUCT ("chm",	 S_CHM_DG_629)
	SET_COMP ("ch_type",	CH_SDCCH)
	SET_COMP ("ch_mode",	0xFF)
ENDSTRUCT

BEGIN_PSTRUCT ("chm",	 S_CHM_SPEECH_FULL)
	SET_COMP ("ch_type",	CH_TCH_F)
	SET_COMP ("ch_mode",	CHM_SPEECH)
ENDSTRUCT

BEGIN_PSTRUCT ("chm",	 S_CHM_SPEECH_AMR_FULL)
	SET_COMP ("ch_type",	CH_TCH_F)
	SET_COMP ("ch_mode",	CHANNEL_MODE_AMR)
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	 S_NCELLS_0)
	SET_COMP ("no_of_ncells",	0x00)
	SET_COMP ("arfcn",	A_ARFCN_NCELLS_0)	/* array of 6 short */
	SET_COMP ("rx_lev",	A_RX_NCELLS_0)	/* array of 6 byte */
	SET_COMP ("bsic",	A_BSIC_NCELLS_0)	/* array of 6 byte */
	SET_COMP ("time_alignmt",	A_TIME_MPH_NCELLS_FTA_B)	/* array of 6 long */
	SET_COMP ("frame_offset",	A_FRAME_NCELLS_3)	/* array of 6 long */
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	 S_NCELLS_3)
	SET_COMP ("no_of_ncells",	0x02)
	SET_COMP ("arfcn",	A_ARFCN_NCELLS_3)	/* array of 6 short */
	SET_COMP ("rx_lev",	A_RX_NCELLS_3)	/* array of 6 byte */
	SET_COMP ("bsic",	A_BSIC_NCELLS_3)	/* array of 6 byte */
	SET_COMP ("time_alignmt",	A_TIME_NCELLS_3)	/* array of 6 long */
	SET_COMP ("frame_offset",	A_FRAME_NCELLS_3)	/* array of 6 long */
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	S_NCELLS_667_0)
	SET_COMP ("no_of_ncells",	0x02)
	SET_COMP ("arfcn",	A_ARFCN_NCELLS_667_0)
	SET_COMP ("rx_lev",	A_RX_NCELLS_667_0)
	SET_COMP ("bsic",	A_BSIC_NCELLS_667_0)
	SET_COMP ("time_alignmt",	A_TIME_NCELLS_667_0)
	SET_COMP ("frame_offset",	A_FRAME_NCELLS_667_0)
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	 S_NCELLS_6)
	SET_COMP ("no_of_ncells",	0x06)
	SET_COMP ("arfcn",	A_ARFCN_NCELLS_1900)	/* array of 6 short */
	SET_COMP ("rx_lev",	A_RXLEV_NCELLS_1900)	/* array of 6 byte */
	SET_COMP ("bsic",	A_BSIC_NCELLS_1900)
	SKIP_COMP ("time_alignmt")
	SKIP_COMP ("frame_offset")
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	 S_NCELLS_4)
	SET_COMP ("no_of_ncells",	0x01)
	SET_COMP ("arfcn",	A_ARFCN_NCELLS_4)	/* array of 6 short */
	SET_COMP ("rx_lev",	A_RX_NCELLS_4)	/* array of 6 byte */
	SET_COMP ("bsic",	A_BSIC_NCELLS_4)	/* array of 6 byte */
	SET_COMP ("time_alignmt",	A_TIME_NCELLS_6_FTA)	/* array of 6 long */
	SET_COMP ("frame_offset",	A_FRAME_NCELLS_3)	/* array of 6 long */
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	 S_NCELLS_6_FTA)
	SET_COMP ("no_of_ncells",	0x06)
	SET_COMP ("arfcn",	A_ARFCN_NCELLS_6_FTA)	/* array of 6 short */
	SET_COMP ("rx_lev",	A_RX_NCELLS_6_FTA)	/* array of 6 byte */
	SET_COMP ("bsic",	A_BSIC_NCELLS_6_FTA)	/* array of 6 byte */
	SET_COMP ("time_alignmt",	A_TIME_NCELLS_6_FTA)	/* array of 6 long */
	SET_COMP ("frame_offset",	A_FRAME_NCELLS_3)	/* array of 6 long */
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	 S_MPH_NCELLS_FTA_B)
	SET_COMP ("no_of_ncells",	0x06)
	SET_COMP ("arfcn",	A_ARFCN_MPH_NCELLS_FTA_B)	/* array of 6 short */
	SET_COMP ("rx_lev",	A_RX_MPH_NCELLS_FTA_B)		/* array of 6 byte */
	SET_COMP ("bsic",	A_BSIC_MPH_NCELLS_FTA_B)	/* array of 6 byte */
	SET_COMP ("time_alignmt",	A_TIME_MPH_NCELLS_FTA_B)	/* array of 6 long */
	SET_COMP ("frame_offset",	A_FRAME_NCELLS_3)	/* array of 6 long */
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	 S_MPH_NCELLS_FTA_C)
	SET_COMP ("no_of_ncells",	0x06)
	SET_COMP ("arfcn",	A_ARFCN_MPH_NCELLS_FTA_C)	/* array of 6 short */
	SET_COMP ("rx_lev",	A_RX_MPH_NCELLS_FTA_B)		/* array of 6 byte */
	SET_COMP ("bsic",	A_BSIC_MPH_NCELLS_FTA_C)	/* array of 6 byte */
	SET_COMP ("time_alignmt",	A_TIME_MPH_NCELLS_FTA_B)	/* array of 6 long */
	SET_COMP ("frame_offset",	A_FRAME_NCELLS_3)	/* array of 6 long */
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	 S_MPH_NCELLS_FTA_D)
	SET_COMP ("no_of_ncells",	0x06)
	SET_COMP ("arfcn",	A_ARFCN_MPH_NCELLS_FTA_D)	/* array of 6 short */
	SET_COMP ("rx_lev",	A_RX_MPH_NCELLS_FTA_D)		/* array of 6 byte */
	SET_COMP ("bsic",	A_BSIC_MPH_NCELLS_FTA_D)	/* array of 6 byte */
	SET_COMP ("time_alignmt",	A_TIME_MPH_NCELLS_FTA_B)	/* array of 6 long */
	SET_COMP ("frame_offset",	A_FRAME_NCELLS_3)	/* array of 6 long */
ENDSTRUCT

BEGIN_PSTRUCT ("ncells",	 S_MPH_NCELLS_FTA_E)
	SET_COMP ("no_of_ncells",	0x04)
	SET_COMP ("arfcn",	A_ARFCN_MPH_NCELLS_FTA_E)	/* array of 6 short */
	SET_COMP ("rx_lev",	A_RX_MPH_NCELLS_FTA_E)		/* array of 6 byte */
	SET_COMP ("bsic",	A_BSIC_MPH_NCELLS_FTA_E)	/* array of 6 byte */
	SET_COMP ("time_alignmt",	A_TIME_MPH_NCELLS_FTA_B)	/* array of 6 long */
	SET_COMP ("frame_offset",	A_FRAME_NCELLS_3)	/* array of 6 long */
ENDSTRUCT

/* DL/26.09.02: EOTD */

BEGIN_PSTRUCT ("assist_data",	ARFCN_14_EOTD)
	SET_COMP ("arfcn",	ARFCN_14)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("assist_data",	ARFCN_124_EOTD)
	SET_COMP ("arfcn",	ARFCN_124)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("assist_data",	ARFCN_1_EOTD)
	SET_COMP ("arfcn",	ARFCN_1)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("assist_data",	ARFCN_32_EOTD)
	SET_COMP ("arfcn",	ARFCN_32)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("assist_data",	ARFCN_26_EOTD)
	SET_COMP ("arfcn",	ARFCN_26)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("assist_data",	ARFCN_64_EOTD)
	SET_COMP ("arfcn",	ARFCN_64)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_sc_res",	eotd_sc_res_ok)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("arfcn",	ARFCN_23)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_sc_res1",	eotd_sc_res1_ok)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("arfcn",	ARFCN_23)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res",	eotd_nc_res_14)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("arfcn",	ARFCN_14)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res",	eotd_nc_res_124)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("arfcn",	ARFCN_124)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res",	eotd_nc_res_1)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("arfcn",	ARFCN_1)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res",	eotd_nc_res_32)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("arfcn",	ARFCN_32)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res",	eotd_nc_res_26)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("arfcn",	ARFCN_26)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res",	eotd_nc_res_64)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_0)
	SET_COMP ("arfcn",	ARFCN_64)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	S_PLMN_SEARCH_RES_123_32)
	SET_COMP ("v_plmn",	V_PLMN_PRES)

SET_COMP ("mcc",	A_SEARCH_RES_MCC_123)

SET_COMP ("mnc",	A_SEARCH_RES_MNC_32)

ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	S_PLMN_SEARCH_RES_123_33)
	SET_COMP ("v_plmn",	V_PLMN_PRES)

SET_COMP ("mcc",	A_SEARCH_RES_MCC_123)

SET_COMP ("mnc",	A_SEARCH_RES_MNC_33)

ENDSTRUCT

BEGIN_PSTRUCT ("plmn",	S_PLMN_SEARCH_RES_VOID)
	SKIP_COMP ("v_plmn")

SKIP_COMP ("mcc")

SKIP_COMP ("mnc")

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_000_000)
	SET_COMP ("arfcn",	CONST_47)

SET_COMP ("rx_lev",	CONST_53)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_000_001)
	SET_COMP ("arfcn",	CONST_1)

SET_COMP ("rx_lev",	CONST_30)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_000_002)
	SET_COMP ("arfcn",	CONST_124)

SET_COMP ("rx_lev",	CONST_13)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_000_003)
	SET_COMP ("arfcn",	CONST_46)

SET_COMP ("rx_lev",	CONST_6)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_001_000)
	SET_COMP ("arfcn",	CONST_1)

SET_COMP ("rx_lev",	CONST_53)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_002_000)
	SET_COMP ("arfcn",	CONST_0)

SET_COMP ("rx_lev",	CONST_53)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_002_001)
	SET_COMP ("arfcn",	CONST_1)

SET_COMP ("rx_lev",	CONST_46)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_000)
	SET_COMP ("arfcn",	CONST_1)

SET_COMP ("rx_lev",	CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_001)
	SET_COMP ("arfcn",	CONST_2)

SET_COMP ("rx_lev",	CONST_2)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_002)
	SET_COMP ("arfcn",	CONST_3)

SET_COMP ("rx_lev",	CONST_3)

- ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_003)
	SET_COMP ("arfcn",	CONST_4)

SET_COMP ("rx_lev",	CONST_4)

- ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_004)
	SET_COMP ("arfcn",	CONST_5)

SET_COMP ("rx_lev",	CONST_5)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_005)
	SET_COMP ("arfcn",	CONST_6)

SET_COMP ("rx_lev",	CONST_6)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_006)
	SET_COMP ("arfcn",	CONST_7)

SET_COMP ("rx_lev",	CONST_7)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_007)
	SET_COMP ("arfcn",	CONST_8)

SET_COMP ("rx_lev",	CONST_8)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_008)
	SET_COMP ("arfcn",	CONST_9)

SET_COMP ("rx_lev",	CONST_9)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_009)
	SET_COMP ("arfcn",	CONST_10)

SET_COMP ("rx_lev",	CONST_10)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_010)
	SET_COMP ("arfcn",	CONST_11)

SET_COMP ("rx_lev",	CONST_11)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_011)
	SET_COMP ("arfcn",	CONST_12)

SET_COMP ("rx_lev",	CONST_12)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_012)
	SET_COMP ("arfcn",	CONST_13)

SET_COMP ("rx_lev",	CONST_13)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_013)
	SET_COMP ("arfcn",	CONST_14)

SET_COMP ("rx_lev",	CONST_14)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_014)
	SET_COMP ("arfcn",	CONST_15)

SET_COMP ("rx_lev",	CONST_15)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_015)
	SET_COMP ("arfcn",	CONST_16)

SET_COMP ("rx_lev",	CONST_16)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_016)
	SET_COMP ("arfcn",	CONST_17)

SET_COMP ("rx_lev",	CONST_17)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_017)
	SET_COMP ("arfcn",	CONST_18)

SET_COMP ("rx_lev",	CONST_18)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results", 	S_EMO_MEAS_RES_003_018)
	SET_COMP ("arfcn",	CONST_19)

SET_COMP ("rx_lev",	CONST_19)

- ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_019)
	SET_COMP ("arfcn",	CONST_20)

SET_COMP ("rx_lev",	CONST_20)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results",	S_EMO_MEAS_RES_003_020)
	SET_COMP ("arfcn",	CONST_21)

SET_COMP ("rx_lev",	CONST_21)

ENDSTRUCT

- /*

## Arrays of Structures

- */

BEGIN_STRUCT_ARRAY (SA_cod_prop_M3_ER, 2)
	S_COD_PROP_4_ER,
	S_COD_PROP_5_ER
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M4_ER, 3)
	S_COD_PROP_4_ER,
	S_COD_PROP_5_ER,
	S_COD_PROP_5_ER
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M4_ALC, 3)
	S_COD_PROP_6_ALC,
	S_COD_PROP_7_ALC,

S_COD_PROP_8_ALC
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M2_ALC, 1)
	S_COD_PROP_6_ALC
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M3_ER_ALR, 3)
	S_COD_PROP_4_ER,
	S_COD_PROP_5_ER,
	S_COD_PROP_0
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M4_ER_ALR, 3)
	S_COD_PROP_4_ER,
	S_COD_PROP_5_ER,
	S_COD_PROP_5_ER
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M4_ALC_ALR, 3)
	S_COD_PROP_6_ALC,
	S_COD_PROP_7_ALC,

S_COD_PROP_8_ALC
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M2_ALC_ALR, 3)
	S_COD_PROP_6_ALC,
	S_COD_PROP_0,
	S_COD_PROP_0
ENDARRAY

BEGIN_MSTRUCT ("multirate_conf",	S_multirate_conf_ER_AFS)
	SET_COMP ("tlv_len",	TLV_LEN_4)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	0x4D)
	SET_COMP ("cod_prop",	SA_COD_PROP_M4_ER)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	S_multirate_conf_ER_AHS)
	SET_COMP ("tlv_len",	TLV_LEN_3)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	0x25)
	SET_COMP ("cod_prop",	SA_COD_PROP_M3_ER)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	S_multirate_conf_ALC_AFS)
	SET_COMP ("tlv_len",	TLV_LEN_4)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	0xE1)
	SET_COMP ("cod_prop",	SA_COD_PROP_M4_ALC)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	S_multirate_conf_ALC_AHS)
	SET_COMP ("tlv_len",	TLV_LEN_2)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	0x18)
	SET_COMP ("cod_prop",	SA_COD_PROP_M2_ALC)
ENDSTRUCT

BEGIN_STRUCT_ARRAY (SA_cod_prop_M1, 3)
	S_COD_PROP_0,
	S_COD_PROP_0,
	S_COD_PROP_0
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M2, 1)
	S_COD_PROP_1
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M3, 2)
	S_COD_PROP_1,
	S_COD_PROP_2
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_M4, 3)
	S_COD_PROP_1,
	S_COD_PROP_2,
	S_COD_PROP_3
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_1, 3)
	S_COD_PROP_0,
	S_COD_PROP_0,
	S_COD_PROP_0
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_2, 3)
	S_COD_PROP_1,
	S_COD_PROP_0,
	S_COD_PROP_0
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_3, 3)
	S_COD_PROP_1,
	S_COD_PROP_2,
	S_COD_PROP_0
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_4, 3)
	S_COD_PROP_1,
	S_COD_PROP_2,
	S_COD_PROP_3
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_702D, 3)
	S_COD_PROP_3,
	S_COD_PROP_2,
	S_COD_PROP_1
ENDARRAY

BEGIN_STRUCT_ARRAY (SA_cod_prop_702E, 2)
	S_COD_PROP_1,
	S_COD_PROP_2
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_PLMN, 12)
	S_PLMN_122_FOUND,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_PLMN_123_32_FOUND, 12)
	S_PLMN_123_32_FOUND,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY,
	S_PLMN_EMPTY
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_NCELLS_3_SACCH, 6)
	S_NCELL_32_0_6,
	S_NCELL_42_1_5,
	S_NCELL_0_0_0,
	S_NCELL_0_0_0,
	S_NCELL_0_0_0,
	S_NCELL_0_0_0
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_NCELLS_3_SACCH_1900, 6)
	S_NCELL_63_8_1,
	S_NCELL_63_3_7,
	S_NCELL_61_6_5,
	S_NCELL_56_7_3,
	S_NCELL_36_5_1,
	S_NCELL_32_1_3
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_NCELLS_3_SACCH_DG_186, 6)
	S_NCELL_32_0_6,
	S_NCELL_42_0_5,
	S_NCELL_0_0_0,
	S_NCELL_0_0_0,
	S_NCELL_0_0_0,
	S_NCELL_0_0_0
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_NCELLS_6_SACCH, 6)
	S_NCELL_63_3_9,
	S_NCELL_63_6_15,
	S_NCELL_63_2_13,
	S_NCELL_61_5_11,
	S_NCELL_45_1_9,
	S_NCELL_35_4_13
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_NCELLS_FTA_B, 6)
	S_NCELL_63_4_9,
	S_NCELL_63_1_15,
	S_NCELL_57_0_11,
	S_NCELL_38_6_9,
	S_NCELL_32_5_15,
	S_NCELL_28_3_13
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_NCELLS_FTA_C, 6)
	S_NCELL_63_4_9,
	S_NCELL_63_2_13,
	S_NCELL_57_3_11,
	S_NCELL_38_6_9,
	S_NCELL_32_5_15,
	S_NCELL_28_1_13
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_NCELLS_FTA_D, 6)
	S_NCELL_63_7_9,
	S_NCELL_60_1_15,
	S_NCELL_59_4_13,
	S_NCELL_51_0_11,
	S_NCELL_34_5_9,
	S_NCELL_31_3_15
ENDARRAY

BEGIN_STRUCT_ARRAY (AS_NCELLS_FTA_E, 6)
	S_NCELL_63_2_9,
	S_NCELL_63_0_15,
	S_NCELL_63_3_13,
	S_NCELL_57_1_11,
	S_NCELL_0_0_0,
	S_NCELL_0_0_0
ENDARRAY

/*AS_NCELLS_FTA_E maybe S_NCELL_31_3_5*/

/* DL/26.09.02: EOTD */

BEGIN_STRUCT_ARRAY (NCELL_EOTD_6, 6)
	ARFCN_14_EOTD,
	ARFCN_124_EOTD,
	ARFCN_1_EOTD,
	ARFCN_32_EOTD,
	ARFCN_26_EOTD,
	ARFCN_64_EOTD
ENDARRAY

BEGIN_STRUCT_ARRAY (EOTD_RESULT_6, 6)
	eotd_nc_res_14,
	eotd_nc_res_124,
	eotd_nc_res_1,
	eotd_nc_res_32,
	eotd_nc_res_26,
	eotd_nc_res_64
ENDARRAY

BEGIN_STRUCT_ARRAY (S_PLMN_SEARCH_RES, 12)
	S_PLMN_SEARCH_RES_123_33,
	S_PLMN_SEARCH_RES_123_32,
	S_PLMN_SEARCH_RES_VOID,
	S_PLMN_SEARCH_RES_VOID,
	S_PLMN_SEARCH_RES_VOID,
	S_PLMN_SEARCH_RES_VOID,
	S_PLMN_SEARCH_RES_VOID,
	S_PLMN_SEARCH_RES_VOID,
	S_PLMN_SEARCH_RES_VOID,
	S_PLMN_SEARCH_RES_VOID,
	S_PLMN_SEARCH_RES_VOID,
	S_PLMN_SEARCH_RES_VOID
ENDARRAY

- BEGIN_STRUCT_ARRAY (S_EMO_MEAS_RES_000,4)

- S_EMO_MEAS_RES_000_000,

S_EMO_MEAS_RES_000_001,

S_EMO_MEAS_RES_000_002,

S_EMO_MEAS_RES_000_003

- ENDARRAY

- BEGIN_STRUCT_ARRAY (S_EMO_MEAS_RES_001,1)

- S_EMO_MEAS_RES_001_000

- ENDARRAY

- BEGIN_STRUCT_ARRAY (S_EMO_MEAS_RES_002,2)

- S_EMO_MEAS_RES_002_000,

- S_EMO_MEAS_RES_002_001

- ENDARRAY

- BEGIN_STRUCT_ARRAY (S_EMO_MEAS_RES_003,21)

- S_EMO_MEAS_RES_003_000,

- S_EMO_MEAS_RES_003_001,

- S_EMO_MEAS_RES_003_002,

- S_EMO_MEAS_RES_003_003,

- S_EMO_MEAS_RES_003_004,

- S_EMO_MEAS_RES_003_005,

- S_EMO_MEAS_RES_003_006,

- S_EMO_MEAS_RES_003_007,

- S_EMO_MEAS_RES_003_008,

- S_EMO_MEAS_RES_003_009,

- S_EMO_MEAS_RES_003_010,

- S_EMO_MEAS_RES_003_011,

- S_EMO_MEAS_RES_003_012,

- S_EMO_MEAS_RES_003_013,

- S_EMO_MEAS_RES_003_014,

- S_EMO_MEAS_RES_003_015,

- S_EMO_MEAS_RES_003_016,

- S_EMO_MEAS_RES_003_017,

- S_EMO_MEAS_RES_003_018,

- S_EMO_MEAS_RES_003_019,

- S_EMO_MEAS_RES_003_020

ENDARRAY

/*

## Message Structures

- */

BEGIN_MSTRUCT ("ncell",	 S_NCELL_31_3_15)
	SET_COMP ("rx_lev_ncell",	31)
	SET_COMP ("bcch_ncell",	3)
	SET_COMP ("bsic",	15)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_1)
	SET_COMP ("tlv_len",	TLV_LEN_1)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	SET_AMR_1)
	SKIP_COMP ("cod_prop")
ENDSTRUCT

BEGIN_MSTRUCT ("pow_cmd", 	POW_CMD_7)
	SET_COMP ("pow", 	7)

ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc", 	CHAN_DESC_13_1)
	SET_COMP ("chan_type", 	TCH_F_NO_ADD)
	SET_COMP ("tn", 	TN_1)
	SET_COMP ("tsc", 	TSC_5)
	SET_COMP ("hop", 	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio", 	MAIO_0)
	SET_COMP ("hsn", 	HSN_1)

ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	 CHANNEL_DESC_FACCH3_HALF)
	SET_COMP ("chan_type",	TCH_H_S1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_12)
	SET_COMP ("hsn",	HSN_14)
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	 CHANNEL_DESC_FACCH4_HALF)
	SET_COMP ("chan_type",	TCH_H_S1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_2)
	SET_COMP ("hsn",	HSN_4)
ENDSTRUCT

BEGIN_MSTRUCT ("freq_list_after", 	FREQ_LIST_AFTER_13_1)
	SET_COMP ("flist", 	CHAN_LIST_13_1)

ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_2)
	SET_COMP ("tlv_len",	TLV_LEN_2)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	SET_AMR_2)
	SET_COMP ("cod_prop",	SA_cod_prop_M2)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_3)
	SET_COMP ("tlv_len",	TLV_LEN_3)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	SET_AMR_3)
	SET_COMP ("cod_prop",	SA_cod_prop_M3)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_4)
	SET_COMP ("tlv_len",	TLV_LEN_4)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	SET_AMR_4)
	SET_COMP ("cod_prop",	SA_cod_prop_M4)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_4_ICMI)
	SET_COMP ("tlv_len",	TLV_LEN_4)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI_S)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	SET_AMR_4)
	SET_COMP ("cod_prop",	SA_cod_prop_M4)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_4_702A)
	SET_COMP ("tlv_len",	TLV_LEN_4)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI_S)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	SET_AMR_5)
	SET_COMP ("cod_prop",	SA_cod_prop_M4)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_4_702B)
	SET_COMP ("tlv_len",	TLV_LEN_4)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI_S)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	SET_AMR_0)
	SET_COMP ("cod_prop",	SA_cod_prop_M4)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_4_702C)
	SET_COMP ("tlv_len",	TLV_LEN_4)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI_S)
	SET_COMP ("st_mode",	ST_MODE_4)
	SET_COMP ("set_amr",	SET_AMR_3)
	SET_COMP ("cod_prop",	SA_cod_prop_M4)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_4_702D)
	SET_COMP ("tlv_len",	TLV_LEN_4)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI_S)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	SET_AMR_4)
	SET_COMP ("cod_prop",	SA_cod_prop_702D)
ENDSTRUCT

BEGIN_MSTRUCT ("multirate_conf",	 S_multirate_conf_4_702E)
	SET_COMP ("tlv_len",	TLV_LEN_3)
	SET_COMP ("mr_vers",	MR_VERS)
	SET_COMP ("nscb",	NSCB)
	SET_COMP ("icmi",	ICMI_S)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("set_amr",	SET_AMR_4)
	SET_COMP ("cod_prop",	SA_cod_prop_702E)
ENDSTRUCT

BEGIN_MSTRUCT ("page_mode",	 PAGING_NORMAL)
	SET_COMP ("pm",	PAGING_NORM)
ENDSTRUCT

BEGIN_MSTRUCT ("start_time",	 START_TIME_1)
	SET_COMP ("t1",	T1)
	SET_COMP ("t3",	T3)
	SET_COMP ("t2",	T2)
ENDSTRUCT

BEGIN_MSTRUCT ("cell_desc",	 CELL_DESC_1)
	SET_COMP ("bcch_arfcn_hi",	BCCH_ARFCN_CELL_DESC_HI)
	SET_COMP ("ncc",	BSIC_CELL_DESC_NCC)
	SET_COMP ("bcc",	BSIC_CELL_DESC_BCC)
	SET_COMP ("bcch_arfcn_lo",	BCCH_ARFCN_CELL_DESC_LO)
ENDSTRUCT

BEGIN_MSTRUCT ("cell_desc",	 CELL_DESC_BAD_BCCH)
	SET_COMP ("bcch_arfcn_hi",	BCCH_ARFCN_CELL_DESC_HI_BAD)
	SET_COMP ("ncc",	BSIC_CELL_DESC_NCC)
	SET_COMP ("bcc",	BSIC_CELL_DESC_BCC)
	SET_COMP ("bcch_arfcn_lo",	BCCH_ARFCN_CELL_DESC_LO_BAD)
ENDSTRUCT

BEGIN_MSTRUCT ("cell_opt_bcch",	 CELL_OPT_BCCH_1)
	SET_COMP ("pow_ctrl",	POW_CTRL_YES)
	SET_COMP ("dtx_b",	B_DTX_SHALL_USE)
	SET_COMP ("rlt",	RLT_64)
ENDSTRUCT

BEGIN_MSTRUCT ("cell_select",	 CELL_SELECT_1)
	SET_COMP ("cell_resel_hyst",	CELL_HYST_4_DB)
	SET_COMP ("ms_txpwr_max_cch",	MS_TXPWR_MAX_CCH_02)
	SET_COMP ("acs",	ACS_USE_SI4)
	SET_COMP ("neci",	NECI_YES)
	SET_COMP ("rxlev_access_min",	RX_LEV_ACCESS_MIN_16)
ENDSTRUCT

BEGIN_MSTRUCT ("cell_select",	 CELL_SELECT_2)
	SET_COMP ("cell_resel_hyst",	CELL_HYST_4_DB)
	SET_COMP ("ms_txpwr_max_cch",	MS_TXPWR_MAX_CCH_02)
	SET_COMP ("acs",	ACS_USE_SI4)
	SET_COMP ("neci",	NECI_YES)
	SET_COMP ("rxlev_access_min",	RX_LEV_ACCESS_MIN_22)
ENDSTRUCT

BEGIN_MSTRUCT ("cell_select",	 CELL_SELECT_3)
	SET_COMP ("cell_resel_hyst",	CELL_HYST_4_DB)
	SET_COMP ("ms_txpwr_max_cch",	MS_TXPWR_MAX_CCH_02)
	SET_COMP ("acs",	ACS_USE_SI4)
	SET_COMP ("neci",	NECI_YES)
	SET_COMP ("rxlev_access_min",	RX_LEV_ACCESS_MIN_24)
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	 CHANNEL_DESC_SDCCH)
	SET_COMP ("chan_type",	SDCCH_4_S1)
	SET_COMP ("tn",	TN_3)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_12)
	SET_COMP ("hsn",	HSN_14)
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	 CHANNEL_DESC_SDCCH2)
	SET_COMP ("chan_type",	SDCCH_4_S1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_115)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	CHANNEL_DESC_SDCCH2_1800)
	SET_COMP ("chan_type",	SDCCH_4_S1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_811)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	CHANNEL_DESC_SDCCH2_1800_1)
	SET_COMP ("chan_type",	SDCCH_4_S1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_812)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	CHANNEL_DESC_SDCCH2_1900_1)
	SET_COMP ("chan_type",	SDCCH_4_S1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_612)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	 CHANNEL_DESC_BAD)
	SET_COMP ("chan_type",	SDCCH_4_S1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_800)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	 CHANNEL_DESC_FACCH2)
	SET_COMP ("chan_type",	TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_115)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	 CHANNEL_DESC_HALFRATE)
	SET_COMP ("chan_type",	TCH_H_S1)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_NO)
	SET_COMP ("arfcn",	ARFCN_115)
	SKIP_COMP ("maio")
	SKIP_COMP ("hsn")
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	 CHANNEL_DESC_FACCH3)
	SET_COMP ("chan_type",	TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_12)
	SET_COMP ("hsn",	HSN_14)
ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc",	 CHANNEL_DESC_FACCH4)
	SET_COMP ("chan_type",	TCH_F)
	SET_COMP ("tn",	TN_4)
	SET_COMP ("tsc",	TSC_2)
	SET_COMP ("hop",	HOP_YES)
	SKIP_COMP ("arfcn")
	SET_COMP ("maio",	MAIO_2)
	SET_COMP ("hsn",	HSN_4)
ENDSTRUCT

BEGIN_MSTRUCT ("chan_needed",	 CHANNEL_NEEDED_1)
	SET_COMP ("cn2",	CN_ANY_CHAN)
	SET_COMP ("cn1",	CN_ANY_CHAN)
ENDSTRUCT

BEGIN_MSTRUCT ("ciph_key_num",	 CKSN_RESERVED)
	SET_COMP ("key_seq",	CKSN_7)
ENDSTRUCT

BEGIN_MSTRUCT ("ciph_mode_set",	 CIPH_MODE_OFF)
	SET_COMP ("algo_ident",	ALGO_0)
	SET_COMP ("sc",	CIPH_OFF)
ENDSTRUCT

BEGIN_MSTRUCT ("ciph_mode_set",	 CIPH_MODE_ON)
	SET_COMP ("algo_ident",	ALGO_0)
	SET_COMP ("sc",	CIPH_ON)
ENDSTRUCT

BEGIN_MSTRUCT ("ciph_res",	 CIPH_RESP_NO_IMEI)
	SET_COMP ("cr",	INC_IMEISV_NO)
ENDSTRUCT

BEGIN_MSTRUCT ("ciph_res",	 CIPH_RESP_WITH_IMEI)
	SET_COMP ("cr",	INC_IMEISV_YES)
ENDSTRUCT

BEGIN_MSTRUCT ("ctrl_chan_desc",	 CTRL_CHAN_DESC_1)
	SET_COMP ("att",	CCD_ATT_YES)
	SET_COMP ("bs_ag_blks_res",	BS_AG_BLKS_RES_5)
	SET_COMP ("ccch_conf",	CCD_CCCH_1_NOT_COMB)
	SET_COMP ("bs_pa_mfrms",	BS_PA_MFRMS_2)
	SET_COMP ("t3212",	T3212_36_MIN)
ENDSTRUCT

BEGIN_MSTRUCT ("freq_chan_seq",	FREQ_CHAN_SEQ_1)
	SET_COMP ("low_arfcn",	FREQ_CHAN_SEQ_1_LOW_ARFCN)
	SET_COMP ("inc_skip",	FREQ_CHAN_SEQ_1_INC_SKIP)
ENDSTRUCT

BEGIN_MSTRUCT ("loc_area_ident",	 LOC_AREA_IDENT_122_2147)
	SET_COMP ("mcc",	MCC_122)
	SET_COMP ("mnc",	MSG_MNC_33)
	SET_COMP ("lac",	LAC_2147)
ENDSTRUCT

BEGIN_MSTRUCT ("loc_area_ident",	 LOC_AREA_IDENT_122_36_2147)
	SET_COMP ("mcc",	MCC_122)
	SET_COMP ("mnc",	MSG_MNC_36)
	SET_COMP ("lac",	LAC_2147)
ENDSTRUCT

BEGIN_MSTRUCT ("loc_area_ident",	 LOC_AREA_IDENT_122_35_2147)
	SET_COMP ("mcc",	MCC_122)
	SET_COMP ("mnc",	MSG_MNC_35)
	SET_COMP ("lac",	LAC_2147)
ENDSTRUCT

BEGIN_MSTRUCT ("loc_area_ident",	 LOC_AREA_IDENT_122_34_2147)
	SET_COMP ("mcc",	MCC_122)
	SET_COMP ("mnc",	MSG_MNC_34)
	SET_COMP ("lac",	LAC_2147)
ENDSTRUCT

BEGIN_MSTRUCT ("loc_area_ident",	 LOC_AREA_IDENT_122_2147_V)
	SET_COMP ("mcc",	MCC_122)
	SET_COMP ("mnc",	MSG_MNC_32)
	SET_COMP ("lac",	LAC_2147)
ENDSTRUCT

BEGIN_MSTRUCT ("loc_area_ident",	 LOC_AREA_IDENT_123_2147)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MSG_MNC_32)
	SET_COMP ("lac",	LAC_2147)
ENDSTRUCT

BEGIN_MSTRUCT ("loc_area_ident",	 LOC_AREA_IDENT_123_2147_V)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MSG_MNC_33)
	SET_COMP ("lac",	LAC_2147)
ENDSTRUCT

BEGIN_MSTRUCT ("loc_area_ident",	 LOC_AREA_IDENT_123_2148)
	SET_COMP ("mcc",	MCC_123)
	SET_COMP ("mnc",	MSG_MNC_32)
	SET_COMP ("lac",	LAC_2148)
ENDSTRUCT

BEGIN_MSTRUCT ("meas_result",	 MEAS_RESULT_NCELL_3)
	SET_COMP ("ba_used",	BA_0)
	SET_COMP ("dtx_used",	DTX_NOT_USED)
	SET_COMP ("rxlev_full",	RX_LEV_20)
	SET_COMP ("meas_valid",	MEAS_VALID_YES)
	SET_COMP ("rxlev_sub",	RX_LEV_20)
	SET_COMP ("rxqual_full",	RX_QUAL_1)
	SET_COMP ("rxqual_sub",	RX_QUAL_1)
	SET_COMP ("num_ncell",	TWO_NCELLS)
	SET_COMP ("ncell",	AS_NCELLS_3_SACCH)
ENDSTRUCT

BEGIN_MSTRUCT ("meas_result",	MEAS_RESULT_NCELL_1900)
	SET_COMP ("ba_used",	BA_1)
	SET_COMP ("dtx_used",	DTX_NOT_USED)
	SET_COMP ("rxlev_full",	RX_LEV_20)
	SET_COMP ("meas_valid",	MEAS_VALID_YES)
	SET_COMP ("rxlev_sub",	RX_LEV_20)
	SET_COMP ("rxqual_full",	RX_QUAL_1)
	SET_COMP ("rxqual_sub",	RX_QUAL_1)
	SET_COMP ("num_ncell",	SIX_NCELLS)
	SET_COMP ("ncell",	AS_NCELLS_3_SACCH_1900)
ENDSTRUCT

BEGIN_MSTRUCT ("meas_result",	 MEAS_RESULT_NCELL_3_DG_186)
	SET_COMP ("ba_used",	BA_0)
	SET_COMP ("dtx_used",	DTX_NOT_USED)
	SET_COMP ("rxlev_full",	RX_LEV_20)
	SET_COMP ("meas_valid",	MEAS_VALID_YES)
	SET_COMP ("rxlev_sub",	RX_LEV_20)
	SET_COMP ("rxqual_full",	RX_QUAL_1)
	SET_COMP ("rxqual_sub",	RX_QUAL_1)
	SET_COMP ("num_ncell",	TWO_NCELLS_7)
	SKIP_COMP ("ncell")
ENDSTRUCT

BEGIN_MSTRUCT ("meas_result",	 MEAS_RESULT_NCELL_0)
	SET_COMP ("ba_used",	BA_0)
	SET_COMP ("dtx_used",	DTX_NOT_USED)
	SET_COMP ("rxlev_full",	RX_LEV_20)
	SET_COMP ("meas_valid",	MEAS_VALID_YES)
	SET_COMP ("rxlev_sub",	RX_LEV_20)
	SET_COMP ("rxqual_full",	RX_QUAL_1)
	SET_COMP ("rxqual_sub",	RX_QUAL_1)
	SET_COMP ("num_ncell",	NO_NCELLS_7)
	SKIP_COMP ("ncell")
ENDSTRUCT

BEGIN_MSTRUCT ("meas_result",	 MEAS_RESULT_NCELL_6_FTA)
	SET_COMP ("ba_used",	BA_1)
	SET_COMP ("dtx_used",	DTX_NOT_USED)
	SET_COMP ("rxlev_full",	RX_LEV_20)
	SET_COMP ("meas_valid",	MEAS_VALID_YES)
	SET_COMP ("rxlev_sub",	RX_LEV_20)
	SET_COMP ("rxqual_full",	RX_QUAL_1)
	SET_COMP ("rxqual_sub",	RX_QUAL_1)
	SET_COMP ("num_ncell",	SIX_NCELLS)
	SET_COMP ("ncell",	AS_NCELLS_6_SACCH)
ENDSTRUCT

BEGIN_MSTRUCT ("meas_result",	 MEAS_RESULT_NCELL_FTA_B)
	SET_COMP ("ba_used",	BA_1)
	SET_COMP ("dtx_used",	DTX_NOT_USED)
	SET_COMP ("rxlev_full",	RX_LEV_20)
	SET_COMP ("meas_valid",	MEAS_VALID_YES)
	SET_COMP ("rxlev_sub",	RX_LEV_20)
	SET_COMP ("rxqual_full",	RX_QUAL_1)
	SET_COMP ("rxqual_sub",	RX_QUAL_1)
	SET_COMP ("num_ncell",	SIX_NCELLS)
	SET_COMP ("ncell",	AS_NCELLS_FTA_B)
ENDSTRUCT

BEGIN_MSTRUCT ("meas_result",	 MEAS_RESULT_NCELL_FTA_C)
	SET_COMP ("ba_used",	BA_1)
	SET_COMP ("dtx_used",	DTX_NOT_USED)
	SET_COMP ("rxlev_full",	RX_LEV_20)
	SET_COMP ("meas_valid",	MEAS_VALID_YES)
	SET_COMP ("rxlev_sub",	RX_LEV_20)
	SET_COMP ("rxqual_full",	RX_QUAL_1)
	SET_COMP ("rxqual_sub",	RX_QUAL_1)
	SET_COMP ("num_ncell",	SIX_NCELLS)
	SET_COMP ("ncell",	AS_NCELLS_FTA_C)
ENDSTRUCT

BEGIN_MSTRUCT ("meas_result",	 MEAS_RESULT_NCELL_FTA_D)
	SET_COMP ("ba_used",	BA_1)
	SET_COMP ("dtx_used",	DTX_NOT_USED)
	SET_COMP ("rxlev_full",	RX_LEV_20)
	SET_COMP ("meas_valid",	MEAS_VALID_YES)
	SET_COMP ("rxlev_sub",	RX_LEV_20)
	SET_COMP ("rxqual_full",	RX_QUAL_1)
	SET_COMP ("rxqual_sub",	RX_QUAL_1)
	SET_COMP ("num_ncell",	SIX_NCELLS)
	SET_COMP ("ncell",	AS_NCELLS_FTA_D)
ENDSTRUCT

BEGIN_MSTRUCT ("meas_result",	 MEAS_RESULT_NCELL_FTA_E)
	SET_COMP ("ba_used",	BA_1)
	SET_COMP ("dtx_used",	DTX_NOT_USED)
	SET_COMP ("rxlev_full",	RX_LEV_20)
	SET_COMP ("meas_valid",	MEAS_VALID_YES)
	SET_COMP ("rxlev_sub",	RX_LEV_20)
	SET_COMP ("rxqual_full",	RX_QUAL_1)
	SET_COMP ("rxqual_sub",	RX_QUAL_1)
	SET_COMP ("num_ncell",	FOUR_NCELLS)
	SET_COMP ("ncell",	AS_NCELLS_FTA_E)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_alloc",	 MOBILE_ALLOCATION_1)
	SET_COMP ("mac",	MOB_ALLOC_1)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_alloc",	 MOBILE_ALLOCATION_ONE_CHAN)
	SET_COMP ("mac",	MOB_ALLOC_ONE_CHAN)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_alloc",	 MOBILE_ALLOCATION_2)
	SET_COMP ("mac",	MOB_ALLOC_2)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_alloc",	 MOBILE_ALLOCATION_3)
	SET_COMP ("mac",	MOB_ALLOC_3)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_alloc",	 MOBILE_ALLOCATION_4)
	SET_COMP ("mac",	MOB_ALLOC_4)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_alloc",	 MOBILE_ALLOCATION_EGSM)
	SET_COMP ("mac",	MOB_ALLOC_EGSM)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_alloc",	MOBILE_ALLOCATION_1800)
	SET_COMP ("mac",	MOB_ALLOC_1800)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_alloc",	MOBILE_ALLOCATION_1800_FAIL)
	SET_COMP ("mac",	MOB_ALLOC_1800_FAIL)
ENDSTRUCT

/*

note that a5_1 has inverse logic

*/

BEGIN_MSTRUCT ("mob_class_2",	 MOB_CLASS2_NORMAL)
	SET_COMP ("rev_lev",	PHASE_2)
	SET_COMP ("es_ind",	SUPPORTED)
	SET_COMP ("a5_1",	NOT_SUPPORTED)
	SET_COMP ("rf_pow_cap",	RF_CLASS_4)
	SET_COMP ("ps",	NOT_SUPPORTED)
	SET_COMP ("ss_screen",	SS_SCREEN_1)
	SET_COMP ("mt_pp_sms",	SUPPORTED)
	SET_COMP ("vbs",	NOT_SUPPORTED)
	SET_COMP ("vgcs",	NOT_SUPPORTED)
	SET_COMP ("egsm",	NOT_SUPPORTED)
	SET_COMP ("class3",	SUPPORTED)
	SET_COMP ("lcsva",	NOT_SUPPORTED)
	SET_COMP ("ucs2_treat",	SUPPORTED)
	SET_COMP ("solsa",	NOT_SUPPORTED)
	SET_COMP ("cmsp",	SUPPORTED)
	SET_COMP ("a5_3",	NOT_SUPPORTED)
	SET_COMP ("a5_2",	SUPPORTED)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_class_2",	 MOB_CLASS2_900)
	SET_COMP ("rev_lev",	PHASE_2)
	SET_COMP ("es_ind",	SUPPORTED)
	SET_COMP ("a5_1",	NOT_SUPPORTED)
	SET_COMP ("rf_pow_cap",	RF_CLASS_4)
	SET_COMP ("ps",	NOT_SUPPORTED)
	SET_COMP ("ss_screen",	SS_SCREEN_1)
	SET_COMP ("mt_pp_sms",	SUPPORTED)
	SET_COMP ("vbs",	NOT_SUPPORTED)
	SET_COMP ("vgcs",	NOT_SUPPORTED)
	SET_COMP ("egsm",	NOT_SUPPORTED)
	SET_COMP ("class3",	SUPPORTED)
	SET_COMP ("lcsva",	NOT_SUPPORTED)
	SET_COMP ("ucs2_treat",	SUPPORTED)
	SET_COMP ("solsa",	NOT_SUPPORTED)
	SET_COMP ("cmsp",	SUPPORTED)
	SET_COMP ("a5_3",	NOT_SUPPORTED)
	SET_COMP ("a5_2",	SUPPORTED)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_class_2",	 MOB_CLASS2_EGSM)
	SET_COMP ("rev_lev",	PHASE_2)
	SET_COMP ("es_ind",	SUPPORTED)
	SET_COMP ("a5_1",	NOT_SUPPORTED)
	SET_COMP ("rf_pow_cap",	RF_CLASS_4)
	SET_COMP ("ps",	NOT_SUPPORTED)
	SET_COMP ("ss_screen",	SS_SCREEN_1)
	SET_COMP ("mt_pp_sms",	SUPPORTED)
	SET_COMP ("vbs",	NOT_SUPPORTED)
	SET_COMP ("vgcs",	NOT_SUPPORTED)
	SET_COMP ("egsm",	SUPPORTED)
	SET_COMP ("class3",	SUPPORTED)
	SET_COMP ("lcsva",	NOT_SUPPORTED)
	SET_COMP ("ucs2_treat",	SUPPORTED)
	SET_COMP ("solsa",	NOT_SUPPORTED)
	SET_COMP ("cmsp",	SUPPORTED)
	SET_COMP ("a5_3",	NOT_SUPPORTED)
	SET_COMP ("a5_2",	NOT_SUPPORTED)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_class_2",	MOB_CLASS_1800)
	SET_COMP ("rev_lev",	PHASE_2)
	SET_COMP ("es_ind",	SUPPORTED)
	SET_COMP ("a5_1",	NOT_SUPPORTED)
	SET_COMP ("rf_pow_cap",	RF_CLASS_0)
	SET_COMP ("ps",	NOT_SUPPORTED)
	SET_COMP ("ss_screen",	SS_SCREEN_1)
	SET_COMP ("mt_pp_sms",	SUPPORTED)
	SET_COMP ("vbs",	NOT_SUPPORTED)
	SET_COMP ("vgcs",	NOT_SUPPORTED)
	SET_COMP ("egsm",	NOT_SUPPORTED)
	SET_COMP ("class3",	SUPPORTED)
	SET_COMP ("lcsva",	NOT_SUPPORTED)
	SET_COMP ("ucs2_treat",	SUPPORTED)
	SET_COMP ("solsa",	NOT_SUPPORTED)
	SET_COMP ("cmsp",	SUPPORTED)
	SET_COMP ("a5_3",	NOT_SUPPORTED)
	SET_COMP ("a5_2",	SUPPORTED)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_class_2",	MOB_CLASS_1900)
	SET_COMP ("rev_lev",	PHASE_2)
	SET_COMP ("es_ind",	SUPPORTED)
	SET_COMP ("a5_1",	NOT_SUPPORTED)
	SET_COMP ("rf_pow_cap",	RF_CLASS_0)
	SET_COMP ("ps",	NOT_SUPPORTED)
	SET_COMP ("ss_screen",	SS_SCREEN_1)
	SET_COMP ("mt_pp_sms",	SUPPORTED)
	SET_COMP ("vbs",	NOT_SUPPORTED)
	SET_COMP ("vgcs",	NOT_SUPPORTED)
	SET_COMP ("egsm",	NOT_SUPPORTED)
	SET_COMP ("class3",	SUPPORTED)
	SET_COMP ("lcsva",	NOT_SUPPORTED)
	SET_COMP ("ucs2_treat",	SUPPORTED)
	SET_COMP ("solsa",	NOT_SUPPORTED)
	SET_COMP ("cmsp",	SUPPORTED)
	SET_COMP ("a5_3",	NOT_SUPPORTED)
	SET_COMP ("a5_2",	SUPPORTED)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_class_2",	 MOB_CLASS2_212)
	SET_COMP ("rev_lev",	PHASE_2)
	SET_COMP ("es_ind",	SUPPORTED)
	SET_COMP ("a5_1",	NOT_SUPPORTED)
	SET_COMP ("rf_pow_cap",	RF_CLASS_4)
	SET_COMP ("ps",	NOT_SUPPORTED)
	SET_COMP ("ss_screen",	SS_SCREEN_1)
	SET_COMP ("mt_pp_sms",	SUPPORTED)
	SET_COMP ("vbs",	NOT_SUPPORTED)
	SET_COMP ("vgcs",	NOT_SUPPORTED)
	SET_COMP ("egsm",	SUPPORTED)
	SET_COMP ("class3",	SUPPORTED)
	SET_COMP ("lcsva",	NOT_SUPPORTED)
	SET_COMP ("ucs2_treat",	SUPPORTED)
	SET_COMP ("solsa",	NOT_SUPPORTED)
	SET_COMP ("cmsp",	SUPPORTED)
	SET_COMP ("a5_3",	NOT_SUPPORTED)
	SET_COMP ("a5_2",	SUPPORTED)
ENDSTRUCT

BEGIN_MSTRUCT ("mob_ident",	 MOBILE_IDENTITY_IMSI2)
	SET_COMP ("ident_type",	TYPE_IMSI)
	SET_COMP ("odd_even",	EVEN)
	SET_COMP ("ident_dig",	M_IMSI_08158912)
	SKIP_COMP ("tmsi_1")
ENDSTRUCT

BEGIN_MSTRUCT ("mob_ident",	 MOBILE_IDENTITY_IMSI_HPLMN)
	SET_COMP ("ident_type",	TYPE_IMSI)
	SET_COMP ("odd_even",	ODD)
	SET_COMP ("ident_dig",	M_IMSI_1233247114912)
	SKIP_COMP ("tmsi_1")
ENDSTRUCT

BEGIN_MSTRUCT ("mob_ident",	 MOBILE_IDENTITY_IMSI_TEST)
	SET_COMP ("ident_type",	TYPE_IMSI)
	SET_COMP ("odd_even",	ODD)
	SET_COMP ("ident_dig",	M_IMSI_0010147114912)
	SKIP_COMP ("tmsi_1")
ENDSTRUCT

BEGIN_MSTRUCT ("mob_ident",	 MOBILE_IDENTITY_IMEISV)
	SET_COMP ("ident_type",	ID_TYPE_IMEISV)
	SET_COMP ("odd_even",	EVEN)
	SET_COMP ("ident_dig",	M_IMEI)
	SKIP_COMP ("tmsi_1")
ENDSTRUCT

BEGIN_MSTRUCT ("mob_ident",	 MOBILE_IDENTITY_TMSI)
	SET_COMP ("ident_type",	TYPE_TMSI)
	SET_COMP ("odd_even",	EVEN)
	SKIP_COMP ("ident_dig")
	SET_COMP ("tmsi_1",	TMSI_1)
ENDSTRUCT

BEGIN_MSTRUCT ("pow_cmd",	 POWER_COMMAND_05)
	SET_COMP ("pow",	POW_05)
ENDSTRUCT

BEGIN_MSTRUCT ("pow_cmd_access",	 POW_05_HO)
	SET_COMP ("atc",	POW_05_HO_ATC)
	SET_COMP ("pow",	POW_05_HO_POW)
ENDSTRUCT

BEGIN_MSTRUCT ("rach_ctrl",	 RACH_CTRL_1)
	SET_COMP ("max_retrans",	MAX_RETRANS_1)
	SET_COMP ("tx_integer",	SPREAD_TRANS_3)
	SET_COMP ("cell_bar_access",	BARRED_NO)
	SET_COMP ("re",	REESTAB_YES)
	SET_COMP ("ac",	ACC_0005)
ENDSTRUCT

BEGIN_MSTRUCT ("rach_ctrl",	 RACH_CTRL_2)
	SET_COMP ("max_retrans",	MAX_RETRANS_1)
	SET_COMP ("tx_integer",	SPREAD_TRANS_3)
	SET_COMP ("cell_bar_access",	BARRED_YES)
	SET_COMP ("re",	REESTAB_YES)
	SET_COMP ("ac",	ACC_0005)
ENDSTRUCT

BEGIN_MSTRUCT ("req_ref",	 REQUEST_REFERENCE_1)
	SET_COMP ("ra",	RA_1)
	SET_COMP ("t1",	T1)
	SET_COMP ("t3",	T3)
	SET_COMP ("t2",	T2)
ENDSTRUCT

BEGIN_MSTRUCT ("req_ref",	 REQUEST_REFERENCE_2)
	SET_COMP ("ra",	RA_2)
	SET_COMP ("t1",	T1)
	SET_COMP ("t3",	T3)
	SET_COMP ("t2",	T2)
ENDSTRUCT

BEGIN_MSTRUCT ("req_ref",	 REQUEST_REFERENCE_3)
	SET_COMP ("ra",	RA_3)
	SET_COMP ("t1",	T1)
	SET_COMP ("t3",	T3)
	SET_COMP ("t2",	T2)
ENDSTRUCT

BEGIN_MSTRUCT ("synch_ind",	 SYNCH_IND_0)
	SET_COMP ("nci",	NCI_0)
	SET_COMP ("rot",	ROT_0)
	SET_COMP ("si",	SYI_NON_SYNCH)
ENDSTRUCT

BEGIN_MSTRUCT ("synch_ind",	 SYNCH_IND_1)
	SET_COMP ("nci",	NCI_1)
	SET_COMP ("rot",	ROT_1)
	SET_COMP ("si",	SYI_NORM_SYNCH)
ENDSTRUCT

BEGIN_MSTRUCT ("synch_ind",	 SYNCH_IND_2)
	SET_COMP ("nci",	NCI_1)
	SET_COMP ("rot",	ROT_1)
	SET_COMP ("si",	SYI_PRE_SYNCH)
ENDSTRUCT

BEGIN_MSTRUCT ("time_advance",	 TIMING_ADVANCE_27)
	SET_COMP ("ta",	TIME_ADV_27)
ENDSTRUCT

BEGIN_MSTRUCT ("time_advance",	 TIMING_ADVANCE_10)
	SET_COMP ("ta",	TIME_ADV_10)
ENDSTRUCT

BEGIN_MSTRUCT ("si3_rest_oct",	 S_SI3_REST_EMPTY)
	SKIP_COMP ("opt_sel_par"	)
	SKIP_COMP ("pow_offs"	)
	SKIP_COMP ("si2ter_ind"	)
	SKIP_COMP ("es_ind_tag"	)
	SKIP_COMP ("if_and_where"	)
	SKIP_COMP ("gprs_indic"	)
ENDSTRUCT

BEGIN_MSTRUCT ("si4_rest_oct",	 S_SI4_REST_EMPTY)
	SKIP_COMP ("opt_sel_par"	)
	SKIP_COMP ("pow_offs"	)
	SKIP_COMP ("gprs_indic"	)
	SKIP_COMP ("lsa_param"	)
	SKIP_COMP ("cell_ident"	)
	SKIP_COMP ("lsa_id_info"	)
ENDSTRUCT

BEGIN_MSTRUCT ("si3_rest_oct",	 S_SI3_REST_LOW)
	SET_COMP ("opt_sel_par",	S_SI34_OPT_SEL_PAR)
	SKIP_COMP ("pow_offs"	)
	SKIP_COMP ("si2ter_ind"	)
	SKIP_COMP ("es_ind_tag"	)
	SKIP_COMP ("if_and_where"	)
	SKIP_COMP ("gprs_indic"	)
ENDSTRUCT

BEGIN_MSTRUCT ("si4_rest_oct",	 S_SI4_REST_LOW)
	SET_COMP ("opt_sel_par",	S_SI34_OPT_SEL_PAR)
	SKIP_COMP ("pow_offs"	)
	SKIP_COMP ("gprs_indic"	)
	SKIP_COMP ("lsa_param"	)
	SKIP_COMP ("cell_ident"	)
	SKIP_COMP ("lsa_id_info"	)
ENDSTRUCT

BEGIN_MSTRUCT ("opt_sel_par",	 S_SI34_OPT_SEL_PAR)
	SET_COMP ("cell_bar_qual",	CBQ_YES)
	SKIP_COMP ("cell_resel_offs"	)
	SKIP_COMP ("temp_offs"	)
	SKIP_COMP ("penalty_time"	)
ENDSTRUCT

BEGIN_MSTRUCT ("ia_rest_oct",	 S_IA_REST_1)
	SET_COMP ("ia_rest_oct_par",	S_IA_REST_OCT_PAR)
ENDSTRUCT

BEGIN_MSTRUCT ("ia_rest_oct_par",	 S_IA_REST_OCT_PAR)
	SET_COMP ("flag",	0x00)
	SET_COMP ("ia_freq_par",	S_IA_FREQ_PAR)
	SKIP_COMP ("ia_assign_par")
ENDSTRUCT

BEGIN_MSTRUCT ("ia_freq_par",	S_IA_FREQ_PAR)
	SET_COMP ("fp_len",	0x02)
	SET_COMP ("maio",	0x10)
	SET_COMP ("mac",	A_MAC_IA)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_32_0_6)
	SET_COMP ("rx_lev_ncell",	0x20)
	SET_COMP ("bcch_ncell",	0x00)
	SET_COMP ("bsic",	0x06)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	S_NCELL_63_8_1)
	SET_COMP ("rx_lev_ncell",	63)
	SET_COMP ("bcch_ncell",	8)
	SET_COMP ("bsic",	1)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	S_NCELL_63_3_7)
	SET_COMP ("rx_lev_ncell",	63)
	SET_COMP ("bcch_ncell",	3)
	SET_COMP ("bsic",	7)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	S_NCELL_61_6_5)
	SET_COMP ("rx_lev_ncell",	61)
	SET_COMP ("bcch_ncell",	6)
	SET_COMP ("bsic",	5)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	S_NCELL_56_7_3)
	SET_COMP ("rx_lev_ncell",	56)
	SET_COMP ("bcch_ncell",	7)
	SET_COMP ("bsic",	3)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	S_NCELL_36_5_1)
	SET_COMP ("rx_lev_ncell",	36)
	SET_COMP ("bcch_ncell",	5)
	SET_COMP ("bsic",	1)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	S_NCELL_32_1_3)
	SET_COMP ("rx_lev_ncell",	32)
	SET_COMP ("bcch_ncell",	1)
	SET_COMP ("bsic",	3)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_42_1_5)
	SET_COMP ("rx_lev_ncell",	0x2A)
	SET_COMP ("bcch_ncell",	0x01)
	SET_COMP ("bsic",	0x05)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_42_0_5)
	SET_COMP ("rx_lev_ncell",	0x2A)
	SET_COMP ("bcch_ncell",	0x00)
	SET_COMP ("bsic",	0x05)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_63_3_9)
	SET_COMP ("rx_lev_ncell",	63)
	SET_COMP ("bcch_ncell",	3)
	SET_COMP ("bsic",	9)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_63_6_15)
	SET_COMP ("rx_lev_ncell",	63)
	SET_COMP ("bcch_ncell",	6)
	SET_COMP ("bsic",	15)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_61_5_11)
	SET_COMP ("rx_lev_ncell",	61)
	SET_COMP ("bcch_ncell",	5)
	SET_COMP ("bsic",	11)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	S_NCELL_45_1_9)
	SET_COMP ("rx_lev_ncell",	45)
	SET_COMP ("bcch_ncell",	1)
	SET_COMP ("bsic",	9)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	S_NCELL_35_4_13)
	SET_COMP ("rx_lev_ncell",	35)
	SET_COMP ("bcch_ncell",	4)
	SET_COMP ("bsic",	13)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_0_0_0)
	SKIP_COMP ("rx_lev_ncell")
	SKIP_COMP ("bcch_ncell")
	SKIP_COMP ("bsic")
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_63_4_9)
	SET_COMP ("rx_lev_ncell",	0x3F)
	SET_COMP ("bcch_ncell",	0x04)
	SET_COMP ("bsic",	0x09)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_63_1_15)
	SET_COMP ("rx_lev_ncell",	0x3F)
	SET_COMP ("bcch_ncell",	0x01)
	SET_COMP ("bsic",	0x0F)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_63_2_13)
	SET_COMP ("rx_lev_ncell",	63)
	SET_COMP ("bcch_ncell",	2)
	SET_COMP ("bsic",	13)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_57_0_11)
	SET_COMP ("rx_lev_ncell",	0x39)
	SET_COMP ("bcch_ncell",	0x00)
	SET_COMP ("bsic",	0x0B)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_57_3_11)
	SET_COMP ("rx_lev_ncell",	0x39)
	SET_COMP ("bcch_ncell",	0x03)
	SET_COMP ("bsic",	0x0B)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_38_6_9)
	SET_COMP ("rx_lev_ncell",	0x26)
	SET_COMP ("bcch_ncell",	0x06)
	SET_COMP ("bsic",	0x09)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_32_5_15)
	SET_COMP ("rx_lev_ncell",	0x20)
	SET_COMP ("bcch_ncell",	0x05)
	SET_COMP ("bsic",	0x0F)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_28_1_13)
	SET_COMP ("rx_lev_ncell",	0x1C)
	SET_COMP ("bcch_ncell",	0x01)
	SET_COMP ("bsic",	0x0D)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_28_3_13)
	SET_COMP ("rx_lev_ncell",	0x1C)
	SET_COMP ("bcch_ncell",	0x03)
	SET_COMP ("bsic",	0x0D)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_63_7_9)
	SET_COMP ("rx_lev_ncell",	0x3F)
	SET_COMP ("bcch_ncell",	0x07)
	SET_COMP ("bsic",	0x09)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_60_1_15)
	SET_COMP ("rx_lev_ncell",	0x3C)
	SET_COMP ("bcch_ncell",	0x01)
	SET_COMP ("bsic",	0x0F)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_59_4_13)
	SET_COMP ("rx_lev_ncell",	0x3B)
	SET_COMP ("bcch_ncell",	0x04)
	SET_COMP ("bsic",	0x0D)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_51_0_11)
	SET_COMP ("rx_lev_ncell",	0x33)
	SET_COMP ("bcch_ncell",	0x00)
	SET_COMP ("bsic",	0x0B)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell", S_NCELL_34_5_9)
	SET_COMP ("rx_lev_ncell",	0x22)
	SET_COMP ("bcch_ncell",	0x05)
	SET_COMP ("bsic",	0x09)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_63_2_9)
	SET_COMP ("rx_lev_ncell",	0x3F)
	SET_COMP ("bcch_ncell",	0x02)
	SET_COMP ("bsic",	0x09)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_63_0_15)
	SET_COMP ("rx_lev_ncell",	0x3F)
	SET_COMP ("bcch_ncell",	0x00)
	SET_COMP ("bsic",	0x0F)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_63_3_13)
	SET_COMP ("rx_lev_ncell",	0x3F)
	SET_COMP ("bcch_ncell",	0x03)
	SET_COMP ("bsic",	0x0D)
ENDSTRUCT

BEGIN_MSTRUCT ("ncell",	 S_NCELL_57_1_11)
	SET_COMP ("rx_lev_ncell",	0x39)
	SET_COMP ("bcch_ncell",	0x01)
	SET_COMP ("bsic",	0x0B)
ENDSTRUCT

BEGIN_MSTRUCT ("freq_list_after",	 S_FREQ_LIST_AFTER_2)
	SET_COMP ("flist",	A_FREQ_LIST_AFTER_2)
ENDSTRUCT

BEGIN_MSTRUCT ("freq_list_before",	 S_FREQ_LIST_BEFORE_1)
	SET_COMP ("flist",	A_FREQ_LIST_BEFORE_1)
ENDSTRUCT

BEGIN_MSTRUCT ("ext_meas_res",	S_EXT_MEAS_RES_000)

SET_COMP ("sc_used",	CONST_1)
	SET_COMP ("dtx_used",	CONST_1)
	SET_COMP ("rx_lev_ncell",	A_EXT_MEAS_RES_000_LEV)
ENDSTRUCT

BEGIN_MSTRUCT ("ext_meas_res",	S_EXT_MEAS_RES_001)

SET_COMP ("sc_used",	CONST_0)
	SET_COMP ("dtx_used",	CONST_1)
	SET_COMP ("rx_lev_ncell",	A_EXT_MEAS_RES_001_LEV)
ENDSTRUCT

BEGIN_MSTRUCT ("ext_meas_res",	S_EXT_MEAS_RES_002)

SET_COMP ("sc_used",	CONST_0)
	SET_COMP ("dtx_used",	CONST_1)
	SET_COMP ("rx_lev_ncell",	A_EXT_MEAS_RES_002_LEV)
ENDSTRUCT

BEGIN_MSTRUCT ("ext_meas_res",	S_EXT_MEAS_RES_003)

SET_COMP ("sc_used",	CONST_1)
	SET_COMP ("dtx_used",	CONST_1)
	SET_COMP ("rx_lev_ncell",	A_EXT_MEAS_RES_003_LEV)
ENDSTRUCT

# TEST CASES

## Routing (internal)

### RR000: Setup the routing and PCO view for the RR test

Description:	Routings for the RR tests are set. All stored BCCH info are cleared.

Preamble:	none

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (CC RESET)
COMMAND (SS RESET)
COMMAND (SMS RESET)
COMMAND (MM RESET)
COMMAND (RR RESET)
COMMAND (DL RESET)
COMMAND (SIM RESET)
COMMAND (RRLP RESET)
COMMAND (PL RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (RR REDIRECT CLEAR)
COMMAND (DL REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (RRLP REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)
	 |		  |		  |
COMMAND (MMI REDIRECT CC NULL)
COMMAND (MMI REDIRECT SS NULL)
COMMAND (MMI REDIRECT SMS NULL)
COMMAND (MMI REDIRECT PL NULL))
COMMAND (MMI REDIRECT RR NULL)
	 |		  |		  |
COMMAND (MM REDIRECT MMI NULL)
COMMAND (MM REDIRECT RR NULL)
COMMAND (MM REDIRECT DL NULL)
	 |		  |		  |
COMMAND (RR REDIRECT PL TAP)
COMMAND (RR REDIRECT DL TAP)
COMMAND (RR REDIRECT MM TAP)

COMMAND (RR REDIRECT LC TAP)
COMMAND (RR REDIRECT RRLP TAP)
COMMAND (RR REDIRECT MMI TAP)
COMMAND (RR CONFIG NO_SYS_TIME)
	 |		  |		  |
COMMAND (DL REDIRECT RR NULL)
COMMAND (DL REDIRECT PL NULL)
	 |		  |		  |
COMMAND (PL REDIRECT RR NULL)
COMMAND (PL REDIRECT DL NULL)
COMMAND (PL REDIRECT MMI NULL)
	 |		  |		  |
COMMAND (TAP REDIRECT TAP RR)
COMMAND (RR CONFIG CLEAR_BCCH_INFO)
(1)	 |	RR_SYNC_IND	  |		  |
	 |	(clear BCCH info)	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_SYNC_IND	ciph	NOT_PRESENT_8BIT
		mm_info	NOT_USED
		bcch_info	S_BCCH_INFO_CLEAR
		synccs	NOT_PRESENT_16BIT
		chm	NOT_USED

History:	04.07.97	PZ	Initial
	24-Mar-03	MSB	clear all stored BCCH infos.

## Cell Selection

### RR001: Start cell selection (std = 1)

Description:	A cell selection without BCCH information is started. 
Variant A: no SIM is inserted.
Variant B: SIM is inserted, test SIM card, IMSI in the HPLMN, TMSI
Variant C: SIM is inserted, normal SIM card, IMSI in the HPLMN, no TMSI
Variant D: SIM is inserted, test SIM card, IMSI in the HPLMN, no TMSI
Variant E: SIM is inserted, normal SIM card, IMSI in the HPLMN, TMSI

Preamble:	RR000

Variants:	<A>....<E>

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
<A>	plmn	PLMN_ID_EMPTY
<B>	plmn	PLMN_ID_123
<C>	plmn	PLMN_ID_123
<D>	plmn	PLMN_ID_123
<E>	plmn	PLMN_ID_123
<A>	op	OP_MODE_EMPTY
<B>	op	OP_MODE_TEST_SIM
<C>	op	OP_MODE_NORMAL
<D>	op	OP_MODE_TEST_SIM
<E>	op	OP_MODE_NORMAL
<A>	cksn	CKSN_NOT_PRES
<B>	cksn	CKSN_NOT_PRES
<C>	cksn	CKSN_NOT_PRES
<D>	cksn	CKSN_NOT_PRES
<E>	cksn	CKSN_6
<A>	kcv	KCV_EMPTY
<B>	kcv	KCV_12345678
<C>	kcv	KCV_12345678
<D>	kcv	KCV_12345678
<E>	kcv	KCV_12345678
<A>	accc	ACC_CTRL_CLASS_0000
<B>	accc	ACC_CTRL_CLASS_0008
<C>	accc	ACC_CTRL_CLASS_0008
<D>	accc	ACC_CTRL_CLASS_4000
<E>	accc	ACC_CTRL_CLASS_0000
<A>	imsi_struct	MOBILE_ID_NOT_SET
<B>	imsi_struct	MOBILE_ID_IMSI_HPLMN
<C>	imsi_struct	MOBILE_ID_IMSI_HPLMN
<D>	imsi_struct	MOBILE_ID_IMSI_HPLMN
<E>	imsi_struct	MOBILE_ID_IMSI_HPLMN
<A>	tmsi_struct	MOBILE_ID_NOT_SET
<B>	tmsi_struct	MOBILE_ID_TMSI
<C>	tmsi_struct	MOBILE_ID_NOT_SET
<D>	tmsi_struct	MOBILE_ID_NOT_SET
<E>	tmsi_struct	MOBILE_ID_TMSI
<A>	thplmn	TIME_HPLMN_EMPTY
<B>	thplmn	TIME_HPLMN_VALID
<C>	thplmn	TIME_HPLMN_VALID
<D>	thplmn	TIME_HPLMN_VALID
<E>	thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

History:	04-Jul-97	DL	Initial
	31-Aug-98	LE	rr_activate_req adapted
	06-Jan-99	LE	optimised cell selection
	12-May-00	DG	RR_ACTIVATE_REQ: cell_test added
	19-Jan-01	DG	RR_ACTIVATE_REQ: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR002: Start cell selection with or without BCCH information (std 6)

Description:	A cell selection without BCCH information is started. 
Variant A: no BCCH information on SIM, , PCM sets channel.
Variant B: BCCH information on SIM GSM 900 channel not set, PCM sets channel
Variant C: BCCH information on SIM GSM 900 channel set, PCM sets channel

Preamble:	RR000

Variants:	<A>....<C>

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
	plmn	PLMN_ID_123
	op	OP_MODE_NORMAL
	cksn	CKSN_NOT_PRES
	kcv	KCV_12345678
	accc	ACC_CTRL_CLASS_0008
	imsi_struct	MOBILE_ID_IMSI_HPLMN
	tmsi_struct	MOBILE_ID_TMSI
	thplmn	TIME_HPLMN_VALID
<A>	bcch_info	S_BCCH_INFO_EMPTY
<B>	bcch_info	S_BCCH_INFO_NO_24
<C>	bcch_info	S_BCCH_INFO_24
	cell_test	CELL_TEST_DISABLE
	gprs_indication	NOT_USED

- MPH_POWER_REQ
	pch_interrupt	PCH_INTERRUPT
	freq_bands	NOT_USED

- MPH_POWER_CNF
	num_of_chan	CHANNELS_2
	arfcn	ARFCN_24_527
	rx_lev	RXLEV_22_21

- MPH_BSIC_REQ
<A>	arfcn	ARFCN_527
<B>	arfcn	ARFCN_527
<C>	arfcn	ARFCN_24

History:	15-Feb-00	LE	Initialselection
	12-May-00	DG	RR_ACTIVATE_REQ: cell_test added
	19-Jan-01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	05-Jun-01	MSE	adapted to TAP2

### RR003: Start cell selection with or without BCCH information (std 5)

Description:	A cell selection without BCCH information is started. 
Variant A: no BCCH information on SIM, ignore PCM.
Variant B: BCCH information on SIM GSM 900 channel not set, PCM sets no channel
Variant C: BCCH information on SIM GSM 900 channel set, PCM sets channel

Preamble:	RR000

Variants:	<A>....<C>

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
	plmn	PLMN_ID_123
	op	OP_MODE_NORMAL
	cksn	CKSN_NOT_PRES
	kcv	KCV_12345678
	accc	ACC_CTRL_CLASS_0008
	imsi_struct	MOBILE_ID_IMSI_HPLMN
	tmsi_struct	MOBILE_ID_TMSI
	thplmn	TIME_HPLMN_VALID
<A>	bcch_info	S_BCCH_INFO_EMPTY
<B>	bcch_info	S_BCCH_INFO_NO_24
<C>	bcch_info	S_BCCH_INFO_24
	cell_test	CELL_TEST_DISABLE
	gprs_indication	NOT_USED

- MPH_POWER_REQ
	pch_interrupt	PCH_INTERRUPT
	freq_bands	NOT_USED

- MPH_POWER_CNF
	num_of_chan	CHANNELS_2
	arfcn	ARFCN_24_527
	rx_lev	RXLEV_22_21

- MPH_BSIC_REQ
	arfcn	ARFCN_24

History:	15-Feb-00	LE	Initial
	12-May-00	DG	RR_ACTIVATE_REQ: cell_test added
	19-Jan-01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	05-Jun-01	MSE	adapted to TAP2

### RR004: Start cell selection without BCCH information (std 4)

Description:	A cell selection without BCCH information is started.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=4)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SI 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SI 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SI 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SI 4)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(14)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(15)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
	plmn	PLMN_ID_123
	op	OP_MODE_NORMAL
	cksn	CKSN_NOT_PRES
	kcv	KCV_12345678
	accc	ACC_CTRL_CLASS_0008
	imsi_struct	MOBILE_ID_IMSI_HPLMN
	tmsi_struct	MOBILE_ID_TMSI
	thplmn	TIME_HPLMN_VALID
	bcch_info	S_BCCH_INFO_EMPTY
	cell_test	CELL_TEST_DISABLE
	gprs_indication	NOT_USED

- MPH_POWER_REQ
	pch_interrupt	PCH_INTERRUPT
	freq_bands	NOT_USED

- MPH_POWER_CNF
	num_of_chan	CHANNELS_2
	arfcn	A_ARFCN_527_601
	rx_lev	RXLEV_22_21

- MPH_BSIC_REQ
	arfcn	ARFCN_527

- MPH_BSIC_CNF
	arfcn	ARFCN_527
	bsic	BSIC_6
	cs	CS_NO_ERROR

- MPH_UNITDATA_IND
	arfcn	ARFCN_527
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_1
	ti	TI_0
	cell_chan_desc	CELL_ALLOC_1024
	rach_ctrl	RACH_CTRL_1
	si1_rest_oct	NOT_USED
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_527
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_2
	ti	TI_0
	neigh_cell_desc	NCELL_DESC_1
	ncc_permit	NCC_PERMITTED_1
	rach_ctrl	RACH_CTRL_1
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_527
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_3
	ti	TI_0
	cell_ident	CELL_IDENT_3748
	loc_area_ident	LOC_AREA_IDENT_123_2147
	ctrl_chan_desc	CTRL_CHAN_DESC_1
	cell_opt_bcch	CELL_OPT_BCCH_1
	cell_select	CELL_SELECT_1
	rach_ctrl	RACH_CTRL_1
	si3_rest_oct	S_SI3_REST_EMPTY
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_527
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_4
	ti	TI_0
	loc_area_ident	LOC_AREA_IDENT_123_2147
	cell_select	CELL_SELECT_1
	rach_ctrl	RACH_CTRL_1
	chan_desc	CHANNEL_DESC_SDCCH2_1800
	mob_alloc	NOT_USED
	si4_rest_oct	S_SI4_REST_EMPTY
	}

- MPH_CLASSMARK_REQ
	classmark	CLASS_MS_1800

- MPH_IDLE_REQ
	mod	MODE_CELL_SELECTION
	arfcn	ARFCN_527
	ext_bcch	NOT_USED
	comb_ccch	CCD_CCCH_1_NOT_COMB
	tn	TN_0
	dlt	DLT_23
	pg	PG_0
	bs_ag_blocks_res	BS_AG_BLKS_RES_5
	bs_pa_mfrms	BS_PA_MFRMS_2
	power	MS_TXPWR_MAX_CCH_02
	ncc_permitted	NCC_PERMITTED_1
	reorg_only	NOT_USED
	eotd_avail	CONST_0
	gprs_support	NOT_USED

- MPH_CBCH_REQ
	cbch	PRR_CHANNEL_TYPE_3_CB_1800

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_EMPTY
	sync_only	NOT_USED

- RR_ACTIVATE_CNF
	op	OP_MODE_NORMAL
	mm_info	MM_INFO_1
	cid	CELL_IDENT_3748
	plmn	PLMN_ID_123
	lac	LAC_2147
	power	NOT_USED
	gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
	mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	27-Jun-02	MPA	initial

### RR010 : Network Sync Timeout

Description:	PL cannot synchronise to the specified ARFCN and so returns a MPH_BSIC_CNF with the cause BCCH_NO_AVAIL. RR then proceeds to send the next BSIC id to be read.

Preamble:	RR002C

MM		 RR		PL/DL
	 |		  |		  |
TIMEOUT (10000) 
(1)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *===============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
	arfcn	NOT_USED
	bsic	NOT_USED
	cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
	arfcn	ARFCN_527

History:	19-Mar-03	ZMM	initial

### RR011: Listen to SYS INFO (without SIM)

Description:	PL detects a BCCH carrier and RR listen to the BCCH messages. 
Variant A: cell is barred.
Variant B: C1 < 0
Variant C: BCCH ok
Variant D: BCCH ok, low priority cell

Preamble:	RR001A

Variants:	<A>....<D>

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_CNF
	num_of_chan	CHANNELS_3
	arfcn	ARFCN_67_32_124
<A>	rx_lev	RXLEV_22_21_20
<B>	rx_lev	RXLEV_10_8_4
<C>	rx_lev	RXLEV_22_21_20
<D>	rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
	arfcn	ARFCN_67

- MPH_BSIC_CNF
	arfcn	ARFCN_67
	bsic	BSIC_6
	cs	CS_NO_ERROR

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_1
	ti	TI_0
	cell_chan_desc	CELL_CHAN_DESC_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_2
	ti	TI_0
<A>	neigh_cell_desc	NCELL_DESC_1
<B>	neigh_cell_desc	NCELL_DESC_1
<C>	neigh_cell_desc	NCELL_DESC_1
<D>	neigh_cell_desc	NCELL_DESC_8
	ncc_permit	NCC_PERMITTED_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_3
	ti	TI_0
	cell_ident	CELL_IDENT_3748
	loc_area_ident	LOC_AREA_IDENT_123_2147
	ctrl_chan_desc	CTRL_CHAN_DESC_1
	cell_opt_bcch	CELL_OPT_BCCH_1
	cell_select	CELL_SELECT_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
<A>	si3_rest_oct	S_SI3_REST_EMPTY
<B>	si3_rest_oct	S_SI3_REST_EMPTY
<C>	si3_rest_oct	S_SI3_REST_EMPTY
<D>	si3_rest_oct	S_SI3_REST_LOW
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_4
	ti	TI_0
	loc_area_ident	LOC_AREA_IDENT_123_2147
	cell_select	CELL_SELECT_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
	chan_desc	NOT_USED
	mob_alloc	NOT_USED
<A>	si4_rest_oct	S_SI4_REST_EMPTY
<B>	si4_rest_oct	S_SI4_REST_EMPTY
<C>	si4_rest_oct	S_SI4_REST_EMPTY
<D>	si4_rest_oct	S_SI4_REST_LOW
	}

History:	04.07.97	DL	Initial
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct:
				SI3_REST_DEF > SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR012: Listen to SYS INFOs (with SIM)

Description:	Listening to a BCCH carrier.
Variant A: cell is barred
Variant B: C1 < 0
Variant C: wrong PLMN
Variant D: BCCH ok
Variant E: BCCH ok, but low priority cell
Variant F: BCCH ok with CBCH

Preamble:	RR001B

Variants:	<A>....<F>

MM		RR		PL/DL
	|		|		|
(1)	|		|	MPH_POWER_CNF	|
	|			*<==========================*
(2)	|		|	MPH_BSIC_REQ	|
	|			*==========================>*
(3)	|		|	MPH_BSIC_CNF	|
	|			*<==========================*
(4)	|		|	MPH_UNITDATA_IND	|
	|		|	  (SYS INFO TYPE 1)	|
	|			*<==========================*
(5)	|		|	MPH_UNITDATA_IND	|
	|		|	  (SYS INFO TYPE 2)	|
	|			*<==========================*
(6)	|		|	MPH_UNITDATA_IND	|
	|		|	  (SYS INFO TYPE 3)	|
	|			*<===========================*
(7)	|		|	MPH_UNITDATA_IND	|
	|		|	  (SYS INFO TYPE 4)	|
	|			*<==========================*
	|		|		|

Parametrization

Primitive	Parameter	Value

- MPH_POWER_CNF
	num_of_chan	CHANNELS_3
	arfcn	ARFCN_67_32_124
<A>	rx_lev	RXLEV_22_21_20
<B>	rx_lev	RXLEV_10_8_4
<C>	rx_lev	RXLEV_22_21_20
<D>	rx_lev	RXLEV_22_21_20
<E>	rx_lev	RXLEV_22_21_20
<F>	rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
	arfcn	ARFCN_67

- MPH_BSIC_CNF
	arfcn	ARFCN_67
	bsic	BSIC_5
	cs	CS_NO_ERROR

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_1
	ti	TI_0
	cell_chan_desc	CELL_CHAN_DESC_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
<E>	rach_ctrl	RACH_CTRL_1
<F>	rach_ctrl	RACH_CTRL_1
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_2
	ti	TI_0
<A>	neigh_cell_desc	NCELL_DESC_2
<B>	neigh_cell_desc	NCELL_DESC_2
<C>	neigh_cell_desc	NCELL_DESC_2
<D>	neigh_cell_desc	NCELL_DESC_2
<E>	neigh_cell_desc	NCELL_DESC_1
<F>	neigh_cell_desc	NCELL_DESC_1
	ncc_permit	NCC_PERMITTED_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
<E>	rach_ctrl	RACH_CTRL_1
<F>	rach_ctrl	RACH_CTRL_1
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_3
	ti	TI_0
	cell_ident	CELL_IDENT_3748
<A>	loc_area_ident	LOC_AREA_IDENT_123_2147
<B>	loc_area_ident	LOC_AREA_IDENT_123_2147
<C>	loc_area_ident	LOC_AREA_IDENT_122_2147
<D>	loc_area_ident	LOC_AREA_IDENT_123_2147
<E>	loc_area_ident	LOC_AREA_IDENT_123_2147
<F>	loc_area_ident	LOC_AREA_IDENT_123_2147
	ctrl_chan_desc	CTRL_CHAN_DESC_1
	cell_opt_bcch	CELL_OPT_BCCH_1
	cell_select	CELL_SELECT_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
<E>	rach_ctrl	RACH_CTRL_1
<F>	rach_ctrl	RACH_CTRL_1
<A>	si3_rest_oct	S_SI3_REST_EMPTY
<B>	si3_rest_oct	S_SI3_REST_EMPTY
<C>	si3_rest_oct	S_SI3_REST_EMPTY
<D>	si3_rest_oct	S_SI3_REST_EMPTY
<E>	si3_rest_oct	S_SI3_REST_LOW
<F>	si3_rest_oct	S_SI3_REST_EMPTY
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_4
	ti	TI_0
<A>	loc_area_ident	LOC_AREA_IDENT_123_2147
<B>	loc_area_ident	LOC_AREA_IDENT_123_2147
<C>	loc_area_ident	LOC_AREA_IDENT_122_2147
<D>	loc_area_ident	LOC_AREA_IDENT_123_2147
<E>	loc_area_ident	LOC_AREA_IDENT_123_2147
<F>	loc_area_ident	LOC_AREA_IDENT_123_2147
	cell_select	CELL_SELECT_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
<E>	rach_ctrl	RACH_CTRL_1
<F>	rach_ctrl	RACH_CTRL_1
<A>	chan_desc	NOT_USED
<B>	chan_desc	NOT_USED
<C>	chan_desc	NOT_USED
<D>	chan_desc	NOT_USED
<E>	chan_desc	NOT_USED
<F>	chan_desc	CHANNEL_DESC_SDCCH2
	mob_alloc	NOT_USED
<A>	si4_rest_oct	S_SI4_REST_EMPTY
<B>	si4_rest_oct	S_SI4_REST_EMPTY
<C>	si4_rest_oct	S_SI4_REST_EMPTY
<D>	si4_rest_oct	S_SI4_REST_EMPTY
<E>	si4_rest_oct	S_SI4_REST_LOW
<F>	si4_rest_oct	S_SI4_REST_EMPTY
	}

History:	04.07.97	DL	Initial
	10.04.00	LE (DG)	values changed: si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct:
				SI3_REST_DEF > SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR013: Listen to SYS INFOs (with SIM)

Description:	Listen to a second BCCH carrier.
Variant A: Cell barred
Variant B: C1 < 0
Variant C: Wrong PLMN
Variant D: BCCH ok
Variant E: BCCH ok, Neighbourcell Description Range 128 Format
Variant F: BCCH ok, Neighbourcell Description Range 256 Format
Variant G: BCCH ok, Neighbourcell Description Range 512 Format
Variant H: BCCH ok, Neighbourcell Description Range 1024 Format
Variant I: BCCH ok, Neighbourcell Description Variable Format.

Preamble:	RR027

Variants:	<A>....<I>

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
	arfcn	ARFCN_32
	bsic	BSIC_5
	cs	CS_NO_ERROR

- MPH_UNITDATA_IND
	arfcn	ARFCN_32
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_1
	ti	TI_0
	cell_chan_desc	CELL_CHAN_DESC_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
<E>	rach_ctrl	RACH_CTRL_1
<F>	rach_ctrl	RACH_CTRL_1
<G>	rach_ctrl	RACH_CTRL_1
<H>	rach_ctrl	RACH_CTRL_1
<I>	rach_ctrl	RACH_CTRL_1
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_32
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_2
	ti	TI_0
<A>	neigh_cell_desc	NCELL_DESC_1
<B>	neigh_cell_desc	NCELL_DESC_1
<C>	neigh_cell_desc	NCELL_DESC_1
<D>	neigh_cell_desc	NCELL_DESC_1
<E>	neigh_cell_desc	NCELL_DESC_128
<F>	neigh_cell_desc	NCELL_DESC_256
<G>	neigh_cell_desc	NCELL_DESC_512
<H>	neigh_cell_desc	NCELL_DESC_1024
<I>	neigh_cell_desc	NCELL_DESC_VAR
	ncc_permit	NCC_PERMITTED_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
<E>	rach_ctrl	RACH_CTRL_1
<F>	rach_ctrl	RACH_CTRL_1
<G>	rach_ctrl	RACH_CTRL_1
<H>	rach_ctrl	RACH_CTRL_1
<I>	rach_ctrl	RACH_CTRL_1
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_32
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_3
	ti	TI_0
	cell_ident	CELL_IDENT_3748
<A>	loc_area_ident	LOC_AREA_IDENT_123_2147
<B>	loc_area_ident	LOC_AREA_IDENT_123_2147
<C>	loc_area_ident	LOC_AREA_IDENT_122_2147
<D>	loc_area_ident	LOC_AREA_IDENT_123_2147
<E>	loc_area_ident	LOC_AREA_IDENT_123_2147
<F>	loc_area_ident	LOC_AREA_IDENT_123_2147
<G>	loc_area_ident	LOC_AREA_IDENT_123_2147
<H>	loc_area_ident	LOC_AREA_IDENT_123_2147
<I>	loc_area_ident	LOC_AREA_IDENT_123_2147
	ctrl_chan_desc	CTRL_CHAN_DESC_1
	cell_opt_bcch	CELL_OPT_BCCH_1
<A>	cell_select	CELL_SELECT_1
<B>	cell_select	CELL_SELECT_2
<C>	cell_select	CELL_SELECT_1
<D>	cell_select	CELL_SELECT_1
<E>	cell_select	CELL_SELECT_1
<F>	cell_select	CELL_SELECT_1
<G>	cell_select	CELL_SELECT_1
<H>	cell_select	CELL_SELECT_1
<I>	cell_select	CELL_SELECT_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
<E>	rach_ctrl	RACH_CTRL_1
<F>	rach_ctrl	RACH_CTRL_1
<G>	rach_ctrl	RACH_CTRL_1
<H>	rach_ctrl	RACH_CTRL_1
<I>	rach_ctrl	RACH_CTRL_1
	si3_rest_oct	S_SI3_REST_EMPTY
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_32
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_4
	ti	TI_0
<A>	loc_area_ident	LOC_AREA_IDENT_123_2147
<B>	loc_area_ident	LOC_AREA_IDENT_123_2147
<C>	loc_area_ident	LOC_AREA_IDENT_122_2147
<D>	loc_area_ident	LOC_AREA_IDENT_123_2147
<E>	loc_area_ident	LOC_AREA_IDENT_123_2147
<F>	loc_area_ident	LOC_AREA_IDENT_123_2147
<G>	loc_area_ident	LOC_AREA_IDENT_123_2147
<H>	loc_area_ident	LOC_AREA_IDENT_123_2147
<I>	loc_area_ident	LOC_AREA_IDENT_123_2147
<A>	cell_select	CELL_SELECT_1
<B>	cell_select	CELL_SELECT_2
<C>	cell_select	CELL_SELECT_1
<D>	cell_select	CELL_SELECT_1
<E>	cell_select	CELL_SELECT_1
<F>	cell_select	CELL_SELECT_1
<G>	cell_select	CELL_SELECT_1
<H>	cell_select	CELL_SELECT_1
<I>	cell_select	CELL_SELECT_1
<A>	rach_ctrl	RACH_CTRL_2
<B>	rach_ctrl	RACH_CTRL_1
<C>	rach_ctrl	RACH_CTRL_1
<D>	rach_ctrl	RACH_CTRL_1
<E>	rach_ctrl	RACH_CTRL_1
<F>	rach_ctrl	RACH_CTRL_1
<G>	rach_ctrl	RACH_CTRL_1
<H>	rach_ctrl	RACH_CTRL_1
<I>	rach_ctrl	RACH_CTRL_1
	chan_desc	NOT_USED
	mob_alloc	NOT_USED
	si4_rest_oct	S_SI4_REST_EMPTY
	}

History:	04.07.97	DL	Initial
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	08.02.01	DG	si3/4_rest_oct:
				SI3_REST_DEF > SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR021: BCCH carrier not suitable (cell barred, without SIM)

Description:	The SYS INFO messages have reported that the BCCH carrier is not suitable due to the cell being barred. RR resumes the search for a BCCH carrier by issuing a MPH-NEXT request primitive.

Preamble:	RR011A

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR022: BCCH carrier not suitable (C1 <= 0, without SIM)

Description:	The SYS INFO messages have reported that the BCCH carier is not suitable as the path loss criterion is not greater than 0. RR resumes the search for a BCCH carrier by issuing a MPH-NEXT request primitive.

Preamble:	RR011B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR023: BCCH carrier not suitable (cell barred, with SIM, second cell)

Description:	The SYS INFO messages have reported that the BCCH carier is not suitable as the cell is barred. RR resumes the search for a BCCH carrier by issuing a MPH-NEXT request primitive. (Ref.)

Preamble:	RR013A

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_124

History:	04.07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR024: BCCH carrier not suitable (C1 <= 0, with cell, second cell)

Description:	The SYS INFO messages have reported that the BCCH carier is not suitable as the path loss criterion is not greater than 0. RR resumes the search for a BCCH carrier by issuing a MPH-NEXT request primitive. (Ref.)

Preamble:	RR013B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_124

History:	04.07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR025: BCCH carrier not suitable (incorrect network, with SIM, second cell)

Description:	The SYS INFO messages have reported that the BCCH carier is not suitable as the network is incorrect. RR resumes the search for a BCCH carrier by issuing a MPH-NEXT request primitive. (Ref.)

Preamble:	RR013C

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_124

History:	04. 07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR027: BCCH carrier not suitable (cell barred, with SIM)

Description:	The SYS INFO messages have reported that the BCCH carier is not suitable as the cell is barred. RR resumes the search with a MPH-BSIC-REQ request using the neighbourcell description of the previous attempt.

Preamble:	RR012A

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04. 07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR028: BCCH carrier not suitable (C1 <= 0, with SIM)

Description:	The SYS INFO messages have reported that the BCCH carier is not suitable as the path loss criterion is to low. RR resumes the search with a MPH-BSIC-REQ request using the neighbourcell description of the previous attempt.

Preamble:	RR012B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04. 07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR029: BCCH carrier not suitable (incorrect network, with SIM)

Description:	The SYS INFO messages have reported that the BCCH carier is not suitable as the network is incorrect. RR resumes the search for a BCCH carrier by issuing a MPH-BSIC_REQ request primitive.

Preamble:	RR012C

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04. 07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR030: Suitable BCCH carrier found (with SIM)

Description:	The SYS INFO messages have reported a suitable BCCH carrier. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive.

Preamble:	RR012D

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_2
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04. 07.97	DL	Initial
	15.05.98	VK	MPH_CBCH_REQ
	02.03.00	DG	RR_SYNC_IND
	10.04.00	LE (DG)	value changed: MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	29.05.00	DG	value changed:
				RR_ACTIVATE_CNF:
				MM_INFO_1 > MM_INFO_2
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR033: Suitable BCCH carrier found (without SIM)

Description:	The SYS INFO messages have reported a suitable BCCH carrier. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive.

Preamble:	RR011C

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_LIMITED
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_EMPTY
		mm_info	MM_INFO_1
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_NO_IMSI_NO_TMSI

History:	04.07.97	DL	Initial
	03.03.00	DG	RR_SYNC_IND
	11.04.00	LE (DG)	values changed:
				RR_SYNC_IND/bcch_info:
				NOT_USED > BCCH_INFO_1_PLUS_43
				RR_ACTIVATE_CNF/op:
				OP_MODE_NO_SIM_LIM_SERV >
				OP_MODE_EMPTY
				RR_ACTIVATE_CNF/mm_info:
				MM_INFO_3 >  MM_INFO_1
	12.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_6
	08.06.00	DG	value changed:
				MPH_NEIGHBOURCELL_REQ:
				MPH_NCELL_1_LIM > MPH_NCELL_1
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR034: Suitable BCCH carrier found (low priority cell, without SIM)

Description:	The SYS INFO messages have reported a suitable BCCH carrier, also it is a low priority cell. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive.

Preamble:	RR011D

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_LIMITED
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_8
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_EMPTY
		mm_info	MM_INFO_1
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_NO_IMSI_NO_TMSI

History:	04.07.97	DL	Initial
	03.03.00	DG	RR_SYNC_IND
	13.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_6
	08.06.00	DG	value changed:
				MPH_NEIGHBOURCELL_REQ:
				MPH_NCELL_1_LIM > MPH_NCELL_8
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR035: Suitable BCCH carrier found (with SIM, second cell)

Description:	The SYS INFO messages have reported a suitable BCCH carrier. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive.

Preamble:	RR013D

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_SYNC_IND	  |		  |
	 *<==============================*		  |
(6)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(7)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1
		sync_only	NOT_USED

- RR_SYNC_IND
		ciph	NOT_PRESENT_8BIT
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	NOT_PRESENT_16BIT
		chm	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	03.03.00	DG	RR_SYNC_IND
	13.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR036: Suitable BCCH carrier found - NCell description in 128-range format

Description:	The SYS INFO messages have reported a suitable BCCH carrier with the NCell description in the 128-range format. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive.

Preamble:	RR013E

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1B
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	03.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR037: Suitable BCCH carrier found - NCell description in 256-range format

Description:	The SYS INFO messages have reported a suitable BCCH carrier with the NCell description in the 256-range format. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive.

Preamble:	RR013F

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1B
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR038: Suitable BCCH carrier found - NCell description in 512-range format

Description:	The SYS INFO messages have reported a suitable BCCH carrier with the NCell description in the 512-range format. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive.

Preamble:	RR013G

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1B
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
	mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR039: Suitable BCCH carrier found - NCell description in 1024-range format

Description:	The SYS INFO messages have reported a suitable BCCH carrier with the NCell description in the 1024-range format. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive.

Preamble:	RR013H

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1B
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR040: Suitable BCCH carrier found - NCell description in variable-bit format

Description:	The SYS INFO messages have reported a suitable BCCH carrier with the NCell description in variable-bit format. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive.

Preamble:	RR013I

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1B
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR041: Listen to SYS INFOs, SYS INFO 1 not required

Description:	Listening to a BCCH carrier. System Information 1 is not necessary to select the BCCH. Then in idle mode the system information type 1 message receives. There is no reaction expected by RR.

Preamble:	RR001B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_SYNC_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(12)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(13)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
TIMEOUT (3)
(15)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_CNF
	num_of_chan	CHANNELS_3
	arfcn	ARFCN_67_32_124
	rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
	arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_SYNC_IND
	cs	CS_SYS_INFO_1_NOT_NEEDED
	arfcn	ARFCN_67

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
				si3/4_rest_oct:
				SI3_REST_DEF > SI3_REST_EMPTY
	19.02.01	DG	MPH_ERROR_IND > MPH_SYNC_IND
	05-Jun-01	MSE	adapted to TAP2

### RR070: No BCCH carrier available

Description:	PL signals that no BCCH carrier is available. MM is informed by a RR-ABORT indication.

Preamble:	RR001B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(9)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
			arfcn	ARFCN_67
			bsic	BSIC_0
			cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
			arfcn	ARFCN_32
			bsic	BSIC_0
			cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
			arfcn	ARFCN_124
			bsic	BSIC_0
			cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
			op	OP_MODE_TEST_SIM_NO_SERV
			cause	RRCS_ABORT_CEL_SEL_FAIL
			plmn_avail	NO_PLMN_AVAILABLE
			plmn	NOT_USED
			lac_list	NOT_USED
			rxlevel	NOT_USED
			power	NOT_USED

History:	04.07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	25.01.01	DG	final MPH_BSIC_REQ 
				(NOT_PRESENT_16BIT_MIN1)added
	12.02.01	DG	final MPH_BSIC_REQ replaced
				by MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR071: No BCCH carrier available (limited service possible)

Description:	PL signals that no BCCH carrier is available. MM is informed by a RR-ABORT indication. One network was detected in the preamble.

Preamble:	RR029

MM		 RR		PL/DL
	 |		  |		  |

- |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*

- |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*

- |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*

- |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*

- |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*

- |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*

- |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*

- |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*

- |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*

- |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*

- |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*

- |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*

- |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |

- |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*

|		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
			classmark	CLASS_MS

- MPH_IDLE_REQ
			mod	MODE_CELL_SELECTION
			arfcn	ARFCN_67
			ext_bcch	NOT_USED
			comb_ccch	CCD_CCCH_1_NOT_COMB
			tn	TN_0
			dlt	DLT_10
			pg	PG_0
			bs_ag_blocks_res	BS_AG_BLKS_RES_5
			bs_pa_mfrms	BS_PA_MFRMS_7
			power	MS_TXPWR_MAX_CCH_02
			ncc_permitted	NCC_PERMITTED_FF
			reorg_only	NOT_USED
			eotd_avail	CONST_0
			gprs_support	NOT_USED

- MPH_CBCH_REQ
			cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
			multi_band	NOT_USED
			arfcn	A_MPH_NCELL_1C
			sync_only	NOT_USED

- RR_ABORT_IND
			op	OP_MODE_TEST_SIM_LIM_SERV
			cause	RRCS_ABORT_CEL_SEL_FAIL
			plmn_avail	ONE_PLMN_AVAILABLE
			plmn	AS_PLMN
			lac_list	A_LAC_LIST1
			rxlevel	A_RXLEVEL_22
			power	NOT_USED

- MPH_IDENTITY_REQ
			mid	S_MS_ID_NO_IMSI_NO_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	11.04.00	DG	value changed: MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_6
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	31.05.00	DG	value changed:
				MPH_NEIGHBOURCELL_REQ:
				MPH_NCELL_1_LIM > MPH_NCELL_1
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	30.01.01	DG	MPH_BSIC_REQ/CNF(43)removed,
				ARFCN_124 now succesful
				values changed:
				MPH_IDLE_REQ: 
				DLT_LIMITED -> DLT_23
				BS_PA_MFRMS_7 -> BS_PA_MFRMS_2
				NCC_PERMITTED_FF -> NCC_PERMITTED_1
	08.02.01	DG	si3/4_rest_oct:
				SI3_REST_DEF > SI3_REST_EMPTY
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2

25.02.03	LG	'lac_list' added in rr_abort_ind

### RR072: Message Cell Broadcast

Description:	The SYS INFO messages have reported a suitable BCCH carrier. RR places PL in IDLE mode by issuing a a MPH-IDLE request primitive and informs MM in the form of a RR-ACTIVATE confirmation primitive. The cell contains Message Cell Broadcast.

Preamble:	RR012F

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(6)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(7)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	PRR_CHANNEL_TYPE_3_CB

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04. 07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

### RR080: Cell Selection for Dualband

Description:	A cell selection for dualband is started.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(14)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(15)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS_DUALBAND

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	11.09.98	LE	Initial
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
				new: RR_SYNC_IND
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
				si3/4_rest_oct:
				SI3_REST_DEF > SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR045: No SIM card, suitable if using C1 Offset

Description:	It is no SIM card inserted. The first channel is selected, because RR adds an offset of 4 (C1 offset) to the calculated C1 value.

Preamble:	RR001A

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(12)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(13)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_2
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_LIMITED
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_2
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_EMPTY
		mm_info	MM_INFO_1
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_NO_IMSI_NO_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
	23.05.00	DG	values changed:
				MPH_UNITDATA/sysinfo 2:
				NCELL_DESC_1 > NCELL_DESC_2
				MPH_UNITDATA/sysinfo 3, 4:
				CELL_SELECT_3 > CELL_SELECT_1
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	31.05.00	DG	value changed:
				MPH_NEIGHBOURCELL_REQ:
				MPH_NCELL_1_LIM > MPH_NCELL_2
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
				si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR046: With normal SIM Card, suitable if using C1 Offset

Description:	It is a normal SIM card inserted. The first channel is selected, because RR adds an offset of 4 (C1 offset) to the calculated C1 value.

Preamble:	RR001C

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(12)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(13)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_2
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_2
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_NORMAL
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_NO_TMSI

History:	04. 07.97	DL	Initial
	15.05.98	VK	MPH_CBCH_REQ
	06.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5:
	23.05.00	DG	values changed:
				MPH_UNITDATA/sysinfo 2:
				NCELL_DESC_1 > NCELL_DESC_2
				MPH_UNITDATA/sysinfo 3, 4:
				CELL_SELECT_3 > CELL_SELECT_1
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
				si3/4_rest_oct: SI3_REST_DEF > SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR047: With Test SIM Card, unsuitable although using C1 Offset

Description:	It is a test SIM card inserted. The first channel is not selected, because RR adds no offset of 4 (C1 offset) to the calculated C1 value.

Preamble:	RR001D

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_3
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_3
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04. 07.97	DL	Initial
	15.05.98	VK	MPH_CBCH_REQ
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF > 
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR048: Cell Selection E-GSM

Description:

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(14)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(15)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

Primitive	Parameter	Value

- RR_ACTIVATE_REQ
	plmn	PLMN_ID_123
	op	OP_MODE_TEST_SIM
	cksn	CKSN_NOT_PRES
	kcv	KCV_12345678
	accc	ACC_CTRL_CLASS_0008
	imsi_struct	MOBILE_ID_IMSI_HPLMN
	tmsi_struct	MOBILE_ID_TMSI
	thplmn	TIME_HPLMN_VALID
	bcch_info	S_BCCH_INFO_EMPTY
	cell_test	CELL_TEST_DISABLE
	gprs_indication	NOT_USED

- MPH_POWER_REQ
	pch_interrupt	PCH_INTERRUPT
	freq_bands	NOT_USED

- MPH_POWER_CNF
	num_of_chan	CHANNELS_3
	arfcn	ARFCN_67_32_124
	rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
	arfcn	ARFCN_67

- MPH_BSIC_CNF
	arfcn	ARFCN_67
	bsic	BSIC_5
	cs	CS_NO_ERROR

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_1
	ti	TI_0
	cell_chan_desc	CELL_CHAN_DESC_1
	rach_ctrl	RACH_CTRL_1
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_2
	ti	TI_0
	neigh_cell_desc	NCELL_DESC_2
	ncc_permit	NCC_PERMITTED_1
	rach_ctrl	RACH_CTRL_1
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu	SYS_INFO_3_A

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_4
	ti	TI_0
	loc_area_ident	LOC_AREA_IDENT_123_2147
	cell_select	CELL_SELECT_1
	rach_ctrl	RACH_CTRL_1
	chan_desc	NOT_USED
	mob_alloc	NOT_USED
	si4_rest_oct	S_SI4_REST_EMPTY
	}

- MPH_CLASSMARK_REQ
	classmark	CLASS_MS

- MPH_IDLE_REQ
	mod	MODE_CELL_SELECTION
	arfcn	ARFCN_67
	ext_bcch	NOT_USED
	comb_ccch	CCD_CCCH_1_NOT_COMB
	tn	TN_0
	dlt	DLT_23
	pg	PG_0
	bs_ag_blocks_res	BS_AG_BLKS_RES_5
	bs_pa_mfrms	BS_PA_MFRMS_2
	power	MS_TXPWR_MAX_CCH_02
	ncc_permitted	NCC_PERMITTED_1
	reorg_only	NOT_USED
	eotd_avail	CONST_0
	gprs_support	NOT_USED

- MPH_CBCH_REQ
	cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_2
			sync_only	NOT_USED

- RR_ACTIVATE_CNF
	op	OP_MODE_TEST_SIM
	mm_info	MM_INFO_2
	cid	CELL_IDENT_3748
	plmn	PLMN_ID_123
	lac	LAC_2147
	power	NOT_USED
	gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
	mid	S_MS_ID_IMSI_HPLMN_TMSI

History:		27-Jun-02		MPA		Initial

### RR049: Cell Selection for Testcase 13.1

Description:

Preamble:	RR000

Variants:	<A>....<B>

MM		 RR		PL/DL
COMMAND (PL CONFIG STD=6)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(14)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(15)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_TEST
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_TEST
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu	SYS_INFO_1_15

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu	SYS_INFO_2_15

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
<A>		sdu	SYS_INFO_3_15
<B>		sdu	SYS_INFO_4_15

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
<A>		sdu	SYS_INFO_4_15
<B>		sdu	SYS_INFO_3_15

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_COMB
		tn	TN_0
		dlt	DLT_18
		pg	PG_12
		bs_ag_blocks_res	BS_AG_BLKS_RES_0
		bs_pa_mfrms	BS_PA_MFRMS_3
		power	MS_TXPWR_MAX_CCH_0D
		ncc_permitted	NCC_PERMITTED_2
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_EMPTY
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_15
		cid	CELL_IDENT_0001
		plmn	PLMN_ID_TEST
		lac	LAC_0001
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_TEST_TMSI

History:	15-Feb-00	LE	Initial
	14-Apr-00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	15-May-00	DG	RR_ACTIVATE_REQ: cell_test added
	17-May-00	DG	new. RR_SYNC_IND
	12-Jul-00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19-Jan-01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	05-Jun-01	MSE	adapted to TAP2

## Idle Mode Neighbour Cell Procedures

### RR132: Listening to Neighbour Cell

Description:	Layer 1 sends a measurement report with a neighbour cell. RR starts listening to the neighbour cell. First the channel 3 is read. After reading of system information type 3 message on report is waited and then channel 18 is requested.

Preamble:	RR030

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND

- arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- MPH_MEASUREMENT_IND

- arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_3
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_24
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

History:	04.07.97	DL	Initial
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	08.02.01	DG	si3_rest_oct: SI3_REST_DEF > 
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR133: Listening to Neighbour Cell and Network is lost, Limited service only possible

Description:	Layer 1 sends a measurement report with a neighbour cell. RR starts listening to measurement reports. There is a downlink signalling error. RR has not read SI 3 from the neighbour cell. RR starts a cell selection for a suitable cell to camp ON. Just when the cell selection is started an emergency call is started. RR should store this primitive. At the end of Power search, RR does not find a suitable cell to camp on with full service. Only limited service is possible. RR should then try an emergency call with limited service.

Preamble:	RR030

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(6)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(18)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(20)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(21)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(22)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(23)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(24)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(25)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(26)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(27)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(28)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(29)	|	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(30)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
(31)	 |		  |	MPH_RANDOM_ACCESS_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_67

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- RR_ESTABLISH_REQ
		estcs	ESTCS_EMRG_CAL
		sdu	MM_MESSAGE

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_2
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY 
	}

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	NOT_USED
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM_LIM_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	AS_PLMN
		lac_list	A_LAC_LIST1
		rxlevel	A_RXLEVEL_21
		power	NOT_USED

- MPH_IDENTITY_REQ
		mid	S_MS_ID_NO_IMSI_NO_TMSI

- MPH_RANDOM_ACCESS_REQ
		send_mode	NOT_USED

History:	21.06-2003	RB	Initial

## Immediate Assignment

### RR136: RR-Connection Establishment by the MS

Description:	MM starts a connection establishment and RR sends two random bursts.

Preamble:	RR030

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_RANDOM_ACCESS_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu	MM_MESSAGE

- MPH_RANDOM_ACCESS_REQ
		send_mode	SEND_MODE_2_BURSTS

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

History:	04.07.97	DL	Initial

### RR153: Paging for MS (IMSI)

Description:	The base station sends a Paging Request Type 1 message with the IMSI of the mobile station. RR sends two random bursts.

Preamble:	RR030

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_RANDOM_ACCESS_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_35
		rx_lev_sub	RX_LEV_35
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

- MPH_RANDOM_ACCESS_REQ
		send_mode	SEND_MODE_2_BURSTS2

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

History:	04.07.97	DL	Initial
	05.05.98	VK	Revised (one prim removed)
	24.01.01	DG	new: MPH_MEASUREMENT_IND

### RR154: Imm Ass for the MS, T3122 is not running

Description:	The base station sends an Immediate Assignment message for the MS. RR informs the PL and tries to establish the layer 2 connection.

<A>:	no starting time, no rest octets, no hopping, SDCCH
<B>:	no starting time, no rest octets, with hopping, SDCCH
<C>:	no starting time, no rest octets, no hopping, TCH
<D>:	with starting time, no rest octets, no hopping, SDCCH
<E>:	with starting time, with rest octets, no hopping, SDCCH
<F>:	with starting time, with rest octets, with hopping, SDCCH
<G>: 	no starting time, no rest octets, no hopping, SDCCH, 1800 / part of a suite
<H>: 	no starting time, no rest octets, no hopping, SDCCH, 1900 / part of a suite

Variants:	<A>..<H>

Preamble:	<A>RR153

<B>RR153

<C>RR153

<D>RR153

<E>RR153

<F>RR153

<G>none

<H>none

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	NOT_USED
		dl	NOT_USED
		d_t	NOT_USED
		page_mode	PAGING_NORMAL
<A>	chan_desc	CHANNEL_DESC_SDCCH2
<B>	chan_desc	CHANNEL_DESC_SDCCH
<C>	chan_desc	CHANNEL_DESC_FACCH2
<D>	chan_desc	CHANNEL_DESC_SDCCH2
<E>	chan_desc	CHANNEL_DESC_SDCCH2
<F>	chan_desc	CHANNEL_DESC_SDCCH
<G>	chan_desc	CHANNEL_DESC_SDCCH2_1800_1
<H>	chan_desc	CHANNEL_DESC_SDCCH2_1900_1
		pck_chan_desc	NOT_USED
		req_ref	REQUEST_REFERENCE_1
		time_advance	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_1
<A>	start_time	NOT_USED
<B>	start_time	NOT_USED
<C>	start_time	NOT_USED
<D>	start_time	START_TIME_1
<E>	start_time	START_TIME_1
<F>	start_time	START_TIME_1
<G>	start_time	NOT_USED
<H>	start_time	NOT_USED
<A>	ia_rest_oct	NOT_USED
<B>	ia_rest_oct	NOT_USED
<C>	ia_rest_oct	NOT_USED
<D>	ia_rest_oct	NOT_USED
<E>	ia_rest_oct	S_IA_REST_1
<F>	ia_rest_oct	S_IA_REST_1
<G>	ia_rest_oct	NOT_USED
<H>	ia_rest_oct	NOT_USED
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
<A>	start	NO_STARTING_TIME
<B>	start	NO_STARTING_TIME
<C>	start	NO_STARTING_TIME
<D>	start	STARTING_TIME_1
<E>	start	STARTING_TIME_1
<F>	start	STARTING_TIME_1
<G>	start	NO_STARTING_TIME
<H>	start	NO_STARTING_TIME
<A>	ch_type	PRR_CHANNEL_TYPE_3
<B>	ch_type	PRR_CHANNEL_TYPE_2
<C>	ch_type	PRR_CHANNEL_TYPE_4
<D>	ch_type	PRR_CHANNEL_TYPE_3
<E>	ch_type	PRR_CHANNEL_TYPE_3
<F>	ch_type	PRR_CHANNEL_TYPE_2
<G>	ch_type	PRR_CHANNEL_TYPE_3_1800
<H>	ch_type	PRR_CHANNEL_TYPE_3_1900
<A>	ch_type2	NOT_USED
<B>	ch_type2	NOT_USED
<C>	ch_type2	NOT_USED
<D>	ch_type2	NOT_USED
<E>	ch_type2	NOT_USED
<F>	ch_type2	NOT_USED
<G>	ch_type2	NOT_USED
<H>	ch_type2	NOT_USED 
<A>	arfcn	ARFCN_67
<B>	arfcn	ARFCN_67
<C>	arfcn	ARFCN_67
<D>	arfcn	ARFCN_67
<E>	arfcn	ARFCN_67
<F>	arfcn	ARFCN_67
<G>	arfcn	ARFCN_527
<H>	arfcn	ARFCN_527
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_2
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	NOT_USED

- DL_ESTABLISH_REQ
<A>	ch_type	CH_TYPE_SDCCH
<B>	ch_type	CH_TYPE_SDCCH
<C>	ch_type	CH_TYPE_FACCH
<D>	ch_type	CH_TYPE_SDCCH
<E>	ch_type	CH_TYPE_SDCCH
<F>	ch_type	CH_TYPE_SDCCH
<G>	ch_type	CH_TYPE_SDCCH
<H>	ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
<A> 	mob_class_2	MOB_CLASS2_900
<B> 	mob_class_2	MOB_CLASS2_900
<C> 	mob_class_2	MOB_CLASS2_900
<D> 	mob_class_2	MOB_CLASS2_900
<E> 	mob_class_2	MOB_CLASS2_900
<F>	mob_class_2	MOB_CLASS2_900
<G>	mob_class_2	MOB_CLASS_1800
<H>	mob_class_2	MOB_CLASS_1900
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

History:	04.07.97	DL	Initial
	13.04.00	LE (DG)	Value changed:
				DL_ESTABLISH_REQ/mob_class_2:
				MOB_CLASS2_NORMAL >
				MOB_CLASS2_900
	25.01.01	DG	IMMEDIATE ASSIGNMENT:
				tma, dl, d_t, pck_chan_desc added
	12.02.01	DG	new: MPH_SYNC_REQ
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation
	28-Feb02	LG	added preambles by variants

### RR152: Imm Ass for the MS, spare bits unequal 0

Description:	According FTA 26.5.7.1.3

Preamble:	RR153

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu	IMM_ASS_152_SPARE_900

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_3
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_2
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	NOT_USED

- DL_ESTABLISH_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	MOB_CLASS2_900
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

History:	20.01.2003	VK	Derived from RR154A

### RR155: Imm Ass Extended for the MS, T3122 is not running

Description:	The base station sends an Immediate Assignment message for the MS. RR informs the PL and tries to establish the layer 2 connection.
The MS is addressed in the 2nd part of the message, the first part contains an invalid channel description (channel 1)

Preamble:	RR153

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT EXT) |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN_EXT
		ti	TI_0
		page_mode	PAGING_NORMAL
		chan_desc	CHANNEL_DESC_SDCCH2_1800
		req_ref	REQUEST_REFERENCE_2
		time_advance	TIMING_ADVANCE_27
		chan_desc_2	CHANNEL_DESC_SDCCH2
		req_ref_2	REQUEST_REFERENCE_1
		time_advance_2	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_1
		start_time	NOT_USED
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_3
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_2
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	NOT_USED

- DL_ESTABLISH_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	MOB_CLASS2_900
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

History:	14.01.03	MPA	Initial

### RR156: Imm Ass Reject for the MS, T3122 is not running

Description:	The base station rejects the call establishment. RR stops sending of random bursts for the requested time. After waiting for an immediate assignment message longer than T3122 the immediate assignment procedure is stopped. When the waiting time of the immediate assignment reject message is over this is indicated to MM.

Preamble:	RR136

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGN REJ)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_RANDOM_ACCESS_REQ	  |
	 |		  *==============================>*
MUTE (2000)
TIMEOUT (500)
(3)	 |	RR_RELEASE_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
MUTE (7000)
TIMEOUT (500)
(5)	 |	RR_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN_REJ
		ti	TI_0
		page_mode	PAGING_NORMAL
		req_ref	REQUEST_REFERENCE_2
		t3122	WAIT_INDICATION_0A
		req_ref_2	REQUEST_REFERENCE_2
		t3122_2	WAIT_INDICATION_0A
		req_ref_3	REQUEST_REFERENCE_3
		t3122_3	WAIT_INDICATION_0A
		req_ref_4	REQUEST_REFERENCE_1
		t3122_4	WAIT_INDICATION_0A
		}

- MPH_RANDOM_ACCESS_REQ
		send_mode	SEND_MODE_NO_BURSTS

- RR_RELEASE_IND
		cause	RRCS_RND_ACC_DELAY
		sapi	SAPI_0
		gprs_resumption	NOT_USED

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- RR_SYNC_IND
		ciph	NOT_PRESENT_8BIT
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	SYNCCS_T3122_TIM_OUT
		chm	NOT_USED

History:	04.07.97	DL	Initial
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	25.01.01	DG	RR_RELEASE_IND: gprs_resumption added
	06.03.03	LG	MUTE/TIMEOUT inserted,

### RR157: Immediate Assignment E-GSM

Description:

Preamble:	RR048

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_RANDOM_ACCESS_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(10)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_35
	rx_lev_sub	RX_LEV_35
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	NOT_USED
	gprs_sync	NOT_USED

- MPH_PAGING_IND
	identity_type	ID_IMSI
	channel_needed	CN_ANY

- MPH_RANDOM_ACCESS_REQ
	send_mode	SEND_MODE_2_BURSTS2

- MPH_RANDOM_ACCESS_CNF
	frame_no	FRAME_NUMBER_1

- MPH_RANDOM_ACCESS_CNF
	frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_IMM_ASSIGN
	ti	TI_0
	tma	NOT_USED
	dl	NOT_USED
	d_t	NOT_USED
	page_mode	PAGING_NORMAL
	chan_desc	CHANNEL_DESC_SDCCH
	pck_chan_desc	NOT_USED
	req_ref	REQUEST_REFERENCE_1
	time_advance	TIMING_ADVANCE_27
	mob_alloc	MOBILE_ALLOCATION_1
	start_time	NOT_USED
	ia_rest_oct	NOT_USED
	}

- MPH_DEDICATED_REQ
	mod	MODE_IMM_ASSIGN
	start	NO_STARTING_TIME
	ch_type	PRR_CHANNEL_TYPE_2
	ch_type2	NOT_USED
	arfcn	ARFCN_67
	bsic	NOT_USED
	ho_param	NOT_USED
	tr_para	PRR_TR_PARA_2
	ciph	NO_CIPHERING
	amr_conf	NOT_USED

- MPH_DEDICATED_CNF
	dedi_res	NOT_USED

- DL_ESTABLISH_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_PAG_RES
	ti	TI_0
	ciph_key_num	CKSN_RESERVED
	mob_class_2	MOB_CLASS2_212
	mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
	}

- DL_ESTABLISH_CNF
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0

- MPH_SYNC_REQ
	cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
	param	NOT_USED

History:		27-Jun-02		MPA		Initial

### RR158: Immediate Assignment TC 13.1

Description:

Preamble:	RR049A

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_RANDOM_ACCESS_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(10)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_35
		rx_lev_sub	RX_LEV_35
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

- MPH_RANDOM_ACCESS_REQ
		send_mode	SEND_MODE_8_BURSTS

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	NOT_USED
		dl	NOT_USED
		d_t	NOT_USED
		page_mode	PAGING_NORMAL
		chan_desc	CHANNEL_DESC_SDCCH
		pck_chan_desc	NOT_USED
		req_ref	REQUEST_REFERENCE_1
		time_advance	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_ONE_CHAN 
		start_time	NOT_USED
		ia_rest_oct	NOT_USED
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	NOT_USED
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	NOT_USED
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	NOT_USED

- DL_ESTABLISH_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	MOB_CLASS2_212
		mob_ident	MOBILE_IDENTITY_IMSI_TEST
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

History:		27-Jun-02		MPA		Initial

### RR159: Immediate Assignment (Multiband)

Description:

Preamble:	RR153

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	NOT_USED
		dl	NOT_USED
		d_t	NOT_USED
		page_mode	PAGING_NORMAL
		chan_desc	CHANNEL_DESC_SDCCH
		pck_chan_desc	NOT_USED
		req_ref	REQUEST_REFERENCE_1
		time_advance	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_1
		start_time	NOT_USED
		ia_rest_oct	NOT_USED
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_2
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_2
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
			dedi_res	NOT_USED

- DL_ESTABLISH_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	NOT_USED
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

History:		t-Jun-jj		NN		Initial

### RR160: T3126 Timeout, T3122 is not running, Mobile Originated Call

Description:	The base station sends no immediate assignment. RR informs MM.

Preamble:	RR136

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
MUTE (2000)
TIMEOUT (500)
(2)	 |	RR_RELEASE_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- RR_RELEASE_IND
		cause	RRCS_RND_ACC_FAIL
		sapi	SAPI_0
		gprs_resumption	NOT_USED

- MPH_POWER_REQ
 		pch_interrupt	PCH_INTERRUPT
 		freq_bands	NOT_USED

History:	04.07.97	DL	Initial
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	25.01.01	DG	RR_RELEASE_IND: gprs_resumption added
	06.03.03	LG	MUTE/TIMEOUT inserted

### RR161: DLF indiciation crosses connection establishment

Description:	MM starts a connection establishment. At the same time a downlink failure occurs.

Preamble:	RR030

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_RANDOM_ACCESS_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(4)	 |	RR_RELEASE_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_REQ
		estcs	ESTCS_SERV_REQ_BY_MM
		sdu	MM_MESSAGE

- MPH_RANDOM_ACCESS_REQ
		send_mode	SEND_MODE_2_BURSTS

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_67

- RR_RELEASE_IND
		cause	RRCS_ABNORM_UNSPEC
		sapi	SAPI_0
		gprs_resumption	NOT_USED

History:	04.10.02	MPA	Initial

### RR162: Imm Ass Extended for the MS fails

Description:	The base station sends an Immediate Assignment message for the MS. RR informs the PL and tries to establish the layer 2 connection.
The MS is addressed in the 2nd part of the message, but the message is discarded, because there is found an conditional IE error

Preamble:	RR153

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT EXT) |
	 |		  *<==============================*
MUTE (2000)
TIMEOUT (500)
(3)	 |	RR_RELEASE_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN_EXT
		ti	TI_0
		page_mode	PAGING_NORMAL
		chan_desc	CHANNEL_DESC_SDCCH2_1800
		req_ref	REQUEST_REFERENCE_2
		time_advance	TIMING_ADVANCE_27
		chan_desc_2	CHANNEL_DESC_SDCCH2_1800
		req_ref_2	REQUEST_REFERENCE_1
		time_advance_2	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_1
		start_time	NOT_USED
		}

- RR_RELEASE_IND
	cause	RRCS_INT_NOT_PRESENT
	sapi	SAPI_0
	gprs_resumption	NOT_USED

- MPH_POWER_REQ
 		pch_interrupt	PCH_INTERRUPT
 		freq_bands	NOT_USED

History:	14.01.03	MPA	Initial
	06.03.03	LG	MUTE/TIMEOUT inserted

### RR400: Paging for MS, TMSI

Description:	The base station sends a Paging Request Type 1 message with the TMSI of the mobile station. RR sends two random bursts.

Preamble:	RR030

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_RANDOM_ACCESS_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_35
		rx_lev_sub	RX_LEV_35
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPH_PAGING_IND
		identity_type	ID_TMSI
		channel_needed	CN_ANY

- MPH_RANDOM_ACCESS_REQ
		send_mode	SEND_MODE_2_BURSTS2

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

History:	04.07.97	DL	Initial
	24.01.01	DG	new: MPH_MEASUREMENT_IND

### RR403: Imm Ass Extended for the MS

Description:	The base station sends an Immediate Assignment Extended message for the MS. RR informs the PL and tries to establish the layer 2 connection.

<A>:	no starting time, channel description 1 (SDCCH)
<B>:	no starting time, channel description 2 (SDCCH)
<C>:	with starting time, channel description 1 (FACCH)
<D>:	with starting time, channel description 2 (FACCH)

Preamble:	RR153

Variants:	<A>..<D>

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT EXT)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN_EXT
		ti	TI_0
		page_mode	PAGING_NORMAL
<A>	chan_desc	CHANNEL_DESC_SDCCH
<B>	chan_desc	CHANNEL_DESC_FACCH2
<C>	chan_desc	CHANNEL_DESC_FACCH2
<D>	chan_desc	CHANNEL_DESC_SDCCH
<A>	req_ref	REQUEST_REFERENCE_1
<B>	req_ref	REQUEST_REFERENCE_2
<C>	req_ref	REQUEST_REFERENCE_1
<D>	req_ref	REQUEST_REFERENCE_2
		time_advance	TIMING_ADVANCE_27
<A>	chan_desc_2	CHANNEL_DESC_FACCH2
<B>	chan_desc_2	CHANNEL_DESC_SDCCH
<C>	chan_desc_2	CHANNEL_DESC_SDCCH
<D>	chan_desc_2	CHANNEL_DESC_FACCH2
<A>	req_ref_2	REQUEST_REFERENCE_2
<B>	req_ref_2	REQUEST_REFERENCE_1
<C>	req_ref_2	REQUEST_REFERENCE_2
<D>	req_ref_2	REQUEST_REFERENCE_1
		time_advance_2	TIMING_ADVANCE_10
		mob_alloc	MOBILE_ALLOCATION_1
<A>	start_time	NOT_USED
<B>	start_time	NOT_USED
<C>	start_time	START_TIME_1
<D>	start_time	START_TIME_1
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
<A>	start	NO_STARTING_TIME
<B>	start	NO_STARTING_TIME
<C>	start	STARTING_TIME_1
<D>	start	STARTING_TIME_1
<A>	ch_type	PRR_CHANNEL_TYPE_2
<B>	ch_type	PRR_CHANNEL_TYPE_2
<C>	ch_type	PRR_CHANNEL_TYPE_4
<D>	ch_type	PRR_CHANNEL_TYPE_4
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
<A>	tr_para	PRR_TR_PARA_2
<B>	tr_para	PRR_TR_PARA_2B
<C>	tr_para	PRR_TR_PARA_2
<D>	tr_para	PRR_TR_PARA_2B
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	NOT_USED

- DL_ESTABLISH_REQ

- <A>	ch_type	CH_TYPE_SDCCH
<B>	ch_type	CH_TYPE_SDCCH
<C>	ch_type	CH_TYPE_FACCH
<D>	ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	MOB_CLASS2_900
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

History:	30.10.97	LE	Initial
	29.01.01	DG	DL_ESTABLISH_REQ:
				MOB_CLASS2_NORMAL changed to
				MOB_CLASS2_900
	26-Feb-02	OT	Adaptations for AMR implementation

### RR404: Indicate Fieldstrength Jump

Description:	If the fieldstrength of the serving cell is more or equal 10 dBm than during the last location updating request, an indication is forwarded to MM.

Preamble:	RR156

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |	RR_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_35
		rx_lev_sub	RX_LEV_35
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	NOT_USED
		gprs_sync	NOT_USED

- RR_SYNC_IND
	ciph	NOT_PRESENT_8BIT
	mm_info	NOT_USED
	bcch_info	NOT_USED
	synccs	SYNCCS_LUP_RETRY
	chm	NOT_USED

History:	20.01.99	LE	Initial

### RR405: Paging for MS (IMSI) after connection

Description:	The base station sends a Paging Request Type 1 message with the IMSI of the mobile station. RR sends two random bursts. The previous connection has a layer 2 release without acknowledgement.

Preamble:	RR227

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_RANDOM_ACCESS_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

- MPH_RANDOM_ACCESS_REQ
		send_mode	SEND_MODE_2_BURSTS2

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

History:	19.04.99	LE	Initial
	06.03.00	DG	RR_SYNC_IND
	18.04.00	DG	values changed:
				RR_SYNC_IND:
				NOT_USED > MM_INFO_2
				NOT_PRESENT_16BIT >
				SYNCCS_BACK_FROM_DEDICATED
				NOT_USED > CHM_DG_629
	05-Jun-01	MSE	adapted to TAP2

## Channel Assignment

### RR204: start of intracell handover without change in channel mode

Description:	The base station initiates an intracell handover. RR  releases the old connection locally and configures the new channel configuration. There is no change of channel mode.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(ASSIGNMENT COMPLETE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_ASSIGN_CMD
		ti	TI_0
		chan_desc	CHANNEL_DESC_SDCCH2
		pow_cmd	POWER_COMMAND_05
		freq_list_after	NOT_USED
		cell_chan_desc	NOT_USED
		multislot_alloc	NOT_USED
		chan_mode	NOT_USED
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		mob_alloc_after	NOT_USED
		start_time	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	NOT_USED
		chan_desc_before_2	NOT_USED
		freq_chan_seq	NOT_USED
		mob_alloc_before	NOT_USED
		ciph_mode_set	NOT_USED
		vgcs_tmi	NOT_USED
		multirate_conf	S_multirate_conf_1
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_DEDICATED_REQ
		mod	MODE_CHAN_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_3
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_3
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_ASSIGN_COMP
		ti	TI_0
		rr_cause	RR_CAUSE_0
		}

History:	04.07.97	DL	Initial
	18.11.97	LE	adapted to layer 1
	06.05.98	VK	DL_RELEASE_CNF removed
	25.01.01	DG	DL_DATA_REQ added,
				DL_DATA_IND: multislot_alloc,
				chan_mode2…8 added
	12.02.01	DG	DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR209: start of intracell handover with change of channel mode

Description:	The base station initiates an intracell handover. RR  releases the old connection locally and configures the new channel configuration. There is a change of channel mode.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(ASSIGNMENT COMPLETE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_ASSIGN_CMD
		ti	TI_0
		chan_desc	CHANNEL_DESC_FACCH2
		pow_cmd	POWER_COMMAND_05
		freq_list_after	NOT_USED
		cell_chan_desc	NOT_USED
		multislot_alloc	NOT_USED
		chan_mode	CHANNEL_MODE_SPEECH
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		mob_alloc_after	NOT_USED
		start_time	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	NOT_USED
		chan_desc_before_2	NOT_USED
		freq_chan_seq	NOT_USED
		mob_alloc_before	NOT_USED
		ciph_mode_set	NOT_USED
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_DEDICATED_REQ
		mod	MODE_CHAN_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_4
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_13
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
	dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_ASSIGN_COMP
		ti	TI_0
		rr_cause	RR_CAUSE_0
		}

History:	04.07.97	DL	Initial
	06.05.98	VK	DL_RELEASE_CNF removed
	25.01.01	DG	D_ASSIGN_CMD: multislot_alloc,
				chan_mode2…8 added
	26-Feb-02	OT	Adaptations for AMR implementation

### RR700: start of intracell handover with change of channel mode-AMR

Description:	The base station initiates an intracell handover. RR  releases the old connection locally and configures the new channel configuration. There is a change of channel mode. Requires a channel mode pcm entry with speech version 3.

<A>:	one codec mode, w/o initial codec mode, full rate
<B>:	two codec modes, w/o initial codec mode
<C>:	three codec modes, w/o initial codec mode
<D>:	four codec modes, w/o initial codec mode
<E>:	four codec modes, with initial codec mode
<F>:	one codec mode, w/o initial codec mode, half rate
<G>:	full rate ericsson IOT
<H>:	full rate alcatel IOT
<I>: 	half rate ericsson IOT
<J>: 	half rate alcatel IOT

Variants:	<A>....<J>

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
COMMAND (RR CONFIG PCM=<MSCAP,0x33,0,0,0,0,0>)
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(ASSIGNMENT COMPLETE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_ASSIGN_CMD
		ti	TI_0
<A>	chan_desc	CHANNEL_DESC_FACCH2
<B>	chan_desc	CHANNEL_DESC_FACCH2
<C>	chan_desc	CHANNEL_DESC_FACCH2
<D>	chan_desc	CHANNEL_DESC_FACCH2
<E>	chan_desc	CHANNEL_DESC_FACCH2
<F>	chan_desc	CHANNEL_DESC_HALFRATE
<G>	chan_desc	CHANNEL_DESC_FACCH2
<H>	chan_desc	CHANNEL_DESC_FACCH2
<I>		chan_desc	CHANNEL_DESC_HALFRATE
<J>		chan_desc	CHANNEL_DESC_HALFRATE
		pow_cmd	POWER_COMMAND_05
		freq_list_after	NOT_USED
		cell_chan_desc	NOT_USED
		multislot_alloc	NOT_USED
		chan_mode	CHANNEL_MODE_AMR
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		mob_alloc_after	NOT_USED
		start_time	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	NOT_USED
		chan_desc_before_2	NOT_USED
		freq_chan_seq	NOT_USED
		mob_alloc_before	NOT_USED
		ciph_mode_set	NOT_USED
		vgcs_tmi	NOT_USED
<A>	multirate_conf	S_multirate_conf_1
<B>	multirate_conf	S_multirate_conf_2
<C>	multirate_conf	S_multirate_conf_3
<D>	multirate_conf	S_multirate_conf_4
<E>	multirate_conf	S_multirate_conf_4_ICMI
<F>	multirate_conf	S_multirate_conf_1
<G>	multirate_conf	S_MULTIRATE_CONF_ER_AFS
<H>	multirate_conf	S_MULTIRATE_CONF_ALC_AFS
<I> 		multirate_conf	S_MULTIRATE_CONF_ER_AHS
<J>		multirate_conf	S_MULTIRATE_CONF_ALC_AHS
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_DEDICATED_REQ
		mod	MODE_CHAN_ASSIGN
		start	NO_STARTING_TIME
<A>	ch_type	PRR_CHANNEL_TYPE_4
<B>	ch_type	PRR_CHANNEL_TYPE_4
<C>	ch_type	PRR_CHANNEL_TYPE_4
<D>	ch_type	PRR_CHANNEL_TYPE_4
<E>	ch_type	PRR_CHANNEL_TYPE_4
<F>	ch_type	PRR_CHANNEL_TYPE_HALF
<G>	ch_type	PRR_CHANNEL_TYPE_4
<H>	ch_type	PRR_CHANNEL_TYPE_4
<I>		ch_type	PRR_CHANNEL_TYPE_HALF
<J> 	ch_type	PRR_CHANNEL_TYPE_HALF
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_AMR
		ciph	NO_CIPHERING
<A>	amr_conf	S_AMR_conf_1
<B>	amr_conf	S_AMR_conf_2
<C>	amr_conf	S_AMR_conf_3
<D>	amr_conf	S_AMR_conf_4
<E>	amr_conf	S_AMR_conf_4_ICMI
<F>	amr_conf	S_AMR_conf_1
<G>	amr_conf	S_AMR_conf_5_ER_AFS
<H>	amr_conf	S_AMR_conf_6_ALC_AFS
<I>		amr_conf	S_AMR_conf_7_ER_AHS
<J>		amr_conf	S_AMR_conf_8_ALC_AHS

- MPH_DEDICATED_CNF
	dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
<A>	ch_type	CH_TYPE_FACCH
<B>	ch_type	CH_TYPE_FACCH
<C>	ch_type	CH_TYPE_FACCH
<D>	ch_type	CH_TYPE_FACCH
<E>	ch_type	CH_TYPE_FACCH
<F>	ch_type	CH_TYPE_FACCH_HR
<G>	ch_type	CH_TYPE_FACCH
<H>	ch_type	CH_TYPE_FACCH
<I>		ch_type	CH_TYPE_FACCH_HR
<J>		ch_type	CH_TYPE_FACCH_HR
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_ASSIGN_COMP
		ti	TI_0
		rr_cause	RR_CAUSE_0
		}

- DL_ESTABLISH_CNF
<A>	ch_type	CH_TYPE_FACCH
<B>	ch_type	CH_TYPE_FACCH
<C>	ch_type	CH_TYPE_FACCH
<D>	ch_type	CH_TYPE_FACCH
<E>	ch_type	CH_TYPE_FACCH
<F>	ch_type	CH_TYPE_FACCH_HR
|<G>		ch_type	CH_TYPE_FACCH
<H>	ch_type	CH_TYPE_FACCH
<I>		ch_type	CH_TYPE_FACCH_HR
<J>		ch_type	CH_TYPE_FACCH_HR
		sapi	SAPI_0

History:	18. Jan  02	OT	Initial, for AMR

### RR211: start of intracell handover using starting time

Description:	The base station initiates an intracell handover. RR releases the old connection locally and configures the new channel configuration. The Starting Time feature is used.
A : after time : Mobile Allocation		before : Mobile Allocation
B : after time : Mobile Allocation (new CA)	before : Mobile Allocation (new CA)
C : after time : Frequency List		before : Mobile Allocation
D : after time : Frequency List		before : Frequency List
E : after time : Frequency List		before : Frequency Channel Sequence
F: after time : Mobile Allocation		before: as after (used as part of a suite only)

Variants:	<A>....<F>

Preamble:	<A>RR154B
		<B>RR154B
		<C>RR154B
		<D>RR154B
		<E>RR154B
		<F>none

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(ASSIGNMENT COMPLETE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_ASSIGN_CMD
	ti	TI_0
	chan_desc	CHANNEL_DESC_FACCH3
	pow_cmd	POWER_COMMAND_05
<A>	freq_list_after	NOT_USED
<B>	freq_list_after	NOT_USED
<C>	freq_list_after	S_FREQ_LIST_AFTER_2
<D>	freq_list_after	S_FREQ_LIST_AFTER_2
<E>	freq_list_after	S_FREQ_LIST_AFTER_2
<F>	freq_list_after	NOT_USED
<A>	cell_chan_desc	NOT_USED
<B>	cell_chan_desc	CELL_CHAN_DESC_2
<C>	cell_chan_desc	NOT_USED
<D>	cell_chan_desc	NOT_USED
<E>	cell_chan_desc	NOT_USED
<F>	cell_chan_desc	NOT_USED
	multislot_alloc	NOT_USED
	chan_mode	NOT_USED
	chan_mode2	NOT_USED
	chan_mode3	NOT_USED
	chan_mode4	NOT_USED
	chan_mode5	NOT_USED
	chan_mode6	NOT_USED
	chan_mode7	NOT_USED
	chan_mode8	NOT_USED
	chan_desc_after_2	NOT_USED
	chan_mode_2	NOT_USED
<A>	mob_alloc_after	MOBILE_ALLOCATION_2
<B>	mob_alloc_after	MOBILE_ALLOCATION_4
<C>	mob_alloc_after	NOT_USED
<D>	mob_alloc_after	NOT_USED
<E>	mob_alloc_after	NOT_USED
<F>	mob_alloc_after	MOBILE_ALLOCATION_1800
	start_time	START_TIME_1
<A>	freq_list_before	NOT_USED
<B>	freq_list_before	NOT_USED
<C>	freq_list_before	NOT_USED
<D>	freq_list_before	S_FREQ_LIST_BEFORE_1
<E>	freq_list_before	NOT_USED
<F>	freq_list_before	NOT_USED 
	chan_desc_before	CHANNEL_DESC_FACCH4
	chan_desc_before_2	NOT_USED
<A>	freq_chan_seq	NOT_USED
<B>	freq_chan_seq	NOT_USED
<C>	freq_chan_seq	NOT_USED
<D>	freq_chan_seq	NOT_USED
<E>	freq_chan_seq	FREQ_CHAN_SEQ_1
<F>	freq_chan_seq	NOT_USED
<A>	mob_alloc_before	MOBILE_ALLOCATION_3
<B>	mob_alloc_before	MOBILE_ALLOCATION_3
<C>	mob_alloc_before	MOBILE_ALLOCATION_3
<D>	mob_alloc_before	NOT_USED
<E>	mob_alloc_before	NOT_USED
<F>	mob_alloc_before	NOT_USED
	ciph_mode_set	NOT_USED
	}

- DL_SUSPEND_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0

- MPH_DEDICATED_REQ
	mod	MODE_CHAN_ASSIGN
	start	STARTING_TIME_1
<A>	ch_type	PRR_CHANNEL_TYPE_6
<B>	ch_type	PRR_CHANNEL_TYPE_8
<C>	ch_type	PRR_CHANNEL_TYPE_6
<D>	ch_type	PRR_CHANNEL_TYPE_6
<E>	ch_type	PRR_CHANNEL_TYPE_6
<F>	ch_type	PRR_CHANNEL_TYPE_6_1800
<A>	ch_type2	PRR_CHANNEL_TYPE_7
<B>	ch_type2	PRR_CHANNEL_TYPE_9
<C>	ch_type2	PRR_CHANNEL_TYPE_7
<D>	ch_type2	PRR_CHANNEL_TYPE_10
<E>	ch_type2	PRR_CHANNEL_TYPE_11
<F>	ch_type2	PRR_CHANNEL_TYPE_7_1800
<A>	arfcn	ARFCN_67
<B>	arfcn	ARFCN_67
<C>	arfcn	ARFCN_67
<D>	arfcn	ARFCN_67
<E>	arfcn	ARFCN_67
<F>	arfcn	ARFCN_527
	bsic	NOT_USED
	ho_param	NOT_USED
	tr_para	PRR_TR_PARA_3
	ciph	NO_CIPHERING
	amr_conf	NOT_USED

- MPH_DEDICATED_CNF
	dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
	ch_type	CH_TYPE_FACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_ASSIGN_COMP
	ti	TI_0
	rr_cause	RR_CAUSE_0
	}

History:	04.05.98	VK	Initial
	25.01.01	DG	D_ASSIGN_CMD: multislot_alloc,
				chan_mode2…8 added
	26.01.01	DG	DL_DATA_REQ added
	30.01.01	DG	value changed:
				MPH_DEDICATED_REQ:
				PRR_TR_PARA_13 -> PRR_TR_PARA_3
	12.02.01	DG	DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR212: start of intracell handover with E-GSM channel 0

Description:	The base station initiates an intracell handover. RR  releases the old connection locally and configures the new channel configuration. There is no change of channel mode. It is started in E-GSM band with channel number 0.

Preamble:	RR157

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(ASSIGNMENT COMPLETE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_ASSIGN_CMD
		ti	TI_0
		chan_desc	CHANNEL_DESC_FACCH3
		pow_cmd	POWER_COMMAND_05
		freq_list_after	NOT_USED
		cell_chan_desc	CELL_CHAN_DESC_EGSM
		multislot_alloc	NOT_USED
		chan_mode	NOT_USED
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		mob_alloc_after	MOBILE_ALLOCATION_EGSM
		start_time	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	NOT_USED
		chan_desc_before_2	NOT_USED
		freq_chan_seq	NOT_USED
		mob_alloc_before	NOT_USED
		ciph_mode_set	NOT_USED
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_DEDICATED_REQ
		mod	MODE_CHAN_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_EGSM
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_3
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_ASSIGN_COMP
		ti	TI_0
		rr_cause	RR_CAUSE_0
		}

History:	15-Feb-00	LE	Initial
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	17.05.00	DG	new: RR_SYNC_IND
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	25.01.01	DG	IMMEDIATE ASSIGNMENT:
				tma, dl, d_t, pck_chan_desc added;
				MPH_MEASUREMENT_IND added,
				D_ASSIGN_CMD: multislot_alloc,
				chan_mode2…8 added,
				DL_ESTABLISH_REQ: MOB_CLASS2_EGSM
				 -> MOB_CLASS2_212
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	26.02.01	DG	Preamble 001B replaced by local
				RR_ACTIVATE_REQ/MPH_POWER_REQ
	27.02.01	DG	new: MPH_SYNC_REQ
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR214: start of intracell handover for testcase 13.1

Description:	The base station initiates an intracell handover. RR releases the old connection locally and configures the new channel configuration. The parameters are from FTA testcase 13.1

Preamble:	RR158

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_RECONNECT_REQ	  |
	 |		  |	(ASSIGNMENT FAILURE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_ASSIGN_CMD
	ti	TI_0
	chan_desc	CHAN_DESC_13_1
	pow_cmd	POW_CMD_7
	freq_list_after	FREQ_LIST_AFTER_13_1
	cell_chan_desc	NOT_USED
	multislot_alloc	NOT_USED
	chan_mode	CM_SPEECH
	chan_mode2	NOT_USED
	chan_mode3	NOT_USED
	chan_mode4	NOT_USED
	chan_mode5	NOT_USED
	chan_mode6	NOT_USED
	chan_mode7	NOT_USED
	chan_mode8	NOT_USED
	chan_desc_after_2	NOT_USED
	chan_mode_2	NOT_USED
	mob_alloc_after	NOT_USED
	start_time	NOT_USED
	freq_list_before	NOT_USED
	chan_desc_before	NOT_USED
	chan_desc_before_2	NOT_USED
	freq_chan_seq	NOT_USED
	mob_alloc_before	NOT_USED
	ciph_mode_set	NOT_USED
	vgcs_tmi	NOT_USED
	multirate_conf	NOT_USED
	}

- DL_SUSPEND_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0

- DL_RECONNECT_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_ASSIGN_FAIL
	ti	TI_0
	rr_cause	RRC_CHANNEL_MODE
	}

History:	15-Feb-00	LE	Initial
	17-May-00	DG	new: DL_RECONNECT_REQ
				deleted:
				(15) MPH_DEDICATED_REQ,
				(16) MPH_DEDICATED_CNF
				(17) DL_RESUME_REQ
	25.01.01	DG	IMMEDIATE ASSIGNMENT:
				tma, dl, d_t, pck_chan_desc added;
				MPH_MEASUREMENT_IND added,
				DL_ESTABLISH_REQ: MOB_CLASS2_EGSM
				 -> MOB_CLASS2_212
	19.02.01	DG	new: MPH_SYNC_REQ
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR205: successful end of intracell handover, no change in channel mode

Description:	The new connection is established. There was no change in channel mode.

Preamble:	RR204

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

History:	04.07.97	DL	Initial

### RR210: successful end of intracell handover with change in channel mode

Description:	The new connection is established. There was a change in channel mode.

Variants:	<A>....<E>

Preamble:	<A>RR209

<B>RR700A

<C>RR700B

<D>RR700C

<E>RR700D

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(2)	 |	RR_SYNC_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0

- RR_SYNC_IND
		ciph	NOT_USED
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	NOT_PRESENT_16BIT
<A>	chm	S_CHM_SPEECH_FULL
<B>	chm	S_CHM_SPEECH_AMR_FULL
<C>	chm	S_CHM_SPEECH_AMR_FULL
<D>	chm	S_CHM_SPEECH_AMR_FULL
<E>	chm	S_CHM_SPEECH_AMR_FULL

History:	04.07.97	DL	Initial
	12.02.01	DG	new: MPH_SYNC_REQ
	05-Jun-01	MSE	adapted to TAP2
	18. Jan.02	OT	Variants <A>..<B> introduced, <B>..<E> for AMR.
	05.03.03	LG	MPH_SYNC_REQ removed

### RR213: unsuccessful intracell handover due to "frequency not implemented" (1800)

Description:	The base station initiates an intracell handover. RR releases the old connection locally and configures the new channel configuration(initial assignment). This test case is used only as part of a suite.

Preamble:	none

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_RECONNECT_REQ	  |
	 |		  |	(ASSIGNMENT FAILURE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_ASSIGN_CMD
	ti	TI_0
	chan_desc	CHANNEL_DESC_FACCH3
	pow_cmd	POWER_COMMAND_05
	freq_list_after	NOT_USED
	cell_chan_desc	NOT_USED
	multislot_alloc	NOT_USED
	chan_mode	NOT_USED
	chan_mode2	NOT_USED
	chan_mode3	NOT_USED
	chan_mode4	NOT_USED
	chan_mode5	NOT_USED
	chan_mode6	NOT_USED
	chan_mode7	NOT_USED
	chan_mode8	NOT_USED
	chan_desc_after_2	NOT_USED
	chan_mode_2	NOT_USED
	mob_alloc_after	MOBILE_ALLOCATION_1800_FAIL
	start_time	START_TIME_1
	freq_list_before	NOT_USED 
	chan_desc_before	CHANNEL_DESC_FACCH4
	chan_desc_before_2	NOT_USED
	freq_chan_seq	NOT_USED
	mob_alloc_before	NOT_USED
	ciph_mode_set	NOT_USED
	}

- DL_SUSPEND_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0

- DL_RECONNECT_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_ASSIGN_FAIL
	ti	TI_0
	rr_cause	RRC_FREQ_NOT_IMPL
	}

History:	1. Jul  02	MPA	Initial

### RR701: unsuccessful intracell handover without multirate conf IE-AMR

Description:	The base station initiates an intracell handover. RR releases the old connection locally and configures the new channel configuration(initial assignment).

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_RECONNECT_REQ	  |
	 |		  |	(ASSIGNMENT FAILURE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_ASSIGN_CMD
		ti	TI_0
		chan_desc	CHANNEL_DESC_FACCH2
		pow_cmd	POWER_COMMAND_05
		freq_list_after	NOT_USED
		cell_chan_desc	NOT_USED
		multislot_alloc	NOT_USED
		chan_mode	CHANNEL_MODE_AMR
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		mob_alloc_after	NOT_USED
		start_time	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	NOT_USED
		chan_desc_before_2	NOT_USED
		freq_chan_seq	NOT_USED
		mob_alloc_before	NOT_USED
		ciph_mode_set	NOT_USED
		vgcs_tmi	NOT_USED
		multirate_conf	NOT_USED
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- DL_RECONNECT_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_ASSIGN_FAIL
		ti	TI_0
		rr_cause	RR_CAUSE_09
		}

History:	18. Jan  02	OT	Initial, for AMR

### RR702: unsuccessful intracell handover with inconsistent multirate conf IE-AMR

Description:	The base station initiates an intracell handover. RR releases the old connection locally and configures the new channel configuration(initial assignment).

<A>:	acs does not include any codec mode
<B>:	acs does include more than four codec modes
<C>:	one codec mode is not supported by the assigned channel
<D>:	thres & hyst values are not set according 05.09 – wrong order
<E>:	thres & hyst values are not set according 05.09 – less values than (acs-1)

Variants:	<A>....<E>

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_RECONNECT_REQ	  |
	 |		  |	(ASSIGNMENT FAILURE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_ASSIGN_CMD
		ti	TI_0
		chan_desc	CHANNEL_DESC_FACCH2
		pow_cmd	POWER_COMMAND_05
		freq_list_after	NOT_USED
		cell_chan_desc	NOT_USED
		multislot_alloc	NOT_USED
		chan_mode	CHANNEL_MODE_AMR
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		mob_alloc_after	NOT_USED
		start_time	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	NOT_USED
		chan_desc_before_2	NOT_USED
		freq_chan_seq	NOT_USED
		mob_alloc_before	NOT_USED
		ciph_mode_set	NOT_USED
		vgcs_tmi	NOT_USED
<A>	multirate_conf	S_multirate_conf_4_702A
<B>	multirate_conf	S_multirate_conf_4_702b
<C>	multirate_conf	S_multirate_conf_4_702c
<D>	multirate_conf	S_multirate_conf_4_702d
<E>	multirate_conf	S_multirate_conf_4_702E
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- DL_RECONNECT_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_ASSIGN_FAIL
		ti	TI_0
		rr_cause	RRC_CHANNEL_MODE
		}

History:	18. Jan  02	OT	Initial, for AMR

### RR206: new configuration failed, back to old configuration

Description:	Establishment of new configuration failed. RR retries to reconnect the old configuration.

Preamble:	RR204

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_DEDICATED_FAIL_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_FAIL_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	DL_RECONNECT_REQ	  |
	 |		  |	(ASSIGNMENT FAILURE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_RELEASE_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		cs	NOT_USED

- MPH_DEDICATED_FAIL_REQ
		param	NOT_USED

- MPH_DEDICATED_FAIL_CNF
		param	NOT_USED

- DL_RECONNECT_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_ASSIGN_FAIL
		ti	TI_0
		rr_cause	RR_CAUSE_6F
		}

History:	04.07.97	DL	Initial

### RR207: reconnection of old configuration successful

Description:	Establishment of old configuration is successful.

Preamble:	RR206

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

History:	04.07.97	DL	Initial

### RR208: reconnection of old configuration failed

Description:	Establishment of old configuration has failed. RR releases the connection and enters the idle mode.

Preamble:	RR206

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_RELEASE_IND	  |
	 |		  *<==============================*
(2)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(3)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_RELEASE_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		cs	NOT_USED

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM
		cause	RRCS_DATA_LINK_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

History:
	04.07.97	DL	Initial
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

## Handover Command

### RR215: Handover Command

Description:	The base station initiates a handover. RR  releases the old connection locally and configures the new channel configuration.

Preamble:	RR185

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(HANDOVER COMPLETE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_HANDOV_CMD
		ti	TI_0
		cell_desc	CELL_DESC_1
		chan_desc_after	CHANNEL_DESC_FACCH3
		handov_ref	HANDOV_REF_1
		pow_cmd_access	POW_05_HO
		synch_ind	SYNCH_IND_2
		freq_short_list_after	NOT_USED
		freq_list_after	NOT_USED
		cell_chan_desc	CELL_CHAN_DESC_1
		chan_mode	CHANNEL_MODE_SPEECH
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		freq_chan_seq_after	NOT_USED
		mob_alloc_after	MOBILE_ALLOCATION_2
		start_time	START_TIME_1
		time_diff	NOT_USED
		time_advance	NOT_USED
		freq_short_list_before	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	CHANNEL_DESC_FACCH4
		chan_desc_before_2	NOT_USED
		freq_chan_seq_before	NOT_USED
		mob_alloc_before	MOBILE_ALLOCATION_3
		ciph_mode_set	NOT_USED
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_DEDICATED_REQ
		mod	MODE_PRE_SYNC_HANDOVER
		start	STARTING_TIME_1
		ch_type	PRR_CHANNEL_TYPE_6
		ch_type2	PRR_CHANNEL_TYPE_7
		arfcn	ARFCN_32
		bsic	NOT_USED
		ho_param	HO_PARAM_1
		tr_para	PRR_TR_PARA_HO
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
	dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_HANDOV_COMP
		ti	TI_0
		rr_cause	RR_CAUSE_0
		}

History:	19.05.98	VK	Initial
	25.01.01	DG	DL_DATA_IND: chan_mode2…8 added
	12.02.01.	DG	DL_DATA_REQ deleted
	18.Jan.02.	OT	Synch_ind structure renumbered.

26-Feb-02	OT	Adaptations for AMR implementation

### RR216: Handover Command

Description:	The base station initiates a handover. RR  releases the old connection locally and configures the new channel configuration.

Preamble:	RR185

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(HANDOVER COMPLETE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_HANDOV_CMD
		ti	TI_0
		cell_desc	CELL_DESC_1
		chan_desc_after	CHANNEL_DESC_FACCH3
		handov_ref	HANDOV_REF_1
		pow_cmd_access	POW_05_HO
		synch_ind	SYNCH_IND_0
		freq_short_list_after	NOT_USED
		freq_list_after	NOT_USED
		cell_chan_desc	CELL_CHAN_DESC_1
		chan_mode	CHANNEL_MODE_SPEECH
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		freq_chan_seq_after	NOT_USED
		mob_alloc_after	MOBILE_ALLOCATION_2
		start_time	START_TIME_1
		time_diff	NOT_USED
		time_advance	NOT_USED
		freq_short_list_before	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	CHANNEL_DESC_FACCH4
		chan_desc_before_2	NOT_USED
		freq_chan_seq_before	NOT_USED
		mob_alloc_before	MOBILE_ALLOCATION_3
		ciph_mode_set	NOT_USED
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_DEDICATED_REQ
		mod	MODE_ASYNC_HANDOVER
		start	STARTING_TIME_1
		ch_type	PRR_CHANNEL_TYPE_6
		ch_type2	PRR_CHANNEL_TYPE_7
		arfcn	ARFCN_32
		bsic	NOT_USED
		ho_param	HO_PARAM_0
		tr_para	PRR_TR_PARA_ASY_HO
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
	dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_HANDOV_COMP
		ti	TI_0
		rr_cause	RR_CAUSE_0
		}

History:	19.05.98	VK	Initial
	25.01.01	DG	DL_DATA_IND: chan_mode2…8 added
	26-Feb-02	OT	Adaptations for AMR implementation

### RR703: Handover Command - AMR

Description:	The base station initiates a handover. RR  releases the old connection locally and configures the new channel configuration with multirate configuration values.

<A>:	non sync handover, with multirate conf IE & with initial codec mode
<B>:	sync handover, with multirate conf IE & with initial codec mode
<C>:	pre sync handover, with multirate conf IE & with initial codec mode
<D>:	async handover, with multirate conf IE & w/o initial codec mode
<E>:	async handover, w/o multirate conf IE 
<F>:	async handover, with FR->HR and different ACS
<G>:	async handover, with HR -> FR and different ACS 
<H>:	async handover, with FR -> HR and different ACS Ericsson
<I>:	async handover, with FR -> HR and different ACS Alcatel
<J>:	async handover, with HR -> FR and different ACS Ericsson
<K>:	async handover, with HR -> FR and different ACS Alcatel

Variants:	<A>....<F>

Preamble:	<A>RR185
		<B>RR185
		<C>RR185
		<D>RR185
		<E>RR185
		<F> RR700A
		<G>RR700F
		<H>RR700G
		<I>RR700H
		<J> RR700I
		<K> RR700J

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(HANDOVER COMPLETE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_HANDOV_CMD
		ti	TI_0
		cell_desc	CELL_DESC_1
<A>	chan_desc_after	CHANNEL_DESC_FACCH3
<B>	chan_desc_after	CHANNEL_DESC_FACCH3
<C>	chan_desc_after	CHANNEL_DESC_FACCH3
<D>	chan_desc_after	CHANNEL_DESC_FACCH3
<E>	chan_desc_after	CHANNEL_DESC_FACCH3
<F>	chan_desc_after	CHANNEL_DESC_FACCH3_HALF
<G>	chan_desc_after	CHANNEL_DESC_FACCH3
<H> 	chan_desc_after	CHANNEL_DESC_FACCH3_HALF
<I> 		chan_desc_after	CHANNEL_DESC_FACCH3_HALF
<J> 	chan_desc_after	CHANNEL_DESC_FACCH3
<K> 	chan_desc_after	CHANNEL_DESC_FACCH3
		handov_ref	HANDOV_REF_1
		pow_cmd_access	POW_05_HO
<A>	synch_ind	SYNCH_IND_0
<B>	synch_ind	SYNCH_IND_1
<C>	synch_ind	SYNCH_IND_2
<D>	synch_ind	SYNCH_IND_0
<E>	synch_ind	SYNCH_IND_0
<F>	synch_ind	SYNCH_IND_0
<G>	synch_ind	SYNCH_IND_0
<H>	synch_ind	SYNCH_IND_0
<I>		synch_ind	SYNCH_IND_0
<J>		synch_ind	SYNCH_IND_0
<K>	synch_ind	SYNCH_IND_0
		freq_short_list_after	NOT_USED
		freq_list_after	NOT_USED
		cell_chan_desc	CELL_CHAN_DESC_1
		chan_mode	CHANNEL_MODE_AMR
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		freq_chan_seq_after	NOT_USED
		mob_alloc_after	MOBILE_ALLOCATION_2
		start_time	START_TIME_1
		time_diff	NOT_USED
		time_advance	NOT_USED
		freq_short_list_before	NOT_USED
		freq_list_before	NOT_USED
<A>	chan_desc_before	CHANNEL_DESC_FACCH4
<B>	chan_desc_before	CHANNEL_DESC_FACCH4
<C>	chan_desc_before	CHANNEL_DESC_FACCH4
<D>	chan_desc_before	CHANNEL_DESC_FACCH4
<E>	chan_desc_before	CHANNEL_DESC_FACCH4
<F>	chan_desc_before	CHANNEL_DESC_FACCH4_HALF
<G>	chan_desc_before	CHANNEL_DESC_FACCH4
<H>	chan_desc_before	CHANNEL_DESC_FACCH4_HALF
<I>		chan_desc_before	CHANNEL_DESC_FACCH4_HALF
<J>		chan_desc_before	CHANNEL_DESC_FACCH4
<K>	chan_desc_before	CHANNEL_DESC_FACCH4
		chan_desc_before_2	NOT_USED
		freq_chan_seq_before	NOT_USED
		mob_alloc_before	MOBILE_ALLOCATION_3
		ciph_mode_set	NOT_USED
		vgcs_tmi	NOT_USED
<A>	multirate_conf	S_MULTIRATE_CONF_4_ICMI
<B>	multirate_conf	S_MULTIRATE_CONF_4_ICMI
<C>	multirate_conf	S_MULTIRATE_CONF_4_ICMI
<D>	multirate_conf	S_MULTIRATE_CONF_4
<E>	multirate_conf	NOT_USED
<F>	multirate_conf	S_MULTIRATE_CONF_2
<G>	multirate_conf	S_MULTIRATE_CONF_2
<H>	multirate_conf	S_MULTIRATE_CONF_ER_AHS
<I>		multirate_conf	S_MULTIRATE_CONF_ALC_AHS
<J>		multirate_conf	S_MULTIRATE_CONF_ER_AFS
<K>	multirate_conf	S_MULTIRATE_CONF_ALC_AFS
		}

- DL_SUSPEND_REQ
<A>	ch_type	CH_TYPE_SDCCH
<B>	ch_type	CH_TYPE_SDCCH
<C>	ch_type	CH_TYPE_SDCCH
<D>	ch_type	CH_TYPE_SDCCH
<E>	ch_type	CH_TYPE_SDCCH
<F>	ch_type	CH_TYPE_FACCH
<G>	ch_type	CH_TYPE_FACCH_HR
<H>	ch_type	CH_TYPE_FACCH
<I> 		ch_type	CH_TYPE_FACCH
<J>		ch_type	CH_TYPE_FACCH_HR
<K>	ch_type	CH_TYPE_FACCH_HR
		sapi	SAPI_0

- MPH_DEDICATED_REQ
<A>	mod	MODE_ASYNC_HANDOVER
<B>	mod	MODE_SYNC_HANDOVER
<C>	mod	MODE_PRE_SYNC_HANDOVER
<D>	mod	MODE_ASYNC_HANDOVER
<E>	mod	MODE_ASYNC_HANDOVER
<F>	mod	MODE_ASYNC_HANDOVER
<G> 	mod	MODE_ASYNC_HANDOVER
<H> 	mod	MODE_ASYNC_HANDOVER
<I>	 	mod	MODE_ASYNC_HANDOVER
<J> 	mod	MODE_ASYNC_HANDOVER
<K> 	mod	MODE_ASYNC_HANDOVER
		start	STARTING_TIME_1
<A>	ch_type	PRR_CHANNEL_TYPE_6
<B>	ch_type	PRR_CHANNEL_TYPE_6
<C>	ch_type	PRR_CHANNEL_TYPE_6
<D>	ch_type	PRR_CHANNEL_TYPE_6
<E>	ch_type	PRR_CHANNEL_TYPE_6
<F>	ch_type	PRR_CHANNEL_TYPE_HOP_HALF2
<G>	ch_type	PRR_CHANNEL_TYPE_6
<H> 	ch_type	PRR_CHANNEL_TYPE_HOP_HALF2
<I> 		ch_type	PRR_CHANNEL_TYPE_HOP_HALF2
<J> 	ch_type	PRR_CHANNEL_TYPE_6
<K> 	ch_type	PRR_CHANNEL_TYPE_6 
<A>	ch_type2	PRR_CHANNEL_TYPE_7
<B>	ch_type2	PRR_CHANNEL_TYPE_7
<C>	ch_type2	PRR_CHANNEL_TYPE_7
<D>	ch_type2	PRR_CHANNEL_TYPE_7
<E>	ch_type2	PRR_CHANNEL_TYPE_7
<F>	ch_type2	PRR_CHANNEL_TYPE_HOP_HALF
<G>	ch_type2	PRR_CHANNEL_TYPE_7
<H>	ch_type2	PRR_CHANNEL_TYPE_HOP_HALF
<I>		ch_type2	PRR_CHANNEL_TYPE_HOP_HALF
<J>		ch_type2	PRR_CHANNEL_TYPE_7
<K>	ch_type2	PRR_CHANNEL_TYPE_7
		arfcn	ARFCN_32
		bsic	NOT_USED
<A>	ho_param	HO_PARAM_0
<B>	ho_param	HO_PARAM_1
<C>	ho_param	HO_PARAM_1
<D>	ho_param	HO_PARAM_0
<E>	ho_param	HO_PARAM_0
<F>	ho_param	HO_PARAM_0
<G>	ho_param	HO_PARAM_0
<H>	ho_param	HO_PARAM_0
<I>		ho_param	HO_PARAM_0
<J>		ho_param	HO_PARAM_0
<K>	ho_param	HO_PARAM_0
		tr_para	PRR_TR_PARA_AMR_1
		ciph	NO_CIPHERING
<A>	amr_conf	S_AMR_CONF_4_ICMI
<B>	amr_conf	S_AMR_CONF_4_ICMI
<C>	amr_conf	S_AMR_CONF_4_ICMI
<D>	amr_conf	S_AMR_CONF_4
<E>	amr_conf	S_AMR_CONF_0
<F>	amr_conf	S_AMR_CONF_2
<G>	amr_conf	S_AMR_CONF_2
<H>	amr_conf	S_AMR_CONF_7_ER_AHS
<I>		amr_conf	S_AMR_CONF_8_ALC_AHS
<J>		amr_conf	S_AMR_CONF_5_ER_AFS
<K>	amr_conf	S_AMR_CONF_6_ALC_AFS

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
<A>	ch_type	CH_TYPE_FACCH
<B>	ch_type	CH_TYPE_FACCH
<C>	ch_type	CH_TYPE_FACCH
<D>	ch_type	CH_TYPE_FACCH
<E>	ch_type	CH_TYPE_FACCH
<F>	ch_type	CH_TYPE_FACCH_HR
<G>	ch_type	CH_TYPE_FACCH
<H>	ch_type	CH_TYPE_FACCH_HR
<I>		ch_type	CH_TYPE_FACCH_HR
<J>		ch_type	CH_TYPE_FACCH
<K>	ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_HANDOV_COMP
		ti	TI_0
		rr_cause	RR_CAUSE_0
		}

- DL_ESTABLISH_CNF
<A>	ch_type	CH_TYPE_FACCH
<B>	ch_type	CH_TYPE_FACCH
<C>	ch_type	CH_TYPE_FACCH
<D>	ch_type	CH_TYPE_FACCH
<E>	ch_type	CH_TYPE_FACCH
<F>	ch_type	CH_TYPE_FACCH_HR
<G>	ch_type	CH_TYPE_FACCH
<H>	ch_type	CH_TYPE_FACCH_HR
<I>		ch_type	CH_TYPE_FACCH_HR
<J>		ch_type	CH_TYPE_FACCH
<K>	ch_type	CH_TYPE_FACCH
		sapi	SAPI_0

History:	18.Feb.02	OT	Initial

### RR706: Handover Command - AMR - no Channel Mode element

Description:	The base station initiates a handover. RR  releases the old connection locally and configures the new channel configuration with multirate configuration values:

async handover, with HR -> FR and different ACS Alcatel 2

Preamble: 
		RR700J

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(HANDOVER COMPLETE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu	AMR_HO_ALC

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_FACCH_HR
		sapi	SAPI_0

- MPH_DEDICATED_REQ
	 	mod	MODE_ASYNC_HANDOVER
		start	NOT_USED
	 	ch_type	NOT_USED
		ch_type2	NOT_USED
		arfcn	NOT_USED
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	NOT_USED
		ciph	NOT_USED
		amr_conf	S_AMR_conf_6_ALC_AFS2

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_HANDOV_COMP
		ti	TI_0
		rr_cause	RR_CAUSE_0
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0

History:	18.Feb.02	OT	Initial

### RR704: Handover cmd with inconsistent multirate conf IE - AMR

Description:	The base station initiates a handover. RR releases the old connection locally and configures the new channel configuration(handover cmd).

<A>:	acs does not include any codec mode
<B>:	acs does include more than four codec modes
<C>:	one codec mode is not supported by the assigned channel
<D>:	thres & hyst values are not set according 05.09 – wrong order
<D>:	thres & hyst values are not set according 05.09 – less values than (acs-1)

Variants:	<A>....<E>

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_RECONNECT_REQ	  |
	 |		  |	(HANDOVER FAILURE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_HANDOV_CMD
		ti	TI_0
		cell_desc	CELL_DESC_1
		chan_desc_after	CHANNEL_DESC_FACCH3
		handov_ref	HANDOV_REF_1
		pow_cmd_access	POW_05_HO
		synch_ind	SYNCH_IND_0
		freq_short_list_after	NOT_USED
		freq_list_after	NOT_USED
		cell_chan_desc	CELL_CHAN_DESC_1
		chan_mode	CHANNEL_MODE_AMR
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		freq_chan_seq_after	NOT_USED
		mob_alloc_after	MOBILE_ALLOCATION_2
		start_time	START_TIME_1
		time_diff	NOT_USED
		time_advance	NOT_USED
		freq_short_list_before	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	CHANNEL_DESC_FACCH4
		chan_desc_before_2	NOT_USED
		freq_chan_seq_before	NOT_USED
		mob_alloc_before	MOBILE_ALLOCATION_3
		ciph_mode_set	NOT_USED
		vgcs_tmi	NOT_USED
<A>	multirate_conf	S_multirate_conf_4_702A
<B>	multirate_conf	S_multirate_conf_4_702b
<C>	multirate_conf	S_multirate_conf_4_702c
<D>	multirate_conf	S_multirate_conf_4_702d
<E>	multirate_conf	S_multirate_conf_4_702e
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- DL_RECONNECT_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_HANDOV_FAIL
		ti	TI_0
		rr_cause	RR_CAUSE_09
		}

History:	18.Feb. 18, 02	OT	Initial

### RR217: Handover Command with mandatory errors

Description:	The base station initiates a handover with mandatory errors. RR  shall reject the command and answers with a RR-Status message.

Preamble:	RR185

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(RR STATUS)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu	HANDOVER_CMD_ERROR

- DL_DATA_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	B_RR_STATUS
		ti	TI_0
		rr_cause	RR_CAUSE_96
		}

History:	15-Feb-00	LE	Initial

### RR218: Assignment Command with mandatory errors

Description:	The base station initiates a handover with mandatory errors. RR  shall reject the command and answers with a RR-Status message.

Preamble:	RR185

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(RR STATUS)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu	ASSIGNMENT_CMD_ERROR

- DL_DATA_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	B_RR_STATUS
		ti	TI_0
		rr_cause	RR_CAUSE_96
		}

History:	27-Feb-03	ZMM	Initial

## Channel Release

### RR227: Timeout T3110, same cell

Description:	RR releases the connection after timeout of the channel release timer. The cell in idle mode is the same as before. So RR sends no RR ACTIVATE IND to MM.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
(3)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CHANNEL_RELEASE)	  |
	 |		  *<==============================*
(4)	 |		  |	DL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_RELEASE_IND	  |		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_0
		gprs_sync	NOT_USED

- DL_UNITDATA_REQ
			ch_type	CH_TYPE_SACCH
			sapi	SAPI_0
			sdu
			{
			component	RR
			direction	UPLINK
			pd	U_MEAS_REP
			ti	TI_0
			meas_result	MEAS_RESULT_NCELL_0
			}

- DL_DATA_IND
			ch_type	CH_TYPE_SDCCH
			sapi	SAPI_0
			sdu
			{
			component	RR
			direction	DOWNLINK
			pd	D_CHAN_REL
			ti	TI_0
			rr_cause	RR_CAUSE_0
			ba_range	NOT_USED
			}

- DL_RELEASE_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- RR_RELEASE_IND
		cause	RRCS_NORM
		sapi	SAPI_0
		gprs_resumption	NOT_USED

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_2
		sync_only	NOT_USED

History:	04.07.97	DL	Initial
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	25.01.01	DG	RR_RELEASE_IND: gprs_resumption added
	31.01.01	DG	DL_DATA_REQ added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF > 
				SI3_REST_EMPTY
	12.02.01	DG	DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2

### RR229: Release by DL_RELEASE_IND, same cell

Description:	DL releases the connection before timeout of the channel release timer using the DL_RELEASE_IND primitive. The cell in idle mode is the same as before. So RR sends no RR ACTIVATE IND to MM.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (RR CONFIG TIMER_SET=<T3110, 20000>)
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
(3)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CHANNEL_RELEASE)	  |
	 |		  *<==============================*
(4)	 |		  |	DL_RELEASE_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	DL_RELEASE_IND	  |
	 |		  *<==============================*
(6)	 |	RR_RELEASE_IND	  |		  |
	 *<==============================*		  |
(7)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
			arfcn	ARFCN_67
			rx_lev_full	RX_LEV_20
			rx_lev_sub	RX_LEV_20
			rx_qual_full	RX_QUAL_1
			rx_qual_sub	RX_QUAL_1
			dtx	DTX_NOT_USED
			otd	TIME_ADV_27
			valid	TRUE
			fn_offset	FN_OFFSET_1_SEC
			ncells	S_NCELLS_0
			gprs_sync	NOT_USED

- DL_UNITDATA_REQ
			ch_type	CH_TYPE_SACCH
			sapi	SAPI_0
			sdu
			{
			component	RR
			direction	UPLINK
			pd	U_MEAS_REP
			ti	TI_0
			meas_result	MEAS_RESULT_NCELL_0
			}

- DL_DATA_IND
			ch_type	CH_TYPE_SDCCH
			sapi	SAPI_0
			sdu
			{
			component	RR
			direction	DOWNLINK
			pd	D_CHAN_REL
			ti	TI_0
			rr_cause	RR_CAUSE_0
			ba_range	NOT_USED
			}

- DL_RELEASE_REQ
			ch_type	CH_TYPE_SDCCH
			sapi	SAPI_0

- DL_RELEASE_IND
			ch_type	CH_TYPE_SDCCH
			sapi	SAPI_0
			cs	NOT_USED

- RR_RELEASE_IND
			cause	RRCS_NORM
			sapi	SAPI_0
			gprs_resumption	NOT_USED

- MPH_IDLE_REQ
			mod	MODE_CELL_SELECTION
			arfcn	ARFCN_67
			ext_bcch	NOT_USED
			comb_ccch	CCD_CCCH_1_NOT_COMB
			tn	TN_0
			dlt	DLT_23
			pg	PG_0
			bs_ag_blocks_res	BS_AG_BLKS_RES_5
			bs_pa_mfrms	BS_PA_MFRMS_2
			power	MS_TXPWR_MAX_CCH_02
			ncc_permitted	NCC_PERMITTED_1
			reorg_only	NOT_USED
			eotd_avail	CONST_0
			gprs_support	NOT_USED

- MPH_NEIGHBOURCELL_REQ
			multi_band	NOT_USED
			arfcn	A_MPH_NCELL_2
			sync_only	NOT_USED

History:	04.07.97	DL	Initial
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	25.01.01	DG	RR_RELEASE_IND: gprs_resumption added
	31.01.01	DG	DL_DATA_REQ (classmark ch.) added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF > 
				SI3_REST_EMPTY
	12.02.01	DG	DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2

## Layer 3 Messages

### RR230: Downlink Messages

Description:	DL sends complete layer 3 messages to RR. RR checks the protocol discriminator and sends the message to MM.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(LAYER 3 MESSAGE)	  |
	 |		  *<==============================*
(2)	 |	RR_DATA_IND	  |		  |
	 |	(LAYER 3 MESSAGE)	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu	MM_MESSAGE

- RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	MM_MESSAGE

History:	04.07.97	DL	Initial
	31.01.01	DG	DL_DATA_REQ (classmark ch.) added
	12.02.01	DG	DL_DATA_REQ deleted

### RR231: Uplink Messages

Description:	MM sends a complete layer 3 message  to RR. RR forwards the message to DL.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_DATA_REQ	  |		  |
	 |	(LAYER 3 MESSAGE)	  |		  |
	 *==============================>*		  |
(2)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(LAYER 3 MESSAGE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_DATA_REQ
		d1	NOT_USED
		d2	NOT_USED
		sdu	MM_MESSAGE

- DL_DATA_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu	MM_MESSAGE

History:	04.07.97	DL	Initial
	31.01.01	DG	DL_DATA_REQ (classmark ch.) added
	12.02.01	DG	DL_DATA_REQ deleted

## Measurement Report

### RR185: Send Measurement Reports on SACCH

Description:	RR is in state Dedicated. After reception of new measurement information from layer 1 the measurement report is build and forwarded to DL.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(4)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1D
		sync_only	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- DL_UNITDATA_REQ
		ch_type	CH_TYPE_SACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_MEAS_REP
		ti	TI_0
		meas_result	MEAS_RESULT_NCELL_3
		}

History:	04.07.97	DL	Initial
	25.01.01	DG	DL_DATA_REQ added
	05-Jun-01	MSE	adapted to TAP2

### RR186: Send Measurement Reports on SACCH (including 5ter)

Description:	RR is in state Dedicated. After reception of new measurement information from layer 1 the measurement report is build and forwarded to DL.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
(1)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5Ter)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(4)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5TER
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5TER
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_DG_186
		sync_only	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- DL_UNITDATA_REQ
		ch_type	CH_TYPE_SACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_MEAS_REP
		ti	TI_0
		meas_result	MEAS_RESULT_NCELL_3_DG_186
		}

History:	04.07.97	DL	Initial
	14.03.00	DG	values changed:
			(2) MPH_NCELL_5TER > MPH_NCELL_DG_186,
			(4) MEAS_RESULT_NCELL_3 >
			MEAS_RESULT_NCELL_3_DG_186
			(NCELLS_3_SACCH_DG_186)
	25.01.01	DG	DL_DATA_REQ added
	12.02.01	DG	DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2

### RR187: Send Measurement Reports on SACCH (Multiband)

Description:	This is a an extract of testcase 26.11.2.3. RR is in state Dedicated. After reception of new measurement information from layer 1 the measurement report is build and forwarded to DL. 5ter includes DCS 1800, 5 includes GSM 900.

Preamble:	RR159

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5ter)	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(6)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_5
	ti	TI_0
	neigh_cell_desc	NCELL_DESC_5_FTA
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	MULTI_BAND_0
	arfcn	A_MPH_NCELL_5_FTA
	sync_only	NOT_USED

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_5TER
	ti	TI_0
	neigh_cell_desc	NCELL_DESC_5TER_FTA
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	MULTI_BAND_3
	arfcn	A_MPH_NCELL_5TER_5_FTA
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_6_FTA
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_6_FTA
	}

History:	15-Feb-00	LE	Initial
	25.01.01	DG	IMMEDIATE ASSIGNMENT:
				tma, dl, d_t, pck_chan_desc added,
				DL_DATA_REQ added
	12.02.01	DG	MPH_SYNC_REQ added
				DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR188: Send Measurement Reports on SACCH (E-GSM)

Description:	RR is in state Dedicated. After reception of new system information message the neighbourcell list is build and forwarded to Layer 1. 5bis includes GSM 900 plus E-GSM, 5 includes DCS 1800.

Preamble:	RR153

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5bis)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	NOT_USED
		dl	NOT_USED
		d_t	NOT_USED
		page_mode	PAGING_NORMAL
		chan_desc	CHANNEL_DESC_SDCCH
		pck_chan_desc	NOT_USED
		req_ref	REQUEST_REFERENCE_1
		time_advance	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_1
		start_time	NOT_USED
		ia_rest_oct	NOT_USED
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_2
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_2
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
			dedi_res	NOT_USED

- DL_ESTABLISH_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	NOT_USED
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5_FTA_A
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_FTA_A
		sync_only	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5BIS
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5BIS_FTA_A
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_5BIS_FTA_A
		sync_only	NOT_USED

History:	15-Feb-00	LE	Initial
	25.01.01	DG	IMMEDIATE ASSIGNMENT:
				tma, dl, d_t, pck_chan_desc added,
				DL_DATA_REQ added
	12.02.01	DG	MPH_SYNC_REQ added
				DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR189: Send Measurement Reports on SACCH (E-GSM) (II)

Description:	RR is in state Dedicated. After reception of new system information message the neighbourcell list is build and forwarded to Layer 1. The testcase is build according to 26.10.2.1 tc=5.

Preamble:	RR153

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5bis)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(14)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	NOT_USED
		dl	NOT_USED
		d_t	NOT_USED
		page_mode	PAGING_NORMAL
		chan_desc	CHANNEL_DESC_SDCCH
		pck_chan_desc	NOT_USED
		req_ref	REQUEST_REFERENCE_1
		time_advance	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_1
		start_time	NOT_USED
		ia_rest_oct	NOT_USED
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_2
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_2
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
			dedi_res	NOT_USED

- DL_ESTABLISH_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	NOT_USED
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5_FTA_B
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_FTA_B
		sync_only	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5BIS
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5BIS_FTA_B
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_5BIS_FTA_B
		sync_only	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_MPH_NCELLS_FTA_B
		gprs_sync	NOT_USED

- DL_UNITDATA_REQ
		ch_type	CH_TYPE_SACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_MEAS_REP
		ti	TI_0
		meas_result	MEAS_RESULT_NCELL_FTA_B
		}

History:	15-Feb-00	LE	Initial
	25.01.01	DG	IMMEDIATE ASSIGNMENT:
				tma, dl, d_t, pck_chan_desc added,
				DL_DATA_REQ added
	12.02.01	DG	MPH_SYNC_REQ added
				DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR190: Send Measurement Reports on SACCH (E-GSM) (III)

Description:	RR is in state Dedicated. After reception of new system information message the neighbourcell list is build and forwarded to Layer 1. The testcase is build according to 26.10.2.1 tc=7.

Preamble:	RR153

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5bis)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(14)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	NOT_USED
		dl	NOT_USED
		d_t	NOT_USED
		page_mode	PAGING_NORMAL
		chan_desc	CHANNEL_DESC_SDCCH
		pck_chan_desc	NOT_USED
		req_ref	REQUEST_REFERENCE_1
		time_advance	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_1
		start_time	NOT_USED
		ia_rest_oct	NOT_USED
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_2
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_2
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
			dedi_res	NOT_USED

- DL_ESTABLISH_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		dpd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	NOT_USED
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5_FTA_C
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_FTA_C
		sync_only	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5TER
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5TER_FTA_C
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_5TER_FTA_C
		sync_only	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_MPH_NCELLS_FTA_C
		gprs_sync	NOT_USED

- DL_UNITDATA_REQ
		ch_type	CH_TYPE_SACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_MEAS_REP
		ti	TI_0
		meas_result	MEAS_RESULT_NCELL_FTA_C
		}

History:	15-Feb-00	LE	Initial
	25.01.01	DG	IMMEDIATE ASSIGNMENT:
				tma, dl, d_t, pck_chan_desc added,
				DL_DATA_REQ added
	12.02.01	DG	MPH_SYNC_REQ added
				DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR191: Send Measurement Reports on SACCH (E-GSM) (IV)

Description:	RR is in state Dedicated. After reception of new system information message the neighbourcell list is build and forwarded to Layer 1. The testcase is build according to 26.10.2.1 tc=8.

Preamble:	RR153

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5bis)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(14)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	NOT_USED
		dl	NOT_USED
		d_t	NOT_USED
		page_mode	PAGING_NORMAL
		chan_desc	CHANNEL_DESC_SDCCH
		pck_chan_desc	NOT_USED
		req_ref	REQUEST_REFERENCE_1
		time_advance	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_1
		start_time	NOT_USED
		ia_rest_oct	NOT_USED
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_2
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_2
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
			dedi_res	NOT_USED

- DL_ESTABLISH_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	NOT_USED
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5_FTA_D
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_FTA_D
		sync_only	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5TER
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5TER_FTA_D
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_5TER_FTA_D
		sync_only	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_MPH_NCELLS_FTA_D
		gprs_sync	NOT_USED

- DL_UNITDATA_REQ
		ch_type	CH_TYPE_SACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_MEAS_REP
		ti	TI_0
		meas_result	MEAS_RESULT_NCELL_FTA_D
		}

History:	15-Feb-00	LE	Initial
	25.01.01	DG	IMMEDIATE ASSIGNMENT:
				tma, dl, d_t, pck_chan_desc added,
				DL_DATA_REQ added
	12.02.01	DG	MPH_SYNC_REQ added
				DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR192: Send Measurement Reports on SACCH (E-GSM) (V)

Description:	RR is in state Dedicated. After reception of new system information message the neighbourcell list is build and forwarded to Layer 1. The testcase is build according to 26.10.2.1 tc=9.

Preamble:	RR153

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
(1)	 |		  |	MPH_RANDOM_ACCESS_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	(IMMEDIATE ASSIGNMENT)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	DL_ESTABLISH_REQ	  |
	 |		  |	(PAGING RESPONSE)	  |
	 |		  *==============================>*
(6)	 |		  |	DL_ESTABLISH_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |	RR_ESTABLISH_IND	  |		  |
	 *<==============================*		  |
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5bis)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(14)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_CNF
		frame_no	FRAME_NUMBER_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	NOT_USED
		dl	NOT_USED
		d_t	NOT_USED
		page_mode	PAGING_NORMAL
		chan_desc	CHANNEL_DESC_SDCCH
		pck_chan_desc	NOT_USED
		req_ref	REQUEST_REFERENCE_1
		time_advance	TIMING_ADVANCE_27
		mob_alloc	MOBILE_ALLOCATION_1
		start_time	NOT_USED
		ia_rest_oct	NOT_USED
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_2
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_2
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
			dedi_res	NOT_USED

- DL_ESTABLISH_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_PAG_RES
		ti	TI_0
		ciph_key_num	CKSN_RESERVED
		mob_class_2	NOT_USED
		mob_ident	MOBILE_IDENTITY_IMSI_HPLMN
		}

- DL_ESTABLISH_CNF
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_SYNC_REQ
		cs	CS_CLEAN_SYS_INFO

- RR_ESTABLISH_IND
		param	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5_FTA_E
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_FTA_E
		sync_only	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5TER
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_5TER_FTA_E
		}

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	A_MPH_NCELL_5_5TER_FTA_E
		sync_only	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_MPH_NCELLS_FTA_E
		gprs_sync	NOT_USED

- DL_UNITDATA_REQ
		ch_type	CH_TYPE_SACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_MEAS_REP
		ti	TI_0
		meas_result	MEAS_RESULT_NCELL_FTA_E
		}

History:	15-Feb-00	LE	Initial
	25.01.01	DG	IMMEDIATE ASSIGNMENT:
				tma, dl, d_t, pck_chan_desc added,
				DL_DATA_REQ added
	12.02.01	DG	MPH_SYNC_REQ added
				DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2
	26-Feb-02	OT	Adaptations for AMR implementation

### RR193: Send Measurement Reports on SACCH / 1900

Description:	RR is in state Dedicated. After reception of new measurement information from layer 1 the measurement report is build and forwarded to DL. This test case us used only as a part of a suite

Preamble:	none

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 5bis)	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(6)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_527
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_5
	ti	TI_0
	neigh_cell_desc	BCCH_FREQ_LIST_1900
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	MULTI_BAND_0
	arfcn	A_MPH_NCELL_1900a
	sync_only	FALSE

- MPH_UNITDATA_IND
	arfcn	ARFCN_527
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_5BIS
	ti	TI_0
	neigh_cell_desc	BCCH_FREQ_LIST_1900_bis
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	MULTI_BAND_0
	arfcn	A_MPH_NCELL_1900
	sync_only	FALSE

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_527
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_6
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_1900
	}

History:	4-Jul-02	MPA	Initial

## Extended Measurement Report on SACCH (EMO)

### RR250: EMO, Normal case including NC reconfig

Description:	When in dedicated mode or group transmit mode, a mobile station may receive an EXTENDED MEASUREMENT ORDER (EMO) message, from the network. The mobile station shall then, as defined in 3GPP TS 05.08, for one reporting period perform measurements on the frequencies specified by this EMO message. The mobile station shall thereafter send an EXTENDED MEASUREMENT REPORT message. This message contains the measurement results as defined in 3GPP TS 05.08.

- Preamble:	RR185

MM		 RR		PL/DL
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(EXT MEAS REPORT)	  |
	|		  *==============================>*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_MEASUREMENT_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(MEASUREMENT REPORT)	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_000
	}

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_000

- MPH_EMO_MEAS_IND
	ba_id	CONST_129
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_000

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_EXT_MEAS_REPORT
	ti	TI_0
	ext_meas_res	S_EXT_MEAS_RES_000
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1D
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_3
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_3
	}

History:	30.10.02	VK	Initial

### RR251: EMO, 10 sec expiry

Description:	If the mobile station has not started to send its EXTENDED MEASUREMENT REPORT within 10 seconds after the reception of the EMO message, no EXTENDED MEASUREMENT REPORT shall be sent.

Preamble:	RR250

MM		 RR		PL/DL
	|		  |		  |

- |	  |	MPH_UNITDATA_IND	  |
	|	  |	  (EXT MEAS ORDER)	  |
	|	  *<==============================*

- |	  |	MPH_EMO_REQ	  |
	|	  *==============================>*
MUTE (9000)
TIMEOUT (1000)

- |	  |	MPH_NEIGHBOURCELL_REQ	  |
	|	  *==============================>*

- |	  |	MPH_MEASUREMENT_IND	  |
	|	  *<==============================*

- |	  |	DL_UNITDATA_REQ	  |
	|	  |	(MEASUREMENT REPORT)	  |
	|	  *==============================>*
	|	  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_EXT_MEAS_ORDER
		ti	TI_0
		ext_meas_freq	EXT_MEAS_FREQ_000
		}

- MPH_EMO_REQ
		ba_id	CONST_130
		arfcn	A_ARFCN_EMO_000

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1D
		sync_only	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_3
		gprs_sync	NOT_USED

- DL_UNITDATA_REQ
		ch_type	CH_TYPE_SACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_MEAS_REP
		ti	TI_0
		meas_result	MEAS_RESULT_NCELL_3
		}

History:	30.10.02	VK	Initial
	06.03.03	LG	MUTE inserted

### RR252: EMO, before first neighbour cell configuration

Description:	This test case is similar to RR250, except that the EXTENDED MEASUREMENT ORDER (EMO) message is received before the Neighbour Cells are configured. Therefore no MPH_NEIGHBOURCELL_REQ must be sent after the (first) EMO procedure. In order to check this, a second EMO procedue is started.

Preamble:	RR154B

MM		 RR		PL/DL
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(EXT MEAS REPORT)	  |
	|		  *==============================>*

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_EXT_MEAS_ORDER
		ti	TI_0
		ext_meas_freq	EXT_MEAS_FREQ_000
		}

- MPH_EMO_REQ
		ba_id	CONST_129
		arfcn	A_ARFCN_EMO_000

- MPH_EMO_MEAS_IND
		ba_id	CONST_129
		dtx	CONST_1
		meas_results	S_EMO_MEAS_RES_000

- DL_UNITDATA_REQ
		ch_type	CH_TYPE_SACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_EXT_MEAS_REPORT
		ti	TI_0
		ext_meas_res	S_EXT_MEAS_RES_000
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_EXT_MEAS_ORDER
		ti	TI_0
		ext_meas_freq	EXT_MEAS_FREQ_000
		}

- MPH_EMO_REQ
		ba_id	CONST_130
		arfcn	A_ARFCN_EMO_000

History:	30.10.02	VK	Initial

### RR253: EMO, successful Channel Change during EMO

Description:	The mobile station shall abort after a successful channel change any pending measurements or reporting related to an EMO message received on the old channel. The neighbour cell description of the cell valid before EMO is configured.
Note: this test depends on a IUT change requested in ConQuest 8072.

- Preamble:	RR185

MM		 RR		PL/DL
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	DL_DATA_IND	  |
	|		  |	(ASSIGNMENT COMMAND)	  |
	|		  *<==============================*

- |		  |	DL_SUSPEND_REQ	  |
	|		  *==============================>*

- |		  |	MPH_DEDICATED_REQ	  |
	|		  *==============================>*

- |		  |	MPH_DEDICATED_CNF	  |
	|		  *<==============================*

- |		  |	DL_RESUME_REQ	  |
	|		  |	(ASSIGNMENT COMPLETE)	  |
	|		  *==============================>*

- |		  |	DL_ESTABLISH_CNF	  |
	|		  *<==============================*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_MEASUREMENT_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(MEASUREMENT REPORT)	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_000
	}

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_000

- DL_DATA_IND
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_ASSIGN_CMD
	ti	TI_0
	chan_desc	CHANNEL_DESC_SDCCH2
	pow_cmd	POWER_COMMAND_05
	freq_list_after	NOT_USED
	cell_chan_desc	NOT_USED
	multislot_alloc	NOT_USED
	chan_mode	NOT_USED
	chan_mode2	NOT_USED
	chan_mode3	NOT_USED
	chan_mode4	NOT_USED
	chan_mode5	NOT_USED
	chan_mode6	NOT_USED
	chan_mode7	NOT_USED
	chan_mode8	NOT_USED
	chan_desc_after_2	NOT_USED
	chan_mode_2	NOT_USED
	mob_alloc_after	NOT_USED
	start_time	NOT_USED
	freq_list_before	NOT_USED
	chan_desc_before	NOT_USED
	chan_desc_before_2	NOT_USED
	freq_chan_seq	NOT_USED
	mob_alloc_before	NOT_USED
	ciph_mode_set	NOT_USED
	vgcs_tmi	NOT_USED
	multirate_conf	S_multirate_conf_1
	}

- DL_SUSPEND_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0

- MPH_DEDICATED_REQ
	mod	MODE_CHAN_ASSIGN
	start	NO_STARTING_TIME
	ch_type	PRR_CHANNEL_TYPE_3
	ch_type2	NOT_USED
	arfcn	ARFCN_67
	bsic	NOT_USED
	ho_param	NOT_USED
	tr_para	PRR_TR_PARA_3
	ciph	NO_CIPHERING
	amr_conf	NOT_USED

- MPH_DEDICATED_CNF
	dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_ASSIGN_COMP
	ti	TI_0
	rr_cause	RR_CAUSE_0
	}

- DL_ESTABLISH_CNF
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1D
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_3
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_3
	}

- History:	30.10.02	VK	Initial
	04.03.03	LG	removed MPH_SYNC_REQ (see Con Quest 8072)

### RR254: EMO, failed Channel Change with successful reconnection during EMO

Description:	The mobile station shall abort after a successful channel change any pending measurements or reporting related to an EMO message received on the old channel. In case of a successful reconnection to the old channel the EMO procedure continues.
Note: this test depends on a IUT change requested in ConQuest 8072.

- Preamble:	RR185

MM		 RR		PL/DL
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	DL_DATA_IND	  |
	|		  |	(ASSIGNMENT COMMAND)	  |
	|		  *<==============================*

- |		  |	DL_SUSPEND_REQ	  |
	|		  *==============================>*

- |		  |	MPH_DEDICATED_REQ	  |
	|		  *==============================>*

- |		  |	MPH_DEDICATED_CNF	  |
	|		  *<==============================*

- |		  |	DL_RESUME_REQ	  |
	|		  |	(ASSIGNMENT COMPLETE)	  |
	|		  *==============================>*

- |		  |	DL_RELEASE_IND	  |
	|		  *<==============================*

- |		  |	MPH_DEDICATED_FAIL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_DEDICATED_FAIL_CNF	  |
	|		  *<==============================*

- |		  |	DL_RECONNECT_REQ	  |
	|		  |	(ASSIGNMENT FAILURE)	  |
	|		  *==============================>*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(EXT MEAS REPORT)	  |
	|		  *==============================>*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_MEASUREMENT_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(MEASUREMENT REPORT)	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_000
	}

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_000

- DL_DATA_IND
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_ASSIGN_CMD
	ti	TI_0
	chan_desc	CHANNEL_DESC_SDCCH2
	pow_cmd	POWER_COMMAND_05
	freq_list_after	NOT_USED
	cell_chan_desc	NOT_USED
	multislot_alloc	NOT_USED
	chan_mode	NOT_USED
	chan_mode2	NOT_USED
	chan_mode3	NOT_USED
	chan_mode4	NOT_USED
	chan_mode5	NOT_USED
	chan_mode6	NOT_USED
	chan_mode7	NOT_USED
	chan_mode8	NOT_USED
	chan_desc_after_2	NOT_USED
	chan_mode_2	NOT_USED
	mob_alloc_after	NOT_USED
	start_time	NOT_USED
	freq_list_before	NOT_USED
	chan_desc_before	NOT_USED
	chan_desc_before_2	NOT_USED
	freq_chan_seq	NOT_USED
	mob_alloc_before	NOT_USED
	ciph_mode_set	NOT_USED
	vgcs_tmi	NOT_USED
	multirate_conf	S_multirate_conf_1
	}

- DL_SUSPEND_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0

- MPH_DEDICATED_REQ
	mod	MODE_CHAN_ASSIGN
	start	NO_STARTING_TIME
	ch_type	PRR_CHANNEL_TYPE_3
	ch_type2	NOT_USED
	arfcn	ARFCN_67
	bsic	NOT_USED
	ho_param	NOT_USED
	tr_para	PRR_TR_PARA_3
	ciph	NO_CIPHERING
	amr_conf	NOT_USED

- MPH_DEDICATED_CNF
	dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_ASSIGN_COMP
	ti	TI_0
	rr_cause	RR_CAUSE_0
	}

- DL_RELEASE_IND
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	cs	NOT_USED

- MPH_DEDICATED_FAIL_REQ
	param	NOT_USED

- MPH_DEDICATED_FAIL_CNF
	param	NOT_USED

- DL_RECONNECT_REQ
	ch_type	CH_TYPE_SDCCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_ASSIGN_FAIL
	ti	TI_0
	rr_cause	RR_CAUSE_6F
	}

- MPH_EMO_MEAS_IND
	ba_id	CONST_129
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_000

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_EXT_MEAS_REPORT
	ti	TI_0
	ext_meas_res	S_EXT_MEAS_RES_000
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1D
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_3
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_3
	}

- History:	30.10.02	VK	Initial

### RR255: EMO, new system info 5* (new NC config) during EMO

Description:	When in dedicated mode or group transmit mode, a mobile station may receive an EXTENDED MEASUREMENT ORDER (EMO) message, from the network. The mobile station shall then, as defined in 3GPP TS 05.08, for one reporting period perform measurements on the frequencies specified by this EMO message. The mobile station shall thereafter send an EXTENDED MEASUREMENT REPORT message. This message contains the measurement results as defined in 3GPP TS 05.08.
This test checks that a System Information Type 5 Message received during EMO is handled correctly.

- Preamble:	RR185

MM		 RR		PL/DL
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (SYS INFO TYPE 5)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(EXT MEAS REPORT)	  |
	|		  *==============================>*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_MEASUREMENT_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(MEASUREMENT REPORT)	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_000
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_5
	ti	TI_0
	neigh_cell_desc	NCELL_DESC_1_EMO
	}

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_000

- MPH_EMO_MEAS_IND
	ba_id	CONST_129
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_000

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_EXT_MEAS_REPORT
	ti	TI_0
	ext_meas_res	S_EXT_MEAS_RES_000
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1D_EMO
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_3
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_3
	}

History:	30.10.02	VK	Initial

### RR256: EMO, frequencies out of band in EMO request

Description:	When in dedicated mode or group transmit mode, a mobile station may receive an EXTENDED MEASUREMENT ORDER (EMO) message, from the network. The mobile station shall then, as defined in 3GPP TS 05.08, for one reporting period perform measurements on the frequencies specified by this EMO message. The mobile station shall thereafter send an EXTENDED MEASUREMENT REPORT message. This message contains the measurement results as defined in 3GPP TS 05.08.
If the EMO message contains frequencies outside the MS’ frequency band, the MS shall set the corresponding RXLEV value(s) to zero.

- Preamble:	RR185

MM		 RR		PL/DL
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(EXT MEAS REPORT)	  |
	|		  *==============================>*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_MEASUREMENT_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(MEASUREMENT REPORT)	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_001
	}

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_001

- MPH_EMO_MEAS_IND
	ba_id	CONST_129
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_001

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_EXT_MEAS_REPORT
	ti	TI_0
	ext_meas_res	S_EXT_MEAS_RES_001
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1D
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_3
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_3
	}

History:	30.10.02	VK	Initial

### RR257: EMO, Frequency List contains frequency ARFCN=0

Description:	When in dedicated mode or group transmit mode, a mobile station may receive an EXTENDED MEASUREMENT ORDER (EMO) message, from the network. The mobile station shall then, as defined in 3GPP TS 05.08, for one reporting period perform measurements on the frequencies specified by this EMO message. The mobile station shall thereafter send an EXTENDED MEASUREMENT REPORT message. This message contains the measurement results as defined in 3GPP TS 05.08.
If the EMO message contains frequencies outside the MS’ frequency band, the MS shall set the corresponding RXLEV value(s) to zero.
This test checks that ARFCN = 0 is handled correctly.

- Preamble:	RR185

MM		 RR		PL/DL
	|		  |		  |

COMMAND (PL CONFIG STD=2)
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(EXT MEAS REPORT)	  |
	|		  *==============================>*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_MEASUREMENT_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(MEASUREMENT REPORT)	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_002
	}

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_002

- MPH_EMO_MEAS_IND
	ba_id	CONST_129
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_002

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_EXT_MEAS_REPORT
	ti	TI_0
	ext_meas_res	S_EXT_MEAS_RES_002
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1D
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_3
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_3
	}

History:	30.10.02	VK	Initial

### RR258: EMO, too many (more than 21) frequencies

Description:	When in dedicated mode or group transmit mode, a mobile station may receive an EXTENDED MEASUREMENT ORDER (EMO) message, from the network. The mobile station shall then, as defined in 3GPP TS 05.08, for one reporting period perform measurements on the frequencies specified by this EMO message. The mobile station shall thereafter send an EXTENDED MEASUREMENT REPORT message. This message contains the measurement results as defined in 3GPP TS 05.08.
If the EMO contains more than 21 carriers, only the 21 first carriers in the sorted EXTENDED MEASUREMENT FREQUENCY LIST (in the EMO) are measured and reported.

- Preamble:	RR185

MM		 RR		PL/DL
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(EXT MEAS REPORT)	  |
	|		  *==============================>*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_MEASUREMENT_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(MEASUREMENT REPORT)	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_003
	}

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_003

- MPH_EMO_MEAS_IND
	ba_id	CONST_129
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_003

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_EXT_MEAS_REPORT
	ti	TI_0
	ext_meas_res	S_EXT_MEAS_RES_003
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1D
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_3
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_3
	}

History:	30.10.02	VK	Initial

### RR259: EMO, Multiple EMO requests, same sequence number

Description:	If a mobile station receives an EMO message indicating the same value of the sequence code as an EMO message received earlier on the same channel without having received any EMO message indicating a different value of the sequence code in between, that EMO message shall be ignored.

- Preamble:	RR185

MM		 RR		PL/DL
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(EXT MEAS REPORT)	  |
	|		  *==============================>*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_MEASUREMENT_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(MEASUREMENT REPORT)	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_000
	}

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_000

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_000
	}

- MPH_EMO_MEAS_IND
	ba_id	CONST_129
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_000

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_EXT_MEAS_REPORT
	ti	TI_0
	ext_meas_res	S_EXT_MEAS_RES_000
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1D
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_3
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_3
	}

History:	30.10.02	VK	Initial

### RR260: EMO, Multiple EMO requests, different sequence number

Description:	If the mobile station, before the reporting related to an EMO message has started, receives a new EMO message with a different value of the sequence code, any pending measurements or reporting related to the earlier EMO message shall be aborted and the new message treated.

- Preamble:	RR185

MM		 RR		PL/DL
	|		  |		  |

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	MPH_UNITDATA_IND	  |
	|		  |	  (EXT MEAS ORDER)	  |
	|		  *<==============================*

- |		  |	MPH_EMO_REQ	  |
	|		  *==============================>*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	MPH_EMO_MEAS_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(EXT MEAS REPORT)	  |
	|		  *==============================>*

- |		  |	MPH_NEIGHBOURCELL_REQ	  |
	|		  *==============================>*

- |		  |	MPH_MEASUREMENT_IND	  |
	|		  *<==============================*

- |		  |	DL_UNITDATA_REQ	  |
	|		  |	(MEASUREMENT REPORT)	  |
	|		  *==============================>*
	|		  |		  |

- Parametrization

- Primitive	Parameter	Value

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_004
	}

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_000

- MPH_UNITDATA_IND
	arfcn	ARFCN_67
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_EXT_MEAS_ORDER
	ti	TI_0
	ext_meas_freq	EXT_MEAS_FREQ_000
	}

- MPH_EMO_REQ
	ba_id	CONST_130
	arfcn	A_ARFCN_EMO_000

- MPH_EMO_MEAS_IND
	ba_id	CONST_129
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_000

- MPH_EMO_MEAS_IND
	ba_id	CONST_130
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_000

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_EXT_MEAS_REPORT
	ti	TI_0
	ext_meas_res	S_EXT_MEAS_RES_000
	}

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1D
	sync_only	NOT_USED

- MPH_MEASUREMENT_IND
	arfcn	ARFCN_67
	rx_lev_full	RX_LEV_20
	rx_lev_sub	RX_LEV_20
	rx_qual_full	RX_QUAL_1
	rx_qual_sub	RX_QUAL_1
	dtx	DTX_NOT_USED
	otd	TIME_ADV_27
	valid	TRUE
	fn_offset	FN_OFFSET_1_SEC
	ncells	S_NCELLS_3
	gprs_sync	NOT_USED

- DL_UNITDATA_REQ
	ch_type	CH_TYPE_SACCH
	sapi	SAPI_0
	sdu
	{
	component	RR
	direction	UPLINK
	pd	U_MEAS_REP
	ti	TI_0
	meas_result	MEAS_RESULT_NCELL_3
	}

History:	30.10.02	VK	Initial

## Channel Mode Modify

### RR225: Channel Mode Modify Procedure

Description:	The base station configures the channel mode. MM and PL are informed and an acknowledge is sent to the base station.

Preamble:	RR154C

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CHANNEL MODE MODIFY)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_CHANNEL_MODE_REQ	  |
	 |		  *==============================>*
(3)	 |	RR_SYNC_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(CHANNEL MODE ACK)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_CHAN_MOD
		ti	TI_0
		chan_desc	CHANNEL_DESC_SDCCH
		chan_mode	CHANNEL_MODE_SPEECH
		}

- MPH_CHANNEL_MODE_REQ
		mode	CHANNEL_MODE_SPEECH
		ch	CH_SDCCH_4_1
		amr_conf	NOT_USED

- RR_SYNC_IND
		ciph	NOT_PRESENT_8BIT
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	NOT_PRESENT_16BIT
		chm	S_CHM_SPEECH_FULL

- DL_DATA_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_CHAN_MOD_ACK
		ti	TI_0
		chan_desc	CHANNEL_DESC_FACCH2
		chan_mode	CHANNEL_MODE_SPEECH
		}

History:	04.07.97	DL	Initial
	26.01.01	DG	DL_DATA_REQ added
	12.02.01	DG	DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2

### RR705: Channel Mode Modify Procedure - AMR

Description:	The base station configures the channel mode. MM and PL are informed and an acknowledge is sent to the base station.

Preamble:	RR154C

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CHANNEL MODE MODIFY)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_CHANNEL_MODE_REQ	  |
	 |		  *==============================>*
(3)	 |	RR_SYNC_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(CHANNEL MODE ACK)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_CHAN_MOD
		ti	TI_0
		chan_desc	CHANNEL_DESC_SDCCH
		chan_mode	CHANNEL_MODE_AMR
		vgcs_tmi	NOT_USED
		multirate_conf	S_multirate_conf_4
		}

- MPH_CHANNEL_MODE_REQ
		mode	CHANNEL_MODE_AMR
		ch	CH_SDCCH_4_1
		amr_conf	S_AMR_conf_4

- RR_SYNC_IND
		ciph	NOT_PRESENT_8BIT
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	NOT_PRESENT_16BIT
		chm	S_CHM_SPEECH_AMR_FULL

- DL_DATA_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_CHAN_MOD_ACK
		ti	TI_0
		chan_desc	CHANNEL_DESC_FACCH2
		chan_mode	CHANNEL_MODE_AMR
		}

History:	04.07.97	DL	Initial

### RR705A: Channel Mode Modify Procedure – AMR (with inconsistent Multirate IE)

Description:	The base station configures the channel mode with inconsistent Multirate IE. RR will ignore the channel mode modify message and shall not send any channel mode modify ack to the base station.

Preamble:	RR154C

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CHANNEL MODE MODIFY)	  |
	 |		  *<==============================*
	 |		  |		  |
	 |		  |                               |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_CHAN_MOD
		ti	TI_0
		chan_desc	CHANNEL_DESC_SDCCH
		chan_mode	CHANNEL_MODE_AMR
		vgcs_tmi	NOT_USED
		multirate_conf	S_multirate_conf_4_702F
                                                                  }

MUTE (100)

History                    23.07.04	xvmadapp	Initial

## Cipher Mode Setting

### RR226: Cipher Mode Setting Procedure

Description:	The base station configures ciphering. MM and PL are informed and an acknowledge is sent to the base station.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CIPHER MODE SETTING)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_CIPHERING_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(CIHPER MODE COMPLETE)	  |
	 |		  *==============================>*
(4)	 |	RR_SYNC_IND	  |		  |
	 *<==============================*		  |
(5)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CIPHER MODE SETTING)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_CIPHERING_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(CIHPER MODE COMPLETE)	  |
	 |		  *==============================>*
(8)	 |	RR_SYNC_IND	  |		  |
	 *<==============================*		  |
(9)	 |	RR_SYNC_REQ	  |		  |
	 *==============================>*		  |
(10)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CIPHER MODE SETTING)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_CIPHERING_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(CIHPER MODE COMPLETE)	  |
	 |		  *==============================>*
(13)	 |	RR_SYNC_IND	  |		  |
	 *<==============================*		  |
(14)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CIPHER MODE SETTING)	  |
	 |		  *<==============================*
(15)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(RR STATUS)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_CIPH_CMD
		ti	TI_0
		ciph_mode_set	CIPH_MODE_ON
		ciph_res	CIPH_RESP_NO_IMEI
		}

- MPH_CIPHERING_REQ
		ciph	CIPHERING_2

- DL_DATA_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_CIPH_COMP
		ti	TI_0
		mob_ident	NOT_USED
		}

- RR_SYNC_IND
		ciph	CIPH_ON
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	NOT_PRESENT_16BIT
		chm	NOT_USED

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_CIPH_CMD
		ti	TI_0
		ciph_mode_set	CIPH_MODE_OFF
		ciph_res	CIPH_RESP_NO_IMEI
		}

- MPH_CIPHERING_REQ
		ciph	NO_CIPHERING

- DL_DATA_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_CIPH_COMP
		ti	TI_0
		mob_ident	NOT_USED
		}

- RR_SYNC_IND
		ciph	CIPH_OFF
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	NOT_PRESENT_16BIT
		chm	NOT_USED

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_6
		kcv	KCV_00112233
		tmsi_struct	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	NOT_PRESENT_16BIT
		accc	ACC_CTRL_CLASS_0000
		thplmn	TIME_HPLMN_EMPTY

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_CIPH_CMD
		ti	TI_0
		ciph_mode_set	CIPH_MODE_ON
		ciph_res	CIPH_RESP_WITH_IMEI
		}

- MPH_CIPHERING_REQ
		ciph	CIPHERING_3

- DL_DATA_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_CIPH_COMP
		ti	TI_0
		mob_ident	MOBILE_IDENTITY_IMEISV
		}

- RR_SYNC_IND
		ciph	CIPH_ON
		mm_info	NOT_USED
		bcch_info	NOT_USED
		synccs	NOT_PRESENT_16BIT
		chm	NOT_USED

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_CIPH_CMD
		ti	TI_0
		ciph_mode_set	CIPH_MODE_ON
		ciph_res	CIPH_RESP_WITH_IMEI
		}

- DL_DATA_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	B_RR_STATUS
		ti	TI_0
		rr_cause	RR_CAUSE_6F
		}

History:	04.07.97	DL	Initial
	31.01.01	DG	DL_DATA_REQ (classmark ch.) added
	12.02.01	DG	DL_DATA_REQ deleted

## Test Functions

### RR300: Close TCH Loop

Description:	The base station closes the TCH loop. Additional close attempts are ignored by RR.
<A>:  Loop Type not C
<B>:  Loop Type is C.

Preamble:	RR210A

Variants:	<A> ... <B>

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CLOSE TCH LOOP CMD)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_TCH_LOOP_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(CLOSE TCH LOOP ACK)	  |
	 |		  *==============================>*
(4)	 |		  |	DL_DATA_IND	  |
	 |		  |	(CLOSE TCH LOOP CMD)	  |
	 |		  *<==============================*
MUTE (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
<A>	sdu	CLOSE_TCH_LOOP_A
<B>	sdu	CLOSE_TCH_LOOP_C

- MPH_TCH_LOOP_REQ
<A>	tch_loop	LOOP_A
<B>	tch_loop	LOOP_C

- DL_DATA_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	CLOSE_TCH_LOOP_ACK

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	CLOSE_TCH_LOOP_A

History:	12.12.97	LE	Initial
	06.03.03	LG	TIMEOUT replaced by MUTE

### RR301: Open TCH Loop (Loop unequal Type C)

Description:	The base station opens the TCH loop. The loop type is unequal type C. So no open loop acknowledge is expected. Additional open commands are ignored by RR.

Preamble:	RR300A

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(OPEN TCH LOOP CMD)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_TCH_LOOP_REQ	  |
	 |		  *==============================>*
TIMEOUT (3000)
(3)	 |		  |	DL_DATA_IND	  |
	 |		  |	(OPEN TCH LOOP CMD)	  |
	 |		  *<==============================*
MUTE (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	OPEN_TCH_LOOP_CMD

- MPH_TCH_LOOP_REQ
		tch_loop	NOT_PRESENT_8BIT

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	OPEN_TCH_LOOP_CMD

History:	12.12.97	LE	Initial
	06.03.03	LG	TIMEOUT replaced by MUTE

### RR302: Open TCH Loop (Loop equal Type C)

Description:	The base station opens the TCH loop. The loop type is equal type C. So an open loop acknowledge is expected. Additional open commands are ignored by RR.

Preamble:	RR300B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(OPEN TCH LOOP CMD)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_TCH_LOOP_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_DATA_REQ	  |
	 |		  |	(OPEN TCH LOOP CMD)	  |
	 |		  *==============================>*
(4)	 |		  |	DL_DATA_IND	  |
	 |		  |	(OPEN TCH LOOP CMD)	  |
	 |		  *<==============================*
MUTE (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	OPEN_TCH_LOOP_CMD

- MPH_TCH_LOOP_REQ
		tch_loop	NOT_PRESENT_8BIT

- DL_DATA_REQ
 		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	OPEN_TCH_LOOP_ACK

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	OPEN_TCH_LOOP_CMD

History:	12.12.97	LE	Initial
	06.03.03	LG	TIMEOUT replaced by MUTE

### RR303: Test Interface

Description:	The base station sends test interface messages. The messages are forwarded to the lower layer.

Preamble:	RR210A

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(TEST INTERFACE)	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_DAI_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_DATA_IND	  |
	 |		  |	(TEST INTERFACE)	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_DAI_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	DL_DATA_IND	  |
	 |		  |	(TEST INTERFACE)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_DAI_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	DL_DATA_IND	  |
	 |		  |	(TEST INTERFACE)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_DAI_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	TEST_INTERFACE_4

- MPH_DAI_REQ
		device	DAI_AD_DA

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	TEST_INTERFACE_2

- MPH_DAI_REQ
		device	DAI_SPEECH_UL

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	TEST_INTERFACE_1

- MPH_DAI_REQ
		device	DAI_SPEECH_DL

- DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	TEST_INTERFACE_0

- MPH_DAI_REQ
		device	DAI_NO_TEST

History:	12.12.97	LE	Initial

## Error Handling

### RR500: Frequency not implemented

Description:	The base station initiates a handover.

<A>	The BCCH arfcn in the cell channel description is not in the GSM range, it is a PCN 
	channel number.
<B> 	The TCH arfcn in the channel description is not in the GSM range, it is a PCN channel 
	number.

This shall lead to handover failure message with the cause frequency not implemented.

Preamble:	RR154B

Variants:	<A>....<B>

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_RECONNECT_REQ	  |
	 |		  |	(HANDOVER FAILURE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
			ch_type	CH_TYPE_SDCCH
			sapi	SAPI_0
			sdu
			{
			component	RR
			direction	DOWNLINK
			pd	D_HANDOV_CMD
			ti	TI_0
<A>		cell_desc	CELL_DESC_BAD_BCCH
<B>		cell_desc	CELL_DESC_1
<A>		chan_desc_after	CHANNEL_DESC_SDCCH2
<B>		chan_desc_after	CHANNEL_DESC_BAD
			handov_ref	HANDOV_REF_1
			pow_cmd_access	POW_05_HO
			synch_ind	NOT_USED
			freq_short_list_after	NOT_USED
			freq_list_after	NOT_USED
			cell_chan_desc	NOT_USED
			chan_mode	NOT_USED
			chan_mode2	NOT_USED
			chan_mode3	NOT_USED
			chan_mode4	NOT_USED
			chan_mode5	NOT_USED
			chan_mode6	NOT_USED
			chan_mode7	NOT_USED
			chan_mode8	NOT_USED
			chan_desc_after_2	NOT_USED
			chan_mode_2	NOT_USED
			freq_chan_seq_after	NOT_USED
			mob_alloc_after	NOT_USED
			start_time	NOT_USED
			time_diff	NOT_USED
			time_advance	NOT_USED
			freq_short_list_before	NOT_USED
			freq_list_before	NOT_USED
			chan_desc_before	NOT_USED
			chan_desc_before_2	NOT_USED
			freq_chan_seq_before	NOT_USED
			mob_alloc_before	NOT_USED
			ciph_mode_set	NOT_USED
			}

- DL_SUSPEND_REQ
			ch_type	CH_TYPE_SDCCH
			sapi	SAPI_0

- DL_RECONNECT_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_HANDOV_FAIL
		ti	TI_0
		rr_cause	RR_CAUSE_0A
		}

History:	16.05.98	LE	Initial
	25.01.01	DG	DL_DATA_IND: chan_mode2…8 added,
				DL_DATA_REQ added
	12.02.01	DG	DL_DATA_REQ deleted

### RR501: Channel mode unacceptable

Description:	The base station initiates a handover.

<A> The channel type in the channel description is halfrate, not fullrate.
<B> The channel mode is not supported

This shall lead to handover failure message with the cause channel mode unacceptable.

Preamble:	RR154B

Variants:	<A>....<B>

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_RECONNECT_REQ	  |
	 |		  |	(HANDOVER FAILURE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
			ch_type	CH_TYPE_SDCCH
			sapi	SAPI_0
			sdu
			{
			component	RR
			direction	DOWNLINK
			pd	D_HANDOV_CMD
			ti	TI_0
			cell_desc	CELL_DESC_1
<A>		chan_desc_after	CHANNEL_DESC_HALFRATE
<B>		chan_desc_after	CHANNEL_DESC_SDCCH2
			handov_ref	HANDOV_REF_1
			pow_cmd_access	POW_05_HO
			synch_ind	NOT_USED
			freq_short_list_after	NOT_USED
			freq_list_after	NOT_USED
			cell_chan_desc	NOT_USED
<A>		chan_mode	NOT_USED
<B>		chan_mode	CHANNEL_MODE_UNDEF
			chan_mode2	NOT_USED
			chan_mode3	NOT_USED
			chan_mode4	NOT_USED
			chan_mode5	NOT_USED
			chan_mode6	NOT_USED
			chan_mode7	NOT_USED
			chan_mode8	NOT_USED
			chan_desc_after_2	NOT_USED
			chan_mode_2	NOT_USED
			freq_chan_seq_after	NOT_USED
			mob_alloc_after	NOT_USED
			start_time	NOT_USED
			time_diff	NOT_USED
			time_advance	NOT_USED
			freq_short_list_before	NOT_USED
			freq_list_before	NOT_USED
			chan_desc_before	NOT_USED
			chan_desc_before_2	NOT_USED
			freq_chan_seq_before	NOT_USED
			mob_alloc_before	NOT_USED
			ciph_mode_set	NOT_USED
			}

- DL_SUSPEND_REQ
			ch_type	CH_TYPE_SDCCH
			sapi	SAPI_0

- DL_RECONNECT_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_HANDOV_FAIL
		ti	TI_0
		rr_cause	RR_CAUSE_09
		}

History:	16.05.98	LE	Initial
	25.01.01	DG	DL_DATA_IND: chan_mode2…8 added,
				DL_DATA_REQ added
	12.02.01	DG	DL_DATA_REQ deleted

## Registration (Limited Service)

### RR600: Cell with unsufficient SYS INFO

Description:	PL detects a BCCH carrier and RR listen to the BCCH messages. The cell has not enough information. After a timeout the detection of the next channel is requested.

Preamble:	RR001A

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(7)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_2
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted, times changed

### RR601: Cell with unreadable SYS INFO

Description:	PL detects a BCCH carrier and RR listen to the BCCH messages. The cell has only unreadable BCCH messages. After 15 errors the detection of the next channel is requested.

Preamble:	RR001A

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
MUTE(500)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	17.04.00	DG	number of "MPH_ERROR_IND" increased
				from 3 to 15
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR602: Second cell is okay

Description:	PL detects a BCCH carrier and RR listen to the BCCH messages.

Preamble:	RR601

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(10)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(11)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_LIMITED
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_EMPTY
		mm_info	MM_INFO_1
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_NO_IMSI_NO_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_6
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	31.05.00	DG	value changed:
				MPH_NEIGHBOURCELL_REQ:
				MPH_NCELL_1_LIM > MPH_NCELL_1
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indic added
				si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR603: No further BCCH detected, enter no service condition

Description:	PL checks the rest of BCCH carrier. There is no further BCCH carrier and RR enters no service condition.

Preamble:	RR601

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
MUTE(500)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_EMPTY_NO_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

History:	04.07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	25.01.01	DG	final MPH_BSIC_REQ added
	12.02.01	DG	final MPH_BSIC_REQ 
				(NOT_PRESENT_16BIT_MIN1) replaced by
				MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR604: Second Cell with unsufficient SYS INFO

Description:	PL detects a BCCH carrier and RR listen to the BCCH messages. The cell has not enough information. After a timeout the detection of the next channel is requested.

Preamble:	RR600

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(5)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
	arfcn	ARFCN_32
	bsic	BSIC_6
	cs	CS_NO_ERROR

- MPH_UNITDATA_IND
	arfcn	ARFCN_32
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_1
	ti	TI_0
	cell_chan_desc	CELL_CHAN_DESC_1
	rach_ctrl	RACH_CTRL_2
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_32
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_3
	ti	TI_0
	cell_ident	CELL_IDENT_3748
	loc_area_ident	LOC_AREA_IDENT_123_2147
	ctrl_chan_desc	CTRL_CHAN_DESC_1
	cell_opt_bcch	CELL_OPT_BCCH_1
	cell_select	CELL_SELECT_1
	rach_ctrl	RACH_CTRL_2
	si3_rest_oct	S_SI3_REST_EMPTY
	}

- MPH_UNITDATA_IND
	arfcn	ARFCN_32
	fn	NOT_USED
	sdu
	{
	component	RR
	direction	DOWNLINK
	pd	D_SYS_INFO_4
	ti	TI_0
	loc_area_ident	LOC_AREA_IDENT_123_2147
	cell_select	CELL_SELECT_1
	rach_ctrl	RACH_CTRL_2
	chan_desc	NOT_USED
	mob_alloc	NOT_USED
	si4_rest_oct	S_SI4_REST_EMPTY
	}

- MPH_BSIC_REQ
	arfcn	ARFCN_124

History:	04.07.97	DL	Initial
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01.	DG	si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted, times changed

### RR605: Second Cell with unreadable SYS INFO

Description:	PL detects a BCCH carrier and RR listen to the BCCH messages. The cell has only unreadable BCCH messages. After three errors the detection of the next channel is requested.

Preamble:	RR601

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_BSIC_REQ
		arfcn	ARFCN_124

History:	04.07.97	DL	Initial
	15.05.00	DG	number of "MPH_ERROR_IND" increased
				from 3 to 15
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR606: Restart by MM for limited service, successful

Description:	MM starts a new attempt for limited service during the actual attempt.

Preamble:	RR602

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(14)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(15)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
MUTE (40000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_EMPTY
		op	OP_MODE_EMPTY
		cksn	CKSN_NOT_PRES
		kcv	KCV_EMPTY
		accc	ACC_CTRL_CLASS_0000
		imsi_struct	MOBILE_ID_NOT_SET
		tmsi_struct	MOBILE_ID_NOT_SET
		thplmn	TIME_HPLMN_EMPTY
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_LIMITED
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_EMPTY
		mm_info	MM_INFO_1
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_NO_IMSI_NO_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_6
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	31.05.00	DG	value changed:
				MPH_NEIGHBOURCELL_REQ:
				MPH_NCELL_1_LIM > MPH_NCELL_1
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indic added
				si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	TIMEOUT replaced by MUTE

### RR607: Restart by MM for limited service fails, no BSIC

Description:	PL detects no further BCCH carrier and RR enters the no service state.

Preamble:	RR602

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_EMPTY
		op	OP_MODE_EMPTY
		cksn	CKSN_NOT_PRES
		kcv	KCV_EMPTY
		accc	ACC_CTRL_CLASS_0000
		imsi_struct	MOBILE_ID_NOT_SET
		tmsi_struct	MOBILE_ID_NOT_SET
		thplmn	TIME_HPLMN_EMPTY
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_EMPTY_NO_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

History:	04.07.97	DL	Initial

15.05.00	DG	RR_ACTIVATE_REQ: cell_test added

19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
MPH_BSIC_REQ: cell_type, timing_info
added

09.02.01	DG	MPH_BSIC_REQ (NOT_PRESENT_16BIT_MIN1) replaced by MPH_SYNC_REQ

31.03.01	VK	'power' added in rr_abort_ind

05-Jun-01	MSE	adapted to TAP2

16-Nov-01	VK	remove step 7-13 (repeated sync attempt to ARFCN 43 and 124)

25.02.03	LG	'lac_list' added in rr_abort_ind

### RR608: Restart by MM for limited service fails, insufficient SYS INFOS

Description:	PL detects a further BCCH carrier with insufficient SYS INFOS. The detection of the next carrier is started.

Preamble:	RR602

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(9)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_EMPTY
		op	OP_MODE_EMPTY
		cksn	CKSN_NOT_PRES
		kcv	KCV_EMPTY
		accc	ACC_CTRL_CLASS_0000
		imsi_struct	MOBILE_ID_NOT_SET
		tmsi_struct	MOBILE_ID_NOT_SET
		thplmn	TIME_HPLMN_EMPTY
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_2
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted, times changed

### RR609: Restart by MM for limited service fails, unreadable SYS INFOS

Description:	PL detects a further BCCH carrier with unreadable SYS INFOs. The detection of the next carrier is started.

Preamble:	RR602

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(21)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_EMPTY
		op	OP_MODE_EMPTY
		cksn	CKSN_NOT_PRES
		kcv	KCV_EMPTY
		accc	ACC_CTRL_CLASS_0000
		imsi_struct	MOBILE_ID_NOT_SET
		tmsi_struct	MOBILE_ID_NOT_SET
		thplmn	TIME_HPLMN_EMPTY
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	18.04.00	DG	number of "MPH_ERROR_IND" increased
				from 3 to 15
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	05-Jun-01	MSE	adapted to TAP2

### RR618: Restart by MM for full service, successful, with bcch info

Description:	MM starts a new attempt for full service. The request contains BCCH information from the SIM card. The attempt is successful. The channel ARFCN_32 shall be used. It is member of the BCCH information.

Preamble:	RR601

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
MUTE (3000)
(2)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(15)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(16)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
	plmn	PLMN_ID_123
	op	OP_MODE_TEST_SIM
	cksn	CKSN_NOT_PRES
	kcv	KCV_12345678
	accc	ACC_CTRL_CLASS_0008
	imsi_struct	MOBILE_ID_IMSI_HPLMN
	tmsi_struct	MOBILE_ID_TMSI
	thplmn	TIME_HPLMN_VALID
	bcch_info	S_BCCH_INFO_32
	cell_test	CELL_TEST_DISABLE
	gprs_indication	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_POWER_REQ

- pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
	classmark	CLASS_MS

- MPH_IDLE_REQ
	mod	MODE_CELL_SELECTION
	arfcn	ARFCN_32
	ext_bcch	NOT_USED
	comb_ccch	CCD_CCCH_1_NOT_COMB
	tn	TN_0
	dlt	DLT_23
	pg	PG_0
	bs_ag_blocks_res	BS_AG_BLKS_RES_5
	bs_pa_mfrms	BS_PA_MFRMS_2
	power	MS_TXPWR_MAX_CCH_02
	ncc_permitted	NCC_PERMITTED_1
	reorg_only	NOT_USED
	eotd_avail	CONST_0
	gprs_support	NOT_USED

- MPH_CBCH_REQ
	cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1
	sync_only	NOT_USED

- RR_ACTIVATE_CNF
	op	OP_MODE_TEST_SIM
	mm_info	MM_INFO_2
	cid	CELL_IDENT_3748
	plmn	PLMN_ID_123
	lac	LAC_2147
	power	NOT_USED
	gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
	mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	17.04.00	DG	value changed:
			MPH_IDLE_REQ/ext_bcch:
			EXT_BCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
			si3/4_rest_oct:
			NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
			class changed into classmark
			(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
			MPH_BSIC_REQ: cell_type,
 			timing_info added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indic added
			si3/4_rest_oct: SI3_REST_DEF > 
			SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	TIMEOUT replaced by MUTE

### RR619: Restart by MM for full service, successful, without bcch info

Description:	MM starts a new attempt for full service. The request contains no BCCH information from the SIM card. The attempt is successful. The channel ARFCN_67 shall be used. It is the channel with the highest fieldstrength.

Preamble:	RR601

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(15)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(16)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
	plmn	PLMN_ID_123
	op	OP_MODE_TEST_SIM
	cksn	CKSN_NOT_PRES
	kcv	KCV_12345678
	accc	ACC_CTRL_CLASS_0008
	imsi_struct	MOBILE_ID_IMSI_HPLMN
	tmsi_struct	MOBILE_ID_TMSI
	thplmn	TIME_HPLMN_VALID
	bcch_info	S_BCCH_INFO_EMPTY
	cell_test	CELL_TEST_DISABLE
	gprs_indication	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_POWER_REQ

- pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
	classmark	CLASS_MS

- MPH_IDLE_REQ
	mod	MODE_CELL_SELECTION
	arfcn	ARFCN_67
	ext_bcch	NOT_USED
	comb_ccch	CCD_CCCH_1_NOT_COMB
	tn	TN_0
	dlt	DLT_23
	pg	PG_0
	bs_ag_blocks_res	BS_AG_BLKS_RES_5
	bs_pa_mfrms	BS_PA_MFRMS_2
	power	MS_TXPWR_MAX_CCH_02
	ncc_permitted	NCC_PERMITTED_1
	reorg_only	NOT_USED
	eotd_avail	CONST_0
	gprs_support	NOT_USED

- MPH_CBCH_REQ
	cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1C
	sync_only	NOT_USED

- RR_ACTIVATE_CNF
	op	OP_MODE_TEST_SIM
	mm_info	MM_INFO_2
	cid	CELL_IDENT_3748
	plmn	PLMN_ID_123
	lac	LAC_2147
	power	NOT_USED
	gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
	mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	17.04.00	DG	value changed:
			MPH_IDLE_REQ/ext_bcch:
			EXT_BCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
			si3/4_rest_oct:
			NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
			class changed into classmark
			(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
			MPH_BSIC_REQ: cell_type, 				timing_info added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indic added
			si3/4_rest_oct: SI3_REST_DEF > 			SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR620: Restart by MM for full service fails, no BSIC

Description:	PL detects no further BCCH carrier and RR enters the no service state.

Preamble:	RR601

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_BSIC_CNF
			arfcn	ARFCN_32
			bsic	BSIC_0
			cs	CS_NO_ERROR

- MPH_POWER_REQ
			pch_interrupt	PCH_INTERRUPT
			freq_bands	NOT_USED

- MPH_POWER_CNF
			num_of_chan	CHANNELS_3
			arfcn	ARFCN_67_32_124
			rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
			arfcn	ARFCN_67

- MPH_BSIC_CNF
			arfcn	ARFCN_67
			bsic	BSIC_0
			cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
			arfcn	ARFCN_32

- MPH_BSIC_CNF
			arfcn	ARFCN_32
			bsic	BSIC_0
			cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
			arfcn	ARFCN_124

- MPH_BSIC_CNF
			arfcn	ARFCN_124
			bsic	BSIC_0
			cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
			op	OP_MODE_TEST_SIM_NO_SERV
			cause	RRCS_ABORT_CEL_SEL_FAIL
			plmn_avail	NO_PLMN_AVAILABLE
			plmn	NOT_USED
			lac_list	NOT_USED
			rxlevel	NOT_USED
			power	NOT_USED

History:	04.07.97	DL	Initial
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	29.01.01	DG	final MPH_BSIC_REQ 
				(NOT_PRESENT_16BIT_MIN1) added
	13.02.01	DG	final MPH_BSIC_REQ replaced by
				MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR621: Restart by MM for full service fails, insufficient SYS INFOS

Description:	PL detects a further BCCH carrier with insufficient SYS INFOS. The detection of the next carrier is started.

Preamble:	RR601

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(10)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_2
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted, time changed

### RR622: Restart by MM for full service fails , unreadable SYS INFOS

Description:	PL detects a further BCCH carrier with unreadable SYS INFOs. The detection of the next carrier is started.

Preamble:	RR601

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(21)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(22)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(23)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
	plmn	PLMN_ID_123
	op	OP_MODE_TEST_SIM
	cksn	CKSN_NOT_PRES
	kcv	KCV_12345678
	accc	ACC_CTRL_CLASS_0008
	imsi_struct	MOBILE_ID_IMSI_HPLMN
	tmsi_struct	MOBILE_ID_TMSI
	thplmn	TIME_HPLMN_VALID
	bcch_info	S_BCCH_INFO_EMPTY
	cell_test	CELL_TEST_DISABLE
	gprs_indication	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	17.04.00	DG	number of "MPH_ERROR_IND" increased
				from 3 to 15
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	05-Jun-01	MSE	adapted to TAP2

### RR623: Restart by MM for full service fails, limited service available

Description:	PL detects a BCCH carrier and RR listen to the BCCH messages. The cell has a wrong PLMN identification. The third cell is not available. RR selects the second cell for limited service.

Preamble:	RR622

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(17)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(18)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(19)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_LIMITED
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1
		sync_only	NOT_USED

- RR_ABORT_IND
			op	OP_MODE_TEST_SIM_LIM_SERV
			cause	RRCS_ABORT_CEL_SEL_FAIL
			plmn_avail	ONE_PLMN_AVAILABLE
			plmn	AS_PLMN
			lac_list	A_LAC_LIST1
			rxlevel	A_RXLEVEL_21
			power	NOT_USED

- MPH_IDENTITY_REQ
		mid	S_MS_ID_NO_IMSI_NO_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_6
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	31.05.00	DG	value changed:
				MPH_NEIGHBOURCELL_REQ:
				MPH_NCELL_1_LIM > MPH_NCELL_1
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	12.02.01	DG	new: MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR624: Restart by RR for full service is successful

Description:	MM has requested a full service search and RR is in limited service. After timeout of the registration timer RR starts the full service search again. The search is successful and MM is informed.

Preamble:	RR623

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT (1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(17)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(18)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
(19)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_PRESENT_8BIT

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	07.03.00	DG	RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indic added
				si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE/TIMEOUT inserted

### RR625: Restart by RR for full Service fails, no BSIC

Description:	In the preamble MM has requested full service. This fails and the limited service was indicated to MM. After 10 seconds RR starts a new registration attempt from limited service to full service. This fails, because the next carrier is no BCCH. Checking of the next carrier is started. After checking all BCCHs RR enters no service.

Preamble:	RR623

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT (1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(10)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM_NO_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

History:	04.07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	12.02.01	DG	new: MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	25.02.03	LG	'lac_list' added in rr_abort_ind
	06.03.03	LG	MUTE inserted

### RR617: Restart by RR for full Service fails, SC is read again

Description:	In the preamble MM has requested full service. This fails and the limited service was indicated to MM. After 10 seconds RR starts a new registration attempt from limited service to full service. This fails, because the next carrier is no BCCH. Checking of the next carrier is started. After checking all BCCHs RR enters no service.

Preamble:	RR623

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT (1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPH_SYNC_REQ
		cs	CS_STOP_BCCH_READING

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM_LIM_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	AS_PLMN
		lac_list	A_LAC_LIST1
		rxlevel	A_RXLEVEL_21
		power	NOT_USED

History:	04.07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	12.02.01	DG	new: MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	25.02.03	LG	'lac_list' added in rr_abort_ind
	06.03.03	LG	MUTE inserted

### RR626: Restart by RR for full Service fails, insufficient SYS INFOS

Description:	In the preamble MM has requested full service. This fails and the limited service was indicated to MM. After 10 seconds RR starts a new registration attempt from limited service to full service. This fails, because the checked carrier has insufficient SYS INFOS. Checking of the next carrier is started.

Preamble:	RR623

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT (1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(7)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted, time changed

### RR627: Restart by RR for full Service fails, unreadable SYS INFOS

Description:	In the preamble MM has requested full service. This fails and the limited service was indicated to MM. After 10 seconds RR starts a new registration attempt from limited service to full service. This fails, because the checked carrier has unreadable SYS INFOS. Checking of the next carrier is started.

Preamble:	RR623

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT (1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	17.04.00	DG	number of "MPH_ERROR_IND" increased
				from 3 to 15
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	15.02.01	DG	number of "MPH_ERROR_IND" reduced
				from 15 to 4
	06.03.03	LG	MUTE inserted

## Registration (No Service)

### RR610: Restart by RR for limited Service, successful

Description:	In the preamble MM has requested limited service. This fails and the no service was indicated to MM. After 10 seconds RR starts a new registration attempt from no service to limited service. This is successful. The limited mode is configured, MM is informed about limited service and RR enters the idle mode.

Preamble:	RR603

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT(1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(13)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
(14)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_LIMITED
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_EMPTY
		mm_info	MM_INFO_1
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_NO_IMSI_NO_TMSI

History:	04.07.97	DL	Initial
	06.03.00	DG	RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_6
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	31.05.00	DG	value changed:
				MPH_NEIGHBOURCELL_REQ:
				MPH_NCELL_1_LIM > MPH_NCELL_1
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indic added
				si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted

### RR611: Restart by RR for limited Service fails, no BSIC

Description:	In the preamble MM has requested limited service. This fails and the no service was indicated to MM. After 10 seconds RR starts a new registration attempt from no service to limited service. This fails, because the next carrier is no BCCH. Checking of the next carrier is started.

Preamble:	RR603

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT(1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	06.03.03	LG	MUTE inserted

### RR612: Restart by RR for limited Service fails, insufficient SYS INFOS

Description:	In the preamble MM has requested limited service. This fails and the no service was indicated to MM. After 10 seconds RR starts a new registration attempt from no service to limited service. This fails, because the checked carrier has insufficient SYS INFOS. Checking of the next carrier is started.

Preamble:	RR603

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT (1000)
 (1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted, time changed

### RR613: Restart by RR for limited Service fails, unreadable SYS INFOS

Description:	In the preamble MM has requested limited service. This fails and the no service was indicated to MM. After 10 seconds RR starts a new registration attempt from no service to limited service. This fails, because the checked carrier has unreadable SYS INFOS. Checking of the next carrier is started.

Preamble:	RR603

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT (1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	18.04.00	DG	number of "MPH_ERROR_IND" increased
				from 3 to 15
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	06.03.03	LG	MUTE inserted

### RR615: Restart by MM for limited service fails, no BSIC

Description:	During no service condition MM starts a new attempt for limited service. PL detects no further BCCH carrier and RR enters the no service state.

Preamble:	RR603

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_EMPTY
		op	OP_MODE_EMPTY
		cksn	CKSN_NOT_PRES
		kcv	KCV_EMPTY
		accc	ACC_CTRL_CLASS_0000
		imsi_struct	MOBILE_ID_NOT_SET
		tmsi_struct	MOBILE_ID_NOT_SET
		thplmn	TIME_HPLMN_EMPTY
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_EMPTY_NO_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

History:	04.07.97	DL	Initial
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	15.02.01	DG	all MPH_BSIC_REQ/CNF deleted
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR628: Restart by MM for full service, successful, with bcch info

Description:	MM starts a new attempt for full service. The request contains BCCH information from the SIM card. The attempt is successful. The channel ARFCN_32 shall be used. It is member of the BCCH information.

Preamble:	RR615

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(14)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(15)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_32
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ	pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
			num_of_chan	CHANNELS_3
			arfcn	ARFCN_67_32_124
			rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	07.03.00	DG	MPH_POWER_REQ/_CNF, RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indic added
				si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR629: Restart by MM for full service, successful, without bcch info

Description:	MM starts a new attempt for full service. The request contains no BCCH information from the SIM card. The attempt is successful. The channel ARFCN_67 shall be used. It is the channel with the highest fieldstrength.

Preamble:	RR615

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(14)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(15)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	03.04.00	DG	deleted:
				(1) RR_ACTIVATE_REQ,
				(2)MPH_BSIC_REQ
				values changed:
				(1-5,7) ARFCN_67 > ARFCN_32,
				(7)DLT_23>DLT_LIMITED,
				BS_PA_MFRMS_2>BS_PA_MFRMS_7,
				NCC_PERMITTED_1>NCC_PERMITTED_FF
				(9)MPH_NCELL_1>MPH_NCELL_1_LIM,
				added:
				(10)RR_SYNC_IND
				(11)OP_MODE_TEST_SIM>
				OP_MODE_EMPTY,
				NOT_USED>POW_01_DG_629
				(12)MS_ID_IMSI_HPLMN_TMSI>
				MS_ID_NO_IMSI_NO_TMSI
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	31.05.00	DG	value changed:
				MPH_NEIGHBOURCELL_REQ:
				MPH_NCELL_1_LIM > MPH_NCELL_1
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indic added
				si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR630: Restart by MM for full service fails, no BSIC

Description:	PL detects no further BCCH carrier and RR enters the no service state.

Preamble:	RR615

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM_NO_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

History:	04.07.97	DL	Initial
	07.03.00	DG	MPH_POWER_REQ/_CNF, values changed
				(6)/(7) <> (8)/(9)
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	16.02.01	DG	new: MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR631: Restart by MM for full service fails, insufficient SYS INFOS

Description:	PL detects a further BCCH carrier with insufficient SYS INFOS. The detection of the next carrier is started.

Preamble:	RR615

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(9)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_2
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	07.03.00	DG	MPH_POWER_REQ/_CNF,
				(9) value changed 124 > 20
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted, time changed

### RR632: Restart by MM for full service fails, unreadable SYS INFOS

Description:	PL detects a further BCCH carrier with unreadable SYS INFOs. The detection of the next carrier is started.

Preamble:	RR613

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(2)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

History:	04.07.97	DL	Initial
	27.03.00	DG	new:
				(1) MPH_BSIC_CNF
				(8)/(9) MPH_POWER_REQ/CNF
				values changed:
				(3)/(4) ARFCN_67>ARFCN_124
				(10)ARFCN_124>ARFCN_67
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	05-Jun-01	MSE	adapted to TAP2

### RR633: Restart by RR for full service is successful

Description:	MM has requested a full service search and RR is in no service. After timeout of the registration timer RR starts the full service search again. The search is successful and MM is informed.

Preamble:	RR625

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT (1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(13)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
(14)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1E
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	07.03.00	DG	RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indic added
				si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE/TIMEOUT inserted

### RR634: Restart by RR for full Service fails, no BSIC

Description:	In the preamble MM has requested full service. This fails and the no service was indicated to MM. After 10 seconds RR starts a new registration attempt from no service to full service. This fails, because the next carrier is no BCCH. Checking of the next carrier is started.

Preamble:	RR625

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT(1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

History:	04.07.97	DL	Initial
	08.03.00	DG	(5) value changed (124 > 20)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	06.03.03	LG	MUTE inserted

### RR635: Restart by RR for full Service fails, insufficient SYS INFOS

Description:	In the preamble MM has requested full service. This fails and the no service was indicated to MM. After 10 seconds RR starts a new registration attempt from no service to full service. This fails, because the checked carrier has insufficient SYS INFOS. Checking of the next carrier is started.

Preamble:	RR625

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT (1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_124

History:	04.07.97	DL	Initial
	08.03.00	DG	(8) value changed (124 > 20)
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE/TIMEOUT inserted, time changed

### RR636: Restart by RR for full Service fails, unreadable SYS INFOS

Description:	In the preamble MM has requested full service. This fails and the no service was indicated to MM. After 10 seconds RR starts a new registration attempt from no service to full service. This fails, because the checked carrier has unreadable SYS INFOS. Checking of the next carrier is started.

Preamble:	RR625

MM		 RR		PL/DL
	 |		  |		  |
MUTE (9000)
TIMEOUT(1000)
(1)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(2)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_BSIC_REQ
		arfcn	ARFCN_124

History:	04.07.97	DL	Initial
	08.03.00	DG	(8) value changed (124 > 20)
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added
	06.03.03	LG	MUTE inserted

## Registration (Full Service)

### RR637: Start by MM for full service, successful, with bcch info info (used as preamble only)

Description:	MM starts an attempt for full service. The request contains BCCH information from the SIM card. The attempt is successful. The channel ARFCN_32 shall be used. It is member of the BCCH information.
All stored BCCH should be cleared at the end of this tc for appropriate use as preamble of RR642-RR650.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1) 
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |	RR_SYNC_IND	  |		  |
	 |	(BCCH info)	  |		  |
	 *<==============================*		  |
(11)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(15)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(16)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
COMMAND (RR CONFIG CLEAR_BCCH_INFO)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_32
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- RR_SYNC_IND	ciph	NOT_PRESENT_8BIT
		mm_info	NOT_USED
		bcch_info	S_BCCH_INFO_NCELL_DESC_1
		synccs	NOT_PRESENT_16BIT
		chm	NOT_USED

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	07.03.00	DG	RR_SYNC_IND
	13.04.00	LE (DG)	MPH_IDLE_REQ/ext_bcch:
			EXT_BCCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
			si3/4_rest_oct:
			NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
			class changed into classmark
			(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indic added
			MPH_BSIC_REQ: cell_type,
			timing_info added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
			si3/4_rest_oct: SI3_REST_DEF > 			SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR638: Restart by MM for full service, successful, without bcch info

Description:	MM starts a new attempt for full service. The request contains no BCCH information from the SIM card. The attempt is successful. The channel ARFCN_67 shall be used. It is the channel with the highest fieldstrength.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(14)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(15)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	07.03.00	DG	RR_SYNC_IND
	14.04.00	DG	value changed:
			MPH_IDLE_REQ/ext_bcch:
			EXT_BCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
			si3/4_rest_oct:
			NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
			class changed into classmark
			(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
			MPH_BSIC_REQ: cell_type, 				timing_info added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
			si3/4_rest_oct: SI3_REST_DEF > 			SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR639: Restart by MM for full service fails, no BSIC

Description:	PL detects no further BCCH carrier and RR enters the no service state.

Preamble:	RR000

Variants:	<A>....<B>

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(12)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(13)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
<A>		op	OP_MODE_TEST_SIM
<B>		op	OP_MODE_NORMAL
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
<A>	op	OP_MODE_TEST_SIM_NO_SERV
<B>	op	OP_MODE_NORMAL_NO_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
<A>	op	OP_MODE_TEST_SIM
<B>	op	OP_MODE_NORMAL
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

History:	04.07.97	DL	Initial

07.03.00	DG	ARFCN-values changed (6)/(7) <> (8)/(9)

15.05.00	DG	RR_ACTIVATE_REQ: cell_test added

19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added MPH_BSIC_REQ: cell_type, timing_infoadded

31.01.01	DG	final MPH_BSIC_REQ (NOT_PRESENT_16BIT_MIN1) added

13.02.01	DG	final MPH_BSIC_REQ replaced by MPH_SYNC_REQ

31.03.01	VK	'power' added in rr_abort_ind

16-Nov-01	VK	Add variant B, add second RR_ACTIVATE_REQ

25.02.03	LG	'lac_list' added in rr_abort_ind

### RR640: Restart by MM for full service fails, insufficient SYS INFOS

Description:	PL detects a BCCH carrier with insufficient SYS INFOS. The detection of the next carrier is started.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(9)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(21)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(24)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(25)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(26)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_2
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_2
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_124
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_2
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_1
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	RF_CLASS_4
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	23.05.00	DG	RR_ACTIVATE_REQ: cell_test added
			Values changed:
			(9)MPH_BSIC_REQ to
			(14)MPH_UNITDATA_IND:
			ARFCN124 > ARFCN20
			(18)-(20)MPH_UNITDATA_IND:
			NCELL_DESC_1 > NCELL_DESC_2,
			LOC_AREA_IDENT_122_2147 >
			LOC_AREA_IDENT_123_2147
			siX_rest_oct: NOT_USED >
			SIX_REST_DEF
			MPH_IDLE_REQ:
			EXT_BCCH_NOT_LIST > BSIC_6, 
			DLT_LIMITED > DLT_23,
			BS_PA_MFRMS_7 >BS_PA_MFRMS_2;
			NCC_PERMITTED_FF > NCC_PERMITTED_1			MPH_NEIGHBOURCELL_REQ:
			MPH_NCELL_1_LIM > MPH_NCELL_2
			MPH_IDENTITY_REQ:
			MS_ID_NO_IMSI_NO_TMSI >
			MS_ID_IMSI_HPLMN_TMSI
			new:
			RR_SYNC_IND, RR_ACTIVATE_CNF
	26.05.00	DG	values changed:
			si3/4_rest_oct: NOT_USED >
			SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
			class changed into classmark
			(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
			MPH_BSIC_REQ: cell_type, timing_info
			added
	30.01.01	DG	values changed:
			last bunch of Sysinfos3/4:
			SI3_REST_DEF -> SI3_REST_OPT_SEL_PAR
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
			si3/4_rest_oct: SI3_REST_OPT_SEL_PAR >
			SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE/TIMEOUT inserted

### RR641: Restart by MM for full service fails, unreadable SYS INFOS

Description:	PL detects a BCCH carrier with unreadable SYS INFOs. The detection of the next carrier is started. At the end of the testcase RR enters the limited service.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(21)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	07.03.00	DG	value changed: (9) 124 > 20
	18.04.00	DG	number of "MPH_ERROR_IND" increased
				from 3 to 15
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	05-Jun-01	MSE	adapted to TAP2

### RR642: Restart by MM for full service, successful, with bcch info

Description:	MM starts a new attempt for full service. The request contains BCCH information from the SIM card. The attempt is successful. The channel ARFCN_32 shall be used. It is member of the BCCH information.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(12)	 |	RR_SYNC_IND	  |		  |
	 |	(BCCH Info)	  |		  |
	 *<==============================*		  |
(13)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(17)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(18)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_122
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_32
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147_V
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147_V
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- RR_SYNC_IND
		ciph	NOT_PRESENT_8BIT
		mm_info	NOT_USED
		bcch_info	S_BCCH_INFO_NCELL_DESC_1
		synccs	NOT_PRESENT_16BIT
		chm	NOT_USED

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_122
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial	
	16.05.00	DG	RR_ACTIVATE_REQ:
			cell_test added
			value changed:
			MPH_IDLE_REQ/ext_bcch:
			EXT_BCCH_NOT_LIST > BSIC_5
			new: RR_SYNC_IND
	26.05.00	DG	values changed:
			si3/4_rest_oct:
			NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
			class changed into classmark
			(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
			MPH_BSIC_REQ: cell_type, 
			timing_info added
	30.01.01	DG	values changed:
			Sysinfos3/4:
			SI3_REST_DEF -> 
			SI3_REST_OPT_SEL_PAR
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
			si3/4_rest_oct: 					SI3_REST_OPT_SEL_PAR >
			SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted, time changed

### RR643: Restart by MM for full service, successful, without bcch info

Description:	MM starts a new attempt for full service. The request contains no BCCH information from the SIM card. The attempt is successful. The channel ARFCN_67 shall be used. It is the channel with the highest fieldstrength.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |	RR_SYNC_IND	  |		  |
	 |	(BCCH Info)	  |		  |
	 *<==============================*		  |
(11)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(15)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(16)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
	plmn	PLMN_ID_122
	op	OP_MODE_TEST_SIM
	cksn	CKSN_NOT_PRES
	kcv	KCV_12345678
	accc	ACC_CTRL_CLASS_0008
	imsi_struct	MOBILE_ID_IMSI_HPLMN
	tmsi_struct	MOBILE_ID_TMSI
	thplmn	TIME_HPLMN_VALID
	bcch_info	S_BCCH_INFO_EMPTY
	cell_test	CELL_TEST_DISABLE
	gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147_V
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147_V
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- RR_SYNC_IND
	ciph	NOT_PRESENT_8BIT
	mm_info	NOT_USED
	bcch_info	S_BCCH_INFO_NCELL_DESC_1
	synccs	NOT_PRESENT_16BIT
	chm	NOT_USED

- MPH_CLASSMARK_REQ
	classmark	CLASS_MS

- MPH_IDLE_REQ
	mod	MODE_CELL_SELECTION
	arfcn	ARFCN_67
	ext_bcch	NOT_USED
	comb_ccch	CCD_CCCH_1_NOT_COMB
	tn	TN_0
	dlt	DLT_23
	pg	PG_0
	bs_ag_blocks_res	BS_AG_BLKS_RES_5
	bs_pa_mfrms	BS_PA_MFRMS_2
	power	MS_TXPWR_MAX_CCH_02
	ncc_permitted	NCC_PERMITTED_1
	reorg_only	NOT_USED
	eotd_avail	CONST_0
	gprs_support	NOT_USED

- MPH_CBCH_REQ
	cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
	multi_band	NOT_USED
	arfcn	A_MPH_NCELL_1C
	sync_only	NOT_USED

- RR_ACTIVATE_CNF
	op	OP_MODE_TEST_SIM
	mm_info	MM_INFO_2
	cid	CELL_IDENT_3748
	plmn	PLMN_ID_122
	lac	LAC_2147
	power	NOT_USED
	gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
	mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	23.03.00	DG	new:
			(2)/(3):MPH_POWER_REQ/CNF,
			(14) RR_SYNC_IND
	14.04.00	DG	value changed:
			MPH_IDLE_REQ/ext_bcch:
			EXT_BCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
			si3/4_rest_oct:
			NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
			class changed into classmark
			(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
			MPH_BSIC_REQ: cell_type, 
			timing_info added
	30.01.01	DG	values changed:
			Sysinfos3/4:
			SI3_REST_DEF ->
			SI3_REST_OPT_SEL_PAR
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
			si3/4_rest_oct: 
			SI3_REST_OPT_SEL_PAR > 
			SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2

### RR644: Restart by MM for full service fails, no BSIC

Description:	PL detects no further BCCH carrier and RR enters the no service state.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_122
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM_NO_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

History:	04.07.97	DL	Initial
	23.03.00	DG	new:
				(2)/(3)MPH_POWER_REQ/CNF
				values changed:
				(6)/(7) ARFCN_124>ARFCN_32,
				(8)/(9)ARFCN_32>ARFCN_124
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	31.01.01	DG	final MPH_BSIC_REQ
				(NOT_PRESENT_16BIT_MIN1) added
	13.02.01	DG	final MPH_BSIC_REQ replaced by
				MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR645: Restart by MM for full service fails, insufficient SYS INFOS

Description:	PL detects a further BCCH carrier with insufficient SYS INFOS. The detection of the next carrier is started.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(9)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_122
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_2
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	23.03.00	DG	new: (2)/(3)MPH_POWER_REQ/CNF
				value changed: (9) ARFCN_124>ARFCN_32
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted, time changed

### RR646: Restart by MM for full service fails, unreadable SYS INFOS

Description:	PL detects a further BCCH carrier with unreadable SYS INFOs. The detection of the next carrier is started.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(21)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_122
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_67

- MPH_BSIC_REQ
		arfcn	ARFCN_32

History:	04.07.97	DL	Initial
	23.03.00	DG	new: (2)/(3)MPH_POWER_REQ/CNF
				value changed: (9) ARFCN_124>ARFCN_32
	18.04.00	DG	number of "MPH_ERROR_IND" increased
				from 3 to 15
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	05-Jun-01	MSE	adapted to TAP2

### RR647: Restart by MM for limited service, successful

Description:	MM starts a new attempt for limited service during full service.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(16)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(17)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_EMPTY
		op	OP_MODE_EMPTY
		cksn	CKSN_NOT_PRES
		kcv	KCV_EMPTY
		accc	ACC_CTRL_CLASS_0000
		imsi_struct	MOBILE_ID_NOT_SET
		tmsi_struct	MOBILE_ID_NOT_SET
		thplmn	TIME_HPLMN_EMPTY
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_32
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_LIMITED
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_EMPTY
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_NO_IMSI_NO_TMSI

History:	04.07.97	DL	Initial
	24.03.00	DG	values changed:
				(2)/(3) ARFCN_32 > ARFCN_67,
				(4) NOT_PRESENT_16BIT >ARFCN_32
				new:
				(5)/(6) MPH_BSIC_CNF/REQ, 					(11) RR_SYNC_IND
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	16.05.00	DG	value changed: MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	05.06.00	DG	TIMEOUT added
				MPH_NEIGHBOURCELL_REQ:
				value changed:
				MPH_NCELL_1_LIM>MPH_NCELL_EMPTY
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_CNF: gprs_indication added
	19.02.01	DG	(6)MPH_BSIC_REQ(NOT_PRESENT_16BIT)
				replaced by MPH_SYNC_REQ
	05-Jun-01	MSE	adapted to TAP2

### RR648: Restart by MM for limited service fails, no BSIC

Description:	PL detects no further BCCH carrier and RR enters the no service state. is buggy

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_EMPTY
		op	OP_MODE_EMPTY
		cksn	CKSN_NOT_PRES
		kcv	KCV_EMPTY
		accc	ACC_CTRL_CLASS_0000
		imsi_struct	MOBILE_ID_NOT_SET
		tmsi_struct	MOBILE_ID_NOT_SET
		thplmn	TIME_HPLMN_EMPTY
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_EMPTY_NO_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

History:	04.07.97	DL	Initial
	08.03.00	DG	values changed:
				(2)/(3) ARFCN_32 > ARFCN_67
				(4)/(5) ARFCN_67 > ARFCN_32
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	31.01.01	DG	final MPH_BSIC_REQ 
				(NOT_PRESENT_16BIT_MIN1) added
	13.02.01	DG	final MPH_BSIC_REQ replaced by
				MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR649: Restart by MM for limited service fails, insufficient SYS INFOS

Description:	PL detects a further BCCH carrier with insufficient SYS INFOS. The detection of the next carrier is started.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT (1000)
(11)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_EMPTY
		op	OP_MODE_EMPTY
		cksn	CKSN_NOT_PRES
		kcv	KCV_EMPTY
		accc	ACC_CTRL_CLASS_0000
		imsi_struct	MOBILE_ID_NOT_SET
		tmsi_struct	MOBILE_ID_NOT_SET
		thplmn	TIME_HPLMN_EMPTY
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_2
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_2
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_124

History:	04.07.97	DL	Initial
	08.03.00	DG	values changed:
				(2)/(3) ARFCN_32 > ARFCN_67
				(4)/(5) ARFCN_67 > ARFCN_32
				(9) ARFCN_124 > NOT_PRESENT_16BIT
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF >
				SI3_REST_EMPTY
	15.02.01	DG	sysinfos: ARFCN_67 > ARFCN_32,
				last MPH_BSIC_REQ/NOT_PRESENT_16BIT
				replaced by MPH_SYNC_REQ
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE/TIMEOUT inserted

### RR650: Restart by MM for limited service fails, unreadable SYS INFOS

Description:	PL detects a further BCCH carrier with unreadable SYS INFOs. The detection of the next carrier is started.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
MUTE (9000)
TIMEOUT(1000)
(11)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_EMPTY
		op	OP_MODE_EMPTY
		cksn	CKSN_NOT_PRES
		kcv	KCV_EMPTY
		accc	ACC_CTRL_CLASS_0000
		imsi_struct	MOBILE_ID_NOT_SET
		tmsi_struct	MOBILE_ID_NOT_SET
		thplmn	TIME_HPLMN_EMPTY
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_32

- MPH_BSIC_REQ
		arfcn	ARFCN_124

History:	04.07.97	DL	Initial
	08.03.00	DG	values changed:
				(2)/(3) ARFCN_32 > ARFCN_67
				(4)/(5) ARFCN_67 > ARFCN_32
				(9) ARFCN_124 > NOT_PRESENT_16BIT
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	14.02.01	DG	final MPH_BSIC_REQ
				(NOT_PRESENT_16BIT) replaced by
				MPH_SYNC_REQ
	05-Jun-01	MSE	adapted to TAP2
	06.03.03	LG	MUTE inserted

## Registration (Network Search)

### RR657: No Service, Network Search requested by MMI, no PLMNs

Description:	PL searches for the available PLMNs. It will not detect any PLMN. After time-out of the registration timer a new attempt for full service shall be started by RR.

Preamble:	RR630

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(11)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
MUTE (9000)
TIMEOUT(1000)
(12)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(21)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(24)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
(25)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI_NO_SRV
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI_NO_SRV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	NO_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	NOT_USED
		rxlevel	NOT_USED
		power	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	13.03.00	DG	values changed:
				(6)/(7) ARFCN_124 > ARFCN_32
				(8)/(9) ARFCN_32 > ARFCN_124
				new: RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indication added
				si3/4_rest_oct: SI3_REST_ DEF>
				SI3_REST_EMPTY
	16.02.01	DG	new: MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind
	06.03.03	LG	MUTE inserted

### RR658: No Service, Network Search requested by MMI, one PLMN

Description:	PL searches for the available PLMNs. It will detect one PLMN. After time-out of the registration timer a new attempt for full service shall be started by RR.

Preamble:	RR630

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(15)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
MUTE (9000)
TIMEOUT(1000)
(16)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(17)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(21)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(22)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(23)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(24)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(25)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(26)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(27)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(28)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
(29)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI_NO_SRV
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI_NO_SRV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	A_LAC_LIST1
		rxlevel	NOT_USED
		power	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	13.03.00	DG	values changed:
				(6)/(7) ARFCN_124 > ARFCN_32
				(8)/(9) ARFCN_32 > ARFCN_124
				(15) ONE_PLMN_AVAILABLE >
				NO_PLMN_AVAILABLE,
				PLMN_122_FOUND > NOT_USED 
				RXLEVEL_20 > NOT_USED 
				new: RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indication added
				si3/4_rest_oct: SI3_REST_ DEF>
				SI3_REST_EMPTY
	16.02.01	DG	MPH_BSIC_REQ/NOT_PRESENT_16BIT
				replaced by MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind
	06.03.03	LG	MUTE inserted

### RR661: Limited Service, Network Search requested by MMI, no PLMNs

Description:	PL searches for the available PLMNs. It will not detect any PLMN. After time-out of the registration timer a new attempt for full service shall be started by RR.

Preamble:	RR627

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(7)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
MUTE (9000)
TIMEOUT(1000)
(8)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(20)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(21)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(23)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
(24)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI_NO_SRV
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI_LIM_SRV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	A_LAC_LIST1
		rxlevel	NOT_USED
		power	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	13.03.00	DG	(6)/(7) ARFCN_124 > ARFCN_32
				(8)/(9) ARFCN_32 > ARFCN_124
				new: RR_SYNC_IND
	17.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indication added
				si3/4_rest_oct: SI3_REST_ DEF>
				SI3_REST_EMPTY
	15.02.01:	DG	(8)/(9)MPH_BSIC_REQ/CNF/124 deleted,
				new:MPH_SYNC_REQ
	19.02.01:	DG	(9)RR_ABORT_IND: NO_PLMN_AVAIL >
				ONE_PLMN_AVAIL
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind
	06.03.03	LG	MUTE inserted

### RR662: Limited Service, Network Search requested by MMI, one PLMN

Description:	PL searches for the available PLMNs. It will detect one PLMN. After time-out of the registration timer a new attempt for full service shall be started by RR.

Preamble:	RR627

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(6)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(7)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(8)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
MUTE (9000)
TIMEOUT(1000)
(14)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(17)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(20)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(21)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(22)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(23)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(24)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(25)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(26)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(27)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(28)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(29)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
(30)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_SYNC_REQ
		cs	CS_STOP_BCCH_READING

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_TEST_SIM_LIM_SERV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	A_LAC_LIST1
		rxlevel	NOT_USED
		power	NOT_USED

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI_NO_SRV
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI_LIM_SRV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	AS_PLMN
		lac_list	A_LAC_LIST1
		rxlevel	NOT_USED
		power	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	NOT_USED
		}

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_7
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_FF
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	NOT_USED
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	NOT_USED
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

History:	04.07.97	DL	Initial
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	29.05.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
				values changed: (6)/(7): 
				ARFCN_124>ARFCN_32
				(8)/(9): ARFCN_32>ARFCN_124
				(10)/(13): ARFCN_32>ARFCN_124
				new: RR_SYNC_IND
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indication added
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind
	06.03.03	LG	MUTE inserted

### RR667: Full Service, Network Search requested by MMI, select PLMN

Description:	PL searches for the available PLMNs. It will detect two PLMNs. It selects the non-serving cell network.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (RR CONFIG TIMER_SET=<TREG, 400>)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |	RR_SYNC_IND	  |		  |
	 |	(BCCH info)	  |		  |
	 *<==============================*		  |
(11)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(15)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
(16)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
(17)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(18)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(21)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(22)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(24)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(25)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(26)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(27)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(28)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(29)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(30)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(31)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(32)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(33)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(34)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(35)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(36)	 |	RR_SYNC_IND	  |		  |
	 |	(BCCH info)	  |		  |
	 *<==============================*		  |
(37)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(38)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(39)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(40)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(41)	 |	RR_ACTIVATE_CNF	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123_V
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147_V
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147_V
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- RR_SYNC_IND	ciph	NOT_PRESENT_8BIT
		mm_info	NOT_USED
		bcch_info	S_BCCH_INFO_NCELL_DESC_1
		synccs	NOT_PRESENT_16BIT
		chm	NOT_USED

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123_V
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_SYNC_REQ
		cs	CS_STOP_BCCH_READING

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	A_LAC_LIST1
		rxlevel	NOT_USED
		power	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_5
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- RR_SYNC_IND	ciph	NOT_PRESENT_8BIT
		mm_info	NOT_USED
		bcch_info	S_BCCH_INFO_NCELL_DESC_1
		synccs	NOT_PRESENT_16BIT
		chm	NOT_USED

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_124
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1F
		sync_only	NOT_USED

- RR_ACTIVATE_CNF
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_2
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2147
		power	NOT_USED
		gprs_indication	GPRS_NO

History:	15-Feb-00	LE	Initial
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_5
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
	17.05.00	DG	new: RR_SYNC_IND
				value changed:
				(23)MPH_BSIC_REQ:
				ARFCN_32>NOT_PRESENT_16BIT
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_CNF/IND: gprs_indication added
				si3/4_rest_oct: SI3_REST_ DEF>
				SI3_REST_EMPTY
		note: has to be revised /DG 12.07.00
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR670: Network Search, interrupted by cell reselection

Description:	PL searches for the available PLMNs. The network search is interrupted by a cell reselection. The reselection is delayed until the end of the network search.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(3)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(4)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(13)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(14)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(21)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(24)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_32
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_4
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2148
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2148
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_32
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_4
		gprs_sync	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_32
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_4
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	A_LAC_LIST1
		rxlevel	NOT_USED
		power	NOT_USED

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_32
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_4
		gprs_sync	NOT_USED

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	NOT_USED
		pg	NOT_USED
		bs_ag_blocks_res	NOT_USED
		bs_pa_mfrms	NOT_USED
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NOT_USED
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2148
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2148
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_3
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2148
		power	NOT_USED
		gprs_indication	GPRS_NO

History:	04.07.97	DL	Initial
	13.3.00	DG	new: (26) RR_SYNC_IND
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_6
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
				value changed:
				RR_ABORT_IND:
				plmn_avail:
				ONE_PLMN_AVAILABLE > 
				NO_PLMN_AVAILABLE
				plmn:
				PLMN_123_32_FOUND > NOT_USED
				rxlevel:
				RXLEVEL_22 > NOT_USED
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indication added
				si3/4_rest_oct: SI3_REST_ DEF>
				SI3_REST_EMPTY
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR671: Network Search, interrupted by downlink signalling failure

Description:	PL searches for the available PLMNs. The network search is interrupted by a downlink signalling failure. The reselection is started. It fails whyever. After ten seconds a cell selection is started and the mobile comes back to idle mode.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(3)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(4)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
MUTE (9000)
TIMEOUT (1000)
(15)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(18)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(21)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(22)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(23)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(24)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(25)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(26)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(27)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_32
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_4
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2148
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2148
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_32

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	AS_PLMN_123_32_FOUND
		lac_list	A_LAC_LIST1
		rxlevel	A_RXLEVEL_20
		power	NOT_USED

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_32

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	NOT_USED
		pg	NOT_USED
		bs_ag_blocks_res	NOT_USED
		bs_pa_mfrms	NOT_USED
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NOT_USED
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2148
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2148
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_3
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2148
		power	NOT_USED
		gprs_indication	GPRS_NO

History:	15-Feb-00	LE	Initial
	14.04.00	DG	value changed:
				MPH_IDLE_REQ/ext_bcch:
				EXT_BCCH_NOT_LIST > BSIC_6
	15.05.00	DG	RR_ACTIVATE_REQ: cell_test added
				value changed:
				RR_ABORT_IND/rxlevel:
				RXLEVEL_22 > RXLEVEL_21
	17.05.00	DG	new: RR_SYNC_IND
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indication added
				si3/4_rest_oct: SI3_REST_ DEF>
				SI3_REST_EMPTY
	19.02.01	DG	(11)MPH_BSIC_REQ(NOT_PRESENT_16BIT)
				replaced by MPH_SYNC_REQ
				(12)insert: MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind
	06.03.03	LG	MUTE inserted, time changed

### RR672: Network Search, interrupted by downlink signalling failure (II)

Description:	PL searches for the available PLMNs. The network search is interrupted by a downlink signalling failure. During reselection a second MMI PLMN search is started. The MMI PLMN search is delayed until the cell reselection has finished.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(3)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(4)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(15)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(16)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_CLASSMARK_REQ	  |
	 |		  *==============================>*
(21)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	MPH_CBCH_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	MPH_NEIGHBOURCELL_REQ	  |
	 |		  *==============================>*
(24)	 |	RR_ACTIVATE_IND	  |		  |
	 *<==============================*		  |
(25)	 |		  |	MPH_IDENTITY_REQ	  |
	 |		  *==============================>*
(26)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(27)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(28)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(29)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(30)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(31)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_32
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_4
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2148
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2148
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_32

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	AS_PLMN_123_32_FOUND
		lac_list	A_LAC_LIST1
		rxlevel	A_RXLEVEL_20
		power	NOT_USED

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_32

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	NOT_USED
		pg	NOT_USED
		bs_ag_blocks_res	NOT_USED
		bs_pa_mfrms	NOT_USED
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NOT_USED
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_1
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2148
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2148
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_CLASSMARK_REQ
		classmark	CLASS_MS

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	DLT_23
		pg	PG_0
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NCC_PERMITTED_1
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_CBCH_REQ
		cbch	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	NOT_USED
		arfcn	A_MPH_NCELL_1C
		sync_only	NOT_USED

- RR_ACTIVATE_IND
		op	OP_MODE_TEST_SIM
		mm_info	MM_INFO_3
		cid	CELL_IDENT_3748
		plmn	PLMN_ID_123
		lac	LAC_2148
		power	NOT_USED
		gprs_indication	GPRS_NO

- MPH_IDENTITY_REQ
		mid	S_MS_ID_IMSI_HPLMN_TMSI

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	A_LAC_LIST1A
		rxlevel	NOT_USED
		power	NOT_USED

History:	15-Feb-00	LE	Initial
	15-May-00	DG	RR_ACTIVATE_REQ: cell_test added
				value changed:
				RR_ABORT_IND/rxlevel:
				RXLEVEL_22 > RXLEVEL_21
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	RR_ACTIVATE_IND: gprs_indication added
				si3/4_rest_oct: SI3_REST_ DEF>
				SI3_REST_EMPTY
	19.02.01	DG	(11)MPH_BSIC_REQ(NOT_PRESENT_16BIT)
				replaced by MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

### RR673: Network Search, interrupted by downlink signalling failure (III)

Description:	PL searches for the available PLMNs. The network search is interrupted by a downlink signalling failure. During reselection a second MMI PLMN search is started. The MMI PLMN search is delayed until the cell reselection has finished. The cell reselection fails and is followed by a cell selection.

Preamble:	RR637

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(3)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(4)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(11)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
(13)	 |		  |	MPH_ERROR_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_IDLE_REQ	  |
	 |		  *==============================>*
MUTE (9000)
TIMEOUT (1000)
(15)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(17)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(18)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(20)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(21)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(24)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(25)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(26)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(27)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(28)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(29)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(30)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(31)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(32)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(33)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(34)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(35)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
MUTE (9000)
TIMEOUT (1000)
(36)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_32
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_4
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2148
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2148
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	AS_PLMN_123_32_FOUND
		lac_list	A_LAC_LIST1
		rxlevel	A_RXLEVEL_20
		power	NOT_USED

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_32

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_67
		ext_bcch	NOT_USED
		comb_ccch	CCD_CCCH_1_NOT_COMB
		tn	TN_0
		dlt	NOT_USED
		pg	NOT_USED
		bs_ag_blocks_res	NOT_USED
		bs_pa_mfrms	NOT_USED
		power	MS_TXPWR_MAX_CCH_02
		ncc_permitted	NOT_USED
		reorg_only	NOT_USED
		eotd_avail	CONST_0
		gprs_support	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2148
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- RR_ACTIVATE_REQ
		plmn	NOT_USED
		op	OP_MODE_NET_SRCH_MMI
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_123_2148
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_123_2148
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_6
		cs	CS_NO_BCCH_AVAIL

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI_NO_SRV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	ONE_PLMN_AVAILABLE
		plmn	AS_PLMN_123_32_FOUND
		lac_list	A_LAC_LIST1A
		rxlevel	A_RXLEVEL_22
		power	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

History:	15-Feb-00	LE	Initial
	15-May-00	DG	RR_ACTIVATE_REQ: cell_test added
				value changed:
				RR_ABORT_IND/rxlevel:
				RXLEVEL_22 > RXLEVEL_21
	26.05.00	DG	values changed:
				si3/4_rest_oct:
				NOT_USED > SI3_REST_DEF
	19.01.01	DG	RR_ACTIVATE_REQ: gprs_indication added
				MPH_BSIC_REQ: cell_type, timing_info
				added
	08.02.01	DG	si3/4_rest_oct: SI3_REST_ DEF>
				SI3_REST_EMPTY
	20.02.01	DG	MPH_BSIC_REQ(NOT_PRESENT_16BIT)
				replaced by MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind
	06.03.03	LG	MUTE inserted

### RR674: Network Search requested by MMI, five PLMNs

Description:	PL searches for available PLMNs. Five PLMNs are found and reported to MM.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(12)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(13)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(14)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(15)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(16)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(17)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(18)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(19)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(20)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(21)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(22)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(23)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(24)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(25)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(26)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(27)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(28)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(29)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(30)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 1)	  |
	 |		  *<==============================*
(31)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 2)	  |
	 |		  *<==============================*
(32)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 3)	  |
	 |		  *<==============================*
(33)	 |		  |	MPH_UNITDATA_IND	  |
	 |		  |	  (SYS INFO TYPE 4)	  |
	 |		  *<==============================*
(34)	 |		  |	MPH_SYNC_REQ	  |
	 |		  *==============================>*
(35)	 |	RR_ABORT_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_TEST
		op	OP_MODE_NET_SRCH_MMI
		cksn	NOT_USED
		kcv	NOT_USED
		accc	NOT_USED
		imsi_struct	NOT_USED
		tmsi_struct	NOT_USED
		thplmn	NOT_USED
		bcch_info	NOT_USED
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_5
		arfcn	ARFCN_67_64_32_26_124
		rx_lev	RXLEV_24_23_22_13_10

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_34_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_67
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_34_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_64

- MPH_BSIC_CNF
		arfcn	ARFCN_64
		bsic	BSIC_6
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_64
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_64
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_64
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147_V
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_64
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147_V
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_32

- MPH_BSIC_CNF
		arfcn	ARFCN_32
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_35_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_32
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_35_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_26

- MPH_BSIC_CNF
		arfcn	ARFCN_26
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_26
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_26
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_26
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_26
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NCELL_DESC_EMPTY
		ncc_permit	NCC_PERMITTED_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_3748
		loc_area_ident	LOC_AREA_IDENT_122_36_2147
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		si3_rest_oct	S_SI3_REST_EMPTY
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_122_36_2147
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		chan_desc	NOT_USED
		mob_alloc	NOT_USED
		si4_rest_oct	S_SI4_REST_EMPTY
		}

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH_AND_DEACTIVATE

- RR_ABORT_IND
		op	OP_MODE_NET_SRCH_MMI_NO_SRV
		cause	RRCS_ABORT_CEL_SEL_FAIL
		plmn_avail	FIVE_PLMN_AVAILABLE
		plmn	NOT_USED
		lac_list	A_LAC_LIST5
		rxlevel	NOT_USED
		power	NOT_USED

History:	22.01.01	DG	Initial
	08.02.01	DG	si3/4_rest_oct: SI3_REST_DEF>
				SI3_REST_EMPTY
	09.02.01	DG	MPH_BSIC_REQ 
				(NOT_PRESENT_16BIT_MIN1) replaced by
				MPH_SYNC_REQ
	31.03.01	VK	'power' added in rr_abort_ind
	05-Jun-01	MSE	adapted to TAP2
	25.02.03	LG	'lac_list' added in rr_abort_ind

## Miscellaneous

### RR680: Overload primitive store / Start cell selection without BCCH information (std 6)

Description:	Seven cell selections without BCCH information are started. Just to test overload of primitive store.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(3)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(4)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(5)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(6)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(7)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_NORMAL
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_NORMAL
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_NORMAL
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_NORMAL
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_NORMAL
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_NORMAL
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_NORMAL
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

History:	31-07-00	DG	Initial
	05-Jun-01	MSE	adapted to TAP2

### RR681: Overload primitive store  / RR-Connection Establishment by the MS

Description:	MM starts 7 connection establishments. Just to test overload of primitive store.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=1)
(1)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(2)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(3)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(4)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(5)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(6)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
(7)	 |	RR_ESTABLISH_REQ	  |		  |
	 *==============================>*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ESTABLISH_REQ
			estcs	ESTCS_SERV_REQ_BY_MM
			sdu	MM_MESSAGE

- RR_ESTABLISH_REQ
			estcs	ESTCS_SERV_REQ_BY_MM
			sdu	MM_MESSAGE

- RR_ESTABLISH_REQ
			estcs	ESTCS_SERV_REQ_BY_MM
			sdu	MM_MESSAGE

- RR_ESTABLISH_REQ
			estcs	ESTCS_SERV_REQ_BY_MM
			sdu	MM_MESSAGE

- RR_ESTABLISH_REQ
			estcs	ESTCS_SERV_REQ_BY_MM
			sdu	MM_MESSAGE

- RR_ESTABLISH_REQ
			estcs	ESTCS_SERV_REQ_BY_MM
			sdu	MM_MESSAGE

- RR_ESTABLISH_REQ
			estcs	ESTCS_SERV_REQ_BY_MM
			sdu	MM_MESSAGE

History:	01.08.00	DG	Initial

### RR682: Overload primitive store  / Cell Selection

Description:	A cell selection is started. Just to test overload of primitive store.

Preamble:	RR000

MM		 RR		PL/DL
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
	 |		  |		  |
(1)	 |	RR_ACTIVATE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_POWER_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(4)	 |	RR_SYNC_REQ	  |		  |
	 *==============================>*		  |
(5)	 |	RR_SYNC_REQ	  |		  |
	 *==============================>*		  |
(6)	 |	RR_SYNC_REQ	  |		  |
	 *==============================>*		  |
(7)	 |	RR_SYNC_REQ	  |		  |
	 *==============================>*		  |
(8)	 |	RR_SYNC_REQ	  |		  |
	 *==============================>*		  |
(9)	 |	RR_SYNC_REQ	  |		  |
	 *==============================>*		  |
(10)	 |	RR_SYNC_REQ	  |		  |
	 *==============================>*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RR_ACTIVATE_REQ
		plmn	PLMN_ID_123
		op	OP_MODE_TEST_SIM
		cksn	CKSN_NOT_PRES
		kcv	KCV_12345678
		accc	ACC_CTRL_CLASS_0008
		imsi_struct	MOBILE_ID_IMSI_HPLMN
		tmsi_struct	MOBILE_ID_TMSI
		thplmn	TIME_HPLMN_VALID
		bcch_info	S_BCCH_INFO_EMPTY
		cell_test	CELL_TEST_DISABLE
		gprs_indication	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	NOT_USED

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_6
		kcv	KCV_00112233
		tmsi_struct	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	ACC_CTRL_CLASS_0000
		thplmn	TIME_HPLMN_EMPTY

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_6
		kcv	KCV_00112233
		tmsi_struct	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	ACC_CTRL_CLASS_0000
		thplmn	TIME_HPLMN_EMPTY

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_6
		kcv	KCV_00112233
		tmsi_struct	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	ACC_CTRL_CLASS_0000
		thplmn	TIME_HPLMN_EMPTY

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_6
		kcv	KCV_00112233
		tmsi_struct	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	ACC_CTRL_CLASS_0000
		thplmn	TIME_HPLMN_EMPTY

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_6
		kcv	KCV_00112233
		tmsi_struct	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	ACC_CTRL_CLASS_0000
		thplmn	TIME_HPLMN_EMPTY

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_6
		kcv	KCV_00112233
		tmsi_struct	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	ACC_CTRL_CLASS_0000
		thplmn	TIME_HPLMN_EMPTY

- RR_SYNC_REQ
		op	NOT_USED
		cksn	CKSN_6
		kcv	KCV_00112233
		tmsi_struct	NOT_USED
		plmn	NOT_USED
		lac	NOT_USED
		synccs	SYNCCS_TMSI_CKSN_KC_INVAL
		accc	ACC_CTRL_CLASS_0000
		thplmn	TIME_HPLMN_EMPTY

History:	01.08.00	DG	Initial
	05-Jun-01	MSE	adapted to TAP2

### RR683: Overload primitive store / Listen to SYS INFOs (with SIM)

Description:	Listening to a BCCH carrier.BCCH ok. Just to test overload of primitive store.

Preamble:	RR001B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |		  |	MPH_POWER_CNF	  |
	 |		  *<==============================*
(2)	 |		  |	MPH_BSIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_BSIC_CNF	  |
	 |		  *<==============================*
(4)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(5)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(7)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPH_PAGING_IND	  |
	 |		  *<==============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_CNF
		num_of_chan	CHANNELS_3
		arfcn	ARFCN_67_32_124
		rx_lev	RXLEV_22_21_20

- MPH_BSIC_REQ
		arfcn	ARFCN_67

- MPH_BSIC_CNF
		arfcn	ARFCN_67
		bsic	BSIC_5
		cs	CS_NO_ERROR

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

- MPH_PAGING_IND
		identity_type	ID_IMSI
		channel_needed	CN_ANY

History:	01.08.00	DG	Initial
	19.01.01	DG	MPH_BSIC_REQ: cell_type, timing_info
				added

### RR684: Engineering Mode – Assignment Command received & complete (Event number 19 + 22)

Description:	The base station initiates an intracell handover. RR  releases the old connection locally and configures the new channel configuration. There is no change of channel mode.

Preamble:	RR154B

MM		 RR		PL/DL
	 |		  |		  |
(1)	 |	EM_RR_EVENT_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_MEASUREMENT_IND	  |
	 |		  *<==============================*
(3)	 |		  |	DL_UNITDATA_REQ	  |
	 |		  |	(MEASUREMENT REPORT)	  |
	 |		  *==============================>*
(4)	 |		  |	DL_DATA_IND	  |
	 |		  |	(ASSIGNMENT COMMAND)	  |
	 |		  *<==============================*
(5)	 |	EM_DATA_IND	  |		  |
	 *<==============================*		  |
(6)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPH_DEDICATED_CNF	  |
	 |		  *<==============================*
(9)	 |		  |	DL_RESUME_REQ	  |
	 |		  |	(ASSIGNMENT COMPLETE)	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- EM_RR_EVENT_REQ
		bitmask_rr_h	Bitm_H
		bitmask_rr_l	Bitm_L

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_67
		rx_lev_full	RX_LEV_20
		rx_lev_sub	RX_LEV_20
		rx_qual_full	RX_QUAL_1
		rx_qual_sub	RX_QUAL_1
		dtx	DTX_NOT_USED
		otd	TIME_ADV_27
		valid	TRUE
		fn_offset	FN_OFFSET_1_SEC
		ncells	S_NCELLS_0
		gprs_sync	NOT_USED

- DL_UNITDATA_REQ
		ch_type	CH_TYPE_SACCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_MEAS_REP
		ti	TI_0
		meas_result	MEAS_RESULT_NCELL_0
		}

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_ASSIGN_CMD
		ti	TI_0
		chan_desc	CHANNEL_DESC_SDCCH2
		pow_cmd	POWER_COMMAND_05
		freq_list_after	NOT_USED
		cell_chan_desc	NOT_USED
		multislot_alloc	NOT_USED
		chan_mode	NOT_USED
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		mob_alloc_after	NOT_USED
		start_time	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	NOT_USED
		chan_desc_before_2	NOT_USED
		freq_chan_seq	NOT_USED
		mob_alloc_before	NOT_USED
		ciph_mode_set	NOT_USED
		vgcs_tmi	NOT_USED
		multirate_conf	S_multirate_conf_1
		}

- EM_DATA_IND
		entity	EM_ENTITY

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_DEDICATED_REQ
		mod	MODE_CHAN_ASSIGN
		start	NO_STARTING_TIME
		ch_type	PRR_CHANNEL_TYPE_3
		ch_type2	NOT_USED
		arfcn	ARFCN_67
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	PRR_TR_PARA_3
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

- DL_RESUME_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	UPLINK
		pd	U_ASSIGN_COMP
		ti	TI_0
		rr_cause	RR_CAUSE_0
		}

History:	04.07.97	DL	Initial
	18.11.97	LE	adapted to layer 1
	06.05.98	VK	DL_RELEASE_CNF removed
	25.01.01	DG	DL_DATA_REQ added,
				DL_DATA_IND: multislot_alloc,
				chan_mode2…8 added
	12.02.01	DG	DL_DATA_REQ deleted
	05-Jun-01	MSE	adapted to TAP2

23-Oct-01	OT	Initial version for EM.

## E-OTD

### RR800: Successful E-OTD

Description:	RRLC initiates a position measurement request and forwards a list of the neighbour cells in the RRLC-MEAS request primitive. RR in turn forwards this request to ALR (MPH-NCELL-POS request primitive). Successful completion of the position measurement procedure is indicated by receipt of a MPH-NCELL-POS indication primitive containing the results of the cells measured. This result is forwarded to RRLP (RRLC-MEAS indication primitive).

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |	RRLC_MEAS_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_NCELL_POS_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_NCELL_POS_IND	  |
	 |		  *<==============================*
(4)	 |	RRLC_MEAS_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RRLC_MEAS_REQ
		req_id	REQ_ID_0
		arfcn	ARFCN_67
		bsic	BSIC_5
		assist_data	NCELL_EOTD_6

- MPH_NCELL_POS_REQ
		req_id	REQ_ID_0
		ncell_eotd	NCELL_EOTD_6

- MPH_NCELL_POS_IND
		req_id	REQ_ID_0
		eotd_res	EOTD_SUCC
		ta	TAV_EOTD
		fn	FN_OFFSET_1_SEC
		eotd_sc_res	eotd_sc_res_ok
		eotd_sc_res1	eotd_sc_res1_ok
		eotd_nc_res	EOTD_RESULT_6

- RRLC_MEAS_IND
		mcc	MCC_123
		mnc	MNC_32
		lac	LAC_2147
		cell_id	CELL_IDENT_3748
		eotd_mode	EOTD_DEDIC
		req_id	REQ_ID_0
		tav	TAV_EOTD
		fn	FN_OFFSET_1_SEC
		eotd_sc_res	EOTD_SC_RES_OK
		eotd_sc_res1	EOTD_SC_RES1_OK
		eotd_nc_res	EOTD_RESULT_6

History:		26 September 2002	DL	Initial

### RR802: E-OTD Request rejected by ALR

Description:	RRLC initiates a position measurement request and forwards a list of the neighbour cells in the RRLC-MEAS request primitive. RR in turn forwards this request to ALR (MPH-NCELL-POS request primitive). Unsuccessful position measurement is indicated by receipt of a MPH-NCELL-POS indication primitive with the IE eotd_res set to EOTD_REF, indicating an incorrect reference BTS. The unsuccessful operatoin is signalled to RRLP (RRLC-ERROR indication primitive).

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |	RRLC_MEAS_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_NCELL_POS_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_NCELL_POS_IND	  |
	 |		  *<==============================*
(4)	 |	RRLC_ERROR_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RRLC_MEAS_REQ
		req_id	REQ_ID_0
		arfcn	ARFCN_67
		bsic	BSIC_5
		assist_data	NCELL_EOTD_6

- MPH_NCELL_POS_REQ
		req_id	REQ_ID_0
		ncell_eotd	NCELL_EOTD_6

- MPH_NCELL_POS_IND
		req_id	REQ_ID_0
		eotd_res	EOTD_REF
		ta	NOT_USED
		fn	NOT_USED
		eotd_sc_res	NOT_USED
		eotd_sc_res1	NOT_USED
		eotd_nc_res	NOT_USED

- RRLC_ERROR_IND
		cause	LCS_WRONG_BTS

History:		26 September 2002	DL	Initial

### RR804: E-OTD Request rejected by RR – Handover in progress

Description:	Following commencement of handover (receipt of HANDOVER COMMAND message), RRLC initiates a position measurement request and forwards a list of the neighbour cells in the RRLC-MEAS request primitive. RR rejects the request by sending an RRLC-ERROR indication primitive to RRLP.

Preamble:	RR185

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(2)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
(4)	 |	RRLC_MEAS_REQ	  | 		  |
	 *==============================>*		  |
(5)	 |	RRLC_ERROR_IND	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_HANDOV_CMD
		ti	TI_0
		cell_desc	CELL_DESC_1
		chan_desc_after	CHANNEL_DESC_FACCH3
		handov_ref	HANDOV_REF_1
		pow_cmd_access	POW_05_HO
		synch_ind	SYNCH_IND_2
		freq_short_list_after	NOT_USED
		freq_list_after	NOT_USED
		cell_chan_desc	CELL_CHAN_DESC_1
		chan_mode	CHANNEL_MODE_SPEECH
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		freq_chan_seq_after	NOT_USED
		mob_alloc_after	MOBILE_ALLOCATION_2
		start_time	START_TIME_1
		time_diff	NOT_USED
		time_advance	NOT_USED
		freq_short_list_before	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	CHANNEL_DESC_FACCH4
		chan_desc_before_2	NOT_USED
		freq_chan_seq_before	NOT_USED
		mob_alloc_before	MOBILE_ALLOCATION_3
		ciph_mode_set	NOT_USED
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0

- MPH_DEDICATED_REQ
		mod	MODE_PRE_SYNC_HANDOVER
		start	STARTING_TIME_1
		ch_type	PRR_CHANNEL_TYPE_6
		ch_type2	PRR_CHANNEL_TYPE_7
		arfcn	ARFCN_32
		bsic	NOT_USED
		ho_param	HO_PARAM_1
		tr_para	PRR_TR_PARA_HO
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

- RRLC_MEAS_REQ
		req_id	REQ_ID_0
		arfcn	ARFCN_67
		bsic	BSIC_5
		assist_data	NCELL_EOTD_6

- RRLC_ERROR_IND
		cause	LCS_HANDOVER

History:		26 September 2002	DL	Initial

### RR806: E-OTD Operation interrupted due to start of Handover

Description:	Following commencement of handover (receipt of HANDOVER COMMAND message), RRLC initiates a position measurement request and forwards a list of the neighbour cells in the RRLC-MEAS request primitive. RR rejects the request by sending an RRLC-ERROR indication primitive to RRLP.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |	RRLC_MEAS_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MPH_NCELL_POS_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_DATA_IND	  |
	 |		  |	(HANDOVER COMMAND)	  |
	 |		  *<==============================*
(4)	 |		  |	DL_SUSPEND_REQ	  |
	 |		  *==============================>*
(5)	 |	RRLC_ERROR_IND	  | 		  |
	 *<==============================*		  |
(6)	 |		  |	MPH_DEDICATED_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RRLC_MEAS_REQ
		req_id	REQ_ID_0
		arfcn	ARFCN_67
		bsic	BSIC_5
		assist_data	NCELL_EOTD_6

- MPH_NCELL_POS_REQ
		req_id	REQ_ID_0
		ncell_eotd	NCELL_EOTD_6

- DL_DATA_IND
		ch_type	CH_TYPE_SDCCH
		sapi	SAPI_0
		sdu
		{
		component	RR
		direction	DOWNLINK
		pd	D_HANDOV_CMD
		ti	TI_0
		cell_desc	CELL_DESC_1
		chan_desc_after	CHANNEL_DESC_FACCH3
		handov_ref	HANDOV_REF_1
		pow_cmd_access	POW_05_HO
		synch_ind	SYNCH_IND_2
		freq_short_list_after	NOT_USED
		freq_list_after	NOT_USED
		cell_chan_desc	CELL_CHAN_DESC_1
		chan_mode	CHANNEL_MODE_SPEECH
		chan_mode2	NOT_USED
		chan_mode3	NOT_USED
		chan_mode4	NOT_USED
		chan_mode5	NOT_USED
		chan_mode6	NOT_USED
		chan_mode7	NOT_USED
		chan_mode8	NOT_USED
		chan_desc_after_2	NOT_USED
		chan_mode_2	NOT_USED
		freq_chan_seq_after	NOT_USED
		mob_alloc_after	MOBILE_ALLOCATION_2
		start_time	START_TIME_1
		time_diff	NOT_USED
		time_advance	NOT_USED
		freq_short_list_before	NOT_USED
		freq_list_before	NOT_USED
		chan_desc_before	CHANNEL_DESC_FACCH4
		chan_desc_before_2	NOT_USED
		freq_chan_seq_before	NOT_USED
		mob_alloc_before	MOBILE_ALLOCATION_3
		ciph_mode_set	NOT_USED
		}

- DL_SUSPEND_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0

- RRLC_ERROR_IND
		cause	LCS_HANDOVER

- MPH_DEDICATED_REQ
		mod	MODE_PRE_SYNC_HANDOVER
		start	STARTING_TIME_1
		ch_type	PRR_CHANNEL_TYPE_6
		ch_type2	PRR_CHANNEL_TYPE_7
		arfcn	ARFCN_32
		bsic	NOT_USED
		ho_param	HO_PARAM_1
		tr_para	PRR_TR_PARA_HO
		ciph	NO_CIPHERING
		amr_conf	NOT_USED

History:		26 September 2002	DL	Initial

### RR820: sending APDU, no segmentation

Description:
Requirement:
Segmentation shall not be used when an APDU fits into a single APPLICATION INFORMATION 
message of maximum or smaller size.
On receiving an APPLICATION INFORMATION message, the receiving layer 3 entity shall deliver the 
message contents to the identified local application.

Scenario:
An APDU of maximum length (251 octets) shall be sent in a RRLC_DATA_REQ primitive. No 
segmentation should be observed, the APDU is forwarded in exactly one DL_DATA_REQ primitive.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |	RRRRLP_DATA_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	DL_DATA_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- RRRRLP_DATA_REQ
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_820

- DL_DATA_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_820

History:		15 October 2002	VK	Initial

### RR821: sending APDU, with segmentation

Description:
Requirement:
The sending layer 3 entity shall segment an APDU by dividing it into one or more segments exactly 
fitting into maximum sized APPLICATION INFORMATION messages plus a final segment fitting into 
an APPLICATION INFORMATION message of maximum size or smaller. 

Scenario:
An APDU of maximum length + 1 octet (252 octets) shall be sent in a RRLC_DATA_REQ primitive. 
Segmentation should be observed.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |	RRRRLP_DATA_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	DL_DATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_DATA_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

(1) RRRRLP_DATA_REQ
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_821

(2) DL_DATA_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_821a

(3) DL_DATA_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_821b

History:	15 October 2002 VK	Initial

### RR822: sending APDU, three segments

Description:
Requirement:
Once segmented, the resulting APPLICATION INFORMATION messages shall be transferred in 
sequence to the data link layer for transmission, without being intersperced by other level 3 
messages. The first APPLICATION INFORMATION message in the sequence shall indicate "First 
Segment" and "Not Last Segment". Subsequent APPLICATION INFORMATION messages except for the 
last shall indicate "Not First Segment" and "Not Last Segment". The last APPLICATION INFORMATION 
message shall indicate "Not First Segment" and "Last Segment" and shall include a C/R flag as 
provided by the sending application.

Scenario:
An APDU of 3* maximum length *(3*251=753 octets) shall be sent in a RRLC_DATA_REQ primitive, 
immediately followed by another DATP message. Segmentation should be observed, three full APDU 
segments should be generated containing the correct APDU flags. The DTAP message is forwarded 
after the APDU.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |	RRRRLP_DATA_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	DL_DATA_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	DL_DATA_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	DL_DATA_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

(1) RRRRLP_DATA_REQ
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_822

(2) DL_DATA_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822a

(3) DL_DATA_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822b

(4) DL_DATA_REQ
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822c

History:	15 October 2002 VK	Initial

### RR823: receiving APDU, no segmentation

Description:
Requirement:
On receiving an APPLICATION INFORMATION message, the receiving layer 3 entity shall deliver the 
message contents to the identified local application.

Scenario:
An APDU of maximum length (251 octets) shall be received in a DL_DATA_IND primitive and 
forwarded in a RRLC_DATA_IND primitive. No segmentation should be observed. The C/R flag shall 
take on '1'.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(2)	 |	RRRRLP_DATA_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

(1) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_820

(2) RRRRLP_DATA_IND
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_820

History:	15 October 2002 VK	Initial

### RR824: receiving APDU, three segments

Description:
Requirement:
The receiving layer 3 entity shall reassemble any segmented APDU before transfer to the local 
application. 

Scenario:
All three segments of an APDU of length 3*251 octets = 753 octets shall be received in 
DL_DATA_IND primitives and forwarded in one RRLC_DATA_IND primitive. The C/R flag shall take on 
'1'.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(2)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(3)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(4)	 |	RRRRLP_DATA_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

(1) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822a

(2) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822b

(3) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822c

(4) RRRRLP_DATA_IND
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_822

History:	15 October 2002 VK	Initial

### RR825: receiving APDU, three segments, timeout

Description:
Requirement:
The receiver may employ a timer to detect possible loss of APDU segments. If employed, the timer shall be started when the first APDU segment is received and cancelled after the last segment is received.

Scenario:
The first two segments of an APDU shall be received in a DL_DATA_IND primitive. Subsequent segments of the APDU are not received within 5 seconds. No RRLC_DATA_IND primitive is sent. The two segments are discarded.
Another APDU (no segmentation) is received in a DL_DATA_IND primitive and forwarded in a RRLC_DATA_IND primitive.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(2)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
MUTE (5000)
(3)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(4)	 |	RRRRLP_DATA_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

(1) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822a

(2) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822b

(3) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_820

(4) RRRRLP_DATA_IND
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_820

History:	15 October 2002	VK	Initial
	06.03.03	LG	TIMEOUT replaced by MUTE

### RR826: receiving APDU, abnormal, other layer 3 message

Description:
Requirement:
An APPLICATION INFORMATION transfer error shall be detected due to any of the following:
b)      While performing APDU reassembly
-       receipt of any other layer 3 message defined to use SAPI 0 on the main DCCH;

Scenario:
The first segment of an segmented APDU is received in a DL_DATA_IND followed by the reception of 
a DTAP layer 3 message. The received APDU segments are disacrded. Then the remaining segments of 
the APDU are received. No RRLC_DATA_IND is generated.
Another APDU (no segmentation) is received in a DL_DATA_IND primitive and forwarded in a 
RRLC_DATA_IND primitive.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(2)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(3)	 |		  |	DL_DATA_IND	  |
	 |		  |	(LAYER 3 MESSAGE)	  |
	 |		  *<==============================*
(4)	 |	RR_DATA_IND 	  |		  |
	 |	(LAYER 3 MESSAGE)	  |		  |
	 *<==============================*		  |
(5)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
MUTE (500)
(6)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(7)	 |	RRRRLP_DATA_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

(1) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822a

(2) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822b

(3) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	MM_MESSAGE

(4) RR_DATA_IND
		d1	NOT_USED
		d2	NOT_USED
		sdu	MM_MESSAGE

(5) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822c

(6) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_820

(7) RRRRLP_DATA_IND
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_820

History:	15 October 2002	VK	Initial
	06.03.03	LG	MUTE inserted

### RR827: receiving APDU, abnormal, "First or Only Segment"

Description:
Requirement:
An APPLICATION INFORMATION transfer error shall be detected due to any of the following:
b)      While performing APDU reassembly
-       receipt of an APDU or APDU segment indicating "First or Only Segment";

Scenario:
The first segment of an segmented APDU is received in a DL_DATA_IND followed by the reception of 
the first segment of a another segmented APDU. Then remaining APDU-segments are received. A 
RRLC_DATA_IND is generated containing the re-segmented APDU containing the first segment which 
was received at the second position.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
MUTE (200)
(2)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(3)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(4)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(5)	 |	RRRRLP_DATA_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

(1) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_827

(2) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822a

(3) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822b

(4) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_822c

(5) RRRRLP_DATA_IND
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_822

History:	15 October 2002	VK	Initial
	06.03.03	LG	MUTE inserted

### RR828: receiving APDU, abnormal, "First or Only Segment"

Description:
Requirement:
An APPLICATION INFORMATION transfer error shall be detected due to any of the following:
c)      While not performing APDU reassembly, receipt of an APDU segment indicating "not First or 
only segment";

Scenario:
A APDU segment containing the flags "not First or only segment" and "Last or only segment" is 
received.
Then another APDU (no segmentation) is received in a DL_DATA_IND primitive and forwarded in a 
RRLC_DATA_IND primitive.
A APDU segment containing the flags "not First or only segment" and "not Last or only segment" is 
received.
Then another APDU (no segmentation) is received in a DL_DATA_IND primitive and forwarded in a 
RRLC_DATA_IND primitive.

Preamble:	RR154C

RRLP		 RR		 ALR
	 |		  |		  |
(1)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
MUTE (200)
(2)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(3)	 |	RRRRLP_DATA_IND	  |		  |
	 *<==============================*		  |
(4)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
MUTE (200)
(5)	 |		  |	DL_DATA_IND	  |
	 |		  *<==============================*
(6)	 |	RRRRLP_DATA_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

(1) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_828a

(2) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_820

(3) RRRRLP_DATA_IND
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_820

(4) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_828b

(5) DL_DATA_IND
		ch_type	CH_TYPE_FACCH
		sapi	SAPI_0
		sdu	APDU_MSG_DL_820

(6) RRRRLP_DATA_IND
		cr	CONST_1
		sdu	APDU_MSG_RRRLP_820

History:	15 October 2002	VK	Initial
	06.03.03	LG	MUTE inserted

# Suites

INT_Cell_Channel_Desc_INIT: RR000;

INT_Cell_Channel_Desc_CS: RR004;

INT_Cell_Channel_Desc_CON_EST: RR153;

INT_Cell_Channel_Desc_DEDI: RR154G;

INT_Cell_Channel_Desc_DEDI_1900: RR154H;

INT_Cell_Channel_Desc_ASS_CMD: RR211F;

INT_Cell_Channel_Desc_ASS_CMD_FAIL: RR213;

INT_Measurement_Report_1900: RR193;

SUI_Cell_Channel_Desc:
	INT_Cell_Channel_Desc_INIT,
	INT_Cell_Channel_Desc_CS,
	INT_Cell_Channel_Desc_CON_EST,
	INT_Cell_Channel_Desc_DEDI,
	INT_Cell_Channel_Desc_ASS_CMD;

SUI_Cell_Channel_Desc_Fail:
	INT_Cell_Channel_Desc_INIT,
	INT_Cell_Channel_Desc_CS,
	INT_Cell_Channel_Desc_CON_EST,
	INT_Cell_Channel_Desc_DEDI,
	INT_Cell_Channel_Desc_ASS_CMD_FAIL;

SUI_Measurement_Report_1900:
	INT_Cell_Channel_Desc_INIT,
	INT_Cell_Channel_Desc_CS,
	INT_Cell_Channel_Desc_CON_EST,
	INT_Cell_Channel_Desc_DEDI_1900,
	INT_Measurement_Report_1900;

/*

History:	15.07.02	MPA		Initial
*/