GSM Fax & Data Services

Test Specification

UART

Author:	Texas Instruments Berlin AG
	Alt-Moabit 90a
	D-10559 Berlin
	Germany

Date:	28-Jul-2026
Document No.:	8441.412.00.008
File:	input.doc

Table of Contents

0	Document Control	4

0.1	Document History	4

0.2	References	5

0.3	Abbreviations	7

0.4	Terms	10

1	Parameters	11

1.1	Primitive elements	11

1.2	Struct elements	12

1.3	Fields	12

1.3.1	Strings	12

1.3.2	Frames	23

1.3.2.1	Multiplexer Start Up / Close Down	23

1.3.2.2	Unnumbered Acknowledgements (UA)	26

1.3.2.3	Flow Control	31

1.3.2.4	Power Saving Control (PSC)	34

1.3.2.5	DLC Establishment / Release	37

1.3.2.6	Data Frames	41

1.3.2.7	Disconnected Mode (DM) Frames	53

1.3.2.8	Modem Status Commands (MSC)	55

1.3.2.9	Invalid Frames	60

1.3.2.10	Other Frames	69

1.4	Declarations	72

1.5	Arrays	72

1.6	Structs	73

1.6.1	Communication Parameters	73

1.6.2	Service Data Units	75

2	TEST CASES	78

2.1	Setup (UART001 - UART010)	78

2.1.1	UART001:  TAP Setup	78

2.1.2	UART002:  Entity setup	78

2.2	Configuration (UART011 – UART019)	79

2.2.1	UART011:  Setup Connection to PPP (default parameters)	79

2.2.2	UART012:  Setup Connection to PPP (explicit parameter setting)	80

2.3	Data Transmission - Sending (UART020 - UART029)	82

2.3.1	UART020: Send Hello Strings	82

2.3.2	UART021: Send Hello Strings - Ignore String with Invalid Channel Id	84

2.4	Data Transmission - Receiving (UART030 - UART039)	86

2.4.1	UART030: Receive One Letter String	86

2.4.2	UART031: Receive String (Size > N1)	87

2.5	Data Transmission - Mixed Sending/Receiving (UART040 - UART049)	88

2.5.1	UART040: Mixed Receiving/Sending One Letter Strings	88

2.5.2	UART041: Mixed Receiving/Sending Multi Letter Strings	91

3	Multiplexer Tests (UART100 - UART200)	95

3.1	Setup/End Multiplexer Mode	95

3.1.1	UART100: Additional Setup for Multiplexer Tests	95

3.1.2	UART101: ACI Prepares UART to Start Multiplexer	95

3.1.3	UART102: Link Establishment Fails (Timeout T3)	96

3.1.4	UART103: Reception of SABM for Multiplexer Control Channel	97

3.1.5	UART105: Multiplexer Close-down (UE initiated)	98

3.1.6	UART106: Multiplexer Close-down (TE initiated)	99

3.1.7	UART107: Retransmission in Multiplexer Close-down (UE initiated, Timeout T2)	100

3.1.8	UART108: Multiplexer Close-down (UE initiated) - No Response	101

3.1.9	UART109: Start Multiplexer with previous data transfer	104

3.2	DLC Establishment	106

3.2.1	UART123: DLC Establishment Requested by TE	106

3.2.2	UART124: MMI Confirmation of TE Initiated DLC	107

3.2.3	UART125: MMI Cancellation of TE Initiated DLC	108

3.2.4	UART126: Second DLC Successfully Requested by TE	109

3.2.5	UART127: DTI Assignment on first DLC	110

3.2.6	UART128: DTI Assignment on second DLC	111

3.2.7	UART129: DCD request after DTI configuration	112

3.3	Multiplexed DLC Data Transmission	115

3.3.1	UART131: Sending Data on Single DLC (UE->TE)	115

3.3.2	UART132: Receiving Data on Single DLC (TE->UE)	116

3.3.3	UART133: Mixed Sending/Receiving of Data on Two DLCs	117

3.3.4	UART134: Receiving Data on Single DLC (TE->UE) with Flow Control	118

3.4	Unsupported Features/Commands	119

3.4.1	UART140: Reception of Unsupported Command	119

3.5	Modem Status Commands	120

3.5.1	UART150: Reception of MSC	120

3.5.2	UART151: Reception of Break-MSC	121

3.5.3	UART152: Sending Break-MSC	123

3.5.4	UART153: Sending Ring-MSC	124

3.5.5	UART154: Retransmission of Break-MSC	125

3.5.6	UART155: Flow-Control-MSC on reception side	126

3.6	DLC Release	129

3.6.1	UART160: DLC Release Requested by ACI	129

3.6.2	UART161: Confirmation of DLC Release (initiated by UE)	130

3.6.3	UART162: DLC Release Requested by TE	131

3.6.4	UART163: Data Received for Already Released Channel	132

3.6.5	UART164: No Response to Retransmissions of DLC DISC Frame	133

3.6.6	UART165: ACI Initiated Multiplexer Close-down (icl. DLC release)	135

3.6.7	UART166: ACI Initiated Multiplexer Close-down (icl. DLC release and DTI channel)	137

3.6.8	UART167: ACI Initiated Multiplexer Close-down (icl. DLC release and two DTI channels)	139

3.6.9	UART168: DLC Release Requested by TE (two MUX channels)	141

3.6.10	UART169: DLC Release Requested by ACI, following a DTI2_DISCONNECT_REQ	142

3.7	Handling of Invalid Frames	143

3.7.1	UART170: Reception of Corrupted Frame	143

3.7.2	UART171: Reception of Frame for Unknown Channel	144

3.7.3	UART172: Reception of Corrupted Message	145

3.8	HDLC Escape Characters in Multiplexer Frames	146

3.8.1	UART180: Establishment of two additional Multiplexer connections	146

3.8.2	UART181: HDLC Escape character in Address field and Information field	150

3.8.3	UART182: HDLC Escape character in Control field and FCS field	155

4	Escape Sequence Tests (UART300 - UART400)	159

4.1	Setup Escape Sequence Tests	159

4.1.1	UART300: Additional Setup for Escape Sequence Tests	159

4.2	Errors During Escape Sequence Detection	159

4.2.1	UART301: Leading Guard Period Too Short	159

4.2.2	UART302: Escape Sequence Character Mismatch	161

4.2.3	UART303: Escape Sequence Characters Duration Too Long	163

4.2.4	UART304: Trailing Guard Period Too Short	166

4.2.5	UART305: Escape Sequence Character Mismatch without Flow Control primitive	169

4.3	Successful Escape Sequence Detection	171

4.3.1	UART310: Single Connection	171

4.3.2	UART311: Establishment of A Third Multiplexer Connection	173

4.3.3	UART312: Escape Sequence on Three DLCs	175

4.3.4	UART313: Single Connection without FLow Control primitive	181

# Document Control

ã Copyright Texas Instruments AG, 2000-2026.

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Texas Instruments AG reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Texas Instruments AG. Texas Instruments AG accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a licence agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Texas Instruments AG.

Texas Instruments AG

Alt Moabit 90a

10559 Berlin

Germany

Telephone:	+49.30.3983-0

Fax:	+49.30.3983-1300

Internet:	http://www.ti.com

E-mail:	gsm@ti.de

## Document History

Document Id.	Date	Author	Remarks

8441.412.00.001	20-Sep-2000	STW	Initial

8441.412.00.002	27-Jul-2001	IH	add testcases for escape sequence detection

8441.412.00.003	11-Sep-2001	STW	extended internal buffer sizes

8441.412.00.004	08-Oct-2001	TVO	adapted for use with DTI2 SAP/DTILIB

8441.412.00.005	17-Jan-2002	STW	mark testcase 110-118 as "not yet implemented"

8441.412.00.006	12-Mar-2002	TVO	Rename dti and parameter primitives, add UART_PARAMETERS_IND

8441.412.00.007	16-Jan-2003	STW	Error corrections

8441.412.00.008	24-Jan-2003	STW	add TCs for HDLC Escape characters

## References

[1]	GSM 05.02 version 8.0.0 Release 1999
Digital cellular telecommunications system (Phase 2+);
Multiplexing and multiple access on the radio path

[2]	GSM 04.60 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station (MS) - Base Station System (BSS) interface;
Radio Link Control/ Medium Access Control (RLC/MAC) protocol

[3]	GSM 04.08 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface layer 3 specification

[4]	GSM 03.64 version 6.1.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Overall description of the GPRS radio interface; Stage 2

[5]	GSM 03.60 version 6.3.1 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Service description; Stage 2

[6]	GSM 04.07 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface signalling layer 3; General aspects

[7]	GSM 04.64 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station - Serving GPRS Support Node (MS-SGSN)
Logical Link Control (LLC) layer specification

[8]	GSM 05.08 version 6.4.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem link control

[9]	GSM 05.10 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem synchronization

[10]	GSM 03.20 TS 100 929: July 1998 (GSM 03.20 version 6.0.1)
Security related network functions, ETSI

[11]	Draft GSM 03.22: August 1998 (GSM 03.22 version 6.1.0)
Functions related to Mobile Station (MS) in idle mode and group receive mode, ETSI

[12]	GSM 04.65 V6.3.0: Subnetwork Dependant Convergence Protocol

ETSI, March 1999

[13]	ITU-T V42bis ITU-T, Recommendation V.42 bis 1990

[14]	GSM 09.60 GPRS Tunneling Protocol (GTP) across the Gn and Gp Interface

[15]	RFC 1661 IETF STD 51 July 1994
The Point-to-Point Protocol (PPP)

[16]	RFC 1662 IETF STD 51 July 1994
PPP in HDLC-like Framing

[17]	RFC 1570 January 1994
PPP LCP Extensions

[18]	RFC 1989 August 1996
PPP Link Quality Monitoring

[19]	RFC 1332 May 1992
The PPP Internet Protocol Control Protocol (IPCP)

[20]	RFC 1877 December 1995
PPP IPCP Extensions for Name Server Addresses

[21]	RFC 2153 May 1997
PPP Vendor Extensions

[22]	RFC 1334 October 1992
PPP Authentication Protocols (for Password Authentication Protocol only)

[23]	RFC 1994 August 1996
PPP Challenge Handshake Authentication Protocol (CHAP)

[24]	TIA/EIA-136-370
Packet-Data Services – Enhanced General Packet Radio for TIA/EIA-136 (EGPRS-136) - Overview, Telecommunications Industry Association

[25]	TIA/EIA-136-376
Packet-Data Services – EGPRS-136 Mobility Management, Telecommunications Industry Association

[26]	TIA/EIA-136-972
Packet-Data Services – Stage 2 Description, Telecommunications Industry Association

## Abbreviations

ACI	Application Control Interface
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CCI	Compression and Ciphering Interface
CHAP	Challenge Handshake Authentication Protocol
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DCOMP	Identifier of the user data compression algorithm used for the N-DPU
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

E	Extension bit
EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GACI	GPRS Application Control Interface
GMM	GPRS Mobility Management
GP	Guard Period
GRR	GPRS RR
GSM	Global System for Mobile Communication

HDLC	High-level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
IP	Internet Protocol
IPCP	Internet Protocol Control Protocol 
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Ciphering Key

L	Length Indicator
LAI	Location Area Information
LCP 	Link Control Protocol 
LISR	Low level Interrupt Service Routine
LLC	Logical Link Control
LPD	Link Protocol Discriminator
LQM	Link Quality Monitoring

M	More bit used to indicate the last segment of N-DPU
MAC	Medium Access Control
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MT	Mobile Termination

N(R)	Receive Number
N(S)	Send Number
NC	Network Control
NCC	National Colour Code
NCP 	Network Control Protocol
NECI	New Establishment Causes included
N-PDU	Network Protocol Data Unit
NSAPI	Network Layer Service Access Point Identifier

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PACCH	Packet Associated Control Channel
PAP	Password Authentication Protocol
PBCCH	Packet BCCH
PCCCH	Packet CCCH
PCOMP	Identifier of the protocol control information compression algorithm used for the N-DPU
PDCH	Packet Data Channel
PDP	Packet Data Protocol e.g. IP or X.25
PDTCH	Packet Data Traffic Channel
PRACH	Packet RACH
PSI	Packet System Information
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Data Unit
PL	Physical Layer
PLMN	Public Land Mobile Network
PPC	Packet Physical Convergence
PPP 	Point-to-Point Protocol
PTP	Point to Point

QoS	Quality of Service

RACH	Random Access Channel
REJ	Reject Frame
RLC	Radio Link Control
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Stand alone Dedicated Control Channel
SDU	Service Data Unit
SGSN	Serving GPRS Support Node
SIM	Subscriber Identity Module
SM	Session Management
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SNDCP	Subnetwork Dependant Convergence Protocol
SNSM	SNDCP-SM
SS	Supplementary Services

TAP	Test Application Program
TBF	Temporary Block Flow
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TCP	Transmission Control Protocol
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TFI	Temporary Flow Identifier
TLLI	Temporary Logical Link Identifier
TMSI	Temporary Mobile Subscriber Identity
TOM	Tunnelling of Messages
TQI	Temporary Queuing Identifier

UA	Unnumbered Acknowledgement Frame
UART	Universal Asynchronous Receiver Transmitter
UI	Unnumbered Information Frame
USF	Uplink State Flag

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visited Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Parameters

/*

## Primitive elements

*/

BYTE UART_DEVICE		0			/* number of UART device */

BYTE UART_INSTANCES		1			/* total number of UART instances */

BYTE DLCI_A			42			/* example data link connection identifier */

BYTE DLCI_B			1			/* second data connection identifier */

BYTE DLCI_C			3			/* third data connection identifier */

BYTE DLCI_D			0x04			/* special HDLC escape forcing DLCI */

BYTE DLCI_E			0x1f			/* special HDLC escape forcing DLCI */

BYTE DLCI_ANY			0			/* a dummy dlci which is used when dlci is unnecessary */

BYTE DLCI_CONTROL		0			/* data link connection identifier for control channel */

LONG UART_PEER_NAME_DUMMY	0			/* dummy for pointer to peer name string */

BYTE P_ID_PEER_A		DTI_PID_UOS		/* Procotol ID of protocol towards peer A */

BYTE P_ID_PEER_B		DTI_PID_UOS		/* Protocol ID of protocol towards peer B */

BYTE UART_BREAK_LEN		0x02			/* length of break signal */

BYTE LINK_ID_1			0x22			/* an example channel id */

BYTE LINK_ID_2			0x45			/* another example channel id */

BYTE LINK_ID_3			0x33			/* another example channel id */

BYTE LINK_ID_4			0x44			/* another example channel id */

BYTE LINK_ID_5			0x55			/* another example channel id */

/* these are __in no way__ real link_ids. Instead, the variable is used for communication between */

/* the entity and the test environment, here. In previous versions, the tui was used for this, but since */

/* DTI2 primitives do not contain such a parameter any more, the dirty work has been passed on */

/* to the link_id ….. */

SHORT LINK_READDATA_PORT_1		0		/* misused link_id for read_data call */

SHORT LINK_DISABLE_PORT_1		1		/* misused link_id for disable call */

SHORT LINK_ENABLE_PORT_1		2		/* misused link_id for enable call */

SHORT LINK_WRITEDATA_PORT_1		3		/* misused link_id for write_data call */

SHORT LINK_READDATA_PORT_2		10		/* misused link_id for read_data call */

SHORT LINK_DISABLE_PORT_2		11		/* misused link_id for disable call */

SHORT LINK_ENABLE_PORT_2		12		/* misused link_id for enable call */

SHORT LINK_WRITEDATA_PORT_2		13		/* misused link_id for write_data call */

SHORT LINK_UART_OUT_PORT_1		0		/* misused link_id of UART_OUT test interface */

SHORT LINK_UART_OUT_PORT_2		10		/* misused link_id of UART_OUT test interface */

SHORT LINK_PORT_THRESHOLD		9		/* to be able to distinguish between port 1 and port2 */

BYTE DTI_PID_UART_OUT			0		/* protocol identifier of UART_OUT test interface */

BYTE 	ESC_SEQ_CHAR_PLUS		0x2B		/* Escape Sequence Char '+' */

SHORT	GUARD_PERIOD_1000		1000		/* Escape Sequence Guard Period */

/*

Set Default Timer Values for Simulation Testing - Original Values too Fast for TAP

*/

SHORT UART_MUX_T1_DEFAULT_TEST	30

SHORT UART_MUX_T2_DEFAULT_TEST	90

SHORT UART_MUX_T3_DEFAULT_TEST	10

/*

## Struct elements

*/

BYTE COMPAR_SPEED_57600	UART_IO_SPEED_57600			/* baudrate of 57600 */

BYTE COMPAR_SPEED_19200	UART_IO_SPEED_19200			/* baudrate of 19200 */

BYTE COMPAR_SPEED_9600	UART_IO_SPEED_9600			/* baudrate of 9600 */

BYTE COMPAR_BPC_8		UART_IO_BPC_8				/* 8 bit per character */

BYTE COMPAR_NSB_1		UART_IO_SB_1				/* 1 stop bit */

BYTE COMPAR_PARITY_EVEN	UART_IO_PA_EVEN			/* even parity */

BYTE COMPAR_PARITY_NO		UART_IO_PA_NONE		/* no parity no space */

BYTE COMPAR_XON_CHAR		UART_IO_XON_DEFAULT		/* default value for XON character */

BYTE COMPAR_XOFF_CHAR		UART_IO_XOFF_DEFAULT		/* default value for XOFF character */

BYTE FCS_DUMMY_OK		0xcf					/* a valid FCS: UART_GOOD_FCS */

BYTE FCS_DUMMY_INVALID	0xdf					/* an invalid FCS */

BYTE UART_UNKNOWN_COMMAND	0xd7					/* an unsupported command type*/

SHORT OFFSET_0			0x00					/* SDU offset of 0 */

SHORT LEN_HELLO		0x40					/* length of hello strings */

/*

## Fields

### Strings

*/

/* String Hello1

| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a | String "Hello1\n" |
| --- | --- |

*/

FIELD(STRING_HELLO1)

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a

ENDFIELD(STRING_HELLO1, 8)

/* String Hello2

| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a | String "Hello2\n" |
| --- | --- |

*/

FIELD(STRING_HELLO2)

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a

ENDFIELD(STRING_HELLO2, 8)

/* String Hello3

| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x33, 0x0d, 0x0a | String "Hello3\n" |
| --- | --- |

*/

FIELD(STRING_HELLO3)

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x33, 0x0d, 0x0a

ENDFIELD(STRING_HELLO3, 8)

/* String h

| 0x68 | String "h" |
| --- | --- |

*/

FIELD(STRING_H)

0x68

ENDFIELD(STRING_H, 1)

/* String e

| 0x65 | String "e" |
| --- | --- |

*/

FIELD(STRING_E)

0x65

ENDFIELD(STRING_E, 1)

/* String l

| 0x6c | String "l" |
| --- | --- |

*/

FIELD(STRING_L)

0x6c

ENDFIELD(STRING_L, 1)

/* String o

| 0x6f | String "o" |
| --- | --- |

*/

FIELD(STRING_O)

0x6f

ENDFIELD(STRING_O, 1)

/* Field String Hello1 SDU

| 0x40, 0x00 | SDU length in bit (8 Byte = 64 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a | String "Hello1\n\r" |

*/

FIELD(STRING_HELLO1_SDU)

0x40, 0x00,

0x00, 0x00,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a

ENDFIELD(STRING_HELLO1_SDU, 12)

/* Field String 15x Hello1 SDU

| 0xc0, 0x03 | SDU length in bit (120 Byte = 960 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a | 15 times String "Hello1\n\r" |

*/

FIELD(STRING_15XHELLO1_SDU)

0xc0, 0x03,

0x00, 0x00,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a

ENDFIELD(STRING_15XHELLO1_SDU, 124)

/* Field String He SDU

| 0x10, 0x00 | SDU length in bit (2 Byte = 16 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x48, 0x65 | String "He" |

*/

FIELD(STRING_HE_SDU)

0x10, 0x00,

0x00, 0x00,

0x48, 0x65

ENDFIELD(STRING_HE_SDU, 6)

/* Field String 30 octets of Hello1\n\r SDU

| 0xf0, 0x00 | SDU length in bit (30 Byte = 240 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31 | first 30 octets of 15 times String "Hello1\n\r" |

*/

FIELD(STRING_1ST30HELLO1_SDU)

0xf0, 0x00,

0x00, 0x00,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31

ENDFIELD(STRING_1ST30HELLO1_SDU, 34)

/* Field String ll SDU

| 0x10, 0x00 | SDU length in bit (2 Byte = 16 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6c, 0x6c | String "ll" |

*/

FIELD(STRING_LL_SDU)

0x10, 0x00,

0x00, 0x00,

0x6c, 0x6c

ENDFIELD(STRING_LL_SDU, 6)

/* Field String second 30 octets of Hello1\n\r SDU

| 0xf0, 0x00 | SDU length in bit (30 Byte = 240 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c | second 30 octets of 15 times String "Hello1\n\r" |

*/

FIELD(STRING_2ND30HELLO1_SDU)

0xf0, 0x00,

0x00, 0x00,

0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c

ENDFIELD(STRING_2ND30HELLO1_SDU, 34)

/* Field String o1 slash SDU

| 0x18, 0x00 | SDU length in bit (3 Byte = 24 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6f, 0x31, 0x0d | String "o1\" |

*/

FIELD(STRING_O1_SLASH_SDU)

0x18, 0x00,

0x00, 0x00,

0x6f, 0x31, 0x0d

ENDFIELD(STRING_O1_SLASH_SDU, 7)

/* Field String third 40 octets of Hello1\n\r SDU

| 0x40, 0x01 | SDU length in bit (40 Byte = 320 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c | third 40 octets of 15 times String "Hello1\n\r" |

*/

FIELD(STRING_3RD40HELLO1_SDU)

0x40, 0x01,

0x00, 0x00,

0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c

ENDFIELD(STRING_3RD40HELLO1_SDU, 44)

/* Field String n SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x0a | String "n" |

*/

FIELD(STRING_N_SDU)

0x08, 0x00,

0x00, 0x00,

0x0a

ENDFIELD(STRING_N_SDU, 5)

/* Field String reset of Hallo1\n\r SDU

| 0xa0, 0x00 | SDU length in bit (20 Byte = 160 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a | rest octets of 15 times String "Hello1\n\r" |

*/

FIELD(STRING_RESTHALLO1_SDU)

0xa0, 0x00,

0x00, 0x00,

0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a

ENDFIELD(STRING_RESTHALLO1_SDU, 24)

/* Field String hHe SDU

| 0x18, 0x00 | SDU length in bit (3 Byte = 24 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x68, 0x48,0x65 | String "hHe" |

*/

FIELD(STRING_HHE_SDU)

0x18, 0x00,

0x00, 0x00,

0x68, 0x48, 0x65

ENDFIELD(STRING_HHE_SDU, 7)

/* Field String 10x h and 20 octets of Hello1 SDU

| 0xf0, 0x00 | SDU length in bit (30 Byte = 240 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c | 10 times String "h" and 20 octets of "Hello1\n\r" String |

*/

FIELD(STRING_1ST30HHELLO1_SDU)

0xf0, 0x00,

0x00, 0x00,

0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c

ENDFIELD(STRING_1ST30HHELLO1_SDU, 34)

/* Field String llo1 SDU

| 0x20, 0x00 | SDU length in bit (4 Byte = 32 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6c, 0x6c, 0x6f, 0x31 | String "llo1" |

*/

FIELD(STRING_LLO1_SDU)

0x20, 0x00,

0x00, 0x00,

0x6c, 0x6c, 0x6f, 0x31

ENDFIELD(STRING_LLO1_SDU, 8)

/* Field String second 40 octets of Hallo1\n\r SDU

| 0x40, 0x01 | SDU length in bit (40 Byte = 320 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c | second part: 40 octets of "Hallo1\n\r" String |

*/

FIELD(STRING_2ND40HHALLO1_SDU)

0x40, 0x01,

0x00, 0x00,

0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c

ENDFIELD(STRING_2ND40HHALLO1_SDU, 44)

/* Field String \n SDU

| 0x10, 0x00 | SDU length in bit (2 Byte = 16 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x0d, 0x0a | String "\n" |

*/

FIELD(STRING_SLASH_N_SDU)

0x10, 0x00,

0x00, 0x00,

0x0d, 0x0a

ENDFIELD(STRING_SLASH_N_SDU, 6)

/* Field String third 50 octets of Hallo1\n\r SDU

| 0x90, 0x01 | SDU length in bit (50 Byte = 400 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31 | third part: 50 octets of "Hallo1\n\r" String |

*/

FIELD(STRING_3RD50HHALLO1_SDU)

0x90, 0x01,

0x00, 0x00,

0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31

ENDFIELD(STRING_3RD50HHALLO1_SDU, 54)

/* Field String reset of Hallo1\n\r SDU

| 0x50, 0x00 | SDU length in bit (10 Byte = 80 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a | rest of "Hallo1\n\r" String |

*/

FIELD(STRING_RESTHHALLO1_SDU)

0x50, 0x00,

0x00, 0x00,

0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a

ENDFIELD(STRING_RESTHHALLO1_SDU, 14)

/* Field String Hello2 SDU

| 0x40, 0x00 | SDU length in bit (8 Byte = 64 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a | String "Hello2\n" |

*/

FIELD(STRING_HELLO2_SDU)

0x40, 0x00,

0x00, 0x00,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a

ENDFIELD(STRING_HELLO2_SDU, 12)

/* Field String 0x10 0x11 0x12 SDU

| 0x18, 0x00 | SDU length in bit (3 Byte = 24 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x10, 0x11, 0x12 | String "0x10 0x11 0x12" |

*/

FIELD(STRING_101112_SDU)

0x18, 0x00,

0x00, 0x00,

0x10, 0x11, 0x12

ENDFIELD(STRING_101112_SDU, 7)

/* Field String 0x13 0x14 0x15 SDU

| 0x18, 0x00 | SDU length in bit (3 Byte = 24 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x13, 0x14, 0x15 | String "0x13 0x14 0x15" |

*/

FIELD(STRING_131415_SDU)

0x18, 0x00,

0x00, 0x00,

0x13, 0x14, 0x15

ENDFIELD(STRING_131415_SDU, 7)

/* Field String 0x7c 0x7d 0x7e SDU

| 0x18, 0x00 | SDU length in bit (3 Byte = 24 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7c, 0x7d, 0x7e | String "0x7c 0x7d 0x7e" |

*/

FIELD(STRING_7c7d7e_SDU)

0x18, 0x00,

0x00, 0x00,

0x7c, 0x7d, 0x7e

ENDFIELD(STRING_7c7d7e_SDU, 7)

/* Field String 0x7e 0x7f 0x80 SDU

| 0x18, 0x00 | SDU length in bit (3 Byte = 24 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e, 0x7f, 0x80 | String "0x7e 0x7f 0x80" |

*/

FIELD(STRING_7e7f80_SDU)

0x18, 0x00,

0x00, 0x00,

0x7e, 0x7f, 0x80

ENDFIELD(STRING_7e7f80_SDU, 7)

/* Field String Hello3 SDU

| 0x40, 0x00 | SDU length in bit (8 Byte = 64 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x33, 0x0d, 0x0a | String "Hello3\n" |

*/

FIELD(STRING_HELLO3_SDU)

0x40, 0x00,

0x00, 0x00,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x33, 0x0d, 0x0a

ENDFIELD(STRING_HELLO3_SDU, 12)

/* Field String h SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x68 | String "h" |

*/

FIELD(STRING_H_SDU)

0x08, 0x00,

0x00, 0x00,

0x68

ENDFIELD(STRING_H_SDU, 5)

/* Field String h++h SDU

| 0x20, 0x00 | SDU length in bit (4 Byte = 32 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x68, 0x2b, 0x2b, 0x68 | String "h++h" |

*/

FIELD(STRING_HPLUSPLUSH_SDU)

0x20, 0x00,

0x00, 0x00,

0x68, 0x2b, 0x2b, 0x68

ENDFIELD(STRING_HPLUSPLUSH_SDU, 8)

/* Field String 10 times h SDU

| 0x50, 0x00 | SDU length in bit (10 Byte = 80 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68 | String "hhhhhhhhhh" |

*/

FIELD(STRING_10H_SDU)

0x50, 0x00,

0x00, 0x00,

0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68, 0x68

ENDFIELD(STRING_10H_SDU, 14)

/* Field String A SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x41 | String "A" |

*/

FIELD(STRING_A_SDU)

0x08, 0x00,

0x00, 0x00,

0x41

ENDFIELD(STRING_A_SDU, 5)

/* Field String 3 time "+" and "A" SDU

| 0x20, 0x00 | SDU length in bit (4 Byte = 32 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x2b, 0x2b, 0x2b, 0x41 | String "+++A" |

*/

FIELD(STRING_3PLUSA_SDU)

0x20, 0x00,

0x00, 0x00,

0x2b, 0x2b, 0x2b, 0x41

ENDFIELD(STRING_3PLUSA_SDU, 8)

/* Field String AT SDU

| 0x20, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x41, 0x54, 0x0D, 0x0A | String "AT\n" |

*/

FIELD(STRING_AT_SDU)

0x20, 0x00,

0x00, 0x00,

0x41, 0x54, 0x0D, 0x0A,

ENDFIELD(STRING_AT_SDU, 8)

/* Field String e SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x65 | String "e" |

*/

FIELD(STRING_E_SDU)

0x08, 0x00,

0x00, 0x00,

0x65

ENDFIELD(STRING_E_SDU, 5)

/* Field String l SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6c | String "l" |

*/

FIELD(STRING_L_SDU)

0x08, 0x00,

0x00, 0x00,

0x6c

ENDFIELD(STRING_L_SDU, 5)

/* Field String o SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6f | String "o" |

*/

FIELD(STRING_O_SDU)

0x08, 0x00,

0x00, 0x00,

0x6f

ENDFIELD(STRING_O_SDU, 5)

/* Empty String

| 0x00, 0x00 | SDU length in bit (0 Byte = 0 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |

*/

FIELD(EMPTY_STRING_SDU)

0x00, 0x00,

0x00, 0x00

ENDFIELD(EMPTY_STRING_SDU, 4)

/* Field String Hello1Hello2 SDU

| 0x80, 0x00 | SDU length in bit (16 Byte = 128 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a | String "Hello1\nHello2\n" |

*/

FIELD(STRING_HELLO1_HELLO2_SDU)

0x80, 0x00,

0x00, 0x00,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a

ENDFIELD(STRING_HELLO1_HELLO2_SDU, 20)

/* Field String 8x"ABCDEFGH" and 1x"h+cops"

| 0x30, 0x02 | SDU length in bit (70 Byte = 560 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, 0x68, 0x2b, 0x63, 0x6f, 0x70, 0x73 | 8x String "ABCDEFGH" and 1x String "h+cops" |

*/

FIELD(STRING_PLUSCOPS_SDU)

0x30, 0x02,

0x00, 0x00,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x68, 0x2b, 0x63, 0x6f, 0x70, 0x73

ENDFIELD(STRING_PLUSCOPS_SDU, 74)

/* Field String + SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x2B | String "+" |

*/

FIELD(STRING_PLUS_SDU)

0x08, 0x00,

0x00, 0x00,

0x2B

ENDFIELD(STRING_PLUS_SDU, 5)

/* Field String ++ SDU

| 0x10, 0x00 | SDU length in bit (2 Byte = 16 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x2B, 0x2B | String "++" |

*/

FIELD(STRING_PLUSPLUS_SDU)

0x10, 0x00,

0x00, 0x00,

0x2B, 0x2B

ENDFIELD(STRING_PLUSPLUS_SDU, 6)

/* Field String +++ SDU

| 0x18, 0x00 | SDU length in bit (3 Byte = 24 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x2B, 0x2B, 0x2B | String "+++" |

*/

FIELD(STRING_ESC_SEQ_SDU)

0x18, 0x00,

0x00, 0x00,

0x2B, 0x2B, 0x2B

ENDFIELD(STRING_ESC_SEQ_SDU, 7)

/* Field String of 64 octets SDU

| 0x00, 0x02 | SDU length in bit (64 Byte = 200 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48 | String "ABCDEFGHABCDEFGHABCDEFGHABCDEFGHABCDEFGHABCDEFGHABCDEFGHABCDEFGH" |

*/

FIELD(STRING_64_SDU)

0x00, 0x02,

0x00, 0x00,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48

ENDFIELD(STRING_64_SDU, 68)

/*

### Frames

#### Multiplexer Start Up / Close Down

*/

/* SDU: MUX Startup (TE initiated) SABM Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MUX_START0)

0x28, 0x00,

0x00, 0x00,

0x7e,

0x03,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_MUX_START0, 9)

/* SDU: Command to close down multiplexer TE->UE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xc3 | Type: Multiplexer Close Down Cmd |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CMD_TE2UE_CLOSE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xc3,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_CMD_TE2UE_CLOSE, 11)

/* SDU: Command to close down multiplexer UE->TE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0xc3 | Type: Multiplexer Close Down Cmd |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CMD_UE2TE_CLOSE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xc3,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_CMD_UE2TE_CLOSE, 11)

/* SDU: Confirm Multiplexer Close Down, MMI initiated

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0xc1 | Type: Multiplexer Close Down Res |
| 0x01 | Length is 0, the EA bit is set to 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_RES_TE2UE_CLOSE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xc1,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_RES_TE2UE_CLOSE, 11)

/* SDU: Indicate Multiplexer Close Down, TE initiated, Variant A, Multiplexer Close Cmd

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xc3 | Type: Multiplexer Close Down Cmd |
| 0x01 | Length |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CMD_TE2UE_CLOSE0)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xc3,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_CMD_TE2UE_CLOSE0, 11)

/* SDU: Respond to Multiplexer Close Down, TE initiated, Variant A, Multiplexer Close Cmd

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xc1 | Type: Multiplexer Close Down Cmd |
| 0x01 | Length |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_RES_UE2TE_CLOSE0)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xc1,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_RES_UE2TE_CLOSE0, 11)

/* SDU: Indicate Multiplexer Close Down, TE initiated, Variant B, DISC command on DLCI 0

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0x53 | Control Field: DISC command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CMD_TE2UE_CLOSE1)

0x28, 0x00,

0x00, 0x00,

0x7e,

0x03,

0x53,

(0XCF),

0x7e

ENDFIELD(SDU_CMD_TE2UE_CLOSE1, 9)

/*

#### Unnumbered Acknowledgements (UA)

*/

/* SDU: UA Response from UE to TE

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0x73 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UA_UE2TE)

0x28, 0x00,

0x00, 0x00,

0x7e,

0x03,

0x73,

(0XCF),

0x7e

ENDFIELD(SDU_UA_UE2TE, 9)

/* SDU: Negative UA Response from UE to TE

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0x63 | Control Field: neg. UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_NEG_UA_UE2TE)

0x28, 0x00,

0x00, 0x00,

0x7e,

0x03,

0x63,

(0XCF),

0x7e

ENDFIELD(SDU_NEG_UA_UE2TE, 9)

/* SDU: UA Response from TE to UE

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0x73 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UA_TE2UE)

0x28, 0x00,

0x00, 0x00,

0x7e,

0x01,

0x73,

(0XCF),

0x7e

ENDFIELD(SDU_UA_TE2UE, 9)

/* SDU: UA Response from UE to TE for DLCI A

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x73 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UA_UE2TE_DLCI_A)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0x73,

(0XCF),

0x7e

ENDFIELD(SDU_UA_UE2TE_DLCI_A, 9)

/* SDU: UA Response from TE to UE for DLCI A

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x01 | Address Field: DLCI_A, C/R 0, EA 1 |
| 0x73 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UA_TE2UE_DLCI_A)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x01,

0x73,

(0XCF),

0x7e

ENDFIELD(SDU_UA_TE2UE_DLCI_A, 9)

/* SDU: Negative UA Response from UE to TE for DLCI A

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x63 | Control Field: Negative UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_NEG_UA_UE2TE_DLCI_A)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0x63,

(0XCF),

0x7e

ENDFIELD(SDU_NEG_UA_UE2TE_DLCI_A, 9)

/* SDU: Negative UA Response from TE to UE for DLCI A

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x01 | Address Field: DLCI_A, C/R 0, EA 1 |
| 0x63 | Control Field: Negative UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_NEG_UA_TE2UE_DLCI_A)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x01,

0x63,

(0XCF),

0x7e

ENDFIELD(SDU_NEG_UA_TE2UE_DLCI_A, 9)

/* SDU: UA Response from UE to TE for DLCI B

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B << 2) ¦ 0x03 | Address Field: DLCI_B, C/R 1, EA 1 |
| 0x73 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UA_UE2TE_DLCI_B)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x03,

0x73,

(0XCF),

0x7e

ENDFIELD(SDU_UA_UE2TE_DLCI_B, 9)

/* SDU: UA Response from TE to UE for DLCI B

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B << 2) ¦ 0x01 | Address Field: DLCI_B, C/R 0, EA 1 |
| 0x73 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UA_TE2UE_DLCI_B)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x01,

0x73,

(0XCF),

0x7e

ENDFIELD(SDU_UA_TE2UE_DLCI_B, 9)

/* SDU: UA Response from UE to TE for DLCI C

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_C << 2) ¦ 0x03 | Address Field: DLCI_C, C/R 1, EA 1 |
| 0x73 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UA_UE2TE_DLCI_C)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_C << 2) | 0x03,

0x73,

(0XCF),

0x7e

ENDFIELD(SDU_UA_UE2TE_DLCI_C, 9)

/* SDU: UA Response from UE to TE for DLCI D

| 0x30, 0x00 | SDU length in bit (6 Bytes = 48 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x7d, ((DLCI_D << 2) ¦ 0x03) ^ 0x20 | Address Field: DLCI_D, C/R 1, EA 1 |
| 0x73 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UA_UE2TE_DLCI_D)

0x30, 0x00,

0x00, 0x00,

0x7e,

0x7d, ((DLCI_D << 2) | 0x03) ^ 0x20,

0x73,

(0XCF),

0x7e

ENDFIELD(SDU_UA_UE2TE_DLCI_D, 10)

/* SDU: UA Response from UE to TE for DLCI E

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_E << 2) ¦ 0x03 | Address Field: DLCI_E, C/R 1, EA 1 |
| 0x73 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UA_UE2TE_DLCI_E)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_E << 2) | 0x03,

0x73,

(0XCF),

0x7e

ENDFIELD(SDU_UA_UE2TE_DLCI_E, 9)

/*

#### Flow Control

*/

/* SDU: Flow Control On Command, TE -> UE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xa3 | Type: Flow On command |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FLOWONCMD_TE2UE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xa3,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_FLOWONCMD_TE2UE, 11)

/* SDU: Flow Control On Command, UE -> TE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0xa3 | Type: Flow On command |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FLOWONCMD_UE2TE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xa3,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_FLOWONCMD_UE2TE, 11)

/* SDU: Flow Control On Response, UE -> TE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xa1 | Type: Flow On response |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FLOWONRES_UE2TE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xa1,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_FLOWONRES_UE2TE, 11)

/* SDU: Flow Control On Response, TE -> UE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xa1 | Type: Flow On response |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FLOWONRES_TE2UE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xa1,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_FLOWONRES_TE2UE, 11)

/* SDU: Flow Control Off Command, TE -> UE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0x63 | Type: Flow Off command |
| 0x01 | Length is 0, EA bit is 0 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FLOWOFFCMD_TE2UE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0x63,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_FLOWOFFCMD_TE2UE, 11)

/* SDU: Flow Control Off Command, UE -> TE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0x61 | Type: Flow Off command |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FLOWOFFCMD_UE2TE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0x61,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_FLOWOFFCMD_UE2TE, 11)

/* SDU: Flow Control Off Response, UE -> TE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0x61 | Type: Flow Off response |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FLOWOFFRES_UE2TE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0x61,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_FLOWOFFRES_UE2TE, 11)

/* SDU: Flow Control Off Response, TE -> UE

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0x61 | Type: Flow Off response |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FLOWOFFRES_TE2UE)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0x61,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_FLOWOFFRES_TE2UE, 11)

/*

#### Power Saving Control (PSC)

*/

/* SDU: Request Power Saving Mode, MMI initiated

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0x43 | Type |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CMD_UE2TE_PSM)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0x43,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_CMD_UE2TE_PSM, 11)

/* SDU: Positive Response to Power Saving Mode, MMI initiated

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0x41 | Type |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_RES_TE2UE_PSM)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0x41,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_RES_TE2UE_PSM, 11)

/* SDU: Indicate Power Saving Mode, Peer initiated

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0x43 | Type |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CMD_TE2UE_PSM)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0x43,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_CMD_TE2UE_PSM, 11)

/* SDU: Accept Power Saving Mode, Peer initiated

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0x41 | Type |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_RES_UE2TE_PSM)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0x43,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_RES_UE2TE_PSM, 11)

/* SDU: Wake Up from Power Saving Mode, Peer initiated

| 0xe0, 0x00 | SDU length in bit (28 Bytes =224 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e...0x7e | Enough Flags 0111 1110 in order to wake up the multiplexer |

*/

FIELD(SDU_FLAGS_TE2UE)

0xe0, 0x00,

0x00, 0x00,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e

ENDFIELD(SDU_FLAGS_TE2UE, 32)

/* SDU: Accept Wake Up, Peer initiated

| 0x30, 0x00 | SDU length in bit (6 Bytes = 48 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 3, EA 1 |
| 0xff | Control Field: UIH cmd/res frame |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_RES_UE2TE_WAKEUP)

0x30, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_RES_UE2TE_WAKEUP, 10)

/* SDU: Wake Up from Power Saving Mode, MMI initiated

| 0xe0, 0x00 | SDU length in bit (28 Bytes =224 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e...0x7e | Enough Flags 0111 1110 in order to wake up the peer's multiplexer (the number of flags depends on the time the peer needs to wakeup, max. T3 seconds.) Has to be adapted to specific implementation for this test case. |

*/

FIELD(SDU_FLAGS_UE2TE)

0xe0, 0x00,

0x00, 0x00,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e

ENDFIELD(SDU_FLAGS_UE2TE, 32)

/* SDU: Accept Wake Up, MMI initiated

| 0x30, 0x00 | SDU length in bit (6 Bytes = 48 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH cmd/res frame |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_RES_TE2UE_WAKEUP)

0x30, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_RES_TE2UE_WAKEUP, 10)

/*

#### DLC Establishment / Release

*/

/* SDU: Establish DLC A, TE initiated, SABM Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_ESTABLISH_DLCI_A_TE2UE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_ESTABLISH_DLCI_A_TE2UE, 9)

/* SDU: Establish DLC B, TE initiated, SABM Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B << 2) ¦ 0x03 | Address Field: DLCI_B, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_ESTABLISH_DLCI_B_TE2UE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x03,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_ESTABLISH_DLCI_B_TE2UE, 9)

/* SDU: Establish DLC B, UE initiated, SABM Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B << 2) ¦ 0x01 | Address Field: DLCI_B, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_ESTABLISH_DLCI_B_UE2TE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x01,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_ESTABLISH_DLCI_B_UE2TE, 9)

/* SDU: Establish DLC, UE initiated, SABM Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x01 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_ESTABLISH_DLCI_A_UE2TE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x01,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_ESTABLISH_DLCI_A_UE2TE, 9)

/* SDU: Establish DLC C, TE initiated, SABM Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_C << 2) ¦ 0x03 | Address Field: DLCI_C, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_ESTABLISH_DLCI_C_TE2UE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_C << 2) | 0x03,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_ESTABLISH_DLCI_C_TE2UE, 9)

/* SDU: Establish DLC D, TE initiated, SABM Frame

| 0x30, 0x00 | SDU length in bit (6 Bytes = 48 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x7d, ((DLCI_D << 2) ¦ 0x03) ^ 0x20 | Address Field: DLCI_D, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_ESTABLISH_DLCI_D_TE2UE)

0x30, 0x00,

0x00, 0x00,

0x7e,

0x7d, ((DLCI_D << 2) | 0x03) ^ 0x20,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_ESTABLISH_DLCI_D_TE2UE, 10)

/* SDU: Establish DLC E, TE initiated, SABM Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_E << 2) ¦ 0x03 | Address Field: DLCI_E, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_ESTABLISH_DLCI_E_TE2UE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_E << 2) | 0x03,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_ESTABLISH_DLCI_E_TE2UE, 9)

/* SDU: Disconnect DLC A, TE initiated, DISC Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x53 | Control Field: DISC command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_DISC_DLCI_A_TE2UE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0x53,

(0XCF),

0x7e

ENDFIELD(SDU_DISC_DLCI_A_TE2UE, 9)

/* SDU: Disconnect DLC A, UE initiated, DISC Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x01 | Address Field: DLCI_A, C/R 0, EA 1 |
| 0x53 | Control Field: DISC command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_DISC_DLCI_A_UE2TE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x01,

0x53,

(0XCF),

0x7e

ENDFIELD(SDU_DISC_DLCI_A_UE2TE, 9)

/* SDU: Disconnect DLC B, TE initiated, DISC Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B<< 2) ¦ 0x03 | Address Field: DLCI_B, C/R 1, EA 1 |
| 0x53 | Control Field: DISC command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_DISC_DLCI_B_TE2UE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x03,

0x53,

(0XCF),

0x7e

ENDFIELD(SDU_DISC_DLCI_B_TE2UE, 9)

/* SDU: Disconnect DLC B, UE initiated, DISC Frame

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B<< 2) ¦ 0x01 | Address Field: DLCI_B, C/R 0, EA 1 |
| 0x53 | Control Field: DISC command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_DISC_DLCI_B_UE2TE)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x01,

0x53,

(0XCF),

0x7e

ENDFIELD(SDU_DISC_DLCI_B_UE2TE, 9)

/*

#### Data Frames

*/

/* SDU: Send h String in Information Frame on DLCI A, TE -> UE

| 0x30, 0x00 | SDU length in bit (6 Bytes = 48Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI A, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x68, | String "h" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_H_TE2UE_A)

0x30, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0xef,

0x68,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_H_TE2UE_A, 10)

/* SDU: Send Hello1 String in Information Frame on DLCI A, TE -> UE

| 0x68, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI A, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, | String "Hello1\n" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_HELLO1_TE2UE_A)

0x60, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0xef,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_HELLO1_TE2UE_A, 17)

/* SDU: Send Hello1 String in Information Frame on DLCI B, UE -> TE

| 0x68, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B<< 2) ¦ 0x01 | Address Field: DLCI B, C/R 0, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, | String "Hello1\n" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_HELLO1_UE2TE_B)

0x68, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x01,

0xef,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_HELLO1_UE2TE_B, 17)

/* SDU: Send 0x10 0x11 0x12 in Information Frame on DLCI A, UE -> TE

| 0x48, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x01 | Address Field: DLCI A, C/R 0, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x10, 0x7d, 0x11 ^ 0x20, 0x12, | String "0x10 0x11 0x12" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_101112_UE2TE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x01,

0xef,

0x10, 0x7d, 0x11 ^ 0x20, 0x12,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_101112_UE2TE_A, 13)

/* SDU: Send 0x13 0x14 0x15 in Information Frame on DLCI B, UE -> TE

| 0x48, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B << 2) ¦ 0x01 | Address Field: DLCI B, C/R 0, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x7d, 0x13 ^ 0x20, 0x14, 0x15, | String "0x13 0x14 0x15" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_131415_UE2TE_B)

0x48, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x01,

0xef,

0x7d, 0x13 ^ 0x20, 0x14, 0x15,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_131415_UE2TE_B, 13)

/* SDU: Send 0x7c 0x7d 0x7e in Information Frame on DLCI D, UE -> TE

| 0x58, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x7d, ((DLCI_D << 2) ¦ 0x01) ^ 0x20 | Address Field: DLCI D, C/R 0, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x7c, 0x7d, 0x7d ^ 0x20, 0x7d, 0x7e ^ 0x20, | String "0x7c 0x7d 0x7e" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_7c7d7e_UE2TE_D)

0x58, 0x00,

0x00, 0x00,

0x7e,

0x7d, ((DLCI_D << 2) | 0x01) ^ 0x20,

0xef,

0x7c, 0x7d, 0x7d ^ 0x20, 0x7d, 0x7e ^ 0x20,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_7c7d7e_UE2TE_D, 15)

/* SDU: Send 0x7e 0x7f 0x80 in Information Frame on DLCI D, UE -> TE, special XON and XOFF

| 0x58, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_D << 2) ¦ 0x01 | Address Field: DLCI D, C/R 0, EA 1 |
| 0x7d, 0xef ^ 0x20 | Control Field: UIH information frame |
| 0x7d, 0x7e ^ 0x20, 0x7f, 0x80, | String "0x7e 0x7f 0x80" |
| 0x7d, FCS_DUMMY_OK ^ 0x20 | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_7e7f80_UE2TE_D_X)

0x58, 0x00,

0x00, 0x00,

0x7e,

(DLCI_D << 2) | 0x01,

0x7d, 0xef ^ 0x20,

0x7d, 0x7e ^ 0x20, 0x7f, 0x80,

0x7d, (0XCF) ^ 0x20,

0x7e

ENDFIELD(SDU_FRAME_7e7f80_UE2TE_D_X, 15)

/* SDU: Send 0x7c 0x7d 0x7e in Information Frame on DLCI E, UE -> TE, special XON and XOFF

| 0x68, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x7d, ((DLCI_E << 2) ¦ 0x01) ^ 0x20 | Address Field: DLCI E, C/R 0, EA 1 |
| 0x7d, 0xef ^ 0x20 | Control Field: UIH information frame |
| 0x7c, 0x7d, 0x7d ^ 0x20, 0x7d, 0x7e ^ 0x20, | String "0x7c 0x7d 0x7e" |
| 0x7d, FCS_DUMMY_OK ^ 0x20 | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_7c7d7e_UE2TE_E_X)

0x68, 0x00,

0x00, 0x00,

0x7e,

0x7d, ((DLCI_E << 2) | 0x01) ^ 0x20,

0x7d, 0xef ^ 0x20,

0x7c, 0x7d, 0x7d ^ 0x20, 0x7d, 0x7e ^ 0x20,

0x7d, (0XCF) ^ 0x20,

0x7e

ENDFIELD(SDU_FRAME_7c7d7e_UE2TE_E_X, 17)

/* SDU: Send 0x7e 0x7f 0x80 in Information Frame on DLCI E, UE -> TE

| 0x50, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x7d, ((DLCI_E << 2) ¦ 0x01) ^ 0x20 | Address Field: DLCI E, C/R 0, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x7d, 0x7e ^ 0x20, 0x7f, 0x80, | String "0x7e 0x7f 0x80" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_7e7f80_UE2TE_E)

0x50, 0x00,

0x00, 0x00,

0x7e,

0x7d, ((DLCI_E << 2) | 0x01) ^ 0x20,

0xef,

0x7d, 0x7e ^ 0x20, 0x7f, 0x80,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_7e7f80_UE2TE_E, 14)

/* SDU: Send 'h' in Information Frame on DLCI B, TE -> UE

| 0x30, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B<< 2) ¦ 0x01 | Address Field: DLCI B, C/R 0, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x68 | String "h" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_H_TE2UE_B)

0x30, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x01,

0xef,

0x68,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_H_TE2UE_B, 10)

/* SDU: Send '+++' in Information Frame on DLCI B, TE -> UE

| 0x40, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B<< 2) ¦ 0x03 | Address Field: DLCI B, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x2B, 0x2B, 0x2B | String "+++" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_ESC_SEQ_B)

0x40, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x03,

0xef,

0x2B, 0x2B, 0x2B,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_ESC_SEQ_B, 12)

/* SDU: Send '+++' in Information Frame on DLCI C, TE -> UE

| 0x40, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_C<< 2) ¦ 0x03 | Address Field: DLCI C, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x2B, 0x2B, 0x2B | String "+++" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_ESC_SEQ_C)

0x40, 0x00,

0x00, 0x00,

0x7e,

(DLCI_C << 2) | 0x03,

0xef,

0x2B, 0x2B, 0x2B,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_ESC_SEQ_C, 12)

/* SDU: Send '+++' in Information Frame on DLCI A TE -> UE

| 0x40, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A< 2) ¦ 0x03 | Address Field: DLCI A C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x2B, 0x2B, 0x2B | String "+++" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_ESC_SEQ_A)

0x40, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A<< 2) | 0x03,

0xef,

0x2B, 0x2B, 0x2B,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_ESC_SEQ_A, 12)

/* SDU: Send 'l' in Information Frame on DLCI C, TE -> UE

| 0x30, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_C<< 2) ¦ 0x03 | Address Field: DLCI C, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x6C | String "l" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_L_TE2UE_C)

0x30, 0x00,

0x00, 0x00,

0x7e,

(DLCI_C << 2) | 0x03,

0xef,

0x6C,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_L_TE2UE_C, 10)

/* SDU: Send 'e' in Information Frame on DLCI A, TE -> UE

| 0x30, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A<< 2) ¦ 0x03 | Address Field: DLCI A, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x65 | String "e" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_E_TE2UE_A)

0x30, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0xef,

0x65,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_E_TE2UE_A, 10)

/* SDU: Send Hello2 String in Information Frame on DLCI A, UE -> TE

| 0x68, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x01 | Address Field: DLCI A, C/R 0, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a, | String "Hello2\n" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_HELLO2_UE2TE_A)

0x68, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x01,

0xef,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_HELLO2_UE2TE_A, 17)

/* SDU: Send Hello1 String in Information Frame on DLCI A, UE -> TE

| 0x68, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A<< 2) ¦ 0x01 | Address Field: DLCI A, C/R 0, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, | String "Hello1\n" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_HELLO1_UE2TE_A)

0x68, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x01,

0xef,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_HELLO1_UE2TE_A, 17)

/* SDU: Send Hello2 String in Information Frame on DLCI A, TE -> UE

| 0x68, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI A, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a, | String "Hello2\n" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_HELLO2_TE2UE_A)

0x68, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0xef,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x32, 0x0d, 0x0a,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_HELLO2_TE2UE_A, 17)

/* SDU: Send 0x10 0x11 0x12 in Information Frame on DLCI A, TE -> UE

| 0x48, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x10, 0x7d, 0x11 ^ 0x20, 0x12, | Data "0x10 0x11 0x12" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_101112_TE2UE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0xef,

0x10, 0x7d, 0x11 ^ 0x20, 0x12,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_101112_TE2UE_A, 13)

/* SDU: Send 0x7e 0x7f 0x80 in Information Frame on DLCI B, TE -> UE

| 0x48, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B << 2) ¦ 0x03 | Address Field: DLCI B, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x7d, 0x7e ^ 0x20, 0x7f, 0x80, | Data "0x7e 0x7f 0x80" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_7e7f80_TE2UE_B)

0x48, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x03,

0xef,

0x7d, 0x7e ^ 0x20, 0x7f, 0x80,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_7e7f80_TE2UE_B, 13)

/* SDU: Send 0x13 0x14 0x15 in Information Frame on DLCI D, TE -> UE

| 0x50, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x7d, ((DLCI_D << 2) ¦ 0x03) ^ 0x20 | Address Field: DLCI D, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x7d, 0x13 ^ 0x20, 0x14, 0x15 | Data "0x13 0x14 0x15" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_131415_TE2UE_D)

0x50, 0x00,

0x00, 0x00,

0x7e,

0x7d, ((DLCI_D << 2) | 0x03) ^ 0x20,

0xef,

0x7d, 0x13 ^ 0x20, 0x14, 0x15,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_131415_TE2UE_D, 14)

/* SDU: Send 0x7c 0x7d 0x7e in Information Frame on DLCI D, TE -> UE, special XON and XOFF

| 0x60, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_D << 2) ¦ 0x03 | Address Field: DLCI D, C/R 1, EA 1 |
| 0x7d, 0xef ^ 0x20 | Control Field: UIH information frame |
| 0x7c, 0x7d, 0x7d ^ 0x20, 0x7d, 0x7e ^ 0x20 | Data "0x7c 0x7d 0x7e" |
| 0x7d, FCS_DUMMY_OK ^ 0x20 | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_7c7d7e_TE2UE_D_X)

0x60, 0x00,

0x00, 0x00,

0x7e,

(DLCI_D << 2) | 0x03,

0x7d, 0xef ^ 0x20,

0x7c, 0x7d, 0x7d ^ 0x20, 0x7d, 0x7e ^ 0x20,

0x7d, (0XCF) ^ 0x20,

0x7e

ENDFIELD(SDU_FRAME_7c7d7e_TE2UE_D_X, 16)

/* SDU: Send 0x13 0x14 0x15 in Information Frame on DLCI E, TE -> UE, special XON and XOFF

| 0x50, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_E << 2) ¦ 0x03 | Address Field: DLCI E, C/R 1, EA 1 |
| 0x7d, 0xef ^ 0x20 | Control Field: UIH information frame |
| 0x13, 0x14, 0x15 | Data "0x13 0x14 0x15" |
| 0x7d, FCS_DUMMY_OK ^ 0x20 | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_131415_TE2UE_E_X)

0x50, 0x00,

0x00, 0x00,

0x7e,

(DLCI_E << 2) | 0x03,

0x7d, 0xef ^ 0x20,

0x13, 0x14, 0x15,

0x7d, (0XCF) ^ 0x20,

0x7e

ENDFIELD(SDU_FRAME_131415_TE2UE_E_X, 14)

/* SDU: Send 0x7c 0x7d 0x7e in Information Frame on DLCI E, TE -> UE

| 0x50, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_E << 2) ¦ 0x03 | Address Field: DLCI E, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x7c, 0x7d, 0x7d ^ 0x20, 0x7d, 0x7e ^ 0x20 | Data "0x7c 0x7d 0x7e" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_7c7d7e_TE2UE_E)

0x50, 0x00,

0x00, 0x00,

0x7e,

(DLCI_E << 2) | 0x03,

0xef,

0x7c, 0x7d, 0x7d ^ 0x20, 0x7d, 0x7e ^ 0x20,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_7c7d7e_TE2UE_E, 14)

/* SDU: Send AT command string in Information Frame on DLCI A, TE -> UE

| 0x48, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI A, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x41, 0x54, 0x0d, 0x0a, | String "AT\n" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_AT_TE2UE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0xef,

0x41, 0x54, 0x0d, 0x0a,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_AT_TE2UE_A, 13)

/* SDU: Send AT command string in Information Frame on DLCI B, TE -> UE

| 0x48, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B << 2) ¦ 0x03 | Address Field: DLCI B, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x41, 0x54, 0x0d, 0x0a, | String "AT\n" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_AT_TE2UE_B)

0x48, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x03,

0xef,

0x41, 0x54, 0x0d, 0x0a,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_AT_TE2UE_B, 13)

/* SDU: Send AT command string in Information Frame on DLCI C, TE -> UE

| 0x48, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_C << 2) ¦ 0x03 | Address Field: DLCI C, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x41, 0x54, 0x0d, 0x0a, | String "AT\n" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_AT_TE2UE_C)

0x48, 0x00,

0x00, 0x00,

0x7e,

(DLCI_C << 2) | 0x03,

0xef,

0x41, 0x54, 0x0d, 0x0a,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_AT_TE2UE_C, 13)

/* SDU: Send 64 octets in Information Frame on DLCI A, TE -> UE

| 0x28, 0x02 | SDU length in bit (69 octets) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI A, C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, | String "ABCDEFGH" |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, | String "ABCDEFGH" |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, | String "ABCDEFGH" |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, | String "ABCDEFGH" |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, | String "ABCDEFGH" |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, | String "ABCDEFGH" |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, | String "ABCDEFGH" |
| 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48, | String "ABCDEFGH" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_FRAME_64_TE2UE_A)

0x28, 0x02,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0xef,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 0x47, 0x48,

(0XCF),

0x7e

ENDFIELD(SDU_FRAME_64_TE2UE_A, 73)

/*

#### Disconnected Mode (DM) Frames

*/

/* SDU: DM Response from UE to TE for DLCI A

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x1f | Control Field: DM Response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_DM_UE2TE_A)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0x1f,

(0XCF),

0x7e

ENDFIELD(SDU_DM_UE2TE_A, 9)

/* SDU: DM Response from UE to TE for DLCI A, Type 0F

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x0f | Control Field: DM Response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_DM_UE2TE_A_0F)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0x0f,

(0XCF),

0x7e

ENDFIELD(SDU_DM_UE2TE_A_0F, 9)

/* SDU: DM Response from TE to UE for DLCI A

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x01 | Address Field: DLCI_A, C/R 0, EA 1 |
| 0x1f | Control Field: DM Response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_DM_TE2UE_A)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x01,

0x1f,

(0XCF),

0x7e

ENDFIELD(SDU_DM_TE2UE_A, 9)

/* SDU: DM Response from TE to UE for DLCI B

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_B << 2) ¦ 0x01 | Address Field: DLCI_B, C/R 0, EA 1 |
| 0x1f | Control Field: DM Response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_DM_TE2UE_B)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_B << 2) | 0x01,

0x1f,

(0XCF),

0x7e

ENDFIELD(SDU_DM_TE2UE_B, 9)

/*

/* SDU: DM Response from UE to TE for SDU_CORRUPTED_DATA8

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (0x43 << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x0f | Control Field: DM Response for UIH |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_DM_CORRUPTED_DATA8)

0x28, 0x00,

0x00, 0x00,

0x7e,

(0x43 << 2) | 0x03,

0x0f,

(0XCF),

0x7e

ENDFIELD(SDU_DM_CORRUPTED_DATA8, 9)

/*

#### Modem Status Commands (MSC)

*/

/* SDU: Normal Modem Status Cmd, TE->UE

| 0x48, 0x00 | SDU length in bit (9 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type: MSC command |
| 0x05 | Length is 2, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x01 | status for DLCI A |
| 0x8d | V.24 signals: 1000 1101 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSC_CMD_TE2UE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe3,

0x05,

(DLCI_A << 2) | 0x01,

0x8d,

(0XCF),

0x7e

ENDFIELD(SDU_MSC_CMD_TE2UE_A, 13)

/* SDU: Normal Modem Status Res, UE->TE

| 0x48, 0x00 | SDU length in bit (9 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe1 | Type: MSC response |
| 0x05 | Length is 2, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x01 | status for DLCI A |
| 0x8d | V.24 signals: 1000 1101 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSC_RES_UE2TE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe1,

0x05,

(DLCI_A << 2) | 0x01,

0x8d,

(0XCF),

0x7e

ENDFIELD(SDU_MSC_RES_UE2TE_A, 13)

/* SDU: Break Modem Status Cmd, TE->UE

| 0x50, 0x00 | SDU length in bit (10 Bytes = 80 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type: MSC command |
| 0x07 | Length is 3, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x01 | status for DLCI A |
| 0x8d | V.24 signals: 1000 1101 |
| (UART_BREAK_LEN << 4) ¦ 0x03 | high byte: break length, low byte: 3 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSCBREAK_CMD_TE2UE_A)

0x50, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe3,

0x07,

(DLCI_A << 2) | 0x01,

0x8d,

(UART_BREAK_LEN << 4) | 0x03,

(0XCF),

0x7e

ENDFIELD(SDU_MSCBREAK_CMD_TE2UE_A, 14)

/* SDU: Break Modem Status Res, UE->TE

| 0x50, 0x00 | SDU length in bit (10 Bytes = 80 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe1 | Type: MSC response |
| 0x07 | Length is 3, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x01 | status for DLCI A |
| 0x8d | V.24 signals: 1000 1101 |
| (UART_BREAK_LEN << 4) ¦ 0x03 | high byte: break length, low byte: 3 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSCBREAK_RES_UE2TE_A)

0x50, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe1,

0x07,

(DLCI_A << 2) | 0x01,

0x8d,

(UART_BREAK_LEN << 4) | 0x03,

(0XCF),

0x7e

ENDFIELD(SDU_MSCBREAK_RES_UE2TE_A, 14)

/* SDU: Break Modem Status Cmd, UE->TE

| 0x50, 0x00 | SDU length in bit (10 Bytes = 80 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type: MSC command |
| 0x07 | Length is 3, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x01 | status for DLCI A |
| 0x8d | V.24 signals: 10001101 |
| (UART_BREAK_LEN << 4) ¦ 0x03 | high byte: break length, low byte: 3 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSCBREAK_CMD_UE2TE_A)

0x50, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xe3,

0x07,

(DLCI_A << 2) | 0x03,

0x8d,

(UART_BREAK_LEN << 4) | 0x03,

(0XCF),

0x7e

ENDFIELD(SDU_MSCBREAK_CMD_UE2TE_A, 14)

/* SDU: Break Modem Status Res, TE->UE

| 0x50, 0x00 | SDU length in bit (10 Bytes = 80 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe1 | Type: MSC response |
| 0x07 | Length is 3, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x03 | status for DLCI A |
| 0x8d | V.24 signals: 10001101 |
| (UART_BREAK_LEN << 4) ¦ 0x03 | high byte: break length, low byte: 3 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSCBREAK_RES_TE2UE_A)

0x50, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xe1,

0x07,

(DLCI_A << 2) | 0x03,

0x8d,

(UART_BREAK_LEN << 4) | 0x03,

(0XCF),

0x7e

ENDFIELD(SDU_MSCBREAK_RES_TE2UE_A, 14)

/* SDU: Invalid Break Modem Status Res, TE->UE

| 0x50, 0x00 | SDU length in bit (10 Bytes = 80 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe1 | Type: MSC response |
| 0x07 | Length is 3, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x01 | invalid: bit 1 is 0 |
| 0x01 | V.24 signals: invalid, only EA bit set |
| (UART_BREAK_LEN << 4) ¦ 0x03 | high byte: break length, low byte: 3 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSCBREAK_RES_TE2UE_A_INVALID)

0x50, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xe1,

0x07,

(DLCI_A << 2) | 0x01,

0x01,

(UART_BREAK_LEN << 4) | 0x03,

(0XCF),

0x7e

ENDFIELD(SDU_MSCBREAK_RES_TE2UE_A_INVALID, 14)

/* SDU: Ring Modem Status Cmd, UE->TE

| 0x48, 0x00 | SDU length in bit (9 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type: MSC command |
| 0x05 | Length is 2, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x03 | status for DLCI A |
| 0xcd | V.24 signals: 1100 1101 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSCRING_CMD_UE2TE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xe3,

0x05,

(DLCI_A << 2) | 0x03,

0xcd,

(0XCF),

0x7e

ENDFIELD(SDU_MSCRING_CMD_UE2TE_A, 13)

/* SDU: Ring Modem Status Res, TE->UE

| 0x48, 0x00 | SDU length in bit (9 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe1 | Type: MSC response |
| 0x05 | Length is 2, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x03 | status for DLCI A |
| 0xcd | V.24 signals: 1100 1101 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSCRING_RES_TE2UE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xe1,

0x05,

(DLCI_A << 2) | 0x03,

0xcd,

(0XCF),

0x7e

ENDFIELD(SDU_MSCRING_RES_TE2UE_A, 13)

/* SDU: Flow Control Modem Status Cmd, UE->TE

| 0x48, 0x00 | SDU length in bit (9 Bytes = 72 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type: MSC command |
| 0x05 | Length is 2, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x01 | status for DLCI A |
| 0x8f | V.24 signals: 10001101 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSCFCOFF_CMD_UE2TE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xe3,

0x05,

(DLCI_A << 2) | 0x03,

0x8f,

(0XCF),

0x7e

ENDFIELD(SDU_MSCFCOFF_CMD_UE2TE_A, 13)

/* SDU: Normal Modem Status Cmd, UE -> TE

| 0x48, 0x00 | SDU length in bit (9 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type: MSC command |
| 0x05 | Length is 2, EA bit is 1 |
| (DLCI_A << 2) ¦ 0x01 | status for DLCI A |
| 0x8d | V.24 signals: 1000 1101 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_MSC_CMD_UE2TE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

0x01,

0xff,

0xe3,

0x05,

(DLCI_A << 2) | 0x03,

0x8d,

(0XCF),

0x7e

ENDFIELD(SDU_MSC_CMD_UE2TE_A, 13)

/*

#### Invalid Frames

*/

/* SDU: An SDU with completely corrupted data

| 0x00, 0x03 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14, 0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66, 0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14, 0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x660x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14, 0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66, 0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14, 0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66 0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14, 0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66, 0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14, 0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66 | some random data |

*/

FIELD(SDU_CORRUPTED_DATA1)

0x00, 0x03,

0x00,0x00,

0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14,

0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66,
	0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14,

0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66,

0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14,

0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66,
	0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14,

0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66,

0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14,

0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66,
	0x55, 0x67, 0x54, 0x73, 0xff, 0x10, 0x22, 0x14,

0x22, 0x65, 0x34, 0xfe, 0xff, 0x20, 0x30, 0x66

ENDFIELD(SDU_CORRUPTED_DATA1, 100)

/* SDU: MUX Startup (TE initiated) SABM Frame - upper Flag corrupted

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7f | Flag Sequence (corrupt) 0111 1111 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA2)

0x28, 0x00,

0x00, 0x00,

0x7f,

0x03,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA2, 9)

/* SDU: DM Response from TE to UE for DLCI A - lower Flag corrupted

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x01 | Address Field: DLCI_A, C/R 0, EA 1 |
| 0x1f | Control Field: DM Response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x0e | corrupted Flag Sequence 0000 1110 |

*/

FIELD(SDU_CORRUPTED_DATA3)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x01,

0x1f,

(0XCF),

0x0e

ENDFIELD(SDU_CORRUPTED_DATA3, 9)

/* SDU: An SDU with flags only

| 0x00, 0x03 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e | some flags |

*/

FIELD(SDU_CORRUPTED_DATA4)

0x00, 0x03,

0x00,0x00,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e,

0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e, 0x7e

ENDFIELD(SDU_CORRUPTED_DATA4, 100)

/* SDU: MUX Startup (TE initiated) SABM Frame - wrong FCS

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_INVALID | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA6)

0x28, 0x00,

0x00, 0x00,

0x7e,

0x03,

0x3f,

FCS_DUMMY_INVALID,

0x7e

ENDFIELD(SDU_CORRUPTED_DATA6, 9)

/* SDU: MUX Startup (TE initiated) SABM Frame - only two bytes between flags

| 0x20, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x3f | Control Field: SABM command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA7)

0x20, 0x00,

0x00, 0x00,

0x7e,

0x3f,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA7, 8)

/* SDU: Send h String in Information Frame on invalid DLCI, TE -> UE

| 0x30, 0x00 | SDU length in bit (6 Bytes = 48Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (0x43 << 2) ¦ 0x03 | Address Field: DLCI , C/R 1, EA 1 |
| 0xef | Control Field: UIH information frame |
| 0x68, | String "h" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA8)

0x30, 0x00,

0x00, 0x00,

0x7e,

(0x43 << 2) | 0x03,

0xef,

0x68,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA8, 10)

/* SDU: Send Hello1 String in Information Frame on DLCI A - no EA bit

| 0x68, 0x00 | SDU length in bit |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x02 | Address Field: DLCI A, C/R 1, EA 0 |
| 0xef | Control Field: UIH information frame |
| 0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a, | String "Hello1\n" |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA9)

0x60, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x02,

0xef,

0x48, 0x65, 0x6c, 0x6c, 0x6f, 0x31, 0x0d, 0x0a,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA9, 17)

/* SDU: invalid control octet

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x3d | Control Field: invalid |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA11)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0x3d,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA11, 9)

/* SDU: SABM mit P bit set to 0

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| (DLCI_A << 2) ¦ 0x03 | Address Field: DLCI_A, C/R 1, EA 1 |
| 0x2f | Control Field: SABM with P set 0 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA12)

0x28, 0x00,

0x00, 0x00,

0x7e,

(DLCI_A << 2) | 0x03,

0x2f,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA12, 9)

/* SDU: DISC command on DLCI 0, P bit set to 0

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0x43 | Control Field: DISC command, P set 0 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA13)

0x28, 0x00,

0x00, 0x00,

0x7e,

0x03,

0x43,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA13, 9)

/* SDU: UA Response from TE to UE, F bit set to 0

| 0x28, 0x00 | SDU length in bit (5 Bytes = 40 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x01 | Address Field: DLCI 0, C/R 0, EA 1 |
| 0x63 | Control Field: UA response |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA14)

0x28, 0x00,

0x00, 0x00,

0x7e,

0x01,

0x63,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA14, 9)

/* SDU: Message Type has EA bit 0

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe2 | Type: EA bit is 0 |
| 0x05 | Length is 2, EA bit is 1 |
| DLCI_A | status for DLCI A |
| 0x8d | V.24 signals: 1000 1101 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA16)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe2,

0x05,

DLCI_A,

0x8d,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA16, 16)

/* SDU: Message Length has EA bit 0

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type: MSC |
| 0x04 | Length is 2, EA bit is 0 |
| DLCI_A | status for DLCI A |
| 0x8d | V.24 signals: 1000 1101 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA17)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe3,

0x04,

DLCI_A,

0x8d,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA17, 16)

/* SDU: MSC, Peer initiated, corrupted length field

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type |
| 0xf1 | Length is corrupt, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA18)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe3,

0xf1,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA18, 11)

/* SDU: Break Modem Status Cmd, TE->UE, info shorter than length

| 0x50, 0x00 | SDU length in bit (10 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type: MSC command |
| 0x09 | Length is 4, EA bit is 1 |
| DLCI_A | status for DLCI A |
| 0x8d | V.24 signals: 1000 1101 |
| (UART_BREAK_LEN << 4) ¦ 0x03 | high byte: break length, low byte: 3 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA19)

0x50, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe3,

0x09,

DLCI_A,

0x8d,

(UART_BREAK_LEN << 4) | 0x03,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA19, 14)

/* SDU: Break Modem Status Cmd, TE->UE, info longer than length

| 0x50, 0x00 | SDU length in bit (10 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3 | Type: MSC command |
| 0x05 | Length is 2, EA bit is 1 |
| DLCI_A | status for DLCI A |
| 0x8d | V.24 signals: 1000 1101 |
| (UART_BREAK_LEN << 4) ¦ 0x03 | high byte: break length, low byte: 3 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA20)

0x50, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe3,

0x05,

DLCI_A,

0x8d,

(UART_BREAK_LEN << 4) | 0x03,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA20, 14)

/* SDU: Break Modem Status Cmd, TE->UE, frame longer than 64 octets

| 0x30, 0x02 | SDU length in bit (70 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0xe3, 0x05, 0x07, 0x8d, 0x13 | MSC command for DLCI 1 |
| 0xe3, 0x05, 0x0b, 0x8d, 0x13 | MSC command for DLCI 2 |
| 0xe3, 0x05, 0x0f, 0x8d, 0x13 | MSC command for DLCI 3 |
| 0xe3, 0x05, 0x13, 0x8d, 0x13 | MSC command for DLCI 4 |
| 0xe3, 0x05, 0x17, 0x8d, 0x13 | MSC command for DLCI 5 |
| 0xe3, 0x05, 0x1b, 0x8d, 0x13 | MSC command for DLCI 6 |
| 0xe3, 0x05, 0x1f, 0x8d, 0x13 | MSC command for DLCI 7 |
| 0xe3, 0x05, 0x23, 0x8d, 0x13 | MSC command for DLCI 8 |
| 0xe3, 0x05, 0x27, 0x8d, 0x13 | MSC command for DLCI 9 |
| 0xe3, 0x05, 0x2b, 0x8d, 0x13 | MSC command for DLCI 10 |
| 0xe3, 0x05, 0x2f, 0x8d, 0x13 | MSC command for DLCI 11 |
| 0xe3, 0x05, 0x33, 0x8d, 0x13 | MSC command for DLCI 12 |
| 0xe3, 0x05, 0x37, 0x8d, 0x13 | MSC command for DLCI 13 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_DATA21)

0x30, 0x02,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe3, 0x05, 0x07, 0x8d, 0x13,

0xe3, 0x05, 0x0b, 0x8d, 0x13,

0xe3, 0x05, 0x0f, 0x8d, 0x13,

0xe3, 0x05, 0x13, 0x8d, 0x13,

0xe3, 0x05, 0x17, 0x8d, 0x13,

0xe3, 0x05, 0x1b, 0x8d, 0x13,

0xe3, 0x05, 0x1f, 0x8d, 0x13,

0xe3, 0x05, 0x23, 0x8d, 0x13,

0xe3, 0x05, 0x27, 0x8d, 0x13,

0xe3, 0x05, 0x2b, 0x8d, 0x13,

0xe3, 0x05, 0x2f, 0x8d, 0x13,

0xe3, 0x05, 0x33, 0x8d, 0x13,

0xe3, 0x05, 0x37, 0x8d, 0x13,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_DATA21, 74)

/*

#### Other Frames

*/

/* SDU: Unknown Command, Peer initiated

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| UART_UNKNOWN_COMMAND | Type |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UNKNOWN_CMD_TE2UE_A)

0x38, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

UART_UNKNOWN_COMMAND,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_UNKNOWN_CMD_TE2UE_A, 11)

/* SDU: Unsupported Command Response, UE->TE

| 0x48, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e | Flag Sequence 0111 1110 |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| 0x7d, 0x31 = 0x11 (transparency) | Type: unsupported command |
| 0x03 | Length is 1, EA bit is 1 |
| UART_UNKNOWN_COMMAND | Type of the Unsupported Cmd |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_UNSUPP_CMD_UE2TE_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0x7d,

0x31,

0x03,

UART_UNKNOWN_COMMAND,

(0XCF),

0x7e

ENDFIELD(SDU_UNSUPP_CMD_UE2TE_A, 12)

/* SDU: Corrupted/Invalid Frame I

| 0x38, 0x00 | SDU length in bit (7 Bytes = 56 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7f | Flag Sequence 0111 1111 (corrupted!) |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| UART_UNKNOWN_COMMAND | Type |
| 0x01 | Length is 0, EA bit is 1 |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_1)

0x38, 0x00,

0x00, 0x00,

0x7f,

0x03,

0xff,

0x43,

0x01,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_1, 11)

/* SDU: Empty Response UE->TE

| 0x28, 0x00 | SDU length in bit (5 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7f | Flag Sequence 0111 1111 (corrupted!) |
| 0x03 | Address Field: DLCI 0, C/R 1, EA 1 |
| 0xff | Control Field: UIH command |
| FCS_DUMMY_OK | Frame Check Sequence (FCS, dummy) |
| 0x7e | Flag Sequence 0111 1110 |

*/

FIELD(SDU_CORRUPTED_RES_EMPTY)

0x28, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPTED_RES_EMPTY, 9)

/* SDU: Multiframe TE->UE (Data, wrong MSC res, 2. wrong MSC res, 4 x Data)

| 0x80, 0x01 | SDU length in bit (48 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e, (DLCI_A << 2) ¦ 0x03, 0xef, 0x2b, FCS_DUMMY_OK, 0x7e | Data frame "+" |
| 0x7e, 0x03, 0xff, 0xe1, 0x05, (DLCI_A << 2) ¦ 0x03, 0x8f, (0XCF), 0x7e | 1. wrong (C/R is set in address field) MSC response |
| 0x7e, 0x03, 0xff, 0xe1, 0x05, (DLCI_A << 2) ¦ 0x03, 0x8f, FCS_DUMMY_OK, 0x7e, | 2. wrong (C/R is set in address field) MSC response |
| 0x7e, (DLCI_A << 2) ¦ 0x03, 0xef, 0x63, (0XCF), 0x7e | Data frame "c" |
| 0x7e, (DLCI_A << 2) ¦ 0x03, 0xef, 0x6f, FCS_DUMMY_OK, 0x7e | Data frame "o" |
| 0x7e, (DLCI_A << 2) ¦ 0x03, 0xef, 0x70, FCS_DUMMY_OK, 0x7e | Data frame "p" |
| 0x7e, (DLCI_A << 2) ¦ 0x03, 0xef, 0x73, FCS_DUMMY_OK, 0x7e | Data frame "s" |

*/

FIELD(SDU_DMSCMSCDDDD_RES_A)

0x80, 0x01,

0x00, 0x00,

0x7e, (DLCI_A << 2) | 0x03, 0xef, 0x2b, (0XCF), 0x7e,

0x7e, 0x03, 0xff, 0xe1, 0x05, (DLCI_A << 2) | 0x03, 0x8f, (0XCF), 0x7e,

0x7e, 0x03, 0xff, 0xe1, 0x05, (DLCI_A << 2) | 0x03, 0x8f, (0XCF), 0x7e,

0x7e, (DLCI_A << 2) | 0x03, 0xef, 0x63, (0XCF), 0x7e,

0x7e, (DLCI_A << 2) | 0x03, 0xef, 0x6f, (0XCF), 0x7e,

0x7e, (DLCI_A << 2) | 0x03, 0xef, 0x70, (0XCF), 0x7e,

0x7e, (DLCI_A << 2) | 0x03, 0xef, 0x73, (0XCF), 0x7e

ENDFIELD(SDU_DMSCMSCDDDD_RES_A, 52)

/* SDU: Multiframe TE->UE (wrong MSC res, 2. wrong MSC res)

| 0x90, 0x00 | SDU length in bit (18 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e, 0x03, 0xff, 0xe1, 0x05, (DLCI_A << 2) ¦ 0x03, 0x8f, FCS_DUMMY_OK, 0x7e | 1. wrong (C/R is set in address field) MSC response |
| 0x7e, 0x03, 0xff, 0xe1, 0x05, (DLCI_A << 2) ¦ 0x03, 0x8d, FCS_DUMMY_OK, 0x7e, | 2. wrong (C/R is set in address field) MSC response |

*/

FIELD(SDU_MSCMSC_RES_A)

0x90, 0x00,

0x00, 0x00,

0x7e, 0x03, 0xff, 0xe1, 0x05, (DLCI_A << 2) | 0x03, 0x8f, (0XCF), 0x7e,

0x7e, 0x03, 0xff, 0xe1, 0x05, (DLCI_A << 2) | 0x03, 0x8d, (0XCF), 0x7e

ENDFIELD(SDU_MSCMSC_RES_A, 22)

/* SDU: wrong MSC response frame TE->UE

| 0x48, 0x00 | SDU length in bit (9 Bytes) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x7e, 0x03, 0xff, 0xe1, 0x05, (DLCI_A << 2) ¦ 0x03, 0x8f, FCS_DUMMY_OK, 0x7e | 1. wrong (C/R is set in address field) MSC response |

*/

FIELD(SDU_CORRUPT_MSC_RES_A)

0x48, 0x00,

0x00, 0x00,

0x7e,

0x03,

0xff,

0xe1,

0x05,

(DLCI_A << 2) | 0x03,

0x8d,

(0XCF),

0x7e

ENDFIELD(SDU_CORRUPT_MSC_RES_A, 13)

/*

## Declarations

*/

DECLARATION (DTI_CHANNELNAME_PPP)

DECLARATION (DTI_CHANNELNAME_ACI)

DECLARATION(COMPAR_57600_8_E_1_HW)

DECLARATION(COMPAR_57600_8_N_1_HW)

DECLARATION(COMPAR_19200_8_N_1_HW)

DECLARATION(COMPAR_9600_8_N_1_HW)

DECLARATION(COMPAR_XON_CTRL_XOFF_FCS)

/* Array declaration */

DECLARATION ( DTI_PARAMETER_FRAME )

DECLARATION ( DTI_PARAMETER_FRAME_BREAK )

DECLARATION ( DTI_PARAMETER_UART_OUT )

DECLARATION ( DTI_PARAMETER_PEER_A )

DECLARATION ( DTI_PARAMETER_PEER_B )

DECLARATION ( DTI_PARA_ST_LINES_BREAK_OFF)

DECLARATION ( DTI_PARA_ST_LINES_BREAK_ON)

/*

Service Data Units and Content

*/

DECLARATION(STRUCT_HELLO1_SDU)

DECLARATION(STRUCT_HELLO2_SDU)

DECLARATION(STRUCT_HELLO3_SDU)

DECLARATION(STRUCT_H_SDU)

DECLARATION(STRUCT_E_SDU)

DECLARATION(STRUCT_L_SDU)

DECLARATION(STRUCT_O_SDU)

DECLARATION(STRUCT_EMPTY_SDU)

/*

## Arrays

*/

/* DTI Communication Channel Name PPP

| 0x50, 0x50, 0x50, 0x00, 0x00, 0x0 | name value (“PPP\0”) |
| --- | --- |

*/

BEGINARRAY(DTI_CHANNELNAME_PPP, 6)

0x50, 0x50, 0x50, 0x00, 0x00, 0x0

ENDARRAY

/* DTI Communication Channel Name ACI

| 0x41, 0x43, 0x44, 0x00, 0x00, 0x0 | name value (“ACI\0”) |
| --- | --- |

*/

BEGINARRAY(DTI_CHANNELNAME_ACI, 6)

0x41, 0x43, 0x44, 0x00, 0x00, 0x0

ENDARRAY

/*

## Structs

*/

/*

### Communication Parameters

*/

BEGIN_PSTRUCT("comPar", COMPAR_57600_8_E_1_HW)

SET_COMP("speed", COMPAR_SPEED_57600)

SET_COMP("bpc", COMPAR_BPC_8)

SET_COMP("nsb", COMPAR_NSB_1)

SET_COMP("parity", COMPAR_PARITY_EVEN)

SET_COMP("flow_rx", UART_IO_FC_RX_RTS)

SET_COMP("flow_tx", UART_IO_FC_TX_RTS)

SET_COMP("xon_valid", UART_IO_XON_VALID)

SET_COMP("xon", COMPAR_XON_CHAR)

SET_COMP("xoff_valid", UART_IO_XOFF_VALID)

SET_COMP("xoff", COMPAR_XOFF_CHAR)

SET_COMP("esc_valid", UART_IO_ESC_VALID)

SET_COMP("esc_char", ESC_SEQ_CHAR_PLUS)

SET_COMP("esc_gp", GUARD_PERIOD_1000)

ENDSTRUCT

/* set XON and XOFF to important values of Control field and FCS field */

BEGIN_PSTRUCT("comPar", COMPAR_XON_CTRL_XOFF_FCS)

SET_COMP("speed", UART_IO_SPEED_UNDEF)

SET_COMP("bpc", UART_IO_BPC_UNDEF)

SET_COMP("nsb", UART_IO_SB_UNDEF)

SET_COMP("parity", UART_IO_PA_UNDEF)

SET_COMP("flow_rx", UART_IO_FC_RX_UNDEF)

SET_COMP("flow_tx", UART_IO_FC_TX_UNDEF)

SET_COMP("xon_valid", UART_IO_XON_VALID)

SET_COMP("xon", 0xef)

SET_COMP("xoff_valid", UART_IO_XOFF_VALID)

SET_COMP("xoff", (0XCF))

SET_COMP("esc_valid", UART_IO_ESC_UNDEF)

SET_COMP("esc_char", UART_IO_ESC_CHAR_DEFAULT)

SET_COMP("esc_gp", UART_IO_ESC_GP_DEFAULT)

ENDSTRUCT

BEGIN_PSTRUCT("comPar", COMPAR_57600_8_N_1_HW)

SET_COMP("speed", COMPAR_SPEED_57600)

SET_COMP("bpc", COMPAR_BPC_8)

SET_COMP("nsb", COMPAR_NSB_1)

SET_COMP("parity", COMPAR_PARITY_NO)

SET_COMP("flow_rx", UART_IO_FC_RX_RTS)

SET_COMP("flow_tx", UART_IO_FC_TX_RTS)

SET_COMP("xon_valid", UART_IO_XON_VALID)

SET_COMP("xon", COMPAR_XON_CHAR)

SET_COMP("xoff_valid", UART_IO_XOFF_VALID)

SET_COMP("xoff", COMPAR_XOFF_CHAR)

SET_COMP("esc_valid", UART_IO_ESC_VALID)

SET_COMP("esc_char", ESC_SEQ_CHAR_PLUS)

SET_COMP("esc_gp", GUARD_PERIOD_1000)

ENDSTRUCT

BEGIN_PSTRUCT("comPar", COMPAR_9600_8_N_1_HW)

SET_COMP("speed", COMPAR_SPEED_9600)

SET_COMP("bpc", COMPAR_BPC_8)

SET_COMP("nsb", COMPAR_NSB_1)

SET_COMP("parity", COMPAR_PARITY_NO)

SET_COMP("flow_rx", UART_IO_FC_RX_RTS)

SET_COMP("flow_tx", UART_IO_FC_TX_RTS)

SET_COMP("xon_valid", UART_IO_XON_VALID)

SET_COMP("xon", COMPAR_XON_CHAR)

SET_COMP("xoff_valid", UART_IO_XOFF_VALID)

SET_COMP("xoff", COMPAR_XOFF_CHAR)

SET_COMP("esc_valid", UART_IO_ESC_VALID)

SET_COMP("esc_char", ESC_SEQ_CHAR_PLUS)

SET_COMP("esc_gp", GUARD_PERIOD_1000)

ENDSTRUCT

BEGIN_PSTRUCT("comPar", COMPAR_19200_8_N_1_HW)

SET_COMP("speed", COMPAR_SPEED_19200)

SET_COMP("bpc", COMPAR_BPC_8)

SET_COMP("nsb", COMPAR_NSB_1)

SET_COMP("parity", COMPAR_PARITY_NO)

SET_COMP("flow_rx", UART_IO_FC_RX_RTS)

SET_COMP("flow_tx", UART_IO_FC_TX_RTS)

SET_COMP("xon_valid", UART_IO_XON_VALID)

SET_COMP("xon", COMPAR_XON_CHAR)

SET_COMP("xoff_valid", UART_IO_XOFF_VALID)

SET_COMP("xoff", COMPAR_XOFF_CHAR)

SET_COMP("esc_valid", UART_IO_ESC_VALID)

SET_COMP("esc_char", ESC_SEQ_CHAR_PLUS)

SET_COMP("esc_gp", GUARD_PERIOD_1000)

ENDSTRUCT

/* Parameters for sending DTI primitives */

BEGIN_PSTRUCT ("st_lines", DTI_PARA_ST_LINES_BREAK_OFF)
            SET_COMP ("st_flow",		DTI_FLOW_ON)
	SET_COMP ("st_line_sa",		DTI_SA_ON)
	SET_COMP ("st_line_sb",		DTI_SB_ON)
	SET_COMP ("st_break_len",	DTI_BREAK_OFF)
ENDSTRUCT

BEGIN_PSTRUCT ("st_lines", DTI_PARA_ST_LINES_BREAK_ON)
            SET_COMP ("st_flow",		DTI_FLOW_ON)
	SET_COMP ("st_line_sa",		DTI_SA_ON)
	SET_COMP ("st_line_sb",		DTI_SB_ON)
	SET_COMP ("st_break_len",	UART_BREAK_LEN)
ENDSTRUCT

BEGIN_PSTRUCT ("parameters", DTI_PARAMETER_FRAME)
	SET_COMP ("p_id",	DTI_PID_UOS)
	SET_COMP ("st_lines",	DTI_PARA_ST_LINES_BREAK_OFF)
ENDSTRUCT

BEGIN_PSTRUCT ("parameters", DTI_PARAMETER_FRAME_BREAK)
	SET_COMP ("p_id",	DTI_PID_UOS)
	SET_COMP ("st_lines",	DTI_PARA_ST_LINES_BREAK_ON)
ENDSTRUCT

BEGIN_PSTRUCT ("parameters", DTI_PARAMETER_UART_OUT)
	SET_COMP ("p_id",	DTI_PID_UART_OUT)
	SET_COMP ("st_lines",	DTI_PARA_ST_LINES_BREAK_OFF)
ENDSTRUCT

BEGIN_PSTRUCT ("parameters", DTI_PARAMETER_PEER_A)
	SET_COMP ("p_id",	P_ID_PEER_A)
	SET_COMP ("st_lines",	DTI_PARA_ST_LINES_BREAK_OFF)
ENDSTRUCT

BEGIN_PSTRUCT ("parameters", DTI_PARAMETER_PEER_B)
	SET_COMP ("p_id",	P_ID_PEER_B)
	SET_COMP ("st_lines",	DTI_PARA_ST_LINES_BREAK_OFF)
ENDSTRUCT

/*

### Service Data Units

*/

/* String Hello1 SDU

| 0x40 | SDU length in bit (8 Byte = 64 Bit) |
| --- | --- |
| 0x00 | SDU offset in bit |
| STRING_HELLO1 | String "Hello1\n" |

*/

BEGIN_PSTRUCT("sdu", STRUCT_HELLO1_SDU)

SET_COMP("l_buf", LEN_HELLO)

SET_COMP("o_buf", OFFSET_0)

SET_COMP("buf", STRING_HELLO1)

ENDSTRUCT

/* String Hello2 SDU

| 0x40 | SDU length in bit (8 Byte = 64 Bit) |
| --- | --- |
| 0x00 | SDU offset in bit |
| STRING_HELLO2 | String "Hello2\n" |

*/

BEGIN_PSTRUCT("sdu", STRUCT_HELLO2_SDU)

SET_COMP("l_buf", 0x40)

SET_COMP("o_buf", 0x00)

SET_COMP("buf", STRING_HELLO2)

ENDSTRUCT

/* String Hello3 SDU

| 0x40 | SDU length in bit (8 Byte = 64 Bit) |
| --- | --- |
| 0x00 | SDU offset in bit |
| STRING_HELLO3 | String "Hello3\n" |

*/

BEGIN_PSTRUCT("sdu", STRUCT_HELLO3_SDU)

SET_COMP("l_buf", 0x40)

SET_COMP("o_buf", 0x00)

SET_COMP("buf", STRING_HELLO3)

ENDSTRUCT

/* String h SDU

| 0x08 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00 | SDU offset in bit |
| 0x68 | String "h" |

*/

BEGIN_PSTRUCT("sdu", STRUCT_H_SDU)

SET_COMP("l_buf", 0x08)

SET_COMP("o_buf", 0x00)

SET_COMP("buf", STRING_H)

ENDSTRUCT

/* String e SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x65 | String "e" |

*/

BEGIN_PSTRUCT("sdu", STRUCT_E_SDU)

SET_COMP("l_buf", 0x08)

SET_COMP("o_buf", 0x00)

SET_COMP("buf", STRING_E)

ENDSTRUCT

/* String l SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6c | String "l" |

*/

BEGIN_PSTRUCT("sdu", STRUCT_L_SDU)

SET_COMP("l_buf", 0x08)

SET_COMP("o_buf", 0x00)

SET_COMP("buf", STRING_L)

ENDSTRUCT

/* String o SDU

| 0x08, 0x00 | SDU length in bit (1 Byte = 8 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |
| 0x6f | String "o" |

*/

BEGIN_PSTRUCT("sdu", STRUCT_O_SDU)

SET_COMP("l_buf", 0x08)

SET_COMP("o_buf", 0x00)

SET_COMP("buf", STRING_O)

ENDSTRUCT

/* Empty String SDU

| 0x00, 0x00 | SDU length in bit (0 Byte = 0 Bit) |
| --- | --- |
| 0x00, 0x00 | SDU offset in bit |

*/

BEGIN_PSTRUCT("sdu", STRUCT_EMPTY_SDU)

SET_COMP("l_buf", 0x00)

SET_COMP("o_buf", 0x00)

SKIP_COMP("buf")

ENDSTRUCT

# TEST CASES

## Setup (UART001 - UART010)

### UART001:  TAP Setup

Description: TAP commands in order to do the setup for all following test cases.

Preamble:

None

(G)ACI		UART		PPP
	|		|		|

COMMAND (TAP RESET)

COMMAND (MMI RESET)

|		|		  |

COMMAND (TAP REDIRECT CLEAR)

COMMAND (MMI REDIRECT CLEAR)

COMMAND (UART REDIRECT CLEAR)

|		|		  |

COMMAND (UART REDIRECT MMI TAP)

|		|		  |

COMMAND (TAP REDIRECT TAP UART)

|		|		  |

COMMAND (UART RESET)

|		|		  |

Parametrization:

Primitive	Parameter	Value

History:

20-Sep-2000	STW	Initial

### UART002:  Entity setup

Description:

In non-simulation mode, UART sends a UART_PARAMETERS_IND to ACI in pei_init(). Since this cannot be monitored in simulation mode, just do nothing here. Another possibility would be to send the UART_PARAMETERS_IND in simulation mode after reception of an according config primitive.

Preamble:

UART001

ACI/PPP		UART		UART_OUT

|		|		|

(1)	|	UART_PARAMETERS_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_PARAMETERS_IND
	uart_instances	UART_INSTANCES

History:

12-Mar-2002	TVO	Initial

## Configuration (UART011 – UART019)

### UART011:  Setup Connection to PPP (default parameters)

Description:

DTI connection to PPP without any changes of parameters and escape sequence. The RX service issues a GETDATA request in order to signal it is ready to receive to the UART_OUT interface. The DTI connection is confirmed towards the ACI. Then the DTX service initialises the DTI interface flow control by sending an empty primitive to the PPP entity and sending a DTI2_READY_IND.

Preamble:

UART002

ACI/PPP		UART		UART_OUT

|		|		|

(1)	|	UART_DTI_REQ	|		|

|	(UART_CONNECT_DTI)	|		|

*	==========================>	*		|

(2)	|	DTI2_CONNECT_IND	|		|

*	<==========================	*		|

(3)	|	DTI2_CONNECT_RES	|		|

*	==========================>	*		|

(4)	|	UART_DTI_CNF	|		|

|	(UART_CONNECT_DTI)	|		|

*	<==========================	*		|

(5)	|		|	DTI2_GETDATA_REQ	|

|		|	(ENABLE)	|

|		*	==========================>	*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(7)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(8)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_ANY
	direction	FALSE
	link_id	LINK_ID_1
	entity_name	DTI_CHANNELNAME_PPP

- DTI2_CONNECT_IND
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_ANY

- DTI2_GETDATA_REQ
	link_id	LINK_ENABLE_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

20-Sep-2000	STW	Initial

1-Mar-2001	LW	UART_OUT as lower layer

25-Apr-2001	STW	add Enable primitive

11-Jun-2001	STW	add DTI_GETDATA_REQ primitive

08-Oct-2001	TVO	adapted for use with DTI2

### UART012:  Setup Connection to PPP (explicit parameter setting)

Description:

DTI connection to PPP with 57600 baud, 8 bpc, even parity, 1 stopbit and RTS/CTS flow control. The rest is analogous to test case UART012.

Preamble:

UART002

ACI/PPP		UART		UART_OUT

|		|		|

(1)	|	UART_PARAMETERS_REQ	|		|

*	==========================>	*		|

(2)	|	UART_PARAMETERS_CNF	|		|

*	<==========================	*		|

(3)	|	UART_DTI_REQ	|		|

|	(UART_CONNECT_DTI)	|		|

*	==========================>	*		|

(4)	|	DTI2_CONNECT_IND	|		|

*	<==========================	*		|

(5)	|	DTI2_CONNECT_RES	|		|

*	==========================>	*		|

(6)	|	UART_DTI_CNF	|		|

|	(UART_CONNECT_DTI)	|		|

*	<==========================	*		|

(7)	|		|	DTI2_GETDATA_REQ	|

|		|	(ENABLE)	|

|		*	==========================>	*

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(10)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_PARAMETERS_REQ
	device	UART_DEVICE
	comPar	COMPAR_57600_8_E_1_HW

- UART_PARAMETERS_CNF
	device	UART_DEVICE

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_ANY
	direction	FALSE
	link_id	LINK_ID_1
	entity_name	DTI_CHANNELNAME_PPP

- DTI2_CONNECT_IND
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_ANY

- DTI2_GETDATA_REQ
	link_id	LINK_ENABLE_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

25-Sep-2000	STW	Initial

01-Mar-2001	LW	Adapted to UART_OUT

11-Jun-2001	STW	add DTI_GETDATA_REQ primitive

08-Oct-2001	TVO	adapted for use with DTI2

## Data Transmission - Sending (UART020 - UART029)

### UART020: Send Hello Strings

Description:

Send Strings Hello1, Hello2 and Hello3 in non-multiplexer mode: The UART entity receives the strings from PPP. Each string is then sent on the UART_OUT test interface. A DTI2_READY_IND on the test interface is forwarded to PPP.

The Variant A does a setup with default parameters, variant B sets explicit parameters and variant C has an multiplexer mode establishment failure in the preamble.

Variants:

<A>....<C>

Preamble:

<A>UART011

<B>UART012

<C>UART102

(G)ACI/PPP		UART		UART_OUT

|		|		|

(1)	|	DTI2_DATA_TEST_REQ	|		|

|	(Hello1)	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(Hello1)	|

|		*	==========================>*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(5)	|	DTI2_DATA_TEST_REQ	|		|

|	(Hello2)	|		|

*	==========================>	*		|

(6)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(Hello2)	|

|		*	==========================>*

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(8)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(9)	|	DTI2_DATA_TEST_REQ	|		|

|	(Hello3)	|		|

*	==========================>	*		|

(10)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(Hello3)	|

|		*	==========================>*

(11)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(12)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO1_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO1_SDU

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO2_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO2_SDU

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO3_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO3_SDU

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

History:

28-Mar-2001	LW	Initial

08-Oct-2001	TVO	adapted for use with DTI2

### UART021: Send Hello Strings - Ignore String with Invalid Channel Id

Description:

Send Strings Hello1, Hello2 and Hello3 in non-multiplexer mode: The UART entity receives the strings from PPP. The PPP entity has sent the Hello2 string on the DTI interface with an errorneous channel id. Therefore the Hello2 string is ignored since in non-multiplex mode there is only one valid channel id. The two correcty received strings Hello1 and Hello3 are then sent on the UART_OUT test interface. A DTI2_READY_IND on the test interface is forwarded to PPP.

Variants:

<A>....<B>

Preamble:

<A>UART011

<B>UART012

(G)ACI/PPP		UART		UART_OUT

|		|		|

(1)	|	DTI2_DATA_TEST_REQ	|		|

|	(Hello1)	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(Hello1)	|

|		*	==========================>*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(5)	|	DTI2_DATA_TEST_REQ	|		|

|	(Hello2, wrong link_id)	|		|

*	==========================>	*		|

(6)	|	DTI2_DATA_TEST_REQ	|		|

|	(Hello3)	|		|

*	==========================>	*		|

(7)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(Hello3)	|

|		*	==========================>*

(8)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(9)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO1_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO1_SDU

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_2
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO2_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO3_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO3_SDU

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

History:

6-Mar-2001		LW			Initial

08-Oct-2001	TVO	adapted for use with DTI2

## Data Transmission - Receiving (UART030 - UART039)

### UART030: Receive One Letter String

Description:

The UART entity receives String "h"  via the UART_OUT test interface and forwards them to PPP. In variant A the DTI connection was started normally in variant B it was started after a multiplexer setup failure.

Variants:

<A>....<C>

Preamble:

<A>	UART011

<B>	UART020B

<C>	UART020C

(G)ACI/PPP		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

28-Feb-2001	LW	Initial

11-Jun-2001	STW	remove DTI_GETDATA_REQ primitive

08-Oct-2001	TVO	adapted for use with DTI2

### UART031: Receive String (Size > N1)

Description:

The UART entity receives 15 times the String "Hello1"  via the UART_OUT test interface. Since the parameter N1 of the DTX service is initially configured to 10 in non multiplexer simulation mode, this string is forwarded in pieces of 30/40 bytes to the PPP entity.

Note: In variant A the DTI connection was started normally in variant B it was started normally and some data was transmitted before receiving and in variant C it was started after a multiplexer setup failure.

Variants:

<A>....<C>

Preamble:

<A>	UART011

<B>	UART020B

<C>	UART020C

(G)ACI/PPP		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(15x hello1\n\r)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(1st 30-octet part)	|		|

*	<==========================	*		|

(3)	|	DTI2_GETDATA_REQ	|		|

*	==========================>*		|

(4)	|	DTI2_DATA_TEST_IND	|		|

|	(2nd 30-octet part)	|		|

*	<==========================	*		|

(5)	|	DTI2_GETDATA_REQ	|		|

*	==========================>*		|

(6)	|	DTI2_DATA_TEST_IND	|		|

|	(3rd 40-octet part)	|		|

*	<==========================	*		|

(7)	|	DTI2_GETDATA_REQ	|		|

*	==========================>*		|

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|	DTI2_DATA_TEST_IND	|		|

|	(rest of data)	|		|

*	<==========================	*		|

(10)	|	DTI2_GETDATA_REQ	|		|

*	==========================>*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_15XHELLO1_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_1ST30HELLO1_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_2ND30HELLO1_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_3RD40HELLO1_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_RESTHALLO1_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

28-Feb-2001	LW	Initial

11-Jun-2001	STW	remove DTI_GETDATA_REQ primitive

28-Aug-2001	STW	extend to N1=10 octets

11-Sep-2001	STW	extend to minimum size_multiplier=3

11-Oct-2001	TVO	adapted for use with DTI2

## Data Transmission - Mixed Sending/Receiving (UART040 - UART049)

### UART040: Mixed Receiving/Sending One Letter Strings

Description:

The UART entity receives and sends one letter strings. (In variant A it is a completely new DTI connection in variant B some receiving/sending of data was done before on the connection.)

Variants:

<A>....<B>

Preamble:

<A>	UART012

<B>	UART031C

(G)ACI/PPP		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_DATA_TEST_REQ	|		|

|	(e)	|		|

*	==========================>	*		|

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(e)	|

|		*	==========================>	*

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(7)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(10)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(11)	|		|	DTI2_DATA_TEST_IND	|

|		|	(e)	|

|		*	<==========================	*

(12)	|	DTI2_DATA_TEST_IND	|		|

|	(e)	|		|

*	<==========================	*		|

(13)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(14)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(15)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(16)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_E_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_E_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_E_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_E_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

History:

06-Mar-2001	LW	Initial

11-Jun-2001	STW	move DTI_GETDATA_REQ to testcase 012

11-Oct-2001	TVO	adapted for use with DTI2

### UART041: Mixed Receiving/Sending Multi Letter Strings

Description:

The UART entity receives and sends strings with a larger number of bytes than in UART040. Since the test case UART040 is in the preamble, the buffer size of the receive descriptor already has been increased once. Therefore the data flow is not disabled after receiving a one letter string.

Variants:

<A>....<B>

Preamble:

<A>	UART040A

<B>	UART040B

(G)ACI/PPP		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(10x h)	|

|		*	<==========================	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(15x Hello1\n\r)	|

|		*	<==========================	*

(7)	|	DTI2_DATA_TEST_REQ	|		|

|	(Hello2)	|		|

*	==========================>	*		|

(8)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(Hello2)	|

|		*	==========================>	*

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(10)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(11)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(12)	|	DTI2_DATA_TEST_IND	|		|

|	(first 30-octet part)	|		|

*	<==========================	*		|

(13)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(14)	|	DTI2_DATA_TEST_IND	|		|

|	(second 40-octet part)	|		|

*	<==========================	*		|

(15)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(16)	|	DTI2_DATA_TEST_IND	|		|

|	(third 50-octet part)	|		|

*	<==========================	*		|

(17)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(18)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(19)	|	DTI2_DATA_TEST_IND	|		|

|	(rest of data)	|		|

*	<==========================	*		|

(20)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_10H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_15XHELLO1_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO2_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HELLO2_SDU

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_1ST30HHELLO1_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_2ND40HHALLO1_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_3RD50HHALLO1_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_RESTHHALLO1_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

6-Mar-2001	LW	Initial

11-Sep-2001	STW	extende to N1=10 octets and minimum size_multiplier=3

11-Oct-2001	TVO	adapted for use with DTI2

# Multiplexer Tests (UART100 - UART200)

## Setup/End Multiplexer Mode

### UART100: Additional Setup for Multiplexer Tests

Description:

This test case does additional parameter setup which is only required in multiplexer tests. Currently there is nothing to do - it only provides free space where parameter setups/primitive can be put later which are to be used in every multiplexer test.

Preamble:

UART002

ACI		UART		UART_OUT

|		|		|

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

16-Jan-2001		LW			Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART101: ACI Prepares UART to Start Multiplexer

Description:

The (G)ACI starts the TS 27.010 multiplexer operation over a serial link. Only advanced mode with UIH frames ares supported. The UART starts the timer T3 to time waiting for the SABM frame.

Preamble:

UART002

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_START_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_GETDATA_REQ	|

|		|	(ENABLE)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	UART_MUX_START_CNF	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_START_REQ
	device	UART_DEVICE
	mode	UART_MUX_MODE_ADVANCED
	frame_type	UART_MUX_FRAME_UIH
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	t1	UART_MUX_T1_DEFAULT_TEST
	n2	UART_MUX_N2_DEFAULT
	t2	UART_MUX_T2_DEFAULT_TEST
	t3	UART_MUX_T3_DEFAULT_TEST

- DTI2_GETDATA_REQ
	link_id	LINK_ENABLE_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- UART_MUX_START_CNF
	device	UART_DEVICE

History:

16-Jan-2001	LW	Initial

11-Jun-2001	STW	add UART_PARAMETERS_CNF primitive

11-Oct-2001	TVO	adapted for use with DTI2

### UART102: Link Establishment Fails (Timeout T3)

Description:

The timer T3 expires. UART informs ACI that multiplexer operation establishment has failed and the UART returns to normal AT mode.

Preamble:

UART101

ACI		UART		UART_OUT

|		|		|

TIMEOUT(11000)

|		|		|

(1)	|	UART_MUX_CLOSE_IND	|		|

*	<==========================	*		|

(2)	|	UART_DTI_REQ	|		|

|	(UART_CONNECT_DTI)	|		|

*	==========================>	*		|

(3)	|	DTI2_CONNECT_IND	|		|

*	<==========================	*		|

(4)	|	DTI2_CONNECT_RES	|		|

*	==========================>	*		|

(5)	|	UART_DTI_CNF	|		|

|	(UART_CONNECT_DTI)	|		|

*	<==========================	*		|

(6)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(7)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_CLOSE_IND
	device	UART_DEVICE

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_ANY
	direction	FALSE
	link_id	LINK_ID_1
	entity_name	UART_PEER_NAME_DUMMY

- DTI2_CONNECT_IND
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_ANY

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

18-Jan-2001	LW	Initial

11-Jun-2001	STW	add DTI_GETDATA_REQ primitive

11-Oct-2001	TVO	adapted for use with DTI2

### UART103: Reception of SABM for Multiplexer Control Channel

Description:

The terminal equipment sends a SABM frame for the multiplexer control channel. The UART stops its timer T3 and answers with a positive UA frame for the control channel (DLCI 0).

Preamble:

UART101

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(SABM)	|

|		*<	===========================*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UA)	|

|		*	===========================>*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MUX_START0

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_UE2TE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

18-Jan-2001		LW			Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART105: Multiplexer Close-down (UE initiated)

Description:

The ACI requests termination of the multiplexer operation on the link. The peer is informed by sending a multiplexer close down command. A multiplexer close down response is received and confirms the successful reset of the link into normal AT command mode.

Preamble:

UART103

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_CLOSE_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CLOSE response)	|

|		*<	===========================*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(5)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(6)	|	UART_MUX_CLOSE_IND	|		|

*	<==========================*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_CLOSE_REQ
	device	UART_DEVICE

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_RES_TE2UE_CLOSE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- UART_MUX_CLOSE_IND
	device	UART_DEVICE

History:

17-Jan-2001		LW			Initial

8-Mar-2001		LW			Testcase now expects DTI_GETDATA_REQ

11-Oct-2001	TVO	adapted for use with DTI2

### UART106: Multiplexer Close-down (TE initiated)

Description:

The TE requests termination of the multiplexer operation on the link. The MMI is informed and a response is sent to the TE. There a two possibilities in which the TE can request close down of the multiplexer: The first one (variant A) is by using a multiplexer close down command/response and the second one (variant B) is using a DISC command/response with DLCI 0.

Variants:

<A>....<B>

Preamble:

UART103

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CLOSE command)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE response)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(5)	|	UART_MUX_CLOSE_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
<A> 	sdu	SDU_CMD_TE2UE_CLOSE0
<B> 	sdu	SDU_CMD_TE2UE_CLOSE1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
<A>	sdu	SDU_RES_UE2TE_CLOSE0
<B>	sdu	SDU_UA_UE2TE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND	link_id	LINK_WRITEDATA_PORT_1

- UART_MUX_CLOSE_IND
	device	UART_DEVICE

History:

17-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART107: Retransmission in Multiplexer Close-down (UE initiated, Timeout T2)

Description:

The ACI requests termination of the multiplexer operation on the link. The peer is informed by sending a multiplexer close down command. The Timer T2 expires and the first retransmission is done since the timer T3 is not expired yet. A multiplexer close down response is received and confirms the successful reset of the link into normal AT command mode.

Preamble:

UART103

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_CLOSE_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(4)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CLOSE response)	|

|		*<	===========================*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(8)	|	UART_MUX_CLOSE_IND	|		|

*	<==========================*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_CLOSE_REQ
	device	UART_DEVICE

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_RES_TE2UE_CLOSE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

- UART_MUX_CLOSE_IND
	device	UART_DEVICE

History:

24-Jan-2001		LW			Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART108: Multiplexer Close-down (UE initiated) - No Response

Description:

The ACI requests termination of the multiplexer operation on the link. The peer is informed by sending a multiplexer close down command. No close down command response is received - the timer T2 expires and 33 retransmissions are done until T3 expires. Then the UART switches to normal mode.

Preamble:

UART103

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_CLOSE_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(4)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(5)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(6)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(8)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(10)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(11)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(12)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(13)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(14)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(15)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(16)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(17)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(18)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(19)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(20)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(21)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(22)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(23)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(900)

(24)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*=	==========================>*

(25)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(26)	|	UART_MUX_CLOSE_IND	|		|

*	<==========================*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_CLOSE_REQ
	device	UART_DEVICE

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- UART_MUX_CLOSE_IND
	device	UART_DEVICE

History:

13-Mar-2001		LW			Initial

21-Mar-2001		LW			retransmissions until T3 expires

11-Oct-2001	TVO	adapted for use with DTI2

### UART109: Start Multiplexer with previous data transfer

Description:

ACI send first new parameters for serial link. After that ACI starts the TS 27.010 multiplexer operation over a serial link. Only advanced mode with UIH frames ares supported. The UART starts the timer T3 to time waiting for the SABM frame.

Preamble:

UART041A

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_PARAMETERS_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_GETDATA_REQ	|

|		|	(DISABLE)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		|	(ENABLE)	|

|		*	==========================>	*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(5)	|	UART_PARAMETERS_CNF	|		|

*	<==========================	*		|

(6)	|	UART_MUX_START_REQ	|		|

*	==========================>	*		|

(7)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(8)	|	UART_MUX_START_CNF	|		|

*	<==========================	*		|

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(SABM)	|

|		*<	===========================*

(10)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UA)	|

|		*	===========================>*

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_PARAMETERS_REQ
	device	UART_DEVICE
	comPar	COMPAR_57600_8_N_1_HW

- DTI2_GETDATA_REQ
	link_id	LINK_DISABLE_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ENABLE_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- UART_PARAMETERS_CNF
	device	UART_DEVICE

- UART_MUX_START_REQ
	device	UART_DEVICE
	mode	UART_MUX_MODE_ADVANCED
	frame_type	UART_MUX_FRAME_UIH
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	t1	UART_MUX_T1_DEFAULT_TEST
	n2	UART_MUX_N2_DEFAULT
	t2	UART_MUX_T2_DEFAULT_TEST
	t3	UART_MUX_T3_DEFAULT_TEST

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_MUX_START_CNF
	device	UART_DEVICE

- DTI2_DATA_TEST_IND
	link_id	LINK_READDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MUX_START0

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_UE2TE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

History:

26-Apr-2001	STW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

16-Jan-2003	STW	error correction

## DLC Establishment

### UART123: DLC Establishment Requested by TE

Description:

The TE wants to establish a data link connection (DLC) with a not yet allocated (variant A) or already allocated (variant B) data link connection identifier (DLCI). The MMI is notified of the new data link connection.

Variants:

<A>....<B>

Preamble:

<A>UART103

<B>UART109

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(SABM DLCI A)	|

|		*<	===========================*

(2)	|	UART_MUX_DLC_ESTABLISH_IND	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_ESTABLISH_DLCI_A_TE2UE

- UART_MUX_DLC_ESTABLISH_IND
	device	UART_DEVICE
	dlci	DLCI_A
	convergence	UART_MUX_CONVERGENCE_UOS
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_DEFAULT

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

History:

17-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART124: MMI Confirmation of TE Initiated DLC

Description:

The MMI accepts the TE initiated DLC, a confirmation is sent to the TE. In variant B there existed a valid connection with this DLCI before, in variant A it did not.

Variants:

<A>....<B>

Preamble:

<A>UART123A

<B>UART123B

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_DLC_ESTABLISH_RES	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UA)	|

|		*	==========================>	*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_DLC_ESTABLISH_RES
	device	UART_DEVICE
	dlci	DLCI_A
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_UE2TE_DLCI_A

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

History:

17-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART125: MMI Cancellation of TE Initiated DLC

Description:

The MMI rejects the UE initiated DLC, a negative confirmation is sent to the UE. In variant B there existed a valid connection with this DLCI before, in variant A it did not.

Variants:

<A>....<B>

Preamble:

<A>UART123A

<B>UART123B

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_DLC_RELEASE_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DM)	|

|		*	==========================>	*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_DLC_RELEASE_REQ
	device	UART_DEVICE
	dlci	DLCI_A

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DM_UE2TE_A

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

History:

17-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART126: Second DLC Successfully Requested by TE

Description:

The TE wants to establish a second data link connection (DLC) with a not yet allocated data link connection identifier (DLCI). A SABM frame is received in order to set up the connection. In variant A the first DLC request has been completed before this second request in variant B the first request is still pending.

Variants:

<A>....<C>

Preamble:

<A>UART124A

<B>UART124B

<C>UART131G

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(SABM)	|

|		*	<==========================	*

(2)	|	UART_MUX_DLC_ESTABLISH_IND	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	UART_MUX_DLC_ESTABLISH_RES	|		|

*	==========================>	*		|

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UA)	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_ESTABLISH_DLCI_B_TE2UE

- UART_MUX_DLC_ESTABLISH_IND
	device	UART_DEVICE
	dlci	DLCI_B
	convergence	UART_MUX_CONVERGENCE_UOS
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_DEFAULT

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- UART_MUX_DLC_ESTABLISH_RES
	device	UART_DEVICE
	dlci	DLCI_B
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_UE2TE_DLCI_B

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

17-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART127: DTI Assignment on first DLC

Description:

After successfully establishing a DLC the ACI assigns a DTI connection to PPP.

Variants:

<A>....<D>

Preamble:

<A>UART124A

<B>UART124B

<C>UART126A

<D>UART126B

ACI/PPP		UART		UART_OUT

|		|		|

(1)	|	UART_DTI_REQ	|		|

|	(UART_CONNECT_DTI)	|		|

*	==========================>	*		|

(2)	|	DTI2_CONNECT_IND	|		|

*	<==========================	*		|

(3)	|	DTI2_CONNECT_RES	|		|

*	==========================>	*		|

(4)	|	UART_DTI_CNF	|		|

|	(UART_CONNECT_DTI)	|		|

*	<==========================	*		|

(5)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(6)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_A
	direction	FALSE
	link_id	LINK_ID_1
	entity_name	DTI_CHANNELNAME_PPP

- DTI2_CONNECT_IND
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_A

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

13-Mar-2001		LW			Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART128: DTI Assignment on second DLC

Description:

A second DTI connection is setup by ACI.

Variants:

<A>....<B>

Preamble:

<A>	UART127C

<B>	UART127D

ACI/PPP		UART		UART_OUT

|		|		|

(1)	|	UART_DTI_REQ	|		|

|	(UART_CONNECT_DTI)	|		|

*	==========================>	*		|

(2)	|	DTI2_CONNECT_IND	|		|

*	<==========================	*		|

(3)	|	DTI2_CONNECT_RES	|		|

*	==========================>	*		|

(4)	|	UART_DTI_CNF	|		|

|	(UART_CONNECT_DTI)	|		|

*	<==========================	*		|

(5)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(6)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_B
	direction	FALSE
	link_id	LINK_ID_2
	entity_name	DTI_CHANNELNAME_PPP

- DTI2_CONNECT_IND
	link_id	LINK_ID_2
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_2
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_B

- DTI2_READY_IND
	link_id	LINK_ID_2

- DTI2_GETDATA_REQ
	link_id	LINK_ID_2

History:

13-Mar-2001		LW			Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART129: DCD request after DTI configuration

Description:

After successfully establishing a DLC the ACI assigns a DTI connection to PPP.

Preamble:

UART124B

ACI/PPP		UART		UART_OUT

|		|		|

(1)	|	UART_DTI_REQ	|		|

|	(UART_CONNECT_DTI)	|		|

*	==========================>	*		|

(2)	|	DTI2_CONNECT_IND	|		|

*	<==========================	*		|

(3)	|	DTI2_CONNECT_RES	|		|

*	==========================>	*		|

(4)	|	UART_DTI_CNF	|		|

|	(UART_CONNECT_DTI)	|		|

*	<==========================	*		|

(5)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(6)	|	UART_DCD_REQ	|		|

*	==========================>	*		|

(7)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(MSC command)	|

|		*	==========================>	*

(8)	|	UART_DCD_CNF	|		|

*	<==========================	*		|

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(10)	|		|	DTI2_DATA_TEST_IND	|

|		|	(wrong double response)	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UIH empty)	|

|		*	==========================>	*

(12)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(13)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(14)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UIH empty)	|

|		*	==========================>	*

(15)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(16)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_A
	direction	FALSE
	link_id	LINK_ID_1
	entity_name	DTI_CHANNELNAME_PPP

- DTI2_CONNECT_IND
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_A

- DTI2_READY_IND
	link_id	LINK_ID_1

- UART_DCD_REQ
	device	UART_DEVICE
	dlci	DLCI_A
	line_state	UART_LINE_ON

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSC_CMD_UE2TE_A

- UART_DCD_CNF
	device	UART_DEVICE
	dlci	DLCI_A

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_READDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCMSC_RES_A

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CORRUPTED_RES_EMPTY

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CORRUPTED_RES_EMPTY

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

14-May-2001	STW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

## Multiplexed DLC Data Transmission

### UART131: Sending Data on Single DLC (UE->TE)

Description:

The peer entity A sends data on DLCI A. The multiplexer builds an information frame and forwards it to the UART test output interface.

Variants:

<A>....<G>

Preamble:

<A>UART127A

<B>UART127B

<C>UART127C
<D>UART127D
<E>UART128A

<F>UART128B

<G>UART129

PEER_A		UART		UART_OUT

|		|		|

(1)	|	DTI2_DATA_TEST_REQ	|		|

|	(Hello1)	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UIH)	|

|		*	===========================>*

(3)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(4)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_HELLO1_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_HELLO1_UE2TE_A

- DTI2_READY_IND	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

History:

18-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART132: Receiving Data on Single DLC (TE->UE)

Description:

A data frame arrives on DLCI A. The multiplexer extracts the information from the arrived frame and forwards the encapsulated string "Hello2" to peer entity A.

Variants:

<A>....<F>

Preamble:

<A>UART128A

<B>UART128B

<C>UART131A

<D>UART131B

<E>UART131E

<F>UART131F

PEER_A		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_H_TE2UE_A

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

18-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART133: Mixed Sending/Receiving of Data on Two DLCs

Description:

A data frame arrives on DLCI A. The multiplexer extracts the information from the arrived frame and forwards the encapsulated string "Hello2" to peer entity A. Next the peer B wants to transmit some data. It is encapsulated in an appropriate UIH frame and sent out.

Variants:

<A>....<F>

Preamble:

<A>UART128A
<B>UART128B
<C>UART131E

<D>UART131F

<E>UART132E

<F>UART132F

PEER_A/PEER_B		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(Hello2)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(5)	|	DTI2_DATA_TEST_REQ	|		|

|	(Hello1)	|		|

*	==========================>	*		|

(6)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UIH)	|

|		*	==========================>	*

(7)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(8)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
<A>	sdu	SDU_FRAME_HELLO2_TE2UE_A
<B>	sdu	SDU_FRAME_HELLO2_TE2UE_A
<C>	sdu	SDU_FRAME_HELLO2_TE2UE_A
<D>	sdu	SDU_FRAME_HELLO2_TE2UE_A
<E>	sdu	SDU_FRAME_HELLO2_TE2UE_A
<F>	sdu	SDU_FRAME_64_TE2UE_A

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_PEER_A
<A>	sdu	STRING_HELLO2_SDU
<B>	sdu	STRING_HELLO2_SDU
<C>	sdu	STRING_HELLO2_SDU
<D>	sdu	STRING_HELLO2_SDU
<E>	sdu	STRING_HELLO2_SDU
<F>	sdu	STRING_64_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_2
	parameters	DTI_PARAMETER_PEER_B
	sdu	STRING_HELLO1_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_PEER_B
	sdu	SDU_FRAME_HELLO1_UE2TE_B

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_2

History:

18-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART134: Receiving Data on Single DLC (TE->UE) with Flow Control

Description:

A  long data frame arrives on DLCI A. The multiplexer extracts the information from the arrived frame. The DTX entity disables the data flow since its buffer is full. A modem status command for the channel is sent out. DTX forwards the encapsulated string "Hello2" to peer entity A and enables the flow again.

TEST CASE IS STILL INCOMPLETE!

Variants:

<A>....<C>

Preamble:

<A>UART128A

<B>UART131A

<C>UART131C

PEER_A		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH)	|

|		*<	===========================*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(Hello2)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_HELLO2_TE2UE_A

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_HELLO2_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

18-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

## Unsupported Features/Commands

### UART140: Reception of Unsupported Command

Description:

An unsupported command is received from TE. In variant A it is received for an open DLC in variant B it is received with an unassigned DLCI. A non supported command response frame is sent back to the peer entity in both cases.

Variants:

<A>...<B>

Preamble:

<A>	UART128A
<B>	UART103

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(unknown)	|

|		*	<===========================*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(NON SUPP CMD)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_READDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UNKNOWN_CMD_TE2UE_A

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UNSUPP_CMD_UE2TE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

History:

24-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

## Modem Status Commands

### UART150: Reception of MSC

Description:

The peer sends a modem status command for an established DLC. Since it does not contain a ring or break signal, no notification is sent to the ACI.

Preamble:

UART128A

PEER_A		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(MSC command)	|

|		*	<===========================*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(MSC response)	|

|		*	===========================>*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSC_CMD_TE2UE_A

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSC_RES_UE2TE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

24-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART151: Reception of Break-MSC

Description:

The peer sends a modem status command for an established DLC indicating a break signal for DLCI A. The MMI/ACI is notified by the UART entity with an empty DTI2_DATA_IND primitive.

Preamble:

UART150

PEER_A		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(MSC command)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(UART_BREAK_LEN)	|		|

*	<==========================	*		|

(3)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(4)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(MSC response)	|

|		*	==========================>	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCBREAK_CMD_TE2UE_A

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME_BREAK
	sdu	EMPTY_STRING_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCBREAK_RES_UE2TE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

24-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART152: Sending Break-MSC

Description:

The device A requests that a break signal is to be sent on DLCI_A. An appropriate MSC frame is sent to the peer multiplexer.

Preamble:

UART150

PEER_A		UART		UART_OUT

|		|		|

(1)	|	DTI2_DATA_TEST_REQ	|		|

|	(UART_BREAK_LEN)	|		|

*	==========================>*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(MSC command)	|

|		*	===========================>*

(3)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(MSC response)	|

|		*	<===========================*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (3000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME_BREAK
	sdu	EMPTY_STRING_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCBREAK_CMD_UE2TE_A

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCBREAK_RES_TE2UE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

History:

24-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART153: Sending Ring-MSC

Description:

The device A requests that an incomming call indication is to be sent on DLCI_A. An appropriate MSC frame is sent to the peer multiplexer.

Preamble:

UART150

PEER_A		UART		UART_OUT

|		|		|

(1)	|	UART_RING_REQ	|		|

*	==========================>*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(MSC command)	|

|		*	===========================>*

(3)	|	UART_RING_CNF	|		|

*	<==========================*		|

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(MSC response)	|

|		*	<===========================*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (3000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_RING_REQ
	device	UART_DEVICE
	dlci	DLCI_A
	line_state	UART_LINE_ON

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCRING_CMD_UE2TE_A

- UART_RING_CNF
	device	UART_DEVICE
	dlci	DLCI_A

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCRING_RES_TE2UE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

History:

24-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART154: Retransmission of Break-MSC

Description:

The device A requests that a break signal is to be sent on DLCI_A. An appropriate MSC frame is sent to the peer multiplexer. The received MSC response is different from the expected response, therefore the MSC command is retransmitted when the timer T2 expires. Then the correct response message is received and the timer T2 is stopped.

Preamble:

UART150

PEER_A		UART		UART_OUT

|		|		|

(1)	|	DTI2_DATA_TEST_REQ	|		|

|	(UART_BREAK_LEN)	|		|

*	==========================>*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(MSC command)	|

|		*	===========================>*

(3)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(invalid MSC response)	|

|		*	<===========================*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

TIMEOUT (900)

(7)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(MSC command)	|

|		*	===========================>*

(8)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(correct MSC response)	|

|		*	<===========================*

(10)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (3000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME_BREAK
	sdu	EMPTY_STRING_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCBREAK_CMD_UE2TE_A

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCBREAK_RES_TE2UE_A_INVALID

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCBREAK_CMD_UE2TE_A

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCBREAK_RES_TE2UE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

History:

21-Mar-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART155: Flow-Control-MSC on reception side

Description:

DLCI_A receives more characters than it can process. It sends a Flow-Control-MSC to the peer to stop sending.

Preamble:

UART132D

PEER_A		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH e)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(e)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH max size)	|

|		*	<==========================	*

(5)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH h)	|

|		*	<==========================	*

(7)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(FC off)	|

|		*	==========================>	*

(8)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(10)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH multi res)	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UIH empty)	|

|		*	==========================>	*

(12)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

(13)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(14)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UIH empty)	|

|		*	==========================>	*

(15)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(16)	|	DTI2_GETDATA_REQ	|		|
	*	==========================>	*		|

(17)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(MSC command FC on)	|

|		*	==========================>	*

(18)	|	DTI2_DATA_TEST_IND	|		|

|	(h+cops)	|		|

*	<==========================	*		|

(19)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(20)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH wrong FC on res)	|

|		*	<==========================	*

(21)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UIH empty)	|

|		*	==========================>	*

(22)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

(23)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(24)	|	DTI2_GETDATA_REQ	|		|
	*	==========================>	*		|

MUTE (3000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_READDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_E_TE2UE_A

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_E_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_READDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_64_TE2UE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_READDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_H_TE2UE_A

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSCFCOFF_CMD_UE2TE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_READDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DMSCMSCDDDD_RES_A

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CORRUPTED_RES_EMPTY

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CORRUPTED_RES_EMPTY

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_MSC_CMD_UE2TE_A

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_PLUSCOPS_SDU

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_READDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CORRUPT_MSC_RES_A

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CORRUPTED_RES_EMPTY

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_WRITEDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

10-May-2001	STW	Initial

11-Sep-2001	STW	new minimum size_multiplier=3

11-Oct-2001	TVO	adapted for use with DTI2

## DLC Release

### UART160: DLC Release Requested by ACI

Description:

The ACI wants to release a data link connection (DLC) with an allocated data link connection identifier (DLCI). A DISC frame is transmitted in order to release the connection.

Variants:

<A>....<C>

Preamble:

<A> UART127A
<B> UART127C
<C> UART128A

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_DLC_RELEASE_REQ	|		|

*	==========================>	*		|

(2)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_DLC_RELEASE_REQ
	device	UART_DEVICE
	dlci	DLCI_A

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_A_UE2TE

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

17-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

16-Jan-2003	STW	error correction

### UART161: Confirmation of DLC Release (initiated by UE)

Description:

The confirmation for a previously requested release of a DLC arrives.

Variants:

<A>....<F>

Preamble:

<A> UART160A
<B> UART160B
<C> UART160C
<D> UART169A
<E> UART169B
<F> UART169C

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UA)	|

|		*	<==========================	*

(2)	|	UART_MUX_DLC_RELEASE_IND	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_READDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_TE2UE_DLCI_A

- UART_MUX_DLC_RELEASE_IND
	device	UART_DEVICE
	dlci	DLCI_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

History:

17-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART162: DLC Release Requested by TE

Description:

The TE wants to release a data link connection (DLC) with an allocated data link connection identifier (DLCI). A DISC frame is received, a confirmation is sent and the MMI is notified of the DLC release.

Preamble:

UART127C

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DISC)	|

|		*	<==========================	*

(2)	|	UART_MUX_DLC_RELEASE_IND	|		|

*	<==========================	*		|

(3)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UA)	|

|		*	==========================>	*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(5)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_B_TE2UE

- UART_MUX_DLC_RELEASE_IND
	device	UART_DEVICE
	dlci	DLCI_B

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_UE2TE_DLCI_B

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

17-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART163: Data Received for Already Released Channel

Description:

Although the DLC A has previously been released, more data is received. Since the specified channel is in disconnected mode the UART responds with a DM frame.

Variants:

<A>....<F>

Preamble:

<A> UART161A
<B> UART161B
<C> UART161C
<D> UART161D
<E> UART161E
<F> UART161F

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DM)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_HELLO2_TE2UE_A

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DM_UE2TE_A_0F

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

24-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART164: No Response to Retransmissions of DLC DISC Frame

Description:

In test case UART160 a DISC for DLCI_A was sent. Now the timer T1 expires. Because the counter for retransmissions in not zero yet the UART transmitts the DISC frame again and decreases the retransmission counter by one. After three retransmissions the max. number of retransmissions is reached and the channel is close without the response.

Variants:

<A>....<F>

Preamble:

<A> UART160A
<B> UART160B
<C> UART160C
<D> UART169A
<E> UART169B
<F> UART169C

ACI		UART		UART_OUT

|		|		|

TIMEOUT(300)

(1)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(2)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(300)

(3)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(300)

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

TIMEOUT(5000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_A_UE2TE

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_A_UE2TE

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_A_UE2TE

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

24-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART165: ACI Initiated Multiplexer Close-down (icl. DLC release)

Description:

The ACI wants to end the multiplexer operation and sends a UART MUX CLOSE request to the UART entity. Then the UART releases all DLCs currently in use (DLCI_A, DLCI_B) and uses the usual close down procedure (see UART105) to return to the normal AT mode.

Preamble:

UART126A

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_CLOSE_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DM)	|

|		*	<==========================	*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(5)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(6)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DM)	|

|		*	<==========================	*

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(10)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*	==========================>	*

(11)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CLOSE response)	|

|		*	<==========================	*

(13)	|	UART_MUX_CLOSE_IND	|		|

*	<==========================	*		|

(14)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_CLOSE_REQ
	device	UART_DEVICE

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_A_UE2TE

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DM_TE2UE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_B_UE2TE

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DM_TE2UE_B

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_RES_TE2UE_CLOSE

- UART_MUX_CLOSE_IND
	device	UART_DEVICE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

History:

24-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART166: ACI Initiated Multiplexer Close-down (icl. DLC release and DTI channel)

Description:

The ACI wants to end the multiplexer operation and sends a UART MUX CLOSE request to the UART entity. Then the UART releases all DLCs currently in use (DLCI_A, DLCI_B) and uses the usual close down procedure (see UART105) to return to the normal AT mode.

Preamble:

UART127C

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_CLOSE_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DM)	|

|		*	<==========================	*

(4)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DM)	|

|		*	<==========================	*

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*	==========================>	*

(12)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(13)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CLOSE response)	|

|		*	<==========================	*

(14)	|	UART_MUX_CLOSE_IND	|		|

*	<==========================	*		|

(15)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_CLOSE_REQ
	device	UART_DEVICE

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_A_UE2TE

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DM_TE2UE_A

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_B_UE2TE

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DM_TE2UE_B

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_RES_TE2UE_CLOSE

- UART_MUX_CLOSE_IND
	device	UART_DEVICE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

History:

10-Oct-2001	TVO	Initial, adapted from UART165

11-Oct-2001	TVO	adapted for use with DTI2

16-Jan-2003	STW	Error corrections

### UART167: ACI Initiated Multiplexer Close-down (icl. DLC release and two DTI channels)

Description:

The ACI wants to end the multiplexer operation and sends a UART MUX CLOSE request to the UART entity. Then the UART releases all DLCs currently in use (DLCI_A, DLCI_B) and uses the usual close down procedure (see UART105) to return to the normal AT mode.

Preamble:

UART128A

ACI		UART		UART_OUT

|		|		|

(1)	|	UART_MUX_CLOSE_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(3)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DM)	|

|		*	<==========================	*

(4)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DM)	|

|		*	<==========================	*

(9)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(10)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(11)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(12)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(CLOSE command)	|

|		*	==========================>	*

(13)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(14)	|		|	DTI2_DATA_TEST_IND	|

|		|	(CLOSE response)	|

|		*	<==========================	*

(15)	|	UART_MUX_CLOSE_IND	|		|

*	<==========================	*		|

(16)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_MUX_CLOSE_REQ
	device	UART_DEVICE

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_A_UE2TE

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DM_TE2UE_A

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_B_UE2TE

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DM_TE2UE_B

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_2
	cause	DTI_CAUSE_NORMAL_CLOSE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CMD_UE2TE_CLOSE

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_RES_TE2UE_CLOSE

- UART_MUX_CLOSE_IND
	device	UART_DEVICE

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

History:

10-Oct-2001	TVO	Initial, adapted from UART165

11-Oct-2001	TVO	adapted for use with DTI2

### UART168: DLC Release Requested by TE (two MUX channels)

Description:

The TE wants to release a data link connection (DLC) with an allocated data link connection identifier (DLCI). A DISC frame is received, a confirmation is sent and the MMI is notified of the DLC release.

Preamble:

UART128A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DISC)	|

|		*	<==========================	*

(2)	|	DTI2_DISCONNECT_IND	|		|

*	<==========================	*		|

(3)	|	UART_MUX_DLC_RELEASE_IND	|		|

*	<==========================	*		|

(4)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UA)	|

|		*	==========================>	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_B_TE2UE

- DTI2_DISCONNECT_IND
	link_id	LINK_ID_2
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_MUX_DLC_RELEASE_IND
	device	UART_DEVICE
	dlci	DLCI_B

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_UE2TE_DLCI_B

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

17-Jan-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

16-Jan-2003	STW	Error corrections

### UART169: DLC Release Requested by ACI, following a DTI2_DISCONNECT_REQ

Description:

The ACI wants to release a data link connection (DLC) with an allocated data link connection identifier (DLCI). A DISC frame is transmitted in order to release the connection.

Variants:

<A>....<C>

Preamble:

<A> UART127A
<B> UART127C
<C> UART128A

ACI		UART		UART_OUT

|		|		|

(1)	|	DTI2_DISCONNECT_REQ	|		|

*	==========================>	*		|

(2)	|	UART_DTI_IND	|		|

|	(UART_DISCONNECT_DTI)	|		|

*	<==========================	*		|

(3)	|	UART_MUX_DLC_RELEASE_REQ	|		|

*	==========================>	*		|

(4)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DISC)	|

|		*	==========================>	*

(5)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DISCONNECT_REQ
	link_id	LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_A

- UART_MUX_DLC_RELEASE_REQ
	device	UART_DEVICE
	dlci	DLCI_A

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DISC_DLCI_A_UE2TE

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

12-Oct-2001	TVO	Initial

16-Jan-2003	STW	Error corrections

## Handling of Invalid Frames

### UART170: Reception of Corrupted Frame

Description:

The UART entity receives invalid data. It ignores all of it.

<A>	no flags in data
<B>	upper flag corrupted
<C>	lower flag corrupted
<D>	only flags received
<E>	FCS invalid
<F>	only two octets between flags
<G>	EA bit in address not set
<H>	control octet not valid
<I>		SABM frame with P bit set to 0
<J>	DISC frame with P bit set to 0
<K>	UA frame with F bit set to 0
<L>	EA bit in type octet is not set (type > 1 octet not supported)
<M>	EA bit in length octet is not set (length > 1 octet not supported)

<N>	frame longer than 64 octets

Variants:

<A>....<N>

Preamble:

UART128A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(corrupted data)	|

|		*	<===========================*

(2)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
<A>	sdu	SDU_CORRUPTED_DATA1
<B>	sdu	SDU_CORRUPTED_DATA2
<C>	sdu	SDU_CORRUPTED_DATA3
<D>	sdu	SDU_CORRUPTED_DATA4
<E>	sdu	SDU_CORRUPTED_DATA6
<F>	sdu	SDU_CORRUPTED_DATA7
<G>	sdu	SDU_CORRUPTED_DATA9
<H>	sdu	SDU_CORRUPTED_DATA11
<I>	sdu	SDU_CORRUPTED_DATA12
<J>	sdu	SDU_CORRUPTED_DATA13
<K>	sdu	SDU_CORRUPTED_DATA14
<L>	sdu	SDU_CORRUPTED_DATA16
<M>	sdu	SDU_CORRUPTED_DATA17
<N>	sdu	SDU_CORRUPTED_DATA21

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

History:

14-Mar-2001	LW	Initial

27-Aug-2001	STW	add frame longer than 64 octets

11-Oct-2001	TVO	adapted for use with DTI2

### UART171: Reception of Frame for Unknown Channel

Description:

The UART entity receives data for an unknown dlci. A DM frame (type 0F since it is a response for a information and not a command frame) is sent to the peer.

Preamble:

UART128A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(UIH)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DM)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

MUTE (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CORRUPTED_DATA8

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_DM_CORRUPTED_DATA8

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

14-Mar-2001	LW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART172: Reception of Corrupted Message

Description:

The UART entity receives invalid data. It responses an empty UIH Control frame.

<A>	corrupt length field in message
<B>	message information shorter than length (MSC)
<C>	message information longer than length (MSC)

Variants:

<A>....<C>

Preamble:

UART128A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(corrupted message)	|

|		*	<==========================	*

(2)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(empty UIH)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

MUTE (2000)

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
<A>	sdu	SDU_CORRUPTED_DATA18
<B>	sdu	SDU_CORRUPTED_DATA19
<C>	sdu	SDU_CORRUPTED_DATA20

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_CORRUPTED_RES_EMPTY

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

History:

23-Mar-2001	STW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

## HDLC Escape Characters in Multiplexer Frames

### UART180: Establishment of two additional Multiplexer connections

Description:

Two Multiplexer connections are active. In this testcase two additional DLCs are set up. These additional DLCs have special DLCIs to force the use of HDLC escape characters.

Preamble:

UART128B

ACI/DTI_PEER		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(SABM DLCI D)	|

|		*	<==========================	*

(2)	|	UART_MUX_DLC_ESTABLISH_IND	|		|

|	(DLCI D)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(SABM DLCI E)	|

|		*	<==========================	*

(5)	|	UART_MUX_DLC_ESTABLISH_IND	|		|

|	(DLCI E)	|		|

*	<==========================	*		|

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(7)	|	UART_MUX_DLC_ESTABLISH_RES	|		|

|	(DLCI D)	|		|

*	==========================>	*		|

(8)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UA DLCI D)	|

|		*	==========================>	*

(9)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(10)	|	UART_MUX_DLC_ESTABLISH_RES	|		|

|	(DLCI E)	|		|

*	==========================>	*		|

(11)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UA DLCI E)	|

|		*	==========================>	*

(12)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(13)	|	UART_DTI_REQ	|		|

|	(LINK_ID 4)	|		|

*	==========================>	*		|

(14)	|	DTI2_CONNECT_IND	|		|

|	(LINK_ID 4)	|		|

*	<==========================	*		|

(15)	|	UART_DTI_REQ	|		|

|	(LINK_ID 5)	|		|

*	==========================>	*		|

(16)	|	DTI2_CONNECT_IND	|		|

|	(LINK_ID 5)	|		|

*	<==========================	*		|

(17)	|	DTI2_CONNECT_RES	|		|

|	(LINK_ID 5)	|		|

*	==========================>	*		|

(18)	|	UART_DTI_CNF	|		|

|	(LINK_ID 5)	|		|

*	<==========================	*		|

(19)	|	DTI2_READY_IND	|		|

|	(LINK_ID 5)	|		|

*	<==========================	*		|

(20)	|	DTI2_CONNECT_RES	|		|

|	(LINK_ID 4)	|		|

*	==========================>	*		|

(21)	|	UART_DTI_CNF	|		|

|	(LINK_ID 4)	|		|

*	<==========================	*		|

(22)	|	DTI2_READY_IND	|		|

|	(LINK_ID 4)	|		|

*	<==========================	*		|

(23)	|	DTI2_GETDATA_REQ	|		|

|	(LINK_ID 5)	|		|

*	==========================>	*		|

(24)	|	DTI2_GETDATA_REQ	|		|

|	(LINK_ID 4)	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_ESTABLISH_DLCI_D_TE2UE

- UART_MUX_DLC_ESTABLISH_IND
	device	UART_DEVICE
	dlci	DLCI_D
	convergence	UART_MUX_CONVERGENCE_UOS
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_DEFAULT

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_ESTABLISH_DLCI_E_TE2UE

- UART_MUX_DLC_ESTABLISH_IND
	device	UART_DEVICE
	dlci	DLCI_E
	convergence	UART_MUX_CONVERGENCE_UOS
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_DEFAULT

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- UART_MUX_DLC_ESTABLISH_RES
	device	UART_DEVICE
	dlci	DLCI_D
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_UE2TE_DLCI_D

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- UART_MUX_DLC_ESTABLISH_RES
	device	UART_DEVICE
	dlci	DLCI_E
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_UE2TE_DLCI_E

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_D
	direction	FALSE
	link_id	LINK_ID_4
	entity_name	DTI_CHANNELNAME_PPP

- DTI2_CONNECT_IND
	link_id	LINK_ID_4
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_E
	direction	FALSE
	link_id	LINK_ID_5
	entity_name	DTI_CHANNELNAME_PPP

- DTI2_CONNECT_IND
	link_id	LINK_ID_5
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_5
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_E

- DTI2_READY_IND
	link_id	LINK_ID_5

- DTI2_CONNECT_RES
	link_id	LINK_ID_4
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_D

- DTI2_READY_IND
	link_id	LINK_ID_4

- DTI2_GETDATA_REQ
	link_id	LINK_ID_5

- DTI2_GETDATA_REQ
	link_id	LINK_ID_4

History:

23-Jan-2003	STW	Initial

### UART181: HDLC Escape character in Address field and Information field

Description:

Mixed data transfer on all active DLCs. The data contains XON (0x11) and XOFF (0x13) characters, the HDLC Escape (0x7d) character and the HDLC Flag (0x7e) character. So the Information field must be extended with HDLC Escape characters. In case of DLCI D and E the Address field must also be extended with HDLC Escape characters.

Preamble:

UART180

DTI_PEER		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI A)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id1:0x10,0x11,0x12)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI D)	|

|		*	<==========================	*

(5)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id4:0x13,0x14,0x15)	|		|

*	<==========================	*		|

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI E)	|

|		*	<==========================	*

(8)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id5:0x7c,0x7d,0x7e)	|		|

*	<==========================	*		|

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI B)	|

|		*	<==========================	*

(11)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id2:0x7e,0x7f,0x80)	|		|

*	<==========================	*		|

(12)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(13)	|	DTI2_DATA_TEST_REQ	|		|

|	(link_id5:0x7e,0x7f,0x80)	|		|

*	==========================>	*		|

(14)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DLCI E)	|

|		*	==========================>	*

(15)	|	DTI2_DATA_TEST_REQ	|		|

|	(link_id4:0x7c,0x7d,0x7e)	|		|

*	==========================>	*		|

(16)	|	DTI2_DATA_TEST_REQ	|		|

|	(link_id2:0x13,0x14,0x15)	|		|

*	==========================>	*		|

(17)	|	DTI2_DATA_TEST_REQ	|		|

|	(link_id1:0x10,0x11,0x12)	|		|

*	==========================>	*		|

(18)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(19)	|	DTI2_READY_IND	|		|

|	(link_id5)	|		|

*	<==========================	*		|

(20)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DLCI B)	|

|		*	==========================>	*

(21)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI A)	|

|		*	<==========================	*

(22)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(23)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(24)	|	DTI2_READY_IND	|		|

|	(link_id2)	|		|

*	<==========================	*		|

(25)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DLCI D)	|

|		*	==========================>	*

(26)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(27)	|	DTI2_READY_IND	|		|

|	(link_id4)	|		|

*	<==========================	*		|

(28)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DLCI A)	|

|		*	==========================>	*

(29)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(30)	|	DTI2_READY_IND	|		|

|	(link_id1)	|		|

*	<==========================	*		|

(31)	|	DTI2_GETDATA_REQ	|		|

|	(link_id5)	|		|

*	==========================>	*		|

(32)	|	DTI2_GETDATA_REQ	|		|

|	(link_id4)	|		|

*	==========================>	*		|

(33)	|	DTI2_GETDATA_REQ	|		|

|	(link_id1)	|		|

*	==========================>	*		|

(34)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id1:Hello2)	|		|

*	<==========================	*		|

(35)	|	DTI2_GETDATA_REQ	|		|

|	(link_id2)	|		|

*	==========================>	*		|

(36)	|	DTI2_GETDATA_REQ	|		|

|	(link_id1)	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_101112_TE2UE_A

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_101112_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_131415_TE2UE_D

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_4
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_131415_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_7c7d7e_TE2UE_E

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_5
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_7c7d7e_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_7e7f80_TE2UE_B

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_2
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_7e7f80_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_5
	parameters	DTI_PARAMETER_PEER_B
	sdu	STRING_7e7f80_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_PEER_B
	sdu	SDU_FRAME_7e7f80_UE2TE_E

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_4
	parameters	DTI_PARAMETER_PEER_B
	sdu	STRING_7c7d7e_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_2
	parameters	DTI_PARAMETER_PEER_B
	sdu	STRING_131415_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_PEER_B
	sdu	STRING_101112_SDU

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_5

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_PEER_B
	sdu	SDU_FRAME_131415_UE2TE_B

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_HELLO2_TE2UE_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_2

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_PEER_B
	sdu	SDU_FRAME_7c7d7e_UE2TE_D

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_4

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_PEER_B
	sdu	SDU_FRAME_101112_UE2TE_A

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_5

- DTI2_GETDATA_REQ
	link_id	LINK_ID_4

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_HELLO2_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_2

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

24-Jan-2003	STW	Initial

### UART182: HDLC Escape character in Control field and FCS field

Description:

Mixed data transfer on all active DLCs. The data contains XON (0x11) and XOFF (0x13) characters, the HDLC Escape (0x7d) character and the HDLC Flag (0x7e) character. So the Information field must be extended with HDLC Escape characters. In case of DLCI D and E the Address field must also be extended with HDLC Escape characters.

Preamble:

UART181

ACI/DTI_PEER		UART		UART_OUT

|		|		|

(1)	|	UART_PARAMETERS_REQ	|		|

*	==========================>	*		|

(2)	|		|	DTI2_GETDATA_REQ	|

|		|	(DISABLE)	|

|		*	==========================>	*

(3)	|		|	DTI2_GETDATA_REQ	|

|		|	(ENABLE)	|

|		*	==========================>	*

(4)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(5)	|	UART_PARAMETERS_CNF	|		|

*	<==========================	*		|

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI E)	|

|		*	<==========================	*

(7)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id5:0x13,0x14,0x15)	|		|

*	<==========================	*		|

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI D)	|

|		*	<==========================	*

(10)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id4:0x7c,0x7d,0x7e)	|		|

*	<==========================	*		|

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|	DTI2_DATA_TEST_REQ	|		|

|	(link_id4:0x7e,0x7f,0x80)	|		|

*	==========================>	*		|

(13)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DLCI D)	|

|		*	==========================>	*

(14)	|	DTI2_DATA_TEST_REQ	|		|

|	(link_id5:0x7c,0x7d,0x7e)	|		|

*	==========================>	*		|

(15)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(16)	|	DTI2_READY_IND	|		|

|	(link_id4)	|		|

*	<==========================	*		|

(17)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(DLCI E)	|

|		*	==========================>	*

(18)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(19)	|	DTI2_READY_IND	|		|

|	(link_id5)	|		|

*	<==========================	*		|

(20)	|	DTI2_GETDATA_REQ	|		|

|	(link_id5)	|		|

*	==========================>	*		|

(21)	|	DTI2_GETDATA_REQ	|		|

|	(link_id4)	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- UART_PARAMETERS_REQ
	device	UART_DEVICE
	comPar	COMPAR_XON_CTRL_XOFF_FCS

- DTI2_GETDATA_REQ
	link_id	LINK_DISABLE_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ENABLE_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- UART_PARAMETERS_CNF
	device	UART_DEVICE

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_131415_TE2UE_E_X

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_5
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_131415_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_FRAME_7c7d7e_TE2UE_D_X

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_4
	parameters	DTI_PARAMETER_PEER_A
	sdu	STRING_7c7d7e_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_4
	parameters	DTI_PARAMETER_PEER_B
	sdu	STRING_7e7f80_SDU

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_PEER_B
	sdu	SDU_FRAME_7e7f80_UE2TE_D_X

- DTI2_DATA_TEST_REQ
	link_id	LINK_ID_5
	parameters	DTI_PARAMETER_PEER_B
	sdu	STRING_7c7d7e_SDU

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_4

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_PEER_B
	sdu	SDU_FRAME_7c7d7e_UE2TE_E_X

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- DTI2_READY_IND
	link_id	LINK_ID_5

- DTI2_GETDATA_REQ
	link_id	LINK_ID_5

- DTI2_GETDATA_REQ
	link_id	LINK_ID_4

History:

24-Jan-2003	STW	Initial

# Escape Sequence Tests (UART300 - UART400)

## Setup Escape Sequence Tests

### UART300: Additional Setup for Escape Sequence Tests

Description:

This test case may be used for additional parameter setup which is only required in Escape Sequence tests. Currently by ACI the Escape Sequence character and the Guard Period are set within UART_PARAMETERS_REQ.

Variants:

<A>....<B>

Preamble:

<A>UART020B
<B>UART128B

ACI		UART		UART_OUT

|		|		|

|		|		|

Parametrization:

Primitive	Parameter	Value

History:

19-Jul-2001		IH			Initial

11-Oct-2001	TVO	adapted for use with DTI2

16-Oct-2001	TVO	moved parameters to UART_PARAMETERS_REQ

## Errors During Escape Sequence Detection

### UART301: Leading Guard Period Too Short

Description:

The UART receives the Escape Sequence character, but the Guard Period between the last data (h) and the first Escape Sequence character was shorter than requested with the element "guardPeriod" of the UART_PARAMETERS_REQ primitive. The state of the Escape Sequence Detection is reset.

Within the second timeout no DTI2_DATA_TEST_IND primitive (indicating the Escape Sequence Detection) is expected.

Preamble:

UART300A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (500)

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+++)	|

|		*	<==========================	*

(6)	|	DTI2_DATA_TEST_IND	|		|

|	(+++)	|		|

*	<==========================	*		|

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (2000)

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(10)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_ESC_SEQ_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_ESC_SEQ_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

19-Jul-2001	IH	Initial

11-Sep-2001	STW	remove escape sequence if detected

11-Oct-2001	TVO	adapted for use with DTI2

### UART302: Escape Sequence Character Mismatch

Description:

After the minimum Guard Period the first Escape Sequence characters are received. The third character does not fit into the Escape Sequence (not an Escape Sequence character). The state of the Escape Sequence Detection is reset.

The third timeout afterwards (trailing guard period) confirms that no DTI2_DATA_TEST_IND primitive (indicating the Escape Sequence Detection) is received.

Preamble:

UART300A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (1100)

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(++)	|

|		*	<==========================	*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (900)

(7)	|	DTI2_DATA_TEST_IND	|		|

|	(++)	|		|

*	<==========================	*		|

(6)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (2000)

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(8)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUSPLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUSPLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

23-Jul-2001		IH			Initial

11-Sep-2001	STW	remove escape sequence if detected

11-Oct-2001	TVO	adapted for use with DTI2

### UART303: Escape Sequence Characters Duration Too Long

Description:

The three Escape Sequence characters are received within a period of time which is longer than the Guard Period. The state of the Escape Sequence Detection is reset.

Within the final timeout (trailing guard period) a DTI2_DATA_TEST_IND primitive (indicating the Escape Sequence Detection) must not be received.

Preamble:

UART300A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (1100)

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (400)

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (400)

(9)	|	DTI2_DATA_TEST_IND	|		|

|	(++)	|		|

*	<==========================	*		|

(10)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (500)

(11)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(12)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(13)	|	DTI2_DATA_TEST_IND	|		|

|	(+)	|		|

*	<==========================	*		|

(14)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (2000)

(15)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(16)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(17)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(18)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUSPLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

23-Jul-2001		IH			Initial

11-Sep-2001	STW	remove escape sequence if detected

11-Oct-2001	TVO	adapted for use with DTI2

interchanged (12) and (13)

### UART304: Trailing Guard Period Too Short

Description:

First Guard Period and reception of the Escape Sequence characters are in time. The first character of the following AT command however, is received within the trailing guard period. The state of the Escape Sequence Detection is reset.

The DTI2_DATA_TEST_IND primitive (indicating the Escape Sequence Detection) must not be received within the final timeout.

Preamble:

UART300A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (1100)

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (100)

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (100)

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(10)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (100)

(11)	|		|	DTI2_DATA_TEST_IND	|

|		|	(A)	|

|		*	<==========================	*

(12)	|	DTI2_DATA_TEST_IND	|		|

|	(+++A)	|		|

*	<==========================	*		|

(13)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(14)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (2000)

(15)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(16)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(17)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(18)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_A_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_3PLUSA_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

23-Jul-2001	IH	Initial

11-Sep-2001	STW	remove escape sequence if detected

11-Oct-2001	TVO	adapted for use with DTI2

### UART305: Escape Sequence Character Mismatch without Flow Control primitive

Description:

After the minimum Guard Period the first Escape Sequence characters are received. The third character does not fit into the Escape Sequence (not an Escape Sequence character). The state of the Escape Sequence Detection is reset.

Because the upper layer does not send a Flow Control pirimive, UART has to collect one previous sent character, two escape sequence characters and one character sent afterwards.

Preamble:

UART300A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (1100)

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(++)	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (1100)

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(10)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(11)	|	DTI2_DATA_TEST_IND	|		|

|	(h++h)	|		|

*	<==========================	*		|

(12)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUSPLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_HPLUSPLUSH_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

12-Sep-2001	STW	Initial

11-Oct-2001	TVO	adapted for use with DTI2

## Successful Escape Sequence Detection

### UART310: Single Connection

Description:

An Escape Sequence is detected on a single Multiplexer connection. The occurence of the Escape Sequence is indicated to upper layers with an UART_DETECTED_IND primitive. The first character of the following AT command, is forwarded with the next DTI2_DATA_IND primitive.

Preamble:

UART300A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

MUTE (1100)

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(6)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (100)

(7)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(8)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (100)

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(10)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (900)

(11)	|	UART_DETECTED_IND	|		|

*	<==========================	*		|

(12)	|		|	DTI2_DATA_TEST_IND	|

|		|	(A)	|

|		*	<==========================	*

(13)	|	DTI2_DATA_TEST_IND	|		|

|	(A)	|		|

*	<==========================	*		|

(14)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(15)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- UART_DETECTED_IND
	device	UART_DEVICE
	dlci	DLCI_CONTROL
	cause	UART_DETECT_ESC

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_A_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_A_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

23-Jul-2001	IH	Initial

11-Sep-2001	STW	remove escape sequence if detected

11-Oct-2001	TVO	adapted for use with DTI2

### UART311: Establishment of A Third Multiplexer Connection

Description:

Two Multiplexer connections are active. In this testcase a third DLC is set up. The DTI connection is set and the Flow Control primitive DTI2_GETDATA_REQ is sent to enable data reception.

Preamble:

UART300B

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(SABM)	|

|		*	<==========================	*

(2)	|	UART_MUX_DLC_ESTABLISH_IND	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	UART_MUX_DLC_ESTABLISH_RES	|		|

*	==========================>	*		|

(5)	|		|	DTI2_DATA_TEST_REQ	|

|		|	(UA)	|

|		*	==========================>	*

(6)	|		|	DTI2_READY_IND	|

|		*	<==========================	*

(7)	|	UART_DTI_REQ	|		|

|	(UART_CONNECT_DTI)	|		|

*	==========================>	*		|

(8)	|	DTI2_CONNECT_IND	|		|

*	<==========================	*		|

(9)	|	DTI2_CONNECT_RES	|		|

*	==========================>	*		|

(10)	|	UART_DTI_CNF	|		|

|	(UART_CONNECT_DTI)	|		|

*	<==========================	*		|

(11)	|	DTI2_READY_IND	|		|

*	<==========================	*		|

(12)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_ESTABLISH_DLCI_C_TE2UE

- UART_MUX_DLC_ESTABLISH_IND
	device	UART_DEVICE
	dlci	DLCI_C
	convergence	UART_MUX_CONVERGENCE_UOS
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_DEFAULT

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- UART_MUX_DLC_ESTABLISH_RES
	device	UART_DEVICE
	dlci	DLCI_C
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_DATA_TEST_REQ
	link_id	LINK_WRITEDATA_PORT_1
	parameters	DTI_PARAMETER_UART_OUT
	sdu	SDU_UA_UE2TE_DLCI_C

- DTI2_READY_IND
	link_id	LINK_UART_OUT_PORT_1

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_C
	direction	FALSE
	link_id	LINK_ID_3
	entity_name	DTI_CHANNELNAME_PPP

- DTI2_CONNECT_IND
	link_id	LINK_ID_3
	version	DTI_VERSION_10

- DTI2_CONNECT_RES
	link_id	LINK_ID_3
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	UART_DEVICE
	dlci	DLCI_C

- DTI2_READY_IND
	link_id	LINK_ID_3

- DTI2_GETDATA_REQ
	link_id	LINK_ID_3

History:

27-Jul-2001	IH	Initial

11-Oct-2001	TVO	adapted for use with DTI2

### UART312: Escape Sequence on Three DLCs

Description:

Three DLC are active in multiplexer mode of the UART. After the receipt of the last byte of data on each of the DLCs the first Guard Period is detected. The Escape Sequence characters and the trailing Guard Period are detected on each of the three Multiplexer connections nearly simultaneous. The order is DLCI_B, DLCI_C and DLCI_A.
The occurence of the Escape Sequence is indicated to upper layers with an empty DTI2_DATA_IND primitive. On DLCI_A the DTI2_GETDATA_REQ primitive is not sent after receipt of the last byte of data (e). That's why the DTI2_DATA_IND primitive is sent delayed.
The following AT commands, are forwarded with the DTI2_DATA_IND primitives after the Flow Control primitives DTI2_GETDATA_REQ.

Preamble:

UART311

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI_B: h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id2: h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(4)	|	DTI2_GETDATA_REQ	|		|

|	(link_id2)	|		|

*	==========================>	*		|

(5)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI_C: l)	|

|		*	<==========================	*

(6)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id3: l)	|		|

*	<==========================	*		|

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(8)	|	DTI2_GETDATA_REQ	|		|

|	(link_id3)	|		|

*	==========================>	*		|

(9)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI_A: e)	|

|		*	<==========================	*

(10)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id1: e)	|		|

*	<==========================	*		|

(11)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(12)	|	DTI2_GETDATA_REQ	|		|

|	(link_id1)	|		|

*	==========================>	*		|

MUTE (1000)

(13)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI_B: +++)	|

|		*	<==========================	*

(14)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (100)

(15)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI_C: +++)	|

|		*	<==========================	*

(16)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (200)

(17)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI_A: +++)	|

|		*	<==========================	*

(18)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (300)

(19)	|	UART_DETECTED_IND	|		|

|	(link_id2)	|		|

*	<==========================	*		|

(20)	|	DTI2_GETDATA_REQ	|		|

|	(link_id2)	|		|

*	==========================>	*		|

(21)	|	UART_DETECTED_IND	|		|

|	(link_id3)	|		|

*	<==========================	*		|

(22)	|	DTI2_GETDATA_REQ	|		|

|	(link_id3)	|		|

*	==========================>	*		|

(23)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI_B: AT)	|

|		*	<==========================	*

(24)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id2: AT)	|		|

*	<==========================	*		|

(25)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(26)	|	DTI2_GETDATA_REQ	|		|

|	(link_id2)	|		|

*	==========================>	*		|

(27)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI_C: AT)	|

|		*	<==========================	*

(28)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id3: AT)	|		|

*	<==========================	*		|

(29)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(30)	|	DTI2_GETDATA_REQ	|		|

|	(link_id3)	|		|

*	==========================>	*		|

(31)	|	UART_DETECTED_IND	|		|

|	(link_id1)	|		|

*	<==========================	*		|

(32)	|	DTI2_GETDATA_REQ	|		|

|	(link_id1)	|		|

*	==========================>	*		|

(33)	|		|	DTI2_DATA_TEST_IND	|

|		|	(DLCI_A: AT)	|

|		*	<==========================	*

(34)	|	DTI2_DATA_TEST_IND	|		|

|	(link_id1: AT)	|		|

*	<==========================	*		|

(35)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(36)	|	DTI2_GETDATA_REQ	|		|

|	(link_id1)	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_H_TE2UE_B

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_2
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_2

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_L_TE2UE_C

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_3
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_L_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_3

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_E_TE2UE_A

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_E_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_ESC_SEQ_B

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_ESC_SEQ_C

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_ESC_SEQ_A

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- UART_DETECTED_IND
	device	UART_DEVICE
	dlci	DLCI_B
	cause	UART_DETECT_ESC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_2

- UART_DETECTED_IND
	device	UART_DEVICE
	dlci	DLCI_C
	cause	UART_DETECT_ESC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_3

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_AT_TE2UE_B

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_2
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_AT_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_2

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_AT_TE2UE_C

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_3
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_AT_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_3

- UART_DETECTED_IND
	device	UART_DEVICE
	dlci	DLCI_A
	cause	UART_DETECT_ESC

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	SDU_FRAME_AT_TE2UE_A

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_AT_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

27-Jul-2001	IH	Initial

11-Sep-2001	STW	remove escape sequence if detected

11-Oct-2001	TVO	adapted for use with DTI2

16-Jan-2003	STW	Error corrections

### UART313: Single Connection without FLow Control primitive

Description:

An Escape Sequence is detected on a single connection. In former versions of the uart SAP, the occurence of the Escape Sequence could not be indicated immediately to upper layers because there was no Flow Control primive from the upper layer. Escape Sequence and subsequent data were forwarded with one DTI2_DATA_IND primitive. Now, this is basically a simpler form of UART312.

Preamble:

UART300A

ACI		UART		UART_OUT

|		|		|

(1)	|		|	DTI2_DATA_TEST_IND	|

|		|	(h)	|

|		*	<==========================	*

(2)	|	DTI2_DATA_TEST_IND	|		|

|	(h)	|		|

*	<==========================	*		|

(3)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (1100)

(4)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(5)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (100)

(6)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(7)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (100)

(8)	|		|	DTI2_DATA_TEST_IND	|

|		|	(+)	|

|		*	<==========================	*

(9)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

MUTE (900)

(10)	|	UART_DETECTED_IND	|		|

|	(link_id1)	|		|

*	<==========================	*		|

(11)	|		|	DTI2_DATA_TEST_IND	|

|		|	(A)	|

|		*	<==========================	*

(12)	|		|	DTI2_GETDATA_REQ	|

|		*	==========================>	*

(13)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

(14)	|	DTI2_DATA_TEST_IND	|		|

|	(A)	|		|

*	<==========================	*		|

(15)	|	DTI2_GETDATA_REQ	|		|

*	==========================>	*		|

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_H_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_PLUS_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- UART_DETECTED_IND
	device	UART_DEVICE
	dlci	DLCI_CONTROL
	cause	UART_DETECT_ESC

- DTI2_DATA_TEST_IND
	link_id	LINK_UART_OUT_PORT_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_A_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_READDATA_PORT_1

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	LINK_ID_1
	parameters	DTI_PARAMETER_FRAME
	sdu	STRING_A_SDU

- DTI2_GETDATA_REQ
	link_id	LINK_ID_1

History:

12-Sep-2001	STW	Initial

11-Oct-2001	TVO	adapted for use with DTI2