Service Access Point

Include SAP example

| Department: | Aalborg Wireless Center |
| --- | --- |
| Creation Date: | 7 September, 2001 |
| Last Modified: | 16 September, 2003 by Kenneth Skou Pedersen |
| ID and Version: | 8434.404.01.008 |
| Status: | Accepted |

- Document Control

Copyright  2003 Texas Instruments, Inc.

All rights reserved.

Texas Instruments Incorporated and / or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and ser-vices at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and / or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and / or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and / or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and / or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and / or the licensing of software do not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written per-mission of TI.

  - Document History

ID	Author	Date	Status

8434.404.01.001	KKS	7 September, 2001	Being Processed

Initial version.

8434.404.01.002	KKS	11 September, 2001	Being Processed

Example SAP include now compiles

8434.404.01.003	CSH	17 September, 2001	Submitted

Update of document details (title, abbreviation, number, type and/or status).

8434.404.01.004	CSH	19 September, 2001	Submitted

Removed COMPATIBILITY_DEFINES from Pragma section

8434.404.01.005	CSH	28 September, 2001	Submitted

Added missing ref to union example (4.3)

8434.404.01.006	KSP	7 March, 2003	Submitted

Changed to new TI template

8434.404.01.007	KSP	4 August, 2003	Submitted

Document updated with new structures. The SAP document has been compiled to check validity of declarations..

8434.404.01.008	KSP	16 September, 2003	Accepted

Rework after review.

  - References, Abbreviations, Terms

Table of Contents

1	Introduction	5

2	Constants	6

3	Primitives	7

3.1	Exported Parameters	7

4	Parameters	8

4.1	Signed Integer, 8-bits	8

4.2	Unsigned Integer, 8-bits	8

4.3	Linked struct	8

4.4	Struct element2	9

4.5	Union element 2	9

4.6	Union element 3	10

# Introduction

This Include Service Access Point (SAP) is made in order to demonstrate the general use and possibilities of an include SAP. By using an include SAP it is possible to use the same structures or elements in several SAPs without having to declare them in every single SAP which use them. This is done by linking to the desired element or structure in the include SAP. In this include document the actual declarations of the linked types used is placed and therefore only needs to be defined once. This way it is also easy to change a structure used in several SAPs without having to change every single SAP. This document is made from the description of the SAP syntax in [TI 8350.301 SAP Syntax]. As for the SAP example document all the parameters in this document are fictive and is only intended for illustrating different possibilities of the SAP syntax. As a result of this the description of the parameters will not be as thorough and informative as it should be for real SAP document.

As for the normal SAPs it is very important to make thorough descriptions of the parameters as this document also serves as documentation. This also goes for the introduction, which should resemble the one for the normal SAPs and clearly describe the purpose and content of the SAP.

Chapter 2 contains a specification of constants used, which are not specific to the value of a particular parameter. Such constants are typically size fields, specifying array or element sizes.

Chapter 3 contains a list of exported parameters. As this is an include SAP no primitives should be defined in this document. This list also serves as a short name index of all the parameters linked from other SAP documents. All exports of parameters shall be done from this section.

Chapter 4 contains the specifications of the common types. It also contains specifications of parameters with predefined values or value ranges. Cross-references to subtypes may be used within this section for complex type declarations

For a more thorough description of the SAP introduction please refer to the Example SAP document [TI 8350.405].

# Constants

Description:

This section contains declarations of pragmas and constant values. The pragmas are used to modify the behaviour of the TI tool chain and the constants are used throughout the remaining part of the document. In this case the parameters are prefixed with EX_INC and the creation of enumerations in the .val file is enabled. Furthermore two constants are declared of which one is linked to from the example SAP.

Pragma:

| Name | Value | Comment |
| --- | --- | --- |
| PREFIX | EX_INC | Prefix for this document |
| ALLWAYS_ENUM_IN_VAL_FILE | YES | Enumeration values in value file |
| COMPATIBILITY_DEFINES | NO | Compatible to the old #defines |
| CAPITALIZE_TYPENAME | NO | Not to be used |
| ENABLE_GROUP | NO | Grouping will not be used |

Definition:

| Name | Value | Comment |
| --- | --- | --- |
| SIZE_MAX | 0x0010 | Example of a constant used to control the maximum number of elements of an structure (value 16) |
| LINK_CONSTANT | -2 | Example of a constant to be linked in the SAP document. |

History:

1-September-01 	KKS	Initial
18-September-01	CSH	Changed name EXINC_SIZE_MAX to SIZE_MAX because 
		prefixing of constants is working.
18-September-01	CSH	Added a new constant, which is linked from the EX_SAP.
18-September-01	CSH 	COMPATIBILITY_DEFINES.=NO

# Primitives

As this is an include document there are no primitives defined in this SAP. This section however needs to contain some text in order for the SAP to parse. This section contains all the exported parameters, which have been linked from other SAPs. Note that linked CONSTANTS are not listed here but only in the Constants section above. That is, all exported parameters except from CONSTANTS shall be exported from the following section.

Please notice the SAP ID, which is special for the Include SAPs. As opposed to traditional primitives, which are normally declared in this section the ID starts with 0 as opposed to 8. In order to support several include SAPs the last two digits are used as SAP identifier to number the documents as for normal SAPs. That is if another include SAP was to be added along with this the ID would be 0x00000001 and so forth.

## Exported Parameters

Description:

This section contains all the exported type from this include document. In addition to this the developer can use it as a Short Name index. (This message is to keep xGen happy, in the following 2 respects: The primitive section cannot be empty and all parameters should be used)

Definition:

| Short Name | ID | Direction |
| --- | --- | --- |
| EXAMPLE_INC_EXPORT | 0x00000000 | SLA->MAS |

Elements:

| Long Name | Short Name | Ref | Type |
| --- | --- | --- | --- |
| Complex union example | union_element_2 | 4.5 | UNION |
| Linked struct example | linked_struct | 4.3 | STRUCT |

History:

1-September-01	KKS	Initial

# Parameters

This section contains the actual declarations of parameter types, parameter values and parameter ranges linked to by other SAP documents.

## Signed Integer, 8-bits

Description:

This is the definition of an 8-bit signed integer parameter.

Definition:

| Type | Short Name | Comment |
| --- | --- | --- |
| S8 | signed_8_bit | Used by the linked struct |

History:

16-Marts_03	KSP	Initial

## Unsigned Integer, 8-bits

Description:

This is the definition of an 8-bit unsigned integer parameter. This also illustrates the possibility of using the same Definition with three different Short Names.

Definition:

| Type | Short Name | Comment |
| --- | --- | --- |
| U8 | unsigned_8_bit_1 | Used by union_element_3 |
| U8 | unsigned_8_bit_2 | Description of the type |
| U8 | unsigned_8_bit | Used by the linked struct |

History:

1-September-01 	KKS	Initial
16-Marts_03	KSP	Added unsigned_u8

## Linked struct

Description:

This is an example of the definition of a linked STRUCT. The STRUCT is made up of two arrays of simple integers. The first array called unsigned_8_bit is a fixed size array of size 5 and the second called signed_8_bit is a dynamic array of size 1 to 6. For the latter a counter element with prefix c_ is created to determine the actual array size. The type of this counter is dependent of the size of the array.

Definition:

| Type | Short Name | Comment |
| --- | --- | --- |
| STRUCT | linked_struct | Example of linked struct |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Simple integer | unsigned_8_bit | [5] | 4.2 | U8 |
| Simple integer | signed_8_bit | [1..6] | 4.1 | S8 |

History:

16-Marts-03 	KSP	Initial

## Struct element2

Description:

This is an example of the definition of a STRUCT. The struct is made up of a single UNION. In addition to this a union controller with prefix ctrl_ is created to determine which element in the union is used.

Definition:

| Type | Short Name | Comment |
| --- | --- | --- |
| STRUCT | struct_element_2 | Container |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Union example | union_element_3 |  | 4.6 | UNION |

History:

29-August-01 	KKS	Initial
28-September-01	CSH	Adding missing ref

## Union element 2

Description:

This is an example of the definition of a more complex UNION. The UNION contains a U8 element and a STRUCT containing a UNION. Please note that a union cannot contain another union directly due to the creation of the union controller. For more detail on this please refer to the SAP syntax document.

Definition:

| Type | Short Name | Comment |
| --- | --- | --- |
| UNION | union_element_2 | Container |

Elements:

| Tag ID | Long Name | Short Name | Ref | Type |
| --- | --- | --- | --- | --- |
| unsigned_char_example | 8-bit unsigned integer | unsigned_8_bit_1 | 4.2 | U8 |
| struct_2_example | Struct 2 example | struct_element_2 | 4.4 | STRUCT |

History:

1-September-01 	KKS	Initial

## Union element 3

Description:

This is an example of the definition of a simple UNION containing two elements of type U8..

Definition:

| Type | Short Name | Comment |
| --- | --- | --- |
| UNION | union_element_3 | Container |

Elements:

| Tag ID | Long Name | Short Name | Ref | Type |
| --- | --- | --- | --- | --- |
| unsigned_char_1 | 8-bit unsigned integer | unsigned_8_bit_1 | 4.2 | U8 |
| unsigned_char_2 | 8-bit unsigned integer | unsigned_8_bit_2 | 4.2 | U8 |

History:

1-September-01 	KKS	Initial