Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM Protocol Stack

G23

LT-Light

Driver Interface

(Edit via File/Properties/Summary!)

| Document Number: | 8415.005.99.005(Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.6(Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1998-Sep-10 |
| Last changed: | 2003-May-14 by XGUTTEFE |
| File Name: | 8415_005.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1998-Sep-10 | LM et al. |  | 0.1 |  | 1 |
| 1999-Mar-23 | LM et al. |  | 0.2 |  | 2 |
| 1999-Mar-24 | MS et al. |  | 0.3 |  | 3 |
| 1999-Jun-3 | LE et al. |  | 0.4 | Submitted | 4 |
| 2000-Feb-4 | UB et al. |  | 0.5 |  | 5 |
| 2003-May-13 | XINTEGRA |  | 0.6 | Draft |  |

Notes:

- Initial version

- API conform with GDI (8415.026.99.012)

- New document template/English check

- Consistency check

- New template

Table of Contents

1.1	References	4

2	Introduction	5

3	Interface description of the LT driver	5

3.1	Data types	5

3.2	Constants	6

3.3	Functions	7

3.3.1	light_Init – Driver initialization	8

3.3.2	light_Exit – De-initialization of the driver	9

3.3.3	light_SetStatus – Change the status of a light emitting device	10

3.3.4	light_GetStatus – Retrieve the status of the light emitting device	11

Appendices	12

A.	Acronyms	12

B.	Glossary	12

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[C_8415.0026]	8415.026.99.012; March 19, 1999
Generic Driver Interface – Functional Specification; Condat

# Introduction

- G23 is a software package implementing Layers 2 and 3 of the ETSI-defined GSM air interface signaling protocol, and as such represents the part of a GSM mobile station’s protocol software which is both, platform and manufacturer independent. Therefore, G23 can be viewed as a building block providing standardized functionality through generic interfaces for easy integration.

The G23 suite of products consists of the following items:

- Layers 2 and 3 for speech & short message services,

- Layers 2 and 3 for fax & data services,

- Application Control Interface,

- Slim MMI [02.30] and

- Test and integration support tools.

This document describes the functional interface of the G23 Light driver interface. This driver is used to control all light emitting devices of the mobile. Each device is addressed using a device ID. This driver specification defines numerous device IDs that are to be used when this kind of device is supported. The range of device IDs may be extended for individual driver implementations. Examples of light emitting devices in a mobile are the backlight of the LCD display and the keyboard or an LED indicating the mobile status “service available”. The exact meaning of the status value is device dependent. In the case of a display backlight device, the status may be on or off or may be interpreted as the brightness; in the case of a dual color LED the value may be interpreted as a color value (0 = OFF, 1 = green, 128 = orange, 256 = red). Because of the number and kinds of different light emitting devices of a specific mobile, only a few standard status values have been defined.

# Interface description of the LT driver

## Data types

Name	Description

-	-

## Constants

Name	Description

LIGHT_DEVICE_BACKLIGHT	Display/Keyboard backlight device
LIGHT_DEVICE_INSERIVCELED	Device indicating the in-service status
LIGHT_STATUS_OFF	Device is switched off
LIGHT_STATUS_ON	Device is switched on
LIGHT_STATUS_DIMMENED	Brightness of the device is low
LIGHT_STATUS_INTERMEDIATE	Brightness of the device is intermediate
LIGHT_STATUS_BRIGHT	Brightness is bright
LIGHT_STATUS_GREEN	Device emits a green light
LIGHT_STATUS_ORANGE	Device emits an orange light
LIGHT_STATUS_RED	Device emits a red light

## Functions

Name	Description

light_Init	Initialization of EMI
light_Exit	Termination of EMI
light_SetStatus	Change the status of a light emitting device 
light_GetStatus	Retrieve the status of a light emitting device

### light_Init – Driver initialization

Definition:

UBYTE light_Init
(
	void
) ;

Parameters:

Name	Description

-	-

Return values:

Name	Description

DRV_OK	Initialization successful
DRV_INITIALIZED 	Driver already initialized
DRV_INITFAILURE	Initialization failed

Description

The function initializes the internal data of the driver. The function returns DRV_INITIALIZED if the driver has already been initialized and is ready to be used or is already in use. In case of an initialization failure, i.e. the driver cannot be used, the function returns DRV_INITFAILURE.

### light_Exit – De-initialization of the driver

Definition:

void light_Exit
(
	void
) ;

Parameters:

Name	Description

-	-

Return values:

Name	Description

-	-

Description

The function is called when the driver functionality is no longer needed. The function de-allocates the resources.

### light_SetStatus – Change the status of a light emitting device

Definition:

UBYTE light_SetStatus
(
	UBYTE	in_DeviceID
	UBYTE	in_NewStatus
) ;

Parameters:

Name	Description

in_DeviceID	Value identifying a special light emitting device

in_NewStatus	New status of the specified device

Return values:

Name	Description

DRV_OK	Function successful

DRV_INVALID_PARARMS	One or more parameters are out of range or not valid

Description

This function is used to change the status of a specific light emitting device supported by this driver. The device is identified by the parameter in_DeviceID. Depending on the capabilities of the device, the parameter in_NewStatus has different meanings. The following table shows the relationship between predefined devices a possible status values.

| DeviceID | Status Values |
| --- | --- |
| LIGHT_DEVICE_BACKLIGHT | LIGHT_STATUS_OFF LIGHT_STATUS_DIMMED LIGHT_STATUS_INTERMEDIATE LIGHT_STATUS_BRIGHT |
| LIGHT_DEVICE_INSERVICELED | LIGHT_STATUS_OFF LIGHT_STATUS_GREEN LIGHT_STATUS_ORANGE LIGHT_STATUS_RED |

As mentioned in the Introduction chapter, this list is a predefined selection of devices and status values. The list may be extended in a per mobile specific driver specification/implementation.

In any case, if the function succeeds, the driver returns DRV_OK. If the driver does not accept the new status value, it returns DRV_INVALID_PARAMS.

### light_GetStatus – Retrieve the status of the light emitting device

Definition:

UBYTE light_GetStatus
(
	UBYTE	in_DeviceID
	UBYTE *	in_StatusPtr
) ;

Parameters:

Name	Description

in_DeviceID	Value identifying the light emitting device of which the status is to be retrieved

in_StatusPtr	Pointer to the buffer used to store the status

Return values:

Name	Description

DRV_OK	Function successful

DRV_INVALID_PARARMS	One or more parameters are out of range or not valid

Description

This function retrieves the status of a specific light emitting device supported by the driver. A list of device identifiers and status values can be found in Chapter3.2.

If the status of the specified device could be retrieved, the function returns DRV_OK.

If the specified device is unknown, the function returns DRV_INVALID_PARAMS.

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |