Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document – Confidential

GSM Protocol Stack

G23

SER – Serial Interface Driver

Driver Interface

(Edit via File/Properties/Summary!)

| Document Number: | 8415.062.99.004 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.5 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1999-Nov-22 |
| Last changed: | 2003-May-21 by XINTEGRA |
| File Name: | ser_api.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1999-Nov-22 | TSE |  | 0.1 |  | 1 |
| 1999-Dec-07 | TSE |  | 0.2 |  | 2 |
| 1999-Dec-13 | TSE |  | 0.3 |  | 3 |
| 1999-Dec-14 | TSE |  | 0.4 |  | 4 |
| 2003-May-20 | XINTEGRA |  | 0.5 | Draft |  |

Notes:

- Initial version

- Corrections

- Function parameters updated

- Function parameters updated/function SER_SetDevice added

Table of Contents

1.1	References	4

3.1	Data types	5

3.1.1	T_SER_DCB – Device Control Block	5

3.2	Constants	8

3.3	Functions	9

3.3.1	SER_Init – Driver Initialization	10

3.3.2	SER_Exit – Termination of the driver	11

3.3.3	SER_Read - Read data from the driver	12

3.3.4	SER_Write – Transmit data via the serial interface	13

3.3.5	SER_Look – Look at received data of the driver	14

3.3.6	SER_Clear – Clear internal driver buffers	15

3.3.7	SER_Flush – Flush internal driver buffers	16

3.3.8	SER_SetSignal – Setup a Signal	17

3.3.9	SER_ResetSignal – Remove a Signal	18

3.3.10	SER_SetConfig – Setup a device configuration	19

3.3.11	SER_GetConfig – Retrieve a driver configuration	20

3.3.12	SER_CallBack – Callback entry for the driver	21

3.3.13	SER_SetDevice - Set communication device (UART, USART, IRDA)	22

A.	Acronyms	23

B.	Glossary	23

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[C_8415.0026]	8415.026.99.012; March 19, 1999
Generic Driver Interface – Functional Specification; Condat

# Introduction

- G23 is a software package implementing Layers 2 and 3 of the ETSI-defined GSM air interface signaling protocol, and as such represents the part of a GSM mobile station’s protocol software which is both, platform and manufacturer independent. Therefore, G23 can be viewed as a building block providing standardized functionality through generic interfaces for easy integration.

The G23 suite of products consists of the following items:

- Layers 2 and 3 for speech & short message services,

- Layers 2 and 3 for fax & data services,

- Application Control Interface, AT Command Interface

- MMI  and MMI Framework (MFW)

- Test and integration support tools.

This document describes the functional interface of the G23 Standard Serial Receiver Transmitter (SER) driver, API. This driver can be used for common communication purposes and includes UART, USART and IRDA devices. This API of the driver is derived from the generic driver interface specification [C_8415.0026].

NOTE: The driver devices need to be configured after initialization. Only the following functions can be called while the driver is not configured: SER_Clear, SER_SetSignal , SER_SetDevice and SER_GetSignal. All other functions return DRV_NOTCONFIGURED. After the Initialization of the driver the communication device must be chosen via the SER_SetDevice() function, after that the device has to be configured with the SER_SetConfig() function.

# Interface description of the SER driver

## Data types

Name	Description

UBYTE                                                  	unsigned   8 bit integer data type

BYTE                                                     signed       8 bit integer data type
USHORT					unsigned 16 bit integer data type

SHORT					signed     16 bit integer data type

T_SER_DCB		Device Control Block

### T_SER_DCB – Device Control Block

Definition:

typedef struct T_SER_DCB
{
	UBYTE	Port
	USHORT	Baud
	UBYTE	DataBits
	UBYTE	StopBits
	UBYTE	Parity
	UBYTE	FlowControl
	USHORT	RxBufferSize
	USHORT	TxBufferSize
} ;

Description:

The device control block data type contains all parameters used to configure the serial device. The following table contains a list of the data elements and brief descriptions of them.

The parameter Port is used to configure which of the available I/O ports is used with this driver in case more than one port is available.

| Data element | Description |
| --- | --- |
| Baud | Transmission rate in bits/sec. See the following corresponding table. |
| DataBits | Size of a character (5 to 8 bits are possible, refer to the following corresponding table). |
| StopBits | Number of stop bits attached to each character. The following corresponding table shows possible values. |
| Parity | Type Parity checking. The following corresponding table shows the possible values. |
| FlowControl | Type of flow control. The following corresponding table shows the possible values. |
| RxBufferSize | Size of the receiver buffer in bytes. |
| TxBufferSize | Size of the transmitter buffer in bytes. |

Possible transmission rate

SER_BAUD_115200

SER_BAUD_57600

SER_BAUD_38400

SER_BAUD_33900

SER_BAUD_28800

SER_BAUD_19200

SER_BAUD_14400

SER_BAUD_9600

SER_BAUD_4800

SER_BAUD_2400

SER_BAUD_1200

SER_BAUD_600

SER_BAUD_300

SER_BAUD_150

SER_BAUD_75

Table 1

Data bits

SER_CHAR5

SER_CHAR6

SER_CHAR7

SER_CHAR8

Table 2

Stop bits

SER_STOP1

SER_STOP15

SER_STOP2

Table 3

Flow Controls

SER_FLOWNO

SER_FLOWHW

SER_FLOWSW

Table 4

Parity

SER_PARITYNO

SER_PARITYODD

SER_PARITYEVEN

Table 5

## Constants

Name	Description

SER_BAUD_115200	Transmission rate of 11520Kbit/s
SER_BAUD_57600 	Transmission rate of 57600Kbit/s
SER_BAUD_38400 	Transmission rate of 38400Kbit/s
SER_BAUD_33900 	Transmission rate of 33900Kbit/s
SER_BAUD_28800 	Transmission rate of 28800Kbit/s
SER_BAUD_19200 	Transmission rate of 19200Kbit/s
SER_BAUD_14400 	Transmission rate of 14400Kbit/s
SER_BAUD_9600 	Transmission rate of 9600Kbit/s
SER_BAUD_4800 	Transmission rate of 4800Kbit/s
SER_BAUD_2400 	Transmission rate of 2400Kbit/s
SER_BAUD_1200 	Transmission rate of 1200Kbit/s
SER_BAUD_600 	Transmission rate of 600Kbit/s
SER_BAUD_300 	Transmission rate of 300bit/s
SER_BAUD_150 	Transmission rate of 150Kbit/s
SER_BAUD_75 	Transmission rate of 75Kbit/s

SER_PARITYNO	no parity

SER_PARITYODD	odd parity

SER_PARITYEVEN	even parity
SER_CHAR5	5-Bit character
SER_CHAR6	6-Bit character
SER_CHAR7	7-Bit character
SER_CHAR8	8-Bit character
SER_STOP1	One stop bit indicates the end of a character
SER_STOP15	One and a half stop bits indicates the end of a character
SER_STOP2	Two stop bits indicate the end of a character
SER_FLOWNO	No flow control takes place (unsecured transmission)
SER_FLOWHW	Hardware handshake takes place
SER_FLOWSW	Software handshake takes place (used for 3-wire interfaces)

DRV_BUFFER_FULL			The internal buffer is exhausted
DRV_DISABLED				Driver is not enabled
DRV_ENABLED 				Driver is enabled
DRV_NOTCONFIGURED			Driver is not configured
DRV_INITFAILURE			Driver initialization failed
DRV_INITIALIZED				Driver is already initialized
DRV_INTERNAL_ERROR			Unspecified internal driver error
DRV_INPROCESS				The requested function is currently being executed
DRV_INVALID_PARAMS			One or more parameters are out of range or invalid
DRV_NOTCONFIGURED			Driver is not configured
DRV_OK					Return value indicating the function completed successfully

DRV_UNKNOWN				unknown device accessed

DRV_SIGFCT_NOTAVAILABLE		The requested event signaling functionality is not available

DEV_UART				UART Device ID

DEV_USART				USART Device ID

DEV_IRDA				IRDA Device ID

## Functions

Name	Description

SER_Init	Initialization of the serial communication driver
SER_Exit	De-initialization of serial communication driver
SER_Clear	Re-initialize all buffers
SER_Write	Send data via the serial interface 
SER_Read	Read data received via the serial interface 
SER_Look	Read data received via the serial interface, but leave data unchanged
SER_Flush	Flush all buffers
SER_SetSignal	Define a signal used to indicated special events
SER_ResetSignal	Un-define a signal the driver uses to indicate an event
SER_SetConfig	Set a device configuration 
SER_GetConfig	Retrieve a device configuration

SER_Callback			         Callback entry for   the driver

SER_SetDevice			         Set communication device (UART, USART, IRDA)

### SER_Init – Driver Initialization

Definition:

USHORT SER_Init (

USHORT      	DrvHandle

T_DRV_CB_FUNC 	in_SignalCBPtr

T_DRV_EXPORT **	DrvInfo

)

Parameters:

Name	Description

DrvHandle	unique handle for this driver

in_SignalCBPtr	This parameter points to the function that is called at the time an event that is to be signaled occurs. This value can be set to NULL if event signaling should not be possible.

DrvInfo	pointer to the driver parameters (see GDI specification document for a description of T_DRV_EXPORT).

Return values:

Name	Description

DRV_OK	Initialization successful
DRV_INITFAILURE	Initialization failed 
DRV_INITIALIZED	Driver already initialized

Description

The function initializes the module and all connected serial devices.

The driver exports its properties like its name, the functions to access driver functionality and a bitfield called flags by the parameter DrvInfo. If the driver is called by ISR, Bit (0) in the bitfield is set, otherwise this bit is cleared.

The driver stores the DrvHandle and passes it over the SignalID to the calling process every time the callback function is called.

The function returns DRV_INITIALIZED if the driver has already been initialized and is ready to be used or is already in use.

The driver uses a default device and a default configuration until the functions SER_SetDevice() and SER_SetConfig() have been called.

### SER_Exit – Termination of the driver

Definition:

void SER_Exit
(
	void
) ;

Parameters:

Name	Description

-

Return values:

Name	Description

-	-

Description

The function is called when the driver functionality is no longer required. The function “de-allocates” the resources (interrupts, buffers, etc.). The driver terminates regardless of any outstanding data to be sent.

### SER_Read - Read data from the driver

Definition:

USHORT SER_Read
(

void*	out_BufferPtr,
	USHORT*	thr_BufferSizePtr
) ;

Parameters:

Name	Description

out_BufferPtr	This parameter points to the buffer wherein the data is to be copied

thr_BufferSizePtr	On call: number of characters to read. If the function returns DRV_OK, it contains the number of characters read. If the function returns DRV_INPROCESS, it contains 0.

Return values:

Name	Description

DRV_OK	Function successful

DRV_INPROCESS	The driver is currently reading data. The data is incomplete.

DRV_NOTCONFIGURED	The driver is not yet configured

Description

This function is used to read data from the driver. The data is copied into the buffer to which out_BufferPtr points. The parameter *thr_BufferSizePtr contains the size of the buffer in characters.

In the case of a successful completion, the driver’s buffer is cleared. The driver keeps the data available when calling the function drv_Look().

If the driver is not configured, the function returns DRV_ NOTCONFIGURED.

NOTE: When calling the function with a buffer size of 0, the function will return DRV_OK. The size of the buffer needed to store the available data is stored in the parameter *thr_BufferSizePtr. In this case, the out_BufferPtr can be set to NULL.

### SER_Write – Transmit data via the serial interface

Definition:

USHORT SER_Write
(

void*	in_BufferPtr,
	USHORT*	thr_BufferSizePtr
) ;

Parameters:

Name	Description

in_BufferPtr	This parameter points to the buffer that is passed to the driver for further processing

thr_BufferSizePtr	On call: number of characters to write. If the function returns DRV_BUFFER_FULL, it contains the maximum number of characters that can be written. If the function returns DRV_OK, it contains the number of characters written. If the function returns DRV_INPROCESS, it contains 0.

Return values:

Name	Description

DRV_OK	Function successful

DRV_BUFFER_FULL	Not enough space

DRV_INPROCESS	Driver is busy writing data

DRV_NOTCONFIGURED	The driver is not yet configured

Description

This function is used to write data to the driver. The parameter *thr_BufferSizePtr contains the number of characters to write.

In the case of a successful completion, the function returns DRV_OK.

If the data cannot be written because the storage capacity of the driver has been exhausted, the function returns DRV_BUFFER_FULL and the maximum number of characters that can be written in *thr_BufferSizePtr.

If the driver is currently busy writing data and therefore cannot accept further data to be written, it returns DRV_INPROCESS and sets the parameter *thr_BufferSizePtr to 0.

If the driver is not configured, the function returns DRV_ NOTCONFIGURED.

NOTE:	When calling the function with a buffer size of 0, the function will return the number of characters that can be written in the parameter *thr_BufferSizePtr. In this case, the in_BufferPtr can be set to NULL.

### SER_Look – Look at received data of the driver

Definition:

USHORT SER_Look
(

void*	out_BufferPtr,
	USHORT*	thr_BufferSizePtr
) ;

Parameters:

Name	Description

out_BufferPtr	This parameter points to the buffer wherein the data is to be copied

thr_BufferSizePtr	On call: number of characters to read. If the function returns DRV_OK, it contains the number of characters read. If the function returns DRV_INPROCESS, it contains 0.

Return values:

Name	Description

DRV_OK	Function successful

DRV_INPROCESS	The driver is currently reading data. The data is incomplete.

DRV_NOTCONFIGURED	The driver is not yet configured

Description

This function is used to read data from the driver. The data is copied into the buffer to which out_BufferPtr points. The parameter *thr_BufferSizePtr contains the size of the buffer in bytes. The internal buffer is not cleared.

In the case of a successful completion, the function returns DRV_OK and sets the value of *thr_BufferSizePtr to the number of bytes read.

In the case of successful completion, consecutive calls to SER_Look() will lead to identical results. Call SER_Read() to implicitly clear the driver’s internal buffer.

If the driver is not configured, the function returns DRV_ NOTCONFIGURED.

NOTE:	When calling the function with a buffer size of 0, the function will return DRV_OK. The size of the buffer needed to store the available data is stored in the parameter *thr_BufferSizePtr. In this case, the out_BufferPtr can be set to NULL.

### SER_Clear – Clear internal driver buffers

Definition:

USHORT SER_Clear
(

USHORT	in_BufferType
) ;

Parameters:

Name	Description

in_BufferType	Bit-mask used to specify if the read, write or read and write buffer is cleared

Return values:

Name	Description

DRV_OK	Function successful

DRV_INPROCESS	The driver could not complete the clearance of the buffers at once. The driver is busy clearing the buffers.

Description

This function is used to clear the device’s internal buffers. The parameter in_BufferType is used to specify which buffer is to be cleared. The value of in_BufferType can be one of the values or a combination of the values defined in [C_8415.0026]. DRV_BUFTYPE_READ and DRV_BUFTYPE_WRITE can be combined to clear the read and write buffers at once.

### SER_Flush – Flush internal driver buffers

Definition:

USHORT SER_Flush ( void ) ;

Parameters:

Name	Description

-

Return values:

Name	Description

DRV_OK	Function successful

DRV_INPROCESS	The driver could not complete flushing the buffers at once. The driver is busy flushing the buffers.

DRV_NOTCONFIGURED	The driver is not yet configured

Description

This function is used to flush the device’s internal buffers. This means that data that is currently stored in the device’s internal write buffer is written to the USART at once.

If the driver is not configured, the function returns DRV_ NOTCONFIGURED.

### SER_SetSignal – Setup a Signal

Definition:

USHORT SER_SetSignal
(

USHORT	SignalType) ;

Parameters:

Name	Description

SignalType				Signal type to be set

Return values:

Name	Description

DRV_OK	Function completed successfully
DRV_INVALID_PARAMS	One or more parameters are out of range or invalid
DRV_SIGFCT_NOTAVAILABLE	Event signaling functionality is not available

Description

This function is used to define a single or multiple signals that is/are indicated to the process when the event identified in the signal information data type as SignalType occurs. The driver uses only the standard signals defined in [C_8415.0026].

To remove a signal, call the function SER_ResetSignal().

If one of the parameters of the signal information data is invalid, the function returns DRV_INVALID_PARAMS.

If no signal call-back function has been defined at the time of initialization, the driver returns DRV_SIGFCT_NOTAVAILABLE.

### SER_ResetSignal – Remove a Signal

Definition:

USHORT SER_ResetSignal
(

USHORT	SignalType
) ;

Parameters:

Name	Description

SignalType				Signal type to be set

Return values:

Name	Description

DRV_OK	Function completed successfully
DRV_INVALID_PARAMS	One or more parameters are out of range or invalid 
DRV_SIGFCT_NOTAVAILABLE	Event signaling functionality is not available

Description

This function is used to remove previously set single or multiple signals. The signals that are removed are identified by the SignalType. If the SignalType provided cannot be located, the function returns DRV_INVALID_PARAMS.

If no signal call-back function has been defined at the time of initialization, the driver returns DRV_SIGFCT_NOTAVAILABLE.

### SER_SetConfig – Setup a device configuration

Definition:

USHORT SER_SetConfig
(

T_SER_DCB*	in_DCBPtr
) ;

Parameters:

Name	Description

in_DCBPtr	Pointer to the driver control block

Return values:

Name	Description

DRV_OK	Function successfully completed
DRV_INVALID_PARAMS	One or more values are out of range or invalid in that combination

Description

This function is used to configure the serial device (port, transmission rate, flow control, etc). The driver can be configured at any one time. The parameters that can be configured are included in the  device control block T_SER_DCB. For detailed information about the contents of the device control block, refer to Chapter 3.1.1.

If any value of the configuration is out of range or invalid in combination with any other value of the configuration, the function returns DRV_INVALID_PARAMS.

Call the SER_GetConfig() function to retrieve the driver’s configuration.

The driver needs to be configured after initialization. Only the following functions can be called while the driver is not configured: SER_Clear, SER_SetSignal and SER_GetSignal. All other functions return DRV_NOTCONFIGURED.

### SER_GetConfig – Retrieve a driver configuration

Definition:

USHORT SER_GetConfig
(

T_SER_DCB*	out_DCBPtr
) ;

Parameters:

Name	Description

out_DCBPtr	Pointer to the driver control block

Return values:

Name	Description

DRV_OK	Function successfully completed
DRV_NOTCONFIGURED	The driver is not yet configured

Description

This function is used to retrieve the configuration of the serial device. The configuration is returned in the driver control block to which the pointer provided out_DCBPtr points. For detailed information about the contents of the device control block, refer to Chapter 3.1.1. The configuration can be requested at any one time.

If the driver is not configured, the function returns DRV_ NOTCONFIGURED.

### SER_CallBack – Callback entry for the driver

Definition:

void SER_CallBack
(

T_DRV_SIGNAL	* Signal
) ;

Parameters:

Name	Description

Signal	Pointer to the signal information data

Return values:

Name	Description

-
Description

Callback entry for the hardware device driver.

### SER_SetDevice - Set communication device (UART, USART, IRDA)

Definition:

USHORT  SER_SetDevice
(

USHORT ComDevice

) ;

Parameters:

Name	Description

ComDevice		Communication Device (see table below)

Return values:

Name	Description

DRV_OK					Function successfully completed

DRV_INVALID_PARAMS			Device not found

Description

Device Codes :

| ComDevice Value | Description |
| --- | --- |
| DEV_UART | Use UART device driver for communication |
| DEV_USART USART Device | Use USART device driver for communication |
| DEV_IRDA IRDA Device ID | Use IRDA device driver for communication |

This Function sets the communication device used for all functions. The driver uses the corresponding hardware driver, until the function is called again.

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |