WMDRM Test Tool Kit

Contents

1. Introduction	3

2. How to Install WMDRM Test Toolkit	3

3. How to Build Test Code	4

3.1 Set Environment Variables	4

3.2 Build WMDRM Porting Kit	4

3.3 Build Test Code	5

4. How to Run Test Cases	5

4.1 Launch individual test program	5

4.2 Launch a batch of test program	6

5. Breakdown of Test Code	8

5.1 Mapping Test Areas to Target DRM Components	8

5.2 Test Framework	9

5.3 Test Utilities	15

6.Test Areas	16

6.1 Top Level WMDRM Components	17

6.2  DRM Stores	19

6.3 Encryption and Decryption	22

6.4 Utilities	23

6.5 Evaluation	25

6.6 Advanced Features	29

7. Appendix	31

7.1 Files in WMDRM Test Tool Kit	31

7.2 Sample Log Result	36

7.3. Sample BVT Test Report	37

# 1. Introduction

Microsoft WMDRM Test Tool Kit ships along with Microsoft WMDRM Portable Device Kit. The test code is broken down into a number of test areas, according to WMDRM functionalities. Upon installation of WMDRM PD setup, the test code and files are found under the Test folder. The test program can be compiled in the format of executable and run against target WMDRM component.

For a quick start, please refer to readme.txt document under Test\doc folder. The WMDRM Test Tool Kit document is for the audiences who want to have more details of WMDRM Test Tool Kit. The document describes WMDRM Basic Verification Test (BVT) and Functional Automation Testing:

- The procedures of installation and building of WMDRM setup kit;

- The steps for running selected test area against a target WMDRM functionality;

- The break down of the test areas and their corresponding WMDRM target functionality;

- Descriptions of the structure of test framework;

- The details of each WMDRM test area;

This document does not cover the end-to-end test areas. Also, a number of useful test tools around the DRM features, such as drm license viewer, response generator, metering challenge generator, package, and others, are included in WMDRM PD (under the Tools folder) and the details of these tools are described in the WMDRM Portable Device Kit document (see Documents\WMRM_PD.chm).

# 2. How to Install WMDRM Test Toolkit

WMDRM Test Toolkit is a part of WMDRM Portable Device Kit setup. Launch the setup package, follow the instruction on the screens, it will install WMDRM PD Kit at the folder, e.g. c:\WMSDK\WMDDRMPK. The test code and files are under Test sub-folder.

A full list of all the test codes, documents, and test files under test folder are given in 7.1 Files in WMDRM Test Tool Kit.

# 3. How to Build Test Code

## 3.1 Set Environment Variables

To build the source, you must first set some environment variables:

set INCLUDE=<your development environment include path>

set LIB=<your development environment lib path>

For example:

set INCLUDE="c:\Program Files\Microsoft Visual Studio .NET 2003\Vc7\include"

set LIB="c:\Program Files\Microsoft Visual Studio .NET 2003\Vc7\lib"

## 3.2 Build WMDRM Porting Kit

Build WMDRM Portable Device Kit Source Code from Source folder. Refer ReleaseNotes.htm file under C:\WMSDK\WMDDRMPK for more details.

Before compile and build the test code, WMDRM code should be compiled and built first.

## 3.3 Build Test Code

From the Test folder, issue the following commands:

make -f pktestmk.msc init

make -f pktestmk.msc free

make -f pktestmk.msc chk

The "init" operation creates all target folders; the "free" and "chk" operations build release and debug builds respectively.

To clean both build types, issue the following command:

make -f pktestmk.msc clean

The binary files are found at Test\bin\i386 and Test\bind\i386 folders for free build and checked build, respectively.

# 4. How to Run Test Cases

## 4.1 Launch individual test program

Each test area should have its own executable, test script file and dependent filedep.txt file. If a test area requires additional dependent test data files, they are defined in the filedep.txt file. As soon as the executable program and test script file(s), along with all the dependent files if any, are copied to a target folder, the test program is ready to launch.

The structure of test framework will be discussed in details in 5.2 Test Framework. Because of the test framework, each test executable supports several optional command line arguments. It takes the following command arguments:

-bvt 			only run BVT test cases

-func			only run functional test cases

-nolog			no log message

numeric number	only run this test case

Notice that the command argument mentioned here is different than the test data input defined in the test script file. If there is not any command argument, the test executable simply runs all the test cases listed in the test script file.

| cscript RefBlackBoxTest_ansi –bvt | Only run BVT test cases of blackbox test area |
| --- | --- |
| cscript RefBlackBoxTest_ansi –func | Only run functional test cases of blackbox test area |
| cscript RefBlackBoxTest_ansi –bvt –nolog | Only run BVT test cases of blackbox test area, without logging debug messages. |
| cscript RefBlackBoxTest_ansi 9670 | Just run 9670 test case |
| cscript RefBlackBoxTest_ansi | Run all the test cases listed on refbb.tc.txt test script file |

- Use blackbox test area as examples

## 4.2 Launch a batch of test program

To launch multiple test areas, it is suggested to have a script to automate the copying files and launching test programs. Under Test\harness folder, there are two sample VB scripts files: runWMDRMPDPKTest.vbs and SendWMDRMPDPKTestResult.vbs

The runWMDRMPDPKTest.vbs script automates the following items:

- Create c:\ wmdrmpd folder which is required for storing WMDRM store file.

- Examine all test areas listed in pktestareas.txt file;

- For each test area,

  - copy the test area executable;

  - examine the filedep.txt, and copy all the required dependent test files if any.

- Launch the test program;

- Copy the test xml log file to a central folder ready for further processing.

A sample xml log result file is given at 7.2 Sample Log Result. The runWMDRMPDPKTest.vbs script takes the following optional arguments:

- -bvt		Only launch BVT test cases for all the test areas;

- -func 		Only launch functional test cases for all the test areas;

- Testarea	Only launch the given test areas

If no argument is provided, the script will simply launch all the test cases for all the test areas defined in test\harness\pktestareas.txt file.

The script SendWMDRMPDPKTestResult.vbs is used to

- summarized all the test xml log files at a central folder;

- create a test report and;

- send the test report to a destination.

To send the test report by email is an optional feature. To use this feature, the dummy email address in the script should be replaced with a valid email address. Also, your machine needs to run an SMTP virtual server. These are the set up stpes:

:

1. Right click “My Computer” then “Manage”.

2. Go to “Services and Applications/Internet Information Services/Default SMTP Virtual Server”.

3. Right click on it then “Properties”.

4. Go to “Delivery” tab, click “Advanced”

5. In “Smart host”, enter "smtphost".

6. Also make sure the SMTP service is running. At DOS prompt, type

“net start smtpsvc”.

If you don't want to send email, add "-noemail" to the script flag.

After running BVT and functional test cases, all the names of the xml log files which had been copied by the runWMDRMPDPKTest.vbs script are appended with “-bvt” and “-func”, respectivbely. For running a full test, the log xml file name will be the same as the test area name without any appending, e.g. for a blackbox teat area, the xml log file has a name of blackbox.xml, balckbox-bvt.xml, blackbox-func.xml, for full test, BVT test and functional test, respectively.

For example, to summarize BVT test log files, run “cscript SendWMDRMPDPKTestResult.vbs –bvt”. It summarizes all the BVT log files and generates BVT test report. A BVT test report sample is given at 7.3. Sample BVT Test Report

# 5. Breakdown of Test Code

## 5.1 Mapping Test Areas to Target DRM Components

WMDRM primarily consists of Blackbox, crypto, DRM Core, Devcert and DRM Manager components. There are several other components, such as Metering, Sync List, Secure Clock, which can be optionally turned on or off through compile flags (see source\makefile.msc). DRM core component itself wraps a number of functionalities, such as stores, parsers, builders, and some low level utilities. The composition of these functionalities are somewhat reflected in the *.h header files under source/inc folder and *.c files under source\core folder.

The WMDRM test code is also composed of different DRM functional test areas, following the same componentization rules as described above. Table 5.1 shows the breakdown of the WMDRM test code. The detail description of each test area is given in 6. Test Areas section.

Table 5.1. Mapping between test areas and the target DRM components.

| Test Area | Test Description | Target Component |
| --- | --- | --- |
| Blackbox | Blackbox API | Drmblackbox.lib |
| Crypto | Crypto APIs | drmbignum.lib drmcrypto.lib drmpkcrypto.lib |
| Devcert parser | Device certificate parser APIs | drmdevcert.lib |
| Device devcert | Devcert generation | Drmdevicedevcert.lib |
| Drmmanager | Top level exposed APIs | drmmanager.lib |
| ExprEval | Expression evaluator APIs | drmcore.lib |
| Hds | Low level hashed data store APIs | drmcore.lib |
| Header | Header parser APIs to create challenge | drmcore.lib |
| License acquisition | License challenge and response APIs | drmcore.lib |
| License chain | License chain evaluation and binding | drmcore.lib |
| License parser | License parser APIs | drmcore.lib |
| License store | License store APIs | drmcore.lib |
| License store cleanup | Store cleanup API | drmmanager.lib |
| licEval | License evaluator APIs | drmcore.lib |
| Metering | Metering collection, meter challenge and process reset | Drmmetering.lib |
| OEM | OEM implemented APIs | oem_ansi.lib |
| Secure store | Secure store APIs | drmcore.lib |
| Synclist | Sync challenge | drmsynclist.lib |
| Utils | CRT and other utilities | drmcore.lib |
| View Rights | Top level view rights APIs | drmcore.lib |
| XMLBuilder | XML Builder APIs | drmcore.lib |
| XMLParser | XML parser APIs | drmcore.lib |

## 5.2 Test Framework

Before going into detail of each test component, the overall structure of the test framework is described in this section.

    - Structure of Test Framework

The test framework is defined in RefTCLib.lib. The framework source code can be found under the folder test/reftclib.  Some macros definitions used in the test frame can be found at test\inc\tclib.h. One advantage of this framework is its flexibility in adding new test function and scripting test data, while keeping ANSI compliance.

Normally we have one test script file (e.g. myscript.pc.pk.ce.txt) and the test functions are wrapped by using the following macros:

BEGIN_APIMAP(executable_name, myscript_name)

API_ENTRY(MyTest1)

API_ENTRY(MyTest2)

END_APIMAP

If we have multiple test script files, we can build a script file list and then use BEGIN_APIMAP_WITH_SCRIPT_LIST to wrapper the test functions:

BEGIN_TEST_SCRIPT_LIST

TEST_SCRIPT_ENTRY("MyScript_name1")

TEST_SCRIPT_ENTRY("MyScript_name2")

END_TEST_SCRIPT_LIST

BEGIN_APIMAP_WITH_SCRIPT_LIST(executable_name)

API_ENTRY(MyTest1)

API_ENTRY(MyTest2)

END_APIMAP

The following is an example from Blackbox test component, where refbb.pc.pk.ce.txt is the test script file, and test functions are wrapper between BEGIN_APIMAP and END_APIMAP. To add a new test function, simply insert an entry of API_ENTRY.

BEGIN_APIMAP(RefBlackBoxTest_ansi, "refbb")

API_ENTRY(Test_API_GetClientID)

API_ENTRY(Test_VerifyClientID)

API_ENTRY(Test_CreateBindingInfo)

API_ENTRY(Test_API_CanBind)

API_ENTRY(Test_GenRandomBlob)

API_ENTRY(Test_API_HashValue)

API_ENTRY(Test_PKEncryptLarge)

API_ENTRY(Test_API_DecryptLicense)

API_ENTRY(Test_API_CipherKeySetup)

API_ENTRY(Test_BlobOps)

API_ENTRY(Test_API_Encrypt)

API_ENTRY(Test_RenewBlackboxContext)

API_ENTRY(Test_API_Decrypt)

END_APIMAP

    - Result Logging

The test framework supports two types of logging mechanism: COM XML Object, and ANSI file log. The released WMDRM Test Toolkit supports ANSI file log mechanism. To enable ANSI file log mechanism, the compile flag USE_DRMHARNESSLOGGER in test\pktestmk.msc should be set to 0.

Each test area has an xml log file. The name of the log file is the same as the exe module name of this teat area. The result of each test case will be logged in the XML file. If the test case fails, the error code is logged. A sample log results are provided in 7.2 Sample Log Result.

For running a batch of different test areas, it is suggested to use a script to parser all the xml log files and generate a summarized report (see 4. How to Run Test Cases)

    - Test Function

Note that each function takes the same argument format (long argc, char **argv). The data inputs come from the second arguments char** argv, while the first argument defines the number of data inputs. The following is the sample of Test_API_GetVersion test function from Blackbox test component:

DRM_RESULT Test_API_GetVersion(long argc, char **argv)

{

DRM_RESULT dr;

DRM_BB_CONTEXT *pBBContext;

DRM_BYTE bbVersion[VERSION_LEN], *pVersion;

char szBBVersion[40];

ChkArg(argc == 2);

pVersion = argv[0]? bbVersion: NULL;

pBBContext = argv[1]? &g_BBContext: NULL;

ChkDR(DRM_BBX_GetVersion(pVersion, pBBContext));

sprintf(szBBVersion, "%d.%d.%d.%d", bbVersion[0], bbVersion[1], bbVersion[2], bbVersion[3]);

if (strcmp(argv[0], szBBVersion))

dr = DRM_S_FALSE;

ErrorExit:

return dr;

}

Typically, the test function has three parts, namely (1) check and verify the input data, (2) convert the input data and then call WMDRM function, and (3) verify the results.

    - Test Script File

The data inputs are provided by the test script (e.g. refbb.pc.pk.ce.txt). The test framework reads the test script and process the input data. The following is an example of test case defined in the test script. The initial number 9670 is the test case number. It is followed by a short description of the nature of the test case. The test case number must be followed by a delimitor of “;”, which is used to signify to the test framework that this is a new test case.

9670:GetClientID should return the proper client info

Test_API_GetClientID(GLOBAL, NORMAL)

Test_VerifyClientID

9671:GetClientID should reject NULL parameters

Test_API_GetClientID(NULL, NORMAL)=-2147024809(0X80070057 E_INVALIDARG)

Following the line of test case number, the test functions can be scripted in an order that fits the test logic. The test framework itself does not impose a restriction on how many test input data for each test function. Typically, each test function is independent. If there is any inter-action between test functions, it is through global variables or files.

For a negative test case, the expected return code is given to the test case in the test script file. The test framework compares the actual return code with the expected return code. If there is a mismatch, the test case is logged as failed.

The script file can have different extensions and their combinations:

- pc extension: for PC Windows Media Player;

- pk extension: for WMDRM Portable Device

- ce extension for WINCE platform

In the test script file, there is a line

“-------------------BVT ends here----------------------“

that is used to divide BVT test cases from functional test cases.

    - PreTestCase and PostTestCase Functions

For each test case, it always initially calls PreTestCase function before calling any test function defined in the test script file. PreTestCase is a good place to initialize global variables, clean DRM stores. No matter the test case passes or fails, the function of PostTestCase is always called before exiting a test case. PostTestCase is a good place for freeing allocated memory and other cleaning up. It is important to clean up the DRM store generated from previous test case. In this way, each test case is well-defined and reproducible.

The following is an example of PreTestCase and PostTestCase functions from Licstore test area.

DRM_RESULT PreTestCase(long lTCID, char *strTCName)

{

DRM_RESULT dr;

DRM_CONST_STRING wszHDSPath;

ChkDR(SetDeviceEnv(L"devcerttemplate.dat", L"priv.dat", TRUE));

RemoveDRMFile(RMFILE_STORE);

tGetDeviceStorePathname(&wszHDSPath);

ChkDR(OpenHDS(&g_pHdsContext, wszHDSPath.pwszString, TRUE));

ErrorExit:

return dr;

}

DRM_RESULT PostTestCase(long lTCID, char *strTCName)

{

DRM_RESULT dr;

DRM_LST_Close(&g_oContext);

ChkDR(CloseHDS(g_pHdsContext));

g_pHdsContext = NULL;

ErrorExit:

return dr;

}

    - Filedep.txt file

Under each test area folder, there is filedep.txt file. This file is used to define which dependent files should be presence when running a given test program.

The following is an example of filedep.txt file from XMLParser test area.

DailytestXMLParser.pc.pk.ce.txt

XML\XMLLicense1.txt

XML\XMLLicense2.txt

XML\XMLLicense3.txt

XML\XMLLicense4.txt

XML\XMLLicense5.txt

XML\XMLLicense6.txt

XML\XMLLicense7.txt

XML\XMLLicense8.txt

XML\XMLLicense9.txt

XML\XMLLicense10.txt

XML\XMLLicense11.txt

XML\XMLLicense12.txt

XML\XMLLicense13.txt

XML\XMLLicense14.txt

XML\XMLLicense15.txt

XML\XMLLicense16.txt

XML\XMLLicense17.txt

XML\XMLLicense18.txt

XML\XMLLicense19.txt

In order to run testxmlparser_ansi.exe, dailytestXMLParser.pc.pk.ce.txt (the test script file) should be present at the same folder of the executable. Also, all the XMLLicense1-19.txt files (test data files) should be present at XML subfolder of the folder.that the executable resides.

## 5.3 Test Utilities

Several functions commonly used among different test areas are placed under test\tstutils folder (tstutils.lib, tGetClientID.lib and tstlicgen.lib ). Some of test functions are platform related and they are placed under test\toemimp folder (tOEMImp.lib).

Among them the test utility for license generator is of specially worth mentioning. Several functions from high level WMDRM components, such as drmmanager, synclist, viewrights, require to add license to the store or process license which is already existed in the store. By using the tstlicgen.lib utility which is able to generate user-defined license locally, we has the option not to bind the test code with WMRM SDK (license server).

Inside the test areas of drmmanager, viewrights and synclist, there are some test code on how to generate user-defined license, wrapper license to become license response, and call WMDRM process response function to add license to the DRM store. The variables include KID, LID, license rights, seed key, and sign key, leaf/root license.

Like WMRM SDK, it needs a license cert to issue a valid license. The testcerts.txt file at test\common folder contains the required certifications and keys. If the certifications in the testcerts.txt expire, it should renew the license certifications from Microsoft license enrollment server at http://licenseserver.windowsmedia.com. Note that the new certifications will be issued to the registry key at HKLM\Software\Microsoft\WM Rights Manager. Based on the new certifications, it can easily compose a new testcerts.txt file.

Notice that (1) the license generator can created some invalid or special licenses which are otherwise not available from WMRM SDK (license server). These types of licenses are useful for negative test cases; (2) Some device board does not support COM object used in WMRM SDK; and (3) the license locally generated by the license generator can be used for the test case which simulated the process of Direct License Acquisition (DLA), without involving to the hassle of device network issue.

# 6.Test Areas

According to the functionality of WMDRM, the test areas are divided into the following subgroups:

- Top level WMDRM components

  - Drm manager

  - View rights

  - Black box

- DRM Stores

  - Hashed data store

  - License store

  - Secure store

- Encryption/Decryption

  - Crypto

- Utilities

  - XML parser

  - XML builder

  - Utils

- Evaluation

  - Devcert parser

  - License parser

  - Expression Evaluator

  - License evaluation

  - Header

- Advanced features

  - Device Devcert

  - Secure clock

  - License chaining

  - Metering

  - Sync list

## 6.1 Top Level WMDRM Components

This test group consists of drmmanager, viewrights and blackbox. They are the components, the APIs of which may be accessed by an application.

6.1.1 Drm manager

For a typical application, it should only access the APIs exposed by drmmanager component. WMDRM functionality is exposed through the APIs of this component.

| Folder | test\drmmanager |
| --- | --- |
| Test code | testdrmmanager.c dlrb.c |
| Target Component | Drmmanager.lib |
| Feature testing | - Generate license challenge; - Process license response - Bind a license - Set enhanced data - Anti clock rollback - Multiple stores - License chaining - Memory alignment - Encryption and decryption - Output protection level verification |
| Test Script File | DailyTestDrmManager.pc.pk.ce.txt DailyTestDrmManager.pc.pk.txt DailyTestDrmManager.pk.pm.txt |
| Test Data Files | ..\common\testcerts.txt ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat .\files\header1.xml .\files\header2.xml .\files\header_longkid.xml .\files\header_longkid64.xml .\files\header_shortkid.xml .\files\header_shortkid64.xml |

6.1.2 View rights

| Folder | test\Viewrights |
| --- | --- |
| Test code | Viewrights.c |
| Target Component | Drmcore.lib |
| Feature testing | - Get license state data - Check the obtained license state data - Check ASD_IsAllowed |
| Test Script File | ViewRights.pc.pk.ce.txt |
| Test Data Files | ..\common\testcerts.txt |

6.1.3  Blackbox

| Folder | test\Viewrights |
| --- | --- |
| Test code | stdafx.h, binding.c and blackboxtest.c |
| Target Component | Drmblackbox.lib |
| Feature testing | - Get Client ID - Get Version - Get Cert - Check Can bind - Decryption - Encryption - Get bind info |
| Test Script File | Refbb.pc.pk.ce.txt |
| Test Data Files | ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat |

## 6.2  DRM Stores

This test group consists of hashed data store (HDS), license store and secure store. HDS is low level file-based store. It has multiple namespace, included the name spaces for license store, secure store and others. Both license store and secure store are implemented on top of HDS and they only use its corresponding namespace.

6.2.1 Hashed data store

| Folder | test\HDS |
| --- | --- |
| Test code | Testhds.c |
| Target Component | Drmcore.lib |
| Feature testing | - Create and open store; - Create and open namespace - Create, open and delete data slot - Enumerate data slot - Resize data slot - Seek data slot position - Read and write content to the data slot |
| Test Script File | DailytestHDS.pc.pk.ce.txt InvalidArgumenttestHDS.pc.pk.ce.txt NegativeTestHDS.pc.pk.ce.txt |
| Test Data Files | N/A |

6.2.2 License store

| Folder | test\licstore |
| --- | --- |
| Test code | Testlicstore.c |
| Target Component | Drmcore.lib |
| Feature testing | - Open and close license store - Get license - Add license - Enumerate license |
| Test Script File* | testlicstore.pc.pk.ce.txt testlicstore.pc.pk.txt |
| Test Data Files | ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat licenses\regularlicense1.txt licenses\regularlicense2.txt licenses\regularlicense3.txt licenses\regularlicense4.txt licenses\regularlicense5.txt licenses\regularlicense6.txt licenses\regularlicense7.txt licenses\regularlicense8.txt licenses\regularlicense9.txt licenses\regularlicense10.txt licenses\regularlicense11.txt licenses\regularlicense12.txt licenses\regularlicense13.txt licenses\regularlicense14.txt licenses\regularlicense15.txt licenses\regularlicense16.txt licenses\regularlicense17.txt licenses\regularlicense18.txt licenses\regularlicense19.txt licenses\regularlicense20.txt licenses\regularlicense21.txt licenses\regularlicense22.txt licenses\regularlicense23.txt licenses\regularlicense24.txt licenses\regularlicense25.txt licenses\regularlicense26.txt licenses\regularlicense27.txt licenses\regularlicense28.txt licenses\regularlicense29.txt licenses\regularlicense30.txt licenses\regularlicense31.txt licenses\regularlicense32.txt licenses\regularlicense34.txt licenses\regularlicense35.txt licenses\regularlicense36.txt licenses\regularlicense37.txt licenses\regularlicense38.txt licenses\regularlicense39.txt licenses\regularlicense40.txt licenses\regularlicense41.txt licenses\regularlicense42.txt licenses\regularlicense43.txt licenses\regularlicense44.txt licenses\invalidlicense.txt licenses\invalidlicense1.txt licenses\invalidlicense2.txt licenses\invalidlicense3.txt licenses\invalidlicense4.txt licenses\invalidlicense5.txt licenses\invalidlicense6.txt licenses\invalidlicense7.txt licenses\invalidlicense8.txt licenses\invalidlicense9.txt licenses\invalidlicense10.txt licenses\invalidlicense11.txt licenses\invalidlicense12.txt licenses\invalidlicense13.txt licenses\invalidlicense14.txt licenses\invalidlicense15.txt licenses\invalidlicense16.txt licenses\invalidlicense17.txt licenses\perflicense1.txt licenses\perflicense1backup.txt licenses\perflicense2.txt licenses\perflicense3.txt licenses\perflicense4.txt licenses\perflicense5.txt licenses\minlicense1.txt licenses\maxlicense1.txt |

6.2.3 Secure store

| Folder | test\secstore |
| --- | --- |
| Test code | Testsecstore.c |
| Target Component | Drmcore.lib |
| Feature testing | - Get and set secure data - Open and close key token, - Get and set token value - Delete key - Get all data |
| Test Script File | Testsecstore.pc.pk.ce.txt |
| Test Data Files | ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat |

## 6.3 Encryption and Decryption

This test area consists of testing on various encryption and decryption technology. The API functions are from Drmcrypto.lib, drmbignum.lib and drmpkcrypto.lib

6.3.1 Crypto

| Folder | test\crypto |
| --- | --- |
| Test code | Cryptotest.c |
| Target Component | drmbignum.lib drmcrypto.lib drmpkcrypto.lib |
| Feature testing | - PK encryption and decryption - PK sign verification - Get random data - Cipher encryption and decryption - HMAC test - DES key test - SHA test - CBC64 and invert CBC64 tests - RC4 test |
| Test Script File | Crypto.pc.pk.ce.txt |
| Test Data Files | N/A |

## 6.4 Utilities

This test area covers XML builder, XML parser, DRM CRT functions and DRM base64 encode and decode functions. These are utility functions that are widely used across all DRM components.

6.4.1 XML builder

| Folder | test\xmlbuilder |
| --- | --- |
| Test code | Testxmlbuilder.c |
| Target Component | Drmcore.lib |
| Feature testing | - Create document, reallocate document, close document - Open node, close current node - Get node name - Get context size - Add data, add CData, add attribute - Append a node |
| Test Script File | dailytestxmlbuilder.pc.pk.ce.txt |
| Test Data Files | N/A |

6.4.2 XML parser

| Folder | test\xmlparser |
| --- | --- |
| Test code | Testxmlparser.c |
| Target Component | Drmcore.lib |
| Feature testing | - Get XML Node, - get XML subnode - Get node attribute - Get node CData |
| Test Script File | dailytestxmlparser.pc.pk.ce.txt |
| Test Data Files | XML\XMLLicense1.txt XML\XMLLicense2.txt XML\XMLLicense3.txt XML\XMLLicense4.txt XML\XMLLicense5.txt XML\XMLLicense6.txt XML\XMLLicense7.txt XML\XMLLicense8.txt XML\XMLLicense9.txt XML\XMLLicense10.txt XML\XMLLicense11.txt XML\XMLLicense12.txt XML\XMLLicense13.txt XML\XMLLicense14.txt XML\XMLLicense15.txt XML\XMLLicense16.txt XML\XMLLicense17.txt XML\XMLLicense18.txt XML\XMLLicense19.txt |

6.4.3 Utils

| Folder | test\utils |
| --- | --- |
| Test code | drmutiltest.c |
| Target Component | Drmcore.lib |
| Feature testing | - DRM CRT functions - DRM base64 encode - DRM base64 decode |
| Test Script File | utiltest.pc.pk.ce.txt utiltest.pc.pk.txt utiltest.ce.txt |
| Test Data Files | N/A |

## 6.5 Evaluation

This subgroup consists of parser and evaluator to examine the correctness of a targeted content, header, device certificate, license or a portion of expression within a license.

6.5.1 Devcert parser

| Folder | \test\devcertparser |
| --- | --- |
| Test code | dcParser.c |
| Target Component | Drmdevcert.lib |
| Feature testing | - Get dev cert - Verify basic value - Advanced dev cert verification |
| Test Script File | dcparser.pc.pk.ce.txt dcparser.pc.pk.txt |
| Test Data Files | ..\..\samples\devcert.dat |

6.5.2 License parser

| Folder | test\licparser |
| --- | --- |
| Test code | licpsr.c |
| Target Component | Drmcore.lib |
| Feature testing | - Get license enablingbits - Get license event - Get license content revocation - Verify content revocation - Verify signature - Verify cert chain - Get license attribute |
| Test Script File | licpsr.pc.pk.ce.txt licpsr.pc.txt |
| Test Data Files | ..\common\testcerts.txt |

6.5.3 Expression Evaluator

| Folder | test\expreval |
| --- | --- |
| Test code | Expreval.c, expreval.h, licenses.h |
| Target Component | Drmcore.lib |
| Feature testing | - Evaluate drm license expression - Evaluate license expression context setting |
| Test Script File | ExprEval.pc.pk.ce.txt |
| Test Data Files | ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat |

6.5.4 License evaluation

| Folder | test\liceval |
| --- | --- |
| Test code | liceval.c, liceval.h, and license.h |
| Target Component | Drmcore.lib |
| Feature testing | - Evaluate license performance operation - Evaluate license evaluation context setting |
| Test Script File | liceval.pc.pk.ce.txt liceval.pc.txt |
| Test Data Files | ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat |

6.5.5 Header

| Folder | test\header |
| --- | --- |
| Test code | Header.c |
| Target Component | Drmcore.lib |
| Feature testing | - Verify header - Get attribute from header - Get uplink from header |
| Test Script File | header.pc.pk.ce.txt |
| Test Data Files | ModDataHeader.xml ModSigHeader.xml NoKIDHeader.xml NonXMLHeader.xml NoVersionHeader.xml validHeader.xml uplinkHeader.xml twouplinks.xml |

## 6.6 Advanced Features

The following advanced features can be opted by selecting appropriate compile flag. See source\makefile.msc for details.

6.6.1 Device devcert

| Folder | test\devicedevcert |
| --- | --- |
| Test code | Ddctest.c |
| Target Component | Drmdevicedevcert.lib |
| Feature testing | - Create a devcert locally, using devcert template. - Get devcert |
| Test Script File | ddctest.pc.pk.ce.txt ddctest.pc.pk.txt |
| Test Data Files | ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat priv_bad.dat dt_oversize.dat dt_badsec.dat dt_badsign.dat |

6.6.2 Secure clock

| Folder | test\SecureClock |
| --- | --- |
| Test code | Secclock.c |
| Target Component | Drmsecureclock.lib |
| Feature testing | - Create secure clock challenge - Process secure clock response - Get secure clock |
| Test Script File | secclock.pc.pk.ce.txt |
| Test Data Files | ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat secClockResponse.txt |

6.6.3 License chaining

License chaining test is covered within drm manager test component. See 6.1.1.drmmanager test area.

6.6.4 Metering

| Folder | test\Metering |
| --- | --- |
| Test code | Metering.c |
| Target Component | Drmmetering.lib |
| Feature testing | - Create metering challenge - Process metering response |
| Test Script File | metering.pc.pk.ce.txt metering.pc.pk.txt |
| Test Data Files | ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat files\metercert.xml files\metercert2.xml files\badmidmetercert.xml files\badpubkeymetercert.xml files\badmetercert.xml files\badsigmetercert.xml files\badmidmeterresponse.xml files\badtidmeterresponse.xml files\badrecordmeterresponse.xml files\badvaluemeterresponse.xml files\badcmdmeterresponse.xml |

6.6.5 Synclist

| Folder | test\Synclist |
| --- | --- |
| Test code | Testsynclist.c |
| Target Component | Drmsynclist.lib |
| Feature testing | - Create synclist challenge |
| Test Script File | Testsynclist.pc.pk.ce.txt |
| Test Data Files | ..\common\testcerts.txt ..\..\samples\devcerttemplate.dat ..\..\samples\priv.dat Header1.xml |

# 7. Appendix

## 7.1 Files in WMDRM Test Tool Kit

The following table lists all the files Test folder of the setup. It is assumed that Test folder as the current folder.

| Item | Source location and name |
| --- | --- |
| ANSI makefile | .\pktestmk.cmn .\pktestmk.msc |
| Blackbox | .\blackbox\binding.c .\blackbox\blackboxtest.c .\blackbox\stdafx.h .\blackbox\refbb.pc.pk.ce.txt .\backbox\filedep.txt |
| Common | .\common\testcerts.txt |
| Crypto | .\crypto\cryptotest.c .\crypto\crypto.pc.pk.ce.txt .\crypto\filedep.txt |
| DevcertParser | .\devcertparser\dcparser.c .\devcertparser\dcparser.pc.pk.ce.txt .\devcertparser\dcparser.pc.pk.txt .\devcertparser\filedep.txt |
| DeviceDevcert | .\devicedevcert\ddctest.c .\devicedevcert\ddctest.pc.pk.ce.txt .\devicedevcert\ddctest.pc.pk.txt .\devicedevcert\filedep.txt .\devicedevcert\dt_badsec.dat .\devicedevcert\dt_badsign.dat .\devicedevcert\dt_oversize.dat .\devicedevcert\priv_bad.dat |
| Doc | .\doc\WMDRMPD_TestToolkit.doc .\doc\readme.txt |
| DrmManager | .\drmmanager\testdrmmanager.c .\ drmmanager\dlrb.c .\drmmanager\stdafx.h .\drmmanager\dailytestdrmmanager.pc.pk.ce.txt .\drmmanager\dailytestdrmmanager.pk.ce.txt .\drmmanager\dailytestdrmmanager.pk.pm.txt .\drmmanager\filedep.txt .\drmmanager\files\header1.xml .\drmmanager\files\header2.xml .\drmmanager\files\header_longkid.xml .\drmmanager\files\header_longkid64.xml .\drmmanager\files\header_shortkid.xml .\drmmanager\files\header_shortkid64.xml |
| ExprEval | .\expreval\expreval..c .\expreval\expreval..h .\expreval\licenses.h .\expreval\expreval.pc.pk.ce.txt .\expreval\filedep.txt |
| Harness | .\harness\PKtestareas.txt .\harness\runWMDRMPDPKTest.vbs .\harness\SendWMDRMPDPKTestResult.vbs .\harness\runit.bat |
| HDS | .\hds\testhds.c .\hds\dailytesthds.pc.pk.ce.txt .\hds\negativetesthds.pc.pk.ce.txt .\hds\invalidargumenttesthds.pc.pk.ce.txt .\hds\filedep.txt |
| Header | .\header\header.c .\header\header.pc.pk.ce.txt .\header\filedep.txt .\header\ModDataHeader.xml .\header\ModSigHeader.xml .\header\NoKIDHeader.xml .\header\NonXMLHeader.xml .\header\NoVersionHeader.xml .\header\validHeader.xml .\header\uplinkHeader.xml .\header\twouplinks.xml |
| Inc | .\inc\drmchkerr.h .\inc\tclib.h .\inc\teststubcerts.h .\inc\tgetclientID.h .\inc\tlicgen.h .\inc\tOEMImp.h .\inc\tResponsegen.h .\inc\tstutils.h |
| LicEval | .\liceval\liceval.c .\liceval\liceval.h .\liceval\licenses.h .\liceval\filedep.txt .\liceval\liceval.pc.pk.ce.txt .\liceval\liceval.pc.txt |
| LicParser | .\licparser\licpsr.c .\licparser\licpsr.pc.pk.ce.txt .\licparser\filedep.tc.txt |
| Licstore | .\licstore\testlicstore.c .\licstore\testlicstore.pc.pk.ce.txt .\licstore\testlicstore.pc.txt .\licstore\filedep.txt .\licstore\licenses\regularlicense1.txt .\licstore\licenses\regularlicense2.txt .\licstore\licenses\regularlicense3.txt .\licstore\licenses\regularlicense4.txt .\licstore\licenses\regularlicense5.txt .\licstore\licenses\regularlicense6.txt .\licstore\licenses\regularlicense7.txt .\licstore\licenses\regularlicense8.txt .\licstore\licenses\regularlicense9.txt .\licstore\licenses\regularlicense10.txt .\licstore\licenses\regularlicense11.txt .\licstore\licenses\regularlicense12.txt .\licstore\licenses\regularlicense13.txt .\licstore\licenses\regularlicense14.txt .\licstore\licenses\regularlicense15.txt .\licstore\licenses\regularlicense16.txt .\licstore\licenses\regularlicense17.txt .\licstore\licenses\regularlicense18.txt .\licstore\licenses\regularlicense19.txt .\licstore\licenses\regularlicense20.txt .\licstore\licenses\regularlicense21.txt .\licstore\licenses\regularlicense22.txt .\licstore\licenses\regularlicense23.txt .\licstore\licenses\regularlicense24.txt .\licstore\licenses\regularlicense25.txt .\licstore\licenses\regularlicense26.txt .\licstore\licenses\regularlicense27.txt .\licstore\licenses\regularlicense28.txt .\licstore\licenses\regularlicense29.txt .\licstore\licenses\regularlicense30.txt .\licstore\licenses\regularlicense31.txt .\licstore\licenses\regularlicense32.txt .\licstore\licenses\regularlicense34.txt .\licstore\licenses\regularlicense35.txt .\licstore\licenses\regularlicense36.txt .\licstore\licenses\regularlicense37.txt .\licstore\licenses\regularlicense38.txt .\licstore\licenses\regularlicense39.txt .\licstore\licenses\regularlicense40.txt .\licstore\licenses\regularlicense41.txt .\licstore\licenses\regularlicense42.txt .\licstore\licenses\regularlicense43.txt .\licstore\licenses\regularlicense44.txt .\licstore\licenses\invalidlicense.txt .\licstore\licenses\invalidlicense1.txt .\licstore\licenses\invalidlicense2.txt .\licstore\licenses\invalidlicense3.txt .\licstore\licenses\invalidlicense4.txt .\licstore\licenses\invalidlicense5.txt .\licstore\licenses\invalidlicense6.txt .\licstore\licenses\invalidlicense7.txt .\licstore\licenses\invalidlicense8.txt .\licstore\licenses\invalidlicense9.txt .\licstore\licenses\invalidlicense10.txt .\licstore\licenses\invalidlicense11.txt .\licstore\licenses\invalidlicense12.txt .\licstore\licenses\invalidlicense13.txt .\licstore\licenses\invalidlicense14.txt .\licstore\licenses\invalidlicense15.txt .\licstore\licenses\invalidlicense16.txt .\licstore\licenses\invalidlicense17.txt .\licstore\licenses\perflicense1.txt .\licstore\licenses\perflicense1backup.txt .\licstore\licenses\perflicense2.txt .\licstore\licenses\perflicense3.txt .\licstore\licenses\perflicense4.txt .\licstore\licenses\perflicense5.txt .\licstore\licenses\minlicense1.txt .\licstore\licenses\maxlicense1.txt |
| Metering | .\metering\metering.c .\metering\metering.pc.pk.ce.txt .\metering\metering.pc.pk.txt .\metering\filedep.tc.txt .\metering\files\metercert.xml .\metering\files\metercert2.xml .\metering\files\badmidmetercert.xml .\metering\files\badpubkeymetercert.xml .\metering\files\badmetercert.xml .\metering\files\badsigmetercert.xml .\metering\files\badmidmeterresponse.xml .\metering\files\badtidmeterresponse.xml .\metering\files\badrecordmeterresponse.xml .\metering\files\badvaluemeterresponse.xml .\metering\files\badcmdmeterresponse.xml |
| Reftclib | .\reftclib\apiparams.c .\reftclib\apiparams.h .\reftclib\logger.c .\reftclib\logger.h .\reftclib\tclib.c |
| Secstore | .\secstore\testsecstore.c .\secstore\testsecstore.pc.pk.ce.txt .\secstore\filedep.txt |
| SecureClock | .\secureclock\secclock.c .\secureclock\secclock.pc.pk.ce.txt .\secureclock\filedep.txt .\secureclock\secclockresponse.txt |
| Synclist | .\synclist\testsynclist.c .\synclist\testsynclist.pc,pk.ce.txt .\synclist\filedep.txt .\synclist\header1.xml |
| tOEMImp | .\toemimp\toemimp.c .\toemimp\doemimp.c |
| tstUtils | .\tstutils\getclientid\tgetclientid.c .\tstutils\licgen\setclientid.c .\tstutils\licgen\setclientid.h .\tstutils\licgen\tlicgen.c .\tstutils\licgen\tresponsegen.c .\tstutils\utils\stubcert.c .\tstutils\utils\tstutils.c |
| Utils | .\utils\drmutiltest.c .\utils\utiltest.pc.pk.ce.txt .\utils\utiltest.pc.pk.txt .\utils\utiltest.ce.txt .\utils\filedep.txt |
| ViewRights | .\viewrights\viewrights.c .\viewrights\viewrights.pc.pk.ce.txt .\viewrights\filedep.txt |
| XMLBuilder | .\xmlbuilder\testxmlbuilder.c .\xmlbuilder\dailytestxmlbuilder.pc.pk.ce.txt .\xmlbuilder\filedep.txt |
| XMLParser | .\xmlparser\testxmlparser.c .\xmlparser\dailyxmlparser.pc.pk.ce.txt .\xmlparser\filedep.txt .\xmlparser\XML\XMLLicense1.txt .\xmlparser\XML\XMLLicense2.txt .\xmlparser\XML\XMLLicense3.txt .\xmlparser\XML\XMLLicense4.txt .\xmlparser\XML\XMLLicense5.txt .\xmlparser\XML\XMLLicense6.txt .\xmlparser\XML\XMLLicense7.txt .\xmlparser\XML\XMLLicense8.txt .\xmlparser\XML\XMLLicense9.txt .\xmlparser\XML\XMLLicense10.txt .\xmlparser\XML\XMLLicense11.txt .\xmlparser\XML\XMLLicense12.txt .\xmlparser\XML\XMLLicense13.txt .\xmlparser\XML\XMLLicense14.txt .\xmlparser\XML\XMLLicense15.txt .\xmlparser\XML\XMLLicense16.txt .\xmlparser\XML\XMLLicense17.txt .\xmlparser\XML\XMLLicense18.txt .\xmlparser\XML\XMLLicense19.txt |

## 7.2 Sample Log Result

The following is the sample log file from blackbox BVT test.

<LOGSUMMARY>

<LOGRESULTS CaseNo="9670" Result="PASS">

<Title>GetClientID should return the proper client info</Title>

</LOGRESULTS>

<LOGRESULTS CaseNo="9677" Result="PASS">

<Title>CanBind should return DRM_SUCCESS for valid binding info</Title>

</LOGRESULTS>

<LOGRESULTS CaseNo="9685" Result="PASS">

<Title>DecryptLicense should decrypt license encrypted with the blackbox public key</Title>

</LOGRESULTS>

<LOGRESULTS CaseNo="9687" Result="PASS">

<Title>DRMBlackBox_Decrypt should be able to decrypt data encrypted by DRMBlackBox_Encrypt</Title>

</LOGRESULTS>

</LOGSUMMARY>

## 7.3. Sample BVT Test Report

| Test Machine | DRM Test Machine |
| --- | --- |
| header | Pass(7:7:0:0) |
| XMLParser | Pass(4:4:0:0) |
| Secstore | Pass(4:4:0:0) |
| XMLBuilder | Pass(13:13:0:0) |
| LicEval | Pass(12:12:0:0) |
| Utils | Pass(11:11:0:0) |
| ExprEval | Pass(32:32:0:0) |
| licstore | Pass(8:8:0:0) |
| hds | Pass(41:41:0:0) |
| blackbox | Pass(4:4:0:0) |
| Crypto | Pass(10:10:0:0) |
| licparser | Pass(11:11:0:0) |
| drmmanager | Pass(22:22:0:0) |
| synclist | Pass(11:11:0:0) |
| ViewRights | Pass(3:3:0:0) |
| DevcertParser | Pass(2:2:0:0) |
| SecureClock | Pass(6:6:0:0) |
| devicedevcert | Pass(2:2:0:0) |
| Metering | Pass(11:11:0:0) |