Guide to the internals of the Hack-o-Rocket IP router

The router is implemented as a very ad hoc miniature OS kernel.  The code runs

directly from flash and most of the RAM is used for packet buffers.  The

"OS kernel" implemented here is *very* ad hoc and all functionality that runs

under it (including the counterpart of "user processes") is compiled into the

image at build time.  The resulting system is quite rugged, but has virtually

no generality or extensibility.  The intent was not to create a new router OS

platform for the future (there are plenty of those already), but to make some

use out of the only hardware platform we have available today for SDSL hacking,

however suboptimal.

Not counting one-time initialization code, the program consists of "ground

level code", i.e., code that runs at IPL 0 when not interrupted, and a number

of interrupt handlers.  The ground level code consists of a single main loop

which repeatedly calls a fixed set of routines.  Each of these routines does

its job and returns, and so the main loop runs forever.  The routines called

from the main loop are thus effectively "threads" or "processes" running under

our "router OS kernel".  They are not scheduled preemptively, but cooperate by

way of each routine limiting the amount of work it does on one iteration.  Here

is the complete list of these "processes":

* (SDSL versions only) _BtMain(), the bitpump control process mostly unchanged

from Conexant/Mindspeed.  It monitors the bitpump, detects LOS on/off

conditions, runs the bitpump activation sequence and performs temperature/

environment handling for long uptimes.  Functionally this process is a slave

to the next one.

* (SDSL versions only) SDSL activation state machine (sdsl_asm()).  This process

controls the SDSL link at the high level, commanding the lower level bitpump

control process to activate or deactivate.  The ASM is based on the one given

in HDSL standards and in Brooktree's example implementation, adapted for the

SDSL flavors of interest to us.  For Flavor B this ASM looks only at the

bitpump state (i.e., is decoupled from the higher level HDLC interface), but

in the Nokia version the ASM interacts with the interrupt-based code which

uses the SCC to receive and transmit Nokia frames.

* (All versions) Ethernet ARP process (arp_inprocess()).  This process drains

the ARP packet queue described later in this document and processes all

incoming ARP packets, storing information about other nodes and responding to

ARP requests about us.

* (All versions) IP core process (ip_inprocess()).  This process drains the IP

packet queue described later in this document and processes all incoming IP

datagrams to the final disposition of each.

* (Nokia SDSL version only) "Packets to frames" process (p2f_process()).  This

process drains the queue of ready-to-transmit AAL5 packets prepared by the

ATM WAN interface (described later in this document) and turns them into

ready-to-transmit Nokia frames going onto another queue.

* (All versions) Craft terminal process (craft_terminal_process()).  This one

is self-explanatory; its functionality is described in router.doc.

* (Nokia SDSL + EOC dump version only) EOC dump process (eocdump_process()):

specialized debug feature for those interested in further reverse engineering

of the Nokia EOC messages.

The process notion is further reinforced by the fact that as one can see from

the list above, they perform mostly independent functions.

The following interrupt handlers are implemented:

* MC145572 interrupt handler (IDSL only): this interrupt handler comprises the

totality of the physical layer management code for IDSL.  The ISDN chip fully

takes care of itself and the interrupt handler merely allows the user to see

it by printing short messages on the console when certain events occur.

* Bitpump interrupt handler (SDSL only): unchanged from Brooktree, really

minimal, resides completely in libzw.

* Ethernet interrupt handler.  Rx side: reads received Ethernet packets out of

the chip, puts IP and ARP ones on the respective packet queues described

below, discards the rest.  Tx side: doesn't do much, it's mostly PIO.

* SCC1 interrupt handler (HDLC).  Rx side: accepts received WAN packets and

places them on the IP input queue after verifying the encapsulation.

Tx side: frees buffers after they are transmitted by the SCC.

* SCC1 interrupt handler (Nokia ATM): very tricky, described later.

* Timer interrupt handler: simply increments the time base counter.

Packet buffers and queues

This is a central aspect of the router's design.  See buffer.h and bufmgt.c in

the router source code.  Each packet processed by the router is stored in a

packet buffer structure defined in buffer.h.  This structure holds a complete

packet of up to 1500 octets (Ethernet MTU) and a number of housekeeping fields.

There is a fixed total number of packet buffers allocated in RAM and on

initialization they are strung into a free list.  The Ethernet and ATM WAN

interfaces pull a buffer from the free list when one is needed for a received

packet; if the free list is empty, the received packet is discarded.  (The HDLC

WAN interface is similar, but a little more complicated: up to 8 packet buffers

are taken off the free list and posted on the RxBD queue for the SCC in

advance.)  Once captured in a buffer by a receive interrupt handler, the packet

goes onto one of the software packet queues drained by ground level code:

ip_queue or arp_queue.  These queues are unlimited, i.e., the only limit to

their growth if they aren't drained fast enough is receive interrupt handlers

getting no free buffers.

As each packet buffer is drained from ip_queue or arp_queue by ground level

code, it's processed in one fell swoop and disposed of.  The final disposition

may be freeing the buffer in the case of an error or some other condition in

which the packet can't or shouldn't be forwarded, but in the normal case of

packet forwarding (ostensibly the router's main function) the packet buffer

ends up on the output queue for one of the interfaces.

The router never originates any packets completely on its own.  ICMP datagrams

which appear to originate in the router (ping responses and error messages) are

always generated in response to some incoming packet, and are actually made by

replacing the original packet in its buffer.  The same holds for ARP: a

generated ARP reply replaces the corresponding request in its buffer, and when

an IP datagram needs to be sent out on the Ethernet interface but the MAC

address is unknown, the generated ARP request replaces the IP packet in its

buffer.  The ONLY place where new packet buffers are allocated is in the

receive interrupt handlers.

This packet buffer design has the following implications:

* The MTU on the WAN interface (HDLC or AAL5) is assumed and required to be

1500 octets, the same as Ethernet. (The relevant standards don't prescribe

any particular MTU, leaving it to unwritten conventions in practice.) Having

the same MTU on both interfaces, the router expects to pass packets or

fragments straight through without worrying about IP fragmentation/reassembly.

(The MTU definition counts all octets which are part of an IP packet, but not

the local headers and trailers which vary between different networks.)

* If we wanted to implement IP fragmentation and reassembly (e.g., for GRE), we

would need to solve the following problems:

- The current packet buffer system is designed on the principle that

1 packet = 1 buffer, no analogue to BSD mbuf chains.

- Fragmentation would involve allocating extra buffers in the IP ground level

process, a departure from our current design.

- Reassembly of fragmented IP datagrams addressed to us (e.g., for stripping

GRE) requires some place to hold the fragments as they arrive. Our current

packet buffer and queue architecture provides no such place.

* Suppose we receive an IP packet on the Ethernet interface which we route via

another gateway also on the Ethernet side. The proper thing to do in this

case is to send the originator an ICMP redirect, but still forward the packet

back on the Ethernet interface. This action implies turning one packet buffer

into two, hence we can't do that, we can only do one or the other.

* Consider the case of an IP datagram going out on the Ethernet interface and

turning into an ARP request because we don't have the MAC address for the

destination node. A robust implementation would hold the original IP packet

off on the expectation that an ARP reply would normally come very quickly, and

then send the original packet out without loss. We don't do that, instead the

ARP request overwrites the original IP packet in its buffer, so the latter is

lost. We rely on IP's license to lose packets occasionally and on higher

layer retransmissions in this case.

Where packet buffers can be found

At any moment in time each packet buffer can be found in one of the following

places:

* The free list.

* One of the software-constructed packet queues:

- ip_queue

- arp_queue

- (Nokia SDSL version only) atm_wan_outgoing_queue

* Held by the Ethernet interface code waiting for the chip's "ready for Tx"

interrupt (takes the place of the Ethernet interface Tx queue, see below).

* (HDLC versions only): one of the IMP-architectured SCC BD queues:

- The RxBD queue holds packet buffers which are either waiting to be filled

by the SCC or have been filled and are waiting to be picked up by the

interrupt handler.

- The TxBD queue holds packet buffers which are either waiting to be

transmitted by the SCC or have been transmitted and are waiting to be freed

by the interrupt handler.

* Nokia SDSL version:

- One packet buffer may be held by the ATM WAN Rx code as it gathers an AAL5

packet from incoming cells.

- One packet buffer may be held by the "packets to frames" process as ATM

cells are moved from the AAL5 packet into Nokia frames being prepared for

transmission.

* In transit: if a packet buffer isn't held in one of the places listed above,

it's being worked on by the currently executing code. It'll be placed

somewhere more permanent before moving to another packet buffer.

Outgoing packet disposition

The IP and ARP processes which drain the respective packet queues are

responsible for the final disposition of each packet buffer.  In other words,

once ip_inprocess() or arp_inprocess() takes a buffer off the queue, it must do

*all* necessary processing and dispose of it before taking another packet buffer

or returning to let other ground level code run.  Unless it culminates with the

buffer being freed (packet discarded), this processing culminates with the

buffer being passed to either ethernet_sendout() or send_to_wan().  The latter

routines are responsible for disposing of the packet buffer right there and

then, either by transmitting it immediately or putting it on an interface Tx

queue.

Ethernet interface

The Rx side of the CS8900 Ethernet interface is fairly decent in that it

implements a kind of Rx packet queue inside the chip.  However, there is no DMA

(CS8900's ISA DMA support is totally unsuited for the M68K architecture) and

the received packets waiting to be read by the CPU are held inside the actual

chip; as one would expect, the memory space there is very limited.  The packet

data are read and the on-chip buffer space is freed by PIO, this is done in the

Ethernet interrupt handler.  The Ethernet interrupt having the highest priority

in the hardware design helps make this work OK.

The Tx side is more lousy.  The CS8900 chip doesn't implement a sensible Tx

queue (one that would be filled by the software and drained by the hardware),

instead it seems to have been designed with the assumption that the host will

only ever want to send an occasional packet once in a while.  The CPU must

request packet transmission one at a time (no continuous back-to-back

transmission) and the mechanism is again purely PIO.  (It's actually even worse

than for reception in terms of the interrupt structure, read CS8900

documentation if you want the full ugly details.)

Our current Ethernet Tx implementation is as follows: when ethernet_sendout()

is called on an idle system, it goes straight to the CS8900 registers and does

what's called "bid for transmit" in CS8900 parlance.  If the CS8900 is ready

right then, the PIO transfer of the data to the chip happens at the same time

as well, freeing the packet buffer and effectively having it go out without the

interrupt handler ever being involved.  If the CS8900 isn't ready to accept the

data immediately after the "bid for transmit", the packet buffer is held off to

be pushed out by the "ready for Tx" interrupt handler.  In either case once the

packet data are pushed into the chip for transmission, a flag is set indicating

that the Ethernet transmitter is busy, to be cleared by the "Tx done" interrupt

handler.  While this flag is set, ethernet_sendout() discards all subsequent

packets.  In short, our Ethernet Tx code currently implements immediate

transmission only instead of queueing.

It is expected that the current design probably causes significant unnecessary

packet loss; the obvious direction for improvement would be to implement a Tx

packet queue as a pure software construct since the hardware doesn't provide a

real one.

Ethernet ARP implementation

The reason why the HoR router can't be connected to Ethernet segments wider

than /24 is due to our current simplistic ARP implementation.  Our ARP table is

not hashed, instead it has a fixed size of 256 entries and is indexed directly

by the least significant octet of the IP address.  Each entry consists of a

6-byte MAC address (all zeros if unknown) and a timestamp.

Given that successful ARP resolution and correct current information in the ARP

tables on both sides are a prerequisite for successful IP communication between

nodes on an Ethernet segment, it is reasoned that ARP should be given a higher

priority than IP.  The current code implements this "ARP priority" idea by

having arp_inprocess() drain the entire arp_queue before returning whereas

ip_inprocess() drains only one packet from ip_queue per invokation, and having

the main loop call arp_inprocess() right before ip_inprocess().  However, the

current lack of a real Tx queue in the Ethernet interface makes it unclear

whether our "ARP priority" scheme actually achieves anything or not.

HDLC WAN interface

Thanks to the quality of Motorola's SCC design, this is the cleanest of all our

network interfaces.

The HDLC WAN interface is present in the IDSL and SDSL Flavor B versions of the

router and it's treated as being logically independent of the DSL PHY

underneath.  The Tx side of the SCC and of the logical WAN interface is always

running regardless of whether the resulting bit stream is going into the 2B1Q

transmitter or into the rathole.  On the IDSL version the Rx side is also always

on regardless of the link state; the MC145572 automatically substitutes all 1s

for the 2B+D payload when the link is down.  On the SDSL Flavor B version the

SDSL ASM turns HDLC Rx on and off; the latter is the only unidirectional tie

between the SDSL PHY and HDLC WAN layers.

The SCC RxBD queue is prefilled with empty packet buffers in the following ways:

* 8 buffers are posted on it during initialization;

* Whenever someone calls free_buffer() to free a packet buffer and there are

fewer than 8 buffers posted for HDLC Rx, the newly freed buffer is posted

there instead of the free list;

* The HDLC interrupt handler tries to fill the RxBD queue with empty buffers

from the free list after taking the ready ones off.

The SCC TxBD queue double-functions as the software Tx queue for the WAN

interface, i.e., this is the queue that's expected to stay full when traffic in

the Ethernet->DSL direction exceeds DSL bandwidth.

The HDLC WAN interface implements RFC 1490 encapsulation only; each packet is

expected to have a 4-octet header before IP.  The interface/encapsulation

implementation is completely stateless, i.e., no LMI of any kind and no PPP with

its LCP & IPCP.

Nokia SDSL implementation

Our Nokia SDSL implementation consists of the following components:

* SCC-based Nokia frame receiver

* SCC-based Nokia frame transmitter

* ATM WAN interface (pure software construct)

* "Packets to frames" process described earlier

Transmitting Nokia frames with the SCC

Although the conventional network interface design practice is that the receiver

is given a higher priority than the transmitter (such is Motorola's CP design,

for example), in the case of software implementation of telecom-style framers

the opposite holds.  In our case the frame transmitter is most critical: the Tx

frame stream must always continue uninterrupted or else the SDSL ASM on the

other end will tear the link down.

We implement transmission of 432-octet Nokia frames with the SCC as follows.

The TxBD queue is set up with only 2 TxBDs instead of 8 and the SCC Tx interrupt

handler is responsible for fetching the next frame for transmission as each

preceding frame is transmitted.  The following 2 types of frames can be

transmitted:

* Idle frames consisting of 8 ATM idle cells: a complete idle frame is hard-

coded in the flash image.

* Non-idle frames prepared by the "packets to frames" process.

A special set of "prepared frame" buffers (distinct from the packet buffers)

exists with its own free list and queue; the "packets to frames" process takes

these buffers off the free list, fills them and puts them on the queue; the SCC

Tx interrupt handler takes them off the queue.

The buffer address linked to each of the 2 TxBDs can point either to the hard-

coded idle frame in flash or to a more interesting frame in RAM.  The SCC Tx

interrupt handler implements the following algorithm as it turns a TxBD from

not-ready to ready:

* Extract the old buffer address; if it isn't the hard-coded idle frame, free

the corresponding prepared frame buffer.

* Take a buffer from the "prepared frame" buffer queue; if the queue is empty,

use the hard-coded idle frame instead.

* Stick the new buffer address in the TxBD and set it back to ready.

An SCC Tx underrun is a fatal error and causes the SDSL ASM to deactivate

immediately and hopefully recover on the next startup cycle.

Receiving Nokia frames with the SCC

The SCC receiver uses all 8 RxBDs permanently pointed to 8 dedicated buffers.

The SCC Rx interrupt handler performs the following functions:

* Extracts ATM cells out of received frames and passes them to the ATM layer;

* Monitors Rx frame sync and reports LOSW and SCC overrun conditions to the

SDSL ASM.

The SCC hardware achieves sync only once; software (the Rx interrupt handler

just described) is responsible for checking that we remain in sync.  If frame

sync is lost, the robust approach would be to tell the SCC to rehunt and

deactivate the SDSL link only after reasonable efforts to reacquire sync fail.

However, experiment has showed that our hardware platform can't do this: if we

command the SCC receiver to rehunt, the SCC transmitter incurs an underrun which

is even more fatal.  (Evidently the priority scheme in the CP microcode which

gives commands and the receiver higher priority than the transmitter is ill-

suited for our application as explained earlier.)  Therefore, we treat the loss

of sync as unrecoverable, declare LOSW and have the SDSL ASM proceed to

teardown; the only recovery occurs on the next startup cycle.

The Nokia SCC code is coupled fairly tightly to the SDSL ASM and is logically

considered part of the SDSL layer rather than the ATM WAN interface.  The SCC

is completely reinitialized for this peculiar mode of operation on each startup

cycle by the ASM.

ATM WAN interface

The ATM WAN interface is a pure software construct.

The ATM cell Rx entry point is called from the SCC Rx interrupt handler and thus

the Rx side of the ATM WAN interface code runs at SCC interrupt time.  This code

implements the AAL5 packet reassembly state machine based on the last bit of

the PTI field in the ATM cell header.  It takes a packet buffer from the free

list, gathers the packet there, checks AAL5 trailer CRC-32 and encapsulation,

and places the buffer on ip_queue.  CRC-32 is computed into an accumulator as

each cell comes in, hence there is no large delay incurred at interrupt time on

the last cell of a packet.

The encapsulation must be routed IP in RFC 1483 with the LLC-SNAP header

present, i.e., the first 8 payload octets of the first ATM cell of an AAL5

packet are always AA AA 03 00 00 00 08 00.

The Tx side of the ATM WAN interface presents a send_to_wan() function to the

router core.  This function puts AAL5 dressing on the packet and sticks it onto

atm_wan_outgoing_queue, a pure software Tx queue for the interface.  This queue

has a configurable limit on its maximum length, once this limit is reached,

further packets sent to WAN will be dropped instead of lengthening the queue

past the limit.  This queue logically corresponds to the HDLC TxBD queue and is

expected to stay full when traffic in the Ethernet->SDSL direction exceeds SDSL

bandwidth.

This queue is then drained by the "packets to frames" process which prepares

Nokia frames for the SCC transmitter.

Statistics counters

Statistics counters are provided for most interesting events throughout the

stack.  Some of these events occur in ground level code, others occur in

interrupt handlers.

The statistics counters are incremented with simple stats.xxx++ statements

without any special interlocking exercises; nonetheless, this design is not

expected to cause any problems.  Most counters are incremented in only one

place, hence there is no way for the incrementing operation to be interrupted

by any other code touching the same counter.  At the present there is only one

counter (ip_rx_total) which is incremented in two different code paths running

at different IPLs: the Ethernet interrupt handler (IPL 6) and the WAN (SCC1)

interrupt handler (IPL 4).  However, no problems are expected even with this

counter because the incrementing operation is atomic: it has been verified that

gcc compiles all of our counter increments to single ADDQL instructions which

are atomic with respect to interrupts.

Packet trace and the printf() subsystem

As explained in the sw-arch document, access to the printf() subsystem needs to

be serialized and every program in the Hack-o-Rocket software suite needs to

choose the IPL at which it should run.  Because locking our network interface

interrupt handlers out during casual printfs would have been unacceptable, the

printf() subsystem runs at a low IPL in the HoR router.  Therefore, printf()

output is not possible from any interrupt handlers, only from ground level code.

(The only exception is the MC145572 interrupt handler on the IDSL version.  The

printf() subsystem runs at IPL 1 on the IDSL version and at IPL 0 on the SDSL

versions.)

The restriction of using printf() only in ground level code is the reason why

ARP, IP and ICMP are the only layers of the network stack instrumented by the

packet trace debug feature -- these are the layers implemented in ground level

code.  Interesting events occurring in interrupt handlers can only be reported

via statistics counters.

Enabling packet trace causes arp_inprocess() and ip_inprocess() to print short

messages on the console for every packet they process.  Regular printf() is

used which waits if all 4 printf buffers are full, hence packet trace can slow

the router's packet throughput considerably: effectively it becomes limited by

the serial console baud rate, which is normally 9600 baud!

In the absence of packet trace the most voluminous printf() output comes from

the SDSL ASM process and the underlying bitpump control process (in libzw) in

the SDSL versions of the router.  However, unlike packet trace this printf()

output doesn't cause any serious problems in normal operation: the bitpump

control process produces its most voluminous TDEBUG output during activation

proceedings when the SDSL link isn't up and running, therefore, even though the

printf() output incurs waits and starves other ground level processes (such as

ip_inprocess()) of CPU cycles, the latter functions aren't expected to be very

useful while SDSL is down.

Nokia EOC dump debug feature

The Nokia SDSL frame format includes one octet per frame for the EOC and the

messages carried there appear to be framed in octet-based HDLC a la RFC 1662.

When the channel is idle, the octet stays at 7E; valid messages are strings of

non-7E octets with an FCS at the end; one can presume that a 7D escape mechanism

is used like in PPP to allow 7D and 7E to occur in message payloads.  However,

the EOC octet can also be a constant FF if EOC isn't used at all (presumably

corresponding to a disabled transmitter output being "mark") and a robust

implementation should be able to accept any other filler in the absence of

valid messages.

If we knew the meaning of these messages and wanted to process them in some

sensible manner, we would have implemented a proper robust octet-based HDLC

receiver and we would have a reliable guard against non-7E garbage passing as

EOC messages: we would have imposed a limit on the message size and we would be

checking the FCS.  However, because we don't know the meaning of these messages

beyond their basic framing format and all we can do with them is to print them

on the console for pure adoration (or for further reverse engineering),

implementing a proper octet-based HDLC receiver for that would have been a

waste of effort.

Instead a shortcut has been implemented purely as a debug feature, enabled by

compiling the Nokia SDSL version of the HoR router with -DEOCDUMP.  The hack

works as follows: upon successful SDSL frame sync the EOC octet is searched for

a 7E.  Once a 7E is received, EOC is assumed to be active and the receiver is

enabled; all subsequent strings of non-7E octets (regardless of length) are

considered to be EOC messages and dumped on the console.  As there is no limit

imposed on the message length, there is no buffer to hold an entire message.

Instead a small circular buffer is used to pass the EOC bytes between the Nokia

SCC interrupt handler which receives them and the ground level process which

prints them, and the octets are printed as they are received modulo buffer

delay.  This mechanism is designed to print ALL strings of non-7E octets

received on an active EOC without judgment, yet avoid printing an " FF" on the

console every 9 ms when EOC is disabled and transmits a constant FF instead of

7E.  Needless to say, the heuristic is quite error-prone and shouldn't be

enabled except by those specifically interested in studying Nokia EOC.

The standard production version of the Nokia SDSL router (compiled without

-DEOCDUMP) simply ignores the EOC octet in all received Nokia frames; both

versions always transmit a constant FF in the outgoing frames.